ETSI/TC GSM

Title :  Recommendation GSM 01.04

Vocabulary In A GSM PLMN.

Version : 3.0.1

Date : February 1991

List of contents:

1.	GENERAL

2.	VOCABULARY.

3.	ABBREVIATIONS AND ACRONYMS.

4.	ALPHA SORTED INDEX.

Language of original : English

Number of pages :

TABLE OF CONTENTS

.Begin Table C.

1. GENERAL	3

1.1 Scope	3

1.2 Introduction	3

2. VOCABULARY	4

2.1 General	4

2.2 Services	4

2.3 Networks	11

2.4 Stations	14

2.5 Access	16

2.6 Location and handover	18

2.7 Identity and Security	21

2.8 Administrative and commercial definitions	24

2.9 Operations and maintenance	26

2.10 Expressions related to the radiosubsystem	30

2.11 Miscellaneous	36

3. ABBREVIATIONS AND ACRONYMS	37

4. ALPHA SORTED INDEX	45

.End Table C.

1. GENERAL

1.1 Scope

These are the terms, definitions and abbreviations to be used throughout the GSM recommendations.

The definition, or reference to a definition, in this recommendation 01.04 is valid in all recommendations where no definition is given within that recommendation. Where a definition, or a reference to a definition for the same term, is given in both this recommendation 01.04 and in another and the definitions are different, then the definition given in that recommendation takes priority in only that recommendation.

1.2 Introduction

This recommendation consists primarily of those terms and definitions that are considered essential to the understanding and application of the principles of a GSM PLMN.

A number of terms are followed by their usual abbreviation, in parenthesis. A listing by abbreviation is provided in chapter 3 of this recommendation.

A number of terms have a GSM recommendation number indicated after the term (e.g. 03.02). This indicates that reference should be made to that recommendation for the full definitions of the term. Where both a reference and a definition is given in this recommendation, the definitions given here embrace only the essential concepts and on that basis it is considered that they are not inconsistent with the more specialized definitions that appear in those other recommendations.

A number of the terms and definitions in this recommendation are identical to those found in CCITT Recommendation I.112, but they are applied to a GSM PLMN instead. The notes complementing CCITT I.112 Rec. definitions are not considered to be part of the definitions. When applicable they have been reproduced.

References to these definitions are given in parenthesis, for example (110), as an aid to ensuring consistency between the two recommendations in the event of amendments. Whenever necessary GSM notes have been added regarding the application to a GSM PLMN.

Any term in common usage but whose use is deprecated in the sense defined, is shown as in the following example "419 FUNCTIONAL GROUP [FUNCTIONAL GROUPING]".

Where a truncated term is widely used in an understood context the complete term is quoted following the colloquial form, for example, "201 SERVICE, TELECOMMUNICATION SERVICE".

Where  a reference is made to the Radio Regulations the definition has been taken verbatim from that source (except where indicated) even though there may be some aspects that are not applicable to GSM. The Radio Regulations are published by the International Telecommunications Union ITU.and have the effect of an international treaty. For this reason it is deemed inadvisable to alter or truncate such references simply to ensure complete relevance to GSM.

2. VOCABULARY

2.1 General

.t.	Telecommunication (110);

Any .i. 	transmission:; and/or emission and reception of signals representing signs, writing, images and sounds or intelligence of any nature by wire, radio, optical or other electromagnetic systems.

2.2 Services

.t.	Service, telecommunication service (201);

That which is offered by an administration or RPOA to its .i. 	customers :;in order to satisfy a specific telecommunication requirement.

.t.	Radiocommunication service;

A service as defined in this section involving the .i. 	transmission:;, emission and/or reception of radio waves for specific telecommunication purposes.

In these Regulations, unless otherwise stated, any radiocommunication service relates to terrestrial radiocommunication.  (Quoted from the Radio Regulations N° 20).

.t.	Mobile service;

A radiocommunication service between mobile and land stations, or between mobile stations.  (Quoted from the Radio Regulations N° 26).

.t. 	Land Mobile Service;

A mobile service between base stations and land mobile stations, or between land mobile stations.  (Quoted from the Radio Regulations N° 28).

.t.	Bearer service (202);

A type of telecommunication service that provides the capability for the transmission of signals between user─network .i. 	interfaces:;.

Note: The GSM PLMN connection type used to support a bearer service may be identical to that used to support other types of telecommunication service.

.t.	Teleservice (203);

A type of .i. 	telecommunication service:; that provides the complete capability, including terminal equipment functions, for communication between users according to .i. 	protocols :;established by agreement between administrations and/or RP0As.

.t.	Demand service, demand telecommunication service (205);

A type of telecommunication service in which the communication path is established almost immediately, in response to a user request effected by means of .i. 	user─network signalling:;.

.t.	Service attribute, telecommunication service attribute (208);

A specified characteristic of a telecommunication service.

.t.	Basic services;

The telecommunication services excluding the .i. 	supplementary services:; (in accordance with table 1/I.210).

.t. 	Supplementary service;

A modification of, or a supplement to, a basic telecommunication service.

.t.	Calling line identification presentation (CLIP); .a.CLIP	Calling line identification presentation :;

The .i. 	supplementary service :;which provides for the called party with the possibility to receive identification of the calling party.

.t.	Calling line identification restriction (CLIR);.a.CLIR	Calling line identification restriction :;

A .i. 	supplementary service :;offered to the calling party to prevent presentation of the .i.	calling party's:; .i. 	ISDN/MSISDN number :;to the .i. 	called party:;.

.t.	Connected line identification presentation (COLP);.a.COLP	Connected line identification presentation :;

The .i. 	supplementary service :;which provides the calling party with the possibility to receive the .i. 	identification :;of the connected party

.t.	Connected line identification restriction (COLR);.a.COLR	Connected line identification restriction :;

A .i. 	supplementary service :;offered to the connected party to prevent presentation of the connected party's .i. 	ISDN/MSISDN number :;to the .i. 	calling party:;.

.t.	Malicious call identification (MCI);.a.MCI	Malicious call identification :;

The .i. 	supplementary service :;which enables a mobile subscriber to request that the source of an .i. 	incoming call :;is identified and registered in the network.

.t.	Call forwarding unconditional (CFU);.a.CFU	Call forwarding unconditional :;

The .i. 	supplementary service :;which permits a called mobile subscriber to have the network send all .i. 	incoming calls:;, or just those associated with a specific .i. 	Basic service:;, addressed to the called mobile subscriber's .i. 	directory number :;to another directory number. The ability of the served mobile subscriber to originate calls is unaffected. If this service is activated, calls are forwarded no matter what the condition of the termination.

.t.	Call forwarding mobile subscriber busy (CFB);.a.CFB	Call forwarding mobile subscriber busy :;

The .i. 	supplementary service :;which permits a called mobile subscriber to have the network send all .i. 	incoming calls:;, or just those associated with a specific .i. 	Basic service:;, addressed to the called mobile subscriber's .i. 	directory number :;and which meet mobile subscriber busy to another directory number. The ability of the .i. 	served mobile subscriber :;to originate calls is unaffected. If this service is activated, a call is forwarded only if the call meets .i. 	mobile subscriber busy:;.

.t.	Call forwarding on no reply (CFNRy);

The .i. 	supplementary service :;which permits a called mobile subscriber to have the network send all .i. 	incoming calls:;, or just those associated with a specific .i. 	Basic service:;, addressed to the called mobile subscriber's directory number and which meet no reply to another directory number. The ability of the served mobile subscriber to .i. 	originate calls :;is unaffected. If this service is activated, a call is forwarded only if the call meets no reply.

.t.	Call forwarding on mobile subscriber not reachable (CFNRc);

The .i. 	supplementary service :;which permits a called mobile subscriber to have the network send all .i. 	incoming calls:;, or just those associated with a specific basic service, addressed to the called mobile subscriber's directory number, but which cannot be reached due to radio congestion, no paging response or because the subscriber is not registered, to another .i. 	directory number:;.

.t.	Call waiting (CW);

The .i. 	supplementary service :;which offers a mobile subscriber the possibility to be notified of an .i. 	incoming call :;whilst the termination is in the busy state. Subsequently, the subscriber can either answer, reject, or ignore the incoming call.

.t.	Completion of calls to busy subscribers (CCBS);

The .i. 	supplementary service :;which allows a calling mobile subscriber, encountering a .i. 	busy :;called destination to have the call completed when the busy destination becomes not busy,without having to make a new call attempt.

.t.	Closed user group (CUG);

The .i. 	supplementary service :;which provides the possibility for a .i. 	group :;of subscribers, connected to the PLMN and/or the ISDN, to intercommunicate only amongst themselves and, if required, one or more subscribers may be provided with incoming/outgoing access to subscribers outside this group.

.t.	Advice of charge (AoC);

The .i. 	supplementary service :;which provides the possibility for a mobile user who pays for the use of telecommunications services to receive .i. 	charging information :;related to the used .i. 	telecommunication services:;.

.t.	Call hold (HOLD);

The .i. 	supplementary service :;which allows a served mobile subscriber to .i. 	interrupt communication :;on an existing call and then subsequently, if desired, .i. 	re-establish communication:;. The traffic channel remains assigned to the mobile subscriber after the communication is interrupted to allow the origination or possible termination of other calls.

The ".i. 	retrieve:;" operation re─establishes communication on a channel.

.t.	Call transfer (CT);

The .i. 	supplementary service :;which enables the served mobile subscriber to .i. 	transfer :;an established incoming or outgoing call to a .i. 	third party:;. This service differs from the .i. 	Call Forwarding :;supplementary service in that Call Forwarding deals only with incoming calls that have not yet reached the "fully─established" state (i.e., have an established end─to─end connection).

.t.	Three party service (3PTY);

The .i. 	supplementary service :;which enables a mobile subscriber to establish a three party conversation. A mobile subscriber who is active on a call is able to hold that call, make an additional call to a third party, switch from one call to the other as required (privacy being provided between the two calls), and/or release one call and return to the other, or join the two calls together into a three─way conversation.

.t.	Conference call,add on (CONF);

The supplementary service which provides a mobile subscriber with the ability to have a multi─connection call, i.e. a simultaneous communication between more than two parties.

.t.	Barring of all outgoing calls(BAOC);

The supplementary service which makes it possible for a mobile subscriber to prevent all outgoing calls or just those associated with a specific Basic service.,  The ability of the served mobile subscriber to receive calls remains unaffected. The ability to set─up emergency calls remains unaffected .

.t.	Barring of outgoing international calls (BOIC);

The supplementary service which makes it possible for a mobile subscriber to prevent all outgoing international calls or just those associated with a specific basic service. Call set─up possibilities only exist to subscribers of the PLMN(s) and the fixed network(s) of the country where the mobile subscriber is currently located. The present PLMN may be the home PLMN or a visited PLMN respectively, the fixed network may be that of the home PLMN country or that of a visited PLMN country. The ability of the served mobile subscriber to receive calls remains unaffected. The ability to set─up emergency calls remains unaffected .

.t.	Barring of outgoing international calls except those directed to the home PLMN country (BOIC-exHC);

The supplementary service which makes it possible for a mobile subscriber to prevent all outgoing international calls except those directed to the home PLMN country,  or just those associated with a specific basic service. Call set─up possibilities only exist to subscribers of the PLMN(s) and the fixed network(s) of the country where the mobile subscriber is currently located or to subscribers of the home PLMN of the served mobile subscriber and to subscribers of the fixed network(s) in the home PLMN country. The present PLMN may be the home PLMN or a visited PLMN, respectively the fixed network may be that of the home PLMN country or that of a visited PLMN country. The ability of the served mobile subscriber to receive calls remains unaffected. The ability to set─up emergency calls remains unaffected.

.t.	Barring of all incoming calls (BAIC);

The supplementary service which makes it possible for a mobile subscriber to prevent all incoming calls or just those associated with a specific basic service The ability of the served mobile subscriber to originate calls remains unaffected.

.t.	Barring of incoming calls when roaming outside the home PLMN country (BIC-Roam);

This service makes it possible for a mobile subscriber to prevent incoming calls or just those associated with a specific basic service only if the mobile subscriber roams outside the home PLMN country. The ability for the served mobile subscriber to originate calls remains unaffected.

.t.	Mobile access hunting (MAH);

The supplementary service  which enables incoming calls to be distributed over a group of accesses .

.t.	User─to─user signalling (UUS);

The supplementary service which allows a mobile subscriber to send/receive a limited amount or information to/from another PLMN or ISDN subscriber over the signalling channel in association with a call to the other subscriber.

.t.	Provision;

An action to make a service available to a subscriber. The provision may be:

─	general: where the service is made available to all subscribers (subject to compatibility restrictions enforced) without prior arrangements being made with the service provider.

─	prearranged: where the service is made available to an individual subscriber only after the necessary arrangements have been made with the service provider.

.t.	Withdrawal;

An action taken by the service provider to remove an available service from a subscriber's access. The withdrawal may be:

─	general : where the service is removed from all subscribers provided with the service.

─	specific : where the service is removed on an individual basis from subscribers provided with the service.

.t.	Registration  ((supplementary) service registration);

The programming by the service provider or subscriber of information to enable subsequent operation of a service. The programming action involves input of specific supplementary information.

For certain services the registration procedure may cause activation whilst for others the service may already be in the active phase.

.t.	Erasure;

The deletion by the service provider, the subscriber or the system of information stored against a particular service by a previous registration(s).

.t.	Activation;

An action taken by either the service provider, the subscriber or the system to enable a process to run as and when required by the service concerned. The time during which the process is activated is defined as the active phase. During activation the service will be either "operative" or "quiescient" according to whether or not the system is actually using the service, e.g. to forward a call or to apply call waiting indication.

.t.	Deactivation;

An action taken by either the service provider, the subscriber or the system to terminate the process started at the activation.

.t.	Invocation;

An action to invoke the service required, taken by the subscriber (e.g. pressing a specific button) or automatically by the network or terminal as a result of a particular condition (e.g. calling line identification for each incoming call)

.t.	Normal operation with successful; outcome;

Description of the normal operation of the service, the normal served subscriber's actions and the system response. Decision points, timing and call progress signals would be some of the aspects defined for the service if they can be perceived by the subscriber.

.t.	Testing	02.04;

The test procedure allows the subscriber to check whether or not the service is operating as desired. In some cases the use of the service is sufficient, for others a method of testing is included in the control procedure.( see GSM 02.04)

.t.	Interrogation ((supplementary) service interrogation);

The request by the subscriber to the PLMN to provide information about a specific supplementary service. This information can be requested by a

─	Status check

the following values can be returned by the PLMN:

─	not supported

─	activated

─	deactivated

Not all values are applicable to all supplementary services.

─	Data check

This interrogation function compares the data input by the subscriber during an interrogation procedure with the information stored in the PLMN. The PLMN returns a standard tone, announcement or indications (e.g. "check is positive" or "check is negative").

─	Data request

This interrogation function enables the subscriber to obtain confirmation of  input data. The PLMN returns an appropriate announcement or indication (e.g. "the forwarded─to number is etc.").

.t.	Exceptional operation, unsuccessful; outcome;

Abnormal situations not described in "normal operation with successful outcome". Procedures on time─out, unexpected signalling response and other such events would be defined.

.t.	Interworking considerations;

Identification of subscriber perceptions when a call exits from an ISDN/PLMN to another CEPT specified network or enters an ISDN/PLMN from another CEPT specified network.

.t.	Interworking;

The general term used to describe the inter-operation of networks, services, supplementary services etc.

.t.	Service interworking;

The interworking required when the services at the calling and called terminals are different (e.g. a Short Message Service interworking with MHS access).

.t.	Supplementary service interworking;

This the interworking between the same supplementary services in different networks..

.t.	Transparent support of services;

Support of services where the network is unaware of the higher layer protocols, ie the higher layer protocols are transparent to the PLMN. See also GSM 03.10

.t.	Non─transparent support of services;

Support of services where the network makes use of knowledge of the higher layer protocols,ie some aspects of the higher layer protocols are not transparent to the PLMN. See also GSM 03.10.

2.3 Networks

.t.	Network, telecommunication network (305);

A set of nodes and links that provides connections between two or more defined points to facilitate telecommunication between them.

.t.	Integrated services digital network (ISDN) (308);

An integrated services network that provides digital connections between user─network interfaces.)

.t.	Public land mobile network (PLMN);

A network, established and operated by an Administration or its licensed operator(s), for the specific purpose of providing land mobile communication services to the public. It provides communication possibilities for mobile users. For communication between mobile and fixed users interworking with a fixed network is necessary.

.t.	GSM public land mobile network (GSM PLMN);

A PLMN which complies with the GSM recommendations.

.t.	Home PLMN (HPLMN) ;

The PLMN where a subscription is held and therefore which contains the HLR of the subscriber.

.t.	Visited PLMN (VPLMN);

A PLMN whose services can be used temporarily, based on a subscription in another PLMN, the home PLMN.

.t.	Local PLMN (LPLMN);

The LPLMN is the HPLMN or VPLMN depending on  the location of the MS at the time, and is the PLMN with which the MS is  registered via the radio interface.

.t.	GSM PLMN area (GPA);

The geographical area in which a GSM PLMN (see definitions 303, 304) provides telecommunication services according to the GSM Recommendations to mobile users.

Note : CCITT Recommendation Q.1001 does not contain a definition of a PLMN area.

.t.	GSM system area (GSA);

The group of GSM PLMN areas accessible by GSM mobile stations.

Note : The system area according to CCITT Recommendation 0.1001 corresponds to the GSM System Area.

.t.	GSM service area 	03.02;

The area in which a mobile station can be reached by a fixed subscriber, without the subscriber's knowledge of the actual location of the mobile station within the area. A service area may include the areas served by several PLMNS.

A service area may consist of one country, be a part of a country or comprise several countries.

.t.	GSM PLMN fixed infrastructure;

The part of a GSM PLMN consisting of all fixed equipment,including the BSS's,the MSC's and associated functional entities(such as VLR,HLR,AuC,EIR and IWF)..

.t.	Mobile─services switching centre (MSC);

The MSC performs the functions of switching, routing and control of the call and charging and accounting.The MSC also controls the interworking with fixed networks.

.t.	Gateway MSC (GMSC);

An MSC that provides an entry point into the PLMN from another network or service. A gateway MSC is also an Interrogating Node for incoming PLMN calls.

.t.	Connection (309)

A concatenation of transmission channels or telecommunication circuits, switching and other functional units set up to provide for the transfer of signals between two or more points in a telecommunication network, to support a single communication.

.t.	Digital connection (310)

A concatenation of digital transmission channels or digital telecommunication circuits, switching and other functional units set up to provide for the transfer of digital signals between two or more points in a telecommunication network, to support a single communication.

.t.	Switched connection

A connection that is established by means of switching.

GSM note:	In a GSM PLMN a switched connection only supports demand services since no reserved circuit services as defined in CCITT Recommendation I.112, 206 are foreseen.

.t.	GSM PLMN connection (314)	03.10

A connection that is established through a GSM PLMN between specified GSM PLMN reference points.

.t.	GSM PLMN connection type attribute

A specific characteristic of a GSM PLMN connection type whose values distinguish it from another GSM PLMN connection type.

.t.	GSM PLMN connection type,connection type (316)	03.10

A description of a set of GSM PLMN connections which have the same attributes.

.t.	GSM PLMN connection element,connection element (317)

A partition of a GSM PLMN connection,see GSM 03.10 for the basis of partitioning.

.t.	GSM PLMN connection element attribute

An attribute of a GSM PLMN connection element,eg information transfer rate.

.t.	Point to point GSM PLMN connection (320)

A GSM PLMN connection that is established between two specified GSM PLMN reference points.

.t.	Point to multipoint GSM PLMN connection (321)

A GSM PLMN connection that is established between a single specified GSM PLMN reference point, and more than one other specified GSM PLMN reference points.

.t.	Public Data Network (PDN)

A network established and operated by an administration for the specific purpose of providing data transmission services to the public. Circuit switched (CSPDN),packet switched (PSPDN) and leased circuit data transmission services are feasible depending on national regulations. The Public Data Network may carry other services.(Adapted from CCITT Rec. X.15)

.t.	Network interworking

The interworking required between two different networks (e.g. between a PLMN and ISDN, a GSM PLMN and a non─GSM PLMN, between two GSM PLMN) in order to provide an end to end connection.

.t.	Terminating Network

Network which in cooperation with a GSM PLMN provides telecommunication services between their respective users. Examples of terminating networks are the PSTN, the ISDN, PDNs, GSM PLMNs.

.t.	Interworking function (IWF)

A network functional entity which provides interworking (network interworking, service interworking, supplementary service interworking or signalling interworking). It may be a part of one or more logical or physical entities in a GSM PLMN.

.t.	Interrogating Node (IN)

A switching node that interrogates a HLR, to route a call for an MS to the visited MSC.

2.4 Stations

.t.	Station

One or more transmitters or receivers or a combination of transmitters and receivers, including the accessory equipment, necessary at one location for carrying on a radiocommunication service, or the radio astronomy service.

Each station shall be classified by the service in which it operates permanently or temporarily. (Quoted from the Radio Regulations N° 58).

.t.	Mobile Station (MS)

A station in the mobile service intended to be used while in motion or during halts at unspecified points. (Quoted from the Radio Regulations N° 65).

.t.	Land Station

A station in the mobile service not intended to be used while in motion. (Quoted from the Radio Regulations N° 67).

.t.	Base Station (BS)

A land station in the land mobile service. (Quoted from the Radio Regulations No 68).

.t.	Land Mobile Station

A mobile station in the land mobile service capable of surface movement within the geographical limits of a country or continent. (Quoted from the Radio Regulations N° 69).

.t.	GSM Mobile Station (GSM MS)

Equipment intended to access a set of GSM PLMN telecommunication services. Services may be accessed while the equipment capable of surface movement within the GSM system area is in motion or during halts at unspecified points.

Note : This definition has been adapted from the definitions of a Mobile Station and a Land Mobile Station in the Radio Regulations.

.t.	 Mobile Equipment (ME)

The ME is the Mobile Station (MS) without the SIM.

.t.	Base Station System (BSS)

The system of base station equipment (transceivers, controllers, etc..) which is viewed by the MSC through a single interface as defined by the GSM 08 0x series of recommendations, as being the entity responsible for communicating with Mobile Stations in a certain area. The radio equipment of a BSS may cover one or more cells. A BSS may consist of one or more base stations. If an internal interface according to the GSM 08.5x series at recommendations is implemented, then the BSS shall consist of one Base Station Controller (BSC) and several Base Transceiver Stations (BTSs).

The functionality is described in Recommendation GSM 08.02.

.t.	Base Station Controller (BSC)

A network component in the PLMN with the functions for control of one or more Base Transceiver Stations (BTSs).

.t.	Base Transceiver Station (BTS)

A network component which serves one cell, and is controlled by a Base Station Controller. The BTS contains one or more Transceivers (TRXs).

.t.	Transceiver (TRX)

A network component which can serve full duplex communication on 8 full─rate traffic channels according to recommendation GSM 05.02. If Slow Frequency Hopping SFH is not used, then the TRX serves the communication on one RF carrier.

2.5 Access

.t.	User access. user network access (402)

The means by which a user is connected to a telecommunication network in order to use the services and/or facilities of that network.

.t.	Interface (408)

The common boundary between two associated systems.

.t.	Service Access Point (SAP)

In the reference model for Open System Interconnection, Service Access Points (SAPs)of a layer are defined as gates through which services are offerred to an adjacent higher layer.

.t.	Physical interface (411)

The interface between two equipments.

.t.	Access capability. GSM PLMN access capability (416)

The number and type of the access channels at a GSM PLMN access interface that are actually available for telecommunication purposes.

.t.	Terminal Equipment (TE) (417)

Equipment that provides the functions necessary for the operation of the access protocols by the user.

.t.	Terminal adaptor (TA)

A physical entity in the MS providing terminal adaptation functions. (GSM 04.02)

.t.	Mobile Termination (MT)

The part of the Mobile Station which terminates the radio transmission to and from the network and adapts terminal equipment (TE) capabilities to those of the radio transmission.

.t.	Reference point (420)

A conceptual point at the conjunction of two non─overlapping functional groups.

.t.	Reference configuration (421)

A combination of functional groups and reference points that shows possible network arrangements.

.t.	Multipoint access (422)

User access in which more than one terminal equipment is supported by a single mobile termination.

.t.	Functional group, functional grouping (419)

A set of functions that may be performed by a single equipment.

.t.	Terminal adaptation function(TAF)

The terminal adaptation function is a functional entity associated with an MS. The TAF provides the functionality necessary to permit interworking between an MT and Terminal equipment(TE). The function of the TAF depends on the service and the type of TE. The TAF is required to convert the protocols provided by the MT to those used by the TE. The terminal adaptation functions are described in GSM 07.01,07.02 and 07.03.

.t.	Direct access

Interworking to a Private Telecommunciations Network via a dedicated link.

.t.	Mobile Originated (MO)

Call or short message originated from the Mobile Station.

.t.	Mobile Terminated (MT)

Call or short message intended to be delivered to the Mobile Station.

2.6 Location and handover

.t.	Location register (LR)	03.01

The functional unit in which the location information is stored.

.t.	Location information  	03.01

The information, indicating where a mobile station is located in the system area.

.t.	Location registration	03.01

The function whereby PLMNs keep track of the location information of  Mobile Stations located in the system area.

.t.	Home location register (HLR)	03.01

The location register where the current location and all subscriber parameters of a mobile station are permanently stored.

.t.	Visited location register (VLR)	03.01

The location register where all relevant parameters concerning a mobile station are stored as long as the mobile station is in a location area controlled by this register.

.t.	Location area	Q.9/ Q.1001

An area in which a mobile station may move freely without up─dating the location register. A location area may comprise one or several base station areas.

.t.	MSC area	 Q.1001, 03.02

The part of the PLMN covered by an MSC.

An MSC area may consist of one or several location areas.

.t.	Base station area

The part of the GSM area to which a base station gives service.

.t.	Paging, Paging procedure	04.08

The procedure by which a GSM PLMN fixed infrastructure attempts to reach a Mobile Station within its location area , before any other network-initiated procedure can take place.

.t.	Location area identification	03.03

The information indicating the location area in which a cell is located.

.t.	Deregistration, location deregistration

The updating of a location register by removing location information.

.t.	Location updating procedure	09.02

The procedure by which location register updating takes place.

.t.	Location cancellation procedure 	03.01

The procedure by which the location information is removed from a location register.

.t.	Location information requested procedure	03.01

The procedure by which a VLR enquires whether or not a MS should be kept in the register.

.t.	Location information retrieval procedure	03.01

The procedure by which the home location register obtains information on which of its mobile stations are registered with a visited location register.

.t.	Reset procedure

The procedure for recovery of information for a  location register after restart.

.t.	Restart (Restart procedure)

The procedure to recover the information in a location register after the location information is lost due to a failure.

.t.	Restoration, Restoration procedure

The procedure to recover the information in a location register, after the location information is lost due to a failure. It consists of a restart procedure, after which some mechanisms are provided to keep track of the reliability of each element in memory.

.t.	Handover

The action of switching a call in progress from one radio channel to another radio channel. Handover is used to allow established calls to continue by switching them to another radio resource, e.g. when mobile stations move from one base station area to another.

Handover may take place between the following GSM entities; timeslot ,RF carrier, cell, base station, BSS, MSC.

The prefixes "inter" and "intra" are used to describe the type of handover according to context, e.g. inter─cell handover.

These entities are listed in nested order such that "inter─BSS handover" is implicitly inter ─BS, inter─cell, etc.

Additionally, the words "internal" or "external" may prefix the term intra─BSS handover with the following meanings

"Internal intra─BSS handover" is an intra─BSS handover which takes place without reference to the MSC (although the MSC will be informed on completion).

"External intra─BSS handover" is an intra─BSS handover which is controlled by an MSC.

.t.	Basic handover procedure	03.09

A procedure in which a call is handed over from the MSC under which the call was originated to another MSC.

.t.	Subsequent handover procedure	03.09

A procedure in which a call is handed over from an MSC under which the call was not originated to another MSC.

.t.	Cell

The area of radio coverage locally defined as seen by the Mobile Station with a Base Station Identity Code (BSIC) and uniquely defined as seen by the network with a cell global Identification.

2.7 Identity and Security

.t.	Algorithm A3

Cryptographic algorithm that produces SRES, using RAND AND Ki.

.t.	Algorithm A5

Cryptographic algorithm that produces ciphertext out of cleartext, using Kc.

.t.	Algorithm A8

Cryptographic algorithm that produces Kc using RAND and Ki.

.t.	Authentication

The corroboration that a peer entity is the one claimed.

.t.	Authentication centre (AUC)

Component of the fixed part of the PLMN which contains subscriber authentication keys (Ki) and generates security related parameters (RAND, SRES, Kc) ─ depending on  implementation.

.t.	Base station identity code (BSIC)

A block of code, consisting of the PLMN colour code and a base station colour code. One Base Station can have several Base Station Colour Codes.

.t.	Cell Global Identification

A block of code which uniquely identifies a cell within all GSM PLMNs. It consists of the LAI and CI.

.t.	Cell Identity (CI)

A block of code which identifies a cell  within a location area.

.t.	Reserved

.t.	Cipherkey

A sequence of symbols that controls the operation of encipherment and decipherment.

.t.	Cipherkey setting

Mutual agreement between the Mobile Station and the fixed part of the system upon a common cipherkey (Kc) to be used in a subsequent encipherment/ decipherment process.

.t.	Ciphertext

Unintelligible data produced through the use of encipherment.

.t.	Cryptographic  algorithm [ cryptological algorithm]

Mathematical method used in encipherment and decipherment or in non─reversible encipherment.

.t.	Decipherment

The transformation by cryptographic techniques to produce plaintext from ciphertext.

.t.	Encipherment

Transformation by cryptographic techniques to produce ciphertext from plaintext.

.t.	International mobile station equipment identity (IMEI)

Uniquely identifies the mobile station as a piece or assembly of equipment.

.t.	International mobile subscriber identity (IMSI)

Uniquely identifies the subscription. It can serve as a key to derive subscriber information such as directory number(s) from the HLR.

.t.	Key setting

See cipherkey setting.

.t.	Mobile station international ISDN number(MSISDN)	03.03

Uniquely defines the mobile station as an ISDN terminal. It consists of three parts; the country code (CC), the national destination code (NDC) and the subscriber number (SN).

.t.	Mobile Station Roaming Number (MSRN)	03.03

A code which is allocated to a mobile station when registered with a VLR for the purpose of routing calls to the MSC in which area the Mobile Station is located.1. The MSRN is used by the home location register for rerouting calls to the mobile station.

.t.	Personal Identification Number (PIN)

Confidential information which may be used in the authentication between subscriber and SIM to corroborate that the subscriber is the one claimed.

.t.	Plaintext

Unciphered Data.

.t.	RAND

Random number to be used as challenge in a challenge response protocol.

.t.	Random number

Non─predictable number.

.t.	Security feature

Security features protect

i)	the access to the mobile services

ii)	any relevant item from being disclosed on the radio path, mainly in order to ensure the privacy of user related information.

.t.	Signed Response (SRES)

Response to a challenge in the challenge response protocol.

.t.	Reserved

.t.	Stream cipher

Bit by bit binary addition of plaintext bitstream and cipherkey bitstream.

.t.	Subscriber authentication key

Subscriber individual confidential information used in authentication and in the cipher key generation process

.t.	Subscriber identity authentication

The corroboration by the fixed part of the PLMN that the subscriber identity (IMSI, TMSI), transferred by the mobile subscriber within the authentication procedure at the radiopath, is the one claimed.

.t.	Subscriber identity confidentiality

The property that the subscriber identity (IMSI) is not made available or disclosed on the radio interface.

.t.	Subscriber Identity Module (SIM)

Removable module which is inserted into a Mobile Equipment; it is considered as part of the Mobile Station. It contains security related information (IMSI, Ki, PIN), other subscriber related information and the Algorithms A3/A8.

.t.	Temporary Mobile Subscriber Identity (TMSI)	03.03

A unique identity temporarily allocated to visiting mobile subscribers in order to support the subscriber identity confidentiality service.

.t.	Local Mobile Station Identity(LMSI)

A unique identity temporarily allocated to visiting mobile subscribers in order to speed up the search for subscriber data in the VLR, when the MSRN allocation is done on a per call basis (see definition 720)

2.8 Administrative and commercial definitions

.t.	CEPT Administration

The national administration which has signed the "Arrangement instituant la Conférence Européenne des Administrations des Postes et des Télécommunications".

.t.	GSM PLMN operator

An administration or its licensed operator(s) which provides a GSM PLMN and its telecommunication services.

.t.	Service Provider

The organisation through which the subscriber obtains GSM telecommunication services. This may be a network operator or possibly a separate body.

.t.	Subscription

The subscription permits participation in telecommunication services (basic telecommunication services and those supplementary services for which it is necessary).

.t.	Customer

The customer is the individual or entity who, or which, obtains a service from a GSM PLMN operator or an authorized agent and is responsible for payment of all charges and rentals due.

Note : This definition is adapted from CCITT Rec. D.000.

.t.	Subscriber

The definition of this term is identical with that of the term "customer" under 805.

.t.	User

The user is the individual or entity designated by the customer, individually or by class, as having access to the service and having such authorization, individually or by class, as may be required by the GSM PLMN operator or an authorized agent concerned.

Note : This definition is adapted from CCITT Rec. D.000.

.t.	Collection charge

The collection charge is the charge in its national currency collected by a GSM PLMN operator or an authorized agent from its customers for the use of the service. The establishment of the collection charge is a national matter.

Note : This definition is adapted from CCITT Rec. D.000.

.t.	Network access charge

Part of the collection charge, intended to cover cost of service which is not dependent on the actual use of networks/telecommunications services. It may consist of an initial fee and a subscription fee.

.t.	Network utilization charge

Part of the collection charge which is intended to cover use of the networks/telecommunications services. The charge is registered on a per case basis.

.t.	Rental

Payment(s) due to a GSM PLMN operator or an authorized agent for the access to certain services for the designated periods.

Note : This definition is adapted from CCITT Rec. D.000.

.t.	NET (Norme Europeenne de Telecommunications)

An approved conformity specification recommendation of the CEPT or part or parts hereof, which the signatories of the  Memorandum of Understanding (MoU) on European Telecommunications Standards for terminal equipment (Nov. 15; 1985). adopted in accordance with the procedures set down in that MoU. It includes, where appropriate, requirements made necessary in a given country by historical network peculiarities or established national provisions concerning the use of radio frequencies.

.t.	Conformity specification

A document giving a full description of the technical characteristics of the relevant telecommunications terminal equipment (such as safety, technical parameters, functions and procedures and service requirements) together with a precise definition of the test and test methods enabling the conformity of the equipment with the prescribed technical characteristics to be verified.

.t.	Approved testing laboratory

A laboratory which has been accredited by the appropriate Administration or an accrediting body recognized as competent in its country, according to Rec. T/G 01─01, and which is approved by that Administration or State as competent for conducting conformity tests on telecommunications terminal equipment.

.t.	Certificate of conformity

A certificate, issued in accordance with the Recommendation T/SF 48, which indicates that a telecommunications terminal equipment has been tested by an approved testing laboratory, using appropriate test methods, and is in conformity with defined standards, such as NETs, or parts thereof.

.t.	Certifying body

A body which determines whether a test report from an approved testing laboratory shows that a telecommunications terminal equipment has been tested using appropriate test methods and is in conformity with defined standards. If so, the body issues a certificate of conformity.

Note : This body may be the appropriate national authority, or the national Approval Authority such as an independent body or the approved testing laboratory which conducted the tests.

2.9 Operations and maintenance

.t.	Controlled maintenance	CCITT G.106, N° 4123/CEPT T/CS 10-13

A method to sustain (or sustaining) a desired"quality of service" (network technical performance) by systematic application of analysis techniques using centralized supervisory facilities and /or sampling to minimize "preventive maintenance" and to reduce "corrective maintenance".

.t.	Corrective maintenance ; repair	CCITT G.106,N° 4105/CEPT T/CS 10─13

The "maintenance" carried out after "fault" (failure) recognition" and intended to restore an "item" to a state in which it can perform a (its) "required function".

.t.	Data Communication Network (DCN)	CCITT M 2x

A DCN connects Network Elements (NEs) with internal mediation functions or mediation devices (MOs) to the Operations Systems (0Ss).

(The reference model for open systems interconnection ─ CCITT Rec. X. 200 ─ should be followed).

Note: In general, a DCN does not provide all the data communication functions for a TMN. Data communication links may be required as part of a Local Communication Network.

.t.	Fault	GSM  12.00

The inability of an "item" to perform a "required function".

.t.	Failure	GSM 12.00

The termination of the ability of an "item" to perform a "required function".

.t.	Indenture level	CCITT G.106, N°4119

A level of subdivision of an item from the point of view of a "maintenance action".

Note 1:	examples of indenture levels could be a subsystem, a circuit board, a component.

Note 2:	the indenture level depends on the complexity of the item's construction, the accessibility to subitems, skill level of maintenance personnel, test equipment facilities, safety considerations, etc.

.t.	Local Communication Network (LCN)	CCITT M. 2x

A LCN connects NEs to MDs or MDs to MDs within a telecommunication centre. However, for practical reasons, a LCN may connect remote NEs to local MDs.

.t.	Maintenance	CCITT G.106,N° 4103 CEPT T/CS 10─13

The combination of all technical and corresponding administrative actions  including supervision actions, intended to retain an "item" in, or restore it to, a state in which it can perform a "required function".

.t.	Maintenance entity (ME)	CCITT 6. 106,N° 4130

A sub─item of a given "item" defined with the intention that an alarm caused by a "fault" in that sub─item will be unambiguously referrable to the sub─item.

Note	The definitions of CCITT and CEPT seem to be somewhat divergent. Since CEPT Recommendations are referring to CCITT Recommendation G.106, and the CCITT definition is more precise, the CCITT view shall be applied.

.t.	Mediation	CCITT M. 2x

A process that routes and/or acts on information passing between NEs and OSs via DCN. Mediation can be shared among NE (s) and/or OS (s).

The processes that can form mediation can be classified into following five general process categories

1.	Communication control

2.	Protocol conversion and data handling

3.	Communication (passing) of primitive functions

4.	Processes involving decision making

5.	Data storage.

.t.	Network administration	GSM 12.00

(not in the sense of a body)

Network administration is the network related part of the Operator's Service Provider's overall administration and any organization, non─technical action to provide or to help to provide services of a telecommunication network.

.t.	Network Management (NM)	GSM 12.00

NM is all activities which control, monitor and record the use and the performance of resources of a telecommunication network in order to provide telecommunication services to customers/users at a certain level of quality.

.t.	Network Management Centre (NMC)	GSM 12.01/12.07

The NMC node of the GSM TMN provides global and centralised PLMN monitoring and control, by being at the top of the TMN hierarchy and linked to subordinate OMC nodes.

.t.	Operations	CCITT G.106,N°1012

Combination of all technical and corresponding administrative actions intended so that any "item" can perform a "required function", recognizing necessary adaptation to changes in external conditions. (Note: by external conditions are understood, for example, service demand and environmental conditions).

.t.	Operations and Maintenance Centre (OMC)	GSM 12.01/12.07

The OMC node of the GSM TMN provides dynamic O&M monitoring and control of the PLMN nodes operating in the geographical area controlled by the specific OMC.

.t.	Operations System (OS)	CCITT M.2x

There are, at least, three functional types of OSs, i.e. basic, network, and services

─	Basic OSs perform TMN application functions related to Network Elements (NEs) located in specific regions.

─	Network OSs cover the realization of network based TMN application functions by communicating with basic OSs.

─	Service OSs perform specific application functions for managing and individual service.

Basic OSs and Network OSs share infrastructural aspects of the telecommunication network. Service OSs are concerned with service aspects of one or more telecommunication networks.

.t.	Preventive maintenance	CCITT G.106,N°4104

The "maintenance" carried out at predetermined intervals or according (corresponding) to prescribed criteria and intended to reduce the "probability" of "failure" or the (performance) degradation of the functioning of an "item".

.t.	Performance Management	GSM 12.07

Performance Management of PLMN implies the maintaining of quality of service without sacrificing the set financial objectives of the Operator.

.t.	Resource Management	GSM 12.00

Resource Management (by planning functions, procurement etc.) provides the network resources (active and passive telecommunication equipment) and the NM resources.

.t.	Security Management	GSM 12.03

Security Management is the control and distribution of security─relevant information to various users and systems for

─	use in providing security services,

─	reporting on security services and mechanisms,

─	reporting on security─relevant events that have occurred.

.t.	System change control	GSM 12.06

The mechanism which assumes the satisfactory function of the programmes controlling the system also ensures that programme change and updating are satisfactorily carried out.

.t.	TMN Standard Interfaces	CCITT M. 2x

The standard interfaces which provide for the interconnection of NEs, 0Ss, MDs and WSs through the DCN or LCN.

Note: This requires compatible communication protocols and a compatible data representation method for the messages, including compatible generic message definitions for TMN application functions.

.t.	Work Station (WS)	GSM 12.00

The remote device via which O&M personnel executes input and output transactions for NM purposes.

.t.	Administration Centre (ADC)

The ADC node of the TMN provides for PLMN Operator-specific requirements in the administration and commercial areas.

.t.	Telecommunications Management Network (TMN)

The implementation of the Network Management functionality required for the PLMN is in terms of physical entities, the full ste of which constitutes the TMN

2.10 Expressions related to the radiosubsystem

.t.	Absolute RF Channel Number (ARFCN)	05.05

An integer  which defines the absolute RF channel number.

.t.	Active mode	05.08

The state of a MS when processing a call.

.t.	Active part of timeslot	 05.02

Identical with burst.

.t.	Adaptive Frame Alignment	05.08

Means of ensuring that the timeslots received at the BS from active MSs at different distances from the BS are in time alignment.

Transmit timeslot advance period in the MS necessary to ensure this. The value can be assessed by measuring the "roundtrip delay"; this depends on the distance of the MS from the serving BS.

.t.	BCCH allocation (BA)	05.08

The radio frequency channels allocated in a cell for BCCH transmission.

.t.	Bit number (BN)	05.02

Number which identifies the position of a particular bit period within a timeslot.

.t.	Burst	05.02

A period of modulated carrier less than one timeslot. The physical content of a timeslot.

.t.	Call Release	05.08

Forced disconnection after a fixed period when the ACCH data is no longer decoded correctly.

.t.	Cell allocation	05.02

The radio frequency channels allocated to a particular cell.

.t.	Cell Coverage Area	03.30

Area within which a defined quality of reception is provided.

The planned radio coverage of a cell.

.t.	Channel	05.02

A means of one─way transmission. A defined sequence of periods (e.g. timeslots) in a TDMA system; a defined frequency band in an FDMA system; a defined sequence of periods and frequency bands in a frequency hopped system.

.t.	Class IA. lB. II	05.03/05.04

The classification of speech encoder bits depending on the degree of protection needed. Class IA and class IB bits have protection, while Class II bits have no protection. Error detection is performed on class IA bits.

.t.	Control Channels (CCH)	05.01

Channels which carry system management messages.

.t.	Current Serving BS	05.10

BS on one of whose channels (TCH, DCCH, or CCCH) the MS is currently operating.

.t.	Discontinuous Reception (DRX)	05.08

Means of saving battery power (e.g. in HPUs) by periodically and automatically switching the MS receiver on and off.

.t.	Discontinuous Transmission (DTX)	05.08

Means of saving battery power (e.g. in HPUs) and reducing interference by automatically switching the transmitter off when no speech or data are to be sent.

.t.	Downlink	05.02.

Physical link from the BS towards the MS (BS transmits, MS receives).

.t.	Dummy bits	05.04

Internal states of the modulator before and after a burst.

.t.	Dummy Burst (DB)	05.02

A period of carrier less than one timeslot whose modulation is a defined sequence that carries no useful information. Means of filling a timeslot with an RF signal when no information is to be delivered to a channel.

.t.	Ec/No

Ratio of energy per modulating bit to the noise spectral density.

.t.	Frame number (FN)	05.02

Identifies the position of a particular TDMA time─frame within a hyperframe.

.t.	Frequency correction burst (FB)	05.02

Period of RF carrier less than one timeslot whose modulation bit stream allows frequency correction to be performed easily within a mobile station.

.t.	Guard Period	05.05

Period at beginning and end of timeslot during which MS transmission is attenuated.

.t.	Handover	05.08

Change of dedicated channel during call.

.t.	Handover Access Burst	 05.08

Access burst used during handover.

.t.	Handover Execution	05.08/03.09

Signalling message sequence that causes the MS to continue the call in another (predetermined) cell.

.t.	Handover Execution Interval	05.08

Period of time between consecutive handover execution commands.

.t.	Handover Margin	05.08

Hysteresis factor that minimises repetitive handovers between adjacent cells. Means to affect the traffic distribution between cells.

.t.	Handover Strategy	05.08

Procedure defined by an algorithm that prescribes how handover shall be carried out e.g. handover to a cell which offers better signal strength or to a cell which allows communication with less power.

.t.	Hyperframe	05.02

A hyperframe consists of 2048 superframes (see definition 1055) It is the longest recurrent time period of the frame structure (12 533.76s).

.t.	Idle Mode	05.08

State of an MS not actively processing a call.

.t.	Inactive part of timeslot	05.02

Identical with guard period.

.t.	Inter─Cell Handover	05.08

Handover between cells. Change of serving cell.

.t.	Intra─Cell Handover	05.08

Handover between different RF carriers  or different timeslots of the same RF carrier  within one cell.

.t.	Logical channels	05.02

Two classes of logical channels are defined, control channels (CCH) which carry system management messages and traffic channels (TCH) which carry users' speech or data.

.t.	Mobile allocation (MA)	05.02

The radio frequency channels allocated to a MS for use in its hopping sequence.

.t.	Multiframe (Multiple time─frame)	05.01

Two types of multiframe are defined in the system: a 26─frame multiframe with a period of 120ms and a 51─frame multiframe with a period of 3060/13ms.

.t.	Multiple timeframe

See multiframe.

.t.	Network Directed Criteria	05.08

Handover criteria originating from network management requirements, e.g. traffic, precedence requests etc...

.t.	Network Management Requirements	05.08

See network directed criteria.

.t.	Normal Burst (NB)	05.02

A period of modulated carrier less than a timeslot.

.t.	Peak power

Measure of the maximum RF power when averaged over one radio frequency cycle, during the useful part of the burst.

.t.	Power Class (of MS)	05.05

Class of maximum output power to which a MS belongs; e.g. handheld portable, mobile. Defined by the maximum peak power that can be output by a MS. Five power classes are defined.

.t.	Power control Level	05.05

Peak power level (less than or equal to the maximum peak power corresponding to the power class) that a MS outputs on command from the BS of the serving cell.

.t.	Power Control Message	05.08

Message that controls the MS transmitted RF power level (see MS_TXPWR_REQUEST).

.t.	Quarter Bit Number	05.10

Timing of quarter bit periods (12/13 us) within a timeslot.

.t.	Radiofrequency channel (RFCH)

A partition of the system RF spectrum allocation with a defined bandwidth and centre frequency.

.t.	Radio Sub─System Criteria	05.08

Criteria used in the handover and transmitted RF power control processes (e.g. signal strength, signal quality, absolute MS─BS distance).

.t.	Radio Sub─System Directed Handover	05.08

Change of channel during a call because of either degradation of quality of the current serving channel or the availability of another channel which can allow communication at a lower transmitted RF power, or to prevent a MS grossly exceeding the planned cell area.

.t.	Roundtrip Delay	05.08

Time period between transmit and receive instant of a timeslot in the BS, determined by the response behaviour of the MS and the MS─BS distance.

.t.	Serving BS	05.08

BS with which the MS is currently communicating.

.t.	Serving Cell	 05.08

Cell containing the serving BS.

.t.	SID─frame	06.31

Frame which carries the averaged background noise information

.t.	Sub─channel number (SCN)	05.02

One of the parameters defining a particular physical channel in a BS.

.t.	Superframe	05.01

A superframe is 51 traffic/associated control multiframes or 26 broadcast/common control multiframes.(period 6.12s).

.t.	Synchronization burst (SB)	05.02

Period of RF carrier less than one timeslot whose modulation bit stream carries information for the MS to synchronize its time─frame start to that of the received signal.

.t.	Tail bits	05.03/05.04

Known bits that are added at the end of the information stream to allow use of a convolutional code to build a (non─systematic) block code while preserving the minimum distance properties.

.t.	TDMA Frame Number	05.02

The number of a particular TDMA frame in the cyclic TDMA frame numbering range (2715647).

.t.	Timebase Counters	05.10

A set of counters which determine the timing state of signals transmitted by a BS or MS.

.t.	Time─frame	05.01

The repetition period of the (8) timeslots of a particular RF channel (period 60/13 = 4.62 ms approx.).This is referred to as a frame in the definition of multiframes.

.t.	Timeslot	05.01

One of a number of periods in a time─frame (period 15/26ms). Physical channel in a TDMA system.

.t.	Timing Advance	05.10

A signal sent by the BS to the MS. It enables the MS to advance the timing of its transmissions to the BS so as to compensate for propagation delay.

.t.	Timeslot Number (TN)	05.02

Identifies a particular timeslot within a TDMA frame frame.

.t.	Traffic channels	05.02

Channels which carry users' speech or data.

.t.	Training sequence	05.02

Sequence of modulating bits employed to facilitate timing recovery and channel equalisation in the receiver.

.t.	Training sequence code	05.02

Parameter used to select one of a number of training sequences.

.t.	Uplink	05.02

Physical link from MS towards BS (MS transmits, BS receives)

.t.	Useful part of timeslot	05.04

That part of the burst used by the demodulator; differs from the full burst because of the 1/2 bit shift of the I and Q parts of the GMSK signal.

.t.	Voice activity detection (VAD)	05.03/05.04

A process used to identify presence or absence of speech data bits.This is used along with DTX (see definition 1016)

2.11 Miscellaneous

.t.	Mobile Station Feature

A MS feature is a function or a piece of equipment which directly relates to the man─machine operation of the MS.

Three categories of MS features are distinguished: basic, supplementary and additional features.

.t.	Queuing

The procedure in which calls, originating from a MS or terminating in a MS are kept pending for reasons of congestion or when the called party is occupied.

.t.	Off─air─call─set─up (OACSU)

The procedure in which a telecommunication connection is being established whilst the r.f. link between the MS and the BS is not occupied.

.t.	Mobile Application Part (MAP)	09.01/03.01

The internetworking signalling between MSCs and LRs and EIRs. A part of CCITT Signalling System No. 7

.t.	Signalling Interworking

Signalling Interworking is the function required to connect the signalling protocols used in a GSM PLMN to those used in the appropriate fixed network.

.t.	Layer 2 relay (L2R)

A function of an MS and IWF that adapts a user's known layer2 protocol onto RLP for transmission over the GSM radio interface. See also GSM 07.01,07.02 and 07.03.

.t.	Radio Link Protocol (RLP)

An Automatic Repeat reQuest (ARQ) protocol used to reliably transfer user data between an MS and IWF.See also GSM 04.22

3. ABBREVIATIONS AND ACRONYMS

.Begin Table A.

CFB	Call forwarding mobile subscriber busy

CFU	Call forwarding unconditional

CLIP	Calling line identification presentation

CLIR	Calling line identification restriction

COLP	Connected line identification presentation

COLR	Connected line identification restriction

MCI	Malicious call identification

.End Table A.

A

A3	see Algorithm A3 (701)

A5	see Algorithm A5 (702)

A8	see Algorithm A8 (703)

AB 	Access Burst

ACCH	Associated Control Channel

ACM	Address Complete Message

ACU	Antenna Combining Unit

ADC	ADministration Centre (924)

AEF	Additional Elementary Functions

AGCH	Access Grant CHannel

AOC	Advice Of Charge (223)

ARFCN	Absolute Radio Frequency Channel Number (1001)

ARQ	Automatic Request for Retransmission (1007)

AUC	Authentication Centre (705)

AUT(H)	Authentication (704)

B

BA	BCCH Allocation (1005)

BAIC	Barring of All Incoming Calls (231)

BAOC	Barring of All Outgoing Calls (228)

BCC	Base Transceiver Station (BTS) Colour Code	05.02

BCCH	Broadcast Control Channel

BCCH_FREQ_NCELL	Frequency of the RF carrier on which the BCCH of a neighbouring cell is transmitted.	05.08

BCD	Binary Coded Decimal

BCU	(See 08.56)

BIC-Roam	Barring of Incoming Calls when Roaming Outside the Home PLMN Country  (232)

Bm	Full-rate traffic channel

BN	Bit Number (1006)

BNHO	Barring all outgoing calls except those to Home PLMN

BOIC	Barring of Outgoing International Calls (229)

BOIC-exHC	Barring of Outgoing international Calls except those directed to the Home PLMN Country (230)

BS	Base Station (404)

BS_BCCH_SDCCH_C0MB	Logical variable that indicates the combination of dedicated and associated control channels on the same physical channel	05.02

BS_CC_CHANS	Number of basic physical channels supporting common control channels	05.02

BS_G_BLKS_RES	Number of blocks on each common control channel reserved for access grant messages	05.02

BS_PA_MFRMS	Number of multiframes between two transmissions of the same paging message to MSs of the same paging group	05.02

BSC	Base Station Controller (409)	05.02

BSIC	Base Transceiver Station Identity Code (706)	05.02

BSIC─NCELL	BSIC of an adjacent cell	05.08

BSS	Base Station System (408)

BTS	Base Transceiver Station (410)

C

C	Conditional

CA	Cell Allocation

CA_BAND_NUMB	Number of the frequency band that contains the cell allocation	05.05

CBCH	Cell Broadcast CHannel 	05.02

CC	Country Code

CCBS	Completion of Calls to Busy Subscribers (221)

CCCH_GROUP	Group of MSs in idle mode

CCH	Control CHannel (1013)

CCPE	Control Channel Protocol Entity

CELL-BAR-ACCESS	Cell Access Barred

CELL_RESELECT_HYSTERESIS	RXLEV Hysteresis required for Cell Reselection

CFB	Call Forwarding on mobile subscriber Busy (217)

CFNRc	Call Forwarding on MS Not REAchable (219)

CFNRy	Call Forwarding on No Reply (218)

CFU	Call Forwarding Unconditional (216)

CI	Cell Identity (708)

CLIP	Calling Line Identification Presentation (211)

CLIR	Calling Line Identification Restriction (212)

CMD	Command

COLI	Connect Line Identity

COLP	Connected Line Identification Presentation (213)

COLR	Connected Line identification Restriction (214)

COM	Complete

CONF	Conference Call,add on (227)

CONN	Connect

CRE	Call RE─establishment procedure

CSPDN	Circuit Switched Public Data Network(324)

CT	- Channel Tester

- Call Transfer (225)

CUG	Closed User Group (222)

CW	Call Waiting (220)

D

DB	Dummy Burst (1019)

DCCH	Dedicated Control Channel

DCF	Data Communication Function

DCN	Data Communication Network (903)

DET	Detach

DISC	DISConnect

DL	Data Link (layer)

DLCI	Data Link Connection Identifier

DLD	Data Link Discriminator

Dm	Control Channel (ISDN terminology applied to mobile service)

DP	Dialled Pulse

DRX	Discontinuous Reception (Mechanism) (1015)

DTAP	Direct Transfer Application Part

DTE	Data Terminal Equipment

DTMF	Dual Tone Multi─Frequency (signalling)

DTX	Discontinuous Transmission (Mechanism) (1016)

E

EA	External Alarms

Ec/No	Ratio of energy per modulating bit to the noise spectral density (1020)

EIR	Equipment Identity Register

ERR	ERRor

F

FA	Full Allocation

FACCH	Fast ACCH

FACCH/F	Full rate Fast Associated Control Channel

FACCH/H	Half rate Fast Associated Control Channel

FB	Frequency correction Burst (1022)

FCCH	Frequency Correction CHannel

FEC	Forward Error Correction

FN	Frame Number (1021)

G

GMSC	Gateway Mobile Services Switching Centre (313)

GPA	GSM PLMN area (308)

GSA	GSM System Area (309)

GSM	Groupe Special Mobile

GSM MS	GSM Mobile Station (406)

GSM PLMN	GSM Public Land Mobile Network (304)

H

HANDO	Handover

HDLC	High Level Data Link Control

HLR	Home Location Register (604)

HO_MARGIN	SDL Message name for Handover Margin

HOLD	Call Hold (Supplementary Service) (224)

HPLMN	Home PLMN (305)

HPU	Hand Portable Unit

HSN	Hopping Sequence Number	05.02

I

I	Information (frames)

IAM	Initial Address Message

ICB	Incoming Calls Barred

ID	Identification

IDN	Integrated Digital Network

IE	Signalling Information Element

IMEI	International Mobile station Equipment Identity (716)

IMSI	International Mobile Subscriber Identity (717)

IN	Interrogating Node (328)

INU	Interworking Unit

ISDN	Integrated Services Digital Network (302)

IWF	Inter Working Function (327)

K

K	Constraint Length of the Convolutional Code

Kc	Cipher Key

Ki	see "subscriber authentication key" (729)

L

L2R	Layer 2 Relay (1106)

LAC	Location Area Code

LAI	Location Area Identity	05.08

LAN	Local Area Network

LAPDm	Link Access Protocol on the Dm channel

LCN	Local Communication Network (908)

LE	Local Exchange

Lm	Traffic channel with capacity lower than Bm

LPLMN	Local PLMN (307)

LR	Location Register (601)

M

M	Mandatory

MA	Mobile Allocation  (1036)

MACN	Mobile Allocation Channel Number

MAF	Mobile Additional Function

MAH	Mobile Access Hunting (233)

MAI	Mobile Allocation Index	05.02

MAIO	Mobile Allocation Index Offset	05.02

MAP	Mobile Application Part (1104)

MCC	Mobile Country Code

MCI	Malicious Cell Identification (215)

MD	Mediation Device

MDL	(mobile) Management (entity) ─ Data Link (layer)

ME	Maintenance Entity (909)

ME	Mobile Equipment (407)

MEF	Maintenance Entity Function

MIC	Mobile Interface Controller

MM	Man Machine

MME	Mobile Management Entity

MMI	Man Machine Interface

MNC	Mobile Network Code

MO	Mobile Originated (515)

MPH	(mobile) Management (entity) ─ PHysical (layer) [primitive]

MS	Mobile Station (402)

MS_PWR_CLASS	MS PoWer Class. Parameter defining the power class of an MS expressed in the same way as the R parameters	05.08

MS_RANGE_MAX	Mobile Station Range Maximum. Handover criterion to determine serving cell

MS_RXLEV_L	Lower Receive Level. Threshold of RXLEV received from the serving BS below which either power control or handover must take place to improve the cell quality	05.08

MS_TXPWR_CONF MS	Transmitted RF Power Confirmation. Parameter sent by the MS to indicate its current transmitted RF power level

MS_TXPWR_MAX_CCH	Maximum Allowed Transmitted RF Power for MSs to Access the System until commanded otherwise

MS_TXPWR_REQUEST	MS Transmitted RF Power Request. Parameter sent by the BSS that commands the required MS RF Power Level

MSC	Mobile─services Switching Centre (312)

MSCM	Mobile Station Class Mark

MSCU	Mobile Station Control Unit

MSISDN	Mobile Station ISDN Number (719

MSRN	Mobile Station Roaming Number (720)

MT	Mobile Terminated (516)

MT	Mobile Termination (508)

MTP	Message Transfer Part

MUMS	Multi User Mobile Station

N

N/W	Network

NB	Normal Burst (1041)

NBIN	A parameter in the hopping sequence 	05.02

NCELL	Neighbouring (or current serving) Cell	05.08

NDC	National Destination Code

NE	Network Element

NEF	Network Element Function

NET	Norme Europeenne de Télécommunications (812)

NF	Network Function

NM	Network Management (912)

NMC	Network Management Centre (913)

NMSI	National Mobile Station Identification number

NSAP	Network Service Access Point

NT	Network Termination

O

O	Optional

O&M	Operations & Maintenance

OACSU	Off─Air─Call─Set─Up (1103)

OCB	Outgoing Calls Barred

OD	Optional for operators to implement for their aim

OMC	Operations & Maintenance Centre (915)

OS	Operating System (916)

OSI	Open System Interconnection

OSI RM	OSI Reference Model

P

PAD	Packet Assembly/Disassembly facility

PAGING_GROUP	The set of MSs monitoring a particular paging block	05.02

PCH	Paging CHannel

PD	Public Data

PDN	Public Data Networks (324)

PH	PHysical (layer)

PI	Presentation Indicator

PIN	Personal Identification Number (721)

PLMN_PERMITTED	PLMN Permitted for handover purposes	05.08

PLNN	Public Land Mobile Network (303)

PPE	Primative Procedure Entity

Ps	Location Probability

PSPDN	Packet Switched Public Data Network(324)

PSTN	Public Switched Telephone Network

Q

QA	Q (Interface) ─ Adapter

QAF	Q ─ Adapter Function

QOS	Quality Of Service

R

[R	Value of Reduction of the MS Transmitted RF Power relative to the maximum allowed output power of the highest  power class of MS (A)]

RA	RAndom mode request information field

RAB	Random Access Burst

RACH	Random Access CHannel

RADIO_LINK_TIME0UT	The timeout period for radio link failure. Maximum value of the radio link timer	05.08

RADIO_LINK_TIMER	Parameter which is incremented or decremented according to the success with which SACCH messages are decoded			05.08

RAND	RANDom Number (authentication) (723)

REC	RECommendation

REL	RELease

REQ	REQuest

RESELECT_INTERVAL_MIN	Minimum time between cell reselections	05.08

RFC	Radio Frequency Channel

RFCH	Radio Frequency CHannel (1047)

RFN	Reduced TDMA Frame Number	05.02

RLP	Radio Link Protocol (1107)

RNTABLE	Table of 128 integers in the hopping sequence

RSE	Radio System Entity

RXLEV	Received Signal Level	05.08

RXLEV_ACCESS_MIN	The minimum received signal level at a MS for access to a cell	05.08

RXLEV_MIN	The minimum received signal level at a MS from a neighbouring cell for handover to be permitted

RXLEV_NCELL	Received signal level of neighbouring or current serving cell measured on the BCCH carrier	05.08

RXLEV_SERVING_CELL	Received signal level in the serving cell measured on the BCCH carrier	05.08

RXQUAL	Received Signal Quality	05.08

RXQUAL_FULL	Received signal quality assessed over the full set of TDMA frames within a SACCH block	05.08

RXQUAL_SERVING_CELL	Received signal quality of serving cell	05.08

RXQUAL_SUB	Received signal quality assessed over a subset of 12 TDMA frames	05.08

S

S/W	SoftWare

SABM	Set Asynchronous Balanced Mode

SACCH	Slow Associated Control CHannel

SACCH/C4	Slow, SDCCH/4 Associated, Control CHannel

SACCH/C8	Slow, SDCCH/8 Associated, Control CHannel

SACCH/T	Slow, TCH─Associated, Control CHannel

SACCH/TF	Slow, TCH/F─Associated, Control CHannel

SACCH/TH	Slow, TCH/H─associated, Control CHannel

SAP	Service Access Point (503)

SAPI	Service Access Point Indicator

SB	Synchronization  Burst (1056)

SCCP	Signalling Connection Control Part

SCH	Synchronization CHannel

SCN	Sub─Channel Number (1054)

SDCCH	Stand-alone Dedicated Control CHannel

SDCCH/4	Stand─alone Dedicated Control CHannel/4

SDCCH/8	Stand─alone Dedicated Control CHannel/8

SDL	Specification Description Language

SE	Support Entity

SEF	Support Entity Function

SEG	Security Experts Group

SFH	Slow Frequency Hopping

SI	Service Interworking

SIM	Subscriber Identity Module (732)

SLTM	Signalling Link Test Message

SMSCB	Short Message Service Cell Broadcast	05.02

Smt	Short message terminal

SN	Subscriber Number

SP	Signalling Point

SRES	Signed RESponse (authentication) (726)

SS	Supplementary Service

SS	System Simulator

STP	Signalling Transfer Point

T

TA	Terminal Adaptor (507)

TAC	Type Approval Code

TAF	Terminal Adaptation Function (513)

TC	Transaction Capabilities

TCAP	Transaction Capabilities Application Part

TCH	Traffic CHannel

TCH/F	A Full rate TCH.

TCH/F2.4	A Full rate data TCH (<2.4kbit/s).

TCH/F4.8	A Full rate date TCH (4.8kbit/s).

TCH/F9.6	A Full rate data TCH (9.6kbit/s).

TCH/FS	A Full rate Speech TCH.

TCH/H	A Half rate TCH

TCH/H4.8	A Half rate data TCH (4.8kbit/s)

TCH/HS	A Half rate Speech TCH.

TCI	Transceiver Control Interface

TE	Terminal Equipment (506)

Tei	Terminal endpoint identifier

TFA	Transfer Allowed

TFP	Transfer Prohibited

TMN	Telecommunications Management Network (925)

TMSI	Temporary Mobile Subscriber Identity (733)

TN	Timeslot Number (1063)

TPS	Three Party Service

TRX	Transceiver (411)

TS	Time Slot

TSDI	Transceiver Speech & Data Interface

TXPWR	Transmit power; Tx power level in the MS_TXPWR_REQUEST and MS_TXPWR_CONF parameters.

U

UI	Unnumbered Information (Frame)

UPD	Up to Date

UUS	User-to-User Signalling (234)

V

VAD	Voice Activity Detection (1069)	(05.03/05.04/06.32)

VLR	Visitor Location Register (605)

VMSC	Visited MSC, (recommendation not to be used)

VPLMN	Visited PLMN (306)

W

WS	Work Station (923)

-

3PTY	Three ParTY service (226)

4. ALPHA SORTED INDEX

.Begin Table T.

A

001	Absolute RF Channel Number (ARFCN)	05.05

505	Access capability. GSM PLMN access capability (416)

239	Activation

1002	Active mode	05.08

1003	Active part of timeslot	05.02

1004	Adaptive Frame Alignment	05.08

924	Administration Centre (ADC)

223	Advice of charge (AoC)

701	Algorithm A3

702	Algorithm A5

703	Algorithm A8

814	Approved testing laboratory

704	Authentication

705	Authentication centre

B

231	Barring of all incoming calls (BAIC)

228	Barring of all outgoing calls(BAOC)

232	Barring of incoming calls when roaming outside the home PLMN country (BIC-Roam)

229	Barring of outgoing international calls (BOIC)

230	Barring of outgoing international calls except those directed to the home PLMN country (BOIC-exHC)

404	Base Station

608	Base station area

409	Base Station Controller (BSC)

706	Base station identity code (BSIC)

408	Base Station System (BSS)

410	Base Transceiver Station (BTS)

620	Basic handover procedure	03.09

209	Basic services

1005	BCCH allocation (BA)	05.08

205	Bearer service (202)

1006	Bit number (BN)	05.02

1007	Burst	05.02

C

217	Call forwarding mobile subscriber busy (CFB)

219	Call forwarding on mobile subscriber not reachable (CFNRc)

218	Call forwarding on no reply (CFNRy)

216	Call forwarding unconditional (CFU)

224	Call hold (HOLD)

1008	Call Release	05.08

225	Call transfer (CT)

220	Call waiting (CW)

211	Calling line identification presentation (CLIP)

212	Calling line identification restriction (CLIR)

622	Cell

1009	Cell allocation	05.02

1010	Cell Coverage Area	03.30

707	Cell Global Identification

708	Cell Identity (CI)

801	CEPT Administration

815	Certificate of conformity

816	Certifying body

1011	Channel	05.02

710	Cipherkey

711	Cipherkey setting

712	Ciphertext

1012	Class IA. lB. II	05.03/05.04

222	Closed user group (CUG)

808	Collection charge

221	Completion of calls to busy subscribers (CCBS)

227	Conference call,add on (CONF)

813	Conformity specification

213	Connected line identification presentation (COLP)

214	Connected line identification restriction (COLR)

314	Connection (309)

1013	Control Channels (CCH)	05.01

901	Controlled maintenance	CCITT G.106, N° 4123/CEPT T/CS 10-13

902	Corrective maintenance

713	Cryptographic algorithm [cryptological algorithm]

1014	Current Serving BS	05.10

805	Customer

D

903	Data Communication Network (DCN)	CCITT M 2x

240	Deactivation

714	Decipherment

207	Demand service, demand telecommunication service (205)

611	Deregistration, location deregistration

315	Digital connection (310)

514	Direct access

1015	Discontinuous Reception (DRX)	05.08

1016	Discontinuous Transmission (DTX)	05.08

1017	Downlink	05.02.

1018	Dummy bits	05.04

1019	Dummy Burst	05.02

E

1020	Ec/No

715	Encipherment

238	Erasure

245	Exceptional operation, unsuccessful

F

905	Failure	GSM 12.00

904	Fault	GSM  12.00

1021	Frame number (FN)	05.02

1022	Frequency correction burst (FB)	05.02

512	Functional group, functional grouping (419)

G

313	Gateway MSC

406	GSM Mobile Station (GSM MS)

308	GSM PLMN area (GPA)

317	GSM PLMN connection (314)	03.10

321	GSM PLMN connection element attribute

320	GSM PLMN connection element,connection element (317)

318	GSM PLMN connection type attribute

319	GSM PLMN connection type,connection type (316)	03.10

311	GSM PLMN fixed infrastructure

802	GSM PLMN operator

304	GSM public land mobile network (GSM PLMN)

310	GSM service area 	03.02

309	GSM system area (GSA)

1023	Guard Period	05.05

H

619	Handover

1024	Handover	05.08

1025	Handover Access Burst	 05.08

1026	Handover Execution	05.08/03.09

1027	Handover Execution Interval	05.08

1028	Handover Margin	05.08

1029	Handover Strategy	05.08

604	Home location register (HLR)	03.01

305	Home PLMN

1030	Hyperframe	05.02

I

1031	Idle Mode	05.08

1032	Inactive part of timeslot	05.02

906	Indenture level	CCITT G.106, N°4119

302	Integrated services digital network (ISDN) (308)

502	Interface (408)

716	International mobile station equipment identity (IMEI)

717	International mobile subscriber identity (IMSI)

328	Interrogating Node (IN)

244	Interrogation ((supplementary) service interrogation)

247	Interworking

246	Interworking considerations

327	Interworking function (IWF)

1033	Inter─Cell Handover	05.08

1034	Intra─Cell Handover	05.08

241	Invocation

K

718	Key setting

L

204	Land Mobile Service

405	Land Mobile Station

403	Land Station

1106	Layer 2 relay (L2R)

907	Local Communication Network (LCN)	CCITT M. 2x

734	Local Mobile Station Identity(LMSI)

307	Local PLMN (LPLMN)

606	Location area	Q.9/ Q.1001

610	Location area identification	03.03

613	Location cancellation procedure 	03.01

602	Location information  	03.01

614	Location information requested procedure	03.01

615	Location information retrieval procedure	03.01

601	Location register	03.01

603	Location registration	03.01

612	Location updating procedure	09.02

1035	Logical channels	05.02

M

908	Maintenance	CCITT G.106,N° 4103 CEPT T/CS 10─13

909	Maintenance entity (ME)	CCITT 6. 106,N° 4130

215	Malicious call identification (MCI)

910	Mediation	CCITT M. 2x

233	Mobile access hunting (MAH)

1036	Mobile allocation (MA)	05.02

1104	Mobile Application Part (MAP)	09.01/03.01

407	Mobile Equipment

515	Mobile Originated (MO)

203	Mobile service

402	Mobile Station

1101	Mobile Station Feature

719	Mobile station international ISDN number(MSISDN)	03.03

720	Mobile Station Roaming Number (MSRN)	03.03

516	Mobile Terminated (MT)

508	Mobile Termination (MT)

312	Mobile─services switching centre (MSC)

607	MSC area	 Q.1001, 03.02

1037	Multiframe (Multiple time─frame)	05.01

1038	Multiple timeframe

511	Multipoint access (422)

N

812	NET (Norme Europeenne de Telecommunications)

809	Network access charge

911	Network administration	GSM 12.00

1039	Network Directed Criteria	05.08

325	Network interworking

912	Network Management (NM)	GSM 12.00

913	Network Management Centre (NMC)	GSM 12.01/12.07

1040	Network Management Requirements	05.08

810	Network utilization charge

301	Network, telecommunication network (305)

251	Non─transparent support of services

1041	Normal Burst (NB)	05.02

242	Normal operation with successful

O

1103	Off─air─call─set─up (OACSU)

914	Operations	CCITT G.106,N°1012

915	Operations and Maintenance Centre (OMC)	GSM 12.01/12.07

916	Operations System (OS)	CCITT M.2x

P

609	Paging, Paging procedure	04.08

1042	Peak power

918	Performance Management	GSM 12.07

721	Personal Identification Number (PIN)

504	Physical interface (411)

722	Plaintext

323	Point to multipoint GSM PLMN connection (321)

322	Point to point GSM PLMN connection (320)

1043	Power Class (of MS)	05.05

1044	Power control Level	05.05

1045	Power Control Message	05.08

917	Preventive maintenance	CCITT G.106,N°4104

235	Provision

324	Public Data Network (PDN)

303	Public land mobile network (PLMN)

Q

1046	Quarter Bit Number	05.10

1102	Queuing

R

1107	Radio Link Protocol (RLP)

1048	Radio Sub─System Criteria	05.08

1049	Radio Sub─System Directed Handover	05.08

202	Radiocommunication service

1047	Radiofrequency channel (RFCH)

723	RAND

724	Random number

510	Reference configuration (421)

509	Reference point (420)

237	Registration  ((supplementary) service registration)

811	Rental

709	Reserved

727	Reserved

616	Reset procedure

919	Resource Management	GSM 12.00

617	Restart (Restart procedure)

618	Restoration, Restoration procedure

1050	Roundtrip Delay	05.08

S

725	Security feature

920	Security Management	GSM 12.03

503	Service Access Point

208	Service attribute, telecommunication service attribute (208)

248	Service interworking

803	Service Provider

201	Service, telecommunication service (201)

1051	Serving BS	05.08

1052	Serving Cell	 05.08

1053	SID─frame	06.31

1105	Signalling Interworking

726	Signed Response (SRES)

401	Station

728	Stream cipher

806	Subscriber

729	Subscriber authentication key

730	Subscriber identity authentication

731	Subscriber identity confidentiality

732	Subscriber Identity Module (SIM)

804	Subscription

621	Subsequent handover procedure	03.09

1054	Sub─channel number (SCN)	05.02

1055	Superframe	05.01

210	Supplementary service

249	Supplementary service interworking

316	Switched connection

1056	Synchronization burst (SB)	05.02

921	System change control	GSM 12.06

T

1057	Tail bits	05.03/05.04

1058	TDMA Frame Number	05.02

101	Telecommunication (110)

925	Telecommunications Management Network (TMN)

206	Teleservice (203)

733	Temporary Mobile Subscriber Identity (TMSI)	03.03

513	Terminal adaptation function(TAF)

507	Terminal adaptor (TA)

506	Terminal Equipment (TE) (417)

326	Terminating Network

243	Testing	02.04

226	Three party service (3PTY)

1059	Timebase Counters	05.10

1061	Timeslot	05.01

1063	Timeslot Number (TN)	05.02

1060	Time─frame	05.01

1062	Timing Advance	05.10

922	TMN Standard Interfaces	CCITT M. 2x

1064	Traffic channels	05.02

1065	Training sequence	05.02

1066	Training sequence code	05.02

411	Transceiver (TRX)

250	Transparent support of services

U

1067	Uplink	05.02

1068	Useful part of timeslot	05.04

807	User

501	User access. user network access (402)

234	User─to─user signalling (UUS)

V

605	Visited location register (VLR)	03.01

306	Visited PLMN (VPLMN)

1069	Voice activity detection (VAD)	05.03/05.04

W

236	Withdrawal

923	Work Station (WS)	GSM 12.00

.End Table T.