Annex A

Random access procedures

A.1    Description of the procedure

A.1.1  Procedure in the MS

The random access procedure is initiated by layer 3.  The data link layer will receive the 8 bit information content of the random access burst in a DL-RANDOM ACCESS-REQUEST primitive.The primitive will also contain an indication of the type of channel to be used.  The data link layer will then pass the information field to the indicated physical layer entity in a PH-RANDOM ACCESS-REQUEST primitive.

The physical layer will, upon sending the random access burst, send a PH-RANDOM ACCESS-CONFIRM primitive containing the time slot in which the burst was sent.  This information is passed then to layer 3 in a DL-RANDOM ACCESS-CONFIRM primitive.

A.1.2  Procedure in the BS

The physical layer will provide the random access information field and an indication of the time slot in which the random access burst was received to the data link layer in a PH-RANDOM ACCESS-INDICATION primitive.  The primitive will also contain an indication of the type of channel on which the burst was received.  The data link layer will pass the information to layer 3 in a DL-RANDOM-ACCESS-INDICATION primitive.

A.2  Format

The format of the random access message at the data link layer consists of 8 bits. The order of bit transmission is defined in Rec. GSM 04.04.

ANNEX G

Handling of frames with parameter errors in the address, control and length indicator fields.

G.1    General

The parameters contained in frames received with parameter errors in the address, control and length indicator fields shall be ignored and an information field shall be discarded.

For some parameter errors an indication is given to the mobile management entity in an MDL-ERROR-INDICATION primitive.

G.2    Parameter errors in the address field

G.2.1 Unallocated SAPI

No action shall be taken on frames containing an unallocated SAPI.

G.2.2 Wrong value of the C/R bit

- I frame, C/R bit indicates response

- SABM frame, C/R bit indicates response

- UI frame, C/R bit indicates response

- UA frame, C/R bit indicates command

- DISC frame, C/R bit indicates response

- DM frame, C/R bit indicates command

Actions

An MDL-ERROR-INDICATION primitive with cause "frame not implemented" is sent to the mobile management entity.

G.2.3 EA bit set to "0"

Frames with EA bit set to "0" are not allowed in the GSM system.

Actions

An MDL-ERROR-INDICATION primitive with cause "frame not implemented" is sent to the mobile management entity.

G.3    Parameter errors in the control field

G.3.1. Supervisory frames

The control field coding

bit    8    7    6    5    4    3    2    1

N(R)       P/F   1    1    0    1

is not allowed

Actions

An MDL-ERROR-INDICATION primitive with cause "frame not implemented" is sent to the mobile management entity.

G.3.2 Unnumberd frames

The control field coding

Bit   8    7    6    5    4    3    2    1

x    x    x   P/F   1    0    1    1

x    x    x   P/F   0    1    1    1

0    1    0   P/F   1    1    1    1

0    1    1   P/F   1    1    1    1

1    0    0   P/F   1    1    1    1

1    0    1   P/F   1    1    1    1

1    1    0   P/F   1    1    1    1

1    1    1   P/F   1    1    1    1

0    0    1   P/F   0    1    1    1

1    0    0   P/F   0    0    1    1

1    0    1   P/F   0    0    1    1

1    1    0   P/F   0    0    1    1

1    1    1   P/F   0    0    1    1

are not allowed.

Actions

An MDL-ERROR-INDICATION primitive with cause "frame not implemented" is sent to the mobile management entity.

G.4   Parameter errors in the length indicator field

G.4.1 EL bit error

If the EL bit is set to "0", an MDL-ERROR-INDICATION primitive with cause "frame not implemented" is sent to the mobile management entity.

G.4.2 Information frames

If the length indicator of an I frame is set to a numerical value L>N201 or L=0, an MDL-ERROR-INDICATION primitive with cause "I frame with incorrect length" is sent to the mobile management entity.

If the numerical value of L is L<N201 and the M bit is set to "1", then an MDL-ERROR-INDICATION primitive with cause "I frame with incorrect use of M bit" is sent to the mobile management entity.

G.4.3 Supervisory frames

If a supervisory frame is received with L>0 or with the M bit set to "1", an MDL-ERROR-INDICATION primitive with cause "S frame with incorrect parameters" is sent to the mobile management entity.

G.4.4 DISC and DM frames

If a DISC or DM frame is received with L>0 or with the M bit set to "1", an MDL-ERROR-INDICATION primitive with cause "U frame with incorrect parameters" is sent to the mobile management entity.

G.4.5 SABM UA and UI frames

If SABM, UA or UI frames are received with L>N201 or with the M bit set to "1" an MDL-ERROR-INDICATION primitive with cause "U frame with incorrect parameters" is sent to the mobile management entity.