ETSI/GSM			                               Version 3.1.0

RECOMMENDATION GSM 04.08 - DCS

Title:	Mobile Radio Interface Layer 3 Specification

(based on version 3.13.0 of GSM 04.08)

Date: October 1991

(detailed list of contents given on next pages)

List of Contents:

1		General

2		Overview of control procedures

3		RR management procedures

4		Elementary procedures for mobility management

5		Elementary procedures for circuit-switched call control

6		Packet communication across the radio interface

7		Structured procedures

8		Handling of error conditions

9		Message functional definitions and contents

10		General message format and information elements coding

11		List of system parameters

Annex B	Compatibility checking

Annex C	Low layer information coding principles

Annex D	Examples of bearer capability information element coding

Annex E	Comparison between Q.931 and Rec. GSM 04.08

Annex F	GSM specific cause values for radio resource management

Annex G	GSM specific cause values for mobility management

Annex H	Cause values for call control

Annex J	Algorithm to encode frequency list information elements

Number of pages: 82

Detailed list of contents:

1 GENERAL

1.1 Scope of the Recommendation	15

3 RR-MANAGEMENT PROCEDURES

3.2 Idle mode procedures		28

3.2.1 MS side

3.2.2 Network side

4 ELEMENTARY PROCEDURES FOR MOBILITY MANAGEMENT

4.4 MM-specific procedures

4.4.1 Location updating procedure	74

4.4.4.6   Location updating not accepted by the network	79

9 MESSAGES FUNCTIONAL DEFINITIONS AND CONTENTS

9.1 Messages for radio resource management	176

9.1.2 Assignment command	179

9.1.14 Handover command	192

9.1.30a System information type 2bis                                    212a

9.1.33a System information type 5bis                                    215a

10  GENERAL MESSAGE FORMAT AND INFORMATION ELEMENTS CODING

10.4 Message type			280

10.5 Other information elements

10.5.2 Radio resources management information elements	304

10.5.2.1 Cell Channel description	306

10.5.2.2 Cell description	308

10.5.2.5 Channel description	314

10.5.2.9 Frequency channel sequence	323

10.5.2.11 Measurement ressults	326

10.5.2.12 Mobile Allocation	330

10.5.2.13 Neighbour cells description	333

10.5.3.6 Reject cause	357

Annex G - GSM specific cause values for mobility management	435

Annex J - Algorithm to encode frequency list information elements              446a

GSM 04.08-DCS - 3.1.0 - page 15

1         GENERAL

This Recommendation specifies the procedures used at the radio interface (Reference Point Um, see GSM Rec. 04.02) for call control, mobility management and Radio Resource  management.

When the mention for "further study" or "FS" or "FFS" is present throughout this document this means this is not relevant for ETSI-GSM phase 1 standard.

These procedures are defined in terms of messages exchanged over the control channels of the radio interface. The control channels are described in Rec. GSM 04.03.

The structured functions and procedures of this protocol and the relationship with other layers and entities are described in general terms in Rec. GSM 04.07.

1.1       Scope of the Recommendation

The procedures currently described in this Recommendation are for the call control of circuit-switched connections, the control of packet-mode communication, mobility management and radio resource management. The transport of other message-based information flows is a subject for further study.

Procedures for supplementary services are contained in Recommendation GSM 04.10.

Note 1:  The term "layer 3" is used for the functions and protocol described in this Recommendation.

The terms "data link layer" and "layer 2" are used interchangeably to refer to the layer immediately below layer 3.

This delta recommendation only includes modified parts concerning specifically the DCS 1800 short term solution (phase 1).

GSM 04.08-DCS - 3.1.0 - page 28

3.2       Idle mode procedures

3.2.1     MS side

In idle mode, the MS listens to the BCCH and to the paging sub-channel for the paging group the MS belongs to (Cf. Rec. GSM 03.13); it measures the radio propagation for connection with other cells.

Measurements are treated to assess the need of a cell change as specified in Rec. GSM 05.08. When a cell change is decided, the MS switches to the BCCH of the new cell. The broadcast information is then checked to verify the allowance to attach to this cell (cf. section 3.2.2). If allowed, the cell change is confirmed, and the broadcast information is then treated for mobility management actions (cf. section 4). Similarly, physical contexts are updated (list of neighbouring cells frequencies, thresholds for some actions, etc... cf. Rec. GSM 05.08 and section 3.2.2).

3.2.2     Network side

3.2.2.1   System information broadcasting

SYSTEM INFORMATION TYPE 1 to 4 messages, and optionaly TYPE 2bis, are regularly broadcast by the network on the BCCH. Based on this information the Mobile Station is able to decide whether and how it may gain access to the system via the current cell.

Note: The exact order of sending SYSTEM INFORMATION messages on the BCCH is specified in Rec. GSM 05.02.

The same information is sent in SYSTEM INFORMATION TYPE 5 and, 6 and optionaly 5bis, on the SACCH just after handover, and whenever there is no other use of that channel.

The information broadcast may be grouped in the following classes:

- information giving unique identification of the current network, location area and cell;

- information used for cell selection and candidate cell measurements for handover procedures;

- information describing the current control channel structure;

- information controlling the random access channel utilisation; and

- information defining different options supported within the cell.

3.2.2.2   Paging

The network is required to send valid layer 3 messages continuously on all paging subchannels.

GSM 04.08-DCS - 3.1.0 - page 74

4.4.1     Location updating procedure

The location updating procedure is a general procedure which is used for the following purposes:

-    normal location updating (described in this section);

-    periodic updating (see section 4.4.2);

-    IMSI attach (see section 4.4.3).

The normal location updating procedure is used to update the registration of the actual Location Area of a Mobile Station in  the network.  The location updating type information element in the LOCATION UPDATING REQUEST message shall indicate normal location updating. The location updating procedure is only performed if a subscriber identity module  is  available  in the  Mobile Station. It is initiated when the Location Area Identification received on the BCCH of the current serving cell (chosen as specified in Rec. GSM 05.08) differs from the value  stored , e.g. if:

i)   the Mobile Station has been switched on in a Location Area   different from the one stored in a non volatile memory or when there is no stored Location Area Identification;

ii)  the Mobile Station moves across the boundaries of a Location Area while being in the "idle" state;

The location updating procedure is also started if the network  indicates that the Mobile Station is unknown in the VLR as a response to MM-connection establishment request.

To limit the number of location updating attempts made, where location updating is unsuccessful, an attempt counter is used. The attempt counter is reset when a Mobile Station is switched on or a SIM card is inserted.

Upon successful location updating the Mobile Station stores the received Location Area Identification in a non-volatile memory and the attempt counter shall be reset.

The detailed handling of the attempt counter is described in 4.4.4.5 to 4.4.4.8.

The Mobile Equipment shall contain a list of "forbidden location areas for national roaming". The list shall be erased when the MS is switched off or when the SIM is removed, and periodically (with period in the range 12 to 24 hours). The location area identification received on the BCCH that triggered the location updating request shall be added to the list whenever a location update reject message is received with the cause "National roaming not allowed in this location area". The list shall accommodate 10 or more location area identifications. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

The  cell selection processes in the different states are described in Rec. GSM 05.08.

The location updating procedure is always initiated by the Mobile  Station.

GSM 04.08-DCS - 3.1.0 - page 79

4.4.4.6   Location updating not accepted by the network

If the location updating cannot be accepted the network sends a LOCATION UPDATING REJECT message to the Mobile Station.

The Mobile Station receiving a LOCATION UPDATING REJECT message shall stop the timer T3210, store the reject cause, start T3240 and enter the location updating rejected state awaiting the release of the RR-connection triggered by the network. Upon the release of the RR-connection the MS shall take the following actions depending on the stored reject cause:

# 2 (IMSI unknown in HLR), or

# 3 (Illegal MS):

For the Mobile Station this corresponds to authentication failure. The Mobile Station shall delete any TMSI, stored LAI, cipher key and cipher key sequence number and enter the "idle, no IMSI" state. In this state the Mobile Station shall act in the following way:

- not start any normal location updating attempt

- not perform periodic updating

- not perform IMSI detach if powered down

- reject any requests from CM entities for MM-connections except emergency calls

- not respond to paging

GSM 04.08-DCS - 3.1.0 - page 80

# 11 (PLMN not allowed):

# 12 (Location Area not allowed):

The Mobile Station shall delete any TMSI, cipher key and cipher key sequence number, reset the attempt counter and in the case of normal location updating, store the LAI received on the BCCH that triggered the location updating request, and then enter the "idle, roaming not allowed" state and memorize which of the above mentioned causes has been received. In this state, it shall in case of "Location Area not allowed":

- perform normal location updating when a new location area is entered - not perform periodic updating

- not perform IMSI detach

- not perform IMSI attach if activated in the same location area

- reject any requests from CM entities for MM-connections except emergency calls

- respond to paging (with IMSI) when roaming not allowed in this area.

In case of "PLMN not allowed", it shall:

- perform normal location updating when a new PLMN is entered

- not perform periodic updating

- not perform IMSI detach

- not perform IMSI attach if activated in the same location area

- reject any requests from CM entities for MM connections except emergency calls, and

- it may respond to paging (with IMSI).

# 13 (National roaming not allowed in this location area)

The Mobile Station shall reset the attempt counter, add to the "list of forbidden location areas for national roaming", the location area identification received on the BCCH that triggered the location updating request, and perform the cell selection procedure as defined in GSM 05.08 - DCS, with the HPLMN as the selected PLMN (see  GSM 02.11 - DCS).

Other values are considered as abnormal cases and the specification of the MS behaviour in those cases is given in section 4.4.4.8.

GSM 04.08-DCS - 3.1.0 - page 176

9.1 Messages for radio resources management

Table 9.1/GSM 04.08 summarizes the messages for radio resources management.

┌───────────────────────────────────────────────────────┐

│                                                       │

│  Channel establishment messages:           Reference  │

│                                                       │

│  ADDITIONAL ASSIGNMENT                     9.1.1      │

│  IMMEDIATE ASSIGNMENT                      9.1.17     │

│  IMMEDIATE ASSIGNMENT EXTENDED             9.1.18     │

│  IMMEDIATE ASSIGNMENT REJECT               9.1.19     │

│                                                       │

│  Ciphering messages:                       Reference  │

│                                                       │

│  CIPHERING MODE COMMAND                    9.1.9      │

│  CIPHERING MODE COMPLETE                   9.1.10     │

│                                                       │

│  Handover messages:                        Reference  │

│                                                       │

│  ASSIGNMENT COMMAND                        9.1.2      │

│  ASSIGNMENT COMPLETE                       9.1.3      │

│  ASSIGNMENT FAILURE                        9.1.4      │

│  HANDOVER ACCESS                           9.1.13     │

│  HANDOVER COMMAND                          9.1.14     │

│  HANDOVER COMPLETE                         9.1.15     │

│  HANDOVER FAILURE                          9.1.16     │

│  PHYSICAL INFORMATION                      9.1.27     │

│                                                       │

│  Channel release messages:                 Reference  │

│                                                       │

│  CHANNEL RELEASE                           9.1.7      │

│  PARTIAL RELEASE                           9.1.25     │

│  PARTIAL RELEASE COMPLETE                  9.1.26     │

│                                                       │

│  Paging messages:                          Reference  │

│                                                       │

│  PAGING REQUEST TYPE 1                     9.1.21     │

│  PAGING REQUEST TYPE 2                     9.1.22     │

│  PAGING REQUEST TYPE 3                     9.1.23     │

│  PAGING RESPONSE                           9.1.24     │

│                                                       │

└───────────────────────────────────────────────────────┘

TABLE 9.1/GSM 04.08

Messages for radio resources management

GSM 04.08-DCS - 3.1.0 - page 177

┌───────────────────────────────────────────────────────┐

│                                                       │

│  System information messages:              Reference  │

│                                                       │

│  SYSTEM INFORMATION TYPE 1                 9.1.29     │

│  SYSTEM INFORMATION TYPE 2                 9.1.30     │

│  SYSTEM INFORMATION TYPE 2bis              9.1.30a    │

│  SYSTEM INFORMATION TYPE 3                 9.1.31     │

│  SYSTEM INFORMATION TYPE 4                 9.1.32     │

│  SYSTEM INFORMATION TYPE 5                 9.1.33     │

│  SYSTEM INFORMATION TYPE 5bis              9.1.33a    │

│  SYSTEM INFORMATION TYPE 6                 9.1.34     │

│                                                       │

│  Miscellaneous messages:                   Reference  │

│                                                       │

│  CHANNEL MODE MODIFY                       9.1.5      │

│  CHANNEL MODE MODIFY ACKNOWLEDGE           9.1.6      │

│  CHANNEL REQUEST                           9.1.8      │

│  CLASSMARK CHANGE                          9.1.11     │

│  FREQUENCY REDEFINITION                    9.1.12     │

│  MEASUREMENT REPORT                        9.1.20     │

│  SYNCHRONISATION CHANNEL INFORMATION       9.1.28     │

│  RR-STATUS                                 9.1.27a    │

│                                                       │

└───────────────────────────────────────────────────────┘

TABLE 9.1/GSM 04.08

Messages for radio resources management

(continued)

GSM 04.08-DCS - 3.1.0 - page 179

9.1.2     Assignment command

This message is sent on the main DCCH by the network to the mobile station to change the channel configuration to another independent dedicated channel configuration, no timing adjustment needed. See Table 9.3/GSM 04.08

Message type: ASSIGNMENT COMMAND

Significance: dual

Direction:    network to mobile station

┌──────────────────────┬─────────┬─────────┬────┬──────┐

│ Information element  │Reference│Direction│Type│Length│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Protocol discriminator│  10.2   │ n -> ms │ MF │ ┐    │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Transaction identifier│  10.3   │ n -> ms │ MF │ │ 2  │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Message type          │  10.4   │ n -> ms │ MF │ ┘    │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel description   │10.5.2.5 │ n -> ms │ MF │  3   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Power command         │10.5.2.16│ n -> ms │ MF │  1   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Frequency list      7)│10.5.2.9a│ n -> ms │ OV │ 4-131│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Cell channel descr. 6)│10.5.2.1 │ n -> ms │ OF │  17  │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel mode        1)│10.5.2.6 │ n -> ms │ OF │  2   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel description 2)│10.5.2.5 │ n -> ms │ OF │  4   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel mode 2      3)│10.5.2.6a│ n -> ms │ OF │  2   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Mobile allocation   4)│10.5.2.12│ n -> ms │ OV │ 2-10 │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Starting time       5)│10.5.2.20│ n -> ms │ OF │  3   │

└──────────────────────┴─────────┴─────────┴────┴──────┘

TABLE 9.3/GSM 04.08

ASSIGNMENT COMMAND message content

GSM 04.08-DCS - 3.1.0 - page 180

Note 1: This information element appears if the channel mode is changed for the channel defined in the mandatory part of the message

Note 2: This information element appears in the case of a so-called intracell handover or an assignment occuring after a call reestablishment if the MS carries two connections (on two dedicated channels, e.g. Lm+Lm).

The connection using the channel previously defined in the mandatory part of an ASSIGNMENT COMMAND or HANDOVER COMMAND message shall use the channel defined in the mandatory part of the ASSIGNMENT COMMAND message defining the new configuration.

The first indicated channel (i.e. in the mandatory part) carries the main DCCH. The SACCH used is the one associated with that channel.

Note 3: this information element apears if te channel mode is changed for the channel defined in the optional channel description information element.

Note 4: This information element appears in the case of frequency hopping. It applies to all assigned channels. This element shall not be included if the frequency list information element is present.

Note 5: This information element appears in particular if a frequency change is in progress.

Note 6: This information element if present must be used to decode the mobile allocation in the same message and in subsequent messages.

Note 7: This information element provides the mobile allocation used for frequency hopping. If this element is present, then the cell channel description information element is not required, and the mobile allocation information element shall not be included.

GSM 04.08-DCS - 3.1.0 - page 192

9.1.14    Handover command

This message is sent on the main DCCH by the network to the mobile station to change the dedicated channel configuration, timing adjustment needed. See Table 9.15/GSM 04.08.

Message type: HANDOVER COMMAND

Significance: dual

Direction:    network to mobile station

┌──────────────────────┬─────────┬─────────┬────┬──────┐

│ Information element  │Reference│Direction│Type│Length│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Protocol discriminator│  10.2   │ n -> ms │ MF │ ┐    │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Transaction identifier│  10.3   │ n -> ms │ MF │ │ 2  │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Message type          │  10.4   │ n -> ms │ MF │ ┘    │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Cell description      │10.5.2.2 │ n -> ms │ MF │  2   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel description   │10.5.2.5 │ n -> ms │ MF │  3   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Handover reference    │10.5.2.10│ n -> ms │ MF │  1   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Power command         │10.5.2.16│ n -> ms │ MF │  1   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Synchronisation       │10.5.2.21│ n -> ms │ OF │  1   │

│indication          1)│         │         │    │      │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Frequency short     9)│10.5.2.9b│ n -> ms │ OF │  10  │

│list                  │         │         │    │      │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Frequency list      9)│10.5.2.9a│ n -> ms │ OV │ 4-131│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Cell channel          │10.5.2.1 │ n -> ms │ OF │  17  │

│description         2)│         │         │    │      │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel mode        3)│10.5.2.6 │ n -> ms │ OF │  2   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel description 4)│10.5.2.5 │ n -> ms │ OF │  4   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Channel mode 2      5)│10.5.2.6a│ n -> ms │ OF │  2   │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Frequency channel     │10.5.2.9 │ n -> ms │ OF │  10  │

│sequence          6 ) │         │         │    │      │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Mobile allocation 7)9)│10.5.2.12│ n -> ms │ OV │ 2-10 │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Starting time     8 ) │10.5.2.20│ n -> ms │ OF │  3   │

└──────────────────────┴─────────┴─────────┴────┴──────┘

TABLE 9.15/GSM 04.08

HANDOVER COMMAND message content

GSM 04.08-DCS - 3.1.0 - page 193

Note 1: If this information element is omitted, the default value is "non-synchronized".

Note 2: This information element appears if frequency hopping is used on the new cell.

Note 3: this element appears if the channel mode is changed for the channel defined in the mandatory part of the message.

Note 4: This information element appears if the MS carries two connections (on two dedicated channels, e.g. Lm+Lm).

The connection using the channel previously defined in the mandatory part of an ASSIGNMENT COMMAND or HANDOVER COMMAND message shall use the channel defined in the mandatory part of the HANDOVER COMMAND message defining the new configuration.

The first indicated channel (i.e. in the mandatory part) carries the main DCCH. The SACCH used is the one associated with that channel.

Note 5: This element appears if the channel mode is changed for the channel defined in the optional channel description information element.

Note 6: This element is a combination of mobile allocation element and cell channel description element. It is designed to allow the sending of the HANDOVER COMMAND in one signalling block for systems using frequency hopping. If this element is present, then the cell channel description and mobile allocation information elements are not required. This element shall not be included for DCS1800 application.

Note 7: This information element appears if frequency hopping is used on the new cell. If it appears, it applies to all assigned channels. This information element cannot appear if the cell channel description information element is not present.

Note 8: This information element appears if a frequency change is in progress. It refers to the new cell time.

Note 9: One and only one of thhese information elements appears if the allocated channels are frequency hopping channels. They apply to all allocated channels

GSM 04.08-DCS - 3.1.0 - page 212a

9.1.30a    System information type 2bis

This message is sent optionnally on the BCCH by the network to all mobile stations within the cell giving information of control of the RACH and of the extension of the BCCH allocation in the neighbour cells. See Table 9.31a/GSM 04.08.

Message type: SYSTEM INFORMATION TYPE 2bis

Significance: dual

Direction:    network to mobile station

┌──────────────────────┬─────────┬─────────┬────┬──────┐

│ Information element  │Reference│Direction│Type│Length│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Protocol discriminator│  10.2   │ n -> ms │ MF │ ┐    │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Transaction identifier│  10.3   │ n -> ms │ MF │ │ 2  │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Message type          │  10.4   │ n -> ms │ MF │ ┘    │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Neighbour cells desc- │10.5.2.13│ n -> ms │ MF │  16  │

│-ription (extension)  │         │         │    │      │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│RACH control          │10.5.2.17│ n -> ms │ MF │  3   │

│parameters            │         │         │    │      │

└──────────────────────┴─────────┴─────────┴────┴──────┘

TABLE 9.31a/GSM 04.08

SYSTEM INFORMATION TYPE 2bis message content

GSM 04.08-DCS - 3.1.0 - page 215a

9.1.33a    System information type 5bis

This message is sent optionnally on the SACCH by the network to mobile stations within the cell giving information on the extension of the BCCH allocation in the neighbour cells. See Table 9.34a/GSM 04.08.

Message type: SYSTEM INFORMATION TYPE 5bis

Significance: dual

Direction:    network to mobile station

┌──────────────────────┬─────────┬─────────┬────┬──────┐

│ Information element  │Reference│Direction│Type│Length│

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Protocol discriminator│  10.2   │ n -> ms │ MF │ ┐    │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Transaction identifier│  10.3   │ n -> ms │ MF │ │ 2  │

├──────────────────────┼─────────┼─────────┼────┤ │    │

│Message type          │  10.4   │ n -> ms │ MF │ ┘    │

├──────────────────────┼─────────┼─────────┼────┼──────┤

│Neighbour cells descr-│10.5.2.13│ n -> ms │ MF │  16  │

│-ription (extension)  │         │         │    │      │

└──────────────────────┴─────────┴─────────┴────┴──────┘

TABLE 9.34a/GSM 04.08

SYSTEM INFORMATION TYPE 5bis message content

GSM 04.08-DCS - 3.1.0 - page 280

10.4      Message Type

The purpose of the message type is to identify the function of the message being sent.

The message type is the third part of every message . The message type is coded as shown in Figure 10.4/GSM 04.08 and Tables 10.3-10.5/GSM 04.08.

Bit 8 is reserved for possible future use as an extension bit.

Bit 7 in the MM- and CM-messages sent from the mobile station is reserved for the send sequence number N(SD) (see section 2.4.1) In all other messages bit 7 is set to 0.

8     7     6     5     4     3     2     1

┌─────┬─────┬───────────────────────────────────┐

│  0  │N(SD)│      Message type                 │ octet 1

├─────┴─────┴───────────────────────────────────┤

FIGURE 10.4/04.08

Message type

Messages with different protocol discriminators are permitted to have the same message type i.e. the function of a message is determined by the protocol discriminator and the message type together.

GSM 04.08-DCS - 3.1.0 - page 281

┌───────────────────────────────────────────────────┐

│ 8 7 6 5 4 3 2 1                                   │

│                                                   │

│ 0 0 1 1 1 - - -  Channel establishment messages:  │

│           0 1 1  - ADDITIONAL ASSIGNMENT          │

│           1 1 1  - IMMEDIATE ASSIGNMENT           │

│           0 0 1  - IMMEDIATE ASSIGNMENT EXTENDED  │

│           0 1 0  - IMMEDIATE ASSIGNMENT REJECT    │

│                                                   │

│ 0 0 1 1 0 - - -  Ciphering messages:              │

│           1 0 1  - CIPHERING MODE COMMAND         │

│           0 1 0  - CIPHERING MODE COMPLETE        │

│                                                   │

│ 0 0 1 0 1 - - -  Handover messages:               │

│           1 1 0  - ASSIGNMENT COMMAND             │

│           0 0 1  - ASSIGNMENT COMPLETE            │

│           1 1 1  - ASSIGNMENT FAILURE             │

│           0 1 1  - HANDOVER COMMAND               │

│           1 0 0  - HANDOVER COMPLETE              │

│           0 0 0  - HANDOVER FAILURE               │

│           1 0 1  - PHYSICAL INFORMATION           │

│                                                   │

│ 0 0 0 0 1 - - -  Channel release messages:        │

│           1 0 1  - CHANNEL RELEASE                │

│           0 1 0  - PARTIAL RELEASE                │

│           1 1 1  - PARTIAL RELEASE COMPLETE       │

│                                                   │

│ 0 0 1 0 0 - - -  Paging messages:                 │

│           0 0 1  - PAGING REQUEST TYPE 1          │

│           0 1 0  - PAGING REQUEST TYPE 2          │

│           1 0 0  - PAGING REQUEST TYPE 3          │

│           1 1 1  - PAGING RESPONSE                │

│                                                   │

└───────────────────────────────────────────────────┘

TABLE 10.3/GSM 04.08 (page 1 of 2)

Message types for radio resources management

GSM 04.08-DCS - 3.1.0 - page 282

┌───────────────────────────────────────────────────┐

│ 8 7 6 5 4 3 2 1                                   │

│                                                   │

│ 0 0 0 1 1 - - -  System information messages:     │

│           0 0 1  - SYSTEM INFORMATION TYPE 1      │

│           0 1 0  - SYSTEM INFORMATION TYPE 2      │

│           0 1 1  - SYSTEM INFORMATION TYPE 3      │

│           1 0 0  - SYSTEM INFORMATION TYPE 4      │

│           1 0 1  - SYSTEM INFORMATION TYPE 5      │

│           1 1 0  - SYSTEM INFORMATION TYPE 6      │

│                                                   │

│ 0 0 0 0 0 - - -  System information messages:     │

│           0 1 0  - SYSTEM INFORMATION TYPE 2bis   │

│           1 0 1  - SYSTEM INFORMATION TYPE 5bis   │

│                                                   │

│ 0 0 0 1 0 - - -  Miscellaneous messages:          │

│           0 0 0  - CHANNEL MODE MODIFY            │

│           1 1 1  - CHANNEL MODE MODIFY ACKNOWLEDGE│

│           1 0 0  - FREQUENCY REDEFINITION         │

│           1 0 1  - MEASUREMENT REPORT             │

│           1 1 0  - CLASSMARK CHANGE               │

└───────────────────────────────────────────────────┘

TABLE 10.3/GSM 04.08 (page 2 of 2)

Message types for radio resources management

Note: Bit 8 is reserved for possible future use as an extension bit.

GSM 04.08-DCS - 3.1.0 - page 304

10.5.2    Radio resources management information elements.

For the radio resources management information elements listed below, the coding of the information element identifier bits is summarized in Table 10.13/GSM 04.08.

┌──────────────────────────────────────┬─────────┬─────────┐

│                                      │Reference│Length in│

│                                      │ section │ octets  │

│                                      │         │   1)    │

├──────────────────────────────────────┼─────────┼─────────┤

│8 7 6 5 4 3 2 1                       │         │         │

│                                      │         │         │

│1 : : : - - - -  Type 1 info elements │         │         │

│  0 0 1 - - - -  Cipher mode setting  │10.5.2.7 │   F1    │

│  0 1 1 - - - -  Page mode            │10.5.2.14│   F1    │

│  1 0 1 - - - -  Synchronisation      │10.5.2.21│   F1    │

│                 indication           │         │         │

│                                      │         │         │

│0 : : : : : : :  Type 3 & 4 info      │         │         │

│                 elements             │         │         │

│  0 0 0 0 0 1 0  Frequency short list │10.5.2.9b│   F10   │

│  0 0 0 0 1 0 1  Frequency list       │10.5.2.9a│ Max 131 │

│  1 1 0 0 0 0 1  Cell description     │10.5.2.2 │   F3    │

│  1 1 0 0 0 1 0  Cell channel         │10.5.2.1 │   F17   │

│                 description          │         │         │

│  1 1 0 0 0 1 1  Channel mode         │10.5.2.6 │   F2    │

│  1 1 0 0 1 0 0  Channel description  │10.5.2.5 │   F4    │

│  1 1 0 0 1 1 0  Channel mode 2       │10.5.2.6a│   F2    │

│  1 1 0 1 0 0 0  Control channel      │10.5.2.8 │   F4    │

│                 description          │         │         │

│  1 1 0 1 0 0 1  Frequency channel    │10.5.2.9 │   F10   │

│                 sequence             │         │         │

│  1 1 0 1 0 1 0  Handover reference   │10.5.2.10│   F2    │

│  1 1 0 1 0 1 1  Cell options         │10.5.2.3 │   F2    │

│  1 1 0 1 1 0 0  Cell selection       │10.5.2.4 │   F3    │

│                 parameters           │         │         │

└──────────────────────────────────────┴─────────┴─────────┘

TABLE 10.13/GSM 04.08 (page 1 of 2)

Information element identifier coding for radio

resources management information elements

GSM 04.08 v. 3.8.0 - page 305

┌──────────────────────────────────────┬─────────┬─────────┐

│                                      │Reference│Length in│

│                                      │ section │ octets  │

│                                      │         │   1)    │

├──────────────────────────────────────┼─────────┼─────────┤

│8 7 6 5 4 3 2 1                       │         │         │

│                                      │         │         │

│  1 1 1 0 0 0 1  Measurement results  │10.5.2.11│   F17   │

│  1 1 1 0 0 1 0  Mobile allocation    │10.5.2.12│ Max. 10 │

│  1 1 1 0 1 0 0  Neighbour cells      │10.5.2.13│   F17   │

│                 description          │         │         │

│  1 1 1 0 1 0 1  Power command        │10.5.2.16│   F2    │

│  1 1 1 0 1 1 0  PLMN permitted       │10.5.2.15│   F2    │

│  1 1 1 1 0 0 0  RACH control param.  │10.5.2.17│   F4    │

│  1 1 1 1 0 0 1  Request reference    │10.5.2.18│   F4    │

│  1 1 1 1 0 1 0  RR cause             │10.5.2.19│   F2    │

│  1 1 1 1 1 0 0  Starting time        │10.5.2.20│   F3    │

│  1 1 1 1 1 0 1  Timing advance       │10.5.2.22│   F2    │

│  1 1 1 1 1 1 0  TMSI                 │10.5.2.23│   F5    │

│  1 1 1 1 1 1 1  Wait indication      │10.5.2.24│   F2    │

│                                      │         │         │

│  All other values are reserved  2)   │         │         │

└──────────────────────────────────────┴─────────┴─────────┘

TABLE 10.13/GSM 04.08 (page 2 of 2)

Information element identifier coding for radio

resources management information elements

Note 1: For fixed length information elements the length is indicated as F length value e.g. F3. For variable length information elements the length is indicated as Max length value e.g. Max 10.

The indicated length is the length included information element identifier and a possible length indicator. When an information element is mandatory in a message the length is reduced with 1 octet as the information element identifier is stripped off.

Note 2: The reserved value with bits 5-8 coded "0000" are for future information elements for which comprehension by the receiver is required (see section 8.8.1 Unrecognized information element).

GSM 04.08-DCS - 3.1.0 - page 306

10.5.2.1  Cell channel description

The purpose of the cell channel description information element is to provide the absolute radio frequency  channel numbers used in a cell the reference frequency list to be used to decode the mobile allocation information element.

The cell channel description information element is coded as shown in Figure 10.20/GSM 04.08 and Table 10.14/GSM 04.08.

The cell channel description is a type 3 information element with 17 octets length.

There are several formats for the cell channel description information element, distinguished by the "format indicator" subfield. Some formats are frequency bit maps, the others use a special encoding scheme.

10.5.2.1.1	General description

Figure 10.20/04.08 shows only a special bit numbering. The different general format is described in Table 10.14/04.08.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┼─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│     │     │ EXT-│  0  │     │     │     │     │

│ Bit │ Bit │ IND │spare│ Bit │ Bit │ Bit │ Bit │ octet 2

│ 128 │ 127 │     │     │ 124 │ 123 │ 122 │ 121 │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ octet 3

│ 120 │ 119 │ 118 │ 117 │ 116 │ 115 │ 114 │ 113 │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │octet 17

│ 008 │ 007 │ 006 │ 005 │ 004 │ 003 │ 002 │ 001 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.20/GSM 04.08

Cell Channel description information element (general format)

GSM 04.08-DCS - 3.1.0 - page 307

┌─────────────────────────────────────────────────────┐

│ CA-NO, Cell allocation number (octet 2)             │

│ Bits                                                │

│ 8 7                                                 │

│ 0 0      Band number 0                              │

│                                                     │

│ All other values are reserved                       │

│                                                     │

│ EXT-IND, Extension indication (octet 2, bit 6)      │

│                                                     │

│ This bit indicates whether the information element  │

│ carries the complete CA or whether a complementary  │

│ information element is sent in another message.     │

│                                                     │

│   Note : This indicator is set to 1 in the two      │

│          cell channels   description information    │

│          fields when two are needed to describe the │

│          frequency list.                            │

│                                                     │

│ Bit 6                                               │

│                                                     │

│   0       The information elements carries the      │

│           complete CA                               │

│   1       The information elements carries only a   │

│           part of the CA                            │

│                                                     │

│ FORMAT-ID, Format Identifier (Bit 128 and next)     │

│                                                     │

│ The different formats are distinguished by the bits │

│ of higher number. The possible values are the       │

│ following :                                         │

│                                                     │

│  Bit Bit Bit Bit Bit          format notation       │

│  128 127 124 123 122                                │

│                                                     │

│   0   0   X   X   X               bit map 0         │

│   1   0   0   X   X               1024 range        │

│   1   0   1   0   0               512 range         │

│   1   0   1   0   1               256 range         │

│   1   0   1   1   0               128 range         │

│   1   0   1   1   1               variable bit map  │

│                                                     │

│                                                     │

│ All other combinations are reserved for future use. │

│                                                     │

│ The signification of the remaining bits depends on  │

│ FORMAT-ID. The different cases are specified in     │

│ the next sections.                                  │

│                                                     │

└─────────────────────────────────────────────────────┘

TABLE 10.14/GSM 04.08

Cell channel description information element, general format

GSM 04.08-DCS - 3.1.0 - page 307a

10.5.2.1.2	Bit map 0 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│  0     0  │EXT- │  0  │ CA  │ CA  │ CA  │ CA  │

│   CA-NO   │  IND│spare│ARFCN│ARFCN│ARFCN│ARFCN│ octet 2

│FORMAT-ID  │     │     │ 124 │ 123 │ 122 │ 121 │

├─────┬─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │

│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ octet 3

│ 120 │ 119 │ 118 │ 117 │ 116 │ 115 │ 114 │ 113 │

├─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │ CA  │

│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│octet 17

│ 008 │ 007 │ 006 │ 005 │ 004 │ 003 │ 002 │ 001 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.20a/GSM 04.08

Cell Channel description information element, bit map 0 format

┌─────────────────────────────────────────────────────┐

│ CA-NO, Cell allocation number (octet 2)             │

│ Bits                                                │

│ 8 7                                                 │

│ 0 0      Band number 0                              │

│                                                     │

│ All other values are reserved                       │

│                                                     │

│ CA  ARFCN  N,  Cell Allocation Absolute RF Channel  │

│ Number N (octet 2 etc.)                             │

│                                                     │

│ For  a  RF channel with ARFCN = N belonging to the  │

│ cell allocation the CA ARFCN N bit is coded with a  │

│ "1"; N = 1, 2, .. , 124.                            │

│                                                     │

│ For  a  RF channel with ARFCN = N not belonging to  │

│ the  cell  allocation  the CA ARFCN N bit is coded  │

│ with a "0"; N = 1, 2 .. , 124.                      │

└─────────────────────────────────────────────────────┘

TABLE 10.14a/GSM 04.08

Cell channel description information element, bit map 0 format

GSM 04.08-DCS - 3.1.0 - page 307b

10.5.2.1.3	Range 1024 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────┬─────┬───────────┤

│  1     0  │EXT- │  0  │  0  │     │           │

│ FORMAT-ID │  IND│spare│FORMA│ F0  │    W(1)   │ octet 2

│           │     │     │T-ID │     │(high part)│

├───────────┴─────┴─────┴─────┴─────┴───────────┤

│              W(1)  (low part)                 │ octet 3

├───────────────────────────────────────────────┤

│              W(2)  (high part)                │ octet 4

├─────┬─────────────────────────────────────────┤

│ W(2)│                W(3)                     │ octet 5

│(low)│             (high part)                 │

├─────┴─────┬───────────────────────────────────┤

│    W(3)   │            W(4)                   │ octet 6

│ (low part)│         (high part)               │

├───────────┼───────────────────────────────────┤

│    W(4)   │            W(5)                   │ octet 7

│ (low part)│         (high part)               │

├───────────┼───────────────────────────────────┤

│    W(5)   │            W(6)                   │ octet 8

│ (low part)│         (high part)               │

├───────────┼───────────────────────────────────┤

│    W(6)   │            W(7)                   │ octet 9

│ (low part)│         (high part)               │

├───────────┼───────────────────────────────────┤

│    W(7)   │            W(8)                   │ octet 10

│ (low part)│         (high part)               │

├─────┬─────┴───────────────────────────────────┤

│ W(8)│               W(9)                      │ octet 11

│(low)│                                         │

├─────┴───────────────────────────────────┬─────┤

│              W(10)                      │W(11)│ octet 12

│                                         │high │

├───────────────────────────────────┬─────┴─────┤

│          W(11)                    │   W(12)   │ octet 13

│        (low part)                 │(high part)│

├─────────────────────────────┬─────┴───────────┤

│   W(12) (low part)          │W(13) (high part)│ octet 14

├───────────────────────┬─────┴─────────────────┤

│   W(13) (low part)    │   W(14) (high part)   │ octet 15

├─────────────────┬─────┴───────────────────────┤

│ W(14) (low part)│         W(15) (high part)   │ octet 16

├───────────┬─────┴─────────────────────────────┤

│  W(15)    │             W(16)                 │ octet 17

│(low part) │                                   │

└───────────┴───────────────────────────────────┘

FIGURE 10.20b/GSM 04.08

Cell Channel description information element (1024 range format)

GSM 04.08-DCS - 3.1.0 - page 307c

┌─────────────────────────────────────────────────────────┐

│                                                         │

│ F0, frequency 0 indicator (octet 2, bit 4) :            │

│                                                         │

│ 0      frequency of ARFCN 0 is not in the list          │

│ 1      frequency of ARFCN 0 is in the list              │

│                                                         │

│ W(i), i from 1 to 16 (octet 2 to 17) :                  │

│                                                         │

│ Each W(i) encodes a non negative integer in binary      │

│ format.                                                 │

│                                                         │

│ If W(k) is null, W(k+1) to W(16) must be null also.     │

│                                                         │

│ Each non null W(k) allows to compute, together with     │

│ some previous W(i) the ARFCN F(k) of a frequency in     │

│ the set. The computation formulas are given in section  │

│ 10.5.2.9a.3.                                            │

└─────────────────────────────────────────────────────────┘

TABLE 10.14b/GSM 04.08

Cell channel description information element, range 1024 format

GSM 04.08-DCS - 3.1.0 - page 307d

10.5.2.1.4 Range 512 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │ EXT-│  0  │  1     0     0  │ORIG-│

│ FORMAT-ID │ IND │spare│    FORMAT-ID    │ARFCN│ octet 2

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN (middle part)              │ octet 3

├─────┬─────────────────────────────────────────┤

│ORIG-│               W(1)                      │ octet 4

│ARFCN│            (high part)                  │

│ low │                                         │

├─────┴─────┬───────────────────────────────────┤

│   W(1)    │         W(2)                      │ octet 5

│ (low part)│      (high part)                  │

├───────────┼───────────────────────────────────┤

│   W(2)    │         W(3)                      │ octet 6

│ (low part)│      (high part)                  │

├───────────┼───────────────────────────────────┤

│   W(3)    │         W(4)                      │ octet 7

│ (low part)│      (high part)                  │

├─────┬─────┴───────────────────────────────────┤

│W(4) │                W(5)                     │ octet 8

│ low │                                         │

├─────┴───────────────────────────────────┬─────┤

│          W(6)                           │ W(7)│ octet 9

│                                         │ high│

├───────────────────────────────────┬─────┴─────┤

│          W(7)                     │  W(8)     │ octet 10

│       (low part)                  │(high part)│

├───────────────────────┬───────────┴───────────┤

│   W(8) (low part)     │  W(9) (high part)     │ octet 11

├───────────┬───────────┴───────────────────────┤

│    W(9)   │            W(10)                  │ octet 12

│(low part) │                                   │

├───────────┴───────────────────────┬───────────┤

│               W(11)               │  W(12)    │ octet 13

│                                   │(high part)│

├───────────────────────┬───────────┴───────────┤

│  W(12) (low part)     │  W(13)  (high part)   │ octet 14

├───────────┬───────────┴───────────────────────┤

│    W(13)  │            W(14)                  │ octet 15

│(low part) │                                   │

├───────────┴───────────────────────┬───────────┤

│               W(15)               │  W(16)    │ octet 16

│                                   │(high part)│

├─────────────────┬─────────────────┴───────────┤

│ W(16) (low part)│           W(17)             │ octet 17

└─────────────────┴─────────────────────────────┘

FIGURE 10.20c/GSM 04.08

Cell Channel description information element (512 range format)

GSM 04.08-DCS - 3.1.0 - page 307e

┌─────────────────────────────────────────────────────────┐

│  ORIG-ARFCN, origin ARFCN (octet 2, 3 and 4)            │

│                                                         │

│  This field encodes the ARFCN of a frequency in the     │

│  set. This value is also used to decode the rest of the │

│  element.                                               │

│                                                         │

│                                                         │

│                                                         │

│  W(i), i from 1 to 17 (octet 4 to 17) :                 │

│                                                         │

│  Each W(i) encodes a non negative integer in binary     │

│  format.                                                │

│                                                         │

│  If W(k) is null, W(k+1) to W(17) must be null also.    │

│                                                         │

│  Each non null W(k) allows to compute, together with    │

│  some previous W(i) the ARFCN F(k) of a frequency in    │

│  the set. The computation formulas are given in section │

│   10.5.2.9a.4.                                          │

│                                                         │

└─────────────────────────────────────────────────────────┘

TABLE 10.14c/GSM 04.08

Cell channel description information element, range 512 format

GSM 04.08-DCS - 3.1.0 - page 307f

10.5.2.1.5 Range 256 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │EXT- │  0  │  1     0     1  │ORIG-│

│ FORMAT-ID │  IND│spare│    FORMAT-ID    │ARFCN│ octet 2

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN  (middle part)             │ octet 3

├─────┬─────────────────────────────────────────┤

│ORIG-│                                         │ octet 4

│ARFCN│              W(1)                       │

│ low │          (high part)                    │

├─────┼─────────────────────────────────────────┤

│W(1) │              W(2)                       │ octet 5

│low  │                                         │

├─────┴───────────────────────────────────┬─────┤

│                    W(3)                 │W(4) │ octet 6

│                                         │high │

├─────────────────────────────┬───────────┴─────┤

│      W(4) (low part)        │ W(5) (high part)│ octet 7

├─────────────────┬───────────┴─────────────────┤

│ W(5) (low part) │    W(6)  (high part)        │ octet 8

├─────┬───────────┴───────────────────────┬─────┤

│W(6) │          W(7)                     │W(8) │ octet 9

│low  │                                   │high │

├─────┴─────────────────┬─────────────────┴─────┤

│   W(8)  (low part)    │   W(9) (high part)    │ octet10

├─────┬─────────────────┴───────────┬───────────┤

│W(9) │      W(10)                  │ W(11)     │ octet 11

│low  │                             │(high part)│

├─────┴───────────┬─────────────────┴───────────┤

│ W(11) (low part)│            W(12)            │ octet 12

├─────────────────┴────────────┬────────────────┤

│          W(13)               │W(14)(high part)│ octet 13

├───────────┬──────────────────┴─────────┬──────┤

│W(14)      │          W(15)             │ W(16)│ octet 14

│ low       │                            │ high │

├───────────┴────┬───────────────────────┼──────┤

│    W(16)       │        W(17)          │ W(18)│ octet 15

│  (low part)    │                       │ high │

├────────────────┼───────────────────────┼──────┤

│    W(18)       │        W(19)          │ W(20)│ octet 16

│  (low part)    │                       │ high │

├────────────────┼───────────────────────┼──────┤

│    W(20)       │        W(21)          │  0   │ octet 17

│  (low part)    │                       │      │

└────────────────┴───────────────────────┴──────┘

FIGURE 10.20d/GSM 04.08

Cell channel description information element, range 256 format

GSM 04.08-DCS - 3.1.0 - page 307g

┌─────────────────────────────────────────────────────────┐

│   ORIG-ARFCN, origin ARFCN (octet 2, 3 and 4)           │

│                                                         │

│   This field encodes the ARFCN of a frequency in the    │

│   set. This value is also used to decode the rest of the│

│   element.                                              │

│                                                         │

│                                                         │

│   W(i), i from 1 to 21 (octet 4 to 17) :                │

│                                                         │

│   Each W(i) encodes a non negative integer in binary    │

│   format.                                               │

│                                                         │

│   If W(k) is null, W(k+1) to W(21) must be null also.   │

│                                                         │

│   Each non null W(k) allows to compute, together with   │

│   some previous W(i) the ARFCN F(k) of a frequency in   │

│   the set. The computation formulas are given in        │

│   section 10.5.2.9a.5.                                  │

│                                                         │

└─────────────────────────────────────────────────────────┘

TABLE 10.14d/GSM 04.08

Cell channel description information element, range 256 format

GSM 04.08-DCS - 3.1.0 - page 307h

10.5.2.1.6 Range 128 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │EXT- │  0  │  1     1     0  │ORIG-│

│ FORMAT-ID │  IND│spare│    FORMAT-ID    │ARFCN│ octet 2

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│                ORIG-ARFCN                     │ octet 3

│              (middle part)                    │

├─────┬─────────────────────────────────────────┤

│ORIG-│                                         │ octet 4

│ARFCN│              W(1)                       │

│ low │                                         │

├─────┴─────────────────────────────┬───────────┤

│                    W(2)           │   W(3)    │ octet 5

│                                   │(high part)│

├───────────────────────┬───────────┴───────────┤

│  W(3) (low part)      │  W(4) (high part)     │

├─────┬─────────────────┴───────────┬───────────┤

│W(4) │     W(5)                    │  W(6)     │ octet 7

│low  │                             │(high part)│

├─────┴───────────┬─────────────────┴───────────┤

│  W(6) (low part)│          W(7)               │ octet 8

├─────────────────┴─────┬───────────────────────┤

│        W(8)           │         W(9)          │ octet 9

├───────────────────────┼───────────────────────┤

│        W(10)          │         W(11)         │ octet 10

├───────────────────────┼───────────────────────┤

│        W(12)          │         W(13)         │ octet 11

├───────────────────────┼───────────────────────┤

│        W(14)          │         W(15)         │ octet 12

├─────────────────┬─────┴───────────┬───────────┤

│        W(16)    │      W(17)      │   W(18)   │ octet 13

│                 │                 │(high part)│

├─────┬───────────┴─────┬───────────┴─────┬─────┤

│W(18)│     W(19)       │     W(20)       │W(21)│ octet 14

│low  │                 │                 │high │

├─────┴─────┬───────────┴─────┬───────────┴─────┤

│   W(21)   │        W(22)    │       W(23)     │ octet 15

│(low part) │                 │                 │

├───────────┴─────┬───────────┴─────┬───────────┤

│     W(24)       │  W(25)          │   W(26)   │ octet 16

│                 │                 │(high part)│

├─────┬───────────┴─────┬───────────┴─────┬─────┤

│W(26)│     W(27)       │     W(28)       │  0  │ octet 17

│low  │                 │                 │spare│

└─────┴─────────────────┴─────────────────┴─────┘

FIGURE 10.20e/GSM 04.08

Cell channel description information element, range 128 format

GSM 04.08-DCS - 3.1.0 - page 307i

┌─────────────────────────────────────────────────────────┐

│   ORIG-ARFCN, origin ARFCN (octet 2, 3 and 4)           │

│                                                         │

│   This field encodes the ARFCN of a frequency in the    │

│   set. This value is also used to decode the rest of the│

│   element.                                              │

│                                                         │

│                                                         │

│   W(i), i from 1 to 28 (octet 4 to 17) :                │

│                                                         │

│   Each W(i) encodes a non negative integer in binary    │

│   format.                                               │

│                                                         │

│   If W(k) is null, W(k+1) to W(28) must be null also.   │

│                                                         │

│   Each non null W(k) allows to compute, together with   │

│   some previous W(i) the ARFCN F(k) of a frequency in   │

│   the set. The computation formulas are given in        │

│   section 10.5.2.9a.6.                                  │

└─────────────────────────────────────────────────────────┘

TABLE 10.14e/GSM 04.08

Cell channel description information element, range 128 format

GSM 04.08-DCS - 3.1.0 - page 307j

10.5.2.1.7 Variable bit map format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     1     0  │ octet 1

│     │       Cell channel description IEI      │

├─────┴─────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │EXT- │  0  │  1     1     1  │ORIG-│

│ FORMAT-ID │ IND │spare│    FORMAT-ID    │ARFCN│ octet 2

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│                ORIG-ARFCN                     │ octet 3

│              (middle part)                    │

├─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│ORIG-│     │     │     │     │     │     │     │

│ARFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│ octet 4

│ low │  1  │  2  │  3  │  4  │  5  │  6  │  7  │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│octet 17

│ 104 │ 105 │ 106 │ 107 │ 108 │ 109 │ 110 │ 111 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.20f/GSM 04.08

Cell channel description information element, variable bit map format

GSM 04.08-DCS - 3.1.0 - page 307k

┌─────────────────────────────────────────────────────────┐

│ ORIG-ARFCN, origin ARFCN (octet 2, 3 and 4)             │

│                                                         │

│ This field encodes the ARFCN of a frequency in the      │

│ set. This value is also used as origin of the           │

│ bit map.                                                │

│                                                         │

│ RRFCN N, relative radio frequency channel number N      │

│ (octet 4 etc.)                                          │

│                                                         │

│ For a RF channel with ARFCN = (ORIG-ARFCN + N) mod 1024 │

│ belonging to the set, RRFCN N bit is coded with a       │

│ "1"; N = 1, 2, .. , 111                                 │

│                                                         │

│ For a RF channel with ARFCN = (ORIG-ARFCN + N) mod 1024 │

│ not belonging to the set, RRFCN N bit is coded with a   │

│ "0"; N = 1, 2, .. , 111                                 │

└─────────────────────────────────────────────────────────┘

TABLE 10.14f/GSM 04.08

Cell channel description information element, variable bit map format

GSM 04.08-DCS - 3.1.0 - page 308

10.5.2.2  Cell description

The purpose of the cell description information element  is to provide a minimum description of a cell, e.g. to  allow the mobile station to use its preknowledge about synchronization.

The  cell description information element is coded as  shown in Figure 10.21/GSM 04.08 and Table 10.15/GSM 04.08.

The  cell  description is a type 3 information element  with 3 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     0     0     1  │ octet 1

│     │         cell description IEI            │

├─────┴─────┬─────────────────┬─────────────────┤

│BCCH ARFCN │                 │                 │

│(high part)│                 │                 │ octet 2

│  BA-NO    │       NCC       │       BCC       │

├───────────┴─────────────────┴─────────────────┤

│          BCCH carrier number                  │ octet 3

│          BCCH ARFCN (low part)                │

└───────────────────────────────────────────────┘

FIGURE 10.21/GSM 04.08

Cell description information element

GSM 04.08-DCS - 3.1.0 - page 309

┌─────────────────────────────────────────────────────┐

│ BA-NO, BCCH allocation number (octet 2)             │

│ Bits                                                │

│ 8 7                                                 │

│ 0 0        Band number 0                            │

│                                                     │

│ All other values are reserved                       │

│                                                     │

│ NCC, PLMN colour code (octet 2)                     │

│ The   NCC   field   is   coded   as   the   binary  │

│ representation  of  the PLMN colour code (see Rec.  │

│ GSM 03.03)                                          │

│                                                     │

│ BCC, BS colour code (octet 2)                       │

│ The   BCC   field   is   coded   as   the   binary  │

│ representation  of  the  BS colour code  (see Rec.  │

│ GSM 03.03).                                         │

│                                                     │

│ BCCH carrier number (octet 3)                       │

│ BCCH ARFCN (octet 2, bits 7 and 8, and octet 3)     │

│                                                     │

│ The BCCH carrier ARFCN field is coded as the        │

│ binary representation of the BCCH carrier           │

│ absolute RF channel number (see rec 05.05).         │

│                                                     │

│ Range : 1 to 124 0 to 1023                          │

│                                                     │

│ All other values are reserved.                      │

│                                                     │

└─────────────────────────────────────────────────────┘

TABLE 10.15/GSM 04.08

Cell description information element

GSM 04.08-DCS - 3.1.0 - page 314

10.5.2.5  Channel description

The  purpose of the channel description information  element is  to  provide  a description  of  an  allocatable  channel together with its SACCH.

The  channel  description information element  is  coded  as shown in Figure 10.24/GSM 04.08 and Table 10.18/GSM 04.08.

The channel description is a type 3 information element with 4 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     1     0     0  │ octet 1

│     │         Channel description IEI         │

├─────┴───────────────────────┬─────────────────┤

│        Channel type         │                 │ octet 2

│      and TDMA offset        │       TN        │

├─────────────────┬─────┬─────┴─────────────────┤

│                 │H=1->│  MAIO (high part)     │

│      TSC        ├─ H ─┼ ─ ─ ─ ─ ─ ┬ ─ ─ ─ ─ ─ ┤ octet 3

│                 │     │           │  FB-NO    │

│                 │     │           │  ARFCN    │

│                 │H=0->│  spare    │(high part)│

├───────────┬─────┴─────┴───────────┴───────────┤

│  MAIO     │                                   │

│(low part) │              HSN                  │

├ ─ ─ ┬ ─ ─ ┴ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤ octet 4

│spare│          ARFCN                          │

│             ARFCN (low part)                  │

└───────────────────────────────────────────────┘

FIGURE 10.24/GSM 04.08

Channel description information element

GSM 04.08-DCS - 3.1.0 - page 315

┌──────────────────────────────────────────────────────┐

│ Channel type and TDMA offset (octet 2)               │

│   Bits                                               │

│ 8 7 6 5 4                                            │

│ 0 0 0 0 1  Bm + ACCHs                                │

│ 0 0 0 1 T  Lm + ACCHs                                │

│ 0 0 1 T T  SDCCH/4 + SACCH/C4                        │

│ 0 1 T T T  SDCCH/8 + SACCH/C8                        │

│                                                      │

│ The  T bits indicate the subchannel number coded in  │

│ binary.                                              │

│                                                      │

│ All other values are reserved.                       │

│                                                      │

│ TN, Timeslot number (octet 2)                        │

│ The  TN field is coded as the binary representation  │

│ of the timeslot number.                              │

│                                                      │

│ Range: 0 to 7.                                       │

│                                                      │

│                                                      │

│ TSC, Training Sequence Code (octet 3)                │

│ The TSC field is coded as the binary representation  │

│ of the Training Sequence Code                        │

│ H, Hopping channel (octet 3)                         │

│ Bit                                                  │

│ 5                                                    │

│ 0        Single RF channel                           │

│ 1        RF hopping channel                          │

│                                                      │

│ Note: The  value  of H affects the semantics of the  │

│       channel  selector  field.                      │

│                                                      │

│ Channel selector (octet 3 and 4)                     │

│ H = "0": The channel selector field consists of     │

│          the cell allocation number, FB-NO, and the  │

│          absolute RF channel number                  │

│                                                      │

│       Octet 3                                        │

│        Bits                                          │

│        4 3                                           │

│        0 0    Spare                                  │

│                                                      │

│        FB-NO, (octet 3)                              │

│        Bits                                          │

│        2 1                                           │

│        0 0    Band number 0                          │

│        All other values are reserved.                │

│                                                      │

│        Octet 4                                       │

│        bit                                           │

│         8                                            │

│         0   Spare                                    │

│                                                      │

└──────────────────────────────────────────────────────┘

TABLE 10.18/GSM 04.08

Channel description information element

GSM 04.08-DCS - 3.1.0 - page 316

┌──────────────────────────────────────────────────────┐

│       ARFCN, (octet 3, bits 2 and 1, and             │

│               octet 4, bits 7 8 to 1)                 │

│       The ARFCN is coded as the binary representation│

│       of the absolute RF channel number (see Rec     │

│       GSM 05.05)                                     │

│                                                      │

│        Range 1 to 124 0 to 1023.                    │

│                                                      │

│        All other values are reserved.                │

│                                                      │

│ H = "1": The channel selector field                  │

│          consists  of the mobile                     │

│          allocation index offset, MAIO, and the      │

│          hopping sequence number, HSN.               │

│                                                      │

│           MAIO, (octet 3 bit 4 to 1 high part and    │

│                  octet 4 bit 8 to 7 low part)        │

│           The MAIO field is coded as the binary rep- │

│           resentation of the mobile allocation index │

│           offset.                                    │

│                                                      │

│          Range: 0 to 63.                             │

│                                                      │

│          All other values are reserved.              │

│                                                      │

│          HSN, (octet 4 bit 6 to 1)                   │

│          The HSN field is coded as the binary repres-│

│          entation of the hopping sequence number.    │

│                                                      │

│          Range 0 to 63.                              │

└──────────────────────────────────────────────────────┘

TABLE 10.18/GSM 04.08

Channel description information element

GSM 04.08-DCS - 3.1.0 - page 323

10.5.2.9  Frequency channel sequence

The purpose of the frequency channel sequence is to  provide the  absolute  radio frequency channel numbers used  in  the mobile  hopping sequence. This information element can only be used for radio frequency channels in GSM band 0 the primary GSM band, ie for channels of ARFCN between 1 and 124.

The frequency channel sequence information element is  coded as shown in Figure 10.28/GSM 04.08 and Table  10.22/GSM 04.08.

The frequency channel sequence is a type 3 information element with 10 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     1     0     0     1  │ octet 1

│     │      Frequency channel sequence IEI     │

├─────┼─────────────────────────────────────────┤

│  0  │             Lowest ARFCN                │ octet 2

│spare│                                         │

├─────┴─────────────────┬───────────────────────┤

│  inc skip of ARFCN 01 │  inc skip of ARFCN 02 │ octet 3

│                       │                       │

├───────────────────────┼───────────────────────┤

:

:

:

:

├───────────────────────┼───────────────────────┤

│  inc skip of ARFCN 15 │  inc skip of ARFCN 16 │octet 10

│                       │                       │

└───────────────────────┴───────────────────────┘

FIGURE 10.28/GSM 04.08

Frequency channel sequence information element

GSM 04.08-DCS - 3.1.0 - page 324

┌──────────────────────────────────────────────────────┐

│ RXLEV-NCELL  i, Received signal strength on the i'th │

│ neighbouring  cell (octet 5, 7, 8, 9, 10, 11, 12, 13,│

│ 14, 15 and 16)                                       │

│ The   RXLEV-NCELL  field  is  coded  as  the  binary │

│ representation of a value N. N corresponds according │

│ to  the  mapping  defined  in  Rec. GSM 05.08 to the │

│ received  signal  strength  on the i'th neighbouring │

│ cell. See note 1 & 2.                                │

│                                                      │

│ Range: 0 to 63.                                      │

│                                                      │

│ BCCH-FREQ-NCELL   i,   BCCH   carrier  of  the  i'th │

│ neighbouring  cell (octet 6, 8, 10, 12, 14, 15, 16   │

│ and 17)                                              │

│ The  BCCH-FREQ-NCELL  i field is coded as the binary │

│ representation  of  of  the  position  of  the  i'th │

│ neighbouring  cells BCCH carrier in the BCCH channel │

│ list provided by the reference neigbour cells        │

│ description information element or elements.         │

│  The BCCH channel list is the list of absolute       │

│ RF channel numbers for which the BA ARFCN bit in the │

│ neighbour  cells  description information element is │

│coded with a "1". The absolute RF channel numbers is are │

│ placed  in  increasing  order  in  the list with the │

│ lowest frequency in position 0. See note 1 & 2.      │

│                                                      │

│  Range: 0 to 31.                                     │

│                                                      │

│ BSIC-NCELL i, Base station identity code of the i'th │

│ neighbouring cell (octet 6, 7, 8, 9, 10, 11, 13, 15  │

│ and 17)                                              │

│ The  BSIC-NCELL  i  field  is  coded  as  the binary │

│ representation  of the base station identity code of │

│ the i'th neighbouring cell. See note 1 & 2.          │

│                                                      │

│  Range: 0 to 63.                                     │

│                                                      │

│ Note 1: If  the  field  extends  over two octets the │

│         highest  numbered bit of the lowest numbered │

│         octet is the most significant and the lowest │

│         numbered  bit  of the highest numbered octet │

│         is the least significant.                    │

│                                                      │

│ Note 2: If  NO-NCELL-M < 6 the remaining RXLEV-NCELL │

│         i,  BS- FREQ-NCELL i and BSIC-NCELL i fields │

│         (NO-NCELL-M < i <= 6) should be coded with a │

│         "0" in each bit.                             │

└──────────────────────────────────────────────────────┘

TABLE 10.24/GSM 04.08

Measurement results information element (continued)

GSM 04.08-DCS - 3.1.0 - page 325

┌──────────────────────────────────────────────────────┐

│ MA  C  i,  Mobile  allocation RF channel i (octet 3  │

│ etc.), i = 1, 2,..., NF                              │

│ The MA C i bit indicates whether or not the Mobile   │

│ allocation frequency list includes the  i'th         │

│ frequency in the cell allocation frequency list      │

│ provided by the reference cell channel description   │

│ information element or elements. NF denotes the      │

│ number  of  frequencies  in  the cell allocation     │

│ frequency list.  The absolute RF channel numbers are │

│ placed  in  increasing order in the list with the    │

│ lowest frequency in position 0.                      │

│ The  RF  channels  represented  in  the  MA C i bit  │

│ fields   are   those  which  in  the  cell  channel  │

│ description information element are coded with "1"s  │

│ in  the  CA  ARFCN  N bit fields. If NF denotes the  │

│ number  of  frequencies  in  the  mobile allocation  │

│ field then :                                         │

│                                                      │

│          MA C i = CA ARFCN N(i);  i = 1, 2,..., NF.  │

│                                                      │

│ N(i)  is an increasing function of i, i.e. the order │

│ of  appearance  of  the  RF  channels  in the mobile │

│ allocation   field  is  the  same  as  in  the  cell │

│ allocation  field  in  the  cell channel description │

│ information element.                                 │

│                                                      │

│ For  a RF channel belonging to the mobile allocation │

│ the  MA  C  i bit is coded with a "1"; i = 1, 2,..., │

│ NF.                                                  │

│                                                      │

│ For  a  RF  channel  not  belonging  to  the  mobile │

│ allocation  the  MA C i bit is coded with a "0"; i = │

│ 1, 2,..., NF.                                        │

│                                                      │

│ If  NF mod 8 <> 0 then bits NF to 8n in octet 3 must │

│ be coded with a "0" in each.                         │

└──────────────────────────────────────────────────────┘

TABLE 10.25/GSM 04.08

Mobile allocation information element

GSM 04.08-DCS - 3.1.0 - page 325a

10.5.2.9a	Frequency list

The purpose of the frequency list information element is to provide the list of the absolute radio frequency channel numbers used in a frequency hopping sequence.

The frequency list information element a type 4 information element.

There are several formats for the frequency list information element, distinguished by the "format indicator" subfield. Some formats are frequency bit maps, the others use a special encoding scheme.

10.5.2.9a.1	General description

┌─────────────────────────────────────────────────────┐

│ FORMAT-ID, Format Identifier (part of octet 3)      │

│                                                     │

│ The different formats are distinguished by the      │

│ FORAMT-ID field.  The possible values are the       │

│ following :                                         │

│                                                     │

│  Bit Bit Bit Bit Bit          format notation       │

│   8   7   4   3   2                                 │

│                                                     │

│   0   0   X   X   X               bit map 0         │

│   1   0   0   X   X               1024 range        │

│   1   0   1   0   0               512 range         │

│   1   0   1   0   1               256 range         │

│   1   0   1   1   0               128 range         │

│   1   0   1   1   1               variable bit map  │

│                                                     │

│                                                     │

│ All other combinations are reserved for future use. │

│                                                     │

│ The signification of the remaining bits depends on  │

│ FORMAT-ID. The different cases are specified in     │

│ the next sections.                                  │

│                                                     │

└─────────────────────────────────────────────────────┘

TABLE 10.22a/GSM 04.08

Cell channel description information element, general format

GSM 04.08-DCS - 3.1.0 - page 325b

10.5.2.9a.2 Bit map 0 format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │            Frequency list IEI           │

├─────┴─────────────────────────────────────────┤

│  0     0     0     1     0     0     0     0  │ octet 2

│    Length of frequency list contents          │

├───────────┬───────────┬─────┬─────┬─────┬─────┤

│  0     0  │  0     0  │     │     │     │     │

│           │   spare   │ARFCN│ARFCN│ARFCN│ARFCN│ octet 3

│FORMAT-ID  │           │ 124 │ 123 │ 122 │ 121 │

├─────┬─────┼─────┬─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ octet 4

│ 120 │ 119 │ 118 │ 117 │ 116 │ 115 │ 114 │ 113 │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│ARFCN│octet 18

│ 008 │ 007 │ 006 │ 005 │ 004 │ 003 │ 002 │ 001 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.28a/GSM 04.08

Frequency list information element, bit map 0 format

GSM 04.08-DCS - 3.1.0 - page 325c

┌─────────────────────────────────────────────────────┐

│ ARFCN  N, Absolute RF Channel                       │

│ Number N (octet 3 etc.)                             │

│                                                     │

│ For  a  RF channel with ARFCN = N belonging to the  │

│ frequency list the ARFCN N bit is coded with a      │

│ "1"; N = 1, 2, .. , 124.                            │

│                                                     │

│ For  a  RF channel with ARFCN = N not belonging to  │

│ the frequency list the ARFCN N bit is coded         │

│ with a "0"; N = 1, 2 .. , 124.                      │

│                                                     │

└─────────────────────────────────────────────────────┘

TABLE 10.22b/GSM 04.08

Frequency list information element, bit map 0 format

GSM 04.08-DCS - 3.1.0 - page 325d

10.5.2.9a.3 Range 1024 format

The information element contains a header, and W(1) to W(M) for some M. If, due to octet boundaries, some bits are not used at the end of the last octet, these bits must be set to 0.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │       Frequency list IEI                │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│    Length of frequency list contents          │

├───────────┬─────┬─────┬─────┬─────┬───────────┤

│  1     0  │  0  │  0  │  0  │     │           │

│ FORMAT-ID │spare│spare│FORMA│ F0  │   W(1)    │ octet 3

│           │     │     │T-ID │     │(high part)│

├───────────┴─────┴─────┴─────┴─────┴───────────┤

│                W(1)  (low part)               │ octet 4

├───────────────────────────────────────────────┤

W(2)  to W(3)  are on 9 bits, when present

W(4)  to W(7)  are on 8 bits, when present

W(8)  to W(15) are on 7 bits, when present

W(16) to W(31) are on 6 bits, when present

W(2k) to W(2k+1-1) are on 10-k bits when present

and so on

.                                               .

.                                               .

└───────────┴───────────────────────────────────┘

FIGURE 10.28b/GSM 04.08

Frequency list information element (Range 1024 format)

GSM 04.08-DCS - 3.1.0 - page 325e

┌─────────────────────────────────────────────────────────┐

│                                                         │

│ F0, frequency 0 indicator (octet 3, bit 4) :            │

│                                                         │

│ 0     frequency of ARFCN 0 is not in the list           │

│ 1     freauency of ARFCN 0 is in the list               │

│                                                         │

│ W(i), i from 1 to M (octet 3 and next) :                │

│                                                         │

│ Each W(i) encodes a non negative integer in binary      │

│ format.                                                 │

│                                                         │

│ If W(k) is null, W(i) for i>k must be null also.        │

│                                                         │

│ Each non null W(k) allows to compute, together with     │

│ some previous W(i) the ARFCN F(k) of a frequency in     │

│ the set. The first computation formulas are given       │

│ hereafter, with the following conventions :             │

│                                                         │

│   Wi denotes W(i);                                      │

│   Fi denotes F(i);                                      │

│   +  indicates the natural integer addition;            │

│   *  indicates the natural integer multiplication;      │

│   n mod m  indicates the remainder of the euclidian     │

│     division of n by m, ie 0 ≤ (n mod m) ≤ m-1 and      │

│     there exists k such that  n = (k*m) + (n mod m);    │

│                                                         │

│   n smod m  indicates the offset remainder of the       │

│     euclidian division of n by m, ie                    │

│     1 ≤ (n smod m) ≤ m and there exists k such that     │

│     n = (k*m) + (n smod m);                             │

│                                                         │

│    F1 =  W1 smod 1023                                   │

│    F2 = (W1 - 512 + W2) smod 1023                       │

│    F3 = (W1       + W3) smod 1023                       │

│    F4 = (W1 - 512 + (W2 - 256 + W4) smod 511) smod 1023 │

│    F5 = (W1       + (W3 - 256 + W5) smod 511) smod 1023 │

│    F6 = (W1 - 512 + (W2       + W6) smod 511) smod 1023 │

│    F7 = (W1       + (W3       + W7) smod 511) smod 1023 │

│    F8  = (W1 - 512 + (W2 - 256 + (W4 - 128 + W8 )       │

│          smod 255) smod 511) smod 1023                  │

└─────────────────────────────────────────────────────────┘

TABLE 10.22c/GSM 04.08

Frequency list information element, range 1024 format

GSM 04.08-DCS - 3.1.0 - page 325f

┌─────────────────────────────────────────────────────────┐

│   F9  = (W1       + (W3 - 256 + (W5 - 128 + W9 )        │

│          smod 255) smod 511) smod 1023                  │

│   F10 = (W1 - 512 + (W2       + (W6 - 128 + W10)        │

│          smod 255) smod 511) smod 1023                  │

│   F11 = (W1       + (W3       + (W7 - 128 + W11)        │

│          smod 255) smod 511) smod 1023                  │

│   F12 = (W1 - 512 + (W2 - 256 + (W4       + W12)        │

│          smod 255) smod 511) smod 1023                  │

│   F13 = (W1       + (W3 - 256 + (W5       + W13)        │

│          smod 255) smod 511) smod 1023                  │

│   F14 = (W1 - 512 + (W2       + (W6       + W14)        │

│          smod 255) smod 511) smod 1023                  │

│   F15 = (W1       + (W3       + (W7       + W15)        │

│          smod 255) smod 511) smod 1023                  │

│   F16  = (W1 - 512 + (W2 - 256 + (W4 - 128 +            │

│              (W8 - 64 + W16) smod 127)                  │

│          smod 255) smod 511) smod 1023                  │

└─────────────────────────────────────────────────────────┘

TABLE 10.22c/GSM 04.08

Frequency list information element, range 1024 format (continued)

┌─────────────────────────────────────────────────────────┐

│ More generally, the computation of F(K) can be done     │

│ with the following program, using ADA language          │

│ (declarative parts are skipped and should be obvious):  │

│                                                         │

│                                                         │

│ INDEX := K;                                             │

│ J := GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);     │

│ N := W(INDEX);                                          │

│ while INDEX>1 loop                                      │

│   if 2*INDEX < 3*J then                                 │

│     INDEX := INDEX - J/2;              -- left child    │

│     N := (N + W(PARENT) - 1024/J - 1) mod               │

│           (2048/J - 1) + 1;                             │

│   else                                -- right child    │

│     INDEX := INDEX - J;                                 │

│     N := (N + W(PARENT) - 1) mod (2048/J - 1) + 1;      │

│   end if;                                               │

│   J := J/2;                                             │

│ end loop;                                               │

│ F(K) := N;                                              │

└─────────────────────────────────────────────────────────┘

GSM 04.08-DCS - 3.1.0 - page 325g

10.5.2.9a.4 Range 512 format

The information element contains a header, and W(1) to W(M) for some M. If, due to octet boundaries, some bits are not used at the end of the last octet, these bits must be set to 0.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │       Frequency list IEI                │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│    Length of frequency list contents          │

├───────────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │  0  │  0  │  1     0     0  │ORIG-│

│ FORMAT-ID │spare│spare│    FORMAT-ID    │ARFCN│ octet 3

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN (middle part)              │ octet 4

├─────┬─────────────────────────────────────────┤

│ORIG-│               W(1)                      │ octet 5

│ARFCN│            (high part)                  │

│ low │                                         │

├─────┴─────┬───────────────────────────────────┤

│   W(1)    │         W(2)                      │ octet 6

│ (low part)│      (high part)                  │

├───────────┼───────────────────────────────────┤

W(2)  to W(3)  are on 8 bits, when present

W(4)  to W(7)  are on 7 bits, when present

W(8)  to W(15) are on 6 bits, when present

W(16) to W(31) are on 5 bits, when present

W(2k) to W(2k+1-1) are on 9-k bits when present

and so on

.                                               .

.                                               .

└───────────┴───────────────────────────────────┘

FIGURE 10.28c/GSM 04.08

Frequency list information element (Range 512 format)

GSM 04.08-DCS - 3.1.0 - page 325h

┌─────────────────────────────────────────────────────────┐

│   ORIG-ARFCN, origin ARFCN (octet 3, 4 and 5)           │

│                                                         │

│   This field encodes the ARFCN of a frequency in the    │

│   set. This value is also used to decode the rest of the│

│   element.                                              │

│                                                         │

│                                                         │

│ W(i), i from 1 to M (octet 5 and next) :                │

│                                                         │

│ Each W(i) encodes a non negative integer in binary      │

│ format.                                                 │

│                                                         │

│ If W(k) is null, W(i) for i>k must be null also.        │

│                                                         │

│ Each non null W(k) allows to compute, together with     │

│ some previous W(i) the ARFCN F(k) of a frequency in     │

│ the set. The first computation formulas are given       │

│ hereafter, with the following conventions :             │

│                                                         │

│   Wi denotes W(i); W0 denotes the value of ORIG-ARFCN   │

│   Fi denotes F(i);                                      │

│   +  indicates the natural integer addition;            │

│   *  indicates the natural integer multiplication;      │

│   n mod m  indicates the remainder of the euclidian     │

│     division of n by m, ie 0 ≤ (n mod m) ≤ m-1 and      │

│     there exists k such that  n = (k*m) + (n mod m);    │

│                                                         │

│   n smod m  indicates the offset remainder of the       │

│     euclidian division of n by m, ie                    │

│     1 ≤ (n smod m) ≤ m and there exists k such that     │

│     n = (k*m) + (n smod m);                             │

│                                                         │

│ F1  = (W0 + W1) mod 1024                                │

│ F2  = (W0 + (W1 - 256 + W2) smod 511) mod 1024          │

│ F3  = (W0 + (W1       + W3) smod 511) mod 1024          │

│ F4  = (W0 + (W1 - 256 + (W2 - 128 + W4) smod 255)       │

│        smod 511) mod 1024                               │

│ F5  = (W0 + (W1       + (W3 - 128 + W5) smod 255)       │

│        smod 511) mod 1024                               │

│ F6  = (W0 + (W1 - 256 + (W2       + W6) smod 255)       │

│        smod 511) mod 1024                               │

│ F7  = (W0 + (W1       + (W3       + W7) smod 255)       │

│        smod 511) mod 1024                               │

│ F8  = (W0 + (W1 - 256 + (W2 - 128 + (W4 - 64 + W8 )     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F9  = (W0 + (W1       + (W3 - 128 + (W5 - 64 + W9 )     │

│        smod 127) smod 255) smod 511) mod 1024           │

└─────────────────────────────────────────────────────────┘

TABLE 10.22d/GSM 04.08

Frequency list information element, range 512 format

GSM 04.08-DCS - 3.1.0 - page 325i

┌─────────────────────────────────────────────────────────┐

│ F10 = (W0 + (W1 - 256 + (W2       + (W6 - 64 + W10)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F11 = (W0 + (W1       + (W3       + (W7 - 64 + W11)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F12 = (W0 + (W1 - 256 + (W2 - 128 + (W4      + W12)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│                                                         │

│ F13 = (W0 + (W1       + (W3 - 128 + (W5      + W13)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F14 = (W0 + (W1 - 256 + (W2       + (W6      + W14)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F15 = (W0 + (W1       + (W3       + (W7      + W15)     │

│        smod 127) smod 255) smod 511) mod 1024           │

│ F16 = (W0 + (W1 - 256 + (W2 - 128 + (W4 - 64 +          │

│       (W8 - 32 + W16)                                   │

│      smod 63) smod 127) smod 255) smod 511) mod 1024    │

│ F17 = (W0 + (W1       + (W3 - 128 + (W5 - 64 +          │

│       (W9 - 32 + W17)                                   │

│      smod 63) smod 127) smod 255) smod 511) mod 1024    │

└─────────────────────────────────────────────────────────┘

TABLE 10.22d/GSM 04.08

Frequency list information element, range 512 format (continued)

┌─────────────────────────────────────────────────────────┐

│ More generally, the computation of F(K) can be done     │

│ with the following program, using ADA language          │

│ (declarative parts are skipped and should be obvious):  │

│                                                         │

│                                                         │

│ INDEX := K;                                             │

│ J := GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);     │

│ N := W(INDEX);                                          │

│ while INDEX>1 loop                                      │

│   if 2*INDEX < 3*J then               -- left child     │

│     INDEX := INDEX - J/2;                               │

│     N := (N + W(PARENT) - 512/J - 1) mod                │

│           (1024/J - 1) + 1;                             │

│   else                                -- right child    │

│     INDEX := INDEX - J;                                 │

│     N := (N + W(_INDEX) - 1) mod (1024/J - 1) + 1;      │

│   end if;                                               │

│   J := J/2;                                             │

│ end loop;                                               │

│ F(K) := (W(0) + N) mod 1024;                            │

└─────────────────────────────────────────────────────────┘

GSM 04.08-DCS - 3.1.0 - page 325j

10.5.2.9a.5	Range 256 format

The information element contains a header, and W(1) to W(M) for some M. If, due to octet boundaries, some bits are not used at the end of the last octet, these bits must be set to 0.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │       Frequency list IEI                │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│    Length of frequency list contents          │

├───────────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │  0  │  0  │  1     0     1  │ORIG-│

│ FORMAT-ID │spare│spare│    FORMAT-ID    │ARFCN│ octet 3

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN (middle part)              │ octet 4

├─────┬─────────────────────────────────────────┤

│ORIG-│               W(1)                      │ octet 5

│ARFCN│            (high part)                  │

│ low │                                         │

├─────┼─────────────────────────────────────────┤

│W(1) │               W(2)                      │ octet 6

│low  │                                         │

├─────┴─────────────────────────────────────────┤

W(2)  to W(3)  are on 7 bits, when present

W(4)  to W(7)  are on 6 bits, when present

W(8)  to W(15) are on 5 bits, when present

W(16) to W(31) are on 4 bits, when present

W(2k) to W(2k+1-1) are on 8-k bits when present

and so on

.                                               .

.                                               .

└───────────┴───────────────────────────────────┘

FIGURE 10.28d/GSM 04.08

Frequency list information element (Range 256 format)

GSM 04.08-DCS - 3.1.0 - page 325k

┌──────────────────────────────────────────────────────────────────┐

│                                                                  │

│   ORIG-ARFCN, origin ARFCN (octet 3, 4 and 5)                    │

│                                                                  │

│   This field encodes the ARFCN of a frequency in the             │

│   set. This value is also used to decode the rest of the element.│

│                                                                  │

│                                                                  │

│ W(i), i from 1 to M (octet 5 and next) :                         │

│                                                                  │

│ Each W(i) encodes a non negative integer in binary format.       │

│                                                                  │

│ If W(k) is null, W(i) for i>k must be null also.                 │

│                                                                  │

│ Each non null W(k) allows to compute, together with              │

│ some previous W(i) the ARFCN F(k) of a frequency in              │

│ the set. The first computation formulas are given                │

│ hereafter, with the following conventions :                      │

│                                                                  │

│   Wi denotes W(i);  W0 denotes the value of ORIG-ARFCN           │

│   Fi denotes F(i);                                               │

│   +  indicates the natural integer addition;                     │

│   *  indicates the natural integer multiplication;               │

│   n mod m  indicates the remainder of the euclidian              │

│     division of n by m, ie 0 ≤ (n mod m) ≤ m-1 and               │

│     there exists k such that  n = (k*m) + (n mod m);             │

│                                                                  │

│   n smod m  indicates the offset remainder of the                │

│     euclidian division of n by m, ie                             │

│     1 ≤ (n smod m) ≤ m and there exists k such that              │

│     n = (k*m) + (n smod m);                                      │

│                                                                  │

│                                                                  │

│ F1  = (W0 + W1) mod 1024                                         │

│ F2  = (W0 + (W1 - 128 + W2) smod 255) mod 1024                   │

│ F3  = (W0 + (W1       + W3) smod 255) mod 1024                   │

│ F4  = (W0 + (W1 - 128 + (W2 - 64 + W4) smod 127) smod 255)       │

│      mod 1024                                                    │

│ F5  = (W0 + (W1       + (W3 - 64 + W5) smod 127) smod 255)       │

│      mod 1024                                                    │

│ F6  = (W0 + (W1 - 128 + (W2      + W6) smod 127) smod 255)       │

│      mod 1024                                                    │

│ F7  = (W0 + (W1       + (W3      + W7) smod 127) smod 255)       │

│      mod 1024                                                    │

└──────────────────────────────────────────────────────────────────┘

TABLE 10.22e/GSM 04.08

Frequency list information element, range 256 format

GSM 04.08-DCS - 3.1.0 - page 325l

┌──────────────────────────────────────────────────────────────────┐

│                                                                  │

│ F8  = (W0 + (W1 - 128 + (W2 - 64 + (W4 - 32 + W8 ) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F9  = (W0 + (W1       + (W3 - 64 + (W5 - 32 + W9 ) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F10 = (W0 + (W1 - 128 + (W2      + (W6 - 32 + W10) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F11 = (W0 + (W1       + (W3      + (W7 - 32 + W11) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F12 = (W0 + (W1 - 128 + (W2 - 64 + (W4      + W12) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F13 = (W0 + (W1       + (W3 - 64 + (W5      + W13) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F14 = (W0 + (W1 - 128 + (W2      + (W6      + W14) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F15 = (W0 + (W1       + (W3      + (W7      + W15) smod 63)      │

│      smod 127) smod 255) mod 1024                                │

│ F16 = (W0 + (W1 - 128 + (W2 - 64 + (W4 - 32 + (W8  - 16 + W16)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

│ F17 = (W0 + (W1       + (W3 - 64 + (W5 - 32 + (W9  - 16 + W17)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

│ F18 = (W0 + (W1 - 128 + (W2      + (W6 - 32 + (W10 - 16 + W18)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

│ F19 = (W0 + (W1       + (W3      + (W7 - 32 + (W11 - 16 + W19)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

│ F20 = (W0 + (W1 - 128 + (W2 - 64 + (W4      + (W12 - 16 + W20)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

│ F21 = (W0 + (W1       + (W3 - 64 + (W5      + (W13 - 16 + W21)   │

│      smod 31) smod 63) smod 127) smod 255) mod 1024              │

└──────────────────────────────────────────────────────────────────┘

TABLE 10.22e/GSM 04.08

Frequency list information element, range 256 format (continued)

┌─────────────────────────────────────────────────────────┐

│ More generally, the computation of F(K) can be done     │

│ with the following program, using ADA language          │

│ (declarative parts are skipped and should be obvious):  │

│                                                         │

│ INDEX := K;                                             │

│ J := GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);     │

│ N := W(INDEX);                                          │

│ while INDEX>1 loop                                      │

│   if 2*INDEX < 3*J then                -- left child    │

│     INDEX := INDEX - J/2;                               │

│     N := (N + W(INDEX) - 256/J - 1) mod                 │

│           (512/J - 1) + 1;                              │

│   else                                -- right child    │

│     INDEX := INDEX - J;                                 │

│     N := (N + W(INDEX) - 1) mod (512/J - 1) + 1;        │

│   end if;                                               │

│   J := J/2;                                             │

│ end loop;                                               │

│ F(K) := (W(0) + N) mod 1024;                            │

└─────────────────────────────────────────────────────────┘

GSM 04.08-DCS - 3.1.0 - page 325m

10.5.2.9a.6	Range 128 format

The information element contains a header, and W(1) to W(M) for some M. If, due to octet boundaries, some bits are not used at the end of the last octet, these bits must be set to 0.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │       Frequency list IEI                │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│    Length of frequency list contents          │

├───────────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │  0  │  0  │  1     1     0  │ORIG-│

│ FORMAT-ID │spare│spare│    FORMAT-ID    │ARFCN│ octet 3

│           │     │     │                 │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN (middle part)              │ octet 4

├─────┬─────────────────────────────────────────┤

│ORIG-│               W(1)                      │ octet 5

│ARFCN│            (high part)                  │

│ low │                                         │

├─────┼─────────────────────────────────────────┤

W(2)  to W(3)  are on 6 bits, when present

W(4)  to W(7)  are on 5 bits, when present

W(8)  to W(15) are on 4 bits, when present

W(16) to W(31) are on 3 bits, when present

W(2k) to W(2k+1-1) are on 7-k bits when present

and so on

.                                               .

.                                               .

└───────────┴───────────────────────────────────┘

FIGURE 10.28e/GSM 04.08

Frequency list information element (Range 128 format)

GSM 04.08-DCS - 3.1.0 - page 325n

┌──────────────────────────────────────────────────────────────────┐

│                                                                  │

│   ORIG-ARFCN, origin ARFCN (octet 3, 4 and 5)                    │

│                                                                  │

│   This field encodes the ARFCN of a frequency in the             │

│   set. This value is also used to decode the rest of the element.│

│                                                                  │

│                                                                  │

│ W(i), i from 1 to M (octet 5 and next) :                         │

│                                                                  │

│ Each W(i) encodes a non negative integer in binary format.       │

│                                                                  │

│ If W(k) is null, W(i) for i>k must be null also.                 │

│                                                                  │

│ Each non null W(k) allows to compute, together with              │

│ some previous W(i) the ARFCN F(k) of a frequency in              │

│ the set. The first computation formulas are given                │

│ hereafter, with the following conventions :                      │

│                                                                  │

│   Wi denotes W(i); W0 denotes the value of ORIG-ARFCN            │

│   Fi denotes F(i);                                               │

│   +  indicates the natural integer addition;                     │

│   *  indicates the natural integer multiplication;               │

│   n mod m  indicates the remainder of the euclidian              │

│     division of n by m, ie 0 ≤ (n mod m) ≤ m-1 and               │

│     there exists k such that  n = (k*m) + (n mod m);             │

│                                                                  │

│   n smod m  indicates the offset remainder of the                │

│     euclidian division of n by m, ie                             │

│     1 ≤ (n smod m) ≤ m and there exists k such that              │

│     n = (k*m) + (n smod m);                                      │

│                                                                  │

│                                                                  │

│ F1 = (W0 + W1) mod 1024                                          │

│ F2 = (W0 + (W1 - 64 + W2) smod 127) mod 1024                     │

│ F3 = (W0 + (W1       + W3) smod 127) mod 1024                    │

│ F4 = (W0 + (W1 - 64 + (W2 - 32 + W4) smod 63) smod 127)          │

│      mod 1024                                                    │

│ F5 = (W0 + (W1       + (W3 - 32 + W5) smod 63) smod 127)         │

│      mod 1024                                                    │

│ F6 = (W0 + (W1 - 64 + (W2      + W6) smod 63) smod 127)          │

│      mod 1024                                                    │

│ F7 = (W0 + (W1       + (W3      + W7) smod 63) smod 127)         │

│      mod 1024                                                    │

└──────────────────────────────────────────────────────────────────┘

TABLE 10.22f/GSM 04.08

Frequency list information element, range 128 format

GSM 04.08-DCS - 3.1.0 - page 325o

┌──────────────────────────────────────────────────────────────────┐

│ F8  = (W0 + (W1 - 64 + (W2 - 32 + (W4 - 16 + W8 ) smod 31)       │

│      smod 63) smod 127) mod 1024                                 │

│ F9  = (W0 + (W1       + (W3 - 32 + (W5 - 16 + W9 ) smod 31)      │

│      smod 63) smod 127) mod 1024                                 │

│ F10 = (W0 + (W1 - 64 + (W2      + (W6 - 16 + W10) smod 31)       │

│      smod 63) smod 127) mod 1024                                 │

│ F11 = (W0 + (W1       + (W3      + (W7 - 16 + W11) smod 31)      │

│      smod 63) smod 127) mod 1024                                 │

│ F12 = (W0 + (W1 - 64 + (W2 - 32 + (W4      + W12) smod 31)       │

│      smod 63) smod 127) mod 1024                                 │

│ F13 = (W0 + (W1       + (W3 - 32 + (W5      + W13) smod 31)      │

│      smod 63) smod 127) mod 1024                                 │

│ F14 = (W0 + (W1 - 64 + (W2      + (W6      + W14) smod 31)       │

│      smod 63) smod 127) mod 1024                                 │

│ F15 = (W0 + (W1       + (W3      + (W7      + W15) smod 31)      │

│      smod 63) smod 127) mod 1024                                 │

│ F16 = (W0 + (W1 - 64 + (W2 - 32 + (W4 - 16 + (W8  - 8 + W16)     │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F17 = (W0 + (W1       + (W3 - 32 + (W5 - 16 + (W9  - 8 + W17)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F18 = (W0 + (W1 - 64 + (W2      + (W6 - 16 + (W10 - 8 + W18)     │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F19 = (W0 + (W1       + (W3      + (W7 - 16 + (W11 - 8 + W19)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F20 = (W0 + (W1 - 64 + (W2 - 32 + (W4      + (W12 - 8 + W20)     │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F21 = (W0 + (W1       + (W3 - 32 + (W5      + (W13 - 8 + W21)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F22 = (W0 + (W1 - 64 + (W2      + (W6      + W(14 - 8 + W22)     │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F23 = (W0 + (W1       + (W3      + (W7      + (W15 - 8 + W23)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F24 = (W0 + (W1 - 64 + (W2 - 32 + (W4 - 16 + (W8       + W24)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F25 = (W0 + (W1       + (W3 - 32 + (W5 - 16 + (W9       + W25)   │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F26 = (W0 + (W1 - 64 + (W2      + (W6 - 16 + (W10      + W26)    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F27 = (W0 + (W1       + (W3      + (W7 - 16 + (W11      + W27)   │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F28 = (W0 + (W1 - 64 + (W2 - 32 + (W4      + (W12      + W28     │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

│ F29 = (W0 + (W1       + (W3 - 32 + (W5      + (W13      + W29    │

│      smod 15) smod 31) smod 63) smod 127) mod 1024               │

└──────────────────────────────────────────────────────────────────┘

TABLE 10.22f/GSM 04.08

Frequency list information element, range 128 format

GSM 04.08-DCS - 3.1.0 - page 325p

┌─────────────────────────────────────────────────────────┐

│                                                         │

│ More generally, the computation of F(K) can be done     │

│ with the following program, using ADA language          │

│ (declarative parts are skipped and should be obvious):  │

│                                                         │

│                                                         │

│ INDEX := K;                                             │

│ J := GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);     │

│ N := W(INDEX);                                          │

│ while INDEX>1 loop                                      │

│   if 2*INDEX < 3*J then                -- left child    │

│     INDEX := INDEX - J/2;                               │

│     N := (N + W(INDEX) - 128/J - 1) mod                 │

│           (256/J - 1) + 1;                              │

│   else                                -- right child    │

│     INDEX := INDEX - J;                                 │

│     N := (N + W(INDEX) - 1) mod (256/J - 1) + 1;        │

│   end if;                                               │

│   J := J/2;                                             │

│ end loop;                                               │

│ F(K) := (W(0) + N) mod 1024;                            │

└─────────────────────────────────────────────────────────┘

GSM 04.08-DCS - 3.1.0 - page 325q

10.5.2.9a.7	Variable bit map format

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  0     0     0     0     1     0     1  │ octet 1

│     │       Frequency list IEI                │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│    Length of frequency list contents          │

├───────────┬─────┬─────┬─────────────────┬─────┤

│  1     0  │  0  │  0  │  1     1     1  │ORIG-│

│ FORMAT-ID │spare│spare│    FORMAT-ID    │ARFCN│ octet 3

│           │     │     │  (continued)    │ high│

├───────────┴─────┴─────┴─────────────────┴─────┤

│         ORIG-ARFCN (middle part)              │ octet 4

├─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│ORIG-│     │     │     │     │     │     │     │

│ARFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│ octet 5

│ low │  1  │  2  │  3  │  4  │  5  │  6  │  7  │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│RRFCN│ octet k

│8k-40│8k-39│8k-38│8k-37│8k-36│8k-35│8k-34│8k-33│

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.28f/GSM 04.08

Cell channel description information element, variable bit map format

┌─────────────────────────────────────────────────────────┐

│ ORIG-ARFCN, origin ARFCN (octet 3, 4 and 5)             │

│                                                         │

│ This field encodes the ARFCN of a frequency in the      │

│ set. This value is also used as origin of the           │

│ bit map.                                                │

│                                                         │

│ RRFCN N, relative radio frequency channel number N      │

│ (octet 5 etc.)                                          │

│                                                         │

│ For a RF channel with ARFCN = (ORIG-ARFCN + N) mod 1024 │

│ belonging to the set, RRFCN N bit is coded with a       │

│ "1"; N = 1, 2, .. ,  8M+7  with 1 ≤ M ≤ 127             │

│                                                         │

│ For a RF channel with ARFCN = (ORIG-ARFCN + N) mod 1024 │

│ not belonging to the set, RRFCN N bit is coded with a   │

│ "0"; N = 1, 2, .. ,  8M+7  with 1 ≤ M ≤ 127             │

└─────────────────────────────────────────────────────────┘

TABLE 10.22g/GSM 04.08

Cell channel description information element, variable bit map format

GSM 04.08-DCS - 3.1.0 - page 325r

10.5.2.9b	Frequency short list

The purpose of the frequency short list information element is to provide the list of the absolute radio frequency channel numbers used in a frequency hopping sequence, in a small fixed length information element to obtain when possible the HANDOVER COMMAND message in a single block.

The frequency short list information element a type 3 information element of 10 octet length.

This element is encoded exactly as the frequency list information element, except that it has a fixed length instead of a variable length.

GSM 04.08-DCS - 3.1.0 - page 326

10.5.2.11 Measurement results

The purpose of the measurement results information element is to provide the results of the measurements made by the mobile station on the serving cell and the neighbour cells.

The measurement results information element is coded as shown in Figure 10.30/GSM 04.08 and Table 10.24/GSM 04.08.

The measurement results is a type 3 information element with 17 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     1     0     0     0     1  │ octet 1

│     │        Measurement results IEI          │

├─────┼─────┬───────────────────────────────────┤

│ BA- │ DTX │     RXLEV-FULL-SERVING-CELL       │ octet 2

│ USED│USED │                                   │

├─────┼─────┼───────────────────────────────────┤

│  0  │MEAS-│     RXLEV-SUB-SERVING-CELL        │octet 3

│spare│VALID│                                   │

├─────┼─────┴───────────┬─────────────────┬─────┤

│  0  │                 │                 │ NO- │

│spare│                 │                 │NCELL│

│     │   RXQUAL-FULL   │    RXQUAL-SUB   │  M  │octet 4

│     │   SERVING-CELL  │   SERVING-CELL  │(high│

│     │                 │                 │part)│

├─────┴─────┬───────────┴─────────────────┴─────┤

│NO-NCELL-M │                                   │octet 5

│(low part) │        RXLEV-NCELL 1              │

├───────────┴─────────────────┬─────────────────┤

│      BCCH-FREQ-NCELL 1      │  BSIC-NCELL 1   │octet 6

│                             │  (high part)    │

├─────────────────┬───────────┴─────────────────┤

│  BSIC-NCELL 1   │       RXLEV-NCELL 2         │octet 7

│   (low part)    │        (high part)          │

├─────┬───────────┴─────────────────┬───────────┤

│RXLEV│                             │           │

│NCELL│                             │BSIC-NCELL │octet 8

│  2  │    BCCH-FREQ-NCELL 2        │    2      │

│(low │                             │(high part)│

│part)│                             │           │

├─────┴─────────────────────────────┴───────────┤

FIGURE 10.30/GSM 04.08

Measurement results information element

GSM 04.08-DCS - 3.1.0 - page 327

├───────────┴───────────┬───────────────────────┐

│     BSIC-NCELL 2      │    RXLEV-NCELL 3      │octet 9

│      (low part)       │     (high part)       │

├───────────┬───────────┴─────────────────┬─────┤

│  RXLEV-   │                             │BSIC-│

│  NCELL 3  │                             │NCELL│

│           │       BCCH-FREQ-NCELL 3     │  3  │octet 10

│ (low part)│                             │(high│

│           │                             │part)│

├───────────┴─────────────────┬───────────┴─────┤

│       BSIC-NCELL 3          │  RXLEV-NCELL 4  │octet 11

│       (low part)            │   (high part)   │

├─────────────────┬───────────┴─────────────────┤

│  RXLEV-NCELL 4  │       BCCH-FREQ-NCELL 4     │octet 12

│   (low part)    │                             │

├─────────────────┴─────────────────┬───────────┤

│                                   │RXLEV-NCELL│

│          BSIC-NCELL 4             │     5     │octet 13

│                                   │(high part)│

├───────────────────────┬───────────┴───────────┤

│     RXLEV-NCELL 5     │   BCCH-FREQ-NCELL 5   │octet 14

│       (low part)      │      (high part)      │

├─────┬─────────────────┴─────────────────┬─────┤

│BCCH-│                                   │RXLEV│

│FREQ-│                                   │NCELL│

│NCELL│           BSIC-NCELL 5            │  6  │octet 15

│5(low│                                   │(high│

│part)│                                   │part)│

├─────┴───────────────────────┬───────────┴─────┤

│     RXLEV-NCELL 6           │BCCH-FREQ-NCELL 6│octet 16

│      (low part)             │   (high part)   │

├───────────┬─────────────────┴─────────────────┤

│BCCH-FREQ- │                                   │octet 17

│  NCELL 6  │          BSIC-NCELL 6             │

│(low part) │                                   │

└───────────┴───────────────────────────────────┤

FIGURE 10.30/GSM 04.08

Measurement results information element (continued)

GSM 04.08-DCS - 3.1.0 - page 328

┌───────────────────────────────────────────────────────┐

│ BA-USED (octet 2), the value of the BA-IND field of   │

│ the neighbour cells description information element   │

│ defining the BCCH allocation used for the coding of   │

│ BCCH-FREQ-NCELL fields. Range 0 to 1                  │

│                                                       │

│ DTX-USED (octet 2)                                    │

│ This  bit  indicates  whether or not the MS used DTX  │

│ during the previous measurement period.               │

│ Bit                                                   │

│  7                                                    │

│  0     DTX was not used                               │

│  1     DTX was used                                   │

│                                                       │

│ RXLEV-FULL-SERVING-CELL and RXLEV-SUB-SERVING-CELL,   │

│ Received signal strength on serving cell, measured    │

│ respectively on all slots and on a subset of slots    │

│ (see Rec. GSM 05.08)                                  │

│ (octets 2 and 3)                                      │

│ The RXLEV-FULL-SERVING-CELL and RXLEV-SUB-SERVING-CELL│

│ fields are coded as the binary                        │

│ representation of a value N. N corresponds according  │

│ to the mapping defined in  Rec. GSM 05.08 to the      │

│ received signal strength on the serving cell.         │

│                                                       │

│ Range: 0 to 63                                        │

│                                                       │

│ MEAS-VALID  (octet 3)                                 │

│ This bit indicates if the measurement results for the │

│ dedicated channel are valid or not                    │

│ bit                                                   │

│  7                                                    │

│  0      The measurement results are valid             │

│  1      the measurement results are not valid         │

│                                                       │

│ RXQUAL-FULL-SERVING-CELL  and RXQUAL-SUB-SERVING-CELL │

│  received signal quality                              │

│ on  serving cell, measured respectively on all slots  │

│ and  on  a  subset of the slots (see Rec. GSM 05.08)  │

│ (octet 4)                                             │

└───────────────────────────────────────────────────────┘

TABLE 10.24/GSM 04.08

Measurement results information element

GSM 04.08 - v. 3.8.0 - page 329

┌──────────────────────────────────────────────────────┐

│ RXLEV-NCELL  i, Received signal strength on the i'th │

│ neighbouring  cell (octet 5, 7, 8, 9, 10, 11, 12, 13,│

│ 14, 15 and 16)                                       │

│ The   RXLEV-NCELL  field  is  coded  as  the  binary │

│ representation of a value N. N corresponds according │

│ to  the  mapping  defined  in  Rec. GSM 05.08 to the │

│ received  signal  strength  on the i'th neighbouring │

│ cell. See note 1 & 2.                                │

│                                                      │

│ Range: 0 to 63.                                      │

│                                                      │

│ BCCH-FREQ-NCELL   i,   BCCH   carrier  of  the  i'th │

│ neighbouring  cell (octet 6, 8, 10, 12, 14, 15, 16   │

│ and 17)                                              │

│ The  BCCH-FREQ-NCELL  i field is coded as the binary │

│ representation  of  of  the  position  of  the  i'th │

│ neighbouring  cells BCCH carrier in the BCCH channel │

│ list provided by the reference neigbour cells        │

│ description information element or elements.         │

│  The BCCH channel list is the list of absolute       │

│ RF channel numbers for which the BA ARFCN bit in the │

│ neighbour  cells  description information element is │

│coded with a "1". The absolute RF channel numbers is are │

│ placed  in  increasing  order  in  the list with the │

│ lowest frequency in position 0. See note 1 & 2.      │

│                                                      │

│  Range: 0 to 31.                                     │

│                                                      │

│ BSIC-NCELL i, Base station identity code of the i'th │

│ neighbouring cell (octet 6, 7, 8, 9, 10, 11, 13, 15  │

│ and 17)                                              │

│ The  BSIC-NCELL  i  field  is  coded  as  the binary │

│ representation  of the base station identity code of │

│ the i'th neighbouring cell. See note 1 & 2.          │

│                                                      │

│  Range: 0 to 63.                                     │

│                                                      │

│ Note 1: If  the  field  extends  over two octets the │

│         highest  numbered bit of the lowest numbered │

│         octet is the most significant and the lowest │

│         numbered  bit  of the highest numbered octet │

│         is the least significant.                    │

│                                                      │

│ Note 2: If  NO-NCELL-M < 6 the remaining RXLEV-NCELL │

│         i,  BS- FREQ-NCELL i and BSIC-NCELL i fields │

│         (NO-NCELL-M < i <= 6) should be coded with a │

│         "0" in each bit.                             │

└──────────────────────────────────────────────────────┘

TABLE 10.24/GSM 04.08

Measurement results information element (continued)

GSM 04.08-DCS - 3.1.0 - page 330

10.5.2.12  Mobile allocation

The purpose of the mobile allocation information element is to provide that part of the RF channels belonging to the cell allocation (coded with a "1" in the cell channel description information element) which is used in the mobile hopping sequence.

The mobile allocation information element is coded as shown in Figure 10.31/GSM 04.08 and Table 10.25/GSM 04.08.

The mobile allocation is a type 4 information element with 10 octets length maximal.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     1     0     0     1     0  │ octet 1

│     │     Mobile allocation IEI               │

├─────┴─────────────────────────────────────────┤

│                                               │ octet 2

│       Length of mobile allocation contents    │

├─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │

│  C  │  C  │  C  │  C  │  C  │  C  │  C  │  C  │ octet 3

│ 8n  │8n-1 │8n-2 │8n-3 │8n-4 │8n-5 │8n-6 │8n-7 │

├─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │ MA  │

│  C  │  C  │  C  │  C  │  C  │  C  │  C  │  C  │octet n+2

│ 008 │ 007 │ 006 │ 005 │ 004 │ 003 │ 002 │ 001 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.31/GSM 04.08

Mobile allocation information element.

GSM 04.08-DCS - 3.1.0 - page 331

┌──────────────────────────────────────────────────────┐

│ MA  C  i,  Mobile  allocation RF channel i (octet 3  │

│ etc.), i = 1, 2,..., NF                              │

│ The MA C i bit indicates whether or not the Mobile   │

│ allocation frequency list includes the  i'th         │

│ frequency in the cell allocation frequency list      │

│ provided by the reference cell channel description   │

│ information element or elements. NF denotes the      │

│ number  of  frequencies  in  the cell allocation     │

│ frequency list.  The absolute RF channel numbers are │

│ placed  in  increasing order in the list with the    │

│ lowest frequency in position 0.                      │

│ The  RF  channels  represented  in  the  MA C i bit  │

│ fields   are   those  which  in  the  cell  channel  │

│ description information element are coded with "1"s  │

│ in  the  CA  ARFCN  N bit fields. If NF denotes the  │

│ number  of  frequencies  in  the  mobile allocation  │

│ field then :                                         │

│                                                      │

│          MA C i = CA ARFCN N(i);  i = 1, 2,..., NF.  │

│                                                      │

│ N(i)  is an increasing function of i, i.e. the order │

│ of  appearance  of  the  RF  channels  in the mobile │

│ allocation   field  is  the  same  as  in  the  cell │

│ allocation  field  in  the  cell channel description │

│ information element.                                 │

│                                                      │

│ For  a RF channel belonging to the mobile allocation │

│ the  MA  C  i bit is coded with a "1"; i = 1, 2,..., │

│ NF.                                                  │

│                                                      │

│ For  a  RF  channel  not  belonging  to  the  mobile │

│ allocation  the  MA C i bit is coded with a "0"; i = │

│ 1, 2,..., NF.                                        │

│                                                      │

│ If  NF mod 8 <> 0 then bits NF to 8n in octet 3 must │

│ be coded with a "0" in each.                         │

└──────────────────────────────────────────────────────┘

TABLE 10.25/GSM 04.08

Mobile allocation information element

GSM 04.08-DCS - 3.1.0 - page 333

10.5.2.13  Neighbour cells description

The purpose of the neighbour cells description information element is to provide the absolute radio frequency channel numbers of the BCCH carriers to be monitored by the MSs in the cell.

The neighbour cells description information element is coded in Figure 10.32/GSM 04.08 and Table 10.26/GSM 04.08. as the cell channel description information element, as specified in section 10.5.2.1, with the exception of bit 5 of octet 2. Figure 10.32/GSM 04.08 and Table 10.26/GSM 04.08 contains the difference of specifications.

The neighbour cells description information element is a type 3 information element with 17 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     1     0     0     1     0     0  │ octet 1

│     │     Neighbour cells description IEI     │

├─────┼─────┬─────┬─────┬─────┬─────┬─────┬─────┤

│     │     │ EXT-│ BA- │     │     │     │     │

│ Bit │ Bit │ IND │ IND │ Bit │ Bit │ Bit │ Bit │ octet 2

│ 128 │ 127 │     │     │ 124 │ 123 │ 122 │ 121 │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ octet 3

│ 120 │ 119 │ 118 │ 117 │ 116 │ 115 │ 114 │ 113 │

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

├─────┼─────┼─────┼─────┼─────┼─────┼─────┼─────┤

│     │     │     │     │     │     │     │     │

│ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │ Bit │octet 17

│ 008 │ 007 │ 006 │ 005 │ 004 │ 003 │ 002 │ 001 │

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

FIGURE 10.32/GSM 04.08

Neighbour cells description information element

GSM 04.08-DCS - 3.1.0 - page 334

┌─────────────────────────────────────────────────────┐

│ EXT-IND, Extension indication (octet 2, bit 6)      │

│                                                     │

│ This bit indicates that the BA is incomplete, and   │

│ that a complementary BA is sent in another message. │

│                                                     │

│ Bit 6                                               │

│                                                     │

│   0       The BA is complete                        │

│   1       The BA is incomplete                      │

│                                                     │

│ BA-IND, BCCH allocation sequence number indication  │

│ (octet 2). Range 0 to 1.                            │

│                                                     │

│                                                     │

│ FORMAT-ID, Format Identifier (Bit 128 and next)     │

│                                                     │

│ The different formats are distinguished by the bits │

│ of higher number. The possible values are the       │

│ following :                                         │

│                                                     │

│  Bit Bit Bit Bit Bit          format notation       │

│  128 127 124 123 122                                │

│                                                     │

│   0   0   X   X   X               bit map 0         │

│   1   0   0   X   X               1024 range        │

│   1   0   1   0   0               512 range         │

│   1   0   1   0   1               256 range         │

│   1   0   1   1   0               128 range         │

│   1   0   1   1   1               variable bit map  │

│                                                     │

│                                                     │

│ All other combinations are reserved for future use. │

│                                                     │

│ The signification of the remaining bits depends on  │

│ FORMAT-ID. The different cases are specified as     │

│ in section 10.5.2.1.                                │

│                                                     │

└─────────────────────────────────────────────────────┘

TABLE 10.26/GSM 04.08

Neighbour cells description information element

GSM 04.08-DCS - 3.1.0 - page 357

10.5.3.6  Reject cause

The purpose of the reject cause information element is to indicate the reason why a request from the mobile station is rejected by the network.

The reject cause information element is coded as shown in Figure 10.49/GSM 04.08 and Table 10.44/GSM 04.08.

The reject cause is a type 3 information element with 2 octets length.

8     7     6     5     4     3     2     1

┌─────┬─────────────────────────────────────────┐

│  0  │  1     0     0     0     1     0     0  │ octet 1

│     │          reject cause IEI               │

├─────┴─────────────────────────────────────────┤

│                                               │

│                 reject cause value            │ octet 2

└───────────────────────────────────────────────┘

FIGURE 10.49/GSM 04.08

Reject cause information element

GSM 04.08-DCS - 3.1.0 - page 358

┌───────────────────────────────────────────────────────┐

│ Reject cause value (octet 2)                          │

│      Bits                                             │

│ 8 7 6 5 4 3 2 1                                       │

│ 0 0 0 0 0 0 0 1  Unallocated TMSI                     │

│ 0 0 0 0 0 0 1 0  IMSI unknown in HLR                  │

│ 0 0 0 0 0 0 1 1  Illegal MS                           │

│ 0 0 0 0 0 1 0 0  IMSI unknown in VLR                  │

│ 0 0 0 0 0 1 0 1  IMEI not accepted                    │

│ 0 0 0 0 1 0 1 1  PLMN not allowed                     │

│ 0 0 0 0 1 1 0 0  Location Area not allowed            │

│ 0 0 0 0 1 1 0 1  National roaming not allowed in this │

│                    location area                      │

│ 0 0 0 1 0 0 0 1  Network failure                      │

│ 0 0 0 1 0 1 1 0  Congestion                           │

│ 0 0 1 0 0 0 0 0  Service option not supported         │

│ 0 0 1 0 0 0 0 1  Requested service option             │

│                    not subscribed                     │

│ 0 0 1 0 0 0 1 0  Service option temporarily           │

│                    out of order                       │

│ 0 0 1 0 0 1 1 0  Call cannot be identified            │

│ 0 1 0 1 1 1 1 1  Invalid message, unspecified         │

│ 0 1 1 0 0 0 0 0  Mandatory information element        │

│                    error                              │

│ 0 1 1 0 0 0 0 1  Message type non-existent            │

│                    or not implemented                 │

│ 0 1 1 0 0 0 1 0  Message not compatible with          │

│                    the call state or not implemented  │

│ 0 1 1 0 0 0 1 1  Information element non-existent     │

│                    or not implemented                 │

│ 0 1 1 0 0 1 0 0  Invalid information element contents │

│ 0 1 1 0 0 1 0 1  Message not compatible with          │

│                    the call state                     │

│ 0 1 1 0 1 1 1 1  Protocol error, unspecified          │

│                                                       │

│ All other values are reserved.                        │

│                                                       │

│ Note: The  listed reject cause values are defined in  │

│       Annex G.                                        │

└───────────────────────────────────────────────────────┘

TABLE 10.44/GSM 04.08

Reject cause information element

GSM 04.08-DCS - 3.1.0 - page 435

ANNEX G

(to Recommendation GSM 04.08)

GSM specific cause values for mobility management

G.1  Causes related to MS identification

Cause value = 1  Unallocated TMSI.

This cause is sent to the MS if the MS identifies itself by a TMSI which is not allocated in the relevant location area and open identification is not requested.

Cause value = 2  IMSI unknown in HLR

This cause is sent to the MS if the MS is not known (registered) in the HRL.

Cause value = 3  Illegal MS

This cause is sent to the MS when the MS does not pass the authentication check, i.e. the SRES received from the MS is different from that generated by the network.

Cause value = 4  IMSI unknown in VLR

This cause is sent to the MS when the given IMSI is not known at the VLR.

Cause value = 5  IMEI not accepted

This cause is sent to the MS if the IMEI given cannot be accepted by the network.

G.2  Cause related to subscription options

Cause value = 11 PLMN not allowed

This cause is sent to the MS if it requests location updating in a PLMN where the MS, by subscription is not allowed to operate.

Cause value = 12 Location Area not allowed

This cause is sent to the MS if it requests location updating  in a Locationn area where the MS, by subscription, is not allowed to operate.

Cause value = 13 National roaming not allowed in this location area

This cause is sent to an MS which requests location updating in a location area of a PLMN which offers national roaming to that MS, but not in that specific location area.

GSM 04.08-DCS - 3.1.0 - page 436

G.3  Causes related to PLMN specific network failures and congestion

Cause value = 17 Network failure

This cause is sent to the MS if the MSC cannot service an MS generated request because of PLMN failures, e.g. problems in MAP.

Cause value = 22  Congestion

This cause is sent if the service request cannot be actioned because of congestion (e.g. no channel, facility busy/congested etc.)

G.4  Causes related to nature of request

Cause value = 32 Service option not supported

This cause is sent when the MS requests a service/facility in the CM SERVICE REQUEST message which is not supported by the PLMN.

Cause value = 33 Requested service option not subscribed

This cause ist sent when the MS requests a service option for which it has no subscription.

Cause value = 34 Service option temporarily out of order

This cause is sent when the MSC cannot service the request because of temporary outage of one or more functions required for supporting the service.

Cause value = 38 Call cannot be identified

This cause is sent when the network cannot identify the call associated with a call re- establishment request.

GSM 04.08-DCS - 3.1.0 - page 437

G.5  Causes related to invalid messages

Cause value = 96 Mandatory information element error.

See Annex H, sect. H.6.1.

Cause value = 97 Message type non-existent or not implemented.

see Annex H, sect. H.6.2.

Cause value = 98 Message non compatible with call state or message type non-existent or not implemented.

see Annex H, sect. H.6.3.

Cause value = 99 Information element non-existent or not implemented

See Annex H, sect. H.6.4.

Cause value = 100 Invalid information element contents

See Annex H, sect. H.6.5.

Cause value = 101 Message not compatible with call state

See Annex H, sect. H.6.6.

Cause value = 111 Protocol error, unspecified

See Annex H, sect. H.6.8.

GSM 04.08-DCS - 3.1.0 - page 446a

ANNEX J

(to Recommendation GSM 04.08)

ALGORITHM TO ENCODE FREQUENCY LIST INFORMATION ELEMENTS

J.1	Introduction

Some information elements encode frequency lists with a special method. The main specification specifies the meaning of the fields and hence the way to decode them, but the corresponding encoding algorithm is difficult to infer from the decoding algorithm. This annex is intended as an help for implementers of the encoding algorithm.

It could be shown that any set of frequency with less or the same number of frequencies as the number of words can be encoded with a careful choice of F1, F2, and so on, ie that a set of Wi can be found so that the decoding algorithm given in the main section will give back the frequency set. The right order is not the order of the frequency values.

J.2	General principle

The encoding algorithm is based on a recursive dichotomy of both the range (ie the set of values that are possible) and the subset (the values to encode).

GSM 04.08-DCS - 3.1.0 - page 446b

The dichotomy is best understood if the range is seen as a circle. For instance, for the 1023 range :

Figure 1 : Circular arrangement of 0..1022

The dichotomy consists in finding a value in the subset such that the diameter determined by this value splits the subset in two equal or nearly equal sub-subsets. In the following case, we see that value 250 is acceptable (the two sub-subsets have 3 elements), when value 290 is not acceptable (the two sub-subsets have 4 and 2 elements) :

Figure 2 : Example of dichotomy

GSM 04.08-DCS - 3.1.0 - page 446c

The pivot value is part of the information field, then the two sub-subsets are renumbered and the same algorithm is applied again on each of them. Because the range is halved at each step, the number of bits needed to encode a pivot value is 1 bit less than the number of bits needed to encode the parent pivot value.

The convention is that if the number of values is even, the left sub-subset (that is to say the values that can be expressed as the pivot value minus some integer between 1 and half the range) will have 1 element more than the right subset.

At each step the subset is numbered from 0 to the range minus 1. The coding in the information field of the pivot value is its value as renumbered, plus 1. Value 0 is reserved to indicate no element.

The order of appearance in the information field of the successive pivot values is particular. If we present the values as organised as a tree, with the left child being the pivot of the left sub-subset and the right child the pivot of the right sub-subset, the order of appearance is given by the following tree :

1

/                    \

2                                3

/       \                      /          \

4              6               5                7

/   \          /    \         /    \             /   \

8     12       10      14     9       13          11    15

(and so on)

This order as been chosen so that

a)	whatever the number N of elements in the set, the meaningful nodes are the first N and the value for all nodes from N+1 on are null (if sent),

b)	the tree and all subtrees are balanced.

Important properties of these trees are used in the algorithms (with generation 1 corresponding to the root) :

Generation g contains 2g-1 nodes, and their indices are 2g-1 to 2g-1;

For generation g, nodes 2g-1 to 2g-1+2g-2-1 are left children, the others are right children;

If node k belongs to generation g, its left child is node k + 2g-1 , and its right child is k + 2g;

Reciprocally, if k is a left child from generation g, its parent node is node k - 2g-2, and if k is a right child of generation g, its parent is node k - 2g-1.

GSM 04.08-DCS - 3.1.0 - page 446d

J.3	Performances

The number of bits needed to encode a given set of values depends of the number of values and on the range thay can span.

For the application on the BCCH and the SACCH (CA and BA information elements), 16 octets are available, and the number of frequencies that can be encoded in one information element is the following :

Range		Number

of frequencies

513 to 1024		2 to 16 (17 if frequency 0 is in)

257 to 512	 	2 to 18

129 to 256		2 to 22

113 to 128		2 to 29

up to 112			any

With two messages (for the BA), and if applying the same dichotomy principle to split the set between the two information elements, the number of frequencies that can be encoded is the following

Range		Number

of frequencies

513 to 1024		2 to 36

257 to 512	 	2 to 44

225 to 256		2 to 58

up to 224			any

The fixed length version of the frequency list information element allows the following :

Range		Number

of frequencies

513 to 1024		2 to 7 (8 if frequency 0 is in)

257 to 512	 	2 to 8

129 to 256		2 to 9

57 to 128			2 to 12

up to 56			any

GSM 04.08-DCS - 3.1.0 - page 446e

The number of frequencies as a function of the range and the length in octets of the variable length frequency list information element (including the message type and length fields) is given by the following table :

Range	513 to	257 to	129 to	up to	bit map

1024		512		256		128		range

octets

5 	1		1		1		1		8

6 	2		2		3		3		16

7 	3		3		4		4		24

8 	4		4		5		6		32

9 	5		6		6		8		40

10	6		7		8		10		48

11	7		8		9		12		56

12	9		9		11		14		64

13	10		11		13		16		72

14	11		12		14		18		80

15	12		13		16		21		88

16	13		15		18		24		96

17	14		16		20		26		104

18	16		18		22		29		112

19	17		19		24		32		120

20	18		21		26		--		128

21	20		22		28				136

22	21		24		30				144

23	22		26		32				152

24	24		27		34				160

25	25		29		37				168

26	26		30		40				176

27	28		32		42				184

28	29		34		45				192

29	30		36		48				200

30	32		38		50				208

31	33		40		53				216

32	35		42		56				224

Table 04.08/J.1 : Performance of the variable length frequency list information element

GSM 04.08-DCS - 3.1.0 - page 446f

J.4	Encoding algorithm

The choice is done recursively as given by the following programs, written in ADA :

Let us define the recursive procedure :

procedure ENCODE_SUBTREE(in INDEX : INTEGER;

in SET : SET_OF_VALUE;

in RANGE : INTEGER);

This procedure is given a set of integer values and an index. It chooses one of those values and computes the corresponding W(INDEX) (considered as a global variable), it splits the set less the value in two equal or nearly equal subsets, and calls itself recursively for each of those subsets, with suitable INDEX.

Assumption : all values in SET lie (inclusively) between 0 and RANGE-1, and they are all distinct.

As written, the program does not assume special values for the range. With a range such as 2k-1, some expressions can be simplified.

Declarative part :

INDEX_IN_SET : INTEGER;

begin

First the program tests the leaf conditions :

if SET'SIZE=0 then

W(INDEX) := 0;

return;

elsif SET'SIZE=1 then

W(INDEX) := 1 + SET(1);

return;

end if;

The following program finds a value in the set such that exactly SET'SIZE/2 values from the set are between this value plus 1 and this value plus half the range :

declare

N : INTEGER;

J : INTEGER;

begin

for I in 1..SET'SIZE loop

N:=0;

for J in 1..SET'SIZE loop

if (SET(J)-SET(I)) mod RANGE < (RANGE-1)/2 then

N := N+1;

GSM 04.08-DCS - 3.1.0 - page 446g

end if;

end loop;             if N-1 = SET'SIZE/2 then

The test compares N-1 because the possible pivot value is counted.

INDEX_IN_SET := I;

exit;

end if;

end loop;

end;

INDEX_IN_SET is then the index in the list of the pivot value.

The following sets W(INDEX)

W(INDEX) := SET(INDEX_IN_SET) + 1;

Then the program does the same thing for the two halves of the range delimited by W(INDEX) and W(INDEX)+RANGE/2. First the left subset :

declare

SUBSET : SET_OF_VALUE(1..SET'SIZE/2);

SUBSET_INDEX : INTEGER;

ORIGIN_VALUE : INTEGER;

begin

ORIGIN_VALUE := (SET(INDEX_IN_SET] + (RANGE-1)/2

+ 1) mod RANGE;

SUBSET_INDEX:=1;

for I in 1..SET'SIZE loop

if (SET(I)-ORIGIN_VALUE) mod RANGE) < RANGE/2 then

SUBSET(SUBSET_INDEX) :=

(SET(I) - ORIGIN_VALUE) mod RANGE;

SUBSET_INDEX := SUBSET_INDEX + 1;

end if;

end loop;

ENCODE_SUBTREE(

INDEX := INDEX +

GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX),

SET := SUBSET,

RANGE := RANGE/2);

end;

Then the right subset :

declare

SUBSET : SET_OF_VALUE(1..(SET'SIZE-1)/2);

SUBSET_INDEX : INTEGER;

ORIGIN_VALUE : INTEGER;

begin

ORIGIN_VALUE := (SET(INDEX_IN_SET] + 1) mod RANGE;

SUBSET_INDEX:=1;

for I in 1..SET'SIZE loop

if (SET(I)-ORIGIN_VALUE) mod RANGE) < RANGE/2 then

SUBSET(SUBSET_INDEX) :=

(SET(I) - ORIGIN_VALUE) mod RANGE;

GSM 04.08-DCS - 3.1.0 - page 446h

SUBSET_INDEX := SUBSET_INDEX + 1;

end if;

end loop;

ENCODE_SUBTREE(

INDEX := INDEX +

2*GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX),

SET := SUBSET,

RANGE := (RANGE-1)/2);

end;

end ENCODE_SUBTREE;

The initial call of the procedure depends on the format. Given some set to encode, the first problem is to verify that it can be encoded, and by so doing to choose the format.

First the encoding process must find the minimum range of the set, that is to say the minimum value R such that there exists one frequency F0 in the set such that all frequencies in the set can be written (F0 + N) mod 1024, with some N, 0 ≤ N ≤ R-1. The choice of the format depends on R and the number of frequencies : the 512 range format can be chosen only if R≤512, the 256 range format can be chosen only if R≤256, the 128 range format can be chosen only if R≤128.

If the chosen format is "1024 range", then the program must first check if frequency 0 is in the set. If so the F0 subfield is set to 1, and frequency 0 is removed from the set. Otherwise, the F0 subfield is set to 0. Then ENCODE_SUBTREE is called with INDEX := 1, SET set to the set of values equal to the ARFCN of all frequencies minus 1, and RANGE := 1023.

If the chosen format is "512 range", "256 range" or "128 range", F0 is chosen as ORIG-ARFCN and ENCODE_SUBTREE is called with INDEX := 1, SET set to the set of values equal to the ARFCN of all frequencies except F0, minus F0+1, and RANGE set respectively to 511, 255 or 127.

GSM 04.08-DCS - 3.1.0 - page 446i

J.5	Decoding

The decoding algorithm as given below in the inverse transform of the program given in the previous section, for the specific case where the original range is a power of 2 minus 1. It is given a set of integer values W(i), and an original range R, and it builds a set of values from 0..R-1.

The program is here written so that the fact that it is the inverse of the encoding program needs no more proof.

procedure DECODE(in W : array <> of INTEGER;

out SET : SET_OF_VALUE;

in ORIGINAL_RANGE : INTEGER);

-- local variables

INDEX : 1..W'SIZE; 	  RANGE : INTEGER;

N : INTEGER;

begin

for K in 1..W'SIZE loop

The next loop follows the tree from child to parent, from the node of index K to the root (index 1). For each iteration the node of index INDEX is tackled. The corresponding range is RANGE, and N is the value of the element in the range defined by the node.

The data are set to their initial values :

INDEX := K;

RANGE := ORIGINAL_RANGE/

GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);

N := W(INDEX) - 1;

while INDEX>1 loop

Due to the assumption that the original range is a power of two nimus one, the range for the parent node can be easily computed, and does not depend whether the current node is a left or right child :

RANGE := 2*RANGE + 1;

Let us note J := 2g-1, g being the generation of node INDEX. We have J = GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX). The numbering used in the tree is such that the nodes of index J to J + J/2 - 1 are left children, and the nodes of index J/2 to J+J-1 are right children. Hence an easy test to distinguish left and right children:

if 2*INDEX <

3*GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX)

then         -- left child

GSM 04.08-DCS - 3.1.0 - page 446j

The next computation gives the index of the parent node of the node of index INDEX, for a left child :

INDEX := INDEX -

GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX)/2;

The next formula is the inverse of the renumbering appearing in the encoding for a left child. It gives the value of the parent node in the range defined by the grand-parent node:

N := (N + W(INDEX) - 1 + (RANGE-1)/2 + 1)

mod RANGE;

else                          -- right child

The next computation gives the index of the parent node of the node of index INDEX, for a right child :

INDEX := INDEX -

GREATEST_POWER_OF_2_LESSER_OR_EQUAL_TO(INDEX);

The next formula is the inverse of the renumbering appearing in the encoding for a right child:

N := (N + W(INDEX) - 1 + 1) mod RANGE;

end if;

end loop;

F(K) := N + 1;

end loop;

end;

A careful study will show that the programs given in the main part of the Recommendation are equivalent to the one presented here. The main difference is the use of different remanent variables to remove most of the calls to the function giving the greatest power of 2 lesser or equal to some integer.

The decoding must be terminated by the correction specific to the format.

J.7	A detailed example

Let us take the following subset of 16 elements of the set [0..1023] :

[13, 71, 122, 191, 251, 321, 402, 476, 521, 575, 635, 701, 765, 831, 906, 981]

Range 1024 format will be used. Frequency 0 is not is the set, thus field F0 is set to 0. The set is renumbered, so as to give a subset of 0..1022 : [12, 70, 121, 190, 250, 320, 401, 475, 520, 574, 634, 700, 764, 830, 905, 980].

GSM 04.08-DCS - 3.1.0 - page 446k

For the first node (corresponding to W(1)), the value 121 satisfies the requirements. The opposite value is 121 + 511 = 633. There are 8 values between 122 and 633 (namely the lefthand subset 634, 700, 764, 830, 905, 980, 12 and 70), and 7 values between 634 and 120 (namely the righthand subset 190, 250, 320, 401, 475, 520 and 574).

The encoded value W(1) is 121 + 1, ie 122.

The second node (corresponding to W(2)) is the lefthand child of the first node. The corresponding subtree has to encode for the lefthand subset, renumbered beginning at 633. This gives the following 8 element subset of 0..510, ordered as resulting from the example of algorithm : [402, 460, 1, 67, 131, 197, 272, 347]. Out of these values, 1 splits the set in 4 and 3, and the encoded value W(2) is 2.

Similarily, the third node (W(3)) is the righthand child of the first node and then the corresponding subtree encodes for the righthand subset, renumbered starting at 122. This gives the following set of 0..510 : [68, 128, 198, 279, 353, 398, 452]. Out of these values, 68 splits the set into 3 and 3, and the encoded value W(3) is 69.

The same method is applied for all nodes, giving the following encoded values per node :

node          value					node          value

1			122				9			 83

2			  2				10			  3

3			 69				11			 24

4			204				12			 67

5			 75				13			 54

6			 66				14			 64

7			 60				15			 70

8			 70				16			  9

The encoding then consists in formatting, in that order :

122 on 10 bits, then 2 and 69 on 9 bits each, then 204, 75,  66 and 60 on 8 bits each, then 70, 83, 3, 24, 67, 54, 64 and 70 on 7 bits each, and finally 9 on 6 bits.

Conversely the decoding can be done easily. For instance for node 2, the original value is

(122 - 512 + 2) smod 1023 = 635

For node 14, we have as original value :

(122 - 512 + (2 + (66 + 64)smod 255)smod 511)smod 1023  	= 765