ETSI/GSM

Recommendation:         GSM 04.12

Title:                  Short Message Service Cell Broadcast (SMSCB)

Support on the Mobile Radio Interface

Date:                   February 1992

List of contents:

1.      Scope

2.      General description

3.      Message format on the BSS-MS interface

3.1  General

3.2  Format convention

3.3  Message content

Original language:      English

Number of pages:        4

1       Scope

This recommendation describes how the Short Message Service - Cell Broadcast (SMSCB) (Teleservice 23 as specified in GSM Rec 02.03) is supported over the mobile radio interface.

2.      General Description

SMSCB is a service in which short messages may be broadcast from a PLMN to MS's.  SMSCB messages come from different sources (e.g. traffic reports, weather reports). The source and subject of the message is identified by a message identifier in the SMSCB header. A sequence number in the SMSCB header enables the MS to determine when a new message of a given source is available. An MS can read the header and then decide whether or not to read the rest of the message.

SMSCB messages are not acknowledged by the MS. Reception of SMSCB by the MS is only possible in idle mode, and the service is designed so as to minimise the adverse impact on the operation of DRX in the MS. The geographical area over which each SMSCB message is transmitted is selected by the PLMN operator, by agreement with the provider of the information.

3.      Message Format on BTS-MS Interface

3.1       General

The SMSCB message at the Application layer consists of an 88 octet block of information,  as defined in Rec 03.41.The 88 octet block is segmented into four 22 octet blocks. A 1 octet header message type is added.The overall blocks are thus 23 octets in length.

The message is sent on the channel allocated as CBCH by GSM Rec 05.02. The timing of the messages is defined in GSM Rec 05.02.

3.2       Format convention

3.2.1     Numbering convention

The basic convention used in this recommendation is illustrated in fig 1. The bits are grouped into octets. the bits of an octet are shown horizontally and are numbered from 1 to 8. Multiple octets are shown vertically and are numbered from 1 to 23.

Bit No     8    7   6   5    4    3    2    1

┌─────────────────────────────────────┐

Octet No  1    │                                     │

├─────────────────────────────────────┤

2    │                                     │

├─────────────────────────────────────┤

23   │     SMSCB Information               │

└─────────────────────────────────────┘

Figure 1/GSM 04.12 Format convention

3.2.2     Order of bit transmission

The message is sent on the CBCH as defined in Rec. GSM 05.02 using the coding defined for that channel.

The order of bit transmission is defined in Rec. GSM 04.04.

3.3       Message content

The 23 octet blocks are coded as follows:

Bit No     8    7   6   5    4    3    2    1

┌─────────────────────────────────────┐

Octet No  1    │      Message type                   │

├─────────────────────────────────────┤

2-23 │      SMSCB information              │

└─────────────────────────────────────┘

Figure 2/GSM 04.12 Message content

3.3.1     Message Type

The purpose of the Message Type is to identiify the function of the message being sent. The message type is coded as shown in figur 3/GSM 04.12.

Bit No     8    7   6   5     4    3    2    1

┌──────┬───────┬─────┬────────────────┐

Octet No  1    │ Spare│L P  D │Spare│Sequence Number │

│      │0   1  │     │                │

└──────┴───────┴─────┴────────────────┘

Bit No     4    3    2    1

┌──────────────────┐

│ 0    0    0    0 │    first block

│ 0    0    0    1 │    second block

│ 0    0    1    0 │    third block

│ 0    0    1    1 │    fourth block

│ 1    1    1    1 │    Mesage does not contain valid

└──────────────────┘    SMSCB information

all other values are reserved

Table 1/GSM 04.12

3.3.2     SMSCB information

The SMSCB information coding is defined in Rec. GSM 03.41