GSM Recommendation:  05.03

Title: CHANNEL CODING

Release Date: October, 1994

CONTENTS:

1.	SCOPE

2.	GENERAL

2.1  General Organisation

2.2  Naming Convention

3.	TRAFFIC CHANNELS (TCH)

3.1  Speech channel at full rate (TCH/FS)

3.2  Speech channel at half rate (TCH/HS)  (tba)

3.3  Data channel at full rate, 9.6 kbit/s services (TCH/F9.6)

3.4  Data channel at full rate, 4.8 kbit/s services (TCH/F4.8)

3.5  Data channel at half rate, 4.8 kbit/s services (TCH/H4.8)

3.6  Data channel at full rate, ≤ 2.4 kbit/s services (TCH/F2.4)

3.7  Data channel at half rate, ≤2.4 kbit/s  services (TCH/H2.4)

4.	CONTROL CHANNELS

4.1  Slow associated control channel (SACCH)

4.2  Fast associated control channel (FACCH)

4.3  Broadcast, paging and access grant channels

(BCCH, PCH and AGCH)

4.4  Stand-alone dedicated control channel (SDCCH)

4.5  Random access channel (RACH)

4.6  Synchronisation channel (SCH)

4.7  Handover Access Burst

ANNEXES:  1. Summary of channel types

2. Summary of polynomials for convolutional codes

3. Examples   (to be added)

Number of pages: 22

Note: (tba) indicates a section to be added later.

Recommendation 05.03

CHANNEL CODING

1.		SCOPE:

A reference configuration of the transmission chain is shown in Rec.05.01. According to this reference configuration, this recommendation  specifies  the data blocks given to the encryption unit.

It includes the specification of encoding, reordering,

interleaving and the stealing flag. It does not specify

the channel decoding method.

The definition is given for each kind of logical channel, starting from the data provided to the channel encoder by the speech coder, the data terminal equipment, or the controler of the MS or BS. The definitions of the logical channel types used in this recommendation are given in Rec.05.02, a summary is in annex 1.

2.			GENERAL

2.1		GENERAL ORGANISATION:

Each channel has its own coding and interleaving scheme. However, the channel coding and interleaving is organised in order to have as much as possible a unified decoder structure.

Each channel uses, in this order, the following sequence of operations:

-  The information bits are coded with a systematic

block code, building words of information + parity

bits.

-  These information + parity bits are encoded with a

convolutional code, building the coded bits.

-  Reordering and interleaving the coded bits, and

adding a stealing flag, gives the interleaved bits.

All those operations are made block by block, the size of which depends on the channel. However, most of the channels use at one point a common structure which is a block of 456 coded bits, interleaved and mapped onto bursts in a very similar way for all of them. Figure 1 gives a diagram showing the general structure of the channel coding.

This block of 456 coded bits is the basic structure of the channel coding scheme. In the case of speech TCH, this block carries the information of one speech frame. In case of control channels, it carries one message.

figure 1 :  CHANNEL CODING AND INTERLEAVING ORGANISATION

In the case of fast ACCH, such a block is stolen from the TCH, used for an ACCH message, and is inserted in the TCH interleaving stucture. Each block of 456 coded bits has in addition a stealing flag (8 bits), indicating if the block belongs to the TCH or to the fast ACCH. In case of slow ACCH, BCCH or CCCH, this stealing flag is dummy.

Some cases do not fit in the general organisation, and do not use the block of 456 coded bits. They are the random access messages of the RACH on uplink and the synchronisation information broadcast on the SCH.

2.2   Naming Convention

For ease of understanding a naming convention for bits is given for use throughout the recommendation :

-  General naming

"k" and "j" for numbering of bits in data blocks and

bursts.

"Kx" gives the amount of bits in one block, where "x"

refers to the data type

"n" is used for numbering of delivered data blocks

where

"N" marks a certain data block

"B" is used for numbering of bursts or blocks where

"B0" marks the first burst or block carrying bits from

the data block with n = 0 (first data block in the

transmission)

- Data delivered to the encoding unit (interface 1 in

fig. 1) :

d (n, k) or d (k) for  k = 0,1,.....,Kd-1

n = 0,1,.....,N,N+1,......

- Data after the first encoding step

(block code, cyclic code ; interface 2 in fig. 1) :

u(n,k) or u (k) for  k = 0,1,...,Ku-1

n = 0,1,...,N,N+1,...

- Data after the second encoding step

(convolutional code ; interface 3 in fig. 1) :

c(n,k) or c(k) for  k = 0,1,.....,Kc-1

n = 0,1,.....,N,N+1,....

- Interleaved data :

i(B,k) for  k = 0,1,....,Ki-1

B = B0, B0+1,....

- Bits in one burst (interface 4 in fig. 1) :

e(B,k) for  k = 0,1,......,114,115

B = B0, B0 + 1,....

3	TRAFFIC CHANNELS (TCH):

Two kinds of traffic channels are considered: speech and data. Both of them use the same general structure (see fig.1), and in both cases, a piece of information can be stolen by the fast ACCH.

3.1	Speech channel at full rate (TCH/FS):

The speech coder delivers to the channel encoder a sequence of blocks of data. In case of a full rate speech TCH, one block of data corresponds to one speech frame. Each block contains 260 information bits,including 182 bits of class 1 (protected bits), and 78 bits of class 2 (no protection), (see Tables 2 and 3)

The bits delivered by the speech codec are labeled {d(0), d(1),..., d(259)}, defined in the order of decreasing importance, as specified in 06.10.

3.1.1 Parity and tailing for a speech frame:

a) Parity bits:

The 50 first bits of class 1 are protected by three parity bits used for error detection. These parity bits are added to the 50 bits, according to a degenerate (shortened) cyclic code (53,50,2), using the generator polynomial:

g(D) = D3 + D + 1

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D52 + d(1)D51 +.... + d(49)D3 + p(0)D2 + p(1)D+ p(2)

where p(0), p(1), p(2) are the parity bits, when

divided by g(D), yields a remainder equal to 1 + D + D2

b) Tailing bits and reordering:

The information and parity bits of class 1 are reordered, defining 189 information + parity + tail bits of class 1, {u(0), u(1),..., u(188)} defined by:

u(k) = d(2k)  and  u(184-k) = d(2k+1)  for k=0,1,...,90

u(91+k) = p(k)                         for k=0,1,2

u(k) = 0   (tail bits)         for k=185, 186, 187, 188

3.1.2 Convolutional encoder:

The class 1 bits are encoded with the 1/2 rate convolutional code defined by the polynomials:

G0 = 1 + D3+ D4

G1 = 1 + D + D3+ D4

The coded bits {c(0), c(1),..., c(455)} are then defined by:

- class 1 : c(2k)   = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)

for k=0,1,.....,188   and u(k)=0 if k<0

- class 2 : c(378+k) = d(182+k)    for k=0,1,....,77

3.1.3 Interleaving

The coded bits are reordered and interleaved according to the following rule :

i(B,j)  = c(n,k)

for  k  = 0,1,....,455

n  = 0,1,....,N,N+1,....

B  = B0 + 4. n + k mod (8)

j  = 2[(49k) mod 57] + [(k mod 8) div 4]

The result of the interleaving is a distribution of the reordered 456 bits of a given data block, n=N, over 8 blocks using the even numbered bits of the first 4 blocks (B = B0 +  4N + 0,1,2,3) and odd numbered bits of the last 4 blocks (B = B0 + 4 N + 4,5,6,7). The reordered bits of the following data block, n = N+1, use the even numbered bits of the blocks B = B0 + 4 N + 4,5,6,7 and the odd numbered bits of the blocks B= B0 + 4 (N+2) + 0,1,2,3. Continuing with the next data blocks shows that one block always carries 57 bit of data from one data blocks (n =N) and 57 bit of data from the next block (n = N+1), where the bits from the data block with the higher number always are the even numbered data bits, else the odd numbered. The block of coded data is interleaved "block diagonal", where a new data block starts every 4th block and is distributed over 8 blocks.

3.1.4 Mapping on a Burst

The mapping is given by the rule :

e(B,j) = i(B,j) and e(B, 59+j) = i(B,57 + j)

for j = 0,... 56

and

e(B,57) = hl(B) and e(B,58) = hu(B)

The two bits, labeled hl(B) and hu(B) on burst number B

are flags used for indication of control channel

signalling.

- hl(B) = 0 and hu(B) = 0 indicates, that all bits

in burst B belong to a traffic channel frame.

- hu(B)  =  1 indicates that all even numbered bits

are used for signalling purposes.

- hl(B)  =  1 indicates that all odd numbered bits are

used for signalling purposes.

In a traffic channel these flags indicate stolen bits

for signalling purposes (see section 4.2.5).

3.2	Speech channel at half rate (TCH/HS):

To be defined for a future evolution of the system.

3.3	Data channel at full rate, 9.6 kbit/s services:

The definition of a 12.0 kbit/s data flow for data services is given in Recommendation 04.21.

3.3.1 Interface with user unit:

The user unit delivers to the encoder a bit stream organised in blocks of 60 information bits (data frames) every 5 ms. Four such blocks are dealt with together in the coding process {d(0),...,d(239)}. For non-transparent services those four blocks will align with one 240-bit RLP frame.

3.3.2 Block code:

The block of 4 * 60 information bits is not encoded, but only increased with 4 tail bits equal to 0 at the end of

the block.

u(k)=d(k)     k=0,...,239

u(k)=0        k=240,...,243

3.3.3 Convolutional encoder:

This block of 244 bits {u(0),...,u(243)} is encoded with the punctured convolutional code of rate 1/2 defined by the following polynomials:

G0 = 1 + D3+ D4

G1 = 1 + D + D3+ D4

resulting in 488 coded bits {C(0), C(1),..., C(487)} with

C(2k)  = u(k)+       +u(k-3)+u(k-4)

C(2k+1)= u(k)+ u(k-1)+u(k-3)+u(k-4)

for   k = 0,...243    ; u(k)=0,K<0

The code is punctured in such a way that the following 32 coded bits:

{C(11 + 15j); j = 0,..,31} are not transmitted.

The result is a block of 456 coded bits, {c(0),..., c(455)}

3.3.4 Interleaving:

The coded bits are reordered and interleaved according

to the following rule :

i(B,j) = c(n,k)

for      k = 0,....,455

n = 0,1,...,N,N+1,....

B = Bo +4n + k mod (19) + k div 114

j = k mod (19) + 19  [k mod (6)]

The result of the interleaving is a distribution of the

reordered 114 bit of a given data block, n = N, over 19

blocks, 6 bits equally distributed in each block, in a

diagonal way over consecutive blocks.

The block of coded data is interleaved "diagonal", where

a new block of coded data starts with every block and is

distributed over 19 blocks.

3.3.5 Mapping on a Burst

The mapping is done as specified for TCH/FS in section

3.1.4 On bitstealing by a FACCH, see section 4.2.5.

3.4	Data channel at full rate, 4.8 kbit/s services (TCH/F4.8):

The definition of a 6.0 kbit/s data flow for data services is given in Recommendation 04.21.

3.4.1 Interface with user unit:

The user unit delivers to the encoder a bit stream organised in blocks of 60 information bits (data frames) every 10 ms, {d(0),..., d(59)}.

3.4.2 Block code:

Sixten bits equal to 0 are added to the 60 information bits, the result being a block of 76 bits, {u(0),..., u(75)}, with:

u(19k+p) = d(15k+p)   for k = 0,...,3  and  p = 0,...,14;

u(19k+p) = 0          for k = 0,...,3  and  p = 15,...,18.

Two such blocks forming a block of 152 bits

{u'(0),...,u'(151)}                       are dealt with together in the rest of the coding process

u'(k)     = u1(k) , k=0,..,75 (u1 = 1:st block)

u'(k+76)  = u2(k) , k=0,..,75 (u2 = 2:nd block)

3.4.3 Convolutional encoder:

This block of 152 bits is encoded with the convolutional code of rate 1/3 defined by the following polynomials:

G1 = 1 + D  + D3 + D4

G2 = 1 + D2 + D4

G3 = 1 + D  + D2 + D3 + D4

The result is a block of 3 * 152 = 456 coded bits, {c(0),...c(455)},

c(3k)    = u'(k)+u'(k-1)+        u'(k-3)+u'(k-4)

c(3k+1)  = u'(k)+        u'(k-2)+        u'(k-4)

c(3k+2)  = u'(k)+u'(k-1)+u'(k-2)+u'(k-3)+u'(k-4)

for   k = 0,...,151  ;   u'(k)=0, k<0

3.4.4 Interleaving:

The interleaving is done as specified for the TCH/F.9.6 in section 3.3.4

3.4.5 Mapping on a Burst

The mapping is done as specified for the TCH/FS in section 3.1.4. On bitstealing for signalling purposes by a FACCH, see  section 4.2.5.

3.5	 Data channel at half rate, 4.8 kbit/s services

(TCH/H4.8):

The definition of a 6.0 kbit/s data flow for data services is given in Recommendation 04.21.

3.5.1 Interface with user unit:

The user unit delivers to the encoder a bit stream organised in blocks of  60 information bits (data frames) every 10 ms. Four such blocks are dealt with together in the coding process  {d(0),...,d(239)}.

For non transparent services those four blocks shall align with one complete 240-bit RLP frame.

3.5.2 Block code:

The block encoding is done as specified for the TCH/F9.6 in section 3.3.2.

3.5.3 Convolutional encoder:

The convolutional encoding is done as specified for the TCH/F9.6 in section 3.3.3.

3.5.4 Interleaving:

The interleaving is done as specified for the TCH/F9.6 in section 3.3.4.

3.5.5 Mapping on a Burst

The mapping is done as specified for the TCH/FS in section 3.1.4. On bitstealing for signalling purposes by a FACCH, see section 4.2.5.

3.6	Data channel at full rate, 2.4 kbit/s and less

services (TCH/F2.4):

The definition of a 3.6 kbit/s data flow for data services is given in Recommendation 04.21.

3.6.1 Interface with user unit:

The user unit delivers to the encoder a bit stream organised in blocks of 36 information bits (data frames) every 10 ms. Two such blocks are dealt with together in the coding process{d(0),...,d(71)}.

3.6.2 Block code:

This block of 72 information bits is not encoded, but only increased with four tail bits equal to 0 at the end of the block.

u(k) = d(k) , k= 0,...,71

u(k) = 0    , k=72,...,75

3.6.3 Convolutional encoder:

This block of 76 bits {u(0),...u(75)} is encoded with the convolutional code of rate 1/6 defined by the following polynomials:

G1 = 1 + D  + D3 +D4

G2 = 1 + D2 + D4

G3 = 1 + D  + D2 + D3 + D4

G1 = 1 + D  + D3 + D4

G2 = 1 + D2 + D4

G3 = 1 + D  + D2 + D3 + D4

The result is a block of 456 coded bits :

{c(0), c(1),...,c(455)}, defined by

c(6k)   = c(6k+3) = u(k)+u(k-1)+       u(k-3)+u(k-4)

c(6k+1) = c(6k+4) = u(k)+       u(k-2)+       u(k-4)

c(6k+2) = c(6k+5) = u(k)+u(k-1)+u(k-2)+u(k-3)+u(k-4)

for   k = 0,...,75  ;    u(k) = 0, k<0

3.6.4 Interleaving:

The interleaving is done as specified for the TCH/FS in section 3.1.3

3.6.5 Mapping on a Burst

The mapping is done as specified for the TCH/FS in section 3.1.4.

3.7   Data channel at half rate, ≤2.4 kbit/s services (TCH/H2.4):

The definition of a 3.6 kbit/s data flow for data services      is given in Recommendation 04.21

3.7.1 Interface with user unit:

The user unit delivers to the encoder a bit stream organised in blocks of 36 information bits (data frames) every 10 ms. Two such blocks are dealt with together in the coding process, {d(0),...,d(71)}.

3.7.2 Block code:

The block of 72 information bits is not encoded, but only increased with 4 tail bits equal to 0, at the end of the block.

Two such blocks forming a block of 152 bits {u(0),...,u(151)} are dealt with together in the rest of the coding process.

u(k)    = d1(k) , k=0,...,75 (d1 = 1:st information block)

u(k+76) = d2(k) , k=0,...,75 (d2 = 2:nd information block)

u(k)    =  0    , k=72,...,75,148,...,151

3.7.3 Convolutional encoder:

The convolutional encoding is done as specified for the TCH/F4.8 in section 3.4.3.

3.7.4 Interleaving:

The interleaving is done as specified for the TCH/F4.8 in section 3.4.4.

3.7.5 Mapping on a Burst

The mapping is done as specified for the TCH/FS in section 3.1.4. On bit stealing for signalling purposes by a FACCH, see section 4.2.5.

4.	CONTROL CHANNELS :

4.1	  Slow associated control channel (SACCH):

4.1.1 Block constitution:

The message delivered to the encoder has a fixed size of 184 information bits {d(0),...,d(183)}. It is  delivered on a burst mode.

4.1.2 Block code:

The block of 184 information bits is encoded, using a shortened binary cyclic code (FIRE code), with the following generator polynomial :

g(D) = (D23 + 1).(D17 + D3 + 1)

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

u(0).D223 + u(1).D222 + ... + u(222).D + u(223)

where {u(0), u(1),..., u(183)} are the information bits  (u(k)=d(k), k=0,...,183) and {u(184), u(185),..., u(223)} are the parity bits ( u(k+184)=p(k), k=0,...,39 ), when divided by g(D) yields a remainder equal to 1 + D + D2 +...+ D39.

The result is a block of 224 bits, completed by 4 tail bits equal to 0 at the end of the block : {u(0), u(1),...,u(223)}, with u(224) to u(227) equal to 0.

4.1.3 Convolutional encoder:

This block of 228 bits is encoded with the 1/2 rate convolutional code (identical to the one used for TCH/FS) defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D  + D3 + D4

This results in a block of 456 coded bits : {c(0),...,c(455)} defined by

c(2k)   = u(k)+       u(k-3)+u(k-4)

c(2k+1) = u(k)+u(k-1)+u(k-3)+u(k-4)

for   k = 0,...,227  ;   u(k)=0, k<0

4.1.4 Interleaving:

The coded bits are reordered and interleaved according to the following rule

i(B,j)  = c(n,k)

for k   = 0,1......,455

n   = 0,1......,N,N+1,....

B   = B0 + 4 . n + k mod (4)

j   = 2 [ (49k) mod 57 ] + [ ( k mod 8 ) div 4 ]

The result of the reordering of bits is the same as given for a TCH/FS (section 3.1.3) as can be seen from the evaluation of the bit number-index j, distributing the 456 bits over 4 blocks on even numbered bits and 4 blocks on odd numbered bits. The resulting 4 blocks are built by putting blocks with even numbered bits and blocks with odd numbered bits together into one block.

The block of coded data is interleaved "block rectangular" where a new data block starts every 4th block and is distributed over 4 blocks.

4.1.5 Mapping on a Burst

The mapping is given by the rule

e(B,j) =i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,...,56

and

e(B,57) = h1(B) and e(B,58) = hu(B)

The two bits labeled h1(B) and hu(B) on burst number B are flags used for indication of control channel signalling. They are set to "1" for a SACCH.

4.2	Fast associated control channel (FACCH):

4.2.1 Block constitution:

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

4.2.2 Block code:

The block encoding is done as specified for the SACCH in section 4.1.2.

4.2.3 Convolutional encoder :

The convolutional encoding is done as specified for the SACCH in section 4.1.3.

4.2.4 Interleaving:

The interleaving is done as specified for the TCH/FS in section 3.1.3.

4.2.5 Mapping on a Burst

A FACCH frame of 456 coded bits is mapped on 8 consecutive bursts as specified for the TCH/FS in section 3.1.4. As a FACCH is transmitted on bits which are stolen in a burst from the traffic channel, the even numbered bits in the first 4 bursts and the odd numbered bits of the last 4 bursts are stolen.

To indicate this to the receiving device the flags h1(B) and hu(B) have to be set according to the following rule :

hu(B)   =   1 for the first 4 burst

(even numbered bits are stolen)

h1(B)   =   1 for the last 4 bursts

(odd numbered bits are stolen)

The consequences of this bitstealing by a FACCH is for a

- speech channel (TCH/FS) and data channel (TCH/F2.4) :

One full frame of data is stolen by the FACCH.

- data channels with 114 coded bits (TCH/F9.6 and

TCH/H4.8) :

The bitstealing by a FACCH disturbs 3 bit belonging to      the same data block in each of the 8 disturbed bursts.       Thus a maximum of 24 bit of one data block may be disturbed.

- Data channels with 228 coded bits (TCH/F4.8 and

TCH/H2.4) :

The bitstealing by a FACCH disturbs in the worst case 6    bit belonging to the same data block in each of the 8   disturbed bursts. Thus a maximum of 48 bit belonging to one data block may be disturbed.

4.3	Broadcast, Paging and Access grant channels

(BCCH, PCH, AGCH):

The coding scheme used for the broadcast, paging and

access grant messages is the same as for the SACCH

messages, specified in 4.1.

4.4	Stand-alone dedicated control channel:

The coding scheme used for the dedicated control channel messages is the same as for SACCH messages, specified in 4.1.

4.5	Random access channel (RACH):

The burst carrying the random access uplink message has a different structure. It contains 8 information bits d(0),...,d(7).

Six parity bits p(0) to p(5) are defined in such a way that in GF(2) the binary polynomial d(0)D13 +...+ d(7)D6 + p(0)D5 +...+ p(5), when divided by D6+D5+D3+D2+D+1 yields a remainder equal to D5+D4+D3+D2+D+1.

The six bits of the BSIC, B(0) to B(5), of the BS to which the Random Access is intended, are added bitwise modulo 2 to the six parity bits. P(0) to P(5). This results in six colour bits, C(0) to C(5) defined as C(k) = b(k) + p(k) (k= 0 to 5) where

b(0) = MSB of PLMN colour code

b(5) = LSB of BS colour code.

This defines {u(0), ..., u(17)} by:

u(k)    = d(k)  (k = 0 to 7)

u(8+k)  = c(k)  (k = 0 to 5)

u(14+k) = 0     (k = 0 to 3)  (tail bits)

The bits {e(0), .., e(35)} are obtained by the same convolutional code of rate 1/2 as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D  + D3 + D4

and with

e(2k)   = u(k)+       u(k-3)+u(k-4)

e(2k+1) = u(k)+u(k-1)+u(k-3)+u(k-4)

for  k = 0, ...,17 ;    u(k)=0, k<0

4.6	Synchronisation channel (SCH):

The burst carrying the synchronisation information on the downlink BCCH has a different structure. It contains 25 information bits {d(0), ..., d(24)}, 10 parity bits {p(0),..., p(9)} and 4 tail bits. The precise ordering of the information bits is given in Rec. 04.08, Section 9.1.28 ("Synchronization channel informaion")

The ten parity bits p(0) to p(9) are defined in such a way that in GF(2) the  binary  polynomial d(0)D34 +...+ d(24)D10 + p(0)D9 +...+ p(9), when divided by D10+D8+D6+D5+D4+D2+1 yields a remainder equal to D9+D8+D7+D6+D5+D4+D3+D2+D+1.

Thus the encoded bits {u(0),..., u(38)} are:

u(k)    = d(k)  (k = 0 to 24)

u(25+k) = p(k)  (k = 0 to 9)

u(35+k) = 0	  (k = 0 to 3)  (tail bits)

The bits {e(0),..., e(77)} are obtained by the same convolutional code of rate 1/2 as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D  + D3 + D4

and with

e(2k)   = u(k)+       u(k-3)+u(k-4)

e(2k+1) = u(k)+u(k-1)+u(k-3)+u(k-4)

for  k = 0,....,77 ;    u(k)=0, k<0

4.7  Handover Access Burst

The encoding of this burst is as defined in section 4.5 for the random access channel (RACH). The BSIC used shall be the BSIC of the BS to which the HO is done.

TABLE 1

REORDERING AND PARTITIONING A CODED BLOCK INTO 8 SUBBLOCKS

TABLE 2

ENCODER OUTPUT PARAMETERS IN ORDER OF OCCURENCE

AND BIT ALLOCATION WITHIN THE SPEECH FRAME OF 260 BITS/20 ms

TABLE 3

SUBJECTIVE IMPORTANCE OF ENCODED BITS

(Parameter names and bit indices refer to table 2)

ANNEX 1

SUMMARY OF CHANNEL TYPES

TCH/FS  :  full rate speech trafic channel

TCH/F9.6:  9.6 kbit/s full rate data traffic channel

TCH/F4.8:  4.8 kbit/s full rate data traffic channel

TCH/H4.8:  4.8 kbit/s half rate data traffic channel

TCH/F2.4:  ≤ 2.4 kbit/s full rate data traffic channel

TCH/H2.4:  ≤ 2.4 kbit/s half rate data traffic channel

SACCH :  slow associated control channel

FACCH :  fast associated control channel

SDCCH :  stand-alone dedicated control channel

BCCH  :  broadcast control channel

PCH   :  paging channel

AGCH  :  access grant channel

RACH  :  random access channel

SCH   :  synchronisation channel

NOTE :

With respect to Recs 05.01 and 05.02 the subcategories of channels(eg.  FACCH/F, FACCH/H,...) are not distinguished since they use the same coding scheme as the main channel type.

ANNEX 2

SUMMARY OF POLYNOMIALS USED FOR CONVOLUTIONAL CODES

G0 = 1+ D3 + D4              TCH/FS, TCH/F9.6, TCH/H4.8,           			                        SDCCH, BCCH, PCH,SACCH, FACCH,

AGCH, RACH, SCH

G1 = 1 + D + D3 + D4         TCH/FS, TCH/F9.6, TCH/H4.8,  									           SACCH, FACCH, SDCCH, BCCH, PCH,

AGCH, RACH, SCH, TCH/F4.8, 										           TCH/F2.4, TCH/H2.4

G2 = 1 + D2 + D4 		        TCH/F4.8, TCH/F2.4, TCH/H2.4

G3 = 1 + D + D2 + D3 + D4    TCH/F4.8, TCH/F2.4, TCH/H2.4

ANNEX  3

EXAMPLES

Several examples are given for different channels. In each case, the blocks of information bits are chosen as follows :

{u(0),u(1),........} is defined by :

u(k)=1				       for k ε [0,8]

u(k)= u(k-5) + u(k-9)   for k ε [9,∞]

1.		Speech TCH:

An example is given on the following page for a speech TCH, with x1=182 bits in class 1 and x2=78 bits in class 2.

The content of two consecutive speech frames A and B, chosen at random as explained above, is processed as explained in 3.1, and the four bursts which are in common with those two speech frames are presented :

┌───────────────────────┐

B0 B1 B2 B3 B4 B5 B6 B7  speech frame B

└───────────────────────┘

┌───────────────────────┐

A0 A1 A2 A3 A4 A5 A6 A7              speech frame A

└───────────────────────┘

^  ^  ^  ^

4 bursts common to frame A and B

In the burst description, the label PR represents the 26 bits of preamble. The stealing flags of both frames are set to 0(=00000000).

The tail bits (two 0 at each end of a burst) are represented.