ETSI/GSM

GSM Recommendation:	05.04

version:	3.1.2

Title:	MODULATION

Date:	February 1992

CONTENTS:

1. SCOPE

2. MODULATION FORMAT

Number of pages: 3

MODULATION

1	SCOPE

The modulator receives the bits from the encryption unit, see Rec.05.01, and produces an RF signal. The filtering of the RF signal necessary to obtain the spectral purity is not defined, neither are the tolerances associated with the theoretical filter requirements specified. These are contained in Rec.05.05.

2	MODULATION FORMAT

2.1	Modulating bit rate:

The modulating bit rate is 1/T = 1625/6 kbit/s (ie app. 270.833 kbit/s).

2.2	Start and stop of the burst:

Before the first bit of the bursts as defined in 05.02 enters the modulator, the modulator has an internal state as if a modulating bit stream consisting of consecutive ones (di = 1) had entered the differential encoder. Also after the last bit of the time slot, the modulator has an internal state as if a modulating bit stream consisting of consecutive ones (di = 1) had continued to enter the differential encoder. These bits are called dummy bits and define the start and the stop of the active and the useful part of the burst as illustrated in Fig.1. Nothing is specified about the actual phase of the modulator output signal outside the useful part of the burst.

.G.REC0504.EPS;15,487 cm;5 cm;EPS

Figure 1: Relation between active part of burst, tail bits and dummy bits. For the normal burst the useful part lasts for 147 modulating bits.

2.3	Differential encoding:

Each data value di = [0,1] is differentially encoded. The output of the differential encoder is:

^

di = di ┼ di-1	(di ╬ {0, 1})	(1)

where ┼ denotes modulo 2 addition.

The modulating data value ai input to the modulator is:

^

ai = 1 -	2di	(ai ╬ {-1, +1})	(2)

2.4	Filtering:

The modulating data values ai as represented by Dirac pulses excite a linear filter with impulse response defined by:

g(t) = h(t) * rect(t/T)	(3)

where the function rect(x) is defined by:

rect(t/T) = 1/T	for |t| < T/2	(4)

rect(t/T) = 0	otherwise

and * means convolution. h(t) is defined by:

__

h(t) = exp(-t2/(2s2T2))/((╓2p)sT)	(5)

_____

where s = ╓ln(2)/2pBT	and BT = 0.3	(6)

where B is the 3 dB bandwidth of the filter with impulse response h(t), and T is the duration of one input data bit. This theoretical filter is associated with tolerances defined in 05.05.

2.5	Output phase:

The phase of the modulated signal is:

t-iT

j(t) = S ai p h	≥ g(u)du	(7)

i	-Ñ

where the modulating index h is 1/2 (maximum phase change in radians is p/2 per data interval).

2.6	Modulation:

The modulated RF carrier, except for start and stop of the TDMA burst may therefore be expressed as:

______

x(t) = ╓(2Ec/T) cos(2pf0t + j(t) + j0)	(8)

where Ec is the energy per modulating bit, f0 is the centre frequency and j0 is a random phase and is constant during one burst.