GSM recommendation: 06.10

Title: GSM full rate speech transcoding

Date: January 1991

List of contents: 1. General

2. Transmission characteristics

3. Functional description of the RPE-LTP codec

4. Computational details of the RPE-LTP codec

5. Digital test sequences

Annex 1. Codec performance

Annex 2. Subjective relevance of the speech

coder output bits

Annex 3. Format for test sequence distribution

NOTE: This Recommendation is a reproduction of recommendation

T/L/03/11 "13 kbit/s Regular Pulse Excitation - Long Term

Prediction - Linear Predictive Coder for use in the

Pan-European Digital Mobile Radio System".

Floppy disks containing the digital test sequences described

in section 5 can be distributed by ETSI Secretariat on request.

ETSI

B.P. 152

F 06561 Valbonne Cedex

France

Tel	+33 92 94 42 00

Fax	+33 93 65 47 16

+33 93 65 28 17

Language of original: English

Number of pages: 93

***** This page is intentionally left blank ! *****

Detailed list of contents

*************************

1. GENERAL

1.1. SCOPE

1.2. OUTLINE DESCRIPTION

1.3. FUNCTIONAL DESCRIPTION OF AUDIO PARTS

1.4. PCM FORMAT CONVERSION

1.5. PRINCIPLES OF THE RPE-LTP ENCODER

1.6. PRINCIPLES OF THE RPE-LTP DECODER

1.7. SEQUENCE AND SUBJECTIVE IMPORTANCE OF ENCODED PARAMETERS

2. TRANSMISSION CHARACTERISTICS

2.1. PERFORMANCE CHARACTERISTICS OF THE ANALOGUE/DIGITAL

INTERFACES

2.2. TRANSCODER DELAY

3. FUNCTIONAL DESCRIPTION OF THE RPE-LTP CODEC

3.1. FUNCTIONAL DESCRIPTION OF THE RPE-LTP ENCODER

3.1.1. Offset compensation

3.1.2. Preemphasis

3.1.3. Segmentation

3.1.4. Autocorrelation

3.1.5. Schur Recursion

3.1.6. Transformation of reflection coefficients to Log.-Area

Ratios

3.1.7. Quantization and coding of Log.-Area Ratios

3.1.8. Decoding of the quantized Log.-Area Ratios

3.1.9. Interpolation of Log.-Area Ratios

3.1.10. Tranformation of Log.-Area Ratios into reflection

coefficients

3.1.11. Short term Analysis Filtering

3.1.12. Sub-segmentation

3.1.13. Calculation of the LTP parameters

3.1.14. Coding/Decoding of the LTP lags

3.1.15. Coding/Decoding of the LTP gains

3.1.16. Long term analysis filtering

3.1.17. Long term synthesis filtering

3.1.18. Weighting Filter

3.1.19. Adaptive sample rate decimation by RPE grid selection

3.1.20. APCM quantization of the selected RPE sequence

3.1.21. APCM inverse quantization

3.1.22. RPE grid positioning

3.2. DECODER

3.2.1. RPE decoding section

3.2.2. Long Term Prediction section

3.2.3. Short term synthesis filtering section

3.2.4. Postprocessing

4. COMPUTATIONAL DETAILS OF THE RPE-LTP CODEC

4.1. DATA REPRESENTATION AND ARITHMETIC OPERATIONS

4.2. FIXED POINT IMPLEMENTATION OF THE RPE-LTP CODER

4.2.0. Scaling of the input variable

4.2.1. Downscaling of the input signal

4.2.2. Offset compensation

4.2.3. Preemphasis

4.2.4. Autocorrelation

4.2.5. Computation of the reflection coefficients

4.2.6. Transformation of reflection coefficients to Log.-Area

Ratios

4.2.7. Quantization and coding of the Log.-Area Ratios

4.2.8. Decoding of the coded Log.-Area Ratios

4.2.9. Computation of the quantized reflection coefficients

4.2.9.1. Interpolation of the LARpp[1..8] to get the LARp[1..8]

4.2.9.2. Computation of the rp[1..8] from the interpolated

LARp[1..8]

4.2.10. Short term analysis filtering

4.2.11. Calculation of the LTP parameters

4.2.12. Long term analysis filtering

4.2.13. Weighting filter

4.2.14. RPE grid selection

4.2.15. APCM quantization of the selected RPE sequence

4.2.16. APCM inverse quantization

4.2.17. RPE grid positioning

4.2.18. Update of the reconstructed short term residual signal

dp[-120..-1]

4.3. FIXED POINT IMPLEMENTATION OF THE RPE-LTP DECODER

4.3.1. RPE decoding section

4.3.2. Long term synthesis filtering

4.3.3. Computation of the decoded reflection coefficients

4.3.4. Short term synthesis filtering section

4.3.5. Deemphasis filtering

4.3.6. Upscaling of the output signal

4.3.7. Truncation of the output variable

4.4. TABLES USED IN  THE FIXED POINT IMPLEMENTATION OF THE RPE-LTP

CODER AND DECODER

5. DIGITAL TEST SEQUENCES

5.1. INPUT AND OUTPUT SIGNALS

5.2. CONFIGURATION FOR THE APPLICATION OF THE TEST SEQUENCES

5.2.1. Configuration 1 (encoder only)

5.2.2. Configuration 2 (Decoder only)

5.3. TEST SEQUENCES

5.3.1. Test sequences for configuration 1

5.3.2. Test sequences for configuration 2

ANNEX 1. CODEC PERFORMANCE

A1.1. INTRODUCTION

A1.2. SPEECH PERFORMANCE

A1.2.1. Single encoding

A1.2.2. Speech performance when interconnected with coding systems

on an analogue basis

A1.2.2.1. Performance with 32 kbit/s ADPCM (G.721)

A1.2.2.2. Performance with another RPE-LTP codec

A1.2.2.3. Performance with encoding other than RPE-LTP and 32

kbit/s ADPCM (G.721)

A1.3. NON-SPEECH PERFORMANCE

A.1.3.1. Performance with single sine waves

A1.3.2. Performance with DTMF tones

A1.3.3. Performance with information tones

A1.3.4. Performance with voice-band data

A1.4. DELAY

A1.5. REFERENCES

ANNEX 2. SUBJECTIVE RELEVANCE OF THE SPEECH CODER OUTPUT BITS

ANNEX 3. FORMAT FOR TEST SEQUENCE DISTRIBUTION

A3.1. TYPE OF FILES PROVIDED

A3.2. FILE FORMAT DESCRIPTION

1. GENERAL

=======

1.1. SCOPE

-----

The transcoding procedure specified in this recommendation is

applicable for the full-rate traffic channel (TCH) in the Pan-

European Digital Mobile Radio (DMR) system. The use of this

transcoding scheme for other applications has not been considered.

In recommendation GSM 06.01, a reference configuration for the

speech transmission chain of the Pan-European DMR system is shown.

According to this reference configuration, the speech encoder

takes its input as a 13 bit uniform PCM signal either from the

audio part of the mobile station or on the network side, from the

PSTN via an 8 bit/A-law to 13 bit uniform PCM conversion. The

encoded speech at the output of the speech encoder is delivered to

a channel encoder unit which is specified in Rec.GSM 05.03. In the

receive direction, the inverse operations take place.

This recommendation describes the detailed mapping between input

blocks of 160 speech samples in 13 bit uniform PCM format to

encoded blocks of 260 bits and from encoded blocks of 260 bits to

output blocks of 160 reconstructed speech samples.  The sampling

rate is 8000 sample/s leading to an average bit rate for the

encoded bit stream of 13 kbit/s. The coding scheme is the

so-called Regular Pulse Excitation - Long Term prediction - Linear

Predictive Coder, here-after referred to as RPE-LTP.

The recommendation also specifies the conversion between A-law PCM

and 13 bit uniform PCM. Performance requirements for the audio

input and output parts are included only to the extent that they

affect the transcoder performance. The recommendation also

describes the codec down to the bit level, thus enabling the

verification of compliance to the recommendation to a high degree

of confidence by use of a set of digital test sequences. These

test sequences are also described and are available on floppy

disks.

1.2. OUTLINE DESCRIPTION

-------------------

The recommendation is structured as follows:

Section 1.3 contains a functional description of the audio parts

including the A/D and D/A functions. Section 1.4 describes the

conversion between 13 bit uniform and 8 bit A-law samples.

Sections 1.5 and 1.6 present a simplified description of the

principles of the RPE-LTP encoding and decoding process

respectively. In section 1.7, the sequence and subjective

importance of encoded parameters are given.

Section 2 deals with the transmission characteristics of the audio

parts that are relevant for the performance of the RPE-LTP codec.

Some transmission characteristics of the RPE-LTP codec are also

specified in section 2. Section 3 presents the functional descrip-

tion of the RPE-LTP coding and decoding procedures, whereas

section 4 describes the computational details of the algorithm.

Procedures for the verification of the correct functioning of the

RPE-LTP are described in section 5.

Performance and network aspects of the RPE-LTP codec are contained

in annex 1.

1.3. FUNCTIONAL DESCRIPTION OF AUDIO PARTS

-------------------------------------

The analogue-to-digital and digital-to-analogue conversion will in

principle comprise the following elements:

1) Analogue to uniform digital

- microphone,

- input level adjustment device,

- input anti-aliasing filter,

- sample-hold device sampling at 8 kHz,

- analogue-to-uniform digital conversion to 13 bits

representation.

The uniform format shall be represented in two's complement.

2) Uniform digital to analogue

- conversion from 13 bit /8kHz uniform PCM to analogue,

- a hold device,

- reconstruction filter including x/sin x correction,

- output level adjustment device,

- earphone or loudspeaker.

In the terminal equipment, the A/D function may be achieved either

- by direct conversion to 13 bit uniform PCM format.

- or by conversion to 8 bit/A-law companded format, based on a

standard A-law codec/filter according to CCITT rec.

G.711/714, followed by the 8-bit to 13-bit conversion

according to the procedure specified in section 1.4.

For the D/A operation, the inverse operations take place.

In the latter case it should be noted that the specifications in

CCITT recommendation G.714 are concerned with PCM equipment

located in the central parts of the network. When used in the

terminal equipment, this specification does not on its own ensure

sufficient out-of-band attenuation.

The specification of out-of-band signals is defined in section 2

between the acoustic signal and the digital interface to take into

account that the filtering in the terminal can be achieved both by

electronic and acoustical design.

1.4. PCM FORMAT CONVERSION

---------------------

The conversion between 8 bit A-law companded format and the 13-bit

uniform format shall be as defined in CCITT Recommendation G.721,

section 4.2.1, sub-block EXPAND and section 4.2.7, sub-block

COMPRESS. The parameter LAW = 1 should be used.

1.5. PRINCIPLES OF THE RPE-LTP ENCODER

---------------------------------

A simplified block diagram of the RPE-LTP encoder is shown in Fig

1.1. In this diagram the coding and quantization functions are not

shown explicitly.

The input speech frame, consisting of 160 signal samples (uniform

13 bit PCM samples), is first pre-processed to produce an

offset-free signal, which is then subjected to a first order

pre-emphasis filter. The 160 samples obtained are then analyzed to

determine the coefficients for the short term analysis filter (LPC

analysis).  These parameters are then used for the filtering of

the same 160 samples. The result is 160 samples of the short term

residual signal. The filter parameters, termed reflection

coefficients, are transformed to log.area ratios, LARs, before

transmission.

For the following operations, the speech frame is divided into 4

sub-frames with 40 samples of the short term residual signal in

each. Each sub-frame is processed blockwise by the subsequent

functional elements.

Before the processing of each sub-block of 40 short term residual

samples, the parame-ters of the long term analysis filter, the LTP

lag and the LTP gain, are estimated and updated in the LTP

analysis block, on the basis of the current sub-block of the

present and a stored sequence of the 120 previous reconstructed

short term residual samples.

A block of 40 long term residual signal samples is obtained by

subtracting 40 estimates of the short term residual signal from

the short term residual signal itself. The resulting block of 40

long term residual samples is fed to the Regular Pulse Excitation

analysis which performs the basic compression function of the

algorithm.

As a result of the RPE-analysis, the block of 40 input long term

residual samples are represented by one of 4 candidate

sub-sequences of 13 pulses each. The subsequence selected is

identified by the RPE grid position (M). The 13 RPE pulses are

encoded using Adaptive Pulse Code Modulation (APCM) with

estimation of the sub-block amplitude which is transmitted to the

decoder as side information.

The RPE parameters are also fed to a local RPE decoding and recon-

struction module which produces a block of 40 samples of the quan-

tized version of the long term residual signal.

By adding these 40 quantized samples of the long term residual to

the previous block of short term residual signal estimates, a

reconstructed version of the current short term residual signal is

obtained.

The block of reconstructed short term residual signal samples is

then fed to the long term analysis filter which produces the new

block of 40 short term residual signal estimates to be used for

the next sub-block thereby completing the feedback loop.

1.6. PRINCIPLES OF THE RPE-LTP DECODER

---------------------------------

The simplified block diagram of the RPE-LTP decoder is shown in

fig 1.2. The decoder includes the same structure as the feed-back

loop of the encoder. In error-free transmission, the output of

this stage will be the reconstructed short term residual samples.

These samples are then applied to the short term synthesis filter

followed by the de-emphasis filter resulting in the reconstructed

speech signal samples.

1.7. SEQUENCE AND SUBJECTIVE IMPORTANCE OF ENCODED PARAMETERS

--------------------------------------------------------

As indicated in fig 1.1 the three different groups of data are

produced by the encoder are:

- the short term filter parameters,

- the Long Term Prediction (LTP) parameters

- the RPE parameters

The encoder will produce this information in a unique sequence and

format, and the decoder must receive the same information in the

same way. In table 1.1, the sequence of output bits b1 to b260 and

the bit allocation for each parameter is shown.

The different parameters of the encoded speech and their

individual bits have unequal importance with respect to subjective

quality. Before being submitted to the channel encoder, the bits

have to be rearranged in the sequence importance as given in table

1.2. The ranking has been determined by subjective testing and the

procedure used is described in annex 2.

==================================================================

Parameter  Parameter   Parameter          Var.  Number    Bit no.

number      name               name  of bits  (LSB-MSB)

==================================================================

==================================================================

1                        LAR 1     6     b1  - b6

2                        LAR 2     6     b7  - b12

FILTER         3      Log. Area         LAR 3     5     b13 - b17

4      ratios            LAR 4     5     b18 - b22

PARAMETERS     5      1 - 8             LAR 5     4     b23 - b26

6                        LAR 6     4     b27 - b30

7                        LAR 7     3     b31 - b33

8                        LAR 8     3     b34 - b36

==================================================================

Sub-frame no.1

==================================================================

LTP            9      LTP lag             N1      7     b37 - b43

PARAMETERS    10      LTP gain            b1      2     b44 - b45

------------------------------------------------------------------

11      RPE grid position   M1      2     b46 - b47

RPE           12      Block amplitude   Xmax1     6     b48 - b53

PARAMETERS    13      RPE-pulse no.1    x1(0)     3     b54 - b56

14      RPE-pulse no.2    x1(1)     3     b57 - b59

..      ...                                  ...

25      RPE-pulse no.13   x1(12)    3     b90 - b92

==================================================================

Sub-frame no.2

==================================================================

LTP           26      LTP lag             N2      7     b93 - b99

PARAMETERS    27      LTP gain            b2      2     b100- b101

------------------------------------------------------------------

28      RPE grid position   M2      2     b102- b103

RPE           29      Block amplitude   Xmax2     6     b104- b109

PARAMETERS    30      RPE-pulse no.1    x2(0)     3     b110- b112

31      RPE-pulse no.2    x2(1)     3     b113- b115

..      ...                                  ...

42      RPE-pulse no.13   x2(12)    3     b146- b148

==================================================================

Table 1.1a. Encoder output parameters in order of occurrence and

bit allocation within the speech frame of 260 bits/20

ms

--

Sub-frame no.3

==================================================================

LTP           43      LTP lag              N3     7     b149- b155

PARAMETERS    44      LTP gain             b3     2     b156- b157

------------------------------------------------------------------

45      RPE grid position    M3     2     b158- b159

RPE           46      Block amplitude    Xmax3    6     b160- b165

PARAMETERS    47      RPE-pulse no.1     x3(0)    3     b166- b168

48      RPE-pulse no.2     x3(1)    3     b169- b171

..      ...                                  ...

59      RPE-pulse no.13    x3(12)   3     b202- b204

==================================================================

Sub-frame no.4

==================================================================

LTP           60      LTP lag              N4     7     b205- b211

PARAMETERS    61      LTP gain             b4     2     b212- b213

------------------------------------------------------------------

62      RPE grid position    M4     2     b214- b215

RPE           63      Block amplitude    Xmax4    6     b216- b221

PARAMETERS    64      RPE-pulse no.1     x4(0)    3     b222- b224

65      RPE-pulse no.2     x4(1)    3     b225- b227

..      ...                                  ...

76      RPE-pulse no.13    x4(12)   3     b258- b260

==================================================================

Table 1.1b. Encoder output parameters in order of occurrence and

bit allocation within the speech frame of 260 bits/20

ms

--

==================================================================

Order of bit    Parameter         Parameter        Bit number

importance        name              number

==================================================================

Class Ia

==================================================================

1      Log.area ratio 1           1                b6

2,3,4,5   Block amplitude       12,29,46,63   b53,b109,b165,b221

6      Log.area ratio 1           1                b5

7      Log.area ratio 2           2                b12

8      Log.area ratio 3           3                b17

.      Log.area ratio 1           1                b4

.      Log.area ratio 2           2                b11

Log.area ratio 3           3                b16

Log.area ratio 4           4                b22

LTP lag                9,26,43,60   b43,b99,b155,b211

Block amplitude        12,29,46,63  b52,b108,b164,b220

Log.area ratio 2,5,6     2,5,6          b10,b26,b30

LTP lag                9,26,43,60   b42,b98,b154,b210

LTP lag                9,26,43,60   b41,b97,b153,b209

LTP lag                9,26,43,60   b40,b96,b152,b208

LTP lag                9,26,43,60   b39,b95,b151,b207

Block amplitude        12,29,46,63  b51,b107,b163,b219

Log.area ratio 1           1                b3

.      Log.area ratio 4           4                b21

.      Log.area ratio 7           7                b33

..49,50    LTP lag                9,26,43,60   b38,b94,b150,b206

==================================================================

Class Ib

==================================================================

51,52    Log.area ratio 5,6        5,6            b25,b29

.      LTP gain               10,27,44,61  b45,b101,b157,b213

.      LTP lag                9,26,43,60   b37,b93,b149,b205

Grid position          11,28,45,62  b47,b103,b159,b215

Log.area ratio 1           1                b2

Log.area ratio 2,3,8,4  2,3,8,4       b9,b15,b36,b20

Log.area ratio 5,7        5,7            b24,b32

LTP gain              10,27,44,61   b44,b100,b156,b212

Block amplitude       12,29,46,63   b50,b106,b162,b218

RPE pulses              13..25       b56,b59,..,b92

RPE pulses              30..42      b112,b115,..,b148

RPE pulses              47..59      b168,b171,..,b204

RPE pulses              64..76      b224,b227,..,b260

Grid position         11,28,45,62   b46,b102,b158,b214

Block amplitude       12,29,46,63   b49,b105,b161,b217

RPE pulses              13..25       b55,b58,..,b91

.      RPE pulses              30..42      b111,b114,..,b147

.      RPE pulses              47..59      b167,b170,..,b203

...182   RPE pulses              64..67     b223,b226,b229,b232

==================================================================

Table 1.2a. Subjective importance of encoded bits (the parameter

and bit numbers refer to table 1.1)

-----------------------------------

==================================================================

Class II

==================================================================

183...   RPE pulses              68..76      b235,b238,..,b259

Log.area ratio 1           1                b1

Log.area ratio 2,3,6     2,3,6          b8,b14,b28

Log.area ratio 7           7                b31

Log.area ratio 8           8                b35

Log.area ratio 8,3        8,3             b34,b13

Log.area ratio 4           4                b19

Log.area ratio 4,5        4,5             b18,b23

Block amplitude       12,29,46,63   b48,b104,b160,b216

RPE pulses              13..25        b54,b57,..,b90

RPE pulses              30..42      b110,b113,..,b146

RPE pulses              47..59      b166,b169,..,b202

RPE pulses              64..76      b222,b225,..,b258

259,260   Log.area ratio 2,6        2,6             b7,b27

==================================================================

Table 1.2b. Subjective importance of encoded bits (the parameter

and bit numbers refer to table 1.1)

-----------------------------------

NOTE: The subdivisions in table 1.2 indicate the border between protection classes Ia, Ib and II as defined in recommendation GSM 05.03.

2. TRANSMISSION CHARACTERISTICS

============================

This section specifies the necessary performance characteristics

of the audio parts for proper functioning of the speech trancoder.

Some transmission performance characteristics of the RPE-LTP

transcoder are also given to assist the designer of the speech

transcoder function. The information given here is redundant and

the detailed specifications are contained in recommendation GSM

11.10.

The performance characteristics are referred to the 13 bit uniform

PCM interface.

NOTE: To simplify the verification of the specifications, the

performance limits may be referred to an A-law measurement

interface according to CCITT Rec-ommendation G.711.  In this

way, standard measuring equipments for PCM systems can be

utilized for measurements.  The relationship between the 13

bit format and the A-law companded shall follow the

procedures defined in section 1.4.

2.1. PERFORMANCE CHARACTERISTICS OF THE ANALOGUE/DIGITAL

INTERFACES

----------

Concerning 1) discrimination against out-of-band signals (sending)

and 2) spurious out-of-band signals (receiving), the same

requirements as defined in ETSI standard TE 04-15 (digital

telephone, candidate NET33) apply.

2.2. TRANSCODER DELAY

----------------

Consider a back to back configuration where the parameters

generated by the encoder are delivered to the speech decoder as

soon as they are available.

The transcoder delay is defined as the time interval between the

instant a speech frame of 160 samples has been received at the

encoder input and the instant the corresponding 160 reconstructed

speech samples have been out-put by the speech decoder at an 8 kHz

sample rate.

The theoretical minimum delay which can be achieved is 20 ms.  The

requirement is that the transcoder delay should be less than 30

ms.

3. FUNCTIONAL DESCRIPTION OF THE RPE-LTP CODEC

===========================================

The block diagram of the RPE-LTP-coder is shown in fig 3.1. The

individual blocks are described in the following sections.

3.1. FUNCTIONAL DESCRIPTION OF THE RPE-LTP ENCODER

---------------------------------------------

The Preprocessing section of the RPE-LTP encoder comprises the

following two sub-blocks:

* Offset compensation (3.1.1)

* Preemphasis (3.1.2)

The LPC analysis section of the RPE-LTP encoder comprises the

following five sub-blocks:

* Segmentation (3.1.3)

* Auto-Correlation (3.1.4)

* Schur Recursion (3.1.5)

* Transformation of reflection coefficients to Log.-Area Ratios

(3.1.6)

* Quantization and coding of Log.-Area Ratios (3.1.7)

The Short term analysis filtering section of the RPE-LTP comprises

the following four sub-blocks:

* Decoding of the quantized Log.-Area Ratios (LARs) (3.1.8)

* Interpolation of Log.-Area Ratios (3.1.9)

* Transformation of Log.-Area Ratios into reflection coefficients

(3.1.10)

* Short term analysis filtering (3.1.11)

The Long Term Predictor (LTP) section comprises 4 sub-blocks

working on subsegments (3.1.12) of the short term residual

samples.

* Calculation of LTP parameters (3.1.13)

* Coding of the LTP lags (3.1.14) and the LTP gains (3.1.15)

* Decoding of the LTP lags (3.1.14) and the LTP gains (3.1.15)

* Long term analysis filtering (3.1.16), and

Long term synthesis filtering (3.1.17)

The RPE encoding section comprises five different sub-blocks:

* Weighting filter (3.1.18)

* Adaptive sample rate decimation by RPE grid selection (3.1.19)

* APCM quantization of the selected RPE sequence (3.1.20)

* APCM inverse quantization (3.1.21)

* RPE grid positioning (3.1.22)

PREPROCESSING SECTION

3.1.1. Offset compensation

-------------------

Prior to the speech encoder an offset compensation,by a notch

filter is applied in order to remove the offset of the input

signal so to produce the offset-free signal sof.

sof(k) = so(k) - so(k-1) + alpha*sof(k-1)         (3.1.1)

alpha = 32735*2-15

3.1.2. Preemphasis

-----------

The signal sof is applied to a first order FIR preemphasis

filter leading to the input signal s of the analysis section.

s(k) = sof(k) - beta*sof(k-1)                     (3.1.2)

beta= 28180*2-15

LPC ANALYSIS SECTION

3.1.3. Segmentation

------------

The speech signal s(k) is divided into non-overlapping frames

having a length of T0 = 20 ms (160 samples). A new LPC-analysis

of order p=8 is performed for each frame.

3.1.4. Autocorrelation

---------------

The first p+1 = 9 values of the Auto-Correlation function are

calculated by

159

ACF(k)=    >   s(i)s(i-k)     ,k = 0,1...,8          (3.2)

i=k

3.1.5. Schur Recursion

---------------

The reflection coefficients are calculated as shown in Fig 3.2

using the Schur Recursion algorithm. The term "reflection

coefficient" comes from the theory of linear prediction of speech

(LPC), where a vocal tract representation consisting of series of

uniform cylindrical sections is assumed. Such a representation can

be described by the reflection coefficents or the area ratios of

connected sections.

3.1.6. Transformation of reflection coefficients to Log.-Area

Ratios

------

The reflection coefficients r(i), (i=1..8), calculated by the

Schur algorithm, are in the range

-1 <= r(i) <= + 1

Due to the favourable quantization characteristics, the reflection

coefficients are converted into Log.-Area Ratios which are

strictly defined as follows:

1 + r(i)

Logarea(i) = log10  (----------)                   (3.3)

1 - r(i)

Since it is the companding characteristic of this transformation

that is of importance, the following segmented approximation is

used.

r(i)                       ;          |r(i)| < 0.675

LAR(i) = sign[r(i)]*[2|r(i)|-0.675] ; 0.675 <= |r(i)| < 0.950

sign[r(i)]*[8|r(i)|-6.375] ; 0.950 <= |r(i)| <= 1.000

(3.4)

with the result that instead of having to divide and obtain the

logarithm of particular values, it is merely necessary to

multiply, add and compare these values.

The following equation (3.5) gives the inverse transformation.

LAR'(i)                           ; |LAR'(i)|<0.675

r'(i)=sign[LAR'(i)]*[0.500*|LAR'(i)|

+0.337500]                        ; 0.675<=|LAR'(i)|<1.225

sign[LAR'(i)]*[0.125*|LAR'(i)|

+0.796875]                        ; 1.225<=|LAR'(i)|<=1.625

(3.5)

3.1.7. Quantization and coding of Log.-Area Ratios

-------------------------------------------

The Log.-Area Ratios LAR(i) have different dynamic ranges and

different asymmetric distribution densities. For this reason, the

transformed coefficients LAR(i) are limited and quantized

differently according to the following equation (3.6), with LARc(i) denoting the quantized and integer coded version of LAR(i).

LARc(i) = Nint{A(i)*LAR(i) + B(i)}                 (3.6)

with

Nint{z} := int{z+sign{z}*0.5}                      (3.6a)

Function Nint defines the rounding to the nearest integer value,

with the coefficients A(i), B(i), and different extreme values of

LARc(i) for each coefficient LAR(i) given in table 3.1.

-------------------------------------------------

|LAR No i |  A(i)  |  B(i)  | Minimum | Maximum |

|         |        |        | LARc(i) | LARc(i) |

|---------|--------|--------|---------|---------|

|    1    | 20.000 |  0.000 |   -32   |   +31   |

|    2    | 20.000 |  0.000 |   -32   |   +31   |

|    3    | 20.000 |  4.000 |   -16   |   +15   |

|    4    | 20.000 | -5.000 |   -16   |   +15   |

|    5    | 13.637 |  0.184 |   - 8   |   + 7   |

|    6    | 15.000 | -3.500 |   - 8   |   + 7   |

|    7    |  8.334 | -0.666 |   - 4   |   + 3   |

|    8    |  8.824 | -2.235 |   - 4   |   + 3   |

-------------------------------------------------

Table 3.1. Quantization of the Log.-Area Ratios LAR(i)

-------------------------------------------

SHORT-TERM ANALYSIS FILTERING SECTION

The current frame of the speech signal s is retained in memory

until calculation of the LPC parameters LAR(i) is completed. The

frame is then read out and fed to the short term analysis filter

of order p=8. However, prior to the analysis filtering operation,

the filter coefficients are decoded and preprocessed by

interpolation.

3.1.8. Decoding of the quantized Log.-Area Ratios

------------------------------------------

In this block the quantized and coded Log.-Area Ratios (LARc(i)) are decoded according to equation (3.7).

LAR''(i) = ( LARc(i) - B(i) )/ A(i)                (3.7)

3.1.9. Interpolation of Log.-Area Ratios

---------------------------------

To avoid spurious transients which may occur if the filter

coefficients are changed abruptly, two subsequent sets of

Log.-Area Ratios are interpolated linearly. Within each frame of

160 analysed speech samples the short term analysis filter and the

short term synthesis filter operate with four different sets of

coefficients derived according to table 3.2.

-----------------------------------------------

|    k    |      LAR'J(i) =                   |

|---------|-----------------------------------|

|  0...12 | 0.75*LAR''J-1(i) + 0.25*LAR''J(i) |

| 13...26 | 0.50*LAR''J-1(i) + 0.50*LAR''J(i) |

| 27...39 | 0.25*LAR''J-1(i) + 0.75*LAR''J(i) |

| 40..159 |                         LAR''J(i) |

-----------------------------------------------

Table 3.2. Interpolation of LAR parameters (J=actual segment)

--------------------------------------------------

3.1.10. Transformation of Log.-Area Ratios into reflection

coefficients

------------

The reflection coefficients are finally determined using the

inverse transformation according to equation (3.5).

3.1.11. Short Term Analysis Filtering

-----------------------------

The Short term analysis filter is implemented according to the

lattice structure depicted in fig 3.3.

d0(k) = s(k)                                          (3.8a)

u0(k) = s(k)                                          (3.8b)

di(k) = di-1(k)  + r'i*ui-1(k-1)  with i=1,...8       (3.8c)

ui(k) = ui-1(k-1) + r'i*di-1(k)   with i=1,...8       (3.8d)

d(k ) = d8(k)                                         (3.8e)

LONG-TERM PREDICTOR (LTP) SECTION

3.1.12. Sub-segmentation

----------------

Each input frame of the short term residual signal contains 160

samples, corresponding to 20 ms. The long term correlation is

evaluated four times per frame, for each 5 ms subsegment. For

convenience in the following, we note j=0,...,3 the sub-segment

number, so that the samples pertaining to the j-th sub-segment of

the residual signal are now denoted by d(kj+k) with j = 0,...,3; kj = k0 + j*40 and k = 0,...,39 where k0 corresponds to the first value of the current frame.

3.1.13. Calculation of the LTP parameters

---------------------------------

For each of the four sub-segments a long term correlation lag Nj, (j=0,...,3), and an associated gain factor bj, (j=0,...,3)

are determined. For each sub-segment, the determination of these

parameters is implemented in three steps.

1) The first step is the evaluation of the cross-correlation

Rj(lambda) of the current sub-segment of short term residual

signal d(kj+i),(i=0,...,39) and the previous samples of the

reconstructed short term residual signal d'(kj+i),

(i=-120,...,-1):

39                          j  = 0,...3

Rj(lambda) =  > d(kj+i)*d'(kj+i-lambda); kj = k0 + j*40

i=0                         lambda = 40,...,120

(3.9)

The cross-correlation is evaluated for lags lambda greater than

or equal to 40 and less than or equal to 120, ie corresponding

to samples outside the current sub-segment and not delayed by

more than two sub-segments.

2) The second step is to find the position Nj of the peak of

the cross-correlation function within this interval:

Rj(Nj) = max { Rj(lambda); lambda = 40..120 };

j = 0,...,3

(3.10)

3) The third step is the evaluation of the gain factor bj

according to:

bj =    Rj(Nj) / Sj(Nj);       j = 0,...,3      (3.11)

with

39

Sj(Nj) =  > d'2 (kj+i-Nj);  j = 0,...,3        (3.12)

i=0

It is clear that the last 120 samples of the reconstructed

short term residual signal d'(kj+i),(i=-120,...,-1) must be

retained until the next sub-segment so as to allow the

evaluation of the relations (3.9),...,(3.12).

3.1.14. Coding/Decoding of the LTP lags

-------------------------------

The long term correlation lags Nj,(j=0,...,3) can have values

in the range (40,...,120), and so must be coded using 7 bits with:

Ncj = Nj;                        j = 0,...,3     (3.13)

At the receiving end, assuming an error free transmission, the

decoding of these values will restore the actual lags:

Nj' = Ncj;                       j = 0,...,3     (3.14)

3.1.15. Coding/Decoding of the LTP gains

--------------------------------

The long term prediction gains bj,(j=0,...,3) are encoded with

2 bits each, according to the following algorithm:

if            bj <= DLB(i)  then  bcj  = 0;  i=0

if DLB(i-1) < bj <= DLB(i)  then  bcj  = i;  i=1,2      (3.15)

if DLB(i-1) < bj            then  bcj  = 3;  i=3

where DLB(i),(i=0,...,2) denotes the decision levels of the

quantizer, and bcj represents the coded gain value. Decision

levels and quantizing levels are given in table 3.3.

-----------------------------------------

|   | Decision level | Quantizing level |

| i |     DLB(i)     |      QLB(i)      |

|---|----------------|------------------|

| 0 |      0.2       |       0.10       |

| 1 |      0.5       |       0.35       |

| 2 |      0.8       |       0.65       |

| 3 |                |       1.00       |

-----------------------------------------

Table 3.3. Quantization table for the LTP gain

-----------------------------------

The decoding rule is implemented according to:

bj' =  QLB(bcj) ;  j = 0,...,3         (3.16)

where QLB(i),(i=0,...,3) denotes the quantizing levels, and bj'

represents the decoded gain value (see table 3.3).

3.1.16. Long term analysis filtering

----------------------------

The short term residual signal d(k0+k),(k=0,...,159) is

processed by sub-segments of 40 samples. From each of the four

sub-segments (j=0,...,3) of short term residual samples, denoted

here d(kj+k), (k=0,...,39), an estimate d"(kj+k),

(k=0,...,39) of the signal is subtracted to give the long term

residual signal e(kj+k), (k=0,...,39) (see fig 3.1):

j  = 0,...,3

e(kj+k) = d(kj+k) - d"(kj+k)   ;   k  = 0,...,39       (3.17)

kj = k0 + j*40

Prior to this subtraction, the estimated samples d"(kj+k) are

computed from the previously reconstructed short term residual

samples d', adjusted to the current sub-segment LTP lag Nj' and

weighted with the sub-segment LTP gain bj':

j = 0,...,3

d"(kj+k) = bj'*d'(kj+k-Nj')  ;  k = 0,...,39           (3.18)

kj = k0 + j*40

3.1.17. Long term synthesis filtering

-----------------------------

The reconstructed long term residual signal e'(k0+k),(k=0,...,159) is processed by sub-segments of 40 samples. To each sub-segment, denoted here e'(kj+k), (k=0,...,39), the estimate d"(kj+k), (k=0,...,39) of the signal is added to give the reconstructed short term residual signal d'(kj+k),(k=0,...,39):

j  = 0,...,3

d'(kj+k) = e'(kj+k) + d"(kj+k)   ; k = 0,...,39       (3.19)

kj = k0 + j*40

RPE ENCODING SECTION

3.1.18. Weighting Filter

----------------

A FIR 'block filter' algorithm is applied to each sub-segment by

convolving 40 samples e(k) with the impulse response H(i) ;

i=0,...,10 (see table 3.4).

------------------------------------------------------------------

|  i     |    5   |  4 (6) |  3 (7) |  2 (8) |   1 (9) |  0 (10) |

|--------|--------|--------|--------|--------|---------|---------|

|H(i)*213|  8192  |  5741  |  2054  |     0  |   -374  |   -134  |

------------------------------------------------------------------

|H(Omega=0)| = 2.779;

Table 3.4. Impulse response of block filter (weighting filter)

---------------------------------------------------

The conventional convolution of a sequence having 40 samples with

an 11-tap impulse response would produce 40+11-1=50 samples. In

contrast to this, the 'block filter' algorithm produces the 40

central samples of the conventional convolution operation. For

notational convenience the block filtered version of each

sub-segment is denoted by x(k), k=0,...,39.

10

x(k) = > H(i) * e(k+5-i)      with k = 0,...,39       (3.20)

i=0

NOTE: e(k+5-i) = 0  for k+5-i<0 and k+5-i>39.

3.1.19. Adaptive sample rate decimation by RPE grid selection

-----------------------------------------------------

For the next step, the filtered signal x is down-sampled by a

ratio of 3 resulting in 3 interleaved sequences of lengths 14, 13

and 13, which are split up again into 4 sub-sequences xm of

length 13:

xm(i) = x(kj+m+3*i)           ;  i = 0,...,12          (3.21)

m = 0,...,3

with m denoting the position of the decimation grid. According to

the explicit solution of the RPE mean squared error criterion, the

optimum candidate sub-sequence xM is selected which is the one

with the maximum energy

12

EM = max  >  xm2(i)       ;  m = 0,...,3              (3.22)

m   i=0

The optimum grid position M is coded as Mc with 2 bits.

3.1.20. APCM quantization of the selected RPE sequence

----------------------------------------------

The selected sub-sequence xM(i) (RPE sequence) is quantized,

applying APCM (Adaptive Pulse Code Modulation). For each RPE

sequence consisting of a set of 13 samples xM(i) ,the maximum

xmax of the absolute values |xM(i)| is selected and quantized

logarithmically with 6 bits as xmaxc as given in table 3.5.

--------------------------------------------------------------

|     xmax     | x'max |xmaxc||                |       |     |

|              |       |     ||       xmax     | x'max |xmaxc|

|--------------|-------|-----||----------------|-------|-----|

|    0 ..   31 |    31 |  0  ||  2048 ..  2303 |  2303 | 32  |

|   32 ..   63 |    63 |  1  ||  2304 ..  2559 |  2559 | 33  |

|   64 ..   95 |    95 |  2  ||  2560 ..  2815 |  2815 | 34  |

|   96 ..  127 |   127 |  3  ||  2816 ..  3071 |  3071 | 35  |

|  128 ..  159 |   159 |  4  ||  3072 ..  3327 |  3327 | 36  |

|  160 ..  191 |   191 |  5  ||  3328 ..  3583 |  3583 | 37  |

|  192 ..  223 |   223 |  6  ||  3584 ..  3839 |  3839 | 38  |

|  224 ..  255 |   255 |  7  ||  3840 ..  4095 |  4095 | 39  |

|  256 ..  287 |   287 |  8  ||  4096 ..  4607 |  4607 | 40  |

|  288 ..  319 |   319 |  9  ||  4608 ..  5119 |  5119 | 41  |

|  320 ..  351 |   351 | 10  ||  5120 ..  5631 |  5631 | 42  |

|  352 ..  383 |   383 | 11  ||  5632 ..  6143 |  6143 | 43  |

|  384 ..  415 |   415 | 12  ||  6144 ..  6655 |  6655 | 44  |

|  416 ..  447 |   447 | 13  ||  6656 ..  7167 |  7167 | 45  |

|  448 ..  479 |   479 | 14  ||  7168 ..  7679 |  7679 | 46  |

|  480 ..  511 |   511 | 15  ||  7680 ..  8191 |  8191 | 47  |

|  512 ..  575 |   575 | 16  ||  8192 ..  9215 |  9215 | 48  |

|  576 ..  639 |   639 | 17  ||  9216 .. 10239 | 10239 | 49  |

|  640 ..  703 |   703 | 18  || 10240 .. 11263 | 11263 | 50  |

|  704 ..  767 |   767 | 19  || 11264 .. 12287 | 12287 | 51  |

|  768 ..  831 |   831 | 20  || 12288 .. 13311 | 13311 | 52  |

|  832 ..  895 |   895 | 21  || 13312 .. 14335 | 14335 | 53  |

|  896 ..  959 |   959 | 22  || 14336 .. 15359 | 15359 | 54  |

|  960 .. 1023 |  1023 | 23  || 15360 .. 16383 | 16383 | 55  |

| 1024 .. 1151 |  1151 | 24  || 16384 .. 18431 | 18431 | 56  |

| 1152 .. 1279 |  1279 | 25  || 18432 .. 20479 | 20479 | 57  |

| 1280 .. 1407 |  1407 | 26  || 20480 .. 22527 | 22527 | 58  |

| 1408 .. 1535 |  1535 | 27  || 22528 .. 24575 | 24575 | 59  |

| 1536 .. 1663 |  1663 | 28  || 24576 .. 26623 | 26623 | 60  |

| 1664 .. 1791 |  1791 | 29  || 26624 .. 28671 | 28671 | 61  |

| 1792 .. 1919 |  1919 | 30  || 28672 .. 30719 | 30719 | 62  |

| 1920 .. 2047 |  2047 | 31  || 30720 .. 32767 | 32767 | 63  |

--------------------------------------------------------------

Table 3.5. Quantization of the block maximum xmax

--------------------------------------

For the normalization, the 13 samples are divided by the decoded

version x'max of the block maximum. Finally, the normalized

samples

x'(i) = xM(i)/x'max ;    i=0,...,12                (3.23)

are quantized uniformly with three bits to xMc(i) as given in

table 3.6.

-------------------------------------------

|        x'*215     |  xM'*215  |   xMc   |

| (Interval-limits) |           |(Channel)|

|-------------------|-----------|---------|

| -32768 ... -24577 |    -28672 | 0 = 000 |

| -24576 ... -16385 |    -20480 | 1 = 001 |

| -16384 ...  -8193 |    -12288 | 2 = 010 |

|  -8192 ...     -1 |     -4096 | 3 = 011 |

|                   |           |         |

|      0 ...   8191 |      4096 | 4 = 100 |

|   8192 ...  16383 |     12288 | 5 = 101 |

|  16384 ...  24575 |     20480 | 6 = 110 |

|  24576 ...  32767 |     28672 | 7 = 111 |

-------------------------------------------

Table 3.6. Quantization of the normalized RPE-samples

------------------------------------------

3.1.21. APCM inverse quantization

-------------------------

The xMc(i) are decoded to xM'(i) and denormalized using the

decoded value x'maxc leading to the decoded sub-sequence

x'M(i).

3.1.22. RPE grid positioning

--------------------

The quantized sub-sequence is upsampled by a ratio of 3 by

inserting zero values according to the grid position given with

Mc.

3.2. DECODER

-------

The decoder comprises the following 4 sections. Most of the

sub-blocks are also needed in the encoder and have been described

already. Only the short term tynthesis filter and the deemphasis

filter are added in the decoder as new sub-blocks.

* RPE decoding section (3.2.1)

* Long Term Prediction section (3.2.2)

* Short term synthesis filtering section (3.2.3)

* Postprocessing (3.2.4)

The complete block diagram for the decoder is shown in fig 3.4.

The variables and parameters of the decoder are marked by the

index r to distinguish the received values from the encoder

values.

3.2.1. RPE decoding section

--------------------

The input signal of the long term synthesis filter (reconstruction

of the long term residual signal) is formed by decoding and

denormalizing the RPE-samples (APCM inverse quantization - 3.1.21)

and by placing them in the correct time position (RPE grid

positioning - 3.1.22). At this stage, the sampling frequency is

increased by a factor of 3 by inserting the appropriate number of

intermediate zero-valued samples.

3.2.2. Long Term Prediction section

----------------------------

The the reconstructed long term residual signal er' is applied

to the long term synthesis filter (see 3.1.16 and 3.1.17) which

produces the reconstructed short term residual signal dr' for

the short term synthesiser.

3.2.3. Short term synthesis filtering section

--------------------------------------

The coefficients of the short term synthesis filter (see fig 3.5)

are reconstructed applying the identical procedure to that in the

encoder (3.1.8 - 3.1.10). The short term synthesis filter is

implemented according to the lattice structure depicted in fig

3.5.

sr(0)(k) = dr'(k)                                     (3.24a)

sr(i)(k) = sr(i-1)(k) - rr'(9-i) * v8-i(k-1);  i=1,...,8

(3.24b)

v9-i(k)  = v8-i(k-1) + rr'(9-i) * sr(i)(k);    i=1,...,8

(3.24c)

sr'(k)   = sr(8)(k)                                   (3.24d)

v0(k)    = sr(8)(k)                                   (3.24e)

3.2.4. Postprocessing

--------------

The output of the synthesis filter sr(k) is fed into the IIR-

deemphasis filter leading to the output signal sro.

sro(k) = sr(k) + beta*sro(k-1) ;  beta= 28180*2-15    (3.25)

4. COMPUTATIONAL DETAILS OF THE RPE-LTP CODEC

==========================================

4.1. DATA REPRESENTATION AND ARITHMETIC OPERATIONS

---------------------------------------------

Only two types of variables are used along the implementation of

the RPE-LTP algorithm in fixed point arithmetic. These two types

are:

Integer on 16 bits;

Long integer on 32 bits;

This assumption simplifies the detailed description and allows the

maximum reach of precision.

In different places of the recommendation, different scaling

factors are used according to different operations. To help the

reader in the comparison of corresponding floating point and fixed

point values given in section 3 and 4 comments of the format:

/* var  = integer(  real_var * scalefactor ) */

are used at several points of section 4. var is the rounded fixed

point representation of the floating point representation of var

(real_var) using the given scaling factor.

In the description, input signal samples, coded parameters and

output signal samples are represented by 16 bit words. At the

receiving part it must therefore be ensured that only valid bits

(13 bits for samples signal and two to seven bits for coded

parameters) are used. In verification tests, the testing system

may introduce random bit at non valid places inside these samples

(3 LSBs) or parameters (MSBs) to test this function. In the

digital test sequences all non valid bits are set to 0.

The following part of this section describes the required set of

arithmetic operations to implement the RPE-LTP algorithm in fixed

point.

For arithmetics operations or variables with a long integer type

(32 bit) a prefix L_ is used in order to distinguish them from the

16 bit variables or arithmetic operations.

All the names of the variables are identical to those of the

functional description of the RPE-LTP Codec (Section 3) but

variables like x', x'' are respectively called:

x' -----> xp

x''-----> xpp

in order to avoid any confusing notation.

NOTE: The x',x" variables are examples but are not used within the

following description.

The following notations are used in the arithmetic operations:

Square brackets ( [..] ) are used for arrays and when needed, the

starting index and the ending index are put inside the bracket.

For example x[0..159] means that x is an array of 160 words of 16

bits with beginning index 0 and ending index 159 and x[k] is an

element of the array x[0..159].

All functions' names are underlined. For example add( x, y) means

that we perform the addition of x and y.

<< n : denotes a n-bit arithmetic shift left operation (zero

fill) on variables of type short or long; if n is less

than 0, this operation becomes an arithmetic right shift

of -n.

>> n : denotes a n-bit arithmetic right shift operation (sign

extension ) on variables of type short or long; if n is

less than 0, this operation becomes an arithmetic left

shift of -n (zero fill).

a > b  : denotes the "greater than" condition;

a >= b : denotes the "greater than or equal" condition;

a < b  : denotes the "less than" condition;

a <= b : denotes the "less than or equal" condition;

a == b : denotes the "equal to" condition.

The basic structure of the FOR-NEXT loop is used in this

description for loop computation; the declaration is:

|== FOR k= start to end:

|    inner computation;

|== NEXT k:

Also the IF.. ELSE IF structure is used throughout this detailed

description. The basic structure is:

IF (condition1) THEN statement1;

ELSE IF ( condition2) THEN statement2;

ELSE IF ( condition3) THEN statement3;

The word EXIT is used to exit immediately from a procedure.

The following arithmetic operations are defined:

add( var1, var2)     : performs the addition (var1+var2) with

: overflow control and saturation; the result

: is set at +32767 when overflow occurs or at

: -32768 when underflow occurs.

sub( var1, var2)     : performs the subtraction (var1-var2) with

: overflow control and saturation; the result

: is set at +32767 when overflow occurs or at

: -32768 when underflow occurs.

mult( var1, var2)  : performs the multiplication of var1 by var2

: and gives a 16 bits result which is scaled ie

: mult( var1,var2 ) = (var1 times var2) >> 15

: and mult( -32768, -32768 ) = 32767

mult_r( var1, var2): same as mult but with rounding ie

: mult_r( var1, var2 ) =

: ( (var1 times var2) + 16384 ) >> 15

: and mult_r( -32768, -32768 ) = 32767

abs( var1 )      : absolute value of var1; abs(-32768) = 32767

div( var1, var2)   : div produces a  result which is the

: fractional integer division of var1 by var2;

: var1 and var2 must be positive and var2 must

: be greater or equal to var1; The result is

: positive (leading bit equal to 0) and

: truncated to 16 bits. if var1 == var2 then

: div( var1, var2 ) = 32767

L_mult(var1, var2) : L_mult is a 32 bit result for the

: multiplication of var1 times var2 with a one

: bit shift left. L_mult( var1, var2 ) =

| ( var1 times var2 ) << 1. The condition

| L_mult( -32768, -32768 ) does not occur in

| the algorithm.

L_add(L_var1, L_var2): 32 bits addition of two 32 bits variables

: (L_var1 + L_var2) with overflow control and

: saturation; the result is set at 2147483647

: when overflow occurs and at -2147483648

: when underflow occurs.

L_sub(L_var1,L_var2): 32 bits subtraction of two 32 bits variables

: (L_var1 - L_var2) with overflow control and

: saturation; the result is set at 2147483647

: when overflow occurs and at -2147483648

: when underflow occurs.

norm( L_var1 )     : norm produces the number of left shifts

: needed to normalize the 32 bits variable

: L_var1 for positive values on the interval

: with  minimum of 1073741824 and maximum of

: 2147483647 and for negative values on the

: interval with  minimum of -2147483648 and

: maximum of -1073741824; in order to normalize

: the result, the following operation must be

: done: L_norm_var1 = L_var1 << norm(L_var1)

L_var2 = var1;      : deposit the 16 bits of var1 in the LSB 16

: bits of L_var2 with sign extension.

var2 = L_var1;      : extract the 16 LSB bits of L_var1 to put in

: var2.

When a constant is used in an operation on 32 bits, it must be

first sign-extended on 32 bits.

4.2. FIXED POINT IMPLEMENTATION OF THE RPE-LTP CODER

-----------------------------------------------

The RPE-LTP coder works on a frame by frame basis. The length of

the frame is equal to 160 samples. Some computations are done once

per frame (analysis) and some others for each of the four

sub-segments (40 samples).

In the following detailed description, procedure 4.2.0 to 4.2.10

are done once per frame to produce at the output of the coder the

LARc[1..8] parameters which are the coded LAR coefficients and

also to realize the inverse filtering operation for the entire

frame (160 samples of signal d[0..159]). These parts produce at

the output of the coder:

| LARc[1..8]     : Coded LAR coefficients

|--> These parameters are calculated and sent once per

frame.

Procedure 4.2.11 to 4.2.18 are to be executed four times per

frame. That means once for each sub-segment RPE-LTP analysis of 40

samples. These parts produce at the output of the coder:

| Nc        : LTP lag;

| bc        : Coded LTP gain;

| Mc        : RPE grid selection;

| xmaxc          : Coded maximum amplitude of the RPE

sequence;

| xMc[0..12]     : Codes of the normalized RPE samples;

|--> These parameters are calculated and sent four times

per frame.

PREPROCESSING SECTION

4.2.0. Scaling of the input variable

-----------------------------

After A-law to linear conversion (or directly from the A to D

converter) the following scaling is assumed for input to the

RPE-LTP algorithm:

S.v.v.v.v.v.v.v.v.v.v.v.v.x.x.x ( 2's complement format).

Where S is the sign bit, v a valid bit, and x a "don't care"

bit.

The original signal is called sop[..];

4.2.1. Downscaling of the input signal

-------------------------------

|== FOR k=0 to 159:

|    so[k]  = sop[k] >> 3;

|    so[k]  = so[k] << 2;

|== NEXT k:

4.2.2. Offset compensation

-------------------

This part implements a high-pass filter and requires extended

arithmetic precision for the recursive part of this filter.

The input of this procedure is the array so[0..159] and the output

the array sof[0..159].

|== FOR k = 0 to 159:

| Compute the non-recursive part.

|    s1 = sub( so[k], z1 );

|    z1 = so[k];

| Compute the recursive part.

|    L_s2 = s1;

|    L_s2 = L_s2 << 15;

| Execution of a 31 by 16 bits multiplication.

|    msp = L_z2 >> 15;

|    lsp = L_sub( L_z2, ( msp << 15 ) );

|    temp = mult_r( lsp, 32735 );

|    L_s2 = L_add( L_s2, temp );

|    L_z2 = L_add(  L_mult( msp, 32735 ) >> 1, L_s2 );

| Compute sof[k] with rounding.

|    sof[k] =  L_add( L_z2, 16384 )  >> 15;

|== NEXT k:

Keep z1 and L-z2 in memory for the next frame.

Initial value: z1=0; L_z2=0;

4.2.3. Preemphasis

-----------

|== FOR k=0 to 159:

|    s[k] = add( sof[k], mult_r( mp, -28180 ) );

|    mp = sof[k];

|== NEXT k:

Keep mp in memory for the next frame.

Initial value: mp=0;

LPC ANALYSIS SECTION

4.2.4. Autocorrelation

---------------

The goal is to compute the array L_ACF[k]. The signal s[i] must be

scaled in order to avoid an overflow situation.

Dynamic scaling of the array s[0..159].

Search for the maximum.

smax = 0;

|== FOR k = 0 to 159:

|    temp = abs( s [k] );

|    IF ( temp > smax ) THEN smax = temp;

|== NEXT k;

Computation of the scaling factor.

IF ( smax == 0 ) THEN scalauto = 0;

ELSE  scalauto = sub( 4, norm( smax << 16 )  );

Scaling of the array s[0..159].

IF ( scalauto > 0 ) THEN

| temp = 16384 >> sub( scalauto,1);

|== FOR k = 0 to 159:

|    s[k] = mult_r( s[k], temp);

|== NEXT k:

Compute the L-ACF[..].

|== FOR k=0 to 8:

|    L_ACF[k] = 0;

|==== FOR i=k to 159:

|      L_temp = L_mult( s[i], s[i-k] );

|      L_ACF[k] = L_add( L_ACF[k], L_temp );

|==== NEXT i:

|== NEXT k:

Rescaling of the array s[0..159].

IF ( scalauto > 0 ) THEN

|== FOR k = 0 to 159:

|    s[k] = s[k] << scalauto;

|== NEXT k:

4.2.5. Computation of the reflection coefficients

------------------------------------------

Schur recursion with 16 bits arithmetic.

IF( L_ACF[0] == 0 ) THEN

|== FOR i = 1 to 8:

|    r[i] = 0;

|== NEXT i:

|    EXIT; /continue with section

4.2.6/

temp = norm( L_ACF[0] );

|== FOR k=0 to 8:

|    ACF[k] = ( L_ACF[k] << temp ) >> 16;

|== NEXT k:

Initialize array P[..] and K[..] for the recursion.

|== FOR i=1 to 7:

|    K[9-i] = ACF[i];

|== NEXT i:

|== FOR i=0 to 8:

|    P[i] = ACF[i];

|== NEXT i:

Compute reflection coefficients.

|== FOR n=1 to 8:

|    IF( P[0] < abs( P[1] ) ) THEN

|                                    |== FOR i = n to 8:

|                                    |    r[i] = 0;

|                                    |== NEXT i:

|                                    | EXIT; /continue with

|                                    |       section 4.2.6/

|    r[n] = div( abs( P[1] ), P[0] );

|    IF ( P[1] > 0 ) THEN r[n] = sub( 0, r[n] );

|

|    IF (  n ==  8 ) THEN EXIT; /continue with

section 4.2-6/

|    Schur recursion.

|    P[0] = add( P[0], mult_r( P[1], r[n] ) );

|==== FOR m=1 to 8-n:

|      P[m] = add( P[m+1], mult_r( K[9-m], r[n] ) );

|      K[9-m] = add( K[9-m], mult_r( P[m+1], r[n] ) );

|==== NEXT m:

|

|== NEXT n:

NOTE: The following lines gives one correct implementation of the

div(num, denum) arithmetic operation. Compute div which is

the integer division of num by denum: with denum >= num > 0.

L_num = num;

L_denum = denum;

div =0;

|== FOR k = 0 to 14:

|    div= div << 1;

|    L_num = L_num << 1;

|    IF ( L_num >= L_denum) THEN

|                         | L_num=L_sub(L_num, L_denum);

|                         | div = add( div ,1 );

|== NEXT k:

4.2.6. Transformation of reflection coefficients to Log.-Area

Ratios

------

The following scaling for r[..] and LAR[..] has been used:

/* r[..] = integer( real_r[..]*32768. );  -1. <= real_r <1. */

/*                                                          */

/* LAR[..] = integer( real_LAR[..]*16384. );                */

/*                                                          */

/* with  -1.625 <= real_LAR <= 1.625                        */

Computation of the LAR[1..8] from the r[1..8].

|== FOR i = 1 to 8:

|    temp = abs( r[i] );

|    IF ( temp < 22118 ) THEN temp = temp >> 1;

|       ELSE  IF ( temp < 31130 ) THEN

|              temp= sub(temp, 11059);

|          ELSE temp = sub( temp, 26112 ) << 2;

|    LAR[i] = temp;

|    IF ( r[i] < 0 ) THEN LAR[i] = sub( 0, LAR[i] );

|== NEXT i:

4.2.7. Quantization and coding of the Log.-Area Ratios

-----------------------------------------------

This procedure needs four tables; the following equations give the

optimum scaling for the constants:

/* A[1..8]= integer( real_A[1..8]*1024); 8 values (see table4.1)*/

/*                                                              */

/* B[1..8]= integer( real_B[1..8]*512);  8 values (see table4.1)*/

/*                                                              */

/* MAC[1..8]= maximum of the LARc[1..8]; 8 values (see table4.1)*/

/*                                                              */

/* MIC[1..8]= minimum of the LARc[1..8]; 8 values (see table4.1)*/

Computation for quantizing and coding the LAR[1..8].

|== FOR i =1 to 8:

|    temp= mult( A[i], LAR[i] );

|    temp= add( temp, B[i] );

|    temp= add( temp, 256);     for rounding

|    LARc[i]= temp >> 9;

|  Check IF LARc[i] lies between MIN and MAX

|     IF ( LARc[i] > MAC[i] ) THEN LARc[i] = MAC[i];

|     IF ( LARc[i] < MIC[i] ) THEN LARc[i] = MIC[i];

|    LARc[i] = sub( LARc[i], MIC[i] );  /See note below/

|== NEXT i:

NOTE: The equation is used to make all the LARc[i] positive.

SHORT TERM ANALYSIS FILTERING SECTION

4.2.8. Decoding of the coded Log.-Area Ratios

--------------------------------------

This procedure requires for efficient implementation two tables.

/* INVA[1..8]=integer((32768*8)/(real_A[1..8]);                */

/*                                       8 values (table 4.2 ) */

/* MIC[1..8]=minimum value of the LARc[1..8];                  */

/*                                       8 values (table 4.1)  */

Compute the LARpp[1..8].

|== FOR i=1 to 8:

|    temp1 = add( LARc[i], MIC[i] ) << 10; /See note below/

|    temp2 = B[i] << 1;

|    temp1 = sub( temp1, temp2);

|    temp1 = mult_r( INVA[i], temp1);

|    LARpp[i] = add( temp1, temp1);

|== NEXT i:

NOTE: The addition of MIC[i] is used to restore the sign of

LARc[i].

4.2.9. Computation of the quantized reflection coefficients

----------------------------------------------------

Within each frame of 160 analysed speech samples the short term

analysis and synthesis filters operate with four different sets of

coefficients, derived from the previous set of decoded

LARs(LARpp(j-1)) and the actual set of decoded LARs (LARpp(j)).

4.2.9.1. Interpolation of the LARpp[1..8] to get the LARp[1..8]

------------------------------------------------------

For k_start = 0 to k_end = 12.

|==== FOR i= 1 to 8:

|     LARp[i] = add((LARpp(j-1)[i] >> 2),(LARpp(j)[i] >> 2));

|     LARp[i] = add( LARp[i] , ( LARpp(j-1)[i]  >> 1 ) );

|==== NEXT i:

For k_start = 13 to k_end = 26.

|==== FOR i= 1 to 8:

|     LARp[i] = add((LARpp(j-1)[i] >> 1),(LARpp(j)[i] >> 1 ));

|==== NEXT i:

For k_start = 27 to k_end = 39.

|==== FOR i= 1 to 8:

|     LARp[i] = add((LARpp(j-1)[i] >> 2),(LARpp(j)[i] >> 2 ));

|     LARp[i] = add( LARp[i] , ( LARpp(j)[i]  >> 1 ) );

|==== NEXT i:

For k_start = 40 to k_end = 159.

|==== FOR i= 1 to 8:

|     LARp[i] =  LARpp(j)[i];

|==== NEXT i:

Initial value: LARpp(j-1)[1..8]=0;

4.2.9.2. Computation of the rp[1..8] from the interpolated

LARp[1..8]

----------

The input of this procedure is the interpolated LARp[1..8] array.

The reflection coefficients, rp[i], are used in the analysis

filter and in the synthesis filter.

|== FOR i=1 to 8:

|    temp = abs( LARp[i] );

|    IF ( temp < 11059 ) THEN temp = temp << 1;

|        ELSE IF (temp < 20070) THEN

|                 temp = add(temp, 11059);

|           ELSE temp = add( (temp >> 2), 26112 );

|    rp[i] = temp;

|    IF ( LARp[i] < 0 ) THEN rp[i] = sub( 0, rp[i] );

|== NEXT i:

4.2.10. Short term analysis filtering

-----------------------------

This procedure computes the short term residual signal d[..] to be

fed to the RPE-LTP loop from the s[..] signal and from the local

rp[..] array (quantized reflection coefficients). As the call of

this procedure can be done in many ways (see the interpolation of

the LAR coefficient), it is assumed that the computation begins

with index k_start (for arrays d[..] and s[..]) and stops with

index k_end (k_start and k_end are  defined in 4.2.9.1). This

procedure also needs to keep the array u[0..7] in memory for each

call.

|== FOR k = k_start to k_end:

|    di = s[k]

|    sav = di;

|==== FOR i = 1 to 8:

|      temp = add( u[i-1], mult_r( rp[i], di ) );

|      di = add( di, mult_r( rp[i], u[i-1] ) );

|      u[i-1] = sav;

|      sav = temp;

|==== NEXT i:

|    d[k] = di;

|== NEXT k:

Keep the array u[0..7] in memory.

Initial value: u[0..7]=0;

LONG TERM PREDICTOR (LTP) SECTION

4.2.11. Calculation of the LTP parameters

---------------------------------

This procedure computes the  LTP gain (bc) and the LTP lag (Nc)

for the long term analysis filter. This is done by calculating a

maximum of the cross-correlation function between the current

sub-segment short  term residual signal d[0..39] (output of the

short term analysis filter; for simplification the index of this

array begins at 0 and ends at 39 for each sub-segment of the

RPE-LTP analysis) and the previous reconstructed short term

residual signal dp[-120..-1]. A dynamic scaling must be performed

to avoid overflow.

Search of the optimum scaling of d[0..39].

dmax = 0;

|== FOR k = 0 to 39:

|    temp = abs( d[k] );

|    IF ( temp > dmax ) THEN dmax = temp;

|== NEXT k:

temp = 0;

IF ( dmax == 0 ) THEN scal = 0;

ELSE temp = norm( dmax << 16 );

IF ( temp > 6 ) THEN scal = 0;

ELSE scal = sub( 6, temp );

Initialisation of a working array wt[0..39].

|== FOR k = 0 to 39:

|    wt[k] = d[k] >> scal;

|== NEXT k:

Search for the maximum cross-correlation and coding of the LTP lag.

L_max = 0;

Nc = 40;   (index for the maximum cross-correlation)

|== FOR lambda = 40 to 120:

|    L_result = 0;

|==== FOR k = 0 to 39:

|      L_temp = L_mult( wt[k], dp[k-lambda] );

|      L_result = L_add( L_temp, L_result );

|==== NEXT k:

|    IF ( L_result > L_max) THEN

|                                | Nc = lambda;

|                                | L_max = L_result ;

|== NEXT lambda:

Rescaling of L-max.

L_max = L_max >> ( sub( 6, scal ) );

Initialisation of a working array wt[0..39].

|== FOR k = 0 to 39:

|    wt[k] = dp[k-Nc] >> 3;

|== NEXT k:

Compute the power of the reconstructed short term residual signal

dp[..].

L_power = 0;

|== FOR k =0 to 39:

|    L_temp = L_mult( wt[k], wt[k] );

|    L_power = L_add( L_temp, L_power );

|== NEXT k:

Normalization of L-max and L-power.

IF ( L_max <= 0 ) THEN

| bc = 0;

| EXIT; /cont. with 4.2-12/

IF ( L_max >= L_power ) THEN

| bc = 3;

| EXIT; /cont. with 4.2-12/

temp = norm( L_power );

R = ( L_max << temp ) >> 16;

S = ( L_power << temp ) >> 16;

Coding of the LTP gain.

Table 4.3a must be used to obtain the level DLB[i] for the

quantization of the LTP gain b to get the coded version bc.

|== FOR bc = 0 to 2:

|      IF (R <= mult(S, DLB[bc])) THEN EXIT; /cont. with

4.2.12/

|== NEXT bc;

bc = 3;

Initial value: dp[-120..-1]=0;

4.2.12. Long term analysis filtering

----------------------------

In this part, we have to decode the bc parameter to compute the

samples of the estimate dpp[0..39]. The decoding of bc needs the

use of table 4.3b. The long term residual signal e[0..39] is then

calculated to be fed to the RPE encoding section.

Decoding of the coded LTP gain.

bp = QLB[bc];

Calculating the array e[0..39] and the array dpp[0..39].

|== FOR k = 0 to 39:

|    dpp[k] = mult_r( bp, dp[k-Nc] );

|    e[k] = sub( d[k], dpp[k] );

|== NEXT k:

RPE ENCODING SECTION

4.2.13. Weighting filter

----------------

The coefficients of the weighting filter are stored in a table

(see table 4.4). The following scaling is used:

/* H[0..10] = integer( real_H[0..10]*8192 ); */

Initialisation of a temporary working array wt[0..49].

|== FOR k= 0 to 4:

|    wt[k] = 0;

|== NEXT k:

|== FOR k = 5 to 44:

|    wt[k] = e[k-5];

|== NEXT k:

|== FOR k= 45 to 49:

|    wt[k] = 0;

|== NEXT k:

Compute the signal x[0..39].

|== FOR k= 0 to 39:

|      L_result =  8192; /rounding of the output

of the filter/

|==== FOR i = 0 to 10:

|      L_temp = L_mult( wt[k+i], H[i] );

|      L_result = L_add( L_result, L_temp );

|==== NEXT i:

|      L_result = L_add(L_result,L_result); /scaling (x2)/

|      L_result = L_add(L_result,L_result); /scaling (x4)/

|    x[k] = L_result >> 16;

|== NEXT k:

4.2.14. RPE grid selection

------------------

The signal x[0..39] is used to select the RPE grid which is

represented by Mc.

EM =0;

Mc = 0;

|== FOR m = 0 to 3:

|    L_result = 0;

|==== FOR i = 0 to 12:

|      temp1 = x[m+(3*i)] >> 2;

|      L_temp = L_mult( temp1, temp1 );

|      L_result = L_add( L_temp, L_result );

|==== NEXT i:

|    IF ( L_result > EM) THEN

|                                | Mc = m;

|                                | EM = L_result;

|== NEXT m:

Down-sampling by a factor 3 to get the selected xM[0..12] RPE

sequence.

|== FOR i = 0 to 12:

|    xM[i] = x[Mc +(3*i)];

|== NEXT i:

4.2.15. APCM quantization of the selected RPE sequence

----------------------------------------------

Find the maximum absolute value xmax of xM[0..12].

xmax = 0;

|== FOR i = 0 to 12:

|    temp = abs( xM[i] ) ;

|    IF ( temp > xmax ) THEN xmax = temp;

|== NEXT i:

Quantizing and coding of xmax to get xmaxc.

exp = 0;

temp = xmax >> 9;

itest = 0;

|== FOR i = 0 to 5:

|    IF ( temp <= 0 ) THEN itest = 1;

|    temp = temp >> 1;

|    IF ( itest == 0 ) THEN exp = add( exp, 1 ) ;

|== NEXT i:

temp = add( exp, 5 ) ;

xmaxc = add( ( xmax >> temp ), ( exp << 3 ) ) ;

Quantizing and coding of the xM[0..12] RPE sequence to get the

xMc[0..12].

This computation uses the fact that the decoded version of xmaxc

can be calculated by using the exponent and the mantissa part of

xmaxc (logarithmic table).

So, this method avoids any division and uses only a scaling of the

RPE samples by a function of the exponent. A direct multiplication

by the inverse of the mantissa (NRFAC[0..7] found in table 4.5)

gives the 3 bit coded version xMc[0..12] of the RPE samples.

Compute exponent and mantissa of the decoded version of

xmaxc.

exp = 0 ;

IF ( xmaxc > 15 ) THEN exp = sub( ( xmaxc >> 3 ), 1 ) ;

mant = sub( xmaxc , ( exp << 3 ) );

Normalize mantissa 0 <= mant <= 7.

IF ( mant == 0 ) THEN | exp = -4;

| mant = 15;

ELSE | itest = 0;

|== FOR i = 0 to 2:

|    IF ( mant > 7 ) THEN itest = 1;

|    IF (itest == 0) THEN mant = add((mant << 1),1);

|    IF ( itest == 0 ) THEN exp = sub( exp, 1 );

|== NEXT i:

mant = sub( mant, 8 );

Direct computation of xMc[0..12] using table 4.5.

temp1= sub( 6, exp );  /normalization by the exponent/

temp2  =  NRFAC[mant];  /see table 4.5 (inverse mantissa)/

|== FOR i = 0 to 12:

|    temp = xM[i] << temp1;

|    temp = mult( temp , temp2 );

|    xMc[i] = add( ( temp >> 12 ), 4 );  /See note below/

|== NEXT i:

NOTE: This equation is used to make all the xMc[i] positive.

Keep in memory exp and mant for the following inverse APCM

quantizer.

4.2.16. APCM inverse quantization

-------------------------

This part is for decoding the RPE sequence of coded xMc[0..12]

samples to obtain the xMp[0..12] array. Table 4.6 is used to get

the mantissa of xmaxc (FAC[0..7]).

temp1 = FAC[mant];  see 4.2-15 for mant

temp2= sub( 6, exp );  see 4.2-15 for exp

temp3= 1 << sub( temp2, 1 );

|== FOR i =0 to 12:

|    temp = sub( ( xMc[i] << 1 ), 7 );  /See note below/

|    temp = temp << 12;

|    temp = mult_r( temp1, temp );

|    temp = add( temp, temp3 );

|    xMp[i] = temp >> temp2;

|== NEXT i;

NOTE: This subtraction is used to restore the sign of xMc[i].

4.2.17. RPE grid positioning

--------------------

This procedure computes the reconstructed long term residual

signal ep[0..39] for the LTP analysis filter. The inputs are the

Mc which is the grid position selection and the xMp[0..12] decoded

RPE samples which are upsampled by a factor of 3 by inserting zero

values.

|== FOR k = 0 to 39:

|    ep[k] = 0;

|== NEXT k:

|== FOR i = 0 to 12:

|    ep[Mc +(3*i)] = xMp[i];

|== NEXT i:

4.2.18. Update of the reconstructed short term residual signal

dp[-120..-1]

------------

This procedure adds the reconstructed long term residual signal

ep[0..39] to the estimated signal dpp[0..39] from the long term

analysis filter to compute the recontsructed short term residual

signal dp[-40..-1]; also the reconstructed short term residual

array dp[-120..-41] is updated.

|== FOR k = 0 to 79:

|    dp[-120+k] = dp[-80+k];

|== NEXT k:

|== FOR k = 0 to 39:

|    dp[-40+k] = add( ep[k], dpp[k] );

|== NEXT k:

Keep the array dp[-120..-1] in memory for the next sub-segment.

Initial value: dp[-120..-1]=0;

4.3. FIXED POINT IMPLEMENTATION OF THE RPE-LTP DECODER

-------------------------------------------------

Only the synthesis filter and the de-emphasis procedure are

different from the procedures found in the RPE-LTP coder.

Procedures 4.3.1 and 4.3.2 are executed for each sub-segment (four

times per frame). Procedures 4.3.3, 4.3.4 and 4.3.5 are executed

once per frame.

4.3.1. RPE decoding section

--------------------

Procedures 4.2.15 (only the part to get mant and exp of xmaxc),

4.2.16 and 4.2.17 are used to obtain the reconstructed long term

residual signal erp[0..39] signal from the received parameters for

each sub-segment (ie Mcr, xmaxcr, xmcr[0..12]).

4.3.2. Long term synthesis filtering

-----------------------------

This procedure uses the bcr and Ncr parameter to realize the long

term synthesis filtering. The decoding of bcr needs the use of

table 4.3b.

- Nr is the received and decoded LTP lag.

- An array drp[-120..39] is used in this procedure.

The elements for -120 to -1 of the array drp are kept in memory

for the long term synthesis filter. For each sub-segment (40

samples), this procedure computes the drp[0..39] to be fed to the

synthesis filter.

Check the limits of Nr.

Nr = Ncr;

IF ( Ncr < 40 ) THEN Nr = nrp;

IF ( Ncr > 120 ) THEN Nr = nrp;

nrp= Nr;

Keep the nrp value for the next sub-segment.

Initial value: nrp=40;

Decoding of the LTP gain bcr.

brp = QLB[bcr]

Computation of the reconstructed short term residual signal

drp[0..39].

|== FOR k = 0 to 39:

|    drpp = mult_r( brp, drp[k-Nr] );

|    drp[k] = add( erp[k], drpp );

|== NEXT k:

Update of the reconstructed short term residual signal

drp[-1..-120].

|== FOR k = 0 to 119:

|    drp[-120+k] = drp[-80+k];

|== NEXT k:

Keep the array drp[-120..-1] for the next sub-segment.

Initial value: drp[-120..-1]=0;

4.3.3. Computation of the decoded reflection coefficients

--------------------------------------------------

This procedure (which is executed once per frame) is the same as

the one described in the CODER part.

For decoding of the received LARcr[1..8], see procedure 4.2.8.

For the interpolation of the decoded Log.-Area Ratios, see

procedure 4.2.9.1 and for the computation of the reflection

coefficients rrp[1..8], see procedure 4.2.9.2.

4.3.4. Short term synthesis filtering section

--------------------------------------

This procedure uses the drp[0..39] signal and produces the

sr[0..159] signal which is the output of the short term synthesis

filter. For ease of explanation, a temporary array wt[0..159] is

used.

Initialisation of the array wt[0..159].

For the first sub-segment in a frame:

|== FOR k = 0 to 39:

|     wt[k] = drp[k];

|== NEXT k:

For the second sub-segment in a frame:

|== FOR k = 0 to 39:

|     wt[40+k] = drp[k];

|== NEXT k:

For the third sub-segment in a frame:

|== FOR k = 0 to 39:

|     wt[80+k] = drp[k];

|== NEXT k:

For the fourth sub-segment in a frame:

|== FOR k = 0 to 39:

|     wt[120+k] = drp[k];

|== NEXT k:

As the call of the short term synthesis filter procedure can be

done in many ways (see the interpolation of the LAR coefficient),

it is assumed that the computation begins with index k_start (for

arrays wt[..] and sr[..]) and stops with index k_end (k_start and

k_end are defined in 4.2.9.1). The procedure also needs to keep

the array v[0..8] in memory between calls.

|== FOR k = k_start to k_end:

|    sri = wt[k];

|==== FOR i = 1 to 8:

|      sri = sub( sri, mult_r( rrp[9-i], v[8-i] ) );

|      v[9-i] = add( v[8-i], mult_r( rrp[9-i], sri ) );

|==== NEXT i:

|    sr[k] = sri;

|    v[0] = sri;

|== NEXT k:

Keep the array v[0..8] in memory for the next call.

Initial value: v[0..8]=0;

POSTPROCESSING

4.3.5. Deemphasis filtering

--------------------

|== FOR k = 0 to 159:

|    temp = add( sr[k], mult_r( msr, 28180 ) );

|    msr = temp;

|    sro[k] = msr;

|== NEXT k:

Keep msr in memory for the next frame.

Initial value: msr=0;

4.3.6. Upscaling of the output signal

------------------------------

|== FOR k = 0 to 159:

|    srop[k] = add( sro[k], sro[k] );

|== NEXT k:

4.3.7. Truncation of the output variable

---------------------------------

|== FOR k = 0 to 159:

|    srop[k] = srop[k] >> 3;

|    srop[k] = srop[k] << 3;

|== NEXT k:

The output format is the following:

S.v.v.v.v.v.v.v.v.v.v.v.v.0.0.0  (2's complement).

Where S is the sign bit, v a valid bit.

NOTE: When a linear to A-law compression is needed, then the

sub-block COMPRESS of CCITT G721 recommendation must be used

with inputs:

SR = srop[k] >> 3;

LAW = 1;

4.4. TABLES USED IN  THE FIXED POINT IMPLEMENTATION OF THE RPE-LTP

CODER AND DECODER

-----------------

----- ---------------------------------------------------

|  i  |    A[i]  |    B[i]  |    MIC[i]    |    MAC[i]    |

|-----|----------|----------|--------------|--------------|

|  1  |  20480   |       0  |      -32     |       31     |

|-----|----------|----------|--------------|--------------|

|  2  |  20480   |       0  |      -32     |       31     |

|-----|----------|----------|--------------|--------------|

|  3  |  20480   |    2048  |      -16     |       15     |

|-----|----------|----------|--------------|--------------|

|  4  |  20480   |   -2560  |      -16     |       15     |

|-----|----------|----------|--------------|--------------|

|  5  |  13964   |      94  |       -8     |        7     |

|-----|----------|----------|--------------|--------------|

|  6  |  15360   |   -1792  |       -8     |        7     |

|-----|----------|----------|--------------|--------------|

|  7  |   8534   |    -341  |       -4     |        3     |

|-----|----------|----------|--------------|--------------|

|  8  |   9036   |   -1144  |       -4     |        3     |

---------------------------------------------------------

Table 4.1. Quantization of the Log.-Area Ratios

------------------------------------

----- -------------

|  i  |     INVA[i] |

|-----|-------------|

|  1  |     13107   |

|-----|-------------|

|  2  |     13107   |

|-----|-------------|

|  3  |     13107   |

|-----|-------------|

|  4  |     13107   |

|-----|-------------|

|  5  |     19223   |

|-----|-------------|

|  6  |     17476   |

|-----|-------------|

|  7  |     31454   |

|-----|-------------|

|  8  |     29708   |

---- --------------

Table 4.2. Tabulation of 1/A[1..8]

-----------------------

-----------------

|  bc  | DLB[bc]  |

|------|----------|

|   0  |   6554   |

|------|----------|

|   1  |  16384   |

|------|----------|

|   2  |  26214   |

|------|----------|

|   3  |  32767   |

-----------------

Table 4.3a. Decision level of the LTP gain quantizer

----------------------------------------

--------------------

|  bc  |  QLB[bc]    |

|------|-------------|

|   0  |    3277     |

|------|-------------|

|   1  |   11469     |

|------|-------------|

|   2  |   21299     |

|------|-------------|

|   3  |   32767     |

--------------------

Table 4.3b. Quantization levels of the LTP gain quantizer

---------------------------------------------

----------------

|  i  |    H[i]  |

|-----|----------|

|  0  |   -134   |

|-----|----------|

|  1  |   -374   |

|-----|----------|

|  2  |      0   |

|-----|----------|

|  3  |   2054   |

|-----|----------|

|  4  |   5741   |

|-----|----------|

|  5  |   8192   |

|-----|----------|

|  6  |   5741   |

|-----|----------|

|  7  |   2054   |

|-----|----------|

|  8  |      0   |

|-----|----------|

|  9  |   -374   |

|-----|----------|

| 10  |   -134   |

----------------

Table 4.4. Coefficients of the weighting filter

------------------------------------

----------------

|  i  | NRFAC[i] |

|-----|----------|

|  0  |  29128   |

|-----|----------|

|  1  |  26215   |

|-----|----------|

|  2  |  23832   |

|-----|----------|

|  3  |  21846   |

|-----|----------|

|  4  |  20165   |

|-----|----------|

|  5  |  18725   |

|-----|----------|

|  6  |  17476   |

|-----|----------|

|  7  |  16384   |

----------------

Table 4.5. Normalized inverse mantissa used to compute xM/xmax

---------------------------------------------------

----------------

|  i  |  FAC[i]  |

|-----|----------|

|  0  |  18431   |

|-----|----------|

|  1  |  20479   |

|-----|----------|

|  2  |  22527   |

|-----|----------|

|  3  |  24575   |

|-----|----------|

|  4  |  26623   |

|-----|----------|

|  5  |  28671   |

|-----|----------|

|  6  |  30719   |

|-----|----------|

|  7  |  32767   |

----------------

Table 4.6. Normalized direct mantissa used to compute xM/xmax

--------------------------------------------------

5. DIGITAL TEST SEQUENCES

======================

This chapter provides information on the digital test sequences

that have been designed to help in the verification of

implementations of the RPE-LTP codec. Copies of these sequences

are available (see annex 3).

5.1. INPUT AND OUTPUT SIGNALS

------------------------

Table 5.1 defines the input and output signals for the test

sequences. The words defined in this table use 16 bits. The left

or right justification is indicated in the table.

The codewords described in the table correspond to one frame of

coder input or decoder output signal; i.e for 20 ms of input

signal the 76 codewords are obtained at the output of the coder

and 76 codewords provided at the input of the decoder will yield

20 ms of output signal in the decoder.

5.2. CONFIGURATION FOR THE APPLICATION OF THE TEST SEQUENCES

-------------------------------------------------------

Two configurations are appropriate in order to test an

implementation of the RPE-LTP coder. The first one is for testing

the coder part of the RPE-LTP; it means that a sop[..] signal is

provided at the input of the encoder that furnishes frames of

coded parameters. This output has to be checked against some

reference file. The other configuration is for testing the decoder

part of the RPE-LTP; in this case frames of coded parameters (see

table 5.1) are sent to the RPE-LTP decoder that furnishes the

srop[..] signal. These samples have to be checked against a

reference file.

5.2.1. Configuration 1 (encoder only)

------------------------------

A reset signal (RS) must be applied to the RPE-LTP encoder under

test to set all internal variables to the exact states specified

in section 4 of this recommendation prior to the start of an input

test sequence in order to obtain the correct output values for

this test. This test must be done in real time with a sampling

rate of 8 kHz at the input of the encoder under test (see figure

5.1). All the necessary hardware and software should be installed

by the user in order to capture in real time the output coded

parameters of the RPE-LTP encoder and to compare them to the

dedicated reference file.

| 8 kHz sampling rate

v

--------------

RS --->| RPE-LTP      |  Coded parameters

| Encoder      |------------/------->  Comparison

Input:   ----/->| under test   |           2-7 bits

13 bits  --------------

Figure 5.1. Configuration 1: RPE-LTP encoder under test

-------------------------------------------

5.2.2. Configuration 2 (Decoder only)

------------------------------

Figure 5.2 shows a RPE-LTP decoder under test. In the same way as

described in the coder part, a reset signal (RS) must be used

before the processing of the first frame of coded parameters. The

decoder must be tested for a continuous output with a sampling

rate of 8 kHz. At the input of the decoder, the 76 parameters must

be sent in a time interval of 20 ms.

| 8 kHz sampling rate

v

------------

RS ---->| RPE-LTP    |  Output signal: srop[k]

| Decoder    |-----------/-------> Comparison

Input: coded ---/-->| under test |           13 bits

parameters      ------------

2-7 bits

Figure 5.2. Configuration 2: RPE-LTP decoder under test

-------------------------------------------

5.3. TEST SEQUENCES

--------------

5.3.1. Test sequences for configuration 1

----------------------------------

For configuration 1, four types of input test sequence are

provided:

1. Sequence for testing the overflow controls in the encoder

2. Sequence for testing the LPC part of the encoder

3. Sequence for testing the LTP part of the encoder

4. Sequence for testing various critical parts of the

algorithm

Sequence 1 uses a large number of saturated samples. The residual

LPC signal reaches very high values, which has two effects on the

processing:

- occurrence of a large number of overflows in

addition/subtraction operations. Table 5.2 describes each

overflow point and the number of occurrences for each.

- the excitation RPE samples have a large dynamic range and the 64

codewords of the sub-block maximum are each obtained at least

once on output.

Sequence 2 focuses successively on each reflection coefficient

calculated in the Schur recursion. Table 5.3 shows which frames

deal with which reflection coefficient and its dynamic range. The

Log.-Area codewords output by the coder cover the full range of

their possible values except the 2nd LARc that does not reach the

value 0 and 63 (min and max). The maximum value (63) is however

obtained in sequence 4.

Sequence 3 tests the long term predictor part of the algorithm. It

has been generated by exciting a sharply resonant filter with a

periodic train of impulses; this produces a pitched signal. Each

part corresponding to a given pitch is 128 ms (4 blocks of 256

words) long. The pitch periods have been randomly drawn in the

range [2,15] ms and the random order is shown in table 5.4.

Sequence 4 accounts for various remaining non tested points of the

algorithm where implementing errors may be suspected. Table 5.5

and 5.6 summarize the critical points that this sequence has been

designed to check (ie where the three previous sequences were

ineffective). Table 5.5 shows the list of tested points where

errors can be detected. Each tested error is described and the

frame number corresponding to the first occurrence of a divergence

between the exact and the degraded algorithm is also indicated.

Table 5.6 illustrates three paths of the algorithm that are never

explored during the processing of the three previous sequences;

the table shows which condition leads to each path and the number

of associated occurrences in sequence 4.

Notice finally one point where special care must be taken:

- A small degradation (ie +/- 1) of DLB[2] (the third decision

level of the LTP gain quantizer (see table 4.3a) is unable to

provide any noticeable effect on the output of the four

sequences described above).

5.3.2. Test sequences for configuration 2

----------------------------------

Five types of input test sequence are provided for this

configuration. Four sequences obtained in configuration 1 at the

output of the encoder (coded parameters) are used as input for the

decoder under test in configuration 2.

Table 5.7 gives the list of tested overflow points and their

occurrence on sequence 1 for this configuration.

Sequence 5 is provided to scan all possible codes for each

parameter. This sequence is an artificial sequence and does not

correspond to any encoder output. The codewords have been randomly

generated and cover the entire range of codewords values.

Moreover, the delay value Nr belonging to [40,120] in an

error-free transmission condition, takes in this sequence its

value in [0,127]. In this case the decoder behaviour on

non-allowed values of Nr will be tested.

==================================================================

Name      |               Description               |Justification

----------+-----------------------------------------+-------------

ENCODER INPUT

----------+-----------------------------------------+-------------

sop[k]    | 13 bits: encoder input signal.          |    left

----------+-----------------------------------------+-------------

OUTPUT PARAMETERS

----------+-----------------------------------------+-------------

LARc[1]   | 6 bits : 1st Log.-Area Ratio            |    right

LARc[2]   | 6 bits : 2nd Log.-Area Ratio            |    right

LARc[3]   | 5 bits : 3rd Log.-Area Ratio            |    right

LARc[4]   | 5 bits : 4th Log.-Area Ratio            |    right

LARc[5]   | 4 bits : 5th Log.-Area Ratio            |    right

LARc[6]   | 4 bits : 6th Log.-Area Ratio            |    right

LARc[7]   | 3 bits : 7th Log.-Area Ratio            |    right

LARc[8]   | 3 bits : 8th Log.-Area Ratio            |    right

----------------------------------------------------+-------------

Sub-frame no 1

----------------------------------------------------+-------------

Nc        | 7 bits : LTP lag                        |    right

bc        | 2 bits : LTP gain                       |    right

Mc        | 2 bits : RPE grid position              |    right

xmaxc     | 6 bits : Block amplitude                |    right

xMc[0..12]| 3 bits : RPE pulses index 0 to 12       |    right

----------------------------------------------------+-------------

Sub-frame no 2

----------------------------------------------------+-------------

Nc        | 7 bits : LTP lag                        |    right

bc        | 2 bits : LTP gain                       |    right

Mc        | 2 bits : RPE grid position              |    right

xmaxc     | 6 bits : Block amplitude                |    right

xMc[0..12]| 3 bits : RPE pulses index 0 to 12       |    right

----------------------------------------------------+-------------

Sub-frame no 3

----------------------------------------------------+-------------

Nc        | 7 bits : LTP lag                        |    right

bc        | 2 bits : LTP gain                       |    right

Mc        | 2 bits : RPE grid position              |    right

xmaxc     | 6 bits : Block amplitude                |    right

xMc[0..12]| 3 bits : RPE pulses index 0 to 12       |    right

----------------------------------------------------+-------------

Sub-frame no 4

----------------------------------------------------+-------------

Nc        | 7 bits : LTP lag                        |    right

bc        | 2 bits : LTP gain                       |    right

Mc        | 2 bits : RPE grid position              |    right

xmaxc     | 6 bits : Block amplitude                |    right

xMc[0..12]| 3 bits : RPE pulses index 0 to 12       |    right

==================================================================

Table 5.1a. Signals used in digital test sequences

--------------------------------------

==================================================================

DECODER OUTPUT

----------+-----------------------------------------+-------------

srop[k]   | 13 bits: decoder output signal. The 3   |    left

| LSB's of the 16 bits are equal to 0.    |

==================================================================

Table 5.1b. Signals used in digital test sequences

--------------------------------------

================================================================

Overflow point                                 No of occurrences

================================================================

Short term analysis filter (4.2.10)

1st add                                       1059

2nd add                                        134

----------------------------------------------------------------

LTP parameters computation (4.2.11)

Abs( d[k] )                                       5

----------------------------------------------------------------

Long term analysis filter (4.2.12)

sub                                              11

----------------------------------------------------------------

Weighting filter (4.2.13)

scaling the result (both x2 and x4)             302

----------------------------------------------------------------

APCM quantizer (4.2.15)

Find max abs of xm: Abs                          49

----------------------------------------------------------------

Update of Array dp of the long term analysis filter

(4.2.18)

add                                             126

================================================================

Table 5.2. List of tested overflow points for sequence 1 (coder

part)

-----

===================================================

Reflection Coeff.|    Frames       | Dynamic range

-----------------+-----------------+---------------

1           |      1-135      | -32564,32558

2           |    136-311      | -32356,32242

3           |    316-423      | -32157,32744

4           |    424-524      | -31594,31960

5           |    525-633      | -31697,31735

6           |    634-738      | -30055,31575

7           |    739-839      | -29090,31386

8           |    840-944      | -31052,31208

===================================================

Table 5.3.

Table 5.3 gives the position of the frames dedicated to the study of each reflection coefficient and dynamic range of the coefficient for sequence 2 in configuration 2.

----------------------------------------------

==================================================================

86  56  68 120  52  93  20  66  82 115 114  60  42  45  17  64  16

88  83  63  90  73  23  77 100  33  29 106  35  67  57 103 116  30

71  69  81  47  32  97  65  62 111  49 109  25  96  50  54  91  85

99  70  76  46  26  34 104 108 107  22 119  48  58  37  72 110  27

24  36  87  51  59  38  21  44 113  39  61  53  18  40  94 105  55

112 75  98 118  41  80  31  74  28  84  89  79  43 101  95  19  78

117 92 102

==================================================================

Table 5.4. Pitch periods of sequence 3 (configuration 1)

---------------------------------------------

==================================================================

|    Error checked :      | No of the

Test point            |incorrect   / correct    | 1st frame

|statement   / statement  | with error

----------------------------+-------------------------+----------

Autocorrelation function   |k=0 to 158  /  k=0 to 159|     27

(4.2.4)                    |                         |

|                         |

Computation of the reflect.|if( P[0] <= / if( P[0] < |    514

coefficients (4.2.5)       |abs(P[1]) ) / abs(P[1]) )|

|                         |

Quantization and coding    | A[4] + 1   /    A[4]    |     21

of the LARs (4.2.7)        | A[5] - 1   /    A[5]    |     35

| A[5] + 1   /    A[5]    |    430

| A[6] - 1   /    A[6]    |    427

| A[8] - 1   /    A[8]    |      8

| MAC[2] - 1 /   MAC[2]   |     24

| MAC[2] + 1 /   MAC[2]   |    516

|                         |

Comput. of the rp from the |    11058   /   11059    |     19

interp. LARp (4.2.9)       |    20069   /   20070    |     25

|                         |

Calc. of the LTP parameters|                         |

->Search of the opt scaling| k= 0 to 38 / k= 0 to 39 |     32

(4.2.11)                   |                         |

->Coding of the LTP gain   |    mult_r  /   mult     |    373

(4.2.11)                   | DLB[0] + 1 /   DLB[0]   |    511

| DLB[1] + 1 /   DLB[1]   |    373

|                         |

ADPCM inverse quantizer    | FAC[2] + 1 /   FAC[2]   |    422

(4.2.16)                   | FAC[3] - 1 /   FAC[3]   |    179

| FAC[4] + 1 /   FAC[4]   |     74

| FAC[5] - 1 /   FAC[5]   |    439

| FAC[5] + 1 /   FAC[5]   |     74

| FAC[6] - 1 /   FAC[6]   |    479

| FAC[6] + 1 /   FAC[6]   |    330

| FAC[7] - 1 /   FAC[7]   |    139

==================================================================

Table 5.5. Errors specially detected by sequence 4/Config 1

------------------------------------------------

================================================================

Test point                  |    Number of

|   occurrences

------------------------------------------------+---------------

Autocorrelation function (4.2.4)               |        8

condition smax == 0                            |

|

Computation of the reflection coefficients :   |

-> condition L_ACF[0] == 0     (4.2.5)         |        8

-> condition P[0] < abs(P[1])  (4.2.5)         |        4

================================================================

Table 5.6. Paths specially explored by sequence 4/Config 1

-----------------------------------------------

=================================================================

Overflow Point                      Nb of occurrences

Sequence 1   Sequence 2    Sequence 3

=================================================================

Long term synthesis filter

(4.3.2) : add                  126            0             0

-----------------------------------------------------------------

Short term synthesis filter

(4.3.4) :

1st add:             4499            0             0

2nd add:              405            1             0

-----------------------------------------------------------------

De-emphasize filter

(4.3.5)  : add                  89            0             0

-----------------------------------------------------------------

Scaling of the output signal

(4.3.6)  : add               16691           339           19

=================================================================

Table 5.7. List of tested overflows points for sequence 1 (decoder

part)

-----

ANNEX 1. CODEC PERFORMANCE

=================

A1.1. INTRODUCTION

------------

The purpose of this annex is to give a broad outline of the

performance of the RPE-LTP codec with other parts of the digital

network. Some general guidance is also offered on non-voice

services.

A1.2. SPEECH PERFORMANCE

------------------

Planning rules for digital processes are defined in terms of

quantizing distortion units (qdu) which can be realized from the

following formula (reference 1) using the assumption that the

formula accuracy represents the determination of qdus from QN

measurements:

QN = 37 - 15 log10(n)      ,where n is the qdu       (A1.1)

By definition 1 qdu is the quantization distortion arising from

one commercial PCM codec.

NOTE: The subjective testing methodology to determine QN for the

RPE-LTP codec was consistent with current CCITT methods

(reference 2).

A1.2.1. Single encoding

---------------

Under error-free transmission conditions the perceived quality of

the RPE-LTP codec (see fig A1.1) is lower than both codecs

conforming to recommendations CCITT G.711 and CCITT G.721.

Table A1.1 indicates the relative performance of the codec and can

be compared with codecs conforming to recommendations CCITT G.711

and CCITT G.721.

The performance of the RPE-LTP codec has been found to be

substantially unaffected down to a carrier to interference (C/I)

ratio of 10 dB, but may be considered to have acceptable

performance down to 7 dB. Smaller C/I ratios produce unacceptable

degradation of speech performance and should be avoided.

NOTE 1: It should be noted that there are doubts as to whether the

simulations which generated the error pattern properly

represent real operating conditions. The C/I values quoted

should therefore only be considered as parameters of this

simulation. They may not correspond to real radio

interference conditions. Results from early GSM validation

hardware show that the C/I values which give the

performance quoted may be several dBs higher. Some error

statistics of the simulations are shown in table A1.2.

NOTE 2: The real condition C/I = 10 dB is believed to correspond

to about 90 % coverage.

============================================================

Codec                         QN (dB)              qdu

============================================================

G.711                          37                   1

(64 kbit/s, A-law PCM)

------------------------------------------------------------

G.721

(32 kbit/s, ADPCM)             29                 3.5 (*)

------------------------------------------------------------

RPE-LTP                       23-25               7-8 (*)

============================================================

(*) Commercial A-law PCM input and output circuitry included.

NOTE: The qdu value for the RPE-LTP codec is a conservative

estimate. At present there are no specific CCITT rules for

determining qdus for encoding below 32 kbit/s.

Table A1.1. Relative levels of speech performance under error-free

conditions

----------

===========================================================

Simulated C/I ratio:         10 dB       7 dB       4 dB

===========================================================

Total number of errors

in class I (182 bits

protected by a 1/2 rate      0.016%       0.61%      4.1%

code)

-----------------------------------------------------------

Total number of errors

in class II (78 bits         4.5%         8.3%       13.0%

unprotected)

-----------------------------------------------------------

Number of "frame erasure"

indications by CRC             1           15         95

-----------------------------------------------------------

Number of "frame erasures"

not detected by CRC            1           14         76

===========================================================

NOTE: The total number of frames was 750. CRC means Cyclic

Redundancy Check.

Table A1.2. Bit error statistics for C/I test conditions

--------------------------------------------

A1.2.2. Speech performance when interconnected with coding systems

on an analogue basis

--------------------

A1.2.2.1. Performance with 32 kbit/s ADPCM (G.721)

----------------------------------------

The speech performance of the RPE-LTP codec when interconnected

with encoding at 32 kbit/s (see fig A1.3 and A1.4) decreases in

accordance with the formula in section A1.2, and appears to obey

the law of additivity when qdus have been determined for the

individual codecs.

A1.2.2.2. Performance with another RPE-LTP codec

--------------------------------------

The speech performance of the RPE-LTP codec when interconnected

with another codec of the same type (see fig A1.2) is lower than

that of A1.2.2.1. It again appears to obey the law of additivity

when qdus have been determined for the individual codecs.

A1.2.2.3. Performance with encoding other than RPE-LTP and 32

kbit/s ADPCM (G.721)

--------------------

No information is available on this point, so great care must be

exercised when interconnection is made to codecs with encoding

different from that of A1.2.2.1 and A1.2.2.2.

A1.3. NON-SPEECH PERFORMANCE

----------------------

It should be noted that the RPE-LTP speech codec is an adaptive

system which has been optimised for speech inputs. Great care must

be taken when making measurements with non-speech signals beause

the normal assumptions of time invariance and linearity cannot be

made.

A.1.3.1. Performance with single sine waves

----------------------------------

Detailed experiments have shown that the RPE-LTP codec will pass

sine waves with segmental signal to noise ratios generally in

excess of 20 dB in the frequency range of 100 - 2000 Hz. However,

in some cases reproduction above 2000 Hz is not as good.

It should be noted that sine waves above 1300 Hz may be reproduced

with significant fluctuations in amplitude and frequency due to

the adaptive sub-sampling technique employed. This results in

irregularities in the measured frequency response.

A typical frequency response measured with A-law PCM input

circuitry is shown in fig A1.5. If 13 bit linear PCM input

circuitry is used, the irregularity is less.

A1.3.2. Performance with DTMF tones

---------------------------

It has been shown that the RPE-LTP codec transfers DTMF signals of

80 ms duration. However, questions like minimum allowable signal

duration, pause duration and the behaviour in the presence of

transmission errors have not been investigated.

A1.3.3. Performance with information tones

----------------------------------

Experiments have shown that network originated signalling tones,

conforming to recommendation CCITT Q.35 (reference 3), are easily

recognizable when passed through the RPE-LTP codec.

A1.3.4. Performance with voice-band data

--------------------------------

Tests have shown that voice-band data transmission does not work

satisfactorily with 1200 bit/s modems according to recommendation

CCITT V.23. Voice-band data according to recommendation CCITT V.21

(300 bit/s) will not be subject to any significant degradation.

This behaviour has been tested for one RPE-LTP link (encoder-decoder). The effect of transmission errors has not been tested.

A1.4. DELAY

-----

The theoretical minimum delay of the RPE-LTP codec is 20 ms.

However, practical realizations may have an additional processing

time in the order of 3 - 8 ms.

Fig A1.5. Frequency response for RPE-LTP codec (with commercial

A-law PCM input and output circuitry)

-------------------------------------

A1.5. REFERENCES

----------

1. CCITT: "Subjective performance assessment of digital processes

using the Modulated Noise Reference Unit (MNRU)", Annex

C, Supplement no 14, Red book, volume V, 1985.

2. CCITT: "Subjective performance assessment of digital processes

using the Modulated Noise Reference Unit (MNRU)", Annex

A, Supplement no 14, Red book, volume V, 1985.

3. CCITT: "Technical characteristics of tones for the telephone

service", recommendation Q.35, Red book, volume VI.1,

1985.

ANNEX 2. SUBJECTIVE RELEVANCE OF THE SPEECH CODER OUTPUT BITS

====================================================

Since no valid objective quality criterion for speech signals is

available, the only way to build up such a relevance table is to

perform listening tests. The procedure described below was used to

obtain the relevance classification given in table A2.1 of the

recommendation.

To classify a single bit, say bit i of parameter k, a short speech

signal (2 sec) was encoded, then this bit was inverted in each

frame (the other bits were left unchanged) and the resulting bit

stream was fed into the speech decoder. The listeners had to

compare the quality of the signal with the quality of six

reference signals with different levels of distortion. Repeating

this procedure for all bits would result in a subdivision of the

260 bits into six relevance classes.

It can be observed that many of the bits have the same physical

meaning and it can be expected that bits with the same meaning

have the same relevance (eg the MSB's of the RPE samples). Relying

on this assumption, only one of the equivalent parameters was

considered. Since there are 13 parameters with different physical

meaning with 56 bits in total, the number of tests is reduced from

260 to 56.

The reference signals were the same speech signal distorted by

inverting one of the six bits of LAR coefficient number one. This

resulted in an adequate quantization of distortion levels ranging

from "not intelligible" (MSB inverted) to "negligible distortion"

(LSB inverted).

The test was carried out using three listeners and one female

speaker. Since the three listeners came to rather similar results,

no more listeners were considered to be required. Averaging the

three outcomes led to the relevance table given in table A2.1,

where the order of all bits between two successive bits of the

first parameter (LAR 1) are arbitrarily chosen.

==================================================================

Importance      Parameter         Parameter         Bit number

class           name              number

==================================================================

1      Log.area ratio 1           1                b6

Block amplitude       12,29,46,63   b53,b109,b165,b221

------------------------------------------------------------------

Log.area ratio 1           1                b5

2      Log.area ratio 2           2                b12

Log.area ratio 3           3                b17

------------------------------------------------------------------

Log.area ratio 1           1                b4

Log.area ratio 2           2                b11

Log.area ratio 3           3                b16

Log.area ratio 4           4                b22

LTP lag                9,26,43,60   b43,b99,b155,b211

3      Block amplitude        12,29,46,63  b52,b108,b164,b220

Log.area ratio 2,5,6     2,5,6          b10,b26,b30

LTP lag                9,26,43,60   b42,b98,b154,b210

LTP lag                9,26,43,60   b41,b97,b153,b209

LTP lag                9,26,43,60   b40,b96,b152,b208

LTP lag                9,26,43,60   b39,b95,b151,b207

------------------------------------------------------------------

Block amplitude        12,29,46,63  b51,b107,b163,b219

Log.area ratio 1           1                b3

Log.area ratio 4           4                b21

Log.area ratio 7           7                b33

4      LTP lag                9,26,43,60   b38,b94,b150,b206

Log.area ratio 5,6        5,6            b25,b29

LTP gain               10,27,44,61  b45,b101,b157,b213

LTP lag                9,26,43,60   b37,b93,b149,b205

Grid position          11,28,45,62  b47,b103,b159,b215

==================================================================

Table A2.1a. Subjective importance of encoded bits (the parameter

and bit numbers refer to table 1.1)

-----------------------------------

==================================================================

Importance      Parameter         Parameter         Bit number

class           name              number

==================================================================

Log.area ratio 1           1                b2

Log.area ratio 2,3,8,4  2,3,8,4       b9,b15,b36,b20

Log.area ratio 5,7        5,7            b24,b32

LTP gain              10,27,44,61   b44,b100,b156,b212

Block amplitude       12,29,46,63   b50,b106,b162,b218

RPE pulses              13..25        b56,b59,..,b92

RPE pulses              30..42      b112,b115,..,b148

RPE pulses              47..59      b168,b171,..,b204

5      RPE pulses              64..76      b224,b227,..,b260

Grid position         11,28,45,62   b46,b102,b158,b214

Block amplitude       12,29,46,63   b49,b105,b161,b217

RPE pulses              13..25        b55,b58,..,b91

RPE pulses              30..42      b111,b114,..,b147

RPE pulses              47..59      b167,b170,..,b203

RPE pulses              64..67     b223,b226,b229,b232

RPE pulses              68..76      b235,b238,..,b259

------------------------------------------------------------------

Log.area ratio 1           1                 b1

Log.area ratio 2,3,6     2,3,6           b8,b14,b28

Log.area ratio 7           7                 b31

Log.area ratio 8           8                 b35

Log.area ratio 8,3        8,3              b34,b13

Log.area ratio 4           4                 b19

6      Log.area ratio 4,5        4,5              b18,b23

Block amplitude       12,29,46,63   b48,b104,b160,b216

RPE pulses              13..25         b54,b57,..,b90

RPE pulses              30..42       b110,b113,..,b146

RPE pulses              47..59       b166,b169,..,b202

RPE pulses              64..76       b222,b225,..,b258

Log.area ratio 2,6        2,6              b7,b27

==================================================================

Table A2.1b. Subjective importance of encoded bits (the parameter

and bit numbers refer to table 1.1)

-----------------------------------

ANNEX 3. FORMAT FOR TEST SEQUENCE DISTRIBUTION

=====================================

A3.1. TYPE OF FILES PROVIDED

----------------------

Three types of files are provided:

- Files for input of the encoder                     : SEQxx.INP

- Files for input of decoder or comparison

with encoder output:                               : SEQxx.COD

- Files for comparison with the decoder output       : SEQxx.OUT

Two diskettes are provided containing all the digital test

sequences.

The 1st flexible disk contains the SEQ01.INP, SEQ01.COD,

SEQ01.OUT, SEQ02.INP, SEQ02.COD, SEQ02.OUT files. The 2nd flexible

disk contains the SEQ03.INP, SEQ03.COD, SEQ03.OUT, SEQ04.INP,

SEQ04.COD, SEQ04.OUT, SEQ05.COD, SEQ05.OUT files. Table A3.1 gives

the contents of the two distribution flexible disks and also the

size in bytes and the number of frames for each test sequence

file.

A3.2. FILE FORMAT DESCRIPTION

-----------------------

All the files are written in binary using 16 bit words. This means

that input samples (sop[k]), output samples (srop[k]) and coded

parameters use 2 bytes each. Hence the sizes of the files are

directly related to the number of processed frames.

For files with .INP or .OUT extension type:

Size (in bytes) = No of frames * 160 * 2 ;

For files with .COD extension type:

Size (in bytes) = No of frames * 76 * 2;

Table A3.1 shows the size of all the files written in direct

binary format.

The diskette is formatted according to the high capacity (1.2 Mb)

specifications for MS/DOS PC-AT compatible computers.

---------------------------------------------------------------

|  DISK No   |      FILE       |    FRAMES     |   SIZE (bytes) |

|------------+-----------------+---------------+----------------|

|            |    SEQ01.INP    |               |   186880       |

|    1       |    SEQ01.COD    |      584      |    88768       |

|            |    SEQ01.OUT    |               |   186880       |

|------------+-----------------+---------------+----------------|

|            |    SEQ02.INP    |               |   303040       |

|    1       |    SEQ02.COD    |      947      |   143944       |

|            |    SEQ02.OUT    |               |   303040       |

|------------+-----------------+---------------+----------------|

|            |    SEQ03.INP    |               |   215360       |

|    2       |    SEQ03.COD    |      673      |   102296       |

|            |    SEQ03.OUT    |               |   215360       |

|------------+-----------------+---------------+----------------|

|            |    SEQ04.INP    |               |   166400       |

|    2       |    SEQ04.COD    |      520      |    79040       |

|            |    SEQ04.OUT    |               |   166400       |

|------------+-----------------+---------------+----------------|

|    2       |    SEQ05.COD    |       64      |     9728       |

|            |    SEQ05.OUT    |               |    20480       |

-------------+-----------------+---------------+----------------

Table A3.1. Contents of diskettes and size of files

---------------------------------------