Recommendation: GSM 06.11

Title: Substitution and muting of lost frames for full-rate speech

traffic channels

Date: February, 1992

List of contents: 0. SCOPE

1. GENERAL

2. REQUIREMENTS

3. EXAMPLE SOLUTION

Language of original: English

Number of pages: 5

Detailed contents list

**********************

0. SCOPE

=====

1. GENERAL

=======

2. REQUIREMENTS

============

2.1. FIRST LOST SPEECH FRAME

2.2. SUBSEQUENT LOST SPEECH FRAMES

2.3. FIRST LOST SID FRAME

2.4. SUBSEQUENT LOST SID FRAMES

3. EXAMPLE SOLUTION

================

0. SCOPE

=====

This recommendation defines a frame substitution and muting

procedure which shall be used by the RX DTX handler when one or

more lost speech or SID frames are received from the radio

subsystem.

The requirements of this recommendation are mandatory for

implementation in all GSM Base Station Systems and Mobile

Stations.

1. GENERAL

=======

The definitions of terms used in this recommendation can be found

in recommendation GSM 06.31.

The purpose of the frame substitution is to conceal the effect of

lost frames.

The purpose of muting the output in the case of several lost

frames is to indicate the breakdown of the channel to the user.

2. REQUIREMENTS

============

2.1. FIRST LOST SPEECH FRAME

-----------------------

Normal decoding of lost speech frames would result in very

unpleasant noise effects. In order to improve the subjective

quality, the first lost speech frame shall be substituted with

either a repetition or an extrapolation of the previous good

speech frame(s). Lost speech frames shall not be delivered to the

speech decoder, nor shall the output be muted directly.

2.2. SUBSEQUENT LOST SPEECH FRAMES

-----------------------------

For subsequent lost speech frames, a muting technique shall be

used that will gradually decrease the output level, resulting in

silencing of the output after a maximum of 320 ms. Section 3 gives

an example solution.

2.3. FIRST LOST SID FRAME

--------------------

A single lost SID frame shall be substituted by the last valid SID

frame and the procedure for valid SID frames be applied as

described in recommendation GSM 06.31.

2.4. SUBSEQUENT LOST SID FRAME

-------------------------

For the second lost SID frame, a muting technique shall be used on

the comfort noise that will gradually decrease the output level,

resulting in silencing of the output after a maximum of 320 ms.

Section 3 gives an example solution.

For subsequent lost SID frames, the muting of the output shall be

maintained.

3. EXAMPLE SOLUTION

================

For guidance, an example solution is given.

The first lost speech frame is replaced at the speech decoder

input by the previous good speech frame. Normal decoding is then

performed.

The muting procedure to be used in the case of subsequent lost

speech frames or for comfort noise frames following the second

lost SID frame is as follows:

The pseudo-logarithmic encoded block amplitude Xmaxcr

(recommendation GSM 06.10), coded on the interval from 0 to 63, is

decreased with a constant value d=4 in each frame, down to the

lowest possible value. Consequently, Xmaxcr will be reduced

gradually, and the output muted after a maximum of 320 ms. The

grid position parameters are chosen randomly between 0 and 3

during this time.

For subsequent unusable frames, after the frame where Xmaxcr

reached the lowest possible value, "silence frames" are passed

from the RX DTX handler to the speech decoder to guarantee a low

output level under all conditions. The silence frame is defined in

table 3-1.

Log area ratio 1 = 42

Log area ratio 2 = 39

Log area ratio 3 = 21

Log area ratio 4 = 10

Log area ratio 5 = 9

Log area ratio 6 = 4

Log area ratio 7 = 3

Log area ratio 8 = 2

_

LTP gain         = 0      |

LTP lag          = 40     |

|

Grid position    = 1      |

Block amplitude  = 0      |

|

RPE pulse no. 1  = 3      |

RPE pulse no. 2  = 4      |

RPE pulse no. 3  = 3      |

RPE pulse no. 4  = 4      |

RPE pulse no. 5  = 4      |

|- repeated for each subsegment

RPE pulse no. 6  = 3      |

RPE pulse no. 7  = 3      |

RPE pulse no. 8  = 3      |

RPE pulse no. 9  = 3      |

RPE pulse no. 10 = 4      |

RPE pulse no. 11 = 4      |

RPE pulse no. 12 = 3      |

RPE pulse no. 13 = 3     _|

Table 3-1. Encoded parameters (recommendation GSM 06.10) of the

silence frame

-------------