Recommendation: GSM 06.12

Title: Comfort noise aspects for full-rate speech traffic channels

Date: February, 1992

List of contents: 0. SCOPE

1. GENERAL

2. FUNCTIONS ON THE TRANSMIT SIDE

3. FUNCTIONS ON THE RECEIVE SIDE

Language of original: English

Number of pages: 6

Detailed list of contents

*************************

0. SCOPE

=====

1. GENERAL

=======

2. FUNCTIONS ON THE TRANSMIT SIDE

==============================

2.1. BACKGROUND ACOUSTIC NOISE EVALUATION

2.2. SID-FRAME ENCODING

3. FUNCTIONS ON THE RECEIVE SIDE

=============================

3.1. COMFORT NOISE GENERATION AND UPDATING.

0. SCOPE

=====

This recommendation gives the detailed requirements for the

correct operation of the background acoustic noise evaluation,

noise parameter encoding/decoding and comfort noise generation in

GSM Mobile Stations and Base Station Systems during Discontinuous

Transmission (DTX) on full rate speech traffic channels.

The requirements described in this recommendation are mandatory

for implementation in all GSM Mobile Stations. The receiver

requirements are mandatory for implementation in all GSM Base

Station Systems, the transmitter requirements only for those where

downlink DTX will be used.

1. GENERAL

=======

The definitions of terms used in this recommendation can be found

in recommendation GSM 06.31.

The overall operation of Discontinuous Transmission is described

in recommendation GSM 06.31.

A basic problem when using DTX is that the background acoustic

noise, which is transmitted together with the speech, would

disappear when the radio transmission is cut, resulting in a

modulation of the background noise. Since the DTX switching can

take place rapidly, it has been found that this effect can be very

annoying for the listener - especially in a car environment with

high background noise levels. In bad cases the speech may be

hardly intelligible.

This recommendation specifies the way to overcome this problem by

generating on the receive side synthetic noise similar to the

transmit side background noise. The parameters of this so called

comfort noise are estimated on the transmit side and transmitted

to the receive side before the radio transmission is cut and at a

regular low rate afterwards. This allows the comfort noise to

adapt to the changes of the noise on the transmit side.

2. FUNCTIONS ON THE TRANSMIT SIDE

==============================

The comfort noise evaluation algorithm uses the unquantized block

amplitude and Log Area Ratio (LAR) parameters of the full rate

speech encoder, defined in section 4.2.15 and 4.2.6 of

recommendation GSM 06.10. These parameters give information on the

level and the spectrum of the background noise, respectively.

The evaluated comfort noise parameters are encoded into a special

frame, called a SID (Silence Descriptor) frame, for transmission

to the receive side.

The SID frame also serves to initiate the comfort noise generation

on the receive side, as a SID frame is always sent at the end of a

speech burst, ie before the radio transmission is cut.

The scheduling of SID or speech frames on the radio path is

described in recommendation GSM 06.31.

2.1. BACKGROUND ACOUSTIC NOISE EVALUATION

------------------------------------

The comfort noise parameters to be encoded into a SID frame are

calculated over N=4 consecutive frames marked with VAD=0, as

follows:

The Log Area Ratio parameters shall be averaged according to the

equation:

mean (LAR(i)) = 1/N SUM [n=1 to N] LAR[j-n](i)      i=1,2..8

where LAR[j](i) is the i'th Log Area Ratio coefficient of the

current frame j and j-n indicates the previous frames.

The block amplitude parameter shall be averaged according to the

equation:

mean (xmax) = 1/(4N) SUM [n=1 to N] SUM [i=1 to 4] xmax[j-n](i)

i = 1,2,..8

where xmax[j](i) is the block amplitude in subsegment i of the

current frame. The SID frame containing these averaged parameters

is passed to the Radio Subsystem instead of frame number j.

2.2. SID-FRAME ENCODING

------------------

The SID-frame encoding algorithm exploits the fact that only some

of the 260 bits in a frame are needed to code the comfort noise

parameters. The other bits can then be used to mark the SID-frame

by means of a fixed bit pattern, called the SID code word.

The log area ratio coefficients are replaced by the mean (LAR(i))

values defined above and encoded as described in recommendation

GSM 06.10.

The block amplitude values are replaced by the mean (xmax) value

defined above, repeated four times inside the frame and encoded as

described in recommendation GSM 06.10.

The SID code word consists of 95 bits which are all zero. The bits

of the SID code word are inserted in the SID field defined as the

positions of those 95 bits of the encoded RPE-pulses Xmc, which

are in the error protection class I (see recommendation GSM 06.10,

table 1.2.a).

The remaining bits in the SID frame are set to zero. The use of

these bits is for further study.

3. FUNCTIONS ON THE RECEIVE SIDE

=============================

The situations in which comfort noise shall be generated on the

receive side are defined in recommendation GSM 06.31.  Generally

speaking, the comfort noise generation is started or updated

whenever a valid SID frame is received.

3.1. COMFORT NOISE GENERATION AND UPDATING

-------------------------------------

The comfort noise generation procedure uses the RPE-LTP speech

decoder algorithm defined in recommendation GSM 06.10.

When comfort noise is to be generated, then the various encoded

parameters are set as follows:

The RPE pulses (Xmcr) are replaced by a locally generated random

integer sequence, uniformly distributed between 1 and 6.

Also the grid position parameters (Mcr) are set to random integer

values, uniformly distributed between 0 and 3.

The LTP gain values (bcr) are set to 0.

The LTP lag values (Ncr) of the 4 subsegments are set to 40, 120,

40 and 120 respectively.

The 4 block amplitude values (Xmaxcr) used are those received in

the SID frame.

The log area ratio parameters (LARcr) used are those received in

the SID frame.

With these parameters, the speech decoder now performs the

standard operations described in recommendation GSM 06.10 and

synthesizes comfort noise.

Updating of the comfort noise parameters occurs each time a valid

SID frame is received, as described in recommendation GSM 06.31.

When updating the comfort noise, the parameters above should

preferably be interpolated over a few frames to obtain smooth

transitions.