Recommendation: GSM 06.31

Title: Discontinuous Transmission (DTX) for full-rate speech

traffic channels

Date: January 1991

List of contents: 0. SCOPE

1. GENERAL

2. TRANSMIT SIDE

3. RECEIVE SIDE

Language of original: English

Number of pages: 13

This page is intentionally left blank !

Detailed contents list

**********************

0. SCOPE

=====

1. GENERAL

=======

1.1. GENERAL ORGANISATION

1.2. NAMING CONVENTION

1.2.1. Definition of general terms

1.2.2. Definition of terms on the receive side

2. TRANSMIT SIDE

=============

2.1. GENERAL OPERATION

2.1.1. Functions of the TX DTX handler

2.1.2. Functions of the TX radio subsystem

3. RECEIVE SIDE

============

3.1. GENERAL OPERATION

3.1.1. Functions of the RX radio subsystem

3.1.2. Functions of the RX DTX handler

0. SCOPE

=====

This recommendation gives a description of the general baseband

operation of full rate speech traffic channels in the transmitter

and in the receiver of GSM Mobile Stations and Base Station

Systems during Discontinuous Transmission (DTX).

For clarity, the description is structured according the block

diagrams in figure 1 and 2. Except in the case described next,

this structure of distributing the various functions between

system entities is not mandatory for implementation, as long as

the operation on the air interface and on the speech decoder

output remains the same.

In the case of Base Station Systems where the speech transcoder is

located remotely in the Base Station Controller, the

implementation of the interfaces between the DTX Handlers and the

Radio Subsystem (RSS) as described in this recommendation together

with all their flags is mandatory, being a part of the A-bis-

interface as described in recommendation GSM 08.60. In this case

the various flags also serve to avoid additional delays.

The DTX functions described in this recommendation are mandatory

for implementation in all GSM Mobile Stations. The receiver

requirements are mandatory for implementation in all GSM Base

Station Systems, the transmitter requirements only for those where

downlink DTX will be used.

DTX shall be in operation in GSM Mobile Stations if commanded so

by the network, see recommendation GSM 04.08.

1. GENERAL

=======

Discontinuous Transmission is a mechanism which allows the radio

transmitter to be switched off most of the time during speech

pauses for the following two purposes:

- to save power in the Mobile Station

- to reduce the overall interference level on the air.

1.1. GENERAL ORGANISATION

--------------------

The overall DTX mechanism described in this recommendation

requires the following functions:

-  a Voice Activity Detector on the transmit side,

-  evaluation of the background acoustic noise on the transmit

side, in order to transmit characteristic parameters to the

receive side,

-  generation on the receive side of a similar noise, called

comfort noise, during periods where the radio transmission is

cut.

The Voice Activity Detector is defined in recommendation GSM 06.32

"Voice Activity Detection",  the comfort noise functions in

recommendation GSM 06.12 "Comfort Noise Aspects". Both are based

partly on the speech transcoder and its internal variables,

defined in recommendation GSM 06.10 "GSM Full Rate Speech

Transcoding".

In addition to these functions, if the parameters arriving at the

receive side are detected to be seriously corrupted by errors, the

speech or comfort noise must be generated from substituted data in

order to avoid seriously annoying effects for the listener. This

function is defined in recommendation GSM 06.11 "Substitution and

Muting of Lost Frames".

An overall description of the speech processing parts can be found

in recommendation GSM 06.01 "Speech processing functions - General

description".

1.2. NAMING CONVENTION

-----------------

This section lists the definitions of terms relevant for the DTX

functions, as used in this and the recommendations mentioned

above.

1.2.1. Definition of general terms

---------------------------

Frame: Time interval of 20 msec. corresponding to the time

segmentation of the full rate speech transcoder (recommendation

GSM 06.10), also used as a short term for a traffic frame.

Traffic frame: Block of 260 information bits (see recommendation

GSM 05.03) transmitted on the full rate speech traffic channel.

SID (Silence Descriptor) frame: Frame characterized by the SID

code word. It conveys information on the acoustic background

noise.

SID code word: Fixed bit pattern defined in recommendation GSM

06.12, for labelling a traffic frame as a SID frame.

SID field: The bit positions defined in recommendation GSM 06.12,

of the SID codeword within a SID frame.

Speech frame: Traffic frame that cannot be classified as a SID

frame.

1.2.2. Definition of terms on the receive side

---------------------------------------

Bad traffic frame: A traffic frame flagged BFI=1 (Bad Frame

Indication) by the Radio Subsystem.

Good traffic frame: A traffic frame flagged BFI=0 by the Radio

Subsystem.

Good speech frame: A good traffic frame which is not an accepted

SID frame.

Accepted SID frame: Traffic frame in which the SID field deviates

in less than 16 bit positions from the SID code word (flag SID=2

or SID=1).

Valid SID frame: Good traffic frame in which the SID field

deviates in less than 2 bit positions from the SID code word (flag

SID=2). This frame is valid for updating of comfort noise

parameters at any time.

Invalid SID frame: Accepted SID frame with BFI=1, or accepted SID

frame with BFI=0, in which the SID field deviates in more than 1

bit position from the SID code word (flag SID=1). This frame is

not valid for updating comfort noise parameters, but the frame

conveys information that comfort noise generations should be

started or continued.

Unusable frame: A bad traffic frame that is not an accepted SID

frame.

Lost SID frame: An unusable frame received when the RX DTX Handler

is generating comfort noise and a SID frame is expected (Time

Alignment Flag, TAF=1).

Lost speech frame: An unusable frame received when the RX DTX

Handler is passing on traffic frames directly to the speech

decoder.

2. TRANSMIT SIDE

=============

A block diagram of the transmit side DTX functions is shown in

fig 2-1 below.

.P.,23 li

2.1. GENERAL OPERATION

-----------------

The TX DTX Handler continuously passes traffic frames,

individually marked by a flag SP, to the Radio Subsystem. This

binary flag is redundant to the SID code word labelling. SP=1

indicates a speech frame, SP=0 a SID frame.

The scheduling of the frames for transmission on the air interface

is controlled by the radio subsystem alone, on the basis of the SP

flag as described next.

2.1.1. Functions of the TX DTX handler

-------------------------------

To allow an exact verification of the TX DTX handler functions, all frames before the reset of the system have to be treated as if there would have been speech frames for an infinitely long time. Therefore, the first N frames after the reset are always marked with SP=1, even if VAD=0 (hangover period, see below).

The Voice Activity Detector must be operating all the time in

order to assess whether the input signal contains speech or not.

The output is a binary flag (VAD=1 or VAD=0, respectively) on a

frame by frame basis (see recommendation GSM 06.32).

The VAD flag controls indirectly, via the TX DTX Handler

operations described below, the overall DTX operation on the

transmit side.

Whenever VAD=1, the speech encoder output frame shall be passed

directly to the radio subsystem, marked with SP=1.

At the end of a speech burst (transition VAD=1 to VAD=0), it takes

N+1 consecutive frames to make a new updated SID frame available (see recommendation GSM 06.12). Normally, the first N speech encoder output frames after the end of the speech burst shall therefore be passed directly to the radio subsystem, marked with SP=1 ("hangover period"). The first new SID

frame is then passed to the RSS as frame N+1 after the end of the

speech burst, marked with SP=0 (see fig 2-2).

.P.,30 li

If, however, at the end of the speech burst, less than 24 frames

have elapsed since the last SID frame was computed and passed to the RSS, then this last last SID frame shall repeatedly be passed to the RSS, until a new updated SID frame is available (N+1 consecutive frames marked with VAD=0).This reduces the activity on the air in cases where short

background noise spikes are taken for speech, by avoiding the

"hangover" waiting for the SID frame computation (see also fig 2-3: Note that fig 2-3 shows as example the longest possible speech burst without hangover).

.P.,30 li

Once the first SID frame after the end of a speech burst has been

computed and passed to the Radio Subsystem, the TX DTX Handler

shall continuously compute and pass updated SID frames to the Radio

Subsystem, marked with SP=0 as long as VAD remains VAD=0.

Consequently, the speech encoder must be operating all the time.

2.1.2. Functions of the TX radio subsystem

-----------------------------------

The following traffic frames shall be scheduled for transmission:

1) all frames marked with SP=1,

2) the first one with SP = 0 after one or more frames with SP=1,

3) those marked with SP=0 and aligned with the SACCH multiframe

structure as described in rec. GSM 05.08.

This has the overall function, that the radio transmission is cut

after the transmission of a SID frame when the speaker stops

talking. During speech pauses the transmission is resumed at

regular intervals for transmission of one SID frame, in order to

update the generated comfort noise on the receive side (and to

improve the measurement of the link quality by the radio

subsystem).

If a SID frame (SP=0), scheduled for transmission is stolen for

signalling (FACCH) purposes, then the subsequent frame shall be

scheduled for transmission instead.

3. RECEIVE SIDE

============

A block diagram of the receive side DTX functions is shown in

fig 3-1 below.

.P.,23 li

3.1. GENERAL OPERATION

-----------------

Whatever their context (speech, SID, FACCH or none), the Radio

Subsystem continuously passes the received traffic frames to the

RX DTX handler, individually marked by various preprocessing

functions with 3 flags. These are the BFI, the SID and the TAF

flags described below, which serve to classify the traffic frame

according to the list of terms defined in section 1.2.2. This

classification, summarized in table 3-1 below, in turn allows the

RX DTX Handler to determine in a simple way how the received frame

is to be handled.

==================================

BFI |              SID

|----------------------------

|      2   |    1   |   0

==================================

0   |    Valid |        | Good

|     SID  |        | speech

|    frame |        | frame

----------------+        +--------

1   |      Invalid      |Unusable

|      SID frame    | frame

==================================

Table 3-1. Classification of traffic frames

--------------------------------

3.1.1. Functions of the RX radio subsystem

-----------------------------------

The binary BFI flag (Bad Frame Indication, see also recommendation

GSM 05.05) indicates whether the traffic frame is considered to

contain meaningful information bits (BFI=0) or not (BFI=1). In the

context of this recommendation, a FACCH frame is considered not to

contain meaningful bits and must also be marked with BFI=1. The

BFI flag must fulfill the performance requirements of

recommendation GSM 05.05, section 6.4.

The ternary SID flag is the output of a SID frame detector, which

compares bit by bit the relevant bits of the received traffic

frame (the SID field) with the SID code word defined in

recommendation GSM 06.12. The flag is coded as follows, where n

designates the number of bit deviations:

SID=2  when n < 2

SID=1  when 2 <= n < 16

SID=0  when n >= 16

The binary TAF flag (Time Alignment Flag) marks with TAF=1 those

traffic frames that are aligned with the SACCH multiframe

structure as described in the recommendations referenced in

section 2.1.2.

3.1.2. Functions of the RX DTX handler

-------------------------------

The RX DTX Handler is responsible for the overall DTX operation on

the receive side, which shall be as follows:

* Whenever a good speech frame is detected, the DTX Handler shall

pass it directly on to the speech decoder.

* When lost speech or lost SID frames are detected, the

substitution and muting procedure defined in recommendation GSM

06.11 shall be applied.

* Valid SID frames shall result in comfort noise generation, as

defined in recommendation GSM 06.12, until the next SID frame is

expected (TAF=1) or good speech frames are detected. During this

period, the RX DTX handler shall ignore any unusable frames

delivered by the Radio Subsystem.

* An invalid SID frame shall be substituted by the last valid SID

frame and the procedure for valid SID frames be applied.

NOTE: If the first SID frame after a speech burst (a series of

good speech frames) is invalid, then the comfort noise

parameters can be taken from the last valid SID frame or

from the last received good speech frame which, because of

the VAD hangover time (see recommendation GSM 06.32), may be

supposed to contain noise only.