ETSI TC GSM

Title : Recommendation GSM 08.04

Mobile Switching Center (MSC) to

Base Station System (BSS) Interface ;

Layer 1 : Structure of Phisical Circuits

Version : 3.0.3

Date : 15 January, 1990

List of Contents:

1. INTRODUCTION

2. LAYER 1 SPECIFICATION

Language of Original :  English

Number of Pages:

1. Introduction

This recommendation defines the structure of the physical layer (layer 1) of the BSS-MSC interface for supporting traffic channels. Use of the physical layer for supporting Signalling System No.7 signalling links is covered in recommendation GSM 08.06.

The physical layer is the lowest layer in the OSI Reference Model and it supports all functions required for transmission of bit streams on the physical medium.

For this recommendation only digital transmission will be considered, the use of analogue transmission is a national concern.

2. Layer 1 Specification

All CCITT recommendations referred to are Red Book.

Layer 1 shall utilise digital transmission at a rate of 2048

kbit/sec with a frame structure of 32 * 64kbit/sec time slots, as specified in recommendation CCITT G.705 section 3.

Therefore the physical/electrical characteristics are defined in CCITT recommendation G.703.

The functional characteristics are defined in recommendation CCITT G.732 section 2 and 3.

Fault conditions should be treated in accordance with recommendation CCITT G.732 section 4.

Speech encoding shall be the A-law as defined in recommendation CCITT G.711.

The idle pattern must be transmitted on every timeslot that is not assigned to a channel, and to every timeslot of a channel that is not allocated to a call. The idle pattern shall be 01010100.

Synchronisation at the BSS for the transmitted 2048 kbit/sec bit stream shall be derived from the received 2048 kbit/sec bit stream.

Data encoding is covered in recommendation GSM 08.20.

Note :	A predetermined number of the 64kbit/sec time slots may be used for signalling, to one or more base station systems.