European digital cellular telecommunications system (Phase 2);

Mobile Application Part (MAP) specification

(GSM 09.02)

Contents

.Begin Table C.

Foreword	19

1	General	21

1.1	Scope	21

1.2	Normative references	21

1.3	Definitions and abbreviations	27

2	Configuration of the mobile network	28

2.1	The entities of the mobile system	28

2.1.1	The Home Location Register (HLR)	28

2.1.2	The Visitor Location Register (VLR)	28

2.1.3	The Mobile-services Switching Centre (MSC)	29

2.1.4	The Base Station System (BSS)	29

2.1.5	The Gateway MSC (GMSC)	29

2.1.6	The SMS Gateway MSC	29

2.1.7	The SMS Interworking MSC	29

2.1.8	The Equipment Identity Register (EIR)	29

2.2	Configuration of a Public Land Mobile Network (PLMN)	30

2.3	Interconnection between PLMNs	30

2.4	The interfaces within the mobile service	30

2.4.1	Interface between the HLR and the VLR (D-interface)	30

2.4.2	[Spare]	30

2.4.3	Interface between the VLR and its associated MSC(s) (B-interface)	30

2.4.4	Interface between VLRs (G-interface)	30

2.4.5	Interface between the HLR and the MSC (C-interface)	31

2.4.6	Interface between MSCs (E-interface)	31

2.4.7	Interface between the MSC and Base Station Systems (A-interface)	31

2.4.8	Interface between MSC and EIR (F-interface)	31

2.5	Splitting of the data storage	31

3	Overload and compatibility overview	33

3.1	Overload control	33

3.1.1	Overload control for MSC (outside MAP)	33

3.1.2	Overload control for MAP entities 	33

3.1.3	Congestion control for Signalling System No. 7	36

3.2	Compatibility	36

3.2.1	General	36

3.2.2	Strategy for selecting the Application Context (AC) version	36

3.2.2.1	Proposed method	36

3.2.2.2	Managing the version look-up table	37

3.2.2.3	Optimizing the method	38

4	Requirements concerning the use of SCCP and TC	39

4.1	Use of SCCP	39

4.1.1	SCCP Class	39

4.1.2	Sub-system number (SSN)	39

4.1.3	SCCP addressing	39

4.1.3.1	Introduction	39

4.1.3.2	The Mobile-services Switching Centre (MSC)	40

4.1.3.2.1	MSC interaction during handover	40

4.1.3.2.2	MSC for short message routing	40

4.1.3.3	The Home Location Register (HLR)	40

4.1.3.3.1	During Call Set-up	40

4.1.3.3.2	Location updating	41

4.1.3.3.3	VLR requests and responses	41

4.1.3.4	The Visitor Location Register (VLR)	41

4.1.3.4.1	Local MSC's transactions	41

4.1.3.4.2	HLR request	41

4.1.3.5	The Interworking MSC for short message service	41

4.1.3.6	The Equipment Identity Register (EIR)	41

4.1.3.7	Summary table	42

4.2	Use of TC	43

5	General on MAP services	45

5.1	Terminology and definitions	45

5.2	Modelling principles	45

5.3	Common MAP services	46

5.3.1	MAP-OPEN service	47

5.3.2	MAP-CLOSE service	50

5.3.3	MAP-DELIMITER service	50

5.3.4	MAP-U-ABORT service	51

5.3.5	MAP-P-ABORT service	52

5.3.6	MAP-NOTICE service	54

5.4	Sequencing of services	54

5.5	General rules for mapping of services onto TC	56

5.5.1	Mapping of common services	56

5.5.2	Mapping of user specific services	58

5.6	Definition of parameters	58

5.6.1	Common parameters	60

5.6.1.1	Invoke Id	60

5.6.1.2	Linked Id	60

5.6.1.3	Provider error	60

5.6.1.4	User error	60

5.6.2	Numbering and identification parameter	64

5.6.2.1	IMSI	64

5.6.2.2	TMSI	64

5.6.2.3	IMEI	64

5.6.2.4	Previous location area Id	64

5.6.2.5	Stored location area Id	64

5.6.2.6	Current location area Id	64

5.6.2.7	Target location area Id	64

5.6.2.8	Target cell Id	64

5.6.2.10	Originating entity number	64

5.6.2.11	MSC number	64

5.6.2.12	Target MSC number	64

5.6.2.13	HLR number	64

5.6.2.14	VLR number	65

5.6.2.15	HLR Id	65

5.6.2.16	LMSI	65

5.6.2.17	MS ISDN	65

5.6.2.18	OMC Id	65

5.6.2.19	Roaming number	65

5.6.2.20	[Spare]	65

5.6.2.21	Handover number	65

5.6.2.22	Forwarded-to number	65

5.6.2.23	Forwarded-to subaddress	65

5.6.2.24	Called number	65

5.6.2.25	Calling number	65

5.6.2.26	Originally dialled number	66

5.6.2.27	Service centre address	66

5.6.2.28	Zone Code	66

5.6.2.29	MSIsdn-Alert	66

5.6.3	Subscriber management parameters	67

5.6.3.1	Category	67

5.6.3.2	Equipment status	67

5.6.3.3	Bearer service	67

5.6.3.4	Teleservice	67

5.6.3.5	Basic Service Group	67

5.6.3.6	GSM bearer capability	67

5.6.3.7	Subscriber status	67

5.6.3.8	CUG outgoing access indicator	67

5.6.3.9	Operator Determined Barring General Data	67

5.6.3.10	ODB HPLMN Specific Data	68

5.6.3.11	Regional Subscription Data	68

5.6.3.12	Regional Subscription Response	68

5.6.3.13	Roaming Restriction Due To Unsupported Feature	68

5.6.4	Supplementary services parameters	69

5.6.4.1	SS code	69

5.6.4.2	SS-status	69

5.6.4.3	SS data	69

5.6.4.4	Override Category	69

5.6.4.5	CLI Restriction Option	70

5.6.4.6	Forwarding Options	70

5.6.4.7	No reply condition timer	70

5.6.4.8	CUG info	70

5.6.4.9	CUG subscription	70

5.6.4.10	CUG interlock	70

5.6.4.11	CUG index	70

5.6.4.12	CUG Feature	71

5.6.4.13	Inter CUG options	71

5.6.4.14	Intra CUG restrictions	71

5.6.4.15	Forwarding information	71

5.6.4.16	Forwarding feature	72

5.6.4.17	Forwarding data	72

5.6.4.18	Call barring information	72

5.6.4.19	Call barring feature	72

5.6.4.20	New password	72

5.6.4.21	Current password	72

5.6.4.22	Guidance information	73

5.6.4.23	[Spare]	73

5.6.4.24	SS info	73

5.6.4.25-5.6.4.35 [Spare]	73

5.6.4.36	USSD Data Coding Scheme	73

5.6.4.37	USSD String	73

5.6.5	Call parameters	74

5.6.5.1	Call reference	74

5.6.6	Radio parameters	74

5.6.6.1-5.6.6.6 [Spare]	74

5.6.6.7	HO-Number Not Required	74

5.6.7	Authentication parameters	74

5.6.7.1	Authentication set list	74

5.6.7.2	Rand	74

5.6.7.3	Sres	74

5.6.7.4	Kc	74

5.6.7.5	[Spare]	74

5.6.7.6	Cksn	74

5.6.7.7	Ciphering mode	74

5.6.8	Short message parameters	75

5.6.8.1	SM-RP-DA	75

5.6.8.2	SM-RP-OA	75

5.6.8.3	MWD status	75

5.6.8.4	SM-RP-UI	75

5.6.8.5	SM-RP-PRI	75

5.6.8.6	SM Delivery outcome	75

5.6.8.7	More Messages To Send	75

5.6.8.8	Alert Reason	76

5.6.9	Access and signalling system related parameters	76

5.6.9.1	BSS-apdu	76

5.6.9.2	CM service type	76

5.6.9.3	Access connection status	76

5.6.9.4	External Signal Information	76

5.6.9.5	Access signalling information	76

5.6.9.6	Location update type	76

5.6.9.7	Protocol ID	77

5.6.9.8	Network signal information	77

5.6.10	System operations parameters	78

5.6.10.1	Network resources	78

5.6.10.2	Trace reference	78

5.6.10.3	Trace type	78

5.7	Representation of a list of a basic parameter in service-primitives	78

6	Mobility services	79

6.1	Location Management Services	79

6.1.1	MAP_UPDATE_LOCATION_AREA service	79

6.1.1.1	Definition	79

6.1.1.2	Service primitives	79

6.1.1.3	parameter definitions and use	79

6.1.2	MAP_UPDATE_LOCATION service	81

6.1.2.1	Definition	81

6.1.2.2	Service primitives	81

6.1.2.3	Parameter definitions and use	81

6.1.3	MAP_CANCEL_LOCATION service	83

6.1.3.1	Definition	83

6.1.3.2	Service primitives	83

6.1.3.3	Parameter definitions and use	83

6.1.4	MAP_SEND_IDENTIFICATION service	84

6.1.4.1	Definition	84

6.1.4.2	Service primitives	84

6.1.4.3	Parameter definitions and use	84

6.1.5	MAP_DETACH_IMSI service	85

6.1.5.1	Definition	85

6.1.5.2	Service primitives	85

6.1.5.3	Parameter definitions and use	85

6.1.6	MAP_PURGE_MS service	86

6.1.6.1	 Definition	86

6.1.6.2	Service primitives	86

6.1.6.3	Parameter definitions and use	86

6.2	Paging and Search	87

6.2.1	MAP_PAGE service	87

6.2.1.1	Definition	87

6.2.1.2	Service primitives	87

6.2.1.3	Parameter definitions and use	87

6.2.2	MAP_SEARCH_FOR_MS service	89

6.2.2.1	Definition	89

6.2.2.2	Service primitives	89

6.2.2.3	Parameter definitions and use	89

6.3	Access Management Services 	91

6.3.1	MAP_PROCESS_ACCESS_REQUEST service	91

6.3.1.1	Definition	91

6.3.1.2	Service primitives	91

6.3.1.3	Parameter definitions and use	91

6.4	Handover Services	93

6.4.1	MAP_PREPARE_HANDOVER service	93

6.4.1.1	Definition	93

6.4.1.2	Service primitives	93

6.4.1.3	Parameter use	93

6.4.2	MAP_SEND_END_SIGNAL service	95

6.4.2.1	Definition	95

6.4.2.2	Service primitives	95

6.4.2.3	Parameter use	95

6.4.3	MAP_PROCESS_ACCESS_SIGNALLING service	96

6.4.3.1	Definition	96

6.4.3.2	Service primitives	96

6.4.3.3	Parameter use	96

6.4.4	MAP_FORWARD_ACCESS_SIGNALLING service	97

6.4.4.1	Definition	97

6.4.4.2	Service primitives	97

6.4.4.3	Parameter use	97

6.4.5	MAP_PREPARE_SUBSEQUENT_HANDOVER service	98

6.4.5.1	Definition	98

6.4.5.2	Service primitives	98

6.4.5.3	Parameter use	98

6.4.6	MAP_ALLOCATE_HANDOVER_NUMBER service	99

6.4.6.1	Definition	99

6.4.6.2	Service primitives	99

6.4.6.3	Parameter use	99

6.4.7	MAP_SEND_HANDOVER_REPORT service	100

6.4.7.1	Definition	100

6.4.7.2	Service primitives	100

6.4.7.3	Parameter use	100

6.5	Authentication management services	101

6.5.1	MAP_AUTHENTICATE service	101

6.5.1.1	Definition	101

6.5.1.2	Service primitives	101

6.5.1.3	Parameter use	101

6.5.2	MAP_SEND_AUTHENTICATION_INFO service	102

6.5.2.1	Definition	102

6.5.2.2	Service primitives	102

6.5.2.3	Parameter use	102

6.6	Security management services	103

6.6.1	MAP_SET_CIPHERING_MODE service	103

6.6.1.1	Definitions	103

6.6.1.2	Service primitives	103

6.6.1.3	Parameter use	103

6.7	International mobile equipment identities management services	104

6.7.1	MAP_CHECK_IMEI service	104

6.7.1.1	Definition	104

6.7.1.2	Service primitives	104

6.7.1.3	Parameter use	104

6.7.2	MAP_OBTAIN_IMEI service	106

6.7.2.1	Definition	106

6.7.2.2	Service primitives	106

6.7.2.3	Parameter use	106

6.8	Subscriber management services	107

6.8.1	MAP-INSERT-SUBSCRIBER-DATA service	107

6.8.1.1	Definition	107

6.8.1.2	Service primitives	107

6.8.1.3	Parameter use	108

6.8.2	MAP-DELETE-SUBSCRIBER-DATA service	112

6.8.2.1	Definition	112

6.8.2.2	Service primitives	112

6.8.2.3	Parameter use	112

6.9	Identity management services	114

6.9.1	MAP-PROVIDE-IMSI service	114

6.9.1.1	Definition	114

6.9.1.2	Service primitives	114

6.9.1.3	Parameter use	114

6.9.2	MAP-FORWARD-NEW-TMSI service	115

6.9.2.1	Definition	115

6.9.2.2	Service primitives	115

6.9.2.3	Parameter use	115

6.10	Fault recovery services	116

6.10.1	MAP_RESET service	116

6.10.1.1	Definition	116

6.10.1.2	Service primitives	116

6.10.1.3	Parameter Definition and Use	116

6.10.2	MAP_FORWARD_CHECK_SS_INDICATION service	117

6.10.2.1	Definition	117

6.10.2.2	Service primitives	117

6.10.2.3	Parameter Definition and Use	117

6.10.3	MAP_RESTORE_DATA service	118

6.10.3.1	Definition	118

6.10.3.2	Service primitives	118

6.10.3.3	Parameter definitions and use	118

7	Operation and Maintenance services	120

7.1	Subscriber tracing services	120

7.1.1	MAP-ACTIVATE-TRACE-MODE service	120

7.1.1.1	Definition	120

7.1.1.2	Service primitives	120

7.1.1.3	Parameter usa	120

7.1.2	MAP-DEACTIVATE-TRACE-MODE service	122

7.1.2.1	Definition	122

7.1.2.2	Service primitives	122

7.1.2.3	Parameter usa	122

7.1.3	MAP-TRACE-SUBSCRIBER-ACTIVITY service	123

7.1.3.1	Definition	123

7.1.3.2	Service primitives	123

7.1.3.3	Parameter usa	123

7.2	Other operation and maintenance services	124

7.2.1	MAP-SEND-IMSI service	124

7.2.1.1	Definition	124

7.2.1.2	Service primitives	124

7.2.1.3	Parameter usa	124

8	Call handling services	125

8.1	MAP_SEND_INFO_FOR_INCOMING_CALL service	125

8.1.1	Definition	125

8.1.2	Service primitives	125

8.1.3	Parameter use	125

8.2	MAP_SEND_INFO_FOR_OUTGOING_CALL Service	128

8.2.1	Definition	128

8.2.2	Service primitives	128

8.2.3	Parameter use	128

8.3	MAP_SEND_ROUTING_INFORMATION service	130

8.3.1	Definition	130

8.3.2	Service primitives	130

8.3.3	Parameter use	130

8.4	MAP_PROVIDE_ROAMING_NUMBER service	133

8.4.1	Definition	133

8.4.2	Service primitives	133

8.4.3	Parameter use	133

8.5	MAP_COMPLETE_CALL service	135

8.5.1	Definition	135

8.5.2	Service primitives	135

8.5.3	Parameter use	135

8.6	MAP_PROCESS_CALL_WAITING service	137

8.6.1	Definition	137

8.6.2	Service primitives	137

8.6.3	Parameter use	137

9	Supplementary services related services	139

9.1	MAP_REGISTER_SS service	139

9.1.1	Definition	139

9.1.2	Service primitives	139

9.1.3	Parameter use	139

9.2	MAP_ERASE_SS service	141

9.2.1	Definition	141

9.2.2	Service primitives	141

9.2.3	Parameter use	141

9.3	MAP_ACTIVATE_SS service	143

9.3.1	Definition	143

9.3.2	Service primitives	143

9.3.3	Parameter use	143

9.4	MAP_DEACTIVATE_SS service	145

9.4.1	Definitions	145

9.4.2	Service primitives	145

9.4.3	Parameter use	145

9.5	MAP_INTERROGATE_SS service	147

9.5.1	Definitions	147

9.5.2	Service primitives	147

9.5.3	Parameter use	147

9.6	MAP_INVOKE_SS service	149

9.6.1	Definitions	149

9.6.2	Service primitives	149

9.6.3	Parameter use	149

9.7	MAP_REGISTER_PASSWORD service	150

9.7.1	Definitions	150

9.7.2	Service primitives	150

9.7.3	Parameter use	150

9.8	MAP_GET_PASSWORD service	151

9.8.1	Definitions	151

9.8.2	Service primitives	151

9.8.3	Parameter use	151

9.9	MAP_PROCESS_UNSTRUCTURED_SS_REQUEST_SERVICE	152

9.9.1	Definitions	152

9.9.2	Service primitives	152

9.9.3	Parameter use	152

9.10	MAP_UNSTRUCTURED_SS_REQUEST service	154

9.10.1	Definitions	154

9.10.2	Service primitives	154

9.10.3	Parameter use	154

9.11	MAP_UNSTUCTURED_SS_NOTIFY service	156

9.11.1	Definitions	156

9.11.2	Service primitives	156

9.11.3	Parameter use	156

10	Short message service management services	158

10.1	MAP-SEND-ROUTING-INFO-FOR-SM service	158

10.1.1	Definition	158

10.1.2	Service primitives	158

10.1.3	Parameter use	158

10.2	MAP-FORWARD-SHORT-MESSAGE service	160

10.2.1	Definition	160

10.2.2	Service primitives	160

10.2.3	Parameter use	160

10.3	MAP-REPORT-SM-DELIVERY-STATUS service	162

10.3.1	Definition	162

10.3.2	Service primitives	162

10.3.3	Parameter use	162

10.4	MAP-READY-FOR-SM service	164

10.4.1	Definition	164

10.4.2	Service primitives	164

10.4.3	Parameter use	164

10.5	MAP-ALERT-SERVICE-CENTRE service	166

10.5.1	Definition	166

10.5.2	Service primitives	166

10.5.3	Parameter use	166

10.6	MAP-INFORM-SERVICE-CENTRE service	167

10.6.1	Definition	167

10.6.2	Service primitives	167

10.6.3	Parameter use	167

10.7	MAP-SEND-INFO-FOR-MT-SMS service	168

10.7.1	Definition	168

10.7.2	Service primitives	168

10.7.3	Parameter use	168

10.8	MAP-SEND-INFO-FOR-MO-SMS service	169

10.8.1	Definition	169

10.8.2	Service primitives	169

10.8.3	Parameter use	169

11	General	170

11.1	Overview	170

11.2	Underlying services	170

11.3	Model	170

11.4	Conventions	170

12	Elements of procedure	171

12.1	Dialogue establishment	171

12.1.1	Receipt of a MAP-OPEN request primitive	171

12.1.2	Receipt of a TC-BEGIN indication	172

12.1.3	Receipt of a MAP-OPEN response	176

12.1.4	Receipt of the first TC-CONTINUE ind	176

12.1.5	Receipt of a TC-END ind	176

12.1.6	Receipt of a TC-U-ABORT ind	177

12.1.7	Receipt of a TC-P-ABORT ind	177

12.2	Dialogue continuation	177

12.2.1	Sending entity	177

12.2.2	Receiving entity	177

12.3	Dialogue termination	178

12.3.1	Receipt of a MAP-CLOSE request	178

12.3.2	Receipt of a TC-END indication	178

12.4	User Abort	178

12.4.1	MAP-U-ABORT request	178

12.4.2	TC-U-ABORT ind	178

12.5	Provider Abort	179

12.5.1	MAP PM error situation	179

12.5.2	TC-P-ABORT ind	179

12.5.3	TC-U-ABORT ind	179

12.6	Procedures for MAP specific services	179

12.6.1	Service invocation	180

12.6.2	Service invocation receipt	180

12.6.3	Service response	181

12.6.4	Receipt of a response	182

12.6.4.1	Receipt of TC-RESULT-NL indication	182

12.6.4.2	Receipt of TC-RESULT-L indication	182

12.6.4.3	Receipt of TC-U-ERROR indication	183

12.6.4.4	Receipt of a TC-INVOKE indication	183

12.6.4.5	Receipt of TC-U-REJECT indication	184

12.6.4.6	Receipt of a TC-L-REJECT indication	184

12.6.4.7	Receipt of a TC-L-CANCEL indication	184

12.6.5	Other events	185

12.6.5.1	Receipt of a TC-U-REJECT	185

12.6.5.2	Receipt of a TC-R-REJECT indication	185

12.6.5.3	Receipt of a TC-L-REJECT indication	185

12.6.6	Parameter checks	185

12.6.7	Returning state machines to idle	186

12.6.8	Load control	186

13	Mapping on to TC services	187

13.1	Dialogue control	187

13.1.1	Directly mapped parameters	187

13.1.2	Use of other parameters of dialogue handling primitives	187

13.1.2.1	Dialogue Id	187

13.1.2.2	Application-context-name	187

13.1.2.3	User information	187

13.1.2.4	Component present	187

13.1.2.5	Termination	187

13.1.2.6	P-Abort-Cause	187

13.1.2.7	Quality of service	188

13.2	Service specific procedures	188

13.2.1	Directly mapped parameters	188

13.2.2	Use of other parameters of component handling primitives	188

13.2.2.1	Dialogue Id	188

13.2.2.2	Class	188

13.2.2.3	Linked Id	189

13.2.2.4	Operation	189

13.2.2.5	Error	191

13.2.2.6	Parameters	191

13.2.2.7	Time out	191

13.2.2.8	Last component	191

13.2.2.9	Problem code	192

13.2.2.9.1	Mapping to MAP user-error	192

13.2.2.9.2	Mapping to MAP provider-error parameter	192

13.2.2.9.3	Mapping to diagnostic parameter	193

14	Abstract syntax of the MAP protocol	221

14.1	General	221

14.1.1	Encoding Rules	221

14.1.2	Use of TC	221

14.1.3	Use of information elements defined outside MAP	222

14.1.4	Compatibility considerations	222

14.1.5	Structure of the Abstract Syntax of MAP	223

14.2	Operation packages	226

14.2.1	General aspects	226

14.2.2	Packages specifications	227

14.2.2.1	Location updating	227

14.2.2.2	Location cancellation	227

14.2.2.3	Roaming number enquiry	227

14.2.2.4	Information retrieval	228

14.2.2.5	Inter-VLR information retrieval	228

14.2.2.6 - 14.2.2.9 [spare]	228

14.2.2.10	Interrogation	228

14.2.2.11	[spare]	228

14.2.2.12	Handover Control	229

14.2.2.13	Subscriber Data management stand alone	229

14.2.2.14	Equipment management	229

14.2.2.15	Subscriber Data management	230

14.2.2.16	Location register restart	230

14.2.2.17	Tracing stand-alone	230

14.2.2.18	Functional SS handling	230

14.2.2.19	Tracing	231

14.2.2.20	Binding	231

14.2.2.21	Unstructured SS handling	231

14.2.2.22	Short message relay services	232

14.2.2.23	Short message gateway services	232

14.2.2.24 - 14.2.2.25 [spare]	232

14.2.2.26	Message waiting data management	232

14.2.2.27	Alerting	233

14.2.2.28	Data restoration	233

14.2.2.29	Purging	233

14.3	Application contexts	234

14.3.1	General aspects	234

14.3.2	Access-context definitions	235

14.3.2.1	[spare]	235

14.3.2.2	Location Updating	235

14.3.2.3	Location Cancellation	235

14.3.2.4	Roaming number enquiry	236

14.3.2.5	[spare]	236

14.3.2.6	Location Information Retrieval	236

14.3.2.7 - 14.3.2.10 [spare]	236

14.3.2.11	Location registers restart	236

14.3.2.12	Handover control	237

14.3.2.13	[spare]	237

14.3.2.14	Equipment Management	237

14.3.2.15	Information retrieval	237

14.3.2.16	Inter-VLR information retrieval	238

14.3.2.17	Stand Alone Subscriber Data Management	238

14.3.2.18	Tracing	238

14.3.2.19	Network functional SS handling	239

14.3.2.20	Network unstructured SS handling	239

14.3.2.21	Short Message Gateway	239

14.3.2.22	Mobile originating Short Message Relay	240

14.3.2.23	[spare]	240

14.3.2.24	Short message alert	240

14.3.2.25	Short message waiting data management	240

14.3.2.26	Mobile terminating Short Message Relay	241

14.3.2.27	MS purging	241

14.3.3	ASN.1 Module for application-context-names	241

14.4	MAP Dialogue Information	244

14.5	MAP Operation and Error codes	246

14.6	MAP Operation and Error Types	252

14.6.1	Mobile Service Operations	252

14.6.2	Operation and Maintenance Operations	258

14.6.3	Call Handling Operations	260

14.6.4	Supplementary Service Operations	262

14.6.5	Short Message Service Operations	267

14.6.6	Errors	270

14.7	MAP Constants and Data Types	275

14.7.1	Mobile Service Data Types	275

14.7.2	Operation and Maintenance Data Types	282

14.7.3	Call Handling Data Types	284

14.7.4	Supplementary Service Data Types	286

14.7.5	Supplementary Service Codes	292

14.7.6	Short Message Data Types	295

14.7.7	Error Data Types	298

14.7.8	Common Data Types	300

14.7.9	Teleservice Codes	305

14.7.10	Bearer Service Codes	307

15	General on MAP user procedures	309

15.1	Introduction	309

15.2	Common aspects of user procedure descriptions	309

15.2.1	General conventions	309

15.2.2	Naming conventions	309

15.2.3	Convention on primitives parameters	311

15.2.3.1	Open service	311

15.2.3.2	Close service	311

15.2.4	Version handling at dialogue establishment	311

15.2.4.1	Behaviour at the initiating side	311

15.2.4.2	Behaviour at the responding side	312

15.2.5	Abort Handling	312

15.2.6	SDL conventions	312

15.3	Interaction between MAP Provider and MAP Users	313

16	Mobility procedures	314

16.1	Location management Procedures	314

16.1.1	Location updating	319

16.1.1.1	General	319

16.1.1.2	Detailed procedure in the MSC	323

16.1.1.3	Detailed procedure in the VLR	327

16.1.1.4	Detailed procedure in the HLR	340

16.1.1.5	Send Identification	344

16.1.1.5.1	General	344

16.1.1.5.2	Detailed procedure in the VLR	344

16.1.1.5.3	Detailed procedure in the PVLR	344

16.1.1.6	The Process Update Location VLR	346

16.1.1.7	The Process Subscriber Present HLR	348

16.1.2	Location Cancellation	350

16.1.2.1	General	350

16.1.2.2	Detailed procedure in the HLR	350

16.1.2.3	Detailed procedure in the VLR	351

16.1.3	Detach IMSI	354

16.1.3.1	General	354

16.1.3.2	Detailed procedure in the MSC	354

16.1.3.3	Detailed procedure in the VLR	355

16.1.4	Purge MS	358

16.1.4.1	General	358

16.1.4.2	Detailed procedure in the VLR	358

16.1.4.3	Detailed procedure in the HLR	359

16.2	Handover procedure	362

16.2.1	General	362

16.2.2	Handover procedure in MSC-A	366

16.2.2.1	Basic handover	366

16.2.2.2	Handling of access signalling	366

16.2.2.3	Other procedures in stable handover situation	366

16.2.2.4	Subsequent handover	367

16.2.2.5	SDL Diagrams	367

16.2.3	Handover procedure in MSC-B	380

16.2.3.1	Basic handover	380

16.2.3.2	Allocation of handover number	380

16.2.3.3	Handling of access signalling	380

16.2.3.4	Other procedures in stable handover situation	380

16.2.3.5	Subsequent handover	380

16.2.3.6	SDL Diagrams	381

16.2.4	Handover error handling macro	393

16.2.5	Handover procedure in VLR	395

16.2.5.1	Allocation of handover number	395

16.2.5.2	SDL Diagrams	395

16.3	Fault recovery procedures	398

16.3.1	VLR fault recovery procedures	398

16.3.2	HLR fault recovery procedures	401

16.3.3	VLR restoration: the restore data procedure in the HLR	406

16.4	Macro Insert_Subs_Data_Framed_HLR	408

17	Operation and maintenance procedures	410

17.1	General	410

17.1.1	Tracing Co-ordinator for the VLR	410

17.1.2	Subscriber Data Management Co-ordinator for the VLR	412

17.2	Tracing procedures	414

17.2.1	Procedures in the HLR	416

17.2.1.1	Subscriber tracing activation procedure	416

17.2.1.2	Subscriber tracing deactivation procedure	419

17.2.2	Procedures in the VLR	422

17.2.2.1	Subscriber tracing activation procedure	422

17.2.2.2	Subscriber tracing deactivation procedure	424

17.2.2.3	Subscriber tracing procedure	426

17.2.3	Procedures in the MSC	426

17.2.3.1	Subscriber tracing procedure	426

17.3	Subscriber Data Management procedures	426

17.3.1	Procedures in the HLR	427

17.3.1.1	Subscriber deletion procedure	427

17.3.1.2	Subscriber data modification procedure	429

17.3.2	Procedures in the VLR	432

17.3.2.1	Subscriber deletion procedure	432

17.3.2.2	Subscriber data modification procedure	432

17.4	Subscriber Identity procedure	435

17.4.1	Subscriber identity procedure in the HLR	435

17.4.2	Subscriber identity procedure in the VLR	437

18	Call handling procedures	439

18.1	General	439

18.2	Retrieval of routing information	446

18.2.1	General	446

18.2.2	Procedures in the MSC	447

18.2.3	Procedures in the HLR	451

18.2.4	Procedures in the VLR	458

18.3	MS terminating calls	465

18.3.1	General	465

18.3.2	Procedures in the MSC	466

18.3.3	Procedures in the VLR	481

18.4	MS originated calls	493

18.4.1	General	493

18.4.2	Procedures in the MSC	495

18.4.3	Procedures in the VLR	500

19	Supplementary services procedures	505

19.0.1	Functional Supplementary Service Process Co-ordinator for MSC	505

19.0.2	Functional Supplementary Service Process Co-ordinator for VLR	507

19.0.3	Functional Supplementary Service Process Co-ordinator for HLR	510

19.1	Registration procedure	513

19.1.1	General	513

19.1.2	Procedures in the MSC	514

19.1.3	Procedures in the VLR	516

19.1.4	Procedures in the HLR	519

19.2	Erasure procedure	522

19.2.1	General	522

19.2.2	Procedures in the MSC	522

19.2.3	Procedures in the VLR	523

19.2.4	Procedures in the HLR	523

19.3	Activation procedure	524

19.3.1	General	524

19.3.2	Procedures in the MSC	525

19.3.3	Procedures in the VLR	527

19.3.4	Procedures in the HLR	530

19.4	Deactivation procedure	533

19.4.1	General	533

19.4.2	Procedures in the MSC	534

19.4.3	Procedures in the VLR	534

19.4.4	Procedures in the HLR	534

19.5	Interrogation procedure	535

19.5.1	General	535

19.5.2	Procedures in the MSC	536

19.5.3	Procedures in the VLR	536

19.5.4	Procedures in the HLR	540

19.6	Invocation procedure	540

19.6.1	General	540

19.6.2	Procedures in the MSC	541

19.6.3	Procedures in the VLR	544

19.7	Register password procedure	546

19.7.1	General	546

19.7.2	Procedures in the MSC	547

19.7.3	Procedures in the VLR	547

19.7.4	Procedures in the HLR	547

19.8	Mobile Initiated USSD procedure	550

19.8.1	General	550

19.8.2	Procedures in the MSC	550

19.8.3	Procedures in the VLR	554

19.8.4	Procedures in HLR	559

19.9	Network initiated ussd procedure	562

19.9.1	General	562

19.9.2	Procedure in MSC	562

19.9.3	Procedure in VLR	567

19.9.4	Procedure in HLR	574

19.10	Common macros for section 19	577

19.10.1	SS Password handling macros	577

19.10.2	SS Error handling macros	580

20	Short Message Service Procedures	584

20.1	General	584

20.1.1	Mobile originated short message service Co-ordinator for the MSC	584

20.1.2	Short message Gateway Co-ordinator for the HLR	586

20.2	The mobile originated short message transfer procedure	588

20.2.1	Procedure in the servicing MSC	589

20.2.2	Procedure in the VLR	593

20.2.3	Procedure in the interworking MSC	595

20.3	The mobile terminated short message transfer procedure	597

20.3.1	Procedure in the Servicing MSC	599

20.3.2	Procedures in the VLR	608

20.3.3	Procedures in the HLR	612

20.3.4	Procedures in the gateway MSC	616

20.4	The Short Message Alert procedure	624

20.4.1	Procedures in the Servicing MSC	627

20.4.2	Procedures in the VLR	629

20.4.2.1	The Mobile Subscriber is present	629

20.4.2.2	The Mobile Equipment has memory available	629

20.4.3	Procedures in the HLR	631

20.4.4	Procedures in the Interworking MSC	633

20.5	The SM delivery status report procedure	635

20.5.1	Procedures in the HLR	635

20.5.2	Procedures in the gateway MSC	637

20.6	Common procedures for the short message section	639

20.6.1	The macro Report_SM_Delivery_Stat_HLR	639

21	General macro description	641

21.1	MAP open macros	641

21.1.1	Macro Receive_Open_Ind	641

21.1.2	Macro Receive_Open_Confirmation	642

21.2	Macros to check the content of indication and confirmation primitives	646

21.2.1	Macro Check_Indication	646

21.2.2	Macro Check_Confirmation	646

21.3	The Page macros	649

21.3.1	Macro PAGE_MSC	649

21.3.2	Macro Search_for_ms_MSC	650

21.4	Macros for handling an Access Request	654

21.4.1	Macro Process_Access_Request_MSC	654

21.4.2	Macro Process_Access_Request_VLR	660

21.4.3	Macro Identification Procedure	662

21.5	Authentication macros and processes	667

21.5.1	Macro Authenticate_MSC	667

21.5.2	Macro Authenticate_VLR	667

21.5.3	Process Obtain_Authentication_Sets_VLR	667

21.5.4	Macro Obtain_Authent_Para_VLR	668

21.5.5	Process Obtain_Auth_Sets_HLR	669

21.6	IMEI Handling Macros	676

21.6.1	Macro Check_IMEI_MSC	676

21.6.2	Macro Check_IMEI_VLR	676

21.6.3	Process Check_IMEI_EIR	677

21.6.4	Macro Obtain_IMEI_MSC	677

21.6.5	Macro Obtain_IMEI_VLR	678

21.7	Insert Subscriber Data Macros	685

21.7.1	Macro Insert_Subs_Data_VLR	685

21.7.2	Process Insert_Subs_Data_Stand_Alone_HLR	687

21.7.3	Macro Wait_for_Insert_Subs_Data_Cnf	690

21.7.4	Process Send_Insert_Subs_Data	692

21.8	Request IMSI Macros	694

21.8.1	Macro Obtain_IMSI_MSC	694

21.8.2	Macro Obtain_IMSI_VLR	696

21.9	Tracing macros	698

21.9.1	Macro Trace_Subscriber_Activity_MSC	698

21.9.2	Macro Trace_Subscriber_Activity_VLR	700

21.9.3	Macro Activate_Tracing_VLR	702

21.9.4	Macro Control_Tracing_HLR	704

21.10	Short Message Alert procedures	706

21.10.1	Subscriber_Present_VLR process	706

21.10.2	Macro Alert_Service_Centre_HLR	708

21.11	Co-ordinating process in the HLR to handle messages sent with the application context information retrieval	711

Annex A (informative): Cross-reference for abstract syntaxes of MAP	714

Annex B (informative): Fully expanded ASN.1 sources for abstract syntaxes of MAP	752

B.1	Fully Expanded ASN.1 Source of MAP-Protocol/TCAPMessages	752

B.2	Fully Expanded ASN.1 Source of MAP-DialogueInformation	778

! ! !    based upon version 4.6.0    ! ! !	778

Annex C (informative): Formal protocol incompatibilities between versions 1 & 2 of MAP	781

C.1	Introduction	781

C.2	Deletion of operations and errors	781

C.2.1	Deletion of operation RegisterChargingInfo	781

C.2.2	Deletion of operation ForwardSS-Notification	781

C.2.3	Deletion of error InsufficientBearerCapabilities	781

C.3	Deletion of errors for operations	781

C.3.1	Error NegativePW-Check for operation RegisterSS	781

C.3.2	Error NegativePW-Check for operation EraseSS	781

C.3.3	Error NegativePW-Check for operation InterrogateSS	781

C.3.4	Errors SS-SubscriptionViolation, SS-Incompatibility and CUG-Reject for operation InvokeSS	782

C.3.5	Error CUG-Reject for operation SendRoutingInfoForSM	782

C.4	Changes to definitions of data types	782

C.4.1	CUG-Feature	782

C.4.2	CUG-FeatureList	782

C.4.3	CUG-Info	782

C.4.4	CUG-RejectCause	782

C.4.5	IMSI	782

C.4.6	ISDN-AddressString	782

C.4.7	Password	783

C.4.8	RequestParameter	783

C.4.9	RequestParameterList	783

C.4.10	SentParameter	783

C.4.11	SentParameterList	783

C.4.12	SS-Info	783

C.4.13	SS-InfoList	783

C.4.14	SubscriberData	783

C.5	Changes to parameters of errors	783

C.5.1	CUG-Reject	783

C.6	Changes to parameters of operations	784

C.6.1	InvokeSS	784

C.6.2	InsertSubscriberData	784

C.6.3	RegisterSS	784

C.6.4	SendInfoForIncomingCall	784

C.6.5	SendInfoForOutgoingCall	784

C.6.6	SendParameters	784

C.6.7	SendRoutingInfoForSM	784

C.7	Changes to results of operations	785

C.7.1	ActivateSS	785

C.7.2	DeactivateSS	785

C.7.3	EraseSS	785

C.7.4	GetPassword	785

C.7.5	InterrogateSS	785

C.7.6	RegisterSS	785

C.7.7	SendParameters	786

C.7.8	SendRoutingInfoForSM	786

C.8	Changes to errors of operations	786

C.8.1	SendInfoForOutgoingCall	786

C.8.2	SendRoutingInfo	786

History	787

.End Table C.

Blank page

Foreword

This section will be provided later.

This European Telecommunication Standard (ETS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI) and is now submitted for the Public Enquiry (PE) phase of the ETSI standards approval procedure.

This draft ETS specifies the Mobile Application Part (MAP), the requirements for the signalling system and procedures within the European digital cellular telecommunications system (Phase 2) at application level.

The introduction to this ETS gives an overview of the four sections of this ETS.

This draft ETS corresponds to GSM Technical Specification (GSM-TS) GSM 09.02 version 4.7.0.

The specification from which this draft ETS has been derived was originally based on CEPT documentation, hence the presentation of this draft ETS may not be entirely in accordance with the ETSI/PNE rules. As a consequence, this draft ETS may undergo further editorial changes during the PE phase.

Reference is made within this draft ETS to GSM Technical Specifications (GSM-TSs)(NOTE).

NOTE:	TC-SMG has produced documents which give the technical specifications for the implementation of the European digital cellular telecommunications system. Historically, these documents have been identified as GSM Technical Specifications (GSM-TSs). These TSs may have subsequently become I-ETSs (Phase 1), or ETSs (Phase 2), whilst others may become ETSI Technical Reports (ETRs). GSM-TSs are, for editorial reasons, still referred to in current GSM ETSs.

Blank page

1	General

1.1	Scope

It will be necessary to transfer between entities of a Public Land Mobile Network (PLMN) information specific to the PLMN in order to deal with the specific behaviour of roaming Mobile Stations. The Signalling System No. 7 specified by CCITT is used to transfer this information.

This Technical Specification describes the requirements for the signalling system and the procedures needed at the application level in order to fulfil these signalling needs.

Sections 1 to 4 are related to general aspects such as terminology, mobile network configuration and other protocols required by MAP.

MAP consists of a set of MAP services which are provided to MAP service-users by a MAP service-provider.

┌────────────────────────┐          ┌───────────────────────┐

│    MAP service-user    │          │    MAP service-user   │

└────────────┬───────────┘          └───────────┬───────────┘

│                                  │

─┼─       Service Interface        ─┼─

│                                  │

┌────────────┴──────────────────────────────────┴───────────┐

│                    MAP service-provider                   │

└───────────────────────────────────────────────────────────┘

Figure 1.1/1: Modelling principles

Sections 5 to 10 of this Technical Specification describe the MAP services.

Sections 11 to 14 define the MAP protocol specification and the behaviour of service provider (protocol elements to be used to provide MAP services, mapping on to TC service primitives, abstract syntaxes...).

Sections 15 to 21 describe the MAP user procedures which make use of MAP services.

1.2	Normative references

This ETS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this ETS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 100): "European digital cellular telecommunications system (Phase 2); Abbreviations and acronyms".

[2]	GSM 02.01 (prETS 300 500): "European digital cellular telecommunications system (Phase 2); Principles of telecommunication services supported by a GSM Public Land Mobile Network (PLMN)".

[3]	GSM 02.02 (prETS 300 501): "European digital cellular telecommunications system (Phase 2); Bearer Services (BS) Supported by a GSM Public Land Mobile Network (PLMN)".

[4]	GSM 02.03 (prETS 300 502): "European digital cellular telecommunications system (Phase 2); Teleservices Supported by a GSM Public Land Mobile Network (PLMN)".

[5]	GSM 02.04 (prETS 300 503): "European digital cellular telecommunications system (Phase 2); General on supplementary services".

[6]	GSM 02.09 (prETS 300 506): "European digital cellular telecommunications system (Phase 2); Security aspects".

[7]	GSM 02.16 (prETS 300 508): "European digital cellular telecommunications system (Phase 2); International Mobile station Equipment Identities (IMEI)".

[8]	GSM 02.41 (prETS 300 513): "European digital cellular telecommunications system (Phase 2); Operator determined barring".

[9]	GSM 02.81 (prETS 300 514): "European digital cellular telecommunications system (Phase 2); Line identification supplementary services - Stage 1".

[10]	GSM 02.82 (prETS 300 515): "European digital cellular telecommunications system (Phase 2); Call Forwarding (CF) supplementary services - Stage 1".

[11]	GSM 02.83 (prETS 300 516): "European digital cellular telecommunications system (Phase 2); Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 1".

[12]	GSM 02.84 (prETS 300 517): "European digital cellular telecommunications system (Phase 2); Multi Party (MPTY) supplementary services - Stage 1".

[13]	GSM 02.85 (prETS 300 518): "European digital cellular telecommunications system (Phase 2); Closed User Group (CUG) supplementary services - Stage 1".

[14]	GSM 02.86 (prETS 300 519): "European digital cellular telecommunications system (Phase 2); Advice of charge (AoC) supplementary services - Stage 1".

[15]	GSM 02.88 (prETS 300 520): "European digital cellular telecommunications system (Phase 2); Call Barring (CB)  supplementary services - Stage 1".

[16]	GSM 02.90 (prETS 300 521): "European digital cellular telecommunication system (Phase 2); Unstructured supplementary services operation - Stage 1".

[17]	GSM 03.03 (prETS 300 523): "European digital cellular telecommunications system (Phase 2); Numbering, addressing and identification".

[18]	GSM 03.04 (prETS 300 524): "European digital cellular telecommunications system (Phase 2); Signalling requirements relating to routeing of calls to mobile subscribers".

[19]	GSM 03.07 (prETS 300 525): "European digital cellular telecommunications system (Phase 2); Restoration procedures".

[20]	GSM 03.08 (prETS 300 526): "European digital cellular telecommunications system (Phase 2); Organisation of subscriber data".

[21]	GSM 03.09 (prETS 300 527): "European digital cellular telecommunications system (Phase 2); Handover procedures".

[22]	GSM 03.11 (prETS 300 529): "European digital cellular telecommunications system (Phase 2); Technical realization of supplementary services".

[23]	GSM 03.12 (prETS 300 530): "European digital cellular telecommunications system (Phase 2); Location registration procedures".

[24]	GSM 03.20 (prETS 300 534): "European digital cellular telecommunications system (Phase 2); Security related network functions".

[25]	GSM 03.40 (prETS 300 536): "European digital cellular telecommunications system (Phase 2); Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[26]	GSM 03.41 (prETS 300 537): "European digital cellular telecommunications system (Phase 2); Technical realization of the Short Message Service Cell Broadcast (SMSCB)".

[27]	GSM 03.81 (prETS 300 542): "European digital cellular telecommunications system (Phase 2); Line identification supplementary services - Stage 2".

[28]	GSM 03.82 (prETS 300 543): "European digital cellular telecommunications system (Phase 2); Call Forwarding (CF) supplementary services - Stage 2".

[29]	GSM 03.83 (prETS 300 544): "European digital cellular telecommunications system (Phase 2); Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 2".

[30]	GSM 03.84 (prETS 300 545): "European digital cellular telecommunications system (Phase 2); Multi Party (MPTY) supplementary services - Stage 2".

[31]	GSM 03.85 (prETS 300 546): "European digital cellular telecommunications system (Phase 2); Closed User Group (CUG) supplementary services - Stage 2".

[32]	GSM 03.86 (prETS 300 547): "European digital cellular telecommunications system (Phase 2); Advice of Charge (AoC) supplementary services - Stage 2".

[33]	GSM 03.88 (prETS 300 548): "European digital cellular telecommunications system (Phase 2); Call Barring (CB) supplementary services - Stage 2".

[34]	GSM 03.90 (prETS 300 549): "European digital cellular telecommunications system (Phase 2); Unstructured supplementary services operation - Stage 2".

[35]	GSM 04.08 (prETS 300 557): "European digital cellular telecommunications system (Phase 2); Mobile radio interface layer 3 specification".

[36]	GSM 04.10 (prETS 300 558): "European digital cellular telecommunications system (Phase 2); Mobile radio interface layer 3  Supplementary services specification  General aspects".

[37]	GSM 04.11 (prETS 300 559): "European digital cellular telecommunications system (Phase 2); Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[38]	GSM 04.80 (prETS 300 564): "European digital cellular telecommunications system (Phase 2); Mobile radio interface layer 3 supplementary services specification  Formats and coding".

[39]	GSM 04.81 (prETS 300 565): "European digital cellular telecommunications system (Phase 2); Line identification supplementary services - Stage 3".

[40]	GSM 04.82 (prETS 300 566): "European digital cellular telecommunications system (Phase 2); Call Forwarding (CF) supplementary services - Stage 3".

[41]	GSM 04.83 (prETS 300 567): "European digital cellular telecommunications system (Phase 2); Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 3".

[42]	GSM 04.84 (prETS 300 568): "European digital cellular telecommunications system (Phase 2); Multi Party (MPTY) supplementary services - Stage 3".

[43]	GSM 04.85 (prETS 300 569): "European digital cellular telecommunications system (Phase 2); Closed User Group (CUG) supplementary services - Stage 3".

[44]	GSM 04.86 (prETS 300 570): "European digital cellular telecommunications system (Phase 2); Advice of Charge (AoC) supplementary services - Stage 3".

[45]	GSM 04.88 (prETS 300 571): "European digital cellular telecommunications system (Phase 2); Call Barring (CB) supplementary services - Stage 3".

[46]	GSM 04.90 (prETS 300 572): "European digital cellular telecommunications system (Phase 2); Unstructured supplementary services operation - Stage 3".

[47]	GSM 08.02 (prETS 300 587-2): "European digital cellular telecommunications system (Phase 2); Base Station System - Mobile-services Switching Centre (BSS - MSC) interface  Interface principles".

[48]	GSM 08.06 (prETS 300 589): "European digital cellular telecommunications system (Phase 2); Signalling transport mechanism specification for the Base Station System - Mobile-services Switching Centre (BSS - MSC) interface".

[49]	GSM 08.08 (prETS 300 590): "European digital cellular telecommunications system (Phase 2); Mobile Switching Centre - Base Station System (MSC - BSS) interface  Layer 3 specification".

[50]	GSM 09.01 (ETR 109): "European digital cellular telecommunications system (Phase 2); General network interworking scenarios".

[51]	GSM 09.02: "European digital cellular telecommunications system (Phase 1); Mobile Application Part (MAP) specification".

[52]	GSM 09.03 (prETS 300 600): "European digital cellular telecommunications system (Phase 2); Signalling requirements on interworking between the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) and the Public Land Mobile Network (PLMN)".

[53]	GSM 09.04 (prETS 300 601): "European digital cellular telecommunications system (Phase 2); Interworking between the Public Land Mobile Network (PLMN) and the Circuit Switched Public Data Network (CSPDN)".

[54]	GSM 09.05 (prETS 300 602): "European digital cellular telecommunications system (Phase 2); Interworking between the Public Land Mobile Network (PLMN) and the Packet Switched Public Data Network (PSPDN) for Packet Assembly/Disassembly facility (PAD) access".

[55]	GSM 09.06 (prETS 300 603): "European digital cellular telecommunications system (Phase 2);  Interworking between a Public Land Mobile Network (PLMN) and a Packet Switched Public Data Network/Integrated Services Digital Network (PSPDN/ISDN) for the support of packet switched data transmission services".

[56]	GSM 09.07 (prETS 300 604): "European digital cellular telecommunications system (Phase 2); General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)".

[57]	GSM 09.08 (prETS 300 604): "European digital cellular telecommunications system (Phase 2); Application of the Base Station System Application Part (BSSAP) on the E-interface".

[58]	GSM 09.10 (prETS 300 605): "European digital cellular telecommunications system (Phase 2); Information element mapping between Mobile Station - Base Station System and BSS ─ Mobile-services Switching Centre (MS - BSS - MSC)  Signalling procedures and the Mobile Application Part (MAP)".

[59]	GSM 09.11 (prETS 300 606): "European digital cellular telecommunications system (Phase 2); Signalling interworking for supplementary services".

[60]	GSM 09.90 (ETR 111): "European digital cellular telecommunications system (Phase 2); Interworking between Phase 1 infrastructure and Phase 2 Mobile Stations (MS)".

[61]	GSM 12.02 (prETS 300 613): "European digital cellular telecommunications system (Phase 2); Subscriber, Mobile Equipment (ME) and services data administration".

[62]	ETS 300 102-1 (1990): "Integrated Services Digital Network (ISDN); User-network interface layer 3 specifications for basic call control".

[63]	ETS 300 136 (1992): "Integrated Services Digital Network (ISDN); Closed User Group (CUG) supplementary service description".

[64]	ETS 300 138 (1992): "Integrated Services Digital Network (ISDN); Closed User Group (CUG) supplementary service Digital Subscriber Signalling System No.one (DSS1) protocol".

[65]	ETS 300 287: "Integrated Services Digital Network (ISDN); CCITT Signalling System No.7 Transaction Capabilities Application Part (TCAP)(version 2)".

[66]	ETR 060: "Signalling Protocols and Switching (SPS); Guide-lines for using Abstract Syntax Notation One (ASN.1) in telecommunication application protocols".

[67]	CCITT Recommendation E.164: "Numbering plan for the ISDN era".

[68]	CCITT Recommendation E.212: "Identification plan for land mobile stations".

[69]	CCITT Recommendation E.213: "Telephone and ISDN numbering plan for land mobile stations".

[70]	CCITT Recommendation E.214: "Structuring of the land mobile global title for the signalling connection control part".

[71]	CCITT Recommendation Q.669: "Interworking between the Digital Subscriber Signalling System Layer 3 protocol and the Signalling System No.7 ISDN  User part".

[72]	CCITT Recommendation Q.711: "Specifications of Signalling System No.7; Functional description of the signalling connection control part".

[73]	CCITT Recommendation Q.712: "Definition and function of SCCP messages".

[74]	CCITT Recommendation Q.713: "Specifications of Signalling System No.7; SCCP formats and codes".

[75]	CCITT Recommendation Q.714: "Specifications of Signalling System No.7; Signalling connection control part procedures".

[76]	CCITT Recommendation Q.716: "Specifications of Signalling System No.7; Signalling connection control part (SCCP) performances".

[77]	CCITT Recommendation Q.721 (1988): "Specifications of Signalling System No.7; Functional description of the Signalling System No.7 Telephone user part".

[78]	CCITT Recommendation Q.722 (1988): "Specifications of Signalling System No.7; General function of Telephone messages and signals".

[79]	CCITT Recommendation Q.723 (1988): "Specifications of Signalling System No.7; Formats and codes".

[80]	CCITT Recommendation Q.724 (1988): "Specifications of Signalling System No.7; Signalling procedures".

[81]	CCITT Recommendation Q.725 (1988): "Specifications of Signalling System No.7; Signalling performance in the telephone application".

[82]	CCITT Recommendation Q.761 (1988): "Specifications of Signalling System No.7; Functional description of the ISDN user part of Signalling System No.7".

[83]	CCITT Recommendation Q.762 (1988): "Specifications of Signalling System No.7; General function of messages and signals".

[84]	CCITT Recommendation Q.763 (1988): "Specifications of Signalling System No.7; Formats and codes".

[85]	CCITT Recommendation Q.764 (1988): "Specifications of Signalling System No.7; Signalling procedures".

[86]	CCITT Recommendation Q.767: "Specifications of Signalling System No.7; Application  of the ISDN user part of CCITT signalling System No.7 for international ISDN interconnections".

[87]	CCITT Recommendation Q.771: "Specifications of Signalling System No.7; Functional description of transaction capabilities".

[88]	CCITT Recommendation Q.772: "Specifications of Signalling System No.7; Transaction capabilities information element definitions".

[89]	CCITT Recommendation Q.773: "Specifications of Signalling System No.7; Transaction capabilities formats and encoding".

[90]	CCITT Recommendation Q.774: "Specifications of Signalling System No.7; Transaction capabilities procedures".

[91]	CCITT Recommendation Q.775: "Specifications of Signalling System No.7; Guide-lines for using transaction capabilities".

[92]	CCITT Recommendation X.200: "Reference Model of Open systems interconnection for CCITT Applications".

[93]	CCITT Recommendation X.208 (1988): "Specification of Abstract Syntax Notation One (ASN.1)".

[94]	CCITT Recommendation X.209 (1988): "Specification of basic encoding rules for Abstract Syntax Notation One (ASN.1)".

[95]	CCITT Recommendation X.210: "Open systems interconnection layer service definition conventions".

1.3	Definitions and abbreviations

Abbreviations used in this specification are listed in GSM 01.04.

2	Configuration of the mobile network

2.1	The entities of the mobile system

To provide the mobile service as it is defined, it is necessary to introduce some specific functions. These functional entities can be implemented in different equipments or integrated. In any case, exchanges of data occur between these entities.

2.1.1	The Home Location Register (HLR)

This functional entity is a data base in charge of the management of mobile subscribers. A PLMN may contain one or several HLRs; it depends on the number of mobile subscribers, on the capacity of the equipment and on the organization of the network. All subscription data are stored there. The main information stored there concerns the location of each mobile station in order to be able to route calls to the mobile subscribers managed by each HLR. All management interventions occur on this data base. The HLRs have no direct control of MSCs.

Two numbers attached to each mobile subscription are stored in the HLR:

-	IMSI;

-	MSISDN.

The data base contains other information such as:

-	location information (VLR number);

-	basic telecommunication services subscription information;

-	service restrictions (e.g. roaming limitation);

-	supplementary services; the tables contain the parameters attached to these services.

The organization of the subscriber data is detailed in TS GSM 03.08.

2.1.2	The Visitor Location Register (VLR)

An MS roaming in an MSC area is controlled by the Visitor Location Register in charge of this area. When an MS appears in a location area it starts a location updating procedure. The MSC in charge of that area notices this registration and transfers to the Visitor Location Register the identity of the location area where the MS is situated. A VLR may be in charge of one or several MSC areas.

The VLR also contains the information needed to handle the calls set up or received by the MSs registered in its data base (in some cases the VLR may have to obtain additional information from the HLR); the following elements can be found in its tables:

-	the IMSI;

-	the MSISDN;

-	the TMSI, if applicable;

-	the location area where the mobile station has been registered. This will be used to call the station;

-	supplementary service parameters.

The information is passed between VLR and HLR by the procedures described in TS GSM 03.12.

The organization of the subscriber data is detailed in TS GSM 03.08.

2.1.3	The Mobile-services Switching Centre (MSC)

The Mobile-services Switching Centre is an exchange which performs all the switching functions for mobile stations located in a geographical area designated as the MSC area. The main difference between an MSC and an exchange in a fixed network is that the MSC has to take into account the impact of the allocation of radio resources and the mobile nature of the subscribers and has to perform, for example, the following procedures:

-	procedures required for the location registration (see TS GSM 03.12);

-	procedures required for hand-over (see TS GSM 03.09).

2.1.4	The Base Station System (BSS)

The BSS is the sub-system of Base Station equipment (transceivers, controllers, etc...) which is viewed by the MSC through a single interface (A-interface) with the functionality described in TS GSM 08.02.

2.1.5	The Gateway MSC (GMSC)

In the case of incoming calls to the PLMN, if the fixed network is unable to interrogate the HLR, the call is routed to an MSC. This MSC will interrogate the appropriate HLR and then route the call to the MSC where the mobile station is located. The MSC which then performs the routing function to the actual location of the mobile is called the Gateway MSC.

The choice of which MSCs can act as Gateway MSCs is a network operator matter (e.g. all MSCs or some designated MSCs).

See also TS GSM 03.04.

2.1.6	The SMS Gateway MSC

The SMS GMSC is the interface between the Mobile Network and the network which provides access to the Short Message Service Centre, for short messages to be delivered to mobile stations.

The choice of which MSCs can act as SMS Gateway MSCs is a network operator matter (e.g. all MSCs or some designated MSCs).

2.1.7	The SMS Interworking MSC

The SMS IWMSC is the interface between the Mobile Network and the network which provides access to the Short Message Service Centre, for short messages submitted by mobile stations.

The choice of which MSCs can act as SMS Interworking MSCs is a network operator matter (e.g. all MSCs or some designated MSCs).

2.1.8	The Equipment Identity Register (EIR)

This functional unit is a data base in charge of the management of the equipment identities of the mobile stations; see also TS GSM 02.16.

2.2	Configuration of a Public Land Mobile Network (PLMN)

The basic configuration of a Public Land Mobile Network is presented in figure 2.2/1. In this figure the most general solution is described in order to define all the possible interfaces which can be found in any PLMN. The specific implementation in each network may be different: some particular functions may be implemented in the same equipment and then some interfaces may become internal interfaces. In any case the configuration of a PLMN must have no impact on the relationship with the other PLMNs.

In this configuration, all the functions are considered implemented in different equipments. Therefore, all the interfaces are external and need the support of the Mobile Application Part of the Signalling System No. 7 to exchange the data necessary to support the mobile service. From this configuration, all the possible PLMN organizations can be deduced.

2.3	Interconnection between PLMNs

Since the configuration of a PLMN does not have any impact on other PLMNs, the signalling interfaces specified can be implemented both between the entities within a PLMN and between different PLMNs.

However, this interface is not fully operational specified. It is strongly recommended not to implement the B-interface as an external interface.

2.4	The interfaces within the mobile service

2.4.1	Interface between the HLR and the VLR (D-interface)

This interface is used to exchange the data related to the location of the mobile station and to the management of the subscriber. The main service provided to the mobile subscriber is the capability to set up or to receive calls within the whole service area. To support that purpose the location registers have to exchange data. The VLR informs the HLR on the registration of a mobile station managed by the latter and provides it with the relevant location information. The HLR sends to the VLR all the data needed to support the service to the mobile station. The HLR then calls the previous VLR to inform it that it can cancel the location registration of this station because of the roaming of the mobile.

Exchanges of data may also occur when the mobile subscriber requires a particular service, when he wants to change some data attached to his subscription or when some parameters of the subscription are modified by administrative means.

2.4.2	[Spare]

2.4.3	Interface between the VLR and its associated MSC(s) (B-interface)

The VLR is the location and management data base for the mobile stations roaming in the area controlled by the associated MSC(s). Whenever the MSC needs data related to a given mobile station currently located in its area, it interrogates the VLR. When a mobile station initiates a location updating procedure with an MSC, the MSC informs its VLR which stores the relevant information in its tables. This procedure occurs whenever a mobile roams to another location area. Also, for instance when a subscriber activates a specific supplementary service or modifies some data attached to a service, the MSC transfers (via the VLR) the request to the HLR, which stores these modifications and updates the VLR if required.

2.4.4	Interface between VLRs (G-interface)

When an MS initiates a location updating using TMSI, the VLR can fetch the IMSI and authentication set from the previous VLR.

2.4.5	Interface between the HLR and the MSC (C-interface)

When the fixed network is not able to perform the interrogation procedure needed to set up a call to a mobile subscriber, the Gateway MSC has to interrogate the HLR of the called subscriber to obtain the roaming number of the called MS (see TS GSM 03.04).

To forward a short message to a mobile subscriber, the SMS Gateway MSC has to interrogate the HLR to obtain the MSC number where the MS is located.

2.4.6	Interface between MSCs (E-interface)

When a mobile station moves from one MSC area to another during a call, a handover procedure has to be performed in order to continue the communication. For that purpose the MSCs involved have to exchange data to initiate and then to realize the operation.

This interface is also used to forward short messages.

2.4.7	Interface between the MSC and Base Station Systems (A-interface)

The description of this interface is contained in the 08-series of GSM Technical Specifications.

The BSS-MSC interface carries information concerning:

-	BSS management;

-	call handling;

-	location management.

2.4.8	Interface between MSC and EIR (F-interface)

This interface is used when an MSC wants to check an IMEI.

2.5	Splitting of the data storage

The data attached to each MS management, operation and location are stored in the Location Registers. Some data are duplicated in the HLR and in the VLR, but others may be stored only in one place.

A detailed description of the data organization can be found in TS GSM 03.08.

.G.C:\USER\MAP\MAP-470\EPS02\RESEAU.EPS;16 cm;22,691 cm;EPS

(to be printed in POSTSCRIPT, not available in any other format)

Figure 2.2/1: Configuration of a PLMN

3	Overload and compatibility overview

3.1	Overload control

There is a requirement for an overload/congestion control for all entities of the Public Land Mobile Network and the underlying Signalling System No. 7.

3.1.1	Overload control for MSC (outside MAP)

For the entity MSC the following two procedures (outside MAP) may be applied to control the processor load:

-	ISDN

CCITT Recommendation Q.764 (Automatic Congestion Control, ACC), applicable to reduce the mobile terminating traffic;

-	BSSAP

TS GSM 08.08 (A-interface Flow Control), applicable to reduce the mobile originating traffic.

3.1.2	Overload control for MAP entities

For all MAP entities, especially the HLR, the following overload control method is applied:

If overload of a MAP entity is detected requests for certain MAP operations (see tables 3.1/1 and 3.1/2) may be ignored by the responder. The decision as to which MAP Operations may be ignored is made by the MAP service provider and is based upon the priority of the application context.

Since most of the affected MAP operations are supervised in the originating entity by TC timers (medium) an additional delay effect is achieved for the incoming traffic.

If overload levels are applicable in the Location Registers the MAP operations should be discarded taking into account the priority of their application context (see table 3.1/1 for HLR and table 3.1/2 for MSC/VLR; the lowest priority is discarded first).

The ranking of priorities given in the tables 3.1/1 and 3.1/2 is not normative. The tables can only be seen as a proposal which might be changed due to network operator/implementation matters.

Responder = HLR 	Initiating Entity

Priority

high

Mobility Management

networkLocUp	VLR

(updateLocation/v2),

(restoreData/v2),

(sendParameters/v1)

infoRetrieval 	VLR

(sendAuthenticationInfo/v2),

(sendIMSI/v2),

(sendParameters /v1)

MsPurging 	VLR

(purgeMS/v2)

Short Message Service

shortMsgGateway 	GMSC

(sendRoutingInfoforSM),

(reportSM-DeliveryStatus)

mwdMngt 	VLR

(alertServiceCentre/v2),

(readyForSM/v2),

(noteSubscriberPresent/v1)

Mobile Terminating Traffic

locationInfoRetrieval 	GMSC

(sendRoutingInfo)

Subscriber Controlled Inputs (Suppl. Services)

networkFunctionalSs 	VLR

(eraseSS),

(deactivateSS),

(interrogateSS),

(processUnstructuredSS-Data/v1),

(beginSubscriberActivity/v1)

networkUnstructuredSs 	VLR

(processSS-Request/v2),

(registerSS/v1),

(eraseSS/v1),

(activateSS/v1),

(deactivateSS/v1),

(registerPassword/v1),

(interrogateSS/v1),

(processUnstructuredSS-Data/v1),

(beginSubscriberActivity/v1)

Priority

low

NOTE:	Application context name is the last but one component of the object identifier.

Operation names are given in brackets with "/vn" appended to vn only operations.

Table 3.1/1: Priorities of Application Contexts for HLR as Responder

Responder = MSC/VLR	Initiating Entity

Priority

high

Handover

handoverControl 	MSC

(performHandover),

(forwardAccessSignalling),

(traceSubscriberActivity)

Mobility and Location Register Management

locationCancellation 	HLR

(cancelLocation)

reset        	HLR

(reset)

interVlrInfoRetrieval 	VLR

(sendIdentification)

subscriberDataMngt 	HLR

(insertSubscriberData),

(deleteSubscriberData)

tracing     	HLR

(activateTraceMode),

(deactivateTraceMode)

Short Message Service

shortMsgMO-Relay 	MSC

(forwardSM)

shortMsgMT-Relay 	MSC

(forwardSM)

Mobile Terminating Traffic

roamingNumberEnquiry 	HLR

(provideRoamingNumber)

Subscriber Controlled Inputs (Suppl. Services)

networkUnstructuredSs 	HLR

(processSS-Request/v2),

(registerSS/v1),

(eraseSS/v1),

(activateSS/v1),

(deactivateSS/v1),

(registerPassword/v1),

(interrogateSS/v1),

(processUnstructuredSS-Data/v1),

(beginSubscriberActivity/v1)

Priority

low

NOTE:	Application context name is the last but one component of the object identifier.

Operation names are given in brackets with "/vn" appended to vn only operations.

Table 3.1/2: Priorities of Application Contexts for MSC/VLR as Responder

3.1.3	Congestion control for Signalling System No. 7

The requirements of SS7 Congestion control have to be taken into account as far as possible.

Means which could be applied to achieve the required traffic reductions are described in sections 3.1.1 and 3.1.2.

3.2	Compatibility

3.2.1	General

This specification of the Mobile Application Part is designed in such a way that an implementation which conforms to it can also conform to the Mobile Application Part operational version 1 specifications, except on the MSC-VLR interface.

A version negotiation mechanism based on the use of an application-context-name is used to negotiate the protocol version used between two entities for supporting a MAP-user signalling procedure.

When starting a signalling procedure, the MAP-user supplies an application-context-name to the MAP-provider. This name refers to the set of application layer communication capabilities required for this dialogue. This refers to the required TC facilities (i.e. version 1 or 2) and the list of operation packages (i.e. set of operations) from which operations can be invoked during the dialogue.

A version one application-context-name may only be transferred to the peer user in a MAP-U-ABORT to an entity of version two or higher (i.e. to trigger a dialogue which involves only communication capabilities defined for MAP operational version 1).

If the proposed application-context-name can be supported by the responding entity the dialogue continues on this basis otherwise the dialogue is refused an the initiating user needs to start a new dialogue which involve another application-context-name which require less communication capabilities but provides similar functionalities (if possible).

When a signalling procedure can be supported by several application contexts which differs by their version number, the MAP-User need to select a name. It can either select the name which corresponds to the highest version it supports or follow a more specific strategy so that the number of protocol fallbacks due to version compatibility problems be minimized.

3.2.2	Strategy for selecting the Application Context (AC) version

A method should be used to minimize the number of protocol fall-backs which would occur sometimes if the highest supported AC-Name were always the one selected by GSM entities when initiating a dialogue. The following method is an example which can be used mainly at transitory phase stage when the network is one of mixed phase entities.

3.2.2.1	Proposed method

A table (table 1) may be set up by administrative action to define the highest application context (AC) version supported by each destination; a destination may be another node within the same or a different PLMN, or another PLMN considered as a single entity. The destination may be defined by an E.164 number or an E.214 number derived from an IMSI. The table also includes the date when each destination is expected to be able to handle at least one AC of the MAP version 2 protocol. When this date is reached, the application context supported by the node is marked as "unknown", which will trigger the use of table 2.

A second table (table 2) contains an entry for each destination which has an entry in table 1. For a given entity, the entry in table 2 may be a single application context version or a vector of different versions applying to different application contexts for that entity. Table 2 is managed as described in section 3.2.2.2.

The data for each destination will go through the following states:

a)	the version shown in table 1 is "version 1"; table 2 is not used;

b)	the version shown in table 1 is "unknown"; table 2 is used, and maintained as described in section 3.2.2.2;

c)	when the PLMN operator declares that an entity (single node or entire PLMN) has been upgraded to support all the MAP version 2 ACs defined for the relevant interface, the version shown in table 1 is set to "version 2" by administrative action; table 2 is no longer used, and the storage space may be recovered.

3.2.2.2	Managing the version look-up table

WHEN it receives a MAP-OPEN ind the MAP-User determines the originating entity number either using the originating address parameter or the originating reference parameter or retrieving it from the subscriber data using the IMSI or the MSISDN.

IF the entity number is known

THEN

It updates (if required) the associated list of highest supported ACs

ELSE

It creates an entry for this entity and includes the received AC-name in the list of highest supported ACs.

WHEN starting a procedure, the originating MAP-user looks up its version control table.

IF the destination address is known and not timed-out

THEN

It retrieves the appropriate AC-name and uses it

IF the dialogue is accepted by the peer

THEN

It does not modify the version control table

ELSE (this should never occur)

It starts a new dialogue with the common highest version supported (based on information implicitly or explicitly provided by the peer).

It replace the old AC-name by the new one in the list of associated highest AC supported.

ELSE

It uses the AC-name which corresponds to the highest version it supports.

IF the dialogue is accepted by the peer

THEN

It adds the destination node in its version control table and includes the AC-Name in the list of associated highest AC supported.

ELSE

It starts a new dialogue with the common highest version supported (based on information implicitly or explicitly provided by the peer).

IF the destination node was not known

THEN

It adds the destination node in its version control table and includes the new AC-Name in the list of associated highest AC supported.

ELSE

It replaces the old AC-name by the new one in the list of highest supported AC and reset the timer.

3.2.2.3	Optimizing the method

A table look-up may be avoided in some cases if both the HLR and the VLR stores for each subscriber the version of the AC-name used at location updating. Then:

-	for procedures which make use of the same application-context, the same AC-name (thus the same version) can be selected (without any table look-up) when the procedure is triggered.

-	for procedures which make use of a different application-context but which includes one of the packages used by the location updating AC, the same version can be selected (without any table look-up) when the procedure is triggered.

For HLR:

-	Roaming number enquiry procedure;

-	Location cancellation procedure;

-	Subscriber data modification (stand alone).

For VLR:

-	Data Restoration.

4	Requirements concerning the use of SCCP and TC

4.1	Use of SCCP

The Mobile Application Part makes use of the services offered by the Signalling Connection Control Part of signalling System No. 7. CCITT White Book or Blue Book Recommendations Q.711 to Q.716 should be consulted for the full specification of SCCP.

4.1.1	SCCP Class

MAP will only make use of the connectionless classes (0 or 1) of the SCCP.

4.1.2	Sub-system number (SSN)

The Application Entities (AEs) defined for MAP consist of several Application Service Elements (ASEs) and are addressed by sub-system numbers (SSNs). The SSN for MAP are:

00000101	for the whole MAP (reserved for possible future use);

00000110	HLR;

00000111	VLR;

00001000	MSC;

00001001	EIR;

00001010	is allocated for evolution (possible Authentication centre).

4.1.3	SCCP addressing

4.1.3.1	Introduction

Within the GSM System there will be a need to communicate between entities within the same PLMN and in different PLMNs. Using the Mobile Application Part (MAP) for this function implies the use of Transaction Capabilities (TC) and the Signalling Connection Control Part (SCCP) of CCITT Signalling System No. 7.

Only the entities which should be addressed are described below. The format and coding of address parameters carried by the SCCP for that purpose shall comply with CCITT Recommendation Q.713 with the following restrictions:

The entity number is always taken as the originating address.

1)	Intra-PLMN addressing

-	SSN indicator = 1 (MAP SSNs always included);

-	A Global title or a point code may also be included; If none of these parameters is present the MTP address (i.e. DPC, OPC) are used.

2)	Inter-PLMN addressing

a)	Destination address

-	SSN indicator = 1 (MAP SSN always included);

-	Global title indicator = 0100 (Global title includes translation type, numbering plan, encoding scheme and nature of address indicator);

-	The translation type field will be coded "00000000" (Not used);

-	Routing indicator = 0 (Routing on global title).

b)	Originating address

-	SSN indicator = 1 (MAP SSNs always included);

-	Point code indicator = 0;

-	Global title indicator = 0100 (Global title includes translation type, numbering plan, encoding scheme and nature of address indicator);

-	The translation type field will be coded "00000000" (Not used);

-	Routing indicator = 0 (Routing on Global Title).

In response messages the calling party address of the initial message is used as called party address for Inter-PLMN addressing.

In case of intra-PLMN strategy the MTP DPC is used if the SCCP calling party address contains only a SSN; otherwise the above protocol applies.

The following sections describe the method of SCCP addressing appropriate for each entity both for the simple intra-PLMN case and where an inter-PLMN communication is required. The following entities are considered:

The Mobile-services Switching Centre (MSC);

The Home location Register (HLR);

The Visitor Location Register (VLR);

The Gateway Mobile-services Switching Centre (GMSC);

The Interworking Mobile-services Switching Centre (IWMSC).

4.1.3.2	The Mobile-services Switching Centre (MSC)

There are several cases where it is necessary to address the MSC.

4.1.3.2.1	MSC interaction during handover

The address is derived from the target Cellid.

4.1.3.2.2	MSC for short message routing

When a short message has to be routed to a mobile station, the GMSC addresses the VMSC by an MSC identity received from the HLR which complies with E.164 rules.

For MS originating short message, the IWMSC address is derived from the Service Centre address.

4.1.3.3	The Home Location Register (HLR)

There are several cases where the HLR has to be addressed:

4.1.3.3.1	During Call Set-up

When a call is initiated the HLR of the called mobile subscriber will be interrogated to discover the whereabouts of the Mobile Station. The addressing required by the SCCP will be derived from the MSISDN dialled by the calling subscriber. The dialled number will be translated into either an SPC, in the case of communications within a PLMN, or a Global Title if other networks are involved (i.e. if the communication is across a PLMN boundary).

If of the calling subscriber is on the fixed network, the interrogation can be initiated from the Gateway MSC of the home PLMN in the general case. If the topology of the network allows it, the interrogation could be initiated from any Signalling Point which has MAP capabilities, e.g. local exchange, outgoing International Switching Centre (ISC), etc.

4.1.3.3.2	Location updating

When a Mobile Station registers for the first time in a VLR, location updating is initiated and the VLR has to communicate with the MS's HLR. The only data for addressing purposes that the VLR has is contained in the IMSI and addressing information for SCCP must be derived from it.

If the HLR is in the same PLMN as the VLR, local translation tables may exist to derive a SPC. For location updating via the international PSTN/ISDN signalling network, the Global title must be derived from the IMSI, using the principles contained in CCITT Recommendation E.214 and the Numbering Plan Indicator (NPI) value referenced by the SCCP Specifications. A summary of the translation from the IMSI (CCITT Recommendation E.212) to Global Title (described in CCITT Recommendation E.214) is shown below:

-	E.212 Mobile Country Code translates to E.164 Country Code;

-	E.212 Mobile Network Code translates to E.164 National Destination Code;

-	E.212 Mobile Subscriber Identification Number (MSIN) carried unchanged if within the E.164 number maximum length and terminated by the ST signal (15 digits + ST). If the Global title is more than 15 digits the number is truncated to 15 by deleting the least significant digits.

This translation will be done either at the application or at SCCP level in the VLR. The Global title thus derived will be used to address the HLR.

4.1.3.3.3	VLR requests and responses

In this case a dialogue has already taken place so the VLR will already have stored the SCCP address for the roaming subscriber's HLR.

This may be an SPC, or it may be a Global title consisting of the MSISDN or the E.164 number allocated to the HLR.

4.1.3.4	The Visitor Location Register (VLR)

There are several cases when the VLR needs to be addressed:

4.1.3.4.1	Local MSC's transactions

The local MSC when it needs to start a dialogue knows the address of the VLR.

4.1.3.4.2	HLR request

The HLR will only request information from a VLR if it is aware of the existence of one of its subscribers in that VLR area. This implies that the MS has registered with that VLR and location updating has taken place. Within the location updating message, the VLR would have sent its E.164 number. This will be used to derive a Global Title or SPC for the communication.

4.1.3.5	The Interworking MSC for short message service

The IWMSC is the interface between the mobile network and the network to access to the short message service centre. This exchange has an E.164 address known in the HLR or in the MSC.

4.1.3.6	The Equipment Identity Register (EIR)

The EIR address is either unique or could be derived from IMEI. The type of address is not defined.

4.1.3.7	Summary table

The following table summarizes the SCCP address used for invoke operations. As a principle, within a PLMN either an SPC or a GT may be used (network operation option), whereas when addressing an entity outside the PLMN the GT must be used. The address type mentioned in the table (e.g. MSISDN) is used as GT or to derive the SPC.

For a response, the originating address passed in the invoke is used. For extra-PLMN addressing the entity number is used as GT; for intra-PLMN addressing an SPC derived from the entity number may be used instead. When using an SPC, the SPC may be taken directly from MTP.

to	fixed	HLR	VLR	MSC	EIR

from			network

fixed			-	E:GT	-	-	-

network			T:MSISDN

home					I:SPC/GT

location		-	-	E:GT	-	-

register				T:VLR

NUMBER

I:SPC/GT	I:SPC/GT

visitor			-	E:GT	E:GT	-	-

location			T:IMSI/MSISDN/	T:VLR

register			HLR NUMBER/	NUMBER

(NOTE)

mobile-services		I:SPC/GT	I:SPC/GT	I:SPC/GT	I:SPC/GT

switching	-	E:GT	E:GT	E:GT	E:GT

centre				T:MSISDN	T:VLR	T:MSC	T:EIR

NUMBER	 NUMBER	NUMBER

Application context name	5.3.1

Destination address	5.3.1

Destination reference	5.3.1

Diagnostic information	5.3.4

Originating address	5.3.1

Originating reference	5.3.1

Problem diagnostic	5.3.6

Provider reason	5.3.5

Refuse reason	5.3.1

Release method	5.3.2

Responding address	5.3.1

Result	5.3.1

Source	5.3.5

Specific information	5.3.1/5.3.2/5.3.4

User reason	5.3.4

Following is an alphabetic list of parameters contained in this section:

.Begin Table I.

Access connection status

Access signalling information

Alert Reason 	5.6.8.8

Authentication set list

Basic Service Group

Bearer service

BSS-apdu

Call barring feature 	5.6.4.19

Call barring information

Call reference

Called number

Calling number

Category

Ciphering mode

Cksn

CLI Restriction 	5.6.4.5

CM service type

CUG Feature 	5.6.4.7

CUG index

CUG info

CUG interlock

CUG outgoing access indicator

CUG subscription

Current location area Id

Current password

Equipment status

External Signal Information 	5.6.9.4

Forwarded-to number

Forwarded-to subaddress

Forwarding data

Forwarding feature 	5.6.4.16

Forwarding information

Forwarding Options 	5.6.4.6

GSM bearer capability

Guidance information 	5.6.4.22

Handover number

HLR Id

HLR number

HO-Number Not Required	5.6.6.7

IMEI

IMSI

Inter CUG options 	5.6.4.13

Intra CUG restrictions

Invoke Id

Kc

Linked Id

LMSI 	5.6.2.16

Location update type 	5.6.9.6

More Messages To Send

MS ISDN

MSIsdn-Alert 	5.6.2.29

MSC number

MWD status

Network resources

Network signal information 	5.6.9.8

New password

No reply condition timer

ODB General Data

ODB HPLMN Specific Data 	5.6.3.10

OMC Id

Originally dialled number 	5.6.2.26

Originating entity number

Override Category

Previous location area Id

Protocol Id 	5.6.9.7

Provider error

Rand

Regional Subscription Data 	5.6.3.11

Regional Subscription Response 	5.6.3.12

Roaming number

Roaming Restriction Due To Unsupported Feature 	5.6.3.13

Service centre address

SM Delivery Outcome

SM-RP-DA

SM-RP-OA

SM-RP-PRI

SM-RP-UI

Sres

SS Code

SS Data

SS Info

SS-status

Stored location area Id

Subscriber status

Target cell Id

Target location area Id

Target MSC number

Teleservice

TMSI

Trace reference

Trace type

User error 	5.6.1.4

USSD Data Coding Scheme	5.6.4.36

USSD String	5.6.4.37

VLR number 	5.6.2.14

Zone Code

.End Table I.

5.6.1	Common parameters

The following set of parameters are used in several MAP service-primitives:

5.6.1.1	.i. Invoke Id

This parameter identifies corresponding service primitives. The parameter is supplied by the MAP service-user and must be unique over each service-user/service-provider interface.

5.6.1.2	.i. Linked Id

This parameter us used for linked services and it takes the value of the invoke Id of the service linked to.

5.6.1.3	.i. Provider error

This parameter is used to indicate a protocol related type of error:

-	duplicated invoke Id;

-	not supported service;

-	mistyped parameter;

-	resource limitation;

-	initiating release, i.e. the peer has already initiated release of the dialogue and the service has to be released;

-	unexpected response from the peer;

-	service completion failure;

-	no response from the peer;

-	invalid response received.

5.6.1.4	User error

This parameter can take values as follows:

NOTE:	The values are grouped in order to improve readability; the grouping has no other significance.

a)	Generic error;

-	system failure, i.e. a task cannot be performed because of a problem in another entity. The type of entity or network resource may be indicated by use of the network resource parameter;

-	data missing, i.e. an optional parameter required by the context is missing;

-	unexpected data value, i.e. the data type is formally correct but its value or presence is unexpected in the current context;

-	resource limitation;

-	initiating release, i.e. the receiving entity has started the release procedure;

-	facility not supported, i.e. the requested facility is not supported by the PLMN.

b)	Identification or numbering problem;

-	unknown subscriber, i.e. no such subscription exists;

-	number changed, i.e. the subscription does not exist for that number any more;

-	unknown MSC;

-	unidentified subscriber, i.e. if the subscriber is not contained in the database and it has not or cannot be established whether or not a subscription exists;

-	unallocated roaming number;

-	unknown BSS;

-	unknown equipment;

-	unknown location area.

c)	Subscription problem;

-	roaming not allowed, i.e. a location updating attempt is made in an area not covered by the subscription;

-	illegal subscriber, i.e. illegality of the access has been established by use of authentication procedure;

-	bearer service not provisioned;

-	teleservice not provisioned;

-	illegal equipment, i.e. the IMEI check procedure has shown that the IMEI is blacklisted or not whitelisted.

d)	Handover problem;

-	invalid target base station, i.e. the identified base station does not exist;

-	no radio resource available, i.e. the radio path cannot be established due to radio congestion;

-	no handover number available;

-	subsequent handover failure, i.e. handover to a third MSC failed for some reason.

e)	Operation and maintenance problem;

-	tracing buffer full, i.e. tracing cannot be performed because the tracing capacity is exceeded.

f)	Call set-up problem;

-	no roaming number available, i.e. a roaming number cannot be allocated because all available numbers are in use;

-	absent subscriber, i.e. the subscriber has activated the detach service or the system detects the absence condition. If this user error value is used for the short message service, an additional indication whether or not the message waiting data flag has been set, may be included;

-	busy subscriber, provides additional information indicating one of three possible categories of Busy Subscriber:

-	Network Determined User Busy (NDUB)

This condition occurs when a call is about to be offered and the information (i.e. Traffic) channel is busy/not available and the maximum number of total calls has been reached. The value of maximum number of calls is 1 for the basic call. When the supplementary service Call Waiting is applicable, the value is n+1, where n is the maximum number of calls that can be waiting (ref TS GSM 02.01).

-	More Calls Allowed

This condition occurs when a call is about to be offered and the called MS is already busy but the maximum number of total calls has not yet been reached (ref TS GSM 02.01).

-	User Determined User Busy (UDUB)

This condition occurs when a call is offered to an MS and the MS responds "User Busy" because the subscribers resources (terminal or person using them) are busy (ref TS GSM 02.01).

-	no subscriber reply;

-	radio congestion;

-	impossible call completion, i.e. the call can neither be established nor forwarded and none of the above causes will be reported. A list of SS code parameter (see section ) may be appended in order to indicate which supplementary services are active.

-	forwarding violation, i.e. the call has already been forwarded the maximum number of times that is allowed;

-	CUG reject, i.e. the call does not pass a CUG check; additional information may also be given in order to indicate rejection due to e.g. incoming call barred or non-CUG membership.

g)	Supplementary services problem;

-	call barred. Optionally, additional information may be included for indicating either that the call meets a barring condition set by the subscriber or that the call is barred for operator reasons.

-	illegal SS operation, i.e. an operation which violates the rules applicable to the particular supplementary service;

-	SS error status, i.e. the operation on the supplementary service is incompatible with the current status of the service. The current status may be given as additional information (by use of the SS-status parameter defined in section );

-	SS not available, i.e. the service is not available in the current location area;

-	SS subscription violation, i.e. the action requested on the supplementary service is in contradiction with the subscription;

-	SS incompatibility, i.e. an action on one supplementary service is incompatible with the status of another supplementary service. The identity and status of the conflicting service may also be indicated. The additional information may contain the SS code parameter (see section ), the Basic Service Group parameter (see section ) and the SS status parameter (see section );

-	negative password check, i.e. the requested password has not been provided or it has been provided but does not match the valid one;

-	password registration failure, i.e. failure due to wrong inputs from the subscriber. More diagnostic may be given:

-	invalid password format;

-	new password mismatch;

-	Number of Password Attempts; i.e. the mobile user has attempted to access password controlled supplementary services using an erroneous password more than three times;

-	USSD Busy; the mobile station is not able to process an Unstructered Supplementary Service Data operation due to an on-going MMI input of the user or an already existing call independent supplementary service transaction;

-	Unknown Alphabet; the alphabet/language used for the unstructered information in an Unstructured Supplementary Service Data operation is not known by the network or the mobile station.

h)	Short message problem

-	SM delivery failure with detailed reason as follows:

-	memory capacity exceeded;

-	MS protocol error;

-	MS not equipped;

-	unknown service centre (SC);

-	SC congestion;

-	invalid SME address;

-	subscriber is not an SC subscriber;

-	and possibly detailed diagnostic information, coded as specified in TS GSM 03.40, under SMS-SUBMIT-REPORT and SMS-DELIVERY-REPORT. If the SM entity which returns the SM delivery failure error includes detailed diagnostic information, it shall be forwarded in the MAP_FORWARD_SHORT_MESSAGE response.

-	message waiting list full, i.e. no further SC address can be added to the message waiting list.

5.6.2	Numbering and identification parameter

5.6.2.1	.i. IMSI

This parameter is the International Mobile Subscriber Identity defined in TS GSM 03.03.

5.6.2.2	.i. TMSI

This parameter is the Temporary Mobile Subscriber Identity defined in TS GSM 03.03.

5.6.2.3	.i. IMEI

This parameter is the International Mobile Equipment Identity defined in TS GSM 03.03.

5.6.2.4	.i. Previous location area Id

This parameter refers to the identity of the location area from which the subscriber has roamed.

5.6.2.5	.i. Stored location area Id

This parameter refers to the location area where the subscriber is assumed to be located.

5.6.2.6	.i. Current location area Id

This parameter is used to indicate the location area in which the subscriber is currently located.

5.6.2.7	.i. Target location area Id

This parameter refers to the location area into which the subscriber intends to roam.

5.6.2.8	.i. Target cell Id

This parameter refers to the identity of the cell to which a call has to be handed over.

5.6.2.10	.i. Originating entity number

This parameter refers to an application layer identification of a system component in terms of its associated ISDN number.

5.6.2.11	.i. MSC number

This parameter refers to the ISDN number of an MSC.

5.6.2.12	.i. Target MSC number

This parameter refers to the ISDN number of an MSC to which a call has to be handed over.

5.6.2.13	.i. HLR number

This parameter refers to the ISDN number of an HLR.

5.6.2.14	.i. VLR number

This parameter refers to the ISDN number of a VLR.

5.6.2.15	.i. HLR Id

This parameter refers to the identity of an HLR derived from the IMSI defined in CCITT Recommendation E.212.

5.6.2.16	.i. LMSI

This parameter refers to a local identity allocated by the VLR to a given subscriber for internal management of data in the VLR.

5.6.2.17	.i. MS ISDN

This parameter refers to one of the ISDN numbers assigned to a mobile subscriber in accordance with CCITT Recommendation E.213.

5.6.2.18	.i. OMC Id

This parameter refers to the identity of an operation and maintenance centre.

5.6.2.19	.i. Roaming number

This parameter refers to the roaming number as defined in CCITT Recommendation E.213.

5.6.2.20	[Spare]

5.6.2.21	.i. Handover number

This parameter refers to the number used for routing a call between MSCs during handover.

5.6.2.22	.i. Forwarded-to number

This parameter refers to the address to which a call is to be forwarded. This may include a subaddress.

5.6.2.23	.i. Forwarded-to subaddress

This parameter refers to the sub-address attached to the address to which a call is to be forwarded.

5.6.2.24	.i. Called number

This parameter refers to a number called by the mobile subscriber.

5.6.2.25	.i. Calling number

This parameter refers to a calling party address.

5.6.2.26	.i. Originally dialled number

This parameter refers to the number dialled by the calling party in order to reach a mobile subscriber.

5.6.2.27	.i. Service centre address

This parameter represents the address of a short message service centre.

5.6.2.28	.i. Zone Code

This parameter is used to define location areas into which the subscriber is allowed or not allowed to roam (regional subscription). With a complete list of zone codes the VLR is able to determine for all its location areas whether roaming is allowed or not.

5.6.2.29	MSIsdn-Alert

This parameter refers to the MSISDN stored in a Message Waiting Data File in the HLR. It is used to alert the Service Centre when the MS is again attainable.

5.6.3	Subscriber management parameters

5.6.3.1	.i. Category

This parameter refers to the calling party category as defined in CCITT Recommendation Q.767.

5.6.3.2	.i. Equipment status

This parameter refers to the status of the mobile equipment as defined in TS GSM 02.16.

5.6.3.3	.i. Bearer service

This parameter may refer to a single bearer service, a set of bearer services or to all bearer services as defined in TS GSM 02.02.

5.6.3.4	.i. Teleservice

This parameter may refer to a single teleservice, a set of teleservices or to all teleservices as defined in TS GSM 02.03.

5.6.3.5	.i. Basic Service Group

This parameter refers to the Basic Service Group either as bearer service (see section ) or teleservice (see section ).

5.6.3.6	.i. GSM bearer capability

This parameter refers to the GSM bearer capability information element defined in TS GSM 04.08.

5.6.3.7	.i. Subscriber status

This parameter refers to the barring status of the subscriber:

-	service granted;

-	Operator Determined Barring.

5.6.3.8	.i. CUG outgoing access indicator

This parameter represents the Outgoing access as defined in ETS 300 136.

5.6.3.9	.i. Operator Determined Barring General Data

This parameter refers to the set of subscribers features that the network operator or the service provider can regulate. This set only includes those limitations that can be controlled in the VLR.

-	All outgoing calls barred;

-	International outgoing calls barred;

-	International outgoing calls except those to the home PLMN country barred;

-	Premium rate (information) outgoing calls barred;

-	Premium rate (entertainment) outgoing calls barred;

-	Supplementary service access barred.

5.6.3.10	.i. ODB HPLMN Specific Data

This parameter refers to the set of subscribers features that the network operator or the service provider can regulate only when the subscriber is registered in the HPLMN. This set only includes those limitations that can be controlled in the VLR.

-	Operator Determined Barring Type 1;

-	Operator Determined Barring Type 2;

-	Operator Determined Barring Type 3;

-	Operator Determined Barring Type 4.

5.6.3.11	.i. Regional Subscription Data

This parameter defines the regional subscription area in which the subscriber is allowed to roam. It consists of a list of Zone Codes (see section 5.6.2.28)

5.6.3.12	.i. Regional Subscription Response

This parameter indicates either that the regional subscription data cannot be handled or that the current MSC area is entirely restricted because of regional subscription.

5.6.3.13	.i. Roaming Restriction Due To Unsupported Feature

This parameter defines that a subscriber is not allowed to roam in the current MSC area. It may be used by the HLR if a feature or service is indicated as unsupported by the VLR.

5.6.4	Supplementary services parameters

5.6.4.1	.i. SS code

This parameter may refer to one supplementary service or a set of supplementary services as defined in TS GSM 02.04. For MAP version 2 this includes:

-	Calling Line Identification Presentation Service (CLIP);

-	Calling Line Identification Restriction Service (CLIR);

-	Connected Line Identification Presentation Service (COLP);

-	Connected Line Identification Restriction Service (COLR);

-	All Call Forwarding services;

-	Call Waiting (CW);

-	Call Hold (HOLD);

-	Multi-Party service (MPTY);

-	Closed User Group (CUG);

-	All Charging services;

-	All Call Restriction services.

5.6.4.2	.i. SS-status

This parameter refers to the status of individual supplementary services with regard to:

-	SS-quiescent/operative (quiescent/operative);

-	SS-provision (provisioned/not provisioned);

-	SS-registration (registered/not registered);

-	SS-activation (not active/active).

Each of these indicators shall be set to the semantically correct value. The status of any of these indicators shall not imply any information that is assigned to a different SS-status indicator. If one of the indicators listed above is not applicable to a particular supplementary service, its value is irrelevant.

5.6.4.3	.i. SS data

This parameter refers to the necessary set of information required in order to characterise one supplementary service:

-	SS code		(see section );

-	SS status (if applicable)	(see section );

-	Override subscription option	(see section );

-	CLI Restriction		(see section 5.6.4.5);

-	Basic Service Group Code	(see section ).

5.6.4.4	.i. Override Category

This parameter refers to the subscription option Override category attached to a supplementary service. It can take the following two values:

-	Enabled;

-	Disabled.

5.6.4.5	CLI Restriction Option

This parameter refers to the subscription option Restriction mode attached to the CLIR supplementary service. It can take the following three values:

-	Permanent;

-	Temporary (Default Restricted);

-	Temporary (Default Allowed).

5.6.4.6	.i. Forwarding Options

This parameter refers to a forwarding option attached to a supplementary service. It can take one of the following values:

-	notification to forwarding party (see TS GSM 02.82 for the meaning of this parameter);

-	notification to calling party (see TS GSM 02.82 for the meaning of this parameter);

-	Forwarding reason (see TS GSM 02.82 for the meaning of this parameter).

5.6.4.7	.i. No reply condition timer

This parameter refers to the no reply condition timer for call forwarding on no reply.

5.6.4.8	.i. CUG info

This parameter refers to the overall information required for operation for each CUG:

-	CUG subscriptionlist;

-	CUG feature.

5.6.4.9	.i. CUG subscription

This parameter refers to the set of basic information for each CUG defined in that subscription. The following information is stored:

-	CUG index;

-	CUG interlock;

-	Intra CUG restrictions;

-	Basic Service Group List.

5.6.4.10	.i. CUG interlock

This parameter represents the CUG interlock code defined in ETS 300 138.

5.6.4.11	.i. CUG index

This parameter represents the CUG index defined in ETS 300 138.

5.6.4.12	.i. CUG Feature

This parameter contains two parameter which are associated with the Basic Service Group. If the Basic Service Group Code is not present the features applies to all Basic Services. The following two parameters are included:

-	Preferential CUG indicator;

which indicates which CUG index is to be used at outgoing call set-up using the associated Basic Service Group;

-	Inter CUG Option;

describes whether it for the associated Basic Service Group is allowed to make calls outside the CUG and whether incoming calls are allowed.

-	Basic Service Group.

See TS GSM 02.85 for meaning of this parameter.

5.6.4.13	Inter CUG options

This parameter indicates the subscribers ability to make and receive calls outside a specific closed user group. It takes the following values:

-	CUG only facility (only calls within CUG are allowed);

-	CUG with outgoing access (calls outside CUG allowed);

-	CUG with incoming access (calls from outside CUG into CUG allowed);

-	CUG with both incoming and outgoing access (all calls allowed).

5.6.4.14	.i. Intra CUG restrictions

This parameter describes whether or not it is allowed to originate calls to or to receive calls from within the CUG. It can take the following values:

-	no CUG restrictions;

-	CUG incoming calls barred;

-	CUG outgoing calls barred.

5.6.4.15	.i. Forwarding information

This parameter represents the information related to each call forwarding service:

-	the SS-code (see section 5.6.4.1) of the relevant call forwarding service;

-	if required, a list of forwarding feature parameters (see section 5.6.4.16). The list contains one item per Basic Service Group code against which the supplementary service is registered.

5.6.4.16	.i. Forwarding feature

This parameter applies to each combination of call forwarding service and Basic Service Group and contains the following information, as required:

-	Basic Service Group	(see section );

-	SS-status		(see section );

-	forwarded-to number	(see section );

-	forwarded-to subaddress	(see section );

-	forwarding options		(see section );

-	no reply condition timer	(see section ).

5.6.4.17	.i. Forwarding data

This parameter refers to the data required by the network for invoking a call forwarding service:

-	forwarded-to number	(see section );

-	if required, forwarding options	(see section ).

5.6.4.18	.i. Call barring information

This parameter contains for each call barring service:

-	SS-code			(see section );

-	a list of call barring feature parameters	(see section ).

The list contains one item per Basic Service Group which the supplementary services is active.

5.6.4.19	.i. Call barring feature

This parameter gives the status of call barring services as applicable to each Basic Service Group. The parameter contains the following information:

-	Basic Service Group	(see section );

-	SS status		(see section ).

5.6.4.20	.i. New password

This parameter refers to the password which the subscriber just registered in the network.

5.6.4.21	.i. Current password

This parameter refers to a password used by the subscriber for supplementary service control.

5.6.4.22	.i. Guidance information

This parameter refers to guidance information given to a subscriber who is requested to provide a password. One of the following information may be given:

-	"enter password";

This information is used for checking of the old password.

-	"enter new password";

This information is used during password registration for the request of the first new password.

-	"enter new password again";

This information is used during password registration for the request of the new password again for verification.

5.6.4.23	[Spare]

5.6.4.24	.i. SS info

This parameter refers to all the information related to a supplementary service and is a choice between:

-	forwarding information	(see section );

-	call barring information	(see section );

-	CUG info		(see section );

-	SS-data		(see section ).

5.6.4.25-5.6.4.35 [Spare]

5.6.4.36	.i. USSD Data Coding Scheme

This parameter contains the information of the alphabet and the language used for the unstructured information in an Unstructured Supplementary Service Data operation. The coding of this parameter is according to the SMS Data Coding Scheme as specified in TS GSM 03.41.

5.6.4.37	.i. USSD String

This parameter contains a string of unstructured information in an Unstructured Supplementary Service Data operation. The string is sent either by the mobile user or the network. The contents of a string sent by the mobile station are interpreted by the network as specified in TS GSM 02.90.

5.6.5	Call parameters

5.6.5.1	.i. Call reference

This parameter refers to a call reference allocated by a call control MSC.

5.6.6	Radio parameters

5.6.6.1-5.6.6.6 [Spare]

5.6.6.7	.i. HO-Number Not Required

This parameter indicates that no handover number allocation is necessary.

5.6.7	Authentication parameters

5.6.7.1	.i. Authentication set list

This parameter represents a list of sets of authentication parameters for a given subscriber:

-	Rand;

-	Sres;

-	Kc.

5.6.7.2	.i. Rand

This parameter represents a random number used for authentication.

5.6.7.3	.i. Sres

This parameter represents the response to an authentication request.

5.6.7.4	.i. Kc

This parameter refers to a key used for ciphering purposes.

5.6.7.5	[Spare]

5.6.7.6	.i. Cksn

This parameter refers to a ciphering key sequence number.

5.6.7.7	.i. Ciphering mode

This parameter refers to the ciphering mode which is associated with a radio channel. It may take values as follows:

-	no encryption;

-	identification of specific ciphering algorithm.

5.6.8	Short message parameters

5.6.8.1	.i. SM-RP-DA

This parameter represents the destination address used by the short message service relay sub-layer protocol. It can be either of the following:

-	IMSI			(see section 5.6.2.1);

-	MS-ISDN		(see section 5.6.2.17);

-	roaming number		(see section 5.6.2.19);

-	service centre address	(see section 5.6.2.27).

5.6.8.2	.i. SM-RP-OA

This parameter refers to the originating address used by the short message service relay sub-layer protocol. It can be either of the following:

-	MS-ISDN		(see section 5.6.2.17);

-	service centre address	(see section 5.6.2.27).

5.6.8.3	.i. MWD status

This parameter indicates whether or not the address of the originator service centre is already contained in the Message Waiting Data file. In addition, it contains the status of the Memory Capacity Exceeded Flag (MCEF) and the status of the Mobile subscriber Not Reachable Flag (MNRF).

5.6.8.4	.i. SM-RP-UI

This parameter represents the user data field carried by the short message service relay sub-layer protocol.

5.6.8.5	.i. SM-RP-PRI

This parameter is used to indicate whether or not delivery of the short message shall be attempted when a service centre address is already contained in the Message Waiting Data file.

5.6.8.6	.i. SM Delivery outcome

This parameter indicates the cause for setting the message waiting data. It can take one of the following values:

-	Absent subscriber;

-	MS memory capacity exceeded;

-	Successful transfer.

5.6.8.7	.i. More Messages To Send

This parameter is used to indicate whether or not the service centre has more short messages to send.

5.6.8.8	Alert Reason

This parameter is used to indicate the reason why the service centre is alerted. It can take one of the following values:

-	MS present;

-	Memory Available.

5.6.9	Access and signalling system related parameters

5.6.9.1	.i. BSS-apdu

This parameter includes one or two concatenated complete 08.06 messages, as described in technical specifications GSM 03.09 and GSM 09.10. The Protocol ID indicates that the message or messages are according to technical specification GSM 08.06. For the coding of the messages see technical specifications GSM 08.06 and GSM 08.08.

5.6.9.2	.i. CM service type

This parameter identifies the service category being requested by the subscriber:

-	mobile originating call;

-	emergency call establishment;

-	short message service;

-	mobile originating call re-establishment;

-	mobile terminating call;

-	SS request.

5.6.9.3	.i. Access connection status

This parameter represents the following access connection status information:

-	RR-connection status (established/not established);

-	ciphering mode (on/off);

-	authentication status (authenticated/not authenticated).

5.6.9.4	External Signal Information

This parameter contains concatenated information elements (including tag and length) which are defined by a common protocol version, preceded by the associated protocol ID. It is used to transport information of the indicated protocol via MAP interfaces.

5.6.9.5	.i. Access signalling information

This parameter refers to any set of information elements imported from TS GSM 04.08.

5.6.9.6	Location update type

This parameter refers to the location update type (normal, periodic or IMSI attach) contained in the GSM 04.08 LOCATION REGISTRATION REQUEST message.

5.6.9.7	Protocol ID

This parameter refers to the protocol to which the coding of the content of the associated External Signal Information conforms.

The following values are defined:

-	04.08;

-	08.06;

-	ETS300102-1.

This value indicates the protocol defined by ETS 300 102-1 (EDSS1).

5.6.9.8	Network signal information

This parameter is transported as external signal information. The protocol ID shall be set to "ETS 300 102-1".

The network signal information may include the following information elements as defined in TS GSM 09.07:

-	ISDN BC; the tag and length are defined by ETS 300 102-1.

For the content, see GSM 09.07.

-	HLC; the tag and length are defined by ETS 300 102-1.

For the content, see GSM 09.07.

-	LLC; the tag and length are defined by ETS 300 102-1.

For the content, see GSM 09.07.

They are contained in the Signal Information parameter according to figure 5.6/1 (irrespective of the order):

┌─────────────────────────┐

│┌───────────────────────┐│

││      ISDN BC TAG      ││

│├───────────────────────┤│

││         LENGTH        ││

│├───────────────────────┤│

││        CONTENT        ││

│└───────────────────────┘│

│┌───────────────────────┐│

││        HLC TAG        ││

│├───────────────────────┤│

││         LENGTH        ││

│├───────────────────────┤│

││        CONTENT        ││

│└───────────────────────┘│

│┌───────────────────────┐│

││        LLC TAG        ││

│├───────────────────────┤│

││         LENGTH        ││

│├───────────────────────┤│

││        CONTENT        ││

│└───────────────────────┘│

└─────────────────────────┘

Figure 5.6/1: Network signal information parameter

5.6.10	System operations parameters

5.6.10.1	.i. Network resources

This parameter refers to a class or type of network resource:

-	PLMN;

-	HLR;

-	VLR (current or previous);

-	MSC (controlling or current);

-	EIR;

-	radio sub-system.

5.6.10.2	.i. Trace reference

This parameter represents a reference associated with a tracing request. The parameter is managed by OMC.

5.6.10.3	.i. Trace type

This parameter identifies the type of trace. Trace types are fully defined in TS GSM 12.02. Some trace types are standardised in TS GSM 12.02; others are defined for PLMN operator-specific use.

5.7	Representation of a list of a basic parameter in service-primitives

In some service-primitives several instances of a basic parameter of section 5.6 are required. In the service descriptions such cases will be represented as

ParameterNameLIST

in the tables where ParameterName refers to one of the parameters defined in section 5.6. This corresponds to the following construction rule:

┌──────────────<───────────────┐

│    ┌────────────────────┐    │

───────────┴────┤      Parameter     ├────┴──────

└────────────────────┘

Figure 5.7/1: Construction of Lists

6	Mobility services

6.1	Location Management Services

6.1.1	MAP_UPDATE_LOCATION_AREA service

6.1.1.1	Definition

This service is used between MSC and VLR to update location information in the network. It is initiated by a MS when changing the location area or at first registration. The detailed conditions are given in TS GSM 03.12.

The MAP_UPDATE_LOCATION_AREA service is a confirmed service using the primitives from table 6.1/1.

6.1.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Target location area Id	M	M(=)

Location update type	M	M(=)

IMSI	C	C(=)

TMSI	C	C(=)

Previous location area Id	C	C(=)

CKSN	C	C(=)

User error			C	C(=)

Provider error				O

Table 6.1/1: MAP_UPDATE_LOCATION_AREA

6.1.1.3	parameter definitions and use

Invoke Id

See definition in section 5.6.1.

Target location area Id

See definition in section 5.6.2.

Location update type

See definition in section 5.6.9.

IMSI

See definition in section 5.6.2. It is up to the mobile station to provide either IMSI or TMSI, but one shall be present.

TMSI

See definition in section 5.6.2. It is up to the mobile station to provide either IMSI or TMSI, but one shall be present.

Previous location area Id

See definition in section 5.6.2. This parameter is provided if the updating is not a first registration.

CKSN

See definition in section 5.6.7. The CKSN is given if TMSI is used.

User error

One of the following error causes defined in section 5.6.1 is sent by the user in case of location area updating failures, depending on the failure reason:

-	unknown subscriber;

This cause is used if the subscriber is not known in the VLR and even a correlated request to the subscriber's HLR gives a negative result (i.e. the IMSI is not allocated to a subscriber).

-	unknown location area;

This cause is used if the target location area identity given is not known in the VLR.

-	roaming not allowed;

This cause is used if the MS is not allowed to roam into the target location area indicated in the MAP_UPDATE_LOCATION_AREA Req. The cause will be qualified according to the roaming restriction reason, i.e. one of "National Roaming Not Allowed", "PLMN Not Allowed", "Location Area Not Allowed", or "Operator Determined Barring".

-	illegal subscriber;

This error is sent if a correlated authentication procedure has not authenticated the subscriber.

-	illegal equipment;

This error is sent if an IMEI check failed, i.e. the IMEI is blacklisted or not whitelisted.

-	system failure;

-	unexpected data value.

Provider error

For definition of provider errors see section 5.6.1.

6.1.2	MAP_UPDATE_LOCATION service

6.1.2.1	Definition

This service is used by the VLR to update the location information stored in the HLR.

The MAP_UPDATE_LOCATION service is a confirmed service using the service primitives given in table 6.1/2.

6.1.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

MSC Address	M	M(=)

VLR number	M	M(=)

LMSI	U	C(=)

HLR number			C	C(=)

User error			C	C(=)

Provider error				O

Table 6.1/2: MAP_UPDATE_LOCATION

6.1.2.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2.

MSC Address

See definition in section 5.6.2. The MSC address is used for short message delivery only and for each incoming call setup attempt the MSRN will be requested from the VLR.

VLR number

See definition in section 5.6.2.

LMSI

See definition in section 5.6.2. It is an operator option to provide the LMSI.

HLR number

See definition in section 5.6.2. The presence of this parameter is mandatory in case of successful HLR updating.

User error

In case of unsuccessful updating, an error cause shall be returned by the HLR. The following error causes defined in section 5.6.1 may be used, depending on the nature of the fault:

-	unknown subscriber;

-	roaming not allowed;

This cause will be sent if the MS is not allowed to roam into the PLMN indicated by the VLR number. The cause is qualified by the roaming restriction reason "PLMN Not Allowed" or "Operator Determined Barring". If no qualification is received (HLR with MAP Version 1), "PLMN Not Allowed" is taken as default.

-	system failure;

-	unexpected data value.

Provider error

For definition of provider errors see section 5.6.1.

6.1.3	MAP_CANCEL_LOCATION service

6.1.3.1	Definition

This service is used between HLR and VLR to delete a subscriber record from the VLR. It may be invoked automatically when a MS moves from one VLR area to another, to remove the subscriber record from the old VLR, or by the HLR operator to enforce a location updating from the VLR to the HLR, e.g. on withdrawal of a subscription.

The MAP_CANCEL_LOCATION service is a confirmed service using the primitives defined in table 6.1/3.

6.1.3.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

LMSI	C	C(=)

User error			C	C(=)

Provider error				O

Table 6.1/3: MAP_CANCEL_LOCATION

6.1.3.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2.

LMSI

See definition in section 5.6.2. The LMSI shall be included if it has been received from VLR.

User error

If the cancellation fails, an error cause is to be returned by the VLR. The following error cause defined in section 5.6.1 shall be used:

-	unexpected data value.

Provider error

For definition of provider errors see section 5.6.1.

6.1.4	MAP_SEND_IDENTIFICATION service

6.1.4.1	Definition

The MAP_SEND_IDENTIFICATION service is used between a VLR and a previous VLR to retrieve IMSI and authentication sets for a subscriber registering afresh in that VLR.

The MAP_SEND_IDENTIFICATION service is a confirmed service using the service primitives defined in table 6.1/4.

6.1.4.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

TMSI	M	M(=)

IMSI			C	C(=)

Authentication set			U	C(=)

User error			C	C(=)

Provider error				O

Table 6.1/4: MAP_SEND_IDENTIFICATION

6.1.4.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

TMSI

See definition in section 5.6.2.

IMSI

See definition in section 5.6.2. The IMSI is to be returned if the service succeeds.

Authentication set

See definition in section 5.6.7. If the service succeeds a list of up to five authentication sets is returned, if there are any available.

User error

This parameter is mandatory if the service fails. The following error cause defined in section 5.6.1 may be used, depending on the nature of the fault:

-	unidentified subscriber.

Provider error

For definition of provider errors see section 5.6.1.

6.1.5	MAP_DETACH_IMSI service

6.1.5.1	Definition

The MAP_DETACH_IMSI service is used by the MSC to indicate to the VLR that a MS is no longer reachable. The network needs this information e.g. to reject an incoming call without initiating paging on the radio path.

The MAP_DETACH_IMSI service is a non-confirmed service using the service primitives defined in table 6.1/5.

6.1.5.2	Service primitives

Parameter name	Request	Indication

Invoke Id	M	M(=)

IMSI	C	C(=)

TMSI	C	C(=)

Table 6.1/5: MAP_DETACH_IMSI

6.1.5.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. It is up to the mobile station to provide either IMSI or TMSI as subscriber identity, but one shall be present.

TMSI

See definition in section 5.6.2. It is up to the mobile station to provide either IMSI or TMSI as subscriber identity, but one shall be present.

6.1.6	MAP_PURGE_MS service

6.1.6.1	 Definition

This service is used between the VLR and the HLR to cause the HLR to mark its data for an MS so that any request for routing information for a mobile terminated call or a mobile terminated short message will be treated as if the MS is not reachable. It is invoked when the subscriber record for the MS is to be deleted in the VLR, either by MMI interaction or automatically, e.g. because the MS has been inactive for several days.

The MAP_PURGE_MS service is a confirmed service using the primitives defined in table 6.1/6.

6.1.6.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

VLR number	M	M(=)

Provider error				O

Table 6.1/6: MAP_PURGE_MS

6.1.6.3	Parameter definitions and use

Invoke ID

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2.

VLR number

See definition in section 5.6.2.

Provider error

See definition of provider errors in section 5.6.1.

6.2	Paging and Search

6.2.1	MAP_PAGE service

6.2.1.1	Definition

This service is used between VLR and MSC to initiate paging of a MS for mobile terminated call set-up, mobile terminated short message or unstructured SS notification.

The MAP_PAGE service is a confirmed service using the primitives from table 6.2/1.

6.2.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

Stored location area Id	M	M(=)

TMSI	U	C(=)

User error			C	C(=)

Provider error				O

Table 6.2/1: MAP_PAGE

6.2.1.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. The IMSI is used to define the paging subgroup. If the TMSI is not supplied, paging on the radio path uses the IMSI as an identifier.

Stored location area Id

See definition in section 5.6.2.

TMSI

See definition in section 5.6.2. The TMSI is included if paging on the radio channel is to use the TMSI as an identifier.

User error

The following error causes defined in section 5.6.1 may be sent by the user in case of a paging error, depending on the failure reason:

-	absent subscriber;

-	unknown location area;

-	busy subscriber;

-	system failure;

This corresponds to the case where there is no call associated with the MAP_PAGE service, i.e. if the call has been released but the dialogue to the VLR has not been aborted.

-	unexpected data value.

Provider error

See definition in section 5.6.1.

6.2.2	MAP_SEARCH_FOR_MS service

6.2.2.1	Definition

This service is used between VLR and MSC to initiate paging of a MS in all location areas of that VLR. It is used if the VLR does not hold location area information confirmed by radio contact.

The MAP_SEARCH_FOR_MS service is a confirmed service using the primitives from table 6.2/2.

6.2.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

Current location area Id			C	C(=)

User error			C	C(=)

Provider error				O

Table 6.2/2: MAP_SEARCH_FOR_MS

6.2.2.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. The IMSI is used to identify the subscriber when paging on the radio path.

Current location area Id

See definition in section 5.6.2. In case of successful outcome of the service, i.e. if the MS responds to paging, the Location Area Id of the area in which the MS responded is given in the response.

User error

The following error causes defined in section 5.6.1 shall be sent by the user if the search procedure fails, depending on the failure reason:

-	absent subscriber;

This error cause is returned by the MSC if the MS does not respond to the paging request.

-	busy subscriber;

-	system failure;

This corresponds to the case where there is no call associated with the MAP_SEARCH_FOR_MS service, i.e. if the call has been released but the dialogue to the VLR has not been aborted.

-	unexpected data value.

Provider error

See definition in section 5.6.1.

6.3	Access Management Services

6.3.1	MAP_PROCESS_ACCESS_REQUEST service

6.3.1.1	Definition

This service is used between MSC and VLR to initiate processing of a MS access to the network, e.g. in case of mobile originated call set-up or after being paged by the network.

The MAP_PROCESS_ACCESS_REQUEST service is a confirmed service using the primitives from table 6.3/1.

6.3.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

CM service type	M	M(=)

Access connection status	M	M(=)

Current Location Area Id	M	M(=)

TMSI	C	C(=)

Cksn	C	C(=)

IMSI	C	C(=)	C	C(=)

IMEI	C	C(=)	C	C(=)

MSISDN			U	C(=)

User error			C	C(=)

Provider error				O

Table 6.3/1: MAP_PROCESS_ACCESS_REQUEST

6.3.1.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

CM service type

See definition in section 5.6.9.

Access connection status

See definition in section 5.6.9.

Current Location Area Id

See definition in section 5.6.2. This parameter is used to update the VLR in case of previous VLR failure.

TMSI

See definition in section 5.6.2. Either TMSI or IMSI as received from the MS are included in the Request/Indication, but one shall be present. In case of CM Service Type "Emergency Call Establishment", the IMEI may replace IMSI/TMSI.

Cksn

See definition in section 5.6.7. In case of access with TMSI, the Cksn shall be present.

IMSI

See definition in section 5.6.2. Either TMSI or IMSI as received from the MS are included in the Request/Indication, but one shall be present. In case of CM Service Type "Emergency Call Establishment", the IMEI may replace IMSI/TMSI.

In the Resopnse/Confirmation, the IMSI is to be sent in case of successful outcome of the service. In case of CM Service Type "Emergency Call Establishment", IMEI may replace IMSI.

IMEI

See definition in section 5.6.2. The IMEI may replace IMSI/TMSI in the Request/Indication and IMSI in the Response/Confirmation only in case the CM Service Type indicates "Emergency Call Establishment".

MSISDN

See definition in section 5.6.2. The MSISDN is included in case of successful outcome of the service as an operator option, e.g. if it is needed at the MSC for charging purposes in case of call forwarding.

User error

One of the following error causes defined in section 5.6.1 shall be sent by the user if the access request fails, depending on the failure reason:

-	unidentified subscriber;

-	illegal subscriber;

This error is sent if a correlated authentication procedure has not authenticated the subscriber.

-	illegal equipment;

This error is sent if an IMEI check failed, i.e. the IMEI is blacklisted or not whitelisted.

-	roaming not allowed;

This cause is used after VLR restart if the subscriber has no subscription for the current location area, e.g. due to regional subscription. The cause will be qualified by "location area not allowed" or "national roaming not allowed", respectively.

-	unknown location area;

-	system failure;

-	unexpected data value.

Provider error

For definition of provider errors see section 5.6.1.

6.4	Handover Services

6.4.1	MAP_PREPARE_HANDOVER service

6.4.1.1	Definition

This service is used between MSC-A and MSC-B (E-interface) when a call is to be handed over from MSC-A to MSC-B.

The MAP_PREPARE_HANDOVER service is a confirmed service using the primitives from table 6.4/1.

6.4.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Target Cell Id	C	C(=)

HO-NumberNotRequired	C	C(=)

BSS-APDU	C	C(=)	C	C(=)

Handover Number			C	C(=)

User-error			C	C(=)

Provider-error				O

Table 6.4/1: MAP_PREPARE_HANDOVER

6.4.1.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

Target Cell Id

For definition of this parameter see section 5.6.2. This parameter is only included if the service is not in an ongoing transaction.

HO-Number Not Required

For definition of this parameter see section 5.6.6.

BSS-APDU

For definition of this parameter see section 5.6.9.

Handover Number

For definition of this parameter see section 5.6.2. This parameter shall be returned, unless the parameter HO-NumberNotRequired is sent.

User-error

For definition of this parameter see section 5.6.1. The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	No handover number available;

-	System failure;

-	Unexpected data value;

-	DataMissing.

Provider-error

See definition of provider errors in section 5.6.1.

6.4.2	MAP_SEND_END_SIGNAL service

6.4.2.1	Definition

This service is used between MSC-B and MSC-A (E-interface) indicating that the radio path has been established by MSC-B to the MS. MSC-A retains then the main control of the call until it clears.

The response is used by MSC-A to inform MSC-B that all resources for the call can be released in MSC-B, either because the call has been released in MSC-A or because the call has been successfully handed over from MSC-B to another MSC.

The MAP_SEND_END_SIGNAL service is a confirmed service using the primitives from table 6.4/2.

6.4.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

BSS-APDU	M	M(=)

Provider Error				O

Table 6.4/2: MAP_SEND_END_SIGNAL

6.4.2.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

BSS-APDU

For definition of this parameter see section 5.6.9.

Provider Error

For definition of this parameter see section 5.6.1.

6.4.3	MAP_PROCESS_ACCESS_SIGNALLING service

6.4.3.1	Definition

This service is used between MSC-B and MSC-A (E-interface) to pass information received on the A─interface in MSC-B to MSC-A.

The MAP_PROCESS_ACCESS_SIGNALLING service is a non-confirmed service using the primitives from table 6.4/3.

6.4.3.2	Service primitives

Parameter name	Request	Indication

Invoke Id	M	M(=)

BSS-APDU	M	M(=)

Table 6.4/3: MAP_PROCESS_ACCESS_SIGNALLING

6.4.3.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

BSS-APDU

For definition of this parameter see section 5.6.9.

6.4.4	MAP_FORWARD_ACCESS_SIGNALLING service

6.4.4.1	Definition

This service is used between MSC-A and MSC-B (E-interface) to pass information to be forwarded to the A-interface of MSC-B.

The MAP_FORWARD_ACCESS_SIGNALLING service is a non-confirmed service using the primitives from table 6.4/4.

6.4.4.2	Service primitives

Parameter name	Request	Indication

Invoke Id	M	M(=)

BSS-APDU	M	M(=)

Table 6.4/4: MAP_FORWARD_ACCESS_SIGNALLING

6.4.4.3	Parameter use

For the definition and use of all parameters and errors, see section 5.6.1

Invoke Id

For definition of this parameter see section 5.6.1.

BSS-APDU

For definition of this parameter see section 5.6.9.

6.4.5	MAP_PREPARE_SUBSEQUENT_HANDOVER service

6.4.5.1	Definition

This service is used between MSC-B and MSC-A (E-interface) to inform MSC-A that it has been decided that a handover to either MSC-A or a third MSC (MSC-B') is required.

The MAP_PREPARE_SUBSEQUENT_HANDOVER service is a confirmed service using the primitives from table 6.4/5.

6.4.5.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Target Cell Id	M	M(=)

Target MSC Number	M	M(=)

BSS-APDU	M	M(=)	C	C(=)

User-error			C	C(=)

Provider-error				O

Table 6.4/5: MAP_PREPARE_SUBSEQUENT_HANDOVER

6.4.5.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

Target Cell Id

For definition of this parameter see section 5.6.2.

Target MSC Number

For definition of this parameter see section 5.6.2.

BSS-APDU

For definition of this parameter see section 5.6.9.

User-error

For definition of this parameter see section 5.6.1. The following error causes defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unknown MSC;

-	Subsequent handover failure;

-	Unexpected data value;

-	Data Missing.

Provider-error

For definition of this parameter see section 5.6.1.

6.4.6	MAP_ALLOCATE_HANDOVER_NUMBER service

6.4.6.1	Definition

This service is used between MSC and VLR (B-interface) to request a handover number.

The MAP_ALLOCATE_HANDOVER_NUMBER service is a confirmed service using the primitives from table 6.4/6.

6.4.6.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

User-error			C	C(=)

Provider-error				O

Table 6.4/6: MAP_ALLOCATE_HANDOVER_NUMBER

6.4.6.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

User-error

For definition of this parameter see section 5.6.1. The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	No handover number available.

Provider-error

For definition of this parameter see section 5.6.1.

6.4.7	MAP_SEND_HANDOVER_REPORT service

6.4.7.1	Definition

This service is used between VLR and MSC-B (B-interface) to transfer the handover number to be forwarded to and used by MSC-A.

The MAP_SEND_HANDOVER_REPORT service is a confirmed service using the primitives from table 6.4/7.

6.4.7.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Handover Number	M	M(=)

Linked Id	M	M(=)

Provider Error				O

Table 6.4/7: MAP_SEND_HANDOVER_REPORT

6.4.7.3	Parameter use

Invoke Id

For definition of this parameter see section 5.6.1.

Handover Number

For definition of this parameter see section 5.6.2.

Linked Id

For definition of this parameter see section 5.6.1. This service is linked with MAP_ALLOCATE_HANDOVER_NUMBER.

Provider-error

For definition of this parameter see section 5.6.1.

6.5	Authentication management services

6.5.1	MAP_AUTHENTICATE service

6.5.1.1	Definition

This service is used between the VLR and the MSC when the VLR receives a MAP service indication from the MSC concerning a location registration, call set-up, operation on a supplementary service or a request from the MSC to initiate authentication.

The service is a confirmed service and consists of four service primitives.

6.5.1.2	Service primitives

The service primitives are shown in table 6.5/1

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

RAND	M	M(=)

CKSN	M	M(=)

SRES			M	M(=)

Provider-error				O

Table 6.5/1: MAP_AUTHENTICATE parameters

6.5.1.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

RAND

See section 5.6.7 for the use of this parameter.

CKSN

See section 5.6.7 for the use of this parameter.

SRES

See section 5.6.7 for the use of this parameter.

Provider-error

See section 5.6.1 for the use of this parameter.

6.5.2	MAP_SEND_AUTHENTICATION_INFO service

6.5.2.1	Definition

This service is used between the VLR and the HLR for the VLR to retrieve authentication information from the HLR. The VLR requests some sets of RAND/SRES/Kc vectors.

If the HLR cannot provide the VLR with triplets, an empty response is returned. The VLR may then re-use old authentication triplets.

Security related network functions are further described in GSM 03.20.

The service is a confirmed service and consists of four service primitives.

6.5.2.2	Service primitives

The service primitives are shown in table 6.5/2.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

AuthenticationSetList			C	C(=)

User-error			C	C(=)

Provider-error				O

Table 6.5/2: MAP_SEND_AUTHENTICATION_PARAMETERS parameters

6.5.2.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

IMSI

See section 5.6.2 for the use of this parameter.

AuthenticationSetList

A set of one to five authentication vectors are transferred from the HLR to the VLR, if the outcome of the service was successful.

User-error

One of the following error causes defined in section 5.6.1 shall be sent by the user in case of unsuccessful outcome of the service, depending on the respective failure reason:

-	unknown subscriber;

-	unexpected data value;

-	system failure;

-	data missing.

Provider-error

See section 5.6.1 for the use of this parameter.

6.6	Security management services

6.6.1	MAP_SET_CIPHERING_MODE service

6.6.1.1	Definitions

This service is used between the VLR and the MSC to set the ciphering mode and to start ciphering if applicable. It is called when another service requires that information is to be sent on the radio path in encrypted form.

The service is a non-confirmed service and consists of two service primitives.

6.6.1.2	Service primitives

The service primitives are shown in table 6.6/1

Parameter name	Request	Indication

Invoke id	M	M(=)

Ciphering mode	M	M(=)

Kc	C	C(=)

Table 6.6/1: MAP_SET_CIPHERING_MODE parameters

6.6.1.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

Ciphering mode

See section 5.6.7 for the use of this parameter.

Kc

The Kc parameter should be included when the ciphering mode parameter indicates that ciphering must be performed.

6.7	International mobile equipment identities management services

6.7.1	MAP_CHECK_IMEI service

6.7.1.1	Definition

This service is used between the VLR and the MSC and between the MSC and the EIR to request check of IMEI. If tThe IMEI is not available in the MSC, it is requested from the MS and transferred to the EIR in the service request.

The service is a confirmed service and consists of four service primitives.

6.7.1.2	Service primitives

The service primitives are shown in table 6.7/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

IMEI	C	C(=)	C	C(=)

Equipment status			C	C(=)

User-error			C	C(=)

Provider-error				O

Table 6.7/1: MAP_CHECK_IMEI parameters

6.7.1.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

IMEI

See section 5.6.2 for the use of this parameter. The parameter shall not be included in the service request between the VLR and the MSC, but is mandatory in the service request from the MSC to the EIR. It is not included in the service response from the EIR to the MSC, but is mandatory in the service response from the MSC to the VLR on successful outcome.

Equipment status

See section 5.6.4 for the use of this parameter. This parameter is sent by the responder in case of successful outcome of the service.

User-error

One of the following error causes defined in section 5.6.1 shall be sent by the user in case of unsuccessful outcome of the service, depending on the respective failure reason:

-	unknown equipment;

This error is returned by the responder when the IMEI is not known in the EIR.

-	system failure;

-	unexpected data value.

Provider-error

See section 5.6.1 for the use of this parameter.

6.7.2	MAP_OBTAIN_IMEI service

6.7.2.1	Definition

This service is used between the VLR and the MSC to request the IMEI. If the IMEI is not available in the MSC, it is requested from the MS.

The service is a confirmed service and consists of four service primitives.

6.7.2.2	Service primitives

The service primitives are shown in table 6.7/2.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

IMEI			C	C(=)

User-error			C	C(=)

Provider-error				O

Table 6.7/2: MAP_OBTAIN_IMEI parameters

6.7.2.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

IMEI

See section 5.6.2 for the use of this parameter. The parameter IS included in the service response from the MSC to the VLR on successful outcome of the service.

User-error

If the service fails, the VLR sends the user error System Failure (see section 5.6.1) to the MSC.

Provider-error

See section 5.6.1 for the use of this parameter.

6.8	Subscriber management services

6.8.1	MAP-INSERT-SUBSCRIBER-DATA service

6.8.1.1	Definition

This service is used by an HLR to update a VLR with certain subscriber data in the following occasions:

-	The operator has changed the subscription of one or more supplementary services, basic services or data of a subscriber. Note that in case of withdrawal of a Basic or Supplementary service this primitive shall not be used.

-	The operator has applied, changed or removed Operator Determined Barring.

-	The subscriber has changed data concerning one or more supplementary services by using a subscriber procedure.

-	The HLR provides the VLR with subscriber parameters at location updating of a subscriber or at restoration.

It is a confirmed service and consists of the primitives shown in table 6.8/1.

6.8.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	C	C(=)

MSISDN	C	C(=)

Category	C	C(=)

Subscriber status	C	C(=)

Bearer service List	C	C(=)	C	C(=)

Teleservice List	C	C(=)	C	C(=)

Forwarding information List	C	C(=)

Call barring information List	C	C(=)

CUG information List	C	C(=)

SS data List	C	C(=)

Operator Determined Barring

General data	C	C(=)	C	C(=)

Operator Determined

Barring HPLMN data	C	C(=)

Roaming Restriction Due To

Unsupported Feature	C	C(=)

Regional Subscription Data	C	C(=)

SS code List			C	C(=)

Regional Subscription Response			C	C(=)

User error			U	C(=)

Provider error				O

Table 6.8/1: MAP-INSERT-SUBSCRIBER-DATA

6.8.1.3	Parameter use

All parameters are described in section 5.6. The following clarifications are applicable:

IMSI

It is only included if the service is not used in an ongoing transaction (e.g. location updating).

MSISDN

It is included either at location updating or when it is changed. The MSISDN sent shall be the basic MSISDN.

Category

It is included either at location updating or when it is changed.

Subscriber Status

It is included either at location updating or when it is changed.

To apply, remove or update Operator Determined Barring Categories the Subscriber Status is set to Operator Determined Barring. In this case ODB General Data and/or ODB HPLMN Data shall also be present.

To remove all Operator Determined Barring Categories the Subscriber Status shall be set to Service Granted.

Bearer service List

A list of Bearer service parameters (Bearer service is defined in section 5.6). A Bearer service parameter must be the code for an individual Bearer service, except in the cases described below.

The codes for the Bearer service groups "allAlternateSpeech-DataCDA" and "allAlternateSpeech-DataCDS" shall, if applicable, be sent from the HLR to the VLR as a pair. The codes for the Bearer service groups "allSpeechFollowedByDataCDA" and "allSpeechFollowedByDataCDS" shall, if applicable, be sent from the HLR to the VLR as a pair.

If it is included in the Request/Indication, it includes either all Bearer services subscribed (at location updating or at restoration) or only the ones changed (at subscriber data modification).

If the VLR receives an Indication containing any Bearer service parameters which it does not support/allocate it returns them in the Response to the HLR and therefore discards the unsupported Bearer services (no error is sent back), except in the cases described below.

If the VLR receives the codes for the Bearer service groups "allAlternateSpeech-DataCDA" and "allAlternateSpeech-DataCDS" and supports one or more of the circuit-switched synchronous or asynchronous data rates specified for simple data bearer services, it shall accept the bearer service codes, and not return them in the response to the HLR. If the VLR receives the codes for the Bearer service groups "allSpeechFollowedByDataCDA" and "allSpeechFollowedByDataCDS" and supports one or more of the circuit-switched synchronous or asynchronous data rates specified for simple data bearer services, it shall accept the bearer service codes, and not return them in the response to the HLR. If the VLR does not support any of the circuit-switched synchronous or asynchronous data rates specified for simple data bearer services, and receives the pair of codes for "allAlternateSpeech-DataCDA" and "allAlternateSpeech-DataCDS" or the pair of codes for "allSpeechFollowedByDataCDA" and "allSpeechFollowedByDataCDS", it shall reject the pair of codes by returning them in the response to the HLR.

Teleservice List

A list of Teleservice parameters (Teleservice is defined in section 5.6). A Teleservice parameter must be the code for an individual Teleservice.

If it is included in the Request/Indication, it contains either all Teleservices subscribed (at location updating or at restoration) or the ones changed (at subscriber data modification).

If the VLR receives an Indication containing any Teleservice parameters which it does not support/allocate it returns them in the Response to the HLR and therefore discards the unsupported Teleservices (no error is sent back).

Forwarding information List

A list of Forwarding information parameters (Forwarding information is defined in section 5.6). It includes Call Forwarding services either at location updating or at restoration or when they are changed. The Forwarding information parameter can only include the code for an individual call forwarding supplementary service.

If the VLR receives an Indication containing any Call Forwarding service codes which it does not support/allocate it returns them to the HLR in the parameter SS_code_List and therefore discards the unsupported Call Forwarding service codes (no error is sent back).

Call barring information List

A list of Call barring information parameters (Call barring information is defined in section 5.6). It includes Call Barring services either at location updating or at restoration or when they are changed. The call barring information can only include the code for an individual call barring supplementary service.

If the VLR receives an Indication containing any Call Barring service codes which it does not support/allocate it returns them to the HLR in the parameter SS_code_List and therefore discards the unsupported Call Barring service codes (no error is sent back).

CUG information List

A list of CUG information list parameters (CUG information is defined in section 5.6). It includes CUG information either at location updating or at restoration or when it is changed.

If the VLR does not support the CUG service it returns its code to the HLR in the parameter SS_code_List and therefore discards the received information (no error is sent back).

SS data List

A list of SS data parameters (SS data is defined in section 5.6). It is sent for any other supplementary service than Call Forwarding, Call Barring and CUG either at location updating or at restoration or when they are changed. The SS data parameter can only include the code for an individual supplementary service.

If the VLR receives an Indication containing any Supplementary service codes which it does not support/allocate it returns them to the HLR in the parameter SS_code_List and therefore discards the unsupported service codes received (no error is sent back).

Operator Determined Barring General data

If it is included in a Request/Indication, it includes all the Operator Determined Barring categories that may be applied to a subscriber registered in any PLMN. This parameter is only included in a Request/Indication when the parameter Subscriber Status is set to the value Operator Determined Barring. Note that all General Operator Determined Barring Categories shall be set to their actual status.

If the VLR receives an Indication containing Operator Determined Barring General Data which shows that the subscriber is subject to barring not supported by the VLR, it returns Operator Determined Barring General Data in the Response to the HLR to show the barring categories which are not supported by the VLR.

Operator Determined Barring HPLMN data

It includes all the Operator Determined Barring categories that may be applied only to a subscriber registered in the HPLMN. Therefore, it is only transferred to the VLR when the subscriber is roaming into the HPLMN and when the parameter Subscriber Status is set to the value Operator Determined Barring. Note that all HPLMN Operator Determined Barring Categories shall be set to their actual status.

Roaming Restriction Due To Unsupported Feature

The HLR may decide to include this parameter in the request if certain services or features are indicated as not supported by the MSC/VLR (e.g. Advice of Charge Charging Level).

If this parameter is sent to the VLR the MSC area is restricted by the HLR and the VLR.

Regional Subscription Data

If included in the Insert Subscriber Data request this parameter defines the subscriber's subscription area for the addressed VLR (as defined in section 5.6). It contains a list of up to 10 Zone Codes. The HLR shall send only those Zone Codes which are stored against the CC and NDC of the VLR to be updated.

NOTE:	Support of this parameter is a network operator option and it will not be sent to networks which do not support Regional Subscription.

After the regional subscription data are inserted the VLR shall derive whether its location areas are allowed or not. If the whole MSC area is restricted it will be reported to HLR by returning the Regional Subscription Response.

The VLR returns a Regional Subscription Response indicating that a problem with the Zone Code has been detected in one of the following cases:

-	Too Many Zone Codes: more than 10 zone codes are to be stored in the VLR;

-	Regional Subscription Not Supported by the VLR;

-	Zone Codes Conflict: the VLR detects that the zone codes indicate conflicting service permission for a location area.

Zone codes which have no mapping to location areas shall be ignored.

If a sequence of MAP_INSERT_SUBSCRIBER_DATA services is used during a dialogue, Regional Subscription Data shall be accepted only in one service. Regional Subscription Data received in a subsequent service shall be rejected with the error Unexpected Data Value.

If Regional Subscription Data are not included in any MAP_INSERT_SUBSCRIBER_DATA Service, there is no restriction of roaming due to Regional Subscription.

SS code List

The list of SS code parameters that are provided to a subscriber but are not supported/allocated by the VLR (SS code is defined in section 5.6). The list can only include individual SS codes that were sent in the service request.

Regional Subscription Response

If included in the response this parameter indicates one of:

-	MSC Area Restricted entirely because of regional subscription;

-	Too Many Zone Codes to be inserted;

-	Zone Codes Conflict;

-	Regional Subscription not Supported by the VLR.

If the VLR determines after insertion of Regional Subscription Data that the entire MSC area is restricted, the VLR shall respond with a Regional Subscription Response indicating MSC Area Restricted. Otherwise MSC Area Restricted is not sent. The HLR shall check whether the current MSC area is no longer restricted.

User error

Only one of the following values is applicable:

-	Unidentified subscriber;

-	Data missing;

-	Unexpected data value.

6.8.2	MAP-DELETE-SUBSCRIBER-DATA service

6.8.2.1	Definition

This service is used by an HLR to remove certain subscriber data from a VLR if the subscription of one or more supplementary	services or basic services is withdrawn. Note that this service is not used in case of erasure or deactivation of supplementary services.

It is a confirmed service and consists of the primitives shown in table 6.8/2.

6.8.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

Basic service List	C	C(=)

CUG Interlock List	C	C(=)

SS code List	C	C(=)

Roaming Restriction Due To

Unsupported Feature	C	C(=)

Regional Subscription Data	C	C(=)

Regional Subscription Response			C	C(=)

User error			C	C(=)

Provider error				O

Table 6.8/2: MAP-DELETE-SUBSCRIBER-DATA

6.8.2.3	Parameter use

All parameters are described in section 5.6. The following clarifications are applicable:

Basic service List

A list of Basic service parameters (Basic service is defined in section 5.6). It is used when the deletion applies to one or several or all basic services.

SS code List

A list of SS code parameters (SS code is defined in section 5.6). It is used when the deletion applies to one or several or all supplementary services.

There are three possible options:

-	Deletion of Basic Service(s);

The parameter Basic service List is only included.

-	Deletion of supplementary service(s);

The parameter SS code List is only included.

-	Deletion of Basic and Supplementary Services;

Both Basic service List and SS code List are included.

CUG Interlock list

A List of CUG Interlocks (CUG Interlock is defined in section 5.6) addressing specific CUGs. This parameters are used when the deletion applies to one or several CUGs out of the subscribed CUGs. CUG data assigned to the listed interlocks will be deleted.

If all subscribed CUGs are to be deleted the SS Code List shall be used.

Roaming Restriction Due To Unsupported Feature

This parameter is used if Roaming Restriction Due To Unsupported Feature is deleted from the subscriber data. This may occur if unsupported features or services are removed from the subscriber data in the HLR.

If this parameter is sent the VLR shall check if the current Location Area is possibly allowed now.

Regional Subscription Data

Contains a list of Zone Codes (as defined section 5.6) and is used if one or more or all Zone Codes are deleted from the subscriber data. When the indicated zone codes are deleted the VLR shall check for its location areas whether they are allowed or not. If the whole MSC area is restricted it will be reported to HLR by returning the Regional Subscription Response.

Note that support of this parameter is a network operator option and it will not be sent to networks which do not support Regional Subscription.

If the VLR is not able to handle the zone code information, this shall be indicated in the Regional Subscription Response.

The VLR returns a Regional Subscription Response indicating that a problem with the Zone Code has been detected in one of the following cases:

-	Regional Subscription not Supported by the VLR;

-	Zone Codes Conflict: the VLR detects that the zone codes indicate conflicting serice permission for a location area.

Zone codes wihch are not stored shall be ignored and no Regional Subscription Response will be returned.

Regional Subscription Response

If included in the Delete Subscriber Data response this parameter indicates one of:

-	MSC Area Restricted;

-	Zone Codes Conflict;

-	Regional Subscription not Supported.

User error

Only one of the following values is applicable:

-	Unidentified subscriber;

-	Data missing;

-	Unexpected data value.

6.9	Identity management services

6.9.1	MAP-PROVIDE-IMSI service

6.9.1.1	Definition

This service is used by a VLR in order to get, via the MSC, the IMSI of a subscriber (e.g. when a subscriber has identified itself with a TMSI not allocated to any subscriber in the VLR).

It is a confirmed service and consists of the primitives shown in table 6.9/1.

6.9.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI			C	C(=)

User error			C	C(=)

Provider error				O

Table 6.9/1: MAP-PROVIDE-IMSI

6.9.1.3	Parameter use

All parameters are described in section 5.6. The following clarifications are applicable:

IMSI

This parameter is received when the request is successfully carried out. It contains the requested IMSI.

User error

Only one of the following values is applicable:

-	Absent subscriber.

6.9.2	MAP-FORWARD-NEW-TMSI service

6.9.2.1	Definition

This service is used by a VLR to allocate, via MSC, a new TMSI to a subscriber during an ongoing transaction (e.g. call set-up, location updating or supplementary services operation).

It is a confirmed service and consists of the primitives shown in table 6.9/2.

6.9.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

TMSI	M	M(=)

Provider error				O

Table 6.9/2: MAP-FORWARD-NEW-TMSI

6.9.2.3	Parameter use

The parameter TMSI is described in section 5.6.

6.10	Fault recovery services

6.10.1	MAP_RESET service

6.10.1.1	Definition

This service is used by the HLR, after a restart, to indicate to a list of VLRs that a failure occurred.

The MAP_RESET service is a non-confirmed service using the service primitives defined in table 6.10/1

6.10.1.2	Service primitives

Parameter name	Request	Indication

Invoke Id	M	M(=)

HLR number	M	M(=)

HLR Id LIST	U	C(=)

Table 6.10/1: MAP_RESET

6.10.1.3	Parameter Definition and Use

Invoke Id

See definition in section 5.6.1

HLR number

See definition in section 5.6.2.

HLR Id LIST

The HLR Id List is a list of HLR Id. If the parameter is present in the indication, the VLR may base the retrieval of subscribers to be restored on their IMSI: the subscribers affected by the reset are those whose IMSI leading digits are equal to one of these numbers. If the parameter is absent, subscribers to be restored are those for which the OriginatingEntityNumber received at location updating time matches the equivalent parameter of the Reset Indication.

6.10.2	MAP_FORWARD_CHECK_SS_INDICATION service

6.10.2.1	Definition

This service may be used by an HLR as an implementation option, to indicate to a mobile subscriber that Supplementary services parameters may have been altered, e.g. due to a restart. If received from the HLR, the VLR shall forward this indication to the MSC, which in turn forwards it to the MS. The HLR only sends this indication after successful completion of the subscriber data retrieval from HLR to VLR that ran embedded in a MAP_UPDATE_LOCATION procedure.

The MAP_FORWARD_CHECK_SS_INDICATION service is a non-confirmed service using the service primitives defined in table 6.10/2.

6.10.2.2	Service primitives

Parameter name	Request	Indication

Invoke Id	M	M(=)

Table 6.10/2: MAP_FORWARD_CHECK_SS_INDICATION

6.10.2.3	Parameter Definition and Use

Invoke Id

See definition in section 5.6.1

6.10.3	MAP_RESTORE_DATA service

6.10.3.1	Definition

This service is invoked by the VLR on receipt of a MAP_PROVIDE_ROAMING_NUMBER indication for an unknown IMSI, or for a known IMSI with the indicator "Confirmed by HLR" set to "Not confirmed". The service is used to update the LMSI in the HLR, if provided, and to request the HLR to send all data to the VLR that are to be stored in the subscriber's IMSI record.

The MAP_RESTORE_DATA service is a confirmed service using the service primitives defined in table 6.10/3.

6.10.3.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

LMSI	U	C(=)

HLR number			C	C(=)

MS Not Reachable Flag			C	C(=)

User error			C	C(=)

Provider error				O

Table 6.10/3: MAP_RESTORE_DATA

6.10.3.3	Parameter definitions and use

Invoke Id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2.

LMSI

See definition in section 5.6.2. It is an operator option to make use of the LMSI received from VLR.

HLR number

See definition in section 5.6.2. The presence of this parameter is mandatory in case of successful outcome of the service.

MS Not Reachable Flag

See definition in section 5.6.8. This parameter shall be present in case of successful outcome of the service, if the "MS Not Reachable flag" was set in the HLR.

User error

In case of unsuccessful outcome,of the service, an error cause shall be returned by the HLR. The following error causes defined in section 5.6.1 may be used, depending on the nature of the fault:

-	unknown subscriber;

-	system failure;

-	unexpected data value;

-	data missing.

Provider error

For definition of provider errors see section 5.6.1.

7	Operation and Maintenance services

7.1	Subscriber tracing services

7.1.1	MAP-ACTIVATE-TRACE-MODE service

7.1.1.1	Definition

This service is used between the HLR and the VLR to activate subscriber tracing in the VLR.

The MAP-ACTIVATE-TRACE-MODE service is a confirmed service using the primitives from table 7.1/1.

7.1.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

IMSI	C	C(=)

Trace Reference	M	M(=)

Trace Type	M	M(=)

OMC Id	U	C(=)

User Error			C	C(=)

Provider Error				O

Table 7.1/1: MAP-ACTIVATE-TRACE-MODE

7.1.1.3	Parameter usa

Invoke id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. The IMSI is a mandatory parameter in a standalone operation.

Trace Reference

See definition in section 5.6.10.

Trace Type

See definition in section 5.6.10.

OMC Id

See definition in section 5.6.2. The use of this parameter is an operator option.

User error

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unidentified Subscriber;

-	Facility Not Supported;

-	Tracing Buffer Full;

-	System Failure;

-	Unexpected Data Value;

-	Data missing.

Provider Error

For definition of provider errors see section 5.6.1.

7.1.2	MAP-DEACTIVATE-TRACE-MODE service

7.1.2.1	Definition

This service is used between the VLR and the HLR for deactivating subscriber tracing in the VLR.

The MAP-DEACTIVATE-TRACE-MODE service is a confirmed service using the primitives from table 7.1/2.

7.1.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

IMSI	C	C(=)

Trace Reference	M	M(=)

User Error			C	C(=)

Provider Error				O

Table 7.1/2: MAP-DEACTIVATE-TRACE-MODE

7.1.2.3	Parameter usa

Invoke id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. The IMSI is a mandatory parameter in a standalone operation.

Trace Reference

See definition in section 5.6.10.

User error

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unidentified Subscriber;

-	Facility Not Supported;

-	System Failure;

-	Unexpected Data Value;

-	Data missing.

Provider Error

For definition of provider errors see section 5.6.1.

7.1.3	MAP-TRACE-SUBSCRIBER-ACTIVITY service

7.1.3.1	Definition

This service is used between the VLR and the MSC to activate the subscriber tracing in the MSC.

The MAP-TRACE-SUBSCRIBER-ACTIVITY service is a non-confirmed service using the primitives from table 7.1/3.

7.1.3.2	Service primitives

Parameter name	Request	Indication

Invoke id	M	M(=)

IMSI	C	C(=)

Trace Reference	M	M(=)

Trace Type	M	M(=)

OMC Id	U	C(=)

Table 7.1/3: MAP-TRACE-SUBSCRIBER-ACTIVITY

7.1.3.3	Parameter usa

Invoke id

See definition in section 5.6.1.

IMSI

See definition in section 5.6.2. The controlling MSC shall provide either the IMSI or the IMEI to the servicing MSC.

Trace Reference

See definition in section 5.6.10.

Trace Type

See definition in section 5.6.10.

OMC Id

See definition in section 5.6.2. The use of this parameter is an operator option.

7.2	Other operation and maintenance services

7.2.1	MAP-SEND-IMSI service

7.2.1.1	Definition

This service is used by a VLR in order to fetch the IMSI of a subscriber in case of some Operation & Maintenance procedure where subscriber data are needed in the Visited PLMN and MSISDN is the only subscriber's identity known.

It is a confirmed service and consists of the primitive shown in figure 7.2/1.

7.2.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	M	M(=)

IMSI			C	C(=)

User error			C	C(=)

Provider error				O

Table 7.2/1: MAP-SEND-IMSI

7.2.1.3	Parameter usa

All parameters are described in section 5.6. The following clarifications are applicable:

User error

Only one of the following values is applicable:

-	Unknown subscriber;

-	Unexpected data value;

-	Data missing.

8	Call handling services

8.1	MAP_SEND_INFO_FOR_INCOMING_CALL service

8.1.1	Definition

This service is used between an MSC and VLR. It is invoked by an MSC receiving an incoming call (call to MS) to request the VLR for information to proceed and set up the call.

This is a confirmed service using the primitives described in table 8.1/1.

8.1.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke ID	M	M(=)	M(=)	M(=)

MSRN	M	C(=)

Bearer Service	C	C(=)

Tele Service	C	C(=)

Dialled Number	C	C(=)

CUG Interlock	C	C(=)	U	C(=)

CUG Outgoing Access	C	C(=)	C	C(=)

Number of Forwarding	C	C(=)

Incoming Call Setup Result			C	C(=)

Forwarding Data			C	C(=)

MSISDN			U	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.1/1: MAP_SEND_INFO_FOR_INCOMING_CALL parameters

8.1.3	Parameter use

See section 5.6 for a definition of the parameters used, in addition to the following.

MSRN

This is the Mobile Subscriber Roaming Number. It is the identity of the MS to receive the incoming call.

Bearer Service

If this information is present in the User Service Information element of the ISUP Initial Address Message, it will be passed on to the VLR without change. Note, either a Bearer Service or Tele Service will be provided, not both.

Tele Service

If this information is present in the User Service Information element of the ISUP Initial Address Message it will be passed on to the VLR without change.

Dialled Number

If the information is available, this is the number originally dialled by the calling subscriber.

CUG Interlock

This parameter will only be present in the request and indication if the CUG information is received from ISUP. This parameter will only be present in the response and confirm, if the Incoming Call Setup Result indicates forward and the call is to be treated as a CUG Call, following the CUG Call Authorisation check in the VLR.

CUG Outgoing Access

This parameter may only be present in the request and indication if the CUG Interlock Code is also provided. This information is received from ISUP. This parameter will only be present in the response and confirm, if the Incoming Call Setup Result indicates forward and the call is to be treated as a CUG Call, following the CUG Call Authorisation check in the VLR.

Number of Forwarding

If the call has already been forwarded this parameter will contain a count of the number of times the call has been forwarded, if it is provided by ISUP.

Incoming Call Setup Result

This parameter if present will indicate either success or forward. If the result indicates forward then the Forwarding Data and optionally MSISDN must also be present.

Forwarding Data

This is the address of the subscriber to whom the call should be connected. The parameter will contain the forwarded-to-number and if required the forwarding options.

MSISDN

This is the mobile subscriber ISDN identity of the called party. This is an operator dependant inclusion.

User Error

This parameter is sent by the responder when an error is detected and if present, takes one of the following values:

-	Unallocated Roaming Number;

The MS has been identified by an MSRN, and this MSRN is not allocated.

-	Unknown Subscriber;

-	Absent Subscriber;

This indicates that there is no call forwarding and either the IMSI detached flag was set in the VLR or paging was unsuccessful.

-	Impossible Call Completion;

This indicates that the call will not be forwarded and shall be released, e.g. call forwarding on busy subscriber and call waiting services are not active.

-	Forwarding Violation;

A call forwarding condition applies but the incoming call has already been forwarded as many times as allowed.

-	CUG Reject;

The value of this error cause will indicate the reason for CUG reject.

-	Unexpected Data Value;

-	Data Missing;

-	System Failure.

See section 5.6 for a definition of these reasons.

Provider Error

These are defined in section 5.6.

8.2	MAP_SEND_INFO_FOR_OUTGOING_CALL Service

8.2.1	Definition

This service is used between an MSC and VLR. The service is invoked by an MSC to retrieve the required information from a VLR to enable an outgoing call set up request (call from an MS) to be actioned.

This is a confirmed service using the primitives defined in table 8.2/1.

8.2.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Called Number	M	M(=)

Bearer Service	C	C(=)

Tele Service	C	C(=)

CUG Index	C	C(=)

Suppress Pref CUG	C	C(=)

Suppress CUG OA	C	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.2/1: MAP_SEND_INFO_FOR_OUTGOING_CALL parameters

8.2.3	Parameter use

See section 5.6 for a definition of the parameters used, in addition to the following.

Bearer Service

If this information is received from the Mobile Station, it is passed on unchanged to the VLR. Note, either a Bearer Service or Teleservice will be provided, not both.

Tele Service

If this information is received from the Mobile Station, it is passed on unchanged to the VLR.

CUG Index

This parameter will only be present if it is received from the mobile station, it is passed unchanged to the VLR.

Suppress Pref CUG

This parameter will only be present if it is received from the mobile station, it is passed unchanged to the VLR.

Suppress CUG OA

This parameter will only be present if it is received from the mobile station, it is passed unchanged to the VLR.

User Error

This parameter is sent by the responder when an error is detected and if present, takes one of the following values:

-	Unknown Subscriber;

-	Bearer Service Not Provisioned;

-	Teleservice Not Provisioned;

A subscription check has been performed and the call has not passed that check due to incompatibility with regard to the requested service. Depending on the nature of the incompatibility, either of these messages will be returned.

-	Call Barred;

This error will indicate that either outgoing calls are barred for this MS or that calls are barred due to Operator Determined Barring (see GSM 02.41 for a definition of this network feature).

-	CUG Reject;

The value of this error cause will indicate the reason for CUG reject.

-	Data Missing;

-	Unexpected Data Value;

-	System Failure.

See section 5.6 for a definition of these reasons.

Provider Error

These are defined in section 5.6.

8.3	MAP_SEND_ROUTING_INFORMATION service

8.3.1	Definition

This service is used between the Gateway MSC and the HLR. The service is invoked by the Gateway MSC to perform the interrogation of the HLR in order to route a call towards the called MS.

This is a confirmed service using the primitives defined in table 8.3/1.

8.3.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	M	M(=)

CUG Interlock	C	C(=)	C	C(=)

CUG Outgoing Access	C	C(=)	C	C(=)

Number of Forwarding	C	C(=)

Network Signal Info	C	C(=)

IMSI			C	C(=)

MSRN			C	C(=)

Forwarding Data			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.3/1: MAP_SEND_ROUTING_INFORMATION parameters

8.3.3	Parameter use

See section 5.6 for a definition of the parameters used in addition to the following.

MSISDN

This is the Mobile Subscriber ISDN number assigned to the called subscriber.

CUG Interlock

If this information is received from ISUP it is transparently passed to the HLR in the request and indication primitives. This parameter will only be present in the response and confirm primitive if the call is to be treated as a CUG Call, following the CUG Call Authorisation check in the HLR.

CUG Outgoing Access

This parameter may only be present if the CUG Interlock code is also provided. This information is received from ISUP and is transparently passed to the HLR in the request and indication primitives. This parameter will only be present in the response and confirm primitive if the call is to be treated as a CUG Call, following the CUG Call Authorisation check in the HLR.

Number of Forwarding

If this information is received from ISUP it is transparently passed to the HLR.

Network Signal Info

This parameter is defined in section 5.6.

IMSI

This is the identity of the subscriber to receive the call. If present this indicates success.

MSRN

This parameter may be returned if there are no call forwarding supplementary services active and the information is available. This parameter will not be present if the parameter Forwarding Data is returned.

Forwarding Data

This parameter will be returned if a call forwarding supplementary service is active. The parameter will not be present if the MSRN is returned.

User Error

This parameter is sent by the responder when an error is detected and if present, takes one of the following values:

-	Unknown Subscriber;

-	Number changed;

-	Call Barred;

This error will indicate that either incoming calls are barred for this MS or that calls are barred due to Operator Determined Barring (see GSM 02.41 for a definition of this network feature).

-	CUG Reject;

The value of this error cause will indicate the reason for CUG Reject.

-	Bearer Service Not Provisioned;

-	Teleservice Not Provisioned;

A subscription check has been performed and the call has not passed the check due to incompatibility with regard to the requested service. Depending on the nature of the incompatibility, either of these messages will be returned.

-	Facility Not Supported;

-	Absent Subscriber;

This indicates that the location of the MS is not known (either the station is not registered and there is no location information available or the Provide Roaming Number procedure fails due to IMSI detached flag being set) and the call forwarding on MS not reachable service is not active.

-	Forwarding Violation;

-	System Failure;

-	Data Missing;

-	Unexpected Data Value.

See section 5.6 for a definition of these errors.

Provider Error

These are defined in section 5.6.

8.4	MAP_PROVIDE_ROAMING_NUMBER service

8.4.1	Definition

This service is used between the HLR and VLR. The service is invoked by the HLR to request a VLR to send back a roaming number to enable the HLR to instruct the GMSC to route an incoming call to the called MS.

This is a confirmed service which uses the Primitives described in table 8.4/1.

8.4.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

IMSI	M	M(=)

MSC Number	M	M(=)

MSISDN	C	C(=)

LMSI	C	C(=)

GSM Bearer Capability	C	C(=)

Network Signal Info	C	C(=)

Roaming Number			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.4/1: MAP_PROVIDE_ROAMING_NUMBER parameters

8.4.3	Parameter use

See section 5.6 for a definition of the parameters used, in addition to the following.

IMSI

This is the IMSI of the called Subscriber.

MSC Number

This is the ISDN number assigned to the MSC currently serving the Mobile Station. The MSC number will have been stored in the HLR as provided at location updating.

MSISDN

If present this is the Mobile Subscriber ISDN identity of the called subscriber. This is the MSISDN contained in the MAP-SEND-ROUTING-INFORMATION request.

LMSI

The Local Mobile Station Identity shall be provided if the VLR had previously updated the HLR with this reference.

GSM Bearer Capability

This information is passed according to the rules specified in GSM 09.07.

There may be two GSM Bearer Capabilities supplied.

Network Signal Info

This parameter is defined in section 5.6.

The information is taken from the Network Signal Info parameter of the MAP_SEND_ROUTING_INFORMATION primitive indication (see section 8.3.3).

Roaming Number

If present this parameter indicates success.

User Error

This parameter is sent by the responder when an error is detected and if present, takes one of the following values:

-	Absent Subscriber;

This error will be returned if the IMSI detach flag is set.

-	No Roaming Number Available;

-	Facility Not Supported;

-	System Failure;

-	Data Missing;

-	Unexpected Data Value.

See section 5.6 for a definition of these reasons.

Provider Error

These are defined in section 5.6.

8.5	MAP_COMPLETE_CALL service

8.5.1	Definition

This service is used between the VLR and MSC. The service is invoked by a VLR to request the MSC to set up a call to an MS in the case of an incoming call, and from an MS for an outgoing call.

This is a confirmed service which uses the Primitives described in table 8.5/1.

8.5.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	C	C(=)

IMEI	C	C(=)

Category	C	C(=)

CUG Index	C	C(=)

GSM Bearer Capability	C	C(=)

Network Signal Info	C	C

No Reply Condition Time	C	C(=)

SS Data list	C	C(=)

CUG Interlock	C	C(=)

CUG Outgoing Access	C	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.5/1: MAP_COMPLETE_CALL parameters

8.5.3	Parameter use

See section 5.6 for a definition of the parameters used, in addition to the following.

MSISDN

This is the Mobile Subscriber ISDN identity assigned to the calling subscriber for mobile originating calls or called subscriber for mobile terminating calls. For outgoing Emergency Calls only, this parameter may not be present.

IMEI

This is the International Mobile Equipment Identity of the MS. This parameter will only be present for outgoing Emergency Calls when the calling subscriber cannot be identified. Use of the IMEI in these circumstances is an operator option.

Category

This is the category of the calling subscriber for outgoing call setup only. This information will always be present for an outgoing call.

CUG Index

In the case of an Incoming Call, this parameter will only be present, if CUG information has been received in the incoming call setup request. The CUG Index will be passed to the called subscriber in the call setup signalling. This information may also be present for an outgoing call, this indicates that a CUG has been implicitly invoked. As an operator option, this information may be returned to the calling user but will not be passed to the call destination (ref GSM 03.85).

GSM Bearer Capability

This is the Bearer Capability received by the VLR in the MAP-PROVIDE-ROAMING-NUMBER service. Two GSM Bearer Capabilities may be supplied. This parameter will only be present for incoming call setup and if the information is available.

Network Signal Info

This will only be present if the information is available and GSM BC is not provided. This is the Network Signal Info received by the VLR in the MAP-PROVIDE-ROAMING-NUMBER service. This parameter is only applicable for incoming call setup.

No Reply Condition Time

This parameter is only included for incoming call setup and if the call forwarding on no reply service is active.

SS Data List

This is a list of SS Data and will only be passed if the information is available (i.e. phase 2 supplementary services such as number identification services, CUG and Advice of Charge).

CUG Interlock

This parameter will only be present for outgoing call setup. This is the result of the conversion by the VLR, of the CUG Index originally supplied by the MS.

CUG Outgoing Access

This parameter may be present but only if the CUG Interlock code is also provided.

User Error

This parameter is sent by the responder when an error is detected and if present, takes one of the following values:

-	No Subscriber Reply;

-	Radio Congestion;

-	Busy Subscriber;

-	Data Missing;

-	Unexpected Data Value.

See section 5.6 for a definition of these reasons.

Provider Error

These are defined in section 5.6.

8.6	MAP_PROCESS_CALL_WAITING service

8.6.1	Definition

This service is used between the VLR and MSC. It is invoked by the VLR to request the MSC to invoke the Call Waiting service.

This is a confirmed service using the primitives described in table 8.6/1.

8.6.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	M	M(=)

NoReply Condition Time	C	C(=)

SS Data List	C	C(=)

CUG Index	C	C(=)

GSM Bearer Capability	C	C(=)

Network Signal Info	C	C(=)

User Error			C	C(=)

Provider Error				O

Table 8.6/1: MAP_PROCESS_CALL_WAITING parameters

8.6.3	Parameter use

See section 5.6 for a definition of the parameters used, in addition to the following.

MSISDN

This is the connected called line identification.

No Reply Condition Time

This is the No Reply Condition Timer value. The presence of this parameter indicates whether or not the service Call Forwarding on No Reply is active.

SS Data List

This is a list of SS Data and will only be passed if the information is available (i.e. for GSM Phase 2 supplementary services such as number presentation services and Advice of Charge).

CUG Index

This parameter will only be present if the received CUG information in the incoming call setup request from the MSC. It is the result of the translation by the VLR, of the CUG Interlock code.

GSM Bearer Capability

This is the Bearer Capability received by the VLR in the MAP-PROVIDE-ROAMING-NUMBER service. It will only be present if the information is available.

Network Signal Info

This will only be present if the information is available and GSM BC is not provided. This is the Network Signal Info received by the VLR in the MAP-PROVIDE-ROAMING-NUMBER service.

User Error

This parameter is sent by the responder when an error is detected, if present, takes one of the following values:

-	Unexpected data value;

-	Busy Subscriber;

-	No Subscriber Reply.

Provider Error

These are defined in section 5.6.

9	Supplementary services related services

9.1	MAP_REGISTER_SS service

9.1.1	Definition

This service is used between the MSC and the VLR and between the VLR and the HLR to register data related to a supplementary service. The VLR will relay the message to the HLR.

The service is a confirmed service and consists of four service primitives.

9.1.2	Service primitives

The service primitives are shown in table 9.1/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

Forwarded-to number with subaddress	C	C(=)

No reply condition time	C	C(=)

Forwarding Information			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.1/1: MAP_REGISTER_SS parameters

9.1.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This parameter indicates the supplementary service which the mobile subscriber wants to register.

Basic service

This parameter indicates for which basic service group the supplementary service is to be registered. If it is not included, the registration request applies to all basic services.

Forwarded-to number with subaddress

This parameter is obligatory if the registration applies to one or more call forwarding supplementary services. It can optionally include a sub-address.

No reply condition time

This parameter is included if the registration applies to the Call Forwarding on No Reply supplementary service (or a superset of this service) and the mobile subscriber supplies a value for this time.

Forwarding Information

This parameter is returned by the responder at successful outcome of the service, if the registration request concerned one or a group of Call Forwarding supplementary services.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values defined in section 5.6.1:

-	System failure;

-	Data missing;

-	Unexpected data value;

-	Call Barred;

-	Bearer service not provisioned;

This error is returned only if not even a subset of the requested bearer service group has been subscribed to.

-	Teleservice not provisioned;

This error is returned only if not even a subset of the requested teleservice group has been subscribed to.

-	Illegal SS operation;

-	SS error status;

-	SS Not Available;

-	SS incompatibility.

Provider error

See section 5.6.1 for the use of this parameter.

9.2	MAP_ERASE_SS service

9.2.1	Definition

This service is used between the MSC and the VLR and between the VLR and the HLR to erase data related to a supplementary service. The VLR will relay the message to the HLR.

The service is a confirmed service and consists of four service primitives.

9.2.2	Service primitives

The service primitives are shown in table 9.2/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

Forwarding Information			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.2/1: MAP_ERASE_SS parameters

9.2.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This parameter indicates the supplementary service which the mobile subscriber wants to erase.

Basic service

This parameter indicates for which basic service group the supplementary service should be erased. If it is not included, the erasure request applies to all basic services.

Forwarding Information

This parameter is returned by the responder at successful outcome of the service, if the erasure request concerned one or a group of Call Forwarding supplementary services.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values, defined in section 5.6.1:

-	System failure;

-	Data Missing;

-	Unexpected data value;

-	Bearer service not provisioned;

This error is returned only if not even a subset of the requested bearer service group has been subscribed to.

-	Teleservice not provisioned;

This error is returned only if not even a subset of the requested teleservice group has been subscribed to.

-	Call Barred;

-	Illegal SS operation;

-	SS error status;

-	SS Not Available.

Provider error

See section 5.6.1 for the use of this parameter.

9.3	MAP_ACTIVATE_SS service

9.3.1	Definition

This service is used between the MSC and the VLR and between the VLR and the HLR to activate a supplementary service. The VLR will relay the message to the HLR.

The service is a confirmed service and consists of four service primitives.

9.3.2	Service primitives

The service primitives are shown in table 9.3/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

Forwarding Information			C	C(=)

Call barring Information			C	C(=)

SS-data			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.3/1: MAP_ACTIVATE_SS parameters

9.3.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This parameter indicates the supplementary service which the mobile subscriber wants to activate.

Basic service

This parameter indicates for which basic service groups the requested supplementary service(s) should be activated. If it is not included, the activation request applies to all basic services.

Forwarding Information

This parameter is returned by the responder at successful outcome of the service, if the activation request concerned Call Forwarding.

Call barring Information

This parameter is returned by the responder at successful outcome of the service, if the activation request concerned Call Barring.

SS-data

This parameter is returned by the responder at successful outcome of the service, if the activation request concerned for example Call Waiting.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values, defined in section 5.6.1:

-	System failure;

-	Data Missing;

-	Unexpected data value;

-	Bearer service not provisioned;

This error is returned only if not even a subset of the requested bearer service group has been subscribed to.

-	Teleservice not provisioned;

This error is returned only if not even a subset of the requested teleservice group has been subscribed to.

-	Call Barred;

-	Illegal SS operation;

-	SS error status;

-	SS Not Available;

-	SS subscription violation;

-	SS incompatibility;

-	Negative PW check;

-	Number Of PW Attempts Violation.

Provider error

See section 5.6.1 for the use of this parameter.

9.4	MAP_DEACTIVATE_SS service

9.4.1	Definitions

This service is used between the MSC and the VLR and between the VLR and the HLR to deactivate a supplementary service. The VLR will relay the message to the HLR.

The service is a confirmed service and consists of four service primitives.

9.4.2	Service primitives

The service primitives are shown in table 9.4/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

Forwarding information			C	C(=)

Call Barring information			C	C(=)

SS-data			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.4/1: MAP_DEACTIVATE_SS parameters

9.4.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This parameter indicates the supplementary service which the mobile subscriber wants to deactivate.

Basic service

This parameter indicates for which basic service group the requested supplementary service(s) should be deactivated. If it is not included the deactivation request applies to all basic services.

Forwarding Information

This parameter is returned by the responder at successful outcome of the service, if the deactivation request concerned one or a group of Call Forwarding supplementary services.

Call barring Information

This parameter is returned by the responder at successful outcome of the service, if the activation request concerned one or a group of Call Barring supplementary services.

SS-data

This parameter is returned by the responder at successful outcome of the service, for example if the deactivation request concerned the Call Waiting supplementary service.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values, defined in section 5.6.1:

-	System failure;

-	Data Missing;

-	Unexpected data value;

-	Bearer service not provisioned;

This error is returned only if not even a subset of the requested bearer service group has been subscribed to.

-	Teleservice not provisioned;

This error is returned only if not even a subset of the requested teleservice group has been subscribed to.

-	Call Barred;

-	Illegal SS operation;

-	SS error status;

-	SS Not Available;

-	SS subscription violation;

-	Negative PW check;

-	Number Of PW Attempts Violation.

Provider error

See section 5.6.1 for the use of this parameter.

9.5	MAP_INTERROGATE_SS service

9.5.1	Definitions

This service is used between the MSC and the VLR and between the VLR and the HLR to retrieve information related to a supplementary service. The VLR will relay the message to the HLR if necessary.

The service is a confirmed service and consists of four service primitives.

9.5.2	Service primitives

The service primitives are shown in table 9.5/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

SS status			C	C(=)

Basic service Group LIST			C	C(=)

Forwarding feature LIST			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.5/1: MAP_INTERROGATE_SS parameters

9.5.3	Parameter use

For additional information on parameter use refer to the GSM 04.8x and 04.9x-series of technical specifications.

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

The mobile subscriber can only interrogate a single supplementary service per service request.

Basic service

This parameter indicates for which basic service group the given supplementary service is interrogated. If it is not included, the interrogation request applies to all basic services.

SS status

This parameter is included by the responder if:

-	the interrogated supplementary service can only be subscribed for all applicable basic services simultaneously, or

-	the interrogated supplementary service is not active for any of the interrogated basic services.

Basic service group LIST

This parameter LIST is used to include one or a series of basic service groups for which the interrogated supplementary service is active. If the interrogated supplementary service is not active for any of the interrogated (and provisioned) basic service groups, the SS status parameter is returned.

Forwarding feature LIST

The forwarding feature parameter is described in section 5.6.4. A list of one or more forwarding features is returned by the responder when the interrogation request applied to Call Forwarding supplementary service.

If no basic service code parameter is provided within this sequence, the forwarding feature parameter applies to all provisioned basic services.

User error

This error is sent by the responder upon unsuccessful outcome of the interrogation service, and then takes one of the following values, defined in section 5.6.1:

-	System failure;

-	Data Missing;

-	Unexpected data value;

-	Bearer Service not provisioned;

This error is returned only if not even a subset of the interrogated bearer services are provided.

-	Teleservice not provisioned;

This error is returned only if not even a subset of the interrogated teleservices are provided.

-	Call Barred;

-	Illegal SS operation;

-	SS not available.

Provider error

See section 5.6.1 for the use of this parameter.

9.6	MAP_INVOKE_SS service

9.6.1	Definitions

This service is used between the MSC and the VLR to check the subscriber's subscription to a given supplementary service in the VLR, in connection with in-call invocation of that supplementary Service, i.e. after the call set-up phase is finished. For supplementary service invocation during call set-up phase, please refer to the call handling descriptions.

The service is a confirmed service and consists of four service primitives.

9.6.2	Service primitives

The service primitives are shown in table 9.6/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

Basic service	C	C(=)

User error			C	C(=)

Provider error				O

Table 9.6/1: MAP_INVOKE_SS parameters

9.6.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This SS-code can only refer to a single supplementary service, e.g. the Call Hold or Multi Party supplementary services.

Basic service

This parameter indicates for which basic service the supplementary service invocation is required.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values:

-	System Failure;

-	Data Missing;

-	Unexpected data value;

-	Call Barred;

-	Illegal SS operation;

-	SS error status;

-	SS not available.

Provider error

See section 5.6.1 for the use of this parameter.

9.7	MAP_REGISTER_PASSWORD service

9.7.1	Definitions

This service is used between the MSC and the VLR and between the VLR and the HLR if the mobile subscriber requests to register a new password. The VLR will relay the message to the HLR.

The service is a confirmed service and consists of four service primitives.

9.7.2	Service primitives

The service primitives are shown in table 9.7/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

SS-code	M	M(=)

New password			C	C(=)

User error			C	C(=)

Provider error				O

Table 9.7/1: MAP_REGISTER_PASSWORD parameters

9.7.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

SS-code

This parameter indicates for which supplementary service(s) the password should be registered.

New Password

See section 5.6.4 for the use of this parameter.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values, defined in section 5.6.1:

-	System failure;

-	Data Missing;

-	Unexpected data value;

-	Call Barred;

-	SS subscription violation;

-	Password registration failure;

-	Negative PW check;

-	Number Of PW Attempts Violation.

Provider error

See section 5.6.1 for the use of this parameter.

9.8	MAP_GET_PASSWORD service

9.8.1	Definitions

This service is used between the HLR and the VLR and between the VLR and the MSC when the HLR receives a request from the mobile subscriber for an operation on a supplementary service which requires a password from the subscriber. The VLR will relay the message to the MSC.

The service is a confirmed service and consists of four service primitives.

9.8.2	Service primitives

The service primitives are shown in table 9.8/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

Linked id	C	C(=)

Guidance info	M	M(=)

Current password			M	M(=)

Provider error				O

Table 9.8/1: MAP_GET_PASSWORD parameters

9.8.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

Linked Id

See section 5.6.1 for the use of this parameter. If the MAP GET PASSWORD service is used in conjunction with the MAP REGISTER PASSWORD service, this parameter must be present; otherwise it must be absent.

Guidance info

See section 5.6.4 for the use of this parameter.

Current password

See section 5.6.4 for the use of this parameter.

Provider error

See section 5.6.1 for the use of this parameter.

9.9	MAP_PROCESS_UNSTRUCTURED_SS_REQUEST_SERVICE

9.9.1	Definitions

This service is used between the MSC and the VLR and between the VLR and the HLR to relay information in order to allow unstructured supplementary service operation.

The MAP_PROCESS_UNSTRUCTURED_SS_REQUEST service is a confirmed service using the primitives from table 9.9/1.

9.9.2	Service primitives

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

USSD Data Coding Scheme	M	M(=)	C	C(=)

USSD String	M	M(=)	C	C(=)

User error			C	C(=)

Provider error				O

Table 9.9/1: MAP_PROCESS_UNSTRUCTURED_SS_REQUEST parameters

9.9.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

USSD Data Coding Scheme:

See section 5.6.4 for the use of this parameter. The presence of the parameter in the response is dependent on the unstructured supplementary service application. If this parameter is present, then the USSD String parameter has to be present.

USSD String:

See section 5.6.1 for the use of this parameter. The presence of the parameter in the response is dependent on the unstructured supplementary service application. If this parameter is present, then the USSD Data Coding Scheme parameter has to be present.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values defined in section 5.6.1:

-	System failure;

-	Data missing;

-	Unexpected data value;

This error is returned by the responder if it is not able to deal with the contents of the USSD string.

-	Call Barred;

-	Unknown Alphabet.

Provider error

See section 5.6.1 for the use of this parameter.

9.10	MAP_UNSTRUCTURED_SS_REQUEST service

9.10.1	Definitions

This service is used between the HLR and the VLR and between the VLR and the MSC when the invoking entity requires information from the mobile user, in connection with unstructured supplementary service handling.

The MAP_UNSTRUCTURED_SS_REQUEST service is a confirmed service using the primitives from table 9.10/1.

9.10.2	Service primitives

The service primitives are shown in table 9.10/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

USSD Data Coding Scheme	M	M(=)	C	C(=)

USSD String	M	M(=)	C	C(=)

User error			C	C(=)

Provider error				O

Table 9.10/1: MAP_UNSTRUCTURED_SS_REQUEST parameters

9.10.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

USSD Data Coding Scheme:

See section 5.6.4 for the use of this parameter. The presence of the parameter in the response is dependent on the mobile user's MMI input. If this parameter is present, then the USSD String parameter has to be present.

USSD String:

See section 5.6.1 for the use of this parameter. The presence of the parameter in the response is dependent on the mobile user's MMI input. If this parameter is present, then the USSD Data Coding Scheme parameter has to be present.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values defined in section 5.6.1:

-	System failure;

-	Data missing;

-	Unexpected data value;

This error is returned by the responder if it is not able to deal with the contents of the USSD string.

-	Absent Subscriber;

-	Illegal Subscriber;

This error indicates that delivery of the unstructered supplementary service data failed because the mobile station failed authentication.

-	Illegal Equipment;

-	USSD Busy;

-	Unknown Alphabet.

Provider error

See section 5.6.1 for the use of this parameter.

9.11	MAP_UNSTUCTURED_SS_NOTIFY service

9.11.1	Definitions

This service is used between the HLR and the VLR and between the VLR and the MSC when the invoking entity requires a notification to be sent to the mobile user, in connection with unstructured supplementary services handling.

The MAP_UNSTRUCTURED_SS_NOTIFY service is a confirmed service using the primitives from table 9.11/1.

9.11.2	Service primitives

The service primitives are shown in table 9.11/1.

Parameter name	Request	Indication	Response	Confirm

Invoke id	M	M(=)	M(=)	M(=)

USSD Data Coding Scheme	M	M(=)

USSD String	M	M(=)

User error			C	C(=)

Provider error				O

Table 9.11/1: MAP_UNSTRUCTURED_SS_NOTIFY parameters

9.11.3	Parameter use

Invoke id

See section 5.6.1 for the use of this parameter.

USSD Data Coding Scheme:

See section 5.6.4 for the use of this parameter.

USSD String:

See section 5.6.1 for the use of this parameter.

User error

This parameter is sent by the responder upon unsuccessful outcome of the service, and then takes one of the following values defined in section 5.6.1:

-	System failure;

-	Data missing;

-	Unexpected data value;

This error is returned by the responder if it is not able to deal with the contents of the USSD string.

-	Absent Subscriber;

-	Illegal Subscriber;

This error indicates that delivery of the unstructered supplementary service data failed because the mobile station failed authentication.

-	Illegal Equipment;

-	USSD Busy;

-	Unknown Alphabet.

Provider error

See section 5.6.1 for the use of this parameter.

10	Short message service management services

10.1	MAP-SEND-ROUTING-INFO-FOR-SM service

10.1.1	Definition

This service is used between the gateway MSC and the HLR to retrieve the routing information needed for routing the short message to the servicing MSC.

The MAP-SEND-ROUTING-INFO-FOR-SM is a confirmed service using the primitives from table 10.1/1.

10.1.2	Service primitives

The service primitives are shown in table 10.1/1.

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	M	M(=)

SM-RP-PRI	M	M(=)

Service Centre Address	M	M(=)

IMSI			C	C(=)

MSC Number			C	C(=)

LMSI			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 10.1/1: MAP-SEND-ROUTING-INFO-FOR-SM

10.1.3	Parameter use

Invoke id:

See definition in section 5.6.1.

MSISDN:

See definition in section 5.6.2.

SM-RP-PRI:

See definition in section 5.6.8.

Service Centre Address:

See definition in section 5.6.2.

IMSI:

See definition in section 5.6.2. The presence of this parameter is mandatory in a successful case.

MSC Number:

See definition in section 5.6.2. This parameter is provided in a successful response.

LMSI:

See definition in section 5.6.2. It is an operator option to provide this parameter. The LMSI can be provided in a successful response, if the VLR, in which area the subscriber is, makes use of the LMSI.

User error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unknown subscriber;

-	Call Barred;

-	Teleservice Not Provisioned;

-	Absent Subscriber;

-	Facility Not Supported;

-	System failure;

-	Unexpected Data Value;

-	Data missing.

Provider Error:

For definition of provider errors see section 5.6.1.

10.2	MAP-FORWARD-SHORT-MESSAGE service

10.2.1	Definition

This service is used between the gateway MSC and the servicing MSC to forward mobile originated or mobile terminated short messages.

The MAP-FORWARD-SHORT-MESSAGE service is a confirmed service using the service primitives given in table 10.2/1.

10.2.2	Service primitives

The service primitives are shown in table 10.2/1.

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

SM RP DA	M	M(=)

SM RP OA	M	M(=)

SM RP UI	M	M(=)

More Messages To Send	C	C(=)

User Error			C	C(=)

Provider Error				O

Table 10.2/1: MAP-FORWARD-SHORT-MESSAGE

10.2.3	Parameter use

Invoke id:

See definition in section 5.6.1.

SM RP DA:

See definition in section 5.6.8. This parameter can contain either an IMSI or a LMSI in case of the mobile terminated SM. The use of the LMSI is an operator option. The LMSI can be provided if it is received from the HLR. The IMSI is used if the use of the LMSI is not available.

This parameter is omitted in the mobile terminated subsequent SM transfers.

In the mobile originated SM transfer this parameter contains the Service Centre address received from the Mobile Station.

SM RP OA:

See definition in section 5.6.8. The Service Centre address received from the originating Service Centre is inserted in this parameter in the mobile terminated SM transfer.

This parameter is omitted in the mobile terminated subsequent SM transfers.

The MSISDN received from the VLR is inserted in this parameter in the mobile originated SM transfer.

SM RP UI:

See definition in section 5.6.8. The short message relay protocol unit data received from the Service Centre is inserted in this parameter.

More Messages To Send:

See definition in section 5.6.8. The information from MMS indication received from the Service Centre is inserted in this parameter. This is used only in the mobile terminated case.

User error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unidentified subscriber;

-	Absent Subscriber;

-	Facility Not Supported;

-	Illegal Subscriber indicates that delivery of the mobile terminated short message failed because the mobile station failed authentication;

-	Illegal equipment indicates that delivery of the mobile terminated short message failed because an IMEI check failed, i.e. the IMEI was blacklisted or not whitelisted;

-	System Failure;

-	SM Delivery Failure;

-	The reason of the SM Delivery Failure can be one of the following in the mobile terminated SM:

-	memory capacity exceeded in the mobile equipment;

-	protocol error;

-	mobile equipment does not support the mobile terminated short message service.

-	The reason of the SM Delivery Failure can be one of the following in the mobile originated SM:

-	unknown Service Centre address;

-	Service Centre congestion;

-	invalid Short Message Entity address;

-	subscriber not Service Centre subscriber;

-	protocol error.

-	Unexpected Data Value;

-	Data Missing.

Provider Error:

For definition of provider errors see section 5.6.1.

10.3	MAP-REPORT-SM-DELIVERY-STATUS service

10.3.1	Definition

This service is used between the gateway MSC and the HLR. The MAP-REPORT-SM-DELIVERY-STATUS service is used to set the Message Waiting Data into the HLR or to inform the HLR of successful SM transfer after polling. This service is invoked by the gateway MSC.

The MAP-REPORT-SM-DELIVERY-STATUS service is a confirmed service using the service primitives given in table 10.3/1.

10.3.2	Service primitives

The service primitives are shown in table 10.3/1.

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

MSISDN	M	M(=)

Service Centre Address	M	M(=)

SM Delivery Outcome	M	M(=)

MSIsdn-Alert			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 10.3/1: MAP-REPORT-SM-DELIVERY-STATUS

10.3.3	Parameter use

Invoke id:

See definition in section 5.6.1.

MSISDN:

See definition in section 5.6.2.

Service Centre Address:

See definition in section 5.6.2.

SM Delivery Outcome:

See definition in section 5.6.8. This parameter indicates the status of the mobile terminated SM delivery.

MSIsdn-Alert:

See definition in section 5.6.2 This parameter refers to the MSISDN stored in a Message Waiting Data file in the HLR. The presence of this parameter is mandatory in case of successful outcome of the service.

User error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unknown Subscriber;

-	Message Waiting List Full;

-	Unexpected Data Value;

-	Data missing.

Provider Error:

For definition of provider errors see section 5.6.1.

10.4	MAP-READY-FOR-SM service

10.4.1	Definition

This service is used between the MSC and VLR and as well between the VLR and the HLR. The MSC initiates this service if a subscriber indicates memory available situation. The VLR uses the service to indicate this to the HLR.

The VLR initiates this service if a subscriber, whose message waiting flag is active in the VLR, has radio contact in the MSC.

The MAP-READY-FOR-SM service is a confirmed service using the primitives from table 10.4/1.

10.4.2	Service primitives

The service primitives are shown in table 10.4/1.

Parameter name	Request 	Indication	Response	Confirm

Invoke Id	M	M(=)

IMSI	C	C(=)

TMSI	C	C(=)

Alert Reason	M	M(=)

User Error			C	C(=)

Provider Error				O

Table 10.4/1: MAP-READY-FOR-SM

10.4.3	Parameter use

Invoke id:

See definition in section 5.6.1.

IMSI:

See definition in section 5.6.2. The IMSI is used always between the VLR and the HLR. Between the MSC and the VLR the identification can be either IMSI or TMSI.

TMSI:

See definition in section 5.6.2. The identification can be either IMSI or TMSI between MSC and VLR.

Alert Reason:

See definition in section 5.6.8. This parameter indicates if the mobile subscriber is present or the mobile station has memory available.

User error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unknown Subscriber;

-	Facility Not Supported:

-	System Failure;

-	Unexpected Data Value;

-	Data missing;

Provider Error:

For definition of provider errors see section 5.6.1.

10.5	MAP-ALERT-SERVICE-CENTRE service

10.5.1	Definition

This service is used between the HLR and the interworking MSC. The HLR initiates this service, if the HLR detects that a subscriber, whose MSISDN is in the Message Waiting Data file, is active or the mobile station has memory available.

The MAP-ALERT-SERVICE-CENTRE service is a confirmed service using the primitives from table 10.5/1.

10.5.2	Service primitives

The service primitives are shown in table 10.5/1.

Parameter name	Request 	Indication	Response	Confirm

Invoke Id	M	M(=)

MSIsdn-Alert	M	M(=)

Service Centre Address	M	M(=)

User Error			C	C(=)

Provider Error				O

Table 10.5/1: MAP-ALERT-SERVICE-CENTRE

10.5.3	Parameter use

Invoke id:

See definition in section 5.6.1.

MSIsdn-Alert:

See definition in section 5.6.2. The provided MSISDN shall be the one which is stored in the Message Waiting Data file.

Service Centre Address:

See definition in section 5.6.2.

User error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	System Failure;

-	Unexpected Data Value;

-	Data missing;

Provider Error:

For definition of provider errors see section 5.6.1.

10.6	MAP-INFORM-SERVICE-CENTRE service

10.6.1	Definition

This service is used between the HLR and the gateway MSC to inform the Service Centre which MSISDN number is stored in the Message Waiting Data file. If the stored MSISDN number is not the same than the one received from the gateway MSC in the MAP-SEND-ROUTING-INFO-FOR-SM service primitive the stored MSISDN number is included in the message.

Additionally the status of MCEF and MNRF flags and the inclusion of the particular Service Centre address in the Message Waiting Data list is informed to the gateway MSC when appropriate.

The MAP-INFORM-SERVICE-CENTRE service is a non-confirmed service using the primitives from table 10.6/1.

10.6.2	Service primitives

The service primitives are shown in table 10.6/1.

Parameter name	Request	Indication

Invoke Id	M	M(=)

MSIsdn-Alert	C	C(=)

MWD Status	C	C(=)

Table 10.6/1: MAP-INFORM-SERVICE-CENTRE

10.6.3	Parameter use

Invoke id:

See definition in section 5.6.1.

MSIsdn-Alert:

See definition in section 5.6.2 This parameter refers to the MSISDN stored in a Message Waiting Data file in the HLR.

MWD Status:

See definition in section 5.6.8. This parameter indicates the status of the MCEF and MNRF flags and the status of the particular SC address presence in the Message Waiting Data list.

10.7	MAP-SEND-INFO-FOR-MT-SMS service

10.7.1	Definition

This service is used between the MSC and the VLR. The service is invoked by the MSC receiving an mobile terminated short message to request subscriber related information from the VLR.

The MAP-SEND-INFO-FOR-MT-SMS service is a confirmed service using the primitives from table 10.7/1.

10.7.2	Service primitives

The service primitives are shown in table 10.7/1.

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

SM RP DA	M	M(=)

MSISDN			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 10.7/1: MAP-SEND-INFO-FOR-MT-SMS

10.7.3	Parameter use

Invoke id:

See definition in section 5.6.1.

SM RP DA:

See definition in section 5.6.8. This parameter shall contain either an IMSI or a LMSI.

MSISDN:

See definition in section 5.6.2.

User Error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Unknown subscriber;

-	Unidentified Subscriber;

-	Absent subscriber;

-	Unexpected Data Value;

-	Data Missing;

-	Illegal subscriber;

-	Illegal equipment;

-	System Failure.

Provider Error:

For definition of provider errors see section 5.6.1.

10.8	MAP-SEND-INFO-FOR-MO-SMS service

10.8.1	Definition

This service is used between the MSC and the VLR. The service is invoked by the MSC which has to handle a mobile originated short message request to request the subscriber related information from the VLR.

The MAP-SEND-INFO-FOR-MO-SMS service is a confirmed service using the primitives from table 10.8/1.

10.8.2	Service primitives

The service primitives are shown in table 10.8/1.

Parameter name	Request	Indication	Response	Confirm

Invoke Id	M	M(=)	M(=)	M(=)

Service Centre Address	M	M(=)

MSISDN			C	C(=)

User Error			C	C(=)

Provider Error				O

Table 10.8/1: MAP-SEND-INFO-FOR-MO-SMS

10.8.3	Parameter use

Invoke id:

See definition in section 5.6.1.

Service Centre Address:

See definition in section 5.6.2.

MSISDN:

See definition in section 5.6.2.

User Error:

The following errors defined in section 5.6.1 may be used, depending on the nature of the fault:

-	Teleservice Not Provisioned;

-	Call Barred;

-	Unexpected Data Value;

-	Data Missing.

Provider Error:

For definition of provider errors see section 5.6.1.

11	General

11.1	Overview

Sections 11 to 14 specify the protocol elements to be used to provide the MAP services described in section 5.

Section 12 specifies the elements of procedures for the MAP protocol. Section 13 specifies the mapping on to TC service primitives. Section 14 specifies the application contexts, operation packages and abstract syntaxes for the MAP protocol as well as the encoding rules to be applied.

11.2	Underlying services

The MAP protocol relies on the services provided by the Transaction Capabilities (TC) of signalling system number 7, as referenced in section 4.

11.3	Model

The MAP protocol machine (MAP PM) can be modelled as a collection of service state machines (SSMs) - one per MAP specific service invoked - coordinated by a MAP dialogue control function with its one state machine: MAP dialogue state machine (DSM). There are two types of Service State Machines: Requesting Service State Machines (RSM) and Performing Service State Machines (PSM).

A new invocation of a MAP PM is employed on the receipt of a MAP-OPEN request primitive or a TC-BEGIN indication primitive. Each invocation controls exactly one MAP dialogue. For each MAP specific service invoked during a dialogue, a MAP RSM is created at the requestor's side and a MAP PSM is created at the performer's side.

This modelling is used only to facilitate understanding and the MAP behaviour descriptions and is not intended to suggest any implementation. SDL descriptions are organized according to this model.

How the MAP-service-user and the MAP refer to a MAP dialogue (i.e. a MAP PM invocation) is a local implementation matter.

How TC dialogue identifiers are assigned to a MAP PM invocation is also a local implementation matter.

11.4	Conventions

The behaviour of the MAP PM depends on the application-context-name associated with the dialogue. One major difference is that the MAP requests the transfer of the application-context-name by TC only for those contexts which do not belong to the so-called "version one context set".

The "version one context set" is a set of application-contexts which model the behaviour of a MAP V1 implementation according to the latest phase 1 version of TS GSM 09.02. This set is defined in section 12.

12	Elements of procedure

12.1	Dialogue establishment

The establishment of a MAP dialogue involves two MAP-service-users, one that is the dialogue-initiator and one that is the dialogue-responder.

This procedure is driven by the following signals:

-	A MAP-OPEN request primitive from the dialogue-initiator;

-	A TC-BEGIN indication primitive occurring at the responding side;

-	A MAP-OPEN response primitive from the dialogue-responder;

-	The first TC-CONTINUE indication primitive occurring at the initiating side;

and under specific conditions:

-	A TC-END indication primitive occurring at the initiating side;

-	A TC-U-ABORT indication primitive occurring at the initiating side;

-	A TC-P-ABORT indication primitive occurring at the initiating side.

12.1.1	Receipt of a MAP-OPEN request primitive

On receipt of a MAP-OPEN request primitive the behaviour of the MAP PM shall be as follows:

If the application-context-name belongs to the MAP version 1 contexts set, the MAP PM shall construct an argument for the beginSubscriberActivity operation if a user data parameter is present and request the invocation of this operation, using the TC-INVOKE service (NOTE).

NOTE:	In practice this should only occur when the application-context is related to supplementary service handling.

The MAP PM shall then accept zero, one or several user request primitives until a MAP-DELIMITER request primitive is received.

For each user request primitive, the MAP PM shall request the invocation of the associated operation using the TC-INVOKE service. See section 12.6 for a description of the associated SSMs.

On receipt of the MAP-DELIMITER request primitive the MAP PM shall issue a TC-BEGIN request primitive. If the application-context-name does not belong to the MAP version 1 context set, this parameter as well as the user information parameter (if any) shall be mapped to the corresponding TC-BEGIN parameters.

The requesting MAP PM waits for a TC indication primitive and does not accept any other primitive from its user, except a MAP-U-ABORT request or a MAP-CLOSE request with the release method parameter set to "pre-arranged release".

12.1.2	Receipt of a TC-BEGIN indication

On receipt of a TC-BEGIN indication primitive, the MAP PM shall:

-	If no application-context-name is included in the primitive and if the "Components present" indicator indicates "no components", issue a TC-U-ABORT request primitive(NOTE 2). The local MAP-User is not informed.

-	If no application-context-name is included in the primitive and if presence of components is indicated, wait for the first TC-INVOKE primitive, and derive a version 1 application-context-name from the operation code according to table 12.1/1 (NOTE 1).

NOTE 1:	In some cases, it may be necessary to analyse the operation argument.

Then:

a)	If no application-context-name can be derived (i.e. the operation code does not exist in MAP V1 specifications), the MAP PM shall issue a TC-U-ABORT request primitive (NOTE 2). The local MAP-User is not informed.

b)	If an application-context-name can be derived and if it is acceptable from a load control point of view, the MAP PM shall:

i)	If this primitive requests the beginSubscriberActivity operation, the MAP PM shall check whether more components have been received associated with this operation. If more components are present, the MAP PM shall issue a MAP-OPEN indication primitive with the version 1 application-context-name "networkFunctionalSsContext-v1". The Destination-reference shall include the IMSI taken from the argument of the beginSubscriberActivity operation; the Originating-reference shall cover the originatingEntityNumber.

A beginSubscriberActivity operation that is not associated with any other Component shall be rejected by the the MAP PM by issuing a TC-U-ABORT request primitive (NOTE 2). The local MAP-User shall not be informed.

ii)	otherwise, the MAP PM shall issue a MAP-OPEN indication primitive with the version 1 application-context-name set according to table 12.1/1. DestinationReference and OriginatingReference must not be included in the MAP-OPEN indication primitive.

Then the MAP PM shall function in a way that the dialogue responding MAP behaves as specified in the GSM phase 1 protocol (latest version of TS GSM 09.02 phase 1).

NOTE 2:	If no AARQ apdu was included in the BEGIN message, TC (Component Sub-layer) will not include an AARE apdu or an ABRT apdu in a TR-U-ABORT request primitive that is to be issued on receipt of a TC-U-ABORT request primitive from the local MAP service provider.

c)	If an application-context-name can be derived but if it is not acceptable from a load control point of view, the MAP PM shall ignore this dialogue request and not inform the MAP-user.

-	If a version 1 application-context-name is included, the MAP PM shall issue a TC-U-ABORT request primitive with abort-reason "User-specific" and user-information "MAP-ProviderAbortInfo" indicating "abnormalDialogue". The local MAP-user shall not be informed.

-	If an application-context-name different from version 1 is included in the primitive and if User-information is present, the User-information must constitute a syntactically correct MAP-OPEN dialogue PDU. Otherwise a TC-U-ABORT request primitive with abort-reason "User-specific" and user-information "MAP-ProviderAbortInfo" indicating "abnormalDialogue" shall be issued and the local MAP-user shall not be informed.

-	If no User-information is present it is checked whether presence of User Information in the TC-BEGIN indication primitive is required for the received application-context-name. If User Information is required but not present, a TC-U-ABORT request primitive with abort-reason "User-specific" and user-information "MAP-ProviderAbortInfo" indicating "abnormalDialogue" shall be issued. The local MAP-user shall not be informed.

-	If an application-context-name different from version 1 is received in a syntactically correct TC-BEGIN indication primitive but is not acceptable from a load control point of view, the MAP PM shall ignore this dialogue request. The MAP-user is not informed.

-	If an application-context-name different from version 1 is received in a syntactically correct TC-BEGIN indication primitive and if it is acceptable from a load control point of view, the MAP PM shall check whether the application-context-name is supported.

NOTE 3:	Unknown application-context-names are treated like unsupported ones.

If it is, the MAP PM shall issue a MAP-OPEN indication primitive with all parameters (application-context-name included) set according to the value of the corresponding parameter of the TC-BEGIN indication primitive.

The MAP PM shall then process any other indication primitives received from TC as described in section 12.6. Once all the received components have been processed, the MAP PM shall inform the local MAP service user by a MAP-DELIMITER indication primitive.

If the TC-BEGIN indication primitive is not associated with any component, the MAP PM shall inform the MAP User by a MAP-DELIMITER indication primitive.

Once all the received primitives have been processed, the MAP PM does not accept any primitive from the provider, except a TC-P-ABORT indication, and waits for a MAP-OPEN response primitive from its user.

-	If an application-context-name different from version 1 is received in a syntactically correct TC-BEGIN indication primitive and if it is acceptable from a load control point of view but the application-context-name is not supported, the MAP PM shall issue a TC-U-ABORT request primitive with abort-reason indicating "application-context-not-supported". If an alternative application-context-name cannot be offered, the received application-context-name shall be returned in the TC-U-ABORT Req primitive.

In the following cases an alternative application-context can be offered and its name included in the TC-U-ABORT Req primitive:

a)	If an application-context of version 2 or higher is requested, but only version 1 application-context supported, then the v1 application context shall be returned.

b)	If an application-context of version 3 or higher is requested, but only version 2 application-context supported, then the v2 application context shall be returned.

Operation		Application-context-name (NOTE 1)

updateLocation		networkLocUpContext-v1

cancelLocation		locationCancellationContext-v1

provideRoamingNumber	roamingNumberEnquiryContext-v1

insertSubscriberData	subscriberDataMngtContext-v1

deleteSubscriberData	subscriberDataMngtContext-v1

sendParameters		infoRetrievalContext-v1

networkLocUpContext-v1 (NOTE 2)

beginSubscriberActivity	networkFunctionalSsContext-v1

sendRoutingInformation	locationInfoRetrievalContext-v1

performHandover		handoverControlContext-v1

reset		resetContext-v1

activateTraceMode	tracingContext-v1

deactivateTraceMode	tracingContext-v1

sendRoutingInfoForSM	shortMsgGatewayContext-v1

forwardSM		shortMsgRelayContext-v1

reportSM-deliveryStatus	shortMsgGatewayContext-v1

noteSubscriberPresent	mwdMngtContext-v1

alertServiceCentreWithoutResult	shortMsgAlertContext-v1

checkIMEI		EquipmentMngtContext-v1

NOTE 1:	These symbolic names refer to object identifier value defined in section 14 and allocated to each application-context used for the MAP.

NOTE 2:	The choice between the application contexts is based on the parameters received in the operation.

Table 12.1/1: Mapping of V1 operation codes on to application-context-names

12.1.3	Receipt of a MAP-OPEN response

On receipt of a MAP-OPEN response primitive indicating that the dialogue is accepted, the MAP PM shall build a MAP-Accept PDU if the user-information parameter is included in the response primitive and accept any MAP specific service request or service response until a MAP-DELIMITER request or a MAP-CLOSE request is received from the MAP user. The MAP PM shall process the MAP specific primitives as described in section 12.6. The MAP PM shall then issue a TC-CONTINUE request primitive after it receives the MAP-DELIMITER request primitive if no MAP-CLOSE request primitive has been received, otherwise it shall issue a TC-END request primitive. In both cases the MAP-Accept PDU (if any) is included in the user-information parameter of the TC primitive.

If the dialogue is not associated with a version 1 application context, the MAP PM shall include the application-context-name in the TC primitive.

If no MAP-CLOSE request has been received, the MAP PM waits for a request primitive from its user or an indication primitive from TC.

On receipt of a MAP-OPEN response primitive indicating that the dialogue is not accepted, the MAP PM shall build a MAP-Refuse PDU and request its transfer using the TC-END req primitive.

12.1.4	Receipt of the first TC-CONTINUE ind

On receipt of the first TC-CONTINUE indication primitive for a dialogue, the MAP PM shall check the value of the application-context-name parameter (if any). If this value matches the one used in the MAP-OPEN request primitive, the MAP PM shall issue a MAP-OPEN confirm primitive with the result parameter indicating "accepted", then process the following TC component handling indication primitives as described in section 12.6, and then waits for a request primitive from its user or an indication primitive from TC, otherwise it shall issue a TC-U-ABORT request primitive with a MAP-providerAbort PDU indicating "abnormal dialogue" and a MAP-P-ABORT indication primitive with the "provider-reason" parameter indicating "abnormal dialogue".

12.1.5	Receipt of a TC-END ind

On receipt of a TC-END indication primitive in the dialogue initiated state, the MAP PM shall check the value of the application-context-name parameter (if any). If this value does not match the one used in the MAP-OPEN request primitive, the MAP PM shall discard any following component handling primitive and shall issue a MAP-P-ABORT indication primitive with the "provider-reason" parameter indicating "abnormal dialogue", otherwise it shall perform the following additional checks:

-	if a MAP-Refuse PDU is included as user-information, issue a MAP-OPEN confirm primitive with the result parameter set to "refused";

-	if no MAP dialogue PDU or a MAP-Accept PDU is included as user-information, issue a MAP-OPEN confirm primitive with the result parameter set to "accepted" and process the following TC component handling indication primitives as described in section 12.6;

then it shall issue a MAP-CLOSE indication primitive, once all the components handling primitives have been processed.

12.1.6	Receipt of a TC-U-ABORT ind

On receipt of a TC-U-ABORT indication primitive in the "Dialogue Initiated" state with an abort-reason parameter indicating "ApplicationContextNotSupported", the MAP PM shall issue a MAP-OPEN confirmation primitive with the result parameter indicating "Dialogue Refused" and the refuse-reason parameter indicating "ApplicationContextNotSupported".

On receipt of a TC_U_ABORT indication primitive in the "Dialogue Initiated" state with an abort-reason parameter indicating "User Specific" and without user information, the MAP PM shall issue a MAP-OPEN confirmation primitive with the result parameter indicating "Dialogue Refused" and the refuse-reason parameter indicating "Potential Version Incompatibility".

Receipt of a TC-U-ABORT indication primitive with abort-reason "User Specific" and with user information is described as part of abnormal termination (see section 12.4.2).

12.1.7	Receipt of a TC-P-ABORT ind

On receipt of a TC-P-ABORT indication primitive in the "Dialogue Initiated" state with a P-abort parameter indicating "Incorrect Transaction Portinon", the MAP PM shall issue a MAP-OPEN confirmation primitive with the result parameter indicating "Dialogue Refused" and the refuse reason parameter indicating "Potential Version Incompatibility".

On receipt of a TC-P-ABORT indication primitive in the "Dialogue Initiated" state with a P-abort parameter indicating "No Common Dialogue Portion", the MAP PM shall issue a MAP-P-ABORT indication primitive with the provider reason parameter indicating "Version Incompatibility".

Receipt of a TC-P-ABORT indication primitive with another P-abort parameter value is described as part of abnormal termination (see section 12.5.2).

12.2	Dialogue continuation

Once established the dialogue is said to be in a continuation phase.

Both MAP users can request the transfer of MAP APDUs until one of them requests the termination of the dialogue.

12.2.1	Sending entity

The MAP PM shall accept any MAP specific service request or response primitives and process them as described in section 12.6.

On receipt of a MAP-DELIMITER request primitive, the MAP PM shall issue a TC-CONTINUE request primitive.

12.2.2	Receiving entity

On receipt of a TC-CONTINUE indication primitive the MAP PM shall accept zero, one or several TC component handling indication primitives and process them as described in section 12.6.

12.3	Dialogue termination

Both the dialogue-initiator and the dialogue-responder have the ability to request the termination of a dialogue after it has been established.

The dialogue termination procedure is driven by the following events:

-	A MAP-CLOSE request primitive;

-	A TC-END indication primitive.

12.3.1	Receipt of a MAP-CLOSE request

On receipt of a MAP-CLOSE request primitive, the MAP PM shall issue a TC-END request primitive and, if applicable, return to idle the associated active SSMs. Note that the TC-END request primitive will trigger the transmission of components associated with any user specific request or response primitives which may have been issued after the last MAP-DELIMITER request.

12.3.2	Receipt of a TC-END indication

On receipt of a TC-END indication primitive, the MAP shall accept any component handling indication primitives and process them as described in section 12.6.

Once all the received primitives have been processed, the MAP PM shall return to idle the associated SSMs and issue a MAP-CLOSE indication primitive.

12.4	User Abort

Both the dialogue-initiator and the dialogue-responder have the ability to abort a dialogue at any time.

The user abort procedure is driven by one of the following events:

-	A MAP-U-ABORT request primitive;

-	A TC-U-ABORT indication primitive carrying a MAP-user-abort PDU.

12.4.1	MAP-U-ABORT request

On receipt of a MAP-U-ABORT request the MAP PM shall construct a MAP-user-abort PDU from the user-reason and diagnostic parameters and issue a TC-U-ABORT request primitive. All state machines associated with the dialogue are returned to idle.

12.4.2	TC-U-ABORT ind

On receipt of a TC-U-ABORT indication carrying a MAP-user-abort PDU, the MAP PM shall issue a MAP-U-ABORT indication primitive. The user-reason and diagnostic information elements are mapped to the corresponding parameters of the MAP-U-ABORT indication primitive.

All state machines associated with the dialogue are returned to idle.

12.5	Provider Abort

The MAP has the ability to abort a dialogue at both the dialogue-initiator side and the dialogue-responder side.

The provider abort procedure is driven by one of the following events:

-	A MAP PM error situation;

-	A TC-P-ABORT indication primitive;

-	A TC-U-ABORT indication primitive carrying a MAP-abort PDU.

12.5.1	MAP PM error situation

In the case of an abnormal situation detected at the MAP level during an established dialogue, the MAP PM shall:

-	issue a MAP-P-ABORT indication primitive with the appropriate value of the provider-reason parameter.;

-	construct a MAP-abort PDU from the value of these parameters and request its transfer using a TC-U-ABORT request primitive.

12.5.2	TC-P-ABORT ind

On receipt of a TC-P-ABORT indication, the MAP PM shall issue a MAP-P-ABORT indication primitive.

All state machines associated with the dialogue are returned to idle.

12.5.3	TC-U-ABORT ind

On receipt of a TC-U-ABORT indication carrying a MAP-abort PDU, the MAP PM shall issue a MAP-P-ABORT indication primitive, with the appropriate value of the provider-reason parameter. The source parameter shall indicate "MAP-provider".

All state machines associated with the dialogue are returned to idle.

12.6	Procedures for MAP specific services

This section describes the MAP procedures for MAP specific services.

These procedures are driven by the following types of events

-	a MAP specific request or a MAP specific MAP response primitive;

-	a component handling primitive from TC.

A Service State Machine is activated on receipt of one of the following signals:

-	a MAP request primitive, which activates a requesting SSM;

-	a TC-INVOKE indication primitive without linked identifier, which activates a responding SSM.

For component handling primitives there are two types of events:

-	Events which activate a Service State Machine or which can be related to an existing one. The procedure elements driven by these events are described in sections 12.6.1 to 12.6.4.

-	Events which cannot be related to a Service State Machine. The procedure elements driven by these events are described in section 12.6.5.

12.6.1	Service invocation

The MAP specific procedures are initiated by the MAP request primitives.

On receipt of a MAP request primitive, the MAP PM shall build an operation argument from the parameters received in the request primitive and request the invocation of the associated operation using the TC-INVOKE procedure. If a linked ID parameter is inserted in the primitive this indicates a child service and implies that the operation on which the service is mapped is linked to the operation on which the parent service is mapped.

The mapping of MAP specific services on to remote operations is given in table 13.2/1.

12.6.2	Service invocation receipt

On receipt of a TC-INVOKE indication primitive, the MAP PM shall:

-	If the invoke ID is already in use by an active service, request the transfer of a reject component using the TC-U-REJECT request primitive with the appropriate problem code (duplicated invokeID) and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event received from the peer".

-	If the operation code does not correspond to an operation supported by the application-context, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (unrecognized operation), and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event received from the peer".

-	If a linked ID is included, perform the following checks: If the operation referred to by the linked ID does not allow linked operations or if the operation code does not correspond to a permitted linked operation, issue a TC-U-REJECT request primitive with the appropriate problem code (linked response unexpected or unexpected linked operation) and issue a MAP-NOTICE indication primitive with an appropriate diagnostic "abnormal event received from the peer".

-	If the type of the argument is not the one defined for the operation, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter), and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event from the peer".

-	If the type of the argument is correct but the values of the information elements it contains do not permit the type of MAP service being invoked to be determined, request the transfer of an error component using the TC-U-ERROR request primitive with an error code set to "unexpected data value" and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event from the peer".

NOTE 1:	These checks are only relevant when there is not a one-to-one mapping between a service and an operation.

-	If the type of the argument is correct but information elements required for the service being invoked are missing, request the transfer of an error component using the TC-U-ERROR request primitive with an error code set to "data missing" and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event from the peer".

NOTE 2:	These checks are only relevant when there is not a one-to-one mapping between a service and an operation.

-	If the type of the argument is correct but contains information elements which are not relevant for the type of MAP service being invoked, request the transfer of an error component using the TC-U-ERROR request primitive with an error code set to "unexpected data value" and issue a MAP-NOTICE indication primitive with a diagnostic parameter set to "abnormal event from the peer".

NOTE 3:	These checks are only relevant when there is not a one-to-one mapping between a service and an operation.

-	Otherwise, issue the relevant MAP indication primitive to the MAP-service-user. If the service is to be user confirmed, the MAP PM waits for the corresponding response primitive.

12.6.3	Service response

For user confirmed services, the MAP PM shall accept a MAP response primitive and shall:

-	If no error indication is included in the primitive and the service maps on to a class 1 or 3 operation, construct a result information element from the parameters received and request its transfer using the TC-RESULT-L service and optionally the TC-RESULT-NL service.

The TC-RESULT-NL services shall be used when the user specific parameters of the response primitives cannot be transferred in a single signalling frame and no segmenting mechanism is available from the underlying layers. The MAP PM shall issue one or several TC-RESULT-NL request primitives followed by a TC-RESULT-L primitive. The user parameters shall be split so that each portion contains sufficient information to construct a value compatible with the type defined for the result of the associated operation.

-	If no error indication is included in the primitive and the service response maps on to a class 4 linked operation, construct an operation argument from the parameters received and request its transfer using the TC-INVOKE service for this class 4 linked operation. The operation to be invoked is deduced from the value of the result parameter of the service primitive.

-	If an error indication is included in the primitive and the service maps on to a class 1 or 2 operation, either issue a TC-U-REJECT request primitive if the user error parameter indicates "resource limitation" or "initiating release", or construct an error parameter from the parameters received and request its transfer using the TC-U-ERROR request primitive. The error code should be the one associated with the value of the user error parameter of the response primitive.

NOTE:	The only user-errors that a MAP user can generate in addition to the list of errors attached to the operation which is associated with the service are: resource limitation and initiating release. Any other abnormal situation is detected either by the TC entity or by the MAP entity.

-	If an error indication is received and the operation maps on to a class 3 operation, or if no error indication is received but the service maps on to a class 2 operation which has no class 4 linked operation, return the local service state machine to idle without requesting any service from TC.

12.6.4	Receipt of a response

A component handling indication primitive is considered as driving a response for a confirmed service if the invoke ID parameter value matches the one stored for the service, or if the linked ID parameter value matches the one stored for the service and the operation invoked is a class 4 operation. On receipt of a response (except a TC-L-CANCEL indication) for an unconfirmed service the MAP PM shall issue a MAP-NOTICE indication primitive with the appropriate provider-error (return result unexpected or return error unexpected).

12.6.4.1	Receipt of TC-RESULT-NL indication

If the type of the partial result parameter is not compatible with the one defined for the complete result of this operation, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter) and issue a confirm primitive with the provider-error parameter set to "invalid response received". The MAP PM shall also issue a TC-U-CANCEL request primitive so that all subsequent result components for this operation are discarded by TC.

Otherwise, store the value of the partial result parameter and wait for subsequent TC-RESULT-NL indication primitives until a TC-RESULT-L indication primitive is received.

12.6.4.2	Receipt of TC-RESULT-L indication

If the type of the result parameter is not the one defined for the result of this operation, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter), and issue a confirm primitive with the provider-error parameter set to "invalid response received".

If the type of the result parameter is correct but does not contain all the information elements required by the service associated with the invocation, issue a confirm primitive with the provider-error parameter set to "invalid response received".

NOTE 1:	These checks are only relevant when there is not a one-to-one mapping between a service and an operation.

If the type of the result parameter is correct but contains information elements which are not relevant for the service associated with the invocation are missing, issue a confirm primitive with the provider-error parameter set to "invalid response received".

NOTE 2:	These checks are only relevant when there is not a one-to-one mapping between a service and an operation.

Otherwise, issue a MAP confirm primitive to the MAP-service-user mapping the result parameter of the TC-RESULT-L primitive on to the MAP specific parameters.

If partial results have been previously received, the value of the partial result parameters shall also be taken into account before performing the three previous checks.

12.6.4.3	Receipt of TC-U-ERROR indication

If the error code is not defined for the MAP or is not one associated with the operation referred to by the invoke identifier, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (unrecognized error or unexpected error), and issue a confirm primitive with the provider-error parameter set to "invalid response received".

If the type of the error parameter is not the one defined for this error, request the transfer of a reject component using the TC-U-REJECT request primitive, with the appropriate problem code (mistyped parameter), and issue a confirm primitive with the provider-error parameter set to "invalid response received".

If the type of the error parameter is correct but does not contain all the information elements required by the service associated with the invocation, issue a confirm primitive with the provider-error parameter set to "invalid response received".

NOTE 1:	In some cases, it may be necessary to analyse the operation argument.

If the type of the error parameter is correct but its value includes information elements which are not relevant for the service associated with the invocation, issue a confirm primitive with the provider-error parameter set to "invalid response received".

NOTE 2:	In some cases, it may be necessary to analyse the operation argument.

Otherwise, issue a MAP confirm primitive to the MAP-service-user with the user-error parameter set according to the received error code. If applicable the error parameter is mapped to the diagnostic parameter.

12.6.4.4	Receipt of a TC-INVOKE indication

A TC-INVOKE indication primitive is considered as carrying a possible response to a specific service if the linked ID refers to an active specific service and the associated operation is a class 4 operation. Note that the presence of a linked ID parameter in a TC-INVOKE primitive requesting a non class 4 operation indicates a child service whose procedures are the same as the procedures for the parent service.

On receipt of a TC-INVOKE indication confirming an active service, the MAP PM shall:

-	If the operation code is not defined for MAP or if the operation referred to by the linked ID does not allow linked operations or if the operation code does not correspond to an allowed linked operation, issue a TC-U-REJECT request primitive with the appropriate problem code (unrecognised operation, linked response unexpected or unexpected linked operation). If the service is confirmed, the MAP shall also issue a Confirm primitive with provider error indication "unexpected response from the peer", otherwise it may issue a MAP-NOTICE indication primitive with an appropriate diagnostic "abnormal event received from the peer".

Otherwise issue a confirm primitive mapping the operation argument parameter to the user specific parameters and setting the result parameter according to the operation code of the linked operation.

12.6.4.5	Receipt of TC-U-REJECT indication

On receipt of a TC-U-REJECT indication primitive which affects a pending service, the MAP PM shall issue a MAP confirm primitive to the MAP-service-user with the appropriate value of the provider-error or user-error parameter.

The mapping of TC invoke problem codes on to MAP provider-error and user-error parameter values is described in section 13.

12.6.4.6	Receipt of a TC-L-REJECT indication

This event occurs when the local TC detects a protocol error in an incoming component which affects an active specific service.

On receipt of a TC-L-REJECT indicating "return result problem, unexpected return result", the MAP shall issue a confirm primitive with the parameter provider-error indicating "unexpected response from the peer".

On receipt of a TC-L-REJECT indicating "return error problem, unexpected error result", the MAP shall issue a confirm primitive with the parameter provider-error indicating "unexpected response from the peer".

NOTE that when the problem code indicates a general problem, it is considered that the event cannot be related to an existing SSM even if the invoke Id is provided by TC. This is because whether the invoke Id refers to a local or remote invocation is ambiguous. The behaviour of the MAP PM in such a case is described in section 12.6.5.3.

12.6.4.7	Receipt of a TC-L-CANCEL indication

On receipt of a TC-L-CANCEL indication, the MAP PM shall:

-	If the associated operation is a class 1 operation, issue a confirm primitive with the provider-error cause indicating: "no response from the peer".

-	If the associated operation is a class 2 operation and no linked operations are defined for this operation, issue a confirm primitive without parameter (i.e. indicating implicitly the successful completion of the service).

-	If the associated operation is a class 2 operation and has linked operations but none of them has been invoked, issue a confirm primitive with the provider-error parameter indicating "service completion failure".

-	If the associated operation is a class 2 operation and a linked operation invocation has already been received in response to this operation, ignore the primitive.

-	If the associated operation is a class 3 operation, issue a confirm primitive with the provider-error cause indicating: "service completion failure".

-	If the associated operation is a class 4 operation, ignore the primitive.

NOTE:	When a TC-L-CANCEL ind primitive is received before the dialogue has been confirmed (i.e. no backward message is received by the dialogue initiator node), the MAP PM shall first issue a MAP-OPEN Cnf primitive with the result parameter indicating "accepted" (which means that the dialogue is considered as being implicitly accepted). Then, as indicated above, the TC-L-CANCEL Indication is interpreted according to the class of the operation to which it refers.

12.6.5	Other events

This section describes the behaviour of the MAP PM on receipt of a component handling indication primitive which cannot be related to any service or which does not affect a pending one. The MAP user is only informed that an abnormal event occurred during the associated dialogue. It is up to the MAP user to abort, continue or terminate the dialogue.

12.6.5.1	Receipt of a TC-U-REJECT

On receipt of a TC-U-REJECT indication primitive which does not affect an active SSM (i.e. indicating a return result or return error problem), the MAP PM shall issue a MAP-NOTICE indication primitive with the diagnostic parameter set to "response rejected by the peer".

This is also applicable for invoke problems related to a class 4 linked operation.

12.6.5.2	Receipt of a TC-R-REJECT indication

On receipt of a TC-R-REJECT indication (i.e. when a protocol error has been detected by the peer TC entity) which does not affect an active SSM, the MAP PM shall either discard this indication or issue a MAP-NOTICE indication primitive with the provider-error indicating "abnormal event detected by the peer".

In case of notification, it is up to the MAP user to continue, abort or terminate the dialogue. Note also that for MAP V1 the reject component is received in an END message and therefore the dialogue is terminated anyway.

12.6.5.3	Receipt of a TC-L-REJECT indication

On receipt of a TC-L-REJECT indication primitive (i.e. when a protocol error has been detected by the local TC entity) which cannot be related to an active SSM, the MAP PM shall either discard this indication or issue a MAP-NOTICE indication primitive with the provider-error indicating "abnormal event received from the peer".

In case of notification, it is up to the MAP user to continue, or to terminate the dialogue and implicitly trigger the transmission of the reject component or to abort the dialogue.

12.6.6	Parameter checks

As described in the previous sections, the MAP PM performs a set of checks to ensure the correctness of the information elements received; these are:

-	check if the syntax and encoding (NOTE) of the operation argument, result or error parameter are correct.

NOTE:	Depending on the implementation, encoding problems on the TC user portion may be detected at TC level or by the MAP user. In the second case the problem is reported in a similar manner to a syntactical problem.

The syntax shall be considered incorrect if a mandatory information element is missing in any constructed element or if the value of an information element is out of the range defined for the type it is supposed to belong to;

-	if there is not a one-to-one mapping between a service and an operation:

i)	check if the value of the information elements (generally a single one) permits the MAP PM to determine the service associated with the operation invocation;

ii)	check that there are no information elements which are irrelevant for the indication or a confirm primitive to be issued;

-	check if all the information elements required to built an indication or a confirm primitive are available.

However some additional checks may have to be performed by the MAP user (see section 15).

12.6.7	Returning state machines to idle

Unlike TC invocation state machines, service state machines exist at both requestor and performer side.

A service state machine at the requestor side is returned to idle when the MAP-specific confirm primitive is issued or when the dialogue terminates.

A service state machine at the performer side is returned to idle on receipt of a MAP-specific response primitive from the MAP user, when the dialogue terminates or at expiry of an implementation dependent watch-dog timer which is started when the state machine is created.

12.6.8	Load control

As stated in the previous sections, before issuing a MAP-OPEN indication primitive the MAP PM performs a check to verify if there are sufficient resources to open the dialogue taking into account possible overload conditions.

The decision is based on the priority allocated to the application-context whose name is explicitly included in the TC-BEGIN indication primitive or implied by the first operation invocation when V1 contexts are in use. How a V1 application-context-name is derived from an operation code is described in table 12.1/1.

The priority level affected to each application-context is described in section 3 table 3.1/1.

13	Mapping on to TC services

13.1	Dialogue control

Dialogue control services are mapped to TC dialogue handling services. The TC-UNI service is not used by the MAP PM.

13.1.1	Directly mapped parameters

The following parameters of the MAP-OPEN request and indication primitives are directly mapped on to the corresponding parameters of the TC-BEGIN primitives:

-	destination address;

-	originating address.

13.1.2	Use of other parameters of dialogue handling primitives

13.1.2.1	Dialogue Id

The value of this parameter is associated with the MAP PM invocation in an implementation dependent manner.

13.1.2.2	Application-context-name

If it does not belong to the MAP V1 context set. The application-context-name parameter of a MAP primitive is mapped to the application-context-name parameter of TC dialogue handling primitives according to the rules described in section 12.1. Otherwise the application-context-name parameter of TC dialogue handling primitives is never included.

13.1.2.3	User information

The user information parameter of TC dialogue primitives is used to carry the MAP dialogue APDUs.

13.1.2.4	Component present

This parameter is used by the MAP PM as described in CCITT Recommendation Q.771. It is not visible to the MAP user.

13.1.2.5	Termination

The value of this parameter of the TC-END request primitive is set by the MAP PM on the basis of the release method parameter of the MAP-CLOSE request primitive.

13.1.2.6	P-Abort-Cause

Values of the P-abort-cause parameter are mapped to the values of the provider-reason parameter of the MAP-P-ABORT indication primitive according to table 13.1/1, except in the dialogue initiated phase for the "incorrectTransactionPortion" and "noCommonDialoguePortion" values which are mapped to the "potential incompatibility problem" value of the refuse-reason parameter of the MAP-OPEN cnf primitive. The source parameter in the MAP-P-ABORT ind takes the value "TC problem".

13.1.2.7	Quality of service

The quality of service of TC request primitives is set by the MAP to the following value:

-	No return option requested;

-	Sequencing requested.

TC P-Abort cause	MAP provider-reason

unrecognized message type	provider malfunction

unrecognized transaction Id	supporting dialogue released

badlyFormattedTransactionPortion	provider malfunction

incorrectTransactionPortion	provider malfunction (NOTE)

resourceLimitation	resource limitation

abnormalDialogue	provider malfunction

noCommonDialoguePortion	version incompatibility

NOTE:	Or version incompatibility in the dialogue initiated phase.

Table 13.1/1: Mapping of P-Abort cause in TC-P-ABORT indication on to provider-reason in MAP-P-ABORT indication

13.2	Service specific procedures

Specific services are mapped to TC component handling services.

13.2.1	Directly mapped parameters

The Invoke Id parameter of the MAP request and indication primitive is directly mapped on to the Invoke Id parameter of the component handling primitives.

13.2.2	Use of other parameters of component handling primitives

13.2.2.1	Dialogue Id

The value of this parameter is associated with the MAP PM invocation in an implementation dependent manner.

13.2.2.2	Class

The value of this parameter is set by the MAP PM according to the type of the operation to be invoked.

13.2.2.3	Linked Id

When a service response is mapped to a class 4 operation, the value of this parameter is set by the MAP PM and corresponds to the value assigned by the user to the initial service request (i.e. the value of the invoke ID parameter of the request primitive). Otherwise if such a parameter is included in MAP request/indication primitives it is directly mapped to the linked ID parameter of the associated TC─INVOKE request/indication primitives.

13.2.2.4	Operation

When mapping a request primitive on to a Remote Operations PDU (invoke), the MAP PM shall set the operation code according to the mapping described in table 13.2/1.

When mapping a response primitive on to a Remote Operations service, the MAP PM shall set the operation code of the TC-RESULT-L/NL primitive (if required) to the same value as the one received at invocation time.

MAP-SERVICE	operation

MAP-ACTIVATE-SS	activateSS

MAP-ACTIVATE-TRACE-MODE	activateTraceMode

MAP-ALERT-SERVICE-CENTRE	alertServiceCentre

MAP-CANCEL-LOCATION	cancelLocation

MAP-CHECK-IMEI	checkIMEI

MAP-DEACTIVATE-SS	deactivateSS

MAP-DEACTIVATE-TRACE-MODE 	deactivateTraceMode

MAP-DELETE-SUBSCRIBER-DATA	deleteSubscriberData

MAP-ERASE-SS	eraseSS

MAP-FORWARD-ACCESS-SIGNALLING	forwardAccessSignalling

MAP-FORWARD-CHECK-SS-INDICATION	forwardCheckSsIndication

MAP-FORWARD-SHORT-MESSAGE	forwardSM

MAP-GET-PASSWORD	getPassword

MAP-INFORM-SERVICE-CENTRE	informServiceCentre

MAP-INSERT-SUBSCRIBER-DATA	insertSubscriberData

MAP-INTERROGATE-SS	interrogateSs

MAP-READY-FOR-SM	readyForSM

MAP-PREPARE-HANDOVER	prepareHandover

MAP-PREPARE-SUBSEQUENT-HANDOVER	prepareSubsequentHandover

MAP-PROCESS-ACCESS-SIGNALLING	processAccessSignalling

MAP-PROCESS-UNSTRUCTURED-SS-REQUEST	processUnstructuredSS-Request

MAP-PROVIDE-ROAMING-NUMBER	provideRoamingNumber

MAP-PURGE-MS	purgeMS

MAP-REGISTER-PASSWORD	registerPassword

MAP-REGISTER-SS	registerSS

MAP-RESET	reset

MAP-RESTORE-DATA	restoreData

MAP-SEND-END-SIGNAL 	sendEndSignal

MAP-SEND-AUTHENTICATION-INFO	sendParameter (NOTE)

sendAuthenticationInfo

MAP-SEND-IMSI	sendIMSI

MAP-SEND-IDENTIFICATION	sendIdentification

MAP-SEND-ROUTING-INFO-FOR-SM	sendRoutingInfoForSM

MAP-SEND-ROUTING-INFORMATION	sendRoutingInformation

MAP-REPORT-SM-DELIVERY-REPORT	reportSmDeliveryReport

MAP-UNSTRUCTURED-SS-NOTIFY	unstructuredSS-Notify

MAP-UNSTRUCTURED-SS-REQUEST	unstructuredSS-Request

MAP-UPDATE-LOCATION	updateLocation

NOTE:	In V1 Context only.

Table 13.2/1: Mapping of MAP specific services on to MAP operations

13.2.2.5	Error

The error parameter in a TC-U-ERROR indication primitive is mapped to the user-error parameter in the MAP confirm primitive of the service associated with the operation to which the error is attached.

The user-error parameter in MAP response primitives is mapped to the error parameter of the TC─U─ERROR request primitive, except for "initiating-release" and "resource-limitation" which are mapped to the problem code parameter of the TC-U-REJECT request primitive.

13.2.2.6	Parameters

The parameters of MAP specific request and indication primitives are mapped to the argument parameter of TC-INVOKE primitives.

The parameters of MAP specific response and confirm primitives are mapped to the result parameter of TC-RESULT-L primitives, the parameter of TC-U-ERROR primitives or the argument of TC-INVOKE primitives when mapping on linked class 4 operations is used.

13.2.2.7	Time out

The value of this parameter is set by the MAP PM according to the type of operation invoked.

13.2.2.8	Last component

This parameter is used by the MAP PM as described in CCITT Recommendation Q.711. It is not visible from the MAP user.

13.2.2.9	Problem code

13.2.2.9.1	Mapping to MAP user-error

The following values of the user error parameter are mapped as follows to values of the TC problem code parameter. These values are generated by the MAP user. This mapping is valid from the TC-U-REJECT indication primitive to the MAP confirm service primitive and from the MAP response service primitive to the TC-U-REJECT request primitive.

MAP user-error		TC problem code

resource limitation	resource limitation

initiating release		initiating release

Table 13.2/2: Mapping of MAP user-error parameter on to TC problem code in TC-U-REJECT primitives

13.2.2.9.2	Mapping to MAP provider-error parameter

The following values of the TC problem code parameter of the TC-U-REJECT indication primitive are mapped as follows to values of the MAP provider-error parameter of the MAP confirm primitive.

TC problem code	MAP provider-error

duplicated invoke Id	duplicated invoke id

unrecognized operation	service not supported

mistyped parameter	mistyped parameter

Table 13.2/3: Mapping of TC problem code in TC-U-REJECT on to MAP provider-error parameter

The following values of the problem code parameters of the TC-L-REJECT primitive are mapped to values of the provider-error parameter of the MAP confirm primitive as follows:

TC problem code	MAP provider-error

return result unexpected	unexpected response from the peer

return error unexpected	unexpected response from the peer

Table 13.2/4: Mapping of TC problem code in TC-L-REJECT on to MAP provider-error parameter

13.2.2.9.3	Mapping to diagnostic parameter

The following values of the problem code parameter of the TC-R-REJECT and TC-U-REJECT primitive are mapped to values of the diagnostic parameter of the MAP-NOTICE indication primitive as follows:

TC problem code	MAP diagnostic

General problem		abnormal event detected by the peer

Invoke problem

- unrecognized linked ID	- abnormal event detected by the peer

- linked response unexpected	- response rejected by the peer

- unexpected linked operation	- response rejected by the peer

Return result problem

- unrecognized invoke ID	- response rejected by the peer

- return result unexpected	- response rejected by the peer

- mistyped parameter	- response rejected by the peer

Return error problem

- unrecognized invoke ID	- response rejected by the peer

- return error unexpected	- response rejected by the peer

- unrecognized error	- response rejected by the peer

- unexpected error	- response rejected by the peer

- mistyped parameter	- response rejected by the peer

Table 13.2/5: Mapping of TC problem code of TC-R-REJECT and TC-U-REJECT on to diagnostic parameter

The following values of the problem code parameter of the TC-L-REJECT primitive are mapped to values of the diagnostic parameter of the MAP-NOTICE indication primitive as follows:

TC problem code	MAP diagnostic

General problems:	- abnormal event received from the peer

Invoke problem:

- unrecognized linked ID	- abnormal event received from the peer

Return result problem:

- unrecognized invoke ID	- abnormal event received from the peer

Return error problem:

- unrecognized invoke ID	- abnormal event received from the peer

Table 13.2/6: Mapping of TC problem code of TC-L-REJECT on to diagnostic parameter

SDL Descriptions

The following SDL specification describes a system which includes three blocs: MAP-user, MAP─provider and TC.

Such a system resides in each network component supporting MAP and communicates with its peers via the lower layers of the signalling network which are part of the environment.

Only the MAP-provider is fully described in this section. The various type of processes which form the MAP-User bloc and the TC bloc are described respectively in sections 15 to 21 of this specification and in CCITT Recommendation Q.774.

The MAP-Provider bloc communicates with the MAP_USER via two channels U1 and U2. Via U1 the MAP-provider receives the MAP request and response primitives. Via U2 it sends the MAP indication and confirm primitives.

The MAP-Provider bloc communicates with TC via two channels P1 and P1. Via P1 the MAP-Provider sends all the TC request primitives. Via P2 it receives all the TC indication primitives.

The MAP-Provide bloc is composed on the four following types of processes:

a)	MAP_DSM: This type of process handles a dialogue. There exists one process instance per MAP dialogue.

b)	LOAD_CTRL: This type of process is in charge of load control. There is only one instance of this process in each system.

c)	PERFORMING_MAP_SSM: This type of process handle a MAP service performed during a dialogue. Instance of this process are created by the an instance of the MAP_DSM process for each MAP-service to be performed.

d)	REQUESTING_MAP_SSM: This type of process handle a MAP service requested during a dialogue. Instance of this process are created by the an instance of the MAP_DSM process for each requested MAP-service.

A process MAP_DSM exchange external signals with other blocs as well as internal signals with the other processes of the MAP-Provider bloc. The external signals are either MAP service primitives or TC service primitives.

The signal routes used by the various processes are organized as follows:

a)	A process MAP_DSM receives and sends events from/to the MAP_user via signal route User1/User2. These routes uses respectively channel U1 and U2.

b)	A process MAP_DSM receives and sends events from/to the TC via signal route Tc1/Tc2. These routes uses respectively channel P1 and P2.

c)	A process MAP_DSM receives and sends events from/to the LOAD_CTRL process via signal route Load1/Load2. These routes are internal.

d)	A process MAP_DSM sends events to the PERFORMING_MAP_SSM processes via signal route Intern1. This route is internal.

e)	A process MAP_DSM sends events to the REQUESTING_MAP_SSM processes via signal route Intern2. This route is internal.

f)	A process MAP_PERFORMING_SSM sends events to the MAP_USER via signal route User4. This route uses channel U2.

g)	A process MAP_PERFORMING_SSM sends events to TC via signal route Tc3. This route uses channel P1.

h)	A process MAP_REQUESTING_SSM sends events to the MAP_USER via signal route User5. This route uses channel U2.

i)	A process MAP_REQUESTING_SSM sends events to TC via signal route Tc4. This route uses channel P1.

Figure 13.2/1: System MAP_STACK

.G.C:\USER\MAP\MAP-470\EPS13\132_1.EPS;18.2 cm;23 cm;EPS

Figure 13.2/2: Block MAPPROVIDER

.G.C:\USER\MAP\MAP-470\EPS13\132_2.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 1: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_31.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 2: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_32.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 3: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_33.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 4: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_34.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 5: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_35.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 6: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_36.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 7: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_37.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 8: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_38.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 9: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_39.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 10: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_3A.EPS;18.2 cm;23 cm;EPS

Figure 13.2/3 sheet 11: Process MAP_DSM

.G.C:\USER\MAP\MAP-470\EPS13\132_3B.EPS;18.2 cm;23 cm;EPS

Figure 13.2/4 sheet 1: Procedure PROCESS_COMPONENTS

.G.C:\USER\MAP\MAP-470\EPS13\132_41.EPS;18.2 cm;23 cm;EPS

Figure 13.2/4 sheet 2: Procedure PROCESS_COMPONENTS

.G.C:\USER\MAP\MAP-470\EPS13\132_42.EPS;18.2 cm;23 cm;EPS

Figure 13.2/4 sheet 3: Procedure PROCESS_COMPONENTS

.G.C:\USER\MAP\MAP-470\EPS13\132_43.EPS;18.2 cm;23 cm;EPS

Figure 13.2/4 sheet 4: Procedure PROCESS_COMPONENTS

.G.C:\USER\MAP\MAP-470\EPS13\132_44.EPS;18.2 cm;23 cm;EPS

Figure 13.2/5: Process LOAD_CTRL

.G.C:\USER\MAP\MAP-470\EPS13\132_5.EPS;18.2 cm;23 cm;EPS

Figure 13.2/6 sheet 1: Process PERFORMING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_61.EPS;18.2 cm;23 cm;EPS

Figure 13.2/6 sheet 2: Process PERFORMING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_62.EPS;18.2 cm;23 cm;EPS

Figure 13.2/6 sheet 3: Process PERFORMING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_63.EPS;18.2 cm;23 cm;EPS

Figure 13.2/7 sheet 1: Process REQUESTING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_71.EPS;18.2 cm;23 cm;EPS

Figure 13.2/7 sheet 2: Process REQUESTING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_72.EPS;18.2 cm;23 cm;EPS

Figure 13.2/7 sheet 3: Process REQUESTING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_73.EPS;18.2 cm;23 cm;EPS

Figure 13.2/7 sheet 4: Process REQUESTING_MAP_SSM

.G.C:\USER\MAP\MAP-470\EPS13\132_74.EPS;18.2 cm;23 cm;EPS

14	Abstract syntax of the MAP protocol

14.1	General

This section specifies the Abstract Syntaxes for the Mobile Application Part as well as the associated set of Operations and Errors, using the Abstract Syntax Notation One (ASN.1), defined in CCITT Recommendation X.208 (1988) with additions as defined in section 14.1.4 on Compatibility Considerations and the OPERATION and ERROR external MACROs, defined in CCITT Recommendation Q.773.

The Mobile Application Part protocol is defined by two Abstract Syntaxes.

-	One Abstract Syntaxes which encompass all Operations and Errors identified by the various MAP subsystem numbers.

This Abstract Syntax represents the set of values each of which is a value of the ASN.1 type TCAPMessages. MessageType as defined in CCITT Recommendation Q.773 with the ANY DEFINED BY clauses resolved by the Operation and Error codes included in the ASN.1 module MAP-Protocol. However, only the subset of this abstract syntax which is required by the procedures defined for an entity needs to be supported.

-	One Abstract Syntax identified by the OBJECT IDENTIFIER value MAP-DialogueInformation.map-DialogueAS.

This Abstract Syntax represents the set of values each of which is a value of the ASN.1 type MAP-DialogueInformation.MAP-DialoguePDU. Such a value of the ASN.1 single-ASN.1-type element is contained within the user-information element of the TCAPMessages.DialoguePortion ASN.1 type. This Abstract Syntax name is to be used as a direct reference.

14.1.1	Encoding Rules

The encoding rules which are applicable to the defined Abstract Syntaxes are the Basic Encoding Rules for Abstract Syntax Notation One, defined in CCITT Recommendation X.209 with the same exceptions as in CCITT Recommendation Q.773 section 4 Message Representation.

NOTE:	When the definite form is used for length encoding, a data value of length less than 128 octets must have the length encoded in the short form.

When the long form is employed to code a length, the minimum number of octets shall be used to code the length field.

OCTET STRING values and BIT STRING values must be encoded in a primitive form.

14.1.2	Use of TC

The mapping of OPERATION and ERROR to TC components is defined in ETS 300 287 (version 2) which is based on CCITT Recommendation Q.773 (1992).

NOTE:	The class of an operation is not stated explicitly but is specified as well in the ASN.1 operation type definition.

Class 1: RESULT and ERROR appear in ASN.1 operation type definition.

Class 2: only ERROR appears in ASN.1 operation type definition.

Class 3: only RESULT appears in ASN.1 operation type definition.

Class 4: both RESULT and ERROR do not appear in ASN.1 operation type definition.

The ASN.1 data type which follows the keywords "ARGUMENT" or "RESULT" (for OPERATION and ERROR) is always optional from a syntactic point of view. However, except when specifically mentioned, it has to be considered as mandatory from a semantic point of view.

When an optional element is missing in an invoke component or in an inner data structure while it is required by the context, an error component is returned if specified in the operation type; the associated type of error is DataMissing. This holds also when the entire parameter of an invoke component is missing while it is required by the context.

NOTE:	When a mandatory element is missing in the parameter or inner data structure of any component, a reject component is returned (if the dialogue still exists). The problem code to be used is "Mistyped parameter".

The Timer Values used in the operation type definitions are indicated as ASN.1 comment. The Timer Value Ranges are:

s	= from 3 seconds to 10 seconds;

m	= from 15 seconds to 30 seconds;

ml	= from 1 minute to 10 minutes;

l	= from 28 hours to 38 hours.

14.1.3	Use of information elements defined outside MAP

An information element or a set of information elements (messages) transparently carried in the Mobile Application Part but defined in other recommendation/technical specifications are handled in one of the following ways:

i)	The contents of each information element (without the octets encoding the identifier and the length in the recommendation/technical specification where it is defined) is carried as the value of an ASN.1 NamedType derived from the OCTET STRING data type. Additionally, the internal structure may be explained by means of comments. In case of misalignment the referred to recommendation/technical specification takes precedence.

ii)	The complete information element (including the octets encoding the identifier and the length in the recommendation/technical specification where it is defined) or set of information elements and the identity of the associated protocol are carried as the value of the ExternalSignalInfo data type defined in this technical specification. Where more than one information element is carried, the information elements are sent contiguously with no filler octets between them.

14.1.4	Compatibility considerations

The following ASN.1 modules are conformant with CCITT Recommendation X.208 (1988), but in addition Ellipsis Notation ("..." - notation) is used as described in ISO 8824-1/PDAM 3.2 wherever future protocol extensions are foreseen.

The "..." construct only applies to a SEQUENCE. An entity supporting a version greater 1 shall not reject unsupported extension following "..." of that SEQUENCE. The Encoding Rules from section 14.1.1 apply to every element of the whole Transfer Syntax especially to the ASN.1 type EXTERNAL.

Note that the Ellipses Notation is not supported in a version 1 context. Therefore a SEQUENCE sent in a version 1 context shall not include any element following "...".

NOTE:	Every extension has to follow the superset mechanism defined in ETR 060.

Any change other than in the extension area leads to a new Application Context version.

PLMN specific extensions shall follow the extension marker and shall be tagged using PRIVATE tags up to and including 30.

The Abstract Syntaxes of MAP version 2 are in general an extension of the Abstract Syntaxes of MAP version 1. They are therefore (in general) backward compatible with MAP v1 from the Abstract Syntax point of View. Exceptions from this backward compatible changes of the Abstract Syntax were made only for the following reasons:

i)	information element was not used by a GSM phase 1 service due to missing or unstable phase 1 service description (e.g. Closed User Group);

ii)	information element used only on the MSC-VLR Interface (e.g. SendInformationForIncomingCallSetUp);

iii)	changes with impact only in the error situation (e.g. different Cause Values);

iv)	changes on the MAP-AbortInfo.

Changes without impact on the Transfer Syntax are not indicated (e.g. name changes, introduction of extension marker).

Information Elements added in existing Constructs for version 2 or information elements kept only for compatibility with version 1 are indicated by ASN.1 comments These comments state which subset of the abstract syntax must be used in the negotiated application context.

14.1.5	Structure of the Abstract Syntax of MAP

For each MAP parameter which has to be transferred by a MAP Protocol Data Unit (MAP message), there is a PDU field (an ASN.1 NamedType) whose ASN.1 identifier has the same name as the corresponding parameter, except for the differences required by the ASN.1 notation (blanks between words are removed or replaced by hyphen, the first letter of the first word is lower-case and the first letter of the following words are capitalized, e.g. "no reply condition time" is mapped to "noReplyConditionTime"). Additionally some words may be abbreviated as follows:

bs	basic service

ch	call handling

cug	closed user group

ho	handover

ic	incoming call

id	identity

info	information

ms	mobile service

oc	outgoing call

om	operation & maintenance

pw	Password

sm	short message service

ss	supplementary service

The MAP protocol is composed of several ASN.1 modules dealing with either operations, errors, data types, and, if applicable, split into those dealing with mobile services, call handling services, supplementary services and short message services. For operations and errors no values are assigned, but only the operation and error types in order to allow use of the defined types also by other protocols (e.g. TS GSM 04.80). The values (operation codes and error codes) are defined in a separate module. The ASN.1 source lines are preceded by line-numbers at the left margin in order to enable the usage of the cross-reference in appendix A.

The module containing the definition of the operation packages for MAP is:

1.	MAP-OperationPackages.

The module containing the definition of the application contexts for MAP is:

2.	MAP-ApplicationContexts.

The module containing the data types for the Abstract Syntax to be used for TCAPMessages.DialoguePortion for MAP is:

3.	MAP-DialogueInformation.

The module containing the operation codes and error codes for MAP is:

4.	MAP-Protocol.

The modules containing all operation type definitions for MAP are:

5.	MAP-MobileServiceOperations;

6.	MAP-OperationAndMaintenanceOperations;

7.	MAP-CallHandlingOperations;

8.	MAP-SupplementaryServiceOperations;

9.	MAP-ShortMessageServiceOperations.

The module containing all error type definitions for MAP is:

10.	MAP-Errors.

Modules containing all data type definitions for MAP are:

11.	MAP-MS-DataTypes;

12.	MAP-OM-DataTypes;

13.	MAP-CH-DataTypes;

14.	MAP-SS-DataTypes;

15.	MAP-SS-Code;

16.	MAP-SM-DataTypes;

17.	MAP-ER-DataTypes;

18.	MAP-CommonDataTypes;

19.	MAP-TS-Code;

20.	MAP-BS-Code.

References are made also to modules defined outside of this technical specification. They are defined in the technical specification Mobile Services Domain and technical specification Transaction Capability respectively:

MobileDomainDefinitions;

TCAPMessages;

DialoguePDUs.

In the word-text of ASN.1 Modules hidden text is used for marking the begin (.$modulename), interrupt (.#), continuation (.$) and end (.#END) of ASN.1 Source. This allows to automatically extract the ASN.1 Sources. These markers should not be deleted! In addition, hidden text is also used to overcome some compilerrestrictions (e.g. Empty Result not supported due to LL1-Problem).

In addition no linebreak should occur when printing a paragraph in ASN.1 source, otherwise the line-numbering of word is inconsistent with the line-numbering in Appendix A.

For Completeness the module MAP-Frame is included as hidden text.

.$MAP-Frame

DEFINITIONS ::=

BEGIN

IMPORTS

Component,

MessageType

FROM TCAPMessages

dialogue-as-id,

DialoguePDU

FROM DialoguePDUs

updateLocation

FROM MAP-Protocol

map-DialogueAS,

MAP-DialoguePDU

FROM MAP-DialogueInformation

map-ac

FROM MAP-ApplicationContexts

;

ZZZZ-Dummy ::= NULL

.#END

14.2	Operation packages

14.2.1	General aspects

This section describes the operation-packages which are used to build the application-contexts defined in section 14.3.

Each operation-package is a specification of the roles of a pair of communicating objects (i.e. a pair of MAP-Providers), in term of operations which they can invoke of each other.

The grouping of operations into one or several packages does not necessarily imply any grouping in term of Application Service Elements.

The following ASN.1 MACRO is used to describe operation-packages in this section:

OPERATION-PACKAGE MACRO ::=

BEGIN

TYPE NOTATION ::= Symmetric | ConsumerInvokes SupplierInvokes |

empty

VALUE NOTATION ::= value(VALUE OBJECT IDENTIFIER)

Symmetric ::= "OPERATIONS" "{" OperationList "}"

ConsumerInvokes ::= "CONSUMER INVOKES" "{" OperationList "}"

SupplierInvokes ::= "SUPPLIER INVOKES" "{" OperationList "}" | empty

OperationList ::= Operation | OperationList "," Operation

Operation ::= value(OPERATION)

END

Since the application-context definitions provided in section 14.3 uses only an informal description technique, only the type notation is used in the following sections to define operation-packages.

The following definitions are used throughout this section:

-	v1-only operation: An operation which shall only be used in v1 application-contexts.

-	v2-only operation: An operation which shall only be used in v2 application-contexts.

-	v2-only package: An operation package which contains solely v2-only operations.

-	v1-operation: An operation whose specification has not been modified since the MAP v1 specifications or if the modifications are not considered as affecting V1 implementations.

-	v1-equivalent operation: The version of an operation which excludes all the information elements and errors which has been added since MAP v1 specifications.

-	v1-package: An operation package which contains only v1- operations.

The names of v1-packages are suffixed by "-v1" while other names are suffixed by "-v2".

For each operation package which is not v2-only and which does not include only v1-operations, there is a v1-equivalent package. Except when a definition is explicitly provided in the following sub-sections, the v1-equivalent package includes the v1-equivalent operations of the operations which are belong to this package.

14.2.2	Packages specifications

14.2.2.1	Location updating

This operation package includes the operations required for location management procedures between HLR and VLR.

LocationUpdatingPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

updateLocation}

SUPPLIER INVOKES {

forwardCheckSs-Indication}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.2	Location cancellation

This operation package includes the operations required for location cancellation and MS purging procedures between HLR and VLR.

LocationCancellationPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

cancelLocation}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.3	Roaming number enquiry

This operation package includes the operations required for roaming number enquiry procedures between HLR and VLR.

RoamingNumberEnquiryPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

provideRoamingNumber}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.4	Information retrieval

This operation package includes the operations required for information retrieval procedures between HLR and VLR.

InfoRetrievalPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

sendAuthenticationInfo,

sendIMSI}

The v1-equivalent package is defined as follows:

InfoRetrievalPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is HLR or VLR if Consumer is VLR

CONSUMER INVOKES {

sendParameters}

14.2.2.5	Inter-VLR information retrieval

This operation package includes the operations required for inter VLR information retrieval procedures.

InterVlrInfoRetrievalPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is VLR

CONSUMER INVOKES {

sendIdentification}

The v1-equivalent package is : InfoRetrievalPackage-v1

14.2.2.6 - 14.2.2.9 [spare]

14.2.2.10	Interrogation

This operation package includes the operations required for interrogation procedures between MSC and HLR.

InterrogationPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is MSC

CONSUMER INVOKES {

sendRoutingInfo}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.11	[spare]

14.2.2.12	Handover Control

This operation package includes the operations required for handover procedures between MSCs.

HandoverControlPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is MSCB if Consumer is MSCA

CONSUMER INVOKES {

prepareHandover,

forwardAccessSignalling}

SUPPLIER INVOKES {

sendEndSignal,

processAccessSignalling,

prepareSubsequentHandover}

The v1-equivalent package is defined as follows.

HandoverControlPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is MSCB if Consumer is MSCA

CONSUMER INVOKES {

performHandover,

forwardAccessSignalling,

traceSubscriberActivity}

SUPPLIER INVOKES {

sendEndSignal,

noteInternalHandover,

processAccessSignalling,

performSubsequentHandover}

14.2.2.13	Subscriber Data management stand alone

This operation package includes the operations required for stand alone subscriber data management procedures between HLR and VLR.

SubscriberDataMngtStandAlonePackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

insertSubscriberData,

deleteSubscriberData}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.14	Equipment management

This operation package includes the operations required for equipment management procedures between EIR and MSC.

EquipmentMngtPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is EIR if Consumer is MSC

CONSUMER INVOKES {

checkIMEI}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.15	Subscriber Data management

This operation package includes the operations required for subscriber data management procedures between HLR and VLR.

SubscriberDataMngtPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

insertSubscriberData}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.16	Location register restart

This operation package includes the operations required for location register restart procedures between HLR and VLR.

ResetPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

reset}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.17	Tracing stand-alone

This operation package includes the operations required for stand alone tracing procedures between HLR and VLR.

TracingStandAlonePackage-v1 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

activateTraceMode,

deactivateTraceMode}

14.2.2.18	Functional SS handling

This operation package includes the operations required for functional supplementary services procedures between VLR and HLR.

FunctionalSsPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

registerSS,

eraseSS,

activateSS,

deactivateSS,

registerPassword,

interrogateSS}

SUPPLIER INVOKES {

getPassword}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.19	Tracing

This operation package includes the operations required for tracing procedures between HLR and VLR.

TracingPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is VLR if Consumer is HLR

CONSUMER INVOKES {

activateTraceMode}

14.2.2.20	Binding

This operation package includes the operation required to initialize a supplementary service procedure between VLR and HLR.

BindingPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

beginSubscriberActivity}

This package is v1 only.

14.2.2.21	Unstructured SS handling

This operation package includes the operations required for unstructured supplementary services procedures between VLR and HLR.

UnstructuredSsPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

processUnstructuredSS-Request}

SUPPLIER INVOKES {

unstructuredSS-Request,

unstructuredSS-Notify}

The v1-equivalent package is defined as follows:

UnstructuredSsPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

processUnstructuredSS-Data}

14.2.2.22	Short message relay services

This operation package includes the operations required for short message relay service procedures between IWMSC and VMSC or between GMSC and MSC.

ShortMsgRelayPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is IWMSC if Consumer is MSC

-- Supplier is MSC if Consumer is GMSC

CONSUMER INVOKES {

forwardSM}

The v1-equivalent package can be determined according to the rules described in section 14.2.1.

14.2.2.23	Short message gateway services

This operation package includes the operations required for short message service gateway procedures between MSC and HLR.

ShortMsgGatewayPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is GMSC

CONSUMER INVOKES {

sendRoutingInfoForSM,

reportSM-DeliveryStatus}

SUPPLIER INVOKES {

informServiceCentre}

The v1-equivalent package is defined as follows:

ShortMsgGatewayPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is GMSC

CONSUMER INVOKES {

sendRoutingInfoForSM

reportSMDeliveryStatus}

14.2.2.24 - 14.2.2.25 [spare]

14.2.2.26	Message waiting data management

This operation package includes the operations required for short message waiting data procedures between HLR and VLR.

MwdMngtPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

readyForSM}

The v1-equivalent package is defined as follows:

MwdMngtPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

noteSubscriberPresent}

14.2.2.27	Alerting

This operation package includes the operations required for alerting between HLR and IWMSC.

AlertingPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is IWMSC if Consumer is HLR

CONSUMER INVOKES {

alertServiceCentre}

The v1-equivalent package is defined as follows.

AlertingPackage-v1 ::= OPERATION-PACKAGE

-- Supplier is IWMSC if Consumer is HLR

CONSUMER INVOKES {

alertServiceCentreWithoutResult}

14.2.2.28	Data restoration

This operation package includes the operations required for VLR data restoration between HLR and VLR.

DataRestorationPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

restoreData}

The v1-equivalent package is: InfoRetrievalPackage-v1

14.2.2.29	Purging

This operation package includes the operations required for purging between HLR and VLR.

PurgingPackage-v2 ::= OPERATION-PACKAGE

-- Supplier is HLR if Consumer is VLR

CONSUMER INVOKES {

purgeMS}

This Package is v2 only.

14.3	Application contexts

14.3.1	General aspects

To each dialogue established by a MAP-user is associated an application-context. In this specification each application-context is assigned a name which is supplied in the MAP-OPEN Req primitive by the MAP-User and transmitted to the peer under certain circumstances.

The following ASN.1 MACRO is used to describe the main aspects of application-contexts in the following sub-sections:

APPLICATION-CONTEXT MACRO ::=

BEGIN

TYPE NOTATION ::= Symmetric | InitiatorConsumerOf

ResponderConsumerOf | empty

VALUE NOTATION ::= value(VALUE OBJECT IDENTIFIER)

Symmetric ::= "OPERATIONS OF" "{" PackageList "}"

InitiatorConsumerOf ::= "INITIATOR CONSUMER OF" "{" PackageList "}"

ResponderConsumerOf ::= "RESPONDER CONSUMER OF" "{" PackageList "}"

| empty

PackageList ::= Package | PackageList "," Package

Package ::= value(OPERATION-PACKAGE)

| type -- shall reference a package type

END

The following definitions are used throughout this section:

-	v1-application-context: An application-context which contains only v1-packages and uses only TC v1 facilities.

-	v1-compatible application-context: An application-context which is a v1-application-context or which uses TC V2 facilities with only v1-packages.

-	v1 context set: the set of v1-application-contexts defined in this specification.

The name of v1-application-contexts are suffixed by "-v1" while other names are suffixed by "-v2".

Application-contexts which do not belong to the v1 context set uses v2 TC facilities.

The last component of the application-context-name (i.e. the last component of the object identifier value) assigned to application-context which belong to the v1 context set indicates explicitly "version1".

For each application-context which does not belong to the "v1 context set" there is v1-equivalent application context. This is a v1-application-context which includes the v1-equivalents of the packages included in the original context.

Each application-context uses the abstract-syntax associated with the operation-packages it includes and uses the transfer-syntax derived from it by applying the encoding rules defined in section 14.1.1.

Ac which do not belong to the v1 context set require the support of the abstract-syntax identified by the following object identifier value: MAP-DialogueInformation.map-Dialogue-AS defined in section 14.4.

14.3.2	Access-context definitions

14.3.2.1	[spare]

14.3.2.2	Location Updating

This application context is used between HLR and VLR for location updating procedures.

networkLocUpContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is VLR

INITIATOR CONSUMER OF {

LocationUpdatingPackage-v2,

DataRestorationPackage-v2}

RESPONDER CONSUMER OF {

SubscriberDataMngtPackage-v2

TracingPackage-v1}

::= {map-ac networkLocUp(1) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac networkLocUp(1) version1(1)}

14.3.2.3	Location Cancellation

This application context is used between HLR and VLR for location cancellation procedures.

locationCancellation-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is HLR

INITIATOR CONSUMER OF {

LocationCancellationPackage-v2}

::= {map-ac locationCancel(2) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

map-ac locationCancel(2) version1(1)

14.3.2.4	Roaming number enquiry

This application context is used between HLR and VLR for roaming number enquiry procedures.

roamingNumberEnquiryContext-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is HLR

INITIATOR CONSUMER OF {

RoamingNumberEnquiryPackage-v2}

::= {map-ac roamingNbEnquiry(3) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac roamingNbEnquiry(3) version1(1)}

14.3.2.5	[spare]

14.3.2.6	Location Information Retrieval

This application-context is used between GMSC and HLR when retrieving location information.

locationInfoRetrievalContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is GMSC

INITIATOR CONSUMER OF {

InterrogationPackage-v2}

::= {map-ac locInfoRetrieval(5) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac locInfoRetrieval(5) version1(1)}

14.3.2.7 - 14.3.2.10 [spare]

14.3.2.11	Location registers restart

This application context is used for location register restart procedures.

resetContext-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is HLR

INITIATOR CONSUMER OF {

ResetPackage-v2}

::= {map-ac reset(10) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac reset(10) version1(1)}

14.3.2.12	Handover control

This application context is used for handover procedures between MSC

handoverControlContext-v2 APPLICATION-CONTEXT

-- Responder is MSCB if Initiator is MSCA

INITIATOR CONSUMER OF {

HandoverControlPackage-v2}

::= {map-ac handoverControl(11) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac handoverControl(11) version1(1)}

14.3.2.13	[spare]

14.3.2.14	Equipment Management

This application context is used for equipment checking between MSC and EIR:

equipmentMngtContext-v2 APPLICATION-CONTEXT

-- Responder is EIR if Initiator is MSC

INITIATOR CONSUMER OF {

EquipmentMngtPackage-v2}

::= {map-ac equipmentMngt(13) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac equipmentMngt(13) version1(1)}

14.3.2.15	Information retrieval

This application context is used for information retrieval between HLR and VLR.

infoRetrievalContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is VLR

INITIATOR CONSUMER OF {

InfoRetrievalPackage-v2}

::= {map-ac infoRetrieval(14) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

-- Responder is HLR if Initiator is VLR

{map-ac infoRetrieval(14) version1(1)}

14.3.2.16	Inter-VLR information retrieval

This application context is used for information retrieval between VLRs.

interVlrInfoRetrievalContext-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is VLR

INITIATOR CONSUMER OF {

InterVlrInfoRetrievalPackage-v2}

::= {map-ac interVlrInfoRetrieval(15) version2(2)}

The v1-equivalent application-context is:

-- Responder is VLR if Initiator is VLR

{map-ac infoRetrieval(14) version1(1)}

14.3.2.17	Stand Alone Subscriber Data Management

This application context is used for stand alone subscriber data management between HLR and VLR:

subscriberDataMngtContext-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is HLR

INITIATOR CONSUMER OF {

SubscriberDataMngtStandAlonePackage-v2}

::= {map-ac subscriberDataMngt(16) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac subscriberDataMngt(16) version1(1)}

14.3.2.18	Tracing

This application context is used for stand alone tracing control procedures:

tracingContext-v2 APPLICATION-CONTEXT

-- Responder is VLR if Initiator is HLR

INITIATOR CONSUMER OF {

TracingStandAlonePackage-v1}

::= {map-ac tracing(17) version2(2)}

This application-context is v1-compatible.

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac tracing(17) version1(1)}

14.3.2.19	Network functional SS handling

This application context is used for functional-like SS handling procedures between VLR and HLR.

networkFunctionalSsContext-v2 APPLICATION-CONTEXT

-- Responder is HLR, Initiator is VLR

INITIATOR CONSUMER OF {

FunctionalSsPackage-v2}

::= {map-ac networkFunctionalSs(18) version2(2)}

The v1-equivalent application-context is defined as follows:

networkFunctionalSsContext-v1 APPLICATION-CONTEXT

-- Responder is HLR, Initiator is VLR

INITIATOR CONSUMER OF {

FunctionalSsPackage-v1,

UnstructuredSsPackage-v1,

BindingPackage-v1}

::= {map-ac networkFunctionalSs(18) version1(1)}

14.3.2.20	Network unstructured SS handling

This application context is used for handling stimuli-like procedures between HLR and VLR.

networkUnstructuredSsContext-v2 APPLICATION-CONTEXT

-- Responder is HLR, Initiator is VLR

-- Responder is VLR, Initiator is HLR

OPERATIONS OF {

UnstructuredSsPackage-v2}

::= {map-ac networkUnstructuredSs(19) version2(2)}

The v1-equivalent application-context is: networkFunctionalSsContext-v1.

14.3.2.21	Short Message Gateway

This application context is used for short message gateway procedures.

shortMsgGatewayContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is GMSC

INITIATOR CONSUMER OF {

ShortMsgGatewayPackage-v2}

::= {map-ac shortMsgGateway(20) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac shortMsgGateway(20) version1(1)}

14.3.2.22	Mobile originating Short Message Relay

This application context is used for mobile originating short message relay procedures.

shortMsgMO-RelayContext-v2 APPLICATION-CONTEXT

-- Responder is IWMSC if Initiator is MSC

INITIATOR CONSUMER OF {

ShortMsgRelayPackage-v2}

::= {map-ac shortMsgMO-Relay(21) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac --shortMsgRelay--21 version1(1)}

14.3.2.23	[spare]

14.3.2.24	Short message alert

This application context is used for short message alerting procedures.

shortMsgAlertContext-v2 APPLICATION-CONTEXT

-- Responder is IWMSC if Initiator is HLR

INITIATOR CONSUMER OF {

AlertingPackage-v2}

::= {map-ac shortMsgAlert(23) version2(2)}

The following application-context-name is symbolically assigned to the v1-equivalent application-context:

{map-ac shortMsgAlert(23) version1(1)}

14.3.2.25	Short message waiting data management

This application context is used for short message waiting data management procedures.

mwdMngtContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is VLR

INITIATOR CONSUMER OF {

MwdMngtPackage-v2}

::= {map-ac mwdMngt(24) version2(2)}

The following application-context-name is assigned to the v1-equivalent application-context:

{map-ac mwdMngt(24) version1(1)}

14.3.2.26	Mobile terminating Short Message Relay

This application context is used for mobile terminating short message relay procedures.

shortMsgMT-RelayContext-v2 APPLICATION-CONTEXT

-- Responder is MSC if Initiator is GMSC

INITIATOR CONSUMER OF {

ShortMsgRelayPackage-v2}

::= {map-ac shortMsgMT-Relay(25) version2(2)}

The v1-equivalent application-context is: shortMsgRelayContext_v1.

14.3.2.27	MS purging

This application context is used between HLR and VLR for MS purging procedures.

msPurgingContext-v2 APPLICATION-CONTEXT

-- Responder is HLR if Initiator is VLR

INITIATOR CONSUMER OF {

purgingPackage-v2}

::= {map-ac msPurging(27) version2(2)}

This application-context is v2 only.

14.3.3	ASN.1 Module for application-context-names

The following ASN.1 module summarizes the application-context-name assigned to MAP application-contexts.

.$MAP-ApplicationContexts {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ApplicationContexts (2) version2 (2)}

DEFINITIONS

::=

BEGIN

-- EXPORTS everything

IMPORTS

gsm-NetworkId,

ac-Id

FROM MobileDomainDefinitions {

ccitt (0) identified-organization (4) etsi (0) mobileDomain (0)

mobileDomainDefinitions (0) version1 (1)}

;

-- application-context-names

map-ac  OBJECT IDENTIFIER ::= {gsm-NetworkId ac-Id}

networkLocUpContext-v2  OBJECT IDENTIFIER ::=

{map-ac networkLocUp(1) version2(2)}

networkLocUpContext-v1  OBJECT IDENTIFIER ::=

{map-ac networkLocUp(1) version1(1)}

locationCancellation-v2  OBJECT IDENTIFIER ::=

{map-ac locationCancel(2) version2(2)}

locationCancellation-v1  OBJECT IDENTIFIER ::=

{map-ac locationCancel(2) version1(1)}

roamingNumberEnquiryContext-v2  OBJECT IDENTIFIER ::=

{map-ac roamingNbEnquiry(3) version2(2)}

roamingNumberEnquiryContext-v1  OBJECT IDENTIFIER ::=

{map-ac roamingNbEnquiry(3) version1(1)}

locationInfoRetrievalContext-v2  OBJECT IDENTIFIER ::=

{map-ac locInfoRetrieval(5) version2(2)}

locationInfoRetrievalContext-v1  OBJECT IDENTIFIER ::=

{map-ac locInfoRetrieval(5) version1(1)}

resetContext-v2  OBJECT IDENTIFIER ::=

{map-ac reset(10) version2(2)}

resetContext-v1  OBJECT IDENTIFIER ::=

{map-ac reset(10) version1(1)}

handoverControlContext-v2  OBJECT IDENTIFIER ::=

{map-ac handoverControl(11) version2(2)}

handoverControlContext-v1  OBJECT IDENTIFIER ::=

{map-ac handoverControl(11) version1(1)}

equipmentMngtContext-v2  OBJECT IDENTIFIER ::=

{map-ac equipmentMngt(13) version2(2)}

equipmentMngtContext-v1  OBJECT IDENTIFIER ::=

{map-ac equipmentMngt(13) version1(1)}

infoRetrievalContext-v2  OBJECT IDENTIFIER ::=

{map-ac infoRetrieval(14) version2(2)}

infoRetrievalContext-v1  OBJECT IDENTIFIER ::=

{map-ac infoRetrieval(14) version1(1)}

interVlrInfoRetrievalContext-v2  OBJECT IDENTIFIER ::=

{map-ac interVlrInfoRetrieval(15) version2(2)}

subscriberDataMngtContext-v2  OBJECT IDENTIFIER ::=

{map-ac subscriberDataMngt(16) version2(2)}

subscriberDataMngtContext-v1  OBJECT IDENTIFIER ::=

{map-ac subscriberDataMngt(16) version1(1)}

tracingContext-v2  OBJECT IDENTIFIER ::=

{map-ac tracing(17) version2(2)}

tracingContext-v1  OBJECT IDENTIFIER ::=

{map-ac tracing(17) version1(1)}

networkFunctionalSsContext-v2  OBJECT IDENTIFIER ::=

{map-ac networkFunctionalSs(18) version2(2)}

networkFunctionalSsContext-v1  OBJECT IDENTIFIER ::=

{map-ac networkFunctionalSs(18) version1(1)}

networkUnstructuredSsContext-v2  OBJECT IDENTIFIER ::=

{map-ac networkUnstructuredSs(19) version2(2)}

shortMsgGatewayContext-v2  OBJECT IDENTIFIER ::=

{map-ac shortMsgGateway(20) version2(2)}

shortMsgGatewayContext-v1  OBJECT IDENTIFIER ::=

{map-ac shortMsgGateway(20) version1(1)}

shortMsgMO-RelayContext-v2  OBJECT IDENTIFIER ::=

{map-ac shortMsgMO-Relay(21) version2(2)}

shortMsgRelayContext-v1  OBJECT IDENTIFIER ::=

{map-ac --shortMsgRelay--21 version1(1)}

shortMsgAlertContext-v2  OBJECT IDENTIFIER ::=

{map-ac shortMsgAlert(23) version2(2)}

shortMsgAlertContext-v1  OBJECT IDENTIFIER ::=

{map-ac shortMsgAlert(23) version1(1)}

mwdMngtContext-v2  OBJECT IDENTIFIER ::=

{map-ac mwdMngt(24) version2(2)}

mwdMngtContext-v1  OBJECT IDENTIFIER ::=

{map-ac mwdMngt(24) version1(1)}

shortMsgMT-RelayContext-v2  OBJECT IDENTIFIER ::=

{map-ac shortMsgMT-Relay(25) version2(2)}

msPurgingContext-v2  OBJECT IDENTIFIER ::=

{map-ac msPurging(27) version2(2)}

.#END

14.4	MAP Dialogue Information

.$MAP-DialogueInformation {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-DialogueInformation (3) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

map-DialogueAS,

MAP-DialoguePDU

;

IMPORTS

gsm-NetworkId,

as-Id

FROM MobileDomainDefinitions {

ccitt (0) identified-organization (4) etsi (0) mobileDomain (0)

mobileDomainDefinitions (0) version1 (1)}

AddressString

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network(1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

-- abstract syntax name for MAP-DialoguePDU

map-DialogueAS  OBJECT IDENTIFIER ::=

{gsm-NetworkId as-Id map-DialoguePDU (1) version1 (1)}

MAP-DialoguePDU ::= CHOICE {

map-open	[0] MAP-OpenInfo,

map-accept	[1] MAP-AcceptInfo,

map-close	[2] MAP-CloseInfo,

map-refuse	[3] MAP-RefuseInfo,

map-userAbort	[4] MAP-UserAbortInfo,

map-providerAbort	[5] MAP-ProviderAbortInfo}

MAP-OpenInfo ::= SEQUENCE {

destinationReference	[0] AddressString	OPTIONAL,

originationReference	[1] AddressString	OPTIONAL,

...}

MAP-AcceptInfo ::= SEQUENCE {

...}

MAP-CloseInfo ::= SEQUENCE {

...}

MAP-RefuseInfo ::= SEQUENCE {

reason	Reason,

...}

Reason ::= ENUMERATED {

noReasonGiven  (0),

invalidDestinationReference  (1),

invalidOriginatingReference  (2)}

MAP-UserAbortInfo ::= SEQUENCE {

map-UserAbortChoice	MAP-UserAbortChoice,

...}

MAP-UserAbortChoice ::= CHOICE {

userSpecificReason	[0] NULL,

userResourceLimitation	[1] NULL,

resourceUnavailable	[2] ResourceUnavailableReason,

applicationProcedureCancellation	[3] ProcedureCancellationReason}

ResourceUnavailableReason ::= ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)}

ProcedureCancellationReason ::= ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}

MAP-ProviderAbortInfo ::= SEQUENCE {

map-ProviderAbortReason	MAP-ProviderAbortReason,

...}

MAP-ProviderAbortReason ::= ENUMERATED {

abnormalDialogue  (0),

invalidPDU  (1)}

.#END

14.5	MAP Operation and Error codes

.$MAP-Protocol {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Protocol (4) version2 (2)}

DEFINITIONS

::=

BEGIN

IMPORTS

UpdateLocation,

CancelLocation,

PurgeMS,

SendIdentification,

PerformHandover,

PrepareHandover,

SendEndSignal,

ProcessAccessSignalling,

ForwardAccessSignalling,

PerformSubsequentHandover,

PrepareSubsequentHandover,

SendAuthenticationInfo,

CheckIMEI,

SendParameters,

InsertSubscriberData,

DeleteSubscriberData,

Reset,

ForwardCheckSS-Indication,

RestoreData

FROM MAP-MobileServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MobileServiceOperations (5)

version2 (2)}

ActivateTraceMode,

DeactivateTraceMode,

TraceSubscriberActivity,

NoteInternalHandover,

SendIMSI

FROM MAP-OperationAndMaintenanceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OperationAndMaintenanceOperations (6)

version2 (2)}

SendRoutingInfo,

ProvideRoamingNumber

FROM MAP-CallHandlingOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CallHandlingOperaations (7)

version2 (2)}

RegisterSS,

EraseSS,

ActivateSS,

DeactivateSS,

InterrogateSS,

ProcessUnstructuredSS-Data,

ProcessUnstructuredSS-Request,

UnstructuredSS-Request,

UnstructuredSS-Notify,

RegisterPassword,

GetPassword,

BeginSubscriberActivity

FROM MAP-SupplementaryServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SupplementaryServiceOperations (8)

version2 (2)}

SendRoutingInfoForSM,

ForwardSM,

ReportSM-DeliveryStatus,

NoteSubscriberPresent,

AlertServiceCentreWithoutResult,

AlertServiceCentre,

InformServiceCentre,

ReadyForSM

FROM MAP-ShortMessageServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ShortMessageServieOperations (9)

version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

NumberChanged,

UnknownBaseStation,

UnknownMSC,

UnidentifiedSubscriber,

UnknownEquipment,

RoamingNotAllowed,

IllegalSubscriber,

IllegalEquipment,

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

InvalidTargetBaseStation,

NoRadioResourceAvailable,

NoHandoverNumberAvailable,

SubsequentHandoverFailure,

TracingBufferFull,

NoRoamingNumberAvailable,

AbsentSubscriber,

CallBarred,

ForwardingViolation,

CUG-Reject,

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

SS-SubscriptionViolation,

SS-Incompatibility,

UnknownAlphabet,

USSD-Busy,

PW-RegistrationFailure,

NegativePW-Check,

NumberOfPW-AttemptsViolation,

SM-DeliveryFailure,

MessageWaitingListFull

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

;

-- location registration operation codes

updateLocation  UpdateLocation ::= localValue 2

cancelLocation  CancelLocation ::= localValue 3

purgeMS  PurgeMS ::= localValue 67

-- NU1 purgeMS must not be used in version 1

sendIdentification  SendIdentification::= localValue 55

-- NU1 sendIdentification must not be used in version 1

-- handover operation codes

performHandover  PerformHandover ::= localValue 28

-- NU>1 performHandover must not be used in version greater 1

prepareHandover  PrepareHandover ::= localValue 68

-- NU1 prepareHandover must not be used in version 1

sendEndSignal  SendEndSignal ::= localValue 29

processAccessSignalling  ProcessAccessSignalling ::= localValue 33

forwardAccessSignalling  ForwardAccessSignalling ::= localValue 34

performSubsequentHandover  PerformSubsequentHandover ::=

localValue 30

-- NU>1 performSubsequentHandover must not be used in version greater 1

prepareSubsequentHandover  PrepareSubsequentHandover ::=

localValue 69

-- NU1 prepareSubsequentHandover must not be used in version 1

-- authentication operation codes

sendAuthenticationInfo  SendAuthenticationInfo ::= localValue 56

-- NU1 sendAuthenticationInfo must not be used in version 1

-- IMEI MANAGEMENT operation codes

checkIMEI  CheckIMEI ::= localValue 43

-- subscriber management operation codes

sendParameters  SendParameters ::= localValue 9

-- NU>1 sendParameters must not be used in version greater 1

insertSubscriberData  InsertSubscriberData ::= localValue 7

deleteSubscriberData  DeleteSubscriberData ::= localValue 8

-- fault recovery operation codes

reset  Reset ::= localValue 37

forwardCheckSS-Indication  ForwardCheckSS-Indication ::=

localValue 38

restoreData  RestoreData ::= localValue 57

-- NU1 restoreData must not be used in version 1

-- operation and maintenance operation codes

activateTraceMode  ActivateTraceMode ::= localValue 50

deactivateTraceMode  DeactivateTraceMode ::= localValue 51

traceSubscriberActivity  TraceSubscriberActivity ::= localValue 52

-- NU>1 traceSubscriberActivity must not be used in version greater 1

noteInternalHandover  NoteInternalHandover ::= localValue 35

-- NU>1 noteInternalHandover must not be used in version greater 1

sendIMSI  SendIMSI ::= localValue 58

-- NU1 sendIMSI must not be used in version 1

-- call handling operation codes

sendRoutingInfo  SendRoutingInfo ::= localValue 22

provideRoamingNumber  ProvideRoamingNumber ::= localValue 4

-- supplementary service handling operation codes

registerSS  RegisterSS ::= localValue 10

eraseSS  EraseSS ::= localValue 11

activateSS  ActivateSS ::= localValue 12

deactivateSS  DeactivateSS ::= localValue 13

interrogateSS  InterrogateSS ::= localValue 14

processUnstructuredSS-Data  ProcessUnstructuredSS-Data ::=

localValue 19

-- NU>1 processUnstructuredSS-Data must not be used in version greater 1

processunstructuredSS-Request  ProcessUnstructuredSS-Request ::=

localValue 59

-- NU1 processUnstructuredSS-Request must not be used in version 1

unstructuredSS-Request  UnstructuredSS-Request ::= localValue 60

-- NU1 UnstructuredSS-Request must not be used in version 1

unstructuredSS-Notify  UnstructuredSS-Notify ::= localValue 61

-- NU1 UnstructuredSS-Notify must not be used in version 1

registerPassword  RegisterPassword ::= localValue 17

getPassword  GetPassword ::= localValue 18

beginSubscriberActivity  BeginSubscriberActivity ::= localValue 54

-- NU>1 beginSubscriberActivity must not be used in version greater 1

-- short message service operation codes

sendRoutingInfoForSM  SendRoutingInfoForSM ::= localValue 45

forwardSM  ForwardSM ::= localValue 46

reportSM-DeliveryStatus  ReportSM-DeliveryStatus ::= localValue 47

noteSubscriberPresent  NoteSubscriberPresent ::= localValue 48

-- NU>1 noteSubscriberPresent must not be used in version greater 1

alertServiceCentreWithoutResult  AlertServiceCentreWithoutResult ::=

localValue 49

-- NU>1 alertServiceCentreWithoutResult must not be used in

-- version greater 1

informServiceCentre  InformServiceCentre ::= localValue 63

-- NU1 informServiceCentre must not be used in version 1

alertServiceCentre  AlertServiceCentre ::= localValue 64

-- NU1 alertServiceCentre must not be used in version 1

readyForSM  ReadyForSM ::= localValue 66

-- NU1 readyForSM must not be used in version 1

-- generic error codes

systemFailure  SystemFailure ::= localValue 34

dataMissing  DataMissing ::= localValue 35

unexpectedDataValue  UnexpectedDataValue ::= localValue 36

facilityNotSupported  FacilityNotSupported ::= localValue 21

-- identification and numbering error codes

unknownSubscriber  UnknownSubscriber ::= localValue 1

numberChanged  NumberChanged ::= localValue 44

unknownBaseStation  UnknownBaseStation ::= localValue 2

unknownMSC  UnknownMSC ::= localValue 3

unidentifiedSubscriber  UnidentifiedSubscriber ::= localValue 5

unknownEquipment  UnknownEquipment ::= localValue 7

-- subscription error codes

roamingNotAllowed  RoamingNotAllowed ::= localValue 8

illegalSubscriber  IllegalSubscriber ::= localValue 9

illegalEquipment  IllegalEquipment ::= localValue 12

bearerServiceNotProvisioned  BearerServiceNotProvisioned ::=

localValue 10

teleserviceNotProvisioned  TeleserviceNotProvisioned ::=

localValue 11

-- handover error codes

invalidTargetBaseStation  InvalidTargetBaseStation ::= localValue 23

noRadioResourceAvailable  NoRadioResourceAvailable ::= localValue 24

noHandoverNumberAvailable  NoHandoverNumberAvailable ::=

localValue 25

subsequentHandoverFailure  SubsequentHandoverFailure ::=

localValue 26

-- operation and maintenance error codes

tracingBufferFull  TracingBufferFull ::= localValue 40

-- call handling error codes

noRoamingNumberAvailable  NoRoamingNumberAvailable ::= localValue 39

absentSubscriber  AbsentSubscriber ::= localValue 27

callBarred  CallBarred ::= localValue 13

forwardingViolation  ForwardingViolation ::= localValue 14

cug-Reject  CUG-Reject ::= localValue 15

-- supplementary service error codes

illegalSS-Operation  IllegalSS-Operation ::= localValue 16

ss-ErrorStatus  SS-ErrorStatus ::= localValue 17

ss-NotAvailable  SS-NotAvailable ::= localValue 18

ss-SubscriptionViolation  SS-SubscriptionViolation ::= localValue 19

ss-Incompatibility  SS-Incompatibility ::= localValue 20

unknownAlphabet  UnknownAlphabet ::= localValue 71

-- NU1 UnknownAlphabet must not be used in version 1

ussd-Busy  USSD-Busy ::= localValue 72

-- NU1 ussd-Busy must not be used in version 1

pw-RegistrationFailure  PW-RegistrationFailure ::= localValue 37

negativePW-Check  NegativePW-Check ::= localValue 38

numberOfPW-AttemptsViolation  NumberOfPW-AttemptsViolation ::=

localValue 43

-- NU1 numberOfPW-AttemptsViolation must not be used in version 1

-- short message service error codes

sm-DeliveryFailure  SM-DeliveryFailure ::= localValue 32

messageWaitingListFull  MessageWaitingListFull ::= localValue 33

.#END

14.6	MAP Operation and Error Types

14.6.1	Mobile Service Operations

.$MAP-MobileServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MobileServiceOperations (5)

version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

-- location registration operations

UpdateLocation,

CancelLocation,

PurgeMS,

SendIdentification,

-- handover operations

PerformHandover,

PrepareHandover,

SendEndSignal,

ProcessAccessSignalling,

ForwardAccessSignalling,

PerformSubsequentHandover,

PrepareSubsequentHandover,

-- authentication management operations

SendAuthenticationInfo,

-- IMEI management operations

CheckIMEI,

-- subscriber management operations

SendParameters,

InsertSubscriberData,

DeleteSubscriberData,

-- fault recovery operations

Reset,

ForwardCheckSS-Indication,

RestoreData

;

IMPORTS

OPERATION

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownSubscriber,

UnknownBaseStation,

UnknownMSC,

UnidentifiedSubscriber,

UnknownEquipment,

RoamingNotAllowed,

InvalidTargetBaseStation,

NoRadioResourceAvailable,

NoHandoverNumberAvailable,

SubsequentHandoverFailure

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

UpdateLocationArg,

UpdateLocationRes,

CancelLocationArg,

PurgeMS-Arg,

SendIdentificationRes,

PerformHO-Arg,

PrepareHO-Arg,

PerformHO-Res,

PrepareHO-Res,

PerformSubsequentHO-Arg,

PrepareSubsequentHO-Arg,

SendAuthenticationInfoArg,

SendAuthenticationInfoRes,

EquipmentStatus,

SendParametersArg,

SentParameterList,

InsertSubscriberDataArg,

InsertSubscriberDataRes,

DeleteSubscriberDataArg,

DeleteSubscriberDataRes,

ResetArg,

RestoreDataArg,

RestoreDataRes

FROM MAP-MS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version2 (2)}

ExternalSignalInfo,

TMSI,

IMEI

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

-- location registration operations

UpdateLocation ::= OPERATION				--Timer m--

ARGUMENT

updateLocationArg	UpdateLocationArg

RESULT

updateLocationRes	UpdateLocationRes

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

RoamingNotAllowed}

CancelLocation ::= OPERATION				--Timer m--

ARGUMENT

cancelLocationArg	CancelLocationArg

RESULT

ERRORS {

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnidentifiedSubscriber}

-- NU>1 UnidentifiedSubscriber must not be used in version

-- greater 1

PurgeMS ::= OPERATION				--Timer m--

ARGUMENT

purgeMS-Arg	PurgeMS-Arg

RESULT ERRORS {}

SendIdentification::= OPERATION				--Timer s--

ARGUMENT

tmsi	TMSI

RESULT

sendIdentificationRes	SendIdentificationRes

ERRORS {

DataMissing,

UnidentifiedSubscriber}

-- handover operations

PrepareHandover ::= OPERATION				--Timer s--

ARGUMENT

prepareHO-Arg	PrepareHO-Arg

RESULT

prepareHO-Res	PrepareHO-Res

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

NoHandoverNumberAvailable}

PerformHandover ::= OPERATION				--Timer s--

ARGUMENT

performHO-Arg	PerformHO-Arg

RESULT

performHO-Res	PerformHO-Res

ERRORS {

SystemFailure,

UnexpectedDataValue,

UnknownBaseStation,

InvalidTargetBaseStation,

NoRadioResourceAvailable,

NoHandoverNumberAvailable}

SendEndSignal ::= OPERATION				--Timer l--

ARGUMENT

bss-APDU	ExternalSignalInfo

-- optional

-- OA1 bss-APDU must be absent in version 1

-- OP>1 bss-APDU must be present in version greater 1

RESULT ERRORS {}

ProcessAccessSignalling ::= OPERATION			--Timer s--

ARGUMENT

bss-APDU	ExternalSignalInfo

ForwardAccessSignalling ::= OPERATION			--Timer s--

ARGUMENT

bss-APDU	ExternalSignalInfo

PrepareSubsequentHandover ::= OPERATION			--Timer m--

ARGUMENT

prepareSubsequentHO-Arg	PrepareSubsequentHO-Arg

RESULT

bss-APDU	ExternalSignalInfo

ERRORS {

UnexpectedDataValue,

DataMissing,

UnknownMSC,

SubsequentHandoverFailure}

PerformSubsequentHandover ::= OPERATION			--Timer m--

ARGUMENT

performSubsequentHO-Arg	PerformSubsequentHO-Arg

RESULT

accessSignalInfo	ExternalSignalInfo

ERRORS {

UnexpectedDataValue,

UnknownBaseStation,

UnknownMSC,

InvalidTargetBaseStation,

SubsequentHandoverFailure}

-- authentication management operations

SendAuthenticationInfo ::= OPERATION			--Timer m--

ARGUMENT

sendAuthenticationInfoArg	SendAuthenticationInfoArg

RESULT

sendAuthenticationInfoRes	SendAuthenticationInfoRes

-- optional

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownSubscriber}

-- IMEI management operations

CheckIMEI ::= OPERATION				--Timer m--

ARGUMENT

imei	IMEI

RESULT

equipmentStatus	EquipmentStatus

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

-- NU>1 UnexpectedDataValue must not be used in version greater 1

UnknownEquipment}

-- subscriber management operations

SendParameters ::= OPERATION				--Timer m--

ARGUMENT

sendParametersArg	SendParametersArg

RESULT

sentParameterList	SentParameterList

-- optional

-- nothing is returned, if no requested parameter is

-- available or exists

ERRORS {

UnexpectedDataValue,

UnknownSubscriber,

UnidentifiedSubscriber}

InsertSubscriberData ::= OPERATION				--Timer m--

ARGUMENT

insertSubscriberDataArg	InsertSubscriberDataArg

RESULT

insertSubscriberDataRes	InsertSubscriberDataRes

-- optional

-- OA1 insertSubscriberDataRes must be absent in version 1

ERRORS {

DataMissing,

UnexpectedDataValue,

UnidentifiedSubscriber}

DeleteSubscriberData ::= OPERATION				--Timer m--

ARGUMENT

deleteSubscriberDataArg	DeleteSubscriberDataArg

RESULT

deleteSubscriberDataRes	DeleteSubscriberDataRes

-- optional

-- OA1 deleteSubscriberDataRes must be absent in version 1

ERRORS {

DataMissing,

UnexpectedDataValue,

UnidentifiedSubscriber}

-- fault recovery operations

Reset ::= OPERATION				--Timer m--

ARGUMENT

resetArg	ResetArg

ForwardCheckSS-Indication ::= OPERATION		--Timer s--

RestoreData ::= OPERATION				--Timer m--

ARGUMENT

restoreDataArg	RestoreDataArg

RESULT

restoreDataRes	RestoreDataRes

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownSubscriber}

.#END

14.6.2	Operation and Maintenance Operations

.$MAP-OperationAndMaintenanceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OperationAndMaintenanceOperations (6)

version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

ActivateTraceMode,

DeactivateTraceMode,

TraceSubscriberActivity,

NoteInternalHandover,

SendIMSI

;

IMPORTS

OPERATION

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

UnidentifiedSubscriber,

TracingBufferFull

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

ActivateTraceModeArg,

DeactivateTraceModeArg,

TraceSubscriberActivityArg,

NoteInternalHO-Arg

FROM MAP-OM-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OM-DataTypes (12) version2 (2)}

ISDN-AddressString,

IMSI

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ComonDataTypes (18) version2 (2)}

;

ActivateTraceMode ::= OPERATION				--Timer m--

ARGUMENT

activateTraceModeArg	ActivateTraceModeArg

RESULT

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnidentifiedSubscriber,

TracingBufferFull}

DeactivateTraceMode ::= OPERATION				--Timer m--

ARGUMENT

deactivateTraceModeArg	DeactivateTraceModeArg

RESULT

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnidentifiedSubscriber}

TraceSubscriberActivity ::= OPERATION			--Timer s--

ARGUMENT

traceSubscriberActivityArg	TraceSubscriberActivityArg

NoteInternalHandover ::= OPERATION				--Timer s--

ARGUMENT

noteInternalHO-Arg	NoteInternalHO-Arg

SendIMSI ::= OPERATION				--Timer m--

ARGUMENT

msisdn	ISDN-AddressString

RESULT

imsi		IMSI

ERRORS {

DataMissing,

UnexpectedDataValue,

UnknownSubscriber}

.#END

14.6.3	Call Handling Operations

.$MAP-CallHandlingOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CallHandlingOperations (7)

version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

SendRoutingInfo,

ProvideRoamingNumber

;

IMPORTS

OPERATION

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

NumberChanged,

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

NoRoamingNumberAvailable,

AbsentSubscriber,

CallBarred,

ForwardingViolation,

CUG-Reject

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

SendRoutingInfoArg,

SendRoutingInfoRes,

ProvideRoamingNumberArg

FROM MAP-CH-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13) version2 (2)}

ISDN-AddressString

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

SendRoutingInfo ::= OPERATION				--Timer m--

ARGUMENT

sendRoutingInfoArg	SendRoutingInfoArg

RESULT

sendRoutingInfoRes	SendRoutingInfoRes

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

NumberChanged,

-- NU1 NumberChanged must not be used in version 1

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

AbsentSubscriber,

CallBarred,

CUG-Reject,

-- NU1 CUG-Reject must not be used in version 1

ForwardingViolation}

ProvideRoamingNumber ::= OPERATION				--Timer m--

ARGUMENT

provideRoamingNumberArg	ProvideRoamingNumberArg

RESULT

roamingNumber	ISDN-AddressString

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

AbsentSubscriber,

NoRoamingNumberAvailable}

.#END

14.6.4	Supplementary Service Operations

.$MAP-SupplementaryServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SupplementaryServiceOperations (8)

version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

RegisterSS,

EraseSS,

ActivateSS,

DeactivateSS,

InterrogateSS,

ProcessUnstructuredSS-Data,

ProcessUnstructuredSS-Request,

UnstructuredSS-Request,

UnstructuredSS-Notify,

RegisterPassword,

GetPassword,

BeginSubscriberActivity

;

IMPORTS

OPERATION

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownSubscriber,

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

CallBarred,

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

SS-SubscriptionViolation,

SS-Incompatibility,

PW-RegistrationFailure,

NegativePW-Check,

NumberOfPW-AttemptsViolation,

UnknownAlphabet,

USSD-Busy,

AbsentSubscriber,

IllegalSubscriber,

IllegalEquipment

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

RegisterSS-Arg,

SS-Info,

SS-ForBS-Code,

InterrogateSS-Res,

SS-UserData,

USSD-Arg,

USSD-Res,

Password,

GuidanceInfo,

BeginSubscriberActivityArg

FROM MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

SS-Code

FROM MAP-SS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version2 (2)}

;

-- supplementary service handling operations

RegisterSS ::= OPERATION				--Timer m--

ARGUMENT

registerSS-Arg	RegisterSS-Arg

RESULT

ss-Info	SS-Info

-- optional

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownSubscriber,

-- NU>1 UnknownSubscriber must not be used in version greater 1

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

CallBarred,

-- NU1 CallBarred must not be used in version 1

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

SS-SubscriptionViolation,

-- NU>1 SS-SubscriptionViolation must not be used in version

-- greater 1

SS-Incompatibility}

EraseSS ::= OPERATION				--Timer m--

ARGUMENT

ss-ForBS	SS-ForBS-Code

RESULT

ss-Info	SS-Info

-- optional

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

-- NU>1 UnknownSubscriber must not be used in version greater 1

BearerServiceNotProvisioned,

-- NU1 BearerServiceNotProvisioned must not be used in version 1

TeleserviceNotProvisioned,

-- NU1 TeleserviceNotProvisioned must not be used in version 1

CallBarred,

-- NU1 CallBarred must not be used in version 1

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

-- NU1 SS-NotAvailable must not be used in version 1

SS-SubscriptionViolation

-- NU>1 SS-SubscriptionViolation must not be used in version

-- greater 1

}

ActivateSS ::= OPERATION				--Timer m--

ARGUMENT

ss-ForBS	SS-ForBS-Code

RESULT

ss-Info	SS-Info

-- optional

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

-- NU>1 UnknownSubscriber must not be used in version greater 1

BearerServiceNotProvisioned,

-- NU1 BearerServiceNotProvisioned must not be used in version 1

TeleserviceNotProvisioned,

-- NU1 TeleserviceNotProvisioned must not be used in version 1

CallBarred,

-- NU1 CallBarred must not be used in version 1

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

SS-SubscriptionViolation,

SS-Incompatibility,

NegativePW-Check,

NumberOfPW-AttemptsViolation}

-- NU1 NumberOfPW-AttemptsViolation must not be used in version 1

DeactivateSS ::= OPERATION				--Timer m--

ARGUMENT

ss-ForBS	SS-ForBS-Code

RESULT

ss-Info	SS-Info

-- optional

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

-- NU>1 UnknownSubscriber must not be used in version greater 1

BearerServiceNotProvisioned,

-- NU1 BearerServiceNotProvisioned must not be used in version 1

TeleserviceNotProvisioned,

-- NU1 TeleserviceNotProvisioned must not be used in version 1

CallBarred,

-- NU1 CallBarred must not be used in version 1

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

-- NU1 SS-NotAvailable must not be used in version 1

SS-SubscriptionViolation,

NegativePW-Check,

NumberOfPW-AttemptsViolation}

-- NU1 NumberOfPW-AttemptsViolation must not be used in version 1

InterrogateSS ::= OPERATION				--Timer m--

ARGUMENT

ss-ForBS	SS-ForBS-Code

RESULT

interrogateSS-Res	InterrogateSS-Res

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

-- NU>1 UnknownSubscriber must not be used in version greater 1

BearerServiceNotProvisioned,

-- NU1 BearerServiceNotProvisioned must not be used in version 1

TeleserviceNotProvisioned,

-- NU1 TeleserviceNotProvisioned must not be used in version 1

CallBarred,

-- NU1 CallBarred must not be used in version 1

IllegalSS-Operation,

SS-NotAvailable}

ProcessUnstructuredSS-Data ::= OPERATION		--Timer m--

ARGUMENT

ss-UserData	SS-UserData

RESULT

ss-UserData	SS-UserData

-- optional

ERRORS {

SystemFailure,

UnexpectedDataValue}

ProcessUnstructuredSS-Request ::= OPERATION	--Timer ml--

ARGUMENT

ussd-Arg	USSD-Arg

RESULT

ussd-Res	USSD-Res

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

UnknownAlphabet,

CallBarred}

UnstructuredSS-Request ::= OPERATION	--Timer ml--

ARGUMENT

ussd-Arg	USSD-Arg

RESULT

ussd-Res	USSD-Res

-- optional

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

AbsentSubscriber,

IllegalSubscriber,

IllegalEquipment,

UnknownAlphabet,

USSD-Busy}

UnstructuredSS-Notify ::= OPERATION		--Timer ml--

ARGUMENT

ussd-Arg	USSD-Arg

RESULT

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

AbsentSubscriber,

IllegalSubscriber,

IllegalEquipment,

UnknownAlphabet,

USSD-Busy}

RegisterPassword ::= OPERATION			--Timer ml--

ARGUMENT

ss-Code	SS-Code

RESULT

newPassword	Password

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

CallBarred,

-- NU1 CallBarred must not be used in version 1

SS-SubscriptionViolation,

PW-RegistrationFailure,

NegativePW-Check,

NumberOfPW-AttemptsViolation}

-- NU1 NumberOfPW-AttemptsViolation must not be used in version 1

LINKED {

GetPassword}

GetPassword ::= OPERATION				--Timer m--

ARGUMENT

guidanceInfo	GuidanceInfo

RESULT

currentPassword	Password

BeginSubscriberActivity ::= OPERATION			--Timer m--

ARGUMENT

beginSubscriberActivityArg	BeginSubscriberActivityArg

.#END

14.6.5	Short Message Service Operations

.$MAP-ShortMessageServiceOperations {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ShortMessageServiceOperations (9)

version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

SendRoutingInfoForSM,

ForwardSM,

ReportSM-DeliveryStatus,

NoteSubscriberPresent,

AlertServiceCentreWithoutResult,

AlertServiceCentre,

InformServiceCentre,

ReadyForSM

;

IMPORTS

OPERATION

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

UnidentifiedSubscriber,

IllegalSubscriber,

IllegalEquipment,

TeleserviceNotProvisioned,

AbsentSubscriber,

CallBarred,

SM-DeliveryFailure,

MessageWaitingListFull

FROM MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

RoutingInfoForSM-Arg,

RoutingInfoForSM-Res,

ForwardSM-Arg,

ReportSM-DeliveryStatusArg,

AlertServiceCentreArg,

InformServiceCentreArg,

ReadyForSM-Arg

FROM MAP-SM-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version2 (2)}

ISDN-AddressString,

IMSI

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

SendRoutingInfoForSM ::= OPERATION				--Timer m--

ARGUMENT

routingInfoForSM-Arg	RoutingInfoForSM-Arg

RESULT

routingInfoForSM-Res	RoutingInfoForSM-Res

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber,

TeleserviceNotProvisioned,

AbsentSubscriber,

CallBarred}

ForwardSM ::= OPERATION			--Timer ml--

ARGUMENT

forwardSM-Arg	ForwardSM-Arg

RESULT

ERRORS {

SystemFailure,

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

FacilityNotSupported,

UnidentifiedSubscriber,

IllegalSubscriber,

IllegalEquipment,

-- NU1 IllegalEquipment must not be used in version 1

AbsentSubscriber,

SM-DeliveryFailure}

ReportSM-DeliveryStatus ::= OPERATION			--Timer s--

ARGUMENT

reportSM-DeliveryStatusArg	ReportSM-DeliveryStatusArg

RESULT

storedMSISDN	ISDN-AddressString

-- optional

-- OA1 storedMSISDN must be absent in version 1

-- OP>1 storedMSISDN must be present in version greater 1

ERRORS {

DataMissing,

-- NU1 DataMissing must not be used in version 1

UnexpectedDataValue,

UnknownSubscriber,

MessageWaitingListFull}

NoteSubscriberPresent ::= OPERATION				--Timer s--

ARGUMENT

imsi		IMSI

AlertServiceCentreWithoutResult ::= OPERATION	--Timer s--

ARGUMENT

alertServiceCentreArg	AlertServiceCentreArg

AlertServiceCentre ::= OPERATION				--Timer s--

ARGUMENT

alertServiceCentreArg	AlertServiceCentreArg

RESULT

ERRORS {

SystemFailure,

DataMissing,

UnexpectedDataValue}

InformServiceCentre ::= OPERATION				--Timer s--

ARGUMENT

informServiceCentreArg	InformServiceCentreArg

ReadyForSM ::= OPERATION				--Timer m--

ARGUMENT

readyForSM-Arg	ReadyForSM-Arg

RESULT

ERRORS {

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

UnknownSubscriber}

.#END

14.6.6	Errors

.$MAP-Errors {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-Errors (10) version2 (2)}

DEFINITIONS

::=

BEGIN

EXPORTS

-- generic errors

SystemFailure,

DataMissing,

UnexpectedDataValue,

FacilityNotSupported,

-- identification and numbering errors

UnknownSubscriber,

NumberChanged,

UnknownBaseStation,

UnknownMSC,

UnidentifiedSubscriber,

UnknownEquipment,

-- subscription errors

RoamingNotAllowed,

IllegalSubscriber,

IllegalEquipment,

BearerServiceNotProvisioned,

TeleserviceNotProvisioned,

-- handover errors

InvalidTargetBaseStation,

NoRadioResourceAvailable,

NoHandoverNumberAvailable,

SubsequentHandoverFailure,

-- operation and maintenance errors

TracingBufferFull,

-- call handling errors

NoRoamingNumberAvailable,

AbsentSubscriber,

CallBarred,

ForwardingViolation,

CUG-Reject,

-- supplementary service errors

IllegalSS-Operation,

SS-ErrorStatus,

SS-NotAvailable,

SS-SubscriptionViolation,

SS-Incompatibility,

UnknownAlphabet,

USSD-Busy,

PW-RegistrationFailure,

NegativePW-Check,

NumberOfPW-AttemptsViolation,

-- short message service errors

SM-DeliveryFailure,

MessageWaitingListFull

;

IMPORTS

ERROR

FROM TCAPMessages {

ccitt recommendation q 773 modules (2) messages (1) version2 (2)}

SS-Status,

SS-SubscriptionOption

FROM MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

RoamingNotAllowedCause,

CallBarringCause,

CUG-RejectCause,

SS-IncompatibilityCause,

PW-RegistrationFailureCause,

SM-DeliveryFailureCause

FROM MAP-ER-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version2 (2)}

NetworkResource

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

-- generic errors

SystemFailure ::= ERROR

PARAMETER

networkResource	NetworkResource

-- optional

DataMissing ::= ERROR

UnexpectedDataValue ::= ERROR

FacilityNotSupported ::= ERROR

-- identification and numbering errors

UnknownSubscriber ::= ERROR

NumberChanged ::= ERROR

UnknownBaseStation ::= ERROR

UnknownMSC ::= ERROR

UnidentifiedSubscriber ::= ERROR

UnknownEquipment ::= ERROR

-- subscription errors

RoamingNotAllowed ::= ERROR

PARAMETER

roamingNotAllowedCause	RoamingNotAllowedCause

-- optional

-- OA1 roamingNotAllowedCause must be absent in version 1

-- OP>1 roamingNotAllowedCause must be present in version greater 1

IllegalSubscriber ::= ERROR

IllegalEquipment ::= ERROR

BearerServiceNotProvisioned ::= ERROR

TeleserviceNotProvisioned ::= ERROR

-- handover errors

InvalidTargetBaseStation ::= ERROR

NoRadioResourceAvailable ::= ERROR

NoHandoverNumberAvailable ::= ERROR

SubsequentHandoverFailure ::= ERROR

-- operation and maintenance errors

TracingBufferFull ::= ERROR

-- call handling errors

NoRoamingNumberAvailable ::= ERROR

AbsentSubscriber ::= ERROR

PARAMETER

mwd-Set	BOOLEAN

-- optional

-- OA>1 mwd-Set must be absent in version greater 1

CallBarred ::= ERROR

PARAMETER

callBarringCause	CallBarringCause

-- optional

ForwardingViolation ::= ERROR

CUG-Reject ::= ERROR

PARAMETER

cug-RejectCause	CUG-RejectCause

-- optional

-- supplementary service errors

IllegalSS-Operation ::= ERROR

SS-ErrorStatus ::= ERROR

PARAMETER

ss-Status	SS-Status

-- optional

SS-NotAvailable ::= ERROR

SS-SubscriptionViolation ::= ERROR

PARAMETER

ss-SubscriptionOption	SS-SubscriptionOption

-- optional

SS-Incompatibility ::= ERROR

PARAMETER

ss-IncompatibilityCause	SS-IncompatibilityCause

-- optional

UnknownAlphabet ::= ERROR

USSD-Busy ::= ERROR

PW-RegistrationFailure ::= ERROR

PARAMETER

pw-RegistrationFailureCause	PW-RegistrationFailureCause

NegativePW-Check ::= ERROR

NumberOfPW-AttemptsViolation ::= ERROR

-- short message service errors

SM-DeliveryFailure ::= ERROR

PARAMETER

sm-DeliveryFailureCause	SM-DeliveryFailureCause

MessageWaitingListFull ::= ERROR

.#END

14.7	MAP Constants and Data Types

14.7.1	Mobile Service Data Types

.$MAP-MS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

-- location registration types

UpdateLocationArg,

UpdateLocationRes,

CancelLocationArg,

PurgeMS-Arg,

SendIdentificationRes,

-- handover types

PrepareHO-Arg,

PrepareHO-Res,

PrepareSubsequentHO-Arg,

PerformHO-Arg,

PerformHO-Res,

PerformSubsequentHO-Arg,

-- authentication management types

SendAuthenticationInfoArg,

SendAuthenticationInfoRes,

-- security management types

EquipmentStatus,

-- subscriber management types

SendParametersArg,

SentParameterList,

InsertSubscriberDataArg,

InsertSubscriberDataRes,

DeleteSubscriberDataArg,

DeleteSubscriberDataRes,

-- fault recovery types

ResetArg,

RestoreDataArg,

RestoreDataRes

;

IMPORTS

CUG-InterlockList,

SS-List,

SS-InfoList

FROM MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

ISDN-AddressString,

ExternalSignalInfo,

IMSI,

SubscriberId,

IMEI,

LocationInfo,

HLR-List,

LMSI,

GlobalCellId,

NetworkResource,

Category,

BearerServiceList,

TeleserviceList,

BasicServiceList

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

-- location registration types

UpdateLocationArg ::= SEQUENCE {

imsi			IMSI,

locationInfo	LocationInfo,

vlr-Number	ISDN-AddressString,

lmsi			[10] LMSI	OPTIONAL,

...}

UpdateLocationRes ::= CHOICE {

hlr-Number	ISDN-AddressString,

-- NU>1 hlr-Number must not be used in version greater 1

extensibleUpdateLocationRes	ExtensibleUpdateLocationRes}

-- NU1 extensibleUpdateLocationRes must not be used in version 1

ExtensibleUpdateLocationRes ::= SEQUENCE {

hlr-Number	ISDN-AddressString,

...}

CancelLocationArg ::= CHOICE {

imsi			IMSI,

imsi-WithLMSI	IMSI-WithLMSI}

PurgeMS-Arg ::= SEQUENCE {

imsi			IMSI,

vlr-Number	ISDN-AddressString,

...}

IMSI-WithLMSI ::= SEQUENCE {

imsi			IMSI,

lmsi			LMSI,

...}

SendIdentificationRes ::= SEQUENCE {

imsi			IMSI,

authenticationSetList	AuthenticationSetList	OPTIONAL,

...}

AuthenticationSetList ::= SEQUENCE SIZE (1..5) OF

AuthenticationSet

AuthenticationSet ::= SEQUENCE {

rand			RAND,

sres			SRES,

kc			Kc,

...}

RAND ::= OCTET STRING (SIZE (16))

SRES ::= OCTET STRING (SIZE (4))

Kc ::= OCTET STRING (SIZE (8))

-- handover types

PrepareHO-Arg ::= SEQUENCE {

targetCellId	GlobalCellId	OPTIONAL,

ho-NumberNotRequired	NULL			OPTIONAL,

bss-APDU		ExternalSignalInfo	OPTIONAL,

...}

PerformHO-Arg ::= SEQUENCE {

targetCellId	GlobalCellId,

servingCellId	GlobalCellId,

channelType	ChannelType,

classmarkInfo	ClassmarkInfo,

handoverPriority	[11] HandoverPriority	OPTIONAL,

kc			[12] Kc		OPTIONAL}

ChannelType ::= OCTET STRING (SIZE (1..10))

-- concatenation of

-- speech data indicator

-- channel rate

-- speech encoding algorithm/ data rate

-- and transparency indicator

-- as defined in GSM 08.08

ClassmarkInfo ::= OCTET STRING (SIZE (1..2))

-- classmark information element type 1 or 2 contents as defined

-- in GSM 08.08

HandoverPriority ::= OCTET STRING (SIZE (1))

-- The internal structure is defined in GSM 08.08.

PrepareHO-Res ::= SEQUENCE {

handoverNumber	ISDN-AddressString	OPTIONAL,

bss-APDU		ExternalSignalInfo	OPTIONAL,

...}

PerformHO-Res ::= SEQUENCE {

handoverNumber	ISDN-AddressString,

accessSignalInfo	ExternalSignalInfo}

PrepareSubsequentHO-Arg ::= SEQUENCE {

targetCellId	GlobalCellId,

targetMSC-Number	ISDN-AddressString,

bss-APDU		ExternalSignalInfo,

...}

PerformSubsequentHO-Arg ::= SEQUENCE {

targetCellId	GlobalCellId,

servingCellId	GlobalCellId,

targetMSC-Number	ISDN-AddressString,

classmarkInfo	[10] ClassmarkInfo	OPTIONAL}

-- authentication management types

SendAuthenticationInfoArg ::= IMSI

SendAuthenticationInfoRes ::= AuthenticationSetList

Ki ::= OCTET STRING (SIZE (16))

-- security management types

EquipmentStatus ::= ENUMERATED {

whiteListed  (0),

blackListed  (1),

greyListed  (2)}

-- subscriber management types

SendParametersArg ::= SEQUENCE {

subscriberId	SubscriberId,

requestParameterList	RequestParameterList}

RequestParameterList ::= SEQUENCE SIZE (1..2) OF

RequestParameter

RequestParameter ::= ENUMERATED {

requestIMSI  (0),

requestAuthenticationSet  (1),

requestSubscriberData  (2),

requestKi  (4)}

SentParameterList ::= SEQUENCE SIZE (1..maxNumOfSentParameter) OF

SentParameter

SentParameter ::= CHOICE {

imsi			[0] IMSI,

authenticationSet	[1] AuthenticationSet,

subscriberData	[2] SubscriberData,

ki			[4] Ki}

maxNumOfSentParameter  INTEGER ::= 6

-- This NamedValue corresponds to the maximum number of

-- authentication set which may be returned by a HLR plus 1

InsertSubscriberDataArg ::= SEQUENCE {

imsi			[0] IMSI		OPTIONAL,

COMPONENTS OF	SubscriberData,

...}

SubscriberData ::= SEQUENCE {

msisdn		[1] ISDN-AddressString	OPTIONAL,

category		[2] Category	OPTIONAL,

subscriberStatus	[3] SubscriberStatus	OPTIONAL,

bearerServiceList	[4] BearerServiceList	OPTIONAL,

teleserviceList	[6] TeleserviceList	OPTIONAL,

provisionedSS	[7] SS-InfoList	OPTIONAL,

odb-Data		[8] ODB-Data	OPTIONAL,

-- OA1 odb-Data must be absent in version 1

roamingRestrictionDueToUnsupportedFeature  [9] NULL	OPTIONAL,

-- OA1 RoamingRestrictionDueToUnsupportedFeature must be absent

-- in version 1

regionalSubscriptionData	[10] ZoneCodeList	OPTIONAL

-- OA1 RegionalSubscriptionData must be absent in version 1

}

SubscriberStatus ::= ENUMERATED {

serviceGranted  (0),

operatorDeterminedBarring  (1)}

ODB-Data ::= SEQUENCE {

odb-GeneralData	ODB-GeneralData,

odb-HPLMN-Data	ODB-HPLMN-Data	OPTIONAL,

...}

ODB-GeneralData ::= BIT STRING {

allOG-CallsBarred  (0),

internationalOGCallsBarred  (1),

internationalOGCallsNotToHPLMN-CountryBarred  (2),

premiumRateInformationOGCallsBarred  (3),

premiumRateEntertainementOGCallsBarred  (4),

ss-AccessBarred  (5)} (SIZE (6))

ODB-HPLMN-Data ::= BIT STRING {

plmn-SpecificBarringType1  (0),

plmn-SpecificBarringType2  (1),

plmn-SpecificBarringType3  (2),

plmn-SpecificBarringType4  (3)} (SIZE (4))

ZoneCodeList ::= SEQUENCE SIZE (1..maxNumOfZoneCodes)

OF ZoneCode

ZoneCode ::= OCTET STRING (SIZE (2))

-- internal structure is defined in GSM 03.03

maxNumOfZoneCodes  INTEGER ::= 10

InsertSubscriberDataRes ::= SEQUENCE {

teleserviceList	[1] TeleserviceList	OPTIONAL,

bearerServiceList	[2] BearerServiceList	OPTIONAL,

ss-List		[3] SS-List	OPTIONAL,

odb-GeneralData	[4] ODB-GeneralData	OPTIONAL,

regionalSubscriptionResponse	[5]

RegionalSubscriptionResponse			OPTIONAL,

-- OA1 regionalSubscriptionResponse must be absent in version 1

...}

RegionalSubscriptionResponse ::= ENUMERATED {

msc-AreaRestricted	(0),

tooManyZoneCodes	(1),

zoneCodesConflict	(2),

regionalSubscNotSupported	(3)}

DeleteSubscriberDataArg ::= SEQUENCE {

imsi			[0] IMSI,

basicServiceList	[1] BasicServiceList	OPTIONAL,

ss-List		[2] SS-List	OPTIONAL,

cug-InterlockList	[3] CUG-InterlockList	OPTIONAL,

-- OA1 CUG-InterlockList must be absent in version 1

roamingRestrictionDueToUnsupportedFeature [4] NULL	OPTIONAL,

-- OA1 RoamingRestrictionDueToUnsupportedFeature must be absent

-- in version 1

regionalSubscriptionData	[5] ZoneCodeList	OPTIONAL,

-- OA1 regionalSubscriptionData must be absent in version 1

...}

DeleteSubscriberDataRes ::= SEQUENCE {

regionalSubscriptionResponse	[0]

RegionalSubscriptionResponse			OPTIONAL,

...}

-- fault recovery types

ResetArg ::= SEQUENCE {

networkResource	NetworkResource	OPTIONAL,

-- OP1 networkResource must be present in version 1

-- OA>1 networkResource must be absent in version greater 1

hlr-Number	ISDN-AddressString,

hlr-List		HLR-List		OPTIONAL,

...}

RestoreDataArg ::= SEQUENCE {

imsi			IMSI,

lmsi			LMSI			OPTIONAL,

...}

RestoreDataRes ::= SEQUENCE {

hlr-Number	ISDN-AddressString,

msNotReachable	NULL			OPTIONAL,

...}

.#END

14.7.2	Operation and Maintenance Data Types

.$MAP-OM-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-OM-DataTypes (12) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

ActivateTraceModeArg,

DeactivateTraceModeArg,

TraceSubscriberActivityArg,

NoteInternalHO-Arg

;

IMPORTS

AddressString,

ExternalSignalInfo,

IMSI,

GlobalCellId

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

ActivateTraceModeArg ::= SEQUENCE {

imsi			[0] IMSI		OPTIONAL,

traceReference	[1] TraceReference,

traceType	[2] TraceType,

omc-Id		[3] AddressString	OPTIONAL,

...}

TraceReference ::= OCTET STRING (SIZE (1..2))

TraceType ::= INTEGER

(0..255)

-- Trace types are more fully defined in TS GSM 12.02.

DeactivateTraceModeArg ::= SEQUENCE {

imsi			[0] IMSI		OPTIONAL,

traceReference	[1] TraceReference,

...}

TraceSubscriberActivityArg ::= SEQUENCE {

imsi			[0] IMSI		OPTIONAL,

traceReference	[1] TraceReference,

traceType	[2] TraceType,

omc-Id		[3] AddressString	OPTIONAL,

callReference	[4] CallReference	OPTIONAL}

CallReference ::= OCTET STRING (SIZE (1..3))

NoteInternalHO-Arg ::= SEQUENCE {

handoverType	HandoverType,

targetCellId	[1] GlobalCellId	OPTIONAL,

channelId	[2] ExternalSignalInfo	OPTIONAL}

HandoverType ::= ENUMERATED {

interBSS  (0),

intraBSS  (1)}

.#END

14.7.3	Call Handling Data Types

.$MAP-CH-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

SendRoutingInfoArg,

SendRoutingInfoRes,

ProvideRoamingNumberArg

;

IMPORTS

CUG-Interlock,

ForwardingData

FROM MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

ISDN-AddressString,

ExternalSignalInfo,

IMSI,

LMSI

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

;

CUG-CheckInfo ::= SEQUENCE {

cug-Interlock	CUG-Interlock,

cug-OutgoingAccess	NULL			OPTIONAL,

...}

NumberOfForwarding ::= INTEGER (1..5)

SendRoutingInfoArg ::= SEQUENCE {

msisdn		[0] ISDN-AddressString,

cug-CheckInfo	[1] CUG-CheckInfo	OPTIONAL,

-- OA1 cug-CheckInfo must be absent in version 1

numberOfForwarding	[2] NumberOfForwarding	OPTIONAL,

networkSignalInfo	[10] ExternalSignalInfo	OPTIONAL,

...}

SendRoutingInfoRes ::= SEQUENCE {

imsi			IMSI,

routingInfo	RoutingInfo,

cug-CheckInfo	CUG-CheckInfo	OPTIONAL,

-- OA1 cug-CheckInfo must be absent in version 1

...}

RoutingInfo ::= CHOICE {

roamingNumber	ISDN-AddressString,

forwardingData	ForwardingData}

ProvideRoamingNumberArg ::= SEQUENCE {

imsi			[0] IMSI,

msc-Number	[1] ISDN-AddressString	OPTIONAL,

-- OP>1 msc-Number must be present in version greater 1

msisdn		[2] ISDN-AddressString	OPTIONAL,

previousRoamingNumber	[3] ISDN-AddressString	OPTIONAL,

lmsi			[4] LMSI		OPTIONAL,

gsm-BearerCapability	[5] ExternalSignalInfo	OPTIONAL,

networkSignalInfo	[6] ExternalSignalInfo	OPTIONAL,

...}

.#END

14.7.4	Supplementary Service Data Types

.$MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RegisterSS-Arg,

SS-Info,

SS-Status,

CUG-Index,

CUG-Interlock,

SS-SubscriptionOption,

SS-ForBS-Code,

InterrogateSS-Res,

SS-UserData,

USSD-Arg,

USSD-Res,

Password,

GuidanceInfo,

BeginSubscriberActivityArg,

SS-List,

SS-InfoList,

ForwardingData,

CUG-InterlockList

;

IMPORTS

AddressString,

ISDN-AddressString,

ISDN-SubaddressString,

maxSignalInfoLength,

IMSI,

BasicServiceCode,

BasicServiceGroupList,

maxNumOfBasicServiceGroups

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

SS-Code

FROM MAP-SS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version2 (2)}

;

RegisterSS-Arg ::= SEQUENCE{

ss-Code		SS-Code,

basicService	BasicServiceCode	OPTIONAL,

forwardedToNumber	[4] AddressString	OPTIONAL,

forwardedToSubaddress	[6] ISDN-SubaddressString	OPTIONAL,

-- OA1 forwardedToSubaddress must be absent in version 1

noReplyConditionTime	[5] NoReplyConditionTime	OPTIONAL,

...}

NoReplyConditionTime ::= INTEGER (5..30)

SS-Info ::= CHOICE {

forwardingInfo	[0] ForwardingInfo,

callBarringInfo	[1] CallBarringInfo,

cug-Info		[2] CUG-Info,

-- NU1 cug-info must not be used in version 1

ss-Data		[3] SS-Data}

ForwardingInfo ::= SEQUENCE {

ss-Code		SS-Code		OPTIONAL,

forwardingFeatureList	ForwardingFeatureList,

...}

ForwardingFeatureList ::=

SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

ForwardingFeature

ForwardingFeature ::= SEQUENCE {

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

-- OP1 ss-Status must be present in version 1

forwardedToNumber	[5] ISDN-AddressString	OPTIONAL,

forwardedToSubaddress	[8] ISDN-SubaddressString	OPTIONAL,

-- OA1 forwardedToSubaddress must be absent in version 1

forwardingOptions	[6] ForwardingOptions	OPTIONAL,

noReplyConditionTime	[7] NoReplyConditionTime	OPTIONAL,

...}

SS-Status ::= OCTET STRING (SIZE (1))

-- bits 8765: 0000 (unused)

-- bits 4321: Semantic of one bit shall not be implied

--		    by the semantic of any other bit

-- bit 4: quiescent/operative indicator

--	0  operative

--	1  quiescent

-- bit 3: provision indicator

--	0  not provisioned

--	1  provisioned

-- bit 2: registration indicator

--	0  not registered

--	1  registered

-- bit 1: activation indicator

--	0  not active

--	1  active

ForwardingOptions ::= OCTET STRING (SIZE (1))

-- v1: bits 87: notification to forwarding party

--	00  no notification

--	01  notification

--	10  notification

-- v2: bit 8: notification to forwarding party

--	0  no notification

--	1  notification

-- v2: bit 7: 0 (unused)

-- v1: bits 65: notification to calling party

--	00  no notification

--	01  notification

--	10  notification

-- v2: bit 6: v2: notification to calling party

--	0  no notification

--	1  notification

-- v2: bit 5: 0 (unused)

-- v2: bits 43: forwarding reason

--	00  ms not reachable

--	01  ms busy

--	10  no reply

--	11  unconditional

-- bits 21: 00 (unused)

CallBarringInfo ::= SEQUENCE {

ss-Code		SS-Code		OPTIONAL,

-- OP1 ss-Code must be present in version 1

callBarringFeatureList	CallBarringFeatureList,

...}

CallBarringFeatureList ::=

SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

CallBarringFeature

CallBarringFeature ::= SEQUENCE {

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

-- OP1 ss-Status must be present in version 1

...}

CUG-Info ::= SEQUENCE {

cug-SubscriptionList	CUG-SubscriptionList,

cug-FeatureList	CUG-FeatureList	OPTIONAL,

...}

CUG-InterlockList ::= SEQUENCE SIZE (1..maxNumOfCUG) OF

CUG-Interlock

CUG-SubscriptionList ::= SEQUENCE SIZE (1..maxNumOfCUG) OF

CUG-Subscription

CUG-Subscription ::= SEQUENCE {

cug-Index	CUG-Index,

cug-Interlock	CUG-Interlock,

intraCUG-Options	IntraCUG-Options,

basicServiceGroupList	BasicServiceGroupList	OPTIONAL,

...}

CUG-Index ::= INTEGER (0..32767)

-- The internal structure is defined in ETS 300 138.

CUG-Interlock ::= OCTET STRING (SIZE (4))

IntraCUG-Options ::= ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)}

maxNumOfCUG  INTEGER ::= 10

CUG-FeatureList ::= SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

CUG-Feature

CUG-Feature ::= SEQUENCE {

basicService	BasicServiceCode	OPTIONAL,

preferentialCUG-Indicator	CUG-Index	OPTIONAL,

interCUG-Restrictions	InterCUG-Restrictions,

...}

InterCUG-Restrictions::= OCTET STRING (SIZE (1))

-- bits 876543: 000000 (unused)

-- bits 21

--	00  CUG only facilities

--	01  CUG with outgoing access

--	10  CUG with incoming access

--	11  CUG with both outgoing and incoming access

SS-Data ::= SEQUENCE {

ss-Code		SS-Code		OPTIONAL,

-- OP1 ss-Code must be present in version 1

ss-Status	[4] SS-Status	OPTIONAL,

ss-SubscriptionOption	SS-SubscriptionOption	OPTIONAL,

-- OA1 ss-SubscriptionOption must be absent in version 1

basicServiceGroupList	BasicServiceGroupList	OPTIONAL,

-- OA1 basicServiceGroupList must be absent in version 1

...}

SS-SubscriptionOption ::= CHOICE {

cliRestrictionOption	[2] CliRestrictionOption,

overrideCategory	[1] OverrideCategory}

CliRestrictionOption ::= ENUMERATED {

permanent  (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)}

OverrideCategory ::= ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}

SS-ForBS-Code ::= SEQUENCE {

ss-Code		SS-Code,

basicService	BasicServiceCode	OPTIONAL,

...}

InterrogateSS-Res ::= CHOICE {

ss-Status	[0] SS-Status,

forwardedToNumber	[1] ISDN-AddressString,

-- NU>1 forwardedToNumber must not be used in version greater 1

basicServiceGroupList	[2] BasicServiceGroupList,

forwardingFeatureList	[3] ForwardingFeatureList}

SS-UserData ::= IA5String (SIZE (1.. maxSignalInfoLength))

USSD-Arg ::= SEQUENCE {

ussd-DataCodingScheme	USSD-DataCodingScheme,

ussd-String	USSD-String,

...}

USSD-Res ::= SEQUENCE {

ussd-DataCodingScheme	USSD-DataCodingScheme,

ussd-String	USSD-String,

...}

USSD-DataCodingScheme ::= OCTET STRING (SIZE (1))

-- The structuree of the USSD-DataCodingScheme is described

-- in GSM 03.41

USSD-String ::= OCTET STRING (SIZE (1..maxUSSD-StringLength))

-- The structure of the contents of the USSD-String is dependent

-- on the USSD-DataCodingScheme as described in GSM 03.41.

maxUSSD-StringLength  INTEGER ::= 160

Password ::= NumericString

(FROM ("0"|"1"|"2"|"3"|"4"|"5"|"6"|"7"|"8"|"9"))

(SIZE (4))

GuidanceInfo ::= ENUMERATED {

enterPW  (0),

enterNewPW  (1),

enterNewPW-Again  (2),

badPW-TryAgain  (3),

-- NU>1 badPW-TryAgain must not be used in version greater 1

badPW-FormatTryAgain  (4)}

-- NU>1 badPW-FormatTryAgain must not be used in version greater 1

-- How this information is really delivered to the subscriber

-- (display, announcement, ...) is not part of this

-- specification.

BeginSubscriberActivityArg ::= SEQUENCE {

imsi			IMSI,

originatingEntityNumber	ISDN-AddressString}

SS-List ::= SEQUENCE SIZE (1..maxNumOfSS) OF

SS-Code

maxNumOfSS  INTEGER ::= 30

SS-InfoList ::= SEQUENCE SIZE (1..maxNumOfSS) OF

SS-Info

ForwardingData ::= SEQUENCE {

forwardedToNumber	[5] ISDN-AddressString	OPTIONAL,

-- OP1 forwardedToNumber must be present in version 1

forwardedToSubaddress	[4] ISDN-SubaddressString	OPTIONAL,

-- OA1 forwardedToSubaddress must be absent in version 1

forwardingOptions	[6] ForwardingOptions	OPTIONAL,

...}

.#END

14.7.5	Supplementary Service Codes

.$MAP-SS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version2 (2)}

DEFINITIONS

::=

BEGIN

SS-Code ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- supplementary service, a group of supplementary services, or

-- all supplementary services. The services and abbreviations

-- used are defined in GSM 02.04. The internal structure is

-- defined as follows:

--

-- bits 87654321: group (bits 8765), and specific service

-- (bits 4321)

allSS			SS-Code ::= '00000000'B

-- reserved for possible future use

-- all SS

allLineIdentificationSS	SS-Code ::= '00010000'B

-- reserved for possible future use

-- all line identification SS

clip			SS-Code ::= '00010001'B

-- calling line identification presentation

clir			SS-Code ::= '00010010'B

-- calling line identification restriction

colp			SS-Code ::= '00010011'B

-- connected line identification presentation

colr			SS-Code ::= '00010100'B

-- connected line identification restriction

mci			SS-Code ::= '00010101'B

-- reserved for possible future use

-- malicious call identification

allForwardingSS	SS-Code ::= '00100000'B

-- all forwarding SS

cfu			SS-Code ::= '00100001'B

-- call forwarding unconditional

allCondForwardingSS	SS-Code ::= '00101000'B

-- all conditional forwarding SS

cfb			SS-Code ::= '00101001'B

-- call forwarding on mobile subscriber busy

cfnry			SS-Code ::= '00101010'B

-- call forwarding on no reply

cfnrc			SS-Code ::= '00101011'B

-- call forwarding on mobile subscriber not reachable

allCallOfferingSS	SS-Code ::= '00110000'B

-- reserved for possible future use

-- all call offering SS includes also all forwarding SS

ct				SS-Code ::= '00110001'B

-- reserved for possible future use

-- call transfer

mah			SS-Code ::= '00110010'B

-- reserved for possible future use

-- mobile access hunting

allCallCompletionSS	SS-Code ::= '01000000'B

-- reserved for possible future use

-- all Call completion SS

cw				SS-Code ::= '01000001'B

-- call waiting

hold			SS-Code ::= '01000010'B

-- call hold

ccbs			SS-Code ::= '01000011'B

-- reserved for possible future use

-- completion of call to busy subscribers

allMultiPartySS	SS-Code ::= '01010000'B

-- reserved for possible future use

-- all multiparty SS

multiPTY		SS-Code ::= '01010001'B

-- multiparty

allCommunityOfInterest-SS	SS-Code ::= '01100000'B

-- reserved for possible future use

-- all community of interest SS

cug			SS-Code ::= '01100001'B

-- closed user group

allChargingSS	SS-Code ::= '01110000'B

-- reserved for possible future use

-- all charging SS

aoci			SS-Code ::= '01110001'B

-- advice of charge information

aocc			SS-Code ::= '01110010'B

-- advice of charge charging

allAdditionalInfoTransferSS	SS-Code ::= '10000000'B

-- reserved for possible future use

-- all additional information transfer SS

uus			SS-Code ::= '10000001'B

-- reserved for possible future use

-- UUS user-to-user signalling

allCallRestrictionSS	SS-Code ::= '10010000'B

-- all call restriction SS

barringOfOutgoingCalls	SS-Code ::= '10010001'B

-- barring of outgoing calls

baoc			SS-Code ::= '10010010'B

-- barring of all outgoing calls

boic			SS-Code ::= '10010011'B

-- barring of outgoing international calls

boicExHC		SS-Code ::= '10010100'B

-- barring of outgoing international calls except those directed

-- to the home PLMN

barringOfIncomingCalls	SS-Code ::= '10011001'B

-- barring of incoming calls

baic			SS-Code ::= '10011010'B

-- barring of all incoming calls

bicRoam			SS-Code ::= '10011011'B

-- barring of incoming calls when roaming outside home PLMN

-- Country

allPLMN-specificSS	SS-Code ::= '11110000'B

-- NU1 *plmn-specific* must not be used in version 1

plmn-specificSS-1	SS-Code ::= '11110001'B

plmn-specificSS-2	SS-Code ::= '11110010'B

plmn-specificSS-3	SS-Code ::= '11110011'B

plmn-specificSS-4	SS-Code ::= '11110100'B

plmn-specificSS-5	SS-Code ::= '11110101'B

plmn-specificSS-6	SS-Code ::= '11110110'B

plmn-specificSS-7	SS-Code ::= '11110111'B

plmn-specificSS-8	SS-Code ::= '11111000'B

plmn-specificSS-9	SS-Code ::= '11111001'B

plmn-specificSS-A	SS-Code ::= '11111010'B

plmn-specificSS-B	SS-Code ::= '11111011'B

plmn-specificSS-C	SS-Code ::= '11111100'B

plmn-specificSS-D	SS-Code ::= '11111101'B

plmn-specificSS-E	SS-Code ::= '11111110'B

plmn-specificSS-F	SS-Code ::= '11111111'B

.#END

14.7.6	Short Message Data Types

.$MAP-SM-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RoutingInfoForSM-Arg,

RoutingInfoForSM-Res,

ForwardSM-Arg,

ReportSM-DeliveryStatusArg,

AlertServiceCentreArg,

InformServiceCentreArg,

ReadyForSM-Arg

;

IMPORTS

AddressString,

ISDN-AddressString,

SignalInfo,

IMSI,

SubscriberId,

LocationInfo,

LMSI

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

TeleserviceCode

FROM MAP-TS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version2 (2)}

;

RoutingInfoForSM-Arg ::= SEQUENCE {

msisdn		[0] ISDN-AddressString,

sm-RP-PRI	[1] BOOLEAN,

serviceCentreAddress	[2] AddressString,

teleservice	[5] TeleserviceCode	OPTIONAL,

-- OA>1 teleservice must be absent in version greater 1

...}

RoutingInfoForSM-Res::= SEQUENCE {

imsi			IMSI,

locationInfoWithLMSI		[0] LocationInfoWithLMSI,

mwd-Set		[2] BOOLEAN	OPTIONAL,

-- OA>1 mwd-Set must be absent in version greater 1

...}

LocationInfoWithLMSI ::= SEQUENCE {

locationInfo	LocationInfo,

lmsi		LMSI			OPTIONAL,

...}

ForwardSM-Arg ::= SEQUENCE {

sm-RP-DA	SM-RP-DA,

sm-RP-OA	SM-RP-OA,

sm-RP-UI	SignalInfo,

moreMessagesToSend	NULL			OPTIONAL,

-- OA1 moreMessagesToSend must be absent in version 1

...}

SM-RP-DA ::= CHOICE {

imsi			[0] IMSI,

lmsi			[1] LMSI,

roamingNumber	[3] ISDN-AddressString,

-- NU>1 roaming number must not be used in version greater 1

serviceCentreAddressDA	[4] AddressString,

noSM-SP-DA	[5] NULL}

-- NU1 noSM-SP-DA must not be used in version 1

SM-RP-OA ::= CHOICE {

msisdn		[2] ISDN-AddressString,

serviceCentreAddressOA	[4] AddressString,

noSM-RP-OA	[5] NULL}

-- NU1 noSM-SP-OA must not be used in version 1

ReportSM-DeliveryStatusArg ::= SEQUENCE {

msisdn		ISDN-AddressString,

serviceCentreAddress	AddressString,

sm-DeliveryOutcome	SM-DeliveryOutcome	OPTIONAL,

-- OA1 sm-DeliveryOutcome must be absent in version 1

-- OP>1 sm-DeliveryOutcome must be present in version greater 1

...}

SM-DeliveryOutcome ::= ENUMERATED {

memoryCapacityExceeded  (0),

absentSubscriber  (1),

successfulTransfer  (2)}

AlertServiceCentreArg ::= SEQUENCE {

msisdn		ISDN-AddressString,

serviceCentreAddress	AddressString,

...}

InformServiceCentreArg ::= SEQUENCE {

storedMSISDN	ISDN-AddressString	OPTIONAL,

mw-Status	MW-Status	OPTIONAL,

...}

MW-Status ::= BIT STRING {

sc-AddressNotIncluded  (0),

mnrf-Set  (1),

mcef-Set  (2)} (SIZE (6))

ReadyForSM-Arg ::= SEQUENCE {

subscriberId	SubscriberId,

alertReason	AlertReason,

...}

AlertReason ::= ENUMERATED {

ms-Present  (0),

memoryAvailable  (1)}

.#END

14.7.7	Error Data Types

.$MAP-ER-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

RoamingNotAllowedCause,

CallBarringCause,

CUG-RejectCause,

SS-IncompatibilityCause,

PW-RegistrationFailureCause,

SM-DeliveryFailureCause

;

IMPORTS

SS-Status

FROM MAP-SS-DataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version2 (2)}

SignalInfo,

BasicServiceCode

FROM MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

SS-Code

FROM MAP-SS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-Code (15) version2 (2)}

;

RoamingNotAllowedCause ::= ENUMERATED {

plmnRoamingNotAllowed  (0),

operatorDeterminedBarring  (3)}

CallBarringCause ::= ENUMERATED {

barringServiceActive  (0),

operatorBarring  (1)}

CUG-RejectCause ::= ENUMERATED {

incomingCallsBarredWithinCUG  (0),

subscriberNotMemberOfCUG  (1),

requestedBasicServiceViolatesCUG-Constraints  (5),

calledPartySS-InteractionViolation  (7)}

-- NU1 calledPartySS-InteractionViolation must not be used in

-- version 1

SS-IncompatibilityCause ::= SEQUENCE {

ss-Code		[1] SS-Code	OPTIONAL,

basicService	BasicServiceCode	OPTIONAL,

ss-Status	[4] SS-Status	OPTIONAL,

...}

PW-RegistrationFailureCause ::= ENUMERATED {

undetermined  (0),

invalidFormat  (1),

newPasswordsMismatch  (2)}

SM-DeliveryFailureCause ::= CHOICE {

sm-DeliveryFailureCauseWithDiagnostic

SM-DeliveryFailureCauseWithDiagnostic,

-- NU1 SM-DeliveryFailureCauseWithDiagnostic must not be used in

-- version 1

sm-EnumeratedDeliveryFailureCause

SM-EnumeratedDeliveryFailureCause}

-- NU>1 SM-EnumeratedDeliveryFailureCause must not be used in version

-- greater 1

SM-EnumeratedDeliveryFailureCause ::= ENUMERATED {

memoryCapacityExceeded  (0),

equipmentProtocolError  (1),

equipmentNotSM-Equipped  (2),

unknownServiceCentre  (3),

sc-Congestion  (4),

invalidSME-Address  (5),

subscriberNotSC-Subscriber  (6)}

SM-DeliveryFailureCauseWithDiagnostic ::= SEQUENCE {

sm-EnumeratedDeliveryFailureCause    SM-EnumeratedDeliveryFailureCause,

diagnosticInfo	SignalInfo	OPTIONAL,

...}

.#END

14.7.8	Common Data Types

.$MAP-CommonDataTypes {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version2 (2)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

-- general data types and values

AddressString,

ISDN-AddressString,

ISDN-SubaddressString,

ExternalSignalInfo,

SignalInfo,

maxSignalInfoLength,

-- data types for numbering and identification

IMSI,

TMSI,

SubscriberId,

IMEI,

LocationInfo,

HLR-List,

LMSI,

GlobalCellId,

NetworkResource,

-- data types for subscriber management

Category,

BearerServiceList,

TeleserviceList,

BasicServiceCode,

BasicServiceList,

BasicServiceGroupList,

maxNumOfBasicServiceGroups

;

IMPORTS

TeleserviceCode

FROM MAP-TS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version2 (2)}

BearerServiceCode

FROM MAP-BS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-BS-Code (20) version2 (2)}

;

-- general data types

TBCD-STRING ::= OCTET STRING

-- This type (Telephony Binary Coded Decimal String) is used to

-- represent several digits from 0 through 9, *, #, a, b, c, two

-- digits per octet, each digit encoded 0000 to 1001 (0 to 9),

-- 1010 (*), 1011 (#), 1100 (a), 1101 (b) or 1110 (c); 1111 used

-- as filler when there is an odd number of digits.

-- bits 8765 of octet n encoding digit 2n

-- bits 4321 of octet n encoding digit 2(n-1) +1

AddressString ::= OCTET STRING (SIZE (1..maxAddressLength))

-- This type is used to represent a number for addressing

-- purposes. It is composed of

--	a)	one octet for nature of address, and numbering plan

--		indicator.

--	b)	digits of an address encoded as TBCD-String.

-- a)	The first octet includes a one bit extension indicator, a

--		3 bits nature of address indicator and a 4 bits numbering

--		 plan indicator, encoded as follows:

-- bit 8: 1  (no extension)

-- bits 765: nature of address indicator

--	000  unknown

--	001  international number

--	010  national significant number

--	011  network specific number

--	100  subscriber number

--	101  reserved

--	110  abbreviated number

--	111  reserved for extension

-- bits 4321: numbering plan indicator

--	0000  unknown

--	0001  ISDN/Telephony Numbering Plan (Rec CCITT E.164)

--	0010  spare

--	0011  data numbering plan (Rec CCITT X.121)

--	0100  telex numbering plan (Rec CCITT F.69)

--	0101  spare

--	0110  land mobile numbering plan (REC CCITT E.212)

--	0111  spare

--	1000  national numbering plan

--	1001  private numbering plan

--	1111  reserved for extension

--	all other values are reserved.

-- b)	The following octets representing digits of an address

--		encoded as a TBCD-STRING.

maxAddressLength  INTEGER ::= 20

ISDN-AddressString ::=

AddressString (SIZE (1..maxISDN-AddressLength))

-- This type is used to represent ISDN numbers.

maxISDN-AddressLength  INTEGER ::= 9

ISDN-SubaddressString ::=

OCTET STRING (SIZE (1..maxISDN-SubaddressLength))

-- This type is used to represent ISDN subaddresses.

-- It is composed of

--	a)	one octet for type of subaddress and odd/even indicator.

--	b)	20 octets for subaddress information.

--	a)	The first octet includes a one bit extension indicator, a

--		3 bits type of subaddress and a one bit odd/even indicator,

--		encoded as follows:

--	bit 8: 1  (no extension)

--	bits 765: type of subaddress

--		000  NSAP (X.213/ISO 8348 AD2)

--		010  User Specified

--		All other values are reserved

--	bit 4: odd/even indicator

--		0  even number of address signals

--		1  odd number of address signals

--		The odd/even indicator is used when the type of subaddress

--		is "user specified" and the coding is BCD.

--	bits 321: 000 (unused)

--	b) Subaddress information.

--	The NSAP X.213/ISO8348AD2 address shall be formatted as specified

--	by octet 4 which contains the Authority and Format Identifier

--	(AFI). The encoding is made according to the "preferred binary

--	encoding" as defined in X.213/ISO834AD2. For the definition

--	of this type of subaddress, see Rec. CCITT I.334.

--	For User-specific subaddress, this field is encoded according

--	to the user specification, subject to a maximum length of 20

--	octets. When interworking with X.25 networks BCD coding should

--	be applied.

maxISDN-SubaddressLength  INTEGER ::= 21

ExternalSignalInfo ::= SEQUENCE {

protocolId	ProtocolId,

signalInfo	SignalInfo,

...}

-- Information about the internal structure is given in

-- section 5.6.9.

SignalInfo ::= OCTET STRING (SIZE (1..maxSignalInfoLength))

maxSignalInfoLength  INTEGER ::= 200

-- This NamedValue represents the theoretical maximum number of

-- octets which are available to carry a single data type,

-- without requiring segmentation to cope with the network layer

-- service. However, the actual maximum size available for a data

-- type may be lower, especially when other information elements

-- have to be included in the same component.

ProtocolId ::= ENUMERATED {

gsm-0408  (1),

gsm-0806  (2),

gsm-BSSMAP  (3),

-- NU1 NU2 Value 3 must not be used in version 1 and

-- must not be used in version 2.

ets-300102-1  (4)}

-- data types for numbering and identification

IMSI ::= TBCD-STRING (SIZE (3..8))

-- digits of MCC, MNC, MSIN are concatenated in this order.

TMSI ::= OCTET STRING (SIZE (1..4))

SubscriberId ::= CHOICE {

imsi			[0] IMSI,

tmsi			[1] TMSI}

IMEI ::= TBCD-STRING (SIZE (8))

-- The internal Structure is defined as follows:

--	type approval code: 6 digits

--	manufacturer place: 2 digits

--	individual serial number: 6 digits

--	unassigned: 1 digit

LocationInfo ::= CHOICE {

roamingNumber	[0] ISDN-AddressString,

-- NU>1 roamingNumber must not be used in version greater 1

msc-Number	[1] ISDN-AddressString}

HLR-Id ::= IMSI

-- leading digits of IMSI, i.e. (MCC, MNC, leading digits of

-- MSIN) forming HLR Id defined in GSM 03.03.

HLR-List ::= SEQUENCE SIZE (1..maxNumOfHLR-Id) OF

HLR-Id

maxNumOfHLR-Id  INTEGER ::= 50

LMSI ::= OCTET STRING (SIZE (4))

GlobalCellId ::= OCTET STRING (SIZE (5..7))

-- Refers to Cell Global Identification defined in GSM 03.03.

-- Octets are coded according to GSM 04.08.

-- The internal structure is defined as follows:

--	Mobile Country Code:	3 digits according to Rec E.212

--			1 digit filler (1111)

--	Mobile Network Code:	2 digits according to Rec E.212

--	Location Area Code:	2 octets according to GSM 04.08

--	Cell Identity:	2 octets (CI) according to GSM 04.08

NetworkResource ::= ENUMERATED {

plmn  (0),

hlr  (1),

vlr  (2),

pvlr  (3),

controllingMSC  (4),

vmsc  (5),

eir  (6),

rss  (7)}

-- data types for subscriber management

Category ::= OCTET STRING (SIZE (1))

-- The internal structure is defined in CCITT Rec Q.763.

BearerServiceList ::= SEQUENCE SIZE (1..maxNumOfBearerServices) OF

BearerServiceCode

maxNumOfBearerServices  INTEGER ::= 50

TeleserviceList ::= SEQUENCE SIZE (1..maxNumOfTeleservices) OF

TeleserviceCode

maxNumOfTeleservices  INTEGER ::= 20

BasicServiceCode ::= CHOICE {

bearerService	[2] BearerServiceCode,

teleservice	[3] TeleserviceCode}

BasicServiceList ::= SEQUENCE SIZE (1..maxNumOfBasicServices) OF

BasicServiceCode

maxNumOfBasicServices  INTEGER ::= 70

BasicServiceGroupList ::= SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF

BasicServiceCode

maxNumOfBasicServiceGroups  INTEGER ::= 13

.#END

14.7.9	Teleservice Codes

.$MAP-TS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-TS-Code (19) version2 (2)}

DEFINITIONS

::=

BEGIN

TeleserviceCode ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- teleservice, a group of teleservices, or all teleservices. The

-- services are defined in GSM 02.03.

-- The internal structure is defined as follows:

-- bits 87654321: group (bits 8765) and specific service

-- (bits 4321)

allTeleservices	TeleserviceCode ::= '00000000'B

allSpeechTransmissionServices	TeleserviceCode ::= '00010000'B

telephony		TeleserviceCode ::= '00010001'B

emergencyCalls	TeleserviceCode ::= '00010010'B

allShortMessageServices	TeleserviceCode ::= '00100000'B

shortMessageMT-PP	TeleserviceCode ::= '00100001'B

shortMessageMO-PP	TeleserviceCode ::= '00100010'B

allFacsimileTransmissionServices	TeleserviceCode ::= '01100000'B

facsimileGroup3AndAlterSpeech	TeleserviceCode ::= '01100001'B

automaticFacsimileGroup3	TeleserviceCode ::= '01100010'B

facsimileGroup4	TeleserviceCode ::= '01100011'B

-- NU1 facsimileGroup4 must not be used in version 1

-- The following non-hierarchical Compound Teleservice Groups

-- are defined in TS GSM 02.30:

allDataTeleservices	TeleserviceCode ::= '01110000'B

-- covers Teleservice Groups 'allFacsimileTransmissionServices'

-- and 'allShortMessageServices'

allTeleservices-ExeptSMS	TeleserviceCode ::= '10000000'B

-- covers Teleservice Groups 'allSpeechTransmissionServices' and

-- 'allFacsimileTransmissionServices'

--

-- Compound Teleservice Group Codes are only used in call

-- independent supplementary service operations, i.e. they

-- are not used in InsertSubscriberData or in

-- DeleteSubscriberData messages. Compound Teleservice Group

-- NU1 Codes must not be used in version 1.

allPLMN-specificTS	TeleserviceCode ::= '11010000'B

-- NU1 *plmn-specific* must not be used in version 1

plmn-specificTS-1	TeleserviceCode ::= '11010001'B

plmn-specificTS-2	TeleserviceCode ::= '11010010'B

plmn-specificTS-3	TeleserviceCode ::= '11010011'B

plmn-specificTS-4	TeleserviceCode ::= '11010100'B

plmn-specificTS-5	TeleserviceCode ::= '11010101'B

plmn-specificTS-6	TeleserviceCode ::= '11010110'B

plmn-specificTS-7	TeleserviceCode ::= '11010111'B

plmn-specificTS-8	TeleserviceCode ::= '11011000'B

plmn-specificTS-9	TeleserviceCode ::= '11011001'B

plmn-specificTS-A	TeleserviceCode ::= '11011010'B

plmn-specificTS-B	TeleserviceCode ::= '11011011'B

plmn-specificTS-C	TeleserviceCode ::= '11011100'B

plmn-specificTS-D	TeleserviceCode ::= '11011101'B

plmn-specificTS-E	TeleserviceCode ::= '11011110'B

plmn-specificTS-F	TeleserviceCode ::= '11011111'B

.#END

14.7.10	Bearer Service Codes

.$MAP-BS-Code {

ccitt identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-BS-Code (20) version2 (2)}

DEFINITIONS

::=

BEGIN

BearerServiceCode ::= OCTET STRING (SIZE (1))

-- This type is used to represent the code identifying a single

-- bearer service, a group of bearer services, or all bearer

-- services. The services are defined in GSM 02.02.

-- The internal structure is defined as follows:

--

-- v1: bit 8: Transparency indicator (If applicable)

--	0	transparent (postfix T) or if not applicable

--		(no postfix T or NT)

--	1	non transparent (postfix NT)

--

-- v2: bit 8: 0 (unused)

-- plmn-specific bearer services:

-- bits 87654321: defined by the HPLMN operator

-- rest of bearer services:

--

-- bits 7654321: group (bits 7654), and rate, if applicable

-- (bits 321)

-- symbolic constants are assigned only for v2

allBearerServices	BearerServiceCode ::= '00000000'B

allDataCDA-Services	BearerServiceCode ::= '00010000'B

dataCDA-300bps	BearerServiceCode ::= '00010001'B

dataCDA-1200bps	BearerServiceCode ::= '00010010'B

dataCDA-1200-75bps	BearerServiceCode ::= '00010011'B

dataCDA-2400bps	BearerServiceCode ::= '00010100'B

dataCDA-4800bps	BearerServiceCode ::= '00010101'B

dataCDA-9600bps	BearerServiceCode ::= '00010110'B

allDataCDS-Services	BearerServiceCode ::= '00011000'B

dataCDS-1200bps	BearerServiceCode ::= '00011010'B

dataCDS-2400bps	BearerServiceCode ::= '00011100'B

dataCDS-4800bps	BearerServiceCode ::= '00011101'B

dataCDS-9600bps	BearerServiceCode ::= '00011110'B

allPadAccessCA-Services	BearerServiceCode ::= '00100000'B

padAccessCA-300bps	BearerServiceCode ::= '00100001'B

padAccessCA-1200bps	BearerServiceCode ::= '00100010'B

padAccessCA-1200-75bps	BearerServiceCode ::= '00100011'B

padAccessCA-2400bps	BearerServiceCode ::= '00100100'B

padAccessCA-4800bps	BearerServiceCode ::= '00100101'B

padAccessCA-9600bps	BearerServiceCode ::= '00100110'B

allDataPDS-Services	BearerServiceCode ::= '00101000'B

dataPDS-2400bps	BearerServiceCode ::= '00101100'B

dataPDS-4800bps	BearerServiceCode ::= '00101101'B

dataPDS-9600bps	BearerServiceCode ::= '00101110'B

allAlternateSpeech-DataCDA	BearerServiceCode ::= '00110000'B

allAlternateSpeech-DataCDS	BearerServiceCode ::= '00111000'B

allSpeechFollowedByDataCDA	BearerServiceCode ::= '01000000'B

allSpeechFollowedByDataCDS	BearerServiceCode ::= '01001000'B

-- The following non-hierarchical Compound Bearer Service

-- Groups are defined in TS GSM 02.30:

allDataCircuitAsynchronous	BearerServiceCode ::= '01010000'B

-- covers "allDataCDA-Services", "allAlternateSpeech-DataCDA" and

-- "allSpeechFollowedByDataCDA"

allAsynchronousServices	BearerServiceCode ::= '01100000'B

-- covers "allDataCDA-Services", "allAlternateSpeech-DataCDA",

-- "allSpeechFollowedByDataCDA" and "allPadAccessCDA-Services"

allDataCircuitSynchronous	BearerServiceCode ::= '01011000'B

-- covers "allDataCDS-Services", "allAlternateSpeech-DataCDS" and

-- "allSpeechFollowedByDataCDS"

allSynchronousServices	BearerServiceCode ::= '01101000'B

-- covers "allDataCDS-Services", "allAlternateSpeech-DataCDS",

-- "allSpeechFollowedByDataCDS" and "allDataPDS-Services"

--

-- Compound Bearer Service Group Codes are only used in call

-- independent supplementary service operations, i.e. they

-- are not used in InsertSubscriberData or in

-- DeleteSubscriberData messages.

-- NU1 Compound Bearer Service Group Codes must not be used in

-- version 1.

allPLMN-specificBS	BearerServiceCode ::= '11010000'B

-- NU1 *plmn-specific* must not be used in version 1

plmn-specificBS-1	BearerServiceCode ::= '11010001'B

plmn-specificBS-2	BearerServiceCode ::= '11010010'B

plmn-specificBS-3	BearerServiceCode ::= '11010011'B

plmn-specificBS-4	BearerServiceCode ::= '11010100'B

plmn-specificBS-5	BearerServiceCode ::= '11010101'B

plmn-specificBS-6	BearerServiceCode ::= '11010110'B

plmn-specificBS-7	BearerServiceCode ::= '11010111'B

plmn-specificBS-8	BearerServiceCode ::= '11011000'B

plmn-specificBS-9	BearerServiceCode ::= '11011001'B

plmn-specificBS-A	BearerServiceCode ::= '11011010'B

plmn-specificBS-B	BearerServiceCode ::= '11011011'B

plmn-specificBS-C	BearerServiceCode ::= '11011100'B

plmn-specificBS-D	BearerServiceCode ::= '11011101'B

plmn-specificBS-E	BearerServiceCode ::= '11011110'B

plmn-specificBS-F	BearerServiceCode ::= '11011111'B

.#END

15	General on MAP user procedures

15.1	Introduction

Sections 15 to 21 describe the use of MAP services for GSM signalling procedures. GSM signalling procedures may involve one or several interfaces running one or several application protocols. This specification addresses only the signalling procedures which require at least the use of one MAP service.

When a signalling procedure takes place in the network, an application process invocation is created in each system component involved. Part of the application process invocation acts as a MAP user and handles one or several MAP dialogues. For each dialogue it employs an instance of the MAP service provider. It may also use other communication services to exchange information on other interfaces, but detailed description of these aspects is outside the scope of this specification.

15.2	Common aspects of user procedure descriptions

15.2.1	General conventions

For each signalling procedure this specification provides a brief textual overview accompanied by a flow diagram which represent the functional interactions between system components. Functional interactions are labelled using the MAP service name when the interaction results from a service request or by this service name followed by the symbol "ack" when this interaction results from a service response.

For each of the system components involved, this specification also provides a detailed textual description of the application process behaviour as well as an SDL diagram. SDL diagrams describe the sequence of events, as seen by the MAP-User, which occurs at MAP service provider boundaries as well as external events which occur at other interfaces and which impact on the previous sequence.

External events do not necessarily correspond to the messages of other protocols used in the system component. The MAP-user procedures are described as if a set of interworking functions (IWF) between the MAP-user and the other protocol entities was implemented (see figure 15.2/1). Such interworking functions are assumed to perform either an identity mapping or some processing or translation as required to eliminate information irrelevant to the MAP-user.

The mapping of service primitives on to protocol elements is described in sections 11 to 14.

GSM signalling procedures are built from one or more sub-procedures (e.g. authentication, ciphering, ....). Sub-procedures from which signalling procedures are built are represented using SDL MACRO descriptions.

In case of any discrepancy between the textual descriptions and the SDL descriptions, the latter take precedence.

15.2.2	Naming conventions

Events related to MAP are represented by MAP service primitives. The signal names used in the SDL diagrams are derived from the service primitive names defined in sections 5 to 10, with some lexical transformations for readability and parsability purposes (blanks between words are replaced by underscores, the first letter of each word is capitalized).

Events received and sent on other interfaces are named by appending the message or signal name to a symbol representing the interface type, with some lexical transformations for readability and parsability purposes (blanks between words are replaced by underscores, the first letter of each word is capitalized).

The following symbols are used to represent the interface types:

"I":		For interfaces to the fixed network. "I" stands for ISUP interface.

"A":		For interfaces to BSS (i.e. A-Interfaces);

"OM":		For network management interfaces (communication with OMC, MML interface, ...);

"SC":		For interfaces to a Service Centre;

"HO_CA":	For internal interfaces to the Handover Control Application.

"US":		For a local USSD application.

These naming conventions can be summarized by the following BNF description:

<Event_Name> ::= <MAP_Primitive> | <External_Event>

<MAP_Primitive> ::= <MAP_Open> | <MAP_Close> | <MAP_U_Abort> | <MAP_P_Abort> | <MAP_Specific> | <MAP_Notice>

<MAP_Open> ::= MAP_Open_Req | MAP_Open_Ind | MAP_Open_Rsp | MAP_Open_Cnf

<MAP_Close> ::= MAP_Close_Req | MAP_Close_Ind

<MAP_U_Abort> ::= MAP_U_Abort_Req | MAP_U_Abort_Ind

<MAP_P_Abort> ::= MAP_P_Abort_Ind

<MAP_Notice> ::= MAP_Notice_Ind

<MAP_Specific> ::= <MAP_Req> | <MAP_Ind> | <MAP_Rsp> | <MAP_Cnf>

<MAP_Req> ::= MAP_<Service_Name>_Req

<MAP_Ind> ::= MAP_<Service_Name>_Ind

<MAP_Rsp> ::= MAP_<Service_Name>_Rsp

<MAP_Cnf> ::= MAP_<Service_Name>_Cnf

<External_Event> ::= <Interface_Type>_<External_Signal>

<Interface_Type> ::= I | A | OM | SC | HO AC | US

<External_Signal> ::= <Lexical_Unit>

<Service_Name> ::= <Lexical_Unit>

<Lexical_Unit> ::= <Lexical_Component> | <Lexical_Unit>_ <Lexical_Component>

<Lexical_Component> ::= <Upper_Case_Letter><Letter_Or_Digit_List>

<Letter_Or_Digit_List> ::= <Letter_Or_Digit> | <Letter_Or_Digit_List><Letter_Or_Digit>

<Letter_Or_Digit> ::= <Letter>|<Digit>

<Letter> ::= <Lower_Case_Letter>|<Upper_Case_Letter>

<Upper_Case_Letter> ::= A|B|C|D|E|F|G|H|I|J|K|L|M|N|O|P|Q|R|S|T|U|V|W|X|Y|Z

<Lower_Case_Letter> ::= a|b|c|d|e|f|g|h|i|j|k|l|m|n|o|p|q|r|s|t|u|v|w|x|y|z

<Digit> ::= 1|2|3|4|5|6|7|8|9|0

│

│

O&M (GSM 12.xy)           SC (GSM 03.40)

│                      │

┌────┴──┐                ┌──┴─────┐

│  IWF  │                │  IWF   │

│       │                │        │

└────┬──┘                └─┬──────┘

│                     │

└─OM_<xx>     ┌─SC_<xx>

│        │

┌───┴────────┴───┐

┌───────┐        │                │          ┌───────┐

───A──┤  IWF  ├─A_<xx>─┤    MAP-User    ├──I_<xx>──┤  IWF  ├─ISUP

GSM  │       │        │                │          │       │(CCITT

08.08 └───────┘      ┌─┤                ├──┐       └───────┘ Q.763)

04.08                │ └────────┬───────┘  │       ┌───────────┐

┌────────┐           │          │          │       │ USSD      │

│Handover│           │┌─────────┴───────┐  └US_<xx>┤           │

│Control ├─HO_CA_<xx>┘│   MAP-Provider  │          │Application│

│Applica │            │                 │          └───────────┘

│    tion│            └─────────────────┘

└────────┘

Figure 15.2/1: Interfaces applicable to the MAP-User

15.2.3	Convention on primitives parameters

15.2.3.1	Open service

When the originating and destination reference parameters shall be included in the MAP-OPEN request primitive, their value are indicated as a comment to the signal which represents this primitive.

15.2.3.2	Close service

When a pre-arranged released is requested, a comment is attached to the signal which represents the MAP-CLOSE request primitive. In the absence of comment, a normal release is assumed.

15.2.4	Version handling at dialogue establishment

Unless explicitly indicated in subsequent sections, the following principles regarding version handling procedures at dialogue establishment are applied by the MAP-user:

15.2.4.1	Behaviour at the initiating side

When a MAP user signalling procedure has to be executed, the MAP-user issues a MAP-OPEN request primitive with an appropriate application-context-name. If several names are supported (i.e. several versions) a suitable one is selected using the procedures described in section 3.

If a MAP-CLOSE Confirm primitive to a MAP-OPEN request with a result parameter set to "refused" and a diagnostic parameter indicating "application-context-not-supported" or "potential incompatibility problem", the MAP-User issues a new MAP-OPEN request primitive with the equivalent version one context. This is informally represented in the SDL diagrams by a task symbol indicating "Perform V1 procedure".

15.2.4.2	Behaviour at the responding side

On receipt of a MAP-OPEN indication primitive, the MAP-User analyses the application-context-name.

If it refers to a version one context, the associated V1 procedure is executed, otherwise the associated V2 procedure is executed.

15.2.5	Abort Handling

Unless explicitly indicated in subsequent sections, the following principles are applied by the MAP-user regarding abort handling procedures:

On receipt of a MAP-P-ABORT indication or MAP-U-ABORT Indication primitive from any MAP-provider invocation, the MAP-User issues a MAP-U-ABORT Request primitive to each MAP- provider invocation associated with the same user procedure.

If applicable a decision is made to decide if the affected user procedure has to be retried or not.

15.2.6	SDL conventions

The MAP SDLs make use of a number of SDL concepts and conventions, where not all of them may be widely known. Therefore, this section outlines the use of a few concepts and conventions to improve understanding of the MAP SDLs.

The MAP User SDLs make use of SDL Processes, Procedures and Macros. Processes are independent from each other even if one process starts another one: The actions of both of them have no ordering in time. SDL Procedures and Macros are just used to ease writing of the specification: They contain parts of a behaviour used in several places, and the corresponding Procedure/Macro definition has to be expanded at the position of the Procedure/Macro call.

All Processes are started at system initialization and live forever, unless process creation/termination is indicated explicitly (i.e. a process is created by some other process).

The direction of Input/Output Signals in the SDL graphs is used to indicate the entity to which/from which communication is directed. If a process A communicates in parallel with processes B and C, all Inputs/Outputs to/from B are directed to one side, whereas communication with C is directed to the other side. However, there has been no formal convention used that communication to a certain entity (e.g. a HLR) will always be directed to a certain side (e.g. right).

In each state all those Input Signals are listed, which result in an action and/or state change. If an Input Signal is not listed in a state, receipt of this input should lead to an implicit consumption without any action or state change (according to the SDL rules). This implicit consumption is mainly used for receipt of the MAP DELIMITER indication and for receipt of a MAP CLOSE indication, except for a premature MAP CLOSE.

15.3	Interaction between MAP Provider and MAP Users

Each MAP User is defined by at least one SDL process. On the dialogue initiating side the MAP User will create a new instance of a MAP Provider implicit by issuing a MAP OPEN request. This instance corresponds to a TC Dialogue and lives as long as the dialogue exists (see also section 11.3). There is a fix relation between MAP User and this Provider instance, i.e. all MAP service primitives from the MAP User for this dialogue are sent to this instance and all TC components received by this MAP Provider are mapped onto service primitives sent to this MAP User.

On the receiving side a MAP Provider instance is created implicit by receipt of a TC BEGIN indication. The corresponding MAP User is determined by the Application Context name included in this primitive, i.e. each Application Context is associated with one and only one MAP User. An instance of this User will be created implicit by receiving a MAP OPEN indication. Note that in some cases there exist several SDL Processes for one MAP User (Application Context), e.g. the processes Register_SS_HLR, Erase_SS_HLR, Activate_SS_HLR, Deactivate_SS_HLR, Interrogate_SS_HLR, and Register_Password for the AC Network_Functional_SS_Handling. In these cases, a coordinator process is introduced acting as a MAP User, which in turn starts a sub-process depending on the first MAP service primitive received.

16	Mobility procedures

16.1	Location management Procedures

This section comprises a number of processes to handle the mobile nature of the subscriber. The processes will be addressed by SCCP Sub-System Number (MSC, VLR or HLR) and the Application Context. The following processes are defined in this section:

Process Update Location Area;

Initiator: Update_Location_Area_MSC, section 16.1.1.2;

Responder: Update_Location_Area_VLR, section 16.1.1.3;

Process Update Location;

Initiator: Update_Location_Area_VLR, section 16.1.1.3, or

Update_Location_VLR, section 16.1.1.6;

Responder: Update_Location_HLR, section 16.1.1.4;

Process Send Identification;

Initiator: Update_Location_Area_VLR, section 16.1.1.3;

Responder: Send_Identification_VLR, section 16.1.1.5;

Process Subscriber Present HLR;

Initiator: Subscriber_Present_HLR, section 16.1.1.7;

Responder: Short_Message_Alert_IWMSC, section 20.4.3;

Process Cancel Location;

Initiator: Cancel_Location_HLR, section 16.1.2.2;

Responder: Cancel_Location_VLR, section 16.1.2.3;

Process Detach IMSI;

Initiator: Detach_IMSI_MSC, section 16.3.2;

Responder: Detach_IMSI_VLR, section 16.3.3;

As both the Update Location Area and the Detach IMSI processes use the same application context name, the MAP Provider cannot distinguish between them. Therefore, a Location Management Coordinator Process will act as one user for this application context. This process (one in MSC, one in VLR) will create the Update Location Area or the Detach IMSI process, depending on the first service primitive received in the respective dialogue.

Additionally, a Location Management Coordinator process in the HLR coordinates the two application processes "Update Location HLR" (section 16.1.1.4) and "RESTORE_DATA_HLR" (section 16.3.3) that are addressed by the same application context.

Location Management Coordinator MSC

On receipt of a request for location updating from the A-Interface, the Location Management Coordinator in the MSC will:

-	create the process Update_Location_Area_MSC in case the updating type indicated in the A-Interface primitive indicates normal updating, periodic updating or IMSI Attach;

-	create the process Detach_IMSI_MSC in case the updating type indicated in the A-Interface primitive indicates IMSI Detach.

The respective primitive is then forwarded to the created process. Henceforth, the coordinator will relay all service primitives from provider to the user and vice versa, until a request or indication for dialogue termination is received. This last primitive will be relayed, too, before the Coordinator process returns to idle state.

Location Management Coordinator VLR

On receipt of a dialogue request for the Location Management Application Context (see Receive_Open_Ind macro in section 21.1), the Location_Management_Coordinator will:

-	terminate the procedure in case of parameter problems;

-	revert to the MAP version one procedure in case the MSC indicated version one protocol; or

-	continue as below, if the dialogue is accepted.

Depending on the first service primitive received from the MAP Provider in this dialogue, the user process is created:

-	Update_Location_Area_VLR in case the primitive is a MAP_UPDATE_LOCATION_AREA indication;

-	Detach_IMSI_VLR in case the primitive is a MAP_DETACH IMSI indication.

In case a MAP_U_ABORT, MAP_P_ABORT or a premature MAP_CLOSE indication is received instead, the process returns to idle state. If a MAP_NOTICE indication is received, the dialogue towards the MSC is aborted and the process returns to idle state.

After creation of the user process the service primitive received from the provider is passed to the user process. Henceforth, the coordinator will relay all service primitives from provider to the user and vice versa, until a request or indication for dialogue termination is received. This last primitive will be relayed, too, before the Coordinator process returns to idle state.

Location Management Coordinator HLR

On receipt of a dialogue request for the Location Management Application Context (see Receive_Open_Ind macro in section 21.1), the Location_Management_Coordinator will:

-	terminate the process in case of parameter problems; or

-	revert to MAP version 1 protocol if the VLR requests version 1 protocol; or

-	continue as described in the following, if the dialogue is accepted.

The user process is created depending on the first service primitive received from the MAP service provider within this dialogue:

-	Update_Location_HLR if the primitive is a MAP_UPDATE_LOCATION indication;

-	RESTORE_DATA_HLR if the primitive is a MAP_RESTORE_DATA indication.

If a MAP_NOTICE indication is received instead, the dialogue towards the MSC is terminated and the process returns to idle state.

After creation of the user process the service primitive received from the MAP service-provider is passed to the user process. Henceforth, the coordinator will relay all service primitives from MAP service-provider to the MAP service-user and vice versa, until a request or indication for dialogue termination is received. This last primitive will be relayed, too, before the Coordinator process returns to idle state.

Figure 16.1/1: Process Location_Management_Coordinator_MSC

.G.C:\USER\MAP\MAP-470\EPS16\161_1.EPS;18.2 cm;23 cm;EPS

Figure 16.1/2: Process Location_Management_Coordinator_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_2.EPS;18.2 cm;23 cm;EPS

Figure 16.1/3: Process Location_Management_Coordinator_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_3.EPS;18.2 cm;23 cm;EPS

16.1.1	Location updating

16.1.1.1	General

The location updating procedure is used to update the location information held in the network. This location information is used to route incoming calls, short messages and unstructured supplementary service data to the roaming subscriber. Additionally, this procedure is used to provide the VLR with the information that a subscriber already registered, but being detached, is reachable again (IMSI Attach, see TS GSM 03.12). The use of this Detach / Attach feature is optional for the network operator.

To minimize the updates of the subscriber's HLR, the HLR holds only information about the VLR and MSC the subscriber is attached to. The VLR contains more detailed location information, i.e. the location area the subscriber is actually roaming in. Therefore, the VLR needs to be updated at each location area change (see figure 16.1.1/1 for this procedure), whereas the HLR needs updating only in the following cases:

-	when the subscriber registers in a new VLR, i.e. the VLR has no data for that subscriber;

-	when the subscriber registers in a new location area of the same VLR and new routing information is to be provided to the HLR (change of MSC area);

-	if the indicator "Confirmed by HLR" or the indicator "Location Information Confirmed in HLR" is set to not confirmed because of HLR or VLR restoration, and the VLR receives an indication that the subscriber is present;

If a mobile subscriber registers in a visitor location register (VLR) not holding any information about this subscriber and is identified by a temporary mobile subscriber identity (TMSI) allocated by a previous visitor location register (PVLR), if the PVLR identity can be derived from LAI the new VLR must obtain the IMSI from PVLR to identify the HLR to be updated (see figure 16.1.1/2). If the IMSI cannot be retrieved from PVLR, it is requested from the MS (see figure 16.1.1/3).

The following MAP services are invoked by the location update procedure:

MAP_UPDATE_LOCATION_AREA	(see section 6.1);

MAP_UPDATE_LOCATION		(see section 6.1);

MAP_CANCEL_LOCATION		(see section 6.1);

MAP_INSERT_SUBSCRIBER_DATA	(see section 6.8);

MAP_SEND_IDENTIFICATION		(see section 6.1);

MAP_PROVIDE_IMSI		(see section 6.9);

MAP_AUTHENTICATE		(see section 6.5);

MAP_SET_CIPHERING_MODE	(see section 6.6);

MAP_FORWARD_NEW_TMSI		(see section 6.9);

MAP_CHECK_IMEI		(see section 6.7);

MAP_ACTIVATE_TRACE_MODE	(see section 7.2);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(see section 7.2).

┌────┐       ┌────┐    A      ┌────┐       B         ┌────┐

│ MS │───────│ BS │────┼──────┤MSC │───────┼─────────│VLR │

└────┘       └────┘           └────┘                 └────┘

│                             │                       │

│     A_LU_REQUEST            │                       │

│---------------------------->│    MAP_UPDATE_        │

│                             │──────────────────────>│

│                             │    LOCATION_AREA      │

│         (NOTE 1)            │                       │

│                             │   MAP_AUTHENTICATE   │

│<----------------------------│<──────────────────────│

│                             │ MAP_AUTHENTICATE ack │

│---------------------------->│──────────────────────>│

│                             │       (NOTE 2)        │

│                             │                       │

│                             │MAP_SET_CIPHERING_MODE │

│<----------------------------│<──────────────────────│

│                             │                       │

│                             │ MAP_TRACE_SUBSCRIBER_ │

│                             │     ACTIVITY          │

│                             │<──────────────────────│

│                             │                       │

│                             │    MAP_CHECK_IMEI     │

│<----------------------------│<──────────────────────│

│                             │  MAP_CHECK_IMEI ack   │

│---------------------------->│──────────────────────>│

│                             │                       │

│                             │ MAP_FORWARD_NEW_TMSI │

│<----------------------------│<──────────────────────│

│                             │                       │

│                             │ MAP_UPDATE_LOCATION_ │

│     A_LU_CONFIRM            │<──────────────────────│

│<----------------------------│       AREA ack        ┤

│                             │                       │

│                             │MAP_FORW._NEW_TMSI ack │

│---------------------------->│──────────────────────>│

│                             │                       │

│                             │                       │

NOTE 1:	For details of the procedure on the radio path, see TS GSM 04.08. The services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	Optional services are printed in italics.

Figure 16.1.1/1: Interface and services for location updating when roaming within an visitor location registers area (without need to update HLR)

┌────┐ ┌────┐  A  ┌────┐   B    ┌────┐         D            ┌────┐

│ MS │-│ BS │--┼--│MSC │───┼────│VLR │─────────┼────────────│HLR │

└────┘ └────┘     └────┘        └──┬─┘                      └──┬─┘

│                           │

│     G     ┌────┐    D     │

└─────┼─────┤PVLR├────┼─────┘

└────┘

│ A_LU_REQUEST    │              │              │            │

│---------------->│ MAP_UPDATE_  │              │            │

│                 │─────────────>│              │            │

│                 │ LOCATION_AREA│MAP_SEND_IDENTIFICATION    │

│                 │              │─────────────>│            │

│                 │              │              │            │

│                 │              │MAP_SEND_IDENTIFICATION    │

│                 │              │<─────────────┤            │

│                 │              │  ack                      │

│                 │              │                           │

│                 │              │ MAP_UPDATE_LOCATION       │

│                 │              │──────────────────────>    ┤

│                 │              │                           │

│                 │              │             │MAP_CANCEL_  │

│                 │              │             │ LOCATION    │

│                 │              │             │<────────────┤

│                 │              │             │             │

│                 │              │             │MAP_CANCEL_  │

│                 │              │             │LOCATION ack │

│                 │              │             │────────────>│

│                 │              │             │             │

│                 │              │MAP_ACTIVATE_TRACE_MODE    │

│                MAP_TRACE_SUBSCR│<──────────────────────    │

│                 │  _ACTIVITY   │                           │

│                 │<─────────────│                           │

│                 │              │MAP_ACTIVATE_TRACE_MODE ack│

│                 │              │──────────────────────>    │

│                 │              │                           │

│                 │              │MAP_INSERT_SUBSCRIBER_DATA │

│                 │              │<──────────────────────    ┤

│                 │              │                           │

│                 │              │MAP_INSERT_SUBSCR._DATA ack│

│                 │              │──────────────────────>    ┤

│                 │              │                           │

│                 │              │MAP_UPDATE_LOCATION ack    │

│                 │              │<──────────────────────    ┤

│                 │  MAP_UPDATE_ │                           │

│                 │<─────────────│                           │

│ A_LU_CONFIRM    │LOCATION_AREA ack                         │

│<----------------│              │                           ┤

│                 │              │                           │

│                 │              │                           │

│                 │              │                           │

│                 │              │                           │

│                 │              │                           │

NOTE:	The optional procedures in figure 16.1.1/1 apply here respectively.

Figure 16.1.1/2: Interface and services for location updating when changing the VLR area

┌────┐    ┌────┐   A   ┌────┐      B      ┌────┐      D      ┌────┐

│ MS │----│ BS │---│---│MSC │──────┼──────┤VLR │──────┼──────┤HLR │

└────┘    └────┘       └────┘             └────┘             └────┘

│   G  ┌────┐  D    │

└───┼──┤PVLR├──┼────┘

└────┘

│      A_LU_REQUEST    │                  │                   │

│--------------------->│   MAP_UPDATE_    │                   │

│                      │─────────────────>│                   │

│                      │  LOCATION_AREA   │                   │

│                      │                  │                   │

│                      │ MAP_PROVIDE_IMSI │                   │

│<---------------------│<─────────────────│                   │

│                      │                  │                   │

│                      │ MAP_PROVIDE_IMSI │                   │

│--------------------->│─────────────────>│                   │

│                      │      ack         │MAP_UPDATE_LOCATION│

│                      │                  │──────────────────>│

│                      │                  │                   │

│                      │                  │      │ MAP_CANCEL_│

│                      │                  │      │ LOCATION   │

│                      │                  │      │<───────────┤

│                      │                  │      │            │

│                      │                  │      │ MAP CANCEL │

│                      │                  │      │LOCATION ack│

│                      │                  │      │───────────>│

│                      │                  │                   │

│                      │                  │MAP_ACTIVATE_TRACE_│

│                      │ MAP_TRACE_SUB -  │<──────────────────│

│                      │  SCRIBER_ACTIVITY│    MODE         │

│                      │<─────────────────│                   │

│                      │                  │MAP_ACTIVATE_TRACE_│

│                      │                  │──────────────────>│

│                      │                  │    MODE ack       │

│                      │                  │                   │

│                      │                  │ MAP_INSERT_       │

│                      │                  │<──────────────────│

│                      │                  │ SUBSCRIBER_DATA   │

│                      │                  │                   │

│                      │                  │ MAP_INSERT_       │

│                      │                  │──────────────────>│

│                      │                  │SUBSCRIBER_DATA ack│

│                      │                  │                   │

│                      │                  │MAP_UPDATE_LOCATION│

│                      │   MAP_UPDATE_    │<──────────────────┤

│      A_LU_CONFIRM    │<─────────────────┤      ack          │

│<---------------------│LOCATION_AREA ack │                   │

│                      │                  │                   │

NOTE:	The optional procedures in figure 16.1.1/1 apply here respectively.

Figure 16.1.1/3: Interface and services for location updating involving both a VLR and an HLR, when IMSI can not be retrieved from the previous VLR

16.1.1.2	Detailed procedure in the MSC

Figure 16.1.1/4 shows the MSC process for location register updating, containing macro calls for:

Receive_Open_Cnf		section 21.1;

Authenticate_MSC			section 21.5;

Check_IMEI_MSC			section 21.6;

Obtain_IMSI_MSC			section 21.8;

Trace_Subscriber_Activity_MSC	section 21.9.

For structuring purposes, the second part of the process is placed into the macro Update Location Completion MSC, which is specific to this process (see figure 16.1.1/5).

When the VLR receives a A_LU_REQUEST (normal location updating, periodic location updating or IMSI attach) for a subscriber via the radio path, the MSC opens a dialogue to the VLR (MAP_OPEN request without any user specific parameters) and sends a MAP_UPDATE_LOCATION_AREA request, containing the parameters provided in the A_LU_REQUEST by the MS or BSS (for the parameter mapping see TS GSM 09.10).

If the dialogue is rejected (see Receive_Open_Cnf macro in section 21.1), the MSC will:

-	terminate the procedure on the radio path in case of termination of the MAP procedure;

-	revert to the MAP version one procedure (starting by sending the Update Location Area Message) in case the VLR indicated fall-back to version one protocol; or

-	continue as below, if the dialogue is accepted.

If the dialogue is accepted, the VLR will process this updating request, invoking optionally the MAP_PROVIDE_IMSI, MAP_TRACE_SUBSCRIBER_ACTIVITY, MAP_CHECK_IMEI or the MAP_AUTHENTICATE services first (see section 16.1.1.3 for initiation conditions, section 21 for macros defining the handling of services in the MSC). For these macros there are two possible outcomes:

-	a positive outcome, in which case the process continues waiting for the MAP_UPDATE_LOCATION_AREA confirmation; or

-	an error is reported, in which case the process terminates (not applicable for Trace_Subscriber_Activity_MSC, which has only a positive outcome).

After receiving the MAP_UPDATE_LOCATION_AREA indication and handling these optional services, the VLR will decide whether a new TMSI need to be allocated to the subscriber or not.

Updating without TMSI reallocation

If the VLR does not reallocate the TMSI, the MSC will receive a MAP_UPDATE_LOCATION_AREA confirmation next (figure 16.1.1/4).

-	If there are no parameters with this primitive, updating was successful and a confirmation containing the parameter Roaming Allowed will be sent to the MS.

-	If there is an error cause contained in the received primitive, this cause will be mapped to the corresponding cause in the confirmation sent to the MS (see TS GSM 09.10 for the mapping of messages and causes).

Updating including TMSI reallocation

This case is covered by the macro Update Location Completion MSC given in figure 16.1.1/5. The MSC will upon receipt of a MAP_SET_CIPHERING_MODE request send a ciphering command towards BSS/MS. Thereafter, the MAP_FORWARD_NEW_TMSI indication and the MAP_UPDATE_LOCATION_AREA confirmation are received in arbitrary order, causing a confirmation on the radio path containing both new LAI and new TMSI. If the MAP_UPDATE_LOCATION_AREA confirmation contains any error, the updating request is rejected towards the MS.

-	The MS will confirm receipt of the new TMSI, resulting in an empty MAP_FORWARD_NEW_TMSI response terminating the dialogue.

-	If there is no confirmation received from the A Interface, the dialogue is terminated locally.

Before receiving a MAP_UPDATE_LOCATION_AREA confirmation, the MSC may receive a MAP_CHECK_IMEI indication. Handling of this indication, comprising IMEI request towards the MS and IMEI checking request towards the EIR, is given in the macro description in chapter 21.6. The result may either be to return to the state Wait for TMSI or to return to terminate.

Forwarding the Check SS Indication

When the VLR receives a MAP_FORWARD_CHECK_SS_INDICATION_Ind during the Update LOCATION Area process, this indication is relayed to the MS (see TS GSM 09.11 for detailed interworking) and the MSC remains in the current state.

Abort handling

If the VLR receives a MAP_U_ABORT, a MAP_P_ABORT or a premature MAP_CLOSE indication from the VLR during the location update process, the MSC terminates the process by sending an A_LU_CONFIRM containing the error cause Updating Failure to the MS. If the MSC had already confirmed the location update towards the MS, the process terminates without notification towards the A Interface.

If the MSC receives a MAP_NOTICE indication, it issues a MAP_U_ABORT and terminates also the A interface dialogue, and the process terminates.

When the procedure is terminated abnormally on the radio path, the dialogue towards the VLR is aborted with the appropriate diagnostic information, and the procedure terminates.

Figure 16.1.1/4: Process Update_Location_Area_MSC

.G.C:\USER\MAP\MAP-470\EPS16\161_14.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/5: Process Update_Location_Completion_MSC

.G.C:\USER\MAP\MAP-470\EPS16\161_15.EPS;18.2 cm;23 cm;EPS

16.1.1.3	Detailed procedure in the VLR

Figure 16.1.1/6 shows the process for location updating in the VLR. The following general macros are used:

Receive_Open_Ind				section 21.1;

Receive_Open_Cnf				section 21.1;

Authenticate_VLR					section 21.5;

Check_IMEI_VLR					section 21.6;

Insert_Subscriber_Data_VLR				section 21.7;

Obtain_IMSI_VLR to request the IMSI for the subscriber		section 21.8;

Activate_Tracing_VLR and Trace_Subscriber_Activity_VLR	section 21.9,

Subscriber_Present_VLR				section 21.10.

Additionally, the process specific macro

Location_Update_Completion_VLR, for optional initiation of Ciphering and TMSI reallocation as for acknowledgement of the MAP_UPDATE_LOCATION_AREA service, see figure 16.1.1/7,

and the optional process specific macro

VLR UPDATE HLR to update the HLR and download subscriber data from there, see figure 16.1.1/8,

are invoked by this process.

Process Initiation

The location area updating process will be activated by receiving a MAP_UPDATE_LOCATION_AREA indication from the MSC. If there are parameter errors in the indication, the process is terminated with the appropriate error sent in the MAP_UPDATE_LOCATION_AREA response to the MSC. Else, The behaviour will depend on the subscriber identity received, either an IMSI or an TMSI.

Updating using IMSI

If the subscriber identity is an IMSI, the VLR checks whether the subscriber is unknown (i.e. no IMSI record). If so, an indication that the location information in HLR is not confirmed is set to initiate HLR updating later on. The process will continue in both cases with the authentication check (see below).

Updating using TMSI

If the subscriber identity is a TMSI, the VLR checks whether the previous location area identification (LAI) provided in the primitive received from MSC belongs to an area of this VLR.

-	If so, the TMSI will be checked. In case of location area change within a VLR, the TMSI should be known and the process will continue with the authentication check. Additionally, an indication that the location information in HLR is not confirmed is set and the trace activity status is checked in case the target Location Area Id belongs to a new MSC.

-	If the TMSI is not known or the subscriber data stored are incomplete, e.g. because the new LA belongs to a different VLR or due to VLR restoration, an indication that the data are not confirmed by HLR is set to initiate HLR updating later on.

If the subscriber has not already been registered in the VLR, i.e. the previous LAI belongs to a different VLR, the indicators Confirmed by HLR and Location Information Confirmed in HLR are set to false and the VLR checks whether the identity of the Previous VLR (PVLR) is derivable from the previous LAI.

-	If so, the IMSI and authentication parameters are requested from that VLR using the MAP_SEND_IDENTIFICATION service (see sheet 3 of figure 16.1.1/6), containing the subscriber's TMSI.

If the dialogue is rejected by the PVLR, the process continues requesting the IMSI from the MS. In case the PVLR reverts to the MAP version one dialogue, the VLR will perform the respective procedure of version one, too, with outcomes as for the version two dialogue. Else, the process waits the for the respective MAP_SEND_IDENTIFICATION response from the PVLR:

-	If the IMSI is received in that primitive, the process continues with the authentication check.

-	If the IMSI is not received from the previous VLR for any reason, the dialogue to the PVLR is terminated and the IMSI will be requested from the MS.

-	If a MAP_NOTICE indication is received from the PVLR, the dialogue will be aborted by sending a MAP_U_ABORT indication (Remote Operations Failure), and the process continues requesting the IMSI from the MS.

-	If a MAP_P_ABORT or MAP_U_ABORT indication is received from the MSC while waiting for the MAP_SEND_IDENTIFICATION response, the process is terminated.

-	If a MAP_NOTICE indication is received from the MSC while waiting for the MAP_SEND_IDENTIFICATION response, both the dialogue to the MSC as to the PVLR will be aborted by sending a MAP_U_ABORT indication (Remote Operations Failure), and the process terminates.

-	If the identity of the previous VLR cannot be derived, the process continues by requesting the IMSI from the MS.

Requesting IMSI from the MS

For requesting the IMSI from the MS, the macro Obtain_IMSI_VLR described in section 21.8 is invoked (see figure 16.1.1/6 sheet 3). The outcome will be:

-	OK, i.e. receipt of IMSI, in which case the process continues with the authentication check described below; or

-	receipt of an Absent Subscriber error, indicating that the MS did not respond. In this case the System Failure error is reported in the MAP_UPDATE_LOCATION_AREA response towards the MSC and the updating process is terminated;

-	aborted, i.e. the MSC dialogue has been released while waiting for the IMSI. In this case the updating process is terminated, too.

Authentication check

After a subscriber identity has been received, either in the service indication or by an explicit request procedure, the VLR checks whether authentication of this identity is required (see figure 16.1.1/6 sheet 2). If so, the authentication macro described in section 21.5 is invoked. The outcome of this macro can be:

-	OK, i.e. the subscriber has been authenticated successfully, in which case the process is continued by setting the radio contact flag to confirmed and updating the location information held in the register. Thereafter,

-	if one of the HLR confirmation indicators is set to false, HLR updating is invoked first,

-	else the process continues with the Location Update Completion VLR macro described below, and the register is updated after successful completion of this macro.

-	Illegal subscriber, i.e. there was a mismatch between expected and received SRES. The VLR checks whether authentication had been performed using the TMSI, in which case a new authentication attempt with IMSI may be started (VLR operator option).

-	If so, the process continues by requesting the IMSI from the MS.

-	Else, the Illegal Subscriber error is reported in the MAP_UPDATE_LOCATION_AREA response.

-	Unknown Subscriber, i.e. the IMSI given is unknown in the HLR. In this case, the subscriber data are deleted in the VLR and the same error is returned in the MAP_UPDATE_LOCATION_AREA response.

-	Procedure Error, i.e. the authentication process was unsuccessful for some other reason, e.g. because of a failure while requesting authentication information from the HLR. In this case the System Failure error is reported in the MAP_UPDATE_LOCATION_AREA response.

-	Null, indicating impossible dialogue continuation (e.g. termination of the radio path), and leading to procedure termination without any further action.

Updating the HLR

If the HLR is to be updated, the VLR_Update_HLR macro described below is performed, with one of the following results (see sheet 4 of figure 16.1.1/6):

-	OK, if HLR updating has been completed successfully. The response will contain the HLR number as parameter. Next, the Location_Update_Completion VLR macro is invoked (checking amongst others the roaming restrictions and regional subscription data), and upon successful outcome of this macro the register is updated and the process terminates.

-	Roaming Not Allowed, qualified by PLMN Roaming Not Allowed if the location information indicates a PLMN for which the subscriber has no subscription or if the subscribers HLR cannot be reached (e.g. SS7 links to the subscribers HPLMN do not yet exist). In this case, the error Roaming Not Allowed qualified by PLMN Roaming Not Allowed is sent in the MAP_UPDATE_LOCATION_AREA response. The Subscriber Data are deleted in the VLR.

-	If Roaming Not Allowed was qualified by the parameter Operator Determined Barring, the same value is sent in the MAP_UPDATE_LOCATION_AREA response to the MSC. The subscriber data are deleted in the VLR.

-	Unknown Subscriber, if the subscriber is not known in the HLR. In this case, the subscriber data are deleted in the VLR, and the same error is sent in the MAP_UPDATE_LOCATION_AREA response.

-	Procedure error, if there occurs some other error during HLR updating (e.g. abort of the connection to HLR).

-	If the VLR can proceed in stand alone mode (VLR operator option), the Location Update Completion VLR macro is invoked to complete the VLR updating, and the indicator Not confirmed by HLR remains unchanged.

-	Else, the System Failure error is sent in the MAP_UPDATE_LOCATION_AREA response.

-	Aborted, indicating that during HLR updating the MSC dialogue has been terminated. In this case, the updating process terminates without any further action.

The macro Location Update Completion VLR

This macro completes the VLR updating process. First, the VLR checks whether there is a roaming restriction for the subscriber (see figure 16.1.1/7):

-	If the target LA is not allowed for the subscriber due to national roaming restrictions, the error Roaming Not Allowed with cause National Roaming Not Allowed is returned in the MAP_UPDATE_LOCATION_AREA response towards the MSC.

The subscriber data are not deleted from VLR, to avoid unnecessary HLR updating when roaming into other LAs of the same MSC. An indication that the subscriber is not allowed to roam is set in the VLR (LA Not Allowed Flag set to not allowed). As a consequence the subscriber is not reachable (checked for MTC, SMS and MT USSD) and cannot perform outgoing actions (checked in Access Management).

-	If the target LA is not allowed for the subscriber because of regional subscription data (Zone Code List) or Roaming Restriction Due To Unsupported Feature stored in the VLR, the error Roaming Not Allowed with cause Location Area Not Allowed is returned towards the MSC in the MAP_UPDATE_LOCATION_AREA response.

Also in this case the subscriber data are not deleted from VLR, to avoid unnecessary HLR updating when roaming into other LAs of the same MSC. The LA Not Allowed Flag is set to not allowed in the VLR.

-	If, after check of possible roaming restrictions, the subscriber is allowed to roam in the target LA, the LA Not Allowed Flag is set to allowed (if necessary), the IMSI Detached Flag is set to attached and the process SUBSCRIBER_PRESENT_VLR is started; this may inform the HLR that the subscriber is present again to retry an SMS delivery (see section 16.1.1.7). Thereafter, the VLR checks whether TMSI reallocation is required.

-	If so, the VLR sends a MAP_SET_CIPHERING_MODE request containing:

-	Ciphering Mode (version 1 GSM); and

-	Kc, the cipher key to be used.

-	If IMEI checking is required by the operator, the VLR will invoke the CHECK_IMEI_VLR macro (see section 21.6) to initiate both requesting IMEI from the MS and checking of this IMEI towards the EIR. As result either the service is granted, with process continuation as given below, or the service is rejected, in which case the VLR marks the subscriber as detached and returns an Illegal Equipment error in the MAP_UPDATE_LOCATION_AREA response before the process terminates.

-	The VLR then sends a MAP_FORWARD_NEW_TMSI request containing the new TMSI, and the MAP_UPDATE_LOCATION_AREA response containing no parameters. The process will thereafter wait for the MAP_FORWARD_NEW_TMSI confirm. If this indicates a negative outcome, or if a MAP_P_ABORT or a MAP_U_ABORT primitive is received, the old TMSI is frozen. Subsequent accesses of the MS shall be accepted with both old or new TMSI.

-	If TMSI reallocation is not required, the VLR invokes the CHECK_IMEI_VLR macro (see section 21.6) to initiate both requesting IMEI from the MS and checking of this IMEI towards the EIR, if IMEI Checking is required by the operator. As a result, either the service is granted, in which case the MAP_UPDATE_LOCATION_AREA response is sent without any parameters, or the service is rejected, in which case an Illegal Equipment error is returned in the MAP_UPDATE_LOCATION_AREA response, before the process terminates.

In all cases where the VLR sends a MAP_UPDATE_LOCATION_AREA response to the MSC, the dialogue towards the MSC is terminated by a MAP_CLOSE request with parameter Release Method indicating Normal Release.

The macro VLR Update HLR

This macro is invoked by the VLR process for location updating or by some other process handling the first subscriber access to the network after a register failure in order to perform HLR updating. If the VLR does not know the subscribers HLR (e.g. no IMSI translation exists as there are not yet any SS7 links to the subscribers HPLMN), the error Roaming Not Allowed with cause PLMN Roaming Not Allowed is returned.

If the subscribers HLR can be reached, the VLR opens a dialogue towards the HLR (see figure 16.1.1/8) by sending a MAP_OPEN request without any user specific parameters, together with a MAP_UPDATE_LOCATION request containing the parameters

-	IMSI, identifying the subscriber;

-	Location Info, containing the MSC number;

-	VLR Number, the E.164 address of the VLR, to be used by the HLR when addressing the VLR henceforth (e.g. when requesting an MSRN);

-	the LMSI as an VLR operator option; this is a subscriber identification local to the VLR, used for fast data base access.

In case the HLR rejects dialogue opening (see section 21.1), the VLR will terminate the procedure indicating Procedure Error. If the HLR indicates version one protocol to be used, the VLR will revert to the version one procedure concerning the dialogue with the HLR, with outcomes as for the version two procedure.

If the HLR accepts the dialogue, the HLR will respond with:

-	a MAP_INSERT_SUBSCRIBER_DATA indication, handled by the macro Insert_Subs_Data_VLR defined in section 21.7;

NOTE:	The HLR may repeat this service several times depending on the amount of data to be transferred to the VLR and to replace subscription data in case they are not supported by the VLR.

-	a MAP_ACTIVATE_TRACE_MODE indication, handled by the macro Activate_Tracing_VLR defined in section 21.9;

-	a MAP_FORWARD_CHECK_SS_INDICATION_ind. This indication will be relayed to the MSC without any change of the current state.

-	the MAP_UPDATE_LOCATION confirmation.

-	If this confirmation contains the HLR Number, this indicates that the HLR has passed all information and that updating has been successfully completed. The VLR is updated using the parameters provided in the service and needed by the VLR. If certain parameters are not needed in the VLR, e.g. because some service is not supported, the corresponding data may be discarded. The VLR sets the "Confirmed by HLR" and "Location information confirmed in HLR" indicators to true to indicate successful subscriber data updating.

-	If the confirmation contains an User error cause (Unknown Subscriber, Roaming Not Allowed or some other), the process calling the macro continues accordingly. In the last case, the subscriber data are marked as incomplete by setting the indicators "Confirmed by HLR" and "Location information confirmed in HLR" to false. The same holds if there is a Provider error or a Data error in the confirmation.

-	a MAP_P_ABORT, MAP_U_ABORT, or MAP_CLOSE indication. In these cases, the subscriber data are marked to be incomplete and the process continues as in the case of an error reported by the HLR.

-	a MAP_NOTICE indication. Then, the dialogue towards the HLR is aborted, the subscriber data are marked to be incomplete and the process continues as in the case of an error reported by the HLR.

-	If during HLR updating the VLR receives a MAP_P_ABORT, MAP_U_ABORT or a MAP_CLOSE indication concerning the MSC dialogue, the process is terminated by sending a MAP_U_ABORT request towards the HLR, and subscriber data are marked to be incomplete.

-	If during HLR updating the VLR receives a MAP_NOTICE indication concerning the MSC dialogue, the process is terminated by sending a MAP_U_ABORT request towards both MSC and HLR, and subscriber data are marked to be incomplete.

Abort Handling

If the VLR receives a MAP_NOTICE indication from the MSC while waiting for a MAP service primitive, the VLR will abort the MSC dialogue and any pending HLR dialogue by sending MAP_U_ABORT indications (Remote Operations Failure), and the process is terminated.

Figure 16.1.1/6 sheet 1: Process Update_Location_Area_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_161.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/6 sheet 2: Process Update_Location_Area_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_162.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/6 sheet 3: Process Update_Location_Area_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_163.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/6 sheet 4: Process Update_Location_Area_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_164.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/7: Macro Location_Update_Completion_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_17.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/8 sheet 1: Macro VLR_Update_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_181.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/8 sheet 2: Macro VLR_Update_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_182.EPS;18.2 cm;23 cm;EPS

16.1.1.4	Detailed procedure in the HLR

The following macros are used by this process:

-	Receive_Open_Ind, defined in section 21.1;

-	Check_indication, defined in section 21.2;

-	Insert_Subs_Data_Framed_HLR, described in section 16.4.1;

-	Control_Tracing_HLR, described in section 21.9;

and the processes Cancel_Location_HLR (see section 16.1.2) and Subscriber_Present_HLR (see section 16.1.1.7) are invoked.

The location updating process in the HLR is activated by receipt of a MAP_UPDATE_LOCATION indication (see figure 16.1.1/9).

-	If there is a parameter problem in the indication, the error Unexpected Data Value is returned in the MAP_UPDATE_LOCATION response (see Check_indication macro defined in section 21.2); if the subscriber is not known in the HLR, the error Unknown Subscriber is returned in the response. In either case the process terminates.

-	If the VLR address received in the MAP_UPDATE_LOCATION indication differs from the one actually stored against the subscriber, the Cancel_Location_HLR process is started to cancel the subscriber data in the stored VLR (see section 16.1.2).

The next action will be to check whether the subscriber is allowed to roam into the PLMN indicated by the VLR Number given in the MAP_UPDATE_LOCATION indication.

-	If the subscriber is not allowed to roam into the PLMN, the error Roaming not Allowed with cause PLMN Roaming Not Allowed is returned in the MAP_UPDATE_LOCATION response, and the routing information stored (VLR number, MSC Number, LMSI) is deleted (deregistration).

-	Otherwise the HLR database will be updated with information received in the indication. The HLR sets the "MS purged" flag to False and checks whether tracing is required for that subscriber. This is handled by the macro Control_Tracing_HLR described in section 21.9.

Thereafter, the macro Insert_Subs_Data_Framed_HLR described in section 16.4.1 is invoked. The outcome of this macro may be:

-	aborted, in which case the process terminates;

-	error, in which case the error System Failure is returned in the MAP_UPDATE_LOCATION response and the process terminates;

-	OK, indicating successful outcome of downloading the subscriber data to the VLR.

If certain services required for a subscriber are not supported by the VLR (e.g. Advice of Charge Charging Level), this may result in one of the following outcomes:

-	The HLR stores and sends Roaming Restriction Due To Unsupported Feature, or other induced subscriber data (e.g. a specific barring program), in a subsequent MAP_INSERT_SUSBCRIBER_DATA service. If Roaming Restriction Due To Unsupported Feature is stored in the HLR the MSC Area Restricted Flag shall be set to restricted. This will prevent MT calls, MT SM and MT USSD from being forwarded to the MSC/VLR.

-	When the VLR receives regional subscription data (Zone Code List) it may respond with MSC Area Restricted in the MAP_INSERT_SUSBCRIBER_DATA response. In this case the MSC Area Restricted Flag shall be set to restricted by the HLR. This will prevent MT calls, MT SM and MT USSD from being forwarded to the MSC/VLR.

If neither of the above cases applies the HLR shall set the MSC Area Restricted Flag to not restricted.

The SUBSCRIBER_PRESENT_HLR process is then started to alert the Short Message Service Centre, if required (see section 16.1.7). Additionally, the MAP_FORWARD_CHECK_SS_INDICATION request is sent to inform the subscriber about an uncertain state of his SS data if this is needed due to previous HLR restoration (use of this service may be omitted as an HLR operator option).

Finally the HLR number is returned in the MAP_UPDATE_LOCATION response.

In all cases where the HLR sends a MAP_UPDATE_LOCATION response to the VLR, the dialogue towards the VLR is terminated by a MAP_CLOSE request with parameter Release Method indicating Normal Release.

Figure 16.1.1/9 sheet 1: Process Update_Location_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_191.EPS;18.2 cm;23 cm;EPS

Figure 16.1.1/9 sheet 2: Process Update_Location_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_192.EPS;18.2 cm;23 cm;EPS

16.1.1.5	Send Identification

16.1.1.5.1	General

This service is invoked by a VLR when it receives a MAP_UPDATE_LOCATION_AREA indication containing a LAI indicating that the subscriber was registered in a different VLR (henceforth called the Previous VLR, PVLR). If the identity of the PVLR is derivable for the VLR (usually if both are within the same network), the IMSI and authentication sets are requested from the PVLR (see section 16.1.1.3), using the service described in section 6.1.4.

┌────┐           B            ┌────┐       G         ┌────┐

│MSC │-----------│------------│VLR │───────┼─────────│PVLR│

└────┘                        └────┘                 └────┘

│                             │                       │

│      MAP_UPDATE_LOCATION_   │                       │

│---------------------------->│                       │

│          AREA               │   MAP_SEND_           │

│                             │──────────────────────>│

│                             │   IDENTIFICATION      │

│                             │                       │

│                             │                       │

│                             │   MAP_SEND_           │

│                             │<──────────────────────│

│                             │   IDENTIFICATION ack  │

│                             │                       │

NOTE:	The service shown in dotted lines indicates the trigger provided by other MAP signalling.

Figure 16.1.1/10: Interface and services for Send Identification

16.1.1.5.2	Detailed procedure in the VLR

The VLR procedure is part of the location area updating process described in section 16.1.1.3, see also figure 16.1.1/6 sheet 3.

16.1.1.5.3	Detailed procedure in the PVLR

On receipt of a dialogue request for the Send Identification procedure, (see Receive_Open_Ind macro in section 21.1), the PVLR will:

-	terminate the procedure in case of parameter problems;

-	revert to the MAP version one procedure (Send Parameters with request for IMSI plus authentication sets) in case the VLR indicated version one protocol; or

-	continue as below, if the dialogue is accepted.

The PVLR process receives a MAP_SEND_IDENTIFICATION indication from the VLR (see figure 16.1.1/11), and checks whether the subscriber identity provided is known.

-	If so, the IMSI and - if available - authentication parameters for the subscriber are returned in the MAP_SEND_IDENTIFICATION response.

-	If not, the error Unidentified Subscriber is returned in the MAP_SEND_IDENTIFICATION response.

In all cases where the PVLR sends a MAP_SEND_IDENTIFICATION response to the VLR, the dialogue towards the VLR is terminated by a MAP_CLOSE request with parameter Release Method indicating Normal Release.

Figure 16.1.1/11: Process Send_Identification_PVLR

.G.C:\USER\MAP\MAP-470\EPS16\161_1B.EPS;18.2 cm;23 cm;EPS

16.1.1.6	The Process Update Location VLR

This process is started by some other MAP user process in case the HLR need to be updated due to previous network failure. It is invoked when the subscriber accesses the network, e.g. for mobile originated call set-up, response to paging or supplementary services handling. Here, location updating consists only of invoking the macro VLR_UPDATE_HLR described above (see section 16.1.1.3), which performs HLR updating and downloading of subscriber data.

If updating is successful (OK) the HLR Number is received in the MAP_UPDATE_LOCATION confirm primitive and the process terminates.

If one of the errors Roaming not Allowed or Unknown Subscriber is received instead, all subscriber data are deleted from the VLR before the process terminates.

In case some other error occurs during HLR updating, the process simply terminates. Note, in all error cases the initiating restoration flags in VLR remain false, therefore a new HLR updating attempt will be started later on.

NOTE:	This process will be performed independent from the calling process, no coordination is required.

Figure 16.1.1/12: Process UL_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_1C.EPS;18.2 cm;23 cm;EPS

16.1.1.7	The Process Subscriber Present HLR

The process Subscriber Present HLR is started by the location updating process in HLR to perform actions required for short message alerting. The process checks the Message Waiting Data flag, and if this is set, the macro Alert_Service _Centre_HLR defined in section 21.10 is invoked. This macro will alert all service centres from which there are short messages waiting for this subscriber.

Figure 16.1.1/13: Process Subscriber_Present_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_1D.EPS;18.2 cm;23 cm;EPS

16.1.2	Location Cancellation

16.1.2.1	General

The purpose of this process is to delete a subscriber's record from a previous visitor location register after she has registered with a new visitor location register. The procedure may also be used if the subscriber's record is to be deleted for other operator determined purposes, e.g. withdrawal of subscription, imposition of roaming restrictions or modifications to the subscription which result in roaming restrictions. Location cancellation can be used to enforce location updating including updating of subscriber data in the VLR at the next subscriber access.

In all cases, the process is performed independently of the invoking process (e.g. Location Updating).

The service as described in section 6.1.3 is invoked when an HLR receives a MAP_UPDATE_LOCATION indication from a VLR other than that stored in its table for this subscriber. Additionally the service may be invoked by operator intervention. The MAP_CANCEL_LOCATION service is in any case invoked towards the VLR whose identity is contained in the HLR table.

┌────┐           D            ┌────┐       D         ┌────┐

│VLR ├───────────┼────────────┤HLR │───────┼─────────│PVLR│

└────┘                        └────┘                 └────┘

│                             │                       │

│      MAP_UPDATE_LOCATION    │                       │

│---------------------------->│                       │

│                             │     MAP_CANCEL_       │

│                             │ ─────────────────────>│

│                             │    LOCATION           │

│                             │                       │

│                             │                       │

│                             │  MAP_CANCEL_LOCATION  │

│                             │<───────────────────── │

│                             │          ack          ┤

│                             │                       │

NOTE:	The service shown in dotted lines indicates the trigger provided by other MAP signalling.

Figure 16.1.2/1: Interface and services for Location Cancellation

16.1.2.2	Detailed procedure in the HLR

The location cancellation process is started by an external process as stated above. The HLR opens a dialogue with the VLR whose identity is contained in the HLR table (MAP_OPEN request without any user specific parameters), sending the MAP_CANCEL_LOCATION request primitive (see figure 16.1.2/2), containing the parameters:

-	IMSI, to identify the subscriber to be deleted from that VLR;

-	LMSI, which is included if available in the HLR.

The HLR then waits for the MAP_OPEN confirmation (see macro Receive_Open_Cnf, section 21.1), indicating either:

-	reject of the dialogue (process terminates);

-	reversion to version one (process will be performed according MAP version one); or

-	dialogue acceptance.

When the VLR accepts the dialogue, it will return a MAP_CANCEL_LOCATION confirmation, containing

-	no parameter, indicating successful outcome of the procedure;

-	a user error, provider error or a data error indicating unsuccessful outcome of the procedure.

In case of unsuccessful outcome or if a MAP_P_ABORT indication has been received, the HLR may repeat the MAP_CANCEL_LOCATION request later, where the number of repeat attempts and time in between are HLR operator options.

16.1.2.3	Detailed procedure in the VLR

Opening of the dialogue is described in the macro Receive_Open_Ind in section 21.1, with outcomes:

-	reversion to version one procedure;

-	procedure termination; or

-	dialogue acceptance, with proceeding as below.

When the VLR process receives a MAP_CANCEL_LOCATION indication from the HLR (see figure 16.1.2/3), the parameters are checked first (macro Check_Indication, see section 21.2). In case of parameter problems a respective error is sent in the MAP_CANCEL_LOCATION response.

Thereafter the VLR checks whether the subscriber identity provided is known in the VLR.

-	If so, the data of the subscriber are deleted from VLR table and a MAP_CANCEL_LOCATION response is returned without any parameters.

-	If not, location cancellation is regarded as being successful, too, and the MAP_CANCEL_LOCATION response is returned without any parameters.

In either case, after sending the MAP_CANCEL_LOCATION response the VLR process releases any TMSI which may be associated with the IMSI of the subscriber, terminates the dialogue (MAP_CLOSE with Release Method Normal Release) and returns to the idle state.

Figure 16.1.2/2: Process Cancel_Location_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_22.EPS;18.2 cm;23 cm;EPS

Figure 16.1.2/3: Process Cancel_Location_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_23.EPS;18.2 cm;23 cm;EPS

16.1.3	Detach IMSI

16.1.3.1	General

On receipt of an A_LU_REQUEST (DETACH IMSI) indication from the radio interface this procedure invokes the MAP_DETACH_IMSI service described in section 6.1.5 in order to inform the visitor location register that a subscriber is no longer reachable (see figure 16.1.3/1), e.g. due to switched off station. This information is used by the VLR to reject mobile terminating calls or short messages without sending page messages on the radio path. The service is unconfirmed as it is likely that the MS is switched off before receiving a confirmation.

The detach IMSI feature is optional for the network operator. The MS is informed by the network whether detach IMSI is to be used or not.

┌────┐       ┌────┐    A      ┌────┐       B         ┌────┐

│ MS │───────│ BS │────┼──────│MSC │───────┼─────────│VLR │

└────┘       └────┘           └────┘                 └────┘

│                             │                       │

│        A_LU_Request         │                       │

│---------------------------->│                       │

│        (DETACH IMSI)        │     MAP_DETACH_IMSI   │

│                             │──────────────────────>│

│                             │                       │

NOTE:	The service shown in dotted lines indicates the trigger provided by the radio interface (see TS GSM 09.10).

Figure 16.1.3/1: Interface and services for MAP_DETACH_IMSI

16.1.3.2	Detailed procedure in the MSC

The MAP_DETACH_IMSI service is invoked by the MSC when receiving an A_LU_Request (DETACH IMSI) for a subscriber (see figure 16.1.3/2).

The MSC will open the dialogue to the VLR with a MAP_OPEN request containing no user specific parameters. The MAP_DETACH_IMSI request will contain the following parameter received from the radio side (for the mapping see TS GSM 09.10):

-	Subscriber Id, being either a TMSI or an IMSI.

The MSC then waits for the MAP_OPEN confirmation (see macro Receive_Open_Cnf, section 21.1), indicating either:

-	reject of dialogue (process terminates);

-	reversion to version one (process will be repeated according MAP version one); or

-	dialogue acceptance.

Thereafter, the dialogue is terminated locally by the MSC (MAP_CLOSE request with Release Method Prearranged End).

16.1.3.3	Detailed procedure in the VLR

When the VLR receives a MAP_DETACH_IMSI indication (see figure 16.1.3/3), it first checks the indication data (macro Check_Indication, see section 21.2). Thereafter it is checked whether the subscriber is known:

-	if the subscriber is unknown the VLR ignores the indication;

-	if the subscriber is known in the VLR, the IMSI detached flag is set.

The VLR process will terminate the dialogue locally (MAP_CLOSE request with Release Method Prearranged End).

Figure 16.1.3/2: Process Detach_IMSI_MSC

.G.C:\USER\MAP\MAP-470\EPS16\161_32.EPS;18.2 cm;23 cm;EPS

Figure 16.1.3/3: Process Detach_IMSI_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_33.EPS;18.2 cm;23 cm;EPS

16.1.4	Purge MS

16.1.4.1	General

When the VLR receives an indication on the O&M interface that the MS record is to be purged (either because of administrative action or because te MS has been inactive for an extended period), this procedure invodes the MAP_PURGE_MS service described in section 6.1.6 to request the HLR to set the "MS purged" flag for the MS so that any request for routing information for a mobile terminated call or a mobile terminated short message will be treated as if the MS is not reachable. The message flow is shown in figure 16.1.4/1.

It is optional for the network operator to delete MS records from the VLR, but if the option is used the VLR shall notify the HLR when a record has been deleted.

The O&M process in the VLR must ensure that during the MS purging procedure any other attempt to access the MS record is blocked, to maintain consistency of data.

┌─────┐                    ┌─────┐

│ VLR ├────────────────────┤ HLR │

└──┬──┘                    └──┬──┘

│                          │

│     MAP_PURGE_MS         │

│─────────────────────────>│

│                          │

│     MAP_PURGE_MS_ack     │

│<─────────────────────────│

│                          │

│                          │

Figure 16.1.4/1: Interface and services for MAP_PURGE_MS

16.1.4.2	Detailed procedure in the VLR

When the VLR receives an indication from O&M that an MS record is to be purged, it invokes the MAP_PURGE_MS service (see figure 16.1.4/2).

The VLR opens the dialogue to the HLR with a MAP_OPEN request containing no user specific parameters. The MAP_PURGE_MS request contains the IMSI of the MS which is to be purged and the VLR number.

The VLR then waits for the MAP_OPEN confirmation (see macro Receive_Open_Cnf, section 21.1), indicating one of:

-	rejection of the dialogue (process terminates);

-	reversion to version one (process terminates);

-	dialogure acceptance.

If the HRL accepts the dialogue it returns a MAP_PURGE_MS confirmation, containing no parameter, indication successful outcome of the procedure.

If a MAP_PURGE_MS confirmation containing a provider error, data error or user error, or a MAP_P_ABORT, MAP_NOTICE or premature MAP_CLOSE indication, has been received, the failure is reported to the O&M interface. Successful outcome of the procedure leads to deletion of the subscriber data and freezing of the TMSI, and is reported to the O&M interface.

16.1.4.3	Detailed procedure in the HLR

Opening of the dialogue is described in the macro Receive_Open_Ind in section 21.1. The possible outcomes are:

-	termination of the procedure if the AC indicates a version 1 dialogure, as this procedure is defined only for version 2;

-	termination of the procedure if there is an error;

-	dialogue acceptance, in which case the procedure is as described below.

When the HLR receives a MAP_PURGE_MS indication (see figure 16.1.4/3), it first checks the indication data (macro Check_Indication, see section 21.2). If there is a parameter error the HLR terminates the dialogue by sending a MAP_CLOSE request (local termination). If there is no parameter error the HLR then checks whether the subscriber is known.

-	If the subscriber is unknown, the HLR reports an error to the O&M interface, and terminates the dialogue by sending a MAP_CLOSE request (local termination).

-	If the subscriber is known, the HLR checks whether the purging notification came from the VLR where the MS was last registered.

-	If the received VLR number and the stored VLR number match, the HLR sets the "MS purged" flag for the subscriber and sends a MAP_PURGE_MS response containing an empty result to indicate successful outcome.

-	If the received VLR number and the stored VLR number do not match, the HLR sends a MAP_PURGE_MS response containing an empty result to indicate successful outcome. Since the MS is known by the HLR to be in a different VLR area, it is not appropriate to block mobile terminated calls or short messages to the MS, but the VLR which initiated the purging procedure can safely purge its record for the MS.

In either cases of successful termination the HLR terminates the dialogue by sending a MAP_CLOSE request.

Figure 16.1.4/2: Process Purge_MS_VLR

.G.C:\USER\MAP\MAP-470\EPS16\161_42.EPS;18.2 cm;23 cm;EPS

Figure 16.1.4/3: Process Purge_MS_HLR

.G.C:\USER\MAP\MAP-470\EPS16\161_43.EPS;18.2 cm;23 cm;EPS

16.2	Handover procedure

16.2.1	General

The handover between different MSCs is called Inter-MSC handover. The interfaces involved for Inter-MSC handover are shown in figure 16.2/1. Following two Inter-MSC handover procedures apply:

1)	Basic Inter-MSC handover:

The call is handed over from the controlling MSC, called MSC-A to another MSC, called MSC-B (figure 16.2/1a).

Figure 16.2/2 shows a successful handover between MSC-A and MSC-B including a request for handover number allocation by MSC-B to VLR-B.

2)	Subsequent Inter-MSC handover:

After the call has been handed over from MSC-A to MSC-B, a handover to either MSC-A (figure 16.2/1a) or to a third MSC (MSC-B') (figure 16.2/1b) is necessary in order to continue the connection.

Figure 16.2/3 shows a successful subsequent handover.

┌──────┐   E   ┌──────┐   B   ┌──────┐

│MSC-A ├───┼───┤MSC-B ├───┼───┤VLR-B │

└──────┘       └──────┘       └──────┘

a) Basic handover procedure MSC-A to MSC-B and subsequent handover procedure MSC-B to MSC-A.

┌──────┐   E   ┌──────┐   B   ┌──────┐

│ MSC-A├───┼───┤MSC-B ├───┼───┤VLR-B │

└──┬───┘       └──────┘       └──────┘

│       E   ┌──────┐   B   ┌──────┐

└───────┼───┤MSC-B'├───┼───┤VLR-B'│

└──────┘       └──────┘

b) Subsequent handover procedure MSC-B to MSC-B'.

Figure 16.2/1: Interface structure for handover

The MAP handover procedures achieve the functionality required to set up an MSC-MSC dialogue, to optionally allocate a handover number and to transport BSSAP messages.

The transported BSSAP messages are controlled and handled by the Handover Control Application in the MSCs. This information will be transparent to the MAP protocol. If the MSC receives via the MAP protocol BSSAP messages, this information will be forwarded to the Handover Control Application (shown in the handover SDL diagrams with the internal HO_CA signalling, it is an internal process in the MSC) and vice versa if the Handover Control Application requires the sending of BSSAP messages via the MAP protocol.

For detailed interworking between the A-interface and MAP procedures, see TS GSM 03.09 and TS GSM 09.10.

┌──────┐       E             ┌──────┐         B        ┌──────┐

│MSC-A ├───────┼─────────────┤MSC-B ├─────────┼────────┤VLR-B │

└──────┘                     └──────┘                  └──────┘

│                            │                        │

│  MAP_PREPARE_HANDOVER      │                        │

├─────────────────────────>  │                        │

│                            │  MAP_ALLOCATE_HANDOVER_│

│                            │  NUMBER                │

│                            ├─────────────────────>  │

│                            │                        │

│                            │  MAP_SEND_HANDOVER_    │

│                            │  REPORT                │

│                            │  <─────────────────────┤

│                            │                        │

│  MAP_PREPARE_HANDOVER ack  │                        │

│  <─────────────────────────┤                        │

─┴─                          ─┴─                      ─┴─

─┬─                          ─┬─ MAP_SEND_HANDOVER_   ─┬─

│                            ├─────────────────────>  │

─┴─                          ─┴─ REPORT ack  (NOTE)   ─┴─

─┬─                          ─┬─                      ─┬─

│ MAP_PROCESS_ACCESS_       │                        │

│  SIGNALLING                │                        │

│  <─────────────────────────┤                        │

│ MAP_SEND_END_SIGNAL       │                        │

│  <─────────────────────────┤                        │

─┴─                          ─┴─                      ─┴─

─┬─                          ─┬─                      ─┬─

│ MAP_FORWARD_ACCESS_       │                        │

│ SIGNALLING                │                        │

├─────────────────────────>  │                        │

│ MAP_PROCESS_ACCESS_       │                        │

│ SIGNALLING                │                        │

│  <─────────────────────────┤                        │

─┴─                          ─┴─                      ─┴─

─┬─                          ─┬─                      ─┬─

│ MAP_SEND_END_SIGNAL ack   │                        │

├─────────────────────────>  │                        │

│                            │                        │

NOTE:	This can be sent at any time after the connection between MSC-A and MSC-B is established

Figure 16.2/2: Example of a successful basic handover procedure to MSC-B

E           ┌──────┐          B    ┌──────┐

┌────────┼───────────┤MSC-B'├──────────┼────┤VLR-B'│

│                    └──────┘               └──────┘

┌──────┐    │    E       ┌──────┐         B       ┌──────┐

│MSC-A ├────┴────┼───────┤MSC-B ├─────────┼───────┤VLR-B │

└──────┘                 └──────┘                 └──────┘

│                         │                       │

│                         │                       │

│ MAP_PREPARE_HANDOVER    │                       │

├──────────────────────>  ├                       │

│                         │ MAP_ALLOCATE_HANDOVER_│

│                         │ NUMBER                │

│                         ├────────────────────>  │

│                         │                       │

│                         │ MAP_SEND_HANDOVER_    │

│                         │ REPORT                │

│                         ┤  <────────────────────┤

│ MAP_PREPARE_HANDOVER ack│                       │

│  <──────────────────────┤                       ┤

│                         │                       │

─┴─                       ─┴─                     ─┴─

─┬─                       ─┬─                     ─┬─

│                         │ MAP_SEND_HANDOVER_    │

│                         │ REPORT ack   (NOTE)   │

│                         ├────────────────────>  │

─┴─                       ─┴─                     ─┴─

─┬─                       ─┬─                     ─┬─

│                         │                       │

│ MAP_PROCESS_ACCESS_     │                       │

│ SIGNALLING              │                       │

│  <──────────────────────┤                       │

│                         │                       │

│ MAP_SEND_END_SIGNAL     │                       │

│  <──────────────────────┤                       │

─┴─                       ─┴─                     ─┴─

─┬─                       ─┬─                     ─┬─

│ MAP_PREPARE_SUBSEQUENT- │                       │

│ HANDOVER                │                       │

│  <──────────────────────┤                       │

│                         │       │               │         │

│                         │       │               │         │

│ MAP_PREPARE_HANDOVER    │       │               │         │

├─────────────────────────┼────>  │               │         │

│                         │       │ MAP_ALLOCATE_HANDOVER_  │

│                         │       │ NUMBER                  │

│                         │       ├───────────────┼──────>  │

│                         │       │               │         │

│                         │       │ MAP_SEND_HANDOVER_      │

│                         │       │ REPORT                  │

│                         │       ┤  <────────────┼─────────┤

│ MAP_PREPARE_HANDOVER ack│       │                        │

│  <──────────────────────┼───────┤               │        │

─┴─                        │      ─┴─              │        ─┴─

─┬─                       ─┴─     ─┬─             ─┴─       ─┬─

│                        ─┬─      │              ─┬─        │

│                         │       │               │         │

│                         │       │               │         │

│                         │ MAP_SEND_HANDOVER_    │         │

│                         │ REPORT ack   (NOTE)   │         │

│                         ├───────┼────────────>  │         │

─┴─                        │      ─┴─              │        ─┴─

─┬─                       ─┴─     ─┬─             ─┴─       ─┬─

│                        ─┬─      │              ─┬─        │

│                         │       │               │         │

│ MAP_PREPARE_SUBSEQUENT- │       │               │         │

│ HANDOVER ack            │       │               │         │

├──────────────────────>  │       │               │         │

│                         │       │               │         │

─┴─                        │      ─┴─              │        ─┴─

─┬─                       ─┴─     ─┬─             ─┴─       ─┬─

│                        ─┬─      │              ─┬─        │

│                         │       │               │         │

│ MAP_PROCESS_ACCESS_     │       │               │         │

│ SIGNALLING              │       │               │         │

│  <──────────────────────┼───────┤               │         │

│                         │       │               │         │

│ MAP_SEND_END_SIGNAL     │       │               │         │

│  <──────────────────────┼───────┤               │         │

│                         │       │               │         │

│                         │       │              │         │

│ MAP_SEND_END_SIGNAL ack │       │              │         │

├──────────────────────>  │       │               │         │

─┴─                        │      ─┴─              │        ─┴─

─┬─                       ─┴─     ─┬─             ─┴─       ─┬─

│                        ─┬─      │              ─┬─        │

│                         │       │               │         │

│ The subsequent handover is completed, MSC-B' is           │

│ considered as MSC-B. Any further inter MSC-handover       │

│ is handled as describedfor a basic handover.              │

NOTE:	This can be sent at any time after the connection between MSC-A and MSC-B is established

Figure 16.2/3: Example of a handover towards a third MSC

16.2.2	Handover procedure in MSC-A

This section describes the handover procedure in MSC-A, including the request for a basic handover to another MSC (MSC-B), subsequent handover to a third MSC (MSC-B') or back to the controlling MSC (MSC-A).

16.2.2.1	Basic handover

When MSC-A has decided that a call has to be handed over to MSC-B, the Handover Control Application in MSC-A requests the MAP application to initiate the MAP_PREPARE_HANDOVER request to MSC-B.

MSC-A opens the dialogue to MSC-B with a MAP_OPEN request containing no user specific parameters and sends a MAP_PREPARE_HANDOVER request. This request may optionally contain an indication that a handover number allocation is not required, targetCellId, for compatibility reasons, and all information required by MSC-B to allocate the necessary radio resources.

If MSC-B accepts the dialogue, it returns a MAP_PREPARE_HANDOVER confirmation containing a handover number, unless the request has included the HO-NumberNotRequired parameter, and BSSAP information which is forwarded to and handled by the Handover Control Application in MSC-A.

Optionally MSC-A can receive, after a MAP_PREPARE_HANDOVER confirmation, a MAP_PROCESS_ACCESS_SIGNALLING indication containing BSSAP information.

When the connection has been established between the MS and MSC-B, MSC-A will be informed by a MAP_SEND_END_SIGNAL indication.

When MSC-A wants to clear the connection with BSS-B, an indication from the Handover Control Application is received in the Map Application to send the MAP_SEND_END-SIGNAL response to MSC-B to close the MAP dialogue.

MSC-A may abort the handover procedure at any time (e.g. if the call is cleared).

16.2.2.2	Handling of access signalling

If required, the Handover Control Application in MSC-A requests the MAP application to invoke the MAP_FORWARD_ACCESS_SIGNALLING request containing the information to be transferred to the A-interface of MSC-B (e.g. call control information).

MAP_FORWARD_ACCESS_SIGNALLING is a non-confirmed service.

MSC-B will then forward the required information to the Handover Control Application. The MAP_FORWARD_ACCESS_SIGNALLING is composed in such a way that the information can be passed transparently to the A-interface for call control and mobility management information. Any response received in MSC-B from the A-interface that should be brought to MSC-A will require a new independent request from the Handover Control Application in MSC-B to MSC-A by invoking a MAP_PROCESS_ACCESS_SIGNALLING request.

16.2.2.3	Other procedures in stable handover situation

During a call and after handover, a number of procedures between MSC-A and BSS-B controlled by or reported to MSC-A may be initiated in both directions by invoking a MAP_FORWARD_ACCESS_SIGNALLING request and reception of a MAP_PROCESS_ACCESS_SIGNALLING indication.

16.2.2.4	Subsequent handover

When MSC-A receives a MAP_PREPARE_SUBSEQUENT_HANDOVER request, it will start the procedure of handing the call over to a third MSC (MSC-B'), or back to the controlling MSC (MSC-A). If the new handover procedure towards MSC-B' or MSC-A is successful, the handover control application in MSC-A will request the release of the dialogue towards MSC-B by sending the MAP_SEND_END_SIGNAL confirmation.

16.2.2.5	SDL Diagrams

The SDL diagrams on the following pages describe the user processes in MSC-A for the procedures described in this section.

The services used are defined in section 6.4.

NOTE 1:	The message primitives HO_CA_MESSAGE used in the SDL-Diagrams are used to show the internal co-ordination between the MAP application and the Handover Control Application. For a detailed description of the co-ordination between the applications for the handover procedure, see TS GSM 03.09.

Note that in case of reception of errors from the MSCs (see the Handover error handling macro), the MAP user reports them to the Handover Control Application and does not take any action except in cases explicitly mentioned in the SDL diagrams.

Figure 16.2.2/1 sheet 1: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_211.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 2: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_212.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 3: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_213.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 4: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_214.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 5: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_215.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 6: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_216.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 7: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_217.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 8: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_218.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 9: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_219.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 10: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_21A.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 11: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_21B.EPS;18.2 cm;23 cm;EPS

Figure 16.2.2/1 sheet 12: Process MSC_A_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_21C.EPS;18.2 cm;23 cm;EPS

16.2.3	Handover procedure in MSC-B

This section describes the handover procedure in MSC-B, including the request for a handover from another MSC (MSC-A), subsequent handover to a third MSC (MSC-B') or back to the controlling MSC (MSC-A).

16.2.3.1	Basic handover

Opening of the dialogue is described in the macro Receive_Open_Ind in section 21.1.

When MSC-B process receives a MAP_PREPARE_HANDOVER indication from MSC-A, MSC-B requests its associated VLR to provide a handover number, unless the parameter HO-NumberNotRequired is received in the indication.

When the connection between the MS and MSC-B is established on MSC-B, the Handover Control Application will request the MAP application to indicate this event to MSC-A by invoking the MAP_SEND_END_SIGNAL request. When a call is released, MSC-A will inform MSC-B by MAP_SEND_END_SIGNAL response and the MAP dialogue between MSC-A and MSC-B is closed.

16.2.3.2	Allocation of handover number

When a handover number is required, a MAP_ALLOCATE_HANDOVER_NUMBER request will be sent to the VLR. The handover number is received in the MAP_SEND_HANDOVER_REPORT request, and will be included in the MAP_PREPARE_HANDOVER response to MSC-A.

As soon as the call from MSC-A using the handover number arrives in MSC-B, MSC-B shall release the handover number in the VLR using the MAP_SEND_HANDOVER_REPORT response.

16.2.3.3	Handling of access signalling

If required by the Handover Control Application, MSC-B invokes the MAP_PROCESS_ACCESS_SIGNALLING request containing the information received on the A-interface that should be transferred to MSC-A (e.g. call control information).

MAP_PROCESS_ACCESS_SIGNALLING is a non-confirmed service and any response from MSC-A will require a MAP_FORWARD_ACCESS_SIGNALLING request.

16.2.3.4	Other procedures in stable handover situation

During a call and after handover, a number of procedures between MSC-A and BSS-B controlled by or reported to MSC-A may be initiated by involving access signalling transfer in both directions.

16.2.3.5	Subsequent handover

The procedure is used when the Handover Control Application in MSC-B has decided that a call is to be handed over to another MSC (either back to the controlling MSC (MSC-A) or to a third MSC (MSC-B')).

After the MAP_PREPARE_SUBSEQUENT_HANDOVER response is received from MSC-A, MSC-B will await the disconnection of the call. Once the disconnect is complete, MSC-B will inform its VLR by invoking the MAP_SEND_HANDOVER_REPORT confirmation. VLR-B will then release the allocated handover number.

The subsequent handover procedure is shown in figure 16.2/3.

16.2.3.6	SDL Diagrams

The SDL diagrams on the following pages describe the user process in MSC-B for the procedures described in this section.

The services used are defined in section 6.4.

NOTE 1:	The message primitives HO_CA_MESSAGE in the SDL-diagrams are used to show the internal co-ordination between the MAP application and the Handover Control Application. For a detailed description of the co-ordination between the applications for the handover procedure, see TS GSM 03.09.

NOTE 2:	The order in the SDL diagrams to allocate first the handover number and then the radio resources is not binding.

Figure 16.2.3/1 sheet 1: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_311.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 2: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_312.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 3: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_313.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 4: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_314.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 5: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_315.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 6: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_316.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 7: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_317.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 8: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_318.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 9: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_319.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 10: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_31A.EPS;18.2 cm;23 cm;EPS

Figure 16.2.3/1 sheet 11: Process MSC_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_31B.EPS;18.2 cm;23 cm;EPS

16.2.4	Handover error handling macro

This macro is used for the handover procedures to receive errors from the MSCs and from the Handover Control Application at any state of a handover process.

If a MAP_NOTICE indication is received, the Handover Control Application is informed and the actual situation is kept and the Handover Control Application decides how the handover process should continue. In all other cases the MSC is returned to a "NULL" state.

Figure 16.2.4/1: Macro Receive_error_from_HO_CA_or_MSC

.G.C:\USER\MAP\MAP-470\EPS16\162_41.EPS;18.2 cm;23 cm;EPS

16.2.5	Handover procedure in VLR

16.2.5.1	Allocation of handover number

When receiving the MAP_ALLOCATE_HANDOVER_NUMBER indication, the VLR will determine whether a handover number is available. If no handover number is available, this will be indicated by a MAP_ALLOCATE_HANDOVER_NUMBER response with the appropriate error.

The handover number allocated will otherwise be returned to MSC-B in the MAP_SEND_HANDOVER_REPORT request.

The handover number will be reserved until a MAP_SEND_HANDOVER_REPORT confirmation is received from MSC-B.

16.2.5.2	SDL Diagrams

The SDL diagrams on the following pages describe the user processes in VLR for the procedures described in this section.

The services used are defined in section 6.4.

Figure 16.2.5/1 sheet 1: Process VLR_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_511.EPS;18.2 cm;23 cm;EPS

Figure 16.2.5/1 sheet 2: Process VLR_B_HO

.G.C:\USER\MAP\MAP-470\EPS16\162_512.EPS;18.2 cm;23 cm;EPS

16.3	Fault recovery procedures

After a fault of a location register, the fault recovery procedures ensure that the subscriber data in the VLR become consistent with the subscriber data that are stored in the HLR for the MS concerned and that the location information in HLR and VLR reflect accurately the current location of the MS.

The detailed specification of fault recovery procedures of location registers is given in TS GSM 03.07.

16.3.1	VLR fault recovery procedures

The following processes are involved with the restoration of one IMSI record in the VLR:

-	In case of a location registration request from the MS:

Update_Location_Area_VLR	section 16.1.1.3;

Update_Location_HLR	section 16.1.1.4.

-	In case of a mobile terminated call:

PRN_VLR		section 18.2.4;

RESTORE_DATA_VLR	section 18.2.4;

RESTORE_DATA_HLR	section 16.3.3;

ICS_VLR		section 18.3.3.

After a restart, the VLR shall erase all IMSI records affected by the failure and shall cause all affected TMSIs and all affected LMSIs to become invalid. There will be no subscriber data or location information stored for an affected mobile station until after the VLR has received either a MAP_PROVIDE_ROAMING_NUMBER indication or a MAP_UPDATE_LOCATION_AREA indication for that mobile station. Restoration of subscriber data in the VLR is triggered individually for each IMSI record by receipt of either of these indications.

Reception of either a MAP_UPDATE_LOCATION_AREA indication or a MAP_PROVIDE_ROAMING_NUMBER indication with an IMSI that is unknown in the VLR causes creation of a skeleton IMSI record that is marked as

-	not confirmed by radio contact by the indicator "Confirmed by Radio Contact" (The function of this indicator is described in TS GSM 03.07), and

-	not confirmed by HLR by the indicator "Confirmed by HLR" (The function of this indicator is described in TS GSM 03.07).

A third indicator "Location Information Confirmed in HLR" is allocated to each IMSI record in the VLR (The function of this indicator is described in TS GSM 03.07).

The indicator "Location Information Confirmed in HLR" shall be checked whenever authenticated radio contact with an MS has been established. The status "Not Confirmed" of this indicator shall force the VLR to invoke the MAP_UPDATE_LOCATION service but it shall never cause rejection of a mobile originated request. The status is changed from "Not Confirmed" to "Confirmed" only after successful completion of a MAP_UPDATE_LOCATION procedure for the MS concerned.

If the VLR serves only one MSC, the indicator "Location Information Confirmed in HLR" is only relevant to the HLR restoration procedure and an initial value must be assigned when an IMSI record is created in the VLR:

-	If the IMSI record was created due to a roaming number request, the initial value must be set to "Confirmed".

-	If reception of a MAP_UPDATE_LOCATION_AREA indication causes creation of the IMSI record, the initial value must be "Not Confirmed".

If the VLR serves more than one MSC, the indicator "Location Information Confirmed in HLR" is used in the VLR restoration procedure as well as in the HLR restoration procedure. When an IMSI record is created in the VLR, the indicator must be set to "Not Confirmed".

VLR restoration triggered by a location registration request

Upon receipt of a MAP_UPDATE_LOCATION_AREA indication, the VLR retrieves authentication data from the HLR by using the MAP_SEND_AUTHENTICATION_INFO service if authentication is required and if no authentication data are available in the VLR for the IMSI concerned (see figure 16.1.1/6).

Receipt of a MAP_UPDATE_LOCATION_AREA indication for an MS whose IMSI is unknown in the VLR or whose data stored in the VLR are marked as "Not Confirmed" by the indicator "Confirmed by HLR" and/or by the indicator "Location Information Confirmed in HLR" forces the VLR to invoke the MAP_UPDATE_LOCATION service after successful authentication, if required. The location updating procedure is performed as described in section 16.1.

Any other mobile originated request from an MS whose IMSI is unknown in the VLR or whose subscriber data stored in the VLR are marked as "Not Confirmed" by the indicator "Confirmed by HLR" shall be rejected with error cause "Unidentified Subscriber". This causes the MS to trigger the location registration procedure.

After successful completion of the MAP_UPDATE_LOCATION procedure, the indicators "Confirmed by HLR" and "Location Information Confirmed in HLR" are set to "Confirmed".

The indicator "Confirmed by Radio Contact" is set to "Confirmed" when the radio contact with the MS is authenticated.

VLR restoration triggered by a roaming number request

Figure 16.3/1 illustrates the signalling sequence for restoration of an IMSI record in the VLR triggered by a mobile terminating call set-up.

Upon receipt of a MAP_PROVIDE_ROAMING_NUMBER indication for an IMSI that is unknown in the VLR and for which authentication is required, the VLR retrieves authentication data from the HLR by using the MAP_SEND_AUTHENTICATION_INFO service after an MSRN has been sent to the HLR in the MAP_PROVIDE_ROAMING_NUMBER response.

Receipt of a MAP_PROVIDE_ROAMING_NUMBER indication for an MS whose IMSI is unknown in the VLR or whose data record in the VLR is marked as "Not Confirmed" by the indicator "Confirmed by HLR" forces the VLR to request subscriber data from the HLR by sending a MAP_RESTORE_DATA request which triggers one or more INSERT_SUBSCRIBER_DATA operations from the HLR. The MAP_RESTORE_DATA request may also be used to send the LMSI to the HLR.

The MAP_RESTORE_DATA process in the VLR is described in section 18.2.4.

The MAP_RESTORE_DATA process in the HLR is described in section 16.3.3.

After successful completion of the MAP_RESTORE_DATA procedure, the indicator "Confirmed by HLR" is set to "Confirmed".

If restoration of an IMSI record was triggered by a MAP_PROVIDE_ROAMING_NUMBER indication (i.e. by a mobile terminating call), the VLR has no valid Location Area Identity information for the MS concerned before successful establishment of the first authenticated radio contact. Upon receipt of a MAP_SEND_INFO_FOR_INCOMING_CALL indication from the MSC (see 5 in figure 16.3/1) for an MS whose subscriber data are marked as "Confirmed" by the indicator "Confirmed by HLR" but not confirmed by radio contact, the VLR shall invoke a "MAP_SEARCH_FOR_MS" instead of a "MAP_PAGE".

A MAP_SEARCH_FOR_MS shall also be performed if the VLR receives a MAP_SEND_INFO_FOR_MT_SMS indication from the MSC for an MS whose IMSI record is marked as "Confirmed" by the indicator "Confirmed by HLR" but not confirmed by radio contact.

The indicator "Confirmed by Radio Contact" is set to "Confirmed" when authenticated radio contact caused by a mobile originated or a mobile terminated activity is established.

┌─────┐         (B)         ┌─────┐          D          ┌─────┐

│ MSC ├──────────┼──────────┤ VLR ├──────────┼──────────┤ HLR │

└─────┘     (no external    └─────┘                     └─────┘

│         interface)        │                            │

│                           │                            │

│                           │ MAP_PROVIDE_ROAMING_NUMBER │)

│                           │<───────────────────────────┤)

│                           │ PROVIDE_ROAMING_NUMBER_ack │)1

│                           │───────────────────────────>│)

│                           │                            │

│                           │MAP_SEND_AUTHENTICATION_INFO│)

│                  (NOTE 1) │--------------------------->│)

│                           │SEND_AUTHENTICATION_INFO_ack│)2

│                           │<---------------------------│)

│                           │                            │)

│                           │     MAP_RESTORE_DATA       │

│                           ├───────────────────────────>│

( │                           │                            │

( │                           │  MAP_ACTIVATE_TRACE_MODE   │

( │                  (NOTE 2) │<---------------------------│

( │                           │MAP_ACTIVATE_TRACE_MODE_ack │

( │                           │--------------------------->│

( │                           │                            │

4( │                           │MAP_INSERT_SUBSCRIBER_DATA  │)

( │ 1) see section  18.2.4    │<───────────────────────────│)

( │ 2) see section  21.5      │ INSERT_SUBSCRIBER_DATA_ack │)3

( │ 3) see sections 16.4.1    ├───────────────────────────>│)

( │                 21.7      │                            │)

( │ 4) see sections 16.3.3    │    MAP_RESTORE_DATA_ack    │

( │                 18.2.4    │<───────────────────────────┤

│                           │                            │

───┼───────────────────────────┼────────────────────────────┼─

│                           │                            │

( │MAP_SEND_INFO_FOR_INCOMING_CALL or                      │

( ├──────────────────────────>│                            │

( │ MAP_SEND_INFO_FOR_MT_SMS  │                            │

( │                           │                            │

5( │   MAP_SEARCH_FOR_MS       │                            │

( │<─────────────────────────-│                            │

( │                           │                            │

( │  MAP_SEARCH_FOR_MS_ack    │                            │

( ├──────────────────────────>│                            │

( │                           │                            │

( │MAP_PROCESS_ACCESS_REQUEST │                            │

( ├──────────────────────────>│ 5) see sections 18.3, 20.3 │

│                           │                            │

NOTE 1:	If authentication required.

NOTE 2:	If subscriber tracing active in HLR.

Figure 16.3/1: Procedures related to restoration of VLR in case of mobile terminated call set-up

16.3.2	HLR fault recovery procedures

The following processes are involved with the restart of the HLR:

-	HLR_RESTART			section 16.3.2;

-	REC_RESET_IN_VLR		section 16.3.2.

In the case of a location registration request from the MS, the following processes are involved with the HLR restoration procedure:

-	Update_Location_Area_VLR		section 16.1.1.3;

-	Update_Location_HLR		section 16.1.1.4.

In the case of a mobile originated service request, the

-	Macro Process_Access_Request_VLR	section 21.4.2; and the

-	Process Update_Location_HLR		section 16.1.1.4,

are involved with the HLR restoration procedure.

For the HLR, periodic backup of data to non-volatile memory is mandatory.

Data that have been changed in the period of time after the last backup storage and before the restart of the HLR cannot be recovered by reload from the non-volatile memory. Therefore, a restoration procedure is triggered individually for each IMSI record that has been affected by the HLR fault at the first authenticated radio contact that is established with the MS concerned.

The HLR restoration procedure forces updating of MSC number, VLR number and, if provided by the VLR, LMSI in the HLR. Consistency of subscriber data that are stored in the VLR for an MS that has been affected by a HLR fault with the subscriber data stored in the HLR for this MS will be achieved.

As an implementation option, a notification can be forwarded to the MS to alert the subscriber to check the parameters for supplementary services that allow subscriber controlled input (MAP_FORWARD_CHECK_SS_INDICATION service). If the VLR receives this notification from the HLR it shall forward the notification to the MS.

Figure 16.3/2 illustrates the signalling sequence for HLR restoration.

After a restart, the home location register performs the following actions for the subscriber data records that have been affected by the HLR fault (see figure 16.3/3):

-	reload all data from the non-volatile back-up;

-	if the MAP_FORWARD_CHECK_SS_INDICATION service is implemented, mark each subscriber record "SS Check Required" by setting the "Check SS" indicator;

-	reset the "MS Purged" flag for each of its MSs;

-	send a MAP_RESET request to the VLRs where its MSs are located (see figure 16.3/4).

The MAP_RESET request contains the HLR number and optionally the HLR Identity List.

When receiving a MAP_RESET indication, the VLR will derive all involved MSs of that HLR either from the HLR Identity List (if present), or from the HLR number. The VLR will then mark these MSs with the indicator "Location Information Confirmed in HLR" set to "Not Confirmed" (see figure 16.3/5).

The status "Not Confirmed" of the indicator "Location Information Confirmed in HLR" forces the VLR to invoke the MAP_UPDATE_LOCATION service after establishment of authenticated radio contact with the MS concerned.

The MAP_UPDATE_LOCATION procedure is performed as described in section 16.1.

After receipt of the MAP_UPDATE_LOCATION acknowledge containing the HLR number, the status of the indicator "Location Information Confirmed in HLR" is changed to "Confirmed".

If the MAP_UPDATE_LOCATION procedure is unsuccessful for any reason, the status of the indicator "Location Information Confirmed in HLR" remains unchanged except for the case that the IMSI record in the VLR is deleted because either of the errors "Unknown Subscriber" or "Roaming Not Allowed" has been received from the HLR in response to a MAP_UPDATE_LOCATION request.

┌─────┐          D          ┌─────┐         (B)         ┌─────┐

│ HLR ├──────────┼──────────┤ VLR ├──────────┼──────────┤ MSC │

└─────┘                     └─────┘     (no external    └─────┘

│       MAP_RESET           │         interface)        │

├──────────────────────────>│                           │

│                           │                           │

│                          ││ MAP_UPDATE_LOCATION_AREA  │

│                          ││<──────────────────────────┤

│                          ││MAP_PROCESS_ACCESS_REQUEST │

│   MAP_UPDATE_LOCATION    ││<──────────────────────────┤

┌──┤<─────────────────────────┤│                           │

│  │ MAP_ACTIVATE_TRACE_MODE   │ )                         │

│  │-------------------------->│ ) If subscriber tracing   │

│  │  ACTIVATE_TRACE_MODE_ack  │ ) is active in HLR.       │

│  │<--------------------------│ )                         │

│  │  MAP_INSERT_SUBSCR_DATA   │                           │

│  ├──────────────────────────>│                           │

│  │  INSERT_SUBSCR_DATA_ack   │                           │

│  │<──────────────────────────┤                           │

│  │ MAP_UPDATE_LOCATION_ack   │   MAP_FORWARD_CHECK_SS_   │

└─>├──────────────────────────>┼──────────────────────────>│

│and  MAP_FORWARD_CHECK_SS_ │   INDICATION              │

│     INDICATION (optional) │   (if received from HLR)  │

Figure 16.3/2: Procedures related to restoration of HLR

Figure 16.3/3: Process HLR_RESTART

.G.C:\USER\MAP\MAP-470\EPS16\163_3.EPS;18.2 cm;23 cm;EPS

Figure 16.3/4: Process SEND_RESET_TO_VLR

.G.C:\USER\MAP\MAP-470\EPS16\163_4.EPS;18.2 cm;23 cm;EPS

Figure 16.3/5: Process REC_RESET_IN_VLR

.G.C:\USER\MAP\MAP-470\EPS16\163_5.EPS;18.2 cm;23 cm;EPS

16.3.3	VLR restoration: the restore data procedure in the HLR

The MAP_RESTORE_DATA procedure in the HLR (Process RESTORE_DATA_HLR) is described in this section; the corresponding procedure in the VLR (RESTORE_DATA_VLR) is described in section 18.2.4.

The process RESTORE_DATA_HLR makes use of the following macros:

-	Receive_Open_Ind		section 21.1.1;

-	Check_Indication			section 21.2.1;

-	Insert_Subs_Data_Framed_HLR	section 16.4.1.

The MAP_RESTORE_DATA service is invoked by the VLR after provision of a roaming number in response to a MAP_PROVIDE_ROAMING_NUMBER indication for an unidentified MS (i.e. IMSI unknown in VLR), or for a known MS whose IMSI record is marked as "Not Confirmed" by the indicator "Confirmed by HLR" (see 4 in figure 16.3/1). The process RESTORE_DATA_VLR is shown in figure 18.2/6.

The restore data process in the HLR is activated by receipt of a MAP_RESTORE_DATA indication from the VLR (see figure 16.3/6). If there is a parameter problem in the indication, either of the errors "Unexpected Data Value" or "Data Missing" is returned in the MAP_RESTORE_DATA response; if the subscriber is not known in the HLR, the error "Unknown Subscriber" is returned in the MAP_RESTORE_DATA response. In all of these cases the process in the HLR teminates.

If the MAP_RESTORE_DATA indication is accepted and if the LMSI is received, the HLR updates the LMSI for the IMSI received in the MAP_RESTORE_DATA indication. For this IMSI the HLR sets "subscriber-tracing-not-active-in-VLR" and checks whether tracing is required. This check is handled by the macro "Control_Tracing_HLR" that is described in section 21.9. Thereafter, the macro "Insert_Subs_Data_Framed_HLR" that is described in section 16.4.1 is invoked. The outcome of the macro Insert_Subs_Data_Framed_HLR is one of:

-	abort, in which case the process terminates;

-	error, in which case the HLR returns the error "System Failure" in the MAP_RESTORE_DATA response, and the process terminates;

-	OK, indicating successful outcome of downloading the subscriber data to the VLR.

After successful completion of the framed MAP_INSERT_SUBSCRIBER_DATA procedure, the HLR Number and, if applicable, the "MS Not Reachable Flag" which is used for SMS, are provided in the MAP_RESTORE_DATA response.

Upon receipt of the MAP_RESTORE_DATA confirmation, the VLR behaves as described in section 18.2.4, figure 18.2/6.

Figure 16.3/6: Process RESTORE_DATA_HLR

.G.C:\USER\MAP\MAP-470\EPS16\163_6.EPS;18.2 cm;23 cm;EPS

16.4	Macro Insert_Subs_Data_Framed_HLR

This macro is used by any procedure invoked in HLR which requires the transfer of subscriber data by means of the InsertSubscriberData operation (e.g. Update Location or Restore Data).

The invocation of the operation is done in a dialogue already opened by the framing procedure. Therefore the latter is the one that handles the reception of the open indication and sends the dialogue close request.

The macro calls the process "Send_Insert_Subs_Data" (see section 21.7.4) as many times as it is needed for transferring all subscriber data. This process call is meant to describe two possible behaviours of HLR to handle service requests and confirmations:

-	either the HLR handles requests and confirmations in parallel;or

-	the HLR sends the next request only after receiving the confirmation to the previous one.

Another call is done to the macro "Wait_for_Insert_Subscriber_Data" (see section 21.7.3). There the reception and handling of the service confirmations is described.

The SDL diagram is shown in figure 16.4/1

Figure 16.4/1: Macro Insert_Subs_Data_Framed_HLR

.G.C:\USER\MAP\MAP-470\EPS16\164_1.EPS;18.2 cm;23 cm;EPS

17	Operation and maintenance procedures

17.1	General

The Operation and Maintenance procedures are needed for operating and maintaining the GSM PLMN network.

The following procedures exist for operation and maintenance purposes:

i)	Tracing procedures;

ii)	Subscriber Data Management procedures;

iii)	Subscriber Identity procedures.

The following application contexts refer to complex MAP Users consisting of several processes:

-	subscriberDataManagementContext;

-	tracingContext.

These two application contexts need a co-ordinating process in the VLR as described in the following sections.

17.1.1	Tracing Co-ordinator for the VLR

The MAP_OPEN indication opens the dialogue for the stand-alone tracing procedure when the application context tracingContext is received. If that service is successful, the Co-ordinator can receive the firs service primitive from the MAP_PM. Depending on the received primitive, the user process is created as follows:

-	if the MAP_ACTIVATE_TRACE_MODE indication is received, the process ATM_VLR_Standalone is created;

-	if the MAP_DEACTIVATE_TRACE_MODE indication is received, the process DTM_VLR_Standalone is created.

After creation of the user process the Co-ordinator relays the messages between the MAP_PM and the invoked process until a request or an indication for dialogue termination is received.

The Tracing Co-ordinator is shown in the figure 17.1/1.

Figure 17.1/1: Process Co_Tracing_VLR

.G.C:\USER\MAP\MAP-470\EPS17\171_1.EPS;18.2 cm;23 cm;EPS

17.1.2	Subscriber Data Management Co-ordinator for the VLR

The MAP_OPEN indication opens the dialogue for the stand-alone subscriber data management procedure when the application context subscriberDataManagementContex is received. If that service is successful, the Co-ordinator can receive the firs service primitive from the MAP_PM. Depending on the received primitive, the user process is created as follows:

-	if the MAP_INSERT_SUBSCRIBER_DATA indication is received, the process INS_SUBS_DATA_VLR is created;

-	if the MAP_DELETE_SUBSCRIBER_DATA indication is received, the process Delete_Subscriber_Data_VLR is created.

After creation of the user process the Co-ordinator relays the messages between the MAP_PM and the invoked process until a request or an indication for dialogue termination is received.

The Subscriber_Data_Management Co-ordinator is shown in the figure 17.1/2.

Figure 17.1/2: Process Co_Data_Management_VLR

.G.C:\USER\MAP\MAP-470\EPS17\171_2.EPS;18.2 cm;23 cm;EPS

17.2	Tracing procedures

Three type of tracing procedures exist:

i)	Subscriber tracing management procedures;

ii)	Subscriber tracing procedures;

iii)	Event tracing procedures.

The subscriber tracing management procedures are used for management of the status and the type of the tracing. The subscriber tracing activation procedure is used at location updating or data restoration when the trace mode of a subscriber is set active in the HLR or, as a stand alone procedure, when the subscriber is already registered and the trace mode becomes active in the HLR. The procedures for providing a trace request to the VLR are shown in figures 17.2/1 and 17.2/2.

┌─────┐            ┌──────┐            ┌───────┐

│ VLR │            │ HLR  │            │ OMC   │

└─────┘            └──────┘            └───────┘

│                   │1.                  │

│2.                 ├< ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

├<══════════════════┤                    │

│                 3.│                    │

├══════════════════>┤                  4.│

│                   ├ ─ ─ ─ ─ ─ ─ ─ ─ ─ >┤

│                   │                    │

1.	Subscriber Tracing Activation

2.	MAP_ACTIVATE_TRACE_MODE

3.	MAP_ACTIVATE_TRACE_MODE_ACK

4.	Subscriber Tracing Activation Accepted

Figure 17.2/1: Stand alone subscriber tracing activation procedure

┌─────┐            ┌──────┐

│ VLR │            │ HLR  │

└─────┘            └──────┘

│                   │

│                 1.│

├══════════════════>│

│2.                 │

├<══════════════════╡

│                 3.│

├══════════════════>│

│4.                 │

├<══════════════════╡

│                   │

1.	MAP_UPDATE_LOCATION or MAP_RESTORE_DATA

2.	MAP_ACTIVATE_TRACE_MODE

3.	MAP_ACTIVATE_TRACE_MODE_ACK

4.	MAP_UPDATE_LOCATION_ACK or MAP_RESTORE_DATA_ACK

Figure 17.2/2: Subscriber tracing activation procedure at location updating or data restoration

The HLR sends the trace request (IMSI, trace reference, trace type and identity of the OMC) to the VLR in a MAP_ACTIVATE_TRACE_MODE request. The receipt of this primitive is acknowledged. The acknowledge primitive will indicate that the trace request is accepted by the VLR. If the request is not accepted, the reason will be reported to the HLR.

The subscriber tracing deactivation procedure is used when the trace request of a subscriber is to be cancelled in the VLR. The procedure is shown in figure 17.2/3.

┌─────┐            ┌──────┐            ┌───────┐

│ VLR │            │ HLR  │            │ OMC   │

└─────┘            └──────┘            └───────┘

│                   │1.                  │

│2.                 ├< ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

├<══════════════════┤                    │

│                 3.│                    │

├══════════════════>┤                  4.│

│                   ├ ─ ─ ─ ─ ─ ─ ─ ─ ─ >┤

│                   │                    │

1.	Subscriber Tracing Deactivation

2.	MAP_DEACTIVATE_TRACE_MODE

3.	MAP_DEACTIVATE_TRACE_MODE_ACK

4.	Subscriber Tracing Deactivation Accepted

Figure 17.2/3: Subscriber tracing deactivation procedure

The HLR sends a MAP_DEACTIVATE_TRACE_MODE request to the VLR. The VLR will acknowledge the deactivation. The acknowledge primitive will indicate that the trace request has been deleted by the VLR. If the deactivation is not accepted, the reason will be reported to the HLR.

The subscriber tracing procedures are used when the VLR detects any subscriber related activity for which the trace mode is activated, e.g. receives the MAP_PROCESS_ACCESS_REQUEST indication. The procedure is shown in figure 17.2/4.

┌─────┐            ┌──────┐            ┌───────┐

│ MSC │            │ VLR  │            │ OMC   │

└─────┘            └──────┘            └───────┘

│                 1.│                    │

├══════════════════>┤                    │

│2.                 │                    │

├<══════════════════┤                    │

│                   │                  3.│

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┼ ─ ─ ─ ─ ─ ─ ─ ─ ─ >┤

│                   │                    │

1.	MAP_PROCESS_ACCESS_REQUEST, MAP_UPDATE_LOCATION_AREA,

2.	MAP_TRACE_SUBSCRIBER_ACTIVITY

3.	Subscriber tracing information

Figure 17.2/4: Subscriber tracing procedure in the servicing MSC

The VLR will generate the MAP_TRACE_SUBSCRIBER_ACTIVITY indication. The receiving MSC will send the trace record to the OMC.

[Figure numbers 17.2/5 and 17.2/6 are spare.]

17.2.1	Procedures in the HLR

17.2.1.1	Subscriber tracing activation procedure

When receiving the subscriber tracing mode activation command for a subscriber from the OMC, the HLR will activate tracing, if the subscriber is known and registered in the HLR and the subscriber is roaming in the home PLMN area. The MAP_ACTIVATE_TRACE_MODE request is sent to the VLR where the subscriber is registered.

If the MAP_ACTIVATE_TRACE_MODE confirmation is received indicating an error situation, the errors are mapped to the OMC interface. The activation request may also be repeated.

If the subscriber is known in the HLR, but is deregistered or roaming outside the home PLMN area, the subscriber tracing status is activated in the HLR, but the VLR is not updated.

When receiving a request for location updating or data restoration while the subscriber trace mode is active, the macro Control_Tracing_HLR (see figure 21.9/4) shall be initiated by the location updating process in the HLR.

The subscriber tracing activation process in the HLR is shown in figure 17.2/7.

Figure 17.2/7 sheet 1: Process ATM_HLR

.G.C:\USER\MAP\MAP-470\EPS17\172_71.EPS;18.2 cm;23 cm;EPS

Figure 17.2/7 sheet 2: Process ATM_HLR

.G.C:\USER\MAP\MAP-470\EPS17\172_72.EPS;18.2 cm;23 cm;EPS

17.2.1.2	Subscriber tracing deactivation procedure

When receiving the subscriber trace mode deactivation command for a subscriber from the OMC, the HLR will send the MAP_DEACTIVATE_TRACE_MODE request to the VLR where the subscriber is registered, if the trace mode activation has been carried out. The subscriber tracing in HLR is set to a deactive state.

If the operation is successful, the HLR will set the subscriber tracing in VLR to a deactive state.

If the MAP_DEACTIVATE_TRACE_MODE confirmation is received indicating an error situation, the errors are mapped to the OMC interface. The deactivation request may be also repeated.

The subscriber tracing deactivation procedure is shown in figure 17.2/8.

Figure 17.2/8 sheet 1: Process DTM_HLR

.G.C:\USER\MAP\MAP-470\EPS17\172_81.EPS;18.2 cm;23 cm;EPS

Figure 17.2/8 sheet 2: Process DTM_HLR

.G.C:\USER\MAP\MAP-470\EPS17\172_82.EPS;18.2 cm;23 cm;EPS

17.2.2	Procedures in the VLR

The VLR is involved in the following tracing procedures:

i)	Subscriber tracing activation procedure;

ii)	Subscriber tracing deactivation procedure;

iii)	Subscriber tracing procedure.

17.2.2.1	Subscriber tracing activation procedure

When receiving a MAP_ACTIVATE_TRACE_MODE indication, the VLR will check the parameters and data in the primitive. Data errors are reported as an unexpected data value error or as a data missing error depending on the nature of the error.

If the subscriber is known, the tracing facility is supported and the tracing capacity is not exceeded, the successful report is sent in the MAP_ACTIVATE_TRACE_MODE response primitive.

The MAP_ACTIVATE_TRACE_MODE indication primitive may be received during a location updating or data restoration procedure, so the location updating or restore data process shall use the macro Activate_Tracing_VLR (see figure 21.9/3).

The subscriber tracing activation process in the VLR is shown in figure 17.2/9.

Figure 17.2/9: Process ATM_VLR_Standalone

.G.C:\USER\MAP\MAP-470\EPS17\172_9.EPS;18.2 cm;23 cm;EPS

17.2.2.2	Subscriber tracing deactivation procedure

When receiving a MAP_DEACTIVATE_TRACE_MODE indication, the VLR will check the parameters and data in the primitive. Data errors are reported as an unexpected data value error or as a data missing error depending on the nature of the error.

If the subscriber is known and the tracing facility is supported, the successful report is sent in the MAP_DEACTIVATE_TRACE_MODE response primitive.

The subscriber tracing deactivation procedure in the VLR is shown in figure 17.2/10.

Figure 17.2/10: Process DTM_VLR_Standalone

.G.C:\USER\MAP\MAP-470\EPS17\172_A.EPS;18.2 cm;23 cm;EPS

17.2.2.3	Subscriber tracing procedure

When the VLR receives a MAP_PROCESS_ACCESS_REQUEST, MAP_UPDATE_LOCATION_AREA or MAP_IMSI_ATTACH indication related to any subscriber activity from the MSC, the subscriber tracing procedure may be carried out. The macro Trace_Subscriber_Activity_VLR is shown in figure 21.9/2.

17.2.3	Procedures in the MSC

The MSC is involved in the following tracing procedure:

i)	Subscriber tracing procedure.

17.2.3.1	Subscriber tracing procedure

When receiving the MAP_TRACE_SUBSCRIBER_ACTIVITY indication from the VLR, the MSC stores trace reference, trace type and the identity of the OMC in charge of the trace, and the MSC starts to collect the trace information. The MSC will send the trace record to the OMC.

The macro Trace_Subscriber_Activity_MSC is shown in figure 21.9/1.

17.3	Subscriber Data Management procedures

Two types of subscriber data management procedures exist in the Mobile Application Part

i)	Subscriber Deletion;

ii)	Subscriber Data Modification.

No requirements have been identified for the Subscriber creation and subscriber data interrogation procedures.

The subscriber deletion and subscriber data modification procedures are initiated by the OMC (see figures 17.3/1 and 17.3/2).

┌─────┐            ┌──────┐            ┌───────┐

│ VLR │            │ HLR  │            │ OMC   │

└─────┘            └──────┘            └───────┘

│                   │1.                  │

│2.                 ├< ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

├<══════════════════┤                    │

│                 3.│                    │

├══════════════════>┤                  4.│

│                   ├ ─ ─ ─ ─ ─ ─ ─ ─ ─ >┤

1.	Delete Subscriber

2.	MAP_CANCEL_LOCATION

3.	MAP_CANCEL_LOCATION_ACK

4.	Subscriber Deleted

Figure 17.3/1: Subscriber deletion procedure

In the subscriber deletion procedure the subscriber data should be removed from the VLR and from the HLR. The HLR uses the MAP_CANCEL_LOCATION service.

┌─────┐            ┌──────┐            ┌───────┐

│ VLR │            │ HLR  │            │ OMC   │

└─────┘            └──────┘            └───────┘

│                   │1.                  │

│2.                 ├< ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

├<══════════════════┤                    │

│                 3.│                    │

├══════════════════>┤                  4.│

│                   ├ ─ ─ ─ ─ ─ ─ ─ ─ ─ >┤

1.	Modify Subscriber Data

2.	MAP_CANCEL_LOCATION, MAP_INSERT_SUBSCRIBER_DATA or

MAP_DELETE_SUBSCRIBER_DATA

3.	MAP_CANCEL_LOCATION_ACK,MAP_INSERT_SUBSCRIBER_DATA_ACK or

MAP_DELETE_SUBSCRIBER_DATA_ACK

4.	Subscriber Data Modified

Figure 17.3/2: Subscriber data modification procedure

In the subscriber data modification procedure the subscriber data is modified in the HLR and when necessary also in the VLR. The HLR initiates either the MAP_INSERT_SUBSCRIBER_DATA, MAP_DELETE_SUBSCRIBER_DATA or MAP_CANCEL_LOCATION service depending on the modified data.

17.3.1	Procedures in the HLR

17.3.1.1	Subscriber deletion procedure

When the subscriber deletion request is received from the OMC, the HLR shall delete the subscriber data from the HLR and initiate the MAP_CANCEL_LOCATION request to the VLR where the subscriber is registered.

The subscriber deletion procedure in the HLR is shown in the figure 17.3/3.

Figure 17.3/3: Process Delete_Subscriber_HLR

.G.C:\USER\MAP\MAP-470\EPS17\173_3.EPS;18.2 cm;23 cm;EPS

17.3.1.2	Subscriber data modification procedure

The OMC can modify the subscriber data in several different ways. The modifications can be categorized in five groups:

a)	HLR internal modification, no effect in the VLR;

b)	data shall be modified both in the HLR and VLR;

c)	withdrawal of a basic service or a supplementary service;

d)	modification affects on the roaming of the subscriber and the subscriber shall be removed from the VLR data base;

e)	authentication algorithm or authentication key of the subscriber is modified.

In case "b" the MAP_INSERT_SUBSCRIBER_DATA service is initiated in the HLR.

In case "c" the MAP_DELETE_SUBSCRIBER_DATA service is initiated in the HLR.

In cases "d" and "e" the MAP_CANCEL_LOCATION service is initiated in the HLR.

If the result of a primitive received from the VLR is unsuccessful, the HLR shall initiate re-attempts.

The subscriber data modification procedure in the HLR is shown in the figures 17.3/4, 17.3/5 and 21.7/2.

Figure 17.3/4: Process Modify_Data_HLR

.G.C:\USER\MAP\MAP-470\EPS17\173_4.EPS;18.2 cm;23 cm;EPS

Figure 17.3/5: Macro Delete_Subscriber_Data_HLR

.G.C:\USER\MAP\MAP-470\EPS17\173_5.EPS;18.2 cm;23 cm;EPS

17.3.2	Procedures in the VLR

17.3.2.1	Subscriber deletion procedure

The subscriber deletion procedure in the VLR is described in the section 16.1.

17.3.2.2	Subscriber data modification procedure

When receiving either the MAP_INSERT_SUBSCRIBER_DATA indication or the MAP_DELETE_SUBSCRIBER_DATA indication, the VLR check the parameters and data in the primitive. Data errors are reported as an unexpected data value error or a data missing error depending on the nature of the error.

After receiving the first MAP_INSERT_SUBSCRIBER_DATA indication, the VLR will check the IMSI that is included in the primitive. If the IMSI is unknown, the error "Unidentified subscriber" is returned.

If the updating of the subscriber data is not possible, the VLR will initiate the MAP_U_ABORT request primitive. If the updating is successful, the MAP_CLOSE indication is received from the HLR.

The subscriber data modification procedure in the VLR is shown in the figures 17.3/6, 17.3/7 and 21.7/1.

Figure 17.3/6: Process INS_SUBS_DATA_VLR

.G.C:\USER\MAP\MAP-470\EPS17\173_6.EPS;18.2 cm;23 cm;EPS

Figure 17.3/7: Process Delete_Subscriber_Data_VLR

.G.C:\USER\MAP\MAP-470\EPS17\173_7.EPS;18.2 cm;23 cm;EPS

17.4	Subscriber Identity procedure

In the subscriber identity procedure the IMSI of the subscriber is retrieved from the HLR. The procedure is shown in figure 17.4/1.

┌─────┐   ┌──────┐                ┌──────┐

│ OMC │   │ VLR  │                │ HLR  │

│     │   │      │                │      │

└─────┘   └──────┘                └──────┘

│        1.│                      │

├ ─ ─ ─ ─ >┤                    2.│

│          ├═════════════════════>┤

│          │ 3.                   │

│ 4.       ├<═════════════════════╡

├< ─ ─ ─ ─ ┤                      │

│          │                      │

1.	Identity request

2.	MAP_SEND_IMSI

3.	MAP_SEND_IMSI_ACK

4.	Identity confirm

Figure 17.4/1: The subscriber identity procedure

17.4.1	Subscriber identity procedure in the HLR

When receiving the MAP_SEND_IMSI indication, the HLR will check the parameters and data in the primitive. Data errors are reported as an unexpected data value error or a data missing error depending on the nature of the error.

If the subscriber is known in the HLR, the IMSI is fetched from the database and sent to the VLR. If the MSISDN cannot be identified, unknown subscriber indication is passed to the VLR.

The subscriber identity procedure in the HLR is shown in figure 17.4/2.

Figure 17.4/2: Process Send_IMSI_HLR

.G.C:\USER\MAP\MAP-470\EPS17\174_2.EPS;18.2 cm;23 cm;EPS

17.4.2	Subscriber identity procedure in the VLR

When the IMSI request is received from the OMC, the VLR will send the MAP_SEND_IMSI request to the HLR. The contents of the response is sent to the OMC.

The subscriber identity procedure in the VLR is shown in figure 17.4/3.

Figure 17.4/3: Process Send_IMSI_VLR

.G.C:\USER\MAP\MAP-470\EPS17\174_3.EPS;18.2 cm;23 cm;EPS

18	Call handling procedures

18.1	General

The Call Handling Procedures are used for the following:

i)	for MS terminating or MS originating calls where the MSC (acting as a VMSC) can obtain the required information directly from the visitor location register;

ii)	for MS terminating calls where the MSC (acting as gateway MSC) has to interrogate the home location register in order to obtain routing information. The HLR must retrieve the routing information from the VLR.

To provide a complete description of the Call Handling procedures, reference is made to the associated Call Handling Procedures in the BSS, MS and Fixed Network. These are only shown to indicate the interworking possibilities. Where inconsistency exists then the relevant procedures described in the following recommendations take precedence:

BSS to MSC procedures	GSM 08.08;

MSC to MS procedures	GSM 04.08;

ISUP procedures		CCITT Q.761-764 (1988);

TUP procedures		CCITT Q.721-725 (1988).

It should be noted that although the call handling procedures for TUP and ISUP are referenced, other national/proprietary user parts may be used in the fixed network.

The Call Handling coordination procedures are described in figures 18.1/1, 18.1/2 and 18.1/3. These procedures perform a coordination and distribution function within the MAP User for Call Handling.

The MSC (see figure 18.1/1) process activates the appropriate process depending on the message received and the function performed by the MSC (either MSC or Gateway MSC). The MSC can receive only one message from the MS, a CM Service Request (A_CM_SERV_REQ) for an outgoing call. Receipt of this message will result in the Outgoing Call procedures being activated, this procedure is described in section 18.4. If the MSC is acting as a Gateway MSC then an Initial Address Message (I_IAM) received from the network will activate the Send Routing Information (SRI_MSC) procedure as described in section 18.2. If the MSC is not acting as a Gateway then the incoming Initial Address Message (I_IAM) will activate the incoming call set-up procedure (ICS_MSC) as described in section 18.3.

The Call Handling coordination procedures in the VLR (figure 18.1/2) and HLR (figure 18.1/3) will be activated when a MAP_OPEN_ind is received which requests one of the Call Handling Application Contexts, the Application Contexts assigned to Call Handling are described in section 14.2. If the Application Context requested is not a MAP Version 1 context and there is no error detected in the Open indication, the appropriate process will be activated. For further details of the process activated refer to the following sections:

Incoming Call Set Up	section 18.3;

Outgoing Call Set Up	section 18.4;

Provide Roaming Number	section 18.2;

Send Routing Information	section 18.2.

NOTE:	Where reference is made in this section to a supplementary service being active, the service must be active for the required basic service. If no basic service is present in a construct then all basic services are covered by the construct.

Figure 18.1/1 sheet 1: Process CH_COORD_MSC

.G.C:\USER\MAP\MAP-470\EPS18\181_11.EPS;18.2 cm;23 cm;EPS

Figure 18.1/1 sheet 2: Process CH_COORD_MSC

.G.C:\USER\MAP\MAP-470\EPS18\181_12.EPS;18.2 cm;23 cm;EPS

Figure 18.1/2 sheet 1: Process CH_COORD_VLR

.G.C:\USER\MAP\MAP-470\EPS18\181_21.EPS;18.2 cm;23 cm;EPS

Figure 18.1/2 sheet 2: Process CH_COORD_VLR

.G.C:\USER\MAP\MAP-470\EPS18\181_22.EPS;18.2 cm;23 cm;EPS

Figure 18.1/3 sheet 1: Process CH_COORD_HLR

.G.C:\USER\MAP\MAP-470\EPS18\181_31.EPS;18.2 cm;23 cm;EPS

Figure 18.1/3 sheet 2: Process CH_COORD_HLR

.G.C:\USER\MAP\MAP-470\EPS18\181_32.EPS;18.2 cm;23 cm;EPS

18.2	Retrieval of routing information

18.2.1	General

The procedure is shown in figure 18.2/1; the sequence only assumes successful operation.

Network       Gateway

┌───┐          ┌───┐              ┌───┐                ┌───┐

│   ├──────────┤MSC├──────────────┤HLR├────────────────┤VLR│

└─┬─┘          └─┬─┘              └─┬─┘                └─┬─┘

│I_IAM (NOTE 2)│MAP_SEND_ROUTING_ │                    │

├─────────────>│INFORMATION       │                    │

│ ┌───┐        ├─────────────────>│                    │

│ │MSC├────────┤         (NOTE 1) │MAP_PROVIDE_ROAMING_│

│ └─┬─┘        │                  │ NUMBER             │

│   │          │                  ├───────────────────>│

│   │          │                  │MAP_PROVIDE_ROAMING_│

│   │          │                  │ NUMBER ack         │

│   │          │ MAP_SEND_ROUTING_├<───────────────────┤

│   │          │  INFORMATION ack │                    │

│   │          │<─────────────────┤ MAP_RESTORE_DATA   │

│   │   I-IAM  │                  │                    │

│   ├<─────────┤                  │                    │

│   │          │                  │                    │

Notes:

xxx=	Optional Procedure

NOTE 1:	This service may also be used by an ISDN exchange for obtaining routing information from the HLR.

NOTE 2:	TUP or ISUP may be used in signalling between MSCs, depending on the network type between the MSCs. For further details on the TUP and ISUP procedures refer to the following CCITT Recommendations:

Q.721─725 ─ Telephone User Part (TUP);

Q.761─764 ─ ISDN User Part (ISUP).

Figure 18.2/1: Procedure for retrieval of routing information

It is assumed that signalling towards the fixed network uses the messages IAM (Initial Address Message) and REL (Release). Examples of other interworking cases are not shown in this standard (for further information see CCITT Recommendation Q.699).

The following MAP services are used to retrieve routing information:

MAP_SEND_ROUTING_INFORMATION	see section 8;

MAP_PROVIDE_ROAMING_NUMBER	see section 8;

MAP_RESTORE_DATA		see section 6.

18.2.2	Procedures in the MSC

The procedure is described in figure 18.2/2. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Receive_Open_Cnf		see section 21.1;

Check_Confirmation 		see section 21.2.

NOTE:	If CUG Information is provided in the Initial Address Message (I_IAM) and the PLMN does not provide a CUG capability, the call will be released if the CUG does not have Outgoing Access. In this case the ISUP Cause indicator will be set to #87 (user not member of CUG) and a Release message (I_REL) returned to the originating network. If the CUG has Outgoing Access the call will be treated as an ordinary call, in this case the CUG Information in the Initial Address Message may need to be modified. See Annex E of CCITT Recommendation Q.767 (Approved 1990) for a description of the procedures for a CUG call interworking with a network which does not support the CUG facility. It has been assumed that these checks will be performed by the Gateway MSC prior to invoking MAP procedures.

Successful Outcome

The Gateway MSC will receive an Initial Address Message (I─IAM) indicating an incoming call. To be able to route the call toward the mobile subscriber, via the appropriate MSC, the Gateway MSC will initiate a MAP─SEND─ROUTING─INFORMATION service request to the HLR of the MS to receive the call.

If a routing address can be provided by the HLR, it is returned to the MSC in the MAP─SEND─ROUTING-INFORMATION service response. The routing address can take two forms: MSRN or forwarded─to-number. If a forwarded─to─number is provided, the Gateway MSC will increment the forwarding counter. The Gateway MSC will then construct an Initial Address Message (I─IAM) and send this to the MSC of the area where the MS is currently located. The procedure in the Gateway MSC is then terminated.

Data or Parameter Errors

If any parameter or data errors are detected by the MSC, the procedure for this call will be terminated and a release message (I_REL) returned to the network to release the call.

Service Provider Failure

Once the dialogue between the Gateway MSC and the HLR is established a MAP_P_ABORT service indication may be received at any stage from the MAP Service Provider. This aborts the dialogue, see section 5 for a definition of the abort reasons.

The call will be released by the MSC sending a Release message (I_REL) to the Network. The procedure in the Gateway MSC is then terminated.

Call Released by Calling Subscriber

This may occur at any stage during the call set up procedure. Call release is indicated to the MSC by a Release Message (I_REL). The procedure in the Gateway MSC is then terminated. Any response subsequently received from the HLR will be discarded.

Routing Failure

Failure to provide routing information is reported by the HLR to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service confirm. The User Error parameter will indicate one of the following conditions:

MAP User Error

Unknown Subscriber	ISUP Error	=    1

Absent Subscriber	  "   "	=  18

Call Barred	  "   "	=  21

Forwarding Violation	  "   "	=  21

Number Changed		=  22

CUG Reject	  "   "	=  55

Teleservice/Bearer Service not provisioned		=  57

Facility Not Supported	  "   "	=  69

Data Missing	  "   "	=111

Unexpected Data Value	  "   "	=111

System Failure	  "   "	=111

NOTE:	A suggested mapping is shown above between the MAP User Error value and the ISUP Error Cause, this is only provided to aid understanding. CCITT Q.6xx-series of recommendations provide further guidance for the mapping of Error Cause values (specifically ISUP and TUP errors).

Table 18.2/1: Suggested mapping between ISUP and MAP Errors

Regardless of the reason for routing failure the Gateway MSC will construct a Release message (I_REL), selecting the appropriate Error Cause value, and return this to the network. The procedure in the Gateway MSC will then be terminated.

Figure 18.2/2 sheet 1: Process SRI_MSC

.G.C:\USER\MAP\MAP-470\EPS18\182_21.EPS;18.2 cm;23 cm;EPS

Figure 18.2/2 sheet 2: Process SRI_MSC

.G.C:\USER\MAP\MAP-470\EPS18\182_22.EPS;18.2 cm;23 cm;EPS

18.2.3	Procedures in the HLR

The procedure is shown in figure 18.2/3 and 18.2/4. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Receive_Open_Cnf		section 21.1;

Check_Confirmation		section 21.2;

Receive_Open_Ind		section 21.1;

Check_Indication			section 21.2.

Successful Outcome

When a MAP_SEND_ROUTING_INFORMATION service indication is received from the Gateway MSC, the HLR will first check for parameter or data errors. If the MS is known to the HLR, the HLR will then check if incoming calls are barred for this MS.

The HLR will then proceed with a subscription check if the Network Signal Information is provided.

If CUG Interlock and optionally, CUG Outgoing Access, information is provided by the Gateway MSC and/or the called subscriber is a CUG subscriber the HLR will then perform the mobile terminated CUG call authorisation check. The purpose of this check is to identify a match between calling and called party CUG attributes, whilst enforcing intra_CUG communication restrictions. The procedures for authorising a CUG call is described in more detail in GSM 03.85.

If Call Forwarding Unconditional service is active and the call has not been forwarded more times than allowed, and a CUG call authorisation check is not required the HLR will increment the forwarding counter and provide the routing information for forwarding this call. This information is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

If Call Forwarding Unconditional service is active and CUG information is provided and/or the forwarding subscriber is a CUG subscriber then the HLR will perform a CUG call authorisation check on the forwarding party's outgoing access. The procedures for authorising a CUG check with Call Forwarding interaction are described in more detail in GSM 03.85. On successful completion of the authorisation check, the HLR will increment the forwarding counter and provide the routing information for forwarding this call. The new CUG Information (i.e. output of the CUG call authorisation check) to be used on the forwarding leg of the call will also be provided. This information is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

If the MS is reachable, i.e. if the MS is not purged, the VLR is not restricted and the HLR has a VLR number for the MS, the HLR requests the current VLR to provide a roaming number to be used for further call routing. The HLR will request this information from the VLR by initiating the MAP_PROVIDE_ROAMING_NUMBER service. If this operation was successful, the MSRN is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response. The procedure is then terminated in the HLR.

Parameter or Data Error

If the HLR detects any parameter or data errors in the MAP_SEND_ROUTING_INFORMATION service indication, the User Error "Data Missing" or "Unexpected Data Value" is returned to the Gateway MSC. If the error is detected in the response to the HLR request for a roaming number (MAP_PROVIDE_ROAMING_NUMBER), the User Error "System Failure" is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response.

Service Provider Failure

A MAP_P_Abort service indication may be received from the MAP Service Provider. This will abort the dialogue, see section 5 for a definition of the abort reasons. Depending on the reason for the abort and where the abort originated, the associated dialogue with the MSC may be terminated. Otherwise the procedure in the HLR is terminated.

Call Released by Calling Subscriber

The call may be released by the calling subscriber at any stage during the incoming call set up procedure. The HLR will be informed of this event by the Gateway MSC sending a MAP_U_ABORT service indication. The HLR will then terminate all procedures associated with this call.

MS Not Known

If the MS is not known to the HLR, the User Error Unknown Subscriber is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

Number Changed

If the number of the subscriber is changed inside the PLMN, the User Error "Number Changed" is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

Calls Barred

If the MS is barred from receiving incoming calls, the User Error "Call Barred" is returned to the Gateway MSC. The MS may be barred from receiving a call due to Operator Determined Barring (as defined in TS GSM 02.41) in which case, the Call Barring cause "Operator Barring" is returned to the Gateway MSC. The User Error is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

Subscription Check Failure

If Network Signal Information is provided, the HLR will perform a service subscription check on this information. If this check fails, depending on the reason for failure, the User Error "Bearer Service Not Provisioned" or "Teleservice Not Provisioned" is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

CUG Reject

If the call fails the CUG call authorisation check (this applies to incoming access check and outgoing access if the call has been forwarded - see TS GSM 03.85 for conditions), the User Error "CUG Reject" is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

MS Not Reachable

The MS may not be reachable for the following reasons:

-	there is no location information available for the MS;

-	the MSC Area Restricted Flag is set, indicating that roaming in the MSC area is not allowed;

-	the "MS purged" flag is set, indicating that the MS data have been purged from the VLR;

-	the Provide Roaming Number procedure fails and returns the error Absent Subscriber;

-	the Provide Roaming Number procedure fails and returns the error "No Roaming Number Available".

The HLR will then check if the service Call Forwarding on MS Not Reachable is active. If the service is not active the appropriate User Error ("System Failure" if the VLR returned "No Roaming Number Available", otherwise "Absent Subscriber") is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

If the Call Forwarding on MS Not Reachable service is active, the call has not been forwarded more times than allowed and a CUG call authorisation check is not required, the HLR will increment the forwarding counter and provide the routing information for forwarding this call.

If the Call Forwarding on MS Not Reachable service is active and CUG information is provided and/or the forwarding subscriber is a CUG subscriber then the HLR will perform a CUG call authorisation check on the forwarding party"s outgoing access. The procedures for authorising a CUG check with Call Forwarding interaction are described in more detail in TS GSM 03.85. On successful completion of the authorisation check, the HLR will increment the forwarding counter and provide the routing information for forwarding this call. The new CUG Information (i.e. output of the CUG call authorisation check) to be used on the forwarding leg of the call will also be provided.

This forwarding information is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

Forwarding Violation

If a Call Forwarding Service is active, but the call has already been forwarded as many times as allowed, the User Error "Forwarding Violation" is returned to the Gateway MSC. This is contained in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

No Roaming Number Allocated

The HLR request to the VLR to provide a roaming number may fail for a number of reasons (see section 8.4). If the reason is "Facility Not Supported", the User Error is set to "Facility Not Supported"; otherwise, if the reason for failure is anything other than "Absent Subscriber" or "No Roaming Number Available", the User Error is set to "System Failure". The error is returned to the Gateway MSC in the MAP_SEND_ROUTING_INFORMATION service response. The procedure in the HLR is then terminated.

If the reason for failure is "Absent Subscriber" or "No Roaming Number Available", then the procedure continues as described in the MS Not Reachable section above.

Figure 18.2/3 sheet 1: Process SRI_HLR

.G.C:\USER\MAP\MAP-470\EPS18\182_31.EPS;18.2 cm;23 cm;EPS

Figure 18.2/3 sheet 2: Process SRI_HLR

.G.C:\USER\MAP\MAP-470\EPS18\182_32.EPS;18.2 cm;23 cm;EPS

Figure 18.2/3 sheet 3: Process SRI_HLR

.G.C:\USER\MAP\MAP-470\EPS18\182_33.EPS;18.2 cm;23 cm;EPS

Figure 18.2/4: Macro IC_CUG_AUTH_CHECK

.G.C:\USER\MAP\MAP-470\EPS18\182_4.EPS;18.2 cm;23 cm;EPS

18.2.4	Procedures in the VLR

The procedures are described in figures 18.2/5, 18.2/6 and 18.2/7. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Receive_Open_Cnf		section 21.1;

Check_Confirmation		section 21.2;

Receive_Open_Ind		section 21.1;

Check_Indication			section 21.2;

Obtain_Authent_Sets_VLR		section 21.5;

Insert_Subs_Data_VLR		section 21.7.

Successful Outcome

On reception of the MAP_PROVIDE_ROAMING_NUMBER service indication from the HLR, the VLR will first perform a service provision check to confirm that the service requested is supported by the PLMN.

If the service is provided and the MS is also known, the VLR will check the IMSI detached flag for the MS. If the IMSI detached flag was not set then a MSRN will be allocated. A MSC number will already be stored in the VLR as well as the HLR.

The MSC number provided in the MAP_PROVIDE_ROAMING_NUMBER service indication will be used with the RAD_CONF_IND for the creation of the MSRN. That is, if the HLR provided MSC number does not match the MSC number stored against the concerned IMSI the Radio Confirmed Indicator (RAD_CONF_IND) should be checked to establish if the number has been confirmed by radio contact.

If the RAD_CONF_IND is set to "NOT_CONF", this means that the MSC number is out of date as it has not been confirmed by radio contact, therefore HLR provided MSC number can be used for creation of the MSRN.

If the RAD_CONF_IND is set to "CONF", this means that the MSC number has already been confirmed by radio contact and therefore, is more likely to be correct that the MSC number held by the HLR. The VLR held MSC number can be used for creation of the MSRN.

If no IMSI record exists for the IMSI received in MAP_PROVIDE_ROAMING_NUMBER service indication or if no MSC number is available in the concerned IMSI record, the VLR will use the HLR provided MSC number for creation of MSRN.

NOTE:	MAP version 1 described 2 options for allocating MSRN by the VLR, either at location updating or per terminating call set up. MAP Version 2 only supports one of these options, that of allocating the MSRN per terminating call set-up. The option to allocate MSRN at location updating has been deleted.

The roaming number is returned to the HLR in the MAP_PROVIDE_ROAMING_NUMBER service response. The VLR associates the MSRN with a service indication, if such an indication was contained in the roaming number request from the HLR. This roaming number will be used to set-up the call to the servicing MSC. The procedure is then terminated in the VLR.

If the indicator Confirm by HLR (HLR_CONF_IND) is set to not confirmed, the VLR will also need to request authentication and subscriber data. This is described in the procedure for MS Not Known, below.

Parameter or Data Errors

If any parameter or data errors are detected in the MAP_PROVIDE_ROAMING_NUMBER service indication, the VLR will return "Unexpected Data Value" in the MAP_PROVIDE_ROAMING_NUMBER service response. The procedure is then terminated in the VLR.

Called Subscriber Not Reachable

If the called subscriber is not reachable, either because the IMSI Detach Flag is set or because the called subscriber is not allowed to roam into the current location area, the error "Absent Subscriber" is returned to the HLR. This procedure is then terminated in the VLR.

No Roaming Number Available

If the VLR cannot assign a Roaming Number, the User Error "No Roaming Number Available" is returned to the HLR in the MAP_PROVIDE_ROAMING_NUMBER service response. The procedure is then terminated in the VLR.

Service Not Supported

If the Service Provision check shows that the service is not provided, the error "Facility Not Supported" is returned to the HLR in the MAP_PROVIDE_ROAMING_NUMBER service response. The procedure is then terminated in the VLR.

MS not Known

If the MS is not known by the VLR, this indicates that either a VLR restart has occurred without restoring the back-up after a VLR failure, the MS registers in the VLR in the time period between the last VLR back-up update and the VLR failure or the HLR is in restoration phase and the VLR addressed by the HLR is not the one where the called MS is currently located. The VLR will still proceed to allocate a MSRN (if available) and LMS identity. In addition, a skeleton IMSI record for this MS will be created and the indicators HLR Confirmation and Radio Confirmation set to not confirmed. If the VLR serves two or more MSCs, the VLR will also set the restoration indicator Location Information Confirmed in HLR (LOC_CONF_IND) to not confirmed. These flags will be set to confirmed when a procedure with the relevant entity has been successfully completed. The roaming number is then returned to the HLR in the MAP_PROVIDE_ROAMING_NUMBER service response.

To be able to process the MAP_SEND_INFO_FOR_INCOMING_CALL indication later, the VLR will request separately, for the concerned IMSI, the following:

-	authentication data with a MAP_SEND_AUTHENTICATION_INFO (if authentication is to be performed), this procedure is described in section 21;

-	subscriber data with a MAP_RESTORE_DATA request, this procedure is described in the section below.

These additional procedures are run independently of the provide roaming number procedure, no result is expected in this procedure.

VLR Restoration

The restore data procedure is invoked by the VLR to update subscriber data in the HLR and VLR after VLR restart, when the VLR receives a MAP_PROVIDE_ROAMING_NUMBER indication from the HLR for an unidentified MS, or for a known MS with the indicator "not confirmed by HLR" set. The procedure in the VLR is described in figure 18.2/6.

If the dialogue with the HLR is opened successfully, the process waits for one or more MAP_INSERT_SUBSCRIBER_DATA indication primitives containing the subscriber data. When all the subscriber data have been received, the dialogue with the HLR is terminated by a MAP_RESTORE_DATA confirmation. If the confirmation contains no errors, the VLR stores the subscriber data in the IMSI record, updates the HLR number and the MS Not Reachable Flag which is used for SMS (if appropriate), and sets the indicator "Confirmed by HLR" to "Confirmed". The status of the indicators "Confirmed by by radio contact" and "Location Information Confirmed in HLR" remains unchanged.

The procedure in the VLR is then terminated. For a description of the procedures in the HLR refer to section 16.3.

Figure 18.2/5 sheet 1: Process PRN_VLR

.G.C:\USER\MAP\MAP-470\EPS18\182_51.EPS;18.2 cm;23 cm;EPS

Figure 18.2/5 sheet 2: Process PRN_VLR

.G.C:\USER\MAP\MAP-470\EPS18\182_52.EPS;18.2 cm;23 cm;EPS

Figure 18.2/5 sheet 3: Process PRN_VLR

.G.C:\USER\MAP\MAP-470\EPS18\182_53.EPS;18.2 cm;23 cm;EPS

Figure 18.2/6: Process RESTORE_DATA_VLR

.G.C:\USER\MAP\MAP-470\EPS18\182_6.EPS;18.2 cm;23 cm;EPS

18.3	MS terminating calls

18.3.1	General

The procedure is shown in figure 18.3/1, this is an example sequence which assumes successful operation. It is also assumed that the location of the subscriber has already been determined.

Gateway

┌───┐                        ┌───┐                          ┌───┐

│MSC├────────────────────────┤MSC├──────────────────────────┤VLR│

└─┬─┘  I_IAM (NOTE 1)        └─┬─┘                          └─┬─┘

├───────────────────────────>│MAP_SEND_INFO_FOR_INCOMING_   │

│  ┌───┐        ┌───┐        │                 CALL         │

│  │MS ├────────┤BSS├────────┼─────────────────────────────>│

│  └─┬─┘(NOTE 3)└─┬─┘(NOTE 2)│MAP_PAGE or MAP_SEARCH_FOR_   │

│    │            │          │      MOBILE_SUBSCRIBER       │

│    │            │Paging    │<─────────────────────────────┤

│    │            │<─────────┘                              │

│    │            │Layer 3 info                             │

│    │            ├─────────>│                              │

│    │            │          │  MAP_PROCESS_CALL_           │

│    │            │          │      WAITING                 │

│    │            │          │                              │

│    │            │          │MAP_PROCESS_ACCESS_REQUEST    │

│    │            │          ├─────────────────────────────>│

│    │            │          │ MAP_PROVIDE_IMSI           │

│    │            │          │ MAP_AUTHENTICATION         │

│    │            │          │ MAP_SET_CIPHERING_MODE      │

│    │            │          │  MAP_CHECK_IMEI              │

│    │            │          │                              │

│    │            │          │MAP_PROCESS_ACCESS_REQUEST ack│

│    │            │          │<─────────────────────────────┤

│    │            │          │ MAP_FORWARD_NEW_TMSI       │

│    │            │          │                              │

│    │            │          │                              │

│    │            │          │ MAP_COMPLETE_CALL            │

│    │            │          │<─────────────────────────────┤

│    │      A_SETUP          │                              │

│    │<───────────┼──────────┤                              │

│    │     A_CALL_CONF       │                              │

│    ├────────────┼─────────>│                              │

│    │            │A_ASS_REQ │                              │

│    │            │<─────────┤                              │

│    │            │A_ASS_COM │                              │

│    │            ├─────────>│                              │

│    │      A_ALERT          │                              │

│    ├────────────┼─────────>│                              │

│       I_ACM (NOTE 1)       │                              │

│<───────────────────────────┤                              │

│    │   A_CONN              │                              │

│    ├──────────────────────>│                              │

│    │     A_CONN_ACK        │                              │

│    │<──────────────────────┤                              │

│      I_ANM (NOTE 1)        │MAP_SEND_INFO_FOR_INCOMING_   │

│<───────────────────────────┤               CALL ack       │

│                            │<─────────────────────────────┤

│                            │                              │

<xxx> = Optional Procedure

Figure 18.3/1: Procedure for MS terminating call set-up

NOTES to figure 18.3/1:

NOTE 1:	TUP or ISUP may be used in signalling between MSCs, depending on the network type between the MSCs. For further details on the TUP and ISUP procedures refer to the following CCITT Recommendations:

Q.721-Q.725 - Telephone User Part (TUP);

Q.761-Q.764 - ISDN User Part (ISUP).

It is assumed that signalling towards the fixed network only uses 3 messages IAM (Initial Address Message), ACM (Address Complete Message) and ANM (Answer Message). Examples of other interworking cases are not shown in this standard (for further information see e.g. CCITT Recommendation Q.699).

NOTE 2:	The BSS and MSC message flow shown in this diagram is only an example of the interworking possibilities. For further details of the procedures between the MSC and BSS refer to GSM 08.08 (MSC to BSS Interface, Layer 3).

NOTE 3:	The MS and MSC message flow shown in this diagram is only an example of the interworking possibilities. For further details on the procedures between the MSC and MS refer to GSM 04.08 (Mobile Radio Interface Layer 3 Specification).

The following MAP services are used during MS terminating call set up:

MAP_SEND_INFO_FOR_INCOMING_CALL		see section 8;

MAP_PROCESS_ACCESS_REQUEST		see section 6;

MAP_PAGE				see section 6;

MAP_SEARCH_FOR_MS			see section 6;

MAP_COMPLETE_CALL			see section 8;

MAP_PROCESS_CALL_WAITING		see section 8;

MAP_PROVIDE_IMSI			see section 6;

MAP_AUTHENTICATION			see section 6;

MAP_SET_CIPHERING_MODE		see section 6;

MAP_FORWARD_NEW_TMSI			see section 6;

MAP_CHECK_IMEI			see section 6.

18.3.2	Procedures in the MSC

The procedures are shown in figures 18.3/2, 18.3/3 and 18.3/4. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Receive_Open_Cnf			section 21.1;

Check_Confirmation			section 21.2;

Check_Indication				section 21.2;

Process_Access_Request_MSC		section 21.4;

Page_MSC				section 21.3;

Search_For_MS_MSC			section 21.3.

Successful Outcome

The Gateway MSC will pass the IAM (Initial Address Message) to the appropriate MSC. The Gateway MSC knows the location of the called subscriber from the roaming number.

On receipt of an IAM the MSC will initiate the MAP_SEND_INFO_FOR_INCOMING_CALL service request to request from the VLR, the relevant parameters for the called subscriber to enable the call to be set up. The response to this from the VLR will be a request to the MSC to commence Paging. This request will either be in the form of a MAP_PAGE service indication or a MAP_SEARCH_FOR_MS service indication. If the MSC does not detect that the called MS is in a busy state (see GSM 02.01 for a definition of these states), the paging procedure will be activated.

Successful response to paging from the BSS will be complete layer 3 information as specified in GSM 08.08, this indicates that the addressed MS has responded to the paging. The MSC will then initiate a MAP_PROCESS_ACCESS_REQUEST service request to the VLR to initiate processing of a subscriber access to the network.

On successful completion of the Process Access Request procedure the VLR will send the required parameters for the call set up to proceed. This information will be contained in a MAP_COMPLETE_CALL service indication.

To start call establishment the MSC will send an A_SETUP message to the MS. Following compatibility checking the MS will respond with an A_CALL_CONF message. These messages and the procedures associated with them are described in more detail in GSM 04.08. To complete call establishment the following events occur:

-	channel assignment (refer GSM 08.08);

-	called user alerted (refer GSM 04.08), this will be followed by the MSC sending an Address Complete Message (I_ACM) to the Gateway MSC;

-	called user accepts the call, this will be followed by the MSC sending an Answer Message (I_ANM) to the Gateway MSC;

-	the VLR will close the dialogue with the MSC by returning a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the result parameter indicating success.

Parameter or Data Errors

If any parameter or data errors are detected by the MSC, the procedure for this call will be terminated and a release message (I_REL) returned to the gateway MSC to release the call.

Service Provider Failure

Once the dialogue between the MSC and the VLR is established a MAP_P_ABORT service indication may be received at any stage from the MAP Service Provider. This aborts the dialogue, see section 5 for a definition of the abort reasons.

The call will be released by the MSC sending a Release message (I_REL) to the Gateway MSC. If at the time the procedure is terminated the call has already been confirmed by the MS, i.e. the MS has responded to the set up message (A_SETUP) by returning a call confirmed message (A_CALL_CONF), then the resources associated with the mobile terminating side of the call should also be released (i.e. Mobile Station and Base Station resources). The procedure in the MSC is then terminated.

Call Released by Calling Subscriber

This may occur at any stage during the call set up procedure. Call release is indicated to the MSC by a Release Message (I_REL). The MSC will then terminate the procedure towards the VLR using the MAP_U_ABORT service. The User Reason will indicate Application Procedure Cancellation and the Diagnostic Information will indicate Call Release. If at the time the procedure is terminated the call has already been confirmed by the MS, i.e. the MS has responded to the set up message (A_SETUP) by returning a call confirmed message (A_CALL_CONF), then the resources associated with the mobile terminating side of the call should also be released (i.e. Mobile Station and Base Station resources). The procedure in the MSC is then terminated.

No Paging or Search Response

If the timer for paging expires, paging may be repeated n times. The number of successive paging attempts and the time intervals are a network dependent choice. If the paging fails the MSC informs the VLR in the MAP service response (either MAP_PAGE or MAP_SEARCH_FOR_MS) with the User Error set to Absent Subscriber. If the Call Forwarding on MS Not Reachable service is not active, the VLR will terminate the procedure by returning a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Impossible Call Completion. The MSC will then send a Release message (I_REL) to the Gateway MSC to release the call.

If Call Forwarding on MS Not Reachable is active, then the call will be forwarded as described below in the Call Forwarding section.

Called Subscriber Not Reachable

In response to the MAP_SEND_INFO_FOR_INCOMING_CALL service request the VLR may inform the MSC that the subscriber cannot be reached. If Call Forwarding on MS Not Reachable service is not active, this notification is contained in a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Absent Subscriber. To terminate the procedure, the MSC will send a Release message (I_REL) to the Gateway MSC to release the call.

If Call Forwarding on MS Not Reachable service is active, then the call will be forwarded as described below in the Call Forwarding section.

Access to Called Subscriber Not Possible

The VLR, in response to the request from the MSC to provide information for the called subscriber (MAP_SEND_INFO_FOR_INCOMING_CALL service request) may detect an error condition. This is reported in the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error indicating one of the following:

Unallocated Roaming Number;

Unknown Subscriber;

Unexpected Data Value;

Data Missing;

System Failure.

To terminate the procedure for this call the MSC will send a Release Message (I_REL) to the Gateway MSC to release the call.

Called Subscriber is Busy

Two scenarios need to be considered here:

1)	If, following the paging request from the VLR (either MAP_PAGE or MAP_SEARCH_FOR_MS service indication), the MSC has knowledge of the "busy" state of the subscriber (see GSM 02.01 for a definition of these states), the MSC will set the User Error to Busy Subscriber. Depending on which state is detected the Busy Subscriber error will indicate either More Calls Allowed or Network Determined User Busy (no more calls allowed). This error will be returned to the VLR in the Page response (either MAP_PAGE or MAP_SEARCH_FOR_MS service response).

Depending on the Busy Subscriber state, the VLR may request Call Waiting or Call Forwarding on Busy to be invoked if these services are active. The procedures are described in the sections below.

If neither Call Forwarding on Busy nor Call Waiting service is active, the VLR will terminate the procedure by returning a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Impossible Call Completion. The MSC will then send a Release message (I_REL) to the Gateway MSC to release the call.

2)	If the Called MS enters a busy state after the paging but before the Set up request is sent to the MS (A_SETUP), the MS may respond to a second set up request with either Call Confirmed message (A_CALL_CONF) User Busy (Error Cause 17) or a Release Complete message (A_REL_COM) also indicating User Busy. In either case the MSC will generate a response to the VLR, a MAP_COMPLETE_CALL service confirm, with the User Error indicating Busy Subscriber (User Determined User Busy).

If the Call Forwarding on Busy service is active, then the call will proceed as described in the Call Forwarding section below.

If Call Forwarding on Busy is not active, the VLR will terminate the procedure by returning a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Impossible Call Completion. The MSC will then send a Release message (I_REL) message to the Gateway MSC to release the call.

Call Rejected by Called Subscriber

Following the generation of a Set up message (A_SETUP) the called MS responds with a Release Complete Message (A_REL_COM) with an Error Cause indicating the nature of the error. For Error Cause #17 (User Busy) the handling of this cause is described in the Call Subscriber is Busy section above. For all other Error Causes the MSC will terminate the dialogue in the direction of the VLR sending a MAP_U_ABORT and then release the call sending a Release Message (I_REL) to the Gateway MSC. The procedure in the MSC is then terminated.

Assignment Failure

The BSS may fail to seize a radio channel for one of the following reasons:

-	Use of the terrestrial channel selected by the MSC is impossible;

NOTE:	It is possible another try for radio channel assignment can be attempted after reselection of a new terrestrial channel.

-	Radio Congestion.

The MSC will be informed of this failure by the Assignment Failure message (A_ASS_FAIL). No further action can be taken at this stage, other than return the error Radio Congestion to the VLR in the MAP_COMPLETE_CALL service response. If the VLR confirms that the Call Forwarding on MS Not Reachable service is active, then the call will be forwarded as described below in the Call Forwarding section. Otherwise the VLR will instruct the MSC to terminate the procedure and release the call. The MSC releases the call by sending a Disconnect message (A_DISC) to the MS and a Release message (I_REL) to the Gateway MSC.

No Answer from Called Subscriber

Following the generation of Address Complete Message (I_ACM) there is no response from the Called MS, i.e. the Connect message (A_CONN) is not received within the given time period.

The MSC will generate a response to the VLR, a MAP_COMPLETE_CALL service confirm, with the User Error indicating No Subscriber Reply. If the Call Forwarding on No Reply service is active then the call will proceed as described in the Call Forwarding section below. If the service is not active, the VLR will terminate the procedure by returning a MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Impossible Call completion. The MSC will then send a Release message (I_REL) to the Gateway MSC to release the call.

Call Forwarding

If the VLR is informed that it is not possible to complete the call set up (e.g. absent subscriber, busy subscriber, No Subscriber Reply, Radio Congestion), the VLR will check if any of the appropriate Call Forwarding services are active. If the appropriate Call Forwarding service is active, and the call has not been forwarded more times than allowed, the VLR will provide the relevant forwarding data in the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm. To terminate the procedure, the MSC will set the Redirection Information in the Initial Address message (I_IAM) from the VLR supplied data. If the call is a CUG call, then the CUG Information supplied by the VLR should replace the CUG Information provided in the original Initial Address Message (I_IAM). The reason for this is because it is possible that, as a result of the CUG Call Authorisation Check performed by the VLR, the CUG Information to be provided in the forwarding leg of the call may be different (see GSM 03.85 for details). This Initial Address Message will then be sent to the Gateway MSC for the call to be forwarded.

If the call has already been forwarded as many times as allowed the VLR will return MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set to Forwarding violation. In this instance the MSC will terminate the procedure by sending a Release message (I_REL) to the calling subscriber to release the call.

Call Waiting

If the VLR is informed that it is not possible to complete the call set up because the subscriber is busy, the VLR will check if the Call Waiting service is active. If the Service is active, the VLR will instruct the MSC to commence Call Waiting by sending a set up message to the MS (A_SETUP), this indicates to the MS that another call is waiting. One of the following events will then occur:

-	T1 Timer Expiry

Timer T1 (this is the network timer controlling time between sending of A_SETUP message and receipt of an acknowledge from the MS) times out. The MSC will release the call in the direction of the Calling Subscriber by sending a release message (I_REL) with the cause value set to User Busy (#17), to the Gateway MSC. The Call Waiting procedure in the MSC is then terminated and, as the call has already been released, no error is reported to the VLR. To terminate the Call Handling procedures for this call, the VLR will return to the MSC the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm indicating success.

-	Call Confirmed

The MS responds to the A_SETUP message with a Call Confirmed message (A_CALL_CONF), the cause will indicate User Busy (#17). Once this is received channel assignment (if required) is performed. In the case of assignment failure, the MSC will release the call in the direction of the Calling Subscriber by sending a release message (I_REL) with the cause value set to User Busy (#17), to the Gateway MSC. The Call Waiting procedure in the MSC is then terminated and, as the call has already been released, no error is reported to the VLR. To terminate the Call Handling procedures for this call, the VLR will return to the MSC the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm indicating success.

The call confirm message from the MS will be followed by an Alerting message (A_ALERT) from the MS, this indicates that called user Alerting has commenced. Failure to receive and Alerting message will result in termination of the Call Waiting procedure as described for Assignment Failure in the previous paragraph.

-	Disconnect

The called MS may decide to reject the call, this is indicated to the MSC by the Disconnect Message (A_DISC) with the Error Cause set to #17 (User Busy). In this instance the MSC will terminate the Call Waiting procedure by returning the error Busy Subscriber (User Determined User Busy) to the VLR in the MAP_PROCESS_CALL_WAITING service response. Also, any other error cause returned in the Disconnect message will be handled in the same way.

If the Call Waiting is to proceed, i.e. the call has been confirmed and the Alerting message has been received from the called MS, then one of the following events will occur:

-	Called MS accepts the new call

The called MS may elect to either place the existing call on hold or release the existing call, alternatively the A subscriber may release the call. Regardless of the scenario invoked to free the existing call, acceptance of the call waiting is signalled by the Called MS sending a Connect message (A_CONN) to the MSC. To acknowledge the connection the MSC will respond with a connect acknowledge message (A_CONN_ACK). The MSC also informs the calling user that the call has been answered by returning an Answer message (I_ANM). The Call Waiting procedure has now been completed successfully. As no error is reported, the VLR will terminate the Call Set up procedure for this call by returning MAP_SEND_INFO_FOR_INCOMING_CALL service confirm indicating success.

-	CWT Timer expiry

Timer CWT (this is the network timer controlling the time between receipt of the A_ALERT message and receipt of the A_CONN during waiting call) times out. If a No Reply Condition Timer (NRCT) value is not supplied by the VLR in the Call Waiting request then the procedure can be terminated by the MSC sending a Disconnect message (A_DISC) to the MS. The MSC will release the call in the direction of the calling subscriber by sending a Release message (I_REL), with the cause value set to User Busy (#17), to the Gateway MSC. The Call Waiting procedure is then terminated and, as the call has already been released, no error is reported to the VLR. To terminate the Call Handling procedures for this call, the VLR returns to the MSC the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm indicating success.

If a No Reply Condition Timer value is supplied by the VLR, this indicates that the Call Forwarding on No Reply service is active. In this case the MSC will terminate the Call Waiting procedure by returning MAP_PROCESS_CALL_WAITING service response with the error value set to No Subscriber Reply and await instructions from the VLR for forwarding the call.

-	Disconnect

The Called MS may release the new call, this will be indicated by a Disconnect message (A_DISC). The Call Waiting procedure is terminated and the error Busy Subscriber (User Determined User Busy) is returned to the VLR in the MAP_PROCESS_CALL_WAITING service confirm.

Alerting Failure

A Call Confirmed message (A_CALL_CONF) has been received from the MS but an Alerting message (A_ALERT) has not been received from the MS. The timer associated with the alerting procedure has expired. The MSC will then release the call by sending a Release message (I_REL) to the calling subscriber. The resources associated with the mobile terminating side of the call should also be released (i.e. Mobile Station and Base Station resources). To terminate the procedure in the VLR a MAP_U_ABORT indication is sent. The call handling procedures are then terminated in the MSC for this call.

Access Request Failure

The Access Request procedure may fail for several reasons, either the access request is not accepted (i.e. the subscriber is not recognised or roaming is not allowed) or one of the associated procedures fails (i.e. Authentication, Ciphering, IMEI Check or TMSI reallocation). Failure of this procedure is reported to the MSC in the MAP_PROCESS_ACCESS_REQUEST service confirm (see section 6.3 for a definition of the possible User Errors). The VLR will terminate the call set up procedure by returning MAP_SEND_INFO_FOR_INCOMING_CALL service confirm with the User Error set either to System Failure or Absent Subscriber. The MSC will then terminate the procedure by sending a Release Message (I_REL) to the calling subscriber to release the call.

CUG Failure

If the VLR cannot convert the CUG Interlock Code into an Index the User Error CUG Reject is returned to the MSC in the MAP_SEND_INFO_FOR_INCOMING_CALL service confirm. In this instance the MSC will terminate the procedure by sending a Release Message (I_REL) to the Gateway MSC, with the ISUP Cause Indicator set to the appropriate value. The mapping between the GSM and ISUP reject causes are defined in GSM 09 series.

Figure 18.3/2 sheet 1: Process ICS_MSC

.G.C:\USER\MAP\MAP-470\EPS18\183_21.EPS;18.2 cm;23 cm;EPS

Figure 18.3/2 sheet 2: Process ICS_MSC

.G.C:\USER\MAP\MAP-470\EPS18\183_22.EPS;18.2 cm;23 cm;EPS

Figure 18.3/3 sheet 1: Macro COMPLETE_CALL_IN

.G.C:\USER\MAP\MAP-470\EPS18\183_31.EPS;18.2 cm;23 cm;EPS

Figure 18.3/3 sheet 2: Macro COMPLETE_CALL_IN

.G.C:\USER\MAP\MAP-470\EPS18\183_32.EPS;18.2 cm;23 cm;EPS

Figure 18.3/4 sheet 1: Macro PROCESS_CALL_WAITING

.G.C:\USER\MAP\MAP-470\EPS18\183_41.EPS;18.2 cm;23 cm;EPS

Figure 18.3/4 sheet 2: Macro PROCESS_CALL_WAITING

.G.C:\USER\MAP\MAP-470\EPS18\183_42.EPS;18.2 cm;23 cm;EPS

Figure 18.3/4 sheet 3: Macro PROCESS_CALL_WAITING

.G.C:\USER\MAP\MAP-470\EPS18\183_43.EPS;18.2 cm;23 cm;EPS

18.3.3	Procedures in the VLR

The procedure is shown in figures 18.3/5, 18.3/6 and 18.2/4. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Receive_Open_Cnf		section 21.1;

Check_Confirmation		section 21.2;

Check_Indication			section 21.2;

Process_Access_Request_VLR		section 21.4.

Successful Outcome

When the MAP_SEND_INFO_FOR_INCOMING_CALL service indication is received from the MSC, the VLR will first check the parameters and data for errors. If there are no errors the VLR will check to see if the MS is known to it, the MS will be identified by MSRN. If the MS is known, the Indicator Confirmed by HLR (HLR_CONF_IND) is set to confirmed and the IMSI detached flag is not set, the VLR will initiate paging procedures to locate the MS.

If the VLR holds location information for the MS, the service MAP_PAGE is invoked, containing the IMSI of the subscriber, the Stored Location Area Id and the subscribers TMSI (if available in the VLR). The VLR then waits for the MAP_PROCESS_ACCESS_REQUEST service request, which is confirmation that paging was successful.

If no location information is available in the VLR, e.g. due to previous VLR recovery, the service MAP_SEARCH_FOR_MS is invoked, this request will just contain the IMSI of the subscriber. The VLR then waits for the MAP_SEARCH_FOR_MS service confirm, in the case of success this will contain the Current Location Area Id of the subscribers actual location, causing updating of the Location Area Information in the VLR. In the case of error only an error cause is returned.

Completion of the Paging procedures is followed by the MAP_PROCESS_ACCESS_REQUEST service indication to the VLR, this is to initiate subscriber access to the network for the called subscriber. On receipt of this indication the VLR will commence the Access Request procedure (this is described in section 21). On successful completion of the Process Access Request procedure, the MAP_PROCESS_ACCESS_REQUEST service indication is confirmed to the MSC.

If CUG Interlock code is present (and optionally CUG Outgoing Access indicator) in the MAP_SEND_INFO_FOR_INCOMING_CALL service indication, the VLR will translate the CUG Interlock code to the CUG Index (see GSM 03.85 for the rules regarding the provision of CUG Index).

In addition, if the Call Forwarding on No Reply service is active the NRCT value will be supplied along with

The other parameters for call set up will be sent to the MSC using MAP_COMPLETE_CALL service request. There will be no response from the MSC for successful completion of Complete Call. If no errors are reported by the MSC the VLR will set the result parameter of MAP_SEND_INFO_FOR_INCOMING_CALL service response to success and return this to the MSC. This terminates the procedures in the VLR.

Data or Parameter Errors

If any parameter or data errors are detected by the VLR during the MS terminating call set up procedures, the MAP service User Error is set (either Data Error or Unexpected Data Value) and returned. The procedure is then terminated in the VLR.

Service Provider Failure

Once a dialogue between the MSC and VLR is established, a MAP_P_ABORT service indication may be received, at any stage, from the MAP Service Provider. This will abort the dialogue (see section 5 for a definition of the abort reasons) and the procedure in the VLR terminated.

Call Released by Calling Subscriber

The call may be released by the calling subscriber at any stage during the MS terminating call set up procedure. The VLR will be informed of this event by the MSC sending a MAP_U_ABORT service indication. The VLR will then terminate all procedures associated with this call, this may include sending a MAP_U_ABORT to the HLR if CUG information has been requested.

MS Not Confirmed by Radio Contact

If the MSRN is known to the VLR but the indicator Confirmed by HLR (HLR_CONF_IND) is set to Not Confirmed, the User Error will be set to System Failure and the Result parameter set to Error. This information is returned to the MSC in the MAP_SEND_INFO_FOR_INCOMING_CALL and the procedure in the VLR terminated.

Called Subscriber Not Reachable

If the called subscriber is not reachable, either because the IMSI Detached Flag is set or because the called subscriber is not allowed to roam into the current location area, the VLR will check if the call forwarding on MS not reachable service is active. If the service is active and the call has not been forwarded as many times as allowed, the forwarding address is returned to the MSC in the MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If the call forwarding service is not active the User Error is set to Absent Subscriber and this is returned to the MSC in the MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

Forwarding Violation

If a call forwarding service is active but the call has already been forwarded as many times as allowed the MAP_SEND_INFO_FOR_INCOMING_CALL service response User Error will be set to Forwarding Violation. The result parameter is also set to Error. This information is returned to the MSC and the procedure in the VLR is terminated.

Access Request Failure

The Access Request procedure may fail for several reasons, either the access request is not accepted (i.e. subscriber is not recognised or roaming is not allowed) or one of the associated procedures fails (i.e. Authentication, Ciphering, IMEI Check or TMSI reallocation). The VLR will then terminate the procedure for this call by setting the MAP_SEND_INFO_FOR_INCOMING_CALL service parameter User Error to System Failure or Absent Subscriber (depending on the error returned by the Access Request Procedure) and the Result parameter to Error. This is then returned to the MSC and the procedure in the VLR terminated.

Paging or Call Set up Failure

These procedures may fail for a number of reasons:

1)	Absent Subscriber

If either procedure fails due to absent subscriber the VLR will then determine whether the Call Forwarding on MS Not Reachable service is active. If the service is active, the call has not been forwarded more times than allowed, and a CUG call authorisation check is not required, the VLR will provide the forwarding address to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If CUG information is provided and/or the forwarding subscriber is a CUG subscriber then the VLR will perform a CUG call authorisation check on the forwarding party's outgoing access. The procedures for authorising a CUG call with call forwarding interaction are described in more detail in GSM 03.85. On successful completion of the CUG call authorisation check the VLR will provide the forwarding address and the new CUG Information (i.e. output of the CUG Call Authorisation check) to be used on the forwarding leg of the call, to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If the Call Forwarding on MS Not Reachable service is not active, the MAP_SEND_INFO_FOR_INCOMING_CALL service User Error parameter is set to Absent Subscriber. This is then returned to the MSC and the procedure in the VLR terminated.

2)	Radio congestion or No Subscriber Reply

The failure Radio Congestion will only occur with the Call Set up procedure.

The VLR will determine if a Call Forwarding service is active (e.g. Call Forwarding on MS Not Reachable in the case of radio congestion and Call Forwarding on No Reply for No Subscriber Reply). If the appropriate Call Forwarding service is active, the call has not been forwarded more times than allowed, and a CUG authorisation check is not required, the VLR will provide the forwarding address to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If CUG information is provided and/or the forwarding subscriber is a CUG subscriber then the VLR will perform a CUG call authorisation check on the forwarding party's outgoing access. The procedures for authorising a CUG call with call forwarding interaction are described in more detail in GSM 03.85. On successful completion of the CUG call authorisation check the VLR will provide the forwarding address to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If a Call Forwarding service is not active, the MAP_SEND_INFO_FOR_INCOMING_CALL service User Error parameter is set to Impossible Call Completion. This is then returned to the MSC and the procedure in the VLR terminated.

3)	Busy Subscriber

Three scenarios need to be considered in this category of failure:

-	Network Determined User Busy (NDUB)

This condition occurs when a call is about to be offered and the information (ie, traffic) channel (Bm or Lm) is busy/not available and the maximum number of total calls has been reached. The value of the maximum number of calls is 1 for the basic call. When the supplementary service Call Waiting is applicable, the value is n+1, where n is the maximum number of calls that can be waiting (reference GSM 02.01).

If this error is reported by the MSC, the VLR will check if the Call Forwarding on Busy service is active. If the service is active, and the call has not been forwarded more times than allowed, and a CUG authorisation check is not required, the VLR will provide the forwarding address to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If CUG information is provided and/or the forwarding subscriber is a CUG subscriber then the VLR will perform a CUG call authorisation check on the forwarding party's outgoing access. The procedures for authorising a CUG call with call forwarding interaction are described in more detail in GSM 03.85. On successful completion of the CUG call authorisation check the VLR will provide the forwarding address to the MSC in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure is then terminated in the VLR.

If Call Forwarding on Busy is not active, the MAP_SEND_INFO_FOR_INCOMING_CALL service User Error is set to Impossible Call Completion. This is then returned to the MSC and the procedure in the VLR terminated.

-	More Calls Allowed

This condition occurs when a call is about to be offered and the called MS is already busy but the maximum number of total calls has not yet been reached. (reference GSM 02.01).

If this error is reported by the MSC, the VLR will first check if the Call Waiting service is active. If the service is not active the VLR will then check if the Call Forwarding on Busy service is active. If this service is also not active then the procedure in the VLR for this call will be terminated and MAP_SEND_INFO_FOR_INCOMING_CALL service response is returned to the MSC with the Error Cause set to Impossible Call Completion.

If the Call Waiting service is not active but the Call Forwarding on Busy service is active, then the procedure will continue as if the Network Determined User Busy error had been returned, see above.

If the Call Waiting service is active, the VLR will construct a Call Waiting request. If CUG Information is provided, the VLR will first translate the interlock code to the CUG Index. If there is no match, the MAP_SEND_INFO_FOR_INCOMING_CALL service response is returned to the MSC with the User Error set to System Failure. The procedure in the VLR is then terminated.

If there is a match, the VLR will then check if the Call Forwarding on No Reply service is active. If this service is active and the call has not been forwarded more times that allowed, the No Reply Condition Timer will be provided in the Call Waiting request (this indicates to the MSC that Call Forwarding on No Reply service is active). Should the call have already been forwarded as many times as allowed then a No Reply Condition Timer value will not be supplied.

On successful completion of the CUG and call forwarding checks, the VLR will instruct the MSC to commence Call Waiting by sending MAP_PROCESS_CALL_WAITING service request. If no errors are returned by the MSC, the VLR will return MAP_SEND_INFO_FOR_INCOMING_CALL service response, with the result set to Success. The procedure in the VLR is then terminated.

If Call Waiting was not completed successfully the MSC will respond with MAP_PROCESS_CALL_WAITING service confirm. If the User Error indicates Busy Subscriber, the procedure continues as described for Network Determined User Busy above. If the error No Subscriber Reply is returned, the procedure continues as described under No Subscriber Reply (item 2 above). All other errors returned by the MSC will result in the VLR returning Impossible Call Completion User Error in MAP_SEND_INFO_FOR_INCOMING_CALL service response. The procedure in the VLR is then terminated.

-	User Determined User Busy (UDUB)

This condition occurs when a call is offered to an MS and the MS responds "User Busy" because the subscribers resources (terminal or person using them) are busy (reference GSM 02.01). This busy condition is treated in exactly the same way as Network Determined User Busy, see above.

If the Busy Subscriber error cause does not indicate one of the above states then the VLR will treat this error as More Calls Allowed.

CUG Failure

Failure will be due either to the VLR being unable to convert the Interlock code into an Index, or the CUG Call Authorisation check for call forwarding interaction may not be successful. This will result in VLR terminating the procedure for this call by setting MAP_SEND_INFO_FOR_INCOMING_CALL service User Error to CUG Reject in the case of CUG authorisation failure, and System Failure for failure to convert the CUG Interlock Code to an Index. This is then sent to the MSC, the procedure in the VLR is then terminated.

Figure 18.3/5 sheet 1: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_51.EPS;18.2 cm;23 cm;EPS

Figure 18.3/5 sheet 2: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_52.EPS;18.2 cm;23 cm;EPS

Figure 18.3/5 sheet 3: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_53.EPS;18.2 cm;23 cm;EPS

Figure 18.3/5 sheet 4: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_54.EPS;18.2 cm;23 cm;EPS

Figure 18.3/5 sheet 5: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_55.EPS;18.2 cm;23 cm;EPS

Figure 18.3/5 sheet 6: Process ICS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_56.EPS;18.2 cm;23 cm;EPS

Figure 18.3/6: Macro SEARCH_VLR

.G.C:\USER\MAP\MAP-470\EPS18\183_6.EPS;18.2 cm;23 cm;EPS

18.4	MS originated calls

18.4.1	General

The call can be established in two ways in the GSM. Either with the early assignment or with the off air call set up (OACSU). The difference between these is that in the latter case the radio channel is reserved at the last possible moment, i.e. when the called subscriber answers. In this section only the procedure with the early assignment is shown. The procedure is shown in figure 18.4/1, this is an example sequence which assumes successful operation.

Gateway

┌───┐ NOTE 1 ┌───┐ NOTE 2 ┌───┐              ┌───┐     ┌───┐

│MS ├────────┤BSS├────────┤MSC├──────────────┤VLR├─────┤MSC│

└─┬─┘        └─┬─┘        └─┬─┘              └─┬─┘     └─┬─┘

│        A_CM_SER_REQ     │                  │         │

├────────────┼───────────>┤MAP_PROCESS_ACCESS│         │

│            │            │     _REQUEST     │         │

│            │            ├─────────────────>│         │

│            │            │ MAP_PROVIDE_IMSI │         │

│            │            │ MAP_AUTHENTICATE │         │

│            │            │ MAP_SET_         │         │

│            │            │ CIPHERING_MODE   │         │

│            │            │ MAP_CHECK_IMEI   │         │

│            │            │ MAP_OBTAIN_IMEI  │         │

│            │            │MAP_PROCESS_ACCESS│         │

│            │            │     _REQUEST ack │         │

│            │            ├<─────────────────┤         │

│            │            │ MAP_FORWARD_NEW_ │         │

│            │            │        TMSI      │         │

│    A_CM_SER_ACC (NOTE 4)│                  │         │

├<───────────┼────────────┤                  │         │

│         A_SETUP         │MAP_SEND_INFO_FOR_│         │

├────────────┼───────────>┤OUTGOING_CALL     │         │

│            │            ├─────────────────>│         │

│            │            │                  │         │

│            │            │ MAP_COMPLETE_    │         │

│            │            │   CALL           │         │

│      A_CALL_PROC        ├<─────────────────┤         │

├<───────────┼────────────┤                  │         │

│            │ A_ASS_REQ  │                  │         │

│            ├<───────────┤                  │         │

│            │ A_ASS_COMP │                  │         │

│            ├───────────>┤                  │         │

│            │            │          I_IAM  NOTE 3     │

│            │            ├───────────────────────────>│

│            │            │MAP_SEND_INFO_FOR_│         │

│            │            │OUTGOING_CALL ack │         │

│            │            ├<─────────────────┤         │

│            │            │                  │         │

xxx = Optional Procedure

Figure 18.4/1: Procedure for MS originating call set-up

NOTES to figure 18.4/1:

NOTE 1:	The BSS and MSC message flow shown in this diagram is only an example of the interworking possibilities. For further details of the procedures between the MSC and MS refer to GSM 04.08 (Mobile Radio Interface Layer 3 Specification).

NOTE 2:	The MS and MSC message flow shown in this diagram is only an example of the interworking possibilities. For further details of the procedures between the MSC and BSS refer to GSM 08.08 (Base Station to Switch Interface, Layer 3).

NOTE 3:	TUP and ISUP may be used in signalling between MSCs, depending on the network type between the MSCs. For further details on the TUP and ISUP procedures refer to the following CCITT Recommendations:

Q.721-Q.725  Telephone User Part (TUP);

Q.761-Q.764  ISDN User Part (ISUP).

It is assumed that signalling towards the fixed network only uses one message, IAM (Initial Address Message). Examples of other interworking cases are not shown in this standard (for further information see e.g. CCITT Recommendation Q.699).

NOTE 4:	Acknowledgement of A-CM-SER-REQ can also be performed by invocation of ciphering mode setting (A-CIPH-MOD-CMD).

The following MAP services are used during outgoing call set up:

MAP_PROCESS_ACCESS_REQUEST		see section 6;

MAP_SEND_INFO_FOR_OUTGOING_CALL	see section 8;

MAP_COMPLETE_CALL			see section 8;

MAP_PROVIDE_IMSI			see section 6;

MAP_SET_CIPHERING_MODE		see section 6;

MAP_AUTHENTICATE			see section 6;

MAP_FORWARD_NEW_TMSI			see section 6;

MAP_CHECK_IMEI			see section 6;

MAP_OBTAIN_IMEI			see section 6.

18.4.2	Procedures in the MSC

The procedure is described in figures 18.4/2, 18.4/3 and 18.4/4. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Check_Confirmation		section 21.2;

Process_Access_Request_MSC	section 21.4.

Successful Outcome

On receipt of an A_CM_SERV_REQ indicating an outgoing circuit switched call set up from the MS, the MSC will initiate a MAP_PROCESS_ACCESS_REQUEST service request to the VLR. The VLR will then invoke the Access Request procedures, this is described in section 21.

Once the MAP_PROCESS_ACCESS_REQUEST service indication is accepted the MS will then send to the MSC the call control information (e.g. called party address and bearer capability), this is contained in an A_SETUP message. From this information the MSC will construct a MAP_SEND_INFO_FOR_OUTGOING_CALL service request and send it to the VLR, to request the subscriber parameters for handling the call.

If the request was successful the VLR will respond with a MAP_COMPLETE_CALL service request. This is the instruction to the MSC to proceed with the call set up. To indicate to the calling MS that the call is being proceeded, the MSC will send an A_CALL_PROC message. This message and the procedures associated with it are described in more detail in GSM 04.08.

To complete the call set up procedures, the MSC will commence the connection establishment phase of the call by sending an Initial Address Message (I_IAM) to the Gateway MSC. If this call is to be treated as a CUG call, then the CUG information (Interlock Code and optionally the Outgoing Access indicator) provided in the MAP_COMPLETE_CALL service indication, should be used to construct the CUG information in the Initial Address Message (I_IAM). The reason for this, is because it is possible that the CUG Information returned by the VLR may be different to the CUG Information originally supplied by the MSC, due to the changes required by the CUG Call Authorisation check. The CUG Index may also be returned by the VLR, the presence of this parameter in the MAP_COMPLETE_CALL service indication is confirmation that an implicit CUG has been invoked. This information will not be passed onto the called user but as an operator option, this information may be returned to the Calling User. See GSM 03.85 for further details of the CUG options and checks.

Service Provider Failure

Once the dialogue between the MSC and the VLR is established a MAP_P_ABORT service indication may be received at any stage from the MAP Service Provider. This aborts the dialogue, see section 5 for a definition of the abort reason.

The MSC then clears the call by sending a Disconnect message (A_DISC) to the MS and a Clear Command (A_CLR_CMD) to the BSS to release the associated dedicated resource. The clear procedures are described in more detail in GSM 04.08 (MS) and 08.08 (BSS). The procedure in the MSC is then terminated.

Call Released by Calling Subscriber

The call may be released at any time by the calling subscriber, this is indicated to the MSC by the MS sending a Release Message (either A_REL, A_REL_COM or A_DISC, see GSM 04.08 for a more detailed description of the options available). To terminate the call set up procedures for this call the MSC will send a MAP_U_ABORT service request to the VLR, this will indicate a User Reason of Application Procedure Cancelled and a Diagnostic value of Call Released. The procedure in the MSC is then terminated.

Process Access Request Failure

The VLR may reject the Process Access Request, either the Access Request is not accepted or one of the associated procedures called by the Access Request procedure may fail (i.e. Authentication, Ciphering, IMEI check or TMSI reallocation). Regardless of the reason, the call will be cleared by the MSC rejecting the MS Service Request. This is indicated to the MS by the MSC sending a CM Service Reject message (A_CM_SERV_REJ). The procedure in the MSC is then terminated.

Calling Subscriber Barred

The calling subscriber may be barred from making outgoing calls or roaming (see GSM 02.41 for a description of the call barring conditions), the MSC is informed of this condition by the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service confirm, the User Error will be set to Call Barred with the barring cause indicating either Call Barred or Call Barred for Administrative Reasons. The MSC will instruct the MS to release the call by sending a Release Complete message (A_REL_COMP). The procedure in the MSC is then terminated.

Assignment Failure

The BSS may fail to seize a radio channel for one of the following reasons:

-	Use of the terrestrial channel selected by the MSC is impossible;

-	Radio Congestion.

The MSC will be informed of this failure by the Assignment Failure message (A_ASS_FAIL). The call is then released by the MSC sending a Disconnect message (A_DISC) to the MS to initiate the clear procedure.

The MSC will then inform the VLR of this failure by setting the MAP_COMPLETE_CALL service User Error to Radio Congestion. The VLR will return MAP_SEND_INFO_FOR_OUTGOING_CALL service response with the User Error set to System Failure. The procedure in the MSC is then terminated.

CUG Failure

The serving VLR will perform the outgoing call CUG authorisation, if the call fails this authorisation the User Error CUG Reject is returned by the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service response. The MSC will instruct the MS to release the call by sending a Release Complete message (A_REL_COMP). The procedure in the MSC is then terminated.

Figure 18.4/2 sheet 1: Process OCS_MSC

.G.C:\USER\MAP\MAP-470\EPS18\184_21.EPS;18.2 cm;23 cm;EPS

Figure 18.4/2 sheet 2: Process OCS_MSC

.G.C:\USER\MAP\MAP-470\EPS18\184_22.EPS;18.2 cm;23 cm;EPS

Figure 18.4/4: Macro COMPLETE_CALL_OUT

.G.C:\USER\MAP\MAP-470\EPS18\184_4.EPS;18.2 cm;23 cm;EPS

18.4.3	Procedures in the VLR

The procedure is shown in figures 18.4/5 and 18.4/6. These procedures make use of a number of processes and macros not described in this section. The description of these can be found in the following sections:

Check_Confirmation		section 21.2;

Process_Access_Request_VLR		section 21.4;

Receive_Open_Ind		section 21.1;

Check_Indication			section 21.2.

Successful Outcome

Once the procedures initiated by the MAP_PROCESS_ACCESS_REQUEST service request have been successfully completed, the MSC will then initiate a MAP_SEND_INFO_FOR_OUTGOING_CALL service request. This is to request the VLR to provide information to allow the MSC to handle an outgoing call set up request from a MS. The VLR will check the parameters and data for errors.

If a specific Bearer Service or Teleservice has been requested by the MS, the VLR will then perform the subscription check for the provision of the service. The VLR will also check the activation status of the supplementary services and the status of call barring services (i.e. barring of outgoing calls, conditional barring of outgoing calls or administrative reasons such as bad payer).

If the calling MS is part of a closed user group (CUG), the MSC will have passed any CUG information (received from the MS) to the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service request. The VLR will perform the authorisation checks on the call, this authorisation is determined by the information received from the MSC, the subscription information held by the VLR and the call scenario. It is possible that a CUG call may be implicitly invoked without CUG information being passed to the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service indication. In this instance, the VLR will just return the CUG_Index value to the MSC. The procedures for authorising a CUG call and the information to be returned to the serving MSC are described in more detail in GSM 03.85.

If the calling MS has passed all checks the VLR will instruct the MSC to set up the call by sending a MAP_COMPLETE_CALL service request to the MSC. This request may contain parameters related to the subscription, e.g. supplementary services and basic services. On successful completion of the MAP_COMPLETE_CALL service, this procedure is then terminated in the VLR.

Parameter or data errors

If the VLR detects any parameter or data errors in MAP_SEND_INFO_FOR_OUTGOING_CALL service indication from the MSC, the VLR will reject the service request. The MAP_SEND_INFO_FOR__OUTGOING_CALL service parameter User Error will be set to either Data Missing or Unexpected Data Value. This response is returned to the MSC and the procedures in the VLR will be terminated.

Service Provider Failure

Once a dialogue between the MSC and VLR or VLR and HLR is established, a MAP_P_ABORT service indication may be received, at any stage, from the MAP Service Provider. This will abort the dialogue, see section 5 for a definition of the abort reasons.

Call Released by Calling MS

The call may be released by the calling subscriber at any stage during the outgoing call set up procedure. The VLR will be informed of this event by the MSC sending a MAP_U_ABORT service indication. The VLR will then terminate all procedures associated with the call.

Process Access Request Failure

If any of this procedure fails (e.g. Authentication, IMEI Check, Ciphering or TMSI reallocation) the VLR will terminate the outgoing call set up procedure for this call. The MSC will have already been informed of the failure by the Process Access Request Procedure (see section 21).

Service Subscription Check Failure

If the Bearer Service or Teleservice requested by the MS for this call is not available. The VLR will set the MAP_SEND_INFO_FOR_OUTGOING_CALL service parameter User Error to either Teleservice Not Provisioned or Bearer Service Not Provisioned. This confirmation is then returned to the MSC and the procedures for this call terminated in the VLR.

Calls Barred

If calls for this MS are barred (e.g. all outgoing calls, conditional barring of outgoing calls, roaming barred or calls barred due to Operator Determined Barring as defined in GSM 02.41), the VLR will terminate outgoing call set up procedures. The MAP_SEND_INFO_FOR_OUTGOING_CALL service parameter User Error will be set to Call Barred, this is then returned to the MSC.

CUG Failure

If the call fails the CUG Authorisation check (the procedures for authorising an outgoing CUG call are defined in GSM 03.85), CUG failure will be reported back to the MSC, the MAP_SEND_INFO_FOR_OUTGOING__CALL service parameter User Error will be set to CUG Reject. Outgoing call set up procedures for this call will then be terminated in the VLR.

Call Set up Failure

If, following a MAP_COMPLETE_CALL service request from the VLR, the MSC fails to set up the call this will be reported as an error in MAP_COMPLETE_CALL service confirm. Regardless of the reason for failure, the VLR will set the User Error parameter of MAP_SEND_INFO_FOR_OUTGOING_CALL to System Failure. This service response is then returned to the MSC and the procedures in the VLR terminated.

Figure 18.4/5 sheet 1: Process OCS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\184_51.EPS;18.2 cm;23 cm;EPS

Figure 18.4/5 sheet 2: Process OCS_VLR

.G.C:\USER\MAP\MAP-470\EPS18\184_52.EPS;18.2 cm;23 cm;EPS

Figure 18.4/6: Macro OG_CUG_AUTH_CHECK

.G.C:\USER\MAP\MAP-470\EPS18\184_6.EPS;18.2 cm;23 cm;EPS

19	Supplementary services procedures

The following application contexts exist for handling of supplementary services:

-	accessUnstructuredSsContext;

-	accessFunctionalSsContext.

The accessUnstructuredSsContext refers to a simple MAP users, for which the corresponding MAP process can be identified by the MAP-Provider directly.

However, the accessFunctionalSsContext refers to a complex MAP-User consisting of several processes. For this user, a process co-ordinator is defined for each network entity, in order to identify the correct process to invoke. These processes open and validate the dialogue, then invoke the necessary operation-specific process. These processes are described below.

19.0.1	Functional Supplementary Service Process Co-ordinator for MSC

Upon receipt of a CM-Service request with CM-service type= SS, the MSC initiates the process access request procedure towards the VLR as described in section 21 of this specification.

Once a CM connection is established, the MSC can handle supplementary service indications from the MS. Table 19.0/1 shows the co-ordinating process' reaction on receipt of specific SS service indications on the air interface. After the relevant process is invoked, the received air interface service indication is sent to that process. The creation of service requests on the basis of air interface messages is described in GSM 09.11.

Service indication received		Process invoked

A_REGISTER_SS_ind		REGISTER_SS_MSC

A_ERASE_SS_ind		ERASE_SS_MSC

A_ACTIVATE_SS_ind		ACTIVATE_SS_MSC

A_DEACTIVATE_SS_ind		DEACTIVATE_SS_MSC

A_INTERROGATE_SS_ind		INTERROGATE_SS_MSC

A_REGISTER_PASSWORD		REGISTER_PASSWORD_MSC

Table 19.0/1: Relationship between received service indication and invoked process in the MSC

Figure 19.0/1 shows the co-ordinating process in the MSC.

Figure 19.0/1

.G.C:\USER\MAP\MAP-470\EPS19\119-01.EPS;18.2 cm;23 cm;EPS

19.0.2	Functional Supplementary Service Process Co-ordinator for VLR

Any functional SS process in the VLR starts by the VLR receiving the MAP_PROCESS_ACCESS_REQUEST indication. The VLR then acts as described in section 21 of this specification.

If the Process Access Request was successful, the VLR can handle supplementary service indications from the MSC. Table 19.0/2 shows the co-ordinating process' reaction on receipt of specific SS service indications from the MSC. After the relevant process is invoked, the received service indication is sent to that process, and the co-ordinating process terminates.

Service indication received		Process invoked

MAP_REGISTER_SS_ind		REGISTER_SS_VLR

MAP_ERASE_SS_ind		ERASE_SS_VLR

MAP_ACTIVATE_SS_ind		ACTIVATE_SS_VLR

MAP_DEACTIVATE_SS_ind		DEACTIVATE_SS_VLR

MAP_INTERROGATE_SS_ind	INTERROGATE_SS_VLR

MAP_REGISTER_PASSWORD	REGISTER_PASSWORD_VLR

Table 19.0/2: Relationship between received service indication and invoked process in the VLR

Figure 19.0/2 shows the co-ordinating process in the VLR.

Figure 19.0/2 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-02.EPS;18.2 cm;23 cm;EPS

Figure 19.0/2 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-02.EPS;18.2 cm;23 cm;EPS

19.0.3	Functional Supplementary Service Process Co-ordinator for HLR

Any functional SS process in the HLR starts by the HLR receiving a MAP-OPEN service indication. If that service is successful, the HLR can handle supplementary service indications from the VLR. Table 19.0/3 shows the co-ordinating process' reaction on receipt of specific SS service indications from the VLR. After the relevant process is invoked, the received service indication is sent to that process, and the co-ordinating process terminates.

Service indication received		Process invoked

MAP_REGISTER_SS_ind		REGISTER_SS_HLR

MAP_ERASE_SS_ind		ERASE_SS_HLR

MAP_ACTIVATE_SS_ind		ACTIVATE_SS_HLR

MAP_DEACTIVATE_SS_ind		DEACTIVATE_SS_HLR

MAP_INTERROGATE_SS_ind	INTERROGATE_SS_HLR

MAP_REGISTER_PASSWORD	REGISTER_PASSWORD_HLR

Table 19.0/3: Relationship between received service indication and invoked process in the HLR.

Figure 19.0/3 shows the co-ordinating process in the HLR.

Figure 19.0/3 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-03.EPS;18.2 cm;23 cm;EPS

Figure 19.0/3 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-03.EPS;18.2 cm;23 cm;EPS

19.1	Registration procedure

19.1.1	General

The registration procedure is used to register data related to a supplementary service in the HLR. The registration procedure is a fully transparent communication between the MS and the HLR, except that some services may be invoked as a result of the procedure, as described in the sections below.

The registration procedure is shown in figure 19.1.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_INSERT_SUBSCRIBER_DATA	(defined in sections 6 and 21);

MAP_REGISTER_SS		(defined in section 9).

┌────┐            ┌────┐      B       ┌────┐    D      ┌────┐

│ MS │------------│MSC │──────┼───────│VLR │────┼──────│HLR │

└────┘            └────┘              └────┘           └────┘

│                 │                    │                 │

│  A_CM_SERV_REQ  │                    │                 │

│---------------->│ MAP_PROCESS_ACC_REQ│                 │

│    (NOTE 1)     │───────────────────>...               │

│                 │    (NOTE 2)        │                 │

│                 │                    │                 │

│  A_REGISTER_SS  │  MAP_REGISTER_SS   │                 │

│---------------->│───────────────────>│MAP_REGISTER_SS  │

│                 │                    │────────────────>│

│                 │                    │                 │

│                 │MAP_REGISTER_SS ack │MAP_REGIS_SS ack │

│A_REGISTER_SS ack│<───────────────────│<────────────────│

│<----------------│                    │MAP_INS_SUBS_DATA│

│                 │                    │<────────────────│

│                 │                    │    (NOTE 3)     │

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Services printed in italics are optional.

Figure 19.1.1/1: Interfaces and services for supplementary service registration

19.1.2	Procedures in the MSC

Supplementary service Registration

The A_REGISTER_SS service indication received by the MAP user in the MSC contains the SS-code and any parameters that are related to the supplementary service.

The MAP user transfers the received information to the VLR in the MAP_REGISTER_SS request without checking the contents of the service indication. Rules for the mapping are described in GSM 09.11.

The MSC then awaits the receipt of the MAP_REGISTER_SS confirm from the VLR. The outcome of the procedure is reported to the MS in the A_REGISTER_SS response message as described in GSM 04.8x, 04.9x and 09.11. Finally the SS-connection is released.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in GSM 09.11).

Error handling

If at any time during the supplementary service part of this procedure a MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE or unexpected MAP_CLOSE indication is received from the VLR concerning the process, a CM_RELEASE_COMPLETE indication is sent to the MS (as specified in GSM 09.11). Upon receipt of a MAP_NOTICE indication from the VLR, the MSC must close the VLR dialogue by sending a MAP_CLOSE request. The process is then terminated.

If an A_CM_RELEASE indication is received from the MS, all open transactions shall be released using the MAP_U_ABORT request indicating application procedure cancellation, and the process is terminated.

The registration procedure in the MSC is shown in figure 19.1.2/1.

Figure 19.1.2/1

.G.C:\USER\MAP\MAP-470\EPS19\119-1-21.EPS;18.2 cm;23 cm;EPS

19.1.3	Procedures in the VLR

Supplementary service Registration

When receiving the MAP_REGISTER_SS indication, the MAP user in the VLR transfers the information to the HLR in the MAP_REGISTER_SS request without checking the contents of the service indication.

The VLR then awaits the receipt of the MAP_REGISTER_SS confirm from the HLR. The MAP user in the VLR shall transfer the information contained in this primitive to the MSC in the MAP_REGISTER_SS response without checking its contents.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in GSM 09.11).

Error handling

If at any time during this procedure a MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE or unexpected MAP_CLOSE indication is received from the MSC concerning the process, a MAP_U_ABORT request indicating application procedure cancellation is sent to the HLR (if a connection exists). If a MAP_NOTICE indication was received from the MSC, that dialogue must be closed by sending a MAP_CLOSE request towards the MSC. The process is terminated.

If a MAP_P_ABORT, MAP_U_ABORT or MAP_CLOSE indication is received from the HLR, a MAP_U_ABORT request shall be sent to the MSC terminating the process. If a MAP_NOTICE indication was received from the HLR, that dialogue must be closed by sending a MAP_CLOSE request towards the HLR. The process terminates.

The registration procedure in the VLR is shown in figure 19.1.3/1.

Figure 19.1.3/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-1-31.EPS;18.2 cm;23 cm;EPS

Figure 19.1.3/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-1-31.EPS;18.2 cm;23 cm;EPS

19.1.4	Procedures in the HLR

The procedure in the HLR is initiated when it receives a MAP_REGISTER_SS indication.

The HLR acts as follows:

-	If the operator has barred the subscriber from access to supplementary services, the Call Barred error should be returned to the VLR. The parameter "operatorBarring" shall be included with the error.

The supplementary service request shall then be processed according to GSM 03.11 and the 03.8x and 03.9x-series of specifications. This handling may lead to either a successful result, a partially successful result, or an error being returned.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in GSM 09.11).

-	If the VLR is to be updated after the supplementary service registration, the MAP_INSERT_SUBS_DATA_HLR process shall be initiated.

-	If at any time during this procedure a MAP_P_ABORT, MAP_U_ABORT or MAP_CLOSE indication concerning the process is received from the VLR, the process is terminated. If a MAP_NOTICE indication is received, a MAP_CLOSE request indicating sent towards the VLR.

The registration procedure in the HLR is shown in figure 19.1.4/1.

Figure 19.1.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-1-41.EPS;18.2 cm;23 cm;EPS

Figure 19.1.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-1-41.EPS;18.2 cm;23 cm;EPS

19.2	Erasure procedure

19.2.1	General

The erasure procedure is used to erase data related to a supplementary service in the HLR. The erasure procedure is a fully transparent communication between the MS and the HLR, except that some services may be invoked as a result of the procedure, as described in the sections below.

The erasure procedure is shown in figure 19.2.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_INSERT_SUBSCRIBER_DATA	(defined in sections 6 and 21);

MAP_ERASE_SS			(defined in section 9).

┌────┐            ┌────┐      B       ┌────┐    D       ┌────┐

│ MS │------------│MSC │──────┼───────│VLR │────┼───────│HLR │

└────┘            └────┘              └────┘            └────┘

│                 │                    │                 │

│  A_CM_SERV_REQ  │                    │                 │

│---------------->│ MAP_PROCESS_ACC_REQ│                 │

│    (NOTE 1)     │───────────────────>...               │

│                 │    (NOTE 2)        │                 │

│                 │                    │                 │

│  A_ERASE_SS     │  MAP_ERASE_SS      │                 │

│---------------->│───────────────────>│ MAP_ERASE_SS    │

│                 │                    │────────────────>│

│                 │                    │                 │

│                 │  MAP_ERASE_SS ack  │MAP_ERASE_SS ack │

│ A_ERASE_SS ack  │<───────────────────│<────────────────│

│<----------------│                    │MAP_INS_SUBS_DATA│

│                 │                    │<────────────────│

│                 │                    │    (NOTE 3)     │

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Services printed in italics are optional.

Figure 19.2.1/1: Interfaces and services for supplementary service erasure

19.2.2	Procedures in the MSC

The MSC procedures for erasure are identical to those specified for registration in section 19.1.2. The text and diagrams in section 19.1.2 apply with all references to registration changed to erasure.

19.2.3	Procedures in the VLR

The VLR procedures for erasure are identical to those specified for registration in section 19.1.3. The text and diagrams in section 19.1.3 apply with all references to registration changed to erasure.

19.2.4	Procedures in the HLR

The HLR procedure for erasure is identical to those specified for registration in section 19.1.4. The text and diagrams in section 19.1.4 apply with all references to registration changed to erasure.

19.3	Activation procedure

19.3.1	General

The activation procedure is used to activate a supplementary service in the HLR. The activation procedure is a fully transparent communication between the MS and the HLR, except that some services may be invoked as a result of the procedure, as described in the sections below.

The activation procedure is shown in figure 19.3.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_GET_PASSWORD		(defined in section 9);

MAP_INSERT_SUBSCRIBER_DATA	(defined in sections 6 and 21);

MAP_ACTIVATE_SS		(defined in section 9).

┌────┐ ┌────┐     ┌────┐      B       ┌────┐    D      ┌────┐

│ MS │-│EIR │-----│MSC │──────┼───────│VLR │────┼──────│HLR │

└────┘ └────┘     └────┘              └────┘           └────┘

│                 │                    │                 │

│  A_CM_SERV_REQ  │                    │                 │

│---------------->│MAP_PROCESS_ACC_REQ │                 │

│   (NOTE 1)      │───────────────────>...               │

│                 │    (NOTE 2)        │                 │

│                 │                    │                 │

│  A_ACTIVATE_SS  │  MAP_ACTIVATE_SS   │MAP_ACTIVATE_SS  │

│---------------->│───────────────────>│────────────────>│

│                 │                    │                 │

│  A_GET_PW       │   MAP_GET_PW       │  MAP_GET_PW     │

│<--------------- │<───────────────────│<────────────────│

│                 │                    │                 │

│  A_GET_PW ack   │ MAP_GET_PW ack     │MAP_GET_PW ack   │

│---------------->│───────────────────>│────────────────>│

│                 │                    │                 │

│A_ACTIVATE_SS ack│ MAP_ACTIVATE_SS ack│MAP_ACTIV_SS ack │

│<----------------│<───────────────────│<────────────────│

│                 │                    │MAP_INS_SUBS_DATA│

│                 │                  ...<────────────────│

│                 │                    │     (NOTE 3)    │

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Services printed in italics are optional.

Figure 19.3.1/1: Interfaces and services for supplementary service activation

19.3.2	Procedures in the MSC

The A_ACTIVATE_SS service indication received by the MAP user in the MSC contains the SS-code and any parameters related to the supplementary service.

The MSC transfers the received information to the VLR in the MAP_ACTIVATE_SS request without checking the contents of the service indication. Rules for the mapping are described in GSM 09.11.

The MAP user may subsequently receive the MAP_GET_PASSWORD indication from the VLR. Upon receipt of this indication, the MSC sends the A_GET_PASSWORD message towards the MS and then awaits the response from the MS. When an A_GET_PASSWORD confirm message is received from the MS, the MSC initiates the MAP_GET_PASSWORD response towards the VLR without checking further the contents of the indication. Also see GSM 09.11.

The MSC will receive a MAP_ACTIVATE_SS confirm from the VLR. The outcome of the procedure is reported to the MS in the A_ACTIVATE_SS response message, see GSM 04.8x, 04.9x and 09.11. Finally the SS connection is released.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in TS GSM 09.11).

The handling of MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE and unexpected MAP_CLOSE or A_CM_RELEASE in this procedure is identical to the handling in the Registration procedure in the MSC, see section 19.1.2 of this specification.

The activation procedure in the MSC is shown in figure 19.3.2/1.

Figure 19.3.2/1

.G.C:\USER\MAP\MAP-470\EPS19\119-3-21.EPS;18.2 cm;23 cm;EPS

19.3.3	Procedures in the VLR

Supplementary service Activation

When receiving the MAP_ACTIVATE_SS indication, the MAP user in the VLR transfers the information to the HLR in the MAP_ACTIVATE_SS request without checking the contents of the service indication.

The VLR may then receive the MAP_GET_PASSWORD indication. This information is transferred to the MSC in the MAP_GET_PASSWORD request. If a MAP_GET_PASSWORD confirm primitive is received from the MSC, the VLR initiates the MAP_GET_PASSWORD response towards the HLR.

The VLR will receive the MAP_ACTIVATE_SS confirm from the HLR. The MAP user in the VLR shall transfer the information contained in this primitive to the MSC in the MAP_ACTIVATE_SS response without checking its contents.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in TS GSM 09.11).

Error handling

The handling of MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE and unexpected MAP_CLOSE in this procedure is identical to the handling in the Registration procedure in the VLR, see section 19.1.3 of this specification.

The activation procedure in the VLR is shown in figure 19.3.3/1.

Figure 19.3.3/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-3-31.EPS;18.2 cm;23 cm;EPS

Figure 19.3.3/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-3-31.EPS;18.2 cm;23 cm;EPS

19.3.4	Procedures in the HLR

The procedure in the HLR is initiated when it receives a MAP_ACTIVATE_SS indication.

The HLR acts as follows:

-	If the operator has barred the subscriber from access to supplementary services, the Call Barred error should be returned to the VLR. The parameter "operatorBarring" shall be included with the error.

The supplementary service request shall then be processed according to TS GSM 03.11 and the 03.8x and 03.9x-series of specifications. This handling may lead to either a successful result, a partially successful result, or an error being returned.

During the handling of activation, the get password procedure may be initiated (as specified in TS GSM 03.11). This will involve the sending of a MAP_GET_PASSWORD request to the VLR.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in TS GSM 09.11).

-	If the VLR is to be updated after the supplementary service activation, the MAP_INSERT_SUBS_DATA_HLR process is initiated.

-	Handling of receipt of MAP_P_ABORT, MAP_U_ABORT or MAP_CLOSE indications from the VLR is identical to their handling in the registration procedure, see section 19.1.4 above.

The activation procedure in the HLR is shown in figure 19.3.4/1.

Figure 19.3.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-3-41.EPS;18.2 cm;23 cm;EPS

Figure 19.3.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-3-41.EPS;18.2 cm;23 cm;EPS

19.4	Deactivation procedure

19.4.1	General

The deactivation procedure is used to deactivate a supplementary service in the HLR. The deactivation procedure is a fully transparent communication between the MS and the HLR, except that some services may be invoked as a result of the procedure, as described in the sections below.

The deactivation procedure is shown in figure 19.4.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_GET_PASSWORD		(defined in section 9);

MAP_INSERT_SUBSCRIBER_DATA	(defined in sections 6 and 21);

MAP_DEACTIVATE_SS		(defined in section 9).

┌────┐            ┌────┐      B       ┌────┐    D      ┌────┐

│ MS │------------│MSC │──────┼───────│VLR │────┼──────│HLR │

└────┘            └────┘              └────┘           └────┘

│                 │                    │                 │

│  A_CM_SERV_REQ  │                    │                 │

│---------------->│MAP_PROCESS_ACC_REQ │                 │

│   (NOTE 1)      │───────────────────>...               │

│                 │    (NOTE 2)        │                 │

│                 │                    │                 │

│ A_DEACTIVATE_SS │ MAP_DEACTIVATE_SS  │MAP_DEACTIVATE_SS│

│---------------->│───────────────────>│────────────────>│

│                 │                    │                 │

│  A_GET_PW       │   MAP_GET_PW       │  MAP_GET_PW     │

│<--------------- │<───────────────────│<────────────────│

│                 │                    │                 │

│  A_GET_PW ack   │ MAP_GET_PW ack     │MAP_GET_PW ack   │

│---------------->│───────────────────>│────────────────>│

│                 │                    │                 │

│A_DEACTIV_SS ack │ MAP_DEACTIV_SS ack │MAP_DEACT_SS ack │

│<----------------│<───────────────────│<────────────────│

│                 │                    │MAP_INS_SUBS_DATA│

│                 │                  ...<────────────────│

│                 │                    │     (NOTE 3)    │

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Services printed in italics are optional.

Figure 19.4.1/1: Interfaces and services for supplementary service deactivation

19.4.2	Procedures in the MSC

The MSC procedures for deactivation are identical to those specified for activation in section 19.3.2. The text and diagrams in section 19.3.2 apply with all references to activated changed to deactivated.

19.4.3	Procedures in the VLR

The VLR procedures for deactivation are identical to those specified for activation in section 19.3.3. The text and diagrams in section 19.3.3 apply with all references to activated changed to deactivated.

19.4.4	Procedures in the HLR

The HLR procedures for deactivation are identical to those specified for activation in section 19.3.4. The text and diagrams in section 19.3.4 apply with all references to activated changed to deactivated.

19.5	Interrogation procedure

19.5.1	General

The interrogation procedure is used to retrieve information related to a supplementary service from the VLR or the HLR. It is the VLR which decides whether an interrogation request should be forwarded to the HLR or not. Some non-supplementary service related services may be invoked as a result of the procedure, as described in the sections below.

The interrogation procedure is shown in figure 19.5.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_INTERROGATE_SS		(defined in section 9).

┌────┐            ┌────┐      B       ┌────┐    D       ┌────┐

│ MS │------------│MSC │──────┼───────│VLR │────┼───────│HLR │

└────┘            └────┘              └────┘            └────┘

│  A_CM_SERV_REQ  │                    │                  │

│---------------->│MAP_PROCESS_ACC_REQ │                  │

│   (NOTE 1)      │───────────────────>...                │

│                 │    (NOTE 2)        │                  │

│                 │                    │                  │

│ A_INTERROGATE_SS│ MAP_INTERROGATE_SS │MAP_INTERROGATE_SS│

│---------------->│───────────────────>│─────────────────>│

│                 │                    │                  │

│ A_INTERR_SS ack │ MAP_INTERR_SS ack  │MAP_INTER_SS ack  │

│<----------------│<───────────────────│<─────────────────│

│                 │                    │     (NOTE 3)     │

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Services printed in italics are optional.

Figure 19.5.1/1: Interfaces and services for supplementary service interrogation

19.5.2	Procedures in the MSC

The MSC procedures for interrogation are identical to those specified for registration in section 19.1.2. The text and diagrams in section 19.1.2 apply with all references to registration changed to interrogation.

19.5.3	Procedures in the VLR

Supplementary service Interrogation

When receiving the MAP_INTERROGATE_SS indication, the MAP user acts as follows:

-	If the operator has barred the subscriber from access to supplementary services, the error Call Barred is returned to the MSC. The parameter "operatorBarring" shall be included with the error.

The interrogation is either answered by the VLR or by the HLR, depending on the service interrogated.

a)	Interrogation to be handled by the VLR

The supplementary service request shall then be processed according to TS GSM 03.11 and the 03.8x and 03.9x-series of specifications. This handling may lead to either a successful result, a partially successful result, or an error being returned.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in GSM 09.11).

b)	Interrogation to be handled by HLR

If the interrogation is to be handled by the HLR, on receiving the MAP_INTERROGATE_SS indication, the MAP user in the VLR transfers the information to the HLR in the MAP_INTERROGATE_SS request without further checking the contents of the service indication.

The VLR will receive the MAP_INTERROGATE_SS confirm from the HLR. The MAP user in the VLR shall transfer the information contained in this primitive to the MSC in the MAP_INTERROGATE_SS response without checking its contents.

For call independent SS operations, each message shall only contain a single component. Messages which contain more than one component will be stopped at the air interface (as specified in TS GSM 09.11).

Error handling

Handling of MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE and unexpected MAP_CLOSE in this procedure is identical to the handling in the Registration procedure in the VLR, section 19.1.3. The Interrogation procedure is described in figure 19.5.3/1.

Figure 19.5.3/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-5-31.EPS;18.2 cm;23 cm;EPS

Figure 19.5.3/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-5-31.EPS;18.2 cm;23 cm;EPS

Figure 19.5.3/1 sheet 3

.G.C:\USER\MAP\MAP-470\EPS19\319-5-31.EPS;18.2 cm;23 cm;EPS

19.5.4	Procedures in the HLR

The HLR procedures for erasure are identical to those specified for registration in section 19.1.4. The text and diagrams in section 19.1.4 apply with all references to registration changed to erasure.

19.6	Invocation procedure

19.6.1	General

The invocation procedure is used to check subscription data in the VLR for certain supplementary services which are invoked after the call set-up phase is finished. For invocation of supplementary services which are invoked during the call set-up phase, please refer to the Call Handling procedure descriptions.

The invocation procedure is shown in figure 19.6.1/1. Note that some optional services may be invoked in connection with this procedure, as described in the section below.

The following services are used:

MAP_PROCESS_ACCESS_REQUEST	(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(defined in sections 7 and 21);

MAP_PROVIDE_IMSI		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI		(defined in sections 6 and 21);

MAP_AUTHENTICATE		(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE	(defined in sections 6 and 21);

MAP_CHECK_IMEI		(defined in sections 6 and 21);

MAP_READY_FOR_SM		(defined in sections 10 and 21);

MAP_INVOKE_SS			(defined in section 9).

┌────┐             ┌────┐      B       ┌────┐

│ MS │-------------│MSC │──────┼───────│VLR │

└────┘             └────┘              └────┘

│ A_CM_SERV_REQ    │                    │

│----------------->│MAP_PROCESS_ACC_REQ │

│    (NOTE 1)      │───────────────────>...

│                  │   (NOTE 2)         │

│   A_INVOKE_SS    │                    │

│----------------->│   MAP_INVOKE_SS    │

│   (NOTE 3)       │───────────────────>│

│                  │                    │

│   A_INVOKE_SS    │   MAP_INVOKE_SS    │

│<-----------------│<───────────────────│

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines indicate the trigger provided by the signalling on the radio path, and the signalling triggered on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	A_INVOKESS is a generic message to illustrate any supplementary service invocation request message on the air interface, e.g. BuildMPTY; see GSM 04.80.

Figure 19.6.1/1: Interfaces and services for supplementary service invocation

19.6.2	Procedures in the MSC

Process Access Request

Before the Call Hold or Multi-Party supplementary services can be invoked, a CC connection must be established between the MS and the MSC as described in GSM 04.08 and the Call Handling procedure descriptions within this specification.

When an A_INVOKE_SS request message arrives at the MSC during a call (as described in TS GSM 04.10, 04.8x and 04.9x-series), then if control of subscription to the invoked supplementary service is required, the MSC initiates the process access request procedure towards the VLR as described in section 21 of this specification.

Supplementary service Invocation

If the Process Access Request procedure towards the VLR is successful, the MSC shall forward a MAP_INVOKE_SS service request towards the VLR. This request shall contain the SS-Code of the supplementary service to be invoked, and possibly the Basic service code. Mapping from the A_INVOKE_SS to this service request is described in TS GSM 09.11.

The MSC will receive a MAP_INVOKE_SS confirm from the VLR. If the outcome of the service is successful (i.e. the service confirm is empty), the MSC will invoke the requested supplementary service as described in GSM 02.8x-series, 03.8x and 03.9x-series. If the outcome of the service is unsuccessful, the MSC shall send an appropriate A_INVOKE_SS response towards the MS. The structure of this message is described in TS GSM 09.11 and 04.8x and 04.9x-series.

Error handling

If at any time during this procedure a MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE or MAP_CLOSE indication concerning the process is received from the VLR, the process is terminated. If a MAP_NOTICE indication was received from the VLR, the VLR dialogue must also be aborted by sending a MAP_U_ABORT request indicating Procedure error towards the VLR. Possible signalling to the MS is described in TS GSM 04.10.

If an A_CM_RELEASE indication is received from the MS, all open transactions are released using the MAP_U_ABORT request indicating application procedure cancellation; the process terminates.

The invocation procedure in the MSC is shown in figure 19.6.2/1.

Figure 19.6.2/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-6-21.EPS;18.2 cm;23 cm;EPS

Figure 19.6.2/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-6-21.EPS;18.2 cm;23 cm;EPS

19.6.3	Procedures in the VLR

Process Access Request

When receiving the MAP_PROCESS_ACCESS_REQUEST indication, the VLR acts as described in section 21 of this specification.

Supplementary service Invocation

When receiving the MAP_INVOKE_SS indication, the MAP user acts as follows:

-	If the operator has barred the subscriber from access to supplementary services, the error "Call Barred" is returned to the MSC. The parameter "operatorBarring" shall be included with the error.

-	If any irrelevant information elements (according to the service description) or invalid information element values are present in the service request, then the unexpected data value error is returned to the MSC in the MAP_INVOKE_SS response.

-	If the VLR does not support the invoked supplementary service then the VLR shall respond with the SS Not Available error.

-	If the requested supplementary service cannot be invoked by subscriber actions, then the VLR shall respond with the Illegal SS Operation error.

-	If the subscriber is not provided with (i.e. subscribed to) the requested supplementary service, then the SS error status error (possibly including the SS-Status as parameter) is returned to the MSC in the MAP_INVOKE_SS response.

If all checks are passed the VLR returns an empty MAP_INVOKE_SS response to the MSC, thus indicating that the invocation request was accepted.

If at any time during this procedure a MAP_P_ABORT, MAP_U_ABORT, MAP_NOTICE or unexpected MAP_CLOSE indication concerning the process is received from the MSC, the process terminates. If a MAP_NOTICE indication was received from the MSC, that dialogue must be aborted by sending a MAP_U_ABORT request indicating Procedure error towards the MSC. The process terminates.

The invocation procedure in the VLR is shown in figure 19.6.3/1.

Figure 19.6.3/1

.G.C:\USER\MAP\MAP-470\EPS19\119-6-31.EPS;18.2 cm;23 cm;EPS

19.7	Register password procedure

19.7.1	General

The password registration procedure is used to register a password in the HLR. The password registration procedure is a fully transparent communication between the MS and the HLR, except that some services may be invoked as a result of the procedure, as described below.

The password registration procedure is shown in figure 19.7.1/1.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST		(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY		(defined in sections 7 and 21);

MAP_PROVIDE_IMSI			(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI			(defined in sections 6 and 21);

MAP_AUTHENTICATE			(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE		(defined in sections 6 and 21);

MAP_CHECK_IMEI			(defined in sections 6 and 21);

MAP_READY_FOR_SM			(defined in sections 10 and 21);

MAP_FORWARD_CHECK_SS_INDICATION	(defined in section 6);

MAP_GET_PASSWORD			(defined in section 9).

┌────┐            ┌────┐      B       ┌────┐    D     ┌────┐

│ MS │------------│MSC │──────┼───────│VLR │────┼─────│HLR │

└────┘            └────┘              └────┘          └────┘

│                 │                    │                │

│A_CM_SERV_REQ    │  MAP_PROC_ACC_REQ  │                │

│---------------->│───────────────────>...              │

│   (NOTE 1)      │    (NOTE 2)        │                │

│                 │                    │                │

│  A_REGISTER_PW  │  MAP_REGISTER_PW   │MAP_REGISTER_PW │

│---------------->│───────────────────>│───────────────>│

│                 │                    │                │

│   A_GET_PW      │   MAP_GET_PW       │  MAP_GET_PW    │

│<--------------- │<───────────────────│<───────────────│

│  A_GET_PW ack   │   MAP_GET_PW ack   │MAP_GET_PW ack  │

│---------------->│───────────────────>│───────────────>│

│                 │                    │                │

│   A_GET_PW      │   MAP_GET_PW       │  MAP_GET_PW    │

│<--------------- │<───────────────────│<───────────────│

│  A_GET_PW ack   │   MAP_GET_PW ack   │MAP_GET_PW ack  │

│---------------->│───────────────────>│───────────────>│

│                 │                    │                │

│   A_GET_PW      │   MAP_GET_PW       │  MAP_GET_PW    │

│<--------------- │<───────────────────│<───────────────│

│  A_GET_PW ack   │   MAP_GET_PW ack   │MAP_GET_PW ack  │

│---------------->│───────────────────>│───────────────>│

│                 │                    │                │

│A_REGISTER_PW ack│MAP_REGISTER_PW ack │ MAP_REG_PW ack │

│<----------------│<───────────────────│<───────────────│

NOTE 1:	For details of the procedure on the radio path, see GSM 04.08, 04.10, 04.8x and 04.9x. Services shown in dotted lines are triggers/ triggered signalling on the radio path.

NOTE 2:	For details on the Process Access Request procedure, please refer to section 21 in this specification.

NOTE 3:	Use of each of the three MAP_GET_PASSWORD operations is described in section 19.7.4.

Figure 19.7.1/1: Interfaces and services for supplementary service password registration

19.7.2	Procedures in the MSC

The register password procedure in the MSC is identical to that for activation specified in section 19.3.2. All the text and diagrams in section 19.3.2 apply with all references to activation changed to register password.

19.7.3	Procedures in the VLR

The register password procedure in the VLR is identical to that for activation specified in section 19.3.3. All the text and diagrams in section 19.3.3 apply with all references to activation changed to register password.

19.7.4	Procedures in the HLR

The procedure in the HLR is initiated when it receives a MAP_REGISTER_PASSWORD indication.

The HLR acts as follows:

-	If the operator has barred the subscriber for access to supplementary services, the Call Barred error is returned to the VLR. The parameter "operatorBarring" shall be included with the error.

-	If any irrelevant information elements (according to the service description) or invalid information element values are present, then the unexpected data value error is returned to the VLR in the response. This error should thus be returned if the SS-code provided by the mobile subscriber is not allocated.

The HLR shall then process the MAP_REGISTER_PASSWORD indication as specified in TS GSM 03.11. During the handling of password registration, the password procedure will be initiated (as specified in TS GSM 03.11) This will involve the sending of MAP_GET_PASSWORD requests to the VLR.

-	Handling of receipt of MAP_P_ABORT, MAP_U_ABORT or MAP_CLOSE indications from the VLR is identical to their handling in the registration procedure, see section 19.1.4 above.

The password registration procedure in the HLR is shown in figure 19.7.4/1.

Figure 19.7.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-7-41.EPS;18.2 cm;23 cm;EPS

Figure 19.7.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-7-41.EPS;18.2 cm;23 cm;EPS

19.8	Mobile Initiated USSD procedure

19.8.1	General

The procedure supports supplementary service signalling procedures which can allow PLMN specific services to be introduced.

The message flow for the procedure can be found in TS GSM 03.90.

The following services may be used:

MAP_PROCESS_ACCESS_REQUEST		(defined in sections 6 and 21);

MAP_TRACE_SUBSCRIBER_ACTIVITY		(defined in sections 7 and 21);

MAP_PROVIDE_IMSI			(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI			(defined in sections 6 and 21);

MAP_AUTHENTICATE			(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE		(defined in sections 6 and 21);

MAP_CHECK_IMEI			(defined in sections 6 and 21);

MAP_READY_FOR_SM			(defined in sections 10 and 21);

MAP_UNSTRUCTURED_SS_REQUEST		(defined in section 9);

MAP_UNSTRUCTURED_SS_NOTIFY		(defined in section 9).

The following service is certainly used:

MAP_PROCESS_UNSTRUCTURED_SS_REQUEST	(defined in section 9).

19.8.2	Procedures in the MSC

Before the Process Unstructured SS Request service can be invoked, a call independent CM connection must be created between the MS and the MSC.

Once a CM-connection is established, the MSC may handle the A_PROCESS_UNSTRUCTRUCTURED_SS_REQUEST from the MS. This message contains information input by the user, the message may be fed to an application contained locally in the MSC or to the VLR. The rules for determining this are specified in TS GSM 03.90.

1)	Message Destined for VLR

If the message is destined for the VLR then the MSC shall transfer the message to the VLR using the mapping specified in detail in TS GSM 09.11.

The MSC may subsequently receive one or more MAP_UNSTRUCTURED_SS_REQUEST  or MAP_UNSTRUCTURED_SS_NOTIFY indications from the VLR. These shall be sent transparently to the MS. When a confirmation is received from the MS this shall be returned to the VLR.

When the MSC receives a MAP_PROCESS_UNSTRUCTRUCTURED_SS_REQUEST confirmation from the VLR then it shall pass this to the MS and initiate release of the CM connection.

2)	Message Destined for Local Application

If the message is destined for the local USSD application then the MSC shall transfer the message to the application.

The MSC may subsequently receive one or more requests from the application which correspond to the MAP_UNSTRUCTURED_SS_REQUEST  or MAP_UNSTRUCTURED_SS_NOTIFY indications. These shall be sent transparently to the MS. When a confirmation is received from the MS this shall be returned to the application.

When the MSC receives the result of the original operation from the application then it shall pass this to the MS and initiate release of the CM connection.

Error Handling

Both the MS and the VLR or USSD Application may initiate release of the CM-connection at any time. This is handled as shown in the diagrams.

The procedure in the MSC is shown in figure 19.8.2/1.

Figure 19.8.2/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-8-21.EPS;18.2 cm;23 cm;EPS

Figure 19.8.2/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-8-21.EPS;18.2 cm;23 cm;EPS

19.8.3	Procedures in the VLR

The initiation of the process is shown in section 19.0.2.

Once a MAP dialogue is established, the VLR may handle the MAP_PROCESS_UNSTRUCTRUCTURED_SS_REQUEST from the MSC. This message contains information input by the user, the message may be fed to an application contained locally in the VLR or to the HLR. The rules for determining this are specified in TS GSM 03.90.

Message Destined for HLR

If the message is destined for the HLR then the VLR shall transfer the message transparently to the HLR.

The VLR may subsequently receive one or more MAP_UNSTRUCTURED_SS_REQUEST  or MAP_UNSTRUCTURED_SS_NOTIFY indications from the HLR. These shall be sent transparently to the MSC. When a confirmation is received from the MSC this shall be returned to the HLR.

When the VLR receives a MAP_PROCESS_UNSTRUCTRUCTURED_SS_REQUEST confirmation from the HLR then it shall pass this to the MS and close the MAP provider service.

Message Destined for Local Application

If the message is destined for the local USSD application then the VLR shall transfer the message to the application.

The VLR may subsequently receive one or more requests from the application which correspond to the MAP_UNSTRUCTURED_SS_REQUEST  or MAP_UNSTRUCTURED_SS_NOTIFY indications. These shall be sent transparently to the MSC. When a confirmation is received from the MSC this shall be returned to the application.

When the VLR receives the result of the original operation from the application then it shall pass this to the MSC and initiate release of the CM connection.

Error Handling

Both the MSC and the HLR or USSD Application may initiate release of the MAP service at any time. This is handled as shown in the diagrams.

The procedure in the VLR is shown in figure 19.8.3/1 and 19.8.3/2.

Figure 19.8.3/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-8-31.EPS;18.2 cm;23 cm;EPS

Figure 19.8.3/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-8-31.EPS;18.2 cm;23 cm;EPS

Figure 19.8.3/1 sheet 3

.G.C:\USER\MAP\MAP-470\EPS19\319-8-31.EPS;18.2 cm;23 cm;EPS

Figure 19.8.3/2

.G.C:\USER\MAP\MAP-470\EPS19\119-8-32.EPS;18.2 cm;23 cm;EPS

19.8.4	Procedures in HLR

The initiation of the process is shown in section 19.0.3.

Once a MAP dialogue is established, the HLR may handle the MAP_PROCESS_UNSTRUCTRUCTURED_SS_REQUEST from the VLR. This message contains information input by the user. If the alphabet used for the message is understood then the message shall be fed to an application contained locally in the HLR. If the alphabet is not understood then the error "UnknownAlphabet" shall be returned.

The HLR may subsequently receive one or more requests from the application which correspond to the MAP_UNSTRUCTURED_SS_REQUEST  or MAP_UNSTRUCTURED_SS_NOTIFY indications. These shall be sent transparently to the VLR. When a confirmation is received from the VLR this shall be returned to the application.

When the HLR receives the result of the original operation from the application then it shall pass this to the VLR and initiate release of the CM connection.

Error Handling

Both the VLR and the USSD Application may initiate release of the MAP service at any time. This is handled as shown in the diagrams.

The procedure in the HLR is shown in figure 19.8.4/1.

Figure 19.8.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-8-41.EPS;18.2 cm;23 cm;EPS

Figure 19.8.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-8-41.EPS;18.2 cm;23 cm;EPS

19.9	Network initiated ussd procedure

19.9.1	General

The procedure supports supplementary service signalling procedures which can allow PLMN specific services to be introduced.

The message flow for the procedure can be found in TS GSM 03.90.

The following services may be used:

MAP_PAGE				(defined in sections 6 and 21);

MAP_SEARCH_FOR_MOBILE_SUBSCRIBER	(defined in sections 6 and 21);

MAP_PROCESS_ACCESS_REQUEST		(defined in sections 6 and 21);

MAP_AUTHENTICATE			(defined in sections 6 and 21);

MAP_SET_CIPHERING_MODE		(defined in sections 6 and 21);

MAP_FORWARD_NEW_TMSI			(defined in sections 6 and 21);

MAP_READY_FOR_SM			(defined in sections 10 and 21).

At least one of the following services will certainly be used, and both may be used:

MAP_UNSTRUCTURED_SS_REQUEST		(defined in section 9);

MAP_UNSTRUCTURED_SS_NOTIFY		(defined in section 9).

19.9.2	Procedure in MSC

The procedure may be invoked either by the VLR or by a USSD application local to the MSC. They may start by using either the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY service. If the request is initiated by a local USSD application then the MSC will open a dialigue with the HLR.

In both cases the MSC will initiate a CM connection to the MS (using the page or search macros defined in section 21.3). Once the connection is successfully established the message  received from the VLR or USSD application will be sent to the MS using the mapping specified in TS GSM 09.11.

Following transfer of the message the MSC will wait for a confirmation from the MS. This will be sent to the VLR or USSD application as appropriate.

Following this, the MSC may receive further uses of the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY services, or may receive an indication to release the connection to the MS.

In the event of an error, the connection to the MS shall be released, and the MAP process with the VLR shall be aborted as shown in the diagram.

The procedure in the MSC is shown in figure 19.9.2/1.

Figure 19.9.2/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-9-21.EPS;18.2 cm;23 cm;EPS

Figure 19.9.2/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-9-21.EPS;18.2 cm;23 cm;EPS

Figure 19.9.2/1 sheet 3

.G.C:\USER\MAP\MAP-470\EPS19\319-9-21.EPS;18.2 cm;23 cm;EPS

Figure 19.9.2/1 sheet 4

.G.C:\USER\MAP\MAP-470\EPS19\419-9-21.EPS;18.2 cm;23 cm;EPS

19.9.3	Procedure in VLR

The procedure may be invoked either by the HLR or by a USSD application local to the VLR. They may start by using either the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY service.

In both cases the VLR will first initiate a MAP dialogue with the MSC. When the indication for the unstructured ss request or notify is received then the macro Start_USSD_VLR will be used to page the MS and open a CM connection. Once the the CM connection is successfully established the indication  received from the HLR or USSD application will be sent to the MSC.

Following transfer of the message the VLR will wait for a confirmation from the MSC. This will be sent to the HLR or USSD application as appropriate.

Following this, the VLR may receive further uses of the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY services, or may receive a MAP_CLOSE_ind.

In the event of an error, the MAP process with the MSC shall be released, and if necessary the MAP process with the HLR shall be aborted as shown in the diagram.

The procedure in the VLR is shown in figure 19.9.3/1.

MSC Initiated USSD

If a USSD application in the MSC wishes to use the network initiated USSD procedure, and a connection to the MS does not exist then it shall open a dialogue to the VLR. This dialogue will automatically lead to the VLR performing page and search using the macro Start_USSD_VLR.

Macro Start_USSD_VLR

This macro is used to initiate a CM connection with the MS for transfer of network initiated unstructred SS data.

It first checks for correct data in the VLR. If a problem is found then "Err" is returned.

A page or search procedure (as appropriate) will then be used to contact the MS. Following successful page or search the macro Process_Access_Request_VLR specified in section 21.4 will be used to handle the CM connection establishment.

The macro is shown in figure 19.9.3/2.

Figure 19.9.3/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-9-31.EPS;18.2 cm;23 cm;EPS

Figure 19.9.3/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-9-31.EPS;18.2 cm;23 cm;EPS

Figure 19.9.3/1 sheet 3

.G.C:\USER\MAP\MAP-470\EPS19\319-9-31.EPS;18.2 cm;23 cm;EPS

Figure 19.9.3/1 sheet 4

.G.C:\USER\MAP\MAP-470\EPS19\419-9-31.EPS;18.2 cm;23 cm;EPS

Figure 19.9.3/2 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-9-32.EPS;18.2 cm;23 cm;EPS

Figure 19.9.3/2 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-9-32.EPS;18.2 cm;23 cm;EPS

19.9.4	Procedure in HLR

The procedure may be invoked by the USSD application local to the HLR. It may start by using either the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY service.

In both cases the HLR will first check whether the MS is reachable (i.e. there is a VLR identity stored in the subscriber record, the MS record is not marked as purged and the MS record is not marked "MSC Area Restricted").

If the MS is reachable, the HLR will initiate a MAP dialogue with the VLR. Once the dialogue is successfully established the message received from the USSD application will be sent to the MSC.

Following transfer of the message the HLR will wait for a confirmation from the MSC. This will be sent to the USSD application.

Following this, the HLR may receive further uses of the MAP_UNSTRUCTURED_SS_REQUEST or MAP_UNSTRUCTURED_SS_NOTIFY services, or may receive a MAP_CLOSE_ind.

In the event of an error, the MAP process with the VLR shall be released as shown in the diagram.

The procedure in the HLR is shown in figure 19.9.4/1.

Figure 19.9.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS19\119-9-41.EPS;18.2 cm;23 cm;EPS

Figure 19.9.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS19\219-9-41.EPS;18.2 cm;23 cm;EPS

19.10	Common macros for section 19

The following macros are used for the description of more than one of the supplementary service processes described in section 19:

19.10.1	SS Password handling macros

Macro Get_Password_MSC

This macro is used by the MSC to relay a request for password from the VLR to the MS, and to relay a response from the MS back to the VLR. The macro is described in figure 19.10.1/1.

Macro Get_Password_VLR

This macro is used by the VLR to relay a request for password from the HLR to the MSC, and to relay a response from the MSC back to the HLR. The macro is described in figure 19.10.1/2.

Figure 19.10.1/1

.G.C:\USER\MAP\MAP-470\EPS19\119-1011.EPS;18.2 cm;23 cm;EPS

Figure 19.10.1/2

.G.C:\USER\MAP\MAP-470\EPS19\119-1012.EPS;18.2 cm;23 cm;EPS

19.10.2	SS Error handling macros

Macro Receive_errors_MSC

This macro is used by the MSC to receive signals which should lead to failure if received in any state of a supplementary service process. If the air interface connection is released by the MS, the communication towards the VLR is aborted, and the MSC should return to a stable "NULL" state. If a MAP_NOTICE indication is received from the VLR, or the VLR aborts or unexpectedly closes the connection, then the air interface connection shall be released. The macro is described in figure 19.10.2/1.

Macro Receive_error_from_MSC

This macro is used by the VLR to receive signals from the MSC which should lead to failure if received in any state of a supplementary service process. If a MAP_NOTICE indication is received from the MSC, that connection is closed before the only outcome of the macro, "err" is reported back to the calling process. The macro is described in figure 19.10.2/2.

Macro Receive_error_from_HLR

This macro is used by the VLR to receive signals from the HLR which should lead to failure if received in any state of a supplementary service process. If a MAP_NOTICE indication is received from the MSC, that connection is closed. The macro is described in figure 19.10.2/3.

Figure 19.10.2/1

.G.C:\USER\MAP\MAP-470\EPS19\119-1021.EPS;18.2 cm;23 cm;EPS

Figure 19.10.2/2

.G.C:\USER\MAP\MAP-470\EPS19\119-1022.EPS;18.2 cm;23 cm;EPS

Figure 19.10.2/3

.G.C:\USER\MAP\MAP-470\EPS19\119-1023.EPS;18.2 cm;23 cm;EPS

20	Short Message Service Procedures

20.1	General

The Short Message Service procedures are used to control both mobile originated and mobile terminated short message transfer.

Four procedures exist for short message services:

-	Mobile originated short message service transfer;

-	Mobile terminated short message service transfer;

-	Short message alert procedure;

-	Short message waiting data set procedure.

The following application context refers to a complex MAP user consisting of several processes:

-	shortMessageGatewayContext.

This application context needs a co-ordinating process in the HLR. Additionally a Co-ordinator has to be defined for the mobile originated situation in the MSC, because the A_CM_SERV_REQ message does not distinguish between mobile originated short message transfer and the short message alert procedures.

20.1.1	Mobile originated short message service Co-ordinator for the MSC

The A_CM_SERV_REQ message (TS GSM 04.08) is received from the A interface containing the CM service type. This parameter indicates mobile originated short message service. The service MAP_PROCESS_ACCESS_REQUEST is started.

If the MAP_PROCESS_ACCESS_REQUEST service ends successfully, the mobile station initiates mobile originated short message transfer or alerting indication. Depending on the situation, the appropriate process is initiated as follows:

-	if the A_RP_MO_DATA indication is received, the process MOSM_MSC is initiated (see section 20.2.1);

-	if the A_RP_SM_MEMORY_AVAILABLE indication is received, the process SC_Alert_MSC is initiated (see section 20.4.1).

After creation of the user process the Co-ordinator relays the messages between the A interface and the invoked process until a request or an indication for dialogue termination is received.

The SMS process Co-ordinator is shown in the figure 20.1/1.

Figure 20.1/1

.G.C:\USER\MAP\MAP-470\EPS20\120-11.EPS;18.2 cm;23 cm;EPS

20.1.2	Short message Gateway Co-ordinator for the HLR

The MAP_OPEN indication opens a dialogue for the short message procedure between the gateway MSC and the HLR when the application context shortMessageGatewayContex is received. If that service is successful, the Co-ordinator can receive the first service primitive from the MAP_PM. Depending on the received primitive, the user process is created as follows:

-	if the MAP_SEND_ROUTING_INFO_FOR_SM indication is received, the process Mobile_Terminated_MS_HLR is created;

-	if the MAP_REPORT_SM_DELIVERY_STATUS indication is received, the process Report_SM_delivery_stat_HLR is created.

After creation of the user processs the Co-ordinator relays the messages between the MAP_PM and the invoked process until a request or an indication for dialogue termination is received.

The SM Gateway Co-ordinator is shown in the figure 20.1/2.

Figure 20.1/2

.G.C:\USER\MAP\MAP-470\EPS20\120-12.EPS;18.2 cm;23 cm;EPS

20.2	The mobile originated short message transfer procedure

The mobile originated short message service procedure is used to forward short message from a mobile subscriber to a Service Centre. The mobile originated short message service procedure is shown in figure 20.2/1.

┌─────┐  ┌──────────┐   ┌──────┐   ┌─────────────┐   ┌────┐

│ MS  │  │Servicing │   │ VLR  │   │Interworking │   │ SC │

│     │  │    MSC   │   │      │   │    MSC      │   │    │

└─────┘  └──────────┘   └──────┘   └─────────────┘   └────┘

│        1.│             │              │           │

├ ─ ─ ─ ─ >┤           2.│              │           │

│          ├════════════>┤              │           │

│          │ 3.          │              │           │

│          ├<════════════┤              │           │

│          │             │            4.│           │

│          ╞═════════════╪═════════════>┤         5.│

│          │             │              ├ ─ ─ ─ ─ ─>┤

│          │             │              │6.         │

│          │7.           │              ├<─ ─ ─ ─ ─ ┤

│8.        ├<════════════╪══════════════┤           │

├< ─ ─ ─ ─ ┤             │              │           │

│          │             │              │           │

1.	Short Message (BSSAP; GSM 04.11)

2.	MAP_SEND_INFO_FOR_MO_SMS

3.	MAP_SEND_INFO_FOR_MO_SMS_ACK

4.	MAP_FORWARD_SHORT_MESSAGE

5.	Short message (GSM 03.40)

6.	Short message Acknowledgement (GSM 03.40)

7.	MAP_FORWARD_SHORT_MESSAGE_ACK

8.	Short Message Acknowledgment (BSSAP; GSM 04.11)

Figure 20.2/1: Mobile originated short message transfer

In addition the following MAP services are used:

MAP_PROCESS_ACCESS_REQUEST	(see section 6.3);

MAP_AUTHENTICATE		(see section 6.5);

MAP_SET_CIPHERING_MODE	(see section 6.6);

MAP_PROVIDE_IMSI		(see section 6.9);

MAP_CHECK_IMEI		(see section 6.7);

MAP_FORWARD_NEW_TMSI		(see section 6.9);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(see section 7.1);

MAP_READY_FOR_SM		(see section 10.4).

20.2.1	Procedure in the servicing MSC

The activation of the MAP_PROCESS_ACCESS_REQUEST service is described in the section 20.6.2.

When receiving the short message from the A interface, the MSC sends the MAP_SEND_INFO_FOR_MO_SMS request to the VLR. As a response the MSC will receive the MAP_SEND_INFO_FOR_MO_SMS confirmation from VLR indicating that:

-	the service ends successfully. If the MSC is not itself the IWMSC, the short message transmission towards the IWMSC is initiated using the MAP_FORWARD_SHORT_MESSAGE request.

-	the service ends unsuccessfully. The error cause in the MAP_SEND_INFO_FOR_MO_SMS confirmation indicates the reason for the unsuccessful end. The mapping between MAP error causes and RP_ERROR causes is described in TS GSM 09.10.

If there are data errors in the MAP_SEND_INFO_FOR_MO_SMS confirmation, or there is an operation failure in MAP, the RP_ERROR cause network out of order is forwarded to the mobile station.

If the service MAP_FORWARD_SHORT_MESSAGE is started, the MSC will receive the MAP_FORWARD_SHORT_MESSAGE confirmation from the IWMSC indicating that:

-	the short message has been successfully delivered to the Service Centre. The acknowledgement is sent to the mobile station;

-	one of several error cases has occurred. The mapping between MAP error causes and RP_ERROR causes is described in TS GSM 09.10. The appropriate indication is provided to the mobile station.

If the procedure failed, a provider error or an abort indication is received. The RP_ERROR cause network out of order is provided to the mobile station.

If the MSC itself is the interworking MSC, the short message is forwarded to the Service Centre. In that case the service MAP_FORWARD_SHORT_MESSAGE is not initiated. The acknowledge message from the Service Centre is forwarded to the mobile station (TS GSM 3.40, TS GSM 4.11).

The mobile originated short message service procedure is shown in figure 20.2/2.

Figure 20.2/2 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-22.EPS;18.2 cm;23 cm;EPS

Figure 20.2/2 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-22.EPS;18.2 cm;23 cm;EPS

Figure 20.2/2 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-22.EPS;18.2 cm;23 cm;EPS

20.2.2	Procedure in the VLR

The MAP_PROCESS_ACCESS_REQUEST indication starts the MAP_PROCESS_ACCESS_REQUEST service in the VLR. The application context in the MAP_OPEN indication is mobile originated short message transfer.

If the service MAP_PROCESS_ACCESS_REQUEST is successful, the VLR waits for the next message from the MSC. When receiving the MAP_SEND_INFO_FOR_MO_SMS indication, the VLR acts as follows:

-	if there is incompatibility in the subscription check, the error teleservice not provisioned is returned to the MSC;

-	if the short message transfer would contravene operator determined barring, the call barred error with cause operator barring is returned;

-	if the short message transfer would contravene the supplementary service call barring conditions in the VLR, the call barred error with cause barring service active is returned.

When the mobile subscriber has passed all checks, the MAP_SEND_INFO_FOR_MO_SMS response is initiated and the procedure is terminated in the VLR. The mobile originated short message transfer procedure in the VLR is shown in figure 20.2/3.

Figure 20.2/3

.G.C:\USER\MAP\MAP-470\EPS20\120-23.EPS;18.2 cm;23 cm;EPS

20.2.3	Procedure in the interworking MSC

This procedure applies only when the IWMSC is not the servicing MSC.

When a MAP_FORWARD_SHORT_MESSAGE indication is correctly received, the Interworking MSC invokes forwarding of the short message to the Service Centre. If invalid data content is detected, an unexpected data value error or a data missing error is returned to the servicing MSC.

The outcome of the procedure with the Service Centre is awaited before a MAP_FORWARD_SHORT_MESSAGE response is given back to the servicing MSC:

-	if a short message is accepted by the Service Centre, an acknowledgement is sent back to the servicing MSC

-	if the Service Centre is not identified, the SM delivery failure error is returned to the servicing MSC;

-	if the Service Centre returns an error indication, the SM delivery failure error is returned to the servicing MSC with the error cause and any diagnostic information received from the Service Centre;

-	if the short message cannot be forwarded to the Service Centre or the procedure towards the Service Centre fails for some reason, a system failure error is sent to the servicing MSC.

The mobile originated short message service transfer in the IWMSC is shown in figure 20.2/4.

Figure 20.2/4

.G.C:\USER\MAP\MAP-470\EPS20\120-24.EPS;18.2 cm;23 cm;EPS

20.3	The mobile terminated short message transfer procedure

The mobile terminated short message transfer procedure is used for forwarding a short message or several short messages from a Service Centre to a mobile subscriber. The mobile terminated short message procedure for a single short message transfer is shown in figure 20.3/1.

┌─────┐ ┌─────────┐┌──────┐ ┌──────┐  ┌─────────┐   ┌────┐

│ MS  │ │Servicing││ VLR  │ │ HLR  │  │ Gateway │   │ SC │

│     │ │    MSC  ││      │ │      │  │ MSC     │   │    │

└─────┘ └─────────┘└──────┘ └──────┘  └─────────┘   └────┘

│         │         │        │           │1.        │

│         │         │        │2.         ├< ─ ─ ─ ─ ┤

│         │         │        ├<══════════┤          │

│         │         │        │         3.│          │

│         │         │        ├══════════>┤          │

│         │4.       │        │           │          │

│         ├<════════╪════════╪═══════════┤          │

│         │       5.│        │           │          │

│         ├════════>┤        │           │          │

│         │6.       │        │           │          │

│7.       ├<════════┤        │           │          │

├< ─ ─ ─ ─┤         │        │           │          │

│       8.│         │        │           │          │

├ ─ ─ ─ ─>┤       9.│        │           │          │

│         ├════════>┤        │           │          │

│         │10.      │        │           │          │

│11.      ├<════════┤        │           │          │

├< ─ ─ ─ ─┤         │        │           │          │

│      12.│         │        │           │          │

├ ─ ─ ─ ─>┤         │        │        13.│          │

│         ├═════════╪════════╪══════════>┤       14.│

│         │         │        │           ├ ─ ─ ─ ─ >┤

1.	Short Message (GSM 03.40)

2.	MAP_SEND_ROUTING_INFO_FOR_SM

3.	MAP_SEND_ROUTING_INFO_FOR_SM_ACK

4.	MAP_FORWARD_SHORT_MESSAGE

5.	MAP_SEND_INFO_FOR_MT_SMS

6.	MAP_PAGE/MAP_SEARCH_FOR_MOBILE_SUBSCRIBER

7.	Page (BSSAP; GSM 04.08)

8.	Page response (BSSAP; GSM 04.08)

9.	MAP_PROCESS_ACCESS_REQUEST_ACK and

MAP_SEARCH_FOR_MOBILE_SUBSCRIBER_ACK

10.	MAP_SEND_INFO_FOR_MT_SMS_ACK

11.	Short Message (BSSAP; GSM 04.11)

12.	Short Message Acknowledgement (BSSAP; GSM 04.11)

13.	MAP_FORWARD_SHORT_MESSAGE_ACK

14.	Short Message Acknowledgment (GSM 03.40)

Figure 20.3/1: Mobile terminated short message service procedures

The mobile terminated short message procedure for multiple short message transfer is shown in figure 20.3/2.

┌─────┐ ┌─────────┐┌──────┐ ┌──────┐  ┌─────────┐   ┌────┐

│ MS  │ │Servicing││ VLR  │ │ HLR  │  │ Gateway │   │ SC │

│     │ │    MSC  ││      │ │      │  │ MSC     │   │    │

└──┬──┘ └────┬────┘└───┬──┘ └───┬──┘  └─────┬───┘   └──┬─┘

│         │         │        │           │          │

│         │         │        │           │< ─ ─ ─ ─ │1.

│         │         │        ├<══════════┤2.        │

│         │         │      3.├══════════>┤          │

│         │<════════╪════════╪═══════════┤4.        │

│       5.╞════════>┤        │           │          │

│         │<════════┤6.      │           │          │

│< ─ ─ ─ ─┤7.       │        │           │          │

8.│ ─ ─ ─ ─>│         │        │           │          │

│       9.╞════════>┤        │           │          │

│         │<════════┤10.     │           │          │

│< ─ ─ ─ ─┤11.      │        │           │          │

12.│ ─ ─ ─ ─>│         │        │           │          │

│      13.╞═════════╪════════╪══════════>┤          │

│         │         │        │        14.├ ─ ─ ─ ─ >┤

│         │         │        │           ├< ─ ─ ─ ─ ┤15.

│         │<════════╪════════╪═══════════┤16.       │

│< ─ ─ ─ ─┤17.      │        │           │          │

18.│ ─ ─ ─ ─>│         │        │           │          │

│      19.╞═════════╪════════╪══════════>┤          │

│         │         │        │        20.├ ─ ─ ─ ─ >┤

1.	Short Message (GSM 03.40)

2.	MAP_SEND_ROUTING_INFO_FOR_SM

3.	MAP_SEND_ROUTING_INFO_FOR_SM_ACK

4.	MAP_FORWARD_SHORT_MESSAGE (NOTE 1)

5.	MAP_SEND_INFO_FOR_MT_SMS

6.	MAP_PAGE/MAP_SEARCH_FOR_MOBILE_SUBSCRIBER

7.	Page (BSSAP; GSM 08.08)

8.	Page response (BSSAP; GSM 04.08)

9.	MAP_PROCESS_ACCESS_REQUEST_ACK and

MAP_SEARCH_FOR_MOBILE_SUBSCRIBER_ACK

10.	MAP_SEND_INFO_FOR_MT_SMS_ACK

11.	Short Message (BSSAP; GSM 04.11)

12.	Short Message Acknowledgement (BSSAP; GSM 04.11)

13.	MAP_FORWARD_SHORT_MESSAGE_ACK

14.	Short Message Acknowledgment (GSM 03.40)

15.	Short Message (GSM 03.40)

16.	MAP_FORWARD_SHORT_MESSAGE (NOTE 2)

17.	Short Message (BSSAP; GSM 04.11)

18.	Short Message Acknowledgement (BSSAP; GSM 04.11)

19.	MAP_FORWARD_SHORT_MESSAGE_ACK

20.	Short Message Acknowledgement (GSM 03.40)

NOTE 1:	The More Messages To Send flag is TRUE.

NOTE 2:	The More Messages To Send flag is FALSE

Figure 20.3/2: Mobile terminated short message procedure for multiple short message transfer.

In the multiple short message transfer the service MAP_FORWARD_SHORT_MESSAGE can be used several times. However, the short message transfer is always acknowledged to the Service Centre before the next short message is sent.

In addition the following MAP services are used:

MAP_PROCESS_ACCESS_REQUEST	(see section 6.3);

MAP_PAGE			(see section 6.2);

MAP_SEARCH_FOR_MS		(see section 6.2);

MAP_AUTHENTICATE		(see section 6.5);

MAP_SET_CIPHERING_MODE	(see section 6.6);

MAP_CHECK_IMEI		(see section 6.7);

MAP_FORWARD_NEW_TMSI		(see section 6.9);

MAP_REPORT_SM_DELIVERY_STATUS	(see section 10.3);

MAP_INFORM_SERVICE_CENTRE	(see section 10.6);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(see section 7.1);

MAP_READY_FOR_SM		(see section 10.4).

20.3.1	Procedure in the Servicing MSC

When initiating the dialogue with the servicing MSC, the SMS Gateway MSC must provide the IMSI of the subscriber to whom the short message is directed.

The IMSI can be included either in the Destination Reference of the MAP_OPEN indication received from the SMS Gateway MSC or in the sm-RP-DA information field of the MAP_FORWARD_SHORT_MESSAGE indication.

When receiving a MAP_OPEN indication primitive that is not associated with any MAP service indication primitive and if the dialogue is accepted, the MAP service-user in the servicing MSC issues a MAP_DELIMITER request primitive in order to trigger the local MAP service-provider to confirm the dialogue.

When receiving the first MAP_FORWARD_SHORT_MESSAGE indication from the gateway MSC, the servicing MSC sends the MAP_SEND_INFO_FOR_MT_SMS request primitive to the VLR, if the MAP service primitive is accepted and if short message service is supported in the servicing MSC.

The MAP_FORWARD_SHORT_MESSAGE indication primitive is checked by the macro "Check_Indication". If the received MAP service primitive contains errors, the service is aborted and an unexpected data value error or data missing error is returned to the GMSC.

If the MSC does not support the short message service, the service is aborted in the servicing MSC and the error "Facility Not Supported" is returned to the GMSC.

The subscriber identity information that may be included in the MAP_OPEN indication primitive and in the MAP service indication primitive is checked by the macro "Check_Subscr_Identity_For_MT_SMS" as follows.

If a Destination Reference has been received in the MAP_OPEN indication, an LMSI must be present in the sm-RP-DA information field of the MAP_FORWARD_SHORT_MESSAGE indication. The LMSI shall be included in the sm-RP-DA information field of the MAP_SEND_INFO_FOR_MT_SMS request sent to the VLR; the associated MAP_OPEN request must contain a Destination Reference that carries an IMSI.

Otherwise, if the IMSI is included in the sm-RP-DA information field of the MAP_FORWARD_SHORT_MESSAGE indication, it is mapped into the sm-RP-DA information field of the MAP_SEND_INFO_FOR_MT_SMS request that is sent to the VLR. In this case, the IMSI is not accompanied by an LMSI and neither the MAP_OPEN indication received from the gateway MSC nor the MAP_OPEN request sent to the VLR shall include a Destination Reference.

If a Destination Reference has been received in the servicing MSC and the sm-RP-DA information field of the MAP_FORWARD_SHORT_MESSAGE indication does not include an LMSI or if no Destination Refernce has been received and the sm-RP-DA information field does not cover an IMSI the service is aborted in the servicing MSC and the error "Unexpected Data Value" is returned to the SMS GMSC.

The following responses to the MAP_SEND_INFO_FOR_MT_SMS request may be received from the VLR:

-	absent subscriber, unidentified subscriber or system failure error. The error code is forwarded to the GMSC;

-	unknown subscriber error. The system failure indication is provided to the GMSC;

-	data missing or unexpected data value error. The system failure indication is provided to the GMSC;

-	a provider error or an abort indication. The system failure indication is provided to the GMSC;

-	paging procedure invocation (see section 21.3) reporting the successful outcome of the procedure;

-	search procedure invocation (see section 21.3) reporting the successful outcome of the procedure.

The result of the paging or the search procedure is processed as follows:

-	if the procedure is completed successfully, the MSC will send the MAP_PROCESS_ACCESS_REQUEST request to the VLR (see section 21.4);

-	if the procedure is completed successfully, but the Mobile Station has no mobile terminated Short message transfer capability, the procedure is terminated and SM Delivery failure indication with cause "equipment not SM equipped" is provided to the GMSC;

-	if the procedure ends unsuccessfully, the termination of the procedure is awaited from the VLR. The absent subscriber error is forwarded to the GMSC, but the other error causes are reported as a system failure indication.

If the short message transfer is aborted for any reason, the dialogue with the VLR is aborted. If the procedure with the VLR is aborted by the VLR or by the provider, a system failure indication is provided to the GMSC.

The unsuccessful outcome of the MAP_PROCESS_ACCESS_REQUEST service is reported by using the system failure error to the GMSC.

When the service MAP_PROCESS_ACCESS_REQUEST is carried out, the MSC will receive the MAP_SEND_INFO_FOR_MT_SMS confirmation indicating:

-	the unsuccessful outcome of the procedure. The error indication received from the VLR is forwarded to the GMSC;

-	the successful outcome of the procedure. The MSC initiates forwarding of the short message to the mobile station.

If the primitive itself is badly formatted or data is missing, the system failure error is sent to the GMSC.

If forwarding of the short message is initiated, the MSC awaits the result before one of the following responses is sent back to the GMSC:

-	an acknowledge if the short message has been successfully delivered to the mobile subscriber;

-	an SM delivery failure error containing a parameter indicating either of the following: there is a mobile station protocol error or the mobile station memory capacity is exceeded; detailed diagnostic information (see section 5.6.1.4) may also be carried;

-	a system failure error if the delivery procedure is aborted.

If the more messages to send flag was FALSE or the service MAP-FORWARD_SHORT_MESSAGE ends unsuccessfully, the transaction to the gateway MSC is terminated. Otherwise, the servicing MSC waits for the next short message from the Service Centre.

When receiving the next MAP_FORWARD_SHORT_MESSAGE indication from the gateway MSC the servicing MSC will act as follows:

-	if the received primitive contains errors, the unexpected data value error or data missing error is provided to the gateway MSC;

-	if the more messages to send flag is FALSE, the servicing MSC will start the short message transfer procedure to the mobile subscriber. The successful or unsuccessful outcome of this procedure is reported to the gateway MSC and the transaction is terminated;

-	if the more messages to send flag is TRUE, the servicing MSC will start the short message transfer to the mobile subscriber. If the outcome of this procedure is unsuccessful, the reason is reported to the gateway MSC and the procedure is terminated. If the procedure is successful, it is acknowledged to the gateway MSC and more short messages can be received.

The tracing procedure may be activated. It is described in detail in the section 17.

The mobile terminated short message transfer procedure in the servicing MSC is shown in figures 20.3/3 and 20.3/4. The page and search procedures are shown in figure 21.3/1.

Figure 20.3/3 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-33.EPS;18.2 cm;23 cm;EPS

Figure 20.3/3 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-33.EPS;18.2 cm;23 cm;EPS

Figure 20.3/3 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-33.EPS;18.2 cm;23 cm;EPS

Figure 20.3/4 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-34.EPS;18.2 cm;23 cm;EPS

Figure 20.3/4 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-34.EPS;18.2 cm;23 cm;EPS

Figure 20.3/4 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-34.EPS;18.2 cm;23 cm;EPS

20.3.2	Procedures in the VLR

When receiving the MAP_SEND_INFO_FOR_MT_SMS indication, the VLR will act as follows:

-	the parameters and data in the primitive are checked by the macro "Check_Indication". A data failure is reported as an unexpected data value error or a data missing error depending on the nature of the failure;

-	for mobile terminated short message the mobile subscriber is identified either by the IMSI only or by the IMSI accompanied by the LMSI. The subscriber identity information that may be included in the MAP_OPEN indication primitive and in the MAP service indication primitive is checked by the macro "Check_Subscr_Identity_For_MT_SMS". In the first case, the IMSI is included in the sm-RP-DA information field and the Destination Reference must not be present in the MAP_OPEN primitve. In the latter case the IMSI must be obtained from the Destination Reference of the MAP_OPEN indication primitive and an LMSI must be present in the sm-RP-DA information field of the MAP_SEND_INFO_FOR_MT_SMS indication. If the mobile subscriber is unknown, the unidentified subscriber error is returned;

-	if the "confirmed by the HLR" indicator is set to not confirmed, the unidentified subscriber error is returned;

-	if the IMSI Detached Flag is set to detached or the LA Not Allowed Flag is set to not allowed in the VLR, an absent subscriber error is returned and the mobile station not reachable flag (MNRF) is set;

-	if the MAP_SEND_INFO_FOR_MT_SMS indication has passed all the tests, the VLR will initiate the paging procedure. If the location area identification is known and the "confirmed by radio contact" indicator is set to confirmed, the MAP_PAGE service is used. Otherwise the MAP_SEARCH_FOR_MOBILE_SUBSCRIBER service is started.

The following responses to the paging procedure may be received from the MSC:

-	the MAP_SEARCH_FOR_MOBILE_SUBSCRIBER confirmation indicating a successful outcome, if the search procedure is used. After that the VLR awaits the MAP_PROCESS_ACCESS_REQUEST indication from the MSC;

-	the MAP_PAGE confirmation or MAP_SEARCH_FOR_MOBILE_SUBSCRIBER confirmation indicating unsuccessful outcome. If an absent subscriber error is received, the mobile station not reachable flag (MNRF) is set in the VLR. The errors are forwarded to the MSC in the MAP_SEND_INFO_FOR_MT_SMS response. If the unexpected data value, busy subscriber or unknown location area error is received, the system failure indication is given to the MSC;

-	the MAP_PROCESS_ACCESS_REQUEST indication telling that the outcome of the service MAP_PAGE is successful.

If the paging procedure or process access request procedure or any other procedure invoked fails, the appropriate error is reported to the MSC.

If the process access request procedure is successful, the VLR will send the MAP_SEND_INFO_FOR_MT_SMS response to the MSC and the transaction is terminated in the VLR.

The mobile terminated short message transfer procedure in the VLR is shown in figure 20.3/5.

Figure 20.3/5 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-35.EPS;18.2 cm;23 cm;EPS

Figure 20.3/5 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-35.EPS;18.2 cm;23 cm;EPS

Figure 20.3/5 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-35.EPS;18.2 cm;23 cm;EPS

20.3.3	Procedures in the HLR

The MAP_SEND_ROUTING_INFO_FOR_SM indication is received from the GMSC. The following error cases are reported to the GMSC in the MAP_SEND_ROUTING_INFO_FOR_SM response as an unsuccessful outcome of the procedure:

-	if the necessary parameters and data are not present in the primitive or they are badly formatted, the data missing or unexpected data value error is returned;

-	if the mobile subscriber is unknown, i.e. it cannot be identified from the MSISDN given, an unknown subscriber error is returned;

-	if the short message transfer would contravene operator determined barring, the call barred error with cause operator barring is returned;

-	if the short message transfer would contravene the supplementary service barring, the call barred error with cause barring service active is returned;

-	if the mobile subscription identified by the given MSISDN number does not include the short message service, the teleservice not provisioned error is returned;

-	if the location registration of the mobile subscriber shows that the visited PLMN does not support the MT short message service, the facility not supported error is returned;

-	if no MSC identity is stored for the mobile subscriber or the "MSC Area Restricted Flag" is set or the "MS purged" flag is set, i.e. the MS is not reachable, the MSISDN-Alert and the SC address are included in the MWD (if possible) and the "Absent Subscriber" error is returned.

The priority parameter (SM_RP_PRI) is processed as follows:

-	if the priority is low (SM_RP_PRI = False) and the mobile station not reachable flag (MNRF) is set, an absent subscriber error is returned. The SC-address given in the request will be included in the MWD if possible. The service MAP_INFORM_SERVICE_CENTRE including the parameter MW Status is invoked to indicate whether or not the SC address has been included in the MWD list;

-	if the priority is low (SM_RP_PRI = False), and the MNRF is clear, the routing information is retrieved as described below;

-	if the priority is high (SM_RP_PRI = True) and the MNRF is set, the HLR will send the acknowledge primitive containing the routing information to the gateway MSC. In addition the service MAP_INFORM_SERVICE_CENTRE including the parameter MW Status is invoked to indicate whether or not the SC address is already included in the MWD list.

If the MSISDN-Alert number of the mobile subscriber stored in the MWD is not the same as that received in the MAP_SEND_ROUTING_INFO_FOR_SM indication, the HLR will include in the MAP_INFORM_SERVICE_CENTRE request to the GMSC the MSISDN-Alert number stored.

The MAP_INFORM_SERVICE_CENTRE request is sent also when the MCEF and/or MNRF are set but the routing information is still sent to the GMSC. The status of the flags is indicated in the parameter MW Status.

The routing information is included in a MAP_SEND_ROUTING_INFO_FOR_SM response as follows:

-	the IMSI will be returned to the GMSC together with the MSC number and may be optionally accompanied by the LMSI.

The mobile terminated short message transfer procedure in the HLR is shown in figure 20.3/6.

Figure 20.3/6 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-36.EPS;18.2 cm;23 cm;EPS

Figure 20.3/6 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-36.EPS;18.2 cm;23 cm;EPS

Figure 20.3/6 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-36.EPS;18.2 cm;23 cm;EPS

20.3.4	Procedures in the gateway MSC

The short message handling function of the GMSC will request routing information when a mobile terminated short message is received from a Service Centre. The GMSC sends the MAP_SEND_ROUTING_INFO_FOR_SM request to the HLR containing the subscriber data of the mobile subscriber.

As an outcome of the procedure the MAP_SEND_ROUTING_INFO_FOR_SM confirmation is received indicating:

-	an unsuccessful event indication containing an error.

The mapping between the MAP error causes and the RP_ERROR causes is explained in TS GSM 03.40;

-	a successful event indication containing following parameters:

-	an IMSI optionally accompanied by an LMSI; and

-	a routing address (a servicing MSC address).

The GMSC may also receive a MAP_INFORM_SERVICE_CENTRE indication after the MAP_SEND ROUTING_INFO_FOR_SM confirmation. The parameter MW Status in the message indicates whether or not the Service Centre address is stored in the Message Waiting Data. It also indicates the status of the MCEF and MNRF flags in the HLR.

If the MSISDN-Alert stored in the MWD data is not the same as the one sent to the HLR, the MSISDN-Alert is received in the MAP_INFORM_SERVICE_CENTRE indication. This MSISDN number shall be transferred in a delivery failure report to the SC.

In the abnormal end or in the provider error case the system failure error is provided to the SC.

The forward short message procedure is initiated when the GMSC has obtained the routing information needed to forward a mobile terminated short message to the servicing MSC. If an LMSI has been provided in the MAP_SEND_ROUTING_INFO_FOR_SM confirmation, it can be included in the sm-RP-DA information field of the first MAP_FORWARD_SHORT_MESSAGE request sent to the servicing MSC. In this case, the IMSI must be included in the Destination Reference of the MAP_OPEN request. If the LMSI is not sent by the SMS Gateway MSC, the sm-RP-DA information field in the first MAP_FORWARD_SHORT_MESSAGE request sent to the servicing MSC shall contain the IMSI and the Destination Reference in the MAP_OPEN request shall not be present. The Service Centre address is sent in the parameter SM_RP_OA. The more messages to send flag is set to TRUE or FALSE depending on the information received from the Service Centre.

If the GMSC is the servicing MSC then the MAP service is not initiated. The procedure in the Servicing MSC is described in section 20.3.1 and in the figure 20.3/4.

If the grouping of MAP_OPEN request and MAP_FORWARD_SHORT_MESSAGE request together would need segmenting on SCCP level, these primitives must not be grouped together. The MAP_OPEN request primitive is sent first without any associated MAP service request primitive and the dialogue confirmation must be received before the MAP_FORWARD_SHORT_MESSAGE request is sent.

As a response to the procedure, the GMSC will receive the MAP_FORWARD_SHORT_MESSAGE confirmation indicating:

-	a successful forwarding of the short message. This indication is passed to the SC;

-	unsuccessful forwarding of the short message. The mapping of the MAP error causes to the RP_ERROR causes is explained in TS GSM 09.10 and TS GSM 04.11. The appropriate error indication is sent to the SC.

A provider error is indicated as a system failure error to the SC.

The GMSC invokes the procedure MAP_REPORT_SM_DELIVERY_STATUS, if an absent subscriber, an unidentified subscriber or SM delivery failure with error cause MS memory capacity exceeded indication is received from the servicing MSC, and the corresponding flags received in the MAP_INFORM_SC are not already set or the SC address is not yet included in the MWD set. The gateway MSC may also invoke the procedure when the transfer was successful, if the MNRF and/or MCEF flags were set in the HLR. This procedure is described in detail in section 20.5.

Unexpected data value, system failure and unidentified subscriber errors are indicated as a system failure to the SC. Other errors are indicated using appropriate cause values and diagnostic information between the GMSC and the SC as described in TS GSM 03.40 and GSM 04.11.

If there are more short messages to send in the Service Centre and the previous short message transfer succeeded, then the gateway MSC awaits the next short message.

When receiving the next short message from the SC, the gateway MSC sets the more messages to send flag according to the information received and starts the service MAP_FORWARD_SHORT_MESSAGE again.

If the gateway MSC is the servicing MSC, then the short message transfer to mobile subscriber is started as described in the section 20.3.1.

The mobile terminated short message transfer procedure in the gateway MSC is shown in figure 20.3/7.

Figure 20.3/7 sheet 1

.G.C:\USER\MAP\MAP-470\EPS20\120-37.EPS;18.2 cm;23 cm;EPS

Figure 20.3/7 sheet 2

.G.C:\USER\MAP\MAP-470\EPS20\220-37.EPS;18.2 cm;23 cm;EPS

Figure 20.3/7 sheet 3

.G.C:\USER\MAP\MAP-470\EPS20\320-37.EPS;18.2 cm;23 cm;EPS

Figure 20.3/7 sheet 4

.G.C:\USER\MAP\MAP-470\EPS20\420-37.EPS;18.2 cm;23 cm;EPS

Figure 20.3/7 sheet 5

.G.C:\USER\MAP\MAP-470\EPS20\520-37.EPS;18.2 cm;23 cm;EPS

Figure 20.3/8

.G.C:\USER\MAP\MAP-470\EPS20\120-38.EPS;18.2 cm;23 cm;EPS

20.4	The Short Message Alert procedure

The Short Message Alert procedure is used for alerting the Service Centre when the mobile subscriber is active after a short message transfer has failed because the mobile subscriber is not reachable or when the mobile station has indicated that it has memory capacity to accept a short message.

The Short Message Alert procedure for the case when the mobile subscriber was not reachable is shown in figure 20.4/1.

┌────┐  ┌─────────┐  ┌─────┐  ┌─────┐  ┌───────────┐  ┌────┐

│ MS │  │Servicing│  │ VLR │  │ HLR │  │ Inter-    │  │ SC │

│    │  │    MSC  │  │     │  │     │  │working MSC│  │    │

└──┬─┘  └────┬────┘  └──┬──┘  └──┬──┘  └─────┬─────┘  └──┬─┘

│       1.│          │        │           │           │

│ ─ ─ ─ ─>│        2.│        │           │           │

│         ╞═════════>│      3.│           │           │

│         │          ╞═══════>│           │           │

│         │          ╞<═══════╡4.         │           │

│         │          │      5.╞══════════>│         6.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │      5.╞══════════>│         6.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │      5.╞══════════>│         6.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │        │<══════════╡7.         │

│         │          │        │<══════════╡7.         │

│         │          │        │<══════════╡7.         │

1.	CM Service Request, Page Response or Location Updating (BSSAP; GSM 04.08)

2.	MAP_PROCESS_ACCESS_REQUEST / MAP_UPDATE_LOCATION_AREA

3.	MAP_READY_FOR_SM (Mobile Present) / MAP_UPDATE_LOCATION /

Supplementary Service Control Request

4.	MAP_READY_FOR_SM_ACK

5.	MAP_ALERT_SERVICE_CENTRE (NOTES 1 and 2)

6.	Alert Service Centre (GSM 03.40)

7.	MAP_ALERT_SERVICE_CENTRE_ACK

NOTE 1:	To all Service Centres in the Message Waiting List.

NOTE 2:	The HLR initiates the MAP_ALERT_SERVICE_CENTRE service only if the MS Memory Capacity Exceeded flag is clear.

Figure 20.4/1: Short message alert procedure (Mobile is present)

The Short Message Alert procedure for the case where the mobile station indicates that it has memory capacity to accept one or more short messages is shown in figure 20.4/2.

┌────┐  ┌─────────┐  ┌─────┐  ┌─────┐  ┌───────────┐  ┌────┐

│ MS │  │Servicing│  │ VLR │  │ HLR │  │ Inter-    │  │ SC │

│    │  │    MSC  │  │     │  │     │  │working MSC│  │    │

└──┬─┘  └────┬────┘  └──┬──┘  └──┬──┘  └─────┬─────┘  └──┬─┘

│       1.│          │        │           │           │

│ ─ ─ ─ ─>│        2.│        │           │           │

│         ╞═════════>│      3.│           │           │

│         │          ╞═══════>│           │           │

│         │5.        ╞<═══════╡4.         │           │

│6.       │<═════════╡        │           │           │

│<- - - - │          │        │           │           │

│         │          │      7.╞══════════>│         8.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │      7.╞══════════>│         8.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │      7.╞══════════>│         8.│

│         │          │        │           ├ ─ ─ ─ ─ ─>┤

│         │          │        │<══════════╡9.         │

│         │          │        │<══════════╡9.         │

│         │          │        │<══════════╡9.         │

1.	SM memory capacity available (BSSAP; GSM 04.11)

2.	MAP_READY_FOR_SM (Memory Available)

3.	MAP_READY_FOR_SM (Memory Available)

4.	MAP_READY_FOR_SM_ACK

5.	MAP_READY_FOR_SM_ACK

6.	SM memory capacity available (Acknowledge) (BSSAP; GSM 04.11)

7.	MAP_ALERT_SERVICE_CENTRE (NOTE)

8.	Alert Service Centre (GSM 03.40)

9.	MAP_ALERT_SERVICE_CENTRE_ACK

NOTE:	To all Service Centres in the Message Waiting List.

Figure 20.4/2: Short message alert procedure (Mobile station memory capacity available)

In addition the following MAP services are used in the mobile station memory available case:

MAP_PROCESS_ACCESS_REQUEST	(see section 6.3);

MAP_AUTHENTICATE		(see section 6.5);

MAP_SET_CIPHERING_MODE	(see section 6.6);

MAP_PROVIDE_IMSI		(see section 6.9);

MAP_CHECK_IMEI		(see section 6.7);

MAP_FORWARD_NEW_TMSI		(see section 6.9);

MAP_TRACE_SUBSCRIBER_ACTIVITY	(see section 7.1).

The Short Message Alert procedure when the mobile station indicates successful transfer after polling is shown in figure 20.4/3.

┌─────────┐  ┌─────┐  ┌───────────┐  ┌────┐

│ Gateway │  │ HLR │  │ Inter-    │  │ SC │

│    MSC  │  │     │  │working MSC│  │    │

└────┬────┘  └──┬──┘  └─────┬─────┘  └──┬─┘

│          │           │           │

1.╞═════════>╡           │           │

╞<═════════╡2.         │           │

│        3.╞══════════>│         4.│

│          │           │ ─ ─ ─ ─ ─>┤

│        3.╞══════════>│         4.│

│          │           ├ ─ ─ ─ ─ ─>┤

│        3.╞══════════>│         4.│

│          │           ├ ─ ─ ─ ─ ─>┤

│          │<══════════╡5.         │

│          │<══════════╡5.         │

│          │<══════════╡5.         │

1.	MAP_REPORT_SM_DELIVERY_STATUS (Successful Transfer)

2.	MAP_REPORT_SM_DELIVERY_STATUS_ACK

3.	MAP_ALERT_SERVICE_CENTRE (NOTE)

4.	Alert Service Centre (GSM 03.40)

5.	MAP_ALERT_SERVICE_CENTRE_ACK

NOTE:	To all Service Centres in the Message Waiting List.

Figure 20.4/3: Short message alert procedure (Successful transfer after polling)

20.4.1	Procedures in the Servicing MSC

The activation of the MAP_PROCESS_ACCESS_REQUEST service is described in the section 20.6.2.

After receiving the SM memory capacity available indication, the servicing MSC sends the MAP_READY_FOR_SM request to the VLR indicating memory available. The outcome of that procedure is one of the following:

-	successful acknowledgment. The MSC sends the corresponding message to the mobile station;

-	negative acknowledgment, where the error causes are treated as follows:

-	unexpected data value, data missing and system failure errors are reported as network out of order error to the mobile station;

-	facility not supported is reported as requested facility not implemented error to the mobile station;

-	procedure failure, which is reported as network out of order error to the mobile station.

The short message alert procedure in the MSC for the MS memory capacity available case is shown in figure 20.4/4.

Figure 20.4/4

.G.C:\USER\MAP\MAP-470\EPS20\120-44.EPS;18.2 cm;23 cm;EPS

20.4.2	Procedures in the VLR

20.4.2.1	The Mobile Subscriber is present

When receiving the MAP_PROCESS_ACCESS_REQUEST indication, MAP_UPDATE_LOCATION_AREA indication while the mobile station not reachable flag (MNRF) is set, the VLR will send the MAP_READY_FOR_SM request towards the HLR. The Alert Reason is set to indicate that the mobile subscriber is present. If the authentication procedure is initiated and it fails, the VLR will not initiate the service. The process in VLR is described in detail in the section 21.10.

20.4.2.2	The Mobile Equipment has memory available

The MAP_PROCESS_ACCESS_REQUEST indication starts the MAP_PROCESS_ACCESS_REQUEST service in the VLR. The application context in the MAP_OPEN indication refers to the short message alerting procedure.

If the service MAP_PROCESS_ACCESS_REQUEST is successful, the VLR waits for the next message from the MSC. When receiving the MAP_READY_FOR_SM indication from the MSC, the VLR will check the contents. Data errors are reported to the MSC as an unexpected data value or data missing error, depending on the error. If the primitive passes the data check, the VLR forwards it to the HLR and awaits an acknowledgment.

When receiving the MAP_READY_FOR_SM confirmation from the HLR and the Alert Reason is MS memory available, the VLR will act as follows:

-	the MAP_READY_FOR_SM response is sent to the MSC as follows:

-	an acknowledge in the positive case;

-	system failure error, if unexpected data value, data missing, or unknown subscriber errors are received, otherwise the error cause received from the HLR;

-	a facility not supported error, if the HLR supports MAP V1 only;

-	procedure failure is reported as a system failure error.

The short message alert procedure in the VLR is shown in figures 20.4/5.

Figure 20.4/5

.G.C:\USER\MAP\MAP-470\EPS20\120_45.EPS;18.2 cm;23 cm;EPS

20.4.3	Procedures in the HLR

When receiving the MAP_READY_FOR_SM indication, the HLR will check the contents. Data errors are reported to the VLR as an unexpected data value or a data missing error depending on the error. If the HLR doesn't support the MNRF, MCEF, and MWD a facility not supported error is reported to the VLR. If the IMSI is unknown an unknown subscriber error is reported to the VLR. Otherwise an acknowledgement is returned to the VLR.

If neither the mobile station not reachable flag (MNRF) or the memory capacity exceeded flag (MCEF) is set, the HLR sets a timer and waits for it to expire. This ensures that in the race situation the MAP_REPORT_SM_DELIVERY_STATUS service (as described in the section 20.6) for the same subscriber can be carried out when delayed in the GMSC.

If the Alert Reason indicates the mobile present situation, or when the update location procedure has been successfully completed or Supplementary Service Control request is received, the mobile station not reachable flag is cleared and the service centre alert procedure is initiated. If the memory capacity exceeded flag is set, the mobile station not reachable flag is cleared but the alert procedure is not started.

If the Alert Reason indicates the memory available situation, the HLR initiates the alert procedure. The mobile station not reachable and memory capacity available flags are cleared.

If the MAP_REPORT_SM_DELIVERY_STATUS indication is received and it indicates the successful transfer of the mobile terminated short message, the HLR initiates the alert procedure described in the section 21.10 and clears MCEF and MNRF flags.

The short message alert procedure in the HLR is shown in figures 20.4/6 and 21.10/2.

Figure 20.4/6

.G.C:\USER\MAP\MAP-470\EPS20\120_46.EPS;18.2 cm;23 cm;EPS

20.4.4	Procedures in the Interworking MSC

When a MAP_ALERT_SERVICE_CENTRE indication is correctly received by the IWMSC, the IWMSC will forward the alerting to the given Service Centre if possible.

Data errors are reported to the HLR as an unexpected data value or a data missing error depending on the error.

The short message alert procedure is shown in figure 20.4/7.

Figure 20.4/7

.G.C:\USER\MAP\MAP-470\EPS20\120-47.EPS;18.2 cm;23 cm;EPS

20.5	The SM delivery status report procedure

The SM delivery status report procedure is used to set the Service Centre address into the message waiting list in the HLR because the subscriber is absent or unidentified or the memory capacity is exceeded. The procedure sets the memory capacity exceeded flag in the HLR if the MS memory does not have room for more messages or the mobile station not reachable flag in the case of unidentified or absent subscriber.

Additionally the procedure is used to report the HLR about the successful transfer after the Service Centre has polled the subscriber. This procedure is described also in the section 20.4.

The SM delivery status report procedure is shown in figure 20.5/1.

┌───────┐  ┌─────┐  ┌─────┐  ┌─────────┐  ┌────┐

│Visited│  │ VLR │  │ HLR │  │ Gateway │  │ SC │

│  MSC  │  │     │  │     │  │   MSC   │  │    │

└──┬────┘  └─┬───┘  └───┬─┘  └───┬─────┘  └──┬─┘

│         │          │      1.│           │

╞═════════╪══════════╪═══════>│           │

│         │          │2.      │           │

│         │          │<═══════╡           │

│         │          │      3.│           │

│         │          ╞═══════>│         4.│

│         │          │        │ - - - - ->│

│         │          │        │           │

1.	MAP_FORWARD_SHORT_MESSAGE_ACK/_NACK (Absent subscriber,

unidentified subscriber or memory capacity exceeded)

2.	MAP_REPORT_SM_DELIVERY_STATUS

3.	MAP_REPORT_SM_DELIVERY_STATUS_ACK

4.	Short Message Negative Acknowledgement (GSM 03.40)

Figure 20.5/1: Short message delivery status report procedure

20.5.1	Procedures in the HLR

When the HLR receives a MAP_REPORT_SM_DELIVERY_STATUS indication, it acts as described in the section 20.6, macro Report_SM_Delivery_Stat_HLR.

The short message delivery status report process in the HLR is shown in figure 20.5/2.

Figure 20.5/2

.G.C:\USER\MAP\MAP-470\EPS20\120-52.EPS;18.2 cm;23 cm;EPS

20.5.2	Procedures in the gateway MSC

The GMSC invokes the short message delivery status report procedure if an absent subscriber indication or unidentified subscriber indication or SM delivery failure error indicating MS memory capacity exceeded is received from the servicing MSC during a mobile terminated short message transfer, and the HLR has not indicated that the SC address is included in the MWD. The unidentified subscriber indication is however processed as the absent subscriber indication.

The service is invoked also when the HLR has indicated that either of the flags MCEF or MNRF is set and the SM delivery was successful.

The reason for unsuccessful or successful delivery of the short message is included in the SM Delivery Outcome in the MAP_REPORT_SM_DELIVERY_STATUS request.

The GMSC sends the MAP_REPORT_SM_DELIVERY_STATUS request to the HLR. As a response the GMSC will receive the MAP_REPORT_SM_DELIVERY_STATUS confirmation reporting:

-	successful outcome of the procedure. The acknowledge primitive may contain the MSISDN-Alert number which is stored in the MWD List in the HLR;

-	unsuccessful outcome of the procedure. The system failure indication is forwarded to the SC.

A provider error is indicated as a system failure to the SC.

The procedure towards the Service Centre may also be aborted. If so the operation towards the HLR is also aborted.

The short message delivery status report procedure in the GMSC is shown in figure 20.5/3.

Figure 20.5/3

.G.C:\USER\MAP\MAP-470\EPS20\120-53.EPS;18.2 cm;23 cm;EPS

20.6	Common procedures for the short message section

20.6.1	The macro Report_SM_Delivery_Stat_HLR

This macro is used when the HLR receives a MAP_REPORT_SM_DELIVERY_STATUS indication from the GMSC. The HLR responses to the indication as follows:

-	if invalid data content is detected, an unexpected data value error or a data missing error is returned to the GMSC;

-	if the MSISDN number provided is not recognized by the HLR, an unknown subscriber error is returned to the GMSC;

-	if the MAP_REPORT_SM_DELIVERY_STATUS indication reports a successful SM delivery, the Service Centres in the Message Waiting list are alerted as described in the section 21.10;

-	if the SM Delivery Outcome reports unsuccessful delivery and the inclusion of the SC address in the MWD is not possible, a message waiting list full error is returned to the GMSC;

-	if the SM Delivery Outcome reports unsuccessful delivery and the message waiting list is not full, the given Service Centre address is inserted and an acknowledgement is sent to the GMSC. If the MSISDN-Alert stored in the subscriber data is not the same as that received in the MAP_REPORT_SM_DELIVERY_STATUS indication, the MSISDN-Alert is sent in a response primitive to the GMSC;

-	if the SM Delivery Outcome is MS memory capacity exceeded the HLR sets the memory capacity exceeded flag in the subscriber data and resets the MRNF;

-	if the SM Delivery Outcome is absent subscriber the HLR sets the mobile station not reachable flag in the subscriber data.

The short message delivery status report macro in the HLR is shown in figure 20.6/1.

Figure 20.6/1

.G.C:\USER\MAP\MAP-470\EPS20\120-61.EPS;18.2 cm;23 cm;EPS

21	General macro description

21.1	MAP open macros

21.1.1	Macro Receive_Open_Ind

This macro is used by a MAP service-user procedure when a peer entity requests opening of a dialogue.

If the application context received in the MAP-OPEN indication primitive indicates a context name of the MAP version one context set, the macro terminates respectively.

If an application-context different from version 1 is received, the presence of MAP_OPEN information is checked. If no MAP_OPEN information has been recieved, the MAP_OPEN Response with:

-	Result set to Dialogue Accepted; and

-	Application Context Name set to the received value,

is returned and the macro terminates succesfully.

If MAP_OPEN information is received, the macro "CHECK_REFERENCE" is called in oreder to check whether the received values for Destination Rfenece and Originating Reference correspond with the requirements of the received application-context-name. The outcome of this check is an error, the MAP_OPEN Reponse with:

-	Result set to Dialogue Refused;

-	Refuse Reason set to Invalid Destination Reference or Invalid Originating Reference;

-	Application Context Name set to the highest version supported,

is returned and the macro terminates with unsuccessful outcome.

If the data values received for Destination Reference and Originating Reference are accepted for the associated application-context-name it is checked whether the Destination Reference is known if this check is required by the process that calls the macro.

If the Destination Reference (e.g. a subscribers IMSI) is unknown, the MAP_OPEN Response with

-	Result set to Dialogue Refused;

-	Refuse Reason set to Invalid Destination Reference;

-	Application Context Name set to the highest version supported,

is returned and the macro terminates with unsuccessful outcome.

Else, if the Destination Reference is accepted or if no check is required, the MAP_OPEN Response with

-	Result set to Dialogue Accepted; and

-	Application Context Name set to the received value,

is returned and the macro terminates successfully.

21.1.2	Macro Receive_Open_Confirmation

This macro is used by a user procedures after it requested opening of a dialogue towards a peer entity.

On receipt of a MAP_OPEN Confirmation with a "Result" parameter indicating "Dialogue Accepted", the macro terminates successfully.

If the "Result" parameter indicates "Dialogue Refused", the "Refuse-reason" parameter is examined. If the "Refuse-reason" parameter indicates "Potential Version Incompatibility", the macro terminates in a way that causes restart of the dialogue by using the version 1 protocol.

If the "Refuse-reason" parameter indicates "Application Context Not Supported" and if the received Application Context Name indicates "Version 1", the macro terminates in a way that causes restart of the dialogue by using the version 1 protocol. If the received Application Context Name does not indicate "Version 1", the dialogue is terminated with unsuccessful outcome.

If the "Refuse-reason" parameter indicates neither "Potential Version Incompatibility" nor "Application Context Not Supported", the dialogue is terminated with unsuccessful outcome.

If a MAP_U_ABORT, a MAP_P_ABORT or a MAP_NOTICE Indication is received, the dialogue is terminated with unsuccessful outcome.

Figure 21.1/1

.G.C:\USER\MAP\MAP-470\EPS21\121-11.EPS;18.2 cm;23 cm;EPS

Figure 21.1/2

.G.C:\USER\MAP\MAP-470\EPS21\121-12.EPS;18.2 cm;23 cm;EPS

Figure 21.1/3

.G.C:\USER\MAP\MAP-470\EPS21\121-13.EPS;18.2 cm;23 cm;EPS

21.2	Macros to check the content of indication and confirmation primitives

21.2.1	Macro Check_Indication

If a parameter required by the application is missing from the indication, the macro takes the error exit, with a user error of "Data Missing".

If a parameter not expected by the application is present in the indication, or an expected parameter has a value not in the set of values permitted by the application, the macro takes the error exit, with a user error of "Unexpected Data Value".

Otherwise the macro takes the "OK" exit.

The macro is shown in figure 21.2/1.

21.2.2	Macro Check_Confirmation

If the confirmation contains a Provider error or User error the macro takes the respective exit.

Otherwise, if a parameter required by the application is missing from the confirmation, or a parameter not expected by the application is present in the confirmation, or an expected parameter has a value not in the set of values permitted by the application, the macro takes the Data error exit.

Otherwise the macro takes the "OK" exit.

The macro is shown in figure 21.2/2.

Figure 21.2/1

.G.C:\USER\MAP\MAP-470\EPS21\121-21.EPS;18.2 cm;23 cm;EPS

Figure 21.2/2

.G.C:\USER\MAP\MAP-470\EPS21\121-22.EPS;18.2 cm;23 cm;EPS

21.3	The Page macros

21.3.1	Macro PAGE_MSC

This macro is called in case an incoming call set-up, an unstructured SS notification or a mobile terminating short message is to be delivered to the MS and the current location area identity of the MS is known in VLR (see figure 21.3/1).

When the MSC receives a MAP_PAGE indication, parameter checks are performed first (macro Check_Indication, see section 21.2). In case parameter errors are detected, a MAP_PAGE response containing the appropriate error cause is returned and the macro terminates with unsuccessful outcome.

Thereafter, several checks on the indication content are performed. The macro terminates by returning the MAP_PAGE response with error

Unknown Location Area if the LAI is not known in the MSC;

System Failure if the call, the SMS or SS transaction for this subscriber has been released by the calling subscriber in the meantime.

At next, it is checked if a radio connection for the given IMSI already exists. If so,

-	in case of mobile terminating call set-up it is determined whether the busy condition can be established by the MSC (see GSM 02.01 for a definition of busy states). A MAP_PAGE response with error Busy Subscriber, qualified by either More Calls Allowed or No More Calls Allowed, is returned if the MSC determines that the MS is busy. The macro terminates thereafter with unsuccessful outcome;

-	in case the service requested is short message service or an unstructured SS notification, or if the service is mobile terminating call set-up, but the existing connection is for signalling purpose only (i.e. a service different from call set-up), the access connection status is set according the characteristics of the existing connection (i.e. RR-connection established, ciphering mode on/off, MM connection existing and authenticated or not), and the macro terminates with successful outcome.

If no connection for the given IMSI exists, paging is initiated at the radio interface within all cells of the location area indicated by the VLR, using the TMSI if provided by the VLR, or the IMSI, else. The IMSI will also be used to determine the page group (see GSM 04.08). There are several possible outcomes of paging:

-	The MS responds to paging, causing the access connection status to be set accordingly (i.e. no RR-connection, other values are not regarded then), and the macro terminates with successful outcome.

-	There is no response from the MS, causing sending of a MAP_PAGE response primitive with user error Absent Subscriber before the macro terminates with unsuccessful outcome.

-	The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released in between (indicated in the SDL by the input signal I-REL). The MAP transaction towards the VLR will be released in this case by a MAP_U_ABORT request, and the unsuccessful macro termination will indicate transaction termination.

-	The MAP transaction towards the VLR may be released in between by receiving a MAP_U_ABORT or MAP_P_ABORT indication. The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released then (indicated in the SDL by the output signal I-REL), and the unsuccessful macro termination will indicate transaction termination.

-	A MAP_NOTICE indication may be received on the VLR transaction. The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released then (indicated in the SDL by the output signal I-REL), the MAP transaction towards the VLR is released by sending a MAP_U_ABORT request, and the unsuccessful macro termination will indicate transaction termination.

21.3.2	Macro Search_for_ms_MSC

This macro (see figure 21.3/2) is called in case an incoming call set-up, an unstructured SS notification or a mobile terminating short message is to be delivered to the MS and the current location area identity of the MS is not known in VLR.

When the MSC receives a MAP_SEARCH_FOR_MS Indication, parameter checks are performed first (macro Check_indication, see section 21.2). In case parameter errors are detected, a MAP_SEARCH_FOR_MS response containing the appropriate error cause is returned and the macro terminates with unsuccessful outcome.

Thereafter, it is checked whether the call, the SMS or SS transaction still exists in MSC. The macro terminates by returning the MAP_SEARCH_FOR_MS response with error System Failure if it has been released by the calling subscriber in the meantime.

At next, it is checked if a radio connection for the given IMSI already exists. If so,

-	in case of mobile terminating call set-up it is determined whether the busy condition can be established by the MSC (see GSM 02.01 for a definition of busy states). A MAP_SEARCH_FOR_MS response with error Busy Subscriber, qualified by either More Calls Allowed or No More Calls Allowed, is returned if the MSC determines that the subscriber is busy. The macro terminates thereafter with unsuccessful outcome;

-	in case the service requested is short message service or an unstructured SS notification, or if the service is mobile terminating call set-up, but the existing connection is for signalling purpose only (i.e. a service different from call set-up), a MAP_SEARCH_FOR_MS response containing the IMSI and current location area identification of the called MS is returned to the VLR. The access connection status is set according the characteristics of the existing connection (i.e. RR-connection established, ciphering mode on/off, MM connection existing and authenticated or not), and the macro terminates with successful outcome.

If no connection for the given IMSI exists, paging is initiated at the radio interface within all cells of all location areas of the VLR, using the IMSI to identify the subscriber as to determine the page group (see GSM 04.08). There are several possible outcomes of paging:

-	The MS responds to paging, causing a MAP_SEARCH_FOR_MS response containing the IMSI and current location area identification of the called MS to be returned to the VLR. The access connection status will be set accordingly (i.e. no RR-connection, other values are not regarded then), and the macro terminates with successful outcome.

-	There is no response from the MS, causing sending of a MAP_SEARCH_FOR_MS response primitive with user error Absent Subscriber before the macro terminates with unsuccessful outcome.

-	The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released in between (indicated in the SDL by the input signal I-REL). The MAP transaction towards the VLR will be released in this case by a MAP_U_ABORT request, and the unsuccessful macro termination will indicate transaction termination.

-	The MAP transaction towards the VLR may be released in between by receiving a MAP_U_ABORT or MAP_P_ABORT indication. The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released then (indicated in the SDL by the output signal I-REL), and the unsuccessful macro termination will indicate transaction termination.

-	A MAP_NOTICE indication may be received on the VLR transaction. The call handling connection or MAP transaction on which the call, SMS or unstructured SS notification is waiting for delivery, is released then (indicated in the SDL by the output signal I-REL), the MAP transaction towards the VLR is released by sending a MAP_U_ABORT request, and the unsuccessful macro termination will indicate transaction termination.

Figure 21.3/1

.G.C:\USER\MAP\MAP-470\EPS21\121-31.EPS;18.2 cm;23 cm;EPS

Figure 21.3/2

.G.C:\USER\MAP\MAP-470\EPS21\121-32.EPS;18.2 cm;23 cm;EPS

21.4	Macros for handling an Access Request

These macros are invoked when a mobile station accesses the network, e.g. to set up an outgoing call or when responding to paging. The macro handles identification and authentication of the mobile subscriber as well as invocation of security related features (see GSM Rec. 02.09).

21.4.1	Macro Process_Access_Request_MSC

This macro is invoked by any procedure receiving an access request from the mobile station, e.g. the page response at mobile terminating call set-up or the request for outgoing call set-up.

If no dialogue with the VLR exists (e.g. within the procedure for outgoing call set-up), the MSC will open a dialogue towards the VLR by sending a MAP_OPEN request without any user specific parameters.

In any case, the parameters received from the MS are mapped to a MAP_PROCESS_ACCESS_REQUEST request primitive, containing:

-	the received subscriber identification (IMSI, TMSI) or - in case of emergency call set-up - an IMEI;

-	the CM service type, indicating the type of request;

-	the status of the access connection, i.e. whether a connection to this MS already exists and if so, whether it is already authenticated and ciphered;

-	the current location area id of the mobile station; and

-	the cksn received from the mobile station.

If opening of the dialogue was required, the MSC will wait for the dialogue confirmation (see macro Receive_Open_Confirmation, section 21.1), leading either to:

-	immediate unsuccessful exit from the macro, in case no dialogue is possible;

-	reversion to MAP version one dialogue if indicated by the VLR. The macro terminates with unsuccessful outcome, as the complete dialogue will be covered by the version one procedure, so that no further action from the calling process is required;

-	continuation as given below, if the dialogue is accepted by the VLR.

The MSC waits then for the MAP_PROCESS_ACCESS_REQUEST confirmation. In between, several other indications may be received from the VLR:

-	The MSC may receive a MAP_PROVIDE_IMSI indication, handled by the macro Obtain_IMSI_MSC defined in section 21.8. In case of positive outcome, the procedure continues waiting for the MAP_PROCESS_ACCESS_REQUEST confirmation, else the macro terminates with unsuccessful outcome.

-	The MSC may receive a MAP_AUTHENTICATE indication, handled by the macro Authenticate_MSC defined in section 21.5. In case of positive outcome, the procedure continues waiting for the MAP_PROCESS_ACCESS_REQUEST confirmation, else the macro terminates with unsuccessful outcome.

-	The MSC may receive a MAP_TRACE_SUBSCRIBER_ACTIVITY indication, handled by the macro Trace_Subscriber_Activity_MSC defined in section 21.9.

-	The MSC may receive a MAP_SET_CIPHERING_MODE indication, which will be stored for initiating ciphering later on.

-	The MSC may receive a MAP_CHECK_IMEI indication, handled by the macro Check_IMEI_MSC defined in section 21.6. In case of positive outcome, the procedure continues waiting for the MAP_PROCESS_ACCESS_REQUEST confirmation, else the macro terminates with unsuccessful outcome.

-	The MSC may receive a MAP_Obtain_IMEI indication, handled by the macro Obtain_IMEI_MSC defined in section 21.6. In case of positive outcome, the procedure continues waiting for the MAP_PROCESS_ACCESS_REQUEST confirmation, else the macro terminates with unsuccessful outcome.

-	The MSC may receive a MAP_U_ABORT or MAP_P_ABORT indication, or a premature MAP_CLOSE indication from the VLR. In all these cases, the macro terminates with unsuccessful outcome, after sending the appropriate reject towards the MS (see GSM Technical Specification 09.10).

-	The MSC may receive a MAP_NOTICE indication from the VLR. In this case, the dialogue towards the VLR is terminated by a MAP_U_ABORT primitive, the appropriate reject is sent towards the MS (see GSM Technical Specification 09.10), and the macro terminates with unsuccessful outcome.

-	The MSC may receive an indication for release of the radio path, in which case the dialogue towards the VLR will be terminated by a MAP_U_ABORT primitive, containing the diagnostic information Radio Channel Release.

When the MAP_PROCESS_ACCESS_REQUEST confirmation is received, the parameters of this primitive are checked first. In case of unsuccessful outcome of the service, the MAP User Error received is mapped onto the appropriate radio interface message (see GSM Technical Specification 09.10), before the macro terminates with unsuccessful outcome.

In case of positive outcome of the service, ciphering is initiated on the radio path, if this had been requested by the VLR (see above). Otherwise, if the access request was not triggered by a page response from the MS, the access request is accepted explicitly by sending a CM_Service_Accept message to the MS. If the access request was triggered by a page response from the MS then no CM Service Accept message is sent.

After ciphering has been initiated, the MSC will wait for the MAP_FORWARD_NEW_TMSI indication from the VLR. While waiting, the MSC may receive:

-	a MAP_U_ABORT or MAP_P_ABORT indication, or a premature MAP_CLOSE indication from the VLR. In these cases, the macro terminates with unsuccessful outcome, after sending a release request towards the MS (see GSM Technical Specification 09.10);

-	a MAP_NOTICE indication from the VLR. In this case, the dialogue towards the VLR is terminated by a MAP_U_ABORT primitive, the appropriate reject is sent towards the MS (see GSM Technical Specification 09.10), and the macro terminates with unsuccessful outcome;

-	an indication for release of the radio path, in which case the dialogue towards the VLR will be terminated by a MAP_U_ABORT primitive, containing the diagnostic information Radio Channel Release;

-	a MAP_DELIMITER request from the VLR. This will be taken as a successful outcome of the macro (i.e. the VLR did not require TMSI reallocation), and it terminates successfully;

-	an A_SETUP request from the MS. This will be saved for handling by the procedure which invoked the macro Process_Access_Request_MSC after the macro has terminated.

When the MAP_FORWARD_NEW_TMSI indication is received in the MSC, the TMSI Reallocation Command is sent to the MS, and the MSC waits for an acknowledgement from the MS. In case a positive acknowledgement is received, the MSC sends an empty MAP_FORWARD_NEW_TMSI response primitive to the VLR and terminates successfully. Else, the dialogue is terminated locally (MAP_CLOSE_Req with Release method Prearranged End) without any further action.

If the MSC receives an A_SETUP request while it is waiting for the TMSI acknowledgement from the MS, the A_SETUP is saved for handling by the procedure which invoked the macro Process_Access_Request_MSC after the macro has terminated.

If the dialogue is aborted by the VLR while waiting for the TMSI acknowledgement from the MS, the MSC regards the access request to be failed and terminates with unsuccessful outcome, after sending a release request towards the MS (see GSM Technical Specification 09.10). The same holds for receipt of a MAP_NOTICE indication, in which case the VLR dialogue is terminated by a MAP_U_ABORT request, additionally.

Figure 21.4/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-41.EPS;18.2 cm;23 cm;EPS

Figure 21.4/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-41.EPS;18.2 cm;23 cm;EPS

Figure 21.4/1 sheet 3

.G.C:\USER\MAP\MAP-470\EPS21\321-41.EPS;18.2 cm;23 cm;EPS

21.4.2	Macro Process_Access_Request_VLR

When the VLR receives a MAP_PROCESS_ACCESS_REQUEST indication, the VLR will check this indication first (macro Check_Indication, see section 21.2). In case of negative outcome, the macro will proceed with the error handling described below.

If the indication data are correct, it is checked first whether the subscriber identification (IMSI or TMSI) is known if included.

-	If the identification is not known, the IMSI may be requested from the mobile station, described in the macro Identification_Procedure (see below) with outcome:

-	OK, if a IMSI known in the VLR has been received;

-	Error, if the VLR did not recognize the subscribers's identity. The macro will proceed with the error handling described below;

-	Aborted, if the transaction to the MSC is released. The macro will terminate immediately with unsuccessful.

In case the identity received is an IMEI, the error System Failure is set and the macro proceeds with the error handling described below. (NOTE: Emergency Call with IMEI may be accepted within the error handling phase.)

For a known subscriber the authentication check is performed next (see macro Authenticate_VLR, section 21.5), if required. If a negative result is received, the VLR proceeds on receipt of user error

-	illegal subscriber depending on the identity used for authentication:

In case IMSI is already used or no new authentication attempt with IMSI shall not be performed (operator option), the error Illegal Subscriber is set and the macro proceeds with the error handling described below.

If a new authentication attempt with IMSI shall be performed, the IMSI is requested from the mobile station (macro Obtain_IMSI_VLR, see section 21.8).

-	The authentication will be performed again if a IMSI known in the VLR is received.

-	The error Unidentified Subscriber is set and the macro proceeds with the error handling described below, if the IMSI received is unknown in VLR.

-	If the IMSI request procedure fails for any other reason, the error System Failure is set and the macro proceeds with the error handling described below.

-	If the dialogue has been aborted during the IMSI request, the macro terminates immediately with unsuccessful outcome;

-	unknown subscriber by setting the error Unidentified Subscriber and proceeding with the error handling described below. NOTE: this can occur only in case of data inconsistency between HLR and VLR;

-	procedure error by setting the error System Failure and proceeding with the error handling described below;

-	null (i.e. the dialogue towards the MSC is terminated) by terminating immediately with unsuccessful outcome.

The mobile station access is accepted in case no authentication is required or after successful authentication. Then, the radio confirmation flag is set to confirmed. If the location information is not confirmed in the HLR, HLR updating will be started as an independent process (Update_Location_VLR, see section 16.1.1.6).

If the subscriber record is marked to be not confirmed by HLR, the error Unidentified Subscriber is set and the macro proceeds with the error handling described below.

If roaming is not allowed in the location area indicated in the Current Location Area Id parameter, the error Roaming Not Allowed qualified by the roaming restriction reason is set and the macro proceeds with the error handling described below.

In case roaming is allowed, the IMSI is set to attached and the process for notifying the HLR that the subscriber is present is started if required (Subscriber Present VLR, see section 21.10).

At next, tracing is invoked if required by the operator (macro Trace_Subscriber_Activity_VLR, see section 21.9). Thereafter,

if ciphering is not required, IMEI checking is invoked if required by the operator (see macro Check_IMEI_VLR defined in section 21.6).

The error Illegal Equipment is set in case of unsuccessful outcome of the IMEI check, the subscriber is marked as detached and the macro proceeds with the error handling described below.

The macro terminates immediately with unsuccessful outcome if the MSC dialogue has been released during the IMEI check.

Else, the macro terminates successfully by returning the MAP_PROCESS_ACCESS_REQUEST response containing the IMSI to indicate acceptance of the MS access.

if ciphering is required, the MAP_SET_CIPHERING_MODE request containing

-	the cipher mode indicating the cipher algorithm required, and

-	the cipher key to be used,

is sent to the MSC.

As a further operator option, IMEI checking may be performed next.

The error Illegal Equipment is set in case of unsuccessful outcome of the IMEI check, the subscriber is marked as detached and the macro proceeds with the error handling described below.

The macro terminates immediately with unsuccessful outcome if the MSC dialogue has been released during the IMEI check.

Else, the macro terminates successfully by returning the MAP_PROCESS_ACCESS_REQUEST response containing the IMSI to indicate acceptance of the MS access.

IF no TMSI reallocation is required (again an operator option), the macro terminates thereafter. Else, TMSI reallocation is performed by sending a MAP_FORWARD_NEW_TMSI request, containing the new TMSI as parameter. The old TMSI will be frozen until an acknowledgement from the MS has been received. Before the macro terminates, the VLR will wait for the MAP_FORWARD_NEW_TMSI response, containing no parameters if reallocation has been confirmed by the MS, or a Provider Error, otherwise, in which case the old TMSI is kept frozen to avoid double allocation. In this case, both the old as the new TMSI are subsequently regarded valid when used by the MS.

Error handling

In case some error is detected during handling the access request, a respective error has been set. Before returning this error cause to the MSC in a MAP_PROCESS_ACCESS_REQUEST response, it need to be checked whether this access is for emergency call set-up, as this will require extra treatment.

If the CM Service type given in the MAP_PROCESS_ACCESS_REQUEST indication is emergency call set-up, it is checked whether EC set-up in the particular error situation is permitted (operator option). If so, it is checked whether the IMEI is required, and if so the IMEI is requested from the MS (macro Obtain_IMEI_VLR, see section 21.6).

The macro will terminate immediately with unsuccessful outcome if the MSC transaction has been aborted during the IMEI retrieval.

In case of an error reported back from IMEI retrieval, MAP_PROCESS_ACCESS_REQUEST response containing the error cause set previously is returned to the MSC, the dialogue is closed (MAP_CLOSE request indicating normal release) and the macro terminates with unsuccessful outcome.

When a subscriber identity required by the operator (IMSI or IMEI) is available, the user error set previously is deleted, the respective identity is returned in the MAP_PROCESS_ACCESS_REQUEST response to indicate acceptance of emergency call, and the macro terminates with successful outcome.

In all other cases, the MAP_PROCESS_ACCESS_REQUEST response containing the error cause set previously is returned to the MSC, the dialogue is closed (MAP_CLOSE request indicating normal release) and the macro terminates with unsuccessful outcome.

21.4.3	Macro Identification Procedure

This macro is invoked by the macro Process_Access_Request_VLR in case the subscribers identity is not known in the VLR.

If the identity received from the MS is an IMSI, the error Unidentified Subscriber will be set and reported back to the calling macro (to be sent in the MAP_PROCESS_ACCESS_REQUEST response). The same error is used in case a TMSI was received from the MS, but the operator does not allow open identification of the MS.

If open identification of the MS is allowed, the macro Obtain_IMSI_VLR is invoked, requesting the subscribers IMSI from the MS (see section 21.8), with outcome

OK, in which case it is checked whether for the IMSI received there exists a subscriber record in the VLR. If so, the macro terminates successfully, else the error Unidentified Subscriber will be set and reported back to the calling macro.

Error, in which case the error System Failure will be set and reported back to the calling macro.

Aborted, i.e. the MSC transaction is released, in which the macro terminates accordingly.

Figure 21.4/2 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-42.EPS;18.2 cm;23 cm;EPS

Figure 21.4/2 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-42.EPS;18.2 cm;23 cm;EPS

Figure 21.4/2 sheet 3

.G.C:\USER\MAP\MAP-470\EPS21\321-42.EPS;18.2 cm;23 cm;EPS

Figure 21.4/3

.G.C:\USER\MAP\MAP-470\EPS21\121-43.EPS;18.2 cm;23 cm;EPS

21.5	Authentication macros and processes

The following macros are used in the GSM network in order to enable authentication of a mobile subscriber.

21.5.1	Macro Authenticate_MSC

This macro is used by the MSC to relay a request for authentication transparently from the VLR to the MS, wait for a response from the MS and to relay the response from the MS back to the VLR. If, while the MSC is waiting for the authentication response, the air interface connection is released or a MAP_U_ABORT, MAP_P_ABORT, MAP_CLOSE or MAP_NOTICE indication is received from the VLR, then necessary connections are released and the Error exit is used. The macro is described in figure 21.5/1.

21.5.2	Macro Authenticate_VLR

This macro is used by the VLR to control the authentication of a subscriber. The macro proceeds as follows:

-	If there are not enough authentication triplets in the VLR to perform the authentication, then the macro "Obtain_Authent_Para_VLR" described below is invoked. If this macro fails, then the corresponding error (Unknown Subscriber or Procedure Error) is returned to the calling process.

-	If there are enough authentication triplets in the VLR, or the Obtain_Authent_Para_VLR macro was successful, then a MAP_AUTHENTICATE request is sent to the MSC. This request contains the RAND and CKSN parameters as indicated in the service description.

-	The VLR then waits for a response from the MSC.

-	If a MAP_U_ABORT, MAP_P_ABORT, MAP_CLOSE or MAP_NOTICE indication is received from the MSC in this wait state, then the necessary connections are aborted and the Procedure Error exit is used.

-	If a MAP_AUTHENTICATE confirmation is received by the VLR, it checks whether the received Signed Result (SRES) is identical to the stored one (see GSM 03.20). If this is not the case, the Illegal Subscriber exit is used. If the SRES values are identical, then the OK exit is used.

-	Before exit, the VLR may fetch a new set of triplets from the HLR. This is done by initiating a separate Obtain_Authent_Sets_VLR process described below.

The macro is described in figure 21.5/2.

21.5.3	Process Obtain_Authentication_Sets_VLR

This process is initiated by the VLR to fetch triplets from a subscriber's HLR in a stand-alone, independent manner. The Obtain_Authent_Para_VLR macro described below is simply called; the process is described in figure 21.5/3.

21.5.4	Macro Obtain_Authent_Para_VLR

This macro is used by the VLR to request authentication triplets from the HLR. The macro proceeds as follows:

-	A connection is opened, and a MAP_SEND_AUTHENTICATION_INFO request sent to the HLR, indicating the preferred authentication information: Authentication Sets or the Authentication Key Ki.

-	If the dialogue opening fails, the Procedure Error exit is reported back to the calling process. Otherwise, the VLR waits for the response from the HLR.

-	If a MAP_SEND_AUTHENTICATION_INFO confirmation is received from the HLR, the VLR checks the received data.

If authentication triplets were requested, then the following answers may be received:

-	Authentication sets, in which case the outcome is successful.

-	Empty response, in which case the VLR may re-use old triplets, if allowed by the PLMN operator.

-	Unknown.

If the authentication key Ki were requested, then the following answers may be received:

-	Ki (and possibly the A3-type), in which case the outcome is successful.

-	Authentication sets, in which case the outcome is successful.

-	Empty response, in which case the VLR may re-use old authentication triplets, if allowed by the PLMN operator.

If the VLR cannot re-use old triplets (or that no such triplets are available) then Procedure Error exits is used.

If the outcome was successful or that re-use of old parameters in the VLR is allowed, then the OK exit is used.

If an "Unknown Subscriber" error is included in the MAP_SEND_AUTHENTICATION_INFO confirm, then the Unknown Subscriber exit is used.

-	If a MAP-U-ABORT, MAP_P_ABORT, MAP_NOTICE or unexpected MAP_CLOSE service indication is received, then open connections are aborted, and a Procedure Error is reported back to the calling process.

The macro is described in figure 21.5/4.

21.5.5	Process Obtain_Auth_Sets_HLR

This process is used by the HLR to obtain authentication triplets from the AuC, upon request from the VLR. The process acts as follows:

-	A MAP_SEND_AUTHENTICATION_INFO indication is received by the HLR.

-	The HLR checks the service indication for errors. If any, they are reported to the VLR in the MAP-SEND_AUTHENTICATION_INFO response. If no errors are detected, authentication triplets are fetched from the AuC. Further details are found in GSM 03.20.

-	If errors are detected they are reported to the VLR in the MAP_SEND_AUTHENTICATION_INFO response. Otherwise the authentication triplets are returned.

The process is described in figure 21.5/5.

Figure 21.5/1

.G.C:\USER\MAP\MAP-470\EPS21\121-51.EPS;18.2 cm;23 cm;EPS

Figure 21.5/2

.G.C:\USER\MAP\MAP-470\EPS21\121-52.EPS;18.2 cm;23 cm;EPS

Figure 21.5/3

.G.C:\USER\MAP\MAP-470\EPS21\121-53.EPS;18.2 cm;23 cm;EPS

Figure 21.5/4 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-54.EPS;18.2 cm;23 cm;EPS

Figure 21.5/4 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-54.EPS;18.2 cm;23 cm;EPS

Figure 21.5/5

.G.C:\USER\MAP\MAP-470\EPS21\121-55.EPS;18.2 cm;23 cm;EPS

21.6	IMEI Handling Macros

The following macros are used in the GSM network in order to enable handling and checking of the mobile equipment identity.

21.6.1	Macro Check_IMEI_MSC

This macro is used by the MSC to receive a request from the VLR, relay it to the EIR, and pass the result from the EIR back to the VLR. The macro proceeds as follows:

-	A MAP_CHECK_IMEI service indication containing only the Invoke Id is received from the VLR.

-	If the IMEI is not available in the MSC, it is requested from the MS using the IDENTITY REQUEST message.

-	When the MS releases the radio resources, a MAP_U_ABORT request indicating "Application procedure Cancellation" is sent to the VLR, and the "error" exit of the macro is used.

-	When the IMEI is known, a connection is set up towards the EIR, and a MAP_CHECK_IMEI service request is sent including the IMEI.

-	If the opening of the dialogue fails, a System Failure is reported to the VLR. Otherwise, the MSC waits for a response from the EIR.

-	When the MAP_CHECK_IMEI service confirm is received, it is checked for errors. Any errors discovered in the MSC lead to the corresponding error (Unexpected Data Value or Data Missing) to be reported to the VLR in the MAP_CHECK_IMEI response. Any errors reported from the EIR are sent directly to the VLR in the MAP_CHECK_IMEI service response. If no errors are detected by or reported to the MSC, the IMEI is added to the service MAP_CHECK_IMEI service response returned to the VLR. The "OK" exit is used in all cases.

-	If the EIR reports MAP_P_ABORT, MAP_U_ABORT, MAP_CLOSE or MAP_NOTICE service indications, the MSC releases the necessary connections, reports a System Failure error back to the VLR in the MAP_CHECK_IMEI response, and uses the macro's "OK" exit.

-	If the EIR or VLR sends a MAP_P_ABORT, MAP_U_ABORT, MAP_CLOSE or MAP_NOTICE indication, then the necessary connections are aborted and the macro terminates, reporting "error" to the calling process.

The macro is described in figure 21.6/1.

21.6.2	Macro Check_IMEI_VLR

This macro is used by the VLR to control the check of a mobile equipment's IMEI. The macro proceeds as follows:

-	A MAP_CHECK_IMEI service request is sent to the MSC, including only the Invoke Id.

-	The VLR then waits for the response from the MSC.

-	A MAP_CHECK_IMEI service confirm including either

-	the IMEI and the Equipment Status, or

-	an error,

is received, the VLR checks whether the response requires that an alarm be generated on the Operation and Maintenance interface. Criterion for such alarms are PLMN operator dependent.

-	The VLR then checks whether the response from the MSC means that service is granted to the mobile station. Criterion for granting service depending on the different equipment statuses or errors received in the MAP_CHECK_IMEI service response are also PLMN operator dependent.

-	If the MSC sends a MAP_P_ABORT, MAP_U_ABORT, MAP_CLOSE or MAP_NOTICE indication, then the MSC connection is aborted (if necessary) and the macro terminates, reporting "Aborted" to the calling process.

The macro is described in figure 21.6/2.

21.6.3	Process Check_IMEI_EIR

This process is used by the EIR to obtain the status of a piece of mobile equipment, upon request from the MSC. The process acts as follows:

-	A connection is opened towards the EIR. If the dialogue opening fails, the process terminates.

-	Otherwise, a MAP_CEHCK_IMEI indication is received by the EIR, containing the IMEI to be checked.

-	The EIR checks the service indication for errors. If any, they are reported to the MSC in the MAP-CHECK_IMEI response. If no errors are detected, the EIR data base function is enquired for the status of the given equipment. Further details are found in GSM 02.16.

-	The status of the equipment (white-listed, grey-listed, black-listed or unknown) is returned to the MSC in the MAP_CHECK_IMEI service response.

-	If a MAP_U_ABORT, MAP_P_ABORT, MAP_NOTICE or MAP_CLOSE indication is received from the MSC at any time during this process, the process in the EIR terminates.

The process is described in figure 21.6/3.

21.6.4	Macro Obtain_IMEI_MSC

This macro is used by the MSC to respond to a request from the VLR to provide the IMEI. The macro proceeds as follows:

-	A MAP_OBTAIN_IMEI service indication containing only the Invoke Id is received from the VLR.

-	If the IMEI is not available in the MSC, it is requested from the MS using the IDENTITY REQUEST message.

-	When the IMEI is known, it is returned to the VLR in the MAP_OBTAIN_IMEI service response. The macro terminates at the "OK" exit.

-	If the IMEI cannot be obtained by the MSC, the System Failure error is reported back to the VLR in the MAP_OBTAIN_IMEI service response. The macro terminates at the "OK" exit.

-	If a MAP_P_ABORT, MAP_U_ABORT, MAP_CLOSE or MAP_NOTICE indication is received from the VLR, the necessary connections are released and the macro terminates at the "Error" exit.

The macro is described in figure 21.6/4.

21.6.5	Macro Obtain_IMEI_VLR

This macro is used by the VLR to obtain the IMEI from the MSC, e.g. to enable handling of emergency calls in case of authentication failure (in which case the IMEI may be used by some operators as an alternative to the IMSI). It proceeds as follows:

-	The MAP_OBTAIN_IMEI service request is sent to the MSC, including only the Invoke Id.

-	The VLR then waits for the response from the MSC.

-	If the IMEI is received in the MAP_OBTAIN_IMEI service response, the macro terminates at the "OK" exit.

-	If the System Failure error is reported in the MAP_OBTAIN_IMEI service response, the "Error" exit is used.

-	If the MSC terminates the dialogue using a MAP_P_ABORT, MAP_U_ABORT, MAP_CLOSE or MAP_NOTICE service indication, the necessary connections are released, and the "Aborted" exit is used for termination of the macro.

The macro is shown in figure 21.6/5.

Figure 21.6/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-61.EPS;18.2 cm;23 cm;EPS

Figure 21.6/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-61.EPS;18.2 cm;23 cm;EPS

Figure 21.6/2

.G.C:\USER\MAP\MAP-470\EPS21\121-62.EPS;18.2 cm;23 cm;EPS

Figure 21.6/3

.G.C:\USER\MAP\MAP-470\EPS21\121-63.EPS;18.2 cm;23 cm;EPS

Figure 21.6/4

.G.C:\USER\MAP\MAP-470\EPS21\121-64.EPS;18.2 cm;23 cm;EPS

Figure 21.6/5

.G.C:\USER\MAP\MAP-470\EPS21\121-65.EPS;18.2 cm;23 cm;EPS

21.7	Insert Subscriber Data Macros

21.7.1	Macro Insert_Subs_Data_VLR

This macro describes the reception of the InsertSubscriberData service indication. This macro is used by any procedure that triggers the reception of subscriber data (e.g. Update Location or Restore Data).

If the VLR does not support any Basic or Supplementary service or the network feature Operator Determined Barring, or there is a problem with Regional Subscription Data then it reports it to the HLR.

If the entire MSC area is restricted due to regional subscription this is reported to the HLR.

The SDL diagram is shown in figure 21.7/1.

Figure 21.7/1

.G.C:\USER\MAP\MAP-470\EPS21\121-71.EPS;18.2 cm;23 cm;EPS

21.7.2	Process Insert_Subs_Data_Stand_Alone_HLR

This process is used by HLR to transfer subscriber data to VLR in a stand alone mode, i.e. in its own dialogue. this is done whenever a change of subscriber data is performed either by the operator or by the subscriber and this change has to be reported to VLR.

The process, after opening the dialogue with VLR, sends as many requests of the InsertSubscriberData service as necessary to transfer the subscriber data. The call to the process "Send_Insert_Subs_Data" (see section 21.7.4) is meant to describe two possible behaviours of the HLR when more than one service request has to be sent:

-	either the HLR handles the requests and the confirmations in parallel; or

-	the HLR sends every request after receiving the confirmation to the previous one.

The process also calls the macro "Control_Tracing_HLR" (see section 21.9.4) by means of which the HLR can report the VLR about the activation of the Tracing mode.

The macro "Wait_for_Insert_Subs_Data_Cnf" (see section 21.7.3) is also called in order to handle every single confirmation.

The SDL diagram is shown in figure 21.7/2.

Figure 21.7/2 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-72.EPS;18.2 cm;23 cm;EPS

Figure 21.7/2 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-72.EPS;18.2 cm;23 cm;EPS

21.7.3	Macro Wait_for_Insert_Subs_Data_Cnf

This macro is used by any process or macro that describes the handling of the reception of the Insert_Subscriber_Data service in HLR (e.g. Update Location or Restore Data).

If the VLR reports the non-support of some Basic or Supplementary service or the network feature Operator Determined Barring then three actions are possible:

-	to ignore the information received;

-	to replace the not supported service;

-	or to perform any other internal action.

The SDL diagram is shown in figure 21.7/3.

Figure 21.7/3

.G.C:\USER\MAP\MAP-470\EPS21\121-73.EPS;18.2 cm;23 cm;EPS

21.7.4	Process Send_Insert_Subs_Data

This process is used by any process or macro where the Insert_Subscriber_Data request is sent to VLR.

The SDL diagram is shown in figure 21.7/4.

Figure 21.7/4

.G.C:\USER\MAP\MAP-470\EPS21\121-74.EPS;18.2 cm;23 cm;EPS

21.8	Request IMSI Macros

21.8.1	Macro Obtain_IMSI_MSC

This macro describes the handling of the request received from the VLR to provide the IMSI of a subscriber (e.g. at Location Updating).

The SDL diagram is shown in figure 21.8/1.

Figure 21.8/1

.G.C:\USER\MAP\MAP-470\EPS21\121-81.EPS;18.2 cm;23 cm;EPS

21.8.2	Macro Obtain_IMSI_VLR

This macro describes the way VLR requests the MSC the IMSI of a subscriber (e.g. at Location Updating).

The SDL diagram is shown in figure 21.8/2.

Figure 21.8/2

.G.C:\USER\MAP\MAP-470\EPS21\121-82.EPS;18.2 cm;23 cm;EPS

21.9	Tracing macros

21.9.1	Macro Trace_Subscriber_Activity_MSC

The Trace_Subscriber_Activity_MSC is invoked in the MSC, when the MSC receives the MAP_TRACE_SUBSCRIBER_ACTIVITY indication from the VLR. The data of the primitive is checked and the tracing in the MSC is started if the content includes no errors. No response is returned to the VLR.

The Trace_Subscriber_Activity_MSC macro is described in the figure 21.9/1.

Figure 21.9/1

.G.C:\USER\MAP\MAP-470\EPS21\121-91.EPS;18.2 cm;23 cm;EPS

21.9.2	Macro Trace_Subscriber_Activity_VLR

The macro Trace_Subscriber_Activity_VLR is invoked, if the subscriber activity is detected by the VLR and the tracing is active. The VLR sends MAP_TRACE_SUBSCRIBER_ACTIVITY request to the MSC. No answer is awaited from the MSC.

The Trace_Subscriber_Activity_VLR macro is shown in the figure 21.9/2.

Figure 21.9/2

.G.C:\USER\MAP\MAP-470\EPS21\121-92.EPS;18.2 cm;23 cm;EPS

21.9.3	Macro Activate_Tracing_VLR

The Activate_Tracing_VLR macro is invoked, when the MAP_ACTIVATE_TRACE_MODE indication is received from the HLR. The primitive is processed in the VLR as follows:

-	if the data contains errors, a data missing ar unexpected data value indication is returned to the HLR;

-	if the tracing is not supported, a facility not supported indication is returned to the HLR;

-	if the tracing buffer does not have any space left for the data, a tracing buffer full indication is returned to the HLR;

-	if no errors is detected, the tracing is set active and a positive acknowledge is returned to the HLR.

The Activate_Tracing_VLR macro is described in the figure 21.9/3.

Figure 21.9/3

.G.C:\USER\MAP\MAP-470\EPS21\121-93.EPS;18.2 cm;23 cm;EPS

21.9.4	Macro Control_Tracing_HLR

The Control_Tracing_HLR macro may be invoked in the HLR, if subscriber related activity is detected. If the tracing is active in the HLR and not active in the VLR, the MAP_ACTIVATE_TRACE_MODE request is sent to the VLR.

The MAP_ACTIVATE_TRACE_MODE confirmation from the VLR is processed as follows:

-	if the primitive contains a successful acknowledge, the tracing in VLR is set active;

-	if the primitive contains errors, the tracing in VLR is set deactive.

The Control_Tracing_HLR macro is shown in the figure 21.9/4.

Figure 21.9/4

.G.C:\USER\MAP\MAP-470\EPS21\121-94.EPS;18.2 cm;23 cm;EPS

21.10	Short Message Alert procedures

21.10.1	Subscriber_Present_VLR process

The Subscriber_Present_VLR process is invoked by the VLR, when the mobile subscriber becomes active and the MNRF flag is set. The general description of the short message alert procedures is in the section 20.4.

The VLR sends the MAP_READY_FOR_SM request to the HLR and waits for the HLR to answer. When receiving the answer, the VLR will act as follows:

-	the MNRF flag is cleared if the procedure is successful;

-	the MNRF flag is not cleared if the procedure is not successful.

The Subscriber_Present_VLR process is shown in the figure 21.10/1.

Figure 21.10/1

.G.C:\USER\MAP\MAP-470\EPS21\121-101.EPS;18.2 cm;23 cm;EPS

21.10.2	Macro Alert_Service_Centre_HLR

The Alert_Service_Centre_HLR macro is initiated when the HLR notices that the Service Centre(s) shall be alerted. The macro starts process Alert_Service_Centre_HLR for every SC address in the MWD list.

In the process Alert_Service_Centre_HLR the HLR sends MAP_ALERT_SERVICE_CENTRE request to the appropriate IWMSC. The MWD entry is deleted when the positive acknowledge is received from the IWMSC. The unsuccessful alert may be repeated. The MWD entry should be purged in the unsuccessful case, at least when a suitable time period has expired.

The Alert_Service_Centre_HLR macro is shown in the figure 21.10/2 and the Alert_Service_Centre_HLR process is shown in the figure 21.10/3.

Figure 21.10/2

.G.C:\USER\MAP\MAP-470\EPS21\121-102.EPS;18.2 cm;23 cm;EPS

Figure 21.10/3

.G.C:\USER\MAP\MAP-470\EPS21\121-103.EPS;18.2 cm;23 cm;EPS

21.11	Co-ordinating process in the HLR to handle messages sent with the application context information retrieval

When the "sendInfoCoordinator-HLR" process receives a MAP-OPEN indication primitive with an application context indicating information retrieval, the macro Receive_Open_Ind (figure 21.1/1) is called.

-	if this macro detects an error, the state machine returns to the Null state;

-	if this macro detects the MAP version 1 application context indicating information retrieval, the MAP protocol machine performs the appropriate MAP version 1 dialogue;

-	otherwise the process waits for the arrival of a MAP specific service primitive.

If a MAP-SEND-IMSI ind primitive is received, the process triggers the SEND_IMSI_HLR process (figure 17.4/2), forwards the MAP-SEND-IMSI ind primitive to the SEND_IMSI_HLR process, and enters the state Relay_Info, ready to process any further signals to or from the SEND_IMSI_HLR process.

If a MAP-SEND-AUTHENTICATION-INFO ind primitive is received, the process triggers the Obtain_Auth_Sets_HLR process (figure 16.3/6), forwards the MAP-SEND-AUTHENTICATION-INFO ind primitive to the Obtain_Auth_Sets_HLR process, and enters the state Relay_Info, ready to process any further signals to or from the Obtain_Auth_Sets_HRL process.

When the process is in the Relay_Info state, messages received from the Provider are relayed to the offspring process and messages received from the offspring process are relayed to the provider, until the process is terminated by a MAP-CLOSE. The MAP-CLOSE is relayed before the process returns to the null state.

The process is shown in figure 21.11/1.

Figure 21.11/1 sheet 1

.G.C:\USER\MAP\MAP-470\EPS21\121-111.EPS;18.2 cm;23 cm;EPS

Figure 21.11/1 sheet 2

.G.C:\USER\MAP\MAP-470\EPS21\221-111.EPS;18.2 cm;23 cm;EPS

Annex A (informative): Cross-reference for abstract syntaxes of MAP

Annex A is not part of the standard, it is included for information purposes only.

For every ASN.1 item such as identifier, type-reference or value-reference the cross-reference allows to locate all occurrences by means of module-name and line numbers. For that purpose line numbers are printed at the left margin in front of each ASN.1 source line starting with 1 for every module.

The items are sorted alphabetically in the cross-reference in a case-insensitive manner. Occurrences of an item are its definition and all its usages such as in exports, imports or within a type or value assignment.

For every item additional information is provided such as kind of item (identifier, value reference, type reference), and tag, associated type and value if applicable.

The cross-reference for a root module includes all modules referred to directly or indirectly via imports. The cross-references for the root modules MAP-Protocol/TCAPMessages and MAP-DialoguePDU are included.

ASN.1 P2.03a   Cross Reference Listing for MAP-Protocol             94-05-05  21:55:50

abort...................................identifier of [APPLICATION 7] IMPLICIT Abort

DEFINED in TCAPMessages           :     56

Abort...................................type reference SEQUENCE

DEFINED in TCAPMessages           :     74

USED in TCAPMessages           :     56

absentSubscriber........................value reference AbsentSubscriber, CHOICE VALUE

DEFINED in MAP-Protocol           :    293

AbsentSubscriber........................type reference ERROR

DEFINED in MAP-Errors             :    167

USED in MAP-Protocol           :    109   293

USED in MAP-CallHandlingOperat :     36    71    87

USED in MAP-SupplementaryServi :     54   244   258

USED in MAP-ShortMessageServic :     42    81    98

USED in MAP-Errors             :     50

absentSubscriber........................identifier of Named Number, 1

DEFINED in MAP-SM-DataTypes       :    100

accessSignalInfo........................identifier of ExternalSignalInfo

DEFINED in MAP-MobileServiceOpera :    207

accessSignalInfo........................identifier of ExternalSignalInfo

DEFINED in MAP-MS-DataTypes       :    175

activateSS..............................value reference ActivateSS, CHOICE VALUE

DEFINED in MAP-Protocol           :    208

ActivateSS..............................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    136

USED in MAP-Protocol           :     60   208

USED in MAP-SupplementaryServi :     20

activateTraceMode.......................value reference ActivateTraceMode, CHOICE VALUE

DEFINED in MAP-Protocol           :    188

ActivateTraceMode.......................type reference OPERATION

DEFINED in MAP-OperationAndMainte :     57

USED in MAP-Protocol           :     41   188

USED in MAP-OperationAndMainte :     18

activateTraceModeArg....................identifier of ActivateTraceModeArg

DEFINED in MAP-OperationAndMainte :     59

ActivateTraceModeArg....................type reference SEQUENCE

DEFINED in MAP-OM-DataTypes       :     37

USED in MAP-OperationAndMainte :     41    59

USED in MAP-OM-DataTypes       :     19

AddressString...........................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :     74

USED in MAP-CommonDataTypes    :     21   118

USED in MAP-OM-DataTypes       :     26    41    59

USED in MAP-SS-DataTypes       :     40    62

USED in MAP-SM-DataTypes       :     29    50    80    86    92   105

alertReason.............................identifier of AlertReason

DEFINED in MAP-SM-DataTypes       :    120

AlertReason.............................type reference ENUMERATED

DEFINED in MAP-SM-DataTypes       :    123

USED in MAP-SM-DataTypes       :    120

alertServiceCentre......................value reference AlertServiceCentre, CHOICE VALUE

DEFINED in MAP-Protocol           :    240

AlertServiceCentre......................type reference OPERATION

DEFINED in MAP-ShortMessageServic :    124

USED in MAP-Protocol           :     80   240

USED in MAP-ShortMessageServic :     23

alertServiceCentreArg...................identifier of AlertServiceCentreArg

DEFINED in MAP-ShortMessageServic :    122

alertServiceCentreArg...................identifier of AlertServiceCentreArg

DEFINED in MAP-ShortMessageServic :    126

AlertServiceCentreArg...................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :    103

USED in MAP-ShortMessageServic :     54   122   126

USED in MAP-SM-DataTypes       :     23

alertServiceCentreWithoutResult...value reference AlertServiceCentreWithoutResult, CHOICE VALUE

DEFINED in MAP-Protocol           :    234

AlertServiceCentreWithoutResult.........type reference OPERATION

DEFINED in MAP-ShortMessageServic :    120

USED in MAP-Protocol           :     79   234

USED in MAP-ShortMessageServic :     22

allAdditionalInfoTransferSS.............value reference SS-Code, '10000000'B

DEFINED in MAP-SS-Code            :     99

allAlternateSpeech-DataCDA..............value reference BearerServiceCode, '00110000'B

DEFINED in MAP-BS-Code            :     65

allAlternateSpeech-DataCDS..............value reference BearerServiceCode, '00111000'B

DEFINED in MAP-BS-Code            :     67

allAsynchronousServices.................value reference BearerServiceCode, '01100000'B

DEFINED in MAP-BS-Code            :     78

allBearerServices.......................value reference BearerServiceCode, '00000000'B

DEFINED in MAP-BS-Code            :     36

allCallCompletionSS.....................value reference SS-Code, '01000000'B

DEFINED in MAP-SS-Code            :     68

allCallOfferingSS.......................value reference SS-Code, '00110000'B

DEFINED in MAP-SS-Code            :     58

allCallRestrictionSS....................value reference SS-Code, '10010000'B

DEFINED in MAP-SS-Code            :    106

allChargingSS...........................value reference SS-Code, '01110000'B

DEFINED in MAP-SS-Code            :     91

allCommunityOfInterest-SS...............value reference SS-Code, '01100000'B

DEFINED in MAP-SS-Code            :     85

allCondForwardingSS.....................value reference SS-Code, '00101000'B

DEFINED in MAP-SS-Code            :     49

allDataCDA-Services.....................value reference BearerServiceCode, '00010000'B

DEFINED in MAP-BS-Code            :     38

allDataCDS-Services.....................value reference BearerServiceCode, '00011000'B

DEFINED in MAP-BS-Code            :     46

allDataCircuitAsynchronous..............value reference BearerServiceCode, '01010000'B

DEFINED in MAP-BS-Code            :     75

allDataCircuitSynchronous...............value reference BearerServiceCode, '01011000'B

DEFINED in MAP-BS-Code            :     81

allDataPDS-Services.....................value reference BearerServiceCode, '00101000'B

DEFINED in MAP-BS-Code            :     60

allDataTeleservices.....................value reference TeleserviceCode, '01110000'B

DEFINED in MAP-TS-Code            :     43

allFacsimileTransmissionServices........value reference TeleserviceCode, '01100000'B

DEFINED in MAP-TS-Code            :     35

allForwardingSS.........................value reference SS-Code, '00100000'B

DEFINED in MAP-SS-Code            :     45

allLineIdentificationSS.................value reference SS-Code, '00010000'B

DEFINED in MAP-SS-Code            :     30

allMultiPartySS.........................value reference SS-Code, '01010000'B

DEFINED in MAP-SS-Code            :     79

allOG-CallsBarred.......................identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    265

allPadAccessCA-Services.................value reference BearerServiceCode, '00100000'B

DEFINED in MAP-BS-Code            :     52

allPLMN-specificBS......................value reference BearerServiceCode, '11010000'B

DEFINED in MAP-BS-Code            :     95

allPLMN-specificSS......................value reference SS-Code, '11110000'B

DEFINED in MAP-SS-Code            :    125

allPLMN-specificTS......................value reference TeleserviceCode, '11010000'B

DEFINED in MAP-TS-Code            :     56

allShortMessageServices.................value reference TeleserviceCode, '00100000'B

DEFINED in MAP-TS-Code            :     31

allSpeechFollowedByDataCDA..............value reference BearerServiceCode, '01000000'B

DEFINED in MAP-BS-Code            :     69

allSpeechFollowedByDataCDS..............value reference BearerServiceCode, '01001000'B

DEFINED in MAP-BS-Code            :     71

allSpeechTransmissionServices...........value reference TeleserviceCode, '00010000'B

DEFINED in MAP-TS-Code            :     27

allSS...................................value reference SS-Code, '00000000'B

DEFINED in MAP-SS-Code            :     26

allSynchronousServices..................value reference BearerServiceCode, '01101000'B

DEFINED in MAP-BS-Code            :     84

allTeleservices.........................value reference TeleserviceCode, '00000000'B

DEFINED in MAP-TS-Code            :     25

allTeleservices-ExeptSMS................value reference TeleserviceCode, '10000000'B

DEFINED in MAP-TS-Code            :     46

aocc....................................value reference SS-Code, '01110010'B

DEFINED in MAP-SS-Code            :     96

aoci....................................value reference SS-Code, '01110001'B

DEFINED in MAP-SS-Code            :     94

AuthenticationSet.......................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    125

USED in MAP-MS-DataTypes       :    123   226

authenticationSet.......................identifier of [1] AuthenticationSet

DEFINED in MAP-MS-DataTypes       :    226

authenticationSetList...................identifier of AuthenticationSetList

DEFINED in MAP-MS-DataTypes       :    119

AuthenticationSetList...................type reference SEQUENCE OF

DEFINED in MAP-MS-DataTypes       :    122

USED in MAP-MS-DataTypes       :    119   193

automaticFacsimileGroup3................value reference TeleserviceCode, '01100010'B

DEFINED in MAP-TS-Code            :     37

badlyFormattedTransactionPortion........identifier of Named Number, 2

DEFINED in TCAPMessages           :    105

badlyStructuredComponent................identifier of Named Number, 2

DEFINED in TCAPMessages           :    181

badPW-FormatTryAgain....................identifier of Named Number, 4

DEFINED in MAP-SS-DataTypes       :    282

badPW-TryAgain..........................identifier of Named Number, 3

DEFINED in MAP-SS-DataTypes       :    280

baic....................................value reference SS-Code, '10011010'B

DEFINED in MAP-SS-Code            :    119

baoc....................................value reference SS-Code, '10010010'B

DEFINED in MAP-SS-Code            :    110

barringOfIncomingCalls..................value reference SS-Code, '10011001'B

DEFINED in MAP-SS-Code            :    117

barringOfOutgoingCalls..................value reference SS-Code, '10010001'B

DEFINED in MAP-SS-Code            :    108

barringServiceActive....................identifier of Named Number, 0

DEFINED in MAP-ER-DataTypes       :     51

basicService............................identifier of BasicServiceCode

DEFINED in MAP-SS-DataTypes       :     61

basicService............................identifier of BasicServiceCode

DEFINED in MAP-SS-DataTypes       :     87

basicService............................identifier of BasicServiceCode

DEFINED in MAP-SS-DataTypes       :    161

basicService............................identifier of BasicServiceCode

DEFINED in MAP-SS-DataTypes       :    200

basicService............................identifier of BasicServiceCode

DEFINED in MAP-SS-DataTypes       :    240

basicService............................identifier of BasicServiceCode

DEFINED in MAP-ER-DataTypes       :     64

BasicServiceCode........................type reference CHOICE

DEFINED in MAP-CommonDataTypes    :    259

USED in MAP-CommonDataTypes    :     43   264   269

USED in MAP-SS-DataTypes       :     45    61    87   161   200   240

USED in MAP-ER-DataTypes       :     34    64

BasicServiceGroupList...................type reference SEQUENCE OF

DEFINED in MAP-CommonDataTypes    :    268

USED in MAP-CommonDataTypes    :     45

USED in MAP-SS-DataTypes       :     46   181   221   247

basicServiceGroupList...................identifier of BasicServiceGroupList

DEFINED in MAP-SS-DataTypes       :    181

basicServiceGroupList...................identifier of BasicServiceGroupList

DEFINED in MAP-SS-DataTypes       :    221

basicServiceGroupList...................identifier of [2] BasicServiceGroupList

DEFINED in MAP-SS-DataTypes       :    247

basicServiceList........................identifier of [1] BasicServiceList

DEFINED in MAP-MS-DataTypes       :    304

BasicServiceList........................type reference SEQUENCE OF

DEFINED in MAP-CommonDataTypes    :    263

USED in MAP-MS-DataTypes       :     77   304

USED in MAP-CommonDataTypes    :     44

bearerService...........................identifier of [2] BearerServiceCode

DEFINED in MAP-CommonDataTypes    :    260

BearerServiceCode.......................type reference OCTET STRING

DEFINED in MAP-BS-Code            :     16

USED in MAP-CommonDataTypes    :     55   250   260

USED in MAP-BS-Code            :     36    38    39    40    41    42   43   44   46

47    48    49    50    52    53   54   55   56

57    58    60    61    62    63   65   67   69

71    75    78    81    84    95   97   98   99

100   101   102   103   104   105  106  107  108

109   110   111

bearerServiceList.......................identifier of [4] BearerServiceList

DEFINED in MAP-MS-DataTypes       :    243

bearerServiceList.......................identifier of [2] BearerServiceList

DEFINED in MAP-MS-DataTypes       :    288

BearerServiceList.......................type reference SEQUENCE OF

DEFINED in MAP-CommonDataTypes    :    249

USED in MAP-MS-DataTypes       :     75   243   288

USED in MAP-CommonDataTypes    :     41

bearerServiceNotProvisioned......value reference BearerServiceNotProvisioned, CHOICE VALUE

DEFINED in MAP-Protocol           :    269

BearerServiceNotProvisioned.............type reference ERROR

DEFINED in MAP-Errors             :    142

USED in MAP-Protocol           :    101   269

USED in MAP-CallHandlingOperat :     33    69

USED in MAP-SupplementaryServi :     41    96   121   149   176   203

USED in MAP-Errors             :     36

begin...................................identifier of [APPLICATION 2] IMPLICIT Begin

DEFINED in TCAPMessages           :     53

Begin...................................type reference SEQUENCE

DEFINED in TCAPMessages           :     61

USED in TCAPMessages           :     53

beginSubscriberActivity..............value reference BeginSubscriberActivity, CHOICE VALUE

DEFINED in MAP-Protocol           :    223

BeginSubscriberActivity.................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    290

USED in MAP-Protocol           :     69   223

USED in MAP-SupplementaryServi :     29

beginSubscriberActivityArg..............identifier of BeginSubscriberActivityArg

DEFINED in MAP-SupplementaryServi :    292

BeginSubscriberActivityArg..............type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    288

USED in MAP-SupplementaryServi :     70   292

USED in MAP-SS-DataTypes       :     32

bicRoam.................................value reference SS-Code, '10011011'B

DEFINED in MAP-SS-Code            :    121

blackListed.............................identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    202

boic....................................value reference SS-Code, '10010011'B

DEFINED in MAP-SS-Code            :    112

boicExHC................................value reference SS-Code, '10010100'B

DEFINED in MAP-SS-Code            :    114

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MobileServiceOpera :    178

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MobileServiceOpera :    186

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MobileServiceOpera :    190

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MobileServiceOpera :    196

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MS-DataTypes       :    142

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MS-DataTypes       :    170

bss-APDU................................identifier of ExternalSignalInfo

DEFINED in MAP-MS-DataTypes       :    180

callBarred..............................value reference CallBarred, CHOICE VALUE

DEFINED in MAP-Protocol           :    294

CallBarred..............................type reference ERROR

DEFINED in MAP-Errors             :    173

USED in MAP-Protocol           :    110   294

USED in MAP-CallHandlingOperat :     37    72

USED in MAP-SupplementaryServi :     43    98   125   153   180   207   232   274

USED in MAP-ShortMessageServic :     43    82

USED in MAP-Errors             :     51

callBarringCause........................identifier of CallBarringCause

DEFINED in MAP-Errors             :    175

CallBarringCause........................type reference ENUMERATED

DEFINED in MAP-ER-DataTypes       :     50

USED in MAP-Errors             :     84   175

USED in MAP-ER-DataTypes       :     20

CallBarringFeature......................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    160

USED in MAP-SS-DataTypes       :    158

callBarringFeatureList..................identifier of CallBarringFeatureList

DEFINED in MAP-SS-DataTypes       :    153

CallBarringFeatureList..................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    156

USED in MAP-SS-DataTypes       :    153

callBarringInfo.........................identifier of [1] CallBarringInfo

DEFINED in MAP-SS-DataTypes       :     72

CallBarringInfo.........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    150

USED in MAP-SS-DataTypes       :     72

calledPartySS-InteractionViolation......identifier of Named Number, 7

DEFINED in MAP-ER-DataTypes       :     58

callReference...........................identifier of [4] CallReference

DEFINED in MAP-OM-DataTypes       :     60

CallReference...........................type reference OCTET STRING

DEFINED in MAP-OM-DataTypes       :     62

USED in MAP-OM-DataTypes       :     60

cancelLocation..........................value reference CancelLocation, CHOICE VALUE

DEFINED in MAP-Protocol           :    134

CancelLocation..........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    124

USED in MAP-Protocol           :     18   134

USED in MAP-MobileServiceOpera :     21

cancelLocationArg.......................identifier of CancelLocationArg

DEFINED in MAP-MobileServiceOpera :    126

CancelLocationArg.......................type reference CHOICE

DEFINED in MAP-MS-DataTypes       :    103

USED in MAP-MobileServiceOpera :     75   126

USED in MAP-MS-DataTypes       :     23

category................................identifier of [2] Category

DEFINED in MAP-MS-DataTypes       :    241

Category................................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    246

USED in MAP-MS-DataTypes       :     74   241

USED in MAP-CommonDataTypes    :     40

ccbs....................................value reference SS-Code, '01000011'B

DEFINED in MAP-SS-Code            :     75

cfb.....................................value reference SS-Code, '00101001'B

DEFINED in MAP-SS-Code            :     51

cfnrc...................................value reference SS-Code, '00101011'B

DEFINED in MAP-SS-Code            :     55

cfnry...................................value reference SS-Code, '00101010'B

DEFINED in MAP-SS-Code            :     53

cfu.....................................value reference SS-Code, '00100001'B

DEFINED in MAP-SS-Code            :     47

channelId...............................identifier of [2] ExternalSignalInfo

DEFINED in MAP-OM-DataTypes       :     67

channelType.............................identifier of ChannelType

DEFINED in MAP-MS-DataTypes       :    148

ChannelType.............................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    153

USED in MAP-MS-DataTypes       :    148

checkIMEI...............................value reference CheckIMEI, CHOICE VALUE

DEFINED in MAP-Protocol           :    166

CheckIMEI...............................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    231

USED in MAP-Protocol           :     29   166

USED in MAP-MobileServiceOpera :     38

classmarkInfo...........................identifier of ClassmarkInfo

DEFINED in MAP-MS-DataTypes       :    149

ClassmarkInfo...........................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    161

USED in MAP-MS-DataTypes       :    149   187

classmarkInfo...........................identifier of [10] ClassmarkInfo

DEFINED in MAP-MS-DataTypes       :    187

clip....................................value reference SS-Code, '00010001'B

DEFINED in MAP-SS-Code            :     33

clir....................................value reference SS-Code, '00010010'B

DEFINED in MAP-SS-Code            :     35

cliRestrictionOption....................identifier of [2] CliRestrictionOption

DEFINED in MAP-SS-DataTypes       :    226

CliRestrictionOption....................type reference ENUMERATED

DEFINED in MAP-SS-DataTypes       :    229

USED in MAP-SS-DataTypes       :    226

colp....................................value reference SS-Code, '00010011'B

DEFINED in MAP-SS-Code            :     37

colr....................................value reference SS-Code, '00010100'B

DEFINED in MAP-SS-Code            :     39

Component...............................type reference CHOICE

DEFINED in TCAPMessages           :    124

USED in TCAPMessages           :     47   115

ComponentPortion.....................type reference  [APPLICATION 12] IMPLICIT SEQUENCE OF

DEFINED in TCAPMessages           :    115

USED in TCAPMessages           :     59    63    67    72

components..............................identifier of ComponentPortion

DEFINED in TCAPMessages           :     59

components..............................identifier of ComponentPortion

DEFINED in TCAPMessages           :     63

components..............................identifier of ComponentPortion

DEFINED in TCAPMessages           :     67

components..............................identifier of ComponentPortion

DEFINED in TCAPMessages           :     72

Continue................................type reference SEQUENCE

DEFINED in TCAPMessages           :     69

USED in TCAPMessages           :     55

continue-ME.............................identifier of [APPLICATION 5] IMPLICIT Continue

DEFINED in TCAPMessages           :     55

controllingMSC..........................identifier of Named Number, 4

DEFINED in MAP-CommonDataTypes    :    238

ct......................................value reference SS-Code, '00110001'B

DEFINED in MAP-SS-Code            :     61

cug.....................................value reference SS-Code, '01100001'B

DEFINED in MAP-SS-Code            :     88

cugIC-CallBarred........................identifier of Named Number, 1

DEFINED in MAP-SS-DataTypes       :    191

cugOG-CallBarred........................identifier of Named Number, 2

DEFINED in MAP-SS-DataTypes       :    192

CUG-CheckInfo...........................type reference SEQUENCE

DEFINED in MAP-CH-DataTypes       :     41

USED in MAP-CH-DataTypes       :     50    59

cug-CheckInfo...........................identifier of [1] CUG-CheckInfo

DEFINED in MAP-CH-DataTypes       :     50

cug-CheckInfo...........................identifier of CUG-CheckInfo

DEFINED in MAP-CH-DataTypes       :     59

CUG-Feature.............................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    199

USED in MAP-SS-DataTypes       :    197

cug-FeatureList.........................identifier of CUG-FeatureList

DEFINED in MAP-SS-DataTypes       :    168

CUG-FeatureList.........................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    196

USED in MAP-SS-DataTypes       :    168

cug-Index...............................identifier of CUG-Index

DEFINED in MAP-SS-DataTypes       :    178

CUG-Index...............................type reference INTEGER

DEFINED in MAP-SS-DataTypes       :    184

USED in MAP-SS-DataTypes       :     22   178   201

cug-Info................................identifier of [2] CUG-Info

DEFINED in MAP-SS-DataTypes       :     73

CUG-Info................................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    166

USED in MAP-SS-DataTypes       :     73

cug-Interlock...........................identifier of CUG-Interlock

DEFINED in MAP-CH-DataTypes       :     42

cug-Interlock...........................identifier of CUG-Interlock

DEFINED in MAP-SS-DataTypes       :    179

CUG-Interlock...........................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :    187

USED in MAP-CH-DataTypes       :     25    42

USED in MAP-SS-DataTypes       :     23   172   179

cug-InterlockList.......................identifier of [3] CUG-InterlockList

DEFINED in MAP-MS-DataTypes       :    306

CUG-InterlockList.......................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    171

USED in MAP-MS-DataTypes       :     57   306

USED in MAP-SS-DataTypes       :     36

cug-OutgoingAccess......................identifier of NULL

DEFINED in MAP-CH-DataTypes       :     43

cug-Reject..............................value reference CUG-Reject, CHOICE VALUE

DEFINED in MAP-Protocol           :    296

CUG-Reject..............................type reference ERROR

DEFINED in MAP-Errors             :    180

USED in MAP-Protocol           :    112   296

USED in MAP-CallHandlingOperat :     39    73

USED in MAP-Errors             :     53

cug-RejectCause.........................identifier of CUG-RejectCause

DEFINED in MAP-Errors             :    182

CUG-RejectCause.........................type reference ENUMERATED

DEFINED in MAP-ER-DataTypes       :     54

USED in MAP-Errors             :     85   182

USED in MAP-ER-DataTypes       :     21

CUG-Subscription........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    177

USED in MAP-SS-DataTypes       :    175

cug-SubscriptionList....................identifier of CUG-SubscriptionList

DEFINED in MAP-SS-DataTypes       :    167

CUG-SubscriptionList....................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    174

USED in MAP-SS-DataTypes       :    167

currentPassword.........................identifier of Password

DEFINED in MAP-SupplementaryServi :    288

cw......................................value reference SS-Code, '01000001'B

DEFINED in MAP-SS-Code            :     71

dataCDA-1200bps.........................value reference BearerServiceCode, '00010010'B

DEFINED in MAP-BS-Code            :     40

dataCDA-1200-75bps......................value reference BearerServiceCode, '00010011'B

DEFINED in MAP-BS-Code            :     41

dataCDA-2400bps.........................value reference BearerServiceCode, '00010100'B

DEFINED in MAP-BS-Code            :     42

dataCDA-300bps..........................value reference BearerServiceCode, '00010001'B

DEFINED in MAP-BS-Code            :     39

dataCDA-4800bps.........................value reference BearerServiceCode, '00010101'B

DEFINED in MAP-BS-Code            :     43

dataCDA-9600bps.........................value reference BearerServiceCode, '00010110'B

DEFINED in MAP-BS-Code            :     44

dataCDS-1200bps.........................value reference BearerServiceCode, '00011010'B

DEFINED in MAP-BS-Code            :     47

dataCDS-2400bps.........................value reference BearerServiceCode, '00011100'B

DEFINED in MAP-BS-Code            :     48

dataCDS-4800bps.........................value reference BearerServiceCode, '00011101'B

DEFINED in MAP-BS-Code            :     49

dataCDS-9600bps.........................value reference BearerServiceCode, '00011110'B

DEFINED in MAP-BS-Code            :     50

dataMissing.............................value reference DataMissing, CHOICE VALUE

DEFINED in MAP-Protocol           :    249

DataMissing.............................type reference ERROR

DEFINED in MAP-Errors             :    107

USED in MAP-Protocol           :     89   249

USED in MAP-MobileServiceOpera :     57   118   129   147   159   199  225  238  267

279   298

USED in MAP-OperationAndMainte :     31    63    75    94

USED in MAP-CallHandlingOperat :     28    63    84

USED in MAP-SupplementaryServi :     38    92   116   144   171   198  229  242  256

271

USED in MAP-ShortMessageServic :     34    76    90   110   130   142

USED in MAP-Errors             :     20

dataPDS-2400bps.........................value reference BearerServiceCode, '00101100'B

DEFINED in MAP-BS-Code            :     61

dataPDS-4800bps.........................value reference BearerServiceCode, '00101101'B

DEFINED in MAP-BS-Code            :     62

dataPDS-9600bps.........................value reference BearerServiceCode, '00101110'B

DEFINED in MAP-BS-Code            :     63

deactivateSS............................value reference DeactivateSS, CHOICE VALUE

DEFINED in MAP-Protocol           :    209

DeactivateSS............................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    163

USED in MAP-Protocol           :     61   209

USED in MAP-SupplementaryServi :     21

deactivateTraceMode.....................value reference DeactivateTraceMode, CHOICE VALUE

DEFINED in MAP-Protocol           :    189

DeactivateTraceMode.....................type reference OPERATION

DEFINED in MAP-OperationAndMainte :     69

USED in MAP-Protocol           :     42   189

USED in MAP-OperationAndMainte :     19

deactivateTraceModeArg..................identifier of DeactivateTraceModeArg

DEFINED in MAP-OperationAndMainte :     71

DeactivateTraceModeArg..................type reference SEQUENCE

DEFINED in MAP-OM-DataTypes       :     50

USED in MAP-OperationAndMainte :     42    71

USED in MAP-OM-DataTypes       :     20

deleteSubscriberData....................value reference DeleteSubscriberData, CHOICE VALUE

DEFINED in MAP-Protocol           :    174

DeleteSubscriberData....................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    271

USED in MAP-Protocol           :     32   174

USED in MAP-MobileServiceOpera :     43

deleteSubscriberDataArg.................identifier of DeleteSubscriberDataArg

DEFINED in MAP-MobileServiceOpera :    273

DeleteSubscriberDataArg.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    302

USED in MAP-MobileServiceOpera :     91   273

USED in MAP-MS-DataTypes       :     47

deleteSubscriberDataRes.................identifier of DeleteSubscriberDataRes

DEFINED in MAP-MobileServiceOpera :    275

DeleteSubscriberDataRes.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    315

USED in MAP-MobileServiceOpera :     92   275

USED in MAP-MS-DataTypes       :     48

derivable...............................identifier of InvokeIdType

DEFINED in TCAPMessages           :    167

DestTransactionID...................type reference  [APPLICATION 9] IMPLICIT TransactionID

DEFINED in TCAPMessages           :     98

USED in TCAPMessages           :     65    70    74

diagnosticInfo..........................identifier of SignalInfo

DEFINED in MAP-ER-DataTypes       :     94

dialoguePortion.........................identifier of DialoguePortion

DEFINED in TCAPMessages           :     58

dialoguePortion.........................identifier of DialoguePortion

DEFINED in TCAPMessages           :     62

dialoguePortion.........................identifier of DialoguePortion

DEFINED in TCAPMessages           :     66

dialoguePortion.........................identifier of DialoguePortion

DEFINED in TCAPMessages           :     71

dialoguePortion.........................identifier of DialoguePortion

DEFINED in TCAPMessages           :     77

DialoguePortion.........................type reference  [APPLICATION 11] EXTERNAL

DEFINED in TCAPMessages           :     82

USED in TCAPMessages           :     58    62    66    71    77

dtid....................................identifier of DestTransactionID

DEFINED in TCAPMessages           :     65

dtid....................................identifier of DestTransactionID

DEFINED in TCAPMessages           :     70

dtid....................................identifier of DestTransactionID

DEFINED in TCAPMessages           :     74

duplicateInvokeID.......................identifier of Named Number, 0

DEFINED in TCAPMessages           :    183

eir.....................................identifier of Named Number, 6

DEFINED in MAP-CommonDataTypes    :    240

emergencyCalls..........................value reference TeleserviceCode, '00010010'B

DEFINED in MAP-TS-Code            :     29

End.....................................type reference SEQUENCE

DEFINED in TCAPMessages           :     65

USED in TCAPMessages           :     54

end-ME..................................identifier of [APPLICATION 4] IMPLICIT End

DEFINED in TCAPMessages           :     54

enterNewPW..............................identifier of Named Number, 1

DEFINED in MAP-SS-DataTypes       :    278

enterNewPW-Again........................identifier of Named Number, 2

DEFINED in MAP-SS-DataTypes       :    279

enterPW.................................identifier of Named Number, 0

DEFINED in MAP-SS-DataTypes       :    277

equipmentNotSM-Equipped.................identifier of Named Number, 2

DEFINED in MAP-ER-DataTypes       :     86

equipmentProtocolError..................identifier of Named Number, 1

DEFINED in MAP-ER-DataTypes       :     85

equipmentStatus.........................identifier of EquipmentStatus

DEFINED in MAP-MobileServiceOpera :    235

EquipmentStatus.........................type reference ENUMERATED

DEFINED in MAP-MS-DataTypes       :    200

USED in MAP-MobileServiceOpera :     86   235

USED in MAP-MS-DataTypes       :     40

eraseSS.................................value reference EraseSS, CHOICE VALUE

DEFINED in MAP-Protocol           :    207

EraseSS.................................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    108

USED in MAP-Protocol           :     59   207

USED in MAP-SupplementaryServi :     19

errorCode...............................identifier of ERROR

DEFINED in TCAPMessages           :    158

ets-300102-1............................identifier of Named Number, 4

DEFINED in MAP-CommonDataTypes    :    186

extensibleUpdateLocationRes.............identifier of ExtensibleUpdateLocationRes

DEFINED in MAP-MS-DataTypes       :     96

ExtensibleUpdateLocationRes.............type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :     99

USED in MAP-MS-DataTypes       :     96

ExternalSignalInfo......................type reference SEQUENCE

DEFINED in MAP-CommonDataTypes    :    163

USED in MAP-MobileServiceOpera :    100   178   186   190   196   207

USED in MAP-MS-DataTypes       :     65   142   170   175   180

USED in MAP-CommonDataTypes    :     24

USED in MAP-OM-DataTypes       :     27    67

USED in MAP-CH-DataTypes       :     32    53    74    75

facilityNotSupported....................value reference FacilityNotSupported, CHOICE VALUE

DEFINED in MAP-Protocol           :    251

FacilityNotSupported....................type reference ERROR

DEFINED in MAP-Errors             :    111

USED in MAP-Protocol           :     91   251

USED in MAP-OperationAndMainte :     33    65    77

USED in MAP-CallHandlingOperat :     30    65    86

USED in MAP-ShortMessageServic :     36    78    93   144

USED in MAP-Errors             :     22

facsimileGroup3AndAlterSpeech...........value reference TeleserviceCode, '01100001'B

DEFINED in MAP-TS-Code            :     36

facsimileGroup4.........................value reference TeleserviceCode, '01100011'B

DEFINED in MAP-TS-Code            :     38

forwardAccessSignalling..............value reference ForwardAccessSignalling, CHOICE VALUE

DEFINED in MAP-Protocol           :    149

ForwardAccessSignalling.................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    188

USED in MAP-Protocol           :     25   149

USED in MAP-MobileServiceOpera :     30

forwardCheckSS-Indication..........value reference ForwardCheckSS-Indication, CHOICE VALUE

DEFINED in MAP-Protocol           :    180

ForwardCheckSS-Indication...............type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    289

USED in MAP-Protocol           :     34   180

USED in MAP-MobileServiceOpera :     47

forwardedToNumber.......................identifier of [4] AddressString

DEFINED in MAP-SS-DataTypes       :     62

forwardedToNumber.......................identifier of [5] ISDN-AddressString

DEFINED in MAP-SS-DataTypes       :     90

forwardedToNumber.......................identifier of [1] ISDN-AddressString

DEFINED in MAP-SS-DataTypes       :    245

forwardedToNumber.......................identifier of [5] ISDN-AddressString

DEFINED in MAP-SS-DataTypes       :    301

forwardedToSubaddress...................identifier of [6] ISDN-SubaddressString

DEFINED in MAP-SS-DataTypes       :     63

forwardedToSubaddress...................identifier of [8] ISDN-SubaddressString

DEFINED in MAP-SS-DataTypes       :     91

forwardedToSubaddress...................identifier of [4] ISDN-SubaddressString

DEFINED in MAP-SS-DataTypes       :    303

forwardingData..........................identifier of ForwardingData

DEFINED in MAP-CH-DataTypes       :     65

ForwardingData..........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    300

USED in MAP-CH-DataTypes       :     26    65

USED in MAP-SS-DataTypes       :     35

ForwardingFeature.......................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :     86

USED in MAP-SS-DataTypes       :     84

forwardingFeatureList...................identifier of ForwardingFeatureList

DEFINED in MAP-SS-DataTypes       :     79

ForwardingFeatureList...................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :     82

USED in MAP-SS-DataTypes       :     79   248

forwardingFeatureList...................identifier of [3] ForwardingFeatureList

DEFINED in MAP-SS-DataTypes       :    248

forwardingInfo..........................identifier of [0] ForwardingInfo

DEFINED in MAP-SS-DataTypes       :     71

ForwardingInfo..........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :     77

USED in MAP-SS-DataTypes       :     71

forwardingOptions.......................identifier of [6] ForwardingOptions

DEFINED in MAP-SS-DataTypes       :     93

ForwardingOptions.......................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :    119

USED in MAP-SS-DataTypes       :     93   305

forwardingOptions.......................identifier of [6] ForwardingOptions

DEFINED in MAP-SS-DataTypes       :    305

forwardingViolation.....................value reference ForwardingViolation, CHOICE VALUE

DEFINED in MAP-Protocol           :    295

ForwardingViolation.....................type reference ERROR

DEFINED in MAP-Errors             :    178

USED in MAP-Protocol           :    111   295

USED in MAP-CallHandlingOperat :     38    75

USED in MAP-Errors             :     52

forwardSM...............................value reference ForwardSM, CHOICE VALUE

DEFINED in MAP-Protocol           :    230

ForwardSM...............................type reference OPERATION

DEFINED in MAP-ShortMessageServic :     84

USED in MAP-Protocol           :     76   230

USED in MAP-ShortMessageServic :     19

forwardSM-Arg...........................identifier of ForwardSM-Arg

DEFINED in MAP-ShortMessageServic :     86

ForwardSM-Arg...........................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :     67

USED in MAP-ShortMessageServic :     52    86

USED in MAP-SM-DataTypes       :     21

generalProblem..........................identifier of [0] IMPLICIT GeneralProblem

DEFINED in TCAPMessages           :    170

GeneralProblem..........................type reference INTEGER

DEFINED in TCAPMessages           :    179

USED in TCAPMessages           :    170

getPassword.............................value reference GetPassword, CHOICE VALUE

DEFINED in MAP-Protocol           :    222

GetPassword.............................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    284

USED in MAP-Protocol           :     68   222

USED in MAP-SupplementaryServi :     28   282

GlobalCellId............................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    223

USED in MAP-MS-DataTypes       :     72   140   146   147   178   184   185

USED in MAP-CommonDataTypes    :     36

USED in MAP-OM-DataTypes       :     29    66

greyListed..............................identifier of Named Number, 2

DEFINED in MAP-MS-DataTypes       :    203

gsm-0408................................identifier of Named Number, 1

DEFINED in MAP-CommonDataTypes    :    181

gsm-0806................................identifier of Named Number, 2

DEFINED in MAP-CommonDataTypes    :    182

gsm-BearerCapability....................identifier of [5] ExternalSignalInfo

DEFINED in MAP-CH-DataTypes       :     74

gsm-BSSMAP..............................identifier of Named Number, 3

DEFINED in MAP-CommonDataTypes    :    183

guidanceInfo............................identifier of GuidanceInfo

DEFINED in MAP-SupplementaryServi :    286

GuidanceInfo............................type reference ENUMERATED

DEFINED in MAP-SS-DataTypes       :    276

USED in MAP-SupplementaryServi :     69   286

USED in MAP-SS-DataTypes       :     31

handoverNumber..........................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    169

handoverNumber..........................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    174

handoverPriority........................identifier of [11] HandoverPriority

DEFINED in MAP-MS-DataTypes       :    150

HandoverPriority........................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    165

USED in MAP-MS-DataTypes       :    150

handoverType............................identifier of HandoverType

DEFINED in MAP-OM-DataTypes       :     65

HandoverType............................type reference ENUMERATED

DEFINED in MAP-OM-DataTypes       :     69

USED in MAP-OM-DataTypes       :     65

hlr.....................................identifier of Named Number, 1

DEFINED in MAP-CommonDataTypes    :    235

HLR-Id..................................type reference IMSI

DEFINED in MAP-CommonDataTypes    :    212

USED in MAP-CommonDataTypes    :    217

hlr-List................................identifier of HLR-List

DEFINED in MAP-MS-DataTypes       :    327

HLR-List................................type reference SEQUENCE OF

DEFINED in MAP-CommonDataTypes    :    216

USED in MAP-MS-DataTypes       :     70   327

USED in MAP-CommonDataTypes    :     34

hlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :     94

hlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    100

hlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    326

hlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    336

hold....................................value reference SS-Code, '01000010'B

DEFINED in MAP-SS-Code            :     73

ho-NumberNotRequired....................identifier of NULL

DEFINED in MAP-MS-DataTypes       :    141

illegalEquipment........................value reference IllegalEquipment, CHOICE VALUE

DEFINED in MAP-Protocol           :    268

IllegalEquipment........................type reference ERROR

DEFINED in MAP-Errors             :    140

USED in MAP-Protocol           :    100   268

USED in MAP-SupplementaryServi :     56   246   260

USED in MAP-ShortMessageServic :     40    96

USED in MAP-Errors             :     35

illegalSS-Operation.....................value reference IllegalSS-Operation, CHOICE VALUE

DEFINED in MAP-Protocol           :    301

IllegalSS-Operation.....................type reference ERROR

DEFINED in MAP-Errors             :    188

USED in MAP-Protocol           :    113   301

USED in MAP-SupplementaryServi :     44   100   127   155   182   209

USED in MAP-Errors             :     56

illegalSubscriber.......................value reference IllegalSubscriber, CHOICE VALUE

DEFINED in MAP-Protocol           :    267

IllegalSubscriber.......................type reference ERROR

DEFINED in MAP-Errors             :    138

USED in MAP-Protocol           :     99   267

USED in MAP-SupplementaryServi :     55   245   259

USED in MAP-ShortMessageServic :     39    95

USED in MAP-Errors             :     34

imei....................................identifier of IMEI

DEFINED in MAP-MobileServiceOpera :    233

IMEI....................................type reference TBCD-STRING

DEFINED in MAP-CommonDataTypes    :    200

USED in MAP-MobileServiceOpera :    102   233

USED in MAP-MS-DataTypes       :     68

USED in MAP-CommonDataTypes    :     32

imsi....................................identifier of IMSI

DEFINED in MAP-OperationAndMainte :     92

imsi....................................identifier of IMSI

DEFINED in MAP-ShortMessageServic :    118

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :     87

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :    104

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :    108

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :    113

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :    118

imsi....................................identifier of [0] IMSI

DEFINED in MAP-MS-DataTypes       :    225

imsi....................................identifier of [0] IMSI

DEFINED in MAP-MS-DataTypes       :    235

imsi....................................identifier of [0] IMSI

DEFINED in MAP-MS-DataTypes       :    303

imsi....................................identifier of IMSI

DEFINED in MAP-MS-DataTypes       :    331

IMSI....................................type reference TBCD-STRING

DEFINED in MAP-CommonDataTypes    :    191

USED in MAP-OperationAndMainte :     50    92

USED in MAP-ShortMessageServic :     62   118

USED in MAP-MS-DataTypes       :     66    87   104   108   113   118  191  225  235

303   331

USED in MAP-CommonDataTypes    :     29   197   212

USED in MAP-OM-DataTypes       :     28    38    51    56

USED in MAP-CH-DataTypes       :     33    57    68

USED in MAP-SS-DataTypes       :     44   289

USED in MAP-SM-DataTypes       :     32    56    76

imsi....................................identifier of [0] IMSI

DEFINED in MAP-CommonDataTypes    :    197

imsi....................................identifier of [0] IMSI

DEFINED in MAP-OM-DataTypes       :     38

imsi....................................identifier of [0] IMSI

DEFINED in MAP-OM-DataTypes       :     51

imsi....................................identifier of [0] IMSI

DEFINED in MAP-OM-DataTypes       :     56

imsi....................................identifier of IMSI

DEFINED in MAP-CH-DataTypes       :     57

imsi....................................identifier of [0] IMSI

DEFINED in MAP-CH-DataTypes       :     68

imsi....................................identifier of IMSI

DEFINED in MAP-SS-DataTypes       :    289

imsi....................................identifier of IMSI

DEFINED in MAP-SM-DataTypes       :     56

imsi....................................identifier of [0] IMSI

DEFINED in MAP-SM-DataTypes       :     76

imsi-WithLMSI...........................identifier of IMSI-WithLMSI

DEFINED in MAP-MS-DataTypes       :    105

IMSI-WithLMSI...........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    112

USED in MAP-MS-DataTypes       :    105

incomingCallsBarredWithinCUG............identifier of Named Number, 0

DEFINED in MAP-ER-DataTypes       :     55

incorrectTransactionPortion.............identifier of Named Number, 3

DEFINED in TCAPMessages           :    106

informServiceCentre.....................value reference InformServiceCentre, CHOICE VALUE

DEFINED in MAP-Protocol           :    238

InformServiceCentre.....................type reference OPERATION

DEFINED in MAP-ShortMessageServic :    133

USED in MAP-Protocol           :     81   238

USED in MAP-ShortMessageServic :     24

informServiceCentreArg..................identifier of InformServiceCentreArg

DEFINED in MAP-ShortMessageServic :    135

InformServiceCentreArg..................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :    108

USED in MAP-ShortMessageServic :     55   135

USED in MAP-SM-DataTypes       :     24

initiatingRelease.......................identifier of Named Number, 4

DEFINED in TCAPMessages           :    187

insertSubscriberData....................value reference InsertSubscriberData, CHOICE VALUE

DEFINED in MAP-Protocol           :    173

InsertSubscriberData....................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    259

USED in MAP-Protocol           :     31   173

USED in MAP-MobileServiceOpera :     42

insertSubscriberDataArg.................identifier of InsertSubscriberDataArg

DEFINED in MAP-MobileServiceOpera :    261

InsertSubscriberDataArg.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    234

USED in MAP-MobileServiceOpera :     89   261

USED in MAP-MS-DataTypes       :     45

insertSubscriberDataRes.................identifier of InsertSubscriberDataRes

DEFINED in MAP-MobileServiceOpera :    263

InsertSubscriberDataRes.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    286

USED in MAP-MobileServiceOpera :     90   263

USED in MAP-MS-DataTypes       :     46

interBSS................................identifier of Named Number, 0

DEFINED in MAP-OM-DataTypes       :     70

interCUG-Restrictions...................identifier of InterCUG-Restrictions

DEFINED in MAP-SS-DataTypes       :    202

InterCUG-Restrictions...................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :    205

USED in MAP-SS-DataTypes       :    202

internationalOGCallsBarred..............identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    266

internationalOGCallsNotToHPLMN-CountryBaidentifier of Named Number, 2

DEFINED in MAP-MS-DataTypes       :    267

interrogateSS...........................value reference InterrogateSS, CHOICE VALUE

DEFINED in MAP-Protocol           :    210

InterrogateSS...........................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    191

USED in MAP-Protocol           :     62   210

USED in MAP-SupplementaryServi :     22

interrogateSS-Res.......................identifier of InterrogateSS-Res

DEFINED in MAP-SupplementaryServi :    195

InterrogateSS-Res.......................type reference CHOICE

DEFINED in MAP-SS-DataTypes       :    243

USED in MAP-SupplementaryServi :     64   195

USED in MAP-SS-DataTypes       :     26

intraBSS................................identifier of Named Number, 1

DEFINED in MAP-OM-DataTypes       :     71

intraCUG-Options........................identifier of IntraCUG-Options

DEFINED in MAP-SS-DataTypes       :    180

IntraCUG-Options........................type reference ENUMERATED

DEFINED in MAP-SS-DataTypes       :    189

USED in MAP-SS-DataTypes       :    180

invalidFormat...........................identifier of Named Number, 1

DEFINED in MAP-ER-DataTypes       :     70

invalidSME-Address......................identifier of Named Number, 5

DEFINED in MAP-ER-DataTypes       :     89

invalidTargetBaseStation............value reference InvalidTargetBaseStation, CHOICE VALUE

DEFINED in MAP-Protocol           :    277

InvalidTargetBaseStation................type reference ERROR

DEFINED in MAP-Errors             :    149

USED in MAP-Protocol           :    103   277

USED in MAP-MobileServiceOpera :     65   172   212

USED in MAP-Errors             :     40

invoke..................................identifier of [1] IMPLICIT Invoke

DEFINED in TCAPMessages           :    125

Invoke..................................type reference SEQUENCE

DEFINED in TCAPMessages           :    133

USED in TCAPMessages           :    125

invokeID................................identifier of InvokeIdType

DEFINED in TCAPMessages           :    134

invokeID................................identifier of InvokeIdType

DEFINED in TCAPMessages           :    145

invokeID................................identifier of InvokeIdType

DEFINED in TCAPMessages           :    157

invokeID................................identifier of CHOICE

DEFINED in TCAPMessages           :    166

InvokeIdType............................type reference INTEGER

DEFINED in TCAPMessages           :    175

USED in TCAPMessages           :     47   134   135   145   157   167

invokeProblem...........................identifier of [1] IMPLICIT InvokeProblem

DEFINED in TCAPMessages           :    171

InvokeProblem...........................type reference INTEGER

DEFINED in TCAPMessages           :    183

USED in TCAPMessages           :    171

ISDN-AddressString......................type reference AddressString

DEFINED in MAP-CommonDataTypes    :    117

USED in MAP-OperationAndMainte :     49    90

USED in MAP-CallHandlingOperat :     50    81

USED in MAP-ShortMessageServic :     61   105

USED in MAP-MS-DataTypes       :     64    89    94   100   109   169  174  179  186

240   326   336

USED in MAP-CommonDataTypes    :     22   208   210

USED in MAP-CH-DataTypes       :     31    49    64    69    71    72

USED in MAP-SS-DataTypes       :     41    90   245   290   301

USED in MAP-SM-DataTypes       :     30    48    78    85    91   104   109

ISDN-SubaddressString...................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    123

USED in MAP-CommonDataTypes    :     23

USED in MAP-SS-DataTypes       :     42    63    91   303

kc......................................identifier of Kc

DEFINED in MAP-MS-DataTypes       :    128

Kc......................................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    135

USED in MAP-MS-DataTypes       :    128   151

kc......................................identifier of [12] Kc

DEFINED in MAP-MS-DataTypes       :    151

Ki......................................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    195

USED in MAP-MS-DataTypes       :    228

ki......................................identifier of [4] Ki

DEFINED in MAP-MS-DataTypes       :    228

linkedID................................identifier of [0] IMPLICIT InvokeIdType

DEFINED in TCAPMessages           :    135

linkedResponseUnexpected................identifier of Named Number, 6

DEFINED in TCAPMessages           :    189

lmsi....................................identifier of [10] LMSI

DEFINED in MAP-MS-DataTypes       :     90

lmsi....................................identifier of LMSI

DEFINED in MAP-MS-DataTypes       :    114

lmsi....................................identifier of LMSI

DEFINED in MAP-MS-DataTypes       :    332

LMSI....................................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    221

USED in MAP-MS-DataTypes       :     71    90   114   332

USED in MAP-CommonDataTypes    :     35

USED in MAP-CH-DataTypes       :     34    73

USED in MAP-SM-DataTypes       :     35    64    77

lmsi....................................identifier of [4] LMSI

DEFINED in MAP-CH-DataTypes       :     73

lmsi....................................identifier of LMSI

DEFINED in MAP-SM-DataTypes       :     64

lmsi....................................identifier of [1] LMSI

DEFINED in MAP-SM-DataTypes       :     77

locationInfo............................identifier of LocationInfo

DEFINED in MAP-MS-DataTypes       :     88

LocationInfo............................type reference CHOICE

DEFINED in MAP-CommonDataTypes    :    207

USED in MAP-MS-DataTypes       :     69    88

USED in MAP-CommonDataTypes    :     33

USED in MAP-SM-DataTypes       :     34    63

locationInfo............................identifier of LocationInfo

DEFINED in MAP-SM-DataTypes       :     63

locationInfoWithLMSI....................identifier of [0] LocationInfoWithLMSI

DEFINED in MAP-SM-DataTypes       :     57

LocationInfoWithLMSI....................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :     62

USED in MAP-SM-DataTypes       :     57

mah.....................................value reference SS-Code, '00110010'B

DEFINED in MAP-SS-Code            :     64

MAP-BS-Code.............................module reference

DEFINED in MAP-BS-Code            :      6

USED in MAP-CommonDataTypes    :     56

MAP-CallHandlingOperations..............module reference

DEFINED in MAP-CallHandlingOperat :      6

USED in MAP-Protocol           :     53

MAP-CH-DataTypes........................module reference

DEFINED in MAP-CH-DataTypes       :      6

USED in MAP-CallHandlingOperat :     46

MAP-CommonDataTypes.....................module reference

DEFINED in MAP-CommonDataTypes    :      6

USED in MAP-MobileServiceOpera :    103

USED in MAP-OperationAndMainte :     51

USED in MAP-CallHandlingOperat :     51

USED in MAP-ShortMessageServic :     63

USED in MAP-Errors             :     94

USED in MAP-MS-DataTypes       :     78

USED in MAP-OM-DataTypes       :     30

USED in MAP-CH-DataTypes       :     35

USED in MAP-SS-DataTypes       :     48

USED in MAP-SM-DataTypes       :     36

USED in MAP-ER-DataTypes       :     35

MAP-Errors..............................module reference

DEFINED in MAP-Errors             :      6

USED in MAP-Protocol           :    125

USED in MAP-MobileServiceOpera :     69

USED in MAP-OperationAndMainte :     37

USED in MAP-CallHandlingOperat :     40

USED in MAP-SupplementaryServi :     57

USED in MAP-ShortMessageServic :     46

MAP-ER-DataTypes........................module reference

DEFINED in MAP-ER-DataTypes       :      6

USED in MAP-Errors             :     89

MAP-MobileServiceOperations.............module reference

DEFINED in MAP-MobileServiceOpera :      6

USED in MAP-Protocol           :     36

MAP-MS-DataTypes........................module reference

DEFINED in MAP-MS-DataTypes       :      6

USED in MAP-MobileServiceOpera :     96

MAP-OM-DataTypes........................module reference

DEFINED in MAP-OM-DataTypes       :      6

USED in MAP-OperationAndMainte :     45

MAP-OperationAndMaintenanceOperations...module reference

DEFINED in MAP-OperationAndMainte :      6

USED in MAP-Protocol           :     46

MAP-Protocol............................module reference

DEFINED in MAP-Protocol           :      6

MAP-ShortMessageServiceOperations.......module reference

DEFINED in MAP-ShortMessageServic :      6

USED in MAP-Protocol           :     83

MAP-SM-DataTypes........................module reference

DEFINED in MAP-SM-DataTypes       :      6

USED in MAP-ShortMessageServic :     57

MAP-SS-Code.............................module reference

DEFINED in MAP-SS-Code            :      6

USED in MAP-SupplementaryServi :     76

USED in MAP-SS-DataTypes       :     53

USED in MAP-ER-DataTypes       :     40

MAP-SS-DataTypes........................module reference

DEFINED in MAP-SS-DataTypes       :      6

USED in MAP-SupplementaryServi :     71

USED in MAP-Errors             :     79

USED in MAP-MS-DataTypes       :     60

USED in MAP-CH-DataTypes       :     27

USED in MAP-ER-DataTypes       :     29

MAP-SupplementaryServiceOperations......module reference

DEFINED in MAP-SupplementaryServi :      6

USED in MAP-Protocol           :     70

MAP-TS-Code.............................module reference

DEFINED in MAP-TS-Code            :      6

USED in MAP-CommonDataTypes    :     51

USED in MAP-SM-DataTypes       :     41

maxAddressLength........................value reference INTEGER, 20

DEFINED in MAP-CommonDataTypes    :    115

USED in MAP-CommonDataTypes    :     74

maxISDN-AddressLength...................value reference INTEGER, 9

DEFINED in MAP-CommonDataTypes    :    121

USED in MAP-CommonDataTypes    :    118

maxISDN-SubaddressLength................value reference INTEGER, 21

DEFINED in MAP-CommonDataTypes    :    161

USED in MAP-CommonDataTypes    :    124

maxNumOfBasicServiceGroups..............value reference INTEGER, 13

DEFINED in MAP-CommonDataTypes    :    271

USED in MAP-CommonDataTypes    :     46   268

USED in MAP-SS-DataTypes       :     47    83   157   196

maxNumOfBasicServices...................value reference INTEGER, 70

DEFINED in MAP-CommonDataTypes    :    266

USED in MAP-CommonDataTypes    :    263

maxNumOfBearerServices..................value reference INTEGER, 50

DEFINED in MAP-CommonDataTypes    :    252

USED in MAP-CommonDataTypes    :    249

maxNumOfCUG.............................value reference INTEGER, 10

DEFINED in MAP-SS-DataTypes       :    194

USED in MAP-SS-DataTypes       :    171   174

maxNumOfHLR-Id..........................value reference INTEGER, 50

DEFINED in MAP-CommonDataTypes    :    219

USED in MAP-CommonDataTypes    :    216

maxNumOfSentParameter...................value reference INTEGER, 6

DEFINED in MAP-MS-DataTypes       :    230

USED in MAP-MS-DataTypes       :    221

maxNumOfSS..............................value reference INTEGER, 30

DEFINED in MAP-SS-DataTypes       :    295

USED in MAP-SS-DataTypes       :    292   297

maxNumOfTeleservices....................value reference INTEGER, 20

DEFINED in MAP-CommonDataTypes    :    257

USED in MAP-CommonDataTypes    :    254

maxNumOfZoneCodes.......................value reference INTEGER, 10

DEFINED in MAP-MS-DataTypes       :    284

USED in MAP-MS-DataTypes       :    278

maxSignalInfoLength.....................value reference INTEGER, 200

DEFINED in MAP-CommonDataTypes    :    172

USED in MAP-CommonDataTypes    :     26   170

USED in MAP-SS-DataTypes       :     43   250

maxUSSD-StringLength....................value reference INTEGER, 160

DEFINED in MAP-SS-DataTypes       :    270

USED in MAP-SS-DataTypes       :    266

mcef-Set................................identifier of Named Number, 2

DEFINED in MAP-SM-DataTypes       :    116

mci.....................................value reference SS-Code, '00010101'B

DEFINED in MAP-SS-Code            :     41

memoryAvailable.........................identifier of Named Number, 1

DEFINED in MAP-SM-DataTypes       :    125

memoryCapacityExceeded..................identifier of Named Number, 0

DEFINED in MAP-SM-DataTypes       :     99

memoryCapacityExceeded..................identifier of Named Number, 0

DEFINED in MAP-ER-DataTypes       :     84

MessageType.............................type reference CHOICE

DEFINED in TCAPMessages           :     51

USED in TCAPMessages           :     47

messageWaitingListFull................value reference MessageWaitingListFull, CHOICE VALUE

DEFINED in MAP-Protocol           :    320

MessageWaitingListFull..................type reference ERROR

DEFINED in MAP-Errors             :    226

USED in MAP-Protocol           :    124   320

USED in MAP-ShortMessageServic :     45   114

USED in MAP-Errors             :     69

mistypedComponent.......................identifier of Named Number, 1

DEFINED in TCAPMessages           :    180

mistypedParameter.......................identifier of Named Number, 2

DEFINED in TCAPMessages           :    185

mistypedParameter.......................identifier of Named Number, 2

DEFINED in TCAPMessages           :    194

mistypedParameter.......................identifier of Named Number, 4

DEFINED in TCAPMessages           :    200

mnrf-Set................................identifier of Named Number, 1

DEFINED in MAP-SM-DataTypes       :    115

moreMessagesToSend......................identifier of NULL

DEFINED in MAP-SM-DataTypes       :     71

msc-AreaRestricted......................identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    297

msc-Number..............................identifier of [1] ISDN-AddressString

DEFINED in MAP-CommonDataTypes    :    210

msc-Number..............................identifier of [1] ISDN-AddressString

DEFINED in MAP-CH-DataTypes       :     69

msisdn..................................identifier of ISDN-AddressString

DEFINED in MAP-OperationAndMainte :     90

msisdn..................................identifier of [1] ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    240

msisdn..................................identifier of [0] ISDN-AddressString

DEFINED in MAP-CH-DataTypes       :     49

msisdn..................................identifier of [2] ISDN-AddressString

DEFINED in MAP-CH-DataTypes       :     71

msisdn..................................identifier of [0] ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :     48

msisdn..................................identifier of [2] ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :     85

msisdn..................................identifier of ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :     91

msisdn..................................identifier of ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :    104

msNotReachable..........................identifier of NULL

DEFINED in MAP-MS-DataTypes       :    337

ms-Present..............................identifier of Named Number, 0

DEFINED in MAP-SM-DataTypes       :    124

multiPTY................................value reference SS-Code, '01010001'B

DEFINED in MAP-SS-Code            :     82

mwd-Set.................................identifier of BOOLEAN

DEFINED in MAP-Errors             :    169

mwd-Set.................................identifier of [2] BOOLEAN

DEFINED in MAP-SM-DataTypes       :     58

mw-Status...............................identifier of MW-Status

DEFINED in MAP-SM-DataTypes       :    110

MW-Status...............................type reference BIT STRING

DEFINED in MAP-SM-DataTypes       :    113

USED in MAP-SM-DataTypes       :    110

negativePW-Check........................value reference NegativePW-Check, CHOICE VALUE

DEFINED in MAP-Protocol           :    311

NegativePW-Check........................type reference ERROR

DEFINED in MAP-Errors             :    215

USED in MAP-Protocol           :    121   311

USED in MAP-SupplementaryServi :     50   160   187   278

USED in MAP-Errors             :     64

networkResource.........................identifier of NetworkResource

DEFINED in MAP-Errors             :    104

networkResource.........................identifier of NetworkResource

DEFINED in MAP-MS-DataTypes       :    323

NetworkResource.........................type reference ENUMERATED

DEFINED in MAP-CommonDataTypes    :    233

USED in MAP-Errors             :     93   104

USED in MAP-MS-DataTypes       :     73   323

USED in MAP-CommonDataTypes    :     37

networkSignalInfo.......................identifier of [10] ExternalSignalInfo

DEFINED in MAP-CH-DataTypes       :     53

networkSignalInfo.......................identifier of [6] ExternalSignalInfo

DEFINED in MAP-CH-DataTypes       :     75

newPassword.............................identifier of Password

DEFINED in MAP-SupplementaryServi :    268

newPasswordsMismatch....................identifier of Named Number, 2

DEFINED in MAP-ER-DataTypes       :     71

noCUG-Restrictions......................identifier of Named Number, 0

DEFINED in MAP-SS-DataTypes       :    190

noHandoverNumberAvailable..........value reference NoHandoverNumberAvailable, CHOICE VALUE

DEFINED in MAP-Protocol           :    279

NoHandoverNumberAvailable...............type reference ERROR

DEFINED in MAP-Errors             :    153

USED in MAP-Protocol           :    105   279

USED in MAP-MobileServiceOpera :     67   161   174

USED in MAP-Errors             :     42

noRadioResourceAvailable............value reference NoRadioResourceAvailable, CHOICE VALUE

DEFINED in MAP-Protocol           :    278

NoRadioResourceAvailable................type reference ERROR

DEFINED in MAP-Errors             :    151

USED in MAP-Protocol           :    104   278

USED in MAP-MobileServiceOpera :     66   173

USED in MAP-Errors             :     41

noReplyConditionTime....................identifier of [5] NoReplyConditionTime

DEFINED in MAP-SS-DataTypes       :     65

NoReplyConditionTime....................type reference INTEGER

DEFINED in MAP-SS-DataTypes       :     68

USED in MAP-SS-DataTypes       :     65    94

noReplyConditionTime....................identifier of [7] NoReplyConditionTime

DEFINED in MAP-SS-DataTypes       :     94

noRoamingNumberAvailable............value reference NoRoamingNumberAvailable, CHOICE VALUE

DEFINED in MAP-Protocol           :    292

NoRoamingNumberAvailable................type reference ERROR

DEFINED in MAP-Errors             :    165

USED in MAP-Protocol           :    108   292

USED in MAP-CallHandlingOperat :     35    88

USED in MAP-Errors             :     49

noSM-RP-OA..............................identifier of [5] NULL

DEFINED in MAP-SM-DataTypes       :     87

noSM-SP-DA..............................identifier of [5] NULL

DEFINED in MAP-SM-DataTypes       :     81

noteInternalHandover....................value reference NoteInternalHandover, CHOICE VALUE

DEFINED in MAP-Protocol           :    192

NoteInternalHandover....................type reference OPERATION

DEFINED in MAP-OperationAndMainte :     84

USED in MAP-Protocol           :     44   192

USED in MAP-OperationAndMainte :     21

noteInternalHO-Arg......................identifier of NoteInternalHO-Arg

DEFINED in MAP-OperationAndMainte :     86

NoteInternalHO-Arg......................type reference SEQUENCE

DEFINED in MAP-OM-DataTypes       :     64

USED in MAP-OperationAndMainte :     44    86

USED in MAP-OM-DataTypes       :     22

noteSubscriberPresent..................value reference NoteSubscriberPresent, CHOICE VALUE

DEFINED in MAP-Protocol           :    232

NoteSubscriberPresent...................type reference OPERATION

DEFINED in MAP-ShortMessageServic :    116

USED in MAP-Protocol           :     78   232

USED in MAP-ShortMessageServic :     21

not-derivable...........................identifier of NULL

DEFINED in TCAPMessages           :    168

numberChanged...........................value reference NumberChanged, CHOICE VALUE

DEFINED in MAP-Protocol           :    257

NumberChanged...........................type reference ERROR

DEFINED in MAP-Errors             :    118

USED in MAP-Protocol           :     93   257

USED in MAP-CallHandlingOperat :     32    67

USED in MAP-Errors             :     26

NumberOfForwarding......................type reference INTEGER

DEFINED in MAP-CH-DataTypes       :     46

USED in MAP-CH-DataTypes       :     52

numberOfForwarding......................identifier of [2] NumberOfForwarding

DEFINED in MAP-CH-DataTypes       :     52

numberOfPW-AttemptsViolation....value reference NumberOfPW-AttemptsViolation, CHOICE VALUE

DEFINED in MAP-Protocol           :    312

NumberOfPW-AttemptsViolation............type reference ERROR

DEFINED in MAP-Errors             :    217

USED in MAP-Protocol           :    122   312

USED in MAP-SupplementaryServi :     51   161   188   279

USED in MAP-Errors             :     65

odb-Data................................identifier of [8] ODB-Data

DEFINED in MAP-MS-DataTypes       :    246

ODB-Data................................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    259

USED in MAP-MS-DataTypes       :    246

odb-GeneralData.........................identifier of ODB-GeneralData

DEFINED in MAP-MS-DataTypes       :    260

ODB-GeneralData.........................type reference BIT STRING

DEFINED in MAP-MS-DataTypes       :    264

USED in MAP-MS-DataTypes       :    260   290

odb-GeneralData.........................identifier of [4] ODB-GeneralData

DEFINED in MAP-MS-DataTypes       :    290

odb-HPLMN-Data..........................identifier of ODB-HPLMN-Data

DEFINED in MAP-MS-DataTypes       :    261

ODB-HPLMN-Data..........................type reference BIT STRING

DEFINED in MAP-MS-DataTypes       :    272

USED in MAP-MS-DataTypes       :    261

omc-Id..................................identifier of [3] AddressString

DEFINED in MAP-OM-DataTypes       :     41

omc-Id..................................identifier of [3] AddressString

DEFINED in MAP-OM-DataTypes       :     59

operationCode...........................identifier of OPERATION

DEFINED in TCAPMessages           :    136

operationCode...........................identifier of OPERATION

DEFINED in TCAPMessages           :    147

operatorBarring.........................identifier of Named Number, 1

DEFINED in MAP-ER-DataTypes       :     52

operatorDeterminedBarring...............identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    257

operatorDeterminedBarring...............identifier of Named Number, 3

DEFINED in MAP-ER-DataTypes       :     48

originatingEntityNumber.................identifier of ISDN-AddressString

DEFINED in MAP-SS-DataTypes       :    290

OrigTransactionID..................type reference  [APPLICATION 8] IMPLICIT TransactionID

DEFINED in TCAPMessages           :     97

USED in TCAPMessages           :     61    69

otid....................................identifier of OrigTransactionID

DEFINED in TCAPMessages           :     61

otid....................................identifier of OrigTransactionID

DEFINED in TCAPMessages           :     69

overrideCategory........................identifier of [1] OverrideCategory

DEFINED in MAP-SS-DataTypes       :    227

OverrideCategory........................type reference ENUMERATED

DEFINED in MAP-SS-DataTypes       :    234

USED in MAP-SS-DataTypes       :    227

overrideDisabled........................identifier of Named Number, 1

DEFINED in MAP-SS-DataTypes       :    236

overrideEnabled.........................identifier of Named Number, 0

DEFINED in MAP-SS-DataTypes       :    235

padAccessCA-1200bps.....................value reference BearerServiceCode, '00100010'B

DEFINED in MAP-BS-Code            :     54

padAccessCA-1200-75bps..................value reference BearerServiceCode, '00100011'B

DEFINED in MAP-BS-Code            :     55

padAccessCA-2400bps.....................value reference BearerServiceCode, '00100100'B

DEFINED in MAP-BS-Code            :     56

padAccessCA-300bps......................value reference BearerServiceCode, '00100001'B

DEFINED in MAP-BS-Code            :     53

padAccessCA-4800bps.....................value reference BearerServiceCode, '00100101'B

DEFINED in MAP-BS-Code            :     57

padAccessCA-9600bps.....................value reference BearerServiceCode, '00100110'B

DEFINED in MAP-BS-Code            :     58

parameter...............................identifier of ANY DEFINED BY operationCode

DEFINED in TCAPMessages           :    137

parameter...............................identifier of ANY DEFINED BY operationCode

DEFINED in TCAPMessages           :    148

parameter...............................identifier of ANY DEFINED BY errorCode

DEFINED in TCAPMessages           :    159

Password................................type reference NumericString

DEFINED in MAP-SS-DataTypes       :    272

USED in MAP-SupplementaryServi :     68   268   288

USED in MAP-SS-DataTypes       :     30

performHandover.........................value reference PerformHandover, CHOICE VALUE

DEFINED in MAP-Protocol           :    143

PerformHandover.........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    163

USED in MAP-Protocol           :     21   143

USED in MAP-MobileServiceOpera :     26

performHO-Arg...........................identifier of PerformHO-Arg

DEFINED in MAP-MobileServiceOpera :    165

PerformHO-Arg...........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    145

USED in MAP-MobileServiceOpera :     78   165

USED in MAP-MS-DataTypes       :     31

performHO-Res...........................identifier of PerformHO-Res

DEFINED in MAP-MobileServiceOpera :    167

PerformHO-Res...........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    173

USED in MAP-MobileServiceOpera :     80   167

USED in MAP-MS-DataTypes       :     32

performSubsequentHandover..........value reference PerformSubsequentHandover, CHOICE VALUE

DEFINED in MAP-Protocol           :    150

PerformSubsequentHandover...............type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    203

USED in MAP-Protocol           :     26   150

USED in MAP-MobileServiceOpera :     31

performSubsequentHO-Arg.................identifier of PerformSubsequentHO-Arg

DEFINED in MAP-MobileServiceOpera :    205

PerformSubsequentHO-Arg.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    183

USED in MAP-MobileServiceOpera :     82   205

USED in MAP-MS-DataTypes       :     33

permanent...............................identifier of Named Number, 0

DEFINED in MAP-SS-DataTypes       :    230

plmn....................................identifier of Named Number, 0

DEFINED in MAP-CommonDataTypes    :    234

plmnRoamingNotAllowed...................identifier of Named Number, 0

DEFINED in MAP-ER-DataTypes       :     47

plmn-SpecificBarringType1...............identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    273

plmn-SpecificBarringType2...............identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    274

plmn-SpecificBarringType3...............identifier of Named Number, 2

DEFINED in MAP-MS-DataTypes       :    275

plmn-SpecificBarringType4...............identifier of Named Number, 3

DEFINED in MAP-MS-DataTypes       :    276

plmn-specificBS-1.......................value reference BearerServiceCode, '11010001'B

DEFINED in MAP-BS-Code            :     97

plmn-specificBS-2.......................value reference BearerServiceCode, '11010010'B

DEFINED in MAP-BS-Code            :     98

plmn-specificBS-3.......................value reference BearerServiceCode, '11010011'B

DEFINED in MAP-BS-Code            :     99

plmn-specificBS-4.......................value reference BearerServiceCode, '11010100'B

DEFINED in MAP-BS-Code            :    100

plmn-specificBS-5.......................value reference BearerServiceCode, '11010101'B

DEFINED in MAP-BS-Code            :    101

plmn-specificBS-6.......................value reference BearerServiceCode, '11010110'B

DEFINED in MAP-BS-Code            :    102

plmn-specificBS-7.......................value reference BearerServiceCode, '11010111'B

DEFINED in MAP-BS-Code            :    103

plmn-specificBS-8.......................value reference BearerServiceCode, '11011000'B

DEFINED in MAP-BS-Code            :    104

plmn-specificBS-9.......................value reference BearerServiceCode, '11011001'B

DEFINED in MAP-BS-Code            :    105

plmn-specificBS-A.......................value reference BearerServiceCode, '11011010'B

DEFINED in MAP-BS-Code            :    106

plmn-specificBS-B.......................value reference BearerServiceCode, '11011011'B

DEFINED in MAP-BS-Code            :    107

plmn-specificBS-C.......................value reference BearerServiceCode, '11011100'B

DEFINED in MAP-BS-Code            :    108

plmn-specificBS-D.......................value reference BearerServiceCode, '11011101'B

DEFINED in MAP-BS-Code            :    109

plmn-specificBS-E.......................value reference BearerServiceCode, '11011110'B

DEFINED in MAP-BS-Code            :    110

plmn-specificBS-F.......................value reference BearerServiceCode, '11011111'B

DEFINED in MAP-BS-Code            :    111

plmn-specificSS-1.......................value reference SS-Code, '11110001'B

DEFINED in MAP-SS-Code            :    127

plmn-specificSS-2.......................value reference SS-Code, '11110010'B

DEFINED in MAP-SS-Code            :    128

plmn-specificSS-3.......................value reference SS-Code, '11110011'B

DEFINED in MAP-SS-Code            :    129

plmn-specificSS-4.......................value reference SS-Code, '11110100'B

DEFINED in MAP-SS-Code            :    130

plmn-specificSS-5.......................value reference SS-Code, '11110101'B

DEFINED in MAP-SS-Code            :    131

plmn-specificSS-6.......................value reference SS-Code, '11110110'B

DEFINED in MAP-SS-Code            :    132

plmn-specificSS-7.......................value reference SS-Code, '11110111'B

DEFINED in MAP-SS-Code            :    133

plmn-specificSS-8.......................value reference SS-Code, '11111000'B

DEFINED in MAP-SS-Code            :    134

plmn-specificSS-9.......................value reference SS-Code, '11111001'B

DEFINED in MAP-SS-Code            :    135

plmn-specificSS-A.......................value reference SS-Code, '11111010'B

DEFINED in MAP-SS-Code            :    136

plmn-specificSS-B.......................value reference SS-Code, '11111011'B

DEFINED in MAP-SS-Code            :    137

plmn-specificSS-C.......................value reference SS-Code, '11111100'B

DEFINED in MAP-SS-Code            :    138

plmn-specificSS-D.......................value reference SS-Code, '11111101'B

DEFINED in MAP-SS-Code            :    139

plmn-specificSS-E.......................value reference SS-Code, '11111110'B

DEFINED in MAP-SS-Code            :    140

plmn-specificSS-F.......................value reference SS-Code, '11111111'B

DEFINED in MAP-SS-Code            :    141

plmn-specificTS-1.......................value reference TeleserviceCode, '11010001'B

DEFINED in MAP-TS-Code            :     58

plmn-specificTS-2.......................value reference TeleserviceCode, '11010010'B

DEFINED in MAP-TS-Code            :     59

plmn-specificTS-3.......................value reference TeleserviceCode, '11010011'B

DEFINED in MAP-TS-Code            :     60

plmn-specificTS-4.......................value reference TeleserviceCode, '11010100'B

DEFINED in MAP-TS-Code            :     61

plmn-specificTS-5.......................value reference TeleserviceCode, '11010101'B

DEFINED in MAP-TS-Code            :     62

plmn-specificTS-6.......................value reference TeleserviceCode, '11010110'B

DEFINED in MAP-TS-Code            :     63

plmn-specificTS-7.......................value reference TeleserviceCode, '11010111'B

DEFINED in MAP-TS-Code            :     64

plmn-specificTS-8.......................value reference TeleserviceCode, '11011000'B

DEFINED in MAP-TS-Code            :     65

plmn-specificTS-9.......................value reference TeleserviceCode, '11011001'B

DEFINED in MAP-TS-Code            :     66

plmn-specificTS-A.......................value reference TeleserviceCode, '11011010'B

DEFINED in MAP-TS-Code            :     67

plmn-specificTS-B.......................value reference TeleserviceCode, '11011011'B

DEFINED in MAP-TS-Code            :     68

plmn-specificTS-C.......................value reference TeleserviceCode, '11011100'B

DEFINED in MAP-TS-Code            :     69

plmn-specificTS-D.......................value reference TeleserviceCode, '11011101'B

DEFINED in MAP-TS-Code            :     70

plmn-specificTS-E.......................value reference TeleserviceCode, '11011110'B

DEFINED in MAP-TS-Code            :     71

plmn-specificTS-F.......................value reference TeleserviceCode, '11011111'B

DEFINED in MAP-TS-Code            :     72

preferentialCUG-Indicator...............identifier of CUG-Index

DEFINED in MAP-SS-DataTypes       :    201

premiumRateEntertainementOGCallsBarred..identifier of Named Number, 4

DEFINED in MAP-MS-DataTypes       :    269

premiumRateInformationOGCallsBarred.....identifier of Named Number, 3

DEFINED in MAP-MS-DataTypes       :    268

prepareHandover.........................value reference PrepareHandover, CHOICE VALUE

DEFINED in MAP-Protocol           :    145

PrepareHandover.........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    152

USED in MAP-Protocol           :     22   145

USED in MAP-MobileServiceOpera :     27

prepareHO-Arg...........................identifier of PrepareHO-Arg

DEFINED in MAP-MobileServiceOpera :    154

PrepareHO-Arg...........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    139

USED in MAP-MobileServiceOpera :     79   154

USED in MAP-MS-DataTypes       :     28

prepareHO-Res...........................identifier of PrepareHO-Res

DEFINED in MAP-MobileServiceOpera :    156

PrepareHO-Res...........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    168

USED in MAP-MobileServiceOpera :     81   156

USED in MAP-MS-DataTypes       :     29

prepareSubsequentHandover..........value reference PrepareSubsequentHandover, CHOICE VALUE

DEFINED in MAP-Protocol           :    153

PrepareSubsequentHandover...............type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    192

USED in MAP-Protocol           :     27   153

USED in MAP-MobileServiceOpera :     32

prepareSubsequentHO-Arg.................identifier of PrepareSubsequentHO-Arg

DEFINED in MAP-MobileServiceOpera :    194

PrepareSubsequentHO-Arg.................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    177

USED in MAP-MobileServiceOpera :     83   194

USED in MAP-MS-DataTypes       :     30

previousRoamingNumber...................identifier of [3] ISDN-AddressString

DEFINED in MAP-CH-DataTypes       :     72

problem.................................identifier of CHOICE

DEFINED in TCAPMessages           :    169

processAccessSignalling..............value reference ProcessAccessSignalling, CHOICE VALUE

DEFINED in MAP-Protocol           :    148

ProcessAccessSignalling.................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    184

USED in MAP-Protocol           :     24   148

USED in MAP-MobileServiceOpera :     29

processUnstructuredSS-Data........value reference ProcessUnstructuredSS-Data, CHOICE VALUE

DEFINED in MAP-Protocol           :    211

ProcessUnstructuredSS-Data..............type reference OPERATION

DEFINED in MAP-SupplementaryServi :    212

USED in MAP-Protocol           :     63   211

USED in MAP-SupplementaryServi :     23

processunstructuredSS-Request..value reference ProcessUnstructuredSS-Request, CHOICE VALUE

DEFINED in MAP-Protocol           :    214

ProcessUnstructuredSS-Request...........type reference OPERATION

DEFINED in MAP-SupplementaryServi :    222

USED in MAP-Protocol           :     64   214

USED in MAP-SupplementaryServi :     24

protocolId..............................identifier of ProtocolId

DEFINED in MAP-CommonDataTypes    :    164

ProtocolId..............................type reference ENUMERATED

DEFINED in MAP-CommonDataTypes    :    180

USED in MAP-CommonDataTypes    :    164

provideRoamingNumber....................value reference ProvideRoamingNumber, CHOICE VALUE

DEFINED in MAP-Protocol           :    201

ProvideRoamingNumber....................type reference OPERATION

DEFINED in MAP-CallHandlingOperat :     77

USED in MAP-Protocol           :     52   201

USED in MAP-CallHandlingOperat :     19

provideRoamingNumberArg.................identifier of ProvideRoamingNumberArg

DEFINED in MAP-CallHandlingOperat :     79

ProvideRoamingNumberArg.................type reference SEQUENCE

DEFINED in MAP-CH-DataTypes       :     67

USED in MAP-CallHandlingOperat :     45    79

USED in MAP-CH-DataTypes       :     21

provisionedSS...........................identifier of [7] SS-InfoList

DEFINED in MAP-MS-DataTypes       :    245

purgeMS.................................value reference PurgeMS, CHOICE VALUE

DEFINED in MAP-Protocol           :    135

PurgeMS.................................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    136

USED in MAP-Protocol           :     19   135

USED in MAP-MobileServiceOpera :     22

purgeMS-Arg.............................identifier of PurgeMS-Arg

DEFINED in MAP-MobileServiceOpera :    138

PurgeMS-Arg.............................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    107

USED in MAP-MobileServiceOpera :     76   138

USED in MAP-MS-DataTypes       :     24

pvlr....................................identifier of Named Number, 3

DEFINED in MAP-CommonDataTypes    :    237

pw-RegistrationFailure................value reference PW-RegistrationFailure, CHOICE VALUE

DEFINED in MAP-Protocol           :    310

PW-RegistrationFailure..................type reference ERROR

DEFINED in MAP-Errors             :    211

USED in MAP-Protocol           :    120   310

USED in MAP-SupplementaryServi :     49   277

USED in MAP-Errors             :     63

pw-RegistrationFailureCause.............identifier of PW-RegistrationFailureCause

DEFINED in MAP-Errors             :    213

PW-RegistrationFailureCause.............type reference ENUMERATED

DEFINED in MAP-ER-DataTypes       :     68

USED in MAP-Errors             :     87   213

USED in MAP-ER-DataTypes       :     23

p-abortCause............................identifier of P-AbortCause

DEFINED in TCAPMessages           :     76

P-AbortCause............................type reference  [APPLICATION 10] IMPLICIT INTEGER

DEFINED in TCAPMessages           :    102

USED in TCAPMessages           :     76

rand....................................identifier of RAND

DEFINED in MAP-MS-DataTypes       :    126

RAND....................................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    131

USED in MAP-MS-DataTypes       :    126

readyForSM..............................value reference ReadyForSM, CHOICE VALUE

DEFINED in MAP-Protocol           :    242

ReadyForSM..............................type reference OPERATION

DEFINED in MAP-ShortMessageServic :    137

USED in MAP-Protocol           :     82   242

USED in MAP-ShortMessageServic :     25

readyForSM-Arg..........................identifier of ReadyForSM-Arg

DEFINED in MAP-ShortMessageServic :    139

ReadyForSM-Arg..........................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :    118

USED in MAP-ShortMessageServic :     56   139

USED in MAP-SM-DataTypes       :     25

reason..................................identifier of CHOICE

DEFINED in TCAPMessages           :     75

regionalSubscNotSupported...............identifier of Named Number, 3

DEFINED in MAP-MS-DataTypes       :    300

regionalSubscriptionData................identifier of [10] ZoneCodeList

DEFINED in MAP-MS-DataTypes       :    251

regionalSubscriptionData................identifier of [5] ZoneCodeList

DEFINED in MAP-MS-DataTypes       :    311

regionalSubscriptionResponse............identifier of [5] RegionalSubscriptionResponse

DEFINED in MAP-MS-DataTypes       :    291

RegionalSubscriptionResponse............type reference ENUMERATED

DEFINED in MAP-MS-DataTypes       :    296

USED in MAP-MS-DataTypes       :    292   317

regionalSubscriptionResponse............identifier of [0] RegionalSubscriptionResponse

DEFINED in MAP-MS-DataTypes       :    316

registerPassword........................value reference RegisterPassword, CHOICE VALUE

DEFINED in MAP-Protocol           :    221

RegisterPassword........................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    264

USED in MAP-Protocol           :     67   221

USED in MAP-SupplementaryServi :     27

registerSS..............................value reference RegisterSS, CHOICE VALUE

DEFINED in MAP-Protocol           :    206

RegisterSS..............................type reference OPERATION

DEFINED in MAP-SupplementaryServi :     84

USED in MAP-Protocol           :     58   206

USED in MAP-SupplementaryServi :     18

registerSS-Arg..........................identifier of RegisterSS-Arg

DEFINED in MAP-SupplementaryServi :     86

RegisterSS-Arg..........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :     59

USED in MAP-SupplementaryServi :     61    86

USED in MAP-SS-DataTypes       :     19

reject..................................identifier of [4] IMPLICIT Reject

DEFINED in TCAPMessages           :    128

Reject..................................type reference SEQUENCE

DEFINED in TCAPMessages           :    165

USED in TCAPMessages           :    128

reportSM-DeliveryStatus..............value reference ReportSM-DeliveryStatus, CHOICE VALUE

DEFINED in MAP-Protocol           :    231

ReportSM-DeliveryStatus.................type reference OPERATION

DEFINED in MAP-ShortMessageServic :    101

USED in MAP-Protocol           :     77   231

USED in MAP-ShortMessageServic :     20

reportSM-DeliveryStatusArg..............identifier of ReportSM-DeliveryStatusArg

DEFINED in MAP-ShortMessageServic :    103

ReportSM-DeliveryStatusArg..............type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :     90

USED in MAP-ShortMessageServic :     53   103

USED in MAP-SM-DataTypes       :     22

requestAuthenticationSet................identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    217

requestedBasicServiceViolatesCUG-Constraidentifier of Named Number, 5

DEFINED in MAP-ER-DataTypes       :     57

requestIMSI.............................identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    216

requestKi...............................identifier of Named Number, 4

DEFINED in MAP-MS-DataTypes       :    219

RequestParameter........................type reference ENUMERATED

DEFINED in MAP-MS-DataTypes       :    215

USED in MAP-MS-DataTypes       :    213

requestParameterList....................identifier of RequestParameterList

DEFINED in MAP-MS-DataTypes       :    210

RequestParameterList....................type reference SEQUENCE OF

DEFINED in MAP-MS-DataTypes       :    212

USED in MAP-MS-DataTypes       :    210

requestSubscriberData...................identifier of Named Number, 2

DEFINED in MAP-MS-DataTypes       :    218

reset...................................value reference Reset, CHOICE VALUE

DEFINED in MAP-Protocol           :    179

Reset...................................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    285

USED in MAP-Protocol           :     33   179

USED in MAP-MobileServiceOpera :     46

resetArg................................identifier of ResetArg

DEFINED in MAP-MobileServiceOpera :    287

ResetArg................................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    322

USED in MAP-MobileServiceOpera :     93   287

USED in MAP-MS-DataTypes       :     51

resourceLimitation......................identifier of Named Number, 4

DEFINED in TCAPMessages           :    107

resourceLimitation......................identifier of Named Number, 3

DEFINED in TCAPMessages           :    186

restoreData.............................value reference RestoreData, CHOICE VALUE

DEFINED in MAP-Protocol           :    182

RestoreData.............................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    291

USED in MAP-Protocol           :     35   182

USED in MAP-MobileServiceOpera :     48

restoreDataArg..........................identifier of RestoreDataArg

DEFINED in MAP-MobileServiceOpera :    293

RestoreDataArg..........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    330

USED in MAP-MobileServiceOpera :     94   293

USED in MAP-MS-DataTypes       :     52

restoreDataRes..........................identifier of RestoreDataRes

DEFINED in MAP-MobileServiceOpera :    295

RestoreDataRes..........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    335

USED in MAP-MobileServiceOpera :     95   295

USED in MAP-MS-DataTypes       :     53

result-RR...............................identifier of SEQUENCE

DEFINED in TCAPMessages           :    146

returnError.............................identifier of [3] IMPLICIT ReturnError

DEFINED in TCAPMessages           :    127

ReturnError.............................type reference SEQUENCE

DEFINED in TCAPMessages           :    156

USED in TCAPMessages           :    127

returnErrorProblem......................identifier of [3] IMPLICIT ReturnErrorProblem

DEFINED in TCAPMessages           :    173

ReturnErrorProblem......................type reference INTEGER

DEFINED in TCAPMessages           :    196

USED in TCAPMessages           :    173

returnErrorUnexpected...................identifier of Named Number, 1

DEFINED in TCAPMessages           :    197

ReturnResult............................type reference SEQUENCE

DEFINED in TCAPMessages           :    144

USED in TCAPMessages           :    126   129

returnResultLast........................identifier of [2] IMPLICIT ReturnResult

DEFINED in TCAPMessages           :    126

returnResultNotLast.....................identifier of [7] IMPLICIT ReturnResult

DEFINED in TCAPMessages           :    129

returnResultProblem.....................identifier of [2] IMPLICIT ReturnResultProblem

DEFINED in TCAPMessages           :    172

ReturnResultProblem.....................type reference INTEGER

DEFINED in TCAPMessages           :    192

USED in TCAPMessages           :    172

returnResultUnexpected..................identifier of Named Number, 1

DEFINED in TCAPMessages           :    193

roamingNotAllowed.......................value reference RoamingNotAllowed, CHOICE VALUE

DEFINED in MAP-Protocol           :    266

RoamingNotAllowed.......................type reference ERROR

DEFINED in MAP-Errors             :    131

USED in MAP-Protocol           :     98   266

USED in MAP-MobileServiceOpera :     64   122

USED in MAP-Errors             :     33

roamingNotAllowedCause..................identifier of RoamingNotAllowedCause

DEFINED in MAP-Errors             :    133

RoamingNotAllowedCause..................type reference ENUMERATED

DEFINED in MAP-ER-DataTypes       :     46

USED in MAP-Errors             :     83   133

USED in MAP-ER-DataTypes       :     19

roamingNumber...........................identifier of ISDN-AddressString

DEFINED in MAP-CallHandlingOperat :     81

roamingNumber...........................identifier of [0] ISDN-AddressString

DEFINED in MAP-CommonDataTypes    :    208

roamingNumber...........................identifier of ISDN-AddressString

DEFINED in MAP-CH-DataTypes       :     64

roamingNumber...........................identifier of [3] ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :     78

roamingRestrictionDueToUnsupportedFeaturidentifier of [9] NULL

DEFINED in MAP-MS-DataTypes       :    248

roamingRestrictionDueToUnsupportedFeaturidentifier of [4] NULL

DEFINED in MAP-MS-DataTypes       :    308

routingInfo.............................identifier of RoutingInfo

DEFINED in MAP-CH-DataTypes       :     58

RoutingInfo.............................type reference CHOICE

DEFINED in MAP-CH-DataTypes       :     63

USED in MAP-CH-DataTypes       :     58

routingInfoForSM-Arg....................identifier of RoutingInfoForSM-Arg

DEFINED in MAP-ShortMessageServic :     71

RoutingInfoForSM-Arg....................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :     47

USED in MAP-ShortMessageServic :     50    71

USED in MAP-SM-DataTypes       :     19

routingInfoForSM-Res....................identifier of RoutingInfoForSM-Res

DEFINED in MAP-ShortMessageServic :     73

RoutingInfoForSM-Res....................type reference SEQUENCE

DEFINED in MAP-SM-DataTypes       :     55

USED in MAP-ShortMessageServic :     51    73

USED in MAP-SM-DataTypes       :     20

rss.....................................identifier of Named Number, 7

DEFINED in MAP-CommonDataTypes    :    241

sc-AddressNotIncluded...................identifier of Named Number, 0

DEFINED in MAP-SM-DataTypes       :    114

sc-Congestion...........................identifier of Named Number, 4

DEFINED in MAP-ER-DataTypes       :     88

sendAuthenticationInfo................value reference SendAuthenticationInfo, CHOICE VALUE

DEFINED in MAP-Protocol           :    160

SendAuthenticationInfo..................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    217

USED in MAP-Protocol           :     28   160

USED in MAP-MobileServiceOpera :     35

sendAuthenticationInfoArg...............identifier of SendAuthenticationInfoArg

DEFINED in MAP-MobileServiceOpera :    219

SendAuthenticationInfoArg...............type reference IMSI

DEFINED in MAP-MS-DataTypes       :    191

USED in MAP-MobileServiceOpera :     84   219

USED in MAP-MS-DataTypes       :     36

sendAuthenticationInfoRes...............identifier of SendAuthenticationInfoRes

DEFINED in MAP-MobileServiceOpera :    221

SendAuthenticationInfoRes...............type reference AuthenticationSetList

DEFINED in MAP-MS-DataTypes       :    193

USED in MAP-MobileServiceOpera :     85   221

USED in MAP-MS-DataTypes       :     37

sendEndSignal...........................value reference SendEndSignal, CHOICE VALUE

DEFINED in MAP-Protocol           :    147

SendEndSignal...........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    176

USED in MAP-Protocol           :     23   147

USED in MAP-MobileServiceOpera :     28

sendIdentification......................value reference SendIdentification, CHOICE VALUE

DEFINED in MAP-Protocol           :    137

SendIdentification......................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    141

USED in MAP-Protocol           :     20   137

USED in MAP-MobileServiceOpera :     23

sendIdentificationRes...................identifier of SendIdentificationRes

DEFINED in MAP-MobileServiceOpera :    145

SendIdentificationRes...................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    117

USED in MAP-MobileServiceOpera :     77   145

USED in MAP-MS-DataTypes       :     25

sendIMSI................................value reference SendIMSI, CHOICE VALUE

DEFINED in MAP-Protocol           :    194

SendIMSI................................type reference OPERATION

DEFINED in MAP-OperationAndMainte :     88

USED in MAP-Protocol           :     45   194

USED in MAP-OperationAndMainte :     22

sendParameters..........................value reference SendParameters, CHOICE VALUE

DEFINED in MAP-Protocol           :    171

SendParameters..........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    246

USED in MAP-Protocol           :     30   171

USED in MAP-MobileServiceOpera :     41

sendParametersArg.......................identifier of SendParametersArg

DEFINED in MAP-MobileServiceOpera :    248

SendParametersArg.......................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    208

USED in MAP-MobileServiceOpera :     87   248

USED in MAP-MS-DataTypes       :     43

sendRoutingInfo.........................value reference SendRoutingInfo, CHOICE VALUE

DEFINED in MAP-Protocol           :    200

SendRoutingInfo.........................type reference OPERATION

DEFINED in MAP-CallHandlingOperat :     56

USED in MAP-Protocol           :     51   200

USED in MAP-CallHandlingOperat :     18

sendRoutingInfoArg......................identifier of SendRoutingInfoArg

DEFINED in MAP-CallHandlingOperat :     58

SendRoutingInfoArg......................type reference SEQUENCE

DEFINED in MAP-CH-DataTypes       :     48

USED in MAP-CallHandlingOperat :     43    58

USED in MAP-CH-DataTypes       :     19

sendRoutingInfoForSM....................value reference SendRoutingInfoForSM, CHOICE VALUE

DEFINED in MAP-Protocol           :    229

SendRoutingInfoForSM....................type reference OPERATION

DEFINED in MAP-ShortMessageServic :     69

USED in MAP-Protocol           :     75   229

USED in MAP-ShortMessageServic :     18

sendRoutingInfoRes......................identifier of SendRoutingInfoRes

DEFINED in MAP-CallHandlingOperat :     60

SendRoutingInfoRes......................type reference SEQUENCE

DEFINED in MAP-CH-DataTypes       :     56

USED in MAP-CallHandlingOperat :     44    60

USED in MAP-CH-DataTypes       :     20

SentParameter...........................type reference CHOICE

DEFINED in MAP-MS-DataTypes       :    224

USED in MAP-MS-DataTypes       :    222

sentParameterList.......................identifier of SentParameterList

DEFINED in MAP-MobileServiceOpera :    250

SentParameterList.......................type reference SEQUENCE OF

DEFINED in MAP-MS-DataTypes       :    221

USED in MAP-MobileServiceOpera :     88   250

USED in MAP-MS-DataTypes       :     44

serviceCentreAddress....................identifier of [2] AddressString

DEFINED in MAP-SM-DataTypes       :     50

serviceCentreAddress....................identifier of AddressString

DEFINED in MAP-SM-DataTypes       :     92

serviceCentreAddress....................identifier of AddressString

DEFINED in MAP-SM-DataTypes       :    105

serviceCentreAddressDA..................identifier of [4] AddressString

DEFINED in MAP-SM-DataTypes       :     80

serviceCentreAddressOA..................identifier of [4] AddressString

DEFINED in MAP-SM-DataTypes       :     86

serviceGranted..........................identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    256

servingCellId...........................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    147

servingCellId...........................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    185

shortMessageMO-PP.......................value reference TeleserviceCode, '00100010'B

DEFINED in MAP-TS-Code            :     33

shortMessageMT-PP.......................value reference TeleserviceCode, '00100001'B

DEFINED in MAP-TS-Code            :     32

signalInfo..............................identifier of SignalInfo

DEFINED in MAP-CommonDataTypes    :    165

SignalInfo..............................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    170

USED in MAP-CommonDataTypes    :     25   165

USED in MAP-SM-DataTypes       :     31    70

USED in MAP-ER-DataTypes       :     33    94

sm-DeliveryFailure......................value reference SM-DeliveryFailure, CHOICE VALUE

DEFINED in MAP-Protocol           :    319

SM-DeliveryFailure......................type reference ERROR

DEFINED in MAP-Errors             :    222

USED in MAP-Protocol           :    123   319

USED in MAP-ShortMessageServic :     44    99

USED in MAP-Errors             :     68

sm-DeliveryFailureCause.................identifier of SM-DeliveryFailureCause

DEFINED in MAP-Errors             :    224

SM-DeliveryFailureCause.................type reference CHOICE

DEFINED in MAP-ER-DataTypes       :     73

USED in MAP-Errors             :     88   224

USED in MAP-ER-DataTypes       :     24

sm-DeliveryFailureCauseWithDiagnostic..identifier of SM-DeliveryFailureCauseWithDiagnostic

DEFINED in MAP-ER-DataTypes       :     74

SM-DeliveryFailureCauseWithDiagnostic...type reference SEQUENCE

DEFINED in MAP-ER-DataTypes       :     92

USED in MAP-ER-DataTypes       :     74

sm-DeliveryOutcome......................identifier of SM-DeliveryOutcome

DEFINED in MAP-SM-DataTypes       :     93

SM-DeliveryOutcome......................type reference ENUMERATED

DEFINED in MAP-SM-DataTypes       :     98

USED in MAP-SM-DataTypes       :     93

sm-EnumeratedDeliveryFailureCause.......identifier of SM-EnumeratedDeliveryFailureCause

DEFINED in MAP-ER-DataTypes       :     78

SM-EnumeratedDeliveryFailureCause.......type reference ENUMERATED

DEFINED in MAP-ER-DataTypes       :     83

USED in MAP-ER-DataTypes       :     78    93

sm-EnumeratedDeliveryFailureCause.......identifier of SM-EnumeratedDeliveryFailureCause

DEFINED in MAP-ER-DataTypes       :     93

sm-RP-DA................................identifier of SM-RP-DA

DEFINED in MAP-SM-DataTypes       :     68

SM-RP-DA................................type reference CHOICE

DEFINED in MAP-SM-DataTypes       :     75

USED in MAP-SM-DataTypes       :     68

sm-RP-OA................................identifier of SM-RP-OA

DEFINED in MAP-SM-DataTypes       :     69

SM-RP-OA................................type reference CHOICE

DEFINED in MAP-SM-DataTypes       :     84

USED in MAP-SM-DataTypes       :     69

sm-RP-PRI...............................identifier of [1] BOOLEAN

DEFINED in MAP-SM-DataTypes       :     49

sm-RP-UI................................identifier of SignalInfo

DEFINED in MAP-SM-DataTypes       :     70

sres....................................identifier of SRES

DEFINED in MAP-MS-DataTypes       :    127

SRES....................................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    133

USED in MAP-MS-DataTypes       :    127

ss-AccessBarred.........................identifier of Named Number, 5

DEFINED in MAP-MS-DataTypes       :    270

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SupplementaryServi :    266

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SS-DataTypes       :     60

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SS-DataTypes       :     78

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SS-DataTypes       :    151

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SS-DataTypes       :    216

ss-Code.................................identifier of SS-Code

DEFINED in MAP-SS-DataTypes       :    239

SS-Code.................................type reference OCTET STRING

DEFINED in MAP-SS-Code            :     16

USED in MAP-SupplementaryServi :     75   266

USED in MAP-SS-DataTypes       :     52    60    78   151   216   239   293

USED in MAP-SS-Code            :     26    30    33    35    37    39   41   45   47

49    51    53    55    58    61   64   68   71

73    75    79    82    85    88   91   94   96

99   102   106   108   110   112  114  117  119

121   125   127   128   129   130  131  132  133

134   135   136   137   138   139   140   141

USED in MAP-ER-DataTypes       :     39    63

ss-Code.................................identifier of [1] SS-Code

DEFINED in MAP-ER-DataTypes       :     63

ss-Data.................................identifier of [3] SS-Data

DEFINED in MAP-SS-DataTypes       :     75

SS-Data.................................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    215

USED in MAP-SS-DataTypes       :     75

ss-ErrorStatus..........................value reference SS-ErrorStatus, CHOICE VALUE

DEFINED in MAP-Protocol           :    302

SS-ErrorStatus..........................type reference ERROR

DEFINED in MAP-Errors             :    190

USED in MAP-Protocol           :    114   302

USED in MAP-SupplementaryServi :     45   101   128   156   183

USED in MAP-Errors             :     57

ss-ForBS................................identifier of SS-ForBS-Code

DEFINED in MAP-SupplementaryServi :    110

ss-ForBS................................identifier of SS-ForBS-Code

DEFINED in MAP-SupplementaryServi :    138

ss-ForBS................................identifier of SS-ForBS-Code

DEFINED in MAP-SupplementaryServi :    165

ss-ForBS................................identifier of SS-ForBS-Code

DEFINED in MAP-SupplementaryServi :    193

SS-ForBS-Code...........................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    238

USED in MAP-SupplementaryServi :     63   110   138   165   193

USED in MAP-SS-DataTypes       :     25

ss-Incompatibility......................value reference SS-Incompatibility, CHOICE VALUE

DEFINED in MAP-Protocol           :    305

SS-Incompatibility......................type reference ERROR

DEFINED in MAP-Errors             :    202

USED in MAP-Protocol           :    117   305

USED in MAP-SupplementaryServi :     48   106   159

USED in MAP-Errors             :     60

ss-IncompatibilityCause.................identifier of SS-IncompatibilityCause

DEFINED in MAP-Errors             :    204

SS-IncompatibilityCause.................type reference SEQUENCE

DEFINED in MAP-ER-DataTypes       :     62

USED in MAP-Errors             :     86   204

USED in MAP-ER-DataTypes       :     22

ss-Info.................................identifier of SS-Info

DEFINED in MAP-SupplementaryServi :     88

ss-Info.................................identifier of SS-Info

DEFINED in MAP-SupplementaryServi :    112

ss-Info.................................identifier of SS-Info

DEFINED in MAP-SupplementaryServi :    140

ss-Info.................................identifier of SS-Info

DEFINED in MAP-SupplementaryServi :    167

SS-Info.................................type reference CHOICE

DEFINED in MAP-SS-DataTypes       :     70

USED in MAP-SupplementaryServi :     62    88   112   140   167

USED in MAP-SS-DataTypes       :     20   298

SS-InfoList.............................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    297

USED in MAP-MS-DataTypes       :     59   245

USED in MAP-SS-DataTypes       :     34

ss-List.................................identifier of [3] SS-List

DEFINED in MAP-MS-DataTypes       :    289

ss-List.................................identifier of [2] SS-List

DEFINED in MAP-MS-DataTypes       :    305

SS-List.................................type reference SEQUENCE OF

DEFINED in MAP-SS-DataTypes       :    292

USED in MAP-MS-DataTypes       :     58   289   305

USED in MAP-SS-DataTypes       :     33

ss-NotAvailable.........................value reference SS-NotAvailable, CHOICE VALUE

DEFINED in MAP-Protocol           :    303

SS-NotAvailable.........................type reference ERROR

DEFINED in MAP-Errors             :    195

USED in MAP-Protocol           :    115   303

USED in MAP-SupplementaryServi :     46   102   129   157   184   210

USED in MAP-Errors             :     58

ss-Status...............................identifier of SS-Status

DEFINED in MAP-Errors             :    192

ss-Status...............................identifier of [4] SS-Status

DEFINED in MAP-SS-DataTypes       :     88

SS-Status...............................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :     97

USED in MAP-Errors             :     77   192

USED in MAP-SS-DataTypes       :     21    88   162   218   244

USED in MAP-ER-DataTypes       :     28    65

ss-Status...............................identifier of [4] SS-Status

DEFINED in MAP-SS-DataTypes       :    162

ss-Status...............................identifier of [4] SS-Status

DEFINED in MAP-SS-DataTypes       :    218

ss-Status...............................identifier of [0] SS-Status

DEFINED in MAP-SS-DataTypes       :    244

ss-Status...............................identifier of [4] SS-Status

DEFINED in MAP-ER-DataTypes       :     65

ss-SubscriptionOption...................identifier of SS-SubscriptionOption

DEFINED in MAP-Errors             :    199

ss-SubscriptionOption...................identifier of SS-SubscriptionOption

DEFINED in MAP-SS-DataTypes       :    219

SS-SubscriptionOption...................type reference CHOICE

DEFINED in MAP-SS-DataTypes       :    225

USED in MAP-Errors             :     78   199

USED in MAP-SS-DataTypes       :     24   219

ss-SubscriptionViolation............value reference SS-SubscriptionViolation, CHOICE VALUE

DEFINED in MAP-Protocol           :    304

SS-SubscriptionViolation................type reference ERROR

DEFINED in MAP-Errors             :    197

USED in MAP-Protocol           :    116   304

USED in MAP-SupplementaryServi :     47   103   131   158   186   276

USED in MAP-Errors             :     59

ss-UserData.............................identifier of SS-UserData

DEFINED in MAP-SupplementaryServi :    214

ss-UserData.............................identifier of SS-UserData

DEFINED in MAP-SupplementaryServi :    216

SS-UserData.............................type reference IA5String

DEFINED in MAP-SS-DataTypes       :    250

USED in MAP-SupplementaryServi :     65   214   216

USED in MAP-SS-DataTypes       :     27

storedMSISDN............................identifier of ISDN-AddressString

DEFINED in MAP-ShortMessageServic :    105

storedMSISDN............................identifier of ISDN-AddressString

DEFINED in MAP-SM-DataTypes       :    109

subscriberData..........................identifier of [2] SubscriberData

DEFINED in MAP-MS-DataTypes       :    227

SubscriberData..........................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :    239

USED in MAP-MS-DataTypes       :    227   236

subscriberId............................identifier of SubscriberId

DEFINED in MAP-MS-DataTypes       :    209

SubscriberId............................type reference CHOICE

DEFINED in MAP-CommonDataTypes    :    196

USED in MAP-MS-DataTypes       :     67   209

USED in MAP-CommonDataTypes    :     31

USED in MAP-SM-DataTypes       :     33   119

subscriberId............................identifier of SubscriberId

DEFINED in MAP-SM-DataTypes       :    119

subscriberNotMemberOfCUG................identifier of Named Number, 1

DEFINED in MAP-ER-DataTypes       :     56

subscriberNotSC-Subscriber..............identifier of Named Number, 6

DEFINED in MAP-ER-DataTypes       :     90

subscriberStatus........................identifier of [3] SubscriberStatus

DEFINED in MAP-MS-DataTypes       :    242

SubscriberStatus........................type reference ENUMERATED

DEFINED in MAP-MS-DataTypes       :    255

USED in MAP-MS-DataTypes       :    242

subsequentHandoverFailure..........value reference SubsequentHandoverFailure, CHOICE VALUE

DEFINED in MAP-Protocol           :    281

SubsequentHandoverFailure...............type reference ERROR

DEFINED in MAP-Errors             :    155

USED in MAP-Protocol           :    106   281

USED in MAP-MobileServiceOpera :     68   201   213

USED in MAP-Errors             :     43

successfulTransfer......................identifier of Named Number, 2

DEFINED in MAP-SM-DataTypes       :    101

systemFailure...........................value reference SystemFailure, CHOICE VALUE

DEFINED in MAP-Protocol           :    248

SystemFailure...........................type reference ERROR

DEFINED in MAP-Errors             :    102

USED in MAP-Protocol           :     88   248

USED in MAP-MobileServiceOpera :     56   117   158   169   224   237   297

USED in MAP-OperationAndMainte :     30    62    74

USED in MAP-CallHandlingOperat :     27    62    83

USED in MAP-SupplementaryServi :     37    91   115   143   170   197  219  228  241

255   270

USED in MAP-ShortMessageServic :     33    75    89   129

USED in MAP-Errors             :     19

targetCellId............................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    140

targetCellId............................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    146

targetCellId............................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    178

targetCellId............................identifier of GlobalCellId

DEFINED in MAP-MS-DataTypes       :    184

targetCellId............................identifier of [1] GlobalCellId

DEFINED in MAP-OM-DataTypes       :     66

targetMSC-Number........................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    179

targetMSC-Number........................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    186

TBCD-STRING.............................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :     64

USED in MAP-CommonDataTypes    :    191   200

TCAPMessages............................module reference

DEFINED in TCAPMessages           :      1

USED in MAP-MobileServiceOpera :     53

USED in MAP-OperationAndMainte :     27

USED in MAP-CallHandlingOperat :     24

USED in MAP-SupplementaryServi :     34

USED in MAP-ShortMessageServic :     30

USED in MAP-Errors             :     74

telephony...............................value reference TeleserviceCode, '00010001'B

DEFINED in MAP-TS-Code            :     28

teleservice.............................identifier of [3] TeleserviceCode

DEFINED in MAP-CommonDataTypes    :    261

teleservice.............................identifier of [5] TeleserviceCode

DEFINED in MAP-SM-DataTypes       :     51

TeleserviceCode.........................type reference OCTET STRING

DEFINED in MAP-TS-Code            :     16

USED in MAP-CommonDataTypes    :     50   255   261

USED in MAP-SM-DataTypes       :     40    51

USED in MAP-TS-Code            :     25    27    28    29    31    32   33   35   36

37    38    43    46    56    58   59   60   61

62    63    64    65    66    67   68   69   70

71    72

teleserviceList.........................identifier of [6] TeleserviceList

DEFINED in MAP-MS-DataTypes       :    244

teleserviceList.........................identifier of [1] TeleserviceList

DEFINED in MAP-MS-DataTypes       :    287

TeleserviceList.........................type reference SEQUENCE OF

DEFINED in MAP-CommonDataTypes    :    254

USED in MAP-MS-DataTypes       :     76   244   287

USED in MAP-CommonDataTypes    :     42

teleserviceNotProvisioned..........value reference TeleserviceNotProvisioned, CHOICE VALUE

DEFINED in MAP-Protocol           :    271

TeleserviceNotProvisioned...............type reference ERROR

DEFINED in MAP-Errors             :    144

USED in MAP-Protocol           :    102   271

USED in MAP-CallHandlingOperat :     34    70

USED in MAP-SupplementaryServi :     42    97   123   151   178   205

USED in MAP-ShortMessageServic :     41    80

USED in MAP-Errors             :     37

temporaryDefaultAllowed.................identifier of Named Number, 2

DEFINED in MAP-SS-DataTypes       :    232

temporaryDefaultRestricted..............identifier of Named Number, 1

DEFINED in MAP-SS-DataTypes       :    231

tmsi....................................identifier of TMSI

DEFINED in MAP-MobileServiceOpera :    143

TMSI....................................type reference OCTET STRING

DEFINED in MAP-CommonDataTypes    :    194

USED in MAP-MobileServiceOpera :    101   143

USED in MAP-CommonDataTypes    :     30   198

tmsi....................................identifier of [1] TMSI

DEFINED in MAP-CommonDataTypes    :    198

tooManyZoneCodes........................identifier of Named Number, 1

DEFINED in MAP-MS-DataTypes       :    298

traceReference..........................identifier of [1] TraceReference

DEFINED in MAP-OM-DataTypes       :     39

TraceReference..........................type reference OCTET STRING

DEFINED in MAP-OM-DataTypes       :     44

USED in MAP-OM-DataTypes       :     39    52    57

traceReference..........................identifier of [1] TraceReference

DEFINED in MAP-OM-DataTypes       :     52

traceReference..........................identifier of [1] TraceReference

DEFINED in MAP-OM-DataTypes       :     57

traceSubscriberActivity..............value reference TraceSubscriberActivity, CHOICE VALUE

DEFINED in MAP-Protocol           :    190

TraceSubscriberActivity.................type reference OPERATION

DEFINED in MAP-OperationAndMainte :     80

USED in MAP-Protocol           :     43   190

USED in MAP-OperationAndMainte :     20

traceSubscriberActivityArg..............identifier of TraceSubscriberActivityArg

DEFINED in MAP-OperationAndMainte :     82

TraceSubscriberActivityArg..............type reference SEQUENCE

DEFINED in MAP-OM-DataTypes       :     55

USED in MAP-OperationAndMainte :     43    82

USED in MAP-OM-DataTypes       :     21

traceType...............................identifier of [2] TraceType

DEFINED in MAP-OM-DataTypes       :     40

TraceType...............................type reference INTEGER

DEFINED in MAP-OM-DataTypes       :     46

USED in MAP-OM-DataTypes       :     40    58

traceType...............................identifier of [2] TraceType

DEFINED in MAP-OM-DataTypes       :     58

tracingBufferFull.......................value reference TracingBufferFull, CHOICE VALUE

DEFINED in MAP-Protocol           :    287

TracingBufferFull.......................type reference ERROR

DEFINED in MAP-Errors             :    160

USED in MAP-Protocol           :    107   287

USED in MAP-OperationAndMainte :     36    67

USED in MAP-Errors             :     46

TransactionID...........................type reference OCTET STRING

DEFINED in TCAPMessages           :    100

USED in TCAPMessages           :     47    97    98

undetermined............................identifier of Named Number, 0

DEFINED in MAP-ER-DataTypes       :     69

unexpectedDataValue.....................value reference UnexpectedDataValue, CHOICE VALUE

DEFINED in MAP-Protocol           :    250

UnexpectedDataValue.....................type reference ERROR

DEFINED in MAP-Errors             :    109

USED in MAP-Protocol           :     90   250

USED in MAP-MobileServiceOpera :     58   120   131   160   170   198  209  226  240

255   268   280   299

USED in MAP-OperationAndMainte :     32    64    76    95

USED in MAP-CallHandlingOperat :     29    64    85

USED in MAP-SupplementaryServi :     39    93   118   146   173   200  220  230  243

257   273

USED in MAP-ShortMessageServic :     35    77    92   112   131   143

USED in MAP-Errors             :     21

unexpectedError.........................identifier of Named Number, 3

DEFINED in TCAPMessages           :    199

unexpectedLinkedOperation...............identifier of Named Number, 7

DEFINED in TCAPMessages           :    190

unidentifiedSubscriber................value reference UnidentifiedSubscriber, CHOICE VALUE

DEFINED in MAP-Protocol           :    260

UnidentifiedSubscriber..................type reference ERROR

DEFINED in MAP-Errors             :    124

USED in MAP-Protocol           :     96   260

USED in MAP-MobileServiceOpera :     62   132   148   257   269   281

USED in MAP-OperationAndMainte :     35    66    78

USED in MAP-ShortMessageServic :     38    94

USED in MAP-Errors             :     29

unidirectional.......................identifier of [APPLICATION 1] IMPLICIT Unidirectional

DEFINED in TCAPMessages           :     52

Unidirectional..........................type reference SEQUENCE

DEFINED in TCAPMessages           :     58

USED in TCAPMessages           :     52

unknownAlphabet.........................value reference UnknownAlphabet, CHOICE VALUE

DEFINED in MAP-Protocol           :    306

UnknownAlphabet.........................type reference ERROR

DEFINED in MAP-Errors             :    207

USED in MAP-Protocol           :    118   306

USED in MAP-SupplementaryServi :     52   231   247   261

USED in MAP-Errors             :     61

unknownBaseStation......................value reference UnknownBaseStation, CHOICE VALUE

DEFINED in MAP-Protocol           :    258

UnknownBaseStation......................type reference ERROR

DEFINED in MAP-Errors             :    120

USED in MAP-Protocol           :     94   258

USED in MAP-MobileServiceOpera :     60   171   210

USED in MAP-Errors             :     27

unknownEquipment........................value reference UnknownEquipment, CHOICE VALUE

DEFINED in MAP-Protocol           :    261

UnknownEquipment........................type reference ERROR

DEFINED in MAP-Errors             :    126

USED in MAP-Protocol           :     97   261

USED in MAP-MobileServiceOpera :     63   242

USED in MAP-Errors             :     30

unknownMSC..............................value reference UnknownMSC, CHOICE VALUE

DEFINED in MAP-Protocol           :    259

UnknownMSC..............................type reference ERROR

DEFINED in MAP-Errors             :    122

USED in MAP-Protocol           :     95   259

USED in MAP-MobileServiceOpera :     61   200   211

USED in MAP-Errors             :     28

unknownServiceCentre....................identifier of Named Number, 3

DEFINED in MAP-ER-DataTypes       :     87

unknownSubscriber.......................value reference UnknownSubscriber, CHOICE VALUE

DEFINED in MAP-Protocol           :    256

UnknownSubscriber.......................type reference ERROR

DEFINED in MAP-Errors             :    116

USED in MAP-Protocol           :     92   256

USED in MAP-MobileServiceOpera :     59   121   227   256   300

USED in MAP-OperationAndMainte :     34    96

USED in MAP-CallHandlingOperat :     31    66

USED in MAP-SupplementaryServi :     40    94   119   147   174   201

USED in MAP-ShortMessageServic :     37    79   113   145

USED in MAP-Errors             :     25

unrecognizedComponent...................identifier of Named Number, 0

DEFINED in TCAPMessages           :    179

unrecognizedError.......................identifier of Named Number, 2

DEFINED in TCAPMessages           :    198

unrecognizedInvokeID....................identifier of Named Number, 0

DEFINED in TCAPMessages           :    192

unrecognizedInvokeID....................identifier of Named Number, 0

DEFINED in TCAPMessages           :    196

unrecognizedLinkedID....................identifier of Named Number, 5

DEFINED in TCAPMessages           :    188

unrecognizedMessageType.................identifier of Named Number, 0

DEFINED in TCAPMessages           :    103

unrecognizedOperation...................identifier of Named Number, 1

DEFINED in TCAPMessages           :    184

unrecognizedTransactionID...............identifier of Named Number, 1

DEFINED in TCAPMessages           :    104

unstructuredSS-Notify..................value reference UnstructuredSS-Notify, CHOICE VALUE

DEFINED in MAP-Protocol           :    219

UnstructuredSS-Notify...................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    250

USED in MAP-Protocol           :     66   219

USED in MAP-SupplementaryServi :     26

unstructuredSS-Request................value reference UnstructuredSS-Request, CHOICE VALUE

DEFINED in MAP-Protocol           :    217

UnstructuredSS-Request..................type reference OPERATION

DEFINED in MAP-SupplementaryServi :    234

USED in MAP-Protocol           :     65   217

USED in MAP-SupplementaryServi :     25

updateLocation..........................value reference UpdateLocation, CHOICE VALUE

DEFINED in MAP-Protocol           :    133

UpdateLocation..........................type reference OPERATION

DEFINED in MAP-MobileServiceOpera :    111

USED in MAP-Protocol           :     17   133

USED in MAP-MobileServiceOpera :     20

updateLocationArg.......................identifier of UpdateLocationArg

DEFINED in MAP-MobileServiceOpera :    113

UpdateLocationArg.......................type reference SEQUENCE

DEFINED in MAP-MS-DataTypes       :     86

USED in MAP-MobileServiceOpera :     73   113

USED in MAP-MS-DataTypes       :     21

updateLocationRes.......................identifier of UpdateLocationRes

DEFINED in MAP-MobileServiceOpera :    115

UpdateLocationRes.......................type reference CHOICE

DEFINED in MAP-MS-DataTypes       :     93

USED in MAP-MobileServiceOpera :     74   115

USED in MAP-MS-DataTypes       :     22

ussd-Arg................................identifier of USSD-Arg

DEFINED in MAP-SupplementaryServi :    224

ussd-Arg................................identifier of USSD-Arg

DEFINED in MAP-SupplementaryServi :    236

ussd-Arg................................identifier of USSD-Arg

DEFINED in MAP-SupplementaryServi :    252

USSD-Arg................................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    252

USED in MAP-SupplementaryServi :     66   224   236   252

USED in MAP-SS-DataTypes       :     28

ussd-Busy...............................value reference USSD-Busy, CHOICE VALUE

DEFINED in MAP-Protocol           :    308

USSD-Busy...............................type reference ERROR

DEFINED in MAP-Errors             :    209

USED in MAP-Protocol           :    119   308

USED in MAP-SupplementaryServi :     53   248   262

USED in MAP-Errors             :     62

ussd-DataCodingScheme...................identifier of USSD-DataCodingScheme

DEFINED in MAP-SS-DataTypes       :    253

ussd-DataCodingScheme...................identifier of USSD-DataCodingScheme

DEFINED in MAP-SS-DataTypes       :    258

USSD-DataCodingScheme...................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :    262

USED in MAP-SS-DataTypes       :    253   258

ussd-Res................................identifier of USSD-Res

DEFINED in MAP-SupplementaryServi :    226

ussd-Res................................identifier of USSD-Res

DEFINED in MAP-SupplementaryServi :    238

USSD-Res................................type reference SEQUENCE

DEFINED in MAP-SS-DataTypes       :    257

USED in MAP-SupplementaryServi :     67   226   238

USED in MAP-SS-DataTypes       :     29

ussd-String.............................identifier of USSD-String

DEFINED in MAP-SS-DataTypes       :    254

ussd-String.............................identifier of USSD-String

DEFINED in MAP-SS-DataTypes       :    259

USSD-String.............................type reference OCTET STRING

DEFINED in MAP-SS-DataTypes       :    266

USED in MAP-SS-DataTypes       :    254   259

uus.....................................value reference SS-Code, '10000001'B

DEFINED in MAP-SS-Code            :    102

vlr.....................................identifier of Named Number, 2

DEFINED in MAP-CommonDataTypes    :    236

vlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :     89

vlr-Number..............................identifier of ISDN-AddressString

DEFINED in MAP-MS-DataTypes       :    109

vmsc....................................identifier of Named Number, 5

DEFINED in MAP-CommonDataTypes    :    239

whiteListed.............................identifier of Named Number, 0

DEFINED in MAP-MS-DataTypes       :    201

ZoneCode................................type reference OCTET STRING

DEFINED in MAP-MS-DataTypes       :    281

USED in MAP-MS-DataTypes       :    279

ZoneCodeList............................type reference SEQUENCE OF

DEFINED in MAP-MS-DataTypes       :    278

USED in MAP-MS-DataTypes       :    251   311

zoneCodesConflict.......................identifier of Named Number, 2

DEFINED in MAP-MS-DataTypes       :    299

Annex B (informative): Fully expanded ASN.1 sources for abstract syntaxes of MAP

Annex B is not part of the standard, it is included for information purposes only.

For every (Value)Assignment in the root ASN.1 module all the used defined types and defined values, which are defined within the ASN.1 module or imported from ASN.1 modules, are replaced by the constructs this type or value is composed of.

The fully expanded ASN.1 root module is itself a correct and equivalent representation of the MAP-Protocol.

It allows to see at all the parameters, including all nested ones for a specific operationcode or errorcode at once.

B.1	Fully Expanded ASN.1 Source of MAP-Protocol/TCAPMessages

--         Expanded ASN.1 Module 'MAP-Protocol'

--SIEMENS ASN.1 Compiler       P2.03a (94-05-05 21:45:40)

--             Date: 94-05-05 Time: 21:55:50

MAP-Protocol { 0 identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-Protocol (4)

version2 (2) }

DEFINITIONS

::=

BEGIN

updateLocation OPERATION

ARGUMENT

updateLocationArg SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

locationInfo CHOICE {

roamingNumber [0] IMPLICIT OCTET STRING (SIZE (1..9)),

msc-Number [1] IMPLICIT OCTET STRING (SIZE (1..9))},

vlr-Number OCTET STRING (SIZE (1..9)),

lmsi       [10] IMPLICIT OCTET STRING (SIZE (4)) OPTIONAL,

... }

RESULT

updateLocationRes CHOICE {

hlr-Number OCTET STRING (SIZE (1..9)),

extensibleUpdateLocationRes SEQUENCE {

hlr-Number OCTET STRING (SIZE (1..9)),

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- roamingNotAllowed -- localValue 8}

::= localValue 2

cancelLocation OPERATION

ARGUMENT

cancelLocationArg CHOICE {

imsi       OCTET STRING (SIZE (3..8)),

imsi-WithLMSI SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

lmsi       OCTET STRING (SIZE (4)),

... }}

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 3

purgeMS    OPERATION

ARGUMENT

purgeMS-Arg SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

vlr-Number OCTET STRING (SIZE (1..9)),

... }

::= localValue 67

sendIdentification OPERATION

ARGUMENT

tmsi       OCTET STRING (SIZE (1..4))

RESULT

sendIdentificationRes SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

authenticationSetList SEQUENCE SIZE (1..5) OF

SEQUENCE {

rand       OCTET STRING (SIZE (16)),

sres       OCTET STRING (SIZE (4)),

kc         OCTET STRING (SIZE (8)),

... } OPTIONAL,

... }

ERRORS {

-- dataMissing -- localValue 35,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 55

performHandover OPERATION

ARGUMENT

performHO-Arg SEQUENCE {

targetCellId OCTET STRING (SIZE (5..7)),

servingCellId OCTET STRING (SIZE (5..7)),

channelType OCTET STRING (SIZE (1..10)),

classmarkInfo OCTET STRING (SIZE (1..2)),

handoverPriority [11] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

kc         [12] IMPLICIT OCTET STRING (SIZE (8)) OPTIONAL}

RESULT

performHO-Res SEQUENCE {

handoverNumber OCTET STRING (SIZE (1..9)),

accessSignalInfo SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- unexpectedDataValue -- localValue 36,

-- unknownBaseStation -- localValue 2,

-- invalidTargetBaseStation -- localValue 23,

-- noRadioResourceAvailable -- localValue 24,

-- noHandoverNumberAvailable -- localValue 25}

::= localValue 28

prepareHandover OPERATION

ARGUMENT

prepareHO-Arg SEQUENCE {

targetCellId OCTET STRING (SIZE (5..7)) OPTIONAL,

ho-NumberNotRequired NULL OPTIONAL,

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL,

... }

RESULT

prepareHO-Res SEQUENCE {

handoverNumber OCTET STRING (SIZE (1..9)) OPTIONAL,

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- noHandoverNumberAvailable -- localValue 25}

::= localValue 68

sendEndSignal OPERATION

ARGUMENT

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }

::= localValue 29

processAccessSignalling OPERATION

ARGUMENT

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }

::= localValue 33

forwardAccessSignalling OPERATION

ARGUMENT

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }

::= localValue 34

performSubsequentHandover OPERATION

ARGUMENT

performSubsequentHO-Arg SEQUENCE {

targetCellId OCTET STRING (SIZE (5..7)),

servingCellId OCTET STRING (SIZE (5..7)),

targetMSC-Number OCTET STRING (SIZE (1..9)),

classmarkInfo [10] IMPLICIT OCTET STRING (SIZE (1..2)) OPTIONAL}

RESULT

accessSignalInfo SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }

ERRORS {

-- unexpectedDataValue -- localValue 36,

-- unknownBaseStation -- localValue 2,

-- unknownMSC -- localValue 3,

-- invalidTargetBaseStation -- localValue 23,

-- subsequentHandoverFailure -- localValue 26}

::= localValue 30

prepareSubsequentHandover OPERATION

ARGUMENT

prepareSubsequentHO-Arg SEQUENCE {

targetCellId OCTET STRING (SIZE (5..7)),

targetMSC-Number OCTET STRING (SIZE (1..9)),

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... },

... }

RESULT

bss-APDU   SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... }

ERRORS {

-- unexpectedDataValue -- localValue 36,

-- dataMissing -- localValue 35,

-- unknownMSC -- localValue 3,

-- subsequentHandoverFailure -- localValue 26}

::= localValue 69

sendAuthenticationInfo OPERATION

ARGUMENT

sendAuthenticationInfoArg OCTET STRING (SIZE (3..8))

RESULT

sendAuthenticationInfoRes SEQUENCE SIZE (1..5) OF

SEQUENCE {

rand       OCTET STRING (SIZE (16)),

sres       OCTET STRING (SIZE (4)),

kc         OCTET STRING (SIZE (8)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1}

::= localValue 56

checkIMEI  OPERATION

ARGUMENT

imei       OCTET STRING (SIZE (8))

RESULT

equipmentStatus ENUMERATED {

whiteListed  (0),

blackListed  (1),

greyListed  (2)}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownEquipment -- localValue 7}

::= localValue 43

sendParameters OPERATION

ARGUMENT

sendParametersArg SEQUENCE {

subscriberId CHOICE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

tmsi       [1] IMPLICIT OCTET STRING (SIZE (1..4))},

requestParameterList SEQUENCE SIZE (1..2) OF

ENUMERATED {

requestIMSI  (0),

requestAuthenticationSet  (1),

requestSubscriberData  (2),

requestKi   (4)}}

RESULT

sentParameterList SEQUENCE SIZE (1..6) OF

CHOICE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

authenticationSet [1] IMPLICIT SEQUENCE {

rand       OCTET STRING (SIZE (16)),

sres       OCTET STRING (SIZE (4)),

kc         OCTET STRING (SIZE (8)),

... },

subscriberData [2] IMPLICIT SEQUENCE {

msisdn     [1] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

category   [2] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

subscriberStatus [3] IMPLICIT ENUMERATED {

serviceGranted  (0),

operatorDeterminedBarring  (1)} OPTIONAL,

bearerServiceList [4] IMPLICIT SEQUENCE SIZE (1..50) OF

OCTET STRING (SIZE (1)) OPTIONAL,

teleserviceList [6] IMPLICIT SEQUENCE SIZE (1..20) OF

OCTET STRING (SIZE (1)) OPTIONAL,

provisionedSS [7] IMPLICIT SEQUENCE SIZE (1..30) OF

CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }} OPTIONAL,

odb-Data   [8] IMPLICIT SEQUENCE {

odb-GeneralData BIT STRING {

allOG-CallsBarred  (0),

internationalOGCallsBarred  (1),

internationalOGCallsNotToHPLMN-CountryBarred  (2),

premiumRateInformationOGCallsBarred  (3),

premiumRateEntertainementOGCallsBarred  (4),

ss-AccessBarred  (5)} (SIZE (6)),

odb-HPLMN-Data BIT STRING {

plmn-SpecificBarringType1  (0),

plmn-SpecificBarringType2  (1),

plmn-SpecificBarringType3  (2),

plmn-SpecificBarringType4  (3)} (SIZE (4)) OPTIONAL,

... } OPTIONAL,

roamingRestrictionDueToUnsupportedFeature [9] IMPLICIT NULL OPTIONAL,

regionalSubscriptionData [10] IMPLICIT SEQUENCE SIZE (1..10) OF

OCTET STRING (SIZE (2)) OPTIONAL},

ki         [4] IMPLICIT OCTET STRING (SIZE (16))}

ERRORS {

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 9

insertSubscriberData OPERATION

ARGUMENT

insertSubscriberDataArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)) OPTIONAL,

msisdn     [1] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

category   [2] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

subscriberStatus [3] IMPLICIT ENUMERATED {

serviceGranted  (0),

operatorDeterminedBarring  (1)} OPTIONAL,

bearerServiceList [4] IMPLICIT SEQUENCE SIZE (1..50) OF

OCTET STRING (SIZE (1)) OPTIONAL,

teleserviceList [6] IMPLICIT SEQUENCE SIZE (1..20) OF

OCTET STRING (SIZE (1)) OPTIONAL,

provisionedSS [7] IMPLICIT SEQUENCE SIZE (1..30) OF

CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }} OPTIONAL,

odb-Data   [8] IMPLICIT SEQUENCE {

odb-GeneralData BIT STRING {

allOG-CallsBarred  (0),

internationalOGCallsBarred  (1),

internationalOGCallsNotToHPLMN-CountryBarred  (2),

premiumRateInformationOGCallsBarred  (3),

premiumRateEntertainementOGCallsBarred  (4),

ss-AccessBarred  (5)} (SIZE (6)),

odb-HPLMN-Data BIT STRING {

plmn-SpecificBarringType1  (0),

plmn-SpecificBarringType2  (1),

plmn-SpecificBarringType3  (2),

plmn-SpecificBarringType4  (3)} (SIZE (4)) OPTIONAL,

... } OPTIONAL,

roamingRestrictionDueToUnsupportedFeature [9] IMPLICIT NULL OPTIONAL,

regionalSubscriptionData [10] IMPLICIT SEQUENCE SIZE (1..10) OF

OCTET STRING (SIZE (2)) OPTIONAL,

... }

RESULT

insertSubscriberDataRes SEQUENCE {

teleserviceList [1] IMPLICIT SEQUENCE SIZE (1..20) OF

OCTET STRING (SIZE (1)) OPTIONAL,

bearerServiceList [2] IMPLICIT SEQUENCE SIZE (1..50) OF

OCTET STRING (SIZE (1)) OPTIONAL,

ss-List    [3] IMPLICIT SEQUENCE SIZE (1..30) OF

OCTET STRING (SIZE (1)) OPTIONAL,

odb-GeneralData [4] IMPLICIT BIT STRING {

allOG-CallsBarred  (0),

internationalOGCallsBarred  (1),

internationalOGCallsNotToHPLMN-CountryBarred  (2),

premiumRateInformationOGCallsBarred  (3),

premiumRateEntertainementOGCallsBarred  (4),

ss-AccessBarred  (5)} (SIZE (6)) OPTIONAL,

regionalSubscriptionResponse [5] IMPLICIT ENUMERATED {

msc-AreaRestricted  (0),

tooManyZoneCodes  (1),

zoneCodesConflict  (2),

regionalSubscNotSupported  (3)} OPTIONAL,

... }

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 7

deleteSubscriberData OPERATION

ARGUMENT

deleteSubscriberDataArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

basicServiceList [1] IMPLICIT SEQUENCE SIZE (1..70) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-List    [2] IMPLICIT SEQUENCE SIZE (1..30) OF

OCTET STRING (SIZE (1)) OPTIONAL,

cug-InterlockList [3] IMPLICIT SEQUENCE SIZE (1..10) OF

OCTET STRING (SIZE (4)) OPTIONAL,

roamingRestrictionDueToUnsupportedFeature [4] IMPLICIT NULL OPTIONAL,

regionalSubscriptionData [5] IMPLICIT SEQUENCE SIZE (1..10) OF

OCTET STRING (SIZE (2)) OPTIONAL,

... }

RESULT

deleteSubscriberDataRes SEQUENCE {

regionalSubscriptionResponse [0] IMPLICIT ENUMERATED {

msc-AreaRestricted  (0),

tooManyZoneCodes  (1),

zoneCodesConflict  (2),

regionalSubscNotSupported  (3)} OPTIONAL,

... }

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 8

reset      OPERATION

ARGUMENT

resetArg   SEQUENCE {

networkResource ENUMERATED {

plmn        (0),

hlr         (1),

vlr         (2),

pvlr        (3),

controllingMSC  (4),

vmsc        (5),

eir         (6),

rss         (7)} OPTIONAL,

hlr-Number OCTET STRING (SIZE (1..9)),

hlr-List   SEQUENCE SIZE (1..50) OF

OCTET STRING (SIZE (3..8)) OPTIONAL,

... }

::= localValue 37

forwardCheckSS-Indication OPERATION

::= localValue 38

restoreData OPERATION

ARGUMENT

restoreDataArg SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

lmsi       OCTET STRING (SIZE (4)) OPTIONAL,

... }

RESULT

restoreDataRes SEQUENCE {

hlr-Number OCTET STRING (SIZE (1..9)),

msNotReachable NULL OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1}

::= localValue 57

activateTraceMode OPERATION

ARGUMENT

activateTraceModeArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)) OPTIONAL,

traceReference [1] IMPLICIT OCTET STRING (SIZE (1..2)),

traceType  [2] IMPLICIT INTEGER (0..255),

omc-Id     [3] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unidentifiedSubscriber -- localValue 5,

-- tracingBufferFull -- localValue 40}

::= localValue 50

deactivateTraceMode OPERATION

ARGUMENT

deactivateTraceModeArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)) OPTIONAL,

traceReference [1] IMPLICIT OCTET STRING (SIZE (1..2)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unidentifiedSubscriber -- localValue 5}

::= localValue 51

traceSubscriberActivity OPERATION

ARGUMENT

traceSubscriberActivityArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)) OPTIONAL,

traceReference [1] IMPLICIT OCTET STRING (SIZE (1..2)),

traceType  [2] IMPLICIT INTEGER (0..255),

omc-Id     [3] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

callReference [4] IMPLICIT OCTET STRING (SIZE (1..3)) OPTIONAL}

::= localValue 52

noteInternalHandover OPERATION

ARGUMENT

noteInternalHO-Arg SEQUENCE {

handoverType ENUMERATED {

interBSS    (0),

intraBSS    (1)},

targetCellId [1] IMPLICIT OCTET STRING (SIZE (5..7)) OPTIONAL,

channelId  [2] IMPLICIT SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL}

::= localValue 35

sendIMSI   OPERATION

ARGUMENT

msisdn     OCTET STRING (SIZE (1..9))

RESULT

imsi       OCTET STRING (SIZE (3..8))

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1}

::= localValue 58

sendRoutingInfo OPERATION

ARGUMENT

sendRoutingInfoArg SEQUENCE {

msisdn     [0] IMPLICIT OCTET STRING (SIZE (1..9)),

cug-CheckInfo [1] IMPLICIT SEQUENCE {

cug-Interlock OCTET STRING (SIZE (4)),

cug-OutgoingAccess NULL OPTIONAL,

... } OPTIONAL,

numberOfForwarding [2] IMPLICIT INTEGER (1..5) OPTIONAL,

networkSignalInfo [10] IMPLICIT SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL,

... }

RESULT

sendRoutingInfoRes SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

routingInfo CHOICE {

roamingNumber OCTET STRING (SIZE (1..9)),

forwardingData SEQUENCE {

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [4] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... }},

cug-CheckInfo SEQUENCE {

cug-Interlock OCTET STRING (SIZE (4)),

cug-OutgoingAccess NULL OPTIONAL,

... } OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unknownSubscriber -- localValue 1,

-- numberChanged -- localValue 44,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- absentSubscriber -- localValue 27,

-- callBarred -- localValue 13,

-- cug-Reject -- localValue 15,

-- forwardingViolation -- localValue 14}

::= localValue 22

provideRoamingNumber OPERATION

ARGUMENT

provideRoamingNumberArg SEQUENCE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

msc-Number [1] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

msisdn     [2] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

previousRoamingNumber [3] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

lmsi       [4] IMPLICIT OCTET STRING (SIZE (4)) OPTIONAL,

gsm-BearerCapability [5] IMPLICIT SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL,

networkSignalInfo [6] IMPLICIT SEQUENCE {

protocolId ENUMERATED {

gsm-0408    (1),

gsm-0806    (2),

gsm-BSSMAP  (3),

ets-300102-1  (4)},

signalInfo OCTET STRING (SIZE (1..200)),

... } OPTIONAL,

... }

RESULT

roamingNumber OCTET STRING (SIZE (1..9))

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- absentSubscriber -- localValue 27,

-- noRoamingNumberAvailable -- localValue 39}

::= localValue 4

registerSS OPERATION

ARGUMENT

registerSS-Arg SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)),

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

forwardedToNumber [4] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

forwardedToSubaddress [6] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

noReplyConditionTime [5] IMPLICIT INTEGER (5..30) OPTIONAL,

... }

RESULT

ss-Info    CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- callBarred -- localValue 13,

-- illegalSS-Operation -- localValue 16,

-- ss-ErrorStatus -- localValue 17,

-- ss-NotAvailable -- localValue 18,

-- ss-SubscriptionViolation -- localValue 19,

-- ss-Incompatibility -- localValue 20}

::= localValue 10

eraseSS    OPERATION

ARGUMENT

ss-ForBS   SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)),

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }

RESULT

ss-Info    CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- callBarred -- localValue 13,

-- illegalSS-Operation -- localValue 16,

-- ss-ErrorStatus -- localValue 17,

-- ss-NotAvailable -- localValue 18,

-- ss-SubscriptionViolation -- localValue 19}

::= localValue 11

activateSS OPERATION

ARGUMENT

ss-ForBS   SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)),

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }

RESULT

ss-Info    CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- callBarred -- localValue 13,

-- illegalSS-Operation -- localValue 16,

-- ss-ErrorStatus -- localValue 17,

-- ss-NotAvailable -- localValue 18,

-- ss-SubscriptionViolation -- localValue 19,

-- ss-Incompatibility -- localValue 20,

-- negativePW-Check -- localValue 38,

-- numberOfPW-AttemptsViolation -- localValue 43}

::= localValue 12

deactivateSS OPERATION

ARGUMENT

ss-ForBS   SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)),

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }

RESULT

ss-Info    CHOICE {

forwardingInfo [0] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

forwardingFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... },

... },

callBarringInfo [1] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

callBarringFeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... },

... },

cug-Info   [2] IMPLICIT SEQUENCE {

cug-SubscriptionList SEQUENCE SIZE (1..10) OF

SEQUENCE {

cug-Index  INTEGER (0..32767),

cug-Interlock OCTET STRING (SIZE (4)),

intraCUG-Options ENUMERATED {

noCUG-Restrictions  (0),

cugIC-CallBarred  (1),

cugOG-CallBarred  (2)},

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... },

cug-FeatureList SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

preferentialCUG-Indicator INTEGER (0..32767) OPTIONAL,

interCUG-Restrictions OCTET STRING (SIZE (1)),

... } OPTIONAL,

... },

ss-Data    [3] IMPLICIT SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)) OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}} OPTIONAL,

basicServiceGroupList SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- callBarred -- localValue 13,

-- illegalSS-Operation -- localValue 16,

-- ss-ErrorStatus -- localValue 17,

-- ss-NotAvailable -- localValue 18,

-- ss-SubscriptionViolation -- localValue 19,

-- negativePW-Check -- localValue 38,

-- numberOfPW-AttemptsViolation -- localValue 43}

::= localValue 13

interrogateSS OPERATION

ARGUMENT

ss-ForBS   SEQUENCE {

ss-Code    OCTET STRING (SIZE (1)),

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

... }

RESULT

interrogateSS-Res CHOICE {

ss-Status  [0] IMPLICIT OCTET STRING (SIZE (1)),

forwardedToNumber [1] IMPLICIT OCTET STRING (SIZE (1..9)),

basicServiceGroupList [2] IMPLICIT SEQUENCE SIZE (1..13) OF

CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))},

forwardingFeatureList [3] IMPLICIT SEQUENCE SIZE (1..13) OF

SEQUENCE {

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

forwardedToNumber [5] IMPLICIT OCTET STRING (SIZE (1..9)) OPTIONAL,

forwardedToSubaddress [8] IMPLICIT OCTET STRING (SIZE (1..21)) OPTIONAL,

forwardingOptions [6] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

noReplyConditionTime [7] IMPLICIT INTEGER (5..30) OPTIONAL,

... }}

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- bearerServiceNotProvisioned -- localValue 10,

-- teleserviceNotProvisioned -- localValue 11,

-- callBarred -- localValue 13,

-- illegalSS-Operation -- localValue 16,

-- ss-NotAvailable -- localValue 18}

::= localValue 14

processUnstructuredSS-Data OPERATION

ARGUMENT

ss-UserData IA5String   (SIZE (1..200))

RESULT

ss-UserData IA5String   (SIZE (1..200))

ERRORS {

-- systemFailure -- localValue 34,

-- unexpectedDataValue -- localValue 36}

::= localValue 19

processunstructuredSS-Request OPERATION

ARGUMENT

ussd-Arg   SEQUENCE {

ussd-DataCodingScheme OCTET STRING (SIZE (1)),

ussd-String OCTET STRING (SIZE (1..160)),

... }

RESULT

ussd-Res   SEQUENCE {

ussd-DataCodingScheme OCTET STRING (SIZE (1)),

ussd-String OCTET STRING (SIZE (1..160)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownAlphabet -- localValue 71,

-- callBarred -- localValue 13}

::= localValue 59

unstructuredSS-Request OPERATION

ARGUMENT

ussd-Arg   SEQUENCE {

ussd-DataCodingScheme OCTET STRING (SIZE (1)),

ussd-String OCTET STRING (SIZE (1..160)),

... }

RESULT

ussd-Res   SEQUENCE {

ussd-DataCodingScheme OCTET STRING (SIZE (1)),

ussd-String OCTET STRING (SIZE (1..160)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- absentSubscriber -- localValue 27,

-- illegalSubscriber -- localValue 9,

-- illegalEquipment -- localValue 12,

-- unknownAlphabet -- localValue 71,

-- ussd-Busy -- localValue 72}

::= localValue 60

unstructuredSS-Notify OPERATION

ARGUMENT

ussd-Arg   SEQUENCE {

ussd-DataCodingScheme OCTET STRING (SIZE (1)),

ussd-String OCTET STRING (SIZE (1..160)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- absentSubscriber -- localValue 27,

-- illegalSubscriber -- localValue 9,

-- illegalEquipment -- localValue 12,

-- unknownAlphabet -- localValue 71,

-- ussd-Busy -- localValue 72}

::= localValue 61

registerPassword OPERATION

ARGUMENT

ss-Code    OCTET STRING (SIZE (1))

RESULT

newPassword NumericString  (FROM ("0"|"1"|"2"|"3"|"4"|"5"|"6"|"7"|"8"|"9")|SIZE (4))

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- callBarred -- localValue 13,

-- ss-SubscriptionViolation -- localValue 19,

-- pw-RegistrationFailure -- localValue 37,

-- negativePW-Check -- localValue 38,

-- numberOfPW-AttemptsViolation -- localValue 43}

LINKED {

-- getPassword -- localValue 18}

::= localValue 17

getPassword OPERATION

ARGUMENT

guidanceInfo ENUMERATED {

enterPW     (0),

enterNewPW  (1),

enterNewPW-Again  (2),

badPW-TryAgain  (3),

badPW-FormatTryAgain  (4)}

RESULT

currentPassword NumericString  (FROM ("0"|"1"|"2"|"3"|"4"|"5"|"6"|"7"|"8"|"9")|SIZE (4))

::= localValue 18

beginSubscriberActivity OPERATION

ARGUMENT

beginSubscriberActivityArg SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

originatingEntityNumber OCTET STRING (SIZE (1..9))}

::= localValue 54

sendRoutingInfoForSM OPERATION

ARGUMENT

routingInfoForSM-Arg SEQUENCE {

msisdn     [0] IMPLICIT OCTET STRING (SIZE (1..9)),

sm-RP-PRI  [1] IMPLICIT BOOLEAN,

serviceCentreAddress [2] IMPLICIT OCTET STRING (SIZE (1..20)),

teleservice [5] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... }

RESULT

routingInfoForSM-Res SEQUENCE {

imsi       OCTET STRING (SIZE (3..8)),

locationInfoWithLMSI [0] IMPLICIT SEQUENCE {

locationInfo CHOICE {

roamingNumber [0] IMPLICIT OCTET STRING (SIZE (1..9)),

msc-Number [1] IMPLICIT OCTET STRING (SIZE (1..9))},

lmsi       OCTET STRING (SIZE (4)) OPTIONAL,

... },

mwd-Set    [2] IMPLICIT BOOLEAN OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unknownSubscriber -- localValue 1,

-- teleserviceNotProvisioned -- localValue 11,

-- absentSubscriber -- localValue 27,

-- callBarred -- localValue 13}

::= localValue 45

forwardSM  OPERATION

ARGUMENT

forwardSM-Arg SEQUENCE {

sm-RP-DA   CHOICE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

lmsi       [1] IMPLICIT OCTET STRING (SIZE (4)),

roamingNumber [3] IMPLICIT OCTET STRING (SIZE (1..9)),

serviceCentreAddressDA [4] IMPLICIT OCTET STRING (SIZE (1..20)),

noSM-SP-DA [5] IMPLICIT NULL},

sm-RP-OA   CHOICE {

msisdn     [2] IMPLICIT OCTET STRING (SIZE (1..9)),

serviceCentreAddressOA [4] IMPLICIT OCTET STRING (SIZE (1..20)),

noSM-RP-OA [5] IMPLICIT NULL},

sm-RP-UI   OCTET STRING (SIZE (1..200)),

moreMessagesToSend NULL OPTIONAL,

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unidentifiedSubscriber -- localValue 5,

-- illegalSubscriber -- localValue 9,

-- illegalEquipment -- localValue 12,

-- absentSubscriber -- localValue 27,

-- sm-DeliveryFailure -- localValue 32}

::= localValue 46

reportSM-DeliveryStatus OPERATION

ARGUMENT

reportSM-DeliveryStatusArg SEQUENCE {

msisdn     OCTET STRING (SIZE (1..9)),

serviceCentreAddress OCTET STRING (SIZE (1..20)),

sm-DeliveryOutcome ENUMERATED {

memoryCapacityExceeded  (0),

absentSubscriber  (1),

successfulTransfer  (2)} OPTIONAL,

... }

RESULT

storedMSISDN OCTET STRING (SIZE (1..9))

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- unknownSubscriber -- localValue 1,

-- messageWaitingListFull -- localValue 33}

::= localValue 47

noteSubscriberPresent OPERATION

ARGUMENT

imsi       OCTET STRING (SIZE (3..8))

::= localValue 48

alertServiceCentreWithoutResult OPERATION

ARGUMENT

alertServiceCentreArg SEQUENCE {

msisdn     OCTET STRING (SIZE (1..9)),

serviceCentreAddress OCTET STRING (SIZE (1..20)),

... }

::= localValue 49

informServiceCentre OPERATION

ARGUMENT

informServiceCentreArg SEQUENCE {

storedMSISDN OCTET STRING (SIZE (1..9)) OPTIONAL,

mw-Status  BIT STRING {

sc-AddressNotIncluded  (0),

mnrf-Set    (1),

mcef-Set    (2)} (SIZE (6)) OPTIONAL,

... }

::= localValue 63

alertServiceCentre OPERATION

ARGUMENT

alertServiceCentreArg SEQUENCE {

msisdn     OCTET STRING (SIZE (1..9)),

serviceCentreAddress OCTET STRING (SIZE (1..20)),

... }

ERRORS {

-- systemFailure -- localValue 34,

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36}

::= localValue 64

readyForSM OPERATION

ARGUMENT

readyForSM-Arg SEQUENCE {

subscriberId CHOICE {

imsi       [0] IMPLICIT OCTET STRING (SIZE (3..8)),

tmsi       [1] IMPLICIT OCTET STRING (SIZE (1..4))},

alertReason ENUMERATED {

ms-Present  (0),

memoryAvailable  (1)},

... }

ERRORS {

-- dataMissing -- localValue 35,

-- unexpectedDataValue -- localValue 36,

-- facilityNotSupported -- localValue 21,

-- unknownSubscriber -- localValue 1}

::= localValue 66

systemFailure ERROR

PARAMETER

networkResource ENUMERATED {

plmn        (0),

hlr         (1),

vlr         (2),

pvlr        (3),

controllingMSC  (4),

vmsc        (5),

eir         (6),

rss         (7)}

::= localValue 34

dataMissing ERROR

::= localValue 35

unexpectedDataValue ERROR

::= localValue 36

facilityNotSupported ERROR

::= localValue 21

unknownSubscriber ERROR

::= localValue 1

numberChanged ERROR

::= localValue 44

unknownBaseStation ERROR

::= localValue 2

unknownMSC ERROR

::= localValue 3

unidentifiedSubscriber ERROR

::= localValue 5

unknownEquipment ERROR

::= localValue 7

roamingNotAllowed ERROR

PARAMETER

roamingNotAllowedCause ENUMERATED {

plmnRoamingNotAllowed  (0),

operatorDeterminedBarring  (3)}

::= localValue 8

illegalSubscriber ERROR

::= localValue 9

illegalEquipment ERROR

::= localValue 12

bearerServiceNotProvisioned ERROR

::= localValue 10

teleserviceNotProvisioned ERROR

::= localValue 11

invalidTargetBaseStation ERROR

::= localValue 23

noRadioResourceAvailable ERROR

::= localValue 24

noHandoverNumberAvailable ERROR

::= localValue 25

subsequentHandoverFailure ERROR

::= localValue 26

tracingBufferFull ERROR

::= localValue 40

noRoamingNumberAvailable ERROR

::= localValue 39

absentSubscriber ERROR

PARAMETER

mwd-Set    BOOLEAN

::= localValue 27

callBarred ERROR

PARAMETER

callBarringCause ENUMERATED {

barringServiceActive  (0),

operatorBarring  (1)}

::= localValue 13

forwardingViolation ERROR

::= localValue 14

cug-Reject ERROR

PARAMETER

cug-RejectCause ENUMERATED {

incomingCallsBarredWithinCUG  (0),

subscriberNotMemberOfCUG  (1),

requestedBasicServiceViolatesCUG-Constraints  (5),

calledPartySS-InteractionViolation  (7)}

::= localValue 15

illegalSS-Operation ERROR

::= localValue 16

ss-ErrorStatus ERROR

PARAMETER

ss-Status  OCTET STRING (SIZE (1))

::= localValue 17

ss-NotAvailable ERROR

::= localValue 18

ss-SubscriptionViolation ERROR

PARAMETER

ss-SubscriptionOption CHOICE {

cliRestrictionOption [2] IMPLICIT ENUMERATED {

permanent   (0),

temporaryDefaultRestricted  (1),

temporaryDefaultAllowed  (2)},

overrideCategory [1] IMPLICIT ENUMERATED {

overrideEnabled  (0),

overrideDisabled  (1)}}

::= localValue 19

ss-Incompatibility ERROR

PARAMETER

ss-IncompatibilityCause SEQUENCE {

ss-Code    [1] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

basicService CHOICE {

bearerService [2] IMPLICIT OCTET STRING (SIZE (1)),

teleservice [3] IMPLICIT OCTET STRING (SIZE (1))} OPTIONAL,

ss-Status  [4] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL,

... }

::= localValue 20

unknownAlphabet ERROR

::= localValue 71

ussd-Busy  ERROR

::= localValue 72

pw-RegistrationFailure ERROR

PARAMETER

pw-RegistrationFailureCause ENUMERATED {

undetermined  (0),

invalidFormat  (1),

newPasswordsMismatch  (2)}

::= localValue 37

negativePW-Check ERROR

::= localValue 38

numberOfPW-AttemptsViolation ERROR

::= localValue 43

sm-DeliveryFailure ERROR

PARAMETER

sm-DeliveryFailureCause CHOICE {

sm-DeliveryFailureCauseWithDiagnostic SEQUENCE {

sm-EnumeratedDeliveryFailureCause ENUMERATED {

memoryCapacityExceeded  (0),

equipmentProtocolError  (1),

equipmentNotSM-Equipped  (2),

unknownServiceCentre  (3),

sc-Congestion  (4),

invalidSME-Address  (5),

subscriberNotSC-Subscriber  (6)},

diagnosticInfo OCTET STRING (SIZE (1..200)) OPTIONAL,

... },

sm-EnumeratedDeliveryFailureCause ENUMERATED {

memoryCapacityExceeded  (0),

equipmentProtocolError  (1),

equipmentNotSM-Equipped  (2),

unknownServiceCentre  (3),

sc-Congestion  (4),

invalidSME-Address  (5),

subscriberNotSC-Subscriber  (6)}}

::= localValue 32

messageWaitingListFull ERROR

::= localValue 33

END

B.2	Fully Expanded ASN.1 Source of MAP-DialogueInformation

--    Expanded ASN.1 Module 'MAP-DialogueInformation'

--SIEMENS ASN.1 Compiler       P2.00 (94-01-31 14:46:00)

--             Date: 94-02-15 Time: 12:22:04

MAP-DialogueInformation { ccitt identified-organization(4) etsi(0) mobileDomain(0)

gsm-Network(1) modules(3) map-DialogueInformation(3) version2(2) }

DEFINITIONS

::=

BEGIN

EXPORTS

map-DialogueAS,

MAP-DialoguePDU

;

map-DialogueAS OBJECT IDENTIFIER ::= { ccitt(0) identified-organization(4)

etsi(0) mobileDomain(0) gsm-Network(1) 1 map-DialoguePDU(1) version1(1) }

MAP-DialoguePDU ::= CHOICE {

map-open   [0] IMPLICIT SEQUENCE {

destinationReference [0] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

originationReference [1] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

... },

map-accept [1] IMPLICIT SEQUENCE {

... },

map-close  [2] IMPLICIT SEQUENCE {

... },

map-refuse [3] IMPLICIT SEQUENCE {

reason     ENUMERATED {

noReasonGiven  (0),

invalidDestinationReference  (1),

invalidOriginatingReference  (2)},

... },

map-userAbort [4] IMPLICIT SEQUENCE {

map-UserAbortChoice CHOICE {

userSpecificReason [0] IMPLICIT NULL,

userResourceLimitation [1] IMPLICIT NULL,

resourceUnavailable [2] IMPLICIT ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)},

applicationProcedureCancellation [3] IMPLICIT ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}},

... },

map-providerAbort [5] IMPLICIT SEQUENCE {

map-ProviderAbortReason ENUMERATED {

abnormalDialogue  (0),

invalidPDU  (1)},

... }}

MAP-OpenInfo ::= SEQUENCE {

destinationReference [0] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

originationReference [1] IMPLICIT OCTET STRING (SIZE (1..20)) OPTIONAL,

... }

MAP-AcceptInfo ::= SEQUENCE {

... }

MAP-CloseInfo ::= SEQUENCE {

... }

MAP-RefuseInfo ::= SEQUENCE {

reason     ENUMERATED {

noReasonGiven  (0),

invalidDestinationReference  (1),

invalidOriginatingReference  (2)},

... }

Reason     ::= ENUMERATED {

noReasonGiven  (0),

invalidDestinationReference  (1),

invalidOriginatingReference  (2)}

MAP-UserAbortInfo ::= SEQUENCE {

map-UserAbortChoice CHOICE {

userSpecificReason [0] IMPLICIT NULL,

userResourceLimitation [1] IMPLICIT NULL,

resourceUnavailable [2] IMPLICIT ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)},

applicationProcedureCancellation [3] IMPLICIT ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}},

... }

MAP-UserAbortChoice ::= CHOICE {

userSpecificReason [0] IMPLICIT NULL,

userResourceLimitation [1] IMPLICIT NULL,

resourceUnavailable [2] IMPLICIT ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)},

applicationProcedureCancellation [3] IMPLICIT ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}}

ResourceUnavailableReason ::= ENUMERATED {

shortTermResourceLimitation  (0),

longTermResourceLimitation  (1)}

ProcedureCancellationReason ::= ENUMERATED {

handoverCancellation  (0),

radioChannelRelease  (1),

networkPathRelease  (2),

callRelease  (3),

associatedProcedureFailure  (4),

tandemDialogueRelease  (5),

remoteOperationsFailure  (6)}

MAP-ProviderAbortInfo ::= SEQUENCE {

map-ProviderAbortReason ENUMERATED {

abnormalDialogue  (0),

invalidPDU  (1)},

... }

MAP-ProviderAbortReason ::= ENUMERATED {

abnormalDialogue  (0),

invalidPDU  (1)}

END

Annex C (informative): Formal protocol incompatibilities between versions 1 & 2 of MAP

C.1	Introduction

Annex C is not normative; it presents for information those parts of the MAP version 2 protocol which are not backward compatible with (i.e. a true superset of) the MAP version 1 protocol. For each incompatibility there is a commentary on the impact on the interworking of MAP version 1 and MAP version 2 entities.

SMG have decided that the MAP specification should include the operations and procedures used on the B interface (MSC/VLR) only for modelling purposes; the B interface cannot be implemented as an open interface. Hence any incompatibilities which affect operations used only on the B interface have no impact on the interworking of MAP version 1 and MAP version 2 entities.

C.2	Deletion of operations and errors

This section lists the operations and errors which have been completely removed from the MAP protocol.

C.2.1	Deletion of operation RegisterChargingInfo

There is no known implementation of MAP version 1 which supports this operation. The deletion has been approved by SMG.

C.2.2	Deletion of operation ForwardSS-Notification

There is no known implementation of MAP version 1 which supports this operation. The deletion has been approved by SMG.

C.2.3	Deletion of error InsufficientBearerCapabilities

This error is defined in the MAP version 1 protocol, but it is not specified for use with any operation.

C.3	Deletion of errors for operations

This section lists the cases where an error which is specified for use with a MAP version 1 operation is not specified for use with the same operation in MAP version 2.

C.3.1	Error NegativePW-Check for operation RegisterSS

Password checking is not used for the supplementary services to which registration applies.

C.3.2	Error NegativePW-Check for operation EraseSS

Password checking is not used for the supplementary services to which erasure applies.

C.3.3	Error NegativePW-Check for operation InterrogateSS

Password checking is not used for the interrogation of supplementary services.

C.3.4	Errors SS-SubscriptionViolation, SS-Incompatibility and CUG-Reject for operation InvokeSS

The operation InvokeSS is used only on the B interface.

C.3.5	Error CUG-Reject for operation SendRoutingInfoForSM

Closed User Group does not apply to the short message service.

C.4	Changes to definitions of data types

This section lists in alphabetical order those data types whose definitions have been directly or indirectly changed. For constructed data types, only the components which have been changed are mentioned. The commentary on the end effect of each change is given in sections C.6 (parameters of operations), C.7 (results of operations) or C.8 (errors of operations).

C.4.1	CUG-Feature

The type CUG-Feature was a sequence of four components; these have been deleted and replaced by three new components. It is used for the components of the type CUG-FeatureList.

C.4.2	CUG-FeatureList

The type CUG-FeatureList is a sequence of components of type CUG-Feature. It is used for a component of the type CUG-Info.

C.4.3	CUG-Info

The type CUG-Info is a sequence. One component of the sequence has been replaced by a component of a new type; the other component was a choice between a cug-Feature and a cug-FeatureList, and is now an optional cug-FeatureList. The type CUG-Info is used for a component of the type SS-Info.

C.4.4	CUG-RejectCause

The range of permitted values of the enumerated type CUG-RejectCause has been extended. The type is used for the parameter of the error CUG-Reject.

C.4.5	IMSI

The lower limit of the length of an IMSI has been increased from 2 octets to 3 octets. It is not possible to code a minimum length IMSI (MCC+MNC+MSIN) in 2 octets, so the theoretical lower limit of 2 octets should never be used by a MAP version 1 entity to send an IMSI; if it is, a MAP version 2 entity will treat it as a protocol error. Since this change has no practical impact it is not discussed further.

C.4.6	ISDN-AddressString

The upper limit of the length of an ISDN-AddressString has been reduced from 10 octets to 9 octets. The maximum length of an E.164 number is 15 digits; this can be encoded as a TBCD-string in 8 octets, plus a further octet to hold the type of number and number plan indicator. The cases where the ISDN-AddressString type was used in MAP version 1 to carry anything other than an E.164 number are described in section C.6; the other cases are not discussed further.

C.4.7	Password

In MAP version 1 the type Password was a choice between a printable string of length 4 to 8 octets or a numeric string of length 4 octets. It is now a numeric string of length 4 octets. The type Password is used for the result of the operation GetPassword.

C.4.8	RequestParameter

The enumerated type RequestParameter is no longer allowed to take the value requestCUG-Info. It is used as a component of the type RequestParameterList.

C.4.9	RequestParameterList

The type RequestParameterList is a sequence of components of type RequestParameter. The parameter of the operation SendParameters is a sequence of which one component is of type RequestParameterList.

C.4.10	SentParameter

The type SentParameter is a choice of which one component is of type SubscriberData. It is used as a component of the type SentParameterList.

C.4.11	SentParameterList

The type SentParameterList is a sequence whose components are of type SentParameter. The maximum number of components in the sequence has been reduced from 10 to 6.

The type SentParameterList is used for the result of the operation SendParameters.

C.4.12	SS-Info

The type SS-Info is a choice of which one component is of type CUG-Info. It is used for the result of the operations RegisterSS, EraseSS, ActivateSS and DeactivateSS, and for components of the type SS-InfoList.

C.4.13	SS-InfoList

The type SS-InfoList is a sequence of components of type SS-Info. It is used for a component of the type SubscriberData

C.4.14	SubscriberData

The type SubscriberData is a sequence of which one component is of type SS-InfoList. Components of SubscriberData are used as a component of the parameter of the operation InsertSubscriberData; the type is also used for a component of the type SentParameter.

C.5	Changes to parameters of errors

This section lists in alphabetical order the errors whose parameters have changed.

C.5.1	CUG-Reject

The error CUG-Reject has an optional parameter of type CUG-RejectCause. The error CUG-Reject is used for the operations SendInfoForOutgoingCall and SendRoutingInfo.

C.6	Changes to parameters of operations

This section lists in alphabetical order the operations whose parameters have changed, and gives a commentary on the effect of the changes on each operation.

C.6.1	InvokeSS

The operation InvokeSS is used only on the B interface.

C.6.2	InsertSubscriberData

The parameter of the operation InsertSubscriberData is a sequence of which one component is a sequence of components of SubscriberData. The component of SubscriberData which is affected by the changes listed in section C.4 is CUG-Info.

The CUG supplementary service is not supported by MAP version 1; CUG-Info should therefore not be used as a component of SubscriberData in a dialogue involving a MAP version 1 entity.

C.6.3	RegisterSS

The forwardedToNumber component of the parameter of the operation RegisterSS had a maximum length of 10 octets in MAP version 1, as it was of the type ISDN-AddressString. In MAP version 2 the maximum length is 20 octets, as the type is AddressString. The maximum length (9 octets) of the ISDN-AddressString type in MAP version 2 may not be adequate to hold the forwardedToNumber, which is not necessarily an E.164 number; the user may enter the number using the digits for international access rather than the '+' key.

C.6.4	SendInfoForIncomingCall

The operation SendInfoForIncomingCall is used only on the B interface.

C.6.5	SendInfoForOutgoingCall

The operation SendInfoForOutgoingCall is used only on the B interface.

C.6.6	SendParameters

The operation SendParameters uses as its parameter a sequence of which one component is of type RequestParameter. The value requestCUG-Info can no longer be used for this component. The SendParameters operation is used only when interworking with a MAP version 1 entity, and MAP version 1 does not support the GSM Phase 2 CUG supplementary service, so the SendParameters operation should in any case not be used to request CUG information.

C.6.7	SendRoutingInfoForSM

The cug-Interlock component of the parameter of the operation SendRoutingInfoForSM has been deleted. Closed User Group does not apply to the short message service.

C.7	Changes to results of operations

This section lists in alphabetical order the operations whose results have changed, and gives a commentary on the effect of the changes on each operation.

C.7.1	ActivateSS

The result of the operation ActivateSS is of type SS-Info. One data type used for components of SS-Info has suffered incompatible changes: CUG-Info.

The ActivateSS operation does not apply to the CUG supplementary service, so the cug-Info component of SS-Info should never be present in the result of the operation ActivateSS.

C.7.2	DeactivateSS

The result of the operation DeactivateSS is of type SS-Info. One data type used for components of SS-Info has suffered incompatible changes: CUG-Info.

The DeactivateSS operation does not apply to the CUG supplementary service, so the cug-Info component of SS-Info should never be present in the result of the operation DeactivateSS.

C.7.3	EraseSS

The result of the operation EraseSS is of type SS-Info. One data type used for components of SS-Info has suffered incompatible changes: CUG-Info.

The EraseSS operation does not apply to the CUG supplementary service, so the cug-Info component of SS-Info should never be present in the result of the operation EraseSS.

C.7.4	GetPassword

The result of the operation GetPassword is of type Password. In MAP version 1 this was a choice between a printable string of length 4 to 8 octets or a numeric string of length 4 octets. It is now a numeric string of length 4 octets. The printable string option was never used in MAP version 1, as indicated by a comment in the ASN.1 in the latest phase 1 version of TS GSM 09.02.

C.7.5	InterrogateSS

The result of the InterrogateSS operation is a CHOICE; one of the components of the CHOICE is a list of basic services to which the supplementary service applies, which is used for the Call Barring supplementary service. In MAP version 1 this list can in principle have up to 70 members, the number of individual basic services. However Call Barring can apply to only 13 basic service groups. In MAP version 2 the length of the list of basic service codes which can be returned in the result of the InterrogateSS operation is reduced to 13 to reflect this.

C.7.6	RegisterSS

The result of the operation RegisterSS is of type SS-Info. One data type used for components of SS-Info has suffered incompatible changes: CUG-Info.

The RegisterSS operation does not apply to the CUG supplementary service, so the cug-Info component of SS-Info should never be present in the result of the operation RegisterSS.

C.7.7	SendParameters

The result of the operation SendParameters is of type SentParameterList, which is a sequence of components of type SentParameter. The maximum number of components in the sequence has been reduced from 10 to 6. MAP version 1 could in principle send 10 sets of CUG-Information, but the supplementary service Closed User Group is not defined for GSM Phase 1, and the MAP version 1 signalling protocol will not support Closed User Group as defined for GSM Phase 2, so a MAP version 1 entity should never request parameters for CUG. The maximum number of sent parameters therefore consists of an IMSI and 5 AuthenticationSets - a total of 6.

The type SentParameter is a choice of which one component is of type SubscriberData. The component of SubscriberData which is affected by the changes listed in section C.4 is CUG-Info.

The CUG supplementary service is not supported by MAP version 1; CUG-Info should therefore not be used as a component of SubscriberData in a dialogue involving a MAP version 1 entity.

C.7.8	SendRoutingInfoForSM

The result of the operation SendRoutingInfoForSM is a sequence of which one component was a choice between location information (optionally with an associated LMSId) and forwarding data; the choice of forwarding data has been removed. Call Forwarding does not apply to the Short Message Service.

C.8	Changes to errors of operations

This section lists in alphabetical order the operations whose errors have changed, and gives a commentary on the effect of the changes on each operation.

C.8.1	SendInfoForOutgoingCall

The operation SendInfoForOutgoingCall is used only on the B interface.

C.8.2	SendRoutingInfo

The definition of the type (CUG-RejectCause) used for the optional parameter of the error CUG-Reject has been changed. However the supplementary service Closed User Group is not defined for GSM Phase 1, and the MAP version 1 signalling protocol will not support Closed User Group as defined for GSM Phase 2, so the error CUG-Reject should not be used in a dialogue involving a MAP version 1 entity.

History

Document history

March 1994	Public Enquiry	PE 59:	1994-03-21 to 1994-08-12

April 1994	Change requests approved

by SMG#10