ETSI/TC GSM

Title : Recommendation GSM 09.10-DCS

Information element mapping between MS-BSS/BSS-MSC signalling procedures and the Mobile Application Part

Version 3.0.0 (based on GSM 09.10 version 3.0.2)

Date: 24 January, 1991

List of Contents (underlined sections indicate changes to

GSM 09.10)

1     Scope

2     Classification of interworking cases

3     Transparent case

4     NON-TRANSPARENT CASE

Language of original: English

Number of pages :  8

1. Scope

The scope of this Recommendation is

i)	to provide guidance on information element mapping between information elements contained in layer 3 messages sent on the MS-BSS interface (Recommendation GSM 04.08) and information elements contained in MAP messages (Recommendation GSM 09.02) where the MSC acts as transparent relay of information;

ii)	to provide guidance on mapping as in i) above and additional interworking when the MSC also processes the information.

Note: 	Interworking for supplementary services is given in Recommendation GSM 09.11.

GSM 09.10-DCS consists of GSM 09.10 with the pages in this specification replacing those in GSM 09.10.

2. Classification of interworking cases

2.1 Transparent operations

The following operations in MAP require transparent mapping in

the MSC (see Recommendation GSM 09.02 for definitions and the use of the operations):

- update location area

- detach IMSI

- attach IMSI

- forward new TMSI

- provide IMSI

- authenticate

- perform call control (in MSC-B)

3.2.2 Location area updating

────────┬────────────────────────────────────────────────┬─────

│      04.08                    09.02            │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │     LOC UPD REQ           Invoke: Update       │

message │                           location area        │

│     Location area id          Loc Area ID      │

│     Mobile identity           Subscriber ID    │

│                                                │

│     Mobile station                             │

│     classmark                      -           │  4

│                                                │

│     Cell identity                  -           │  5

│                                                │

│          -                  Target Loc Area ID │  1

│                                                │

│     Ciphering key             Ciphering key    │

│     seq number                seq number       │

│                                                │

│     Location update type           -           │  3

────────┼────────────────────────────────────────────────┼─────

Positive│     LOC UPD ACC           Return result:       │

results │                           Location area up-    │

│                           dating accepted      │

│                                                │

│     Mobile identity                -           │  6

────────┼────────────────────────────────────────────────┼─────

Negative│     LOC UPD REJ           Return error         │

results │                                                │

│     IMSI unknown in HLR   Unknown Subscriber   │

│     Network failure       Unknown Loc Area     │  2

│     Roaming not allowed   Roaming Not Allowed  │

│     PLMN not allowed      "Roaming not allowed"│

│                           with parameter "PLMN │

│                           not allowed"         │

│                                                │

│     National roaming not  "Roaming not allowed"│

│     allowed in this       with parameter       │

│     location area         "National roaming not│

│                           allowed"             │

│                                                │

│     Illegal MS            Illegal MS           │

│     Network failure       System Failure       │

│     Network failure       Reject               │

│     Network failure       U-reject             │

│     Network failure       Abort                │

│     Network failure       L-cancel             │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	This information element is inserted by the MSC.

2)	The Unknown Loc Area error is not generated because of wrong information provided by the MS. The wrong information has been inserted by the MSC.

3)	This parameter is used on the radio path in order to discriminate between normal location updating, periodic registration and IMSI attach.

4)	The classmark is required only for radio resource management and is not transferred to the VLR.

5)	This information  element is not transferred to the VLR.

6)	The mobile identity is inserted by the MSC as received in a forward new TMSI operation.