ETSI/TC GSM

Title : Recommendation GSM 09.10

Information element mapping between MS-BSS/BSS-MSC signalling procedures and the Mobile Application Part

Version 3.0.2

Date: 21 February, 1990

List of Contents

1     Scope

2     Classification of interworking cases

3     Transparent case

4     Non-transparent case

Language of original: English

Number of pages :  17

1. Scope

The scope of this Recommendation is

i)	to provide guidance on information element mapping between information elements contained in layer 3 messages sent on the MS-BSS interface (Recommendation GSM 04.08) and information elements contained in MAP messages (Recommendation GSM 09.02) where the MSC acts as transparent relay of information;

ii)	to provide guidance on mapping as in i) above and additional interworking when the MSC also processes the information.

Note: 	Interworking for supplementary services is given in Recommendation GSM 09.11.

2. Classification of interworking cases

2.1 Transparent operations

The following operations in MAP require transparent mapping in

the MSC (see Recommendation GSM 09.02 for definitions and the use of the operations):

- update location area

- detach IMSI

- attach IMSI

- forward new TMSI

- provide IMSI

- authenticate

- perform call control (in MSC-B)

2.2 Non-transparent operations

Operations in this class require processing in the MSC and information element mapping. The operations include:

- operations related to outgoing call set-up

- operations related to incoming call set-up

- operations related to handover

3. Transparent case

3.1 Interworking in the BSS

All messages will use DTAP (see Recommendation GSM 08.08).

3.2 Interworking in the MSC

3.2.1 General

When receiving a forward message from the MS, the MSC will invoke the desired operation of MAP and establish a cross reference between the Recommendation GSM 04.08 procedure and the MAP procedure in order to return the results of the operation to the MS. The cross reference is deleted when the MSC terminates the MAP operation.

Positive or negative results of the MAP operation are returned in the appropriate message of Recommendation GSM 04.08.

The parameters of the forward Recommendation GSM 04.08 message is mapped by a one-to-one mapping into the parameters of the invoke massage of MAP. However, in some cases parameters received on the radio path may be suppressed at the MSC because they are related to another protocol entity, e.g. in MM-management messages information related to RR-management may be included. Similarly, parameters received in the return result is mapped one-to-one into parameters of the corresponding backward message defined in Recommendation GSM 04.08.

Negative outcome as carried in various MAP messages (TC-U-ERROR, TC-L-REJECT, TC-R-REJECT, TC-U-REJECT, T-CP-ABORT and TC-L-CANCEL, see Recommendation GSM 09.02 for definitions) is mapped into cause values in the required backward unsuccessful message in Recommendation GSM 04.08. In this case several negative results of MAP may be mapped into the same cause in Recommendation GSM 04.08, i.e. without discrimination between these negative results.

Note: 	The MAP procedure entity in the MSC will require a more detailed overview of negative results than the MS for O & M purposes.

These principles are illustrated in Figure 1.

For each of the procedures in para 2.1 above the following format is used to show the mapping.

────────┬────────────────────────────────────────────────┬─────

│      04.08                    09.02            │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │     MS to MSC                 MSC to VLR       │

message │     Message name              Invoke name      │

│     para 1          <----->   para 1           │

│     para 2          <----->   para 2           │

────────┼────────────────────────────────────────────────┼─────

Positive│     MSC to MS                 VLR to MSC       │

result  │     Message name              Return result    │

│     para 1          <----->   para 1           │

│     para 2          <----->   para 2           │

────────┼────────────────────────────────────────────────┼─────

Negative│     MSC to MS                 VLR to MSC       │

result  │     Message name              Return Error     │

│     cause 1         <----->   cause 1          │

│     cause 2         <----->   cause 2          │

│     cause           <----->   Reject           │

│     cause           <----->   U-reject         │

│     cause           <----->   Abort            │

│     cause           <----->   L-cancel         │

────────┴────────────────────────────────────────────────┴─────

Equivalent mapping principles apply for operations invoked by the VLR. However, negative results are then not received on the radio path but generated in the MSC. Therefore, for such operations the interworking for negative results is not shown.

04.08 (08.08)                                  09.02

forward message                               invoke

────────────>                                ─────────>

┌────────┐                                   ┌────────┐

│  para  │                                   │  para  │

└───┬────┘           one-to-one              └───┬────┘

└────────────────────────────────────────>───┘

mapping

return

positive ack                                 result

<───────────                               <───────────

┌────────┐                                   ┌────────┐

│  para  │                                   │  para  │

└────┬───┘           one-to-one              └───┬────┘

└────<──────────────────────────────────────┘

mapping

negative

negative cause        ┌───────────┐            result

<──────────           │Translation│         <─────────

└───┬───┬───┘

┌────────┐                │   │              ┌───────┐

│  cause │                │   │              │ cause │

└───┬────┘                │   │              └───┬───┘

└─────<───────────────┘   └──<───────────────┘

Figure 1  Illustration of mapping principles in the MSC

3.2.2 Location area updating

────────┬────────────────────────────────────────────────┬─────

│      04.08                    09.02            │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │     LOC UPD REQ           Invoke: Update       │

message │                           location area        │

│     Location area id          Loc Area ID      │

│     Mobile identity           Subscriber ID    │

│                                                │

│     Mobile station                             │

│     classmark                      -           │  4

│                                                │

│     Cell identity                  -           │  5

│                                                │

│          -                  Target Loc Area ID │  1

│                                                │

│     Ciphering key             Ciphering key    │

│     seq number                seq number       │

│                                                │

│     Location update type           -           │  3

────────┼────────────────────────────────────────────────┼─────

Positive│     LOC UPD ACC           Return result:       │

results │                           Location area up-    │

│                           dating accepted      │

│                                                │

│     Mobile identity                -           │  6

────────┼────────────────────────────────────────────────┼─────

Negative│     LOC UPD REJ           Return error         │

results │                                                │

│     IMSI unknown in HLR   Unknown Subscriber   │

│     Network failure       Unknown Loc Area     │  2

│     Roaming not allowed   Roaming Not Allowed  │  2

│     Illegal MS            Illegal MS           │

│     Network failure       System Failure       │

│     Network failure       Reject               │

│     Network failure       U-reject             │

│     Network failure       Abort                │

│     Network failure       L-cancel             │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	This information element is inserted by the MSC.

2)	The Unknown Loc Area error is not generated because of wrong information provided by the MS. The wrong information has been inserted by the MSC.

3)	This parameter is used on the radio path in order to discriminate between normal location updating, periodic registration and IMSI attach.

4)	The classmark is required only for radio resource management and is not transferred to the VLR.

5)	This information  element is not transferred to the VLR.

6)	The mobile identity is inserted by the MSC as received in a forward new TMSI operation.

3.2.3 Detach IMSI

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ IMSI DET IND             Invoke: detach IMSI   │

message │                                                │

│ Mobile identity          Subscriber Id         │

────────┼────────────────────────────────────────────────┼─────

Positive│                                                │

result  │                                                │  1

────────┼────────────────────────────────────────────────┼─────

Negative│                                                │

result  │                                                │  2

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	The forward message is not acknowledged

2)	Negative results (reject, U-reject, abort) may be received by MAP. However, these results are not provided to the MS.

3.2.4 Attach IMSI

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │   LOC UPD REC            Invoke: Attach IMSI   │

message │                                                │

│   Location area Id       Loc Area Id           │

│   Mobile identity        Subscriber Id         │

│                                                │

│   Mobile station                               │

│   class mark                   -               │  2

│   Cell identity                -               │  2

│   Cipher key seq number  Cipher key seq number │

│   Location update type         -               │  2

────────┼────────────────────────────────────────────────┼─────

Positive│   LOC UPD ACC            Return result:        │

result  │                          IMSI attach ack       │  1

────────┼────────────────────────────────────────────────┼─────

Negative│   LOC UPD REJ            Return result:        │

result  │                          IMSI attach ack       │

│                                                │

│   IMSI unknown in VLR        Unidentified      │

│                               Subscriber       │

│     IMSI unknown in HLR      Unknown           │

│                               Subscriber       │

│     Network failure          Reject            │

│     Network failure          U-reject          │

│     Network failure          Abort             │

│     Network failure          L-cancel          │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)The acknowledgement does not contain any parameters.

2)These information elements are suppressed by the MSC.

3.2.5 Perform call control

Note:  This section applies to the mapping performed in MSC-B.

────────┬────────────────────────────────────────────────┬─────

│  04.08                    09.02                │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ Any call control or      Invoke: perform       │

message │ mobility management      call control          │  1

│ message                  BSSAPDU               │

────────┼────────────────────────────────────────────────┼─────

Positive│                                                │

result  │                                                │  2

────────┼────────────────────────────────────────────────┼─────

Negative│                          Return error:         │

result  │                                                │

│                          Handover state        │

│                          Undetermined          │  3

│                                                │

│                          Reject                │

│                          U-reject              │

│                          Abort                 │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	All fields received at the BSS-MSC interface are mapped into the BSSAPDU

2)	Return result does not apply. If MSC-A returns a message, this message will arrive in an Invoke: process call control information.

3)	The negative results are not sent on the radio path.

3.2.6 Authentication

The authenticate operation is shown in Figure 2.

MS                         MSC                         VLR

AUT REQ                         INVOKE (authenticate)

<-----------                    <----------------------

AUT RES

------------>

RESULT (auth ack)

----------------->

or

U-ERROR, U-REJECT

- - - - - - - - ->

Figure 2. Authentication operation.

The MSC can only act on an INVOKE (authenticate) if a connection exists to the MS on the radio interface. If such connection does not exist, the MSC shall return a U-ERROR with cause "Absent subscriber. The same applies if the MS does not respond to an AUT REQ message.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ AUT REQ                  Invoke: authenticate  │

message │                                                │

│ RAND                     RAND                  │

│                                                │

│ Ciphering key seq        Ciphering key seq     │

│ number                   number                │

────────┼────────────────────────────────────────────────┼─────

Backward│ AUT RES                  Result authenticate   │

result  │                          ack                   │

│                                                │

│ SRES                     SRES                  │

────────┴────────────────────────────────────────────────┴─────

3.2.7 Retrieval of the IMSI from the MS

The VLR may request open identification of an MS in an INVOKE (provide IMSI) message.

The mapping of information elements is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ ID REQ                   INVOKE: provide IMSI  │

message │                                                │

│     Identity type                              │

│     set to: IMSI                               │  1

────────┼────────────────────────────────────────────────┼─────

Backward│ ID RES                   Result: IMSI ack      │

result  │ IMSI                     Subscriber identity   │

────────┴────────────────────────────────────────────────┴─────

Note:  1)	The INVOKE does not carry any parameters. The identity type is inferred from the invoke name.

The MSC shall return a U-ERROR with cause absent subscriber if

-	there is no radio connection the MS when the invoke is received

-	there is no response from the MS.

3.2.8 Reallocation of TMSI

This operation is invoked by the VLR. The INVOKE (forward new TMSI) contains the new TMSI which is forwarded to the MS in the TMSI REAL CMD. When the MS acknowledges the receipt of the new TMSI, the MSC will return a RESULT message to the VLR.

If there is no radio connection to the MS when receiving the INVOKE, the MSC shall ignore the message.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ TMSI REAL CMD         INVOKE: Forward          │

message │                       new TMSI                 │

│                                                │

│ Mobile identity       TMSI                     │

│                                                │

│ Location area                                  │

│ identification               -                 │

────────┼────────────────────────────────────────────────┼─────

Backward│ TMSI REAL COM         Result: TMSI ack         │

result  │                                                │

│ Empty                 Empty                    │

────────┴────────────────────────────────────────────────┴─────

4. Non-transparent cases

4.1 General

For interworking other than mapping of information fields, see Recommendation GSM 09.09.

4.2 Outgoing call set-up (MS originating call)

Figure 3 shows those elements of a call set-up sequence which will require interworking with the mobile application part. Radio interface messages not requiring interworking with MAP are not shown.

MS                         MSC                         VLR

CM SERV REQ

-------------->

INVOKE (process access request)

---------------------------->

┌────────────────────────┐

│Possibly identification │

│procedure/authentication│

│procedure               │

└────────────────────────┘

CM SERV ACC

(Note 6)                    INVOKE (Start ciphering)

<------------                 <---------------------------

INVOKE (Forward new TMSI (Note 1)

<---------------------------

CM SREV REJ

(Note 3)                   RESULT (Access request accepted)

<------------                 <---------------------------

CIPH MOD CMD

<------------

CIPH MOD COM

------------->

SETUP

------------->

TMSI REAL CMD                 INVOKE (send info)

<-------------                 --------------------------->

(for O/G call set-up)

(Note 5)

TMSI REAL COM

-------------->

RESULT (TMSI ack)

------------------------------>

U-ERROR, (U-)REJECT

<------------------------------

U/P-ABORT, L-CANCEL            (Note 2)

REL COM

<-----------

(Note 4)                         INVOKE (complete call)

<------------------------------

CALL PROC

<-----------

Note 1:	These messages may be received simultaneously.

Note 2:	L-CANCEL is local in the MSC.

Note 3:	CM-SERVICE-REJECT is sent as a response to a negative result in the process access request message.

Note 4:	Release because of negative result.

Note 5:	No sequencing required with other messages. Must be sent after ciphering.

Note 6:	CM SERV ACC is only sent if the ciphering procedure is not invoked.

Figure 3  Part of outgoing call set-up sequence.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ CM-SERV-REQ             INVOKE: Process        │

│                         access request         │

│                                                │

│ CM Service type          CM Service type       │  1

│                                                │

│ Ciphering key            Ciphering key         │

│ sequence number          sequence number       │

│                                                │

│ Mobile identity          Subscriber Id         │

│                                                │

│ Mobile station                -                │

│                                                │

│ Classmark                                      │

│                                                │

│     -                    Access Connection     │

│                         Status                 │  3

────────┼────────────────────────────────────────────────┼─────

Positive│ CM-SERV-ACC              RESULT: Access        │

result  │                          request accepted      │  2

────────┼────────────────────────────────────────────────┼─────

Negative│ CM-SERV-REJ              Return error          │

result  │                                                │

│ IMSI unknown in VLR      Unidentified          │

│                          Subscriber            │

│ IMSI unknown in HLR      Unknown Subscriber    │

│ Network failure          Reject                │

│ Network failure          U-reject              │

│ Network failure          Abort                 │

│ Network failure          L-cancel              │

────────┼────────────────────────────────────────────────┼─────

│                                                │

│ AUT REJ                  Return error          │

│ Illegal MS               Illegal subscriber    │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	Indicates in this case an outgoing call.

2)	The CM-SERV-ACC is sent when the ciphering procedure is no invoked.

3)	Indicates whether or not an RR-connection exists and whether or not ciphering has been started.

The procedure is initiated by the MS sending a CM-SERVICE-REQUEST message. The MSC will forward the service request to the VLR in the process access request message. The VLR may then invoke other operations, e.g. authentication and identification. These operations are defined in para 3.2.6 and 3.2.7.

If the request is for a first MM-connection, the VLR indicates acceptance of the call request by returning the start ciphering message which will initiate the ciphering on the radio path. This interworking is described in para 4.4. If the request is for a parallel MM-connection, then the return result is used to inform the MS in the CM-service acceptance. After cipher mode setting has been completed or the CM SERV ACC message has been returned, the MS will send the SETUP message and information retrieval takes place as shown.

Information element mapping is required between the messages

-	CM-SERVICE-REQUEST to INVOKE (process access request)

-	SETUP to INVOKE (send information for O/G call set-up)

-	U-ERROR, (U-)REJECT, U/P-ABORT or L-CANCEL into REL COM or CM-SERVICE-REJECT.

The information contained in the INVOKE (complete call) is not passed on the radio interface but used in the MSC for connecting the call.

The conversion of information elements is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ SETUP                   Invoke: Sent info      │

message │                         for OG call set-up     │

│                                                │

│ Bearer capabilities           -                │

│ Mobile identity         Subscriber Id          │

│ Called part BCD number  Called Number          │

│ Low layer compatibility      -                 │

│ High layer compatibility      -                │

│                                                │

│ Facility                       -               │  1

│ User-user                       -              │

│                                                │

│ Calling party number BCD                       │

│ number                       -                 │

────────┼────────────────────────────────────────────────┼─────

Positive│                                                │

result  │                                                │  2

────────┼────────────────────────────────────────────────┼─────

Negative│ REL COM                  Return error          │

result  │                                                │

│ Rec GSM 04.10            CallBarred            │

│ Network out of order(38) DataMissing           │

│ Network out of order(38) UnexpectedDataValue   │

│ Network out of order(38) SystemFailure         │

│                                                │

│                          (Reject               │

│ Network out of order(38) (U-reject             │

│                          (Abort                │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	This information is passed if required in separate operations

2)	The call set-up parameters retrieved from the VLR are not sent to the MS. The parameters are contained in the complete call message.

3)	Service information may be included as Bearer Service code and/or teleservice code

4.3 Incoming call set-up (MS terminating call)

Figure 4 shows these elements of the procedure which require interworking between the mobile application part in Recommendation GSM 04.08.

MS                         MSC                              VLR

┌───────────────────────────┐

│Info retrieval             │

└───────────────────────────┘

INVOKE (Page MS)

<--------------------------

PAG REQ

<-----------------

(Note 2)

PAG RES                     INVOKE (Process access request)

------------------>           --------------------------------->

┌────────────────────────────┐

│Possibly authentication     │

│procedure                   │

└────────────────────────────┘

INVOKE (start ciphering)

<--------------------------------

RESULT (Access request accepted)

<--------------------------------

CIPH MODE CMD (Note2)        INVOKE (Forward new TMSI)

<--------------------        <--------------------------------

CIPH MOD COM                 INVOKE (complete call)

-------------------->        <--------------------------------

TMSI REAL CMD (Note3)

<--------------------

SETUP

<--------------------

TMSI REAL COM (Note3)        RESULT (TMSI code)

-------------------->        <--------------------------------

Notes: 1)	All four messages may be received simultaneously.

2)	Required only if an MM-connection does not exist already.

3)	No sequencing required with other messages. Must be sent after ciphering.

Figure 4  Incoming call set-up.

The paging procedure is controlled by the VLR. It may be followed by authentication (para 3.2.6), ciphering (para 4.4) and reallocation of TMSI(para 3.2.8). The SETUP message is sent when the complete call message is received.

Normally there is no interworking between the complete call message and the SETUP message. However, the complete call message may contain bearer service indication which will be used to establish the bearer capabilities at the MSC.

The interworking between Page MS and PAG REQ is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                    09.02                │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ PAG REQ                  INVOKE: (Page MS)     │

message │                                                │

│ Mobile identity          IMSI, TMSI            │  1

────────┼────────────────────────────────────────────────┼─────

│ PAG RES                  INVOKE(Process        │

│                          access request        │

│                                                │

│ Ciphering key            Ciphering key         │

│ sequence number          sequence number       │

│                                                │

│ Mobile identity               -                │

│                                                │

│ Mobile station                                 │

│ class mark                    -                │

────────┴────────────────────────────────────────────────┴─────

Note 1)	If TMSI is included, the TMSI is then also used as mobile identity in PAG REQ. Otherwise the IMSI is used.

4.4 Cipher mode setting

The interworking is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                  09.02                  │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ CIPH MOD CMD           INVOKE (start ciphering)│

│                                                │

│ Cipher mode setting     Cipher mode setting    │

│                                                │

│     -                   Kc                     │  1

────────┼────────────────────────────────────────────────┼─────

Backward│ CIPH MOD COM            None                   │

message │                                                │

────────┴────────────────────────────────────────────────┴─────

Note  1) The key Kc is passed to the BSS.

4.5 Handover

The general principles of operation and interworking are given in Recommendation GSM 03.09. This Recommendation together with Recommendations GSM 08.08, GSM 09.09 and 09.02 give the necessary information for interworking. No direct mapping of information elements is required.

.Begin Table C.

1. Scope	2

2. Classification of interworking cases	2

2.1 Transparent operations	2

2.2 Non-transparent operations	3

3. Transparent case	3

3.1 Interworking in the BSS	3

3.2 Interworking in the MSC	3

3.2.1 General	3

3.2.2 Location area updating	6

3.2.3 Detach IMSI	7

3.2.4 Attach IMSI	7

3.2.5 Perform call control	8

3.2.6 Authentication	8

3.2.7 Retrieval of the IMSI from the MS	9

3.2.8 Reallocation of TMSI	10

4. Non-transparent cases	10

4.1 General	10

4.2 Outgoing call set-up (MS originating call)	10

4.3 Incoming call set-up (MS terminating call)	15

4.4 Cipher mode setting	16

4.5 Handover	17

.End Table C.