European digital cellular telecommunications system (Phase 2);

Information element mapping between

Mobile Station - Base Station System and

BSS - Mobile-services Switching Centre (MS - BSS - MSC)

Signalling procedures and the Mobile Application Part (MAP)

(GSM 09.10)

European digital cellular telecommunication system (Phase 2);

Information element mapping between Mobile Station - Base

Station System and BSS - Mobile-services Switching Centre

(MS - BSS - MSC)  Signalling procedures and the Mobile

Application Part (MAP)

(GSM 09.10)

Contents

.Begin Table C.

Foreword	5

1	Introduction	7

1.1	Scope	7

1.2	Normative references	7

1.3	Definitons and abbreviations	8

2	Classification of interworking cases	8

2.1	Transparent procedures	8

2.2	Non-transparent procedures	9

3	Interworking in the MSC, Transparent case	9

3.1	General	9

3.2	Location area updating	11

3.3	Detach IMSI	12

3.4	Authentication	12

3.5	Retrieval of the IMSI from the MS	13

3.6	Reallocation of TMSI	14

3.7	Retrieval of the IMEI from the MS	14

3.8	Tracing subscriber activity	15

4	Non-transparent cases	15

4.1	General	15

4.2	Outgoing call set-up (MS originating call)	15

4.3	Incoming call set-up (MS terminating call)	21

4.4	Cipher mode setting	23

4.5	Inter-MSC Handover	24

4.5.1	Basic Inter-MSC Handover	24

4.5.2	Subsequent Inter-MSC Handover back to MSC-A	30

4.5.3	Subsequent Inter-MSC Handover to third MSC	36

4.5.4	BSSAP Messages transfer on E-Interface	41

4.5.5	Processing in MSC-B, and information transfer on E-interface	42

4.5.5.1	Encryption Information	43

4.5.5.2	Channel Type	44

4.5.5.3	Classmark	44

4.5.5.4	Downlink DTX-Flag	45

4.5.5.5	Priority	45

4.5.5.6	MSC/BSC-Invoke Trace Information Elements	45

4.5.6	Overview of the Technical Specifications GSM interworking for the Inter-MSC Handover	46

History	47

.End Table C.

Blank page

Foreword

This space is reserved for the foreword of the ETS version of this document.

This draft European Telecommunication Standard (ETS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI) and is now submitted for the Public Enquiry (PE) phase of the ETSI standards approval procedure.

This draft ETS describes information element mapping between Mobile Station - Base Station System and BSS - Mobile-services Switching Centre (MS - BSS - MSC), signalling procedures and the Mobile Application Part (MAP), within the European digital cellular telecommunications system (Phase 2).

This draft ETS corresponds to GSM technical specification GSM 09.10 version 4.1.0.

The specification from which this draft ETS has been derived was originally based on CEPT documentation, hence the presentation of this draft ETS may not be entirely in accordance with the ETSI/PNE rules. As a consequence, this draft ETS may undergo further editorial changes during the PE phase.

NOTE:	TC-SMG has produced documents which give the technical specifications for the implementation of the European digital cellular telecommunications system. Historically, these documents have been identified as  GSM Technical Specifications (GSM-TSs). These TSs may have subsequently become I-ETSs (Phase 1), or ETSs (Phase 2), whilst others may become ETSI Technical Reports (ETRs). GSM-TSs are, for editorial reasons, still referred to in current GSM ETSs.

Blank page

1	Introduction

1.1	Scope

The scope of this Technical Specification is

i)	to provide a detailed specification for the interworking between information elements contained in layer 3 messages sent on the MS-MSC interface (Call Control and Mobility Management parts of TS GSM 04.08) and parameters contained in MAP services sent over the MSC-VLR interface (TS GSM 09.02) where the MSC acts as a transparent relay of information;

ii)	to provide a detailed specification for the interworking between information elements contained in BSSMAP messages sent on the BSC-MSC interface (TS GSM 08.08) and parameters contained in MAP services sent over the MSC-VLR interface (TS GSM 09.02) where the MSC acts as a transparent relay of information;

iii)	to provide a detailed specification for the interworking as in i) and ii) above when the MSC also processes the information.

Interworking for supplementary services is given in TS GSM 09.11. Interworking for the short message service is given in TSs GSM 03.40 and GSM 04.11. Interworking between the call control signalling of TS GSM 04.08 and the PSTN/ISDN is given in TSs GSM 09.03, GSM 09.07 and GSM 09.08. Interworking between the 'A' and 'E' interfaces for inter-MSC handover signalling is given in TSs GSM 09.07 and 09.08.

1.2	Normative references

This ETS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this ETS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 100): "European digital cellular telecommunication system (Phase 2); Abbreviations and acronyms".

[2]	GSM 03.09 (prETS 300 527): "European digital cellular telecommunication system (Phase 2); Handover procedures".

[3]	GSM 03.40 (prETS 300 536): "European digital cellular telecommunication system (Phase 2); Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[4]	GSM 04.08 (prETS 300 557): "European digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3 specification".

[5]	GSM 04.10 (prETS 300 558): "European digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3  Supplementary services specification  General aspects".

[6]	GSM 04.11 (prETS 300 559): "European digital cellular telecommunication system (Phase 2); Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[7]	GSM 08.08 (prETS 300 590): "European digital cellular telecommunication system (Phase 2); Mobile Switching Centre - Base Station System (MSC - BSS) interface  Layer 3 specification".

[8]	GSM 09.02 (prETS 300 599): "European digital cellular telecommunication system (Phase 2); Mobile Application Part (MAP) specification".

[9]	GSM 09.03 (prETS 300 600): "European digital cellular telecommunication system (Phase 2); Signalling requirements on interworking between the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) and the Public Land Mobile Network (PLMN)".

[10]	GSM 09.07 (prETS 300 604): "European digital cellular telecommunication system (Phase 2); General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)".

[11]	GSM 09.08 (prETS 300 626): "European digital cellular telecommunication system (Phase 2); Application of the Base Station System Application Part (BSSAP) on the E-interface"

[12]	GSM 09.11 (prETS 300 606): "European digital cellular telecommunication system (Phase 2); Signalling interworking for supplementary services".

1.3	Definitons and abbreviations

Abbreviations used in this specification are listed in GSM 01.04.

2	Classification of interworking cases

2.1	Transparent procedures

The following MSC procedures require transparent mapping of BSSAP information elements into MAP parameters and vice versa (see  TS GSM 09.02 for definitions and the use of the procedures):

-	update location area

-	detach IMSI

-	forward new TMSI

-	provide IMSI

-	obtain IMEI

-	check IMEI

-	authenticate

-	trace subscriber activity

2.2	Non-transparent procedures

Procedures in this class require processing in the MSC and information element mapping. These procedures include those related to:

-	outgoing call set-up

-	incoming call set-up

-	handover

-	cipher mode setting

3	Interworking in the MSC, Transparent case

3.1	General

When the MSC receives a forward message from the BSS (possibly forwarded transparently from the MS), it will invoke the desired MAP service and establish a cross reference between the BSSAP procedure and the MAP procedure in order to return the result of the operation to the BSS (which may forward it transparently to the MS. The cross reference is deleted when the MSC terminates the MAP procedure.

Positive or negative results of the MAP procedure are returned in the appropriate BSSAP message.

The parameters of the forward BSSAP message are mapped by a one-to-one mapping into the parameters of the MAP service. However, in some cases parameters received on the radio path may be suppressed at the MSC because they are related to another protocol entity, e.g. information related to RR-management may be included in MM-management messages. Similarly, parameters received in the (positive) MAP service response are mapped one-to-one into parameters of the corresponding backward BSSAP message.

A negative outcome, as carried in various MAP services (MAP specific service response, MAP_U_ABORT, MAP_P_ABORT, MAP_NOTICE and premature MAP_CLOSE, see TS GSM 09.02 for definitions) is mapped into a cause value in the required backward BSSAP message. In this case several negative results of MAP may be mapped into the same BSSAP cause value, i.e. without discrimination between these negative results.

Note: 	For O & M purposes, the MAP procedure entity in the MSC may require a more detailed overview of negative results than the MS.

These principles are illustrated in Figure 1.

04.08 (08.08)                                MAP service

forward message                                request

────────────>                                ───────────>

┌───────────┐                                ┌─────────┐

│information│                                │parameter│

│  element  │                                │         │

└─────┬─────┘           one-to-one           └────┬────┘

└─────>────────────────────────────────>────┘

mapping

MAP service

positive ack                                  response

<────────────                               <───────────

┌───────────┐                                ┌─────────┐

│information│                                │parameter│

│  element  │                                │         │

└─────┬─────┘           one-to-one           └────┬────┘

└─────<────────────────────────────────<────┘

mapping

negative

negative cause                                response

<────────────                               <───────────

┌───────────┐                                ┌─────────┐

│   cause   │                                │  cause  │

└─────┬─────┘    one-to-one or many-to-one   └────┬────┘

└─────<────────────────────────────────<────┘

mapping

Figure 1: Illustration of mapping principles in the MSC

For each of the transparent operations listed in section 2.1, the following format is used to show the mapping.

────────┬─────────────────────────────────────────────────┬─────

│ 04.08 or 08.08                  09.02           │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ MS/BSS to MSC                 MSC to VLR        │

message │   message name              MAP service request │

│ information element 1  <--->  parameter 1       │

│ information element 2  <--->  parameter 2       │

────────┼─────────────────────────────────────────────────┼─────

Positive│ MSC to MS/BSS                  VLR to MSC       │

result  │   message name               positive response  │

│ information element 1  <--->  parameter 1       │

│ information element 2  <--->  parameter 2       │

────────┼─────────────────────────────────────────────────┼─────

Negative│ MSC to MS/BSS                  VLR to MSC       │

result  │   message name               negative response  │

│     cause 1            <--->     cause 1        │

│     cause 2            <--->     cause 2        │

│     cause 3            <--->  MAP_U/P_ABORT     │

│     cause 3            <--->   MAP_NOTICE       │

│     cause 3            <--->    MAP_CLOSE       │

────────┴─────────────────────────────────────────────────┴─────

Equivalent mapping principles apply for operations invoked by the VLR towards the BSS/MS. However, negative results are generally not received from the BSS/MS but are generated in the MSC. Therefore, for such operations the interworking for negative results is not normally shown.

3.2	Location area updating

────────┬────────────────────────────────────────────────┬─────

│   08.08/04.08                 09.02            │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ COMPLETE LAYER 3 INFO     MAP_UPDATE_LOCATION_ │

message │ (LOCATION UPDATING          AREA request       │

│    REQUEST)                                    │

│                                                │

│    Location area id         Previous LA Id     │

│    Mobile identity          IMSI or TMSI       │

│    Mobile station                              │

│       classmark 1                  -           │  4

│    Ciphering key            CKSN               │

│      seq number                                │

│    Location update          Location update    │

│      type                     type             │  3

│ Cell identifier             Target LA Id       │  1

│ Chosen channel                     -           │

────────┼────────────────────────────────────────────────┼─────

Positive│ DTAP (LOCATION            MAP_UPDATE_LOCATION  │

results │ UPDATING ACCEPT)            AREA response      │

│                                                │

│    Location area identity          -           │

│    Mobile identity                 -           │  5

│    Follow on proceed               -           │

────────┼────────────────────────────────────────────────┼─────

Negative│ DTAP (LOCATION            MAP_UPDATE_LOCATION  │

results │ UPDATING REJECT)            AREA response      │

│                                                │

│    IMSI unknown in HLR    Unknown subscriber   │

│    Network failure        Unknown LA           │  2

│                           Roaming not allowed: │

│    PLMN not allowed          PLMN not allowed  │

│    LA not allowed            LA not allowed    │

│    National roaming not      National Roaming  │

│     allowed in this LA           not allowed   │

│    PLMN not allowed          Operator          │

│                              determined barring│

│    Illegal MS             Illegal subscriber   │

│    Illegal ME             Illegal equipment    │

│    Network failure        System Failure       │

│    Network failure        Unexpected data value│

│    Network failure        MAP_U/P_ABORT        │

│    Network failure        MAP_NOTICE           │

│    Network failure        MAP_CLOSE            │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	The Target LA Id parameter is derived by the MSC from the Cell identifier information element.

2)	The Unknown LA error is only generated as a result of incorrect information being inserted by the MSC or BSS.

3)	This parameter can be used by the VLR to decide whether (eg) Authentication or IMEI checking is needed.

4)	As the mobile station classmark (1 or 2) is received by the MSC at the establishment of every RR connection, this information need not be stored in the VLR, but it is stored in the MSC as long as the RR connection exists.

5)	The mobile identity is inserted by the MSC if it is received in a MAP_FORWARD_NEW_TMSI service. If a TMSI is included, the MS should respond with a TMSI REALLOCATION COMPLETE message.

3.3	Detach IMSI

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ IMSI DETACH              MAP_DETACH_IMSI       │

message │    INDICATION               request            │

│                                                │

│ Mobile identity             IMSI or TMSI       │

│                                                │

│ Mobile Station                                 │

│   classmark 1                 -                │

────────┼────────────────────────────────────────────────┼─────

Positive│                                                │

result  │                                                │  1

────────┼────────────────────────────────────────────────┼─────

Negative│                                                │

result  │                                                │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	The forward message is not acknowledged

Depending on the state of the MS, the IMSI DETACH INDICATION may be carried in either a DTAP message or a BSSMAP COMPLETE LAYER 3 INFORMATION message.

3.4	Authentication

The message flow for the authentication procedure is shown in Figure 2.

MS                        MSC                                VLR

MAP_AUTHENTICATE request

AUTHENTICATION REQUEST    <----------------------------------

<-----------------------

AUTHENTICATION RESPONSE

----------------------->        MAP_AUTHENTICATE response

---------------------------------->

or

MAP_U/P_ABORT

---------------------------------->

Figure 2. Authentication operation.

The MSC can only act on a MAP_AUTHENTICATE request if an RR connection exists with the MS. If such a connection does not exist, the MSC shall terminate the MAP procedure with a MAP_U_ABORT. The same applies if the MS does not respond to an AUTHENTICATION REQUEST message.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ AUTHENTICATION REQUEST   MAP_AUTHENTICATE      │

message │                             request            │

│                                                │

│ RAND                     RAND                  │

│                                                │

│ Ciphering key seq        CKSN                  │

│ number                                         │

────────┼────────────────────────────────────────────────┼─────

Backward│ AUTHENTICATION REQUEST   MAP_AUTHENTICATE      │

result  │                             response           │

│                                                │

│ SRES                     SRES                  │

────────┴────────────────────────────────────────────────┴─────

If the SRES parameter does not match the value stored in the VLR, then the ongoing MAP procedure shall be terminated with a cause 'illegal subscriber'. This shall cause the MSC to send an AUTHENTICATION REJECT message.

3.5	Retrieval of the IMSI from the MS

The VLR may request open identification of an MS with a MAP_PROVIDE_IMSI request.

The mapping of information elements is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ IDENTITY REQUEST         MAP_PROVIDE_IMSI      │

message │                             request            │

│     Identity type                              │

│     set to: IMSI                               │  1

────────┼────────────────────────────────────────────────┼─────

Backward│ IDENTITY RESPONSE        MAP_PROVIDE_IMSI      │

result  │ Mobile Identity (IMSI)      response           │

────────┴────────────────────────────────────────────────┴─────

Note:  1)	The INVOKE does not carry any parameters. The identity type is inferred from the invoke name.

The MSC shall return a MAP_PROVIDE_IMSI response with user error "absent subscriber" if

-	there is no RR connection with the MS when the MAP service request is received

-	there is no response from the MS.

3.6	Reallocation of TMSI

This operation is invoked by the VLR. The MAP_FORWARD_NEW_TMSI request contains the new TMSI which is forwarded to the MS in the TMSI REALLOCATION COMMAND. When the MS acknowledges the receipt of the new TMSI, the MSC will return a MAP_FORWARD_NEW_TMSI response to the VLR.

If there is no radio connection to the MS when the MSC receives the MAP service request, the MSC shall ignore the message.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ TMSI REALLOCATION     MAP_FORWARD_NEW_TMSI     │

message │    COMMAND               request               │

│                                                │

│ Mobile identity       TMSI                     │

│                                                │

│ Location area                                  │

│ identification               -                 │

────────┼────────────────────────────────────────────────┼─────

Backward│ TMSI REALLOCATION     MAP_FORWARD_NEW_TMSI     │

result  │  COMPLETE                response              │

────────┴────────────────────────────────────────────────┴─────

3.7	Retrieval of the IMEI from the MS

The VLR may use the MAP_OBTAIN_IMEI service to request the MS to supply its IMEI , or may use the MAP_CHECK_IMEI service to request the MSC to check the MS's IMEI. For either MAP service the BSSAP signalling is the same.

The mapping of information elements is as follows:

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │                      (MAP_CHECK_IMEI request   │

message │ IDENTITY REQUEST     (   or                    │

│                      (MAP_OBTAIN_IMEI request  │

│    Identity type                               │

│    set to: IMEI                                │  1

────────┼────────────────────────────────────────────────┼─────

Backward│                      (MAP_CHECK_IMEI response  │

result  │ IDENTITY RESPONSE    (   or                    │

│                      (MAP_OBTAIN_IMEI response │

│                                                │

│ Mobile Identity          IMEI                  │  2

│    (IMEI)                                      │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	The MAP service request does not carry any parameters. The identity type is inferred from the service name.

2)	If the MAP_CHECK_IMEI service was used, the MSC also returns the equipment status to the VLR in the MAP_CHECK_IMEI response, after a successful dialogue with the EIR using the IMEI received from the MS.

The MSC shall terminate the MAP dialogue with the VLR using a MAP_U_ABORT if

-	there is no RR connection with the MS when the MAP service request is received

-	there is no response from the MS.

Note:	The MSC can also obtain the IMEI from a phase 2 MS by including appropriate information in the BSSMAP Cipher Mode Command.

3.8	Tracing subscriber activity

The VLR may request the MSC and/or BSS to record data about the current transaction with an MS.

────────┬────────────────────────────────────────────────┬─────

│  08.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ MSC INVOKE TRACE      MAP_TRACE_SUBSCRIBER_    │

message │                          ACTIVITY request      │

│                                                │

│ Trace type               Trace type            │

│ TriggerId                    -                 │

│ Trace reference          Trace reference       │

│ TransactionId                -                 │

│ Mobile identity(IMSI)    IMSI                  │  1

│ Mobile identity(IMEI)    IMEI                  │  1

│ OMCId                    OMCId                 │

────────┼────────────────────────────────────────────────┼─────

Backward│ none                  none                     │

result  │                                                │

────────┴────────────────────────────────────────────────┴─────

Note:  1)	The VLR may provide either an IMSI or IMEI, but not both.

4	Non-transparent cases

4.1	General

For interworking other than the mapping of information fields, see TS GSM 09.08.

4.2	Outgoing call set-up (MS originating call)

Figure 3 shows those elements of a call set-up sequence which require interworking between BSSAP and MAP. BSSAP messages which do not require interworking with MAP are not shown.

MS                        MSC                                VLR

CM SERVICE REQUEST

----------------------->   MAP_PROCESS_ACCESS_REQUEST request

---------------------------------->

┌──────────────────────────┐

│         Possibly         │

│ identification procedure/│

│ authentication procedure │

└──────────────────────────┘

MAP_SET_CIPHERING_MODE request

<----------------------------------

(Note 1)

MAP_PROCESS_ACCESS_REQUEST response

CM SERVICE ACCEPT      <---------------------------------

<-----------------------         positive result (Note 1)

(Note 2)

CIPHER MODE COMMAND

<-----------------------

(Note 2)

CIPHER MODE COMPLETE

----------------------->

MAP_FORWARD_NEW_TMSI request

TMSI REALLOCATION COMMAND  <----------------------------------

<-----------------------                 (Note 3)

TMSI REALLOCATION COMPLETE

----------------------->      MAP_FORWARD_NEW_TMSI response

---------------------------------->

MAP_PROCESS_ACCESS_REQUEST response

CM SERVICE REJECT      <---------------------------------

<-----------------------     negative result, MAP_U/P_ABORT,

(Note 4)                  MAP_NOTICE, MAP_CLOSE

SETUP (Note 5)

----------------------->     MAP_SEND_INFO_FOR_OUTGOING_CALL

---------------------------------->

request

MAP_COMPLETE_CALL request

CALL PROCEEDING       <----------------------------------

<-----------------------

MAP_SEND_INFO_FOR_OUTGOING_CALL

RELEASE COMPLETE       <----------------------------------

<-----------------------        response, MAP_U/P_ABORT,

(Note 6)                 MAP_NOTICE, MAP_CLOSE

Figure 3  Part of outgoing call set-up sequence.

Notes: 1)	If the MSC received a MAP_SET_CIPHERING_MODE request, it stores it until it receives the MAP_PROCESS_ACCESS_ REQUEST response.

2)	CM SERVICE ACCEPT is sent only if the ciphering procedure is not invoked.

3)	TMSI reallocation need not be sequenced with other messages, but should be sent after ciphering.

4)	CM SERVICE REJECT is sent as a result of a user error parameter in the MAP_PROCESS_ACCESS_REQUEST response, or termination of the MAP dialogue.

5)	The SETUP message is sent after the MS has either received a CM SERVICE ACCEPT or sent a CIPHER MODE COMPLETE.

6)	RELEASE COMPLETE is sent as a result of a user error parameter in the MAP_SEND_INFO_FOR_OUTGOING_CALL response, or termination of the MAP dialogue.

The procedure can be considered in two parts: the handling of the CM SERVICE REQUEST and the handling of the SETUP request.

The procedure is initiated by the MS sending a CM SERVICE REQUEST message. The MSC will forward the service request to the VLR in the MAP_PROCESS_ACCESS_REQUEST request. The VLR may then invoke other operations, e.g. authentication and identification. These operations are defined in sections 3.4 and 3.5.

If there is a positive outcome for the CM SERVICE REQUEST procedure, the VLR always sends a MAP_PROCESS_ACCESS_REQUEST response. If the request is for a first MM-connection and ciphering is required, the MAP_PROCESS_ACCESS_REQUEST response is preceded by a MAP_SET_CIPHERING_MODE request. In this case the MSC sends a CIPHER MODE COMMAND towards the MS. The interworking for cipher mode setting is described in section 4.4. If the request is for an additional MM-connection or for a first MM-connection where ciphering is not required, then the positive MAP_PROCESS_ACCESS_ REQUEST response causes the MSC to send a CM SERVICE ACCEPT message to the MS. After cipher mode setting has been completed or the CM SERVICE ACCEPT message has been returned, the MS will send the SETUP (or EMERGENCY SETUP) message and information retrieval takes place as shown.

A negative outcome for the MAP_PROCESS_ACCESS_REQUEST procedure can be signalled by a MAP_PROCESS_ACCESS_REQUEST response containing a user error parameter, or by terminating the MAP dialogue between the MSC and the VLR.

A positive outcome for the call setup procedure is indicated by a MAP_COMPLETE_CALL request from the VLR to the MSC, which causes the MSC to send a CALL PROCEEDING message towards the MS.

A negative outcome for the call setup procedure can be signalled by a MAP_SEND_INFO_FOR_INCOMING_CALL response or by terminating the dialogue between the MSC and the VLR.

Information element mapping is required between the messages

-	CM SERVICE REQUEST to MAP_PROCESS_ACCESS_REQUEST request

-	SETUP to MAP_SEND_INFO_FOR_OUTGOING CALL request

-	MAP_SEND_INFO_FOR_OUTGOING_CALL response, MAP_U/P_ABORT, MAP_NOTICE or premature MAP_CLOSE to RELEASE COMPLETE or CM SERVICE REJECT.

The information contained in the MAP_COMPLETE_CALL request is not transmitted on the radio interface but is used in the MSC for connecting the call.

The conversion of information elements is as follows:

────────┬────────────────────────────────────────────────┬─────

│  08.08/04.08             09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ COMPLETE LAYER 3 INFO   MAP_PROCESS_ACCESS_    │

│ (CM SERVICE REQUEST)       REQUEST request     │

│                                                │

│    CM Service type      CM Service type        │  1

│    Ciphering key        CKSN                   │

│       sequence number                          │

│    Mobile identity      TMSI or IMSI or IMEI   │

│    Mobile station                              │

│       Classmark 2             -                │

│                                                │

│ Cell identifier         Current LA Id          │  4

│ Chosen channel                -                │

│         -               Access Connection      │

│                            Status              │  3

────────┼────────────────────────────────────────────────┼─────

Positive│ DTAP(CM SERVICE ACCEPT) MAP_PROCESS_ACCESS_    │

result  │                            REQUEST response    │  2

────────┼────────────────────────────────────────────────┼─────

Negative│ DTAP(CM SERVICE REJECT) MAP_PROCESS_ACCESS_    │

result  │                            REQUEST response    │

│                                                │

│    IMSI unknown in VLR     Unidentified        │

│                               Subscriber       │

[       │    Requested service     ???????               │

│       option not                               │

│       subscribed                               │]

│    Illegal ME              Illegal equipment   │

│    Network failure         System failure      │

│    Network failure      MAP_U/P_ABORT          │

│    Network failure      MAP_NOTICE             │

│    Network failure      MAP_CLOSE              │

────────┼────────────────────────────────────────────────┼─────

│ DTAP(AUTHENTICATION     MAP_PROCESS_ACCESS_    │

│    REJECT)                 REQUEST response    │

│                                                │

│                            Illegal subscriber  │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	Indicates, in this case, a mobile originating call establishment or an emergency call establishment.

2)	The CM SERVICE ACCEPT is sent when the ciphering procedure is not invoked.

3)	Indicates whether or not an RR-connection exists and whether or not ciphering has been started.

4)	The Current LA Id parameter is derived by the MSC from the Cell identifier information element.

────────┬────────────────────────────────────────────────┬─────

│  04.08                   09.02                 │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ SETUP                   MAP_SEND_INFO_FOR_     │

message │                         OUTGOING_CALL request  │

│                                                │

│ BC repeat indicator              -             │

│ Bearer capability 1              -             │  3

│ Bearer capability 2              -             │  3

│ Calling party subaddress         -             │

│ Called party BCD number     Called Number      │

│ Called party subaddress          -             │

│ LLC repeat indicator             -             │

│ Low layer compatibility I        -             │

│ Low layer compatibility II       -             │

│ HLC repeat indicator             -             │

│ High layer compatibility i       -             │

│ High layer compatibility ii      -             │

│      -                      Bearer service     │  3

│      -                      Teleservice        │  3

│ Facility                         -             │  1

│      -                      CUG index          │  4

│      -                      Suppress pref CUG  │  4

│      -                      Suppress CUG OA    │  4

│ User-user                        -             │

│ SS version                       -             │

│ CLIRO flag                       -             │

────────┼────────────────────────────────────────────────┼─────

Positive│                                                │

result  │                                                │  2

────────┼────────────────────────────────────────────────┼─────

Negative│ RELEASE COMPLETE         MAP_SEND_INFO_FOR_    │

result  │                          OUTGOING_CALL response│

│                                                │

│ TS GSM 04.10             Call Barred           │

│                             Barring Service    │

│                                Active          │

│ Operator determined      Call Barred           │

│    barring                  Operator Determined│

│                                Barring         │

│ Network out of order     Data Missing          │

│ Network out of order     Unexpected Data Value │

│ Network out of order     System Failure        │

│ Bearer capability        Bearer service not    │

│    not authorized           provisioned        │

│ Bearer capability        Teleservice not       │

│    not authorized           provisioned        │

│ [User not member of CUG] CUG reject            │

│                                                │

│ Network out of order     MAP_U/P_ABORT         │

│ Network out of order     MAP_NOTICE            │

│ Network out of order     MAP_CLOSE             │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	If the Facility IE contains CUG information, the CUG information is transferred to the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service; any other information contained in a Facility IE is transferred to the VLR in a MAP Supplementary Services related service.

2)	The call setup parameters retrieved from the VLR are not sent to the MS. The parameters are carried in the MAP_COMPLETE_CALL service.

3)	The bearer capabilities can be used to derive the bearer/tele service.

4)	CUG information is derived from the contents of the Facility IE.

4.3	Incoming call set-up (MS terminating call)

Figure 4 shows those elements of the procedure which require interworking between MAP and TS GSM 04.08 procedures.

MS                        MSC                                VLR

┌──────────────────────────┐

│       Info retrieval     │

└──────────────────────────┘

MAP_PAGE request or

PAGE REQUEST         <----------------------------------

<-----------------------         MAP_SEARCH_FOR_MS request

(Note 1)

PAGING RESPONSE

----------------------->   MAP_SEARCH_FOR_MS response (Note 2)

---------------------------------->

MAP_PROCESS_ACCESS_REQUEST request

---------------------------------->

┌──────────────────────────┐

│         Possibly         │

│ authentication procedure │

└──────────────────────────┘

MAP_SET_CIPHERING_MODE request

<----------------------------------

(Note 3)

MAP_PROCESS_ACCESS_REQUEST response

CIPHER MODE COMMAND     <---------------------------------

<-----------------------         positive result (Note 4)

(Note 3)

CIPHER MODE COMPLETE

----------------------->

MAP_FORWARD_NEW_TMSI request

TMSI REALLOCATION COMMAND  <----------------------------------

<-----------------------                 (Note 5)

TMSI REALLOCATION COMPLETE

----------------------->      MAP_FORWARD_NEW_TMSI response

---------------------------------->

MAP_COMPLETE_CALL request

SETUP            <----------------------------------

<-----------------------

MAP_SEND_INFO_FOR_INCOMING_CALL

RELEASE COMPLETE       <---------------------------------

<-----------------------        response negative result,

(Note 6)           MAP_U/P_ABORT,MAP_NOTICE, MAP_CLOSE

Figure 4: Incoming call set-up.

Notes: 1)	If an MM connection already exists, the PAGE REQUEST is not sent. If the call can be accepted, the MSC sends a MAP_PROCESS_ACCESS_REQUEST request in response to the MAP_PAGE request. If the call cannot be accepted the MSC sends a MAP_PAGE response containing the error 'busy subscriber'.

2)	Sent only if MAP_SEARCH_FOR_MS was used.

3)	Needed only if a ciphered MM-connection does not exist already.

4)	If the MSC received a MAP_SET_CIPHERING_MODE request, it stores it until it receives the MAP_PROCESS_ACCESS_ REQUEST response.

5)	TMSI reallocation need not be sequenced with other messages, but should be sent after ciphering.

6)	RELEASE COMPLETE is sent as a result of a user error parameter in the MAP_SEND_INFO_FOR_OUTGOING_CALL response, or termination of the MAP dialogue.

The paging procedure is controlled by the VLR. It may be followed by authentication (section 3.4), ciphering (section 4.4) and reallocation of TMSI(section 3.6). The SETUP message is sent when the MAP_COMPLETE_CALL request is received.

Normally there is no interworking between the MAP_COMPLETE_CALL request and the SETUP message. However, the MAP_COMPLETE_CALL request may contain a bearer service indication which will be used to establish the bearer capabilities at the MSC.

The interworking between the MAP_PAGE request or MAP_SEARCH_FOR_MS request and the BSSMAP PAGING REQUEST message is as follows:

────────┬────────────────────────────────────────────────┬─────

│  08.08/04.08              09.02                │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ PAGING REQUEST       MAP_PAGE request or       │

message │                      MAP_SEARCH_FOR_MS request │

│                                                │

│ IMSI                    IMSI                   │

│ TMSI                    TMSI                   │  1

│ Cell identifier         Stored LA Id           │

│       list                                     │

────────┼────────────────────────────────────────────────┼─────

Backward│ COMPLETE LAYER 3 INFO    MAP_PROCESS_ACCESS_   │

message │ (PAGING RESPONSE)           REQUEST request    │

│                                                │

│          -                  CM service type    │  2

│    Ciphering key            CKSN               │

│       sequence number                          │

│    Mobile identity          TMSI or IMSI       │

│    Mobile station                              │

│       classmark 2                  -           │

│ Cell Identifier             Current LA Id      │  3

│          -                  Access connection  │

│                                status          │

│ Chosen channel                     -           │

────────┴────────────────────────────────────────────────┴─────

Notes: 1)	If TMSI is included, the TMSI is used as the mobile identity in the TS GSM 04.08 PAGE REQUEST message, otherwise the IMSI is used as the mobile identity.

2)	In this case the MAP CM service type is set to 'mobile terminating call'.

3)	The Target LA Id parameter is derived by the MSC from the Cell identifier information element.

4.4	Cipher mode setting

The interworking is as follows:

────────┬────────────────────────────────────────────────┬─────

│  08.08                  09.02                  │Notes

────────┼────────────────────────────────────────────────┼─────

Forward │ CIPHER MODE COMMAND    MAP_SET_CIPHERING_MODE  │

│                           request              │

│                                                │

│ Cipher mode setting       Ciphering mode       │

│ Encryption information    Kc                   │  1

────────┼────────────────────────────────────────────────┼─────

Positive│ CIPHER MODE COMPLETE   None                    │

result  │                                                │

────────┼────────────────────────────────────────────────┼─────

Negative│ CIPHER MODE REJECT     None                    │

result  │                                                │

────────┴────────────────────────────────────────────────┴─────

Note:  1)	The key Kc is passed through the BSS to the BTS, but is not passed to the MS.

4.5	Inter-MSC Handover

The general principles of the handover procedures are given in Technical Specification  GSM 03.09. TS GSM 09.10 gives the necessary information for interworking between the GSM 08.08 handover protocol and the GSM 09.02 MAP protocol.

4.5.1	Basic Inter-MSC Handover

When a Mobile Station is handed over between two MSCs, the establishment of a connection between them (described in TS GSM 03.09) requires interworking between A-Interface and E-Interface.

The signalling at initiation, execution, completion of the Basic Inter-MSC handover procedure is shown in figures 5 to 10 with both possible positive or negative outcomes.

BSS-A         MSC-A                     MSC-B

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| ┌────────────────┐

|               |request                  | │Possible Alloc. │

|               |                         | │of a handover   │

|               |                         | │nb. in the VLR-B│

|               |                         | └────────────────┘

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 5: Signalling for Basic Inter-MSC Handover initiation

Possible Positive outcomes

a)	successful radio resources allocation and handover number allocation (if performed):

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE HANDOVER    |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued and successful handover number allocation (if performed). Later successful radio resources allocation indication:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

HANDOVER COMMAND| SIGNALLING request      |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 6: Signalling for Basic Inter-MSC Handover execution (Positive outcomes)

Possible Negative outcomes

c)	user error detected, or handover number allocation unsuccessful (if performed), or component rejection or dialogue abortion performed by MSC-B:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |MAP PREPARE HANDOVER response                |

|               |negative result, MAP CLOSE                   |

|               |<------------------------|                   |

|               |MAP U/P-ABORT            |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

d)	radio resources allocation failure:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

|               | SIGNALLING request      |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

f)	unsuccessful handover execution (Reversion to the old channel):

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|                         |                   |

|FAILURE        |                         |                   |

|               |MAP U -ABORT             |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |                   |

Figure 7: Signalling for Basic Inter-MSC Handover execution (Negative outcomes)

Note 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER COMPLETE  |

|               |                         |<------------------|

|               |MAP SEND END SIGNAL request                  |

|               |<------------------------|                   |

|CLEAR COMMAND  |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 8: Signalling for Basic Inter-MSC Handover completion

Positive outcome

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |MAP SEND END SIGNAL      |                   |

|               |------------------------>|                   |

|               | response                |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         | (Note 1)          |

Figure 9: Signalling for Basic Inter-MSC Handover completion (Positive outcome)

Negative outcome

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               | MAP U/P -ABORT          |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |                   |

Figure 10: Signalling for Basic Inter-MSC Handover completion (Negative outcome)

Note 1:	From interworking between MAP and BSSMAP point of view.

The handover procedure is normally triggered by BSS-A by sending a HANDOVER REQUIRED message on A-Interface to MSC-A. The invocation of the Basic Inter-MSC handover procedure is performed and controlled by MSC-A. The sending of the MAP Prepare-Handover request to MSC-B is triggered in MSC-A upon receipt of the HANDOVER REQUIRED message. For compatibility reason, the cell identity of the cell where the call is to be handed over in MSC-B area, provided in the HANDOVER REQUIRED message, is mapped into targetCellId MAP parameter and the HANDOVER REQUEST message is encapsulated in the bss-APDU MAP parameter of the Prepare-Handover MAP request. MSC-B can invoke another operation towards the VLR-B (allocation of the handover number described in TS GSM 09.02).

The interworking between Prepare Handover and HANDOVER REQUIRED  is as follows:

────────┬─────────────────────────────────────────────────┬───

│   08.08                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ HANDOVER REQUIRED    MAP PREPARE HANDOVER request

message │                                                 │

│                               -ho-NumberNotRequired 1

│  BSSMAP information           -targetCellId     │

│         elements              -bss-APDU(        │   2

│                               HANDOVER REQUEST) │

────────┼─────────────────────────────────────────────────┼─────

Positive│                      MAP PREPARE HANDOVER response

result  │                                                 │   3

│                              -handover number   │

│                              -bss-APDU(         │

│                               QUEUING INDICATION│

│                               or HANDOVER REQUEST

│                               ACKNOWLEDGE or    │

│                               HANDOVER FAILURE) │

│                                                 │

────────┼─────────────────────────────────────────────────┼─────

Negative│ HANDOVER REQUIRED REJECT     MAP PREPARE HANDOVER   4

result  │                                                 │

│ equipment failure            System Failure     │

│ equipment failure            No Handover Number │

│                                 available       │

│ equipment failure            UnexpectedDataValue│

│ equipment failure            Data Missing       │

│                                                 │

│ equipment failure            MAP CLOSE          │

│ equipment failure            MAP U/P -ABORT     │

│                                                 │

Note 1:	The ho-NumberNotRequired parameter is included by MSC-A, when MSC-A decides not to use any circuit connection with MSC-B. No handover number shall be present in the positive result. Any negative response from MSC-B shall not be due to handover number allocation problem.

Note 2:	The process perfomed on the BSSMAP information elements received in the HANDOVER REQUIRED message is described in the GSM Recommendation 08.08.

Note 3:	The response to the Prepare-Handover request can include in its bss-APDU parameter, identifying the GSM 08.08 protocol, either a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, MSC-A shall wait for the radio resources allocation response from MSC-B, transmitted to MSC-A as described in chapter 4.5.4.

In the second case, the positive result triggers in MSC-A the sending on A-Interface of the HANDOVER COMMAND.

In the third case, the positive result triggers in MSC-A one of the following:

-	another handover attempt is initiated by MSC-A

-	optionaly the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of the TS GSM 09.10 and lies in the TS GSM 08.08).

Note 4:	The possible sending of the HANDOVER REQUIRED REJECT message is described in the TS GSM 08.08.

The interworking between Send End Signal and HANDOVER COMPLETE in MSC-B is as follows:

────────┬─────────────────────────────────────────────────┬─────

│   08.08                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ HANDOVER COMPLETE    MAP SEND END SIGNAL request│

message │                                                 │

│                               -bss-APDU(        │

│                               HANDOVER COMPLETE)│

│                                                 │

────────┼─────────────────────────────────────────────────┼─────

Positive│ CLEAR COMMAND        MAP SEND END SIGNAL response

result  │  -Call Control release                          │  1

────────┼─────────────────────────────────────────────────┼─────

Negative│ CLEAR COMMAND                                   │

result  │  -Call Control release        MAP CLOSE         │  2

│  -Call Control release        MAP U/P -ABORT    │

│                                                 │

Note 1:	The positive empty result triggers the clearing of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B. If a circuit connection is used between MSC-A and MSC-B, the 'Call Control release' clearing cause shall only be given to BSS-B when MSC-B has received a clearing indication on its circuit connection with MSC-A.

Note 2:	The abortion of the dialogue or the rejection of the component triggers in MSC-B the clearing of its circuit connection with MSC-A, if any, of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B.

The interworking between Send End Signal and CLEAR COMMAND in MSC-A is as follows:

────────┬─────────────────────────────────────────────────┬─────

│   09.02                          08.08          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ MAP SEND END SIGNAL          CLEAR COMMAND      │

message │ response                                        │

│          -bss-APDU(           - Handover        │

│          HANDOVER COMPLETE)        Successful   │

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                                                 │

The interworking between HANDOVER FAILURE in case of reversion to old channel of the MS and User Abort in MSC-A is as follows:

────────┬─────────────────────────────────────────────────┬─────

│   08.08                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ HANDOVER FAILURE                MAP U -ABORT    │

message │                                                 │

│  - Reversion to old                             │

│    channel                                      │

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                                                 │

4.5.2	Subsequent Inter-MSC Handover back to MSC-A

When a Mobile Station is being handed over back to MSC-A, the procedure (described in TS GSM 03.09) requires interworking between A-Interface and E-Interface.

The signalling at initiation, execution and completion of the Subsequent Inter-MSC handover procedure is shown in figures 11 to 15.

BSS-A         MSC-B                     MSC-A

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE SUBSEQUENT HANDOVER

|REQUIRED       |------------------------>|

|               | request                 |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 11: Signalling for Subsequent Inter-MSC Handover back to MSC-A initiation

Possible Positive outcomes

a)	successful radio resources allocation:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued. Later successful radio resources allocation indication:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |MAP FORWARD ACCESS SIGNALLING                |

|               |<------------------------|                   |

|               | request                 |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

Figure 12: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution (Positive outcome)

Possible Negative outcomes

c)	user error detected, or component rejection or dialogue abortion performed by MSC-A:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|HANDOVER REQUIRED  response negative result                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

d)	component rejection or dialogue abortion performed by MSC-A:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |MAP CLOSE, MAP U/P ABORT |                   |

|               |<------------------------|                   |

|CLEAR COMMAND  |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

e)	radio resources allocation failure:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|HANDOVER REQUIRED  response              |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

f)	radio resources allocation queued. Later unsuccessful radio resources allocation:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP FORWARD ACCESS       |                   |

|               |<------------------------|                   |

|HANDOVER REQUIRED   SIGNALLING request   |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

Figure 13: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution

(Negative outcome)

Note 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

BSS-B         MSC-A                     MSC-B              BSS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

Figure 14: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion

(Successful completion of the procedure)

Note 1:	Positive outcome case shown in figure 9.

BSS-B         MSC-A                     MSC-B              BSS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |                   |

|               |MAP U/P -ABORT           |                   |

|               |<------------------------|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |(Note 1)           |

Figure 15: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion (Unsuccessful completion of the procedure)

Note 1:	Abnormal end of the procedure which triggers the clearing of all resources in MSC-B.

The interworking between Prepare Subsequent Handover and HANDOVER REQUIRED is as follows:

────────┬─────────────────────────────────────────────────┬─────

│   08.08                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ HANDOVER REQUIRED    MAP PREPARE SUBSEQUENT HANDOVER

message │                               request           │  1

│                                                 │

│                               -target MSC number│

│  BSSMAP information           -targetCellId     │

│         elements              -bss-APDU(        │

│                               HANDOVER REQUEST) │

────────┼─────────────────────────────────────────────────┼─────

Positive│ HANDOVER REQUIRED    MAP PREPARE SUBSEQUENT HANDOVER

result  │                              response           │  2

│                              -bss-APDU(         │

│                               QUEUING INDICATION│

│                               or HANDOVER REQUEST

│                               ACKNOWLEDGE or    │

│                               HANDOVER FAILURE) │

────────┼─────────────────────────────────────────────────┼─────

Negative│ HANDOVER REQUIRED REJECT  MAP PREPARE SUBSEQUENT│  3

result  │                           HANDOVER response     │

│ equipment failure            System Failure     │

│ equipment failure            Unknown MSC        │

│ equipment failure            Subsequent Handover│

│                                         Failure │

│ equipment failure            UnexpectedDataValue│

│ equipment failure            Data Missing       │

│                                                 │

│ CLEAR COMMAND                                   │

│                                                 │

│ equipment failure            MAP CLOSE          │

│ equipment failure            MAP U/P -ABORT     │

│                                                 │

Note 1:	The processing performed on the BSSMAP information elements received in the HANDOVER REQUIRED message is out of the scope of this TS. The target MSC number is provided to MSC-A by MSC-B based on the information received from BSS-B.

Note 2:	The response to the Prepare-Subsequent-Handover request can include in its bss-APDU parameter, identifying the GSM 08.08 protocol, either a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, MSC-B shall wait for the radio resources allocation response from MSC-A, transmitted to MSC-B as described in chapter 4.5.4.

In the second case, the positive result triggers in MSC-B the sending on A-Interface of the HANDOVER COMMAND.

In the third case, the positive result triggers in MSC-B one of the following:

-	another handover attempt is initiated by  MSC-B

-	optionally the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of the TS GSM 09.10 and lies in the TS GSM 08.08).

Note 3:	The possible sending of the HANDOVER REQUIRED REJECT message is described in the TS GSM 08.08.

The interworking between Send End Signal Result and HANDOVER COMPLETE in MSC-A is as follows:

────────┬─────────────────────────────────────────────────┬─────

│   08.08                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ HANDOVER COMPLETE    MAP SEND END SIGNAL        │

message │                               response          │

│                                                 │

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                          MAP U/P -ABORT         │  1

│                                                 │

Note 1:	The abortion of the dialogue ends the handover procedure with MSC-B.

4.5.3	Subsequent Inter-MSC Handover to third MSC

When a Mobile Station is being handed over to a third MSC, the procedure (described in TS GSM 03.09) does require one specific interworking case in MSC-A (figure 20) between E-Interface from MSC-B and E-Interface from MSC-B' other than the combination of the ones described in the chapter 4.5.1 and 4.5.2.

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |

|----------->|MAP PREPARE SUSEQUENT |                |

|REQUIRED    |--------------------->|                |

|            |HANDOVER request      |MAP PREPARE HANDOVER

|            |                      |--------------->|

|            |                      |request         |

|            |                      |                |┌───────┐

|            |                      |                |│Possib.│

|            |                      |                |│Alloc. │

|            |                      |                |│of ho. │

|            |                      |                |│number │

|            |                      |                |│ VLR-B │

|            |                      |                |└───────┘

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |-------->|

|            |                      |                |REQUEST  |

|            |                      |                |

Figure 16: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') initiation.

Possible Positive outcomes

a)	successful radio resources allocation:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|   response     |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

|            |                      |                |         |

b)	radio resources allocation queued and successful handover number allocation, if performed. Later successful radio resources allocation indication:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|   response     |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |MAP FORWARD ACCESS    |SIGNALLING request        |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 17: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution

(Positive outcome)

Possible Negative outcomes

c)	user error detected, or component rejection or dialogue abortion performed by MSC-B':

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |response negative result  |

|            |                      |MAP CLOSE       |         |

|            |                      |<---------------|         |

|            |                      |MAP U/P -ABORT  |         |

|            |MAP PREPARE SUBSEQUENT|                |         |

|            |<---------------------|                |         |

|            |HANDOVER response negative             |         |

| HANDOVER   |result                |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

d)	radio resources allocation failure:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |                      |SIGNALLING request        |

|            |MAP FORWARD ACCESS    |                |         |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

Figure 18: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution (Negative outcome)

Note 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

Positive outcome

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |COMPLETE |

|            |                      |MAP SEND END SIGNAL       |

|            |                      |<---------------|         |

|            | MAP SEND END SIGNAL  |                |         |

|            |<---------------------|                |         |

|            | response             |                |         |

| CLEAR      |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 19: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Successful completion of the procedure)

Negative outcome

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |    BSS-B'

|----------->|                      |                |         |

|FAILURE     |MAP PROCESS ACCESS    |                |         |

|            |--------------------->|                |         |

|            |SIGNALLING request (Note 1)            |         |

|            |                      |MAP U -ABORT    |         |

|            |                      |--------------->|         |

|            |                      |                |CLEAR    |

|            |                      |                |-------->|

|            |                      |                |COMMAND  |

|            |                      |                |         |

Figure 20: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Unsuccessful completion of the procedure)

Note 1:	Specific interworking case detailed below.

The specific interworking case in MSC-A compared to the chapters 4.5.1 and 4.5.2 occurs between HANDOVER FAILURE encapsulated in a Process Access Signalling from MSC-B and the abortion of the dialogue with MSC-B' in the case of a reversion to old channel of the MS:

────────┬─────────────────────────────────────────────────┬─────

│   09.02                          09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ MAP PROCESS-SIGNALLING                          │

message │         request                                 │

│                                                 │

│         -bss-APDU(               MAP U -ABORT   │  1

│         HANDOVER FAILURE)                       │

│                                                 │

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                                  MAP U/P -ABORT │  2

│                                                 │

Note 1:	The abortion of the dialogue triggers in MSC-B' the clearing of the circuit connection with MSC-A, if any, and of the Resources between MSC-B' and BSS-B'.The abortion of the dialogue ends the handover procedure with MSC-B'.

Note 2:	The abortion of the dialogue ends the handover procedure with MSC-B.

4.5.4	BSSAP Messages transfer on E-Interface

The following mapping applies to the encapsulation performed in MSC-A.

────────┬─────────────────────────────────────────────────┬─────

│   04.08/08.08                    09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ BSSAP messages       MAP FORWARD ACCESS SIGNALLING

message │                               request           │  1

│                                                 │

│                       -bss-APDU (BSSAP messages)│

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │  2

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                               MAP CLOSE         │

│                               MAP U/P -ABORT    │

│                                                 │

Note 1:	Complete BSSAP messages to be sent on MSC-B - BSS-B interface (BSSMAP or DTAP messages) are embedded into the bss-APDU parameter (see Annex A of TS GSM 08.08 for the description of the set of BSSMAP messages).

Note 2:	The Return Result does not apply. If MSC-B returns a message, this message will arrive in an Invoke: Process Access Signalling.

The following mapping applies to the encapsulation performed in MSC-B.

────────┬─────────────────────────────────────────────────┬─────

│   04.08/08.08                    09.02          │Notes

────────┼─────────────────────────────────────────────────┼─────

Forward │ BSSAP messages       MAP PROCESS ACCESS SIGNALLING

message │                                request          │ 1

│                                                 │

│                       -bss-APDU (BSSAP messages)│

────────┼─────────────────────────────────────────────────┼─────

Positive│                                                 │

result  │                                                 │  2

────────┼─────────────────────────────────────────────────┼─────

Negative│                                                 │

result  │                               MAP CLOSE         │

│ CLEAR COMMAND                                   │

│                                                 │

│ equipment failure             MAP U/P -ABORT    │  3

│                                                 │

Note 1:	Complete BSSAP messages to be sent to MSC-A (BSSMAP or DTAP messages) are embedded into the bss-APDU parameter (see TS GSM 09.08 for the description of the set of BSSMAP messages).

Note 2:	The Return Result does not apply. If MSC-A returns a message, this message will arrive in an Invoke: Forward Access Signalling.

Note 3:	The abortion of the dialogue triggers the clearing of the circuit connection with MSC-A, if any, of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B. The clearing of the Radio Resources (the clearing indication received from BSS-B is transmitted to MSC-A) or the loss of the SCCP connection between MSC-B and BSS-B, triggers in MSC-B the abortion of the dialogue on the E-Interface and the clearing of the circuit connection with MSC-A, if any.

4.5.5	Processing in MSC-B, and information transfer on E-interface

The following parameters require processing (e.g. to store the parameter, to internally generate the parameter) in MSC-B. The relevant BSSMAP procedures are mentioned to ease the comprehension, their detailed description is the scope of the TS GSM 08.08. Each BSSMAP message listed in TS GSM 09.08 being transferred on E-interface shall use the mechanisms given in chapter 4.5.4 and is described in TS GSM 08.08.

4.5.5.1	Encryption Information

A sequence of possible encryption algorithms can be sent to a BSS in Cipher Mode Command or Handover Request. The BSS chooses one of the listed algorithms and reports this back to the MSC in Cipher Mode Complete or Handover Request Acknowledge respectively.

The list of algorithms, the ciphering key and the chosen algorithm shall be stored by MSC-B, and the chosen value sent to MSC-A.

Transfer of Information

If ciphering has not been performed before Inter-MSC Handover, this will be controlled by MSC-A after the completion of Inter-MSC Handover.

Ciphering control towards MSC-B:

If Ciphering has been performed before Inter-MSC Handover:

-	in the  Handover Request BSSMAP message (information included)

The Handover Request Acknowledge should in this case contain the indication of the chosen algorithm.

If Ciphering has NOT been performed before Inter-MSC Handover:

-	in the Cipher Mode Command procedure between MSC-A and MSC-B.

If the encryption algorithm is changed at an intra-BSS handover in BSS-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure.

If the encryption algorithm is changed at an intra-MSC handover in MSC-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure which shall be initiated by MSC-B on reception from BSS-B of the Handover Complete message (the information being previously received in the Handover Request Acknowledge message).

Note also that the chosen encryption value may be contained in the BSSMAP Assignment Complete message. This may happen if the encryption value changes e.g. at a second assignment during a call (e.g. from TCH to SDCCH).

4.5.5.2	Channel Type

Assignment Request and Handover Request (BSSMAP) may give the BSS a choice, in the same way as the Encryption Algorithm above. Depending on the Channel Type Info, the chosen channel may have impact on subsequent handovers, internal in MSC-B and inter-MSC controlled by MSC-A. Some values in channel Type Info indicate that if a particular channel once has been chosen, the same type must be used for the rest of the call.

The Channel Type, and the Chosen Channel shall be stored by MSC-B, and the Chosen Channel transferred to MSC-A.

Transfer of Information

Independently of the type of resource (Signalling only (e.g. SDCCH) or TCH) assigned to the MS, the Channel Type Information is transferred to MSC-B in:

-	the Handover Request BSSMAP message, and the Chosen Channel should be reported back to MSC-A in the Handover Request Acknowledge.

If a new type of resource is to be assigned after Inter-MSC Handover, this can be made with:

-	the BSSMAP Assignment procedure between MSC-A and MSC-B (Chosen Channel in Assignment Complete).

If the Channel Type (the chosen channel) is changed at an intra-BSS handover in BSS-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure.

If the Channel Type (the chosen channel) is changed at an intra-MSC handover in MSC-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure which shall be initiated by MSC-B on reception from BSS-B of the Handover Complete message (the information being previously received in the Handover Request Acknowledge message).

4.5.5.3	Classmark

This information shall be stored by MSC-B and might be received either from MSC-A, or from the MS when the MS initiates a Classmark Update.

Transfer of Information due to Classmark received from MSC-A:

This information shall be stored by MSC-B and is received:

-	in the Handover Request BSSMAP message

If a new type of resource is to be assigned after Inter-MSC Handover, Classmark Information MAY be included:

-	in the BSSMAP Assignment procedure

Transfer of Information, due to "Classmark Signalling Procedures"

This information shall be stored by MSC-B and can be received:

-	Due to a classmark update, either requested from MSC-A (Classmark Request, Classmark Update), or an MS-Initiated Classmark Update.

This can be carried out either with:

-	the BSSMAP Classmark procedure(s).

Apart from these cases there is the "odd" case where a Classmark Update can be received during an Inter-MSC Handover by MSC-B, i.e. before the MS has moved to the new channel controlled by MSC-B. This can be made with transparent transfer of BSSMAP Classmark Update.

4.5.5.4	Downlink DTX-Flag

The parameter shall be stored by MSC-B to be used at internal Handover in MSC-B.

Transfer of Information

Received by MSC-B from MSC-A in either:

If the MS has already been assigned to a TCH for speech before the Inter-MSC Handover, the DTX-flag should be sent in:

-	the Handover Request BSSMAP message

(If the type of resource is not TCH for speech, the DTX-flag shall not be included).

If a new assignment to a TCH for speech after an Inter-MSC Handover is to be performed, this can be made with:

-	the BSSMAP Assignment procedure.

4.5.5.5	Priority

The parameter shall be stored by MSC-B and is received according to below:

Transfer of Information

Received by MSC-B from MSC-A in:

-	the Handover Request BSSMAP message

If a change is needed after an Inter-MSC Handover with:

-	the BSSMAP Assignment procedure.

4.5.5.6	MSC/BSC-Invoke Trace Information Elements

The process to be performed by MSC-B on the information elements of the MSC or BSS Invoke Trace BSSMAP messages is left for further study.

4.5.6	Overview of the Technical Specifications GSM interworking for the Inter-MSC Handover

History

Document history

March 1994	Public Enquiry	PE 59:	1994-03-21 to 1994-08-12

╔══════════════════════════════════════════════════════════════════════════════════════════════════╗

║ PSTN/ISDN                                                                                        ║

║               ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                 ║

║               ▒       ┌───────────────┐                        ▒                                 ║

║ ==============▒====== │ ============= │ ===============        ▒                                 ║

║ PLMN          ▒       │               │                        ▒                                 ║

║               ▒       V               V                        ▒                                 ║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                                ▒                                 ║

║ ▒  BSS-A            MSC-A           MSC-B          BSS-B       ▒     MS                          ║

║ ▒                                                              ▒                                 ║

║ ▒                 ┌───────┐                                    ▒  ┌───────┐                      ║

║ ▒                 │  C M  │                 04.08 (Note)       ▒  │  C M  │                      ║

║ ▒                 ├───────┤<─────────────────────────────────────>├───────┤                      ║

║ ▒ ░░░░░░░░░░░░░░░ │  M M  │ ░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░ ▒  │  M M  │                      ║

║ ▒ ░┌──────┐ 08.08 ├───────┤ 08.08'┌───────┐ 08.08 ┌───────┐  ░ ▒  ├───────┤                      ║

║ ▒ ░│ R R  │<─────>│  R R  │<=====>│  R R  │<─────>│  R R  │  ░ ▒  │  R R  │                      ║

║ ▒ ░└──────┘       └───────┘ (Note)└───────┘       └───────┘  ░ ▒  └───────┘                      ║

║ ▒ ░                A   A             A   A                   ░ ▒                                 ║

║ ▒ ░                │   │    09.10    │   │                   ░ ▒  Note: Subset of 08.08          ║

║ ▒ ░                │   V             V   │                   ░ ▒        procedures as described  ║

║ ▒ ░                │┌─────┐ 09.02 ┌─────┐│                   ░ ▒        in the TS GSM 08.08.     ║

║ ▒ ░                ││MAP/E│<─────>│MAP/E││                   ░ ▒                                 ║

║ ▒ ░                │└─────┘       └─────┘│              03.09░ ▒                                 ║

║ ▒ ░░░░░░░░░░░░░░░░ │ ░░░░░░░░░░░░░░░░░░░ │ ░░░░░░░░░░░░░░░░░░░ ▒  Remark: The A-bis interface and║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒          the link layer         ║

║                   ┌───────────────────────┐                               protocols being out of ║

║                   │       T C A P         │                               the scope of this      ║

║                   └───────────────────────┘                               Specification are not  ║

║    ┌──────────────────────────────────────────────────────┐               shown here.            ║

║    │                      S C C P                         │                                      ║

║    └──────────────────────────────────────────────────────┘                                      ║

║    ┌──────────────────────────────────────────────────────┐                                      ║

║    │                        M T P                         │                                      ║

║    └──────────────────────────────────────────────────────┘                                      ║

╚══════════════════════════════════════════════════════════════════════════════════════════════════╝