ETSI STC GSM3

Recommendation GSM 09.11 			    Version 3.0.1

Signalling interworking for

Supplementary Services

Date	: 8 October 1990

List of contents:					     page

1. Scope		2

2. Categories of Supplementary Services	3

3. Interworking Arrangements	6

3.1	Call related supplementary services management	6

3.2	Call independent supplementary services management	6

4. Mapping between Transaction Sublayer messages and Layer 3

messages	7

5. Mapping of Operation Codes, Error Codes, Parameter Tags

and Parameter Contents	9

5.1	Operation codes	9

5.2	Error codes	9

5.3	Parameter tags and parameter values	9

Note:	This recommendation is only stable with respect to E1 supplementary services.

Language of original: English

Number of pages: 9

SECTION 1

1.  SCOPE

The scope of this recommendation is to provide a detailed specification for interworking between operations on the radio path and operations in MAP for handling of supplementary services.

Reference is made to the following recommendations:

-	GSM 02.04 and GSM 02.81 through 02.88

for definition of supplementary services,

-	GSM 03.11 and GSM 03.81 through 03.88

for technical realisation of supplementary services,

-	GSM 04.10 and GSM 04.80 through 04.88

for radio path signalling procedures for supplementary services,

-	GSM 09.02 (MAP).

Useful material is also found in the following CCITT recommendations:

-	X.208 and X.209 ASN.1,

-	X.219 Remote operation: Service definition,

-	X.229 Remote operation: Protocol specification,

-	Q.771 through Q.775 TCAP.

SECTION 2

2.  CATEGORIES OF SUPPLEMENTARY SERVICES

Management of supplementary services is classified in two

categories:

-	Call related supplementary services management.

This type of management may use either of the message types shown in Table 1/GSM 09.11.

-	Call independent supplementary services management.

This type of management may use either of the message types shown in Table 2/GSM 09.11.

Table 3/GSM 09.11 summarises the operations defined for supplementary services in Recommendations GSM 04.80 and 09.02 and shows the applicability of these operations to the categories above.

Table 4/GSM 09.11 shows whether or not a mobile user is allowed to operate on a supplementary service in a specific way, and whether this operation is call related or call independent.

┌───────────────────────────────────┬───────────────────┐

│   Message type                    │  Reference to GSM │

│                                   │  recommendation   │

├───────────────────────────────────┼───────────────────┤

│   FACILITY                        │  04.80            │

│   HOLD                            │  04.80            │

│   HOLD ACKNOWLEDGE                │  04.80            │

│   HOLD REJECT                     │  04.80            │

│   RETRIEVE                        │  04.80            │

│   RETRIEVE ACKNOWLEDGE            │  04.80            │

│   RETRIEVE REJECT                 │  04.80            │

│   ALERTING                        │  04.08            │

│   CONNECT                         │  04.08            │

│   DISCONNECT                      │  04.08            │

│   RELEASE                         │  04.08            │

│   RELEASE COMPLETE                │  04.08            │

│   SETUP                           │  04.08            │

└───────────────────────────────────┴───────────────────┘

Table 1/GSM 09.11

Messages for call related supplementary services management

┌───────────────────────────────────┬───────────────────┐

│    Message type                   │  Reference to GSM │

│                                   │  recommendation   │

├───────────────────────────────────┼───────────────────┤

│   FACILITY                        │  04.80            │

│   REGISTER                        │  04.80            │

│   RELEASE COMPLETE                │  04.08            │

└───────────────────────────────────┴───────────────────┘

Table 2/GSM 09.11

Messages for call independent supplementary services management

┌─────────────────────────────────┬────────────────────────┐

│   Operation name                │    SS management       │

│                                 │call       │ call       │

│                                 │related    │ independent│

├─────────────────────────────────┼───────────┼────────────┤

│   Register SS                   │Not appl.  │Applicable  │

│   Erase SS                      │Not appl.  │Applicable  │

│   Activate SS                   │Applicable │Applicable  │

│   Deactivate SS                 │Applicable │Applicable  │

│   Interrogate SS                │Not appl.  │Applicable  │

│   Invoke SS                     │Applicable │Not appl.   │

│   Get Password                  │Not appl.  │Applicable  │

│   Register Password             │Not appl.  │Applicable  │

│   Process Unstructured SS data  │Applicable │Applicable  │

│   Forward SS Notification       │Not appl.  │Applicable  │

│   Forward Check SS Indication   │Not appl.  │Applicable  │

└─────────────────────────────────┴───────────┴────────────┘

Note:	Operation names are the same in 09.02 and 04.80, except for Forward SS Notification (09.02) which corresponds to Notify SS in 04.80.

Table 3/GSM 09.11

Applicability of supplementary services operations

╒════════════════════════════════╤═════════╤═════════╤══════════╤════════════╤═════════╤═════════════╤══════════╕

│ Service or group of services   │ Register│ Erase   │Activate  │ Deactivate │ Invoke  │ Interrogate │Register  │

│ in GSM PLMN                    │         │         │          │            │         │             │password  │

╞════════════════════════════════╪═════════╪═════════╪══════════╪════════════╪═════════╪═════════════╪══════════╡

│ Calling line identification    │ NA      │ NA      │ A        │ A          │ A       │ NA          │   NA     │

│ presentation                   │         │         │          │            │         │             │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Calling number identification  │ NA      │ NA      │ Per call │ A          │ A       │ Status      │   NA     │

│ restriction                    │         │         │ /A       │            │         │             │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Connected number identification│ NA      │ NA      │ A        │ A          │ A       │ NA          │   NA     │

│ presentation                   │         │         │          │            │         │             │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Connected number identification│ NA      │ NA      │ A        │ A          │ A       │ Status      │   NA     │

│ restriction                    │         │         │          │            │         │             │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Malicious call identification  │ NA      │ NA      │ A        │ A          │ Per call│ Status      │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Call forwarding services       │ HLR     │ HLR     │ A        │ A          │ A       │ Data req.   │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Call transfer                  │ NA      │ NA      │ A        │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Mobile access hunting          │ NA      │ NA      │ A        │ A          │ A       │ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Call waiting                   │ NA      │ NA      │ HLR      │ HLR        │ A       │ Status      │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Call hold                      │ NA      │ NA      │ A        │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Completion of call to          │ NA      │ NA      │ Per call │ Per call   │ A       │ Status/     │   NA     │

│ busy subscriber                │         │         │          │            │         │ data req.   │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Three party service            │ NA      │ NA      │ A        │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Conference call                │ NA      │ NA      │ A        │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Closed user group              │ NA      │ NA      │ A        │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Advice of charge               │ NA      │ NA      │ A        │ A          │ A       │ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Freephone                      │ HLR     │ HLR     │ Per call │ Per call   │ A       │ Data request│   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Reverse charging               │ NA      │ NA      │ A        │ A          │ Per call│ Status      │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ User to user signalling        │ NA      │ NA      │ Per call │ A          │ Per call│ NA          │   NA     │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Barring of outgoing calls      │ NA      │ NA      │ HLR      │ HLR        │ A       │ Data req./  │   HLR    │

│                                │         │         │          │            │         │ Status      │          │

├────────────────────────────────┼─────────┼─────────┼──────────┼────────────┼─────────┼─────────────┼──────────┤

│ Barring of incoming calls      │ NA      │ NA      │ HLR      │ HLR        │ A       │ Data req./  │   HLR    │

│                                │         │         │          │            │         │ Status      │          │

╘════════════════════════════════╧═════════╧═════════╧══════════╧════════════╧═════════╧═════════════╧══════════╛

Table 4/GSM 09.11

Applicability of operations to supplementary services

Terminology for Table 4/GSM 09.11 :

NA	=	Not Applicable,

A	=	Administrative or Automatic,

Per call	=	per call procedures apply, i.e. call related,

HLR	=	mobile user is allowed to operate on the HLR , i.e. call independent,

Status/data req.	=	mobile user is allowed to interrogate the location registers, i.e. call independent.

SECTION 3

3.  INTERWORKING ARRANGEMENTS

3.1  Call related supplementary services management

Call related supplementary service management is part of call control. Therefore, if management functions are required not only in the MSC but also in the VLR, the interworking is specified in Recommendation GSM 09.02 in relation with the operations for incoming and outgoing call set-up. These interworking arrangements are not considered further here.

3.2  Call independent supplementary services management

For call independent supplementary services management the MSC shall:

-	check the components and layer 3 messages received on

the radio path for formal errors,

-	check the transaction header and the components received in TCAP for formal errors,

-	perform mapping of operation codes, parameter tags, parameter values, invoke id and error codes as required (see below),

-	interwork between transaction sublayer messages in TCAP and layer 3 messages on the radio path (see below).

On the TCAP connection between the MSC and the VLR supplementary services operations are always sent in dialogues which have been opened by another operation (normally the process access request operation). If a supplementary service message is received on the radio path and there is no suitable dialogue with the VLR, the message shall be rejected. Similarly, a supplementary services operation in TCAP is rejected if there is no radio connection with the MS.

SECTION 4

4.  MAPPING BETWEEN TRANSACTION SUBLAYER MESSAGES AND LAYER 3

MESSAGES

The mapping of transaction sublayer messages to layer 3 messages

is shown in Table 5/GSM 09.11; Table 6/GSM 09.11 shows the

reverse mapping. Note that the mapping is not symmetric.

┌──────────────────────────────┬────────────────────────┐

│ Transaction sublayer message │  Layer 3 message       │

├──────────────────────────────┼────────────────────────┤

│ BEGIN  1)                    │ --                     │

│ CONTINUE                     │ FACILITY/REGISTER   2) │

│ END                          │ RELEASE COMPLETE       │

│ ABORT                        │ RELEASE COMPLETE       │

└──────────────────────────────┴────────────────────────┘

Note 1 :	Use of a BEGIN message corresponds to a procedure error, see section 3.2

Note 2 :	CONTINUE is mapped to FACILITY if the "dialogue" on the radio path has been opened by a REGISTER message. Otherwise it is mapped to REGISTER

Table 5/GSM 09.11

Mapping of transaction sublayer messages to layer 3 messages

┌────────────────────────┬──────────────────────────────┐

│ Layer 3 message        │ Transaction sublayer message │

├────────────────────────┼──────────────────────────────┤

│ REGISTER               │ CONTINUE  1)                 │

│ FACILITY               │ CONTINUE                     │

│ RELEASE COMPLETE       │ END                          │

└────────────────────────┴──────────────────────────────┘

Note 1 :	Mapping is only possible if a TCAP dialogue exists, see section 3.2.

Table 6/GSM 09.11

Mapping of layer 3 messages to transaction sublayer messages

The mapping from Transaction Sublayer messages to Layer 3 messages must include a replacement of the tag and length of the Component Portion in the Transaction Sublayer message with the Information element identifier and length of the Facility Information Element for the Layer 3 message. Similarly for the reverse mapping.

All transaction sublayer messages, except the ABORT message, will normally contain one or more components. If components are included, the conversion algorithm of section 5 applies. If a message does not contain a component, then the corresponding message is also sent without a component: messages shall not be withheld by the interworking function.

If an ABORT message is received in TCAP, a RELEASE COMPLETE message is to be sent on the radio path. The RELEASE COMPLETE message shall not contain any component. If a cause is to be provided to the MS, one of the cause codes of recommendation GSM 04.08 shall be used.

If a layer 3 message or a component in the layer 3 message is rejected by the MSC, the MSC shall:

-	return a RELEASE COMPLETE message to the MS. If the reject condition is not associated with a component, one of the cause codes of Recommendation GSM 04.08 shall be inserted. If it is a component (except a REJECT component), a REJECT component with the appropriate problem code shall be inserted in the RELEASE COMPLETE message. If the reject condition concerns a REJECT component the RELEASE COMPLETE message may be empty,

-	terminate the transaction with the VLR by use of an ABORT message.

SECTION 5

5.  MAPPING OF OPERATION CODES, ERROR CODES,PARAMETER TAGS AND

PARAMETER CONTENT

5.1  Operation codes

The same operation codes are used for equivalent operations in

Recommendations GSM 04.80 and GSM 09.02.

5.2  Error codes

The same error codes are used for equivalent error types in

Recommendations GSM 04.80 and GSM 09.02, except for forward SS Notification in Recommendation GSM 09.02 which corresponds to NotifySS in Recommendation GSM 04.80.

The RETURN ERROR components are also constructed in the same way

on both sides of the interface.

5.3  Parameter tags and parameter values

The same parameter tags and parameter values are used for

equivalent parameters in Recommendations GSM 04.80 and GSM 09.02.