TD <>

Draft TS 100 960 proposed V1.2.0 (1997-08)

Digital cellular telecommunications system (Phase 2+);

General Packet Radio Service (GPRS);

GPRS Tunnelling Protocol (GTP)

across the Gn and Gp Interface;

(GSM 09.60 proposed version 1.2.0)

Reference

<Workitem>

Keywords

<keyword[, keyword]>

ETSI Secretariat

Postal address

F-06921 Sophia Antipolis Cedex - FRANCE

Office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

X.400

c= fr; a=atlas; p=etsi; s=secretariat

Internet

secretariat@etsi.fr

http://www.etsi.fr

Copyright Notification

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restrictions extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

Contents

1. Scope

2. Normative references

3. Definitions and abbreviations

3.1 Definitions

3.2 Abbreviations

4. General

5. Signalling Plane

5.1 Signalling protocol

5.2 Signalling Message Formats

5.3 IP Header

5.3.1 Request messages

5.3.2 Response messages

5.4 UDP Header

5.4.1 Request messages

5.4.2 Response messages

5.5 Common Header

5.6 Path Management messages

5.6.1 Echo Request

5.6.2 Echo Response

5.6.3 Path MTU Request

5.6.4 Path MTU Response

5.7 Tunnel Management messages

5.7.1 Create PDP Context Request

5.7.2 Create PDP Context Response

5.7.3 Update PDP Context Request

5.7.4 Update PDP Context Response

5.7.5 Delete PDP Context Request

5.7.6 Delete PDP Context Response

5.7.7 Create AA PDP Context Request

5.7.8 Create AA PDP Context Response

5.7.9 Delete AA PDP Context Request

5.7.10 Delete AA PDP Context Response

5.7.11 Error Indication

5.7.12 PDU Notification

5.7.13 PDU Reject

5.8 Location Management messages

5.8.1 Send Routing Information Request

5.8.2 Send Routing Information Response

5.8.3 Failure Report

5.8.4 Failure Report Acknowledgement

5.8.5 Note MS Present

5.9 Mobility Management messages

5.9.1 Identification Request

5.9.2 Identification Response

5.9.3 SGSN Context Request

5.9.4 SGSN Context Response

5.10 Reliable delivery of signalling messages

5.11 Transmission order

5.12 Information elements

5.12.1 Cause

5.12.2 International Mobile Subscriber Identity (IMSI)

5.12.3 Routing Area Identity (RAI)

5.12.4 Temporary Logical Link Identity (TLLI)

5.12.5 No. of PDP Contexts

5.12.6 Quality of Service (QoS)

5.12.7 BB Protocol

5.12.8 Reordering Required

5.12.9 No. of Triplets

5.12.10 Authentication Triplet

5.12.11 Receive State Variables

5.12.12 GTP Path MTU

5.12.13 End User Address

5.12.14 MM Context

5.12.15 PDP Context

6. Transmission Plane

6.1 Protocol Stack

6.1.1 IP Header

6.1.2 UDP Header

6.1.3 TCP Header

6.1.4 GTP Header

6.2 TCP transmission

6.3 Tunnelling between SGSNs

6.4 Tunnelling between GGSNs

7. State transition diagrams

7.1 General

7.2 State transition tables

7.3 GTP services

7.3.1 SGSN, PDP Context

7.3.2 SGSN, PDP Context for anonymous access

7.3.3 GGSN, PDP Context

7.3.4 GGSN, PDP Context for anonymous access

7.3.5SGSN, MM Context

8. Error handling

8.1 Protocol errors

8.1.1 Message too short

8.1.2 Unknown or erroneous header

8.1.3 Missing mandatory information element

8.1.4 Invalid information element

8.1.5 Unknown information element

8.1.6 Different GTP versions

8.2 Mismatching PDP state in SGSN

8.3 Missing MM context in SGSN

8.4 Missing PDP context in GGSN

9. Inter-PLMN GTP communication over the Gp interface

10. IP, the networking technology used by GTP

10.1 IP fragmentation

10.1.1 MO direction

10.1.2 MT direction

10.1.3 Tunnelling from old to new SGSN

11. GTP parameters

11.1 Timers

11.2 Counters

12. History

3. Definitions and abbretio

3.2 Abbreviation

5. SignalliPlacol

5.2 Signalling MessaFora

5.3.1 Request

5.4 UDP Header	essages

5.4.2 Response sages

5.6 Path Managemeness

5.6.2 Path MTU Responsessages

5.7.1 Create PDP Contextquest

5.7.3 Update PDP Context Request

5.7.5 Delete PDP Context Request

5.7.7 Create AA PDP Context Reque

5.7.9 Delete AA PDP Context Request	e

5.7.11 Error Indication

5.7.13 PDU Reject	ement messages

5.8.1 Send Routingforst

5.8.3 Failure Report	Acknowledgement

5.8.5 Note MS Present	t messages

5.9.1 Identification Rest

5.9.3 SGSN Context Request

5.10 Reliable delivery of sa me

5.12 Information element

5.12.2 Internber

5.12.4 Temporary Logical Link Ident

5.12.6 Quality of Service (

5.12.8 Reordering R

5.12.10 Authentication p

5.12.11 Receive State Variables

5.12.13 End User Addr

5.12.14 MM Context

6. Transmission Plane

6.1 Protocol Stack

6.1.2 UDP Header

6.1.3 TCP Header

6.2 Tunnelling been

7. State transition diagrams

7.2 State tr

7.3.1 SGSN, PDP Cfor anonymous access

7.3.3 GGSN, PDP Context	for anonymous access

7.3.5SGSN, MM Context

8.1 Protocol errorort

8.1.2 Unknown or errousher	n element

8.1.4 Invalid information element

8.1.6 Different GTP versions	SGSN

8.3 Missing MM context in SGSN

9. Inter-PLMN GTP communicatione Gp

10.1 IP fragmentation

10.1.1 MO direction

10.1.3 Tunnelling frold

11.1 Timers	s

12. History

Intellectual Property Rights

This clause is always the first unnumbered clause.

If you have received any information concerning an essential IPR related to this document please indicate the details here.

Foreword

This ETSI Technical Specification (TS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI).

This TS defines the Gn and Gp interfaces for the General Packet Radio Service (GPRS) within the digital cellular telecommunications system (Phase 2+).

# Scope

This GSM Technical Specification defines the Gn and Gp interfaces for the General Packet Radio Service (GPRS).

# Normative references

This specification incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this specification only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 100): "Digital cellular telecommunications system (Phase 2); Abbreviations and acronyms".

[2]	GSM 03.03:  “Digital cellular telecommunications system (Phase 2+); Numbering, addressing and identification”.

[2]	GSM 03.60: "Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Service Description; Stage 2".

[4]	GSM 03.64:  “Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Overall description of the GPRS Radio Interface; Stage 2”

[3]	GSM 04.08: "Digital cellular telecommunications system (Phase 2); Mobile radio interface layer 3 - specification".

[4]	GSM 04.64: “Digital cellular telecommunications system; Mobile Station - Serving GPRS Support Node (MS-SGSN) Logical Link Control (LLC) Layer Specification”

[4]	GSM 09.02: "Digital cellular telecommunications system (Phase 2+); Mobile Application Part (MAP) specification”.

[5]	STD 0005: “Internet Protocol”, J. Postel.

[6]	STD 0006: “User Datagram Protocol”, J. Postel.

[7]	STD 0007: “Transmission Control Protocol”, J. Postel.

[8]	RFC 1191: “Path MTU Discovery”, J. Mogul and S. Deering.

[9]	RFC 1981: “Path MTU Discovery for IPv6”, J. McCann and S. Deering and J. Mogul.

[10]	RFC 2181: “Clarifications to the DNS Specification”, R. Elz and R. Bush.

# Definitions and abbreviations

## Definitions

For the purpose of this GSM Technical Specification, the following definitions apply:

BB Protocol:					See Path Protocol.

G-PDU: 	A T-PDU plus a GTP header. A G-PDU is sent in a path.

GTP tunnel:	A GTP tunnel is defined by two associated PDP Contexts in different GSN nodes and is identified with a Tunnel ID. A GTP tunnel is necessary to forward packets between an external packet data network and a MS user.

MM Context:	The information sets held in MS and GSNs for a GPRS subscriber related to mobility management (MM) (please refer to the MM Context Information Element).

MM Context ID:	IMSI or equivalent for use in conjunction with Anonymous Access (please refer to sectionGTP Signalling Header).

NSAPI:	Network Service Access Point Identifier. A fixed value between 0 and 15, identifying a certain PDP Context. It identifies a PDP context belonging to a specific MM Context ID.

Path:	The UDP/IP tunnel and TCP/IP tunnel are examples of paths that can be used to multiplex GTP tunnels.

Path Protocol:	The Path

PDP:	A Packet Data Protocol (PDP) is a network protocol used by an external packet data network interfacing to GPRS.

PDP Context:	The information sets held in MS and GSNs for a PDP address (please refer to the PDP Context Information Element).

Quality of Service :	Quality of Service may be applicable for the GPRS backbone if the path media supports it. Separate paths with different priorities may be defined between a GSN pairHowever, the possible use of QoS in the GGSN is outside the scope of the GTP specification.

Signalling message:	GTP signalling messages are exchanged between GSN pairs in a path.l. The signalling messages are used to transfer GSN capability information between GSN pairs and to create, update and delete GTP tunnels.

TCP/IP tunnel:	A TCP/IP tunnel is a virtual data link defined by two tunnel end-points and a tunnel end-point is defined by an IP address and a TCP port number. A TCP/IP tunnel can be used instead of a UDP/IP tunnel to tunnel encapsulated T-PDUs but should not be used when the T-PDUs are IP datagrams. TCP/IP tunnels should be used when the T-PDUs are X.25 packet layer protocol packets.

T-PDU: 	An original packet, for example an IP datagram, from a MS or a network node in an external packet data network. A T-PDU is the payload that is tunnelled in the GTP tunnel.

TID:	A Tunnel ID (TID) consists of a MM Context ID and a NSAPI.

UDP/IP tunnel:	A UDP/IP tunnel is a virtual data link defined by two tunnel end-points and a tunnel end-point is defined by an IP address and a UDP port number. A UDP/IP tunnel carries G-PDUs between GSN nodes related to one or more GTP tunnels.

## Abbreviations

Abbreviations used in this specification are listed in GSM 01.04.

For the purpose of this specification the following additional abbreviations apply:

BB	Backbone Bearer

DF	DonÕt Fragment

FFS	For Further Study

GTP	GPRS Tunneling Protocol

IANA	Internet Assigned Number Authority

ICMP	Internet Control Message Protocol

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

MTU	Maximum Transmission Unit

QoS	Quality of Service

TID	Tunnel  IDentifier

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

Gn interface	Interface between GPRS Support Nodes (GSNs) within a PLMN

Gp interface	Interface between GPRS Support Nodes (GSNs) in different PLMNs

# General

This document defines the GPRS Tunnelling Protocol (GTP), i.e. the protocol between GSN nodes in the GPRS backbone network. It includes both the GTP signalling and data transfer procedures.

GTP is defined both for the Gn interface, i.e. the interface between GSNs within a PLMN, and the Gp interface between GSNs in different PLMNs.

GTP allows multiprotocol packets to be tunnelled through the GPRS Backbone between GPRS Support Nodes (GSNs).

In the signalling plane, GTP specifies a tunnel control and management protocol which allows the SGSN to provide GPRS network access for a MS. Signalling is used to create, modify and delete tunnels.

In the transmission plane, GTP uses a tunnelling mechanismto provide a service for carrying user data packets. The choice of path is dependant on whether the user data to be tunnelled requires a reliable link or not.

The GTP protocol is implemented only by  SGSNs and GGSNs. No other systems need to be aware of GTP. GPRS MSs are connected to a SGSN without being aware of GTP.

It is assumed that there will be a many-to-many relationship between SGSNs and GGSNs.  A SGSN may provide service to many GGSNs. A single GGSN may associate with many SGSNs to deliver traffic to a large number of geographically diverse mobile stations.

# Signalling Plane

The signalling plane in this case relates to GPRS Mobility Management functions like for example GPRS Attach, GPRS Routeing Area Update and Activation of PDP Contexts. The signalling between GSN nodes shall be performed by theGPRS Tunnelling Protocol (GTP).

Figure 1: Signalling Plane - Protocol stack

Signalling protocol

The signalling flow shall be logically associated with, but separate from, the tunnels. For each GSN-GSN pair one or more paths exist. One or more tunnels can use each path. GTP shall be the means by which tunnels are established, used, managed and released. A path may be maintained by keep-alive echo messages. This ensures that a connectivity failure between GSNs can be detected in a timely manner.

Figure

## Signalling Message Formats

GTP defines a set of signalling messages between two associated GSNs. The signalling messages to be used are defined in the table below.

Table 1: Signalling messages

| Message Type value | Signalling message | Reference |
| --- | --- | --- |
| 1 | Echo Request | 5.4.1 |
| 2 | Echo Response | 5.4.2 |
| 3 | Path MTU Request | 5.4.3 |
| 4 | Path MTU Response | 5.4.4 |
| 16 | Create PDP Context Request | 5.5.1 |
| 17 | Create PDP Context Response | 5.5.2 |
| 18 | Update PDP Context Request | 5.5.3 |
| 19 | Update PDP Context Response | 5.5.2 |
| 20 | Delete PDP Context Request | 5.5.5 |
| 21 | Delete PDP Context Response | 5.5.6 |
| 22 | Create AA PDP Context Request | 5.5.7 |
| 23 | Create AA PDP Context Response | 5.5.8 |
| 24 | Delete AA PDP Context Request | 5.5.9 |
| 25 | Delete AA PDP Context Response | 5.5.10 |
| 26 | Error Indication | 5.5.11 |
| 27 | PDU Notification | 5.5.12 |
| 28 | PDU Reject | 5.5.13 |
| 32 | Send Routeing Information for GPRS Request | 5.6.1 |
| 33 | Send Routeing Information for GPRS Response | 5.6.2 |
| 34 | Failure Report | 5.6.3 |
| 35 | Failure Report Acknowledgement | 5.6.4 |
| 36 | Note MS Present | 5.6.5 |
| 48 | Identification Request | 5.7.1 |
| 49 | Identification Response | 5.7.2 |
| 50 | SGSN Context Request | 5.7.3 |
| 51 | SGSN Context Response | 5.7.4 |
| 255 | reserved for the transmission plane |  |
| others | reserved for future use |  |

The reserved fields in the messages shall be filled with ones. The reserved fields are intended for future use.

## I

## I

## GTP Signalling Header

The GTP signalling header shall be a fixed format 16 octets header used for all GTP signalling messages.

Version shall be set to 0 to indicate the first version of GTP.

LFN shall be set to 0.

The Message Type shall be set to the unique value that is used for each type of signalling message.

Length shall be the length of the signalling message including the GTP signalling header.

Transaction ID shall be a message number valid for a path that shall be incremented for each GTP signalling request message sent on the path. The Transaction ID in a signalling response message shall be copied from the signalling request message that the GSN is replying to.

TID (see Figure 4: Tunnel ID (TID) format) shall be set to 0 in all Path Management messages (see section 5.4), Location Management messages (see sectioLocation Management messages 5.6) and Mobility Management messages (see secti Mobility Management messages 5.7). In the Tunnel Management messages (see sectn Tunnel Management messages 5.5), TID shall be used to point out the MM and PDP Contexts in the destination GSN.

The GTP signalling header has basically the same format as the GTP transmission header (see section GTP Transmission Header). The Version and Length fields are found at the same offsets and have the same meaning in both headers. The LFN and its related LLC Frame Number are never used in the GTP signalling header but may be used in the GTP transmission header. The TID is sometimes set to 0 in the GTP signalling header but must always be defined in the transmission header of a G-PDU. The Message Type 255 (all ones) is reserved for G-PDUs.

Figure 3: Outline of GTP signalling header

Note 1:	The MCC, MNC and MSIN are parts of the IMSI defined in GSM 03.03. For Anonymous Access, the IMSI shall be replaced by a SGSN number and a locally assigned number.

Note 2:	MSIN digits not used shall be set to FF (HEX).

Figure 4: Tunnel ID (TID) format

The GTP signalling header may be followed by subsequent Information Elements dependent on the type of signalling message.

Figure 5: GTP signalling header followed by subsequent  Information Elements

## Path Management messages

The Path Management messages can be sent between any type of GSN pair.

### Echo Request

An Echo Request may be sent on a path to another GSN to find out if the other node is alive (see  section 9). Echo Request messages mal be sent for each path in use at regular intervals.

A GSN shall be prepared to receive an Echo Request at any time and it shall reply with an Echo Response. A GSN may optionally send Echo Request messages.

The Echo Request contains no Information Elements.

### Echo Response

The message shall be sent as a response of a received Echo Request.

The Echo Response contains no Information Elements.

### Path MTU Request

A Path MTU Request is amessage used to find out the maximum datagram and fragment sizes to optimize the network performance. The path MTU can be used to minimise the necessity of IP fragmentation in the backbone. A GSN shall be prepared to receive a Path MTU Request at any time and it shall reply with a Path MTU Response. A GSN may optionally send Path MTU Request messages.

The Don’t Fragment (DF) bit shall then be explicitly set in the IPv4 header and implicitly set in IPv6 header for the Path MTU Request message. An ICMP message (fragmentation needed and DF set - IPv4 or Packet Too Big - IPv6) should be returned by an intermediate router if the data part is too large. The ICMP message contains the MTU of the next link (always for IPv6 and for IPv4 if the router supports [9])  that is lower than the initial size.

The requester initially assumes that the path MTU is the known MTU of the first hop in the backbone path and adapts the path MTU downwards if any of the ICMP messages above are returned.It is an implementation issue how often the path MTU is negotiated..

The receiver of the Path MTU Request may use the size of latest received Path MTU Request to calculate the path MTU for the path to the peer but should send an own Path MTU Request to the peer. Different path MTU values may be used for each direction depending on the backbone network topology.

The size of the path MTU between a GSN pair shall be calculated as the UDP length (from the UDP header) minus the GTP header size.

A new Path MTU Request with adapted message length shall be sent immediately if any of the ICMP messages above have been received.

Note that the Transaction ID shall always be incremented for each Path MTU Request.

The information element in a Path MTU Request message is described in the table below.

Table 2: Information elements in a Path MTU Request

| Information element | Reference |
| --- | --- |
| GTP Path MTU | 5.10.12 |

### Path MTU Response

The message shall be sent as a response to a received Path MTU Request.  The sender of the Path MTU Request can assume that the Path MTU value, reflected in the size of the GTP Path MTU information element, is correct when a Path MTU Response is received.

The Path MTU Response message contains no Information Elements.

## Tunnel Management messages

The Tunnel Management messages are the control and management messages used to create, update and delete tunnels to be able to route T-PDUs between a MS and an external packet data network via SGSN and GGSN.

### Create PDP Context Request

A Create PDP Context Request shall be sent from a SGSN node to a GGSN node as a part of the GPRS PDP Context Activation procedure. A valid request initiates the creation of a tunnel between a PDP Context in a SGSN and a PDP Context in a GGSN. The GGSN can start to forward T-PDUs from the external data network to the SGSN when the GGSN PDP Context has been activated.

The information elements in a Create PDP Context Request message are described in the table below. If the MS requests a dynamic PDP address and a dynamic PDP address is allowed, then the Protocol Address field in the End User Address information element shall be empty. If the MS requests a static PDP Address then the Protocol Address field in the End User Address information element will contain the static PDP Address. The Quality of Service Profile information element shall be the QoS values negotiated between the MS and the SGSN at PDP Context activation.

The BB Protocol field states if UDP or TCP is requested by the SGSN to tunnel end user data packets. The Reordering Required information element states whether reordering is required or not.

The SGSN can select any suitable GGSN if no GGSN name is defined in the PDP context.n to get the address where to send th Context , the GGSN Name shall be converted to an address. The conversion may be performed with any name-to-address function. The converted address shall be stored in the “GGSN Address in Use” field in the PDP context and be used during the entire lifetime of the PDP context.

Note: 	A DNS query may be used as the name- to- IPaddres mapping of the GGSN. The IP address returned in the DNS response is thenshall be s the “GGSN Address in Use” field in the PDP context. A new query shouldore thNS P address s

Table 4: Information elements in a Create PDP Context Request

| Information element | Reference |
| --- | --- |
| Quality of Service Profile | 5.10.6 |
| End User Address | 5.10.13 |

### Create PDP Context Response

The message shall be sent from a GGSN node to a SGSN node as a response of a Create PDP Context Request. The SGSN can start to forward T-PDUs from the MS to the external data network when the SGSN PDP Context has been activated.

The information elements in a Create PDP Context Response message are described in the table below.

The Cause value indicates if a PDP context has been created in the GGSN or not. A PDP context has not been created in the GGSN if the Cause differs from Request accepted.  Create PDP Context specific Cause values are:

-	‘No resources available’

-	‘Requested address not supported’

No resources available indicates a temporary error condition e.g. all dynamic PDP addresses occupied or no memory available.

Requested address not supported indicates a permanent error condition e.g. when the GGSN does not support the PDP address from the HLR.

If the MS requests a dynamic PDP address and a dynamic PDP address is allowed, then the optional Protocol  Address field in the End User Address information element shall contain the dynamic PDP Address allocated  by the GGSN.

If the MS uses a static PDP address then the End User  Address information element is not included in the response. information ele

The QoS values supplied in the Create PDP Context Request may be negotiated downwards by the GGSN. The negotiated values or the original value from SGSN is inserted in the Quality of Service Profile information element.

The BB Protocol field chooses if UDP or TCP shall be used to transfer the forthcoming encapsulated T-PDUs in the tunnel. Depending on the end user protocol, the GGSN indicates to the SGSN which type of path that shall be used for the tunneling of subsequent T-PDUs. If  TCP is chosen and there already exist one TCP connection between this SGSN-GGSN pair, this TCP connection shall be used. If TCP is chosen and a TCP connection doesn’t exist, the GGSN shall be responsible for establishing one.

Only one TCP connection shall be used between any given GSN-pair, and this TCP connection shall be used to tunnel end user traffic in both directions.

The Reordering Required value supplied in the Create PDP Context Response indicates whether the end user protocol benefits  from packet in sequence delivery and whether the SGSN shall perform reordering or not.

shal

Table 5: Information elements in a Create PDP Context Response

| Information element | Reference |
| --- | --- |
| Cause |  |
| Quality of Service Profile | 5.10.6 |
| BB Protocol | 5.10.7 |
| Reordering required | 5.10.8 |
| End user address (optional) | 5.10.13 |

### Update PDP Context Request

An Update PDP Context Request message shall be sent from a SGSN to a GGSN as part of the GPRS Inter SGSN Routeing Update procedure or the PDP Context Modification procedure. It shall be used to change the QoS, the path and the tunnel. The message shall be sent by the new SGSN at the Inter SGSN Routeing Update procedure. The IP header contains the new SGSN address.

The information elements in an Update PDP Context Request message are described in the table below. The Quality of Service Profile information element shall include the QoS negotiated between the MS and SGSN at PDP Context activation or the new QoS negotiated in the PDP Context Modification procedure.

Table 6: Information elements in an Update PDP Context Request

| Information element | Reference |
| --- | --- |
| Quality of Service Profile | 5.10.6 |

### Update PDP Context Response

The message shall be sent from a GGSN node to a SGSN node as a response of an Update PDP Context Request.

The QoS values supplied in the Create PDP Context Request may be negotiated downwards by the GGSN. The negotiated values or the original value from SGSN is inserted in the Quality of Service Profile information element.

The information elements in an Update PDP Context Response message are described in the table below.

Table 7: Information elements in an Update PDP Context Response

| Information element | Reference |
| --- | --- |
| Quality of Service Profile | 5.10.6 |
| Cause |  |

### Delete PDP Context Request

A Delete PDP Context Request shall be sent from a SGSN node to a GGSN node as part of the GPRS Detach procedure or the GPRS PDP Context Deactivation procedure or from a GGSN node to a SGSN node as part of the PDP Context Deactivation Initiated by GGSN procedure. A request shall be used to deactivate an activated PDP Context.

The Delete PDP Context Request message contains no Information Elements.

### Delete PDP Context Response

The message shall be sent as a response of a Delete PDP Context Request.

The Delete PDP Context Response message contains no Information Elements.

### Create AA PDP Context Request

A Create AA PDP Context Request shall be sent from a SGSN node to a GGSN node as a part of the GPRS Anonymous Access PDP Context Activation procedure. It shall be used to create a tunnel between a PDP Context in a SGSN and a PDP Context in a GGSN. The GGSN may start to forward T-PDUs from the external data network to SGSN when the GGSN PDP Context has been activated.

The information elements in a Create PDP Context Request message are described in the table below.

The Quality of Service Profile information element shall be the QoS values negotiated by the MS and SGSN at Anonymous Access PDP Context activation.

The End User Address contains the requested protocol with the address field left empty. address of the Anonymous Server.

Table 9: Information elements in a Create AA PDP Context Request

| Information element | Reference |
| --- | --- |
| Quality of Service Profile | 5.10.6 |
| End User Address | 5.10.13 |

### Create AA PDP Context Response

The message shall be sent from a GGSN node to a SGSN node as a response of a Create AA PDP Context Request.

The information elements in a Create AA PDP Context Response message are described in the table below.

The QoS values supplied in the Create AA PDP Context Request may be negotiated downwards by the GGSN. The negotiated value or the original value from SGSN is inserted in the Quality of  Service Profile information element.

The BB Protocol decides if UDP or TCP shall be used to transfer the forthcoming encapsulated T-PDUs in the tunnel.

The BB Protocol field chooses if UDP or TCP shall be used to transfer the forthcoming encapsulated T-PDUs in the tunnel. Depending on the end user protocol, the GGSN indicates to the SGSN which type of path that shall be used for the tunneling of subsequent T-PDUs. If  TCP is chosen and there already exist one TCP connection between this SGSN-GGSN pair, this TCP connection shall be used. If TCP is chosen and a TCP connection doesn’t exist, the GGSN shall be responsible for establishing one.

Only one TCP connection shall be used between any given GSN-pair, and this TCP connection shall be used to tunnel end user traffic in both directions.

The Reordering Required value supplied in the Create AA PDP Context Response indicates whether the end user protocol benefits  from packet in sequence delivery and whether the SGSN shall perform reordering or not.

shalThe Protocol  Address field in the End User Address information element contains the dynamic PDP Address allocated  by the GGSN.

Table 10: Information elements in a Create AA PDP Context Response

| Information element | Reference |
| --- | --- |
| Cause |  |
| Quality of Service Profile | 5.10.6 |
| BB Protocol | 5.10.7 |
| Reordering Required | 5.10.8 |
| End User Address | 5.10.13 |

### Delete AA PDP Context Request

A Delete AA PDP Context Request shall be sent from a SGSN node to a GGSN node as part of the GPRS PDP Anonymous Access Context Deactivation procedure. The GGSN may send the request to the SGSN if it detects malicious usage of the service. The request shall be used to deactivate an activated PDP Context.

The Cause information element  indicates whether the SGSN shall request the real identities (i.e. IMSI or IMEI) of the anonymous MS. One of the following Cause values shall be used:

-	“Request IMSI”

-	“Request IMEI”

-	“No identity needed”

Table 11: Information elements in a Delete AA PDP Context Request

| Information element | Reference |
| --- | --- |
| Cause |  |

### Delete AA PDP Context Response

The message shall be sent as a response of a Delete AA PDP Context Request.

The Delete AA PDP Context Response message contains no Information Elements.

### Error Indication

The SGSN sends an Error Indication to the GGSN if no PDP context exists or the PDP context is inactive for a received tunnel PDUthe MS to re-activa The SGSN also sends an Error Indication to the GGSN of no MM context exists for a received tunnel PDU. [FFS: The details how the SGSN can request re-attach with paging.]

The new SGSN sends an Error Indication to the old SGSN if no PDP context exists or the PDP context is inactive for a received tunnel PDU.

The GGSN sends an Error Indication to the SGSN if no PDP context exists for a received tunnel PDU.

The GGSN shall delete its PDP context when an Error Indication is received.

The SGSN shall re-establish the PDP context in GGSN when an Error Indication is received.

The old SGSN shall delete its PDP context when an Error Indication is received.

The TID used in the Error Indication message shall be fetched from the tunnel PDU that triggered this procedure

The Error Indication message contains no Information Elements.

### PDU Notification

When receiving a T-PDU the GGSN checks if a PDP context is established for that PDP address. If no PDP context has been previously established the GGSN may try to deliver the T-PDU by initiating the Network-Requested PDP Context Activation procedure. The criteria, used by the GGSN to determine whether trying to deliver the T-PDU to the MS or not, may be based on subscription information and are outside the scope of GPRS standardisation.

As part of the Network-Requested PDP Context Activation procedure the GGSN sends a PDU Notification message to the SGSN indicated by the HLR, i.e. the current location of the MS. When receiving this message, the SGSN shall be responsible for requesting the MS to activate the indicated PDP Context. The IMSI is inserted in the IMSIpart of the TID in the GTP header of the PDU Notification message. The NSAPI part of the TID is not used and shall be filled with HEX(F).

The End User Address information contains the PDP type and PDP address that the SGSN shall request the MS to activate.

Table 13: Information elements in a PDU Notification

| Information element | Reference |
| --- | --- |
| End User Address | 5.10.13 |

### PDU Reject

If the PDP Context requested by the GGSN can not be established, the SGSN  sends a PDU Reject message as a reply to the PDU Notification message. The Cause value indicates the reason why the PDP Context could not be established:

-	‘IMSI Not Known’.

-	‘MS GPRS Detached’.

-	‘MS Not GPRS Responding’.

-	‘MS Refuses’.

When receiving the PDU Reject message the GGSN may reject or discard the stored T-PDU(s) depending on the PDP type.

After an unsuccessful Network-Requested PDP Context Activation procedure the operator may perform some actions to prevent unnecessary enquires to the HLR. The actions taken depend on the cause of the delivery failure.

-	If the MS is not reachable or the MS refuses the activation request (Cause values ‘MS Not GPRS Responding’ and ‘MS Refuses’) the GGSN may refuse any T-PDU for that PDP address during a certain period. The GGSN may store the SGSN address during a certain period and send any subsequent PDU Notification message to that SGSN. If the IMSI is not known in the SGSN the GGSN has to send an enquiry to the HLR.

-	If the MS is GPRS-detached (Cause value ‘MS GPRS Detached’), the SGSN returns a PDU Reject message to the GGSN with Cause equal to ‘MS GPRS Detached’. If the SGSN has a MM context for that user it sets MNRF to indicate the need to report to the HLR when the next contact with that MS is performed.

The End User Address information element contains the PDP type and PDP address of the PDP context that could not be activated.

Table 14: Information elements in a PDU Reject

| Information element | Reference |
| --- | --- |
| Cause |  |
| End User Address | 5.10.13 |

The TID of the PDU Reject message shall be the same as the TID of the PDU Notification that triggered the reject.

## Location Management messages

In the case where a GGSN does not have a SS7 MAP interface, GTP can be used to relay signalling messages between the GGSN and a GTP-MAP protocol-converting GSN in the GPRS backbone network. The MAP protocol describing these corresponding procedures and messages is described in GSM 09.02 [4]. This alternative method is illustrated in Figure 6.

Figure 6: GGSN - HLR Signalling  via a GTP-MAP protocol-converter in a GSN

This signalling interface can be used to support the Network-Requested PDP Context Activation procedure, which allows the GGSN to initiate the activation of a PDP context. When receiving a T-PDU the GGSN checks if a PDP Context is established for that PDP address. If no PDP context has been previously established the GGSN may try to deliver the T-PDU by initiating the Network-Requested PDP Context Activation procedure. The criteria used by the GGSN to determine whether trying to deliver the T-PDU to the MS may be based on subscription information and are outside the scope of GPRS standardisation.

To support Network-Requested PDP Context Activation the GGSN has to have static PDP information about the PDP address. To determine whether Network-Requested PDP Context Activation is supported for a PDP address, the GGSN checks if there is static PDP information for that PDP address.

Once these checks have been performed the GGSN may initiate the Network-Requested PDP Context Activation procedure.

The network operator may implement the following techniques to prevent unnecessary enquires to the HLR:

-	Implementation of the Mobile Detached Flag for GPRS (MDFG) technique in GGSN, SGSN, and HLR.

-	The GGSN may reject or discard T-PDUs after a previous unsuccessful delivery attempt. This systematic rejection of T-PDUs would be performed during a certain time after the unsuccessful delivery.

-	The GGSN may store the address of the SGSN with which the GGSN established the last PDP context. This would prevent an enquiry to the HLR. This SGSN address would be considered as valid during a certain time.

### Send Routeing Information for GPRS Request

When receiving a T-PDU for a MS where the corresponding PDP Context is not established, the GGSN determines if the Network-Requested PDP Context Activation procedure can be initiated. The GGSN may store the received T-PDU, as well as subsequent T-PDUs received for the same PDP address, during the activation procedure.

In order to obtain location information for the MS, the GGSN may send a Send Routeing Information for GPRS Request message intended for the HLR via a GTP-MAP protocol-converter in a GSN that has a SS7 MAP interface.

Table 15: Information elements in a Send Routeing Information for GPRS Request

| Information element | Reference |
| --- | --- |
| IMSI | 5.10.2 |

### Send Routeing Information for GPRS Response

As a response to the Send Routeing Information for GPRS Request message, the GSN interfacing to the HLR shall return a Send Routeing Information for GPRS Response message, via the same route, to the GGSN indicating the address of the SGSN that currently serves the MS.

The GSN Name information element contains the name of  the SGSN.

Table 16: Information elements in a Send Routeing Information for GPRS Response

| Information element | Reference |
| --- | --- |
| Cause |  |
| IMSI | 5.10.2 |
| GSN Name | 5.10.16 |

### Failure Report

When the GGSN receives a PDU Reject message it shall set MDFG for that PDP address and shall send a Failure Report message to the GSN interfacing to the HLR to request MDFG to be set in the HLR. The HLR shall set (if not already set) MDFG for the IMSI and shall add the GGSN address to the list of GGSNs to report to when activity from the MS, using the IMSI, is detected.

The GSN Name information element contains the name of  the GGSN.

Table 17: Information elements in a Failure Report

| Information element | Reference |
| --- | --- |
| IMSI | 5.10.2 |
| GSN Name | 5.10.16 |

### Failure Report Acknowledgement

The HLR shall acknowledge the receipt of a Failure Report message by returning a Failure Report Acknowledgement message to the GGSN. This message is relayed to the GGSN via a GSN interfacing to the HLR.

Table 18: Information elements in a Failure Report Acknowledgement

| Information element | Reference |
| --- | --- |
| IMSI | 5.10.2 |

### Note MS Present

When the HLR receives a message for a MS that has MDFG set, telling that it is present again, it shall clear MDFG for that MS and shall send a Note MS Present message to all the GGSNs in the list of the subscriber. When the GGSN receives the message, via a GSN interfacing to the HLR, it shall clear the MDFG.

Table 19: Information elements in a Note MS Present

| Information element | Reference |
| --- | --- |
| IMSI | 5.10.2 |

## Mobility Management messages

The Mobility Management messages are the signalling messages, defined in GSM 03.60, that are used at the GPRS Attach and Inter SGSN Routeing Update procedures. The new SGSN derives the address of the old SGSN from the TLLI and old routeing area identity. The address translation mechanism is implementation specific. Some possible translation mechanisms are found in Annex A.

Generally, the purpose of the signalling is to transfer data associated with the MS from the old SGSN to the new SGSN.

The SGSN Context Request/Response shall be used to transfer MM and PDP Context data from the old SGSN to the new SGSN.

### Identification Request

If the MS, at GPRS Attach,  identifies itself with TLLI and it has changed SGSN since detach, the new SGSN shall send an Identification Request message to the old SGSN to request the IMSI.

The TLLI and RAI is a TLLI and an RAI in the old SGSN.

Table 20: Information elements in an Identification Request

| Information element | Reference |
| --- | --- |
| Routeing Area Identity (RAI) | 5.10.3 |
| Temporary Logical Link Identifier (TLLI) | 5.10.4 |

### Identification Response

As a response to an incoming Identification Request message, the SGSN shall respond to the requesting SGSN with an Identification Response message. The response shall contain the requested IMSI and may also contain a number of authentication triplets.

The Authentication Triplet information element is repeated if a set of triplets is sent in the response.

Table 21: Information elements in an Identification Response

| Information element | Reference |
| --- | --- |
| IMSI | 5.10.2 |
| No of Triplets | 5.10.9 |
| Authentication Triplet (optional) | 5.10.10 |

### SGSN Context Request

The new SGSN shall send an SGSN Context Request to the old SGSN to get the MM and PDP Contexts for the MS. The MS is identified by its old RAI and old TLLI values. The TLLI and RAI is a TLLI and an RAI in the old SGSN.

The old SGSN responds with an SGSN Context Response.

The BB Protocol field states if UDP or TCP shall be requested by the new SGSN to tunnel end user data packets.

Table 22: Information elements in a SGSN Context Request

| Information element | Reference |
| --- | --- |
| Routeing Area Identity (RAI) | 5.10.3 |
| Temporary Logical Link Identifier (TLLI) | 5.10.4 |
| BB Protocol | 5.10.7 |

### SGSN Context Response

The old SGSN shall send an SGSN Context Response to the new SGSN as a response to a previous SGSN Context Request. The message contains the IMSI of the MS as well as  the MM and PDP Contexts stored at the old SGSN. The number of PDP Contexts transferred in the message is also included.

sh

Table 23: Information elements in a SGSN Context Response

| Information element | Reference |
| --- | --- |
| Cause |  |
| IMSI | 5.10.2 |
| No of PDP Contexts | 5.10.5 |
| Receive State Variables | 5.10.11 |
| MM Context | 5.10.14 |
| PDP Context | 5.10.15 |

## Reliable delivery of signalling messages

Each path maintains a queue with signalling messages to be sent to the peer. The message at the front of the queue shall be sent with the current Transaction ID, and shall be held until a response it received. The Transaction ID for the path shall then be incremented and shall wrap to 0 from the maximum value. A GSN may have several outstanding requests while waiting for responses.

A signalling message request or response has probably been lost if a response has not been received within the T3-RESPONSE time. The request is then retransmitted if the total number of request attempts is less than N3-REQUESTS times. The timer shall be implemented in the signalling client application as UDP lacks time supervision. A retransmitted request has the same Transaction ID as the first request. The wait time for a response (T3-RESPONSE) and the number of retries (N3-REQUESTS) shall be configurable.  The total wait time, i.e. the number of retries times the wait time for a response, shall be shorter than the MS wait time between retries of Attach and RA Update messages.

A duplicated request resulting from retransmission shall be discarded and not handled as in the normal case, but a new response shall be sent because the previous response may have been lost. Duplicated response messages can simply be discarded.

## Transmission order

The messages in this document shall be transmitted starting with bit 8, most significant bit, of octet 1.

## Information elements

A signalling message may contain several information elements. The TLV (Type, Length, Value) or TV (Type, Value) encoding format shall be used for the GTP information elements. The information elements are sorted, with the Type fields in ascending order, in the signalling messages. The Length field contains the length of the information element including the Type and Length field.

The most significant bit in the Type field is set to 0 when the TV format is used and set to 1 for the TLV format.

Figure 8: Type field for TV and TLV format

Note:	If a dynamic address is requested, the length field shall be set to 3.

Figure

The Protocol Address shall be the address that the MS is identified with from an external packet data network.

Figure

Figure

Note:	Digit 1 contains the first BCD coded digit of the X.121 address.

Figure

Cause

The Cause shall be an indication of the acceptance of a request and shall be included in a response message.

Figure 13: Cause information element

Table 25: Cause values

| Cause | Value |
| --- | --- |
| Request accepted | 0 |
| Non-existent | 1 |
| Invalid message format | 2 |
| IMSI not known | 3 |
| MS is GPRS Detached | 4 |
| MS is not GPRS Responding | 5 |
| MS Refuses | 6 |
| Version not supported | 7 |
| Request IMSI | 8 |
| Request IMEI | 9 |
| No identity needed | 10 |
| No resources available | 11 |
| Requested address not supported | 12 |
| All other values are reserved |  |

### International Mobile Subscriber Identity (IMSI)

The IMSI shall be the subscriber identity of the MS.

Figure 15: IMSI information element

### Routeing Area Identity (RAI)

The former (old) RAI of where the MS were located before the change of routeing area.

Figure 16: RAI information element

### Temporary Logical Link Identity (TLLI)

The TLLI associated with the MS in the old routeing area.

Figure 17: TLLI information element

### No. of PDP Contexts

The number of PDP Contexts, stored at the old SGSN and transferred from there to the new SGSN, for a specific MS.

Figure 18: No. of PDP Contexts information element

### Quality of Service (QoS) Profile

The Quality of Service (QoS) Profile shall include the values of the defined QoS parameters The conteent and the coding of the QoS Profile is FFS.

Figure 19: Quality of Service (QoS) Profile information element

Table 26: Peak Bitrate values

| Peak Bitrate | Value |
| --- | --- |
| All other values are reserved |  |

Table 27: Mean Bitrate values

| Mean Bitrate | Value |
| --- | --- |
| All other values are reserved |  |

Table 28: Delay Class values

| Delay Class | Value |
| --- | --- |
| All other values are reserved |  |

Table 29: Ack/Unack values

| Ack/Unack | Value |
| --- | --- |
| All other values are reserved |  |

### BB Protocol

The BB Protocol describes the bearer that shall be used to carry G-PDUs in the tunnel.

Figure 20: BB Protocol information element

Table 30: BB Protocol values

| BB Protocol | Value |
| --- | --- |
| UDP | 0 |
| TCP | 1 |
| All other values are reserved |  |

### Reordering Required

The Reordering Required information element states whether reordeing by GTP is required or not.

Figure 21: Reordering Required information element

Table 31: Reordering Required values

| Reordeiring required | Value |
| --- | --- |
| No | 0 |
| Yes | 1 |
| All other values are reserved |  |

### No. of Triplets

The number of Authentication Triplets, stored at the old SGSN and transferred from there to the new SGSN, for a specific MS.

Figure 22: No. of Triplets information element

### Authentication Triplet

An Authentication triplet consists of a random string (RAND), a signed response (SRES) and a ciphering key (Kc).

Figure 23: Authentication Triplet information element

### Receive State Variables

The Receive State Variables are the V(R) variables from the LLC in the old SGSN. The V(R) variables may be defined for the logical links in asynchronous balanced mode i.e. SAPI 3, 5, 9 and 11 as defined in GSM 04.64. The V(R) is a value between 0 and 63. The Not Used (NU) value shall be set to 1 and the corresponding V(R) value shall be set to all ones by the old SGSN if the SAPI is not used.

Figure 24: Receive State Variables information element

### GTP Path MTU

The GTP Path MTU is used to fill a Path MTU request with data to find out the path MTU , includ two corresponding GSNs. The value should be null octets. The length of the GTP Path MTU should be the first hop MTU minus the length of the backbone IP header minus the length of the UDP header minus the length of the Common Header minus one octet for the Type value.

Example:

Assume that the first hop is Ethernet with the MTU value 1500 octets, the backbone uses an IPv4 header without options (i.e. 20 octets), the UDP header with 8 octets, the Common Header with 14 octets and 1 octet for the Type value. Then the GTP Path MTU should consist of  1457 (1500-20-8-14-1) octets with null octets.

When a Path MTU Response is received this value can be used to calculate the Path MTU value for the tunnel. The sending GSN now knows that it can send at least 1500 octets without backbone fragmentation.

The IP header, UDP or TCP header and the GTP header will reduce the payload that can be tunnelled.

The path MTU for the GTP/UDP/IP tunnel will in this example be 1464  (1500-20-8-8) octets. The path MTU for the GTP/TCP/IP tunnel will be 1452 (1500-20-20-8) octets. If the MS in this case sends IP datagrams that are larger than 1464 or 1452 octets the SGSN must fragment the datagrams or discard them and return an ICMP message.

Figure 25: GTP Path MTU information element

### End User Address

The purpose of the End User Address information element shall be to supply protocol specific information of the external packet data network accessed by the GPRS subscriber.

If the MS requests a dynamic PDP address and a dynamic PDP address is allowed, then the Protocol Address field in the End User Address information element shall not be present and the length field shall be set to 4. If the MS requests a static PDP address then the Protocol  Address field in the End User Address information shall contain the static PDP Address in the Create PDP Context Request message.

The Protocol Address field in the End User Address information element in the Create PDP Context Response message will contain the dynamic PDP Address allocated  by the GGSN, if the Protocol Address field in the End User Address information element in the Create PDP Context Request contains the value zero.

Figure 26: End User Address information element

In the table below protocol address type values and their respective length are defined.

Table 32: Relations between Address Type and Protocol Address length

| Value | Address Type | Protocol Address length |
| --- | --- | --- |
| 0 | IPv4 | 4 octets |
| 1 | IPv6 | 16 octets |
| 2 | X.121 | 1 - 14 digits |

The Protocol Address shall be the address that the MS is identified with from an external packet data network.

Figure 27: Protocol information element for IPv4

Figure 28: Protocol information element for IPv6

Note:	Digit 1 contains the first BCD coded digit of the X.121 address. If the X.121 address has an odd number of digits, the last BCD digit shall be padded with HEX(F).

Figure 29: Protocol information element for X.25

### MM Context

The MM Context information element contains the Mobility Management; MS and security parameters that are necessary to transfer between SGSNs at the Inter SGSN Routeing Update procedure.

[FFS: This is a preliminary version. The DRX parameters are based on the expected values in GSM 03.64. The Classmark parameters areisased on GSM 03.60 and GSM 04.08. Classmark 2 and Classmark 3 parameters have been included. Revision level, ES IND, PS capability, SS Screening Indicator, SM capability, VBS notification, VGCS notification and Frequence Capability have been excluded since they are considered not to be applicable for GPRS. SM capability is not needed since MT SMS is mandatory in GPRS. Frequence capability seems to be included in the Multiband Support parameter(??).

The following parameters defined for the SGSN MM Context in GSM 03.60 have been excluded:

VLR Address, New SGSN Address, TLLI, Routeing Area, Cell Identity, Compression and Recovery.

The TLLI, Routeing Area and Cell Identity values can be excluded because the values are already in the new SGSN provided by the Routeing Area Update Request that caused the transfer between old to new SGSN.

The New SGSN Address is only applicable in the old SGSN.

The VLR Address can be based on a table mapping of the new LAI (from RAI) to the VLR address.

The Recovery parameter seems to be redundant. It is vaguely defined in 03.60 but our interpretation is that the VLR or HLR shall be updated at the next Routeing Area Update regardless of the type of the Routeing Area Update. Since the HLR will be updated anyhow at the Inter SGSN Routeing Area Update the flag is not necessary for HLR update. The VLR will be updated if the MS is IMSI attached and not otherwise. That is not affected by a Recovery flag.

The Compression will be re-negotiated as part of the link setup to the new SGSN.

The cipher algorithm is called A6 in the description below in order to separate it from the A5 algorithm.]

The IMSI is encoded as the value in the IMSI information element.

The DRX flag indicates if the MS uses DRX or not.

The Mobile Not Reachable (MNR) flag indicates if the new SGSN shall send MAP-READY-FOR-SM or not when the SGSN Context Response has been received.

The usage of the Ciphering Key Sequence Number (CKSN) is described in GSM 04.08. Possible values are 0 to 6. The value 7 is reserved.

The DRX Period is the SPLIT_PG_CYCLE defined in GSM 03.64.

The Non-DRX Timer is a conditional parameter for MSs defined in GSM 03.64.  The value is between 0 - 254 seconds and the value 255 means that the function is not used.

State Time-to-Live is the number of seconds left for the SMM state timer running for the SMM-STANDBY or SMM-READY state. The number of seconds is a value between 0 - (2^31-1). The value 0 means that the SMM state timer shall be restarted with the initial value. The value HEX(FFFFFFFF) means that no SMM state timer shall be started.

The P, E and D bits are set to 1 if the MS supports P-GSM, E-GSM or DCS 1800 respectively. The P-GSM and E-GSM are mutually exclusive.

The RF 1800 is valid if the D bit is set and then contains the RF power capability for the DCS 1800 band. The RF 1800 value is a value between 1 and 3.

The RF 900 is valid if the P or E bit is set and then contains the RF power capability for the GSM 900 band. The RF 900 value is a value between 1 and 5.

Version is the GPRS version that the MS supports. The value 0 is the GPRS version 1.

No of Triplets is the number of authentication and ciphering triplets included in the Triplet array.

Kc is the ciphering key currently used by the old SGSN.

The A6/7 to A6/1 bits indicate which algorithms the MS supports and a specific ciphering algorithm is supported by the MS if the corresponding bit is set to 1.

The Triplet array contains triplets encoded as the value in the Authentication Triplet information element.

Figure 30: MM Context element

Table 33: SMM State values

| SMM State | Value |
| --- | --- |
| SMM-IDLE | 0 |
| SMM-STANDBY | 1 |
| SMM-READY | 2 |
| Value 3 is reserved |  |

Table 34: DRX  vales

| DRX | Value |
| --- | --- |
| Not in use | 0 |
| In use | 1 |

Table 35: MNR values

| MNR | Value |
| --- | --- |
| Don’t send MAP-READY-FOR-SM | 0 |
| Send MAP-READY-FOR-SM | 1 |

Table 36: MS Class values

| MS Class | Value |
| --- | --- |
| C | 0 |
| B | 1 |
| A | 2 |
| Value 3 is reserved |  |

Table 37: Used Cipher values

| Cipher Algorithm | Value |
| --- | --- |
| No ciphering | 0 |
| A6/1 | 1 |
| A6/2 | 2 |
| A6/3 | 3 |
| A6/4 | 4 |
| A6/5 | 5 |
| A6/6 | 6 |
| A6/7 | 7 |

Table 38: Multislot Capability values

| Number of slots | Value |
| --- | --- |
| 1 | 0 |
| 2 | 1 |
| 3 | 2 |
| 4 | 3 |
| 5 | 4 |
| 6 | 5 |
| 7 | 6 |
| 8 | 7 |

### PDP Context

The PDP Context information element contains the Session Management parameters, defined for an external packet data network address, that are necessary to transfer between SGSNs at the Inter SGSN Routeing Update procedure.

Bit 8 of octet 2 is the most significant bit and bit 1 of octet 3 is the least significant bit of the Length field.

NSAPI is a value between 0 and 15.

Reordering Required (Order) indicates whether the SGSN shall reorder T-PDUs before delivering the T-PDUs to the MS.

Dynamic Address Allowed (DAA) indicates whether a static or dynamic PDP address shall be used.

VPLMN Address Allowed (VAA) indicates whether a dynamic PDP address is allowed or not in a VPLMN. This flag shall be set to No if DAA indicates that a static PDP address shall be used.

Quality of Service Subscribed (QoS Sub), Quality of Service Requested (QoS Req) and Quality of Service Negotiated (QoS Neg) are encoded as described in Table 43.

The Sequence Number Down is the number of the next T-PDU that shall be sent from the new SGSN to the MS. The number is associated to the Sequence Number from the GTP Header of an encapsulated T-PDU.

The Sequence Number Up is the number that new SGSN shall use as the Sequence Number in the GTP Header for the next encapsulated T-PDU from the MS to the GGSN.

The GGSN Address is the 4 octet IPv4 address or the 16 octet IPv6 address that the old SGSN has cached after a name-to-address conversion. The GGSN Address is assigned when the PDP context is activated and shall not be changed as long as the PDP contexts is activated.

The GGSN Name is a domain name of the GGSN to be contacted by the SGSN. A domain name consists of labels separated by dots (e.g. ggsn123.operator.gprs.org). A label can be between 1 and 63 octets. The sum of all labels including the dots shall not exceed 255 octets. See also the Name Syntax in RFC 2181.

Figure 31: PDP Context information element

Table 39: State values

| State | Value |
| --- | --- |
| Inactive | 0 |
| Active | 1 |

Table 40: Reordering Required values

| Reordering Required | Value |
| --- | --- |
| No | 0 |
| Yes | 1 |

Table 41: Dynamic Address Allowed values

| Dynamic Address Allowed | Value |
| --- | --- |
| No | 0 |
| Yes | 1 |

Table 42: VPLMN Address Allowed values

| VPLMN Address Allowed | Value |
| --- | --- |
| No | 0 |
| Yes | 1 |

Table 43: QoS Profile values [FFS]

| QoS | Value |
| --- | --- |

### GSN Name

The GSN Name information element contains the Node Name with a name of a GSN.

The Node Name is in the first version of GPRS a domain name which consists of labels separated by dots (e.g. ggsn123.operator.gprs.org). A label can be between 1 and 63 octets. The sum of all labels including the dots shall not exceed 255 octets. See also the Name Syntax in RFC 2181.

Figure 32: GSN Name information element

# Transmission Plane

Tunnels are used to carry encapsulated T-PDUs between a given GSN pair for individual MSs. The key Tunnel ID (TID) which is present in the GTP header shall indicate which tunnel a particular T-PDU belongs to. In this manner, packets are multiplexed and demultiplexed by GTP between a given GSN-GSN pair.  The Tunnel ID value to use in the key field shall be established by the Create PDP Context establishment procedure which takes place on the signalling plane.

## Protocol Stack

The GTP protocol carries T-PDUs through the GPRS backbone. T-PDUs are carried in a tunnel between GSN pairs, encapsulated in G-PDUs. A G-PDU is a packet with a GTP header and a T-PDU. The Path Protocol defines the path and the GTP header defines the tunnel. Several tunnels can be multiplexed on a single path. The frames have the following general structure:

Figure 33: Transmission Plane - Protocol Stack

### .].

### GTP Transmission Header

The format of the GTP transmission header shall be as follows:

Figure 34: GTP Transmission Header format

- Version shall be 0, indicating the first version of the GTP protocol.

- LFN flag: The GTP header includes the optional Frame Number field if the F flag is set to 1.

- reserved: Reserved bits that shall be set to 1.

- Length: Size of the T-PDU including the GTP header size.

- Sequence Number: The sending GSN uses the value 0 for the first T-PDU in a tunnel and increments the Sequence Number for each following T-PDU. The value wraps to 0 after 255. The receiving GSN shall reorder the incoming T-PDUs in sequence if the Reordering Required flag in the PDP context is set.

- LLC Frame Number: The optional LLC Frame Number may be used by the old SGSN, at the Inter SGSN Routeing Update procedure, to inform the new SGSN of the number of the LLC frame which transferred the last segment of the T-PDU. The LLC Frame Number shall be set to HEX(FF) if the LFN flag is 0.

- TID: Contains the tunnel identifier for the tunnel to which this T-PDU belongs. The TID shall be used by the receiving GSN to find the MM and PDP contexts.

- TCP transmission

Each T-PDU shall be encapsulated with one GTP header, and put into one TCP segment, and one IP packet. If the resulting IP packet is too long for the backbone MTU, standard IP fragmenting shall be used, see section 11.1. Note that TCP contains the possibility to use smaller segments instead of using IP fragmentation, but this cannot be used since GTP doesn’t contain any mechanism to reassemble more than one TCP segment to construct the original packet.

## Tunnelling between SGSNs

T-PDUs, stored in the old SGSN and not yet sent to the MS, are tunnelled to the new SGSN as a part of the Inter SGSN Routeing Update procedure described in GSM 03.60. Some T-PDUs may still be on their way from the GGSN to the old SGSN because they have been sent before the tunnels change. These T-PDUs will also be tunnelled to the new SGSN.

## Tunnelling between GGSNs

It is not expected that GTP tunnelling between GGSNs should be specified. MS-to-MS traffic between GGSNs should use the  Gi interface [FFS].

# Path Protocols

[FFS: UDP/TCP ports for the GTP shall be requested from Internet Assigned Number Authority (IANA) by ETSI. Who in ETSI is responsible for this task?]

## UDP/IP

UDP/IP is the only path protocol defined to transfer GTP signalling messages in the first version of GTP. UDP/IP is also the recommended choice to tunnel connection-less T-PDUs.

### UDP Header

#### Signalling request messages

The UDP Destination Port shall be the server port number MMMM [FFS] that is reserved for GTP signalling messages. Extra implementation specific destination ports are possible but all GSNs shall support the server port number MMMM.

The UDP Source Port is a random port, locally assigned at the sending GSN.

#### Signalling response messages

The UDP Destination Port shall be copied from the Source Port of the corresponding request message.

The UDP Source Port shall be copied from the Destination Port of the corresponding request message.

#### Encapsulated T-PDUs

The UDP Destination Port shall be the server port number NNNN [FFS] that is reserved for G-PDUs. Extra implementation specific destination ports are possible but all GSNs shall support the server port number NNNN.

The UDP Source Port is a random port, locally assigned at the sending GSN.

### IP Header

#### Signalling request messages and Encapsulated T-PDUs

The IP Source Address shall be an IP address of the source GSN from which the message is originating.

The IP Destination Address in a GTP signalling request message shall be an IP address of the destination GSN.

#### Signalling response messages

The IP Source Address shall be copied from the IP Destination Address of the corresponding  request message.

The IP Destination Address shall be copied from the IP Source Address of the GTP signalling request message to which this GSN is replying to.

## TCP/IP

TCP/IP is the recommended choice to tunnel connection-oriented T-PDUs.

### TCP Header

The TCP Destination Port shall be the server port number NNNN [FFS] that is reserved for G-PDUs. Extra implementation specific destination ports are possible but all GSNs shall support the server port number NNNN.

The TCP Source Port is a random port, locally assigned at the sending GSN.

### IP Header

The IP Source Address shall be an IP address of the source GSN from which the message is originating.

The IP Destination Address in a GTP signalling request message shall be an IP address of the destination GSN.

State transition diagrams

## General

This chapter deals with handling of normal cases as well as the possible exception cases and error cases in GTP.

The exception handling is presented as state machines in table form. There are five tables:

- GTP session management in an SGSN

- GTP session management for anonymous access in an SGSN

- GTP session management in a GGSN

- GTP session management for anonymous access in an GGSN

- GTP mobility management in an SGSN

Figure

Figure

Figure

Figure

C1: Inter SGSN Routeing Update

The diagram below indicates a pre-condition for an Update PDP Context Response. The Inter SGSN Routeing Update procedure is described in GSM 03.60.

Figure 39: Update PDP Context Response at Inter SGSN Routeing Update

C2: PDP Context Modification

The diagram below indicates another pre-condition for an Update PDP Context Response. The PDP Context Modification procedure is described in GSM 03.60.

Figure 40: Update PDP Context Response at PDP Context Modification

State transition tables

The state transition tables in the following sections have the same format. States are indicated horizontally in the first row and events are read vertically in the first column.

State transitions and actions are represented in the format action, new state.

The notation – means that the event/state combination is not expected.

## GTP services

The GTP layer in SGSN provides general services that are used by the relaying and routeing function. The general GTP services are used as specific events and actions in the following state transition tables.

Figure 44: Funtional relaying and routeing model in SGSN

The services provided by GTP are:

- GT-PDP-ACTIVATE.request: Used to  request the creation of  a specific PDP context in a remote GGSN.

- GT-PDP-ACTIVATE.confirm: Acknowledgement of the outcome of a PDP context creation in the GGSN.

- GT-PDP-MODIFY.request: Used to request the change the SGSN address and/or QoS in a PDP context in a remote GGSN.

- GT-PDP-MODIFY.confirm: Acknowledgement of the outcome of the change of a PDP context.

- GT-PDP-DEACTIVATE.request:  Requests deletion of  the context in a GGSN.

- GT-PDP-DEACTIVATE.confirm: Acknowledgement of the outcome of the deletion of a PDP context.

- GT-PDP-DEACTIVATE.indication: Indicates to the relaying and routeing function that the specified context shall be deactivated.

- GT-AA-PDP-ACTIVATE.request: Requests GTP to create an anonymous PDP context in a GGSN.

- GT-AA-PDP-ACTIVATE.confirm: Acknowledgement of the outcome of the creation of an anonymous PDP context.

- GT-AA-PDP-DEACTIVATE.request: Requests GTP to delete an anonymous PDP context in a GGSN.

- GT-AA-PDP-DEACTIVATE.confirm: Acknowledgement of the outcome of the deletion of an anonymous PDP context.

- GT-IDENTIFICATION.indication: Requests GTP to get information from an old SGSN.

- GT-IDENTIFICATION.confirm: Acknowledgement of the outcome of the information retrieval from an old SGSN.

- GT-DETACH.request:  SMM indicates that an MS has detached from GPRS and requests GTP to delete all active PDP contexts in the GGSNs. ????????????

- GT-GET-CONTEXTS.request: Requests GTP to get the contextsinformatiold SGSN.

- GT-GET-CONTEXTS.confirm: Acknowledgement of the outcome of the contextsinformatil from an old SGSN

- GT-CANCEL.indication: Indication from the relaying and routeing function that the MM context has been cancelled from the HLR or an indication from GTP that the time limit for PDU forwarding has expired and the relaying and routeing function may delete the MM and PDP contexts.

- GT-UNITDATA.indication: The relaying and routeing function provides a PDU received from another protocol stack to be transmitted to another GSN or GTP provides a decapsulated PDU from a tunnel interface.

It is assumed in the state transition diagrams that the relaying and routeing function has the main responsibility to keep track of SMM, MAP and GTP signalling and to maintain the MM and PDP contexts based on the result of the SMM, MAP and GTP operations.

SGSN, PDP Context

Table 44: State transition diagram for a PDP Context in SGSN

|  | State |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Event | S11 Inactive | S12 Activating | S13 Updating | S14 Inactivating | S15 Active |
| GT-PDP-ACTIVATE.request | A102, S12 | -- | -- | -- | A102, S12 |
| GT-PDP-MODIFY.request | -- | -- | -- | -- | A103, S13 |
| GT-PDP-DEACTIVATE.request | A115, S11 | -- | -- | -- | A104, S14 |
| GT-DETACH.request | A115, S11 | A104, S14 | A104, S14 | A101, S14 | A104, S11 |
| ACK | A101, S11 | A105, S15 | A106, S15 | A107,S11 | -- |
| NACK | A101, S11 | A108, S11 | C1:A109,S11 C2:A109,S15 | A107, S11 | -- |
| T3-RESPONSE timeout | -- | A102, S12 | A103, S13 | A104, S14 | -- |
| N3-REQUESTS | -- | A108, S11 | C1:A109,S11 C2:A109,S15 | A107, S101 | -- |
| Error Indication | -- | A101, S12 | A101, S13 | A101, S14 | A102, S12 |
| GT-UNITDATA.indication | -- | -- | -- | -- | A111, S15 |
| G-PDU | A110, S11 | A114, S12 | A114, S13 | A113, S14 | A112, S15 |

Table 45: States for a PDP Context in SGSN

| States | Description |
| --- | --- |
| S11 Inactive | An inactivated (i.e. not valid) PDP context exist in the SGSN for the given ID. |
| S12 Activating | A Create PDP Context Request has been sent to the GGSN associated with the context with the given ID to create a tunnel. The SGSN has not received any answer yet. |
| S13 Updating | An Update PDP Context Request has been sent to the GGSN to update the current location. The SGSN has not received any answer yet. |
| S14 Inactivating | A Delete PDP Context Request has been sent to the GGSN to delete the tunnel associated with the Tunnel ID. The SGSN has not received any answer yet. |
| S15 Active | An activated PDP context exist in the SGSN for the given ID. |

Table 46: Events for a PDP Context in SGSN

| Events | Description |
| --- | --- |
| GT-PDP-ACTIVATE.request | The relaying and routing function should trigger this event when an Activate PDP Context Request message from a MS has been received., |
| GT-PDP-MODIFY.request | The relaying and routing function should trigger this event when a Modify PDP Context Accept from the MS ar a Routeing Area Update Request from the MS has been received.as part of the Inter SGSN |
| GT-PDP-DEACTIVATE.request | The relaying and routing function should trigger this event when a Deactivate PDP Context Request from the MS has been received |
| ACK | The Create/Update/Delete PDP Context Response from GGSN, containing valid parameters values, arrives in time. The Cause is set to Request Accepted. |
| NACK | The Create/Update/Delete PDP Context Response has a Cause differing from Request Accepted or has a parameter value out of bounds. |
| T3-RESPONSE timeout | A GTP signalling request message has been sent and no answer has been received within the time limit defined by the timer T3-RESPONSE. |
| N3-REQUESTS | The maximum number of GTP signalling request messages have been sent. |
| Error Indication | The GGSN has lost its PDP Context and sent an Error Indication to the SGSN. |
| GT-UNITDATA.indication | A PDU, sent from the MS, arrives at the SGSN. |
| G-PDU | A G-PDU, sent from a GGSN or another SGSN and addressed to the MS, arrives at the SGSN. |

Table 47: Actions for a PDP Context in SGSN

| Action | Description |
| --- | --- |
| A101 | - Ignore. |
| - A102 | - PDP context.Map the GGSN name from the PDP context to an IP address. - Send a Create PDP Context Request to the GGSN. - Start timer T-RESPONSE. |
| - A103 | - Get the GGSN IP address from the PDP context. - Send an Update PDP Context Request to the GGSN. - Start timer T-RESPONSE. |
| - A104 | - Get the GGSN IP address from the PDP context. - Send a Delete PDP Context Request to the GGSN. - Start timer T-RESPONSE |
| - A105 | - Stop timer T-RESPONSE. - Issue GT-PDP-ACTIVATE.confirm(SUCCESSFUL). |
| - A106 | - Stop timer T-RESPONSE. - Issue GT-PDP-MODIFY.confirm(SUCCESSFUL). |
| - A107 | - Stop timer T-RESPONSE. - Issue GT-PDP-DEACTIVATE.confirm. |
| - A108 | - Stop timer T-RESPONSE. - Issue GT-PDP-ACTIVATE.confirm(FAILURE). |
| - A109 | - Stop timer T-RESPONSE. - Issue GT-PDP-MODIFY.confirm(FAILURE). |
|  | - GT-PDP-DEACTIVATE.confirm(FAILURE). |
| - A110 | - Send an Error Indication to the GGSN. - Issue GT-PDP-DEACTIVATE.indication |
| - A111 | - Get the GGSN IP address from the PDP context. - Encapsulate the PDU with the GTP header. - Send the encapsulated PDU to the selected GGSN. |
| - A112 | - Decapsulate the PDU. - Issue GT-UNITDATA.indication. |
| - A113 | - Discard the G-PDU. |
| - A114 | - Store the G-PDU until the the Activate or Routin procedures are completed. |

### SGSN, PDP Context for anonymous access

Table 48: State transition diagram for an anonymous PDP Context in SGSN

|  | State |  |  |  |
| --- | --- | --- | --- | --- |
| Event | S21 Inactive | S22 Activating | S23 Inactivating | S24 Active |
| GT-AA-PDP-ACTIVATE.request | A202, S22 | -- | -- | A202, S22 |
| GT-AA-PDP-DEACTIVATE.request | A201, S21 | -- | -- | A203, S23 |
| ACK | A201, S21 | A204, S24 | A205, S21 | -- |
| NACK | A201, S21 | A206, S21 | A205, S21 | -- |
| T3-RESPONSE timeout | -- | A202, S22 | A203, S23 | -- |
| N3-REQUESTS | -- | A206, S21 | A205, S21 | -- |
| Error Indication | -- | A201, S22 | A201, S23 | A207, S22 |
| GT-UNITDATA.indication | -- | -- | -- | A209, S24 |
| T-PDU | A208, S21 | A212, S22 | A211, S23 | A210, S24 |

Table 49: States for an anonymous PDP Context in SGSN

| States | Description |
| --- | --- |
| S21 None | No anonymous PDP context exists in the SGSN for the given ID. |
| S22 Activating | A Create AA PDP Context Request has been sent to the GGSN associated with the anonymous server and the context with the given ID to create a tunnel. The SGSN has not received any answer yet. |
| S23 Inactivating | A Delete AA PDP Context Request has been sent to the GGSN to delete the tunnel associated with the Tunnel ID. The SGSN has not received any answer yet. |
| S24 Active | An activated PDP context exist in the SGSN for the given ID. |

Table 50: Events for an anonymous PDP Context in SGSN

| Events | Description |
| --- | --- |
| GT-AA-PDP-ACTIVATE.request | An Activate AA PDP Context Request message from a MS has been received by the SMM. |
| GT-AA-PDP-DEACTIVATE.request | The READY timer in SMM has expired and SMM indicates that the PDP context shall be deleted. |
| ACK | The Create/Delete AA PDP Context Response from GGSN, containing valid parameters values, arrives in time. The Cause is set to Request Accepted. |
| NACK | The Create/ Delete AA PDP Context Response has a Cause differing from Request Accepted or has a parameter value out of bounds. |
| T3-RESPONSE timeout | A GTP signalling request message has been sent and no answer has been received within the time limit defined by the timer T3-RESPONSE. |
| N3-REQUESTS | The maximum number of GTP signalling request messages have been sent. |
| Error Indication | The GGSN has lost its PDP Context and sent an Error Indication to the SGSN. |
| GT-UNITDATA.indication | A T-PDU, sent from the MS, arrives at the SGSN. |
| G-PDU | A G-PDU, sent from a GGSN or another SGSN and addressed to the MS, arrives at the SGSN. |

Table 51: Actions for an anonymous PDP Context in SGSN

| Action | Description |
| --- | --- |
| A201 | - Ignore. |
| - A202 | - ????????Map the GGSN name from the anonymous PDP context to an IP address. ?????? - Send a Create AA PDP Context Request to the GGSN. - Start timer T-RESPONSE. |
| - A203 | - Get the GGSN IP address from the anonymous PDP context. - Send a Delete AA PDP Context Request to the GGSN. - Start timer T-RESPONSE |
| - A204 | - Stop timer T-RESPONSE. - Issue GT-AA-PDP-ACTIVATE.confirm(SUCCESSFUL). |
| - A205 | - Stop timer T-RESPONSE. - Issue GT-AA-PDP-DEACTIVATE.confirm. |
| - A206 | - Stop timer T-RESPONSE. - Issue GT-AA-PDP-ACTIVATE.confirm(FAILURE). |
| - A207 | - GT-AA-PDP-DEACTIVATE.confirm(FAILURE). - Get thDP context. - Send a Create AA PDP Context Request to the GGSN. - Start timer T-RESPONSE. |
| - A208 | - Send an Error Indication to the GGSN. |
| - A209 | - Get the GGSN IP address from the anonymous PDP context. - Encapsulate the PDU with the GTP header. - Send the encapsulated PDU to the selected GGSN. |
| - A210 | - Decapsulate the PDU. - Issue GT-UNITDATA.indication. |
| - A211 | - Discard the G-PDU. |
| - A212 | - Store the G-PDU until the the Activation procedure is completed. |

### GGSN, PDP Context

Table 52: State transition diagram for a PDP Context in GGSN

|  | State |  |
| --- | --- | --- |
| Event | S31 None | S32 Active |
| Create PDP Context Request | A301, S32 | A302, S32 |
| Update PDP Context Request | A303, S31 | A304, S32 |
| Delete PDP Context Request | A305, S31 | A306, S31 |
| Invalid Create Request | A307, S31 | A308, S31 |
| Invalid Update Request | A309, S31 | A310, S31 |
| Error Indication | -- | A312, S31 |
| G-PDU | A311, S31 | A313, S32 |
| PDU | A314, S31 | A315, S32 |

Table 53: States for a PDP Context in GGSN

| States | Description |
| --- | --- |
| S31 None | - No PDP context exists in the GGSN for the TID in a GTP message - No PDP context exists in the GGSN for the network address or connection identifier in a PDU from an external network. |
| - S32 Active | - An activated PDP context exists in the GGSN for the TID in a GTP message - An activated PDP context exists in the GGSN for the network address or connection identifier in a PDU from an external network. |

Table 54: Events for a PDP Context in GGSN

| Events | Description |
| --- | --- |
| Create PDP Context Request | A Create PDP Context Request with valid information elements is received. |
| Update PDP Context Request | An Update PDP Context Request with valid information elements is received. |
| Delete PDP Context Request | A Delete PDP Context Request with valid information elements is received. |
| Invalid Create Request | A received Create PDP Context Request contains a missing mandatory or an invalid information element. |
| Invalid Update Request | A received Update PDP Context Request contains a missing mandatory or an invalid information element. |
| Error Indication | The SGSN has lost its PDP Context and sent an Error Indication to the GGSN. |
| G-PDU | A G-PDU, sent from the MS via SGSN, has arrived at the GGSN. |
| PDU | A PDU has been received on an interface to an external packet data network. |

Table 55: Actions for a PDP Context in GGSN

| Action | Description |
| --- | --- |
| A301 | - Create a new PDP context. - Insert the values from the information elements of the Create PDP Context Request in the new context. - Send a Create PDP Context Response with Cause set to Request accepted. |
| - A302 | - Replace the values in the existing PDP context with the values from the information elements of the Create PDP Context Request. - Send a Create PDP Context Response with Cause set to Request accepted. |
| - A303 | - Send an Update PDP Context Response with Cause set to Non-existent. |
| - A304 | - Update the PDP context with the values from the information elements of the Update PDP Context Request. - Send an Update PDP Context Response with Cause set to Request accepted. |
| - A305 | - Send a Delete PDP Context Response with Cause set to Request accepted. |
| - A306 | - Send a Delete PDP Context Response with Cause set to Request accepted. - Delete the PDP context. |
| - A307 | - Send a Create PDP Context Response with Cause set to Invalid message format. |
| - A308 | - Send a Create PDP Context Response with Cause set to Invalid message format. - Delete the PDP context. |
| - A309 | - Send an Update PDP Context Response with Cause set to Invalid message format. |
| - A310 | - Send an Update PDP Context Response with Cause set to Invalid message format. - Delete the PDP context. |
| - A311 | - Send an Error Indication. |
| - A312 | - Delete the PDP context. |
| - A313 | - Decide which interface the encapsulated PDU should be relayed to. - Decapsulate the G-PDU. - Encapsulate the PDU depending on characteristics of the external data network. - Send the encapsulated PDU on the selected interface. |
| - A314 | - Discard the PDU. |
| - A315 | - Decide which tunnel to use to a SGSN. - Encapsulate the PDU with the GTP header. - Send the G-PDU on the selected tunnel interface to the SGSN. |

### GGSN, PDP Context for anonymous access

Table 56: State transition diagram for an anonymous PDP Context in GGSN

|  | State |  |
| --- | --- | --- |
| Event | S41 None | S42 Active |
| Create AA PDP Context Request | A401, S42 | -- |
| Delete AA PDP Context Request | A402, S41 | A403, S41 |
| Invalid Create Request | A404, S41 | -- |
| Error Indication | -- | A406, S41 |
| G-PDU | A405, S41 | A407, S42 |
| PDU | A409, S41 | A408, S42 |

Table 57: States for an anonymous PDP Context in GGSN

| States | Description |
| --- | --- |
| S41 None | - No anonymous PDP context exists in the GGSN for the TID in a GTP message - No anonymous PDP context exists in the GGSN for the network address or connection identifier in a PDU from an external network. |
| - S42 Active | - An activated anonymous PDP context exists in the GGSN for the TID in a GTP message - An activated anonymous PDP context exists in the GGSN for the network address or connection identifier in a PDU from an external network. |

Table 58: Events for an anonymous PDP Context in GGSN

| Events | Description |
| --- | --- |
| Create AA PDP Context Request | A Create AA PDP Context Request with valid information elements is received. |
| Delete PDP Context Request | A Delete AA PDP Context Request with valid information elements is received. |
| Invalid Create Request | A received Create AA PDP Context Request contains a missing mandatory or an invalid information element. |
| Error Indication | The SGSN has lost its PDP Context and sent an Error Indication to the GGSN. |
| G-PDU | A G-PDU, sent from the MS via SGSN, has arrived at the GGSN. |
| PDU | A PDU has been received on an interface to an external packet data network. |

Table 59: Actions for an anonymous PDP Context in GGSN

| Action | Description |
| --- | --- |
| A401 | - Create a new anonymous PDP context. - Insert the values from the information elements of the Create AA PDP Context Request in the new anonymous context. - Send a Create AA PDP Context Response with Cause set to Request accepted. |
| - A402 | - Send a Delete AA PDP Context Response with Cause set to Request accepted. |
| - A403 | - Send a Delete AA PDP Context Response with Cause set to Request accepted. - Delete the anonymous PDP context. |
| - A404 | - Send a Create AA PDP Context Response with Cause set to Invalid message format. |
| - A405 | - Send an Error Indication. |
| - A406 | - Delete the anonymous PDP context. |
| - A407 | - Decide which interface the encapsulated PDU should be relayed to. - Decapsulate the G-PDU. - Encapsulate the PDU depending on characteristics of the external data network. - Send the encapsulated PDU on the selected interface. |
| - A408 | - Decide which tunnel to use to a SGSN. - Encapsulate the PDU with the GTP header. - Send the G-PDU on the selected tunnel interface to the SGSN. |
| - A409 | - Discard the PDU. |

### SGSN, MM Context

This section is an addition to the mobility management description in GSM 03.60 and GSM 04.08.

Table 60: State transition diagram for a MM Context in a SGSN

|  | State |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Event | S51 Idle | S52 Standby | S53 Ready | S54 Tunnel | S55 Identity | S56 Context |
| GT-IDENTIFICATION.request | A501, S55 | -- | -- | -- | -- | -- |
| GT-GET-CONTEXTS.request | A502, S56 | -- | -- | -- | -- | -- |
| Identity Request | A503, S51 | A503, S51 | A503, S51 | A503, S51 | -- | -- |
| Identity Response ACK | -- | -- | -- | -- | A504, S51 | -- |
| Identity Response NACK | -- | -- | -- | -- | A505, S51 | -- |
| SGSN Context Request INACTIVE | --061 | A506, S51 | A506, S51 | -- | -- | -- |
| SGSN Context Request ACTIVE | --074 | A507, S54 | A507, S54 | -- | -- | -- |
| SGSN Context Response ACK | -- | -- | -- | -- | -- | A508, S5x |
| SGSN Context Response NACK | -- | -- | -- | -- | -- | A509, S51 |
| T3-RESPONSE timeout | -- | -- | -- | -- | A510, S55 | A511, S56 |
| N3-REQUESTS | -- | -- | -- | -- | A505, S51 | A509, S51 |
| T-PDU | -- | -- | -- | A512, S54 |  |  |
| T3-TUNNEL timeout | -- | -- | -- | A513, S51 |  |  |

Table 61: States for a MM Context in SGSN

| States | Description |
| --- | --- |
| S51 Idle | The MS is not attached to GPRS. This is the MM IDLE (GPRS) State defined in GSM 03.60. |
| S52 Standby | The location of the MS is known on the routeing area level. This is the MM STANDBY state defined in GSM 03.60. |
| S53 Ready | The location of the MS is known on the cell level. This is the MM READY state defined in GSM 03.60. |
| S54 Tunnel | The MM state is STANDBY or READY, the New SGSN Address field in the MM context holds an IP address of another SGSN where downlink T-PDUs shall be tunneled and at least one PDP context is in ACTIVE |
| S55 Identity | The SGSN has sent a GTP Identity Request message to another SGSN and is waiting for the response. |
| S56 Context | The SGSN has sent a GTP SGSN Context Request message to another SGSN and is waiting for the response. |

Table 62: Events for a MM Context in SGSN

| Events | Description |
| --- | --- |
| GT-IDENTIFICATION.request | The relaying and routing function should trigger this event when an MS attaches to the GPRS network.SMM anting to access the GPRS network. |
| GT-GET-CONTEXTS.request | The relaying and routing function should trigger this event when an MS has changed routing area and the MS is unknown in this SGSN.SMM receittached to GPRS but is unknown in this SGSN. |
| Identity Request | A GTP Identity Request message has been received. |
| Identity Response ACK | A GTP Identity Response message with the Cause value set to Request accepted has been received. |
| Identity Response NACK | A GTP Identity Response message with the Cause value set to another value than Request accepted or with an invalid information element has been received. |
| SGSN Context Request INACTIVE | A GTP SGSN Context Request message has been received. No PDP context is in the ACTIVE state. |
| SGSN Context Request ACTIVE | A GTP SGSN Context Request message has been received. At least one PDP context is in the ACTIVE state. |
| SGSN Context Response ACK | A GTP SGSN Context Response message with Cause value set to Request accepted has been received. |
| SGSN Context Response NACK | A GTP SGSN Context Response message with Cause value set to another value than Request accepted or with an invalid information element has been received. |
| T3-RESPONSE timeout | A GTP signalling request message has been sent and no answer has been received within the time limit defined by the timer T3-RESPONSE. |
| N3-REQUESTS | The maximum number of GTP signalling request messages have been sent. |
| T-PDU | A T-PDU has been received from a GGSN. |
| T3-TUNNEL timeout | The T3-TUNNEL timeout indicates that the SGSN shall not forward any GGSN data to another SGSN any longer. |

| Action | Description |
| --- | --- |
| A501 | - Map the RAI to an IP address of an old SGSN. The mapping procedure is implementation specific but a table or DNS can be used for example be used as mapping tools. - Send a GTP Identity Request message to the old SGSN. |
| - A502 | - Map the RAI to an IP address of an old SGSN. - Send a GTP SGSN Context Request message to the old SGSN. |
| - A503 | - Create an Identity Response message. - Copy values of the IMSI and Authentication Triplets fields in the MM context to the Identity Response message. - Send the Identity Response message to the originator of the Identity Request message. |
| - A504 | - Issue GT-IDENTIFICATION.confirm(SUCCESS). |
| - A505 | - Issue GT-IDENTIFICATION.confirm(FAILURE). |
| - A506 | - Create an SGSN Context Response. - Fill the SGSN Context Response with values from the MM and PDP contexts. - Send the SGSN Context Response message to the originator of the SGSN Context Request message. - Issue GT-CANCEL.indication. |
| - A507 | - Create an SGSN Context Response. - Fill the SGSN Context Response with values from the MM and PDP contexts. - Send the SGSN Context Response message to the originator of the SGSN Context Request message. - Start timer T3-TUNNEL |
| - A508 | - Issue GT-GET-CONTEXTS.confirm(SUCCESS). |
| - A509 | - Issue GT-GET-CONTEXTS.confirm(FAILURE). |
| - A510 | - Send another GTP Identity Request message to the old SGSN. |
| - A511 | - Send another GTP SGSN Context Request message to the old SGSN. |
| - 12 | - GT-ATTRE |
| - 13 | - GT-RA-rm( |
| - A5124 | - Reroute the T-PDU to the SGSN which IP address is found in the New SGSN Address field of the MM context. |
| - A513 | - Issue GT-CANCEL.indication. |

# Error handling

Protocol errors

A protocol error is defined as a message with  unknown, unforeseen or erroneous content. The term silently discarded used in the following sub-sections means that the implementation shall discard the message without further processing and should log the event including the erroneous message and should include the error in a statistical counter.

### Message too short

When a message is received that is too short to contain all expected information elements, the message shall be silently discarded.

### Unknown or erroneous header

When a receiving node receives a GTP signalling message or a G-PDU, with any GTP header parameter out of bounds or not defined in the GTP version that the sender claims to use, the receiver shall silently discard the GTP signalling message or G-PDU.

### Missing mandatory information element

The receiver of a request with a missing mandatory information element discards the request and sends a response with Cause set to Invalid message format. The receiver of a response with a missing mandatory information element interprets it as a response with Cause set to Invalid message format.

### Invalid information element

The receiver of a request with an invalid information element value discards the request and sends a response with Cause set to Invalid message format. The receiver of a response with an invalid information element value interprets it as a response with Cause set to Invalid message format.

### Unknown information element

An unknown information element in a message is simply ignored by the receiver of the message.

### Different GTP versions

If a receiving node receives a GTP signalling message of a not supported version, the response is sent with Cause set to Version not supported and the version field in the Common Header shall contain the latest version that the receiver supports. The response shall also be in the format defined in the latest supported version. All GSNs shall be able to support earlier GTP versions.

## Mismatching PDP state in SGSN

A PDP context in SGSN can be inactivated but still activated in GGSN for example due to earlier communcation problems between SGSN and GGSN.

When a PDP context is inactivated in SGSN the following procedure is used. Each step is explained in the following list.

Figure 45: Inactivated PDP context in SGSN

1)	The SGSN receives a G-PDU from a GGSN. The PDP context matching the TID is in the Inactive state.

2)	The SGSN sends a GTP Error Indication to the GGSN. The GGSN shall delete its PDP context when this message is received.

## Missing MM context in SGSN

[FFS: The proposed procedure below for MM recovery may be added to GSM 03.60 section 13.6 Recovery and Restoration procedures instead.]

An MM context in SGSN can be missing for example after a restart of the SGSN. When a MM context is missing in SGSN the following procedure is used. Each step is explained in the following list.

Figure 46: Missing MM context in SGSN

1)	The SGSN receives a G-PDU from a GGSN. There is no MM context matching the TID and the G-PDU is discarded.

2)	The SGSN sends a GTP Error Indication to the GGSN. The GGSN shall delete its PDP context when this message is received.

[FFS: It is not obvious how the following step (i.e. paging) can be done. There are some questions that need to be solved:

- Can the IMSI from TID be used for paging or must it be a TLLI?

- Does the RLC/MAC and BSSGP support paging in a complete SGSN area?

- Is it possible to page the MS without knowing the DRX parameters?

- Can we use the rest octets in the Paging Request to indicate that it is a special type of paging?

- The MS must not enter SMM-READY mode. Instead it should enter SMM-IDLE state and initiate the Attach procedure with a piggy-backed Attach Request in a LLC frame.

- This procedure causes wasted T-PDUs from the GGSN. Can we instead, after a SGSN restart, inform all MSs in the service area to contact the SGSN by temporary reducing the periodic routeing area update timer? It is assumed to have a typical value of some hours and can for example be temporary reduced to 5-10 minutes.]

## Missing PDP context in GGSN

[FFS: The proposed procedures below for SGSN recovery may be added to GSM 03.60 section 13.6 Recovery and Restoration procedures instead.]

A context in GGSN can be missing for example after a restart of the GGSN. When a PDP context is missing in GGSN the following procedure is used. Each step is explained in the following list.

Figure 47: Missing PDP context in GGSN

1)	The GGSN receives a G-PDU from a SGSN. There is no PDP context matching the TID and the G-PDU is discarded.

2)	The GGSN sends a GTP Error Indication to the GGSN.

3)	The SGSN sends a Create PDP Context Request (IMSI, PDP Type, PDP Address, QoS Negotiated, TID) message to the GGSN where the Error Indication message was received from to re-activate the PDP context.

4)	The GGSN returns a Create PDP Context Response (TID, PDP Address, BB Protocol, Cause) message to the SGSN.

# Inter-PLMN GTP communication over the Gp interface

[Here it is intended to describe the possible additions needed for inter-PLMN GTP communication. –Ed.]

# IP, the networking technology used by GTP

[Here it is intended to describe, for example, which version of IP the intra-/inter-PLMN backbone network shall be based upon, which routing protocols shall be used over the Gn interface and Gp interface and how packet fragmentation shall be performed. –Ed.]

## IP version

In the first phase of GPRS, Internet Protocol version 4 (IPv4) [8] shall be the networking technology on which GTP tunneling shall be based.

## IP fragmentation

Here it is described how the fragmentation mechanism shall work together with GTP, when the GPRS backbone is based on IPv4.

However, fragmentation should be avoided if possible. Examples of fragmentation drawbacks are, e.g.:

- Fragmentation is inefficient, since the complete IP header is duplicated in each fragment.

- If one fragment is lost, the complete packet have to be discarded. The reason is that no selective retransmission of fragments is possible.

By using Path MTU discovery the application can find out the MTU, and thereby utilise more efficient segmentation mechanisms in other protocol layers than IP.

### MO direction

SGSN: A packet from an MS shall be encapsulated at the SGSN with a GTP header, UDP or TCP header, and IP header. If the resulting IP packet is larger than the MTU of the first link towards the GGSN, fragmentation of the IP packet shall be performed by the SGSN. The SGSN should preferably fragment the IP packet if it is larger than the MTU of any link between SGSN and GGSN. To determine the smallest MTU, see section Path MTU Request.

Backbone router: Any router in the backbone may fragment the GTP packet if needed, according to IPv4.

GGSN: The GGSN will assemble any IP fragments received from SGSNs, according to IPv4. Note that if any fragment is lost, the whole packet need to be discarded.

### MT direction

GGSN: A packet from an external host shall be encapsulated at the GGSN with a GTP header, UDP or TCP header, and IP header. If the resulting IP packet is larger than the MTU on the first link towards the SGSN, fragmentation of the IP packet shall be performed by the GGSN. The GGSN should preferably fragment the IP packet if it is larger than the MTU of any link between GGSN and SGSN. To determine the smallest MTU, see section Path MTU Request. Fragmentation shall also be performed by the GGSN for all packets larger than what SNDCP can handle (which is 1600 octets minus the BSSGP, LLC and SNDCP headers).

Backbone router: Any router in the backbone may fragment the GTP packet if needed, according to IPv4.

SGSN: The SGSN will assemble any IP fragments received from the GGSN, according to IPv4. Note that if any fragment is lost, the whole packet need to be discarded.

### Tunnelling from old to new SGSN

Old SGSN: A user packet shall be encapsulated with a GTP header, UDP or TCP header, and IP header. If the resulting IP packet is larger than the MTU on the first link towards the new SGSN, fragmentation of the IP packet shall be performed by the old SGSN. The old SGSN should preferably fragment the IP packet if it is larger than the MTU of any link between old and new SGSN. To determine the smallest MTU, see Path MTU Request. Fragmentation shall also be performed by the old SGSN for all packets larger than what SNDCP can handle (which is 1600 octets minus the BSSGP, LLC and SNDCP headers).

Backbone router: Any router in the backbone may fragment the GTP packet if needed, according to IPv4.

New SGSN: The new SGSN will assemble any IP fragments received from the old SGSN, according to IPv4. Note that if any fragment is lost, the whole packet need to be discarded.

# GTP parameters

The product of T3-RESPONSE and N3-REQUESTS shall be less than the minumum value of the Attach timer (T3310) and the Routeing Area Update timer (T3330) defined in GSM 04.08.

## Timers

The timer T3-RESPONSE holds the maximum wait time for a response of a signalling request message. The value of the timer is network dependent.

The timer T3-TUNNEL holds the time when PDUs shall be forwarded from the old SGSN to the new SGSN. The timer is started in the old SGSN when it receives a GTP SGSN Context Request message and there is at least one active PDP context. GTP indicates to SMM when the timer has expired. The value of the timer is network dependent.

## Counters

The counter N3-REQUESTS holds the maximum number of attempts made by GTP to send a signalling request message. The value of the counter is network dependent.

Tables

Table 1: Signalling messages

Table 2: Information elements in a Path MTU Response

Table 4: Information elements in a Create PDP Context Request

Table 5: Information elements in a Create PDP Context Response

Table 6: Information elements in an Update PDP Context Request

Table 7: Information elements in an Update PDP Context Response

Table 9: Information elements in a Create AA PDP Context Request

Table 10: Information elements in a Create AA PDP Context Response

Table 12: Information elements in a PDU Reject

Table 13: Information elements in a Send Routeing Information Request

Table 14: Information elements in a Send Routeing Information Response

Table 15: Information elements in a Failure Report

Table 16: Information elements in a Failure Report Acknowledgement

Table 17: Information elements in a Note MS Present

Table 18: Information elements in an Identification Request

Table 19: Information elements in an Identification Response

Table 20: Information elements in a SGSN Context Request

Table 21: Information elements in a SGSN Context Response

Table 23: Cause values

Table 24: Quality of Service (QoS) values

Table 25: BB Protocol values

Table 26: Reordering Required values

Table 27: Relations between Address Type and Protocol Address length

Table 28: SMM State values

Table 29: DRX values

Table 30: MNR values

Table 31: MS Class values

Table 32: Used Cipher values

Table 33: Multislot Capability values

Table 34: State values

Table 35: Reordering Required values

Table 36: Dynamic Address Allowed values

Table 37: VPLMN Address Allowed values

Table 38: QoS values

Table 39: State transition diagram for a PDP Context in SGSN

Table 40: States for a PDP Context in SGSN

Table 41: Events for a PDP Context in SGSN

Table 42: Actions for a PDP Context in SGSN

Table 43: State transition diagram for an anonymous PDP Context in SGSN

Table 44: States for an anonymous PDP Context in SGSN

Table 45: Events for an anonymous PDP Context in SGSN

Table 46: Actions for an anonymous PDP Context in SGSN

Table 47: State transition diagram for a PDP Context in GGSN

Table 48: States for a PDP Context in GGSN

Table 49: Events for a PDP Context in GGSN

Table 50: Actions for a PDP Context in GGSN

Table 51: State transition diagram for an anonymous PDP Context in GGSN

Table 52: States for an anonymous PDP Context in GGSN

Table 53: Events for an anonymous PDP Context in GGSN

Table 54: Actions for an anonymous PDP Context in GGSN

Table 55: State transition diagram for a MM Context in a SGSN

Table 56: States for a MM Context in SGSN

Table 57: Events for a MM Context in SGSN

Figures

Figure 1: Signalling Plane - Protocol stack

Figure 2: User Datagram Protocol (UDP) header

Figure 3: Outline of Common Header

Figure 4: Tunnel ID (TID) format

Figure 5: Common Header followed by subsequent  Information Elements

Figure 6: GGSN - HLR Signalling  via a GTP-MAP protocol-converter in a GSN

Figure 8: Type field for TV and TLV format

Figure 13: Cause information element

Figure 15: IMSI information element

Figure 16: RAI information element

Figure 17: TLLI information element

Figure 18: No. of PDP Contexts information element

Figure 19: Quality of Service (QoS) information element

Figure 20: BB Protocol information element

Figure 21: Reordering Required information element

Figure 22: No. of Triplets information element

Figure 23: Authentication Triplet information element

Figure 24: Receive State Variables information element

Figure 25: GTP Path MTU information element

Figure 26: End User Address information element

Figure 27: Protocol information element for IPv4

Figure 28: Protocol information element for IPv6

Figure 29: Protocol information element for X.25

Figure 30: MM Context element

Figure 31: PDP Context information element

Figure 32: Transmission Plane - Protocol Stack

Figure 33: GTP Header format

Figure 38: Update PDP Context Response at Inter SGSN Routeing Update

Figure 39: Update PDP Context Response at PDP Context Modification

Figure 43: Funtional relaying and routing model in SGSN

Figure 44: Inactivated PDP context in SGSN

Figure 45: Missing MM context in SGSN

Figure 46: Missing PDP context in GGSN

Annex A (informative):	Seamless inter-PLMN roaming

A possible way to support inter-PLMN roaming is discussed very briefly in this annex.

When an MS roams between two SGSNs within the same PLMN, the new SGSN finds the address to the old SGSN by the association old RA - old SGSN. Thus, each SGSN knows the address to every other SGSN in the PLMN.

When an MS roams from an SGSN to an SGSN in another PLMN, the new SGSN may not itself have access to the address to the old SGSN. Instead, the SGSN transforms the old RA information to a logical name of the form:

RAC.LAC.MNC.MCC.GPRS

The SGSN may then acquire the IP address of the old SGSN from a DNS server, using the logical address. Every PLMN should include one DNS server each. Note that these DNS servers are GPRS internal entities, unknown outside the GPRS system.

The above implies that at least MCC + MNC + RAC + LAC (= RAI) is sent as RA parameter over the radio when an MS roams to another RA.

If the new SGSN for any reason fails to obtain the address of the old SGSN, the same actions as when the corresponding event occurs within one PLMN are taken.

Introducing the DNS concept in GPRS gives a general possibility to use logical names instead of IP addresses when referring to e.g. GSNs, thus providing flexibility in addressing of  PLMN nodes.

Another way to support seamless inter-PLMN roaming is to store the SGSN IP addresses in HLR and request them when necessary.

# History

| Document history |  |  |
| --- | --- | --- |
| Date | Status | Comments |
| November 28th 1996 | Version 0.3.0 | Creation of version 0.3.0. The old text of earlier versions of this document have been replaced by completely new text only covering the Gn interface. |
| January 10th 1997 | Proposed version 0.4.0 | The SGSN-SGSN signalling has been modified. The format of the figures describing the MM and PDP Contexts has been modified to conform with the format of the other figures. |
| March 20th 1997 | Proposed version 0.5.0 | - Description of GTP Exception handling has been added. -The negotiated QoS for a given MS is now reported to the GGSN when creating and modifying a PDP Context. - Information about the MS identity (IMSI) is given to the GGSN when creating the PDP Context. - The Address Type field in the transmission plane GTP header has been removed. - New signalling messages to handle Recovery and restoration procedures have been added - New signalling messages to handle Anonymous Access have been added. - |
| May 28th 1997 | Version 1.0.0 | Conversion to ETSI new regime deliverable style. |
| June 6th 1997 | Proposed version 1.1.0 | - Tunnel ID (TID) to be used instead of TEPI. - Sequence number included in the GTP header. - Information if reordering is required is included. - Possibility to set up and tunnel GTP data either over UDP or TCP. - “Modify PDP Context” messages are renamed to “Update PDP Context” messages. - GTP support for “Network Determined PDP Context Activation” is included. - Optional field in GTP header for “piggy-backing” of LLC sequence numbers when tunnelling T-PDUs between old SGNS and new SGSN at inter SGSN RA Updates is included. - IPv4 fragmentation in relation to GTP is described. -The state transition tables are re-worked. - Minor editorial changes are also made. |
| August 22nd 1997 | Proposed version 1.2.0 | - The description regarding how Error Indication is used has been rephrased. - A new complementary coding format TV (Type and Value) has been included for those Information Elements of fixed length. - DNS use at Create PDP Context is described. - Negotiation regarding QoS and reordering is included. - MM and PDP Contexts are described in more detail. - GTP Servises are described and used in the state transition tables. - The State Transition tables have been revised. - The section on Error Recovery has been renamed to Error Handling and also further expanded. - A new section on GTP parameters has been added. - Minor editorial changes have also been made. |
| Rapporteur for GSM 09.60 is: |  |  |
| Stefan Toth Ericsson Mobile Data Design AB Phone: +46 31 703 60 46 Fax: +46 31 703 60 33 Email: stefan.toth@erv.ericsson.se |  |  |
| This document is written in Microsoft Word version 7.0. |  |  |

There will be two outstanding requests when this action is completed. The Create PDP Context Response shall be ignored and can be recognised with a Transaction ID that is lower than expected.

There will be two outstanding requests when this action is completed. The Update PDP Context Response shall be ignored and can be recognised with a Transaction ID that is lower than expected.

All PDP Contexts shall be checked and those that are active shall be updated by sending Update PDP Request(s) to the corresponding GGSN(s).

There will be two outstanding requests when this action is completed. The Create PDP Context Response shall be ignored and can be recognised with a Transaction ID that is lower than expected.

There will be two outstanding requests when this action is completed. The Update PDP Context Response shall be ignored and can be recognised with a Transaction ID that is lower than expected.

The new MM state in the new SGSN shall be the same as the MM state in the old SGSN.