II.4	RECEIVER

ref: GSM 05.05-DCS

In this section on receiver measurements, the procedures to test equipment which is fitted with an antenna connector, and the procedures to test equipment which can only be used with an integral antenna, are in general combined into one single test description. The sections which contain separate descriptions for testing these two kinds of equipment are blocking and spurious response (section II.4.7).

Tests on Mobile Stations fitted with an integral antenna and having no means of connecting an external antenna are specified in terms of received field strength. In order to test such Mobile Stations without the need for separated access to a calibrated test site an antenna coupling device is defined in General Conditions GC6 of Annex 1 of part GC. The detailed description of its calibration is contained within section II.4.2.2. In practice this antenna coupling device may be used for transmitter measurements described in section II.3, but the calibration factors determined in II.4.2.2 will not be directly usable. The detailed calibration, when needed, for transmission tests are described in the relevant sections of II.3.

Testing philosophy

Certain assumptions concerning the functional mechanisms of GSM receivers have been made in order to define tests that will verify the receiver performance without excessive redundancy and excessive test times.

The receiver functions can be divided into:

-	Analogue RF and IF stages that are affected by input levels, temperature and power supply levels.

-	Demodulator that is affected by input levels and interfering signals.

-	Decoders that are affected by the different logical channels and input levels.

The tests are designed to stress each of these blocks with a minimum of redundancy.

Statistical testing of receiver BER/FER performance

Error Definition

1)  Frame Erasure Ratio (FER)

A frame is defined as erased if the error detection functions in the receiver, operating in accordance with GSM 05.03, indicate an error. For full rate speech this is the result of the 3 bit cyclic redundancy check (CRC) as well as other processing functions that cause a Bad Frame Indication (BFI). For signalling channels it is the result of the FIRE code or any other block code used. For data traffic FER is not defined.

2)  Residual Bit Error Ratio (RBER)

The Residual Bit Error Ratio is defined as the Bit Error Ratio (BER) in frames which have not been declared as erased.

3)  Bit Error Ratio (BER)

The Bit Error Ratio is defined as the ratio of the bits wrongly received to all data bits sent.

Test method

Each test is performed in the following manner:

a)  Set up the required test conditions. Set the parameters Max-samples (maximum number of samples) and Max-events (maximum number of error events) to specific values for each test.

b)  Perform the test and record the number of offered samples (bits or frames sent) and the number of occurred events (bit or frame errors).

c)  Terminate the test and determine the test result ("pass" or "fail") when either of the following conditions become true:

-  Number of offered samples >= Max-samples   --->  "PASS"

-  Number of (error) events  >  Max-events    --->  "Fail"

In practice it may be sufficient to generate "Max-samples" and later look at Number of error events observed.

Test criteria

The limits on number of samples and events shall be defined in order to comply with different requirements :

1)  to keep reasonably low the risk of passing a bad unit through the individual tests;

2)  to have high probability of passing a good unit through the individual tests;

3)  to perform a measurement with a significant statistics;

4)  to keep the test time as low as possible.

The risk of passing a bad unit (point 1) should be kept lower than 0.2%. A unit is generally considered "bad" if its BER (or FER) performance is 1.5 times worse than that specified in AWGN (Additive White Gaussian Noise) and 1.26 times worse than that specified in multipath environment. These values have been adopted (taking into account the expected shapes of the BER performance) in order not to pass a unit with performance worse than the specifications by more than 1 dB.

The probability of passing a good unit (point 2) should be at least 99.7%.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures lead to consider a number of events (point 3) not lower than 200 in AWGN channel and not lower than 600 in multipath environment, and to test a BER (or FER) performance 1.22 times worse than that specified in AWGN and 1.12 times worse than that specified in multipath environment (this corresponds to test a performance at the most 0.5 dB worse than that specified).

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in the following table.

PROPAGATION CONDITIONS	TU3 1.5	TU50	HT100	RA250130

MIN. TEST TIME (sec)	500	 3015	157.5	 6

The table below details, for the different test conditions, the number of events and samples required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

As it can be seen in the table, in the cases in which both FER and RBER have to be tested on the same channel the same time as the FER measurement has been adopted. This is longer than that required for the RBER only according to the the discussed criteria, but allows the relevant accuracy to be improved without increasing the total test time.

TABLE OF TEST CONDITIONS

PROPAGATION/	SPECIFIED	TESTED	Max No	Max No	Prob that	BAD UNIT	Risk that

Type of	Type of	Frequency	FER/BER	FER/BER	 of	 of	GOOD UNIT	BER/FER	BAD UNIT

test	channel	Conditions	%	%	EVENTS	SAMPLES	will pass	 %	will pass

BFI	TCH/FS	STATIC	0.200	0.244	 200	82000	99.719	0.300	0.139

Sensitivity 	TCH/FS	STATIC/FH	0.100*α	0.122*α	200*α	164000	99.717	0.150*α	0.140

,,	TCH/FS Class Ib	STATIC/FH	0.400/α	0.410/α	82000/α 20000000	100.000	0.600/α <0.001

,,	TCH/FS Class II	STATIC/FH	2.000	2.439	200	8200	99.714	3.000	0.001

,,	TCH/FS	TU50/No FH	4.000*α	4.478*α	600*α	13400	99.743	5.040*α	0.133

,,	TCH/FS Class Ib	TU50/No FH	0.300/α	0.320/α	4800/α	1500000	100.000	0.378/α <0.001

,,	TCH/FS Class II	TU50/No FH	8.000	8.333	5000	60000	99.865	 10.080	 <0.001

,,	TCH/FS Class II	HT100/No FH	9.000	9.333	2800	30000	97.826	 11.340	 <0.001

,,	TCH/FS Class II	RA130/No FH	7.000	7.500	1800	24000	99.873	8.820	 <0.001

,,	FACCH	TU50/No FH	9.000	8.955	600	6700	99.788	 10.080	0.102

,,	TCH/F9.6&H4.8	HT100/No FH	0.700	0.784	600	76500	99.721	0.882	0.176

,,	TCH/F4.8	HT100/No FH	0.010	0.011	600	5350000	99.732	0.013	0.197

,,	TCH/F2.4	HT100/No FH	0.001	0.001	150	11900000	99.734	0.002	0.139

,,	TCH/H2.4	HT100/No FH	0.010	0.011	 600	5350000	99.732	0.013	0.197

INPUT LEVEL 	TCH/FS Class II	STATIC	0.010	0.012	 200	1640000	99.716	0.015	0.141

RANGE	TCH/FS Class II	EQ50	3.000	3.250	1950	60000	99.981	3.780	 <0.001

CO-CHANNEL	TCH/FS	TU1.5/No FH 	21.00*α	24.00*α	6000*α	25000	100.000	26.460*α	<0.001

REJECTION	TCH/FS Class Ib	TU1.5/No FH	2.000/α	2.091/α	69000/α	3300000	100.000	2.520/α	<0.001

,,	TCH/FS Class II	TU1.5/No FH	4.000	4.300	86000	2000000	100.000	5.040	<0.001

,,	TCH/FS	TU1.5/FH	3.000*α	3.371*α	600*α	17800	99.797	3.780*α	0.194

,,	TCH/FS Class Ib	TU1.5/FH	0.200/α	0.215/α	4300/α	2000000	100.000	0.252/α	<0.001

,,	TCH/FS Class II	TU1.5/FH	8.000	8.333	100000	1200000	100.000	 10.080	 <0.001

,,	FACCH	TU1.5/No FH 22.000	 24.000	6000	25000	100.000	 27.720	 <0.001

ADJACENT	TCH/FS	 TU50/No FH	3.000*α	3.371*α	600*α	17800	99.797	3.780*α	0.194

CHANNEL	TCH/FS Class Ib	TU50/No FH	0.250/α	0.270/α	5400/α	2000000	100.000	0.315/α	<0.001

,,	TCH/FS Class II	TU50/No FH	8.100	8.333	100000	1200000	100.000	10.206	 <0.001

INTERMOD.	TCH/FS Class II	STATIC	2.000	2.439	200	8200	99.741	3.000	0.122

BLOCKING &	TCH/FS Class II	STATIC	2.000	2.462	32	1300	89.805	4.000	0.167

SPURIOUS			(Note 1) 	2.439	200	8200	99.741	4.000	<0.001

RESP.

Note 1:	In the Table of Test Conditions, for "blocking and spurious responses", theis procedure labelled by (*) will be carried out only if the unit under test fails the previous (less time-consuming) test.

Note 2:	α is a parameter which ranges from 1 to 1.6. The value of α for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions. For example, the value of α may be different for a TU50 sensitivity test and an RA250 RA130 sensitivity test. The value of α is determined by dividing the measured number of events for the FER test by the value of the maximum number of events listed in the table corresponding to α=1; if the result of the division is lower than 1, a value of α=1 shall be used. The probabilities that a good unit will pass and the risks that a bad unit will pass, listed in the table are valid for α=1, and would be slightly different for other values of α.

Note 3:	In order to save time, two tests shown in the table do not comply with the above said constraints: they are the sensitivity test for the TCH/F2.4 channel and the blocking and spurious response test for the TCH/FS (class II bits) channel.

In the first case, a bad unit performs 2 times (instead of 1.26) worse than that specified is accounted, so reducing the required number of events to 150, instead of 600. On the other hand, the specified RBER is in this case 10E-5 and, on the basis of simulations and hardware validation results, doubling this RBER results in a drop in performance of less than 1 dB.

For the blocking test two separate procedures are foreseen : one, less accurate and less time consuming, which does not fulfil the contraints set at points 1) to 3), and a second procedure, which fulfil them, to be adopted only if the unit fails the first one. With the first procedure, the probability of passing a good unit is about 90%, but in case of failure a re-testing is foreseen, which guarantees a greater probability (99.7%). The risk of passing a unit defined as "bad" would be very high with the first procedure if we adopted the aforesaid criteria (about 14%) ; on the other hand, it will drop to 0.17% if we consider as a threshold to reject a bad unit a BER value of twice the specified one. Being a measuremnt in static, non-linear conditions, a factor of 2 should correspond to a degradation lower than 1 dB, and taking also into account that the blocking test is a very long test, it seems reasonable to accept it.

Based on such table, it has been computed that the corresponding total test time is of the order of [65] hours on the average, excluding the test set up time and considering that the blocking test has to be carried out with the more accurate procedure in 10% of the cases.

II.4.1	Bad frame indication performance	ref: GSM 05.05-DCS section 6.4

II.4.1.1	Definition

The performance of the bad frame indication (BFI) is a measure of the effectiveness of the MS under DTX conditions. It includes the effect of the 3 bit Cyclic Redundancy Check (CRC) and all other processing associated with the DTX function. The BFI is measured on a full rate speech TCH (TCH/FS) by counting the number of undetected bad frames whilst the input signal is a randomly modulated carrier described in g) below.

Note:	DTX is used to prevent the MS dropping the call.

II.4.1.2	Method of measurement

a)  A call shall be originated by the SS to the MS, and the MS  shall be made to answer the call.

The call shall occupy a full rate channel and shall be in a non-hopping mode on one of the radio frequencies in the range 60 to 65690-710 (ARFCN). Throughout the test the BCCH shall be transmitted by the SS at an EMF of 28dBµV(  ).

b)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the bad frame indication to the SS.

c)  The SS shall simulate a BSS in DTX mode. During the period when no transmission would occur the SS shall transmit a GSM carrier modulated with random data at a level of 19 dBµV emf(  ). SACCH shall be transmitted normally at a level of 28 dBµV emf(  ). The SID frame shall be transmitted in its correct time interval with valid information at a level of 28 dBµV emf(  ). During transmission of SACCH or SID the random data shall be discontinued.

d)  The SS will proceed to transmit Max-samples of frames of TCH/FS information and will check the bad frame indication (BFI) of the looped back signal from the MS. The SS shall record the number of frames where the bad frame indication is not set. During transmission by the SS of SID the SS shall check that the BFI is not set.

NOTE:	Further explanations on the mechanism of signalling the BFI to the SS will be found in sections III.1.1 and III.1.3.

II.4.1.3	Limits

The BFI performance is accepted if the number of undetected BFIs does not exceed the Max-events:

Max-events 	-  200

Max-samples	-  82000 (excluding SID frames).

During loop back of SID frames no BFI shall be set.

II.4.2	Sensitivity

The reference sensitivity (GSM 05.05-DCS) is the signal level at the MS receiver input at which a certain BER or FER must be achieved.

II.4.2.1 Reference Sensitivity for TCH/FS - compliance	ref: GSM 05.05-DCS section 6.2

II.4.2.1.1	Definition

Compliance with the requirements for reference sensitivity level is tested by setting the receiver at an input signal at the specification limit for reference sensitivity level and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a data signal with a BER or a FER less than the specified values is produced.

II.4.2.1.2	Method of measurement

a)  Set up a call on a traffic channel in the range ARFCN 60 to 65690-710. The SS shall also set up six adjacent cell BCCH's having signal strengths in the range 15 dBµVemf(  ) to 35 dBµVemf(  ). The choice of ARFCN and amplitude for these adjacent cell BCCH's shall follow the guideline given in II.3.2.1 a).

b)  The MS shall be commanded to the maximum power level.

c)  The SS traffic channel shall be the Standard Test Signal C1(see GSM 11.40-DCS) at a MS level of 28 dBµVemf(  ).

d)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS. The fading function of the SS is set to TU50.

e)  The amplitude of the wanted signal shall be set to 1113dBµVemf(  ) for hand portable MSs and to 9dBµVemf(  ) for other MSs.

f)  The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication. Also throughout the test, the SS monitors the status message from the MS to confirm correct reporting of the adjacent cell BCCH's.

g)  The SS determines the number of residual bit error events for the bits of class II, by examining sequences of Max-samples of consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.

h)  The SS determines the number of residual bit error events for the bits of the class Ib, by examining sequences of Max-samples of consecutive bits of class Ib, where bits are taken from those frames for which no bad frame indication was given.

i)  The SS also determines the frame erasure events by examining sequences of Max-samples of consecutive frames and assuming a frame is received successfully when there is no bad frame indication concerning it.

j)  The MS is placed in a climatic test chamber (for extreme test conditions see Annex 1, TC), and steps a) up to including g) are repeated for the following combinations of temperatures and power supply voltages:

Temp		:	Hi	Hi	Lo	Lo

Voltage	:		Hi	Lo	Lo	Hi

k)  Steps a) to j) shall be repeated for a channel in the range 1 to 5513 to 523 (ARFCN) and for a channel in the range 120 to 124874 to 884 (ARFCN).

l)  Steps a) to g) shall be repeated, except under step d) the SS fading function shall be set in turn to RA250130 and HT100.

m)  Steps a) to j) shall be repeated, except at step g) the SS fading function shall be set to static and the MS shall be commanded by the SS into hopping mode using the hopping sequence of GC1 of Annex 1, full band.

The amplitude of the wanted signal shall be set as in step e). All the other time slots, except the active ones, shall be set to 28dBµVemf(  ). This will implicitly test adjacent time slot rejection.

II.4.2.1.3	Limits

The Max-events measured for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the values given in the table below:

Propagation	Propagation	Propagation	Static

Conditions	Conditions	Conditions	Conditions

Channels	TU50		RA250130	HT100

TCH/FS	Max-	Max-	Max-   	Max-	Max-   	Max-	Max-    	Max-

Events	samples	events 	sample	events 	samples	events  	samples

FER	600*α	19100 13400				200*α 	164000

class Ib(RBER)  5000/α	2000000					82000/α    20000000

4800/α	1500000

class II(RBER)  9200	120000	1800 	24000	5200	60000	200	8200

5000	60000	2800  	30000

where α is a parameter which can range from 1 to 1.6. The value of α for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.2.2	Sensitivity Measurements for Equipment without an antenna connector

For equipments fitted with an integral antenna and not provided with means for connection to an external antenna an initial procedure is required before sensitivity measurements can be performed. This procedure consists of calibrating an antenna coupling device. This antenna coupling device is a test fixture, which, when mated with the MS forms a single unit enabling the MS to be tested as if it were a MS with an antenna connector. Once calibrated this antenna coupling device enables all receiver test procedures, except blocking to be identical for equipments with an integral antenna and for equipments with an antenna connector.

The calibration procedure shall be carried out at three frequencies, namely an ARFCN in the range 1 to 5513 to 523, an ARFCN in the range 60 to 65690-710 and an ARFCN in the range 120 to 124874 to 884. The procedure consists of three distinct stages as follows :

1)  Establish the MS antenna radiation pattern for the three selected frequencies.

2)  Calibrate the test range (or anechoic shielded chamber) for the conditions needed in 1).

3)  Determine the antenna coupling device coupling factor.

II.4.2.2.1	Antenna Radiation Pattern

a)  The MS shall be in the anechoic shielded chamber, or on an outdoor test site, on an isolated support in a vertical position at an orientation specified by the manufacturer. This position is the 0 degree position.

A test antenna, connected to the SS shall be in the anechoic shielded chamber, or on the outdoor test site, at a distance of at least 3 metres from the MS.

b)  A call shall be originated by the SS to the MS on a frequency in the range ARFCN 1 to 5513 to 523. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power.

c)  The SS shall, using estimated parameters for the outdoor test site or anechoic shielded chamber, set its output level 'E' (see Fig. II.4-1) to give an MS receiver input level of approximately 32 dBµVemf. This corresponds to a field strength of 55.561.3 dBµV/m at the MS position. The signal shall be the Standard Test Signal C1.

Note:	The absolute value of the received signal level is not critical. The value suggested however will ensure that the MS receiver is operating essentially error free, yet is low enough to avoid any non linear effects in the receiver.

d)  The SS shall use the RXLEV message from the MS to determine a measure of the received field strength. The procedure detailed in the flow chart of figure II.4-1 shall now be followed. The signal level from the SS that just results in the transition from between two values of the parameter RXLEV, defined as RXLEVa to RXLEVb shall be recorded as Ei.

Note:	The actual values of RXLEVa and RXLEVb will need to be recorded, because this transition will be used as the reference point for all further stages of the calibration procedure.

e)  Step d) shall be repeated after the MS has been rotated by n * 45 degrees in the horizontal plane for all values of n in the range 1-7. Ensuring that the same RXLEV transition is used, the signal levels from the SS shall be recorded as Ein.

FIG II.4-1

**********************************************************************************************************************

This space is left for FIG II.4-1

**********************************************************************************************************************

f)  Calculate the effective mean signal level from the RMS value of the eight signal levels obtained in d) and e) above by using the following formula :

┌      8      ┐1/2

│ __________  │

E1  =  │   n=7   1   │

│   Σ  ────   │

└  n=0  Ein²  ┘

g)  Repeat steps b) to f), except in step b) use an ARFCN in the range

60 to 65690-710 to obtain a mean signal level E2. Ensure the same RXLEV transition is used.

h)  Repeat steps b) to f), except in step b) use an ARFCN in the range 120 to 124874 to 884 to obtain a mean signal level E3.

Ensure the same RXLEV transition is used.

II.4.2.2.2	Test Range Calibration

The objective of this step is to determine the actual field strength at the MS corresponding to the three signal levels E1, E2 and E3 established in II.4.2.2.1. The following procedure shall be used :

a)  Replace the MS by a calibrated reception antenna connected to a measuring receiver.

b)  For each frequency used in II.4.2.2.1 measure the field strength Efr corresponding to the respective signal levels Er determined in steps f), g) and h) of II.4.2.2.1 record these values as Ef1, Ef2, Ef3.

II.4.2.2.3	Antenna coupling Device Coupling Factor

The coupling factor of the antenna coupling device is the relationship expressed in dB, between the output signal of the SS and the effective receiver input signal for the MS. The antenna gain is assumed to be OdBi. The procedure shall be as follows :

a)  The MS is fixed to the antenna coupling device and the output of the SS is connected to the antenna coupling device.

b)  A call shall be originated by the SS to the MS using a frequency in the range ARFCN 1 to 5513 to 523. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power, non hopping encrypted mode.

c)  The SS shall, using the procedures of II.4.2.2.1, adjust its output signal level to determine the RXLEVa to RXLEVb transition. This signal level shall be recorded as Ec1.

d)  Repeat steps b) and c) for frequencies in the range ARFCN 60 to 65690-710 and 120 to 124874 to 884. Record the RXLEV transitions as Ec2 and Ec3 respectively.

e)  The antenna coupling device coupling factor F is then calculated from:

┌   Ecn    ┐

Fn = 20 log10 │  ─────── │

└  Efn*Kn  ┘

where Kn = conversion factor of an isotropic antenna expressed as:

µV

────

µV/m

at the frequency corresponding to the ARCFN  ARFCN used.

f)  The mean antenna coupling factor Fm to be used for measurements requiring hopping shall be calculated from the RMS value of all parameters in e) as follows :

┌             3           ┐1/2

Ecm  =  │  ────────────────────   │

└ 1/Ec1² + 1/Ec2² + 1/Ec3²┘

┌             3            ┐1/2

Efm  =  │  ────────────────────    │

└ 1/Ef1² + 1/Ef2² + 1/Ef3² ┘

┌  k1² + k2² + k3²  ┐1/2

km   =  │  ─────────────    │

└         3         ┘

┌   Ecm    ┐

Fm   =  20 log10 │  ─────── │

└  Efm*km  ┘

g)  In all tests in which a handheld MS with integral antenna is the unit under test, the input signal level into the coupling device is determined from :

Ein 	  =	Ereq + F

where: Ein   = input signal level to coupling device (dBµVemf)

Ereq  = signal level required by the test (dBµVemf)

F     = coupling factor at the respective ARFCN (dB)

This is indicated in the test procedures as Ereq, dBµVemf(   ), where the empty parenthesis is to be read as Ein.

II.4.2.3	Sensitivity for data channels and control channels

II.4.2.3.1	Definition

Compliance with the requirements for reference sensitivity on control channels is tested by setting the receiver at an input signal at the specification limit for reference sensitivity and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a FER less than the specified value be produced. For data channels, reference sensitivity is defined in terms of BER.

II.4.2.3.2	Method of measurement

a)  The SS originates a call to the MS on a traffic channel having a ARFCN in the range 60 to 65690-710.

b)  The MS is made to answer the call, and is commanded to it's maximum power level.

c)  The SS output on the traffic channel is set to 28 dBµVemf(  ), producing Standard Test Signal C1.

d)  The SS commands the MS to signal the frame erasure event to the SS.  The fading function of the SS is set to TU50.

e)  The wanted signal is set to a level of 1113 dBµVemf(  ) for hand held MS or 9 dBµV for any other MS.

f)  The SS now sends progress messages, with the user-to-user field filled with random data. The content of the Progress Indicator Information Element should be #3 (Origination address in non-PLMN/ISDN).

g)  The SS  determines the frame erasure events during Max-samples of FACCH frames.

h)  The wanted signal is set to 2830dBµV and the SS commands the MS to a TCH/F9.6 channel. It also commands loop back on the data decoder.

i)  Set the SS fading function to HT100 and the wanted signal to 1113dBµV(  ) for a hand portable MS or 9dBµV for any other MS.

j)  The SS compares transmitted data with received data for Max-samples consecutive bits and records every error bit as an error event.

k)  Steps h) to j) are repeated for data channels of TCH/H4.8, TCH/F4.8, and TCH/F2.4 and TCH/H2.4.

Note 1:	Not all Mobile Stations will be equipped with all data channels. The test procedure will need to be adapted to cater for limited sub sets for Mobile Stations offering a restricted data capability.

Note 2:	The SACCH, SDCCH, AGCH and PCH channels are not tested because the coding is identical to the FACCH. These channels are tested implicitly in layer 3.

II.4.2.3.3	Limits

The Max-events measured for different channels and under the different propagation conditions shall not exceed the values given in the table below:

Channels	Type of	Propagation	Max-events	Max-samples

measurement

FACCH 	FER	TU50	600 	 53506700

TCH/F9.6&H4.8 	BER	HT100	 1400600	 18000076500

TCH/F4.8	BER	HT100	600 	5350000

TCH/F2.4	BER	HT100	150 	 11900000

TCH/H2.4	BER	HT100	600	 5350000

II.4.3	Usable receiver input level range	ref: GSM 05.05-DCS section 6.1

II.4.3.1	Definition

The usable receiver input level range is the range of the radio frequency input level of a specified modulated signal over which bit error ratio or frame erasure ratios stay between specified limits.

II.4.3.2	Method of measurement

a)  The SS shall originate a call to the MS and the MS shall be made to answer the call on a TCH/FS having an ARFCN in the range 60 to 65690-710.

b)  The SS shall be set on the TCH/FS to produce Standard Test Signal C1 at a level of 28 dBµVemf(  ).

c)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input without signalling the frame erasure event (III.1.2.1.1.2).

d)  The SS compares the data that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding.

The SS tests the bit error ratio for the non-protected bits of TCH/FS class II, by examining sequences of Max-samples consecutive bits of class II. The number of error events is recorded.

e)  Step d) shall be repeated with the amplitude of the wanted signal increased to an input levels of respectively 6373dBmicroVoltemf(  ) and 103dBmicroVoltemf(  ) at the receiver input.

e1)  Step d) shall be repeated with the amplitude of the wanted signal increased to an input level of 90dBmicroVoltemf(  )at the receiver input

f)  The SS fading function is set to EQ50.

g)  Step d) shall be repeated with the amplitude of the wanted signal set to respectively 28dBmicroVoltemf(  ) and 73dBmicroVoltemf(  ) at the receiver input.

h)  The Mobile Station is placed in a climatic chamber and steps a) to g) are repeated for the following combinations of extreme test conditions (Annex 1, TC2.2 and TC3):

Temp	:	Hi 	Hi	Lo	Lo

Voltage	:	Hi	Lo	Lo	Hi

II.4.3.3	Limits

The number of error events recorded in this test shall not exceed the

Max-events values given in the table below when Number of samples = Max-samples. This shall apply for any combination of normal and extreme test voltages and ambient temperature, for the different propagation conditions and for any level of input signal to the receiver.

Propagation conditions	Max-events	 Max-samples

Step e)	 Static	200	1640000

Step e1)	 Static	 400	1640000

EQ50	39001950	 120000 60000

II.4.4	Co-channel rejection

II.4.4.1	Definition

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal, both signals being at the nominal frequency of the receiver.

II.4.4.2	Method of Measurement

a)  A call shall be originated by the SS to the MS on a channel in the range ARFCN 60 to 65690-710, and the MS shall be made to answer the call.

The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28dBmicroVoltemf(  ).

The unwanted signal shall have no fixed relationship with the bit transitions of the wanted signal, and it shall be modulated with random data. Its amplitude shall be 9dB below that of the wanted signal. The unwanted signal shall be continuous.

b)  The MS shall be operated in the encrypted mode.

c)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

d)  The SS is set to produce a wanted signal and an independantindependent, uncorrelated interfering (unwanted) signal at the same time.  The fading characteristic of the wanted and the interfering signal shall be TU31.5. The channel frequency shall be in the range 60 to 65690-710 (ARFCN).

e)  The SS compares the modulation of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure ratio compliance for the TCH/FS, by examining Max-samples consecutive frames of TCH/FS information where a frame is assumed to be received successfully if there is no bad frame indication. The number of frame erasure events is recorded.  During the Max-samples of the frame erasure measurement, the RBER of the class II and class Ib bits are also determined.

f)  The SS now sends progress messages, with the user-to-user field filled with random data. The content of the progress information element should be #3 (Origination address in non-PLMN/ISDN). The SS determines the number of error events (frames erasures) for Max-samples of consecutive frames.

g)  Steps d) and e) shall be repeated except that in step d), both the wanted and interferringinterfering signal shall be hopping and the SS shall command the MS into hopping mode with a hop pattern covering at least [10] frequencies in a range not exceeding [5] MHz. The hopping band shall be centeredcentred around an ARFCN in the range 60 to 65690-710, according to the narrow band hop of GC1 of Annex 1.

Note: the frequency range of this test is dependant on the fading simulator.

II.4.4.3	Limits

The number of events recorded in this test shall not exceed the Max-events values given in the table below when the Number of samples = Max-samples.

Channel	Propagation	Type of	Max-events	Max-samples

conditions	measurement

FACCH	TU31.5/No FH	FER	6000	25000

TCH/FS	TU31.5/No FH	FER	6000*α	25000

TCH/FS Class Ib	TU31.5/No FH	RBER	69000/α	3300000

TCH/FS Class II	TU31.5/No FH	RBER	86000	2000000

TCH/FS	TU31.5/FH	FER	600*α	17800

TCH/FS	TU31.5/FH	RBER	4300/α	2000000

Bits class Ib

TCH/FS	TU31.5/FH	RBER	100000	1200000

Bits class II

The parameter α can range from 1 to 1.6. The value of α for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.5	Adjacent channel rejection	ref: GSM 05.05-DCS section 6.3.2

II.4.5.1	Definition

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal in the adjacent channel.

The adjacent channel can be the adjacent in the RF spectrum or in time.  There are therefore two types of adjacent channel selectivity:

1.	Adjacent RF channel selectivity which is specifically tested in this section.

2.	Adjacent Time Slot selectivity, which is implicitly tested during many receiver tests, including the tests in this section.

II.4.5.2	Method of Measurement

a)  A call shall be originated by the SS to the MS, and the MS shall be made to answer the call.

b)  The MS shall be operated in the encrypted mode.

c)  The System Simulator commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

d)  The SS is set to produce a wanted Standard Test Signal C1 and an independantindependent, uncorrelated interfering (unwanted) Standard Test Signal I1 at the same time. The fading characteristic of the wanted and the interfering signal is TU50. The channel frequency shall be in the range 60 to 65690-710 (ARFCN).

The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28 dBmicroVoltemf(  ).

The unwanted signal shall have no fixed relationship with the bit transition of the wanted signal and it shall be modulated with random data. Its amplitude shall be 9 dB above that of the wanted signal. The unwanted signal shall be continuous, with a nominal frequency 200 kHz above the nominal frequency of the wanted signal.

e)  The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure compliance for the TCH/FS by examining Max-samples of consecutive frames. The number of frame erasure events is recorded. During Max-samples of the frame erasure measurement, the RBER of the class II and class Ib bits are also determined.

f)  The measurement of steps d) and e) shall be repeated with the unwanted signal on a frequency at the same displacement from, but below, the frequency of the wanted signal.

g)  The measurement of steps d) to f) shall be repeated for a displacement of the unwanted signal of 400 kHz, and with the amplitude of the unwanted signal 41 dB above the level of the wanted input signal.

h)  The Mobile Station is placed in a climatic test chamber (for extreme test conditions see Annex 1, part TC), and steps d) to g) are repeated for the following combinations of temperatures and power supply voltages:

Temp.		:	Hi	Hi	Lo	Lo

Volt.	:		Hi	Lo	Lo	Hi

II.4.5.3	Limits

Channel	Type of measurement	Max-events	Max-samples

TCH/FS	FER	600*α	178008900

class Ib	RBER	54004200/α	20000001000000

class II	RBER	10000050000	1200000600000

The number of events recorded in this test shall not exceed the Max-events given in the table when the Number of samples = Max-samples.  This shall apply for any combination of normal and extreme test voltages and ambient temperature, and with the interfering signals at either side of the wanted frequency.

The parameter α can range from 1 to 1.6. The value of α for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.6	Intermodulation rejection	ref: GSM 05.05-DCS section 5.2

II.4.6.1	Definition

The intermodulation rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency.

II.4.6.2	Method of measurement

NOTE:	The measurements address the third order intermodulation, which represents the most serious case.

The compliance with the requirements for the bit error ratio of TCH/FS class II bits is checked in order to verify the receiver's intermodulation rejection under normal and extreme test conditions.

a)  A call shall be originated by the SS to the MS and the MS shall be made to answer the call. The call shall be on a TCH in the range 60 to 65690-710 (ARFCN).

b)  The MS shall be operated in the encrypted mode.

c)  The MS shall be operated under normal test conditions.

d)  The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver. The level of the wanted signal shall correspond to 4 dB above the reference sensitivity level (see Table II.4.6-1).

e)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the frame erasure event to the SS.

f)  The SS is set to produce a static wanted signal, and two static interfering (unwanted) signals at the same time. There shall be no correlation in the modulation between the signals.

The first interfering signal shall be on a frequency equal to the centre frequency of an ARFCN four above that of the receiver. This signal shall be static and shall be unmodulated.

The second interfering signal shall be on an ARFCN eight above that of the receiver. This signal shall be static, continuous and shall be modulated by random data.

The amplitude of both the interfering signals shall be set according to Table II.4.6-1.

g)  The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance of class II bits by examining Max-samples of consecutive bits, with bits only taken from those frames which do not signal frame erasure. The number of error events are recorded.

h)  The measurement of step g) shall be repeated with the two unwanted signals having frequencies corresponding to ARFCN four and eight below the ARFCN of the unwanted signal.

i)  Steps a) to h) shall be repeated but with the receiver operating on an ARFCN in the range 1 to 5513 to 523.

j)  Steps a) to h) shall be repeated but with the receiver operating on an ARFCN in the range 120 to 124874 to 884.

k)  The Mobile Station is placed in a climatic test chamber (for conditions see Annex 1, part TC), and steps a) to j) are repeated for the following combinations of temperature and power supply voltage:

Temp. :	Hi	Lo	Hi	Lo

Volt. :	Hi	Lo	Lo	Hi

┌────────────┬─────────────┬─────────────┬──────────────┬──────────────┐

│  MS CLASS  │ MS HANDHELD │   Wanted    │  First       │  Second      │

│            │             │   signal    │  interferer  │  interferer  │

│            │             │ dBµVemf(  ) │  dBµVemf(  ) │  dBµVemf(  ) │

├────────────┼─────────────┼─────────────┼──────────────┼──────────────┤

│  1,2       │     NO     │     1317    │     7464     │     6364     │

│ 3,4,5     │     YES    │     15      │     64       │     63       │

└────────────┴─────────────┴─────────────┴──────────────┴──────────────┘

Table II.4.6-1: Intermodulation test signal levels

Note:	The levels in Table II.4.6-1 are different to those specified  in GSM 05.05 due to the consideration of the effect of modulation sideband noise from the second interferer.

II.4.6.3	Limits

The Number of error events recorded in this test shall not exceed the

Max-events values given below when the Number of samples = Max-samples.

This shall apply under normal condition and under any combination of normal and extreme test voltages and ambient temperature, and with the two interfering signals at either side of the wanted frequency.

Channel	Type of	Max-events	Max-samples

measurement

TCH/FS

Class II Static	RBER	   200	   8200

II.4.7	Blocking and Spurious Response	ref: GSM 05.05-DCS section 5.1

II.4.7.1	Definition

Blocking is a measure for the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation.

II.4.7.2	Method of measurement - equipment with antenna connector

a)  The MS is connected to the SS. The MS is operated under normal test conditions.

b)  A call shall be originated by the SS to the MS, and the MS shall be made to answer the call. The call shall occupy a full rate channel and shall be on one of the radio frequencies in the range 60 to 65690-710 ARFCN.

c)  The MS shall be operated in the encrypted mode.

d)  The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver.

e)  The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

f)  The SS is set to produce a static wanted signal and a static interfering signal at the same time. The level of the wanted signal shall be 1715 dBmicroVoltemf. for handportable MSs or 13 dBmicroVoltemf for all other MSs (this corresponds to 4 dB above the reference sensitivity level.

g)  The unwanted signal shall be a C.W. signal. It shall in turn be on all frequencies in the range 100 kHz to 12.75 GHz at intervals of 200  kHz, with the exclusion of the range FR +/- 600 kHz (FR is the center frequency of the receiver).

h)  The level of the unwanted signal shall be set according to the following table.

FREQUENCY	 LEVEL FOR MSs

WITH POWER CLASSES

3, 4, 5	1, 2

FR +/- 600 kHz to FR +/- 800 kHz	70 dBµV	 7570 dBµV

FR +/- 800 kHz to FR +/- 1.6 MHz	70 dBµV	 8070 dBµV

FR +/- 1.6 MHz to FR +/- 3 MHz	80 dBµV	 9080 dBµV

9151785 MHz to FR - 3 MHz	 90 dBµV	 90 dBµV

FR + 3 MHz to 9801920 MHz	 90 dBµV	 90 dBµV

835 MHz to 915 MHz	113 dBµV	113 dBµV

980 MHz to 1000 MHz	113 dBµV	113 dBµV

100 kHz to 835 MHz	 90 dBµV	 90 dBµV

1000 MHz to 12.75 GHz	 90 dBµV	 90 dBµV

100 kHz to 1705 MHz		113 dBµV

1705 MHz to 1785 MHZ		101 dBµV

1920 MHz to 1980 MHz		101 dBµV

1980 MHZ to 12.75 GHz	 	113 dBµV

Note : This section is the subject of a GSM 2 CR to GSM 30.

i)  The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance for the bits of class II, by examining sequences of Max-samples consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.  The number of error events is recorded.

j)  Steps g) to i) shall be repeated, for the MS operating on an ARFCN in the range 1 to 5513 to 523.

k)  Steps g) to i) shall be repeated, for the MS operating on an ARFCN in the range 120 to 124874 to 884.

II.4.7.3	Method of measurement for equipment with integral antenna

General

If the test below is carried out on an outdoor test site, the necessary precautions should be taken to prevent the signals in the frequency range 30 MHz to 4 GHz from causing harmful interference to services that might be in operation in the immediate vicinity.

Method of measurement

a)  In the anechoic shielded chamber or on the test site a calibrated measuring antenna (vertically polarised) shall be placed, connected to a measuring receiver.

b)  A test antenna, vertically polarised, connected to the System Simulator shall be in the anechoic shielded chamber or on the test site at a distance of at least 3 metres from the MS.

c)  At the position of the measuring antenna, a wanted signal is established with a field strength of 38.5dB46.3dBµV/m.

Note:	This field strength is 4dB above reference sensitivity.

d)  The measuring antenna is replaced by the MS, placed on a non conductive support, in the position for normal use.

Note:	The MS is at this point in the procedure exposed to a wanted signal, at a field strength 4 dB in excess of the the reference sensitivity value.

e)  A call shall be originated by the SS to the MS, and the MS shall be made to answer the call. The call shall occupy a full rate channel on one of the radio frequencies in the range 60 to 65690-710 (ARFCN).

f)  The MS shall be operated in the encrypted mode.

g)  The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be static and at the nominal frequency of the receiver.

h)  The SS is set to produce a static wanted signal and a static interfering signal at the same time. The interfering signal shall be available from a separate output port on the SS. The interfering signal shall be fed to an antenna, selected in accordance with the frequency of the interfering signal. This antenna, radiating the interfering signal shall be in the anechoic shielded chamber or on the test site at a distance of at least 3 metres from the MS, except at frequencies above 900 MHz when a distance of 1 metermetre is acceptable.

i)  The unwanted signal shall be the Standard Test Signal I0. It shall inn turn be on all frequencies frequency in the range 80 MHz to 4 GHz at intervals of 200 kHz, with the exclusion of the range FR +/- 600 kHz (FR is the center frequency of the receiver).

j)  The level of the unwanted signal shall be set according to the table below.

The required field strength of the unwanted signal will be set to the values given in the table below.

Frequency	 Absolute Field Strength

80 MHz to 200 MHz	98.5 dBµV/m

200 MHz to 500 MHz	101.5 dBµV/m

500 MHz to 835 MHz	113.5 dBµV/m

835 MHz to 9151705 MHz	136.5142.3 dBµV/m

1705 MHz to 1785 MHz	130.3 dBµV/m

9151785 MHz to FR - 3 MHz	113.5116.3 dBµV/m

FR +/- 3 MHz to FR +/- 1.6 MHz	103.5109.3 dBµV/m

FR +/- 1.6 MHz to FR +/- 800 kHz	93.599.3 dBµV/m

FR +/- 800 kHz to FR +/- 600 kHz	93.599.3 dBµV/m

FR + 3 MHz to 9801920 MHz	113.5116.3 dBµV/m

1920 MHz to 1980 MHz	130.3 dBµV/m

9801980 MHz to 10002400 MHz	136.5142.3 dBµV/m

10002400 MHz to 4000 MHz	104.5110.3 dBµV/m

k)  The SS compares the data of the signal that it sends to the MS with the data which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication. The SS tests the RBER compliance for the bits of class II, by examining sequences of Max-samples consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given. The number of error events shall be recorded.

l)  Steps i) to k) shall be repeated with the MS operating on an ARFCN in the range 1 to 5513 to 523..

m)  Steps i) to k) shall be repeated with the MS operating on an ARFCN in the range 120 to 124874 to 884.

II.4.7.4	Limits

The number of error events recorded in this test shall not exceed the Max-events values given below when the Number of Samples = Max-samples.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

Channel	Type of	Max-events	Max-samples

measurement

TCH/FS

Class II	RBER	32	1300

For every frequency at which the MS fails the test, the procedure stated in II.4.7.2 or II.4.7.3 shall be repeated by using the following greater values of Max-events and Max-samples:

Max-events - 200		Max-samples - 8200.

A maximum of six failures are allowed in the range FR +/- 45 MHz, with the exclusion of FR +/- 800 kHz (which, if grouped, shall not exceed three 200 kHz channels per group), and a maximum of 24 failures are allowed in the range 100 kHz to 12.75 GHz for the equipment with antenna connector (or 80MHz to 4 GHz for the equipment with integral antenna) outside FR +/- 45 MHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

At those frequencies at which the failures occur the test stated in II.4.7.2 (or II.4.7.3) shall be repeated, but with a level of the unwanted signal set at 70 dBµV emf (or 93.5 99.3 dBµV/m for the equipment with integral antenna). The performance requirement is once again that stated above, that is:

Max-events - 200		Max-samples - 8200