II.6	Link Management

II.6.1	Synchronisation to the System

This chapter specifies the requirements concerning synchronisation of the MS to the system.

II.6.1.1	Network Properties

The MS has to cope with certain properties of the channels it has to synchronize to. These properties are summarized in this section II.6.1.1. All information given primarily reflects the fixed part of the network. However, since these properties affect the requirements for the MS, this information is summarized here in Rec. GSM 11.10-DCS.

The ability of the MS to cope with these network properties is implicitly tested under II.6.1.2 to II.6.1.4.  Thus no explicit measurements are described in this present section.

In the following, the term BTS refers to an entity controlling exactly one BCCH as well as all control and traffic channels associated to that BCCH.

II.6.1.1.1	BTS Frequency Tolerance

II.6.1.1.1.1	Absolute Tolerance

The carrier frequency of any channel transmitted by any BTS may have an absolute frequency error of +/- 0.05 ppm with respect to the nominal values.

II.6.1.1.1.2	Relative Tolerance

For all channels transmitted by one BTS, the carrier frequencies are locked to a common reference.

II.6.1.1.2	BTS Timing Tolerance

The timing tolerance of any two channels transmitted is the delay between the frame structures of these channels measured at the BTS antenna outputs.

For any two channels transmitted by the same BTS, the timing tolerance will be less than 1.0 microsecond. For channels transmitted by different BTSs, the timing tolerance may be undefined.

II.6.1.1.3	Synchronisation between Carrier and Data Clocks

For any channel transmitted by the BTS, the data clock frequencies are locked to the same reference as the carrier.

II.6.1.1.4	Multipath Effects

The MS shall cope with the fact that both the wanted and interfering signals may be influenced by any of the propagation conditions specified in GSM 05.05-DCS.

II.6.1.2	Receive/Transmit Delay

Whenever the MS transmitter is activated, proper timing in relation to the received frame structure must be provided and additionally all specifications laid down in II.3 must be fulfilled.

The absolute receive/transmit delay of the MS is the delay between a common burst reference point within the received and the transmitted RF burst.

Equivalently the delay can be referenced to the modulator input vs. the demodulator output or to the differential encoder input vs. the differential decoder output, provided the measured delay is corrected for the additional delays in the signal path. E.g., in the latter case at least an inherent delay of 2 bit periods due to the differential coding must be subtracted frpom the measured value.

For normal or dummy bursts, the common burst reference point is defined to be the transition from bit 13 to bit 14 of the midamble. For an access burst it is defined to be the transition from bit 48 to bit 49 of the burst.

The numbering of bits within a burst is defined in GSM 05.02 section 5.2 (which defines midambles as modulating bits).

II.6.1.2.1	Absolute Delay and Timing Advance Setting

Definition

The MS must set the TA value signalled to it.

Method of Measurement

The test is carried out under ideal radio conditions (Annex 1, GC3).

The MS does not use DTX, "MAX retrans = 7" and "TX-integer = 3" signalled on the BCCH.

After 10 s the SS pages the MS.

The first 7 channel requests from the MS shall not be answered, the 8th channel request from the MS on the RACH shall be answered by the SS with an immediate ASSIGNMENT, timing advance set to 0.

The MS is then brought into conversation state.

During the conversation state the values 10, 20, 30, 40, 50, 60, 63 and one random value other than these values for timing advance (TA) are signalled to the MS in consecutive SACCH blocks.

The SS shall monitor the TA value set in the L1 header on the uplink SACCH for each timing advance.

The absolute delays for all MS bursts are measured.

Requirements:

The measured receive/transmit delay shall equal the following nominal values with an absolute tolerance of +/- 1 bit period:

access bursts:	3 timeslots (= 45/26 ms)

normal bursts:	3 timeslots (= 45/26 ms) minus the last timing advance value received from the SS.

The MS shall use the new timing advance at the first TDMA frame belonging to the next reporting period after the SACCH frame containing the new TA value.

The value of the TA field in the uplink SACCH L1 header shall correspond to the most recently ordered value.

II.6.1.2.2	Reception Time Tracking Speed

Definition:

Reception time tracking speed is the speed at which the MS adapts its transmit time to a change in the timing of the received signal.

Method of Measurement:

a) The SS shall originate a call to the MS using a traffic channel with an ARFCN in the range 60-65 690-710.  The MS shall answer the call.

b) The SS introduces propagation condition TU50.

c) After 10 seconds the SS sets the input signal level to 11 13dBµVemf(  ) for a handheld and 9dBµVemf(  ) for all other classes. For the last second before step d) the SS takes an average receive/transmit delay of all bursts in that 1s.

d) The SS increases the delay of the transmitted signal to the MS by an 8 microsecond step a 2 bit step (7.4 us) and keeps this delay for 20 seconds.

The SS then decreases the delay by an 8 microsecond step and keeps this delay for 20 seconds.

e) The SS measures the absolute receive/transmit delay for each burst.

f) The SS increases the input signal level to 14 16dBµVemf(  ) and sets multipath condition RA250 130.

g) Steps c) to e) are repeated.

Requirements:

The MS shall adjust its absolute receive/transmit delay in steps of 1/4 bit period. This adjustment shall be performed at intervals of not less than 1 second and not more than 2 seconds until the timing difference is less than 1/2 bit period (rec GSM 05.10).

The MS shall adjust the timing of its transmit burst back to the correct receive/transmit timing delay. All burst timings shall be within the shaded part of the following figure.

See timing figure in GSM 11.10 V3.8.0

II.6.1.3	Access Times During Handover

In this section, the maximum access times allowed for the MS during resynchronization from one channel to another are tested.

II.6.1.3.1	Intra-Cell Handover

Definition

Intra-cell handover occurs when the MS is required to change from one channel/timeslot in the serving cell to another channel/timeslot in the same cell. Intra-cell handover can occur either to a timeslot on a new carrier or to a different timeslot on the same carrier.

Method of Measurement

a) The SS uses ARFCN 1 512 and ARFCN 124 885 to simulate two carriers on the same cell, under ideal radio conditions, with the following properties:

- Relative frequency tolerance: 0

- Relative timing tolerance   : 1/4 Bit.

b) The MS is brought into conversation state on ARFCN 1  ARFCN 512 timeslot 1, DTX not active.

c) The SS sends an ASSIGNMENT COMMAND to the MS allocating a full rate traffic channel on ARFCN 124 885 and timeslot 2 with a Power Command of 5.

d) After it has sent the assignment command the SS shall measure the reception time of bursts received on the new channel and the time that transmission ceases on the old channel.

Requirements:

1) The MS shall transmit its first burst on the new channel within 125ms from the last timeslot of the message block containing the Assignment Command.

2) The MS shall transmit its first burst on the new channel within 25ms from the last complete speech or data frame or message block sent on the old channel.

II.6.1.3.2	Inter-Cell Handover

Definition

Inter-cell handover occurs when the MS is required to change from a channel/timeslot in one cell to another channel/timeslot in a different cell.

Method of Measurement

a) The SS simulates two cells, A and B, under ideal radio conditions. A is the old cell and B is the target for the handover. Appropriate information on both cell A and B is given to the MS on the BCCHs of both cells.

b) The SS uses two channels with the following properties:

Cell A	TN=2	  ARFCN   1 512 with an offset of +267 320Hz

Cell B	TN=0	  ARFCN 124 885 with an offset of -267 320Hz.

This offset is representing worst cases for doppler shift at 250 130km/hr and frequency inaccuracy of 0.05ppm.

c) The simulated BCCHs for the two cells shall have the following differences in timing:

Timer T1	50

Timer T2	15

Timer T3	40

1/4 bit number	17

Timeslots	 2

d) The MS is brought into conversation state on a full rate traffic channel on cell A, DTX not active and timing advance 0.

e) The SS sends a HANDOVER COMMAND on the main DCCH on cell A ordering the MS to go to cell B on a full rate traffic channel (ARFCN=124 885, timeslot 0). Cells A and B are not synchronised cells. The Power Command is set to 5.

f) After the SS has sent the Handover command it shall measure the reception time of bursts received on the new channel and the time that transmission ceases on the old channel.

g) The SS also measures the absolute transmit/receive delay for the access bursts on the new channel.

h) The SS sends the Physical Information with timing advance set to 50. The SS shall then measure the reception time and absolute delay of the bursts transmitted on the new cell.

Requirements

1) The MS shall transmit its first burst on cell B within 125 ms from the last timeslot of the message block containing the Handover Command.

2) The MS shall transmit its first burst on cell B within 25 ms from the last complete speech or data frame or message block sent on cell A.

3) The MS shall transmit using the TA value in the physical information within 50 milliseconds from the end of the last timeslot of the message block containing the new TA value.

4) The measured absolute delay in steps g) and h) shall equal the following nominal values with an absolute tolerance of +/- 1 bit:

access burst  :  	3 timeslots (=45/26 ms)

normal bursts :  	3 timeslots (=45/26 ms) minus the timing advance value received from the SS.

II.6.1.4	Behaviour Under Insufficient Reception Conditions

II.6.1.4.1	Temporary Reception Gaps

Definition:

If reception is interrupted for a certain limited time (temporary reception gap) the MS must maintain the frequency and timing of its transmission within specifications.

Ref: 	GSM 05.10 sections 6.1 and 6.2

GSM 05.08-DCS section 5.2

Method of Measurement

The MS is put into idle state with RADIO_LINK_TIMEOUT = 64 and "DTX OFF" signalled on the BCCH. After 10 s the MS is brought into conversation state. A temporary reception gap is introduced as follows:

a) In a TDMA frame immediately following the transmission of a complete SACCH block the downlink signal is removed for 63 SACCH blocks (the maximum temporary reception gap - the radio link counter S decrements by 1 for each SACCH block not successfully decoded and hence now S=1).

b) The frequency and timing of the MS transmitter are measured immediately before and at least 5 times at approximately equally spaced intervals during the gap, one of these measurements being at the end of the gap.

c) The downlink transmission is resumed for a period sufficient to allow reception of 1 SACCH block (the radio link counter increments by 2 for each SACCH block successfully decoded and hence now S=3).

d) The downlink transmission is again removed for a period equal to at least 3 SACCH blocks. The MS transmission frequency and timing are measured immediately before and during the reception gap (as the MS fails to decode each of these SACCH blocks the radio link counter S decrements by 1 and hence at the end of 3 failed SACCH blocks S=0).

Requirements

1) The MS carrier frequency shall be accurate to within 0.2 ppm compared to signals received from the SS.

2) The receive/transmit delay timing shall be 3 timeslots +/- 1 bit.

3) During the second reception gap the MS shall maintain transmission for a period up to but not exceeding 3 SACCH blocks (when the radio link counter S reaches zero a radio link failure is declared and the MS transmission ceases).

4) During the first, maximum, reception gap the MS transmission timing may have drifted resulting in an error of not greater than +/- 1.3446.048 µs. The error at the start of the reception gap (not greater than +/- 2 µs ) will have been determined by the first measurement of MS transmission frequency and timing.

II.6.1.5	Channel release after unrecoverable errors		ref: GSM 03.06 and 05.08-DCS

Radio link failure

The aim of determining radio link failure in the MS is to ensure that calls with unacceptable voice/data quality, which cannot be improved either by RF power control or handover, are either reestablished or released in a defined manner. In general the parameter that control the forced release should be set such that the forced release will not normally occur until the call has degraded to a quality below that at which the majority of subscribers would have manually released. This ensures that, for example, a call on the edge of a radio coverage area, although of bad quality, can usually be completed if the subscriber wishes.

II.6.1.5.1	Purpose of the test and introduction

This test will verify the management by the MS of the radio link failure criterion: the counter S.

The counter S is initialized with the value of the parameter RADIO_LINK_TIMEOUT.

Call re-establishment is not allowed in any cell.

The parameter is sent on a BCCH inside a message System Information type 3. It can also be sent on SACCH inside a message System Information type 6, but is taken in account by the MS only in case of hand over or channel assignment.

If the MS is unable to decode SACCH message, counter S is decreased by 1; in case of successful reception, counter S is increased by 2, but will not exceed the value of RADIO_LINK_TIMEOUT.

If counter S reaches 0 a radio link failure shall be declared, the MS aborts the RR connection, the call control entity ask for a re-establishment procedure.

The parameter RADIO_LINK_TIMEOUT can take 16 values N with

N = 4 * n while 1<= n <= 16.

between 1 * 4 and 16*4.

II.6.1.5.2	Method of test A

To initialise the counter S in the MS, on the BCCH on which the MS camps a message System Information is sent with the a randomly chosen value N n*4 for the parameter RADIO_LINK_TIMEOUT

1  <=  n  <=  16.

The mobile originates a call according to chapter "structured procedures".

Once tThe call is established, with the system simulator unrecoverable errors will be introduced inside the SACCH messages by using the System Simulator..

The system simulator will send 32 errorfree SACCH messages, then N 4 * n erroneous SACCH messages, and then continuously errorfree SACCH messages.

Repetition of the test:

The test shall be repeated once with a different value of N.

II.6.1.5.3	Requirements A

After receiving the N 4 * n erroneous SACCH messages, the MS shall send a CM re-establishment request message abort the RR-connection, i.e that there is no more activity on the SDCCH channel.

The test will be repeated once with  a different value for n (1 <= n <= 16).

II.6.1.5.4	Method of test B

The counter S is initialised with the a randomly chosen value N of the parameter RADIO_LINK_TIMEOUT.

A call isshalll be established as in method A..

The system simulator is programmed to send 32 errorfree SACCH message. Then it will send 2 SACCH messages with unrecoverable errors followed by one error free SACCH message This step is repeated 64 times.

Repetition of the test:

The test shall be repeated once with a different value of N.

II.6.1.5.5	Requirements B

The MS shall not abort the RR-connection never send CM re-establishment message.

The test will be repeated with different values for n (1 <= n <= 16).

II.6.1.5.6	Method of test C

The counter S is initialised with  thea randomly chosen value N of the parameter RADIO_LINK_TIMEOUT.

A call is shall be established as in Method A.

The system simulator is programmed to send 32 error free SACCH messages. Then it will send 3 SACCH messages with unrecoverable errors followed by 1 error free SACCH message. This step is repeated 4 * n N - 2 times. Then the SS will send continuously errorfree SACCH message.

Repetition of the test:

The test shall be repeated once with a different value of N.

II.6.1.5.7	Requirements C

After receiving the 3 * (N - 2)4 * n erroneous SACCH messages the MS shall send a CM re-establishment request message abort the RR-connection i.e there is no more activity on the SACCH channel.

The test will be repeated once with  a different value for n (1 <= n <= 16).

II.6.1.6	Cell Selection/Reselection

II.6.1.6.1	General Points

GSM 05.08-DCS does not give times for how quickly an MS has to select or reselect a cell. For the purposes of these tests, the following times are allowed:

(a) Time to read and average signal strength on 124374 RF carriers. If onetwo signal strength samples isare read on an RF carrier every TDMA frame, the time to take 5 samples on each is about 34.5 seconds. However the averaging time is 3-5 second or 5 paging block intervals (which equals 5.9 s for the case of BS_PA_MFRMS= 5 frames). 6 seconds will be allowed.

(b) Time to attempt to read SCH on a BCCH carrier. SCH is sent 5 times every 235.4 ms.  We will allow 0.5 s per carrier on which SCH is searched for. Hence for a BA with 16 carriers 8 seconds will be allowed.

(c) Time to read a maiximum of 5all 4 System Information messages on one BCCH carrier. GSM 05.02 specifies that the all system information blocks should be sent every 8 multiframes, i.e. 1.9 s.

(d) Time to do random access (RA) attempts. Assuming a non combined CCCH and MAX retrans = 1, the whole RA attempt process will take around 1 second.

(e) Time between paging blocks for an MS - This is 1.2 s for BS_PA_MFRMS = 5.

For the cell selection tests below, the maximum response time will be (a) + (b) + 4(c) + (d) + (e) = 23.8 s.  A time of 30 s is allowed in the tests below.  For cell reselection, the maximum response time is (a) + (c) + (d) + (e) = 10.1 s.  A time of 12 s is allowed in the tests below. These times are assumed to be after self test has been completed and any PIN entry accepted.

The accuracy of signal level measurements is assumed to be up to +/- 6 dB absolute and +/- 2 dB relative between carriers. The tests below allow, for cases of discrimination between C1 and 0, a difference of at least 8 dB, and for discrimination between C1 values on different carriers, a difference of at least 5 dB.

Note:	The timing tolerances in all the tests shall be -0%, +10%.

II.6.1.6.2	Definition - Cell Selection

Cell Selection is the process in which an MS selects a suitable cell to camp on.  The algorithms to be followed are given in GSM 05.08-DCS section 6.1 to 6.4.

II.6.1.6.3	Method of Test

a) A number of BCCH carriers are transmitted by the SS. The System Information on these carriers is as given in Table II.6-1 unless otherwise stated. The ARFCN and power levels of each of the carriers are as given in case 1 of Table II.6-2, and the System Information parameters on these carriers which are different to those in Table II.6-1 are given in Table II.6-2.

b) Paging messages for the MS are transmitted on each of the BCCH carriers in each of the paging blocks of the MS  (as defined in GSM 05.02). The SS ignores any Random Access (RA) messages sent in response to pages, unless otherwise stated.

c) The MS is switched on.

d) The SS checks that MS makes a response according to Requirement 1 of case 1 of Table II.6-2 within 30 seconds.

e) The MS is turned off.

f) Steps c) to e) are repeated, this time checking that the MS makes a response according to Requirement 2 of case 1 of Table II.6-2 within 30 seconds.

g) Steps a) to f) above are repeated for cases 2 to 4 of Table II.6-2. (The System Information parameters in Table II.6-2 only apply to the case given and are not continued into the next case unless explicitly stated.)

II.6.1.6.4	Requirement

The MS shall respond as in d) and f) above for each case in Table II.6-2.

II.6.1.6.5	Definition - Cell Reselection

Cell reselection is the process in which the MS ensures that it is camped on the most appropriate cell.  The algorithms to be followed are given in GSM 05.08-DCS sections 6.4 to 6.6.

II.6.1.6.6	Method of Test

Note:	The SS looks for a response from the MS, when required, within 12 s, unless otherwise stated.

a) The SS does steps a) to c) of case 1 of the cell selection test above, but with RAM=-93 on carrier 6. (The values of C1 on the 6 BCCH carriers are then respectively 25, 15, 10, 20, -8, <=5.)

b) The SS checks that the MS does at least one RA request on carrier 3 within 30 s followed by at least one RA request on carrier 4 within a further 12 s.

Note:	Carrier 3 has a higher level than Carrier 4 but a lower C1. Therefore initial cell selection should choose Carrier 3 and cell reselection should choose carrier 4.

c) The SS stops all paging messages and then sets CBA=0 on carrier 1. After 6 minutes the SS pages the MS once on carrier 1 and repeatedly on carrier 4 in every paging block of the MS until the end of this test. (6 minutes allows time for the MS to read the BCCH data on carrier 1. The MS should then reselect carrier 1.)

d) The SS checks that the MS makes at least one RA request on carrier 1 followed by at least 1 RA request on carrier 4. (The MS should temporarily reselect carrier 4 because of lack of response to RA requests on carrier 1, and then revert to carrier 1.)

e) The SS transmits carrier 1 at 68 dBmicroVoltemf(  ) for 10 s, 38 dBmicroVoltemf(  ) for 2 s, 68 dBmicroVoltemf(  ) for 10 s and back to 48 dBmicroVoltemf(  ). (This gives a minimum averaged level of 48 dBmicroVoltemf(  ), i.e. minimum C1 of 25.)

f) The SS checks that the MS does not make a RA request on either carrier 1 or carrier 4.

g) The SS waits 10 seconds, then stops transmitting on carrier 1 for 18 seconds, and then transmits as before. (The averaging of signal level can take 6 s, C1 is checked at least every 5 s, and C1 low must be detected for 5 s. Adding 10% tolerance gives 17.6 s.)

h) The SS checks that the MS makes at least 1 RA request on carrier 4.

i) The SS changes the ARFCN of carrier 4 to 18 and then waits 120 seconds (to ensure the MS reselects carrier 1). It then puts random data into 4 successive paging blocks of the MS on carrier 1, and then reverts to sending normal data. (DSC will be initialised to 90/5 = 18, hence after 4 erroneous blocks, DSC will equal 2, hence no signalling failure.)

j) The SS checks that the MS makes no RA requests on either carrier 1 or carrier 4.

k) After at least 16 successive paging blocks of the MS on carrier 1 with normal data, the SS puts random data into 5 successive paging blocks of the MS followed by normal data. (This will cause DSC to reach 0.)

l) The SS checks that the MS makes at least one RA request on carrier 4.

m) The SS sets CBA = 1 on carriers 1 and 4.

n) The SS waits 1 minute and then pages the MS on carriers 1,2,3 and 4.

o) The SS checks that the MS makes at least one RA request on carrier 3 but none on carrier 1.

p) The SS pages the MS on carriers 2 and 4 and checks that the MS makes no RA request on carriers 2 and 4.

q) The SS sets CBA=O on carriers 1 and 4.

r) The SS sets the level on carrier 3 to 33 dBmicroVoltemf(  ) (giving C1 = 5).

s) The SS waits 6 minutes.

t) The SS reduces the level on carrier 1 to 38 dBmicroVoltemf(  ) for 18 seconds and then reverts to 48 dBmicroVoltemf(  ). (This gives C1 = 15 on carrier 1 and 20 on carrier 4.)

u) The SS checks that the MS makes at least one RA request on carrier 4.

v) The SS waits for 30 seconds (to allow the MS to revert to carrier 1).  It then sets LAC on carrier 4 to 1112(Hex) and reduces the level of carrier 1 to 38 dBmicroVoltemf(  ) for 18 seconds, and then reverts to 48 dBmicroVoltemf(  ).

w) The SS checks that the MS makes no RA requests on carriers 1 and 4.

x) The SS reduces the level on carrier 1 to 28 dBmicroVoltemf(  ) for 18 seconds (giving C1 = 5 on carrier 1, i.e. more than CRH below C1 on carrier 4) and then reverts to 48 dBmicroVoltemf(  ).

y) The SS checks that the MS makes at least one RA request on carrier 4.

z) The SS waits for 30 seconds and then switches off all the carriers for 20 seconds. It then sets up the carriers again according to case 1 of Table II.6-2 and pages the MS repeatedly on all the carriers in every paging block of the MS.

aa) The SS checks that the MS makes at least one RA request within 30 s on carrier 3 followed by at least one RA request on carrier 4. (For the reasons given in b) above.)

II.6.1.6.7	Requirement

1) The MS shall respond as indicated in steps b), d), f), h), j), l), o), p), u), w), y) and aa).

II.6.1.6.8	Definition - Cell Selection after Release of TCH and SDCCH

The requirement for cell selection after release of a TCH or SDCCH is given in GSM 05.08-DCS section 6.7.

II.6.1.6.9	Method of Test

a) The SS does steps a) to c) of Case 1 of the cell selection test above, but with no pages being sent and with RAM = -70 on carriers 4 and 6. (This gives negative C1 on both carriers.)

b) The SS sets up a call by paging the MS on BCCH carrier 3 and setting up a call on a TCH with ARFCN of 22534.

c) The SS starts paging the MS repeatedly on all the BCCH carriers.

d) The SS causes a normal channel release.

e) The SS checks that the MS does at least one RA request on carrier 3, but none on carrier 5.

f) Steps a) and b) above are repeated.

g) The TCH is switched off. (This triggers call re-establishment.)

h) The SS checks that the MS does at least one RA request on Carrier 3 within 12 s.

i) Steps a) and b) above are repeated but with RE=1 on carrier 3 and RAM = -100 on carrier 4.

j) The TCH is switched off.

k) The SS checks that the MS does at least one RA request on carrier 4 within 12 s.

II.6.1.6.10	Requirements

The MS shall respond as indicated in steps e), h) and k) above.

II.6.1.6.11	Definition - Cell Selection (Abnormal Cases and Emergency Calls)

Cell selection in abnormal conditions is defined in GSM 05.08-DCS section 6.8.

II.6.1.6.12	Method of Test

a) The SS performs steps a) to c) of case 1 of the cell selection test, but with all BCCH carriers having the same PLMN which is not the home PLMN of the MS. Any automatic reselection of a PLMN other than the MS's home PLMN shall be disabled. No manual reselection of PLMN shall be done during this test.

b) The SS checks that no RA request is made by the MS on carriers 2 or 3.

c) 60 seconds after the MS is switched on an emergency call is initiated on the MS  (30seconds for cell selection of carrier 2 or 3 which have the highest signal strength, 18 seconds for comparison of C1, and 12 seconds for reselection of carrier 4 which has the highest C1).

d) The SS checks that the MS makes a RA request on carrier 4. (Carrier 4 has the highest value of C1 among the unbarred cells.)

e) The SS sets the PLMN of carrier 2 to be the MS's home PLMN and starts paging the MS repeatedly on carrier 2.

f) The SS checks that the MS makes a RA request on carrier 2 within 42 seconds (30 seconds for MS detection of the change in PLMN, and 12 seconds for reselection of carrier 2).

g) The MS is switched off, the SIM is removed, and the MS is switched on again.

h) The SS checks that no further RA requests are made by the MS on carrier 2 within 60 seconds.

i) 60 seconds after the MS is switched on an emergency call is initiated on the MS (30seconds for cell selection of carrier 2 or 3 which have the highest signal strength, 18 seconds for comparison of C1, and 12 seconds for reselection of carrier 4 which has the highest C1).

j) The SS checks that the MS makes a RA request on  carrier 4  within  12 s.

II.6.1.6.13	Requirement

The MS shall respond as indicated in steps b), d), f), h) and j) above.

Parameter	Reference in	Abbre-	Normal Setting

GSM 04.08-DCS	viation

Cell Channel Description	10.5.2.1	Any values

MAX retrans	10.5.2.17	-	1

TX-integer	10.5.2.17	-	Any value

CELL_BAR_ACCESS	10.5.2.17	CBA	0 (i.e. no

barred)

AC CN	10.5.2.17	AC	All O

RE	10.5.2.17	RE	O (i.e. re-establishment allowed)

BA ARFCN	10.5.2.13	BA	All O except for ARFCN=3,9,18,25,41,

43,49,50,54,58,62,

66,70,80,92,124512,543

568,589,602,641,662,683,

696,711,732,754,794,851,

870,871,872,884,

NCC	10.5.2.15	NCC	Any value

Cell Identity	10.5.1.1	-	Any value

MCC, MNC	10.5.1.3	PLMN	Home PLMN of MS

LAC	10.5.1.3	LAC	1111 (Hex)

ATT, B_AG_BLKS_RES,T3212	10.5.2.8	-	  Any values

BS_PA_MFRMS	10.5.2.8	BPM	5 frames

Cell Options	10.5.2.3	-	Any values

CELL_RESELECT_HYSTERESIS    10.5.2.4	CRH	10 dB

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Maximum RF output power of MS.

(P in GSM 05.08-DCS)

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-90 dBm

Table  II.6-1 :  Normal System Information Fields

Case 1	Case 2	Case 3	Case 4

BCCH 	Level	48 dBm	60/30**	60/30**	60/30**

dBµVemf(  )

Carrier 1	Syst Info	CBA=1	MTMC	AC=1	Normal

=1314 dBm	for MS

ARFCN	62543	62543	62543	2543

BCCH	Level	38	65	65	65

dBµVemf(  )

Carrier 2	Syst Info	PLMN not	PLMN not	PLMN not	PLMN not

that of MS	that of MS	that of MS	that of MS

ARFCN	124884	124884	124884	74790

BCCH	Level	38	38	38	38

dBµVemf(  )

Carrier 3	Syst Info	RAM=-85	RAM=-85	RAM=-85	RAM=-85

ARFCN	54696	54696	54696	4604

BCCH	Level	33	33	33	33

dBµVemf(  )

Carrier 4	Syst Info	RAM=-10098	RAM=-10098	RAM=-98100	RAM=-10098

ARFCN	58711	58711	58711	8608

BCCH	Level	43	43	43	43

dBµVemf(  )

Carrier 5	Syst Info	RAM=-62	RAM=-62	RAM=-62	RAM=-62

ARFCN	66_851	66851	66851	16630

BCCH	Level	43/13*	61/31*	61/31*	61/31*

dBµVemf(  )

Carrier 6	Syst Info	RAM=-98	Normal	Normal	Normal

ARFCN	70568	70568	70568	20660

The requirements that apply in the various cases are listed below.

*	Power is alternately the higher level for 0.5 s and the lower level for 3.5 s.

**	Power is alternately the higher level for 3.5 s and the lower level for 0.5 s.

Table  II.6-2 :  Cell Selection Test Parameters

Notes on Table II.6-2

Through these tests the system information type 2 and 2 bis will be tested by forcing neighbour cells description on two messages and verifying the correct cell selection.

Case 1:	The averaging on carrier 6 should result in an average level in the range 25 to 13 dBmicroVoltemf(  ). The values of C1 on the 6 BCCH carriers are respectively: 25,15,10,20,-8,10 to -2. Carrier 3 is the one with the highest level which has C1 > 0, is not barred and is of the correct PLMN.

Case 2:	The averaging on Carriers 1 and 6 should result in an average level in the range of respectively: 60 to 48 dBmicroVoltemf(  ) and 43 to 31 dBmicroVoltemf(  ). Carrier 1 is the carrier with the highest level that satisfies the other constraints for cell selection.

Case 3:	The MS should select Carrier 1 as for Case 2, but because AC is set, the MS should not make any response to the SS.

Case 4:	Any stored BA information will be incorrect. The MS should therefore do a search of all 124374 GSM carriers. It should then read BCCH data on Carrier 2 followed by Carrier 1. As Carrier 2 is the wrong PLMN, the BA on the carrier should be ignored.

Requirement 1 in connection with Table II.6-2

In case 1:	RA response on carrier 3 before a response (if any) on carriers 4 or 6.

In case 2:	RA response on carrier 1 before a response (if any) on carriers 3 or 4.

In case 3:	No RA response on carriers 1, 2, or 3.

In case 4:	RA response on carrier 1 before a response (if any) on carriers 3 or 4.

Requirement 2 in connection with Table II.6-2

In case 1:	No RA response on carriers 1, 2 or 5.

In case 2:	No RA response on carriers 2, 5 or 6.

In case 3:	No RA response on carriers 4, 5 or 6.

In case 4:	No RA response on carriers 2, 5 or 6.

II.6.2	RX Measurement

For evaluating the reception quality (the basis for handover and power control) the following two criteria are used:

-  signal strength (RXLEV)

-  signal quality (RXQUAL).

II.6.2.1	Signal Strength

II.6.2.1.1	Definition

The MS must be capable of measuring the received RF signal strength from reference sensitivity level up to 65dBmicroVolt emf.  The measured RF signal strength has to be mapped by the MS into RXLEV values using the coding scheme specified in GSM 05.08-DCS section 8.1.4.

II.6.2.1.2	Method of Measurement

a) The MS is connected to the SS. This connection will be direct for a MS having an antenna connector, or via an antenna coupling device for a MS with an integral antenna.

b) The SS is set to produce the BCCH of the serving cell and the BCCHs of 6 adjacent cells at 28 dBmicroVolt emf( ). The BCCH of the serving cell shall indicate these BCCHs.

c) After 30 s the MS is brought into conversation state. The envelopes of the TCH of the serving cell and the BCCHs of the neighbouring cells shall be static. The SACCH shall indicate the same neighbouring BCCHs as the BCCH of the serving cell at step b).

The SS shall wait 2 s before step d).

TCH	 BCCH1	BCCH2	BCCH3	 BCCH4	BCCH5	BCCH6

Step 	ARFCN:	1512	62700	124885	 20585	40660	 80735	100810

1+m*21		64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10

2+m*21		64.5-m*10	63.5-m*10	4547-m*10	5557-m*10	75-m*10	1517	 2527

3+m*21		64.5-m*10	62.5-m*10	4547-m*10	5557-m*10	75-m*10	1517	 2527

.		 .	 .	.	.	.	 1517	 2527

.		 .	 .	.	.	.	 1517	 2527

.		 .	 .	.	.	.	 1517	 2527

21+m*21		64.5-m*10	44.546.5-m*10	4547-m*10	5557-m*10	75-m*10	1517	 2527

m = 0, 1, 2, 3, 4.

TABLE  II.6-3 :  Average signal levels at RX input in dBµV(  ) emf

d) The measurement is done in 105 steps.  The initial signal levels of the TCH of the serving cell and the BCCHs of the neighbouring cells shall be adjusted according to Table II.6-3. At each step the SS keeps the signal levels stable for one reporting period.

At steps 1 to 30 the SS shall simulate a base station with DTX off and at steps 31 to 105 the SS shall simulate a base station with DTX on.

At steps 1 to 30 the SS shall check the accuracy of the measured signal strength of serving cell by checking the values of the parameters RXLEV-FULL and RXLEV-SUB. At steps 31 to 105 the SS shall check only the value of the parameter RXLEV-SUB.

At step 84, within every 480 ms reporting period, out of the 4 SACCH and 8 SID timeslots the SS shall transmit the first six active timeslots of the TCH with signal level 42.5 dBmicroVolt emf( ) and the last six active timeslots of the TCH with signal level 26.5 dBmicroVolt emf( ).

e) The MS is placed in the climatic test chamber, and step d) is repeated for the following combinations of temperatures and power supply voltages:

Temp	:	Hi	Hi	Lo	Lo

Voltage	:	Hi	Lo	Lo	Hi

II.6.2.1.3	Requirements

1) Relative Accuracy

1.1) Relative Accuracy over the Frequency Band

For each of the steps 1, 22, 43, 64, 85 the 7 reported RXLEV values shall be checked. The minimum reported RXLEV value shall be subtracted from the maximum reported RXLEV value. The result for each of the five steps shall be less than or equal to 2.

1.2) Relative Accuracy at a single frequency, BCCH1

The reported value of RXLEV shall meet the following formula for signal levels above the sensitivity level:

+1 	 +2 Note 1) 	 +3 Note 2)

RXLEVn = ( RXLEV(m*21+1) - n + m*21 + 1 ) 	{    , 	{     , 	{

-1 	 -1	 -1

where	m = 0, 1, 2, 3, 4  (see Table II.6-3);

n = number of step  so, that  n > m*21  and  n <= (m+1)*21.

n = number of step where n takes all values from n=m*21 to n = (m+1)*21

Note 1:	This tolerance applies to signal levels below 2725 dBµV emf ( )for handhelds and to signal levels below 23 dBµV emf for other types of MSs.

Note 2:	This tolerance applies to signal levels below 1412 dBµV ( )emf forhandhelds and to signal levels below 10 dBµV emf for other types of MSs.

Note 3:	The relaxation for low signal levels is necessary due to the effect of receiver noise in the measurement results.

2) Absolute Accuracy

For each of the steps 1, 22, 43, 64, 85 the reported RXLEV values shall meet the following requirements:

For normal test conditions:

│RXLEVMS - RXLEVc│ <=	(  4  for steps 64, 85;

(  6  for steps 1, 22, 43;

where	RXLEVMS	=  reported RXLEV value;

RXLEVc	=  correct  RXLEV value as stated in the Table II.6-4                   below.

For extreme test conditions:

│RXLEVMS - RXLEVc│ <=  6  for steps 1, 22, 43, 64, 85.

RXLEV		for Received Signal Level

0		less than 3 dBmicroVolt emf

1		3 dBmicroVolt emf to 4 dBmicroVolt emf

2		4 dBmicroVolt emf to 5 dBmicroVolt emf

.			.

.			.

.			.

62		64 dBmicroVolt emf to 65 dBmicroVolt emf

63		greater than 65 dBmicroVolt emf

TABLE  II.6-4 :  Correct values of RXLEV

For input signal levels below sensitivity level the reported value of RXLEV may be any value between that reported at sensitivity level and the actual input signal level, but never a value higher than that reported at sensitivity level.

3) At step 64 the MS shall report RXLEV of the TCH to be 32+/-4 (average of dB values).

II.6.2.2	Signal Quality

Definition

The MS must be capable of measuring the received signal quality, which is specified in terms of bit error ratio (BER) before channel decoding averaged over the reporting period of length of one SACCH multiframe defined in Section 8.4 of GSM 05.08-DCS. The MS has to map this BER into RXQUAL values using the coding scheme defined in Section 8.2.4 of GSM 05.08-DCS.

In order to assess the correct operation of the quality reporting algorithm in the MS, the SS will measure the BER of class II bits and will assume that it is equal to that before channel coding.

The test is performed for two channel propagation conditions: static and TU50. In static conditions two kind of channels are considered: full rate and DTX. In TU50 conditions, only the full-rate channel is considered.

II.6.2.2.1	Test on signal quality under static conditions

II.6.2.2.1.1	Method of measurement

a) The MS shall be connected to the SS.  A call shall be originated by the SS to the MS in the range ARFCN 60-65690-710 and the MS shall be made to answer the call.  The MS is operated under normal test conditions. The SS is set to produce a wanted signal and an independent uncorralated interfering (unwanted) signal at the same time, both with static propagation characteristics. The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and its level shall be 28 dBuVemf ( ). The unwanted signal shall be on the same channel as the wanted signal but shall have no fixed relationship with the bit transitions of the wanted signal, and it shall be modulated with random data. The MS shall be operated in the encrypted mode.

b) The SS commands the MS to create the loop back facility from the receiver channel decoder output to the transmitter encoder input.

c) For each level of the unwanted signal varying, at 1 dB steps, in a range (presumablyfor example from 13 dBuVemf ( ) to 33 dBuVemf ( )) roughly corresponding to a class II bits BER from 0.05% to 20%, the SS counts the number of the class II bits for which 200 errors occur and determines the corresponding BER.

d) A curve of BER of class II bits is drawn vs the ratio of the level of the wanted to the unwanted signal (C/I) and the correspondence of the table below is derived from it.

CASE	AVERAGE BER	C/I

0	 0,09%	(C/I)0

1	 0,28%	(C/I)1

2	 0,57%	(C/I)2

3	 1,13%	(C/I)3

4	 2,26%	(C/I)4

5	 4,53%	(C/I)5

6 	 9,05%	(C/I)6

7	18,10%	(C/I)7

Note:	If the shape of the BER vs the C/I were too sharp, a curve of BER vs the received power level C should be alternatively derived, and C, instead of C/I, should be used in the next steps of the measurement procedure.

e) The level of the unwanted signal is set at a value for which C/I = (C/I)i, with i equal to one out of the numbers 0,1, ... 7.  At such C/I the expected quality band reported by the MS is RXQUAL_i.

f) At the end of each SACCH block, the SS increases by 1 a counter of the number of errors in reporting the RXQUAL band only if such band is different from the expected one (that is RXQUAL_i). Moreover a counter of the number of errors of the class II bits is properly increased.

g) The procedure stated in f) is repeated 20 times. If the ratio between the number of errors of the class II bits and the number of the class II bits occurring within 20 SACCH blocks is different for more than +7% (or -7%) from the value of the BER corresponding to the case i, the level of the unwanted signal shall be changed, at step of -0.2 dB (or +0.2 dB) and, at each step the BER of the class II bits shall be evaluated (by counting at least 200 errors) until the BER becomes again equal to that corresponding to the case i with a tolerance of +7% or -7%.

h) Steps f) and g) shall be repeated until the number of samples of case i is equal to Max-samples_i, or the number of errors in reporting the RXQUAL band is equal to Max-events_i.

i) Steps from e) to h) shall be repeated for each of the (C/I)i values with i = 0, 1, ... 7.

j) The SS is set to simulate a base station in DTX mode according to section 8.3 of GSM 05.08 and steps from e) to i) shall be repeated except that in step g) the number of SACCH blocks will be 240 instead of 20.

II.6.2.2.1.2	Requirements (static conditions)

The number of errors in reporting the RXQUAL bands recorded in each of the considered cases shall not exceed the corresponding value of Max-events shown in the following tables, when the number of samples relevant to the case under test is equal to Max-samples.

The two tables below refer to RXQUAL_FULL bands (relevant to the full-rate channel quality evaluation) and RXQUAL_SUB bands (relevant to the quality evaluation of the channel operating in DTX mode) respectively.

CASE	Expected	Specified	Max-events	Max-samples

RXQUAL_FULL	reporting

error rate

0	RXQUAL_O	10%	200	 1660

1	RXQUAL_1	25%	200	 665

2	RXQUAL_2	15%	200	 1105

3	RXQUAL_3	10%	200	 1660

4	RXQUAL_4	10%	200	 1660

5	RXQUAL_5	 5%	200	 3300

6	RXQUAL_6	 5%	200	 3300

7	RXQUAL_7	 5%	200	 3300

CASE	Expected	Specified	Max-events	Max-samples

RXQUAL_SUB	reporting

error rate

0	RXQUAL_0	35%	300	750

1	RXQUAL_1	75%	400	495

2	RXQUAL_2	70%	400	525

3	RXQUAL_3	65%	400	560

4	RXQUAL_4	45%	300	590

5	RXQUAL_5	35%	300	750

6	RXQUAL_6	25%	200	665

7	RXQUAL_7	15%	200	1105

II.6.2.2.2	Test on signal quality under TU50 propagation conditions

II.6.2.2.2.1	Method of measurement

a) The MS shall be connected to the SS. A call shall be originated by the SS to the MS in the range ARFCN 69060 to 65710 and the MS shall be made to answer the call. The MS is operated under normal test conditions.  The SS is set to produce the standard test signal C1 with TU50 propagation profile. It shall be at the nominal frequency of the receiver at a median level of 28dBuVemf(  ). The MS shall be operated in the encrypted mode.

b) The SS commands the MS to create the loop back facility from the receiver channel decoder output to the transmitter encoder input.

c) The SS counts the number of errors of the class II bits (occurring in 50 SACCH blocks) and the relevant BER is computed. If such BER does not correspond to one out of the 8 different cases shown in the table below, the test shall be continued from step f). On the contrary, if such BER corresponds to one out of the 8 different cases shown in the table below, let us say case i (with i equal to 1 out of the 8 numbers 0, 1, ... 7), the procedure stated from step d) on shall be adopted.

d) The SS records all the RXQUAL bands reported at the end of each of the 50 SACCH blocks. A counter Max-samples_i shall be increased by 50, and the SS verifies whether each of the quality band reported by the MS is equal to RXQUAL i (with i = 0, 1, ... 7), or to one of the adjacent bands RXQUAL_(i-1) (with i = 1, 2, ..., 7) and RXQUAL_(i+1) (with i = 0, 1, ..., 6). For each failure (to be in the correct or one of the adjacent bands) that is found, if any, a counter Max-events_i shall be increased by one.

CASE	AVERAGE BER (%)

OF THE CLASS II BITS

0	< 0.10

1	0.26 TO 0.30

2	0.51 TO 0.64

3	1.0  TO 1.3

4	1.9  TO 2.7

5	3.8  TO 5.4

6	7.6  TO 11

7	> 15

e) Step c) shall be repeated 20 times.

f) The SS is set to produce also an independent, uncorrelated interfering (unwanted) signal with TU50 propagation profile. The unwanted signal shall be on the same channel as the wanted signal but shall have no fixed relationship with the bit transitions of the wanted signal and it shall be modulated with random data. Its median level shall be 25 dB below the median level of the wanted signal.

g) The SS counts the number of error of the class II bits occurring in 50 SACCH blocks and the relevant BER is computed.

h) If such BER does not correspond to one out of the 8 cases shown in the table above, the median level of the unwanted signal shall be increased (or decreased) by 0.6 dB and step g) shall be repeated. On the contrary, if the computed BER correspond to one out of the 8 cases shown in the table above, step d) shall be repeated.

i) Steps g) and h) shall be repeated 20 times.

j) The median level of the unwanted signal is increased at step of 2 dB up to a value of 5 dB above that of the wanted signal and, at each step, the procedure stated from g) to i) shall be repeated.

k) Step j) shall be stopped when Max-samples is greater than 200.

Note:	If, at the end of the whole test, one or more Max-samples_i (with i = 0, 1, ..., 7) is lower than 200, the corresponding case shall be skipped out, being statistically not significant.

II.6.2.2.2.2	Requirements (TU50 conditions)

For each of the examined cases of the table above, the error rate in reporting the quality band is computed as the ratio between the recorded number of reporting errors Max-events_i, and the corresponding number of the samples Max-samples_i.  The test is passed if the computed reporting error rate does not exceed the Max_reporting error rate shown in the table below.

CASE	Expected	Specified	Max_reporting

RXQUAL_FULL	reporting	error rate

error rate

0	 RXQUAL_0/1	 15%	18%

1	 RXQUAL_1/0/2	 15%	18%

2	 RXQUAL_2/1/3	 15%	18%

3	 RXQUAL_3/2/4	 10%	12%

4	 RXQUAL_4/3/5	 10%	12%

5	 RXQUAL_5/4/6	 10%	12%

6	 RXQUAL_6/5/7	 10%	12%

7	 RXQUAL_7/6	 10%	12%

II.6.3	TX Power Control

II.6.3.1	Definition

RF power control is employed to minimise the transmit power whilst maintaining the quality of the radio link. The RF power level to be employed by the MS is indicated by means of the 5 bit TXPWR field sent either in the layer 1 header of each downlink SACCH message block or in a dedicated signalling block.

The MS shall confirm the power level that it is currently employing by setting the MS-TXPWR-CONF field in the uplink SACCH L1 header to its current power setting.

The timing requirement on the MS power control behaviour and the MS-TXPWR-CONF field in the uplink SACCH L1 header will be tested here, while the transmitted output power used by the MS is tested in the transmitter section (see II.3.3).

Timing reference for this test: In this test case TDMA frame number 1 is the first TDMA frame belonging to the reporting period after the SS signals a change in TXPWR level (see GSM 05.08 section 4.7).

II.6.3.2	Method of Measurement

Note:	The maximum peak power (power class, see GSM 05.05 section 4.1.1) appropriate to the MS is known to the system simulator from the previous location updating.

Note:	The method of measuring the MS transmitter output power is given in section II.3.3.

a) The SS sets MS-TXPWR-MAX-CCH = 0 in its SYSTEM INFORMATION TYPE 3 message and " DTX = OFF " signalled on the BCCH. After 10 s the MS is brought into the conversation state on timeslot 0.

b) The SS then signals TXPWR = 1513 to the MS for class 2 or TXPWR= 10 to the MS for class 1.

c) During 1 s immediately after the MS receives the new power command, its transmit power shall be measured and recorded every TDMA frame.The SS shall measure the MS transmit power on TDMA frame 6, 19, 32 and every subsequent 13th TDMA frame to TDMA frame 201.  The SS shall also monitor the MS-TXPWR-CONF field in the uplink SACCH L1 header.

d) The SS now sets TXPWR back to the maximum peak power appropriate to the class of the MS.

e) During 1 s immediately after the MS receives the new power command, its transmit power shall be measured and recorded every TDMA frame.  The SS shall also monitor the MS-TXPWR-CONF field in the uplink SACCH L1 header.Step c) shall be repeated.

f) The SS now sets TXPWR = 8.  After 1 s the SS then sets TXPWR = 9.

For a class 1 mobile the SS now sets the TXPWR = 6, and after 3 s the SS sets TXPWR = 7

For a class 2 mobile the SS now sets the TXPWR = 3, and after 3 s the SS sets TXPWR = 4

g) 13 TDMA frames (60 ms) after the MS receives the power command TXPWR=9, the MS output power is measured. (GSM 05.08 section 4.7)After the SS has set TXPWR = 7 for class 1 and TXPWR = 4 for class 2, the SS shall measure the MS transmit power on TDMA frame 6 (GSM 05.08-DCS section 4.7)

h) The SS returns TXPWR to 68 for a class 1 mobile and to 3 for a class 2 mobile.

i) 13 TDMA frames (60 ms) after the MS receives the power command TXPWR = 8, the MS output power is measured. (GSM 05.08 section 4.7).After the SS has set TXPWR = 6 for a class 1 mobile and to 3 for a class 2 mobile, the SS shall measure the MS transmit power on TDMA frame 6 (GSM 05.08-DCS section 4.7).

j) Then the channel assignment is changed and the demanded power within the channel assignment is set to TXPWR=1510 for class 1 or TXPWR= 13 for class 2. Immediately after the MS has changed the channel, its output power is measured and the MS-TXPWR-CONF field in the uplink SACCH L1 header is monitored by the SS.

II.6.3.3	Requirements

Refer to TABLE II.3.3 for relationship between the POWER CLASS, POWER CONTROL LEVEL, PEAK TRANSMITTED POWER and the relevant TOLERENCES.

1) Step c). The peak transmitted carrier power of TDMA frame 6 shall correspond to one power control step below the maximum value. The peak transmitted carrier power of each subsequently measured TDMA frame shall correspond to one power control step lower than the previously measured TDMA frame until power control level 10 (for class 1) or 13 (for class 2) is reached.After the SS orders the MS to change its output power from its maximum value to 13 dBm (TXPWR = 15) in one step and back to its maximum value in one step the MS shall change its output power at a rate of one nominal power step every 60 ms.  The value of the MS-TXPWR-CONF field in the uplink SACCH L1 header shall correspond to the measured MS transmitted power during the last active TDMA burst of the previous SACCH reporting period (GSM 05.08-DCS section 4.28.2.3).

2) Step e). The peak transmitted carrier power of TDMA frame 6 shall correspond to power control level 14. The peak transmitted carrier power of each subsequently measured TDMA frame shall correspond to one power control step higher than the previous measured TDMA frame until the maximum power control level is reached. The value of the MS TXPWR CONF field in the uplink SACCH L1 header shall correspond to the measured MS transmitted power during the last active TDMA burst of the previous SACCH reporting period.

23) After the SS orders the MS to change its output power by 1 step up or down the measured MS transmitter power after 13 TDMA frames (60 ms) shall correspond to the commanded TXPWR.Steps g) and i). The peak transmitted carrier power of TDMA frame 6 shall corespond to the commanded power control level.

34) Step j). The MS output power measured immediately after the new channel assignment has been set, shall correspond to the commanded power level.

II.6.4	Single frequency reference

The MS is required to use one single frequency reference for both RF generation/reception and baseband signals. A test method to verify this is not available.

Requirement

1) The manufacturer shall demonstrate that the MS is based on one single frequency reference.