II.5.3.9	Testing of structured procedures

Ref: GSM 04.08-DCS sections 7.3.2 through 7.3.4

II.5.3.9.1	General

The purpose of these tests is to complement the testing of elementary procedures to verify that the MS functions also in the "field" and not only on the "test bench".

The elementary procedures are combined into sequences, "structured procedures" as defined in GSM 04.08-DCS section 7, representing true sequences of events for normal MS operation.

Only the main structured procedures are deemed necessary to test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of traffic channel.

In two of the cases the call establishment test is followed by testing of call release initiated by network and mobile respectively.

The tests are performed only for a successful outcome of each elementary procedure.

Time requirements are indicated typically "(T3240-MM-MS)", showing timer identity, sublayer and (MS/NW) side where the timer is used (NW = Network).

The timer values are defined in GSM 04.08-DCS, section 11.

II.5.3.9.2	MS originating call establishment, early assignment, release init. by network

II.5.3.9.2.1	Purpose of the test

a)  To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08-DCS section 7.3.2.1a.

b)  To test the MS ability to release a call, when the release is initiated by the network, following the sequence of elementary signalling procedures given in GSM 04.08-DCS section 7.3.4a.

c)  To verify the MS display of the called number.

II.5.3.9.2.2	Method of test

a)  The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)  The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)  The MS is made to initiate a call and the SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

d)  The SS accepts the service request from the MS via the data link establishment procedure.

e)  The SS sends message AUTHENTICATION REQUEST.

f)  After reception of AUTHENTICATION RESPONSE the SS sends message CIPHERING MODE COMMAND within (T3230-MM-MS) seconds after the first access burst, and starts deciphering.

g)  After reception of CIPHER MODE COMPLETE the SS starts enciphering.

h)  The SS sends the message CALL PROCEEDING within (T303-CC-MS) seconds after the SS has received the message SETUP.

i)  The SS then sends message ASSIGNMENT COMMAND.

j)  Having received message ASSIGNMENT COMPLETE, the SS sends the message ALERTING within (T310-CC-MS) seconds after the sending of CALL PROCEEDING.

k)  Within (T301-CC-MS) seconds after ALERTING the SS sends the CONNECT message.

l)  The SS initiates clearing of the call by sending the message DISCONNECT.

m)  Within (T308-CC-MS) seconds after reception of the message RELEASE the SS sends RELEASE COMPLETE followed, within a further (T3240-MM-MS) seconds, by CHANNEL RELEASE.

MESSAGE IMMEDIATE ASSIGNMENT (GSM 04.08-DCS, 9.1.17) to the MS:

Comment	 Value

Protocol Discriminator	  RR	011

Transaction Identifier	  Not used	  0000

Message Type		0011 1111

Page mode	Normal	00

Channel description

- Channel type ...	SDCCH/SACCH 1(4)   0 0100

- Time slot number	Arbitrary	  xxx

- Training seq.code	(default)

- Hopping	No	  0

- FB no.	 Band no 0	  000

- ARFCN	(default)

Random reference

- Random access info	 As in CHAN REQ	111x xxxx

- N51, N32, N26	As applicable	....

Timing advance	 Arbitrary	 xx xxxx

Mobile allocation	 Length=0 (due to hopping)  0

MESSAGE AUTHENTICATION REQUEST (GSM 04.08-DCS, 9.2.2) to the MS:

Comment	 Value

Protocol Discriminator	  MM	  0101

Transaction Identifier	  Not used	  0000

Message Type		0001 0010

Ciphering key seq.number	As applic.from loc.upd	 xxx

Authent.parameter RAND	  As applicable	 (128 bits)

MESSAGE CIPHERING MODE COMMAND (GSM 04.08-DCS, 9.1.9) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0011 0101

Cipher mode setting	  Start ciphering	 1

MESSAGE CALL PROCEEDING (GSM 04.08-DCS, 9.3.3) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  1xxx

Message Type		0000 0010

MESSAGE ASSIGNMENT COMMAND (GSM 04.08-DCS, 9.1.2) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0010 1110

Channel description

- Channel type ...	Bm+ACCHs	0 0001

- Time slot number	Arbitrary	  xxx

- Training seq.code	  (default)

- Hopping	No	  0

- FB no	  Band no 0	  000

- ARFCN	  (default)

Power level	 As applicable	 x xxxx

Channel mode		  0110 0011

- speech full rate		  0000 0001

MESSAGE ALERTING (GSM 04.08-DCS, 9.3.1) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  1xxx

Message Type		0000 0001

MESSAGE CONNECT (GSM 04.08-DCS, 9.3.5) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  1xxx

Message Type		0000 0111

MESSAGE DISCONNECT (GSM 04.08-DCS, 9.3.7) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  SS orig.	  0xxx

Message Type		0010 0101

Cause

- Coding standard	 GSM	11

- Location	  User	0000

- Cause value	  Normal clearing	001 0000

MESSAGE RELEASE COMPLETE (GSM 04.08-DCS, 9.3.15) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  SS orig.	  0xxx

Message Type		0010 1010

Drafting note: Is the "Cause" information element mandatory here ?

MESSAGE CHANNEL RELEASE (GSM 04.08-DCS, 9.1.7) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0000 1101

RR cause	 Normal release	0000 0000

II.5.3.9.2.3	Requirements

1)  In step c) the MS shall display the dialled number.

2)  In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

3)  In step d) the MS shall send a layer 2 SABM frame containing the CM SERVICE REQUEST message to the SS. (The same message is returned in a layer 2 UA frame from the SS). The SABM frame shall be sent within (T3101-RR-NW) seconds from the time ofafter reception of the IMMEDIATE ASSIGNMENT message.

4)  In step e) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure)

5)  In step f), within 1 second from CIPHERING MODE COMMAND, the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE followed by the SETUP message

Note:	In requirement 5), the time of 1 second is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

6)  In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after the reception of the within (T3107-RR-NW) seconds from ASSIGNMENT COMMAND.

7)  In step j) the ringing indication at the called party shall be recognizable in the MS.

8)  In step k) the MS shall send the message CONNECT ACKNOWLEDGE after reception ofwithin (T313-CC-NW) seconds from CONNECT, and through-connect the speech path in both directions.

9)  In step l) the MS shall send the message RELEASE within (T305-CC-NW) seconds after it has received the message DISCONNECT from the SS, initiating call release.

10)  After In step m) the MS shall enter the idle, updated state within (T3110-RR-MS) seconds after it has received the message CHANNEL RELEASE.

MESSAGE CHANNEL REQUEST (GSM 04.08-DCS, 9.1.8) to the SS:

Comment	  Value

Random reference	 x xxxx

Cause	 Orig. call	 111

MESSAGE CM SERVICE REQUEST (GSM 04.08-DCS, 9.2.7) to the SS:

Comment	  Value

Protocol discriminator	  MM	  0101

Transaction identifier	  Not used	  0000

Message Type		0x10 0100

CM service type	Mob.orig.call est.	 0001

Ciphering key seq.number	As applic.from loc.upd	 xxx

MS classmark 2

- Revision level	  As applicable	 xxx

- Encryption algorithm	  A5	 00

- RF power capability	As applicable	 xxx

- Short message capability As applicable	x

- Frequency band	  Band no 0	  000

Mobile identity

- Odd/even no of digits	 As applicable	x

- Type of identity	TMSI	 100

- Identity digits	 As applicable	....

MESSAGE AUTHENTICATION RESPONSE (GSM 04.08-DCS, 9.2.3) to the SS:

Comment	  Value

Protocol discriminator	  MM	  0101

Transaction identifier	  Not used	  0000

Message Type		0x01 0100

Authentication parameter	SRES, as applicable (32 bits)

MESSAGE CIPHERING MODE COMPLETE (GSM 04.08-DCS, 9.1.10) to the SS:

Comment	  Value

Protocol discriminator	  RR	  0110

Transaction identifier	  Not used	  0000

Message Type		0011 0010

MESSAGE SETUP (GSM 04.08-DCS, 9.3.16) to the SS:

Comment	  Value

Protocol discriminator	  CC	  0011

Transaction identifier	  MS orig.	  0xxx

Message Type		0x00 0101

Bearer capability

- Radio channel req.	 Full rate	01

- Coding standard	 GSM	 0

- Transfer mode	Circuit mode	 0

- Info transfer capab.	  Speech	  000

Mobile identity	Info element may be present

- Odd/even no of digits	 As applicable	x

- Type of identity	TMSI	 100

- Identity digits	 As applicable	....

Called party BCD number

- Type of number	  As applicable	 xxx

- Numbering plan id.	 As applicable	xxxx

- Digits	 As applicable	....

MESSAGE ASSIGNMENT COMPLETE (GSM 04.08-DCS, 9.1.3) to the SS:

Comment	  Value

Protocol discriminator	  RR	  0110

Transaction identifier	  Not used	  0000

Message Type		0010 1001

RR cause		0000 0000

MESSAGE CONNECT ACKNOWLEDGE (GSM 04.08-DCS, 9.3.6) to the SS:

Comment	  Value

Protocol discriminator	  CC	  0011

Transaction identifier	  MS orig.	  0xxx

Message Type		0x00 1111

MESSAGE RELEASE (GSM 04.08-DCS, 9.3.14) to the SS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  SS orig.	  1xxx

Message Type		0x10 1101

II.5.3.9.3	MS originating call establishment, late assignment

II.5.3.9.3.1	Purpose of the test

To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08-DCS section 7.3.2.1b.

II.5.3.9.3.2	Method of test

a)  The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the CALL PROCEEDING message. Thereafter, the following applies:

b)  The SS sends the message ALERTING within (T310-CC-MS) seconds after the message CALL PROCEEDING was sent. The ALERTING message contents shall be the same as in section II.5.3.9.2.2.

c)  The SS then sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.2.2.

d)  Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT within (T301-CC-MS) seconds after message ALERTING was sent. The CONNECT message contents shall be the same as in section II.5.3.9.2.2.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.3.3	Requirements

1)  The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the SETUP message. Thereafter, the following applies:

2)  In step b) the ringing indication at the called party shall be recognizable in the MS as in section II.28.2.  II.16.2.2

3)  In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after within (T3107-RR-NW) seconds from reception of ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.2.3.

4)  In step d) the MS shall send the message CONNECT ACKNOWLEDGE after reception ofwithin (T313-CC-NW) seconds from CONNECT, and through-connect the speech path in both directions. The CONNECT ACKNOWLEDGE message contents shall be the same as in section II.5.3.9.2.3.

II.5.3.9.4	MS terminating call establishment, early assignment, release init.by MS

II.5.3.9.4.1	Purpose of the test

a)  To test the MS ability to respond correctly to a mobile terminating call following the sequence of elementary signalling procedures given in GSM 04.08-DCS section 7.3.2.1b.

b)  To test the MS ability to initiate and fulfillfulfil the release of a call, following the sequence of elementary signalling procedures given in GSM 04.08-DCS section 7.3.4b.

II.5.3.9.4.2	Method of test

a)  The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)  The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)  The MS is paged by means of a PAGING REQUEST TYPE 1 message on the correct paging subchannel.

d)  After reception of the CHANNEL REQUEST the SS returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

e)  The SS accepts the paging response from the MS via the data link establishment procedure.

f)  Immediately after the data link establishment the SS sends AUTHENTICATION REQUEST.

g)  Following the correct AUTHENTICATION RESPONSE from the MS the SS sends CIPHERING MODE COMMAND and then starts ciphering.

Drafting note: Timer values for authentication/ciphering are not specified.

h)  The SS sends the message SETUP immediately after reception of CIPHERING MODE COMPLETE. The SETUP message shall not include the "signal".

i)  The SS then sends message ASSIGNMENT COMMAND.

j)  When the SS has received messages ASSIGNMENT COMPLETE and ALERT off-hook is performed on the MS.

k)  The SS sends message CONNECT ACKNOWLEDGE within (T313-C-MS) seconds after reception of message CONNECT and then enters conversation state.

l)  The MS is initiated to clear the call.

m)  Within (T305-CC-MS) seconds after reception of the message DISCONNECT the SS sends the RELEASE message.

n)  Within (T3240-MM-MS) seconds after reception of the message RELEASE COMPLETE the SS releases the RR connection by sending the message CHANNEL RELEASE.

MESSAGE PAGING REQUEST TYPE 1 (GSM 04.08-DCS, 9.1.21) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0010 0001

Page mode	Normal	00

Mobile identity

- Odd/even no of digits	 As applicable	x

- Type of identity	TMSI	 100

- Identity digits	 As applicable	....

MESSAGE IMMEDIATE ASSIGNMENT (GSM 04.08-DCS, 9.1.17) to the MS:

Comment	  Value

Protocol Discriminator	 RR	0110

Transaction Identifier	 Not used	0000

Message Type		0011 1111

Page mode	  Normal	 00

Channel description

- Channel type ...	  SDCCH/SACCH 1(4)    0 0100

- Time slot number	  Arbitrary	xxx

- Training seq.code	  (default)

- Hopping	  No	0

- FB no	 Band no 0	000

- ARFCN	  (default)

Random reference

- Random access info	As in CHAN REQ	 100x xxxx

- N51, N32, N26	  As applicable

Timing advance	Arbitrary	  xx xxxx

Mobile allocation	Length=0 (due to hopping) 0

MESSAGE AUTHENTICATION REQUEST (GSM 04.08-DCS, 9.2.2) to the MS:

Comment	  Value

Protocol Discriminator	  MM	  0101

Transaction Identifier	  Not used	  0000

Message Type		0001 0010

Ciphering key seq.number	As applic.from loc.upd	 xxx

Authent.parameter RAND	  As applicable	 (128 bits)

MESSAGE CIPHERING MODE COMMAND (GSM 04.08-DCS, 9.1.9) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0011 0101

Cipher mode setting	  Start ciphering	 1

MESSAGE SETUP (GSM 04.08-DCS, 9.3.16) to the MS:

Comment	  Value

Protocol discriminator	  CC	  0011

Transaction identifier	  SS orig.	  0xxx

Message Type		0000 0101

MESSAGE ASSIGNMENT COMMAND (GSM 04.08-DCS, 9.1.2) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0010 1110

Channel description

- Channel type ...	Bm+ACCHs	0 0001

- Time slot number	Arbitrary	  xxx

- Training seq.code	  (default)

- Hopping	No	  0

- FB no	  Band no 0	  000

- ARFCN	  (default)

Power level	 As applicable	 x xxxx

Channel mode	  	0110 0011

- speech full rate	  	0000 0001

MESSAGE CONNECT ACKNOWLEDGE (GSM 04.08-DCS, 9.3.6) to the MS:

Comment	  Value

Protocol discriminator	  CC	  0011

Transaction identifier	  SS orig.	  0xxx

Message Type		0000 1111

MESSAGE RELEASE (GSM 04.08-DCS, 9.3.14) to the MS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  1xxx

Message Type		0010 1101

MESSAGE CHANNEL RELEASE (GSM 04.08-DCS, 9.1.7) to the MS:

Comment	  Value

Protocol Discriminator	  RR	  0110

Transaction Identifier	  Not used	  0000

Message Type		0000 1101

RR cause	 Normal release	0000 0000

II.5.3.9.4.3	Requirements

1)  In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

2)  In step d) the MS shall send a layer 2 SABM frame containing the PAGING RESPONSE message to the SS. (The same message is returned in a Layer 2 UA frame from the SS). The SABM frame shall be sent afterwithin (T3101-RR-NW) seconds from the time of reception of the IMMEDIATE ASSIGNMENT message and within (T3113-RR-NW) seconds from the time of reception of the paging request.

3)  In step f) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure.)

4)  In step g) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

5)  In step h) the MS shall send the message CALL CONFIRMED afterwithin (T303-CC-NW) seconds from the reception of SETUP.

6)  In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel afterwithin (T3107-RR-NW) seconds from the reception of ASSIGNMENT COMMAND.

7)  The MS shall 'ring'.

8)  The MS shall send the message ALERTING afterwithin (T310-CC-NW) seconds from sending the message CALL CONFIRMED.

9)  When off-hook has been performed the MS shall send the message CONNECT afterwithin (T301-CC-NW) seconds from sending the message ALERTING.

10)  In step j) the MS shall through-connect the speech path in both directions.

11)  In step l) the MS shall send the message DISCONNECT at the initiation of call release.

12)  In step m) the MS shall send the message RELEASE COMPLETE afterwithin (T308-CC-NW) seconds from reception of the message RELEASE.

13)  AfterIn step n) the MS shall enter the idle, updated state within (T3110-RR-MS) seconds after it has received the message CHANNEL RELEASE.

MESSAGE CHANNEL REQUEST (GSM 04.08-DCS, 9.1.8) to the SS:

Comment	  Value

Random reference	 x xxxx

Cause	 Answer to paging	 100

MESSAGE PAGING RESPONSE (GSM 04.08-DCS, 9.1.24) to the SS:

Comment	  Value

Protocol discriminator	  RR	  0110

Transaction identifier	  Not used	  0000

Message Type		0010 0111

Ciphering key seq.number	As applic.from loc.upd	 xxx

MS classmark 2

- Revision level	  As applicable	 xxx

- Encryption algorithm	  A5	 00

- RF power capability	As applicable	 xxx

- Short message capability As applicable	x

- Frequency band	  Band no 0	  000

Mobile identity

- Odd/even no of digits	 As applicable	x

- Type of identity	TMSI	 100

- Identity digits	 As applicable	....

MESSAGE AUTHENTICATION RESPONSE (GSM 04.08-DCS, 9.2.3) to the SS:

Comment	  Value

Protocol discriminator	  MM	  0101

Transaction identifier	  Not used	  0000

Message Type		0x01 0100

Authentication parameter	SRES, as applicable (32 bits)

MESSAGE CIPHERING MODE COMPLETE (GSM 04.08-DCS, 9.1.10) to the SS:

Comment	  Value

Protocol discriminator	  RR	  0110

Transaction identifier	  Not used	  0000

Message Type		0011 0010

MESSAGE CALL CONFIRMED (GSM 04.08-DCS, 9.3.2) to the SS:

Comment	  Value

Protocol discriminator	  CC	  0011

Transaction identifier	  SS orig.	  1xxx

Message Type		0x00 1000

MESSAGE ASSIGNMENT COMPLETE (GSM 04.08-DCS, 9.1.3) to the SS:

Comment	  Value

Protocol discriminator	  RR	  0110

Transaction identifier	  Not used	  0000

Message Type		0010 1001

RR cause		0000 0000

MESSAGE ALERTING (GSM 04.08-DCS, 9.3.1) to the SS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  SS orig.	  1xxx

Message Type		0x00 0001

MESSAGE CONNECT (GSM 04.08-DCS, 9.3.5) to the SS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  SS orig.	  1xxx

Message Type		0x00 0111

MESSAGE DISCONNECT (GSM 04.08-DCS, 9.3.7) to the SS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  0xxx

Message Type		0x10 0101

Cause

- Coding standard	 GSM	11

- Location	  user	0000

- Cause value	  normal clearing	001 0000

MESSAGE RELEASE COMPLETE (GSM 04.08-DCS, 9.3.15) to the SS:

Comment	  Value

Protocol Discriminator	  CC	  0011

Transaction Identifier	  MS orig.	  0xxx

Message Type		0x10 1010

II.5.3.9.5	MS terminating call establishment, late assignment

II.5.3.9.5.1	Purpose of the test

To test the MS ability to respond correctly to a mobile terminated call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.3.1b.

II.5.3.9.5.2	Method of test

a)  The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS receiving the CALL CONFIRMED, except that the SETUP message shall include the "Signal" element message. Thereafter, the following applies:

b)  After the SS has received the messages CALL CONFIRMED and ALERTING, off-hook is performed within (T301-CC-NW) seconds after the reception of ALERTING, on the MS.

c)  After the SS has received the message CONNECT the SS sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.4.2.

d)  Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT ACKNOWLEDGE within (T313-CC-MS) seconds after reception of message CONNECT and then enters conversation state. The CONNECT ACKNOWLEDGE message contents shall be the same as in section II.5.3.9.4.2.

Note: 	Clearing of the call is not part of the test procedure.

II.5.3.9.5.3	Requirements

1)  The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the CALL CONFIRM message. Thereafter, the following applies:

2)  In step b) the MS shall 'ring'.

3)  In step b) the MS shall send the message ALERTING afterwithin (T310-CC-NW) seconds from sending the message CALL CONFIRMED. The ALERTING message contents shall be the same as in section II.5.3.9.4.3.

4)  In step b) the MS shall send the message CONNECT within 0.5 seconds from the acceptance of the call (off hook) at the MS. The message contents shall be the same as in section II.5.3.9.4.3.

Note:	In requirement 4), the time of 0.5 second is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

5)  In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception ofwithin (T3107-RR-NW) seconds from ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.4.3.

6)  In step d) the MS shall through-connect the speech path in both directions.

[within (...) milliseconds after it has received the message CONNECT ACKNOWLEDGE].