II.5.3.1	Default conditions and structured sequence of tests

II.5.3.1.1	Default test conditions during L3-tests

During tests in section II.5.3 the following default test conditions shall apply if not otherwise stated within the test description. In the table below, decimal values are normally used. Sometimes a hexadecimal value, indicated with a 'H', or a binary value, indicated with a 'B' is given.

General signalling conditions for all carriers

Cyphering                               yes

General RF-conditions for all carriers

Frequency hopping mode                  Non-hopping

Propagation profile                     Static

Downlink Input Level                    63 dBmicroVolt emf

Uplink output power                     Maximum according to MS power class

Serving cell, BCCH/CCCH carrier

Channel                                 ARFCN 20

Alternative channels                    ARFCN 40 or 60

Serving cell, Traffic channel, SDCCH

Channel                                 ARFCN 30

Alternative channels                    ARFCN 50 or 70

Power Control Indicator (PWRC)          0

Neighbouring cells BCCH/CCCH carriers

Channels (RFCN):                        10, 80, 90, 100, 110, 120

Alternative channels:                   15, 85, 95, 105, 115, 122

Input level                             53 dBmicroVolt emf

Network dependent parameters

Cell identity                           0001H

Mobile country code, MCC                001  (decimal)

Mobile network code, MNC                01   (decimal)

Location area code, LAC                 0001H

BCCH allocation number, BA-NO           00B   band number 0

BCCH allocation sequence number(BA-IND) 0

Cell allocation number, CA-NO           00B   band number 0

PLMN colour code, NCC                   1

BS colour code, BCC                     5

SMS Cell Broadcast                      not active

DTX                                     MS must not use

IMSI Attach-detach allowed              0 MS shall not apply

CCCH-CONF                               set to 1 basic physical channel

used for CCCH, combined with

SDCCHS, encoded as  001B

BS-AG-BLKS-RES                          0 blocks reserved for access grant

BS-PA-MFRMS                             5 paging subgroups, encoded as 011B

CELL-BAR-ACCESS                         0 the cell is not barred

Call-reestablishment allowed (RE)       1

Emergency Call allowed (EC)             0

Access Control Class (AC) (0..9,11..15) 0

Network dependent timers

Radio Link Timeout                      8

T3212 Periodic updating in decihours    Infinite

Access control parameters

Max retrans  (1, 2, 4 or 7)             1

Tx-integer, number of slots             5

CELL-RESELECT-HYSTERESIS                12 dB

MS-TXPWR-MAX-CCH                        minimum level

RXLEV-ACCESS-MIN                        minimum

These informations are provided by system information 1, 2, 3 and 4 messages.

The system information elements which are broadcasted on the SACCH during the dedicated mode should be consistent with those sent on the BCCH when the MS was in idle mode prior to the channel request.

II.5.3.1.2	Structured sequence of the tests

The tests shall be performed in the order as indicated in the following table.

The validity of the tests depends upon the results of the tests performed before.

Channel request (basic test)                       RR   II.5.3.2.1

Immediate assignment                               RR   II.5.3.6.1

IMSI attach/detach (basic)                         RR   II.5.3.2.2

Paging                                             RR   II.5.3.6.2

Test of the mobile station functions in idle mode  RR   II.5.3.3

Frequency redefinition                             RR   II.5.3.6.6

Measurement report (incl. system info not idle)    RR   II.5.3.6.3

Authentication                                     MM   II.5.3.7.2

Cipher mode setting                                RR   II.5.3.6.8

Identification                                     MM   II.5.3.7.3

Sequenced MM/CC message transfer                   ..   II.5.3.2.3

Channel release                                    RR   II.5.3.6.12

Location updating                                  MM   II.5.3.7.4

TMSI reallocation                                  MM   II.5.3.7.1

Classmark change                                   RR   II.5.3.6.11

Call controll (verification on CC state diagram)   CC   II.5.3.8.1

DTMF information transfer                          CC   II.5.3.8.4

Handover                                           RR   II.5.3.6.5

Additional assignment                              RR   II.5.3.6.9

Partial release                                    RR   II.5.3.6.10

Re-establishment                                   CC   II.5.3.8.3

Dedicated channel assignment (during calls)        RR   II.5.3.6.4

Emergency call establishment                       CC   II.5.3.8.2

Transmission mode change                           RR   II.5.3.6.7

Mobility management connection establishment       MM   II.5.3.7.5

Test of Layer 3 error handling                          II.5.3.5

User to user signalling                            CC   II.5.3.8.5

Testing of structured procedures                        II.5.3.9

II.5.3.1.3	General Rules for message parameters

The following rules concerning message parameters apply to section II.5.3:

1)	Those values of parameters which are a consequence of the context of a test and which are not specific to that test need not be defined.

2)	If the value of a parameter of an uplink message (MS to Network) is specifiedin a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless stated otherwise.

3)	An optional field or optional Information Element of a downlink message (Network to MS), the presence of which is not a consequence of a test description, shall be absent in that test.

4)	If an optional field or Information Element is not indicated for the uplink (MS to Network) - unless specified otherwise -, it may be included or not.

5)	The Protocol Discriminator, Transaction Identifier and Message Type of all uplink messages have to be checked.