II.9	TEST OF AUTOCALLING RESTRICTIONS

ref: GSM 02.07

II.9.1	General

It is essential that all autocalling apparatus is prevented from continously dialling a given number, to avoid machines repeatedly disturbing PSTN subscribers in error, or numerous repeat attempts to unobtainable numbers which cause waste of valuable network resources. Two scenarios are considered:

-	Automatic reattempt by the Mobile Station after a single MMI request to dial a number;

-	Policing of automatic reattempt by terminal apparatus connected to a Mobile Station.

II.9.2	Blacklisting of MMI autocalled numbers

Where this function is provided, the following tests will be performed. The prerequisite is that the Layer 3 functions have been tested.

Procedure 1

1a)	The autocalling function on the MS shall be enabled by MMI.

1b)	A number shall be selected and called via MMI.

1c)	The System Simulator shall monitor the occurrence of call requests from the MS over a period of 60 seconds, and respond to each with an arbitrary error cause of Category 3 as defined in GSM 02.07.

Requirements 1

There shall be no automatic call reattempt within 5 seconds. After any reattempt the MS shall return to idle mode and there shall be no further reattempts.

Procedure 2

Procedure 1a) to 1c) shall be repeated, but with the System Simulator responding with an arbitrary error cause of Category 1 or 2 as defined in GSM 02.07, excluding cause no.27 if the MS has implemented that in Category 3, and monitoring the MS over a period of up to 60 minutes as defined in Requirements 2 below.

Requirements 2

Any reattempts by the MS shall be performed with the following minimum delays between attempts:

1st reattempt - 5 s

2nd reattempt - 1 minute

3rd reattempt - 1 minute

4th reattempt - 1 minute

5th reattempt - 3 minutes

6th reattempt - 3 minutes

7th reattempt - 3 minutes

8th reattempt - 3 minutes

9th reattempt - 3 minutes

10th reattempt - 3 minutes

After the 10th reattempt, the MS shall return to idle mode. There shall be no further reattempts, and after a further 4 minutes the tests may be aborted.

If the reattempt sequence exceeds 60 minutes the test may be aborted.

II.9.3	Blacklisting of terminal autocalled numbers

Where this function is provided, the following tests will be performed. The prerequisite is that the Layer 3 functions have been tested.

Procedure 1

A call to number N(x) x=1 shall be requested via the terminal interface.

Requirement 1

The MS shall attempt to set up the call.

Procedure 2

The SS shall respond to the SETUP message from the MS with a RELEASE COMPLETE message containing any one of the cause numbers as defined in Appendix H of GSM 04.08, and indicating 'Unobtainable destination - permanent/long term' as defined in GSM 02.07 Annex 1.

Requirement 2

The MS shall indicate to the terminal that the call attempt failed.

Procedure 3

The call to number N(x) shall again be requested over the terminal interface.

Requirement 3

If the minimum time between reattempts (Table II.9-1) has not expired then the MS shall reject the call request. Procedure 3 is then repeated.

If the minimum time between call attempts has expired and the number N(x) has not been blacklisted then the MS shall attempt to set up the call.  Procedures 2 and 3 are then repeated.

If the maximum number of reattempts  has been exceeded, then the MS shall respond to the call request with an appropriate error message indicating that the number has been blacklisted.

Procedure 4

4a) 	Procedures 1 to 3 are repeated using a different number N(2) in Procedure 1, and with the SS responding in Procedure 2 with a RELEASE COMPLETE message containing any one of the cause numbers defined in Appendix H of GSM 04.08, and indicating 'Unobtainable destination - temporary or Busy destination' as defined in GSM 02.07 Annex 1, excluding cause no.27 'Unobtainable destination - permanent/long term' if the MS has implemented it in Category 3.

4b) 	After the second reattempt to this number N(2), a further coexistent sequence of Procedures 1 to 3 is initiated, using number N(3) in Procedure 1, and with the SS responding in Procedure 2 with a RELEASE COMPLETE message containing any one of the cause numbers defined in Appendix H of GSM 04.08, and indicating 'Unobtainable destination - temporary or Busy destination' as defined in GSM 02.07 Annex 1, excluding cause no.27 'Unobtainable destination - permanent/long term' if the MS has implemented it in Category 3.

Requirement 4

The reattempt timings for calls to numbers N(2) and N(3) shall independently conform to Table II.9-1.

Procedure 5

A call to number N(4) shall be requested via the terminal interface.

Requirement 5

The MS shall attempt to set up the call.

Procedure 6

6a) 	As Procedure 2.

6b) 	After 5 seconds a further call to number N(4) shall be requested via the terminal interface.

6c) 	The SS shall allow the call to be sucessful and exchange the message sequence of GSM 04.08 Fig 7.8a.

The SS shall clear the call following the message sequence of GSM 04.08 Fig 7.10a, with the RELEASE COMPLETE message containing cause number 16.

6d) 	A call to number N(3) shall be requested via the terminal interface.

Requirement 6

The MS shall reject the call request.

Procedure 7

7a) 	Procedures 1 to 3 are repeated using numbers N(4) up to N(B), where B is the maximum capacity of the MS to manage simultaneous blacklisted numbers.

7b) 	A call to any number stored in the blacklist shall be requested via the terminal interface.

Requirement 7

The MS shall reject the call request.

Procedure 8

A call to a further number not in the blacklist shall be requested via the terminal interface.

Requirement 8

The MS shall reject the call request.

Note:	It shall not be possible to delete numbers from the Blacklist using commands from the terminal.  Clearing of the Blacklist shall require MMI intervention and typically powering off the MS.

TABLE II.9-1

CALL ATTEMPT	   │  MINIMUM DURATION BETWEEN CALL ATTEMPTS

│  for call failures of Catagory 3 as defined in

│  GSM 02.07

─────────────────────┼────────────────────────────────────────────────

1st reattempt	   │  5s

Further attempts	   │  Blacklisted

CALL ATTEMPT	   │  MINIMUM DURATION BETWEEN CALL ATTEMPTS

│  for call failures of Catagories 1 and 2 as

│  defined in GSM 02.07

─────────────────────┼────────────────────────────────────────────────

1st reattempt	   │  5s

2nd reattempt	   │  1 minute

3rd reattempt	   │  1 minute

4th reattempt	   │  1 minute

5th reattempt	   │  3 minutes

6th reattempt	   │  3 minutes

7th reattempt	   │  3 minutes

8th reattempt	   │  3 minutes

9th reattempt	   │  3 minutes

10th reattempt	   │  3 minutes

Further attempts	   │  Blacklisted