Aspect  II :  NETWORK  INTERFACING

II.1          General  tests  and  procedures

II.1.1	Test to verify the implementation of support for services

Purpose of the test

The purpose of this test is twofold:

1)	To verify which services are implemented in the MS under test.

2)	To verify that the MS, when it receives a mobile terminated call for a service that it does not support, it will refuse the call. In general this means that the MS shall respond with a RELEASE COMPLETE message.

For the services which it does not reject, the support shall function correctly, and the MS's support for that service is tested, see part I, I.2.1.

Method of test

ref: GSM 04.08 Annex B.3.2; GSM 07.01

Initial conditions

Note:	An MS is constructed to support a finite number of services.  For the support of a specific service, a specific setup of the MS may be required.  An example of this would be to create an MS set-up in which 1 of the R-interfaces is dedicated to handling of automatic fax group 3 only.

All the possible set-ups/configurations for the MS are listed.

One of the possible configurations/set-ups is created.

Procedure

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is paged by means of a PAGING REQUEST TYPE 1 message on the correct paging subchannel.

d)	After reception of the CHANNEL REQUEST the SS returns the IMMEDIATE ASSIGNMENT message.

e)	The SS accepts the paging response from the MS via the data link establishment procedure.

f)	The LTE, connected to the MS's TE interface(s), emulates the type of TE for which the SS will undertake to invoke the MS's support.

Note:	The MS may have various user Access Interfaces, such as:

-	Analogue 4 wire

-	V21     DTE/DCE interface

-	V22     DTE DCE interface

-	V22 bis DTE DCE interface

-	V23     DTE DCE interface

-	V26 ter DTE DCE interface

-	V32     DTE DCE interface

-	X21

-	X25

-	S interface.

g)	The SS sends the message SETUP, invoking the MS's support for one of the bearer- or teleservices, listed in Table II.1.

h)	The SS observes the MS's response.

Note:	CALL CONFIRM indicates that the MS accepts the incoming call as compatible with its configuration.

RELEASE COMPLETE (with a cause field) indicates that the MS rejects the incoming call.

i)	Upon acceptance of the call by the MS, the SS sends RELEASE COMPLETE.

j)	The test is repeated for all other bearer-/teleservices, as listed in Table II.1.

k)	Steps f) to j) are repeated for every set-up of the MS's configuration.

l)	The SS initiates release (not part of this test).

Table II.1 :	List of the Bearerservices and Teleservices, possibly supported by an MS, for which the test shall be carried out

11)   3.1 kHz Ex PLMN                                 T

10)   3.1 kHz Ex PLMN                                 NT

21a)  Data circuit Duplex asynchronous 300  b/s       T

21b)  Data circuit Duplex asynchronous 300  b/s       NT

22a)  Data circuit Duplex asynchronous 1200 b/s       T

22b)  Data circuit Duplex asynchronous 1200 b/s       NT

23a)  Data circuit Duplex asynchronous 1200/75 b/s    T

23b)  Data circuit Duplex asynchronous 1200/75 b/s    NT

24a)  Data circuit Duplex asynchronous 2400 b/s       T

24b)  Data circuit Duplex asynchronous 2400 b/s       NT

25a)  Data circuit Duplex asynchronous 4800 b/s       T

25b)  Data circuit Duplex asynchronous 4800 b/s       NT

26a)  Data circuit Duplex asynchronous 9600 b/s       T

26b)  Data circuit Duplex asynchronous 9600 b/s       NT

31 )  Data circuit Duplex  synchronous 1200 b/s       T

32 )  Data circuit Duplex  synchronous 2400 b/s       T

33 )  Data circuit Duplex  synchronous 4800 bit/s     T

34 )  Data circuit Duplex  synchronous 9600 bit/s     T

41a)  PAD Access circuit  asynchronous  300 bit/s     T

41b)  PAD Access circuit  asynchronous  300 bit/s     NT

42a)  PAD Access circuit  asynchronous 1200 bit/s     T

42b)  PAD Access circuit  asynchronous 1200 bit/s     NT

43a)  PAD Access circuit  asynchronous 1200/75 bit/s  T

43b)  PAD Access circuit  asynchronous 1200/75 bit/s  NT

44a)  PAD Access circuit  asynchronous 2400 bit/s     T

44b)  PAD Access circuit  asynchronous 2400 bit/s     NT

45a)  PAD Access circuit  asynchronous 4800 bit/s     T

45b)  PAD Access circuit  asynchronous 4800 bit/s     NT

46a)  PAD Access circuit  asynchronous 9600 bit/s     T

46b)  PAD Access circuit  asynchronous 9600 bit/s     NT

51a)  Data Packet Duplex   synchronous 2400 bit/s     T

51b)  Data Packet Duplex   synchronous 2400 bit/s     NT

52a)  Data Packet Duplex   synchronous 4800 bit/s     T

52b)  Data Packet Duplex   synchronous 4800 bit/s     NT

53a)  Data Packet Duplex   synchronous 9600 bit/s     T

53b)  Data Packet Duplex   synchronous 9600 bit/s     NT

61a)  Alternate Speech/Unrestricted Digital           T  for unres dig

(here unres dig offers the same service

as bearer services 21-34)

61b)  Alternate Speech/Unrestricted Digital           NT for unres dig

(here unres dig offers the same service

as bearer services 21-26)

71 )  12 kbit/s Unrestricted Digital

81a)  Speech followed by data                         T  for unres dig

(here unres dig offers the same service

as bearer services 21-34)

81b)  Speech followed by data                         NT for unres dig

(here unres dig offers the same service

as bearer services 21-26)

11)   Telephony

12)   Emergency calls

21)   Short message MT/PP

22)   Short message MO/PP

23)   Short message service cell broadcast

31)   Avanced MHS access

41)   Videotex access profile 1

42)   Videotex access profile 2

43)   Videotex access profile 3

51)   Teletex

61)   Alternate Speech and facsimile group 3

62)   Automatic facsimile group 3

Requirements

1)	For each of the bearer capabilities, offered by the network, which are not supported by the MS, the MS shall

-		ignore the call, or

-		reject the offered call using cause nr. 88

("incompatible destination"), or

-		start negotiating,

regardless of the set-up of the MS's configuration.

Note:	Those bearer/teleservice call offerings, which are accepted by the MS, are deemed to be implemented in the MS, and support for these services is verified separately.

II.1.2	Generic call set-up procedure

In the test procedures, described in this specification, unless otherwise stated in the test description, the call set-up procedure shall be as follows:

a)	The MS shall be operated under normal test conditions (see Annex 1, TC.2.1).

b)	The special Test-SIM (see section III.1) shall be inserted and a MS terminating call is established following the steps described in section II.5.3.9.3 "MS terminating call, late assignment" using the parameters of section II.5.3.9.3 with the following exceptions:

-	Ideal radio conditions (see Annex 1, GC1)

-	Timing advance set to 0

-	Non hopping mode

-	encryption on

-	no DTX

-	SDCCH combined with BCCH

-	Full rate speech channel.