Aspect  II :  NETWORK  INTERFACING

II.1          General  tests  and  procedures

II.1.1	verifICATION of support for services

Purpose of the verification

The purpose of the verification is threefold:

1)	To verify which services are implemented in the MS under test, as declared by the manufacturer in the Protocol Implementation Information according to Annex 3.

2)	To verify that the MS, when it receives a mobile terminating call, reacts correctly according to the principles of the single and/or multi numbering scheme as well as for ISDN originated calls.

3)	To verify that the MS, when it receives an incompatible mobile terminating call, reacts correctly according to the principles of the multi numbering scheme as well as for ISDN originated calls.

Method of verification

ref: GSM 04.08 Annex B.3.2; GSM 07.01; GSM 09.07

All the possible configuration set-ups of an MS need to be tested with regard to their support for any of the GSM bearer services and teleservices defined in GSM 02.02 and 02.03.  These services are reflected during the signalling phase within the GSM PLMN in the GSM Bearer Capability Information Element (BC-IE), LLC and HLC, if applicable.  The possible settings of the parameters in the GSM BC-IE, LLC and HLC are shown in GSM 07.01, Annex II.

In the cases of a multi numbering scheme as well as for ISDN originating calls, the SETUP message sent to the MS carries a valid GSM BC-IE (plus the ISDN LLC/HLC, if applicable) of the service under verification, according to Annex II of GSM 07.01.  If the MS does not support the offered service it shall

-	ignore the call or

-	reject the call by responding with a RELEASE COMPLETE message with cause "incompatible destination" or

-	apply the re-negotiation option.

(Note: covering the conflicting statements currently in GSM 04.08: Annex B.3.2 allows "ignoring", whereas section 5.2.2.2 requests a RELEASE COMPLETE message).

In case of the single numbering scheme (e.g. PSTN), where for a mobile terminating call no BC-IE is contained in the SETUP message, the expected (pre-set) 3.1 kHz audio ex PLMN related service shall be indicated in the CALL CONFIRMED message by the MS.

For the services which the MS accepts/indicates, the support is deemed to be implemented in the MS.  The support for these services shall function correctly and is tested specifically, see part I, I.2.

At least one of the services under the multi numbering scheme and at least one ISDN service not declared by the manufacturer in the PICS shall be tested. The correct outcome will be the transmission of a RELEASE COMPLETE message by the MS after the reception of the call SETUP message.

Configuration set-up and setting of service parameters

An MS is constructed to support a finite number of services.  For the support of a particular set of  services, a specific configuration set-up of the MS may be required.  An example of this would be to create an MS configuration set-up in which one interface at the R reference point is to be dedicated to handling automatic fax group 3 only.  In case of single numbering scheme an expected 3.1kHz audio ex PLMN related service is to be pre-set.

The MS may support at its R reference point  various  DTE/DCE interfaces and Procedures, such as:

-	V.21     DTE/DCE interface

-	V.22     DTE DCE interface

-	V.22 bis DTE DCE interface

-	V.23     DTE DCE interface

-	V.26 ter DTE DCE interface

-	V.32     DTE DCE interface

-	X.21     Procedure

-	X.21 bis Procedure

-	V.25 bis Procedure.

However, the setting of the value of the parameter Modem Type in the BC-IE depends exclusively on the actually required type of modem - bound to the requested user data rate signified in the BC-IE - for the individual connection.

In the case of data services, the setting of the parameter values of the BC-IE as outlined in GSM 07.01, Annex II, falls into 3 categories:

-	common to all data and telematic services

-	variable according to the individual service and

non re-negotiable

-	variable, partly related to an individual service,

but re-negotiable.

The setting of the variable values is described as a single path through a set of graphs established for each group of services and type of Information Transfer Capability (ITC)  (see relationship diagrams in GSM 07.01 Annex II, which indicate the individual graph pertinent to the service group, ITC and condition concerned).

MS responses

CALL CONFIRMED indicates that the MS accepts the incoming call.The configuration of the MS is either compatible to the BC-IE, LLC and HLC, if applicable or will enter the type of service signified by the BC-IE.  The MS may also apply the re-negotiation option.

RELEASE COMPLETE (with a cause field) indicates that the MS rejects the incoming call.

No response message from the MS arriving within [15] s at the SS, after having sent the SETUP message, shall be interpreted as if the MS ignores the incoming call.

II.1.2	Verification of non-support for services.

Purpose of the test:

Ensure that on an incompatible incoming call the MS will reject the call.

Initial conditions:

The generic call set-up procedure shall be implemented in accordance with sections II.1.3 or II.1.4 as applicable up to and including step g in section II.5.3.9.4.2.

Procedure:

SS: transmit SETUP

Note: SETUP configured with incompatible BC-IE as declared by the manufacturer

Result checking:

Not applicable

Requirement:

Expect a RELEASE COMPLETE message with cause value 88 - incompatible destination

II.1.3	Procedures for the verification of support for speech calls

For tests in this section the SS shall check that the coding of the information elements contained in all the messages received from the MS under test are in accordance with 04.08 sections 9.3,9.4 and 10.05 An MS shall be considered to have failed the tests if an information element is received which is either incorrectly structured or has an invalid code value.

II.1.3.1	Generic call set-up/release procedures for speech calls

In the test procedures described in this section, unless otherwise stated in the test description, the call set-up/release procedure shall be as follows:

a)	Mobile terminating call

- The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

- The special Test-SIM (see section III.1.6) shall be inserted and a MS terminating call shall be established following the steps described in section II.5.3.9.4 "MS terminating call establishment, early assignment" using the parameters of section II.5.3.9.4.2.

b)	Mobile originating call

- The procedures should be followed as detailed in the above section for the mobile terminating call except that the steps described in section II.5.3.9.3 "MS originating call, late assignment" shall be used.

II.1.3.2	Test for MS terminating speech call acceptable to the MS

Purpose of the test:

Ensure that on a valid incoming call the MS will enter the active state.

Initial condition:

The generic call set-up procedure in section II.1.3.1 should be used up to and including step g in section II.5.3.9.4.2.

Procedure:

SS: transmit a valid SETUP

MS: transmit CALL CONFIRMED

MS: transmit ALERTING

MS: transmit CONNECT

SS: transmit CONNECT ACKNOWLEDGE

SS: transmit STATUS ENQUIRY

MS: transmit STATUS

Result checking:

Analyse the STATUS message

Requirement:

The STATUS message shall contain: call state U10 - active.

II.1.3.3	Test for MS originating speech call - receipt of a CONNECT message

Purpose of the test:

Ensure on receipt of a connect message the MS enters the active state U10 (see 04.08 10.5.4.5)

Initial condition:

The generic call set-up procedure in section II.1.3.1 should be followed up to and including the CIPHERING MODE COMPLETE message of Requirement 5 in II.5.3.9.2.3.

Procedure:

MS: transmit SETUP

SS: transmit CALL PROCEEDING

SS: transmit ALERTING

SS: transmit CONNECT

MS: transmit CONNECT ACKNOWLEDGE

SS: transmit STATUS ENQUIRY

MS: transmit STATUS

Result checking:

Analyse STATUS message

Requirement:

The STATUS message must contain: call state U10 - active.

II.1.4	Generic Call Set-up/Release procedures for data calls

II.1.4.1	Description of data call set-up/release procedures

In the test procedures, described in this specification, unless otherwise stated in the test description, the call set-up/release procedure shall be as follows:

a)	The MS shall be operated under normal test conditions (see Annex 1, TC.2.1).

b1)	Mobile terminating call (MTC):

The special Test-SIM (see section III.1.6) shall be inserted and a call shall be established following the steps described in section II.5.3.9.4 "MS terminating call, early assignment" using the parameters and method of section II.5.3.9.4.2 with the following exceptions:

-	In the ASSIGNMENT COMMAND message the channel mode shall be either:

9.6 kbit/s

or	4.8 kbit/s full rate

or	2.4 kbit/s full rate

-	In step j) the ALERTING message may be replaced by a CONNECT message.subject to the implementation of the mobile as declared in the PIXIT statement

-	In step k) the conversation state shall be replaced by the Data Transfer Phase, see Figures II.1.3-1 and II.1.3-2.

The requirements of section II.5.3.9.4.3 are used with the following exceptions:

.-	Requirement 5) shall be replaced with the following:

The MS shall accept or reject the call by either sending CALL

CONFIRMED or RELEASE COMPLETE messages.

-	Requirement 7) is not applicable

-	Requirement 8) shall not be verified

-	Requirement 9) shall be replaced with the following:

In case of CALL CONFIRMED message the MS shall send a CONNECT

message. [Note: The CONNECT message may be proceeded by ALERT

subject to the implementation in the mobile as declared in the

PIXIT statement]

-	In step j) the MS shall through connect the data path in both directions.

b2)	Mobile originating call (MOC):

The special Test-SIM (see section III.1.6) shall be inserted and a call shall be established following the steps described in section II.5.3.9.3 "MS originating call establishment, late assignment " using the parameters and method of section II.5.3.9.3.2 with the following exceptions:

-	In the ASSIGNMENT COMMAND message the channel mode shall be either:

9.6 kbit/s

or	4.8 kbit/s full rate

or	2.4 kbit/s full rate

-	In section II.5.3.9.3.2, step b) shall be skipped.

-	In section II.5.3.9.3.2 step d), the ALERTING message shall be replaced by a CALL PROCEEDING message.

The requirements in section II.5.3.9.3.3 are changed as follows:

-	Requirement 2) is not applicable.

II.1.4.2	State Diagram Model for GSM data services testing

General

The diagrams in Figures II.1.4-1 and II.1.4-2 are intended as a simplified model to aid in the description of the tests for GSM data services.  It is not intended to replace in any way the state diagrams in other GSM documents defining the various machine states permissible.  The definition of these simplified (compound in some cases) states are defined related to the figures.

Definition of states in Figure II.1.4-1

IDLE	The Mobile Termination (MT) does not have a dedicated channel established to a Base Station and will, in general, be listening to the downlink BCCH.

RR Set-Up	The MT has completed the process of establishing the initial Radio Resource, i.e. a dedicated control channel to the network, thus enabling the set-up of the Mobility Management (MM) entity.

MM Set-Up	The MT has completed the process of establishing the Mobility Management (MM) entity, thus enabling the set-up of the Call Control (CC) entity.

CC Set-Up	The MT has completed the process of establishing the Call Control (CC) entity, thus enabling the establishment of the Data Transfer Phase.

For expansion of this (compound) state see Figure II.1.4-2.

DATA TRANSFER PHASE (T DATA)

This state describes the situation where the MT is transmitting and receiving data to and from the far end user. This state encompasses data services requiring only transparent bearer capabilities.

DATA TRANSFER PHASE (NT DATA)

This state describes the situation where the MT is transmitting and receiving data to and from the far end user. This state encompasses data services requiring only non-transparent bearer capabilities.

DATA TRANSFER PHASE (VOICE)

This state describes the situation where the MT is transmitting and receiving encoded GSM voice data to and from the far end user.

CC Release	The MT has completed the process of releasing the Call Control (CC) entity.

MM Release	The MT has completed the process of releasing the Mobility Management (MM) entity.

RR Release	The MT has completed the process of releasing the Radio Resource.

Definition of states in Figure II.1.4-2

Mobile Originating Call (MOC)

CC procedures perform the Layer 3 functions for a mobile originating call attempt.

Mobile Terminating Call (MTC)

CC procedures perform the Layer 3 functions for a mobile terminating call attempt.

SETUP, MOC	CC has sent the SETUP message.

SETUP, MTC	CC has received the SETUP message.

CALL PROCEEDING

CC of the MT has received the CALL PROCEEDING message, which contains a BC-IE, if the parameter Connection Element needs to be determined.  The radio channel related parameters are still to be determined.

CALL CONFIRMED

CC of the MT has sent the CALL CONFIRMED message, if the MT accepts the call (also when parameter values of the BC-IE are to be re-negotiated).  The radio channel related parameters are still to be determined.

TCH Assignment Pending

The TCH has been assigned.  The radio channel related parameters are now determined.

Pre-Data Transfer Phase

The Synchronisation process and the RLP establishment, respectively, took place, consequently the connection can enter the DATA TRANSFER PHASE (T/NT).

FIGURE  II.1.4-1 / GSM 11.10 :

State Diagram Model for Call Set-up / Release for GSM Data Services Testing

FIGURE  II.1.4-2 / GSM 11.10 :

Expansion of Call Control (CC Set-Up) State as seen in Fig. II.1.4-1 above