II.5.3.3	Test of MS functions in Idle Mode

II.5.3.3.1	Initial conditions

The SIM shall contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

During the tests in II.5.3.3.2 and II.5.3.3.3, the following parameters apply:

RACH control parameters

In cells 1 to 7:

Max retrans = 01, 2 retransmissions

Tx-integer = 0111, [10] slots for spreading

CB, Cell Barred = 0, access is allowed

RE = 1, reestablishment not allowed

AC C00 to AC C15 = 0, access is not barred

In cell 8:

Max retrans = 01, 2 retransmissions

Tx-integer = 0111, [10] slots for spreading

CB, Cell Barred = 1, access is not allowed

RE = 1, reestablishment not allowed

AC C00 to AC C15 = 0, access is not barred

Cell  │    PLMN    │	 BA - ARFCN bit = 1

│    perm.   │

───────┼────────────┼───────────────────────────────┐

1   │  00000100  │  7, 39, 65, 66, 85,  97, 124  │

2   │  00000100  │  8, 40, 67, 68, 86,  98, 123  │

3   │  00000100  │  9, 41, 69, 70, 87,  99, 122  │

4   │  00000100  │ 10, 42, 71, 72, 88, 100, 121  │

5   │  00000100  │ 11, 43, 73, 74, 89, 101, 120  │

6   │  00000100  │ 12, 44, 75, 76, 90, 102, 119  │

7   │  00000100  │ 13, 45, 77, 78, 91, 103, 118  │

8   │  00000100  │ 124                           │

Location area identification

Cell  │ MCC1 │ MCC2 │ MCC3 │ MNC1 │ MNC2 │ LAC  │

──────┼──────┼──────┼──────┼──────┼──────┼──────┤

1   │  0   │  0   │  2   │  0   │  F   │  x   │

2   │  0   │  0   │  3   │  2   │  F   │  x   │

3   │  0   │  0   │  4   │  3   │  F   │  x   │

4   │  0   │  0   │  5   │  4   │  F   │  x   │

5   │  0   │  0   │  6   │  5   │  F   │  x   │

6   │  0   │  0   │  7   │  6   │  F   │  x   │

7   │  0   │  0   │  8   │  7   │  F   │  x   │

8   │  0   │  0   │  1   │  0   │  1   │  x   │  The HPLMN of the MS

Note 1:	'x' denotes any value.

Note 2:	The MS representation of the MCC, MNC on the Handset can be manufacturer dependant.

Note 3:	The NCC values of each cell must be different.

Control channel description and BS options

All 8 cells:

CELL - RESELECT - HYSTERESIS = 010, 4dB RXLEV hysteresis

MS-TXPWR-MAX-CCH is given the value corresponding to the maximum available output power from the MS under test.

RXLEV-ACCESS-MIN = 30

ATT = 0, no IMSI attach and detach

DTX = 0, no discontinuous transmission

BS-AG-BLKS-RES = 1, 1 block reserved for access grant

CCCH-CONF = 001, 1 SDCCHs combined with the CCCH

RADIO-LINK-TIMEOUT = 5, 10 seconds timeout

BS-PA-MFRMS = 010, 4 multiframes periods for paging

T3212 timeout value = H'00

Cell │ level(dBmicroVolt emf) │ BCCH ARFCN

─────┼────────────────────────┼───────────

1  │        +65             │    1

2  │        +63             │    7

3  │        +61             │   39

4  │        +23             │   65

5  │        +59             │   66

6  │        +57             │   85

7  │        +55             │   97

8  │        +53             │  124

Note:	The SIM should contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

II.5.3.3.2	MS indication of available PLMNs

II.5.3.3.2.1	Purpose of the test

To verify that a MS presents the available PLMNs to the user according to the requirements of GSM 05.08 and 02.11, if no suitable cell of the selected PLMN has been found.

II.5.3.3.2.2	Procedure

a)	Each cell is given a different NCC value since each cell has a different MCC and MNC.

b)	The MS is switched on and equipped with a SIM containing default values except for those values listed under section II.5.3.3.1 (initial conditions).

II.5.3.3.2.3	Requirements

1)	The MS must present the user with some indication that regular service is unavailable.

2)	The MS shall present a list of available PLMNs (MCC and MNC values). The list shall include the MCC and MNC of cells 1 to 7, but not of

cell 8.

II.5.3.3.3	MS will send only if BSS is "on air"

II.5.3.3.3.1	Purpose of the test

To verify that the MS will not produce any RF transmission if no BSS is received.

II.5.3.3.3.2	Procedure

a)	The RF-signal for the BCCHs of cell 1 to 8 is switched off.

b)	The SS shall wait 20 seconds to allow the MS to detect the loss of cells.

c)	By MMI, an attempt to originate a call is made.

d)	By MMI, an attempt to originate an emergency call is made.

II.5.3.3.3.3	Requirements

1)	The MS must not give "service indication".

2)	In steps c) and d) the MS shall not produce any RF output.