Aspect III: INDIVIDUAL EQUIPMENT TYPE REQUIREMENTS AND

INTERWORKING

III.1	Special conformance testing functions

III.1.1	General

This chapter specifies those ME functions which are required for conformance testing purposes only. However, except for the Electrical Man Machine Interface EMMI, they are required for every Mobile Station.

For conformance tests, functions are activated via radio interface, Test-SIM or dedicated pins. These functions can only be activated when a Test-SIM is present. In this state, the MS must be able to perform all functions specified in this MS conformity specification; in addition however, the special conformance testing functions must be operational.

The special conformance testing functions of the ME are enabled to operate by use of a dedicated Subscriber Identity Module (Test-SIM, see III.1.6). SIMs, in general, are described in GSM 11.11. The ME recognizes the Test-SIM by the Administrative Data Field.

III.1.2	Activation and Deactivation of Special Test Functions in the MS

These functions can be activated and deactivated from a SS by sending appropriate Layer 3 commands to the MS. The protocol discriminator to be used is defined in GSM 04.08, section 10.2.

The Layer 3 commands are sent on the DCCH. On Layer 2, SAPI 0 is used in acknowledged mode. Furthermore, those commands are processed by the MS only if a Test-SIM is present; otherwise the MS will ignore those commands.

Apart from sending the appropriate deactivation command to the MS the functions can be deactivated by switching off the MS or removing the Test-SIM.

The following test functions can be activated (and deactivated):

- TCH Loop,

- Electrical MMI,

- Test via DAI.

The TCH loop and the test via DAI are test functions which are mutually exclusive.

III.1.2.1	Internal Test Loops

A number of internal test loops is required providing access to isolated functions of the MS without introducing new physical interfaces just for the reason of type approval testing. Fig III.1-1 shows a functional block diagram of a reference MS containing the different test loops.

Note:	It should be emphasized that these test loops only describe the functional behaviour of the MS with respect to its external interfaces; physical implementation of the loops is completely left open to the manufacturer.

A particular loop is activated in an MS by transmitting the appropriate Command message to the MS.

figure file: TESTLOOP.GEM

Fig. III.1-1:  Testloops in the MS

III.1.2.1.1	TCH Loop

Purpose:

To establish a transparent loop for TCH blocks between the output of the channel decoder and the input of the channel encoder in an MS. A TCH must be active between SS and MS. The TCH may be full or half rate, speech or data of any rate specified in the GSM system.

Two types of TCH loop back are required. One includes the signalling of erased frames and is used to determine Frame Erasure Ratio (FER) and Residual Bit Error Ratio (RBER) for speech TCH and Bit Error Ratio (BER) for any data TCH. The second type is required to determine Class II Bit Error Ratio for the speech TCH.

III.1.2.1.1.1	TCH loop including signalling of erased frames

Procedure:

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped and that erased frames are to be signalled by the MS. The SS then starts timer TT01.

If no TCH is active, or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, every good speech frame or any user data frame received by the MS on the specified TCH (downlink) shall be taken from the output of the channel decoder, input to the channel encoder and transmitted on the same TCH (uplink).

If the channel decoder detects a bad speech frame or if the MS decodes the stealing flags as indicating an FACCH frame, then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH (uplink).The FACCH channel shall operate normally.

III.1.2.1.1.2	Speech TCH loop without signalling of erased frames

Procedure

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped. The SS then starts timer TT01.

If no TCH is active or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, any speech frame received by the MS on the specified TCH (downlink) shall be taken from the output of the channel decoder, input to the channel encoder, and transmitted on the same TCH (uplink).

The SS should avoid using the FACCH downlink in this situation until the test is complete.

III.1.2.1.1.3	Deactivating TCH Loops

The SS orders the MS to open any TCH loop by transmitting an OPEN_LOOP_CMD message.

If no loop is closed the MS shall ignore any OPEN_LOOP_CMD message.

If a TCH is looped, the MS shall open the loop.

All channels shall be open for normal use again.

III.1.2.2	Activating and deactivating EMMI

Activating EMMI requires the presence of a Test-SIM. EMMI shall be activated by any of the following:

-	switching on the MS

-	inserting a Test-SIM

-	Layer 3 message on the radio interface (ACT_EMMI_CMD).

When the MS is ready to receive frames, it shall send one XON message.

EMMI shall be deactivated by any of the following:

-	switching off the MS

-	removing the Test-SIM

-	Layer 3 message on the radio interface (DEACT_EMMI).

Note: No XOF shall be sent after deactivation.

The L3 message used on the radio interface to activate the EMMI is the activation command ACT_EMMI_CMD (see III.1.2.4.4).  This message has to be acknowledged by the message ACT_EMMI_ACK on the radio interface sent by the MS (see III.1.2.4.5).

For deactivation of the EMMI in the MS through the radio interface, the message DEACT_EMMI is defined in III.1.2.4.6.  An acknowledgement of this message is not required.

III.1.2.3	Activating and deactivating DAI tests

Purpose: to determine the routing of speech data (DAI or internal, i.e. normal mode) and which device is being tested (speech transcoder / DTX functions or A/D & D/A).

Prerequisites: a dedicated channel must be established if the manufacturer has stated that the DAI is activated by means of the Layer 3 message.

Procedure: the SS sends a TEST_INTERFACE message if the manufacturer has stated that the DAI is activated by means of the Layer 3 message or applies the appropriate control signal on the DAI if the manufacturer has declared that the DAI is activated this way.

When the test mode is established i.e. speech data comes from test interface, each new test function overrides the previous one.

III.1.2.4	Message Definitions and Contents

Note:	For all messages the Protocol discriminator  PD  111 (binary) is used.

III.1.2.4.1	CLOSE_TCH_LOOP_CMD

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

├───────────────────────┼───────────┼──────┼────────┤

│      Sub-channel      │           │  MF  │   1    │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  0  │  0  │  0  │  0  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

and Sub-channel is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  0  │  0  │  Y  │  X  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

X = 0	If there is only one TCH active (so there is no choice) or if sub-channel 0 of two half rate channels is to be looped.

X = 1	If sub-channel 1 of two half rate channels is to be used.

Y = 0	If the looped TCH is a speech channel then the frame erasure is to be signalled.

Y = 1	If the looped TCH is a speech channel then frame erasure is not signalled.

III.1.2.4.2	CLOSE_TCH_LOOP_ACK

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  0  │  0  │  0  │  1  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

III.1.2.4.3	OPEN_LOOP_CMD

Note: OPEN_LOOP_ACKNOWLEDGE message is not provided.

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  0  │  1  │  1  │  0  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

III.1.2.4.4	Command for the activation of the EMMI, ACT_EMMI_CMD

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  1  │  1  │  0  │  0  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

III.1.2.4.5	Acknowledge of the activation of the EMMI, ACT_EMMI_ACK

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  1  │  1  │  0  │  1  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

III.1.2.4.6	Deactivation of the EMMI, DEACT_EMMI

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

└───────────────────────┴───────────┴──────┴────────┘

where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  1  │  0  │  0  │  0  │  0  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

III.1.2.4.7	Test_Interface

┌───────────────────────┬───────────┬──────┬────────┐

│ Information Element   │ Reference │ Type │ Length │

│=======================│===========│======│========│

│Protocol discriminator │ GSM 04.08 │      │        │

│                       │sect. 10.2 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Transaction identifier │ GSM 04.08 │      │   2    │

│                       │sect. 10.3 │  MF  │        │

├───────────────────────┼───────────┼──────┤        │

│Message type           │           │  MF  │        │

├───────────────────────┼───────────┼──────┼────────┤

│Tested device          │           │  MF  │   1    │

└───────────────────────┴───────────┴──────┴────────┘

Where message type is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  1  │  0  │  1  │  0  │  0  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

and Tested Device is:

8     7     6     5     4     3     2     1        bit no.

┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐

│  0  │  0  │  0  │  0  │  0  │  X  │  X  │  X  │     octet 1

└─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘

Tested Devices:

000 = normal operation (no tested device via DAI)

001 = test of speech decoder / DTX functions (downlink)

010 = test of speech encoder / DTX functions (uplink)

100 = test of acoustic devices and A/D & D/A.

All other values are reserved.

III.1.2.5	Timer values

TT01:	To be started when a CLOSE_TCH_LOOP_CMD is sent. To be stopped when the corresponding CLOSE_TCH_LOOP_ACK is received.

Recommended value: [2.5 seconds].

III.1.3	Electrical Man Machine Interface (EMMI)

III.1.3.1	Use of the EMMI

Conformity tests of Mobile Stations are made using the System Simulator specified in Annex 4.

Test signals are sent on the Um interface, and actions of the MS are registered. The Electrical Man Machine Interface (EMMI) is a half duplex communication link between the SS and the MS by which it is possible to automatically register the status, indications and performance of the MS. It is also possible to simulate actions normally made by the user on the keyboard of the MS.

III.1.3.2	Formal aspects

i)	The EMMI is optional for the ME.

ii)	The EMMI is mandatory for the SS.

iii)	If the EMMI is to be used in conformance testing of an MS, it shall be possible to connect the SS to a connector on the MS, or to an adapter connected to the same MS. If an adapter is to be used, it shall be provided by the manufacturer.

iv)	If the MS fulfils the requirements performed with the use of an EMMI, the MS is regarded as having passed that test.

v)	If the MS is rejected in a test performed with EMMI, the test shall be repeated on the same mobile with the device carrying the EMMI to the MS removed. The MS shall be regarded as fulfilling the requirements, if it then passes the test.

vi)	When using the EMMI, the MS does not necessarily conform to the

RF requirements. Therefore, tests concerning Rx and Tx parameters on MSs with integral antenna and cabinet radiation tests for all types of MSs will never be performed with the use of the EMMI.

III.1.3.3	Layered structure of the interface

The definition of the EMMI is divided into three different layers.  On Layer 1 the use of a 25-pole socket with standard electrical characteristics for serial communication is defined. On Layer 2, an extremely simple frame oriented protocol is defined. On Layer 3, messages for control and verification of functions and indications are defined. Each layer is defined independently of surrounding layers, and is therefore easy to replace.

The EMMI protocol structure takes into account that the SS only sends and receives Layer 3 frames when the corresponding step within a testcase is to be performed.

III.1.3.4	Terminology

EMMI       Electrical Man Machine Interface

MI         Message Identifier

ME         Mobile Equipment

MS         Mobile Station

SS         System Simulator

Frame      Used on Layer 2 to transfer messages to and from Layer 3

Message    Information on Layer 3

III.1.3.5	Description of the EMMI

III.1.3.5.1	EMMI, Layer 1

III.1.3.5.1.1	Mechanical and Electrical characteristics

If implemented, the EMMI interface shall use the same connector as the Digital Audio Interface (DAI), described in section III.1.4.3.

The pin assignments for the EMMI shall be as follows:

PIN   │         FUNCTION        │  SOURCE

──────┼─────────────────────────┼──────────

2     │   Transmitted data      │    SS

3     │   Received data         │    MS

7     │   EMMI signal ground    │

The electrical characteristics of the interface shall be as given in section III.1.4.3.2.

III.1.3.5.1.2	Transmission and reception characteristics

The EMMI uses asynchronous serial data transmission with 1 start bit (S),

8 data bits (D1 to D8), no parity and 1 stop bit (E).

┌───────────────────────────────────────────────┐

│ S │ D1 │ D2 │ D3 │ D4 │ D5 │ D6 │ D7 │ D8 │ E │

└───────────────────────────────────────────────┘

------------------------------------------------>

Table 3 :  Use of start and stop bits

The conditions on start and stop characters are defined in REC CCITT V.1.

The transmission rates are: 600, 1200, 2400, 4800, 9600 bits per second. The ME shall support at least one of these speeds.

The SS will adapt its rate (manually or by MMI) to this ME rate.

III.1.3.5.2	EMMI, Layer 2

III.1.3.5.2.1	General Structure

On Layer 2, frames are used to carry data from higher layers.  Frames consists of one or several octets. One frame with variable length is used to carry data from higher layers, and four other frames with the length of one character is used to control the flow of frames.

III.1.3.5.2.2	Control Frames

Special frames have been assigned to control the flow of frames on Layer 2. They are only one octet long.

┌──────┬─────────────────────┬───────────┬─────────────┐

│ Abbr.│ Meaning             │ Dec.value │ Bit pattern │

├──────┼─────────────────────┼───────────┼─────────────┤

│ ACK  │ Acknowledge         │    06     │  0000 0110  │

│ NAK  │ Not Acknowledge     │    21     │  0001 0101  │

│ XON  │ Resume sending data │    17     │  0001 0001  │

│ XOF  │ Stop sending data   │    19     │  0001 0011  │

└──────┴─────────────────────┴───────────┴─────────────┘

Table 4 :  Characters For Flow Control

III.1.3.5.2.3	Frame Structure

Two octets, called characters, indicate start and stop of I-frames.

┌──────┬─────────────────────┬───────────┬─────────────┐

│ Abbr.│ Meaning             │ Dec.value │ Bit pattern │

├──────┼─────────────────────┼───────────┼─────────────┤

│ STX  │ Start of data       │    02     │  0000 0010  │

│ ETX  │ End of data         │    03     │  0000 0011  │

└──────┴─────────────────────┴───────────┴─────────────┘

Table 5 :  Start and Stop Characters

Information from higher layers are transferred in I-frames with the following structure.

┌────────────┬─────────────┬─────────────────┬───────────────────┬───────┐

│ Field name │ No of octet │     Value       │ Start at octet no │ Note  │

├────────────┼─────────────┼─────────────────┼───────────────────┼───────┤

│ Start      │      1      │ Character STX   │         1         │  -    │

│ Length     │      1      │ Length of data  │         2         │  1)   │

│ Data       │    0-255    │ Content of data │         3         │  2)   │

│ Check      │      1      │ Error detection │      Length+2     │  3)   │

│ End        │      1      │ Character ETX   │      Length+3     │  -    │

└────────────┴─────────────┴─────────────────┴───────────────────┴───────┘

Notes:

1)	Length: The total number of data octets in the data field of the frame is calculated. The value shall be in the range of (0..255 decimal). The corresponding binary value is put into the length field.

2)	Data: Data to and from higher layers are in the form of octets (groups of 8 digital bits).

3)	Check: Longitudinal checksum is created by exclusive OR on all characters starting with the Start field and ending with the last octet before the Check field. The value, one octet, is mapped into the Check field.

III.1.3.5.2.4	Flow of I-frames on Layer 2

III.1.3.5.2.4.1	Transmission of Frames

A multiple frame starts with the first octet being the character STX and stops with the last octet with content ETX. The time between the start bits of two consecutive octets shall not be longer than T22.

┌─────────────┐         ┌─────────────┐

│  octet n    │         │  octet n+1  │

└─────────────┘         └─────────────┘

│          T22          │

│<─────────────────────>│

The time between two frames is measured from the start bit of the last octet of the nth frame to the start bit of the first octet of the n+1st frame. The time between two consecutive frames shall not be less than T23.

┌─────┬───--------──┬─────┐         ┌─────┬───---------───┬─────┐

│ STX │   frame n   │ ETX │         │ STX │   frame n+1   │ ETX │

└─────┴───--------──┴─────┘         └─────┴───---------───┴─────┘

│      T23      │

│<─────────────>│

III.1.3.5.2.4.2	Reception of Frames

Start of a frame is defined as (more than T22*2 since previous reception

of octet) AND (Reception of STX).

End of a frame is defined as (Reception of ETX in octet number (length+2)) OR (more than T22*2 since last reception of octet).

III.1.3.5.2.4.3	Use of ACK and NAK on Receiving Side

ACK is used by the MS to acknowledge a frame on receiving side if all the following conditions are fulfilled. Otherwise NAK is used.

1)	The content of Start field is STX.

2)	The content of Check field corresponds to the exclusive OR sum of previous octets in the frame.

3)	The content of the last octet is ETX.

NAK is used by the SS to request retransmission of a frame. Otherwise ACK is used.

Note:	NAK shall not be used for Layer 3 errors, if the Layer 2 frame is correct. If the meaning of a Layer 3 message is undefined or not performable, then the problem is solved with Layer 3 messages.

III.1.3.5.2.4.4	Use of XON and XOF

XON and XOF are used for flow control of Layer 2.

XOF is sent if the unit (MS or SS) due to internal processing is not capable of receiving a following frame when a frame is being received.

XON is sent if XOF has been sent previously, but the reason for that XOF-transmission no longer exists.

III.1.3.5.2.4.5	Parameters on Layer 2

┌───────────┬────────────────┬────────────────┐

│ Bit rate  │  Value of T22  │  Value of T23  │

├───────────┼────────────────┼────────────────┤

│    600    │    25.0  ms    │    58.3  ms    │

│   1200    │    12.5  ms    │    29.2  ms    │

│   2400    │     6.3  ms    │    14.6  ms    │

│   4800    │     3.1  ms    │     7.3  ms    │

│   9600    │     1.6  ms    │     3.6  ms    │

└───────────┴────────────────┴────────────────┘

Table 7:  Timer Values on Layer 2

III.1.3.5.3	EMMI, Layer 3

III.1.3.5.3.1	Message Structure

Messages are used on Layer 3. They are defined by Message Identifiers (MI) in the range of (0..255). The Message Identifier is always the first, and often the only, octet of the message.

┌─────────┬────────────────────────────────────────────────────┐

│ MI      │ Use                                                │

├─────────┼────────────────────────────────────────────────────┤

│   0-49  │ Not used.                                          │

│         │                                                    │

│  50-179 │ General messages. All undefined values reserved    │

│         │ for further evolution of the EMMI.                 │

│         │                                                    │

│ 180-209 │ ME-type dependent blocks, may be used by the SS    │

│         │ as a sender or  receiver, if so requested by the   │

│         │ manufacturer. Undefined values available for the   │

│         │ manufacturer.                                      │

│         │                                                    │

│ 210-239 │ ME-type dependent blocks, never to be used by the  │

│         │ SS in conformance testing. Undefined values        │

│         │ available for the manufacturer.                    │

│         │                                                    │

│ 240-255 │ Reserved for L3 error handling. All undefined      │

│         │ values reserved for further evolution of the EMMI. │

│         │                                                    │

└─────────┴────────────────────────────────────────────────────┘

Note:	Layer 2 is transparent, but to avoid unnecessary interference from Layer 3, MI with the same value as control frames on Layer 2 is not used.

Table 8 :  Use of Message Identifiers

Most of the messages, especially in the direction SS - MS contain only one octet, the Message Indicator. Some of the messages, especially in the direction MS - SS are quite long.

Note:	If the interface is limited only to the minimum required for automatic conformity testing with the electrical man machine interface, then the included L3 messages should be RQTI, KEYS, BEL1, BEL0, HOK1, HOK0, BCAP and RSTI.

┌───────┬──────┬──────────────────────────────────────┬─────────┐

│ MI    │      │                                      │ Source  │

│ value │ Abbr.│ Meaning                              │ MS   SS │

├───────┼──────┼──────────────────────────────────────┼─────────┤

│       │      │                                      │         │

│  051  │ VOL1 │ Increase volume (***)                │      X  │

│  052  │ VOL0 │ Decrease volume (***)                │      X  │

│       │      │                                      │         │

│  053  │ RQTS │ Request for table, status            │      X  │

│  054  │ RQTI │ Request for table, indication        │      X  │

│  055  │ RQPL │ Request for power level              │      X  │

│       │      │                                      │         │

│  056  │ RQBE │ Request for bell status              │      X  │

│  057  │ RQSM │ Request for Short Message            │      X  │

│       │      │                                      │         │

│  058  │ KEYS │ Perform Keystroke Sequence           │      X  │

│       │      │                                      │         │

│  060  │ BEL1 │ Indication User Alert On             │ X       │

│  061  │ BEL0 │ Indication User Alert Off            │ X       │

│       │      │                                      │         │

│  064  │ HOK1 │ Hook On                              │      X  │

│  065  │ HOK0 │ Hook Off                             │      X  │

│       │      │                                      │         │

│  070  │ BCAP │ Selection of Bearer Capability       │      X  │

│       │      │                                      │         │

│  080  │ STPO │ Set power level                      │      X  │

│       │      │                                      │         │

│  091  │ RSTS │ Response table, status               │ X       │

│  092  │ RSTI │ Response table, indication           │ X       │

│  093  │ RSPO │ Response, power level                │ X       │

│       │      │                                      │         │

│  101  │ RXSM │ Received Short Message               │ X       │

│  102  │ RXSN │ No Short Message received            │ X       │

│       │      │                                      │         │

│  240  │ ER00 │ Internal Malfunction Detected        │ X       │

│  241  │ ER01 │ L3 Message not recognized            │ X    X  │

│  242  │ ER02 │ L3 Message not performable           │ X       │

│       │      │                                      │         │

│  255  │ RESE │ Perform  Hardware and Software Reset │      X  │

└───────┴──────┴──────────────────────────────────────┴─────────┘

(***):	Note: Functioning of this should be verified, as the volume control in the ME might be of another type (non-incremental).

Table 9 :  Block types

III.1.3.5.3.2	Definition of Messages

Messages are defined in the order of the value of the Message Identifier.

051 VOL1        Increase volume

052 VOL0        Decrease volume

Increase/decrease volume in the loudspeaker by one step.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

053 RQTS        Request for table, status

054 RQTI        Request for table, indication

055 RQPL        Request for power level

On receiving a request message RQxy, the corresponding table RSxy shall be sent.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

056 RQBE        Request for Bell status

On receiving a request message RQBE, the internal User Alert status (BEL1 or BEL0) shall be sent. BEL1 and BEL0 will indicate whether the ringing or alert procedure has been activated or not. (See also the definition of BEL1 and BEL0).

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

057 RQSM        Request for Short Message

In response to the RQSM Request the MS shall send either the short message type using the message RXSM or, in case of no Short Message received, the message RXSN.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

058 KEYS        Perform Keystroke Sequence

Perform the MS function related to the received Keystroke Sequence.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  First Key

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  N  Key number N-1

└──┴──┴──┴──┴──┴──┴──┴──┘

The possible Keystroke Sequences are based on the Basic Public Man Machine Interface as defined in GSM 02.30. There exists a minimum set of key characters.

The codes associated with these characters are defined as follows:

Key          Code (decimal)

#                   35

*                   42

+                   43

0                   48

1                   49

2                   50

3                   51

4                   52

5                   53

6                   54

7                   55

8                   56

9                   57

END (function)       18

SEND (function)       20

060 BEL1        Indication User Alert On

061 BEL0        Indication User Alert Off

Indication User Alert shall indicate, on request of the SS, the internal status of the alert or ringing procedure. For this purpose, the SS shall send the RQBE (request for bell status) message to the MS.

BEL1 shall indicate that the alert procedure is active.

BEL0 shall indicate that the alert procedure is not active.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

064 HOK1        Hook On

065 HOK0        Hook Off

Control of the Hook. The Hook On/Off command shall action the normal procedure associated with physically lifting the handset and replacing it whatever that maybe.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

070 BCAP        Selection of Bearer Capability

The EMMI BCAP message shall change the default Bearer Capability on all future calls made via the EMMI "Perform Keystroke Sequence" message. The new default Bearer Capability shall be in effect until the ME is switched off or until the ME receives another EMMI BCAP message.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  Bearer Capability

├──┼──┼──┼──┼──┼──┼──┼──┤     (see Note 1)

│                       │

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │

└──┴──┴──┴──┴──┴──┴──┴──┘

Note 1:	The field "Bearer Capability" in the BCAP message is mandatory and is coded bit for bit exactly as the "Bearer Capability" Information Element as described in GSM 04.08 section 10.5.4.4, beginning with octet 2 (length of the Bearer Capability contents).  Because the "Bearer Capability" is mandatory, the first byte of the field shall be the length of the Bearer Capability content and not the Bearer Capability Information Element Identifier.

080 STPO        Set power level

Used to control the Tx power level of the ME.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  Power Level

└──┴──┴──┴──┴──┴──┴──┴──┘     (see Note 1)

Note 1:	The power level is defined as the 2nd octet of the Power Command Information Element in GSM 04.08.

091 RSTS        Response table, status

Response table status is send as an answer to the corresponding request.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│   F1      │   F2      │  2  Status Field

├──┼──┼──┼──┼──┼──┼──┼──┤     (see Note 1)

│         F3            │

└──┴──┴──┴──┴──┴──┴──┴──┘

Note 1:

F1

0 0 0 0      Spare

F2           (1=yes, 0=no)

Bit 4        L2 link on SACCH established

Bit 3        Speech connection on TCH establ

Bit 2        Listening to BCCH

Bit 1        SDCCH established

F3

Bit 8        Frequency Hopping (yes/no)

Bit 7-1      ARFCN of BCCH of serving cell

092 RSTI        Response table, indication

Response table indication send as an answer to the corresponding request.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│         F1         │F2│  2  Indication Field

└──┴──┴──┴──┴──┴──┴──┴──┘     (see Note 1)

Note 1:

F1

0000000       Spare

F2            (yes=1, no=0)

Bit 1         Service Indication On (yes/no)

093 RSPO        Response, power level

Response power level is sent as an answer to the corresponding request.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  Power Level

└──┴──┴──┴──┴──┴──┴──┴──┘     (see Note 1)

Note 1:	The power level is defined as the 2nd octet of the Power Command Information Element in GSM 04.08.

101 RXSM        Received Short Message

The message RXSM has to be sent by the MS in response to the RQSM Request if a Short Message has been received.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  SM-field octet 1

├──┼──┼──┼──┼──┼──┼──┼──┤     (see note 1)

│                       │

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  N  SM field octet N-1

└──┴──┴──┴──┴──┴──┴──┴──┘

Note 1:	The SM field is defined as the content of the Data field Short Message Storage defined in GSM 11.11 paragraph 3.5.1, excluding byte 1, as follows:

1 byte       NULL value

12 bytes      TP Originating Address

12 bytes      TS Service Centre Address

1 byte       TP Protocol Identifier

1 byte       TP Data Coding Scheme

7 bytes      TP Service Centre Time Stamp

1 byte       TP User Data Length

up to 140 bytes      TP User Data.

If the length of the address for TP and for TS service centre is less than 12 bytes, the remaining bytes are filled with null values.

102 RXSN        No Short Message received

The message RXSN has to be sent by the MS in response to the RQSM Request if no Short Message has been received.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

240 ER00        Internal Malfunction Detected

Used to indicate to the SS that the MS has discovered an internal error. This error message is to be handled in the same manner as the Layer 3 error messages (only to be sent to the SS in response to a Layer 3 message received at the MS).

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

├──┼──┼──┼──┼──┼──┼──┼──┤

│                       │  2  Error Indicator

└──┴──┴──┴──┴──┴──┴──┴──┘     (see Note)

Note: The Error Indicator is defined by the manufacturer of the ME.

241 ER01        L3 Message not recognized

Used by the receiver to indicate to the sender that the message was correctly received, but ignored due to the fact that it was not understandable (eg a KEYS message with one or many values which are not part of the standard set of keys for the MSE). The message to the MS implies a request for resending the latest message sent.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

242 ER02        L3 Message not performable

Used by the receiver to indicate to the sender that the message was understood, but not performable due to intentional lack of functionality in the MS (eg as answer to a CALL message containing a type of service which the MS is not able to use) or to a message requesting a change to a state already existent.

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

255 RESE        Perform HW and SW Reset

The MS shall perform a total reset. The MS shall behave as if it has been switched "off" and "on".

8  7  6  5  4  3  2  1      Octet

┌──┬──┬──┬──┬──┬──┬──┬──┐

│                       │  1  Message Identifier

└──┴──┴──┴──┴──┴──┴──┴──┘

III.1.4	Digital Audio interface

III.1.4.1	General

A special interface is required in order to perform the bit exact test of the speech coder/decoder and to test the SLR/RLR performance of the analogue and acoustic devices. It shall be possible to insert and extract speech data in both the transmit and receive directions. The interruption of the normal speech data paths can be commanded either by a Layer 3 message over the air interface or by special control lines in the test interface. The MS need react to only one of these command methods. The manufacturer shall state which method is to be used.

III.1.4.2	Formal aspects

It shall be possible to connect the SS to the ME or to an adaptor connected to the ME. If an adapter is to be used, it shall be provided by the manufacturer of the ME.

When using the DAI, the MS does not necessarily conform to all

RF requirements.

When the DAI is connected the MS shall be able to correctly send and receive on a TCH and associated channels under ideal radio conditions.

Note:	Prior to tests of the speech coder, other functional entities involved in the tests, such as the channel codec or RF units must have been verified successfully.

III.1.4.3	Hardware aspect of the interface

The data exchanged on the interface are 13 bit linear PCM at 8000 samples per second, which, in order to keep the pin-count low, are transferred on a duplex set of serial lines at 104 kbit/s.

One additional line resets the speech transcoder and the speech A/D and D/A functions. Two lines control the data flow direction and working mode of the interface, one mode being normal operation of the MS. These lines are controlled by the SS. Another line, controlled by the MS, clocks the data as required or available.

This is described in detail below.

III.1.4.3.1	Mechanical characteristics of the interface

The interface shall use a 25-pin DSUB socket, detailed in the ISO 2110 document. The ME shall use a female part.

The manufacturer may provide this interface on an external test "adapter".

The pin assignment of the connector shall be as follows:

┌─────────┬───────────────┬────────────────────────────┬──────────────┐

│  Pin    │  Use          │  Function                  │   Source     │

├─────────┼───────────────┼────────────────────────────┼──────────────┤

│  1      │               │  Chassis ground            │     ME       │

│  2-3    │  EMMI         │  signals                   │              │

│  4-6    │  Not used     │                            │              │

│  7      │  EMMI         │  Signal ground             │              │

│  8-10   │  Not used     │                            │              │

│  11     │  DAI          │  Test control 1            │     SS       │

│  12     │  DAI          │  Signal ground             │              │

│  13     │  DAI          │  Test control 2            │     SS       │

│  14-21  │  Not used     │                            │              │

│  22     │  DAI          │  Reset                     │     SS       │

│  23     │  DAI          │  Data                      │     ME       │

│  24     │  DAI          │  Data clock (104 kHz)      │     ME       │

│  25     │  DAI          │  Data                      │     SS       │

└─────────┴───────────────┴────────────────────────────┴──────────────┘

Note: The EMMI interface is optional and is described in section III.1.3.

III.1.4.3.2	Electrical characteristics of the interface

The state of a signal pin is defined by the voltage (V) between the pin and its associated ground as follows:

┌───────────────────────────┬──────────────────────────────┐

│  Logical state            │           Voltage v          │

├───────────────────────────┼──────────────────────────────┤

│  0 or "LOW" or "ON"       │      0   V < v < + 0,8 V     │

│  1 or "HIGH" or "OFF"     │    + 3,5 V < v < + 5   V     │

│  undefined                │    + 0,8 V < v < + 3,5 V     │

│  forbidden                │  v < 0   V,  v > + 5   V     │

└───────────────────────────┴──────────────────────────────┘

III.1.4.3.3	Timing characteristics of the interface

The following timing applies:

┌────────────────────────────────┬──────────────────────────┐

│     Parameter                  │  Value                   │

├────────────────────────────────┼──────────────────────────┤

│     Clock frequency            │  104 kHz +/- 20 ppm      │

│     Duty cycle                 │  40 to 60 %              │

│     Clock rising edge time     │  <  1 microsecond        │

│     Clock falling edge time    │  <  1 microsecond        │

│     Reset pulse duration       │  >= 4 millisecond        │

└────────────────────────────────┴──────────────────────────┘

|<------------------ 9,6 µs ----------------->|

|                                             |

logical state    |                      |<--- 4,8µs +/-1µs --->|

1 or high        |                      |                      |

---------\   |<--- 4,8µs +/-1µs --->|  /---------------\   |

..............\..|......................|./.................\..|.....90%

|\ |                      |/|                  \ |

..............|.\......................./.|...................\......50%

|  \                     /  |                    \

..............|...\.................../...|.....................\....10%

|   |\                 /|   |                      \

|   | \---------------/ |   |                       \------

|   |                   |   |             logical state ^

|<─>|                   |<─>|              0 or  low ───┘

falling                  rising

edge < 1µs               edge < 1µs

Data shall be stable during the period between 3 microseconds before and

1 microsecond after the rising edge of the clock (50% level).

III.1.4.4	Logical interface

The reset signal is active low.

The data consist of 13 bit words in two's complement format, with the most significant bit transmitted first.

Data are read in by the MS or SS at the rising edge and are output by the SS or MS at the falling edge of the clock, as defined in Figure III.1-2.

The clock signal is high when inactive.

The two test control lines determine the routing of the speech data (DAI or internal, i.e. normal mode) and which device is being tested (speech transcoder/DTX functions or A/D & D/A) as follows:

┌───────────────────┬──────────────────────────────────────────────────┐

│ TEST CONTROL LINE │  FUNCTION                                        │

│    1        2     │                                                  │

├───────────────────┼──────────────────────────────────────────────────┤

│   LOW     LOW     │  Normal operation                                │

│   LOW     HIGH    │  Test of speech decoder/DTX functions (downlink) │

│   HIGH    LOW     │  Test of speech encoder/DTX functions (uplink)   │

│   HIGH    HIGH    │  Test of acoustic devices and A/D & D/A          │

└───────────────────┴──────────────────────────────────────────────────┘

The same test-setup may be achieved by the following Layer 3 TEST_INTERFACE message : see III.1.2.3 and III.1.2.4.7.

III.1.4.5	Functionality of the DAI

To initiate a test, the SS shall apply the appropriate test control signals or send the appropriate Layer 3 messages and then, more than 1 second later, apply a reset pulse.

Upon release of the reset pulse, the MS subsequently starts the test by issuing clock pulses when data are required or are ready.

When testing uplink speech transcoding or DTX functions, the first falling clock edge shall request from the SS the first bit of the speech samples to be encoded, the transmission of which shall start at the next earliest possible interleaved block TDMA frame (as defined in GSM 05.02) after the release of the reset pulse.

When testing downlink speech transcoding or DTX functions, the first falling clock edge shall output to the SS the first bit of the speech samples decoded from the first interleaved block TDMA frames, the reception of which is completed subsequently to the release of the reset pulse.

The MS speech transcoders shall be reset by the end of the reset pulse, whenever it occurs, whilst the DAI is in one of the active states (Test of speech decoder/DTX functions (downlink), Test of speech encoder/DTX functions (uplink), Test of acoustic devices and A/D and D/A).

Figure III.1-2 / GSM 11.10 :	DAI Timing

III.1.5	SIM/ME Test interface

III.1.5.1	General

A special interface is required in order to perform the tests of the SIM/ME interface.

III.1.5.2	Formal aspects

It shall be possible to connect the SIM-simulator to the ME. If an adapter is to be used, it shall be provided by the manufacturer of the ME.

When using the SIM-simulator, the ME does not necessarily conform to all

RF requirements.

When the SIM-simulator is connected the ME shall be able to correctly send and receive on a TCH and associated channels under ideal radio conditions (according to GC3 of Annex 1).

III.1.5.3	Hardware and logical aspects of the interface

The signals on this interface are specified in GSM 11.11.

III.1.5.4	Mechanical characteristics of the interface

The interface of the SIM-simulator offers two connection types-

i) a paddle which is inserted into the ME under test in place of an IC-card SIM, and connects with wires to the measuring equipment. The dimensions of the paddle are shown in Figure III.1-3.

ii) a connector with leads of length 12cm, terminated directly at the contacting elements inside the ME under test.

For ME which use the plug-in SIM, or when the paddle cannot be inserted due to constraints imposed by the ME design, then the ME manufacturer shall, for the purpose of testing the SIM/ME interface only, provide the ME with the connector of type ii) and the leads attached directly to the contacting elements. This connector with the flying leads shall be delivered by the test house.

Figure III.1-3 / GSM 11.10 : Dimensions of Full Size Paddle.

III.1.6	Test-SIM

III.1.6.1	General

A Test-SIM is required to allow the ME to operate its special conformance testing functions, and the tests of Section II to be performed.

The functionality may be provided by a SIM Simulator.

III.1.6.2	Requirements of the Test-SIM

1)	The Test-SIM shall conform to GSM 11.11.

2)	Byte 1 of the Administrative Data Field (Identifier 6F AD) shall be set to MS operation mode "Type approval operations", possibly including "Specific facilities" (see GSM 11.11).  This allows the special conformance testing features of the ME, e.g. loopbacks, to be activated.

3)	A Test Algorithm for authentication shall be incorporated.

4)	The value of Ki stored in the SIM shall be known, to allow its value to be programmed into the SS.

III.1.6.3	Definition of the Test Algorithm for Authentication

The following procedure employs bitwise modulo 2 addition ("XOR").

The following convention applies:

In all data transfer the most significant byte is the first byte to be sent; data is represented so that the left most bit is the most significant bit of the most significant byte.

Step 1:

XOR to the challenge RAND, a predefined number Ki, having the same bit length (128 bits) as RAND.  The result RES1 of this is

RES1 = RAND XOR Ki.

Step 2:

The most significant 32 bits of RES1 form SRES.  The next 64 bits of RES1 form Kc.  The remaining 32 bits are not used.