II.5.3.2	Initial tests

II.5.3.2.1	Channel request

The Random Access procedure is specified in section 3.3.1.1 of GSM 04.08. To verify the correct performance of the random access procedure by the MS, two initial tests are performed, described in II.5.3.2.1.1 and in II.5.3.2.1.2.

II.5.3.2.1.1	Test 1

II.5.3.2.1.1.1	Purpose of the test

The purpose of this test is to verify that the MS

(a)	spreads an initial CHANNEL REQUEST message with equal probability on approximately the correct number of time slots,

(b)	spreads retransmissions of a CHANNEL REQUEST message with equal probability on Tx-integer timeslots,

(c)	retransmits exactly Max-retrans times a CHANNEL REQUEST message if the network never responds to a CHANNEL REQUEST message.

Note:	It is not possible to verify that a Mobile Station spreads an initial CHANNEL REQUEST message on exactly the correct number of time slots, because the exact moment is not known when the Mobile station initially starts timer T3120. It is assumed that a Mobile Station that is paged, will initially start timer T3120 in less than 0.95 seconds, and that the time to initially start timer T3120 will not vary more than 5 ms between different performances of the random access procedure.

II.5.3.2.1.1.2	Method of measurement

For each of the two cases:

-	non combined CCCH/SDCCH, and

-	combined CCCH/SDCCH,

(a), (b) and (c) from II.5.3.2.1.1.1 are checked with low values for

Tx-integer and for Max-retrans in test cases 1 and 2.

The tests are repeated in reduced form with randomly chosen values for

Tx-integer and Max-retrans in test cases 3 and 4.

II.5.3.2.1.1.3	Test case 1

II.5.3.2.1.1.3.1	Initial conditions

The SS simulates a BSS with exactly one cell.

a)	The MS has successfully performed a location update.

b.1)	RACH Control Parameters (GSM 04.08, 10.5.2.17)

-	Tx-integer is set at 5 slots used to spread transmission,

(coded as 0010).

-	Max retransmission is set to 1, (coded as 00).

b.2) Control Channel description (GSM 04.08, 10.5.2.8)

-	CCCH-CONF  indicates 1 basic physical channel used for CCCH not combined with SDCCH, (encoded as 000).

II.5.3.2.1.1.3.2	Procedure 1

In the following, let

N0 = max (8, Tx-integer)

T1 = 0,25

K  = 200.

The following procedure shall be repeated for k= 1, ... , K.

a)	The SS sends a PAGING REQUEST TYPE 1 message on the paging subchannel corresponding to the MS's IMSI with the correct TMSI.

b)	The SS measures the number f(0,k) of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of a CHANNEL REQUEST message from the MS.

c)	The SS stores f(0,k).

d)	For i= 1, ..., Max-retrans  the following steps are performed:

d.1)	The SS measures the number f(i,k) of CCCH RACH slots between

-  the moment where the last CHANNEL REQUEST message has been

received and an additional time of T1 seconds has elapsed,

and

-  the reception of a new CHANNEL REQUEST message from the MS.

If f(i,k) exceeds Tx-integer-1 before a new channel request was

received, the complete test is finished with non-conformance.

d.2)	The SS stores f(i,k).

e)	After reception of the last CHANNEL REQUEST message of d), the SS sends an IMMEDIATE ASSIGNMENT REJECT message; the first Request reference information element of this IMMEDIATE ASSIGNMENT REJECT message corresponds to the last CHANNEL REQUEST message received; the first Wait indication information element of this IMMEDIATE ASSIGNMENT REJECT message specifies 0 seconds.

II.5.3.2.1.1.3.3	Analysis of test results

For i= 0, ..., Max-retrans  and  n=0, ..., Max f(i,k)

let S(i,n) = CARD {k │ f(i,k) = n}.

Note:	CARD {k │ f(k) = n} is mathematical notation for  the number of times that f(k) equals n.

II.5.3.2.1.1.3.4	Requirements

1)	All CHANNEL REQUEST messages sent by the MS during the procedure shall specify the establishment cause "Answer to paging"

(encoded as 100).

2)	After step a), the MS shall send a CHANNEL REQUEST message within

1 second.

3)	S(0,n) <= 41  for all n;  there is an integer n0 such that

S(0,n) = 0  for all  n < n0  and all  n > n0 + N0 + 1.

4)	S(1,n) <= 60  for all n.

Note:	The test limit has been computed to give a confidence of 99.74 % that a unit which follows the requirements will pass. The number of samples [200] has been chosen to get a good compromise between the test time and the risk of passing a bad unit. (Increase of the collision rate !)

II.5.3.2.1.1.4	Test case 2

Comment GSM3: define a subcase: there are 3 time slots available, different paging subchannel upon different IMSI or different TMSI

II.5.3.2.1.1.4.1	Initial conditions

The SS simulates a BSS with exactly one cell.

a)	The MS has successfully performed a location update.

b.1)	RACH Control Parameters (GSM 04.08, 10.5.2.17)

-	Tx-integer is set at 5 slots used to spread transmission,

(coded as 0010).

-	Max retransmission is set to 1, (coded as 00).

b.2) Control Channel description (GSM 04.08, 10.5.2.8)

-	CCCH-CONF indicates 1 basic physical channel used for CCCH, combined with SDCCH, (encoded as 001).

II.5.3.2.1.1.4.2	Procedure 2

In the following, let

N0 = max (8, Tx-integer)

T1 = 0,35

K  = 200.

The following procedure shall be repeated for k= 1, ... , K.

a)	The SS sends a PAGING REQUEST TYPE 1 message on the paging subchannel corresponding to the MS's IMSI with the correct TMSI.

b)	The SS measures the number f(0,k) of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of a CHANNEL REQUEST message from the MS.

c)	The SS stores f(0,k).

d)	For i= 1, ..., Max-retrans the following steps are performed:

d.1)	The SS measures the number f(i,k) of CCCH RACH slots between

-  the moment where the last CHANNEL REQUEST message has been

received and an additional time of T1 seconds has elapsed,

and

-  the reception of a new CHANNEL REQUEST message from the MS.

If f(i,k) exceeds Tx-integer-1 before a new CHANNEL REQUEST

message was received, the complete test is finished with

non-conformance.

d.2)	The SS stores f(i,k).

e)	After reception of the last CHANNEL REQUEST message of d), the SS sends an IMMEDIATE ASSIGNMENT REJECT message; the second Request reference information element of this IMMEDIATE ASSIGNMENT REJECT message corresponds to the last CHANNEL REQUEST message received; the second Wait indication information element of this IMMEDIATE ASSIGNMENT REJECT message specifies 0 seconds.

II.5.3.2.1.1.4.3	Analysis of test results

For i= 0, ..., Max-retrans  and n= 0, ..., Max f(i,k)

let S(i,n) = CARD {k │ f(i,k) = n}.

Note:	CARD {k │ f(k) = n} is mathematical notation for the number of times that f(k) equals n.

II.5.3.2.1.1.4.4	Requirements

1)	All CHANNEL REQUEST messages sent by the MS during the procedure shall specify the establishment cause "Answer to paging"

(encoded as 100).

2)	After step a), the MS shall send a CHANNEL REQUEST message within

1 second.

3)	S(0,n) <= 41  for all n;  there is an integer n0 such that

S(0,n) = 0  for all  n < n0  and for all  n > n0 + N0 + 1.

4)	S(1,n) <= 60  for all n.

Note:	The test limit has been computed to give a confidence of 99.74 % that a unit which follows the requirements will pass. The number of samples [200] has been chosen to get a good compromise between the test time and the risk of passing a bad unit. (Increase of the collision rate !)

II.5.3.2.1.1.5	Test case 3

II.5.3.2.1.1.5.1	Initial conditions

The SS simulates a BSS with exactly one cell.

a)	The MS has successfully performed a location update.

b.1)	RACH Control Parameters (GSM 04.08, 10.5.2.17)

-	Tx-integer is randomly chosen in the range

{6,...,12, 14, 16, 20, 25, 32, 50}  (encoded as 0011, ..., 1111).

-	Max retransmission is randomly chosen

in the range {1, 2, 4, 7}, (coded as 00, ..., 11).

b.2)	Control Channel description (GSM 04.08, 10.5.2.8)

-	CCCH-CONF  indicates 1 basic physical channel used for CCCH not combined with SDCCH, (encoded as 000).

c)	The upper rounded value of 230/Max-retrans is computed, let K be this integer.

d.1)	The upper rounded value of 0.5 * Tx-Integer is computed, let s be this integer.

d.2)	A counter S is put to 0.

II.5.3.2.1.1.5.2	Procedure 3

In the following, let

N0 = max(8,Tx-Integer)

T1 = 0.25.

The following procedure shall be repeated for k= 1, ..., K.

a)	The SS sends a PAGING REQUEST TYPE 1 message on the paging subchannel corresponding to the MS's IMSI with the correct TMSI.

b)	The SS measures the number f(0,k) of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of a

CHANNEL REQUEST message from the MS.

c)	For i= 1, ..., Max-retrans the following is performed:

The SS measures the number f(i,k) of CCCH RACH slots between

-	the moment where the last  CHANNEL REQUEST message has been

received and an additional time of T1 seconds has elapsed, and

-	the reception of a new CHANNEL REQUEST message from the MS.

If f(i,k) exceeds Tx-integer-1 before a new  CHANNEL REQUEST message was received, the complete test is finished with non-conformance.

d)	If f(i,k) is greater or equal to s, the SS increments the

counter S by one,

( in other words f(i,k) >= s  ==> S = S+1 ).

e)	Depending on the value of k, step e.1) or e.2) is performed:

e.1)	k < K:

After reception of the last CHANNEL REQUEST message of d), the SS sends an IMMEDIATE ASSIGNMENT REJECT message; the third Request reference information element of this IMMEDIATE ASSIGNMENT MESSAGE corresponds to the last CHANNEL REQUEST message received; the third Wait indication information element of this IMMEDIATE ASSIGNMENT REJECT message specifies

0 seconds.

e.2)	k = K:

After receipt of the last CHANNEL REQUEST message of d), the SS monitors the RACH for 1 second.

II.5.3.2.1.1.5.3	Requirements

1)	All CHANNEL REQUEST messages sent by the MS during the procedure shall specify the establishment cause "Answer to paging"

(encoded as 100).

2)	After step a), the MS shall send a CHANNEL REQUEST message within

1 second.

3)	S / (K * Max-Retrans) shall be inside the following interval:

[ 0.8 - s / Tx-Integer ; 1.2 - s / Tx-Integer ]

4)	For k = K, after step d), the Mobile Station shall send no more CHANNEL REQUEST messages, but perform cell reselection.

Note:	The confidence interval and the number of samples in requirement 3) are chosen in such a way that the possibility of non accepting a correct MS is less than 0.26 %.

II.5.3.2.1.1.6	Test case 4

Test case 4 is identical to case 3, with only the following difference:

II.5.3.2.1.1.6.1	Initial conditions

As an initial condition, CCCH-CONF indicates "1 basic physical channel used for CCCH and SDCCH" (encoded as 001).

All the other initial conditions remain the same.

II.5.3.2.1.1.6.2	Procedure

As II.5.3.2.1.1.5.2, except that T1 is set to 0.35.

II.5.3.2.1.1.6.3	Requirements

As II.5.3.2.1.1.5.3.

II.5.3.2.1.2	Test 2

II.5.3.2.1.2.1	Purpose of the test

Both for the value of timer T3120 and the random reference which is used as a parameter of the CHANNEL REQUEST message, random numbers must be generated in the range 0, ..., 31 (random reference), or, respectively, 0, ..., K-1 where K = 3,...,12,14,16,20,25,32,50.

It must be sure that different MSs of one product series do not produce the same sequences of random values. In order to test that, it is verified that a MS produces different random references for a CHANNEL REQUEST after having been switched off and on.

II.5.3.2.1.2.2	Initial Conditions

The MS is updated idle. The SS simulates a BS with exactly one cell.

The channel configuration will be non-combined CCCH/SDCCH.

II.5.3.2.1.2.3	Procedure

Let K = 7, D = 4.

The following steps a) to d) are performed for k = 1, ..., K:

a)	The MS is switched off.

b)	The MS is switched on.

c)	After the MS is in service, the SS sends a PAGING REQUEST TYPE 1 message on the paging subchannel corresponding to the MS's IMSI with the correct TMSI.

d)	The SS stores the random reference r(k) contained as a parameter in the next CHANNEL REQUEST message sent by the MS.

Note:	8.5 seconds in step c) allow the Mobile Station to camp on the cell, cf. II.6.1.5.1.

II.5.3.2.1.2.4	Requirements

1)	After step c), the MS must send a CHANNEL REQUEST message within one second.

2)	At least D values of r(1), ..., r(K) must be different.

Note:	For the time of 1 second in requirement 1), cf. the note in II.5.3.2.1.1.1.

D has been computed such that the probability of refusing a correct MS is less then 0.027 %.

II.5.3.2.2	IMSI detach/attach (basic)

The procedure IMSI attach is described in GSM 04.08 section 4.4.3.

The procedure IMSI detach is described in GSM 04.08 section 4.3.4.

II.5.3.2.2.1	Purpose of the test

The purpose of the test is to verify that the MS correctly performs IMSI detach/attach procedures when it is required by the network and upon deactivation/activation or SIM removal/insertion and does not perform these procedures when not required.

II.5.3.2.2.2	Method of test

a)	The SS simulates a base station with BCCH/CCCH, with LAI equal to the one stored in the SIM.

b)	The SS sends SYSTEM INFORMATION TYPE 3 providing the ATT flag value.

Message   SYSTEM INFORMATION TYPE 3 (GSM 04.08, 9.1.31)

Information Element	Comment	Value

Protocol Discriminator	RR	0110

Transaction Identifier	not relevant	0000

Message Type		0001 1011

Cell Identity	default value

LAI	the one stored in the SIM

Control Channel Description	ATT flag (bit 7 in octet 2) (see procedures)

Cell Options	default value

Cell Selection Parameters	  "      "

RACH Control Parameters	  "      "

II.5.3.2.2.3	Procedure 1

a)	The broadcasted ATT flag value is set to 0, that is "The MSs in the cell are not allowed to apply IMSI attach and detach".

b)	The MS is brought in "idle updated" state.

c)	The MS is powered off.

d)	The MS is powered on.

II.5.3.2.2.4	Requirements 1

1)	After step c), the MS shall not initiate the IMSI detach procedure.

2)	After step d), the MS shall not initiate the IMSI attach procedure.

II.5.3.2.2.5	Procedure 2

This procedure is only performed if SIM removal is possible without disconnection of the power supply (see PIXIT statement).

a)	The broadcasted ATT flag value is set to 0, that is "The MSs in the cell are not allowed to apply IMSI attach and detach".

b)	The MS is brought in "idle updated" state.

c)	the SIM is removed from the MS.

d)	The SIM is inserted in the MS.

II.5.3.2.2.6	Requirements 2

1)	After step c), the MS shall not initiate the IMSI detach procedure.

2)	After step d), the MS shall not initiate the IMSI attach procedure.

II.5.3.2.2.7	Procedure 3

a)	The broadcasted ATT flag value is set to 1, that is "The MSs in the cell are allowed to apply IMSI attach and detach".

b)	The MS is brought in "idle updated" state.

c)	The MS is powered off.

II.5.3.2.2.8	Requirements 3

1)	After step c), the MS shall establish a RR-connection and sends the IMSI DETACH INDICATION message.

Message   IMSI DETACH IND

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0x00 0001

Mobile Classmark 1	as specified by the manufacturer

Mobile Identity	TMSI

II.5.3.2.2.9	Procedure 4

a)	The MS is switched on.

b)	The SS shall respond to the channel request with IMMEDIATE ASSIGN allocating a SDCCH. It shall initiate no authentication or cipher mode setting but just respond to the location updating request with a LOCATION UPDATED ACCEPT (provided the request is correct) allocating a new TMSI.

II.5.3.2.2.10	Requirements 4

1)	After step a), the MS shall initiate a location updating with the type "IMSI attach"

2)	After step b), the MS shall answer with a TMSI REALLOCATION COMPLETE.

Message   LOCATION UPDATE REQUEST (GSM 04.08, 9.2.13)

Information Element	Comment	Value

Protocol Discriminator	MM		0101

Transaction Identifier	not relevant	0000

Message Type		0x00	1000

Location Updating Type	IMSI attach		0010

Ciphering key sequence number		0000

LAI	the one stored in the SIM

Mobile station Classmark 1	as specified by the manufacturer

Mobile Identity	TMSI

Message   LOCATION UPDATE ACCEPT (GSM 04.08, 9.2.11)

Information Element	Comment	Value

Protocol Discriminator	MM		0101

Transaction Identifier	not relevant	0000

Message Type		0000	0010

LAI	the one stored in the SIM

Mobile Identity	TMSI

Message   TMSI REALLOCATION COMPLETE (GSM 04.08, 9.2.15)

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0x01 1011

II.5.3.2.2.11	Procedure 5

This procedure is only performed if SIM removal is possible without disconnection of the power supply (see PIXIT).

a)	The broadcasted ATT flag value is set to 1, that is "The MSs in the cell are allowed to apply IMSI attach and detach".

b)	The MS is brought in "idle updated" state.

c)	the SIM is removed from the MS.

II.5.3.2.2.12	Requirements 5

1)	After step c), the MS shall establish a RR-connection and sends the IMSI DETACH INDICATION message.

Message   IMSI DETACH IND

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0x00 0001

Mobile Classmark 1	as specified by the manufacturer

Mobile Identity	TMSI

II.5.3.2.2.13	Procedure 6

This procedure is only performed if SIM removal is possible without disconnection of the power supply (see PIXIT).

a)	The MS is switched on and then the SIM is inserted.

b)	The SS shall respond to the channel request with IMMEDIATE ASSIGN allocating a SDCCH. It shall initiate no authentication or cipher mode setting but just respond to the location updating request with a LOCATION UPDATED ACCEPT (provided the request is correct) allocating a new TMSI.

II.5.3.2.2.14	Requirements 6

1)	After step a), the MS shall initiate a location updating with the type "IMSI attach"

2)	After step b), the MS shall answer with a TMSI REALLOCATION COMPLETE.

Message   LOCATION UPDATE REQUEST (GSM 04.08, 9.2.13)

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0x00	1000

Location Updating Type	IMSI attach		0010

Ciphering key sequence number		0000

LAI	the one stored in the SIM

Mobile station Classmark 1	as specified by the manufacturer

Mobile Identity	TMSI

Message   LOCATION UPDATE ACCEPT (GSM 04.08, 9.2.11)

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0000	0010

LAI	the one stored in the SIM

Mobile Identity	TMSI

Message   TMSI REALLOCATION COMPLETE (GSM 04.08, 9.2.15)

Information Element	Comment	Value

Protocol Discriminator	MM	0101

Transaction Identifier	not relevant	0000

Message Type		0x01 1011

II.5.3.2.3	Test of sequenced MM/CC message transfer

II.5.3.2.3.1	General

According to GSM 04.08, section 2.4.1 [Variables and sequence numbers] the RR sublayer of the MS shall have an associated send state variable V(SD) ("Send duplicated") for sending MM and CM messages. This send state variable has been introduced to avoid the duplication of MM and CM messages, e.g. in the case of Handover.

II.5.3.2.3.2	Purpose of the test

-	Check that the V(SD) is set correctly to 0 by the MS at the beginning of the establishment of the first RR connection.

-	Check that the MS increments correctly this variable.

II.5.3.2.3.3	Method of test

a)	The SS originates a link establishment and the following messages are sent on the air-interface:

-	PAGING REQUEST(#1) (SS>MS)

-	CHANNEL REQUEST (MS>SS)

-	IMMEDIATE ASSIGNMENT (SS>MS)

-	PAGING RESPONSE (MS>SS) (Layer 2 establishment).

b)	The SS sends the Mobility Management Message IDENTITY REQUEST.

c)	Step b) is repeated 10 times.

II.5.3.2.3.4	Requirements

1)	The MS shall send an IDENTITY RESPONSE message where N(SD)=0.

2)	In step c), the MS shall send alternately the value 1 and 0 in the N(SD) field of the IDENTITY RESPONSE message.