II.4	RECEIVER

ref: GSM 05.05

In this section on receiver measurements, the procedures to test equipment which is fitted with a permanent antenna connector, and the procedures to test equipment which is designed to only be used with an integral antenna, are in general combined into one single test description.

Tests on Mobile Stations fitted with an integral antenna and having no means of connecting an external antenna are specified in terms of received field strength. In order to perform most tests on such Mobile Stations without the need for separated access to a calibrated test site a temporary antenna connector shall be used as defined in General Conditions GC7 of Annex 1 of part GC. In accordance with ETS 300 086 for equipment of this type, tests of sensitivity are performed only using the integral antenna. The detailed description of its calibration is contained within section II.4.2.2.

In practice the temporary antenna connector may be used for transmitter measurements described in section II.3, but the calibration factors determined in II.4.2.2 will not be directly usable. The detailed calibration, when needed, for transmission tests are described in the relevant sections of II.3.

Testing philosophy

Certain assumptions concerning the functional mechanisms of GSM receivers have been made in order to define tests that will verify the receiver performance without excessive redundancy and excessive test times.

The receiver functions can be divided into:

-	Analogue RF and IF stages that are affected by input levels, temperature and power supply levels.

-	Demodulator that is affected by input levels and interfering signals.

-	Decoders that are affected by the different logical channels and input levels.

The tests are designed to stress each of these blocks with a minimum of redundancy.

Statistical testing of receiver BER/FER performance

Error Definition

1)	Frame Erasure Ratio (FER)

A frame is defined as erased if the error detection functions in the receiver, operating in accordance with GSM 05.03, indicate an error. For full rate speech this is the result of the 3 bit cyclic redundancy check (CRC) as well as other processing functions that cause a Bad Frame Indication (BFI). For signalling channels it is the result of the FIRE code or any other block code used. For data traffic FER is not defined.

2)	Residual Bit Error Ratio (RBER)

The Residual Bit Error Ratio is defined as the Bit Error Ratio (BER) in frames which have not been declared as erased.

3)	Bit Error Ratio (BER)

The Bit Error Ratio is defined as the ratio of the bits wrongly received to all data bits sent.

Test method

Each test is performed in the following manner:

a)	Set up the required test conditions. Set the parameters Max-samples (maximum number of samples) and Max-events (maximum number of error events) to specific values for each test.

b)	Perform the test and record the number of offered samples (bits or frames sent) and the number of occurred events (bit or frame errors).

c)	Terminate the test and determine the test result ("pass" or "fail") when either of the following conditions become true:

-  Number of offered samples >= Max-samples   --->  "PASS"

-  Number of (error) events  >  Max-events    --->  "Fail"

In practice it may be sufficient to generate "Max-samples" and later look at Number of error events observed.

Note:	At least one measurement reporting period must be left, after the conditions for any measurement has been set, before measuring RF performance.

Test criteria

The limits on number of samples and events shall be defined in order to comply with different requirements :

1)	to keep reasonably low the risk of passing a bad unit through the individual tests;

2)	to have high probability of passing a good unit through the individual tests;

3)	to perform a measurement with a significant statistics;

4)	to keep the test time as low as possible.

The risk of passing a bad unit (point 1) should be kept lower than 0.2%. A unit is generally considered "bad" if its BER (or FER) performance is 1.5 times worse than that specified in AWGN (Additive White Gaussian Noise) and 1.26 times worse than that specified in multipath environment. These values have been adopted (taking into account the expected shapes of the BER performance) in order not to pass a unit with performance worse than the specifications by more than 1 dB.

The probability of passing a good unit (point 2) should be at least 99.7%.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures lead to consider a number of events (point 3) not lower than 200 in AWGN channel and not lower than 600 in multipath environment, and to test a BER (or FER) performance 1.22 times worse than that specified in AWGN and 1.12 times worse than that specified in multipath environment (this corresponds to test a performance at the most 0.5 dB worse than that specified).

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in the following table.

┌────────────────────────┬─────────┬────────┬─────────┬─────────┐

│ PROPAGATION CONDITIONS │   TU3   │  TU50  │  HT100  │  RA250  │

├────────────────────────┼─────────┼────────┼─────────┼─────────┤

│ MIN. TEST TIME (sec)   │   500   │   30   │    15   │    6    │

└────────────────────────┴─────────┴────────┴─────────┴─────────┘

The table below details, for the different test conditions, the number of events and samples required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

As it can be seen in the table, in the cases in which both FER and RBER have to be tested on the same channel the same time as the FER measurement has been adopted. This is longer than that required for the RBER only according to the discussed criteria, but allows the relevant accuracy to be improved without increasing the total test time.

TABLE OF TEST CONDITIONS

════════════════════════════════════════════════════════════════════════════════════════════════════════

PROPAGATION/ SPECIFIED TESTED  Max No   Max No  Prob that BAD UNIT Risk that

Type of     Type of         Frequency    FER/BER  FER/BER   of       of    GOOD UNIT  BER/FER  BAD UNIT

test       channel         Conditions      %        %    EVENTS  SAMPLES  will pass     %    will pass

════════════════════════════════════════════════════════════════════════════════════════════════════════

BFI         TCH/FS           STATIC       0.200    0.244     200      82000   99.719    0.300    0.139

Sensitivity TCH/FS           STATIC/FH    0.100*Ó  0.122*Ó   200*Ó   164000   99.717    0.150*Ó  0.140

,,          TCH/FS Class Ib  STATIC/FH    0.400/Ó  0.410/Ó 82000/Ó 20000000  100.000    0.600/Ó <0.001

,,          TCH/FS Class II  STATIC/FH    2.000    2.439     200       8200   99.714    3.000    0.001

,,          TCH/FS           TU50/No FH   6.000*Ó  6.742*Ó   600*Ó     8900   99.825    7.560*Ó  0.162

,,          TCH/FS Class Ib  TU50/No FH   0.400/Ó  0.420/Ó  4200/Ó  1000000   99.919    0.504/Ó <0.001

,,          TCH/FS Class II  TU50/No FH   8.000    8.333   10000     120000   99.999   10.080   <0.001

,,          TCH/FS Class II  HT100/No FH  9.000    9.333    5600      60000   99.779   11.340   <0.001

,,          TCH/FS Class II  RA250/No FH  7.000    7.500    1800      24000   99.873    8.694   <0.001

,,          FACCH            TU50/No FH  13.000   14.634     600       4100   99.899   16.380    0.119

,,          TCH/F9.6&H4.8    HT100/No FH  0.700    0.778    1400     180000   99.995    0.882   <0.001

,,          TCH/F4.8         HT100/No FH  0.010    0.011     600    5350000   99.732    0.013    0.197

,,          TCH/F2.4         HT100/No FH  0.001    0.001     150   11900000   99.734    0.002   <0.001

,,          TCH/H2.4         HT100/No FH  0.010    0.011     600    5350000   99.732    0.013    0.197

INPUT LEVEL TCH/FS Class II  STATIC       0.010    0.012     200    1640000   99.716    0.015    0.141

RANGE       TCH/FS Class II  EQ50         3.000    3.250    3900     120000  100.000    3.780   <0.001

CO-CHANNEL  TCH/FS           TU3/No FH   21.000*Ó 24.000*Ó  6000*Ó    25000  100.000   27.720*Ó <0.001

REJECTION   TCH/FS Class Ib  TU3/No FH    2.000/Ó  2.091/Ó 69000/Ó  3300000  100.000    2.520/Ó <0.001

,,          TCH/FS Class II  TU3/No FH    4.000    4.300   86000    2000000  100.000    5.040   <0.001

,,          TCH/FS           TU3/FH       3.000*Ó  3.371*Ó   600*Ó    17800   99.797    3.780*Ó  0.194

,,          TCH/FS Class Ib  TU3/FH       0.200/Ó  0.215/Ó  4300/Ó  2000000  100.000    0.252/Ó <0.001

,,          TCH/FS Class II  TU3/FH       8.000    8.333  100000    1200000  100.000   10.080   <0.001

,,          FACCH            TU3/No FH   22.000   24.000    6000      25000  100.000   27.720   <0.001

,,          TCH/F9.6 or H4.8 TU3/FH       0.300    0.336     600     178500   99.716    0.378    0.180

,,          TCH/F4.8         TU3/FH       0.010    0.011     600    5350000   99.732    0.0126   0.197

,,          TCH/F2.4         TU3/FH       0.001    0.001     150   11900000   99.734    0.002   <0.001

,,          TCH/H2.4         TU3/FH       0.010    0.011     600    5350000   99.732    0.0126   0.197

ADJACENT    TCH/FS           TU50/No FH   6.000*Ó  6.742*Ó   600*Ó     8900   99.825    7.560*Ó  0.162

CHANNEL     TCH/FS Class Ib  TU50/No FH   0.400/Ó  0.420/Ó  4200/Ó  1000000   99.919    0.504/Ó <0.001

200 KHz     TCH/FS Class II  TU50/No FH   8.000    8.333   50000     600000  100.000   10.080   <0.001

,,          FACCH            TU50/No FH  13.000   14.634     600       4100   99.899   16.380    0.119

ADJACENT    TCH/FS           TU50/No FH  10.200*Ó 11.461*Ó  1020*Ó     8900   99.995   12.852*Ó  0.004

CHANNEL     TCH/FS Class Ib  TU50/No FH   0.720/Ó  0.756/Ó  7560/Ó  1000000   99.999    0.907/Ó <0.001

400 kHz     TCH/FS Class II  TU50/No FH   8.800    9.167   55000     600000  100.000   11.088   <0.001

,,          FACCH            TU50/No FH  10.000   11.215     600       5350   99.836   12.600   <0.105

INTERMOD.   TCH/FS Class II  STATIC       2.000    2.439     200       8200   99.741    3.000    0.122

FACCH            TU50/No FH  13.000   14.634     600       4100   99.899   16.380    0.119

BLOCKING &  TCH/FS Class II  STATIC       2.000    2.439     200       8200   99.741    4.000   <0.001

SPURIOUS    FACCH            TU50/No FH  13.000   14.634     600       4100   99.899   16.380    0.119

RESP.

════════════════════════════════════════════════════════════════════════════════════════════════════════

Note 1:	Ó is a parameter which ranges from 1 to 1.6. The value of Ó for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions. For example, the value of Ó may be different for a TU50 sensitivity test and an RA250 sensitivity test. The value of Ó is determined by dividing the measured number of events for the FER test by the value of the maximum number of events listed in the table corresponding to Ó=1; if the result of the division is lower than 1, a value of Ó=1 shall be used. The probabilities that a good unit will pass and the risks that a bad unit will pass, listed in the table are valid for Ó=1, and would be slightly different for other values of Ó.

Note 2:	In order to save time the sensitivity test and co-channel rejection test for the TCH/F2.4 channel do not comply with the above said constraints. In fact, a bad unit which performs 2 times (instead of 1.26) worse than that specified is accounted for, so reducing the required number of events to 150, instead of 600. On the other hand, the specified RBER is in this case 10E-5 and, on the basis of simulations and hardware validation results, doubling this RBER results in a drop in performance of less than 1 dB.

II.4.1	Bad frame indication performance

ref: GSM 05.05 section 6.4

II.4.1.1	Definition

The performance of the bad frame indication (BFI) is a measure of the effectiveness of the MS under DTX conditions. It includes the effect of the 3 bit Cyclic Redundancy Check (CRC) and all other processing associated with the DTX function. The BFI is measured on a full rate speech TCH (TCH/FS) by counting the number of undetected bad frames whilst the input signal is a randomly modulated carrier described in g) below.

This test is only applicable to MS supporting speech.

Note:	The DTX is used to prevent that the MS would drop the call.

II.4.1.2	Method of measurement

a)	A call shall be originated by the SS to the MS, and the MS  shall be made to answer the call.

The call shall occupy a full rate channel and shall be in a non-hopping mode on one of the radio frequencies in the range 60 to 65 (ARFCN). Throughout the test the BCCH shall be transmitted by the SS at an EMF of 28dBµV(  ).

b)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the bad frame indication to the SS.

c)	The SS shall simulate a BSS in DTX mode. During the period when no transmission would occur the SS shall transmit a GSM carrier modulated with random data at a level of 19 dBµV(  ). SACCH shall be transmitted normally at a level of 28 dBµV(  ). The SID frame shall be transmitted in its correct time interval with valid information at a level of 28 dBµV(  ). During transmission of SACCH or SID the random data shall be discontinued.

d)	The SS will proceed to transmit Max-samples of frames of TCH/FS information and will check the bad frame indication (BFI) of the looped back signal from the MS. The SS shall record the number of frames where the bad frame indication is not set. During transmission by the SS of SID the SS shall check that the BFI is not set.

NOTE:	Further explanations on the mechanism of signalling the BFI to the SS will be found in sections III.1.1 and III.1.3.

II.4.1.3	Limits

The BFI performance is accepted if the number of undetected BFIs does not exceed the Max-events:

Max-events 	-	200

Max-samples	-	82000 (excluding SID frames).

During loop back of SID frames no BFI shall be set.

II.4.2	Sensitivity

The reference sensitivity (GSM 05.05) is the signal level at the MS receiver input at which a certain BER or FER must be achieved.

II.4.2.1	Reference Sensitivity for TCH/FS - compliance

ref: GSM 05.05 section 6.2

II.4.2.1.1	Definition

Compliance with the requirements for reference sensitivity level is tested by setting the receiver at an input signal at the specification limit for reference sensitivity level and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a data signal with a BER or a FER less than the specified values is produced.

II.4.2.1.2	Method of measurement

a)	Set up a call on a traffic channel in the range ARFCN 60 to 65. The SS shall also set up six adjacent cell BCCH's having signal strengths in the range 15 dBµVemf(  ) to 35 dBµVemf(  ). The ARFCNs for the serving and adjacent cell BCCHs shall not be co-channel with, or on adjacent channel to the wanted traffic channel.

Note:	When frequency hopping is used, the traffic channel may fall on any of the ARFCNs defined in Annex 1, GC1.

b)	The MS shall be commanded to the maximum power level.

c)	The SS traffic channel shall be the Standard Test Signal C1 at a MS level of 28 dBµVemf(  ).

d)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS. The fading function of the SS is set to TU50.

e)	The amplitude of the wanted signal shall be set to 11dBµVemf(  ) for hand portable MSs and to 9dBµVemf(  ) for other MSs.

f)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication. Also throughout the test, the SS monitors the status message from the MS to confirm correct reporting of the adjacent cell BCCH's.

g)	The SS determines the number of residual bit error events for the bits of class II, by examining sequences of Max-samples of consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.

h)	The SS determines the number of residual bit error events for the bits of the class Ib, by examining sequences of Max-samples of consecutive bits of class Ib, where bits are taken from those frames for which no bad frame indication was given.

i)	The SS also determines the frame erasure events by examining sequences of Max-samples of consecutive frames and assuming a frame is received successfully when there is no bad frame indication concerning it.

j)	The MS is placed in a climatic test chamber (for extreme test conditions see Annex 1, TC), and steps a) up to including g) are repeated for the following combinations of temperatures and power supply voltages:

Temp		:	Hi	Hi	Lo	Lo

Voltage	:	Hi	Lo	Lo	Hi

k)	Steps a) to j) shall be repeated for a channel in the range 1 to 5 (ARFCN) and for a channel in the range 120 to 124 (ARFCN).

l)	Steps a) to g) shall be repeated, except under step d) the SS fading function shall be set in turn to RA250 and HT100.

m)	Steps a) to j) shall be repeated, except at step d) the SS fading function shall be set to static and the MS shall be commanded by the SS into hopping mode using the hopping sequence of GC1 of Annex 1, full band.

The amplitude of the wanted signal shall be set as in step e). All the other time slots, except the active ones, shall be set to 28dBµVemf(  ). This will implicitly test adjacent time slot rejection.

II.4.2.1.3	Limits

The Max-events measured for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the values given in the table below:

───────────────────────────────────────────────────────────────────────────

Propagation    Propagation    Propagation        Static

Conditions     Conditions     Conditions       Conditions

Channels          TU50           RA250          HT100

───────────────────────────────────────────────────────────────────────────

TCH/FS         Max-   Max-    Max-   Max-    Max-   Max-     Max-    Max-

Events samples events samples events samples  events  samples

───────────────────────────────────────────────────────────────────────────

FER             600*Ó   8900                               200*Ó    164000

───────────────────────────────────────────────────────────────────────────

class Ib(RBER) 4200/Ó 1000000                             82000/Ó  20000000

───────────────────────────────────────────────────────────────────────────

class II(RBER) 10000   120000  1800  24000   5600   60000   200        8200

───────────────────────────────────────────────────────────────────────────

where Ó is a parameter which can range from 1 to 1.6. The value of Ó for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.2.2	Sensitivity Measurements for Equipment without an antenna connector

For equipments fitted with an integral antenna and not provided with a permanent means for connection to an external antenna a calibration procedure is required to allow subsequent measurements to be performed on the temporary antenna connector.

Once calibrated this temporary antenna connector enables all receiver test procedures, except sensitivity tests to be identical for equipments with an integral antenna and for equipments with an antenna connector.

In accordance with ETS 300 086 tests of receiver sensitivity on MSs with no permanent external antenna connector are performed on the integral antenna only.

The calibration procedure shall be carried out at three frequencies, namely an ARFCN in the range 1 to 5, an ARFCN in the range 60 to 65 and an ARFCN in the range 120 to 124. The procedure consists of three distinct stages as follows :

1)	Establish the MS antenna radiation pattern for the three selected frequencies.

2)	Calibrate the test range (or anechoic shielded chamber) for the conditions needed in 1).

3)	Determine the temporary antenna connector coupling factor.

II.4.2.2.1	Antenna Radiation Pattern

a)	The MS shall be in the anechoic shielded chamber, or on an outdoor test site, on an isolated support in a vertical position at an orientation specified by the manufacturer. This position is the

0 degree position.

A test antenna, connected to the SS shall be in the anechoic shielded chamber, or on the outdoor test site, at a distance of at least 3 metres from the MS.

b)	A call shall be originated by the SS to the MS on a frequency in the range ARFCN 1 to 5. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power.

c)	The SS shall, using estimated parameters for the outdoor test site or anechoic shielded chamber, set its output level 'E' (see Fig. II.4-1) to give an MS receiver input level of approximately 32 dBµVemf. This corresponds to a field strength of 55.5 dBµV/m at the MS position. The signal shall be the Standard Test Signal C1.

Note:	The absolute value of the received signal level is not critical. The value suggested however will ensure that the MS receiver is operating essentially error free, yet is low enough to avoid any non linear effects in the receiver.

d)	The SS shall use the RXLEV message from the MS to determine a measure of the received field strength. The procedure detailed in the flow chart of figure II.4-1 shall now be followed. The signal level from the SS that just results in the transition from RXLEVa to RXLEVb shall be recorded as Ei.

Note:	The actual values of RXLEVa and RXLEVb will need to be recorded, because this transition will be used as the reference point for all further stages of the calibration procedure.

e)	Step d) shall be repeated after the MS has been rotated by n * 45 degrees in the horizontal plane. Ensuring that the same RXLEV transition is used, the signal levels from the SS shall be recorded as Ein.

FIG II.4-1

f)	Calculate the effective mean signal level from the RMS value of the eight signal levels obtained in d) and e) above by using the following formula :

┌      8      ┐1/2

│ __________  │

E1  =  │  n=7  1     │

│   õ  ────   │

└  n=0  Ein²  ┘

g)	Repeat steps b) to f), except in step b) use an ARFCN in the range

60 to 65 to obtain a mean signal level E2. Ensure the same RXLEV transition is used.

h)	Repeat steps b) to f), except in step b) use an ARFCN in the range 120 to 124 to obtain a mean signal level E3.

Ensure the same RXLEV transition is used.

II.4.2.2.2	Test Range Calibration

The objective of this step is to determine the actual field strength at the MS corresponding to the three signal levels E1, E2 and E3 established in II.4.2.2.1. The following procedure shall be used :

a)	Replace the MS by a calibrated reception antenna connected to a measuring receiver.

b)	For each frequency used in II.4.2.2.1 measure the field strength Efr corresponding to the respective signal levels Er determined in steps f), g) and h) of II.4.2.2.1 record these values as Ef1, Ef2, Ef3.

II.4.2.2.3	Temporary Antenna Connector Coupling Factor

The coupling factor of the temporary antenna connector is the relationship expressed in dB, between the output signal of the SS and the effective receiver input signal for the MS.

The test sample MS is modified to fit a temporary antenna connector in accordance with Annex 1 GC7. Or alternatively a second MS shall be provided, fitted with such a temporary antenna connector.

Note:	If only one MS is supplied for testing, the tests of radiated spurious emissions (transmit and receive) and receiver sensitivity shall be performed before the MS is modified to accept a temporary antenna connector.

The calibration procedure shall be as follows :

a)	The MS temporary connector is connected to the output of the SS.

b)	A call shall be originated by the SS to the MS using a frequency in the range ARFCN 1 to 5. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power, non hopping encrypted mode.

c)	The SS shall, using the procedures of II.4.2.2.1, adjust its output signal level to determine the RXLEVa to RXLEVb transition. This signal level shall be recorded as Ec1.

d)	Repeat steps b) and c) for frequencies in the range ARFCN 60 to 65 and 120 to 124. Record the RXLEV transitions as Ec2 and Ec3 respectively.

e)	The temporary antenna connector coupling factor F is then calculated from:

┌   Ecn    ┐

Fn = 20 log10 │  ─────── │

└  Efn*Kn  ┘

where Kn = conversion factor of an isotropic antenna

expressed as    µV  at the frequency

µV/m

corresponding to the ARCFN used.

f)	The mean antenna coupling factor Fm to be used for measurements requiring hopping shall be calculated from the RMS value of all parameters in e) as follows :

┌             3           ┐1/2

Ecm  =  │  ────────────────────   │

└ 1/Ec1² + 1/Ec2² + 1/Ec3²┘

┌             3           ┐1/2

Efm  =  │  ────────────────────   │

└ 1/Ef1² + 1/Ef2² + 1/Ef3²┘

┌  k1² + k2² + k3²  ┐1/2

km   =  │  ─────────────    │

└         3         ┘

┌   Ecm    ┐

Fm   =  20 log10 │  ─────── │

└  Efm*km  ┘

g)	In all tests in which a handheld MS with integral antenna is the unit under test, the input signal level into the temporary antenna connector is determined from :

Ein 	  =	Ereq + F

where: Ein   = input signal level to coupling device (dBµVemf)

Ereq  = signal level required by the test (dBµVemf)

F     = coupling factor at the respective ARFCN (dB)

This is indicated in the test procedures as Ereq, dBµVemf(   ), where the empty parenthesis is to be read as Ein.

II.4.2.3	Sensitivity for data channels and control channels

II.4.2.3.1	Definition

Compliance with the requirements for reference sensitivity on control channels is tested by setting the receiver at an input signal at the specification limit for reference sensitivity and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a FER less than the specified value be produced. For data channels, reference sensitivity is defined in terms of BER.

For MS fitted with a permanent antenna connector the method of test below applies.

For MS not fitted with a permanent antenna connector the method of test below applies except:

a)	The MS shall be in the anechoic shielded chamber as described in II.4.2.2.1.

b)	At step e) the wanted signal is generated at the SS to give a field strength Ef2 equivalent to a wanted signal of 11 dBµVemf(  ) for hand held MS or 9 dBµV for any other MS at the receiver front end when the MS antenna radiation pattern and test range characteristics have been allowed for..

II.4.2.3.2	Method of Test

a)	The SS originates a call to the MS on a traffic channel having a ARFCN in the range 60 to 65.

b)	The MS is made to answer the call, and is commanded to it's maximum power level.

c)	The SS output on the traffic channel is set to 28 dBµVemf(  ), producing Standard Test Signal C1.

d)	The fading function of the SS is set to TU50.

e)	The wanted signal is set to a level of 11 dBµVemf(  ) for hand held MS or 9 dBµV for any other MS.

f)	The SS sends the STATUS message. Due to the low signal level the MS may not be able to acknowledge the Layer 2 frame with an RR frame and the SS will repeat the Layer 2 frame. Each repeated L2 frame will be counted and will indicate a frame erasure event.

g)	The SS  determines the frame erasure events during Max-samples of FACCH frames.

h)	The wanted signal is set to 28dBµV and the SS commands the MS to a TCH/F9.6 channel.  It also commands the MS to close its TCH loop, specifying that erased frames are to be signalled by the MS (see section III.1.2.1.1.1) in order to loop back received data from the channel decoder via the channel encoder to the uplink TCH.

i)	Set the SS fading function to HT100 and the wanted signal to 11dBµV(  ) for a hand portable MS or 9dBµV for any other MS.

j)	The SS compares transmitted data with received data for Max-samples consecutive bits and records every error bit as an error event.

k)	Steps h) to j) are repeated for data channels of TCH/H4.8, TCH/F4.8, TCH/F2.4 and TCH/H2.4.

Note 1:	Not all Mobile Stations will be equipped with all data channels.  The test procedure will need to be adapted to cater for limited sub sets for Mobile Stations offering a restricted data capability.

Note 2:	The SACCH, SDCCH, AGCH and PCH channels are not tested because the coding is identical to the FACCH.  These channels are tested implicitly in Layer 3.

II.4.2.3.3	Limits

─────────────────────────────────────────────────────

Channels                    Max-events    Max-samples

─────────────────────────────────────────────────────

FACCH                         600              4100

TCH/F9.6&H4.8                1400            180000

TCH/F4.8                      600           5350000

TCH/F2.4                      150          11900000

TCH/H2.4                      600           5350000

─────────────────────────────────────────────────────

II.4.3	Usable receiver input level range

ref: GSM 05.05 section 6.1

II.4.3.1	Definition

The usable receiver input level range is the range of the radio frequency input level of a specified modulated signal over which bit error ratio or frame erasure ratios stay between specified limits.

This test is only applicable to MS supporting speech.

II.4.3.2	Method of measurement

a)	The SS shall originate a call to the MS and the MS shall be made to answer the call on a TCH/FS having an ARFCN in the range 60 to 65.

b)	The SS shall be set on the TCH/FS to produce Standard Test Signal C1 at a level of 28 dBµVemf(  ).

c)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input without signalling the frame erasure event (III.1.2.1.1.2).

d)	The SS compares the data that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding.

The SS tests the bit error ratio for the non-protected bits of

TCH/FS class II, by examining sequences of Max-samples consecutive bits of class II. The number of error events is recorded.

e)	Step d) shall be repeated with the amplitude of the wanted signal increased to input levels of respectively 63dBmicroVoltemf(  ) and 103dBmicroVoltemf(  ) at the receiver input.

f)	The SS fading function is set to EQ50.

g)	Step d) shall be repeated with the amplitude of the wanted signal set to respectively 28dBmicroVoltemf(  ) and 73dBmicroVoltemf(  ) at the receiver input.

h)	The Mobile Station is placed in a climatic chamber and steps a) to g) are repeated for the following combinations of extreme test conditions (Annex 1, TC2.2 and TC3):

Temp		:	Hi 	Hi	Lo	Lo

Voltage	:	Hi	Lo	Lo	Hi

II.4.3.3	Limits

The number of error events recorded in this test shall not exceed the

Max-events values given in the table below when Number of samples = Max-samples. This shall apply for any combination of normal and extreme test voltages and ambient temperature, for the different propagation conditions and for any level of input signal to the receiver.

────────────────────────────────────────────────────

Propagation conditions      Max-events   Max-samples

────────────────────────────────────────────────────

Static                     200        1640000

EQ50                      3900         120000

────────────────────────────────────────────────────

II.4.4	Co-channel rejection

II.4.4.1	Definition

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal, both signals being at the nominal frequency of the receiver.

II.4.4.2	Method of Measurement

a)	A call shall be originated by the SS to the MS on a channel in the range ARFCN 60 to 65, and the MS shall be made to answer the call. For MS supporting speech this shall be a TCH/FS, for MS not supporting speech any one of the supported TCH/ (F9.6, H4.8, F4.8, F2.4 or H2.4) shall be used and the test of TCH not frequency hopping is not performed.

The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28dBmicroVoltemf(  ).

The unwanted signal shall have no fixed relationship with the bit transitions of the wanted signal, and it shall be modulated with random data. Its amplitude shall be 9dB below that of the wanted signal. The unwanted signal shall be continuous.

b)	The MS shall be operated in the encrypted mode.

c)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS. (ref III.1.2.1.1.1)

d)	The SS is set to produce a wanted signal and an independent, uncorrelated interfering (unwanted) signal at the same time.  The fading characteristic of the wanted and the interfering signal shall be TU3. The channel frequency shall be in the range 60 to 65 (ARFCN).

e)	The SS compares the modulation of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure ratio compliance for the TCH/FS, by examining Max-samples consecutive frames of TCH/FS information where a frame is assumed to be received successfully if there is no bad frame indication. The number of frame erasure events is recorded.  During the Max-samples of the frame erasure measurement, the RBER of the class II and class Ib bits are also determined.

f)	The SS commands the MS to open the TCH loop (ref III.1.2.1.1.1)

g)	The SS sends the STATUS message . Due to the co-channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

h)	Steps c) to e) shall be repeated except that in step d), both the wanted and interfering signal shall be hopping and the SS shall command the MS into hopping mode with a hop pattern covering at least 10 frequencies in a range not exceeding 5 MHz. The hopping band shall be centred around an ARFCN in the range 60 to 65, according to the narrow band hop of GC1 of Annex 1.

II.4.4.3	Limits

The number of events recorded in this test shall not exceed the Max-events values given in the table below when the Number of samples = Max-samples.

──────────────────────────────────────────────────────────────────────

Channel           Propagation    Type of      Max-events   Max-samples

conditions    measurement

──────────────────────────────────────────────────────────────────────

FACCH             TU3/No FH         FER          6000          25000

TCH/FS            TU3/No FH         FER          6000*Ó        25000

TCH/FS Class Ib   TU3/No FH        RBER         69000/Ó      3300000

TCH/FS Class II   TU3/No FH        RBER         86000        2000000

TCH/FS            TU3/FH            FER           600*Ó        17800

TCH/FS            TU3/FH           RBER          4300/Ó      2000000

Bits class Ib

TCH/FS            TU3/FH           RBER        100000        1200000

Bits class II

TCH/F9.6 or H4.8  TU3/FH           RBER           600         178500

TCH/F4.8          TU3/FH           RBER           600        5350000

TCH/F2.4          TU3/FH           RBER           150       11900000

TCH/H2.4          TU3/FH           RBER           600        5350000

──────────────────────────────────────────────────────────────────────

The parameter Ó can range from 1 to 1.6. The value of Ó for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.5	Adjacent channel rejection

ref: GSM 05.05 section 6.3.2

II.4.5.1	Definition

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal in the adjacent channel.

The adjacent channel can be the adjacent in the RF spectrum or in time.  There are therefore two types of adjacent channel selectivity:

1)	Adjacent RF channel selectivity which is specifically tested in this section.

2)	Adjacent Time Slot selectivity, which is implicitly tested in test II.4.2.1.

II.4.5.2	Method of Measurement

For a MS supporting speech step f) is not required.

For a data only MS, steps c) and e) are not required.

a)	A call shall be originated by the SS to the MS, and the MS shall be made to answer the call. For MS's supporting speech a TCH/FS call shall be used, otherwise a data bearer declared in section 2.1.7 of the PIXIT shall be used.

b)	The MS shall be operated in the encrypted mode.

c)	For MS's supporting speech the System Simulator commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

d)	The SS is set to produce a wanted Standard Test Signal C1 and an independent, uncorrelated interfering (unwanted) Standard Test Signal I1 at the same time. The fading characteristic of the wanted and the interfering signal is TU50. The channel frequency shall be in the range 60 to 65 (ARFCN).

The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28 dBmicroVoltemf(  ).

The unwanted signal shall have no fixed relationship with the bit transition of the wanted signal and it shall be modulated with random data. Its amplitude shall be 9 dB above that of the wanted signal. The unwanted signal shall be continuous, with a nominal frequency 200 kHz above the nominal frequency of the wanted signal.

e)	For MS's supporting speech the SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure compliance for the TCH/FS by examining Max-samples of consecutive frames. The number of frame erasure events is recorded.

The SS determines the number of residual bit error events for bits of class Ib and class II by examining sequences of Max-Samples of consecutive bits of class Ib and class II. Bits are only taken from those frames for which no bad frame indication was given.

f)	The SS sends the STATUS message. Due to the adjacent channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

g)	The measurement of steps d) and e) or f), whichever is applicable, shall be repeated with the unwanted signal on a frequency at the same displacement from, but below, the frequency of the wanted signal.

h)	The measurement of steps d) to g) shall be repeated for a displacement of the unwanted signal of 400 kHz, and with the amplitude of the unwanted signal 41 dB above the level of the wanted input signal, and the unwanted signal static.

i)	The Mobile Station is placed in a climatic test chamber (for extreme test conditions see Annex 1, part TC), and steps d) to h) are repeated for the following combinations of temperatures and power supply voltages:

Temp.	:	Hi	Hi	Lo	Lo

Volt.	:	Hi	Lo	Lo	Hi

II.4.5.3	Limits

────────────────────────────────────────────────────────────────────────

Interfer-	Channel	Type of measurement	Max-events	Max-samples

ence at

────────────────────────────────────────────────────────────────────────

200 kHz	TCH/FS		 FER			   600*Ó	     8900

class Ib		RBER			  4200/Ó	  1000000

class II		RBER			 50000	   600000

FACCH		 FER			   600	     4100

────────────────────────────────────────────────────────────────────────

400 kHz	TCH/FS		 FER			  1020*Ó	     8900

class Ib		RBER			  7560/Ó	  1000000

class II		RBER			 55000	   600000

FACCH		 FER			   600	     5350

────────────────────────────────────────────────────────────────────────

The number of events recorded in this test shall not exceed the Max-events given in the table when the Number of samples = Max-samples.  This shall apply for any combination of normal and extreme test voltages and ambient temperature, and with the interfering signals at either side of the wanted frequency.

The parameter Ó can range from 1 to 1.6. The value of Ó for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

Note:	A static unwanted signal is used to avoid a potential problem with the noise floor of the fading simulator.

II.4.6	Intermodulation rejection

ref: GSM 05.05 section 5.2

II.4.6.1	Definition

The intermodulation rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency.

II.4.6.2	Method of measurement

NOTE:	The measurements address the third order intermodulation, which represents the most serious case.

The compliance with the requirements for the bit error ratio of TCH/FS class II bits and FER of FACCH is checked in order to verify the receiver's intermodulation rejection under normal and extreme test conditions.

For a MS supporting speech, step g2) is not required (FACCH not tested).

For a data only MS, steps e) and g1) are not required.

a)	A call shall be originated by the SS to the MS and the MS shall be made to answer the call. The call shall be on a TCH in the range

60 to 65 (ARFCN).

b)	The MS shall be operated in the encrypted mode.

c)	The MS shall be operated under normal test conditions.

d)	The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver. The level of the wanted signal shall correspond to 4 dB above the reference sensitivity level (see Table II.4.6-1).

e)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the frame erasure event to the SS.

f)	For a speech MS the SS is set to produce a static wanted signal, and two static interfering (unwanted) signals at the same time. For a non speech MS the SS is set to produce a TU50 wanted signal, and two static interfering (unwanted) signals at the same time. There shall be no correlation in the modulation between the signals.

The first interfering signal shall be on a frequency equal to the centre frequency of an ARFCN four above that of the receiver. This signal shall be static and shall be unmodulated.

The second interfering signal shall be on an ARFCN eight above that of the receiver. This signal shall be static, continuous and shall be modulated by random data.

The amplitude of both the interfering signals shall be set according to Table II.4.6-1.

g1)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance of class II bits by examining Max-samples of consecutive bits, with bits only taken from those frames which do not signal frame erasure. The number of error events are recorded.

g2)	The SS sends the STATUS message. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

h)	The measurement of step g1) or g2), whichever is applicable, shall be repeated with the two unwanted signals having frequencies corresponding to ARFCN four and eight below the ARFCN of the unwanted signal.

i)	Steps a) to h), as applicable, shall be repeated but with the receiver operating on an ARFCN in the range 1 to 5.

j)	Steps a) to h), as applicable, shall be repeated but with the receiver operating on an ARFCN in the range 120 to 124.

k)	The Mobile Station is placed in a climatic test chamber (for conditions see Annex 1, part TC), and steps a) to j) are repeated for the following combinations of temperature and power supply voltage:

Temp. :	Hi	Lo	Hi	Lo

Volt. :	Hi	Lo	Lo	Hi

┌────────────┬─────────────┬─────────────┬──────────────┬──────────────┐

│  MS CLASS  │ MS HANDHELD │   Wanted    │  First       │  Second      │

│            │             │   signal    │  interferer  │  interferer  │

│            │             │ dBµVemf(  ) │  dBµVemf(  ) │  dBµVemf(  ) │

├────────────┼─────────────┼─────────────┼──────────────┼──────────────┤

│  1,2       │     NO      │     13      │     74       │     63       │

│  3,4,5     │     YES     │     15      │     64       │     63       │

└────────────┴─────────────┴─────────────┴──────────────┴──────────────┘

Table II.4.6-1: Intermodulation test signal levels

Note:	The levels in Table II.4.6-1 are different to those specified  in GSM 05.05 due to the consideration of the effect of modulation sideband noise from the second interferer.

II.4.6.3	Limits

The Number of error events recorded in this test shall not exceed the

Max-events values given below when the Number of samples = Max-samples.

This shall apply under normal condition and under any combination of normal and extreme test voltages and ambient temperature, and with the two interfering signals at either side of the wanted frequency.

─────────────────────────────────────────────────────────────────

Channel		Type of		Max-events	Max-samples

measurement

─────────────────────────────────────────────────────────────────

TCH/FS

Class II Static		RBER			   200	   8200

FACCH			FER			   600	   4100

─────────────────────────────────────────────────────────────────

II.4.7	Blocking and Spurious Response

ref: GSM 05.05 section 5.1

II.4.7.1	Definition

Blocking is a measure of the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation.

The method of test aligns with ETS 300 086.

For MS not supporting speech, step e1) and j1) are not required. For all other MS, step e2) and j2) are not required.

II.4.7.2	Method of measurement

a)	The MS is connected to the SS. The MS is operated under normal test conditions.

b)	A call shall be originated by the SS to the MS, and the MS shall be made to answer the call. The call shall occupy a full rate channel and shall be on one of the radio frequencies in the range 60 to 65 ARFCN.

c)	The MS shall be operated in the encrypted mode.

d)	The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver.

e1)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

e2)	The SS sends the STATUS message. Due to interfering signals, the MS may not be able to acknowledge the Layer 2 frame. Frame erasures are indicated by repeated L2 frames.

f)	For a speech MS the SS is set to produce a static wanted signal and a static interfering signal at the same time. For a non-speech MS the SS is set to produce a TU50 wanted signal and a static interfering signal at the same time. The level of the wanted signal shall be 15 dBmicroVoltemf for hand portable MSs or 13 dBmicroVoltemf for all other MSs (this corresponds to 4 dB above the reference sensitivity level.

g)	The unwanted signal shall be a C.W. signal (Standard test signal I0) of frequency FB. It shall in turn be applied on the subset of frequencies calculated at step h) in the overall range 100 kHz to 12.75 GHz, where FB is an integer multiple of 200 kHz.

Frequencies in the range FR +/- 600 kHz shall, however, be excluded.

Note:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at subharmonic frequencies nFB where n = 2, 3, 4, 5 etc.

h)	The frequencies at which the test shall be performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) below :-

i)	The total frequency range formed by :-

the frequencies between Flo + (IF1 + IF2 + ... +IFn +12.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12.5 MHz).

and

the frequencies + 100 MHz and - 100 MHz from the edge of the relevant receive band.

Measurement shall be made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies mFlo + IF1, mFlo - IF1, where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12.75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

iv)	Outside the range defined in i) above, spot frequencies shall be measured at 10 MHz from the range edge and repeated at 10 MHz intervals from 100 kHz to 12.75 GHz.

Where :-

Flo - local oscillator applied to first receiver mixer

IF1 ... IFn - are the n intermediate frequencies

Flo, IF1, IF2 ... IFn shall be declared by the manufacturer in the PIXIT statement GSM 11.10 Annex 3.

i)	The level of the unwanted signal shall be set according to the following table.

──────────────────────────────────────────────────────────────

FREQUENCY                     LEVEL FOR MSs

WITH POWER CLASSES

3, 4, 5      1, 2

──────────────────────────────────────────────────────────────

FR +/- 600 kHz to FR +/- 800 kHz          70 dBµV     75 dBµV

FR +/- 800 kHz to FR +/- 1.6 MHz          70 dBµV     80 dBµV

FR +/- 1.6 MHz to FR +/- 3 MHz            80 dBµV     90 dBµV

915 MHz to FR - 3 MHz                     90 dBµV     90 dBµV

FR + 3 MHz to 980 MHz                     90 dBµV     90 dBµV

835 MHz  to  915 MHz                    113 dBµV    113 dBµV

980 MHz  to 1000 MHz                    113 dBµV    113 dBµV

100 kHz  to  835 MHz                     90 dBµV     90 dBµV

1000 MHz  to  12.75 GHz                   90 dBµV     90 dBµV

──────────────────────────────────────────────────────────────

Note:	These values differ from GSM 05.05 because of practical signal generator limits in the SS.

j1)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance for the bits of class II, by examining sequences of Max-samples consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.  The number of error events is recorded.

If a failure is indicated it shall be noted and counted towards the allowed exemption totals.

In the case of failures discovered at the predicted frequencies at steps h ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

j2)	The SS determines the number of frame erasure events during Max-samples. If a failure is indicated, it should be noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps h ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

k)	Steps g) to j1) or j2), whichever is applicable, shall be repeated, for the MS operating on an ARFCN in the range 1 to 5.

l)	Steps g) to j1) or j2), whichever is applicable, shall be repeated, for the MS operating on an ARFCN in the range 120 to 124.

II.4.7.3	Reserved

II.4.7.4	Limits

The number of error events recorded in this test shall not exceed the

Max-events values given below when the Number of Samples = Max-samples.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

─────────────────────────────────────────────────────────────────

Channel			Type of		Max-events	Max-samples

measurement

─────────────────────────────────────────────────────────────────

non speech MS

FACCH				FER			600		4100

─────────────────────────────────────────────────────────────────

other MS

TCH/FS

Class II			RBER			200		8200

─────────────────────────────────────────────────────────────────

A maximum of six failures are allowed in the range FR +/- 45 MHz, with the exclusion of FR +/- 800 kHz (which, if grouped, shall not exceed three

200 kHz channels per group).

A maximum of 24 failures are allowed in the range 100 kHz to 12.75 GHz for the equipment with antenna connector outside FR +/- 45 MHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

If the number of failures do not exceed the maximum allowed figures stated before, at those frequencies at which the failures occur the test stated in II.4.7.2 (or II.4.7.3) shall be repeated, but with a level of the unwanted signal set at 70 dBµV emf. The performance requirement is once again that stated above, that is:

(non speech MS)

Max-events		-	600		Max-samples	-	4100

(other MS)

Max-events 	-	200		Max-samples	-	8200

The number of Error Events recorded in this test shall not exceed the

Max-events values given above, when the number of samples = Max-samples.

No failures are allowed at this lower unwanted signal level.