Aspect II:	NETWORK  INTERFACING

II.1	General  tests  and  procedures

II.1.1	introduction to verifICATION of support for services

Purpose of the verification

The purpose of the verification is threefold:

1)	To verify which services are implemented in the MS under test, as declared by the manufacturer in the Protocol Implementation Information according to Annex 3.

2)	To verify that the MS, when it receives a mobile terminating call, reacts correctly according to the principles of the single and/or multi numbering scheme as well as for ISDN originated calls.

3)	To verify that the MS, when it receives an incompatible mobile terminating call, reacts correctly according to the principles of the multi numbering scheme as well as for ISDN originated calls.

Method of verification

ref: GSM 04.08 Annex B.3.2; GSM 07.01; GSM 09.07

All the possible configuration set-ups of an MS need to be tested with regard to their support for any of the GSM bearer services and teleservices defined in GSM 02.02 and 02.03.  These services are reflected during the signalling phase within the GSM PLMN in the GSM Bearer Capability Information Element (BC-IE), LLC and HLC, if applicable.  The possible settings of the parameters in the GSM BC-IE, LLC and HLC are shown in GSM 07.01, Annex II.

In the cases of a multi numbering scheme as well as for ISDN originating calls, the SETUP message sent to the MS carries a valid GSM BC-IE (plus the ISDN LLC/HLC, if applicable) of the service under verification, according to Annex II of GSM 07.01.  If the MS does not support the offered service it shall

-	ignore the call or

-	reject the call by responding with a RELEASE COMPLETE message with cause "incompatible destination" or

-	apply the re-negotiation option.

Note:	covering the conflicting statements currently in GSM 04.08: Annex B.3.2 allows "ignoring", whereas section 5.2.2.2 requests a RELEASE COMPLETE message. However when a call has to be rejected the preferred behaviour is the sending of the RELEASE COMPLETE message.

In case of the single numbering scheme (e.g. PSTN), where for a mobile terminating call no BC-IE is contained in the SETUP message, the expected (pre-set) 3.1 kHz audio ex PLMN related service shall be indicated in the CALL CONFIRMED message by the MS.

For the services which the MS accepts/indicates, the support is deemed to be implemented in the MS.  The support for these services shall function correctly and is tested specifically, see part I, I.2.

Configuration set-up and setting of service parameters

An MS is constructed to support a finite number of services.  For the support of a particular set of  services, a specific configuration set-up of the MS may be required.  An example of this would be to create an MS configuration set-up in which one interface at the R reference point is to be dedicated to handling automatic fax group 3 only.  In case of single numbering scheme an expected 3.1kHz audio ex PLMN related service is to be pre-set.

The MS may support at its R reference point  various  DTE/DCE interfaces and Procedures, such as:

-	V.21     DTE/DCE interface

-	V.22     DTE DCE interface

-	V.22 bis DTE DCE interface

-	V.23     DTE DCE interface

-	V.26 ter DTE DCE interface

-	V.32     DTE DCE interface

-	X.21     Procedure

-	X.21 bis Procedure

-	V.25 bis Procedure.

However, the setting of the value of the parameter Modem Type in the BC-IE depends exclusively on the actually required type of modem - bound to the requested user data rate signified in the BC-IE - for the individual connection.

In the case of data services, the setting of the parameter values of the BC-IE as outlined in GSM 07.01, Annex II, falls into 3 categories:

-	common to all data and telematic services

-	variable according to the individual service and

non re-negotiable

-	variable, partly related to an individual service,

but re-negotiable.

The setting of the variable values is described as a single path through a set of graphs established for each group of services and type of Information Transfer Capability (ITC)  (see relationship diagrams in GSM 07.01 Annex II, which indicate the individual graph pertinent to the service group, ITC and condition concerned).

MS responses

CALL CONFIRMED indicates that the MS accepts the incoming call. The configuration of the MS is either compatible to the BC-IE, LLC and HLC, if applicable or will enter the type of service signified by the BC-IE.  The MS may also apply the re-negotiation option.

RELEASE COMPLETE (with a cause field) indicates that the MS rejects the incoming call.

No response message from the MS arriving within 15 s at the SS, after having sent the SETUP message, shall be interpreted as if the MS ignores the incoming call but it should be noted that in such a case the preferred behaviour is the sending of the RELEASE COMPLETE message.

II.1.2.1	Verification of support and non-support of services.

(Multiple numbering scheme or ISDN)

Purpose of the test:

Ensure that for incoming calls the MS will react in accordance with the manufacturer's declaration for support and non-support of services.

Initial conditions:

For an MS with an external interface the interface shall be setup in such a way that the MS will be able to successfully receive the call for the service in question. The manufacturer must state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted.

The generic call set-up procedure shall be implemented in accordance with sections II.1.3 or II.1.4 as applicable up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

Procedure:

SS: transmit SETUP

Note:	The SETUP shall contain a single BC-IE with either speech or one of the possibilities defined in 07.01 Annex I. If applicable to the tested Bearer Service/Teleservice the SETUP contains LLC-IE and HLC-IE.

The test must be performed with a speech BC and with all the possible Bearer Service/Teleservice described in 07.01 Annex II. For negotiable parameters only one value has to be used, and that value must be supported by the MS. Negotiation is not checked here.

Result checking:

Not applicable

Requirement:

One of the following requirements must be met:

1)	Expect a RELEASE COMPLETE message with cause value 88 - incompatible destination. The MS is deemed not to support the service.

2)	Expect no response within 15 seconds. The MS is deemed not to support the service.

3)	Expect a CALL CONFIRMED message. The MS is deemed to support the service.

II.1.2.2	Verification of support of the single numbering scheme

Purpose of Test

Ensure that the MS will send a valid BC in the CALL CONFIRMED message as a response to an incoming call with no BC element.

Initial conditions

If possible, the MS shall be configured to respond with a specified preferred BC, in the CALL CONFIRMED message, in reply to a SETUP message with no BC element. The manufacturer must state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted. If the MS can not be configured the test is performed with the MS as supplied.

The generic call set-up procedure shall be implemented in accordance with sections II.1.3 or II.1.4 as applicable up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

Procedure

SS:	transmit SETUP

Note:	The SETUP does not contain a BC element.

The test is repeated with the MS configured for all possible preferred Bearer Capabilities, as declared in the PIXIT statement.

Negotiation of parameters is not checked in this test.

Result Checking

Expect a CALL CONFIRMED message

Requirement

The CALL CONFIRMED message shall contain the BC for which the MS is configured, if applicable. Otherwise the BC shall be among those supported by the MS and as declared by the manufacturer.

Where two BCs are contained in the CALL CONFIRMED message, it shall be checked that the combination is allowed, according to GSM 07.01, and that a REPEAT INDICATOR is also included.

II.1.3	Generic call set-up procedures for speech calls

In the test procedures described in this specification, unless otherwise stated in the test description, the call set-up procedure shall be as follows:

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

a)	Mobile terminating call

- The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

- The special Test-SIM (see section III.1.6) shall be inserted and a MS terminating call shall be established following the steps described in section II.5.3.9.5 "MS terminating call establishment, late assignment" using the parameters of section II.5.3.9.4.2 with the following exceptions:

- Ideal radio conditions (see Annex 1, GC3).

- Timing advance set to 0.

b)	Mobile originating call

- The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

- The special Test-SIM (see section III.1.6) shall be inserted and a MS originating call shall be established following the steps described in section II.5.3.9.3 "MS originating call, late assignment" using the parameters of section II.5.3.9.2.2 with the following exceptions:

- Ideal radio conditions (see Annex 1, GC3).

- Timing advance set to 0.

II.1.4	Generic Call Set-up/Release procedures for data calls

II.1.4.1	Description of data call set-up/release procedures

In the test procedures, described in this specification, unless otherwise stated in the test description, the call set-up/release procedure shall be as follows:

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

a)	The MS shall be operated under normal test conditions (see Annex 1, TC.2.1).

The MS shall be operated under ideal radio conditions (see Annex 1, GC3) and the timing advance shall be set to zero.

b1)	Mobile terminating call (MTC):

The special Test-SIM (see section III.1.6) shall be inserted and a call shall be established following the steps described in section II.5.3.9.4 "MS terminating call, early assignment" using the parameters and method of section II.5.3.9.4.2.

The requirements of section II.5.3.9.4.3 are used.

b2)	Mobile originating call (MOC):

The special Test-SIM (see section III.1.6) shall be inserted and a call shall be established following the steps described in section II.5.3.9.3 "MS originating call establishment, late assignment " using the parameters and method of section II.5.3.9.3.2 with the following exceptions:

-	In section II.5.3.9.3.2, step b) shall be skipped.

Note:	This means that the correct behaviour of the MS is tested in the case where the ALERTING message is not sent. The test for the case where it is sent is performed in II.5.3.9.3.2. Hence both cases are covered.

-	In section II.5.3.9.3.2 step d), the words 'after message ALERTING 'shall be replaced by 'after message CALL PROCEEDING'.

II.1.4.2	State Diagram Model for GSM data services testing

General

The diagrams in Figures II.1.4-1 and II.1.4-2 are intended as a simplified model to aid in the description of the tests for GSM data services.  It is not intended to replace in any way the state diagrams in other GSM documents defining the various machine states permissible.  The definition of these simplified (compound in some cases) states are defined related to the figures.

Note:	In step k), of section II.5.3.9.4 "MS terminating call, early assignment", the conversation state shall be replaced by the Data Transfer Phase, see Figures II.1.4-1 and II.1.4-2.

Definition of states in Figure II.1.4-1

IDLE	The Mobile Termination (MT) does not have a dedicated channel established to a Base Station and will, in general, be listening to the downlink BCCH.

RR Set-Up	The MT has completed the process of establishing the initial Radio Resource, i.e. a dedicated control channel to the network, thus enabling the set-up of the Mobility Management (MM) entity.

MM Set-Up	The MT has completed the process of establishing the Mobility Management (MM) entity, thus enabling the set-up of the Call Control (CC) entity.

CC Set-Up	The MT has completed the process of establishing the Call Control (CC) entity, thus enabling the establishment of the Data Transfer Phase.

For expansion of this (compound) state see Figure II.1.4-2.

DATA TRANSFER PHASE (T DATA)

This state describes the situation where the MT is transmitting and receiving data to and from the far end user. This state encompasses data services requiring only transparent bearer capabilities.

DATA TRANSFER PHASE (NT DATA)

This state describes the situation where the MT is transmitting and receiving data to and from the far end user. This state encompasses data services requiring only non-transparent bearer capabilities.

DATA TRANSFER PHASE (VOICE)

This state describes the situation where the MT is transmitting and receiving encoded GSM voice data to and from the far end user.

CC Release	The MT has completed the process of releasing the Call Control (CC) entity.

MM Release	The MT has completed the process of releasing the Mobility Management (MM) entity.

RR Release	The MT has completed the process of releasing the Radio Resource.

Definition of states in Figure II.1.4-2

Mobile Originating Call (MOC)

CC procedures perform the Layer 3 functions for a mobile originating call attempt.

Mobile Terminating Call (MTC)

CC procedures perform the Layer 3 functions for a mobile terminating call attempt.

SETUP, MOC	CC has sent the SETUP message.

SETUP, MTC	CC has received the SETUP message.

CALL PROCEEDING

CC of the MT has received the CALL PROCEEDING message, which contains a BC-IE, if the parameter Connection Element needs to be determined.  The radio channel related parameters are still to be determined.

CALL CONFIRMED

CC of the MT has sent the CALL CONFIRMED message, if the MT accepts the call (also when parameter values of the BC-IE are to be re-negotiated).  The radio channel related parameters are still to be determined.

TCH Assignment Pending

The TCH has been assigned.  The radio channel related parameters are now determined.

Pre-Data Transfer Phase

The Synchronization process and the RLP establishment, respectively, took place, consequently the connection can enter the DATA TRANSFER PHASE (T/NT).

FIGURE  II.1.4-1 / GSM 11.10 :

State Diagram Model for Call Set-up / Release for GSM Data Services Testing

FIGURE  II.1.4-2 / GSM 11.10 :

Expansion of Call Control (CC Set-Up) State as seen in Fig. II.1.4-1 above