1	GENERAL

1.1	Scope

These are the technical characteristics and methods of measurement for mobile stations, for the Pan European Digital Mobile Radio System standardised by the ETSI Technical Committee "Special Mobile Group" (SMG).

In these specifications, a GSM Mobile Station can be:

- a vehicle mounted station;

- a transportable station;

- a handheld station;

- a vehicle mounted/transportable station;

- a vehicle mounted/handheld station.

These specifications cover the minimum characteristics considered necessary in order to provide sufficient equipment performance for mobile equipment in the GSM system and to prevent interference to other services or to other users, and to GSM PLMNs.

They do not necessarily include all the characteristics which may be required by a user, nor do they necessarily represent the optimum performance achievable.

They apply to the public land mobile radio telephone service in the GSM system, using constant envelope modulation and operating on radio frequencies in the 900 MHz band with a channel separation of 200 kHz and carrying 8 full rate traffic channels or 16 half rate channels per carrier according to the TDMA principle.

1.2	Introduction

1.2.1	Relation with other GSM recommendations

This specification is part of the GSM-series of recommendations. This specification neither replaces any of the other GSM recommendations , nor is it created to provide full understanding of (parts of) the GSM system. This specification lists the requirements, and provides the methods of testing for use by test houses when testing a GSM Mobile Station for conformance.

For a full description of the GSM system, please refer to all the GSM recommendations which are grouped into 12 different series. A complete list of the GSM recommendations, on which these conformance test specifications are based, is listed in section 1.5.

If it is judged that there is a difference of interpretation between these conformance test specifications, and any other GSM recommendation, then the other GSM recommendation shall prevail.

1.2.2	Terminology on mobile station configurations

According to the GSM definition, a Mobile Station is the complete configuration that is present in a vehicle, and which may take part in the communication. However, this might not be the mobile station as it is offered to a test house for conformance testing. Whilst the definition above includes terminal equipment which is connected to the "mobile termination", such Terminal Equipment (TE) is in general submitted to a separate type approval procedure, on the basis of Rec. T/TE-04-08.

In general, the GSM Mobile Station, as it will be presented to a test house for conformance testing, is the station without all the additional terminal equipment. Such a piece of hardware is also called a Mobile Termination (MT), but in this specification, the expression Mobile Station (MS) is used for any form of the hardware as it is offered to the test house.

┌─────────────┐                         ┌───────────┐

│ Mobile      │ Interface: S, R, etc..  │ Terminal  │

│ Termination ├─────────────────────────┤ Equipment │

└─────────────┘                         └───────────┘

During the tests, the interfaces of the "MT" will be connected to a "System Simulator" (SS), which will also emulate the TE. For some tests, it may be necessary to establish a pre-configured set-up of the MS. As an example: for reception of automatic fax group 3 to a fax machine on the R-interface, the MS needs configuration information about the presence of such a machine on that interface.

As an alternative, the terminal equipment may be physically integrated.

For a more detailed description of MS-configurations, see GSM 02.06.

1.2.3	Applicability of these specifications

These specifications apply to the unit which includes the hardware to establish a connection across the radio interface.

If a Mobile Station is equipped with a connector, to connect terminal equipment on an S or R interface, then testing of the Mobile Station will include testing of appropriate functioning to and from this connector.

These specifications do not apply to terminal equipment which is to be connected to that connector (which constitutes a public interface), even if it is delivered with the MS.

These specifications do not apply to a MS where the user-interface offers 2-wire or 4-wire PSTN type connection.

1.2.3.1	Application to terminal equipment

If an MS is delivered for conformance testing, and it contains physically integrated terminal equipment, then this specification applies to the complete Mobile Station including that terminal equipment.

These specifications also apply to separate terminal equipment, if it is delivered for conformance testing with the Mobile Station, and it should be connected via a non-public type of interface. The MS is then tested as an MT0. In that case, the specific terminal equipment with which the Mobile Station is tested is documented in the test report.

1.2.4	The System Simulator

In order to bring the MS into operation, it should be necessary to provide the MS with some signals, and the MS's output signals should be analysed. Rather than describing the hardware configuration of the measuring arrangement (and the requirements to it) in each individual test, the part thereof which is contained in the System Simulator (SS) is described in a separate specification GSM 11.40. This System Simulator is a mandatory tool for conformance testing of GSM Mobile Stations.

1.3	Definitions

Only a limited set of definitions, with special relevance to the MS and to conformance testing are included. Further definitions may be found in GSM 01.04.

GSM Mobile Station (MS)	ref: GSM 02.06

Equipment intended to access a set of GSM PLMN telecommunication services. Services may be accessed while the equipment capable of surface movement within the GSM system area is in motion or during halts at unspecified points.

Mobile Termination (MT)

The part of the Mobile Station which terminates the radio transmission to and from the network and adapts terminal equipment (TE) capabilities to those of this radio transmission.

Conformity specification	ref: TG 01-01

A document giving a precise and full description of the technical characteristics of the relevant telecommunications terminal equipment (such as safety, technical parameters, functions and procedures and service requirements) together with a precise definition of the tests and test methods enabling the conformity of the equipment with the prescribed technical characteristics to be verified.

Telecommunications terminal equipment	ref: TG 01-01

Equipment directly or indirectly connected to public telecommunications networks or for use with public telecommunications services.

1.4	TERMINOLOGY

1.4.1	Abbreviations and Acronyms

Abbreviation	Full Term	Reference

GSM

AB 	Access Burst	05.02

AC 	Access Class (C0 to C15)	02.11

ACCH	Associated Control Channel

AGCH	Access Grant CHannel	05.02

ARFCN	Absolute Radio Frequency Channel Number

ARQ	Automatic Request for Retransmission

ATT (flag)	Attach

BA	BCCH Allocation	05.08

BCC	BS Colour Code	03.03

BCCH	Broadcast Control Channel	05.02

BCCH_FREQ_NCELL	Frequency of the RF carrier on which the 	05.08

BCCH of a neighbouring cell is transmitted

BCD	Binary Coded Decimal

BER	Bit Error Ratio	05.05

BFI	Bad Frame Indication 	05.05

Bm	full rate traffic channel

BN	Bit Number 	05.02

BS─AG─BLKS─RES	Number of blocks on each common control	05.02

channel reserved for access grant messages

BS─BCCH─SDCCH─C0MB	Logical variable that indicates the	05.02

combination of dedicated and associated

control channels on the same physical channel

BS_PA_MFRMS	Number of multiframes between two 	05.02

transmissions of the same paging message

to MSs of the same paging group

CC	Call Control	04.07

CCH	Control CHannels	05.01

CELL-BAR-ACCESS	Cell Access Barred

CELL_RESELECT_HYSTERESIS

RXLEV Hysteresis required for Cell Reselection

CM	Connection Management	04.07

CMD	Command

COM	Complete

CONN	Connect

CRC	(3 bit) Cyclic Redundancy Check	05.05/11.10

DCCH	Dedicated Control Channel	04.08

DET	Detach

DISC	DISConnect

DRX	Discontinuous Reception (Mechanism)

DTE	Data Terminal Equipment

DTMF	Dual Tone Multi Frequency (signalling) 	04.08

DTX	Discontinuous Transmission (Mechanism)

Abbreviation	Full Term	Reference

GSM

Ec/No	Ratio of energy per modulating bit to

the noise spectral density

EMMI	Electrical Man Machine Interface

FACCH	Fast Associated Control Channel

FACCH/F	Full rate Fast Associated Control Channel

FACCH/H	Half rate Fast Associated Control Channel

FEC	Forward Error Correction

FER	Frame Erasure Ratio	05.05

FH	Frequency Hopping 	05.05

GMSK	Gaussian Minimum Shift Keying (modulation)

HPU	Hand Portable Unit

IMEI	International Mobile station Equipment Identity	03.03

IMSI	International Mobile Subscriber Identity	03.03

L1	Layer 1

L2R	Layer 2 Relay

L3	Layer 3	04.07

LAC	Location Area Code	03.03

LAI	Location Area Identification	03.03

LAP─Dm	Link Access Protocol on the Dm channel

Lm	Traffic channel with capacity lower than Bm

LPLMN	Local PLMN

LTE	Local Terminal Emulator

MCC	Mobile Country Code	03.03

MM	Mobility Management	04.07

MMI	Man Machine Interface

MNC	Mobile Network Code	03.03

MS	GSM Mobile Station	02.06

MS_TXPWR_MAX_CCH	Maximum Allowed Transmitted RF Power for MSs

to Access the System until commanded otherwise

MT	Mobile Termination

NCC	PLMN Colour Code	03.03

PAD	Packet Assembly/Disassembly facility

PAGING_GROUP	The set of MSs monitoring a particular 	05.02

paging block

PCH	Paging CHannel	05.02

PIN	Personal Identification Number

PLMN	Public Land Mobile Network

PLMN_PERMITTED	PLMN Permitted for handover purposes	05.08

PSPDN	Packet Switched Public Data Network

PSTN	Public Switched Telephone Network

R	Value of Reduction of the MS Transmitted RF

Power relative to the maximum allowed output

power of the highest power class of MS (A)

Abbreviation	Full Term	Reference

GSM

RA	RAndom mode request information field

RACH	Random Access CHannel	05.02

RADIO─LINK─TIME0UT	The timeout period for radio link failure. 	05.08

Maximum value of the radio link timer.

RADIO_LINK_TIMER	Parameter which is incremented or decremented 	05.08

according to the success with which SACCH

messages are decoded

RAND	RANDom Number (authentication)

RBER	Residual Bit Error Ratio	05.05

REL	RELease

REQ	REQuest

RLP	Radio Link Protocol

RMS	Root Mean Square (value)	05.05

RTE	Remote Terminal Emulator

RXLEV	Received Signal Level	05.08

RXLEV_ACCESS_MIN	The minimum received signal level at a MS for 	05.08

access to a cell

RXLEV_MIN	The minimum received signal level at a MS from a

neighbouring cell for handover to be permitted

RXLEV_NCELL	Received signal level of neighbouring or 	05.08

current serving cell measured on the BCCH carrier

RXLEV_SERVING_CELL	Received signal level in the serving cell 	05.08

measured on the BCCH carrier

RXQUAL	Received Signal Quality	05.08

RXQUAL_FULL	Received signal quality assessed over the 	05.08

full set of TDMA frames within a SACCH block

RXQUAL_SERVING_CELL	Received signal quality of serving cell	05.08

RXQUAL_SUB	Received signal quality assessed over a 	05.08

subset of 12 TDMA frames

SABM	Set Asynchronous Balanced Mode

SACCH	Slow Associated Control CHannel

SACCH/C4	Slow, SDCCH/4 Associated, Control CHannel

SACCH/C8	Slow, SDCCH/8 Associated, Control CHannel

SACCH/T	Slow, TCH─Associated, Control CHannel

SACCH/TF	Slow, TCH/F─Associated, Control CHannel

SACCH/TH	Slow, TCH/H─associated, Control CHannel

SAPI	Service Access Point Identifier	04.05

SDCCH	Stand-alone Dedicated Control CHannel	05.02

SDCCH/4	Stand─alone Dedicated Control CHannel/4

SDCCH/8	Stand─alone Dedicated Control CHannel/8

SID	Silence Descriptor	05.02

Abbreviation	Full Term	Reference

GSM

SIM	Subscriber Identity Module

SMSCB	Short Message Service Cell Broadcast	05.02

SRES	Signed RESponse (authentication)

SS	System Simulator	11.40

TA	Terminal Adapter

TAC	Type Approval Code

TCH	Traffic CHannel	05.02

TCH/F	Full rate Traffic CHannel	05.02

TCH/FS	Full rate Traffic CHannel for Speech	05.02

TCH/F2.4	Full rate TCH for <=2.4kbit/s user data	05.02

TCH/F4.8	Full rate TCH for   4.8kbit/s user data	05.02

TCH/F9.6	Full rate TCH for   9.6kbit/s user data	05.02

TCH/H	Half rate Traffic CHannel	05.02

TCH/HS	Half rate Traffic CHannel for Speech	05.02

TCH/H2.4	Half rate TCH for <=2.4kbit/s user data	05.02

TCH/H4.8	Half rate TCH for   4.8kbit/s user data	05.02

TE	Terminal Equipment

Tei	Terminal endpoint identifier

TI	Transaction Identifier

TMSI	Temporary Mobile Subscriber Identity

TN	Timeslot Number

TXPWR	Transmit power: Tx power level in the

MS_TXPWR_REQUEST and MS_TXPWR_CONF parameters

UI	Unnumbered Information (Frame)

VAD	Voice Activity Detection	05.03/05.04/

06.32

V(SD)	SenD state Variable	04.08

1.4.2	Conventions for mathematical notations

Some mathematical terms cannot easily be expressed in ASCII characters. The exceptions used throughout this specification are shown below.

Mathematical signs

The "plus or minus" sign is expressed by "+/-".

The sign "multiplied by" is expressed by "*".

The sign "divided by" is expressed by "/", or the common division bar.

The sign "greater or equal to" is expressed by ">=".

The sign "smaller or equal to" is expressed by "<=".

Roots are expressed by potentials.

Powers to the base 10

Powers to the base 10 are expressed by "10Ex", where x is the potential figure, e.g. 10E-5, 10E6.

1.4.3	Conventions on electrical terms

RF input signal level

In general, the RF input signal level to the MS is expressed in terms of the received field strength E in dBµVemf (assuming a 0 dBi gain antenna). This is related to the power level P in dBm by the following formula

(ref. GSM 05.05):

E (dBµV/m) = P (dBm) + 136.5   (valid for a frequency of 925 MHz).

According to section II.4.2.2.3, in all tests in which a handheld MS normally only equipped with an integral antenna is the unit under test, the equivalent input signal level into a temporary test connector is determined from :

Ein	=	Ereq + F

where:

Ein   = input signal level to a temporary antenna connector (dBµVemf);

Ereq  = signal level required by the test (dBµVemf);

F     = coupling factor (dB) at the respective ARFCN.

Since F has to be determined by each test house individually, Ein cannot be given as a figure in test procedures. If the case of integral antenna is applicable, the input signal level, therefore, is expressed in the test procedures as:

Ereq dBµVemf(   ),

where the empty parenthesis is to be read as Ein.

1.4.4	Terms on test conditions

Radio test conditions

The radio propagation conditions refer to multipath propagation models of GSM 05.05.

They are expressed by typical profiles:

static

rural area               (RA),

hilly terrain            (HT),

urban area               (TU),

or for equalisation test (EQ).

The non-static profiles are also related to typical speeds of movement of the MS expressed in km/h, e.g. TU3, TU50, HT100, EQ50.

The "ideal radio conditions" for this test specification are defined in Annex 1, Part GC General Conditions, GC3.

Environmental test conditions

The following terms are used with their meaning shown for indication of environmental conditions in this specification (ref. Annex 1, Part TC).

Term:			Meaning:

E.T.C.		extreme test conditions

Hi			high

Lo			low

N.T.C.		normal test conditions

Temp			temperature

Volt			voltage

1.5	List of the GSM Specifications, on which this test specification is based

Number	Version	Title

GSM

02.02	 3.2.0	Bearer Services Supported by a GSM PLMN

02.03	 3.4.0	Teleservices Supported by a GSM PLMN

02.04	 3.7.1	Description of Supplementary Services

02.06	 3.2.0	Types of Mobile Stations

02.07	 3.4.1	Mobile Station Features

02.09	 3.1.0	Security Aspects

02.11	 3.7.0	Service Accessibility

02.16	 3.0.1	International MS Equipment Identities

02.17	 3.2.0	Subscriber Identity Modules, Functional Characteristics

02.30	 3.9.0	Man-machine Interface of the Mobile Station

02.40	 3.2.0	Procedures for Call Progress Indications

02.82	 3.6.1	Call Offering Supplementary Services

02.88	 3.6.1	Call Restriction Supplementary Services

03.03	 3.5.0	Numbering, Addressing and Identification

03.05	 3.2.0	Technical performance objectives

03.10	 3.3.0	GSM PLMN Connection Types

03.13	 3.0.2	Discontinuous Reception (DRX) in the GSM System

03.14	 3.0.2	Support of DTMF via the GSM System

03.20	 3.3.2	Security-related Network Functions

03.40	 3.7.0	Technical Realization Short Message Service Point-to-point

03.41	 3.4.0	Technical Realization of Short Message Service Cell Broadcast

03.43	 3.0.1	Technical Realization of Videotex

03.44	 3.0.1	Support of Teletex in a GSM PLMN

03.45	 3.3.0	Technical Realization of Facsimile Group 3 Service - transparent

03.46	 3.2.1	Technical Realization of Facsimile Group 3 Service - non transparent

03.50	 3.2.2	Transmission Planning Aspects of the Speech Service in the GSM PLMN System

04.01	 3.0.1	MS-BSS Interface - General Aspects and Principles

04.02	 3.0.2	GSM PLMN Access Reference Configuration

04.03	 3.0.3	MS-BSS Interface : Channel Structures and Access Capabilities

04.04	 3.3.4	MS-BSS Layer 1 - General Requirements

04.05	 3.1.5	MS-BSS Data Link Layer - General Aspects

04.06	 3.9.0	MS-BSS Data Link Layer Specification

04.07	 3.3.3	Mobile Radio Interface Signalling Layer 3 - General Aspects

04.08	3.13.0	Mobile Radio Interface - Layer 3 Specification

04.10	 3.2.3	Mobile Radio Interface  Layer 3 - Supplementary Services Specification - General Aspects

04.11	 3.3.0	Point-to-point Short Message Service Support on Mobile Radio Interface

04.12	 3.2.1	Cell Broadcast Short Message Service Support on Mobile Radio Interface

04.21	 3.4.0	Rate Adaptation on MS-BSS Interface

04.22	 3.7.0	Radio Link Protocol for Data and Telematic Services on the MS-BSS Interface

04.80	 3.2.0	Mobile Radio Interface Layer 3 - SS Specification - Formats and Coding

04.82	 3.1.3	Mobile Radio Interface Layer 3 - Call Offering SS Specification

04.88	 3.1.3	Mobile Radio Interface Layer 3 - Call Restriction SS Specification

05.01	 3.3.2	Physical Layer on the Radio Path (General Description)

05.02	 3.6.1	Multiplexing and Multiple Access on the Radio Path

05.03	 3.5.1	Channel Coding

05.04	 3.1.2	Modulation

05.05	3.15.0	Radio Transmission and Reception

05.08	 3.7.0	Radio Subsystem Link Control

05.10	 3.5.1	Radio Subsystem Synchronization

06.01	 3.0.0	Speech Processing Functions : General Description

06.10	 3.2.0	GSM Full Rate Speech Transcoding

06.11	 3.0.1	Substitution and Muting of Lost Frames for Full-rate Speech Traffic Channels

06.12	 3.0.1	Comfort Noise Aspects for Full Rate Speech Traffic Channels

06.31	 3.1.0	Discontinuous Transmission (DTX) for Full Rate Speech Traffic Channels

06.32	 3.0.0	Voice Activity Detection

07.01	3.14.0	General on Terminal Adaptation Functions for MSs

07.02	 3.8.0	Terminal Adaptation Functions for Services Using Asynchronous Bearer Capabilities

07.03	 3.4.0	Terminal Adaptation Functions for Services Using Synchronous Bearer Capabilities

09.02	 3.8.0	Mobile Application Part Specification

11.11	3.14.0	Specification of the Internal Logical Organization of the SIM and its Interfaces

11.40	 3.6.0	System Simulator Specification (MS conformance test system)

Aspect I:	FORMAL PROCEDURES AND GENERAL REQUIREMENTS

I.1	Formal procedures

The administrative procedures that govern:

- the accreditation of test houses,

- the issue and use of certificates of conformity,

- the formal approval procedures,

are referenced directly or indirectly, or specified in

*	CEPT Rec. T/R 21-08 relating to type approval procedures and free circulation of GSM Mobile Stations,

*	Council Directive 86/361 on the initial stage of the mutual recognition of type approval for telecommunications terminal equipment.

I.2	Testing and Approval Methodology in general (L1, L2, L3)

I.2.1	Testing of optional functions and procedures

Conformance shall be tested using the test specified in this specification.

Any function or procedure which is optional, as indicated in this specification, shall be subject to a conformance test if it is implemented in the MS.

The means to determine whether an optional function/procedure has been implemented can be by either apparatus supplier's declaration or as a result of performing the conformance tests on the MS under test. In this respect, the test in II.1, where it is verified that a MS refuses towards the network the support of any service that it can not support is of special interest.

Where no declaration is made by the Apparatus Supplier as to the implementation (or not) of an optional function/procedure, and the conformance test reveals that the option is incorrectly (or partially) implemented, the option shall be deemed to have been implemented and the apparatus shall be tested accordingly.

I.2.2	Access

The user - network interface at Um reference point provides the main test access for the purpose of performing conformance tests. The provision of the following special conformance test facilities is mandatory, where applicable:

-	support of special conformance test functions, which are enabled by the insertion of a dedicated Subscriber Identity Module for testing (Test-SIM);

-	provision of a Digital Audio Interface (only for MSs which support speech services, or alternate speech/data services).

-	for equipment which does not have a permanent external 50 ohm antenna connector, a temporary 50 ohm antenna connector shall be provided in accordance with the requirements of Annex 1 GC7.

-	for MS supporting diversity, or for any other reason having more than one RF connector (or temporary connector in the case of an integral antenna MS), the manufacturer shall supply coupling and/or terminating devices so that the tests can be performed via a single transmit/receive RF connection.

Furthermore, provision of an extra special type testing function, the Electrical Man Machine Interface (EMMI), is highly recommended to the manufacturer.

All these special conformance test functions are described in section III.1 of these specifications.

Actions at the user side of the equipment under test (e.g. at the Man-machine Interface, at the S- or R- interface, at the SIM-interface, execution of higher layer processes in the case of data services) shall be used to invoke actions at Layers 1, 2 and 3 of the Dm-channel protocol within the equipment under test.

I.2.3	Different layers

The conformance tests for each layer of the Dm-channel protocol are specified separately and the test configuration(s) to be used in testing each layer is specified in the section of this specification relating to the conformance tests for that layer.

I.2.4	Information to be provided by the Apparatus Supplier

The apparatus supplier shall provide two kinds of information:

-	information with respect to the protocol: Protocol Implementation Conformance Statement (PICS);

-	information with respect to the Man-machine Interface: Protocol Implementation eXtra Information for Testing (PIXIT).

The complete list of the information to be provided by the apparatus supplier is a matter between the apparatus supplier and the test house but an example of the information to be supplied is given for information in Annex 3 of this specification.

I.3	Applicability of the individual chapters

These test specifications contain a number of chapters in which the test procedures are described. The test procedures which are actually carried out on the MS are determined on the basis of the services which are supported via the MS.

A MS is tested only on aspects related to the services for which it provides network access. For all the other services, for which a MS should not deliver support, the MS must produce a rejection, if any attempt to invoke the MS's support for such a service is made.

For a list of bearer and teleservices, supported by GSM PLMNs, see GSM 02.02 and GSM 02.03.

With the application for conformance testing, the applicant indicates which services are supported by the MS. The MS shall be tested in relation with each service, for which it does not refuse support, and it must pass all the corresponding tests.

Since the current GSM Standard does not support half-rate traffic channels, an MS shall not support these channels, and the tests on half-rate traffic channels are not applicable.

All MSs shall undergo the tests in the following sections:

(The order of the tests is recommended.)

II.2	Transceiver

II.3	Transmitter

II.4	Receiver

II.5	Signalling

II.6	Radio-link management

II.8	MS-SIM interface testing

II.1.2.1	Testing of support and non-support of Services.

II.16	MS features

II.12	Supplementary services

II.17	Test of low battery voltage detection

MSs supporting speech teleservices shall in addition undergo the tests in sections:

(The order of the tests is recommended.)

II.11.1	Transmission characteristics

II.13	Speech transcoding functions.

MSs supporting the short message service shall in addition undergo the tests in sections:

II.11.2	Short message service.

I.4	General requirements concerning safety of people and protection of the network and terminal

I.4.1	Interference to car management systems

Note:	It is considered that if other equipment is hardened to the transmitter's wanted output, then the unwanted emissions will not cause further interference.

I.4.2	Requirements concerning environmental conditions for operation

The Mobile Station shall function correctly under ambient temperatures in the range of -20 to +55 degrees Celsius.

There is no special test to verify this.  In some sections of module "Aspect II", tests are being described, which will be carried out under various temperatures within this range.

The Mobile Station shall function correctly, whilst the power supply voltage has any value within a range which is specified in Annex 1, TC2.2, separately for different types of power supply as "extreme test voltages".

There is no special test to verify this.  In some sections of module "Aspect II", tests are being described, which will be carried out under various power supply voltages within this range.