II.5.3.8	Tests related to circuit switched call control

II.5.3.8.1	Circuit Switched Call Control state machine verification

The principle of checking the Call Control functions consists in the validation of each Call Control identified state.

Hence Call Control functions are tested by means of a state table. The state table partly relies upon Fig 5.1a/GSM 04.08.  A square of the table provides:

-	an observable action triggered by a particular event in a defined state,

-	the next state.

This picture describes how to use the state table in the test procedure.

┌──────────────────┐

│ event            │

│                  │<-------- event either generated by the SS

┌──┼──────────────────┤          or automatically within the MS

│U │action            │

│i │                  │<-------- action performed by the MS to be

│  ├──────────────────┤          verified if observable

│  │next state        │<-------- new state to be verified if

└──┴──────────────────┘          observable

^

│

└────────────────────────────── initial state to be verified if

observable

State table element description

A couple event/state can be considered as passed if both the new state and action performed by the MS are right provided they are observable.

State U0 as an initial state is not verified in the tests of II.5.3.8.1.2 (Establishment of an outgoing call).

State U0.1 is never verified.

Each square of the table must be passed through to ensure that all the possibilities of leaving a state to another state are tested.

The steps to be followed within each performed test are:

-  bring the MS into the required state,

-  trigger the tested event,

-  check the MS response and new state.

The pairs event/state which are not mentioned shall result in no observable action or state modification besides the sending of a status message if the event was a message not compatible with the state (cf. GSM 04.08, section 8.5).

The events are either messages sent by the SS or actions performed through the Man Machine Interface (e.g. the depression of a key to send a DTMF).

The following table provides a way of generating the primitives used in this test:

MNCC_SETUP_REQ		MMI action

Request to initiate an outgoing call.

MNCC_START_DTMF_REQ	MMI action

A DTMF key is pressed.

MNCC_STOP_DTMF_REQ	MMI action

A DTMF key is released.

MNCC_ALERT_REQ		MS internally generated when

-  an alerting indication is generated locally at the MS;

-  request to send an ALERTING message to the NW.

MNCC_REJ_REQ		MS internally generated.

The call cannot be accepted by the MS because of

missing compatibility.

MNCC_DISC_REQ		MMI action

hook on

MNCC_REL_REQ		MS internally generated

MMCC_REL_IND		SS action

The SS sends a RELEASE message.

MMCC_EST_CNF		SS action

The SS positively acknowledges the outgoing MM

connection establishment either by establishing the cipher

mode or by sending the message CM SERVICE ACCEPT.

MMCC_ERR_IND		SS action

The SS generates a lower layer failure at Layer 1

or Layer 2 level.

MMCC_SYNC_IND		SS action

The SS assigns a TCH by sending

the message ASSIGNMENT_CMD.

MNCC_CALL_CONF_REQ	MS internally generated

when an incoming SETUP message meets the

MS's bearer capability.

MNCC_MODIFY_REQ	MMI action

MNCC_NOTIFY_REQ

Note:	This list is incomplete.

The states are checked through STATUS_ENQ messages sent by the SS (when feasible both initial and final states are checked).

For the special case of U0, the state is checked by sending STATUS ENQUIRY message with all possible values of Transaction Identifier (seven values) as U0 is the only state in which for every TI the MS will answer with release complete with cause #81.  If U0 is to be verified when no RR connection exists, first a mobile terminating radio connection must be established.

The MS responses are either Call Management messages received by the SS or lower layers functions activated within the MS or MMI actions (e.g. the buzzing of an alerting tone).

A timeout within the MS is triggered by the SS when it does not answer back an MS expected response.

The test sequences may be split in 3 main groups:

-  establishment and release of an outgoing call;

-  establishment and release of an incoming call;

-  in call functions.

Note:	Throughout section II.5.3.8.1, abbreviations for the names of CM messages are used such as "MODIFY_REJ" instead of "MODIFY REJECT". This may cause name conflicts with L2 message names (e.g. "DISC" as an abbreviation for "DISCONNECT"). Unless otherwise indicated, a CM message is meant in such cases.

Remark on verification of transient states:

The following states may be transient, depending on implementation and previous messages:

State U6:

State U6 may be transient if the implementation allows it. Then there is an internal transition:

CALL CONF

U6 ----------------> U9

State U9:

State U9 is not transient if

- the implementation does not support immediate connect and

- an appropriate TCH is not yet assigned and

- the signalling element has not been present in the SETUP.

If the implementation supports immediate connect, there is an internal transition:

CONN

U9 ----------------> U8

If the appropriate TCH is available or the signalling element was present in SETUP, there is an internal transition:

ALERT

U9 ----------------> U7

State U7:

If the implementation allows for automatic connect after an implementation specific time T, then there is an internal transition:

after T, CONN

U7 ----------------> U8

State U12:

U12 is stable if the implementation supports it and progress indicator is present in DISCONNECT and indicates the availability of in-band announcements, otherwise there is an internal transition:

REL

U12 ---------------> U19

A test ending in a transient state (this fact may depend on the implementation) can be executed by verifying that the SS receives the message that accompanies the (next) internal transition (see above).

If a test starts in a transient state, then the test is executed without verification of the starting state.

Note that there are situations of chained transient states, e.g.

CALL CONF       ALERT           CONN

U6 -----------> U9 -----------> U7 -----------> U8

In these cases, still the above described way to execute the tests holds.

II.5.3.8.1.1	Message definition

messages			fields provided

all messages			protocol discriminator

transaction identifier

message type

ALERTING			none

CALL_PROCEED		none

CONNECT			none

CONNECT_ACK			none

DISCONNECT			cause

progress indicator

MODIFY			bearer capability

MODIFY_COM			bearer capability

MODIFY_REJ			cause

bearer capability

NOTIFY				notification indicator

PROGRESS			progress indicator = #8

RELEASE			none

RELEASE_COM			none

SETUP				signal

up to 2 bearer capabilities

progress indicator

START_DTMF			keypad facility

START_DTMF_ACK		keypad facility

START_DTMF_REJ		cause

STOP_DTMF			none

STOP_DTMF_ACK		none

STATUS_ENQ			none

STATUS			cause

call state

Fields provided in the downlink messages

II.5.3.8.1.2	Establishment of an outgoing call

II.5.3.8.1.2.1	Initial conditions

As a minimum requirement the MS is updated and has been given a TMSI, a Ciphering Key and Cipher Key Sequence Number, and the Layer 2, RR and MM functionalities have been verified.

There are as many CM initial conditions as states to be checked.

The tables below describe message exchanges which bring the MS in the requested initial states.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order followed in the test procedure will be U0, U0.1, U1, U3, U4, U10, U12, U19, U11 as seen in the table underneath.

The MS is brought again in the initial state starting with U0 at each new test performed.

TABLE 1

actions			messages exchanged			next

MS SS		state

init outgoing call	CHANN_REQ		---->

IMM_ASSIGN (SDCCH)	   <----		U0

CM_SERV_REQ		---->		U0.1

CIPH_MOD_CMD		   <----

CIPH_MOD_COM		---->

SETUP			---->		U1

CALL_PROC		   <----		U3

ALERT			   <----		U4

ASSIGN (TCH)		   <----

ASSIGN_COM		---->

CONN			   <----

CONN_ACK		---->		U10

DISC			   <----		U12

REL			---->		U19

on hook		(from U10) DISC		---->		U11

TABLE 2

actions			messages exchanged			next

MS SS		state

init outgoing call	CHANN_REQ		---->

IMM_ASSIGN (TCH)	   <----		U0

CM_SERV_REQ		---->		U0.1

CHAN_MOD_MODIFY 1)	   <----

CHAN_MOD_MODIFY_ACK	---->

CIPH_MOD_CMD		   <----

CIPH_MOD_COM		---->

SETUP			---->		U1

CALL_PROC		   <----		U3

ALERT			   <----		U4

CONN			   <----

CONN_ACK		---->		U10

DISC			   <----		U12

REL			---->		U19

on hook		(from U10) DISC		---->		U11

Note 1)	channel mode = speech full rate

TABLE 3

actions			messages exchanged			next

MS SS		state

init outgoing call	CHANN_REQ		---->

IMM_ASSIGN (SDCCH)	   <----		U0

CM_SERV_REQ		---->		U0.1

CIPH_MOD_CMD		   <----

CIPH_MOD_COM		---->

SETUP			---->		U1

AUTH_REQ		   <----

AUTH_RESP		---->

CALL_PROC		   <----		U3

ASSIGN (TCH)		   <----

ASSIGN_COM		---->

ALERT			   <----		U4

CONN			   <----

CONN_ACK		---->		U10

DISC			   <----		U12

REL			---->		U19

on hook		(from U10) DISC		---->		U11

TABLE 4

actions			messages exchanged			next

MS SS		state

init outgoing call	CHANN_REQ		---->

IMM_ASSIGN (TCH)	   <----		U0

CM_SERV_REQ		---->		U0.1

IDENTITY_REQ		   <----

IDENTITY_RESP		---->

CIPH_MOD_CMD		   <----

CIPH_MOD_COM		---->

SETUP			---->		U1

CHAN_MOD_MODIFY 1)	   <----

CHAN_MOD_MODIFY_ACK	---->

CALL_PROC		   <----		U3

ALERT			   <----		U4

CONN			   <----

CONN_ACK		---->		U10

DISC			   <----		U12

REL			---->		U19

on hook		(from U10) DISC		---->		U11

Note 1) channel mode = speech full rate

II.5.3.8.1.2.2	Test procedures

II.5.3.8.1.2.2.1	U0 null state

State U0 is reached using Table 2.

MNCC_SETUP_REQ

2)

U0	MM connection est

1)

U0.1

1)	MS sends CM_SERVICE_REQ, verify the type of call which is asked for "basic" or "emergency" by the MS.

2)	The test is performed for basic call.

II.5.3.8.1.2.2.2	U0.1 MM connection pending

State U0.1 is reached using Table 1.

MNCC_REL_IND		MMNC_EST_CNF

1)		2)

U0.1			send SETUP

3)

U0		U1

1)	SS sends CM_SERVICE_REJ.

2)	SS sends CM_SERVICE_ACC.

3)	With called party BCD number.

State U0.1 is reached using Table 1.

MNCC_ERR_IND

1)

U0.1	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

II.5.3.8.1.2.2.3	U1 call initiated

State U1 is reached using Table 2.

CALL_PROC		REL_COM		timeout T303

received		received	3)		4)

U1	tone generation		MM connection rel		send DISC

1)		2)

U3		U0		U11

1)	Not mandatory, hence not tested.

2)	MS waits for MM layer release initiated by SS.

3)	With a valid cause value among:

related to numbering,

#1  unallocated number

#3  no route to destination

#22 number changed

#28 invalid number format

related to bearer capabilities,

#57 bearer capability not authorized

#58 bearer capability not presently available

#63 service or option not available

#65 bearer service not implemented

#34 no circuit/channel available (call queueing).

4)	Check T303 timer accuracy (T303 +/-20%) between reception of CM SERVICE REQUEST and DISC by SS.

State U1 is reached using Table 4.

MMCC_ERR_IND		ALERTING received	CONN received

1)

U1	MM connection rel		alerting indication		send CONN_ACK

2)

U0		U4		U10

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

2)	Optional, not tested.

State U1 is reached using Table 1.

unknown message

received	1)

U1	send STATUS

2)

U1

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, state U1.

II.5.3.8.1.2.2.4	U3 MS originating call proceeding

State U3 is reached using Table 2.

ALERT received		CONN received		PROGRESS received

3) 4)

U3	tone generation		send CONN_ACK

1)		2)		5) 4)

U4		U10		U3

1)	Not mandatory.

2)	Stop tone generation if any.

3)	Tested with a valid cause value among:

#1  call is not end to end PLMN/ISDN

#2  destination address is not PLMN/ISDN

#4  call has returned to PLMN/ISDN

#8  in band information or appropriate pattern now available.

4)	It is verified that T310 is stopped, ie at T310 timeout no DISC message is sent by the MS.

5)	With progress description value #8 check the availability of in band information at MS side after a TCH has been allocated.

State U3 is reached using Table 2.

DISC received		DISC received		REL received

2)		1)		4)

U3			send REL		send REL_COM

3)			MM connection rel.

U12		U19		U0

1)	Sent without progress indicator.

2)	Sent with progress indicator value #8 in band information available.

3)	This behaviour may happen, otherwise the MS must send a RELEASE and go to state U19.

4)	In case of causes mentioned in GSM 04.08, section 5.4.2.

State U3 is reached using Table 3.

MNCC_DISC_REQ		MMCC_SYNC_IND		timeout T310

(TCH allocation)

U3	send DISC		L2 establishment		send DISC

on FACCH			1)

U11		U3		U11

1)	Check T310 timer accuracy from CALL_PROC emmission to DISC reception by SS.

State U3 is reached using Table 4.

MMCC_ERR_IND

1)

U3	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U3 is reached using Table 1.

unknown message

received	1)

U3	send STATUS

2)

U3

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, and state U3.

II.5.3.8.1.2.2.5	U4 call delivered

State U4 is reached using Table 3.

CONN		MNCC_DISC_REQ

U4	send CONN_ACK		send DISC

1)

U10		U11

1)	Stop alerting indication if applicable.

State U4 is reached using Table 2.

DISC received		DISC received		REL received

2)		1)		4)

U4			send REL		send REL_COM

3)			MM connection rel

U12		U19		U0

1)	Sent without progress indicator.

2)	Sent with progress indicator value #8 in band information available and a TCH channel has been allocated.

3)	This behaviour may happen, otherwise the MS must send a RELEASE and go to state U19.

4)	In case of causes mentioned in GSM 04.08, section 5.4.2.

State U4 is reached using Table 2.

MMCC_ERR_IND

1)

U4	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U4 is reached using Table 3.

MMCC_SYNC_IND

(TCH allocation)

U4	L2 establishment

on FACCH

U4

State U4 is reached using Table 4.

unknown message

received	1)

U4	send STATUS

2)

U4

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, and state U4.

II.5.3.8.1.2.2.6	U10 call active

State U10 is reached using Table 1.

MNCC_DISC_REQ		REL received

1)

U10	send DISC		send REL_COM

MM connection rel

U11		U0

1)	In case of causes mentioned in GSM 04.08, section 5.4.2.

State U10 is reached using Table 2.

DISC received		DISC received

2)		1)

U10			send REL

3)

U12		U19

1)	Sent without progress indicator.

2)	Sent with progress indicator value #8 in band information available.

3)	This behaviour may happen, otherwise the MS must send a RELEASE and go to state U19.

II.5.3.8.1.2.2.7	U11 disconnect request

State U11 is reached using Table 3.

DISC received		REL received		timeout T305

U11	send REL		send REL_COM		send REL

MM connection rel			2) 1)

U19		U0		U19

1)	Check T305 timer accuracy between reception of DISC and REL by SS.

2)	With the same cause value as originally contained in the DISC message. An additional cause information element (#102 recovery on timer expiry) may be included.

State U11 is reached using Table 4.

MMCC_ERR_IND

1)

U11	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U11 is reached using Table 4.

unknown message

received	1)

U11	send STATUS

2)

U11

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, and state U11.

II.5.3.8.1.2.2.8	U12 disconnect indication

The tests of this section are only applicable if the MS implementation has a stable U12.

State U12 is reached using Table 1.

MNCC_REL_REQ		REL received

(on hook)			1)

U12	send REL		send REL_COM

MM connection rel

U19		U0

1)	In case of causes mentioned in GSM 04.08, section 5.4.2.

State U12 is reached using Table 2.

MMCC_ERR_IND

1)

U12	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U12 is reached using Table 3.

unknown message

received	1)

U12	send STATUS

2)

U12

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, and state U12.

II.5.3.8.1.2.2.9	U19 release request

State U19 is reached using Table 4.

1st timeout T308		2nd timeout T308		REL received		REL_COM

2)	received

U19	send REL		MM connection rel		MM connection rel		MM connection rel

1)

U19		U0		U0		U0

1)	Check T308 timer accuracy between the 2 receptions of REL by SS.

2)	With same cause number as originally contained in DISC and optional cause #102 recovery on timer expiry.

State U19 is reached using Table 1.

MMCC_ERR_IND

1)

U19	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

II.5.3.8.1.3	Incoming Call

II.5.3.8.1.3.1	Initial conditions

The tables below describe message exchanges which bring the MS in the requested initial states in case of an incoming call.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order will be U0, U6, U9, U7, U8, U10, U26 etc. as in the following tables.

TABLE 1

actions			messages exchanged			next

MS SS		state

SS sends PAGING

with			CHANN_REQ		---->

IMM_ASSIGN (SDCCH)	   <----		U0

PAG_RESP		---->

AUTH_REQ		   <----

AUTH_RESP		---->

CIPH_MODE_CMD	   <----

CIPH_MODE_COM	---->

SETUP			   <----		U6

CALL_CONF		---->		U9

ALERT			---->		U7

off hook		CONN			---->		U8

ASSIGN (TCH)		   <----

ASSIGN_COM		---->

CONN_ACK		   <----		U10

initiate modify		MODIFY			---->		U26

TABLE 2

actions			messages exchanged			next

MS SS		state

SS sends PAGING

with			CHANN_REQ		---->

IMM_ASSIGN (SDCCH)	   <----		U0

PAG_RESP		---->

CIPH_MODE_CMD	   <----

CIPH_MODE_COM	---->

SETUP			   <----		U6

CALL_CONF		---->		U9

ALERT (Note)		---->		U7

ASSIGN (TCH)		   <----

ASSIGN_COM		---->

CONN			---->		U8

AUTH_REQ		   <----

AUTH_RESP		---->

CONN_ACK		   <----		U10

initiate modify		MODIFY			---->		U26

Note:	It would be possible for the SS to send the ASSIgn command before the ALERT from the MS. In this case the MS shall send the ASSIGN COM before ALERT.

TABLE 3

actions			messages exchanged			next

MS SS		state

SS sends PAGING

with			CHANN_REQ		---->

IMM_ASSIGN (TCH)	   <----		U0

PAG_RESP		---->

AUTH_REQ		   <----

AUTH_RESP		---->

CIPH_MODE_CMD	   <----

CIPH_MODE_COM	---->

CHAN MOD MODIFY 1)	   <----

CHAN MOD MODIFY ACK	---->

SETUP			   <----		U6

CALL_CONF		---->		U9

ALERT			---->		U7

CONN			---->		U8

CONN_ACK		   <----		U10

initiate modify		MODIFY			---->		U26

Note 1):  channel mode = speech full rate

TABLE 4

actions			messages exchanged			next

MS SS		state

SS sends PAGING

with			CHANN_REQ		---->

IMM_ASSIGN (SDCCH)	   <----		U0

PAG_RESP		---->

CIPH_MODE_CMD	   <----

CIPH_MODE_COM	---->

SETUP (Note)		   <----		U6

CALL_CONF		---->		U9

ASSIGN (TCH)		   <----

ASSIGN_COM		---->

ALERT			---->		U7

CONN			---->		U8

AUTH_REQ		   <----

AUTH_RESP		---->

CONN_ACK		   <----		U10

initiate modify		MODIFY			---->		U26

Note: the signal information element is not included.

II.5.3.8.1.3.2    Test procedures

II.5.3.8.1.3.2.1	U0 null state

State U0 is reached using Table 1.

SETUP received		SETUP received		SETUP received		SETUP received

1)		3)		5)		6)

U0			send REL_COM

4)

U6	2)	U0		U6	2)	U6	2)

1)	With no optional parameter provided.

2)	This state may not be observable if MS immediately responds with CALL_CONF.

3)	With bearer capability not supported by the MS.

4)	With one of the following cause field values:

#57 bearer capability not authorized

#58 bearer capability not presently available

#65 bearer service not implemented

#88 incompatible destination.

5)	With signal information present in the SETUP.

6)	With one of the following values for the progress indicator field:

#1 call is not end to end PLMN/ISDN

#3 originating address is not PLMN/ISDN.

II.5.3.8.1.3.2.2	U6 call present

State U6 is reached using Table 2.

MNCC_CALL		MNCC_REJ_REQ		DISC received		REL received

_CONF_REC			1)		5)		5) 4)

U6	send CALL_CONF		send REL_COM				send REL_COM

2)			MM connection rel

U9	3)	U0		U19		U0

1)	If the ME supports the ability to refuse a call.

2)	With cause value #21 call rejected.

3)	May not be observable if MS immediately sends ALERT.

4)	In case of causes mentioned in GSM 04.08, section 5.4.2.

5)	If U6 is transient (implementation option) this is not tested.

State U6 is reached using Table 3.

MMCC_ERR_IND

2) 1)

U6	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

2)	If U6 is transient (implementation option) this is not tested.

State U6 is reached using Table 1.

unknown message

received	3) 1)

U6	send STATUS

2)

U6

1)	The "message type" field is unknown.

2)	With cause value #97 or #98, and

state U6.

3)	If U6 is transient (implementation option) this is not tested.

II.5.3.8.1.3.2.3	U9 MS terminating call confirmed

State U9 is reached using Table 2		State U99 is reached using Table 4

MNCC_ALERT		MNCC_SETUP_RSP	MMCC_SYNC_IND		MNCC_DISC_REQ

_REQ	2)		2)	(TCH assignment)

U9	send ALERT		send CONN		L2 establishment		send DISC

1)			on FACCH

U7		U8		U9		U11

1)	Verify that it is sent only if an alerting indication is given at the MS side.

2)	This primitive is internally generated. The MS is allowed to behave as specified for any of the internally generated primitives.

State U9 is reached using Table 4.

DISC received		REL received

1)		2)

U9	send REL		send REL_COM

MM connection rel

U19		U0

1)	Sent with or without progress indicator.

2)	In case of causes mentioned in GSM 04.08, section 5.4.2.

State U9 is reached using Table 1.

MCC_ERR_IND

1)

U9	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U9 is reached using Table 4.

unknown message

received	1)

U9	send STATUS

2)

U9

1)	The "message type" field is unknown.

2)	With cause #97 message type non existent or #98 message not compatible with control state or message type non-existent state U9.

II.5.3.8.1.3.2.4	U7 call received

State U7 is reached using Table 3.

MNCC_SETUP_RSP	MNCC_DISC_REQ

U7	send CONN		send DISC

U8		U11

1)	With cause #21 call rejected.

State U7 is reached using Table 1.

DISC received		DISC received		REL received

1)		2)		3)

U7	send REL		send REL		send REL_COM

MM connection rel

U19		U19		U0

1)	Sent with progress indicator value #8 in band information available.

2)	Sent without progress indicator.

3)	In case of causes mentioned in GSM 04.08, section 5.4.2 a) or b).

State U7 is reached using Table 2.

MMCC_ERR_IND

1)

U7	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U7 is reached using Table 3.

unknown message

received	1)

U7	send STATUS

2)

U7

1)	The "message type" field is unknown.

2)	With cause #97 or #98 message not compatible with control state or message type non-existent.

State U7 is reached using Table 1.

MMCC_SYNC_IND

(TCH assignment)

U7	L2 establishment

on FACCH

U7

II.5.3.8.1.3.2.5	U8 connect request

State U8 is reached using Table 2.

CONN_ACK		timeout T313		MNCC_DISC_REQ

U8			send DISC		send DISC

U10		U11		U11

State U8 is reached using Table 3.

DISC received		DISC received		REL received

2)		1)		4)

U8			send REL		send REL_COM

3)			MM connection rel

U12		U19		U0

1)	Sent without progress indicator.

2)	Sent with progress indicator value #8 in band information available.

3)	This behaviour may happen, or the MS may send a RELEASE message and then go to state U19 (see GSM 04.08, 5.4.4.1)

4)	In case of causes mentioned in GSM 04.08, section 5.4.2 a) or b).

State U8 is reached using Table 1.

MMCC_ERR_IND

1)

U8	MM connection rel

U0

1)	Due to a low layer failure initiated by the SS (e.g. the SS does not respond at L2 level).

State U8 is reached using Table 1.

MMCC_SYNC_IND		unknown message

(TCH assignment)		received	1)

U8	L2 establishment		send STATUS

on FACCH			2)

U8		U8

1)	The "message type" field is unknown.

2)	With cause #97 message type non existent or #98 message not compatible with control state or message type non-existent.

state U7.

II.5.3.8.1.4	In call functions

II.5.3.8.1.4.1	Test procedures

II.5.3.8.1.4.1.1	U10 call active

State U10 is reached using Table 2.

MNCC_START		START_DTMF_ACK		START_DTMF_REJ		MNCC_STOP

_DTMF_RE		received		received		_DTMF_REQ

U10	send START_DTMF					send STOP_DTMF

3) 2) 1)						1)

U10		U10		U10		U10

1)	Verify To > Tomin, Ts > Ts min.

To time between STOP_DTMF_ACK  and next START_DTMF sent.

Ts time between START_DTMF_ACK and next STOP_DTMF sent.

2)	Verify that this function is only available during the speech phase of a call (as a consequence no DTMF messages are sent on SDCCH).

3)	Verify that the only keypad information sent are (0,...,9,A,B,C,D,#,*).

State U10 is reached using Table 3.

STOP_DTMF_ACK

received

U10

U10

Note: The following test is not yet applicable.

State U10 is reached using Table 1.

MNCC_NOTIFY_REQ	NOTIFY received

1)		2)

U10	send NOTIFY

U10		U10

1)	May never be created by MS.

2)	With notification indicator with successive values: user suspended, user resumed, bearer changed.

State U10 is reached using Table 2.

MMCC_SYNC_IND		MMCC_SYNC_IND

1)		2) 3)

U10

U10		U10

1)	Corresponding to channel assignment or channel mode modify initiated by SS at RM layer.

2)	The MS returns to the old channel after a handover failure.

3)	SS reestablishes correctly the link.

State U10 is reached using Table 1.

MODIFY received		MODIFY received		MODIFY received		MODIFY received

1)		3)		5)		7)

U10	send MODIFY_COM	send MODIFY_COM	send MODIFY_REJ		send MODIFY_REJ

2)				6)		8)

U10		U10		U10		U10

1)	With new mode same as actual mode.

2)	Contains actual mode as bearer capability.

3)	With new mode different from actual mode, but one of those already negotiated and agreed during the establishment phase of the call. This test is not applicable to an MS if it doesn't support dual bearer capability.

4)	(reserved).

5)	With bearer capability not supported by the MS.

6)	With cause value #58 bearer capability not presently available and with old bearer capability.

7)	The bearer capability indicated in the MODIFY message does not belong to that already given in the initiating SETUP message.

8)	With cause value #57 bearer capability not authorized.

Note: The following test is not yet applicable.

State U10 is reached using Table 2.

MNCC_MODIFY_REQ

U10	send MODIFY

2) 1)

U26

1)	Verify that the bearer capability provided in the MODIFY message is one of that already given in the initial SETUP.

2)	Verify that the MS stops sending Bm channel information according to old mode.

II.5.3.8.1.4.1.2	U26 MS originating modify

Note: The tests in this subsection are not yet applicable.

State U26 is reached using Table 1.

MODIFY_COM rec.		MODIFY_REJ rec.		MODIFY_COM rec.		MODIFY_REJ rec.

1)		3)		5)

U26

2)		4)		4)

U10		U10		U26		U26

1)	With cause #58 bearer capability not presently available and with old bearer capabilities.

2)	Verify that the MS resumes sending Bm channel information according to old mode.

3)	With a mode that does not correspond to the requested one.

4)	No action, the message is ignored.

5)	With a mode that does not correspond to the actual one.

State U26 is reached using Table 2.

timeout T323		MMCC_SYNC_IND

3)

U26	send DISC

2) 1)

U11		U26

1)	Check T323 timer accuracy between emission of MODIFY and reception of DISC by SS.

2)	Sent with cause value #102 recovery on timer expiry.

3)	Corresponding to channel assignment or channel mode modify initiated by SS at RM layer.

State U26 is reached using Table 3.

MMCC_SYNC_IND

1) 2)

U26

U26

1)	The MS returns to the old channel after a handover failure.

2)	SS reestablishes correctly the link.

State U26 is reached using Table 1.

unknown message

received	1)

U26	send STATUS

2)

U26

1)	The "message type" field is unknown.

2)	With cause #97 or #98 message not compatible with control state or message type non-existent.

state U26.

II.5.3.8.2	Emergency call establishment

II.5.3.8.2.1	Introduction

Emergency call establishment can be initiated by an MS in any state of the mobility management sub-layer i.e. whether location updating has been successful or failed for any reason, including SIM removal.

The emergency call establishment uses a unique EMERGENCY SETUP message and includes a field identified as emergency in the CHANNEL REQUEST message and the CM SERVICE REQUEST message. An MS shall only perform an emergency call establishment if it is equipped for voice telephony.

It seems obvious that the network would not want to initiate an authentication procedure if IMEI has been sent in the CM SERVICE REQUEST message because the MS has no SIM, so this is not tested, even though it is not specifically prohibited in REC GSM 04.08.

The passages of unique interest to emergency call establishment in REC GSM 04.08 are 5.2.1.2 and 4.5.1.5.

II.5.3.8.2.2	Purpose of the test

The purpose of this test is to verify that the MS can :

- correctly setup an emergency call;

- set up an emergency call if it has no IMSI, TMSI allocated;

- be rejected during an emergency call establishment.

It is not the purpose of this test to verify that the MS can send an emergency call when any MS features have been set. In these cases, as directed in other tests, only the section II.5.3.8.2.4 or 6 should be tested.

II.5.3.8.2.3	Initial conditions

The SS simulates one cell with BCCH, CCCH, SDCCH and TCH.

The MS shall be in the "idle updated" state as after a successful location update in test II.5.3.2.4.

II.5.3.8.2.4	Emergency call establishment (idle updated)

II.5.3.8.2.4.1	Procedure 1

After the initial conditions have been established the number 112 is dialled on the MS to initiate an emergency call.

II.5.3.8.2.4.2	Response requirement 1

The MS sends a Random Access channel request.

Message:	CHANNEL REQUEST.

Information Element		Comment			Value

Establ. cause		Emergency			101 (binary)

reference random		arbitrary

All other requirements of this random access have been tested in II.5.3.3.2.2.

II.5.3.8.2.4.3	Procedure 2

The SS responds to the MS with an immediate assignment message allocating a SDCCH.

Message:	IMMEDIATE ASSIGNMENT.

Information Element		Comment			Value

Protocol discriminator	RR management			0110	(bin)

Transaction identifier					0000	(bin)

Message type		Immediate			0011	1111

assignment			(binary)

Page mode			normal paging

Channel description		SDCCH

Request reference		as sent by the MS

Timing advance

Mobile allocation

Starting time

II.5.3.8.2.4.4	Response requirement 2

The MS goes to the correct SDCCH as tested in II.5.3.3.2.3 except that the message sent on the SDCCH is CM service request with CM service type set to Emergency.

Message:	CM SERVICE REQUEST.

Information Element		Comment			Value

Protocol discriminator	MM message				0101

Transaction identifier	not relevant			 000

Message type						0x10	0100

CM service type		emergency call			0010

Ciphering key sequence	as stored in the MS

number

Mobile station classmark 2	as specified by manufacturer

Mobile identity		TMSI

II.5.3.8.2.4.5	Procedure 3

The SS performs a successful authentication with the MS as tested in II.5.3.7.2.2.

The SS performs a successful cipher mode setting with the MS as tested in II.5.3.6.8.3.1.

II.5.3.8.2.4.6	Response requirement 3

The MS shall consider the cipher mode setting as acceptance of its CM service request and so send an emergency setup message.

Message:	EMERGENCY SETUP

Information Element		Comment			Value

Protocol discriminator	CC					0011

Transaction identifier

Message type						0x00	1110

Bearer capability		Optional

II.5.3.8.2.4.7 	Procedure 4

The SS continues the call setup with the MS as tested in II.5.3.9.2 by :

-	sending a call proceeding message

-	sending an alert message

-	assigning the appropriate full rate or half rate speech traffic channel

-	performing a connect procedure.

The SS shall check that a two way audio path has been established.

The SS then clears the call as tested in II.5.3.9.2 by :

-	sending a disconnect

-	sending a release complete in response to the  MS release

-	sending a channel release.

II.5.3.8.2.5 	Emergency call establishment (Idle Updated)

Exception testing

II.5.3.8.2.5.1	Description

If an MS is equipped for both full rate and half rate speech, test II.5.3.8.2.4 shall be performed for the MS operating at its preferred rate and then repeated with its non-preferred rate. The MS shall send the bearer capability of the non-preferred rate in the emergency setup.

II.5.3.8.2.6 	Emergency call establishment (Idle, no IMSI)

II.5.3.8.2.6.1	Procedure 1

After the initial conditions have been established the SIM is removed. The number 112 is dialled on the MS to initiate an emergency call.

II.5.3.8.2.6.2	Response requirement 1

The MS sends a Random Access Channel request.

Message:	CHANNEL REQUEST

Information Element		Comment			Value

Establ. cause		Emergency			101 (binary)

random reference		arbitrary

All other requirements of this random access have been tested in II.5.3.3.2.2.

II.5.3.8.2.6.3 	Procedure 2

The SS responds to the MS with an immediate assignment message allocating a SDCCH.

Message:	IMMEDIATE ASSIGNMENT

Information Element		Comment			Value

Protocol discriminator	RR Management			0110 (bin)

Transaction identifier	orig. side and RR			0000 (bin)

Message type		Immediate			0011	1111

Assignment			(binary)

Page mode			normal paging			00

Channel description		SDCCH

Request reference		as sent by the MS

Timing advance

Mobile allocation

Starting time

II.5.3.8.2.6.4 	Response requirement 2

The MS goes to the correct SDCCH as tested in II.5.3.3.2.3 except that the message sent on the SDCCH is CM service request.

Message:	CM SERVICE REQUEST.

Information Element		Comment			Value

Protocol discriminator	MM message			0101

Transaction identifier	not relevant			 000

Message type						0x10	0100

CM service type		emergency call			0010

Ciphering key sequence	as stored in the MS

number

Mobile station classmark 2	as specified by manufacturer

Mobile identity		IMEI

Note:	IMEI Mobile Identity.

II.5.3.8.2.6.5	Procedure 3

The SS responds to the MS with a CM service accept message.

Message:	CM SERVICE ACCEPT

Information Element		Comment			Value

Protocol discriminator	MM message			0101

Transaction identifier	not relevant			 000

Message type						0010	0001

II.5.3.8.2.6.6 	Response requirement 3

The MS shall send an Emergency Setup message.

Message:	EMERGENCY SETUP

Information Element		Comment			Value

Protocol discriminator	CC message				0011

Transaction identifier

Message type						0x00	1110

Bearer capability		Optional

II.5.3.8.2.6.7 	Procedure 4

The SS continues the call setup with the MS as tested in II.5.3.9.2 by:

-	sending a call proceeding message

-	sending an alert message

-	assigning the appropriate full rate or half rate speech traffic channel

-	performing a connect procedure.

The SS shall check that a two way audio path has been established.

The SS then clears the call as tested in II.5.3.9.2 by :

-	sending a disconnect

-	sending a release complete in response to the MS release

-	sending a channel release.

II.5.3.8.2.7 	Emergency Call Rejection

The only rejection test for emergency call establishment is for the SIM removed case.

II.5.3.8.2.7.1	Procedure 1

After the initial conditions have been established the SIM is removed. The number 112 is dialled on the MS to initiate an emergency call.

II.5.3.8.2.7.2 	Response requirement 1

The MS sends a Random Access Channel request.

Message:	CHANNEL REQUEST

Information Element		Comment			Value

Establ. cause		Emergency			101 (binary)

random reference		arbitrary

All other requirements of this random access have been tested in II.5.3.3.2.2.

II.5.3.8.2.7.3 	Procedure 2

Message:	IMMEDIATE ASSIGNMENT

Information Element		Comment			Value

Protocol discriminator	RR Management			bin	0110

Transaction identifier	not relevant			0000	(bin)

Message type		Immediate			0011	1111

Assignment			(binary)

Page mode			normal paging

Channel description		SDCCH

Request reference		as sent by the MS

Timing advance

Mobile allocation

Starting time

II.5.3.8.2.7.4	Response Requirement 2

The MS goes to the correct SDCCH as tested in II.5.3.3.2.3 except that the message sent on the SDCCH is CM service request.

Message:	CM SERVICE REQUEST

Information Element		Comment			Value

Protocol discriminator	MM message			0101

Transaction identifier	not relevant			 000

Message type						0x10	0100

CM service type		emergency call			0010

Ciphering key sequence	as stored in the MS

number

Mobile station classmark 2	as specified by manufacturer

Mobile identity		IMEI

Note:	IMEI Mobile Identity.

II.5.3.8.2.7.5 	Procedure 3

The SS responds to the MS with a CM Service Reject message .

Message:	CM SERVICE REJECT

Information Element		Comment			Value

Protocol discriminator	MM message			0101

Transaction identifier	not relevant			 000

Message type						0010	0010

Reject cause		IMEI not accepted		0000	0101

There shall be no emergency call set up from MS.

II.5.3.8.3	Call Re-establishment

II.5.3.8.3.1	Introduction

The call re-establishment procedure is used to re-instate a call which has failed due to a lower layer failure.  It is described in GSM  04.08, sections 4.5.1.6 and 5.5.4.

II.5.3.8.3.2	Purpose of the test

The purpose of these test is to verify that the MS can correctly perform a call re-establishment and that re-establishment attempts are carried out only when this is allowed.

II.5.3.8.3.3	Call present, re-establishment allowed

II.5.3.8.3.3.1	Initial conditions

The SS simulates cells A and B which have equal LAI. The conditions of cells A and B shall allow call re-establishment:

Cell B is not barred, the RACH control parameters Information Element sent in SYSTEM INFORMATION TYPE 1 to 4 of cell B specifies 'Call Reestablishment allowed in the cell', the NCC of cell B is indicated as permitted in the PLMN permitted Information Element of SYSTEM INFORMATION TYPE 2 and 6 of cell A.

II.5.3.8.3.3.2	Procedure

The MS is brought into the idle, updated state in cell A, and a call is established according to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to k).

The SS then stops transmission on the TCH/SACCH. The RF level of cell A is lowered in order to force the MS to select cell B.

In the required re-establishment of the call, after the radio link time-out, the SS applies the following procedure on cell B:

a)	The SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message.

b)	The SS accepts the CM-REESTABLISHMENT REQUEST message from the MS via the data link establishment procedure.

c)	The SS sends message CIPHERING MODE COMMAND and starts deciphering.

d)	After reception of CIPHERING MODE COMPLETE the SS starts enciphering.

e)	The SS then sends message ASSIGNMENT COMMAND.

f)	Having received message ASSIGNMENT COMPLETE, the SS enters conversation state.

The contents of the messages shall be according to section "Structured

procedures, Mobile originating call, early assignment, Method of test".

II.5.3.8.3.3.3	Requirements

After expiry of the radio link timeout the MS shall re-establish the call according to the following procedure:

1)	Before step a) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message with establishment cause set to call re-establishment.

2)	After step a) the MS shall send a Layer 2 SABM frame containing the CM REESTABLISHMENT REQUEST message to the SS.

3)	After step c) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

4)	After step e) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel.

The MS shall then through-connect the speech path in both directions.

The contents of the messages shall be according to section "Structured procedures, Mobile originating call, early assignment, Requirements" with the following change/addition:

In message CHANNEL REQUEST the establishment cause shall be call re-establishment.

Message:	CM-REESTABLISHMENT REQUEST	(GSM 04.08, 9.2.4) to the SS:

Information Element		Comment			Value

Protocol discriminator	MM				0101

Transaction identifier	Not used				0000

Message type						0x10	1000

Ciphering key seq.number	As applic.from loc.upd		xxx

MS classmark 2

- Revision level		As applicable			xxx

- Encryption algorithm	A5				00

- RF power capability	As applicable			xxx

- Short message capability	As applicable			x

- Frequency band		Band no 0			000

Mobile identity

- Odd/even no of digits	As applicable			x

- Type of identity		TMSI				100

- Identity digits		As applicable

Location area identification	Cell B

II.5.3.8.3.4	Call present, re-establishment not allowed

II.5.3.8.3.4.1	Initial conditions

The RE field of the RACH control parameters information element broadcast in messages SYSTEM INFO TYPE 1, 2, 3 and 4 of the serving cell is set to "Call Reestablishment not allowed in the cell".

II.5.3.8.3.4.2	Procedure

a)	The MS is brought into the idle, updated state and a call is established according to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to k).

b)	The SS then stops transmission on the TCH/SACCH.

II.5.3.8.3.4.3	Requirements

1)	The MS shall not try to re-establish the call, but return to the idle, updated state.

II.5.3.8.3.5	Call under establishment, re-establishment allowed

II.5.3.8.3.5.1	Initial conditions

The default conditions of the serving cell shall allow call re-establishment (bit RE is set to 0).

II.5.3.8.3.5.2	Procedure

a)	The MS is brought into the idle, updated state.

b)	Call establishment is started according to the procedures of section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

c)	The SS then stops transmission on the SDCCH/SACCH.

II.5.3.8.3.5.3	Requirements

1)	The MS shall not try to re-establish the call, but return to the idle, updated state.

II.5.3.8.4	DTMF information transfer

Note:	The values [Ts] and [To] are used below. [Ts] is the minimum send duration and [To] is the minimum off time between tones as specified in CEPT T/CS 46-02.

II.5.3.8.4.1	Purpose

The purpose of this test is to verify that the MS can cause DTMF tones to be generated by the network in a correct manner.

II.5.3.8.4.2	Procedure 1

a)	The MS is brought into the idle, updated state and a call is established according to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to l).

b)	The MS is made to send one DTMF character.

c)	The character is acknowledged by the SS using message START DTMF ACK.

Message:	START DTMF ACK	(GSM 04.08, 9.3.18) to the MS:

information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0011	0110

Keypad facility		IEI				0010	1100

IA5 character			xxxx	xxxx

II.5.3.8.4.3	Requirements 1

1)	The MS shall send message START DTMF on the main DCCH.

Message:	START DTMF	(GSM 04.08, 9.3.17) to the SS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				0xxx

Message Type						0x11	0101

Keypad facility		IEI				0010	1100

IA5 character			xxxx	xxxx

II.5.3.8.4.4	Procedure 2

a)	The MS is made to trigger stopping of transmission of the DTMF tone at a point in time greater than or equal to [Ts] after receiving START DTMF ACK.

b)	After reception of message STOP DTMF the SS sends message STOP DTMF ACK on the main DCCH indicating that the sending of the DTMF tone has been stopped.

Message:	STOP DTMF ACK	(GSM 04.08, 9.3.23) to the MS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0011	0010

II.5.3.8.4.5	Requirements 2

1)	The MS shall send message STOP DTMF on the main DCCH at minimum [Ts] after reception of message START DTMF ACK.

Message:	STOP DTMF	(GSM 04.08, 9.3.22) to the SS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				0xxx

Message Type						0x11	0001

II.5.3.8.4.6	Procedure 3

a)	Procedures 1 and 2 are repeated after one another in such a way that the following characters should be transmitted: 0, 1, ... 9, *, # (, A, B, C, D if available on the MS). The time between characters (from STOP DTMF ACK to next START DTMF) shall be greater than [To].

II.5.3.8.4.7	Requirements 3

1)	The requirements 1 and 2 above apply and the MS shall transmit the characters in the sequence in which they were keyed in.

II.5.3.8.4.8	Procedure 4

a)	Procedures 1 and 2 are repeated after one another in such a way that two different, arbitrary, characters are transmitted. The time between characters (from STOP DTMF ACK to next START DTMF) shall be less than [To].

II.5.3.8.4.9	Requirements 4

1)	The MS shall transmit the characters in the sequence in which they were keyed in.

2)	The time between the first STOP DTMF ACK and the second START DTMF shall be at least [To].

II.5.3.8.4.10	Procedure 5

a)	Procedures 1 and 2 are repeated after one another in such a way that two different, arbitrary, characters are transmitted. However, the "sending time" (from START DTMF to STOP DTMF) shall be less than [Ts]. The time between characters (from STOP DTMF ACK to next START DTMF) shall be less than [To].

II.5.3.8.4.11	Requirements 5

1)	The MS shall transmit the characters in the sequence in which they were keyed in.

2)	The second START DTMF message shall be sent at least [To] after the STOP DTMF ACK of the first character.

3)	Each STOP DTMF message shall be sent at least [Ts] after its corresponding START DTMF ACK message.

II.5.3.8.4.12	Procedure 6

The purpose of this procedure is to ensure future compatibility when downlink DTMF has been introduced.

a)	The MS is brought into the idle, updated state and a call is established according to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to l).

b)	The SS sends message START DTMF.

Message:	START DTMF	(GSM 04.08, 9.3.17) to the MS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				0xxx

Message Type						0011	0101

Keypad facility		IEI				0010	1100

IA5 character			xxxx	xxxx

c)	The SS then checks the call state of the MS by sending message STATUS ENQUIRY.

Message:	STATUS ENQUIRY	(GSM 04.08, 9.3.21) to the MS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				0xxx

Message Type						0011	0100

d)	After the MS has reported its call state the SS sends message STOP DTMF.

Message:	STOP DTMF	(GSM 04.08, 9.3.22) to the MS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				0xxx

Message Type						0011	0001

e)	The SS again checks the call state of the MS by sending message STATUS ENQUIRY as above.

II.5.3.8.4.13	Requirement 6

1)	The MS shall not transmit any of the messages START DTMF ACK or STOP DTMF ACK. The MS shall not change its call state. The MS response to message STATUS ENQUIRY shall in both cases be message STATUS.

Message:	STATUS	(GSM 04.08, 9.3.20) to the SS:

Information element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				1xxx

Message Type						0x11	1101

Cause

- Coding standard		GSM				11

- Location			User				0000

- Cause value		30 (Rsp to STAT.ENQ)		 001	1110

Call state

- Coding standard		GSM				11

- Call state value		U10 - Active			  00	1010

Note 1:	DTMF rejection by the network is not tested since no requirements on the MS are defined in GSM 04.08.

Note 2:	There is no test for DTMF in the downlink direction.

II.5.3.8.5	user to user signalling

II.5.3.8.5.1	Purpose of the test

The purpose of this test is to verify that inclusion of the 'user-user' information element in a 'call-control' message causes no adverse effects on MS operation.

The 'user to user' information element is used to convey information between the mobile user and a remote ISDN user.

Note:	There is no test for an MS originating call including a 'user-user' information element since it is not a mandatory MS feature.

II.5.3.8.5.2	Method of test

II.5.3.8.5.2.1	Initial conditions.

1)	The SS shall simulate a BSS with CCCH and BCCH.

2)	The MS shall be placed in the MM-state "idle,updated" (refer location updating accepted II.5.3.6.4.1 ).

II.5.3.8.5.2.2	Procedure 1

a)	The SS shall page the MS and establish a normal call.

b)	The SETUP message from the SS shall contain a 'user-user' information element.

Message:	SETUP	(GSM 04.08, 9.3.15)

Information Element		Comment			Value

Protocol discriminator	call-control			0011

Transaction identifier	originating			0xxx

Message type		setup				0x00	0101

Bearer capabilities		B.C. iei				0000	0100

length				0000	0001

full rate speech			1010	0000

User-user			user-user iei			0111	1110

length				0000	0010

PD (see note)			0000	0000

user-user(see note)		0000	0000

NOTE:	The codings above are for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields.

II.5.3.8.5.2.3	Procedure 2

a)	The SS shall initiate call cleardown and include in the subsequent DISCONNECT message a 'user-user' information element.

Message:	DISCONNECT	(GSM 04.08, 9.4.16)

Information Element		Comment			Value

Protocol discriminator	call-control			0011

Transaction identifier	originating			1xxx

Message type		disconnect			0x10	0101

Cause			(MV)

length (4 bytes)			0000	0010

coding std(gsm)			0110	0000

cause (normal)			1001	0000

User-user			user-user iei			0111	1110

length (4 bytes)			0000	0010

PD (see note)			0000	0000

user-user(see note)		0000	0000

NOTE:	The coding above is for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields.

II.5.3.8.5.3	Requirements

II.5.3.8.5.3.1	Requirement to Procedure 1

1)	The MS shall respond to paging.

2)	The MS shall not be adversely affected by the inclusion of the information element and shall successfully establish a normal call.

II.5.3.8.5.3.2	Requirements to Procedure 2

1)	The MS shall not respond adversely to the inclusion of the 'user-user' information element in the DISCONNECT message and shall continue to clear the call normally.