II.5.3.3	Test of MS functions in idle mode

II.5.3.3.1	Initial conditions

The SIM shall contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

During the tests in II.5.3.3.2 and II.5.3.3.3, the following parameters apply:

RACH control parameters

In cells

GSM900:	1 to 7

DCS1800:	1 to 6:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	[10] slots for spreading

CB, Cell Barred	= 0	access is allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

In cell

GSM900:	8

DCS1800:	7:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	[10] slots for spreading

CB, Cell Barred	= 1	access is not allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

GSM900	DCS1800

Cell	PLMN	BA - ARFCN bit = 1	BA - ARFCN bit = 1

perm.

1	00000100	7	39	65	66	85	97	124	520	580	610	702	703	830	885

2	00000100	8	40	67	68	86	98	123	521	581	612	704	705	831	884

3	00000100	9	41	69	70	87	99	122	522	582	614	706	707	832	883

4	00000100	10	42	71	72	88	100	121	523	583	616	708	709	833	882

5	00000100	11	43	73	74	89	101	120	524	584	618	710	711	844	881

6	00000100	12	44	75	76	90	102	119	525	585	620	712	713	835	880

7	00000100	13	45	77	78	91	103	118	526	586	622	714	715	836	879

8	00000100	124

Location area identification

GSM900 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x

2	0	0	3	2	F	x

3	0	0	4	3	F	x

4	0	0	5	4	F	x

5	0	0	6	5	F	x

6	0	0	7	6	F	x

7	0	0	8	7	F	x

8	0	0	1	0	1	x	The HPLMN of the MS

GSM900 only - end

DCS1800 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x

2	0	0	3	2	F	x

3	0	0	4	3	F	x

4	0	0	5	4	F	x

5	0	0	6	5	F	x

6	0	0	7	6	F	x

7	0	0	1	0	1	x	The HPLMN of the MS

DCS1800 only - end

Note 1:	'x' denotes any value.

Note 2:	The MS representation of the MCC, MNC on the handset can be manufacturer dependant.

Note 3:	The NCC values of each cell must be different.

Control channel description and BS options

All

GSM900:	8 cells

DCS1800:	7 cells:

CELL_RESELECT_HYSTERESIS	= 010	4dB RXLEV hysteresis

MS_TXPWR_MAX_CCH	= value corresponding to the maximum

available output power from MS

RXLEV_ACCESS_MIN	= 30

ATT	= 0	no IMSI attach and detach

DTX	= 0	no discontinuous transmission

BS_AG_BLKS_RES	= 1	1 block reserved for access grant

CCCH_CONF	= 001	1 SDCCH combined with the CCCH

RADIO_LINK_TIMEOUT	= 5	10 s timeout

BS_PA_MFRMS	= 010	4 multiframes periods for paging

T3212 timeout value	= H'00

GSM900	DCS1800

Cell	level	BCCH ARFCN	level	BCCH ARFCN

dBmVemf(  )		dBmVemf(  )

1	+65	1	+65	520

2	+63	7	+63	580

3	+61	39	+61	610

4	+55	65	+55	702

5	+59	66	+59	703

6	+57	85	+57	830

7	+55	97	+55	885

8	+53	124

Note:	The SIM should contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

II.5.3.3.2	MS indication of available PLMNs

II.5.3.3.2.1	Purpose of the test

To verify that a MS presents the available PLMNs to the user according to the requirements of GSM 05.08 and 02.11, if no suitable cell of the selected PLMN has been found.

II.5.3.3.2.2	Procedure

a)	Each cell is given a different NCC value since each cell has a different MCC and MNC.

b)	The MS is switched on and equipped with a SIM containing default values except for those values listed under section II.5.3.3.1 (initial conditions).

II.5.3.3.2.3	Requirements

1)	The MS must present the user with some indication that regular service is unavailable.

2)	The MS shall present a list of available PLMNs (MCC and MNC values). The list shall include the MCC and MNC of

GSM900:	cells 1 to 7, but not of cell 8

DCS1800:	cells 1 to 6, but not of cell 7.

II.5.3.3.3	MS will send only if BSS is "on air"

II.5.3.3.3.1	Purpose of the test

To verify that the MS will not produce any RF transmission if no BSS is received.

II.5.3.3.3.2	Procedure

a)	The RF-signal for the BCCHs of

GSM900:	cell 1 to 8 is switched off

DCS1800:	cell 1 to 7 is switched off.

b)	The SS shall wait 20 s to allow the MS to detect the loss of cells.

c)	By MMI, an attempt to originate a call is made.

d)	By MMI, an attempt to originate an emergency call is made.

Step d) is only performed if the MS supports speech (see PICS/PIXIT statement).

II.5.3.3.3.3	Requirements

1)	The MS must not give "service indication".

2)	In steps c) and d) the MS shall not produce any RF output.