32.	Testing of speech transcoding functions

The test sequences for speech transcoding and DTX tests, both for input and required output, are defined in TS 06.10, section 5, and TS 06.32, section 4. They are available on floppy disks in IBM/AT MS-DOS format from ETSI/PT12

The Digital Audio Interface (DAI) is described in [36.4].

Note:	For a definition of the term "traffic frame" used in this chapter, refer to TS 06.32.

32.1	Downlink speech transcoding

32.1.1	Definition and applicability

Downlink speech transcoding transforms the 13 kbit/s net bit stream obtained by channel decoding the incoming bit stream from the air interface to 13 bit linear PCM.

The requirements and this test apply to all MS supporting TCH/FS.

32.1.2	Conformance requirement

The output bit stream from the seech transcoder shall be continuous and bit by bit exactly the same as the predefined output sequence (SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT).

TS 06.01, 2; TS 06.10, 5.2/5.2.2

32.1.3	Test purpose

To verify that the speech transcoding of the MS can transform all predefined sequences (SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT) at 13 kbits/s level to 104 kbit/s (13 bit linear PCM at 8 kHz) level correctly.

32.1.4	Method of test

32.1.4.1	Initial conditions

DTX is off.

32.1.4.2	Procedure

a)	The SS resets the speech decoder of the MS via the DAI.

b)	The SS sends test sequence SEQ01.COD at 13 kbit/s to the MS via the air interface after passing it through the SS channel encoder.

Note:	These test sequence files contain 16 bit words for all speech encoded parameters and are justified as described in TS 06.10 table 5.1. 76 words are used as input in a period of 20 ms.

c)	The SS records the 104 kbit/s output bit stream from the MS on the digital audio interface.

d)	The test is repeated using the test sequences SEQ03.COD, SEQ04.COD and SEQ05.COD.

32.1.5	Test Requirements

The bit stream output shall be continuous and bit by bit exactly the same as the sequence given in the files SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT.

Note:	These files contain 16 bit words of 13 bit linear PCM left justified.

32.2	Downlink receiver DTX functions

32.2.1	Definition and applicability

The DTX receiver functions consist of a SID frame detector, comfort noise generator functions and lost frame substitution and muting functions.

The requirements and this test apply to all MS supporting TCH/FS.

32.2.2	Conformance requirement

1)	The output level of the decoder has to be constant for an input signal consisting of identical speech frames.

TS 06.10

2/3)	When, after the first lost speech frame subsequent speech frames are lost, a muting technique shall be used that will gradually decrease the output level, resulting in the silencing of the output after a maximum of 320 ms. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

TS 06.01, 6; TS 06.11, 2.1/2.2/3 [for requirement 2 (first part)].

TS 06.01, 6; TS 06.11, 2.1/2.2/3; TS 06.31, 1.2.2/3.1.1 [for requirement 3 (second part)].

4/5)	A valid SID-frame followed by a sequence of lost speech frames shall result in comfort noise generation with constatnt block amplitude parameters. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

TS 06.01, 3/5; TS 06.12, 3/3.1;

TS 06.31, 1.2.2/3.1/3.1.1/3.1.2 [for requirement 4 (first part)].

TS 06.01, 3/5/6; TS 06.11, 2.1/2.2; TS 06.12, 3/3.1;

TS 06.31, 1.2.2/3.1/3.1.1/3.1.2 [for requirement 5 (second part)].

6/7)	An invalid SID-frame followed by a sequence of lost speech frames shall result in comfort noise generation, using the set of parameters from the last valid SID-frame. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

TS 06.01, 3/5; TS 06.12, 3/3.1;

TS 06.31, 1.2.2/3.1/3.1.1/3.1.2 [for requirement 6 (first part)].

TS 06.01, 3/5/6; TS 06.11, 2.1/2.2; TS 06.12, 3/3.1;

TS 06.31, 1.2.2/3.1/3.1.1/3.1.2 [for requirement 7 (second part)].

8)	The energy of the output signal is controlled by the block amplitude parameter, xmaxc.

TS 06.10, 3.1.20/3.1.21/3.2.1;

9/10)	The first SID-frame that is expected and not received shall be substituted by the last valid SID-frame and the procedure for valid SID-frames shall be applied. For the second lost SID-frame, a muting technique shall be used that will gradually decrease the output level, resulting in silencing the output after a maximum of 320 ms. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

TS 05.08, 8.3; TS 06.01, 6; TS 06.11, 2.3/2.4;

TS 06.31, 1.2.2/3.1.1/3.1.2.

32.2.3	Test purpose

1)	To verify that the signal energy at the output of the decoder is constant with a tolerance of +/- 3 dB if a sequence of identical speech frames is applied at the receiver input.

2)	To verify that the muting function of the receiver is within the required limits if a sequence of lost speech frames is applied at the receiver input.

3)	To verify that the muting function of the receiver is within the required limits if a sequence of speech frames with the FACCH flag set is applied at the receiver input.

4)	To verify the function of comfort noise generation when a valid SID-frame is received followed by a sequence of lost speech frames. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

5)	To verify the function of comfort noise generation when a valid SID-frame is received followed by a sequence of speech frames with the FACCH flag set. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

6)	To verify the function of comfort noise generation when an invalid SID-frame is received followed by a sequence of lost speech frames. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

7)	To verify the function of comfort noise generation when an invalid SID-frame is received followed by a sequence of speech frames with the FACCH flag set. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

8)	To verify that the signal energy at the output of the decoder depends on the block amplitude xmaxc of the input frames if a sequence of speech frames is applied to the decoder. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

9)	To verify the SID-frame substitution and muting functions on the comfort noise, if two consecutive expected SID-frames are lost with the other frames being lost speech frames.

10)	To verify the SID-frame substitution and muting functions on the comfort noise, if two consecutive expected SID-frames are lost with the other frames being speech frames with the FACCH flag set.

32.2.4	Method of test

32.2.4.1	Initial conditions

32.2.4.2	Procedure

a)	The SS transmits coded "speech" traffic frames on the air interface after passing them trough the SS channel encoder. They contain a special test signal at 13 kbit/s as defined below. All traffic frames are identical with the exception of some frames which are SID frames as defined in TS 06.32.

b)	The energy of the PCM signal is evaluated (as a mean square average) at the digital audio interface of the MS at 104 kbit/s level (13 bit, 8 kHz linear PCM) and recorded for each block of 20 ms synchronized to the 20 ms speech frame structure.

c)	The SS transmission of the TDMA frames of the TCH/FS on the air interface is ramped "on" and "off" on a traffic frame by traffic frame basis, taking into account the block diagonal interleaving scheme defined in TS 05.03. The first traffic frame in step 1 occurs one frame after the window of the SACCH multiframe (TDMA frame 60 modulo 104), allocated for the SID frame (see TS 05.02 and 05.08). The SACCH will also be transmitted.

Note:	8 timeslots in 8 consecutive TCH/FS TDMA frames are seen as one traffic frame, and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in TS 05.03.

d)	The special test frame is an encoded "speech" traffic frame of 260 bits obtained from white Gaussian noise band limited to 300 - 3400 Hz. When repeated, the special test frame results in a humming sound with a fairly constant level when decoded, and is defined in table .

Table : Table of special test traffic frame for receiver DTX tests

Encoded parameter:	Value:

LARc(1)	38

LARc(2)	42

LARc(3)	24

LARc(4)	20

LARc(5)	10

LARc(6)	9

LARc(7)	5

LARc(8)	3

Sub-block no:

0	1	2	3

Grid position (Mc)	1	3	2	0

Block amplitude (xmaxc)	40	40	40	40

LTP gain (Bc)	0	0	0	0

LTP lag (Nc)	40	120	40	120

RPE pulses (xmc)

- pulse no 1	4	6	6	6

- pulse no 2	4	5	4	3

- pulse no 3	2	1	3	4

- pulse no 4	6	2	1	3

- pulse no 5	3	6	4	1

- pulse no 6	5	1	6	3

- pulse no 7	5	2	5	5

- pulse no 8	5	6	2	1

- pulse no 9	1	3	4	4

- pulse no 10	3	2	4	3

- pulse no 11	5	5	4	5

- pulse no 12	6	1	2	2

- pulse no 13	1	3	4	3

Note:	The signal energy of the decoded special test frame is controlled with the block amplitude parameter (xmaxc). Reducing xmaxc from 40 to 32 reduces the signal energy by 6 dB, and reducing xmaxc from 40 to 24 reduces the signal energy by 12 dB.

e)	The sequence of traffic frames on the air interface is as follows:

e.1)	23 test frames "on".

e.2)	20 frames "off".

e.3)	20 test frames "on".

e.4)	1 SID frame followed by 6 frames "off", another identical SID frame and 23 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

e.5)	1 different SID frame, however with 2 to 15 errors inserted in the SID codeword, followed by 23 frames "off".

e.6)	20 test frames "on", but with the block amplitude parameter xmaxc = 24.

e.7)	1 SID frame followed by 50 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

e.8)	The whole test is repeated, but the frames "off" are replaced by frames "on" with the FACCH flag set.

32.2.5	Test Requirements

1)	In step e.1), the signal energy shall be fairly constant within +/- 3 dB.

2)	In step e.2), the signal energy shall decrease to less than -60 dBm within 17 frames.

3)	In step e.4), comfort noise shall be generated. The same requirements as in step e.1) apply.

4)	In step e.5), the same requirements as in step e.4) apply.

5)	In step e.6), the same requirements as in step e.1) apply. However, the signal energy shall be 12 dB lower.

6)	In step e.7), the signal energy shall be fairly constant within +/- 3 dB for 28 frames. Then the signal energy shall decrease to less than -60 dBm within 16 frames.

7)	In step e.8), the same requirements as in all previous steps apply.

32.3	Uplink speech transcoding

32.3.1	Definition and applicability

Uplink speech transcoding transforms 13 bit linear PCM to the 13 kbit/s net bit stream. This net bit stream is to be channel encoded for transmission on the air interface.

The requirements and this test apply to all MS supporting TCH/FS.

32.3.2	Conformance requirement

The output bit stream from the speech transcoder shall be bit by bit exactly the same as the predefined output sequence (SEQ01.OUT, SEQ02.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT).

TS 06.01, 2; TS 06.10, 5.2/5.2.1

32.3.3	Test purpose

To verify that the speech transcoder on the MS can transform all predefined sequences (SEQ01.INP, SEQ02.INP, SEQ03.INP, SEQ04.INP and SEQ05.INP) at 104 kbit/s (13 bit linear PCM at 8 kHz) level to 13 kbit/s level correctly.

32.3.4	Method of test

32.3.4.1	Initial conditions

DTX is off.

32.3.4.2	Procedure

a)	The SS resets the speech decoder on the MS (see [36.4])

b)	The SS sends a test sequence SEQ01.INP to the MS at 104 kbit/s level via the digital audio interface.

Note:	These files contain 16 bit words for 13 bit linear PCM left justified. See also TS 06.10 table 5.1.

c)	The SS records the 13 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface.

d)	The test is repeated using the test sequences SEQ02.INP, SEQ03.INP, SEQ04.INP and SEQ05.INP.

32.3.5	Test Requirements

The bit stream output shall be bit by bit exactly the same as the sequence given in the files SEQ01.COD, SEQ02.COD, SEQ03.COD, SEQ04.COD and SEQ05.COD.

Note:	These files contain 16 bit words of all the 76 parameters in a speech frame justified as in TS 06.10 table 5.1. 76 codewords shall occur in a frame of 20 ms.

32.4	Uplink transmitter DTX functions

32.4.1	Definition and applicability

The VAD/DTX transmitter functions consist of a Voice Activity Detector (VAD) and a surrounding Discontinuous Transmission (DTX) system introducing additional "speech" traffic frames on the air interface compared to those the VAD itself would classify as speech frames containing real speech. The additional traffic frames on the air are introduced due to:

1)	A "hangover" period at the end of speech bursts in order to be certain that the traffic frames contain only noise and to evaluate the background acoustic noise characteristics when no real speech is present.

2)	Special traffic frames (SID frames) added on the air at regular intervals containing only the evaluated background acoustic noise characteristics. These frames are used for generation of comfort noise in speaker silence periods on the receiving side.

The requirements and this test apply to all MS supporting TCH/FS.

32.4.2	Conformance requirement

The MS VAD and DTX function allows only those frames to be transmitted that are either marked with SP = 1 or that are properly positioned SID-frames.

TS 05.08, 8.3; TS 06.01, 3/4; TS 06.31, 2.1/2.1.1/2.1.2; TS 06.32, 1/2/2.1/2.2.8

32.4.3	Test purpose

To verify that the combination of VAD and DTX operates correctly.

32.4.4	Method of test

32.4.4.1	Initial conditions

A call is set up on a TCH/FS according to the generic call set-up procedure

32.4.4.2	Procedure.

a)	The SS sends a test sequence SPECT_A1.INP of PCM samples, which are grouped into frames of 20 ms synchronized to the TDMA and traffic frame structure on the air interface, on the digital audio interface in the MS at 104 kbits/s (13 bit, 8 kHz linear PCM).

The start of the test sequences is synchronized with the radio transmission on the air interface so that the first traffic frame on the air occurs just after the traffic frame allocated for the SID frame (TDMA frame 56 modulo 104, see TS 05.02 and 05.08).

Note:	8 timeslots in 8 consecutive TCH/FS TDMA frames are seen as one traffic frame and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in TS 05.03.

SEQ0x.INP defined in GSM 06.32.

b)	The SS detects whether or not there is any power transmitted over the radio path on a timeslot basis excluding SACCH frames. The speech frame by speech frame on/off transmission (on = 1) is recorded.

c)	The test is repeated for all test sequences *.INP described in TS 06.32 section 4.

32.4.5	Test Requirements

1)	In step b), the traffic frame on/off sequence recorded shall be bit exact like the sequence of SP flags stored as bit 15 of LAR(2) on the respective reference files *.COD described in TS 06.32, with the following exceptions:

1.1)	The occurrence of a SID frame in its allowed window within the SACCH multiframe as defined in TS 05.08.

1.2)	The occurrence of a SID frame after 1 or more real speech frames consecutively transmitted on the air.

32.5	Speech channel transmission delay

32.5.1	Definition and applicability

The total transmission delay within the various elements of a GSM system are specified as round trip delays. For the MS this would be equivalent to applying an RF equivalent of a speech signal to the MS receiver, closing an acoustic path from the ERP to the MRP, detecting the corresponding RF signal at the MS transmitter output and measuring the time interval between the signal originally fed to the MS receiver and that transmitted by the MS transmitter.

This simple approach cannot be demonstrated to be accurate due to the inherent non linear characteristic of the speech transcoder. The overall delay therefore is split into four identifiable and measurable delays. The delays are respectively:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output.

Each delay is defined and its method of test described in the following sections.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.2	Conformance requirement

The overall speech channel transmission delay shall be less than 144.9 ms.

TS 03.05, 4.2.1

Note:	 Although the delay of 144.9 ms at maximum is not mentioned explicitly, calculations based on the figures indicated in TS 03.05, section 4.2.1 will lead to this delay, when the specific test method used is considered.

32.5.3	Test purpose

To verify that the round trip delay, of a speech channel for a MS, which consists of the sum of:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output

meets the requirements when using the predefined test sequences SEQ01.COD, SEQ03.COD, SEQ04.COD and SEQ05.COD.

32.5.4	Downlink processing delay

32.5.4.1	Definition and applicability

The downlink processing delay is the delay from the first bit of a speech block transmitted from the RF output of the SS up to the last bit of the corresponding speech block received at the DAI on the output of the speech transcoder.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.4.2	Method of test

32.5.4.2.1	Initial conditions

DTX is off.

32.5.4.2.2	Procedure

a)	The test set up is that described in [32.1] for downlink speech transcoding.

b)	The SS transmits one of the test patterns SEQ01.COD, SEQ03.COD, SEQ04.COD or SEQ05.COD to the MS.

c)	The SS measures for each speech block it transmits the time between the first bit at the air interface and the last bit of that speech block on the DAI. This difference is the delay measured.

d)	Step c) is repeated 20 times and the maximum delay measured in ms is the downlink processing delay TDP.

Note:	This is to account for the fact that the processing time may not be constant.

32.5.5	Downlink coding delay

32.5.5.1	Definition and applicability

The downlink coding delay is defined as the delay between the digital representation of an acoustic signal on the DAI and the corresponding acoustic signal at the ERP.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.5.2	Method of test

32.5.5.2.1	Initial conditions

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.5.5.2.2	Procedure

a)	The SS generates on the DAI a digital representation of a sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" ∞1, in the range of 0 to 360 degrees, between the equivalent sine wave generated at the DAI and the sine wave at the input to the artificial ear.

c)	The frequency is increased to 1100 Hz and the resulting phase shift ∞2 noted.

d)	The downlink coding delay TDC is calculated from either:

TDC = (∞2 - ∞1)/36 ms	for ∞2 > ∞1

or

TDC = (∞2 + 360 - ∞1)/36 ms	for ∞2 < ∞1

32.5.6	Uplink processing delay

32.5.6.1	Definition and applicability

The uplink processing delay is the delay from the first bit of a speech block on the DAI to the last bit of that speech block being transmitted on the air interface of the MS.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.6.2	Method of test

32.5.6.2.1	Initial conditions

32.5.6.2.2	Procedure

a)	The test set up is that described in 32.3 for uplink speech transcoding.

b)	The SS sends one of the test patterns SEQ01.INP, SEQ03.INP, SEQ04.INP or SEQ05.INP to the DAI of the MS.

c)	The SS measures the time between the first bit on the DAI, and the last transmitted bit of the block at the air interface for each speech block the SS sends on the DAI. This time difference is the delay measured.

d)	Step c) is repeated 20 times. The maximum delay measured in ms is the uplink coding delay TUP.

Note:	This is to account for the fact that the processing time may not be constant.

32.5.7	Uplink coding delay

32.5.7.1	Definition and applicability

The uplink coding delay is defined as the delay between an acoustic signal at the MRP and the digital representation of that signal on the DAI.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.7.2	Method of test

32.5.7.2.1	Initial conditions

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.5.7.2.2	Procedure

a)	The SS generates an acoustic signal at the artificial mouth of the LRGP, being a pure sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" ∞1, in the range of 0 to 360 degrees, between the signal at the MRP and its digital representation on the DAI.

c)	The SS set the generated frequency to 1100 Hz and measures the resulting phase shift ∞2.

d)	The uplink coding delay TUC is calculated from either:

TDC = (∞2 - ∞1)/36 ms	for ∞2 > ∞1

or

TDC = (∞2 + 360 - ∞1)/36 ms	for ∞2 < ∞1

32.5.8	Test Requirement

The sum of the dealys {TDP + TDC + TUP + TUC} shall be less than 144.9 ms.

Note 1:	This limit includes an allowance of 4*0.25 ms delay from the DAI to the MS transmission path.

Note 2:	No allowances have been made for any delays within the measurement system. These must either be calibrated out or subtracted from the individual delays before performing the sum above.