26.6	Test of the elementary procedures for radio resource management

Note:	For SS implementor : if tests are concatenated, it is important that unused fields in IMMEDIATE ASSIGNMENT REJECT messages do not use Request References that relate to CHANNEL REQUEST messages recently transmitted by the MS.

26.6.1	Immediate Assignment

The immediate assignment procedure is used by the network to establish a dedicated control channel for the MS and network to communicate the detail of the service requested. If the Mobile Station does not implement the procedure correctly, radio resources can be wasted as the Mobile Station might use the wrong channels.

26.6.1.1	Immediate Assignment / SDDCH or TCH assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.3 in phase 1 version.

-	Timing Advance is no longer specified]

Purpose:

To verify that the MS can correctly set up a dedicated SDCCH control channel and that the MS can correctly set up a dedicated TCH/FACCH control channel.

Conformance Requirement:

1.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the SDCCH/8 described in the IMMEDIATE ASSIGNMENT message.

2.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the TCH/FACCH described in the IMMEDIATE ASSIGNMENT message.

Reference(s):

GSM 04.08 section 3.3.1.2.1

Related PICS/PIXIT Statement(s):

Support rate(s) of TCH: TCH/F and/or TCH/H.

Initial Conditions:

System Simulator:

1 cell, except that CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

If TCH/F is supported by the MS, the test is repeated with the SS assigning a TCH/F.

If TCH/H is supported by the MS, the test is repeated with the SS assigning a TCH/H.

Maximum Duration of Test:

6 seconds per value of the execution timer

Expected Sequence:

This sequence is performed for execution counter, K = 1, 2, 3 (unless the TCH is not supported).

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below

4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel

5	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

IMMEDIATE ASSIGNMENT

K=1, SDCCH test : Channel Type = SDCCH/8

K=2, TCH/F test : Channel Type = Bm + ACCHs

K=3, TCH/H test : Channel Type = Lm + ACCHs, subchannel randomly drawn

26.6.1.2	Immediate Assignment / extended assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.5 in phase 1 version.

-	two procedures have been concatenated to one test.

-	in the last part of the test the MS shall continue to send CHANNEL REQUESTs]

-

]

Note:	In these tests the SS must send the immediate assignment messages in due time to allow for the MS to receive them and send a PAGING RESPONSE rather than another random access. This applies to the whole of section 26.

Purpose:

To verify that the MS can correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

To verify that the MS only reacts to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

Conformance Requirement:

1:	The MS shall go to the allocated SDCCH/4 and send a PAGING RESPONSE message containing its identity and its classmark.

2:	The MS shall go to the allocated SDCCH/8 and send a PAGING RESPONSE message containing its identity and its classmark.

3:	Same as test purpose

4:	Same as test purpose

Reference(s):

GSM 04.08 section 3.3.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

1 cell, Max-retrans is set to 7.

Mobile Station:

The MS is in the "idle, updated" state. with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with a TMSI allocated.

Method of Test:

In the first part of the test, the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the n-th CHANNEL REQUEST message (n being randomly chosen by the SS from the set {1, 2 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message, which references one of the last 3 CHANNEL REQUEST messages from the MS. The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

In the second part of the test, the SS again pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the k-th CHANNEL REQUEST message (k being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, instead of referencing one of the last 3 CHANNEL REQUEST messages from the MS, references an earlier CHANNEL REQUEST message. The MS shall then ignore the IMMEDIATE ASSIGNMENT EXTENDED message and continue to send CHANNEL REQUEST messages until the Max-Retrans value has been reached. Then a period of 7 seconds shall elapse in order to allow the MS to perform cell reselection (this allows for the time between the last CHANNEL REQUEST message and the beginning of cell reselection).

In the third part of the test, the CCCH_CONF of the SS is set to non-combined and the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the r-th CHANNEL REQUEST message (r being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, in the second request reference, references one of the last 3 CHANNEL REQUEST messages from the MS. The associated Channel Description allocates SDCCH(S) (S being randomly chosen by the SS from the set {0,1 ... 7}). The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

Maximum Duration of Test:

90 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being

randomly drawn from {1.... 8}

are sent, all with Establ. Cause =

"Answer to paging"

3	MS -> SS	CHANNEL REQUEST

4	SS -> MS	IMM ASSIGN EXT 	see note 1

5	MS -> SS	PAGING RESPONSE

6	SS -> MS	CHANNEL RELEASE

7	--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	k CHANNEL REQUESTs (k being

randomly drawn from the set {4, 5,

8}) are sent all with

10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

11	SS -> MS	IMM ASSIGN EXT	see note 2

12	MS -> SS	CHANNEL REQUEST	8-k CHANNEL REQUESTs are sent, all

with Establ. Cause = "Answer to

13	MS -> SS	CHANNEL REQUEST	paging"

14	SS		The SS verifies that the MS does not

transmit any Layer 2 frames for at least

3 seconds.

15	 SS		The SS sets CCCH_CONF to

non-combined

16	 SS		The SS waits 40 seconds to

allow the MS to perform

cell reselection and to

read the BCCH information

17	SS -> MS	PAG REQ TYPE 1

18	MS -> SS	CHANNEL REQUEST	r CHANNEL REQUESTs (r being

randomly drawn from {4, 5... 8}

are sent, all with Establ. Cause =

"Answer to paging"

19	MS -> SS	CHANNEL REQUEST

20	SS -> MS	IMM ASSIGN EXT 	see note 3

21	MS -> SS	PAGING RESPONSE

22	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

Note 1 : 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {max (1,n-2) ... n}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

Note 2 : 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {1 ... k-3}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

Note 3 : 	The second Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {r-2, r-1, r}, its value being randomly drawn by the SS. The first Request Reference shall be different from any Request Reference the MS has generated in this test.

26.6.1.3	Immediate Assignment / assignment rejection

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.6 in phase 1 version.

-	two procedures have been concatenated to one test.

-	there is an IMMEDIATE ASSIGNMENT REJECT added at the end of the test to avoid a cell reselection.]

Purpose:

To verify that the MS can accept an Immediate Assignment Rejection.

To verify that the MS can respond to paging  after an Immediate Assignment Rejection is received on a different cell.

Conformance Requirement:

1:	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging". After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit during the time indicated in the "Wait Indication" field of the IMMEDIATE ASSIGNMENT REJECT message, and then it shall answer to the new paging requests.

2:	After an assignment rejection, the MS shall perform a cell reselection (idle mode operation) and the MS shall not transmit unless a different cell is selected.

Reference(s):

GSM 04.08, section 3.3.1.2.2.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

2 cells with the same LAI, Max-Retrans is 7.

Mobile Station:

The MS is camped on cell A and is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

The MS is camped on cell B and is in the "idle, updated" state, with a TMSI allocated.

Method of Test:

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the n-th CHANNEL REQUEST (n being an integer from the set {1, 2 ... 8}, randomly chosen by the SS) the SS sends an IMMEDIATE ASSIGNMENT REJECT message, which references one of the last 3 CHANNEL REQUESTs from the MS, and with the Wait Indication set to x seconds (x being an integer from the set {5, 6 ... 255}, randomly chosen by the SS). The SS continues to send paging messages for that mobile station in every block of the mobile station's paging subgroup for x+2 seconds. The MS shall only answer with a CHANNEL REQUEST after x seconds have elapsed.

The SS responds to this CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message with the Wait Indication set to 255 seconds.

Immediately afterwards the SS changes the power levels so the MS selects cell B. After 12 seconds have elapsed the SS pages the MS in cell B and the MS shall answer to this page. In order to avoid another cell reselection the SS then sends another IMMEDIATE ASSIGNMENT REJECT.

Maximum Duration of Test:

5 minutes.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being

.	.	.	randomly drawn from the set {1, 2

.	.	.	 ... 8}) are sent, all with

1+n	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

2+n	SS -> MS	IMM ASSIGN REJ	1st, 3rd and 4th Request

References are different to all

n Request References received from

the MS under test.

2nd Request Reference : see note 1.

2nd Wait Indication = x seconds (x

being randomly drawn from the set

{5 ,6 ... 255}

3+n	SS -> MS	PAG REQ TYPE 1	Sent for more than x seconds

.	.	.

.	.	.

k	SS -> MS	PAG REQ TYPE 1	between x and x+2 seconds (note 2)

k+1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

k+2	SS -> MS	IMM ASSIGN REJ	1st, 2nd and 4th Request

References are different to all

n Request References received from

the MS.

The 3rd Request Reference pertains

to the last CHANNEL REQUEST

sent by the MS.

The 3rd Wait Indication is 255

seconds

k+3	--------	----------	Raise power level of cell B, lower

power level of cell A until the MS

selects cell B

k+4	SS -> MS	PAG REQ TYPE 1	Sent once, 12 seconds after the change

of levels

k+5	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

k+6	SS -> MS	IMM ASSIGN REJ

Note 1 :	The Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer from the set {max(1,n-2) ... n}, its value being randomly drawn by the SS.

Note 2 :	the value of k is not important in this test.

Specific Message Contents:

None.

26.6.1.5	Immediate Assignment / ignore assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.7 in phase 1 version.

-	two tests have been concatenated to one.

(* it is not clear that there were originally two tests : it is possible to do as one test. How have R+S implemented the FTA? *)

-	an IMMEDIATE ASSIGNMENT REJECT has been added to the end of the test in order to avoid a cell reselection

-	Timing Advance is no longer included]

Purpose:

To verify that the MS ignores an assignment for another MS while it is waiting for an assignment of its own.

Conformance Requirement:

1.	An MS waiting for a response from the network, following the sending of a CHANNEL REQUEST, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong frame number.

2.	An MS is waiting for an assignment of its own, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong random access information.

Reference(s):

GSM 04.08 section 3.3.1.2.1

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, max_retrans = 7

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

[The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a wrong Request Reference (in the first run of the test the frame number is wrong, in the repetition it is the random access info that is wrong). It is verified for 2 seconds that the MS does not start signalling on the SDCCH. The MS shall ignore the assignment and send another CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT and repeats the test once.

Note:	The test can be repeated before the 2 seconds have passed. In this case the SS shall continue to monitor the SDCCH assigned in step 3 while at the same time try to establish a new SDCCH.]

The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a slightly different Request Reference (only the frame number is wrong). This IMMEDIATE ASSIGNMENT shall be ignored by the MS. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a slightly different Request Reference (only the random access information is wrong). This IMMEDIATE ASSIGNMENT shall be ignored by the MS. The MS sends a third CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT. It is checked throughout the test that the MS does not start signalling on any SDCCH.

Maximum Duration of Test:

6 seconds.

Expected Sequence:

(*

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Frame number in Request Reference

is 2 too high. The MS shall not

start signalling on the assigned

SDCCH. This is verified for a

period of 2 seconds

4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

5	SS -> MS	IMM ASSIGN REJ

6	SS -> MS	PAG REQ TYPE 1

7	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

8	SS -> MS	IMM ASSIGNMENT	Random access info in Request

Reference is wrong. The MS shall

not start signalling on the

assigned SDCCH. This is verified

for a period of 2 seconds

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGN REJ

*)

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to

paging", Random Reference = X,

sent in frame number, FN

3	SS -> MS	IMM ASSIGNMENT	The Random Access Information is

that included in the CHANNEL

REQUEST message but the frame

number is FN+2. The MS shall not

start signalling on any SDCCH.

4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to

paging", Random Reference = X,

sent in frame number, FN

5	SS -> MS	IMM ASSIGNMENT	The frame number is FN but the

Random Access Information in

the Request Reference is not that

which was sent in the CHANNEL

REQUEST. The MS shall not

start signalling on any SDCCH.

6	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

7	SS -> MS	IMM ASSIGN REJ	Note

(* may need to allow for more channel requests : must also change the default value of max retrans *)

Note:	throughout the test, and for at least 2 seconds after step 5, it shall be checked that the MS does not signal on any SDCCH.

Specific Message Contents:

None.

26.6.2	Test of paging

The Paging procedure is used by the network to cause the Mobile Station to establish a radio connection. Normally the Mobile Station listens to its paging subchannel, but this can be modified by the use of different page modes. The correct implementation of the paging procedure in the Mobile Station is essential for the basic establishment of a connection.

26.6.2.1	Normal paging

26.6.2.1.1	Paging / normal / type 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.3 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

]

(* step 5 has been changed for phase 2 imsi response to imsi paging.*)

(* steps 25 and 26 may be deleted as ignoring IMEI pages is difficult to 'justify'.*)

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 1 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 1 message. It is tested that the MS ignores addressing with IMEI and with a faulty address.

Conformance Requirement:

1.	The MS shall respond correctly to various PAGING REQUEST TYPE 1 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed with its IMSI in the first Mobile Identity field. The optional Mobile Identity field is not present.

1.2.	The MS is addressed with its TMSI in the first Mobile Identity field. The optional Mobile Identity field specifies an IMSI different from that of the MS.

1.3.	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field addresses the MS by its IMSI.

1.4	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field contains the correct TMSI of the MS.

2.	An MS shall ignore PAGING REQUEST TYPE 1 messages with incorrect information, when the page mode is set to normal paging, in the following cases:

2.1.	The MS is addressed with its IMEI in the first Mobile Identity field. The optional Mobile Identity field is not present.

2.2	The MS is addressed with its TMSI in the first Mobile Identity field, but the type of identity in this field is set to "No Identity". The optional Mobile Identity field is not present.

Reference(s):

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 6 times with different PAGING REQUEST TYPE 1 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last 2 cases, it is tested that the MS does not answer to paging that does not address it or that uses its IMEI.

Maximum Duration of Test:

2 min

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of

MS, 2nd Mobile Ident not present.

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

MS, 2nd Mobile Ident contains IMSI

of another MS.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

another MS, 2nd Mobile Ident

contains IMSI of MS.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

another MS, 2nd Mobile Ident

contains TMSI of MS.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

24	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

25	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMEI of

MS, 2nd Mobile Ident not present.

26	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

27	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

MS but with type of identity set

to "no identity", second Mobile

Ident not present.

28	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

(* In steps 26 and 28, it may be sufficient to check for no layer 3 messages rather than checking for no RF-transmission.*)

Specific Message Contents:

None.

26.6.2.1.2	Paging / normal / type 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.5 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

(* imsi response to imsi paging is included.*)

(* imei paging test may not be 'justified' *)

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 2 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 2 message. It is tested that the MS ignores addressing with IMEI and faulty addressing.

Conformance Requirement:

1.	The MS shall respond correctly (by sending CHANNEL REQUEST messages with an Establishment Cause set to "Answer to Paging") to various PAGING REQUEST TYPE 2 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed in the first TMSI field.

1.2	The MS is addressed in the second TMSI field.

1.3	The MS is addressed in the optional Mobile Identity field with its TMSI.

1.4	The MS is addressed in the optional Mobile Identity field with its IMSI.

2.	The MS shall ignore PAGING REQUEST TYPE 2 messages with incorrect information, when the page mode is set to normal paging, in the following cases:

2.1	The MS is addressed in the optional Mobile Identity field with its IMEI.

2.2	The MS is addressed in the optional Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity".

Reference(s):

GSM 04.08 section 3.3.2.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 6 times with different PAGING REQUEST TYPE 2 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or by its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last 2 cases, it is tested that the MS does not answer to paging that does not address it or that uses its IMEI.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses MS, 2nd TMSI

addresses another MS, Mobile

Identity IE not present

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses MS, Mobile Identity

IE not present.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains TMSI of MS.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains IMSI of MS.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

24	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

25	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains IMEI of MS.

26	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

27	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains TMSI of MS

but with type of identity set to

"no identity".

28	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

(* RF transmission ==> L3 messages ?*)

Specific Message Contents:

None.

26.6.2.1.3	Paging / normal / type 3

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.7 in phase 1 version.

-	4 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 3 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested.

Conformance Requirement:

An MS shall respond correctly to various PAGING REQUEST TYPE 3 messages, when the page mode is set to normal paging. The MS shall send CHANNEL REQUEST messages, with an Establishment Cause set to "Answer to Paging", until the network answers. The number of CHANNEL REQUEST messages shall be limited by the parameter Max-retrans. After the assignment procedure, the MS shall send a PAGING RESPONSE message on the channel assigned by the network.

Reference(s):

GSM 04.08 section 3.3.2.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 4 times with different PAGING REQUEST TYPE 3 messages on the paging subchannel which corresponds to the MS's IMSI.

In all the cases the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 3	1st TMSI addresses MS; 2nd, 3rd

and 4th TMSIs address other MSs.

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 3	2nd TMSI addresses MS; 1st, 3rd

and 4th TMSIs address other MSs.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 3	3rd TMSI addresses MS; 1st, 2nd

and 4th TMSIs address other MSs.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 3	4th TMSI addresses MS; 1st, 2nd

and 3rd TMSIs address other MSs.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

24	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.2.2	Paging / extended

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	two procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

-	Instead of specifying a specific page mode , the mode is now chosen arbitrarily.*)

-	It is specified that in step 6 the random reference is not one used by the MS.]

Purpose:

To test that the MS is operating in the extended page mode when this is ordered by the SS in either a PAGING REQUEST message or an IMMEDIATE ASSIGNMENT message.

Conformance Requirement:

1.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

2.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT message on the paging subchannel which corresponds to the MS's identity.

Reference(s):

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends a PAGING REQUEST TYPE 1 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT on the paging subchannel which corresponds to the MS's identity. The random reference is different to those used by the Mobile Station in the last two CHANNEL REQUEST messages. (Phase 2 requires a Mobile Station to react on an IMMEDIATE ASSIGNMENT after a rejection.) The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

[editorial note : If it is an essential feature that the MS ignores paging on other subchannels, later, a test shall be elaborated, which tests that the MS does not take into account the paging reorganisation (only page modes on the MS's own paging subchannel shall be taken into account). However it seems not reasonable to forbid the MS listening to e.g. all subchannels.]

Maximum Duration of Test:

10 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	Mobile Ident : IMSI of another MS,

page mode = "extended paging"

2	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging

subblock. Page mode

is arbitrarily chosen

Mobile Ident : TMSI of the MS.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds

6	SS -> MS	IMM ASSIGNMENT	Sent in the paging subblock of

MS under test. Page mode =

"extended paging", Request

reference chosen arbitrarily by

the SS, but different from all

references used earlier in this

test sequence

7	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging

subblock. Page mode is arbitrarily

chosen. Mobile Ident : TMSI of

the MS.

8	MS -> SS	CHANNEL REQUEST

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.2.2.1	Paging / reorganisation / procedure 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	the paging parameters are chosen   arbitrarily by the SS

-	type of identity in the first PAGING REQUEST TYPE 1 is no longer specified as IMSI.]

(* half of this test can probably be deleted -- however as these are the 2nd and 4th quarters it would disrupt the SS !!! *)

Purpose:

To test that the MS correctly determines its new paging subchannel when the CCCH structure is changed from non-combined to combined.

Conformance Requirement:

1.	An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent on its old CCCH in paging blocks which do not belong to the MS's paging sub-channel.

2.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "1 basic physical channel used for CCCH combined with SDCCH", and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

3.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "2 basic physical channels used for CCCH, not combined with SDCCHs" and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

Reference(s):

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS, with the exceptions that the CCCH-CONF shall not be set to combined and that BS-PA-MFRMS shall not be set to 9..

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined/default range that ensures its paging channel changes when the broadcast parameters are changed.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED message on the MS's paging subchannel, with the page mode element set to "paging reorganisation" and Request References that do not pertain to the MS. Before the MS's original paging subchannel re-occurs, the SS pages it on the CCCH corresponding to the Mobile Station's IMSI with a PAGING REQUEST TYPE 2 message (page mode = normal paging) containing the MS's TMSI in some paging block which does not belong to the Mobile Station's paging subchannel. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) on an arbitrarily selected paging subchannel.

Then the SS starts sending messages with page mode set to "paging reorganization" on all paging subchannels.

The SS pages the MS with its TMSI on an arbitrarily selected paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

The SS changes the paging parameters.

Then the SS sets the page mode to "normal paging".

The SS then waits for the duration of five 51-TDMA multiframes (4 to allow the MS to read all the system information type 1, 2, 3, and 4 messages on the BCCH, and one to calculate the new paging group). Then the MS is paged with a PAGING REQUEST TYPE 1 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Then the MS is paged with a PAGING REQUEST TYPE 2 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

10 sec

Expected Sequence:

This sequence is performed for execution counter, K =1, 2.

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Sent on the MS's paging

channel. Page mode set to "paging

reorganisation". Request Reference

not pertaining to the MS.

2	SS -> MS	PAG REQ TYPE 2	Sent before the MS's original

paging subchannel re-occurs, but

later than the next paging

block of that CCCH.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

6	--------	--------	All L3 messages sent on any paging

subchannel are paging fill

frames specify "paging re

organization.

7	SS -> MS	PAG REQ TYPE 2	Sent on an arbitrarily

selected paging subchannel

Page mode "paging reorganisation"

8	MS -> SS	CHANNEL REQUEST

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

11			Change of paging

parameters in SYS INFO 3 and 4 as

described below for K=1, 2

12			The SS waits until it has

sent all system information

messages (page mode is still

paging reorganisation).

13	--------	--------	All L3 messages sent on any paging

subchannel specify "normal paging

14	--------	--------	Wait 3 seconds

15	SS -> MS	PAG REQ TYPE 1	Sent on the new paging subchannel							of the MS.

16	MS -> SS	CHANNEL REQUEST

17	MS -> SS	CHANNEL REQUEST

18	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

19	SS -> MS	PAG REQ TYPE 2	Sent on the new paging subchannel					of the MS.

20	MS -> SS	CHANNEL REQUEST

21	MS -> SS	CHANNEL REQUEST

22	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

For execution counter K :

K=1:

SYSTEM INFORMATION TYPEs 3 and 4 shall have the Control Channel Description IE changed to:

CCCH-CONF	"1 basic physical channel used for CCCH, combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

K=2:

SYSTEM INFORMATION TYPEs 3 and 4 shall have the Control Channel Description IE changed to:

CCCH-CONF	"2 basic physical channel used for CCCH, not combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

26.6.2.2.2	Paging / reorganisation / procedure 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	It is stated clearly that the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS is operating in the "paging reorganisation" page mode when this is ordered by the SS and the MS is paged in its former access grant channel.

Conformance requirement

An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent in a former Access Grant block.

Reference(s):

GSM 04.08 section 3.3.2.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 1, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined\default range that ensures its paging channel changes when the broadcast parameters are changed.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED on the MS's paging subchannel, with the page mode element set to "paging reorganisation". The MS is then paged immediately in a former Access Grant block with a PAGING REQUEST TYPE 2 message. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

5 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Page mode set to "paging

reorganisation"

2	SS -> MS	PAG REQ TYPE 2	Sent in a former access grant

block.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.2.3	Paging / same as before

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS remembers the page mode from the previous paging request message.

Conformance Requirement:

An MS, after first receiving a message with page mode set to "extended paging" and then another message with page mode set to "same as before", shall remember the page mode from the previous message and answer to paging messages in the next but one paging sub block.

Reference(s):

GSM 04.08 section 3.3.2.1;  GSM 05.02, section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT REJECT on the MS's paging subchannel, with the page mode element set to "extended paging". In the next but one subblock on the same CCCH, nothing addresses the MS. When the MS's specific paging subchannel reoccurs, a PAGING REQUEST TYPE 3 is sent, not addressing the MS under test and with page mode set to "same as before". In the next but one subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

10 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN REJ	Page mode set to "extended paging"

2	SS -> MS	XXXX	In the next but one subblock

nothing addresses the MS

3	SS -> MS	PAG REQ TYPE 3	This is sent in the next paging

subblock on the MS's specific

paging subchannel. The page mode

is set to "same as before", and the MS

under test is not addressed.

4	SS -> MS	PAG REQ TYPE 1	The MS is addressed in this 'next

but one subblock'. Page mode set

to "paging reorganisation"

5	MS -> SS	CHANNEL REQUEST

6	MS -> SS	CHANNEL REQUEST

7	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.3	Test of measurement report

When an RR-connection exists, the MS shall send measurement reports. These reports contain reception characteristics from serving and neighbouring cells. The measurement report procedure is described in section 3.4.1.2 of 04.08.

Note:	The capability to calculate RxLev and RxQual is tested in sections 15 and 16. In this test only the signalling aspect is verified.

[ previous Editor's note : In phase 2, the MS shall be provoked to return the "Neighbour cell information not available" value in the NO-NCELL-M field. It should also be considered whether to use different RF-levels and ARFCN's in the 4 tests]

[editorial note : this section has not been corrected since the last signalling meeting ]

26.6.3.1	Measurement / no neighbours

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

Purpose:

To test that, when the SS gives absolutely no information about neighbouring cells, the MS does not report on neighbouring cells.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH block and the measurement valid indication shall be set to valid (0) within the second block at the latest.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell Identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

[Editor's note : is this the same as "a call in progress" ?]

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. The BA is indicated as empty. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence number	1

BCCH Allocation ARFCN		No channels belong to the BCCH

allocation

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier			0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available

for serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

[Editor's note : is the term "No result" unambigous ? - now coded as sent]

Note 1:	The actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable -- once the MS is in U10 then this can be checked immediately.]

26.6.3.2	Measurement / all neighbours present

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

Purpose:

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS and the place of each BCCH measurement in the message shall be in accordance with the increasing RXLEVs measured for each BCCH.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. All 8 of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained.

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence		1

BCCH Allocation ARFCN		Only channel numbers 2, 8, 14, 20, 26, 32,

38, and 44 belong to the BCCH allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier			0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				6 neighbour cell measurement results

RXLEV-NCELL-1			See note 3

BCCH-FREQ-NCELL-1			Corresponds to N7

BSIC-NCELL-1				Corresponds to N7

RXLEV-NCELL-2			See note 3

BCCH-FREQ-NCELL-2			Corresponds to N6

BSIC-NCELL-2				Corresponds to N6

RXLEV-NCELL-3			See note 3

BCCH-FREQ-NCELL-3			Corresponds to N5

BSIC-NCELL-3				Corresponds to N5

RXLEV-NCELL-4			See note 3

BCCH-FREQ-NCELL-4			Corresponds to N4

BSIC-NCELL-4				Corresponds to N4

RXLEV-NCELL-5			See note 3

BCCH-FREQ-NCELL-5			Corresponds to S1

BSIC-NCELL-5				Corresponds to S1

RXLEV-NCELL-6			See note 3

BCCH-FREQ-NCELL-6			Corresponds to N3

BSIC-NCELL-6				Corresponds to N3

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

Note 3:	The actual RXLEV-values are not checked, but the order of the cells shall be as described above.

26.6.3.3	Measurement / barred cells and non-permitted NCCs

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	one test has been splitted into 4]

Purpose:

To test that, when a combination of normal neighbours, barred cells and non-permitted NCCs is "on air", the MS reports only on normal neighbours.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORTs on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the messages shall contain measurement results only for the BCCH carriers among those for which the MS is allowed to report (in this case 4 BCCHs) and the place of each BCCH measurement in the message shall be in accordance with the increasing RXLEVs measured for each BCCH.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	2	1	020	0004H

Neighbour, N4	-55	3	3	026	0005H

Neighbour, N5	-50	4	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Note:	The BA sent in SYSTEM INFORMATION TYPE 5 does not include N1, N4 and N5. N1 may be the case of a barred cell, N3 simulates the case where another operator is transmitting on the same frequency (e.g. in border areas), N4 & N5 simulate the case where other operators are transmitting on other frequencies.

Mobile Station:

The MS is in the active state of a call (U10).

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. 5 of the 8 BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 4 strongest permitted carriers have been obtained (one of the carriers in the BA belongs to a non-permitted NCC).

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence		1

BCCH Allocation ARFCN		only channel numbers 2, 14, 20, 38, and

44 belong to the BCCH allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier			0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				4 neighbour cell measurement results

RXLEV-NCELL-1			See note 3

BCCH-FREQ-NCELL-1			Corresponds to N7

BSIC-NCELL-1				Corresponds to N7

RXLEV-NCELL-2			See note 3

BCCH-FREQ-NCELL-2			Corresponds to N6

BSIC-NCELL-2				Corresponds to N6

RXLEV-NCELL-3			See note 3

BCCH-FREQ-NCELL-3			Corresponds to S1

BSIC-NCELL-3				Corresponds to S1

RXLEV-NCELL-4			See note 3

BCCH-FREQ-NCELL-4			Corresponds to N2

BSIC-NCELL-4				Corresponds to N2

RXLEV-NCELL-5			00 0000

BCCH-FREQ-NCELL-5			 0 0000

BSIC-NCELL-5				00 0000

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

Note 3:	The actual RXLEV-values are not checked, but the order of the cells shall be as described above.

26.6.3.4	Measurement / DTX

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test have been splitted into 4

-	Handover is no more specified]

Purpose:

To test that, in the case of the MS using DTX and the SS indicating that power control is in use, the MS reports appropriate results.

Conformance Requirement:

After the sending of the HANDOVER COMPLETE, the MS shall continuously send measurement reports in every SACCH blocks, the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS and the place of each BCCH measurement in the message shall be in accordance with the increasing RXLEVs measured for each BCCH. Further, in a quiet environment, the DTX-USED field shall be set by the MS to "DTX used".

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Support for transparent data services only: yes/no.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

In the serving cell, the DTX indicator is set to "MS shall use discontinous transmission"

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

The MS has just completed a handover into the serving cell, S1.

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress on an arbitrary cell, the MS is handed over to cell S1. On cell S1, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH with all 8 of the BCCHs "on air" indicated in the BA. Cell S1 also indicates that DTX shall be used. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained and that DTX has been used. (The MS is positioned in an environment free from acoustic noise.)

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Neighbour Cells Description

- Format Identifier	bit map 0

- BCCH Allocation Sequence	1

- BCCH Allocation ARFCN	only channel numbers 2, 8, 14, 20, 26, 32,

38, and 44 belong to the BCCH allocation.

SYSTEM INFORMATION TYPE 6:

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall use DTX

- Radio-Link-Timeout	8

MEASUREMENT REPORT:

Protocol Discriminator	RR Management

Transaction Identifier	0000

Message Type		MEASUREMENT REPORT

Measurement Results

- BA-used		0

- DTX-used		DTX was used (note 4)

- RXLEV-FULL-SERVING-CELL	See note 1

- RXLEV-SUB-SERVING-CELL	See note 1

- MEAS-VALID		See note 2

- RXQUAL-FULL-SERVING-CELL	See note 1

- RXQUAL-SUB-SERVING-CELL	See note 1

- NO-NCELL-M		6 neighbour cell measurement results

- RXLEV-NCELL-1	See note 3

- BCCH-FREQ-NCELL-1	Corresponds to N7

- BSIC-NCELL-1		Corresponds to N7

- RXLEV-NCELL-2	See note 3

- BCCH-FREQ-NCELL-2	Corresponds to N6

- BSIC-NCELL-2		Corresponds to N6

- RXLEV-NCELL-3	See note 3

- BCCH-FREQ-NCELL-3	Corresponds to N5

- BSIC-NCELL-3		Corresponds to N5

- RXLEV-NCELL-4	See note 3

- BCCH-FREQ-NCELL-4	Corresponds to N4

- BSIC-NCELL-4		Corresponds to N4

- RXLEV-NCELL-5	See note 3

- BCCH-FREQ-NCELL-5	Corresponds to S1

- BSIC-NCELL-5		Corresponds to S1

- RXLEV-NCELL-6	See note 3

- BCCH-FREQ-NCELL-6	Corresponds to N3

- BSIC-NCELL-6		Corresponds to N3

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block after the HANDOVER COMPLETE message at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

Note 3:	The actual RXLEV-values are not checked, but the order of the cells shall be as described above.

Note 4:	For an MS that only supports transparent data services, the value of DTX-used shall not be checked.

(* I assume as there is no voice input to the MS, DTX has to be used for voice mobiles -- this value might be different if the MS is tested in a noisy environment. *)

26.6.4	Test of dedicated channel assignment

An intracell change of channel can be requested by upper layers in order to change the channel type, or it may be initiated by the RR-sublayer, e.g. for an intra cell handover. This change is performed using the dedicated channel assignment procedure.

The purpose of the dedicated channel assignment procedure is to completely modify the physical channel configuration of the MS while staying in the same cell.

This is described in section 3.4.3 of GSM 04.08.

[note : the phase 2 CR from France Telecom is not yet included]

26.6.4.1	Dedicated assignment / Full rate

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.4.2 in phase 1 version

-	the test have been divided into 2 (full & half rate and full rate only

-	N(SD) is used instead of V(SD)]

Purpose:

To test that the MS can seize the channel defined in the ASSIGNMENT COMMAND.

Conformance Requirement:

1.	Upon receipt of an ASSIGNMENT COMMAND for the transitions listed below the MS shall switch to the channel defined in the ASSIGNMENT COMMAND, establish the link and send an ASSIGNMENT COMPLETE message.

1.1	from SDCCH to TCH/F

1.2	from TCH/F to TCH/F.

2.	An MS shall not increment N(SD) when repeating a message, after completion of the assignment procedure.

3.	The MS shall take into account any Starting Time information element in the ASSIGNMENT COMMAND message.

4.	The MS shall establish the link with the power level specified in the ASSIGNMENT COMMAND message.

5.	The MS shall switch to the channel defined in the ASSIGNMENT COMMAND, establish the link and send an ASSIGNMENT COMPLETE message when frequency hopping is used on the allocated channel and not on the previous one and reciprocally.

Reference(s):

GSM 04.08 section 3.4.3.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

The supported channel mode(s) need to be declared.

(*The frequency band that is supported needs to be supplied *)

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS and allocates an SDCCH. Then 2 different channels are assigned with ASSIGNMENT COMMANDs. Each time the MS shall switch to the assigned channel, establish the link with the power level specified in the ASSIGNMENT COMMAND message and send an ASSIGNMENT COMPLETE message.

Then the SS sends a AUTHENTICATION REQUEST message. The MS shall answer with an AUTHENTICATION RESPONSE message, which is not acknowledged on L2 by the SS. Immediately after the AUTHENTICATION RESPONSE message is received, the SS sends an ASSIGNMENT COMMAND. The MS shall switch to the assigned channel, establish the link with the commanded power level and send as ASSIGNMENT COMPLETE message. Then MS shall repeat the AUTHENTICATION RESPONSE message, with the same N(SD) value.

Finally, the SS sends an ASSIGNMENT COMMAND, which includes a Starting Time IE. The MS shall react as specified above, but this shall be done at the time specified in Starting Time IE.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGN	Channel Type = SDCCH/8, FH is not

used, Timeslot Number = n.

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/FS, FH is used,

Timeslot Number = (n+1) mod 8

6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel, with

the requested power level in the

layer 1 header.

7	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/FS, FH is not

used, Timeslot Number = (n+3)mod 8

8	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel, with

the requested power level in the

layer 1 header.

9	SS -> MS	AUTHENT REQUEST

10	MS -> SS	AUTHENT RESPONSE	This message is not L2

acknowledged by the SS

11	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/FS, FH is used,

Timeslot Number = (n+4) mod 8

12	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel, with

the requested power level in the

layer 1 header.

13	MS -> SS	AUTHENT RESPONSE	N(SD) shall be the same as in

step 10

14	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/FS, FH is not

used, Timeslot Number = (n+5) mod 8,

Starting Time = current frame

number + 100 frames

15	MS -> SS	SABM	Shall be sent in frame number =

starting time and on the correct channel

16	SS -> MS	UA

17	MS -> SS	ASSIGN COMPLETE

18	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	As specified in table

- Timeslot Number	As specified in table

- Training Sequence Code	Chosen randomly by the SS

- Hopping	As specified in table

- ARFCN	Within the range supported by the

MS

Power Command

- Power level	Chosen randomly by the SS but with a changed value each time.

Channel Mode

- Mode	Arbitrarily selected from the full

rate capabilities declared for the

MS

Mobile Allocation	If present, chosen randomly by the testhouse

Starting Time	Only used for step 14

26.6.4.2	Dedicated assignment / failure

[ dissimilarities to 11.10 phase 1 - editorial notes :

Purpose:

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel with the previous power level used on the old channel.

Conformance Requirement:

The MS shall correctly apply the assignment failure procedure.

Reference(s):

GSM 04.08 section 3.4.3.3.

Related PICS/PIXIT Statement(s):

Support for TCH/F.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the active state (U10) of a mobile terminated call.

Foreseen Final State of the MS:

The active state (U10) of a mobile terminated call.

Method of Test:

The MS is in the active state (U10) of a mobile terminated call. The SS sends an ASSIGNMENT COMMAND allocating a new TCH/F, but does not activate the new channel. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel with the same power as used before, reconnect the TCH and trigger the establishment of the main signalling link. Then the MS shall send an ASSIGNMENT FAILURE.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, power

command different to that in the

layer 1 header.

The MS attempts (and fails) to establish a

signalling link on the new channel.

2			The MS reestablishes the signalling link on

the old channel.

3	MS -> SS	ASSIGN FAILURE	RR cause value ="Abnormal release,

unspecified", "Abnormal release,

channel unacceptable", "Abnormal

release, timer expired" or

"Abnormal release, no activity on

the radio path". Layer 1 header

has the same power level as that

given in step 1.

Specific Message Contents:

None

26.6.5	Test of handover

With the Handover procedure, it is possible to completely alter the channels allocated to a MS. This makes it possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network, with the MS in a dedicated mode.

Sections 26.6.5.1 - 26.6.5.4 contain generic test procedures to be used for executing successfull Handover tests. Table 1 contains a list of the different combinations of parameters which have to be tested, together with a reference to the appropriate generic test procedure. If a test uses a channel rate which the MS under test does not support, the test shall be skipped. An MS supporting only full rate speech shall consequently pass [17] tests, while an MS supporting both full and half rate speech shall pass [22] tests.

Table 1

From	To	Timing	Start	Syn	State	Section

Adv.	Time	?	of call	26.

TCH/F, no FH	TCH/F, no FH	20	none	no	U10	6.5.1

TCH/F, no FH	TCH/F, no FH	50	none	no	U10	6.5.1

TCH/F, no FH	TCH/F, no FH	20	1.1s	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	20	none	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	50	none	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	20	1.1s	no	U10	6.5.1

SDCCH/8, no FH	SDCCH/8, FH	20	none	no	estab	6.5.2

SDCCH/8, no FH	SDCCH/8, FH	20	1.1s	no	estab	6.5.2

TCH/F, FH	TCH/F, FH	20	none	no	estab	6.5.2

TCH/F, FH	TCH/F, FH	20	1.1s	no	estab	6.5.2

SDCCH/8, FH	TCH/F, no FH	20	none	no	estab	6.5.2

SDCCH/8, FH	TCH/F, no FH	20	1.1s	no	estab	6.5.2

TCH/F, FH	TCH/F, no FH	2k+y	none	yes	U10	6.5.3

TCH/F, FH	TCH/F, no FH	2k+y	1.1s	yes	U10	6.5.3

TCH/H, FH	TCH/H, no FH	2k+y	none	yes	U10	6.5.3

TCH/H, FH	TCH/H, no FH	2k+y	1.1s	yes	U10	6.5.3

SDCCH/8, FH	SDCCH/8, FH	2k+y	none	yes	estab	6.5.4

SDCCH/8, FH	SDCCH/8, FH	2k+y	1.1s	yes	estab	6.5.4

TCH/F, no FH	TCH/F, FH	2k+y	none	yes	estab	6.5.4

TCH/F, no FH	TCH/F, FH	2k+y	1.1s	yes	estab	6.5.4

SDCCH/8, no FH	TCH/F, no FH	2k+y	none	yes	estab	6.5.4

SDCCH/8, no FH	TCH/F, no FH	2k+y	1.1s	yes	estab	6.5.4

Table 2

TCH/FS	TCH/HS	SDCCH

n	10-20	5-10	2-5

In addition to the successfull case of Handover, 2 unsuccessfull cases shall be tested. These tests are described in 26.6.5.5 and 26.6.5.6.

26.6.5.1	Handover / successful / active call / non-synchronized

Purpose:

To test that when the MS is ordered to make a non-synchronized handover it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly handles the values of any Starting Time IE in the HANDOVER COMMAND message and the Timing Advance IE in the PHYSICAL INFORMATION message. To test that the MS activates the new channel correctly.

Conformance Requirement:

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping towards a TCH/F without frequency hopping.

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/H without frequency hopping to a TCH/H with frequency hopping. This does not apply to MSs not supporting TCH/H.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on cell A.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a call on cell B.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included in the message) begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends PHYSICAL INFORMATION with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission time of the PHYSICAL INFORMATION. It shall do so with the correct timing advance. Then the MS shall establish a signalling link and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : as specified in

table 1 of section 26.6.5.

Power Command : 5. Synchronisation

Indication : not synchronised.

Cell Channel Description : appears

if hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

(if no Starting Time is specified then

this IE is not included).

2	MS -> SS	HANDOVER ACCESS	Repeated until reception of PHYSICAL

INFORMATION. Handover Reference as

included in the HANDOVER COMMAND

3	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER

ACCESS. Timing Advance : as

specified in table 1 of section

26.6.5.

4	MS -> SS	SABM	Sent without information field

5	SS -> MS	UA

6	MS -> SS	HANDOVER COMPLETE

Specific Message Contents:

None

26.6.5.2	Handover / successful / cell under establishment / non-synchronized

Purpose:

To test that when the MS is ordered to make a non-synchronized handover, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION from the SS. To test that the MS correctly takes the values of the Timing Advance and Starting Time information elements into account. To test that the MS activates the new channel correctly. To test that the MS correctly retransmits Layer 3 MM or CC messages, that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

Conformance Requirement:

The MS shall correctly apply the handover procedure from SDCCH without frequency hopping to SDCCH with frequency hopping in the non-synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F with frequency hopping in the non-synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from SDCCH with frequency hopping to TCH/F without frequency hopping in the non-synchronized case, during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

Reference(s):

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Foreseen Final State of the MS:

"Idle, updated" with TMSI allocated and camped on cell B.

Method of Test:

A Mobile Originating Call is initiated. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element if it is included) begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends PHYSICAL INFORMATION with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission time of the PHYSICAL INFORMATION. It shall do so with the correct timing advance. Then the MS shall establish a signalling link and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,

NECI not set to 1

3	SS -> MS	IMM ASSIGNMENT	Channel Description and Mobile

Allocation : as specified in table

1 of section 26.6.5.

4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-

ting Call Establishment.

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	MS -> SS	SETUP	Last L2 frame not acknowledged by

the SS.

8	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : not synchronized.

Cell Channel Description : appears

if hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5. (if no

Starting Time is specified then this IE is

not included)

9	MS -> SS	HANDOVER ACCESS	Repeated until reception of PHYSICAL

INFORMATION. Handover Reference as

included in the HANDOVER COMMAND

10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER

ACCESS. Timing Advance : as

specified in table 1 of section

26.6.5.

11	MS -> SS	SABM	Sent without information field

12	SS -> MS	UA

13	MS -> SS	HANDOVER COMPLETE

14	MS -> SS	SETUP	Same N(SD) as in step 7.

15	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.5.3	Handover / successful / active call / finely synchronized

Purpose:

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element and correctly calculating the timing advance to use.

Conformance Requirement:

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F without frequency hopping in the synchronized case when a call is in progress.

The MS shall correctly apply the handover procedure from TCH/H with frequency hopping to TCH/H without frequency hopping in the synchronized case when a call is in progress. This requirement does not apply to MSs not supporting TCH/H.

Reference(s):

GSM 04.08 section 3.4.4.

GSM 05.10, section 6.6

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y [bit periods].

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is 3.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell B). Used power level is 5.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included) send 4 access bursts, at the commanded power level, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link using the correct Timing Advance and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : Synchronized.

Cell

Channel Description : appears if

hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

If no Starting Time is specified then

this IE is not included.

2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in

3	MS -> SS	HANDOVER ACCESS	the HANDOVER COMMAND.

4	MS -> SS	HANDOVER ACCESS

5	MS -> SS	HANDOVER ACCESS

6	MS -> SS	SABM	Sent without information field

7	SS -> MS	UA

8	MS -> SS	HANDOVER COMPLETE

Specific Message Contents:

None

26.6.5.4	Handover / successful / call under establishment / finely synchronized

Purpose:

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element and correctly calculating the timing advance to use. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

Conformance Requirement:

The MS shall correctly apply the handover procedure from SDCCH with frequency hopping to SDCCH with frequency hopping in the finely synchronized case, during call establishment.

If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F with frequency hopping in the synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from SDCCH without frequency hopping to TCH/F without frequency hopping in the synchronized case, during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

Reference(s):

GSM 05.10, section 6.6

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y [bit periods].

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Foreseen Final State of the MS:

"Idle, updated" with TMSI allocated and camped on cell B.

Method of Test:

A Mobile Originating Call is initiated. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element, if included) send 4 access bursts, at the commanded power level, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link using the correct timing advance and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,

NECI not set to 1"

3	SS -> MS	IMM ASSIGNMENT	Channel Description and Mobile

Allocation : as specified in table

1 of section 26.6.5.

4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-

ting Call Establishment.

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	MS -> SS	SETUP	Last L2 frame not acknowledged by

the SS.

8	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : synchronized. Cell

Channel Description : appears if

hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

(If no Starting Time is specified then

this IE is not included)

9	MS -> SS	HANDOVER ACCESS	Handover Reference as included in

10	MS -> SS	HANDOVER ACCESS	the HANDOVER COMMAND

11	MS -> SS	HANDOVER ACCESS

12	MS -> SS	HANDOVER ACCESS

13	MS -> SS	SABM	Sent without information field

14	SS -> MS	UA

15	MS -> SS	HANDOVER COMPLETE

16	MS -> SS	SETUP	Same N(SD) as in step 7

17	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.5.5	Handover / L3-failure

Purpose:

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE

Conformance Requirement:

The MS shall return to the old channel in the case of an handover failure caused by the non reception of the PHYSICAL INFORMATION message. On the old channel the MS shall use the Power Level that it was previously using on that channel.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND with Power Command set to 8 on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS activates the SACCH, but does not send PHYSICAL INFORMATION (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,

full rate

Power Command : 8. Synchronisation

Indication : non synchronized.

2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all

with correct Handover References.

3	MS -> SS	HANDOVER FAILURE	Sent on old channel,

RR cause value ="Abnormal release,

unspecified", "Abnormal release,

channel unacceptable", "Abnormal

release, timer expired" or

"Abnormal release, no activity on

the radio path". Layer 1 header

has the same power level as the

layer 1 header in step 1.

Shall be sent

within 3 seconds from the trans-

mission of HANDOVER COMMAND.

Specific Message Contents:

None

26.6.5.6	Handover / L1-failure

Purpose:

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE

Conformance Requirement:

The MS shalll return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell. On the old channel the MS shall use the Power Level that it was previously using on that channel.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Method of Test:

The MS is in the active state (u10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,

full rate. Synchronisation

Indication : non synchronized.

2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all

with correct Handover References.

3	MS -> SS	HANDOVER FAILURE	Sent on old channel,

RR cause value ="Abnormal release,

unspecified", "Abnormal release,

channel unacceptable", "Abnormal

release, timer expired" or

"Abnormal release, no activity on

the radio path". Layer 1 header

has the same power level as the

layer 1 header in step 1.

Shall be sent

within 3 seconds from the trans-

mission of HANDOVER COMMAND.

Specific Message Contents:

None

26.6.6	Test of frequency redefinition

The Frequency Redefinition procedure is used by the network to change the frequencies and hopping sequences of the allocated channels.

26.6.6.1	Frequency redefinition

Purpose:

To verify that the MS, after receiving a Frequency Redefinition message, starts using the new frequencies and hopping sequence at the time indicated in the message.

Conformance Requirement:

An MS, after receiving a FREQUENCY REDEFINITION message, shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

The behaviour described in the test purpose is applied for each combination of the value T(k) (k = 1,2,3) and for each supported dedicated channel type.

Reference(s):

GSM 04.08 section 3.4.5.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Initial Condition(s):

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs. The cell allocation is set to CA(1) before each execution of this test.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

Test parameters:

ca(1) is set to 64.

A random subset CA(1) of the set {1,...,124} containing ca(1) elements is drawn.

An element B of the set CA(1) is randomly drawn.

A random value ca(2) in the range 20,...,63 is drawn.

A random subset CA(2) of the set {1,...,124} with ca(2) elements and containing B is drawn.

A random value ca(3) in the range 4,...,19 is drawn.

A random subset CA(3) of the set {1,...,124} with ca(3) elements and containing B is drawn.

For j = 1,2,3, values ma(j) in the range j,...,ca(j)-1 and values MAIO(j) in the range 0,...,ma(j)-1 are randomly drawn.

Subsets MA(j) of CA(j) not containing B and having ma(j) elements are randomly drawn.

Let T(1) = 91, T(2) = 42000.

A random value T(3) in the range 92,...,29999 is drawn.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a dedicated channel (TCH/F, TCH/H or SDCCH, as defined by the execution counter R). Then the SS sends a FREQUENCY REDEFINITION message, which modifies the frequencies/hopping sequence to be used by the MS. The MS shall then, at the TDMA frame defined by the contents of the "Starting Time" information element , use the new frequencies/hopping sequence. (The value of T(2) ensures that the MS believes the Starting Time has passed and so the MS shall start transmitting immediately. The range for T(3) ensures that the MS has to wait until the designated frame before starting transmission on the new frequencies.)

The verification is performed at the RF burst level. The MS transmits the standard test signal C1 as defined in GSM 11.40, and for the TCH case, the SS checks the received pattern with the expected pattern. For the SDCCH case the MS transmits fill frames, and the SS checks for each burst whether the burst is transmitted at the right frequency.

[editorial note : how is the MS made to transmit the C1 signal ? By means of loop-back ? ]

Maximum Duration of Test:

3 * (number of supported channels * T(3) + 7)

Expected Sequence:

This sequence is performed for every combination of execution counters K = 1,2,3 and R = 1,2,3:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies without delay.

6	SS -> MS	FREQUENCY REDEF	See description 1 below.

7	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies and that the transmissions

started in the correct frame.

8	SS -> MS	FREQUENCY REDEF	See description 2 below

9	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies and that the transmissions

started in the correct frame.

10	SS -> MS	CHANNEL RELEASE

For	K=1	T(K) = T(1);

K=2	T(K) = T(2);

K=3	T(K) = T(3).

Specific Message Contents:

IMMEDIATE ASSIGNMENT

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CA(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected by test house

TSC						arbitrarily selected by test house

Hopping channel				RF hopping channel

MAIO						MAIO(1)

HSN						0

Request reference				corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MA(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIO(2)

HSN						0

Mobile Allocation					corresponds to set MA(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(k)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CA(2) with 'Format ID'

set to "bit map 0"

FREQUENCY REDEFINITION (Description 2)

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIO(3)

HSN						0

Mobile Allocation					corresponds to set MA(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(k)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CA(3) with 'Format ID'

set to "bit map 0"

26.6.7	Test of transmission mode change

The Transmission Mode Change procedure allows the network to request the MS to change the channel mode for one channel. The channel mode covers the coding, decoding and transcoding mode used on the indicated channel.

26.6.7.1	Transmission mode change

Purpose:

To verify that the MS changes mode of operation and acknowledges this to the network.

Conformance Requirement

1.	For each of the new modes supported by the MS and included in the CHANNEL MODE MODIFY message the MS shall send the CHANNEL MODE MODIFY ACKNOWLEDGE message with this new mode, this applies to the following modes: signalling, speech full rate, speech half rate, data 9.6, data 4.8/F and data 2.4/F.

2:	For each of the new modes supported by the MS and included in the CHANNEL MODE MODIFY message the MS shall send the CHANNEL MODE MODIFY ACKNOWLEDGE message with this new mode, this applies to the following modes: data 4.8/H and data 2.4/H.

This requirement does not apply for MSs not supporting TCH/H.

Reference(s):

GSM 04.08 section 3.4.6.

Related PICS/PIXIT Statement(s):

-

Initial Condition(s):

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is in the active state of a speech call (u10).

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The test starts with the MS having a speech call in progress on a TCH/F channel. The SS then sends a series of CHANNEL MODE MODIFY messages. For every change of mode that the MS is capable to handle, it sends CHANNEL MODE MODIFY ACKNOWLEDGE with the corresponding value.

Only parameter values within the messages are verified. The coding is not tested here.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	CHAN MODE MODIFY	Channel Type = Bm + ACCHs, Channel

Mode = data 9.6.

2	MS -> SS	CHAN MODE MOD ACK	Channel Type = Bm + ACCHs, Channel

Mode = data 9.6. See Note.

3	SS -> MS	CHAN MODE MODIFY	Channel Type = Bm + ACCHs, Channel

Mode = data 4.8/F.

4	MS -> SS	CHAN MODE MOD ACK	Channel Type = Bm + ACCHs, Channel

Mode = data 4.8/F. See Note.

5	SS -> MS	CHAN MODE MODIFY	Channel Type = Bm + ACCHs, Channel

Mode = data 2.4/F.

6	MS -> SS	CHAN MODE MOD ACK	Channel Type = Bm + ACCHs, Channel

Mode = data 2.4/F. See Note.

7	SS -> MS	CHAN MODE MODIFY	Channel Type = Lm + ACCHs, Channel

Mode = data 4.8/H.

8	MS -> SS	CHAN MODE MOD ACK	Channel Type = Lm + ACCHs, Channel

Mode = data 4.8/H. See Note.

9	SS -> MS	CHAN MODE MODIFY	Channel Type = Lm + ACCHs, Channel

Mode = data 2.4/H.

10	MS -> SS	CHAN MODE MOD ACK	Channel Type = Lm + ACCHs, Channel

Mode = data 2.4/H. See Note.

11	SS -> MS	CHANNEL RELEASE

Note:	CHANNEL MODE MODIFY ACKNOWLEDGE is only sent by the MS if it supports the new channel mode.

Specific Message Contents:

None

26.6.8	Test of ciphering mode setting

The Ciphering Mode Setting Procedure can be used by the network to trigger the start and stop of stream ciphering.

26.6.8.1	Ciphering mode / start ciphering

Purpose:

To verify that the MS starts ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "Start Ciphering". To verify that it continues to use the old cipher key after it receives an AUTHENTICATION REQUEST whilst in ciphered mode.

Conformance Requirement:

1:	When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field;

-	the MS responds with a CIPHERING MODE COMPLETE message in ciphered mode;

-	the ciphering uses the cipher key determined during the authentication procedure.

2:	The MS responds to the AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the ciphering key obtained from the previous authentication procedure.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Supported encryption algorithms: A5/1 and/or A5/2

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with an algorithm supported by the MS. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in ciphered mode using the cipher key determined during the authentication procedure, and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

The SS then sends another AUTHENTICATION REQUEST and the MS shall respond with an AUTHENTICATION RESPONSE. The MS shall continue to use the old cipher key.

Finally the SS sends a CHANNEL RELEASE to end the test.

[editorial note : In the old 11.10 it was required that the MS should store the new key, but that was not tested. Should it be tested in phase 2 ? ]

Maximum Duration of Test:

-

Expected Sequence:

This sequence is performed for execution counter, K=1, 2.

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call"

NECI not set to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	AUTHENT REQUEST

5	MS -> SS	AUTHENT RESPONSE

6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start

Ciphering; cipher with algorithm A5/K".

The SS starts deciphering.

7	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using

the cipher key determined in between

steps 4&5. The SS start enciphering

8	MS -> SS	SETUP

9	SS -> MS	AUTHENT REQUEST	Determines a new cipher key

10	MS -> SS	AUTHENT RESPONSE	Sent in ciphered mode using the

cipher key determined in between steps

4&5.

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

CIPHERING MODE COMMAND

For	k = 1, A5/k = A5/1

For	k = 2, A5/k = A5/2

26.6.8.2	Ciphering mode / no ciphering

Purpose:

To verify that the MS does not start ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "No Ciphering".

Conformance Requirement:

When the MS receives a CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering" the MS shall respond in non ciphered mode with a CIPHERING MODE COMPLETE message.

When the CIPHERING MODE COMMAND with Ciphering Mode Setting information element set to "no ciphering" is received as a response to a CM SERVICE REQUEST, the MS shall continue the establishment of the CM service.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with an AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS not to start ciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non-ciphered mode and continue to establish the call with a SETUP message.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call;

NECI not equal to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	AUTHENT REQUEST

5	MS -> SS	AUTHENT RESPONSE

6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No

Ciphering"

7	MS -> SS	CIPH MODE COMPLETE	Sent in non-ciphered mode

8	MS -> SS	SETUP

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.8.3	Ciphering mode / old cipher key

Purpose:

To verify that the MS uses the stored cipher key when it receives a CIPHERING MODE COMMAND without a preceding authentication procedure.

Conformance Requirement:

When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field. Also;

-	the MS responds with a CIPHERING MODE COMPLETE message in the correct ciphered mode;

-	the ciphering shall use the previously stored cipher key

-	in the case of a mobile originating speech caal, the MS shall send a SETUP message after the completion of the ciphering procedure.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Supported encyrption algorithms: A5/1 and/or A5/2.

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout = 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key stored.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the stored cipher key and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call:

NECI not equal to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start

Ciphering", algorithm arbitrarily selected

from those supported by the MS. The SS starts

deciphering.

5	MS -> SS	CIPH MODE COMPLETE	Sent in commanded ciphered mode with

the stored cipher key. The SS

starts enciphering.

6	MS -> SS	SETUP

7	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.9	Test of additional assignment

The Additional Assignment procedure is used to change an MS's channel configuration from Lm + ACCH to Lm + Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

26.6.10	Test of partial release

The Partial Release procedure is used to change an MS's channel configuration from Lm + Lm + ACCH to Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

26.6.11	Test of classmark change

This procedure allows the MS to indicate to the network that a change in the classmark (e.g. due to addition of power amplification) has taken place.

26.6.11.1	Classmark change

Purpose:

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed during a call, the change is communicated on the DCCH to the network.

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed in idle mode, the out of date capabilities are not communicated to the network during RR connection establishment.

Conformance Requirement:

If the RF power capability of the MS is changed during a call, this change shall be signalled to the network.

If the RF power capability of the MS is changed in idle mode, the up to date RF power capability shall be signalled to the network during RR connection establishment.

Reference(s):

GSM 04.08 section 3.4.10.

Related PICS/PIXIT Statement(s):

Support for more than one RF power class: yes/no.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and with no additional power amplification applied.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated and with no additional power amplification applied.

Method of Test:

With the MS in idle mode, the RF power capability shall be changed by the addition of power amplification, after which the MS is made to originate a call. The new RF power capability shall be included in the CM SERVICE REQUEST message. After the call has reached the Call Control state U10, the RF power capability of the MS is changed by removal of the additional power amplification. The MS shall send a CLASSMARK CHANGE message indicating the new RF power capability. The RF power capability is then changed by adding the power amplification. The MS shall again send a CLASSMARK CHANGE message indicating the new RF power capability. The call is then released by the SS.

With the MS in idle mode, the power amplification is removed. The SS then pages the MS, which in the PAGING RESPONSE message shall indicate the correct RF power capability.

Finally the SS transmits a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	Add power amplification

2	--------	--------	The MS shall be made to

originate a call

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call"

NECI not equal to one

4	SS -> MS	IMM ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	The "Mobile Station Classmark 2"

IE shall indicate the new RF power

capability

6	SS -> MS	CM SERVICE ACCEPT

7	MS -> SS	SETUP

8	SS -> MS	CALL PROCEEDING

9	SS -> MS	ALERT

10	SS -> MS	ASSIGNMENT COMMAND

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> MS	CONNECT

13	MS -> SS	CONNECT ACKNOWLEDGE

14	--------	--------	Remove the power amplification

15	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

16	--------	--------	Add power amplification

17	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

18	SS -> MS	CHANNEL RELEASE

19	--------	--------	Remove the power amplification

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

20	SS -> MS	PAG REQ TYPE 1

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT

23	MS -> SS	PAGING RESPONSE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

24	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.12	Test of channel release

The purpose of this procedure is to deactivate the dedicated channels in use. When the channels are released, the MS returns to the CCCH configuration, idle mode.

26.6.12.1	Channel release / SDCCH

Purpose:

To verify that the MS is able to correctly release an SDCCH after having received a CHANNEL RELEASE message.

Conformance Requirement:

After the acknowledgement of the Layer 2 disconnection by the network, the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

20 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.	6	MS -> SS	DISC

7	SS -> MS	UA

--------	--------	The SS verifies for 3 seconds that the

MS does not produce any Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT

11	MS -> SS	PAGING RESPONSE

12	SS -> MS	CHANNEL RELEASE

[Editorial note : A timelimit of 3 seconds for checking the RF-transmissions has been added in all the channel release tests ]

Specific Message Contents:

None

26.6.12.2	Channel release / SDCCH - no L2 ACK

Purpose:

To verify that the MS is able to correctly release a SDCCH after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

Conformance Requirement:

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

25 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.

6	MS - > SS	DISC	The MS shall send at least 2 L2

DISC frames, to which the SS does

not respond. After a period of 2

seconds, the SS verifies for 3

seconds that the MS does not

produce any further Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	SS -> MS	IMM ASSIGNMENT

10	MS -> SS	PAGING RESPONSE

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.12.3	Channel release / TCH-F

Purpose:

To verify that the MS is able to correctly release a full-rate TCH after having received a CHANNEL RELEASE message.

Conformance Requirement:

After the acknowledgement of the Layer 2 disconnection by the network the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

Supported rate of TCH: TCH/F.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

20 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.

6	MS -> SS	DISC

7	SS -> MS	UA

--------	--------	The SS verifies for 3 seconds that the

MS does not produce any Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT

11	MS -> SS	PAGING RESPONSE

12	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.12.4	Channel release / TCH-F - no L2 ACK

Purpose:

To verify that the MS is able to correctly release a TCH/F after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

Conformance Requirement:

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

Supported rate of TCH: TCH/F.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message (with cause "abnormal release, unspecified"), after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

25 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	Cause value = "Abnormal release,

unspecified".

6	MS -> SS	DISC	The MS shall sned at

least 2 L2 DISC frames, to which

the SS does not respond. After a

period of 2 seconds, the SS

verifies for 3 seconds that the MS

does not produce any further

Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

10	MS -> SS	PAGING RESPONSE

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.13	Default contents of GSM 900 layer 3 messages for RR tests

This section contains the default values of L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occurs in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

The following messages are yet to be defined:

DISCONNECT

Default SYSTEM INFORMATION:

Note:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 5 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description			Not present

(CBCH) Mobile Allocation				Not present

Cell Channel Description

-	Format identifier			bit map 0

-	Cell Allocation ARFCN			Channel Numbers 20, 30, 50 and 70.

Cell Identity

-	Cell Identity Value			0001H

Cell Options

-	Power Control Indicator			Power Control Indicator is not set

-	DTX Indicator				MS shall not use DTX

-	Radio-Link-Timeout			8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis			12 dB

-	MX-TXPWR-MAX-CCH			Minimum level

-	ACS					No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI					New establishment causes allowed

-	RXLEV-ACCESS-MIN			Minimum level

Control Channel Description

-	Attach-Detach allowed			No Attach/Detach

-	BS-AG-BLKS-RES			0 blocks reserved for access grant

-	CCCH-CONF				1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS			5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value			Infinite

L2 pseudo length

-	System information 1			21

-	System information 2			22

-	System information 3			18

-	System information 4			12

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Message Type

-	System information 1			00011001

-	System information 2			00011010

-	System information 3			00011011

-	System information 4			00011100

-	System information 5			00011101

-	System information 6			00011110

Neighbour Cells Description

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND				This IE carries the complete BA

NCC Permitted					0000 0010

RACH Control Parameters

-	Max Retrans				Max 1 retrans

-	Tx-integer				5 slots used

-	Cell Barred for Access			Cell is not barred

-	Call Reestablishment Allowed		Not allowed

-	Access Control Class			Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets					Not used (all bits are set to spare)

SI 2 rest octets					Not used (all bits are set to spare)

SI 3 rest octets					Not used (all bits are set to spare)

SI 4 rest octets					Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level				63 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			20

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier				Bir map 0

-	Cell Allocation ARFCN			Channel Number 10

Note:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value			0002H

Default settings for cell B:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			10

Contents of ALERTING message (SS to MS):

Protocol Discriminator				Call Control,

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					00000001

Facility						Not present

Progress indicator				Not present

User-user					Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen randomly by the test house

Frequency List					Not present

Cell Channel Description				Not present

Channel Mode					Not present

Channel Description				Not present

Channel Mode 2					Not present

Mobile Allocation					Not present

Starting Time					Not present

Cipher Mode Setting				Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101001

RR Cause

-	RR Cause Value			Normal Release

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101111

RR Cause

-	RR Cause Value			Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X010010

Ciphering Key Sequence Number

-	Key Sequence				Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value				Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X010100

Authentication Parameter SRES

-	SRES value				As supplied by the SIM

Contents of CALL PROCEEDING message:

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					0X000010

Repeat Indicator					Not present

Bearer Capability 1				Not present

Bearer Capability 2				Not present

Facility						Not Present

Progress indicator				Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010000

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010111

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code		Same as in the CHANNEL MODE MODIFY

message

-	Hopping				Single RF channel

-	Frequency Band			Band number 0

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00001101

RR Cause

-	RR Cause Value			Normal Release

Contents of CHANNEL REQUEST message

Establishment Cause				If in response to paging,

then '100'; if a mobile originating call,

then '111'

Random Reference				5 bits generated randomly by the MS

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110101

Cipher Mode Setting

-	algorithm identifier			cipher with A5/1

-	SC					Start ciphering

Cipher Response					IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110010

Mobile Identity					Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					000100110

Mobile Station Classmark 2			See PICS/PIXIT

Mobile Station Classmark 3			For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					0X100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					0X100100

CM service type					Mobile originating call establishment

Ciphering key sequence number			as allocated in the previous Authentication

Request; if none has been allocated then

"no key is available"

Mobile station classmark 2				See PICS/PIXIT

Mobile Identity					the allocated TMSI, or if not allocated, then

the IMSI

Contents of CONNECT message (SS to MS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (if this entity received the SETUP,

0 otherwise)

Message Type					00000111

Facility						not present

Progress indicator				not present

Connected number				not present

Connected subaddress				not present

User-User					not present

Contents of CONNECT ACKNOWLEDGE message

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					0 (if this entity sent the SETUP, 1 otherwise)

Message Type					0X001111

Contents of HANDOVER ACCESS message:

Handover Reference				Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Set to the BCCH carrier number of cell B.

(one of 10, 20, 80, 90, 100, 110 or 120)

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Chosen randomly by the test house

Handover Reference

-	Handover Reference Value		Chosen arbitrarily by the test house.

Power Command

-	Power level				Chosen arbitrarily by the test house

Synchronisation Indication				Not present

Frequency short list				Not present

Frequency list					Not present

Cell Channel Description				Not present

Channel Mode					Not present

Channel Description				Not present

Channel Mode 2					Not present

Frequency Channel Sequence			Not present

Mobile Allocation					Not present

Starting Time					Not present

Real time difference				Not present

Timing advance					Not present

Cipher Mode setting				Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101100

RR cause					Normal event

Time difference					Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101000

RR cause					Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111111

Page Mode

-	Page Mode				Normal Paging

Channel Description

-	Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping				Single RF channel

-	ARFCN				For non-combined CCCH/SDCCH (see initial

conditions), Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference				Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value			30 bit periods

Mobile Allocation

-	Length				0

Starting Time					Not present

IA rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111001

Page Mode

-	Page Mode				Normal Paging

Channel Description 1

- Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number			For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping				Single RF channel

- ARFCN				For non-combined CCCH/SDCCH (see initial

conditions),Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference 1				Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value			Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset		Same channel type as in Channel

description, but different TDMA offset to the

previous one.

by the testhouse

-	Timeslot Number			equal to the value in Channel description 1.

-	Training Sequence Code		equal to the value in Channel description 1.

-	Hopping				Single RF channel

-	ARFCN				equal to the value in Channel description 1.

Request Reference 2				Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value			Chosen arbitrarily by the test house

Mobile Allocation

-	Length				0

Starting Time					Not present

IAX rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111010

Page Mode

-	Page Mode				Normal Paging

Request Reference				Pertaining to last Channel Request sent

by the MS

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

IAR rest octets					Not used (all bits set to spare)

Contents of LOCATION UPDATING REJECT message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X000100

Reject Cause

-	Reject Cause Value			As described in test

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X001000

Location Updating Type

-	LUT					Normal Location Updating

Ciphering Key Sequence Number

-	Key Sequence				Key sequence number previously allocated

to MS. If not allocated, then set to 'no key

is available'.

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Mobile Station Classmark 1

Mobile Identity

-	odd/even indication			Even

-	Type of Identity			TMSI

-	Identity Digits				TMSI previously allocated to MS

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100001

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile Identity 1

-	odd/even indication			Even

-	Type of Identity			TMSI

-	Identity Digits				TMSI previously allocated to MS

Mobile Identity 2					Not present

P1 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100010

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile Identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value				TMSI not allocated to MS

Mobile Identity					Not present

P2 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100100

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile identity 2

-	TMSI value				TMSI not allocated to MS

Mobile identity 3

-	TMSI value				TMSI not allocated to MS

Mobile identity 4

-	TMSI value				TMSI not allocated to MS

P3 rest octets					Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100111

Ciphering Key Sequence Number

-	Key Sequence				Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication			Even

-	Type of identity			TMSI

-	Identity Digits				TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101101

Timing advance					20 bit periods

Contents of RELEASE message (MS to SS)

(* only used in Classmark change procedure, II.5.3.6.11 *)

(* If CR 11.10-C10 is accepted, this message definition can be deleted.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					0 (if this entity sent the SETUP, 1 otherwise)

Message Type					0X101101

Cause						optional, if present ignore contents other

than the length and bit 8 of the IEI

Second Cause					not present

Facility						optional, if present ignore contents other

than the length and bit 8 of the IEI

User-User					optional, if present ignore contents other

than the length and bit 8 of the IEI

SS version					optional, if present ignore contents other

than the length and bit 8 of the IEI

Contents of RELEASE COMPLETE message (SS to MS)

(* only used in Classmark change procedure, II.5.3.6.11 *)

(* If CR 11.10-C10 is accepted, this message definition can be deleted.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (if this entity received the SETUP,

0 otherwise)

Message Type					0X101010

Cause						same as in the RELEASE message; if it was

not included then this IE is not present

Facility						not present

User-user					not present

Contents of SETUP message; (SS to MS; for full rate speech)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag					0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length					01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

0/1 ext					1 (not extended)

Bearer capability 2				Not present

Facility						Not present

Progress indicator				Not present

Signal						[Not present]

Calling party BCD number				Not present

Calling party subaddress				Not present

Called party BCD number				Not present

Called party subaddress				Not present

LLC repeat indicator				Not present

Low layer compatibility I				Not present

Low layer compatibility II				Not present

HLC repeat indicator				Not present

High layer compatibility i				Not present

High layer compatibility ii				Not present

User-user					Not present

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					any value from the set {0, ..., 6}

TI flag					0

Message Type					0X000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04H

length					01H

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

0/1 ext					1 (not extended)

Bearer capability 2				Not present

Facility						optional, if present ignore contents other

than the length and bit 8 of the IEI

Calling party subaddress				optional, if present ignore contents other

than the length and bit 8 of the IEI

Called party BCD number				Shall be present, however ignore contents

other than the length and bit 8 of the IEI.

(Note that to make a call, at least 3 digits

are needed.)

Called party subaddress				optional, if present ignore contents other

than the length and bit 8 of the IEI

LLC repeat indicator				Not present

Low layer compatibility I				Not present

Low layer compatibility II				Not present

HLC repeat indicator				Not present

High layer compatibility i				Not present

High layer compatibility ii				Not present

User-user					optional, if present ignore contents other

than the length and bit 8 of the IEI

SS version					optional, if present ignore contents other

than the length and bit 8 of the IEI

CLIR override					optional, if present ignore contents other

than bit 8 of the IEI