20.	Cell selection/reselection

20.1	General points

GSM 05.08 does not give times for how quickly an MS has to select or reselect a cell. For the purposes of these tests, the following times are allowed:

GSM900 only - begin

(a)	Time to read and average signal strength on 124 RF carriers. If one signal strength sample is read on an RF carrier every TDMA frame, the time to take 5 samples on each is about 3 s. However the averaging time is 3-5 s or 5 paging block intervals (which equals 5.9 s for the case of BS_PA_MFRMS= 5 frames). 6 s will be allowed.

GSM900 only - end

DCS1800 only - begin

(a)	Time to read and average signal strength on 374 RF carriers. If two signal strength samples are read on an RF carrier every TDMA frame, the time to take 5 samples on each is about 4.5 s. However the averaging time is 3-5 s or 5 paging block intervals (which equals 5.9 s for the case of BS_PA_MFRMS= 5 frames). 6 s will be allowed.

DCS1800 only - end

(b)	Time to attempt to read SCH on a BCCH carrier. SCH is sent 5 times every 235.4 ms. We will allow 0.5 s per carrier on which SCH is searched for. Hence for a BA with 16 carriers 8 s will be allowed.

(c)	Time to read

GSM900:	all 4

DCS1800:	a maximum of 5

System information messages on one BCCH carrier. GSM 05.02 specifies that all system information blocks should be sent every 8 multiframes, i.e. 1.9 s.

(d)	Time to do random access (RA) attempts. Assuming a non combined CCCH and MAX retrans = 1, the whole RA attempt process will take around 1 s.

(e)	Time between paging blocks for an MS - This is 1.2 s for BS_PA_MFRMS = 5.

For the cell selection tests below, the maximum response time will be

(a) + (b) + 4(c) + (d) + (e) = 23.8 s.

A time of 30 s is allowed in the tests below.

For cell reselection, the maximum response time is

(a) + (c) + (d) + (e) = 10.1 s.

A time of 12 s is allowed in the tests below.

DCS1800 only - begin

These times are assumed to be after self test has been completed and any PIN entry accepted.

DCS1800 only - end

The accuracy of signal level measurements is assumed to be up to +/- 6 dB absolute and +/- 2 dB relative between carriers. The tests below allow, for cases of discrimination between C1 and 0, a difference of at least 8 dB, and for discrimination between C1 values on different carriers, a difference of at least 5 dB.

Note:	The timing tolerances in all the tests shall be -0%, +10%.

The initial conditions that apply to all the tests in 20 Cell selection/reselection shall be as follows:

The MS is brought into the idle updated state with the values of PLMN and LAC as in Table II.6-1 and then switched off.

20.2	Cell selection

20.2.1	Definition - Cell selection

Cell selection is the process in which an MS selects a suitable cell to camp on. The algorithms to be followed are given in GSM 05.08 section 6.1 to 6.4.

20.2.2	Method of test

a)	A number of BCCH carriers are transmitted by the SS. The system information on these carriers is as given in Table II.6-1 unless otherwise stated. The ARFCN and power levels of each of the carriers are as given in case 1 of table II.6-2, and the system information parameters on these carriers which are different to those in table II.6-1 are given in table II.6-2.

b)	Paging messages for the MS are transmitted on each of the BCCH carriers in each of the paging blocks of the MS (as defined in GSM 05.02). The SS ignores any random access (RA) messages sent in response to pages, unless otherwise stated.

c)	The MS is switched on.

d)	The SS checks that MS makes a response according to requirement 1 of case 1 of table II.6-2 within 30 s.

e)	The MS is turned off.

f)	Steps c) to e) are repeated, this time checking that the MS makes a response according to requirement 2 of case 1 of table II.6-2 within 30 s.

g)	Steps a) to f) above are repeated for cases 2 to 4 of table II.6-2. (The system information parameters in table II.6-2 only apply to the case given and are not continued into the next case unless explicitly stated.)

20.2.3	Requirement

The MS shall respond as in d) and f) above for each case in table II.6-2.

20.3	Cell reselection

20.3.1	Definition - Cell reselection

Cell reselection is the process in which the MS ensures that it is camped on the most appropriate cell. The algorithms to be followed are given in GSM 05.08 sections 6.4 to 6.6.

20.3.2	Method of test

Note:	The SS looks for a response from the MS, when required, within 12 s, unless otherwise stated.

a)	The SS does steps a) to c) of case 1 of the cell selection test above, but with RAM = -93 on carrier 6. (The values of C1 on the 6 BCCH carriers are then respectively 25, 15, 10, 20, -8, <=5.)

b)	The SS checks that the MS does at least one RA request on carrier 3 within 30 s followed by at least one RA request on carrier 4 within a further 12 s.

Note:	Carrier 3 has a higher level than carrier 4 but a lower C1. Therefore initial cell selection should choose carrier 3 and cell reselection should choose carrier 4.

c)	The SS stops all paging messages and then sets CBA=0 on carrier 1. After 6 minutes the SS pages the MS once on carrier 1 and repeatedly on carrier 4 in every paging block of the MS until the end of this test. (6 minutes allows time for the MS to read the BCCH data on carrier 1. The MS should then reselect carrier 1.)

d)	The SS checks that the MS makes at least one RA request on carrier 1 followed by at least 1 RA request on carrier 4. (The MS should temporarily reselect carrier 4 because of lack of response to RA requests on carrier 1, and then revert to carrier 1.)

e)	The SS transmits carrier 1 at 68 dBmVemf(  ) for 10 s, 38 dBmVemf(  ) for 2 s, 68 dBmVemf(  ) for 10 s and back to 48 dBmVemf(  ). (This gives a minimum averaged level of 48 dBmVemf(  ), i.e. minimum C1 of 25.)

f)	The SS checks that the MS does not make a RA request on either carrier 1 or carrier 4.

g)	The SS waits 10 s, then stops transmitting on carrier 1 for 18 s, and then transmits as before. (The averaging of signal level can take 6 s, C1 is checked at least every 5 s, and C1 low must be detected for 5 s. Adding 10% tolerance gives 17.6 s.)

h)	The SS checks that the MS makes at least 1 RA request on carrier 4.

i)	The SS changes the ARFCN of carrier 4 to 18 and then waits 120 s (to ensure the MS reselects carrier 1). It then puts random data into 4 successive paging blocks of the MS on carrier 1, and then reverts to sending normal data. (DSC will be initialised to 90/5 = 18, hence after 4 erroneous blocks, DSC will equal 2, hence no signalling failure.)

j)	The SS checks that the MS makes no RA requests on either carrier 1 or carrier 4.

k)	After at least 16 successive paging blocks of the MS on carrier 1 with normal data, the SS puts random data into 5 successive paging blocks of the MS followed by normal data. (This will cause DSC to reach 0.)

l)	The SS checks that the MS makes at least one RA request on carrier 4.

m)	The SS sets CBA = 1 on carriers 1 and 4.

n)	The SS waits 1 minute and then pages the MS on carriers 1, 2, 3 and 4.

o)	The SS checks that the MS makes at least one RA request on carrier 3 but none on carrier 1.

p)	The SS pages the MS on carriers 2 and 4 and checks that the MS makes no RA request on carriers 2 and 4.

q)	The SS sets CBA=O on carriers 1 and 4.

r)	The SS sets the level on carrier 3 to 33 dBmVemf(  ) (giving C1 = 5).

s)	The SS waits 6 minutes.

t)	The SS reduces the level on carrier 1 to 38 dBmVemf(  ) for 18 s and then reverts to 48 dBmVemf(  ). (This gives C1 = 15 on carrier 1 and 20 on carrier 4.)

u)	The SS checks that the MS makes at least one RA request on carrier 4.

v)	The SS waits for 30 s (to allow the MS to revert to carrier 1). It then sets LAC on carrier 4 to 1112(Hex) and reduces the level of carrier 1 to 38 dBmVemf(  ) for 18 s, and then reverts to 48 dBmVemf(  ).

w)	The SS checks that the MS makes no RA requests on carriers 1 and 4.

x)	The SS reduces the level on carrier 1 to 28 dBmVemf(  ) for 18 s (giving C1 = 5 on carrier 1, i.e. more than CRH below C1 on carrier 4) and then reverts to 48 dBmVemf(  ).

y)	The SS checks that the MS makes at least one RA request on carrier 4.

20.3.3	Requirement

1)	The MS shall respond as indicated in steps b), d), f), h), j), l), o), p), u), w) and y).

20.4	Cell selection after release of TCH and SDCCH

20.4.1	Definition - Cell selection after release of TCH and SDCCH

Ref: GSM 05.08 section 6.7.

20.4.2	Method of test

a)	The SS does steps a) to c) of case 1 of the cell selection test above, but with no pages being sent and with RAM = -70 on carriers 4 and 6. (This gives negative C1 on both carriers.)

Radio link timeout is set to 8.

b)	The SS sets up a call and brings it into the active state by paging the MS on BCCH carrier 3 and setting up a call on a TCH with ARFCN of

GSM900:	22

DCS1800:	534

c)	The SS starts paging the MS repeatedly on all the BCCH carriers.

d)	The SS causes a normal channel release.

e)	The SS checks that the MS does at least one RA request on carrier 3, but none on carrier 5.

f)	The MS is switched off and steps a) and b) above are repeated.

g)	Transmission on any dedicated channel is switched off. (This triggers call re-establishment.)

h)	The SS checks that the MS does at least one RA request on carrier 3 within 16 s.

i)	The MS is switched off and steps a) and b) above are repeated but with RE = 1 on carrier 3 and the SS stops paging the MS on carrier 3.

j)	The RXLEV ACCESS MIN on carrier 4 is set to RAM = -100.

k)	Transmission on any dedicated channel is switched off.

l)	The SS checks that the MS does at least one RA request on carrier 4 within 16 s.

Note:	Steps g and h are not applicable for an MS not supporting speech, see PIXIT.

20.4.3	Requirements

The MS shall respond as indicated in steps e), h) and l) above.

20.5	Cell selection (Abnormal cases and emergency calls)

20.5.1	Definition - Cell selection (Abnormal cases and emergency calls)

Ref: GSM 05.08 section 6.8.

20.5.2	Method of Test

a)	The SS performs steps a) to c) of 20.3 using table II.6-2 case 1, with the following exceptions:

-	The BCCH carriers belong to the same PLMN, which is not the MS's home PLMN.

-	No paging message from the MS is transmitted on the BCCH carriers.

-	Automatic network selection shall be disabled and the PLMN used in the BCCH shall be in the SIM's forbidden PLMN's list. No manual network selection shall be done during this test.

b)	The SS checks that no RA requests are made by the MS.

c)	60 s after the MS is switched on an emergency call is initiated on the MS (30 s for cell selection of carrier 2 or 3 which have the highest signal strength, 18 s for comparison of C1, and 12 s for reselection of carrier 4 which has the highest C1).

d)	The SS checks that the MS makes RA requests on carrier 4. (Carrier 4 has the highest value of C1 among the unbarred cells.)

e)	The SS sets carrier 2 to have the same PLMN and LAC as in table II.6-1 (changing the BSIC of the carrier) and starts paging the MS repeatedly on carrier 2.

f)	The SS checks that the MS makes RA requests on carrier 2 within 72 s (30 s for MS detection of the change in BSIC, 30 s for decode of BCCH and 12 s for reselection of carrier 2).

g)	The MS is switched off, the SIM is removed, carrier 2 is returned to having the PLMN of all the other BCCH cariers, and the ME is switched on again.

h)	The SS checks that no further RA requests are made by the MS within 60 s.

i)	60 s after the MS is switched on an emergency call is initiated on the MS (30 s for cell selection of carrier 2 or 3 which have the highest signal strength, 18 s for comparison of C1, and 12 s for reselection of carrier 4 which has the highest C1).

j)	The SS checks that the MS makes a RA request on carrier 4 within 12 s.

20.5.3	Requirement

The MS shall respond as indicated in steps b), d), f), h) and j) above.

GSM900	DCS1800

Parameter	Reference	Abbre-	Normal	Normal

GSM 04.08	viation	Setting	Setting

Cell channel description	10.5.2.1	-	Any values	Any values

MAX retrans	10.5.2.17	-	1	1

TX-integer	10.5.2.17	-	Any value	Any value

CELL_BAR_ACCESS	10.5.2.17	CBA	0 (not barred)	0 (not barred)

AC CN	10.5.2.17	AC	All 0	All 0

RE	10.5.2.17	RE	0 (re-establishment	0 (re-establishment

allowed)	allowed)

BA ARFCN	10.5.2.13	BA	All 0 except for	All 0 except for

ARFCN = 3,9,18,25,	ARFCN = 512,543,

41,43,49,50,54,58,	568,589,602,641,662,

62,66,70,80,92,124	683,696,711,732,754,

794,851,870,871,872,

884

NCC	10.5.2.15	NCC	Any value	Any value

Cell Identity	10.5.1.1	-	Any value	Any value

MCC, MNC	10.5.1.3	PLMN	MS Home PLMN	MS Home PLMN

LAC	10.5.1.3	LAC	1111 (Hex)	1111 (Hex)

ATT	10.5.2.8	-	Any values	Any values

BS_AG_BLKS_RES	10.5.2.8	-	Any values	Any values

T3212	10.5.2.8	-	Any values	Any values

BS_PA_MFRMS	10.5.2.8	BPM	5 frames	5 frames

Cell Options	10.5.2.3	-	Any values	Any values

CELL_RESELECT_-	10.5.2.4	CRH	10 dB	10 dB

HYSTERESIS

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Max. output	Max. output

power of MS	power of MS

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-90 dBm	-90 dBm

Table II.6-1 : Normal system information fields

GSM900 only - begin

Case 1	Case 2	Case 3	Case 4

BCCH	Level dBmV emf(  )	48	60/30 **	60/30 **	60/30 **

Carrier 1	Sys info	CBA = 1	MTMC = 13 dBm	AC = 1 for MS	Normal

ARFCN	62	62	62	2

BCCH	Level dBmV emf(  )	38	65	65	65

Carrier 2	Sys info	PLMN not	PLMN not	PLMN not	PLMN not

that of MS	that of MS	that of MS	that of MS

ARFCN	124	124	124	74

BCCH	Level dBmV emf(  )	38	38	38	38

Carrier 3	Sys info	RAM = -85	RAM = -85	RAM = -85	RAM = -85

ARFCN	54	54	54	4

BCCH	Level dBmV emf(  )	33	33	33	33

Carrier 4	Sys info	RAM = -100	RAM = -100	RAM = -100	RAM = -100

ARFCN	58	58	58	8

BCCH	Level dBmV emf(  )	43	43	43	43

Carrier 5	Sys info	RAM = -62	RAM = -62	RAM = -62	RAM = -62

ARFCN	66	66	66	16

BCCH	Level dBmV emf(  )	43/13 *	61/31 *	61/31 *	61/31 *

Carrier 6	Sys info	RAM = -98	RAM = -62	Normal	RAM = -62

ARFCN	70	70	70	20

The requirements that apply in the various cases are listed below.

*	Power is alternately the higher level for 0.5 s and the lower level for 3.5 s.

**	Power is alternately the higher level for 3.5 s and the lower level for 0.5 s.

GSM900 only - end

DCS1800 only - begin

Case 1	Case 2	Case 3	Case 4

BCCH	Level dBmV emf(  )	48	60/30 **	60/30 **	60/30 **

Carrier 1	Sys info	CBA = 1	MTMC = 14 dBm	AC = 1 for MS	Normal

ARFCN	543	543	543	543

BCCH	Level dBmV emf(  )	38	65	65	65

Carrier 2	Sys info	PLMN not	PLMN not	PLMN not	PLMN not

that of MS	that of MS	that of MS	that of MS

ARFCN	884	884	884	790

BCCH	Level dBmV emf(  )	38	38	38	38

Carrier 3	Sys info	RAM = -85	RAM = -85	RAM = -85	RAM = -85

ARFCN	696	696	696	604

BCCH	Level dBmV emf(  )	33	33	33	33

Carrier 4	Sys info	RAM = -98	RAM = -98	RAM = -98	RAM = -98

ARFCN	711	711	711	608

BCCH	Level dBmV emf(  )	43	43	43	43

Carrier 5	Sys info	RAM = -62	RAM = -62	RAM = -62	RAM = -62

ARFCN	851	851	851	630

BCCH	Level dBmV emf(  )	43/13 *	61/31 *	61/31 *	61/31 *

Carrier 6	Sys info	RAM = -98	RAM = -62	Normal	RAM = -62

ARFCN	568	568	568	660

The requirements that apply in the various cases are listed below.

Through these tests system information type 2 and 2 bis will be tested by forcing neighbour cell descriptions on two messages and verifying the correct cell selection.

*	Power is alternately the higher level for 0.5 s and the lower level for 3.5 s.

**	Power is alternately the higher level for 3.5 s and the lower level for 0.5 s.

DCS1800 only - end

Table II.6-2 : Cell selection test parameters

Notes on table II.6-2

Case 1:	The averaging on carrier 6 should result in an average level in the range 25 to 13 dBmVemf(  ). The values of C1 on the 6 BCCH carriers are respectively: 25, 15, 10, 20, -8, 10 to -2. Carrier 3 is the one with the highest level which has C1 > 0, is not barred and is of the correct PLMN.

Case 2:	The averaging on carriers 1 and 6 should result in an average level in the range of respectively: 60 to 48 dBmVemf(  ) and 43 to 31 dBmVemf(  ). Carrier 1 is the carrier with the highest level that satisfies the other constraints for cell selection.

Case 3:	The MS should select carrier 1 as for case 2, but because AC is set, the MS should not make any response to the SS.

Case 4:	Any stored BA information will be incorrect. The MS should therefore do a search of all

GSM900:	124 GSM900 carriers

DCS1800:	374 DCS1800 carriers.

It should then read BCCH data on carrier 2 followed by carrier 1. As carrier 2 is the wrong PLMN, the BA on the carrier should be ignored.

Requirement 1 in connection with table II.6-2

In case 1:	RA response on carrier 3 before a response (if any) on carriers 4 or 6.

In case 2:	RA response on carrier 1 before a response (if any) on carriers 3 or 4.

In case 3:	No RA response on carriers 1, 2, or 3.

In case 4:	RA response on carrier 1 before a response (if any) on carriers 3 or 4.

Requirement 2 in connection with table II.6-2

In case 1:	No RA response on carriers 1, 2 or 5.

In case 2:	No RA response on carriers 2, 5 or 6.

In case 3:	No RA response on carriers 4, 5 or 6.

In case 4:	No RA response on carriers 2, 5 or 6.

Page reserved for revised Cell Selection / reselection section

Page reserved for revised Cell Selection / reselection section

Page reserved for revised Cell Selection / reselection section

Page reserved for revised Cell Selection / reselection section

Page reserved for revised Cell Selection / reselection section