Default tab should be 1.5cm

10.	Generic call set up procedures

10.1	Generic call set-up procedure for Mobile Terminating Speech calls

In the test procedures described in this ETS, unless otherwise stated in the test description, the Mobile Terminating Speech call set-up procedure shall be as described in this section.

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.1.1	Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is in the MM state "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.1.2	Definition of System Information Messages:

The following parameters shall be coded into the system information messages. Parameters shall be coded according to GSM 04.08.

SYSTEM INFORMATION TYPE 1

Information Element			Value/remark

Cell channel description			Includes the hopping sequence ARFCNs,

if hopping is used

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SI1 rest octets				Spare Octets

SYSTEM INFORMATION TYPE 2

Information Element			Value/remark

BCCH Frequency list			Indicates seven surrounding cells on any ARFCN

of the supported band

NCC permitted

NCC PERMITTED			e.g all NCCs permitted

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SYSTEM INFORMATION TYPE 3

Information Element			Value/remark

Cell identity

CI VALUE				0001 hex (not relevant)

Location Area Identification

MCC					001  decimal (not relevant)

MNC					01   decimal (not relevant)

LAC					0001 hex (not relevant)

Control Channel Description

ATT (IMSI att/det)			MS shall not apply (not relevant)

BS-AG-BLKS-RES			0 blocks reserved (not relevant)

CCCH-CONF			Combined CCCH/SDCCH (not relevant)

BS-PA-MFRMS			5 multiframes (not relevant)

T3212					Infinite

Cell options

PWRC				power control not set

DTX					MS must not use DTX

RADIO LINK TIMEOUT		8

Cell selection parameters

CELL RESELECT HYSTERESIS	0 dB

MS-TXPWR-MAX-CCH		Max. output power of MS

RXLEV-ACCESS-MIN			-90 dBm

ACS					There are no additional cell parameters included

in SI7 and SI8

NECI					New establishment cause not supported

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SI3 rest octets

P1					C2 parameters not present

SYSTEM INFORMATION TYPE 4

Information Element			Value/remark

Location Area Identification

MCC					001 decimal (not relevant)

MNC					01 decimal (not relevant)

LAC					0001 hex (not relevant)

Cell selection parameters

CELL RESELECT HYSTERESIS	0 dB

MS-TXPWR-MAX-CCH		Max. output power of MS

RXLEV-ACCESS-MIN			-90 dBm

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

CBCH Channel Description			Omitted

Channel type and TDMA offset	[TBD]

TN					[TBD]

TSC					[TBD]

H					[TBD]

MAIO					[TBD]

FB-NO				[TBD]

HSN					[TBD]

ARFCN				[TBD]

CBCH Mobile Allocation			Omitted

MA					[TBD]

SI4 rest octets

P1					C2 parameters not present

SYSTEM INFORMATION TYPE 5

Information Element			Value/remark

Neighbour cell description			As Cell Channel Description in SI 1

SYSTEM INFORMATION TYPE 6

Information Element			Value/remark

Cell identity

CI VALUE				0001 hex (not relevant)

Location Area Identification

MCC					001 decimal (not relevant)

MNC					01 decimal (not relevant)

LAC					0001 hex (not relevant)

Cell options

PWRC				power control not set

DTX					MS must not use DTX

RADIO LINK TIMEOUT		8

NCC permitted

NCC PERMITTED			e.g all NCCs permitted

10.1.3	Procedure

An MS terminating call on a TCH/FS shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	Message contains the signal IE.

11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING

B13	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B14	MS		The MS is made to accept the call

in a way described in a

PICS/PIXIT statement.

B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	MS		The TCH is through connected in both

directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

10.1.4	Specific Message Contents:

PAGING REQUEST TYPE 1 (GSM 04.08, 9.1.22) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal Paging

Mobile Identity

Odd/even no of digits			As applicable

Type of Identity			TMSI

Identity digits			As applicable

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Transaction Identifier			Not used

Message Type

Ciphering key seq. number			As applicable from location update

Authent. parameter RAND			As applicable (128 bits)

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

SETUP (GSM 04.08, 9.3.23) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

Signal

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Power level				Power control level 7

Channel mode				Speech full rate

CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

10.2	Generic call set-up procedure for Mobile Originating Speech calls

In the test procedures described in this ETS, unless otherwise stated in the test description, the Mobile Originating Speech (MOC) call set-up procedure shall be as described in this clause.

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.2.1	Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see [Annex 1 TC.2.1])

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is in the MM state "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.2.2	Definition of System Information Messages:

See clause 10.1.2.

10.2.3	Procedure

An MS originating call on a TCH/FS shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	MS		"called number" entered

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"originating call".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following messages shall

be sent enciphered.

9	SS		SS starts ciphering.

10	MS -> SS	SETUP

11	SS -> MS	CALL PROCEEDING

12	SS -> MS	ALERTING

13	MS		An alerting indication as defined in an

PICS/PIXIT statement is given by the MS.

14	SS -> MS	ASSIGNMENT COMMAND

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is through connected in both

directions

10.2.4	Specific Message Contents:

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Transaction Identifier			Not used

Message Type

Ciphering key seq. number			As applicable from location update

Authent. parameter RAND			As applicable (128 bits)

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

CALL PROCEEDING (GSM 04.08, 9.3.3) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Repeat Indicator				Omitted

Bearer Capability 1				Omitted

Bearer Capability 2				Omitted

Facility					Omitted

Progress Indicator				Omitted

ALERTING (GSM 04.08, 9.3.1) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Facility					Omitted

Progress Indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Power level				Power control level 7

Channel mode				Speech full rate

CONNECT (GSM 04.08, 9.3.5) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Facility					Omitted

Progress Indicator				Omitted

Connected number				Omitted

Connected Subaddress			Omitted

User-user				Omitted

10.3	Generic call set-up procedure for Mobile Terminating Data calls

In the test procedures described in this ETS, unless otherwise stated in the test description, the Mobile Terminating Data call set-up procedure shall be as described in this section.

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.3.1	Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is in the MM state "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.3.2	Definition of System Information Messages:

See clause 10.1.2.

10.2.3	Procedure

An MS terminating call on a TCH shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	A call is set up according to the required

characteristics of the test procedure.

11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING

B13	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B14	MS		The MS is made to accept the call

in a way described in a

PICS/PIXIT statement.

B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	MS		The TCH is through connected in both

directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

10.3.4	Specific Message Contents:

PAGING REQUEST TYPE 1 (GSM 04.08, 9.1.22) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal Paging

Mobile Identity

Odd/even no of digits			As applicable

Type of Identity			TMSI

Identity digits			As applicable

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Transaction Identifier			Not used

Message Type

Ciphering key seq. number			As applicable from location update

Authent. parameter RAND			Arbitrarily selected

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

SETUP (GSM 04.08, 9.3.23) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

Bearer Capability

Radio Channel Requirement		HR or FR and declared as supported by the MS

(Not "Dual ...")

Connection Element			T or NT and declared as supported by the MS

(Not "Both ...")

Other parameters			Declared as supported by the MS

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Power level				Power control level 7

Channel mode				Proper data rate,

according to BC-IE included in the Set-Up

CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

10.4	Generic call set-up procedure for Mobile Originating Data calls

In the test procedures described in this ETS, unless otherwise stated in the test description, the Mobile Originating Data call set-up procedure shall be as described in this clause.

Note:	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.2.1	Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see [Annex 1 TC.2.1])

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is in the MM state "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.4.2	Definition of System Information Messages:

See clause 10.1.2.

10.4.3	Procedure

An MS originating call on a TCH shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	MS		"called number" entered

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"originating call".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following messages shall

be sent enciphered.

9	SS		SS starts ciphering.

10	MS -> SS	SETUP

11	SS -> MS	CALL PROCEEDING

12	SS -> MS	ALERTING

13	MS		An alerting indication as defined in an

PICS/PIXIT statement is given by the MS.

14	SS -> MS	ASSIGNMENT COMMAND

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is through connected in both

directions

10.4.4	Specific Message Contents:

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Transaction Identifier			Not used

Message Type

Ciphering key seq. number			As applicable from location update

Authent. parameter RAND			Arbitrarily selected.

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

CALL PROCEEDING (GSM 04.08, 9.3.3) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Repeat Indicator				Present if dual BC-IE received and negotiation

necessary, else omitted.

Bearer Capability 1				Present if negotiation necessary (i.e reception of "Dual"

or "Both" for RCR and CE parameters in SETUP),

else omitted

Radio Channel Requirement		FR (in case of "Dual FR (HR) preferred" received)

Connection element			T (in case of "Both T (NT) preferred" received)

Other parameters			Same as sent by the MS in the SETUP, where applicable

Bearer Capability 2				Present if dual BC-IE received and negotiation

necessary, else omitted

Radio Channel Requirement		FR (in case of "Dual FR (HR) preferred" received)

Connection element			T (in case of "Both T (NT) preferred" received)

Other parameters			Same as sent by the MS in the SETUP, where applicable

Facility					Omitted

Progress Indicator				Omitted

ALERTING (GSM 04.08, 9.3.1) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Facility					Omitted

Progress Indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Power level				Power control level 7

Channel mode				Either

Appropriate data rate according to:

the first BC-IE in the SETUP

or

in the first BC-IE in the CALL PROCEEDING if

negotiation took place

or

Speech if the first BC-IE Indicated Speech in the SETUP

CONNECT (GSM 04.08, 9.3.5) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Facility					Omitted

Progress Indicator				Omitted

Connected number				Omitted

Connected Subaddress			Omitted

User-user				Omitted