26.9	Structured procedures

Abbreviations for message names used in this section:

AUTHENTICATION REQ	=	AUTHENTICATION REQUEST

AUTHENTICATION RESP	=	AUTHENTICATION RESPONSE

CIPHER MODE COMMAND	=	CIPHERING MODE COMMAND

CIPHER MODE COMPLETE	=	CIPHERING MODE COMPLETE

26.9.1	Structured procedures / General

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. The term "structured procedure" is defined in TS GSM 04.08, section 7, where also examples of structured procedures are given.

The reason for this test purposes is twofold:

-	The behaviour of the MS in an elementary procedure may depend on the preamble which precedes the elementary procedure.

-	Structured procedures tested in this section are used in other parts of this Technical Specification as preambles to establish the initial conditions for other tests; correct behaviour of an implementation under test in a preamble is essential for the validity of a test.

[It can however be discussed whether to move the tests of this section into 26.8, where they would become CC tests with a certain preamble.]

Mobile originating and terminating calls are tested in cases of both early and late assignment of the traffic channel; in one of the cases call release initiated by the network is tested, in another one, call release initiated by the MS. The tests in this section only cover the successful outcome of elementary procedures (i.e. they do not deal with abnormal cases).

The emergency call service is tested in the following cases:

-	emergency call initiated in the idle, updated state with authentication and ciphering, for all supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network accepting the call, for one supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network rejecting the call, for one supported speech rate.

These tests on emergency calls are only applicable to an MS supporting speech.

For an MS supporting speech the test procedures in 26.9.2, 26.9.3, 26.9.4 and 26.9.5 are performed for speech (teleservice 11, telephony), once for each supported rate (full rate/half rate).

For an MS not supporting speech but supporting at least one teleservice, for each of the test procedures in 26.9.2, 26.9.3, 26.9.4, and 26.9.5 and each supported rate (full rate/half rate) a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice (note that this teleservice is never a dual service).

In cases where a mobile originated call for the tested teleservice can be initiated both

-	via the MMI and

-	via the R or S interface,

procedure 26.9.2 shall be performed when initiating the mobile originated call via the MMI and procedure 26.9.3 shall be performed when initiating the mobile originated call via an appropriate interface (R or S).

[Editorial note:

- Default conditions of 26.1 instead of 5.3.3 are now used.

- If the MS supports both F ad H TCH/F and THC/H, all tests are repeated.

- Immediate assignment: It is now left open which SDCCH is assigned. Most procedures required a SDCCH(4) in phase 1. However it is still specified that the SDCCH is non-hopping.

- The formulations concerning BC1 and BC2 etc are a little bit abstract. But at the moment never a dual service would be chosen for a test because a MS supporting a dual service also supports speech. The negotiation (presence of BC 1 in CALL CONFIRMED or CALL PROCEEDING) might concern the Radio channel requirement field and /or connection element. But 04.08 does at the moment not allow negotiation for RCR in CALL PROCEEDING.

]

26.9.2	Structured porcedures / MS originated call / early assignment

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Through connection not yet finally defined in SMG 3.

- Default parameters no more from 5.3.3, but from 5.3.1.

]

Purpose:

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

4)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an ASSIGNMENT COMMAND message allocating an appropriate TCH, after having completed the traffic channel early assignment procedure by replying with the ASSIGNMENT COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

5)	To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

Conformance requirement(S):

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

4,) The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

- attach the user connection to the radio path;

- return a CONNECT ACKNOWLEDGE message.

5)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

6)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

Reference(s):

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.1.1.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 04.08 section 5.4.4.

Conformance requirement 6:	TS GSM 04.08, section 3.5.1.

Related PICS/PIXIT Statement(s):

- 	Supported rates (full rate/half rate)

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	SS version

-	Supported teleservices

-	Classmark

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Method of Test:

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test:

1 minute

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The "called number" is entered

2	MS		If p1 = Y, the MS must display the

called number in the way defined in

PICS/PIXIT.

3	MS -> SS	CHANNEL REQUEST	Establishment cause is "originating

call and the network does not

set the NECI bit to 1".

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

6	SS -> MS	AUTHENTICATION REQ

7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following messages

shall be sent enciphered.

10	SS		SS starts ciphering.

11	MS -> SS	SETUP

12	SS -> MS	CALL PROCEEDING

13	SS -> MS	ALERTING

14	MS		Depending on the PICS, an alerting indication

is given

15	SS -> MS	ASSIGNMENT COMMAND

16	MS -> SS	ASSIGNMENT COMPLETE

17	SS -> MS	CONNECT

18	MS -> SS	CONNECT ACKNOWLEDGE

19	MS		The appropriate bearer channel is

through connected in both directions.

20	SS -> MS	DISCONNECT

21	MS -> SS	RELEASE

22	SS -> MS	RELEASE COMPLETE

23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

-

26.9.3	Structured procedures / MS originated call / late assignment

Purpose:

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

Conformance Requirement:

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4) The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

- attach the user connection to the radio path;

- return a CONNECT ACKNOWLEDGE message.

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 3.4.3.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 02.07.

Conformance requirement 6:	TS GSM 04.08 sections 5.2.1.1.6 and 5.1.3.

Related PICS/PIXIT statement(s):

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS:

The MS has a MO call in state U10, "active".

Method of Test:

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment.

Maximum Duration of Test:

30 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The "called number" is entered

2	MS

3	MS -> SS	CHANNEL REQUEST	Establishment cause is

"originating call and the

network does not set the NECI bit to 1".

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

6	SS -> MS	AUTHENTICATION REQ

7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

10	SS		SS starts ciphering.

11	MS -> SS	SETUP

12	SS -> MS	CALL PROCEEDING

13	SS -> MS	ALERTING

14	MS		Depending on the PICS, an alerting

indication is given.

15	SS -> MS	ASSIGNMENT COMMAND

16	MS -> SS	ASSIGNMENT COMPLETE

17	SS -> MS	CONNECT

18	MS -> SS	CONNECT ACKNOWLEDGE

19	MS		The appropriate bearer channel is

through connected in both directions.

Specific Message Contents:

-

26.9.4	Structured procedures / MS terminated call / early assignment

Test purpose:

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having sent a PAGING RESPONSE message on the allocated SDCCH, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message not containing a signal information element, returns a CALL CONFIRMED message.

2)	To verify that subsequently, the SS sending an ASSIGNMENT COMMAND message, the MS successfully continues a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE message, and

b)	by continuing the call establishment by either sending one or two CONNECT messages (with equal N(SD)) or sending an ALERTING message, steps a) and b) being performed in any permitted interleaving.

3)	To verify that if after sending a CALL PROCEEDING message, the MS sends an ALERTING message during MTC establishment with early assignment, it generates an alerting indication.

4)	To verify that if an ALERTING had been sent, subsequently, when the user accepts the call (possibly internal action as declared in PICS/PIXIT statement), the MS returns a CONNECT message.

5)	To verify that the MS:

-	if the call is a speech call: after sending the CONNECT message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) CONNECT message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently, the MS can initiate call clearing by sending a DISCONNECT message.

7)	To verify that the MS in this phase of call release, upon receipt of a RELEASE message, returns a RELEASE COMPLETE message.

8)	To verify that subsequently the MS, upon receipt of a CHANNEL RELEASE message, disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

Conformance requirement(s):

1)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

2,3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated (the MS sends an ALERTING message and generates an alerting indication). ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

4)	An MS indicates acceptance of a MT call by sending CONNECT.

5)

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

6)	The MS initiates call clearing of an active call by sending a DISCONNECT message.

7)	The MS in this phase of call release, upon receipt of a RELEASE message, shall return a RELEASE COMPLETE message.

8)	Subsequently the MS, upon receipt of a CHANNEL RELEASE message, shall disconnect the main signalling link.

Reference(s):

Conformance requirement 1:		TS GSM 04.08, section 5.2.2.3.1.

Conformance requirements 2,3:	TS GSM 04.08, section 5.2.2.3.2, 3.4.3.1.

Conformance requirement 4:		TS GSM 04.08 section 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.6 and 5.1.3.

Conformance requirements 6,7,8:	TS GSM 04.08, section 5.4.

Related PICS/PIXIT statement(s):

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Method of Test:

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test:

1 minute

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	Message does not contain the

signal IE.

11	MS -> SS	CALL CONFIRMED

12	SS -> MS	ASSIGNMENT COMMAND

A13	MS -> SS	ASSIGNMENT COMPLETE	During step 12 or

between step 12 and step 13

the MS may send a CONNECT message

on the old channel.

A14	MS -> SS	CONNECT	If during step 12 or

between step 12 and step A13

the MS has send a CONNECT message

on the old channel, both CONNECT

messages must have equal N(SD).

B13	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel.

B14	MS -> SS	ALERTING

B15	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B16	MS		The MS is made to accept the call

in the way described in a

PICS/PIXIT statement.

B17	MS -> SS	CONNECT

18	MS		If the call is a speech call,

the TCH shall be through connected

in both directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

20	MS		If the call is a data call,

the TCH shall be through

connected in both

directions.

21	MS		The MS is made to release the call.

22	MS -> SS	DISCONNECT

23	SS -> MS	RELEASE

24	SS -> MS	RELEASE COMPLETE

25	SS --> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

-

26.9.5	Structured procedures / MS terminated call / late assignment

Test purpose:

1)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

2)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

3)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

4)	To verify that subsequently after receipt of an ASSIGNMENT COMMAND, the MS sends an ASSIGNMENT COMPLETE message.

5)	To verify that subsequently the MS

-	if the call is a speech call: after sending the ASSIGNMENT COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) ASSIGNMENT COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

Conformance Requirement:

TP1,TP2:	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call. The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

TP3:	The MS indicates acceptance of a call by sending a CONNECT message.

TP4:	ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

TP5:

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirementS 1, 2, 3:		TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 4:			TS GSM 04.08, section 3.4.3.1.

Conformance requirement 5:			TS GSM 04.08, section 5.2.2.9.

Related PICS/PIXIT statement(s):

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS:

CC state U10-call active.

Method of Test:

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and a MT call is established with late assignment (after CONNECT).

Maximum Duration of Test:

40 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	Message contains the signal IE.

11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING

B13	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B14	MS		The MS is made to accept the call

in the way described in a

PICS/PIXIT statement.

B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	MS		If the call is a speech call,

the TCH shall be through connected

in both directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

20	MS		If the call is a data call,

the MS shall

through connect the TCH

in both directions.

Specific Message Contents:

-

26.9.6	Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

If the procedures tested in this section are not correctly implemented in the MS, establishment, maintenance and clearing of connections might fail in the essential case of emergency calls.

The tests of this section are only applicable to an MS supporting speech.

26.9.6.1	Structured procedures / emergency call / idle updated

26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate

[Editorial notes:

- Cross references to phase 1:

26.9.6.1.1 was 26.8.2.4

26.9.6.1.2 was 26.8.2.5

26.9.6.2.1 was 26.8.2.6

26.9.6.2.2 was 26.8.2.7

]

Test Purpose:

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

6)	To verify subsequent correct performance of a connect procedure.

7)	To verify that subsequently the MS has through connected the TCH in both directions.

8)	To verify that the call is cleared correctly.

Conformance Requirement:

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5),6)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

7)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

8)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2.

For conformance requirement 4:

TS GSM 04.08, section 5.2.1.2.

For conformance requirement 5 and 6:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 7:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 8:

TS GSM 04.08, section 5.4.

Related PICS/PIXIT Statement(s):

- Speech supported (Y/N).

- Supported rate for speech: (p1 = F/H, F).

- Classmark.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Method of Test:

The MS is made to initiate an emergency call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test:

1 minute

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered

3	MS -> SS	CHANNEL REQUEST	Establishment cause is

emergency call establishment.

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The

CM service type IE indicates

"emergency call establishment".

6	SS -> MS	AUTHENTICATION REQ

7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

10	SS		SS starts ciphering.

11	MS -> SS	EMERGENCY SETUP	If p1 = F/H, the message must contain

one bearer capability IE indicating

in the radio channel requirement field

"dual rate/half rate preferred" or

"dual rate/full rate preferred".

If p1 = F, the message must either

contain no bearer capability IE or

contain one bearer capability IE

indicating in the

radio channel requirement field

"full rate channel".

12	SS -> MS	CALL PROCEEDING

13	SS -> MS	ALERTING

14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is that one

indicated by the EMERGENCY SETUP

message, if that message did not

offer a choice, and the rate is the

preferred one else.

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is

through connected in both directions.

19	SS -> MS	DISCONNECT

20	MS -> SS	RELEASE

21	SS -> MS	RELEASE COMPLETE

23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

-

26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate

The test is performed if the MS supports both TCH/HS and TCH/FS see PICS/PIXIT statement).

It identical to the test in 26.9.2.1 except that the assigned TCH has the non-preferred rate.

[Editorial comments: Is it clear that a dual rate MS offers the choice?

]

26.9.6.2	Structured procedures / emergency call / idle, no IMSI

26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case

[Ed. notes:

- Ph. 1 11.10 required the CKSN stored in the SIM. CR needed?

- Test ref. in ph 1 is 26.8.2.6.

Test Purpose:

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment".

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

Conformance Requirement:

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.2.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

Related PICS/PIXIT Statement(s):

- Speech supported (Y/N).

- Classmark.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Method of Test:

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test:

1 minute.

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered

3	MS -> SS	CHANNEL REQUEST	Establishment cause is

"emergency call".

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The

CM service type IE indicates

"emergency call establishment".

The mobile identity IE specifies the

IMEI of the MS.

The cipher key sequence number IE

indicates "no key is available".

The mobile station classmark IE is as

specified by the manufacturer in a

PICS/PIXIT statement.

4	SS -> MS	CM SERVICE ACCEPT

11	MS -> SS	EMERGENCY SETUP

12	SS -> MS	CALL PROCEEDING

13	SS -> MS	ALERTING

14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one

indicated by the EMERGENCY SETUP

message.

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is through connected in both

directions.

19	SS -> MS	DISCONNECT

20	MS -> SS	RELEASE

21	SS -> MS	RELEASE COMPLETE

23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

-

26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case

[Ed. notes:

- Ph. 1 11.10 required the CKSN stored in the SIM. CR needed?

- Test ref. in ph 1 is 26.8.2.7.

- It is clarified how the SS checks the MS abandoning the emergency call: 5 seconds checking after the CM SERVICE REJECT, but still in dedicated mode, 20 seconds after the disconnection of the main signalling link.

- In order to have a well defined final state, the SS disconnects the main signalling link (forced release).

Test Purpose:

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency call establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE REJECT message from the SS, the MS abandons the emergency call establishment.

Conformance Requirement:

1)	The MS in the "idle, no IMSI" state (no SIM inserted), after the number 112 has been entered, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	In the situation at the end of test purpose 2, when the MS receives a CM SERVICE REJECT message, it shall abandon the emergency call.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4

For conformance requirement 3:

TS GSM 04.08 section 3.4.7, TS GSM 04.08 section 4.5.1.1.

Related PICS/PIXIT statement(s):

- Speech supported (Y/N).

- Classmark.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Method of Test:

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. The SS responds to the CM SERVICE REQUEST from the MS with a CM SERVICE REJECT message specifying in the reject cause IE the reject cause value "IMEI not accepted". The SS then verifies for during 5 seconds that the MS does not send a layer 3 message. Then the call is cleared by the SS. The SS verifies during 20 seconds after disconnection of the main signalling link that the MS does not initiate an RR connection establishment.

Maximum Duration of Test:

1 minute.

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered

3	MS -> SS	CHANNEL REQUEST	Establishment cause is

"emergency call".

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The

CM service type IE indicates

"emergency call establishment".

The mobile identity IE specifies the

IMEI of the MS.

The cipher key sequence number IE

indicates "no key is available".

The mobile station classmark IE is as

specified by the manufacturer in a

PICS/PIXIT statement.

4	SS -> MS	CM SERVICE REJECT	the reject cause IE specifies

reject cause value #5,

"IMEI not accepted".

5	SS		During 5 seconds, the SS verifies

that the MS does not send L3 messages.

6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

7	SS		During 20 seconds, the SS verifies

that the MS does not initiate an

RR connection establishment.

Specific Message Contents:

-

26.9.7	Default contents of messages

[Messages have to reviewed as messages may still change in 04.08. Shall the tests also apply to phase 2+? If so, e.g. new IEs would have to be accepted by SS.]

ALERTING (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

User-user 				Omitted

SS version				Omitted

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

Power Command				As in section 26.1.1

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				normal event

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		Correct for given SRES

CALL CONFIRMED

Information element			Value/remark

Repeat indicator				Omitted

Bearer capability 1 			Omitted if the MS only supports TCH/F. Included if the

MS supports both TCH/F and TCH/H; in that case, the

"radio channel requirement" field specifies "dual rate/half

rate preferred" or "dual rate/full rate preferred" or "half

rate channel", all other parameters are same as in the

bearer capability 1 IE of the received SETUP message.

Bearer capability 2				Omitted

Cause					Omitted

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1				Omitted if the SETUP message did not specify in the

bearer capability 1 IE a connection element value "both,

transparent preferred" or "both, non-transparent

preferred". Otherwise included; in that case the

connection element specifies the value that is appropriate

for the selected teleservice (either value "transparent" or

value "non transparent (RLP)"), all other parameters are

same as in the bearer capability 1 IE of the received

SETUP message.

Bearer Capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			Answer to paging (100)

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHER MODE COMPLETE

Information element			Value/remark

Mobile equipment identity			Omitted

CM SERVICE ACCEPT

Information element			Value/remark

none but message head

CM SERVICE REQUEST

Information element			Value/remark

CM service type				Mobile originating call

establishment or packet mode

connection establishment

Ciphering key sequence number		CKSN of the MS

Mobile station classmark 2			as given by PICS.

Mobile identity				TMSI of MS

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number				Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

Connected subaddress			Omitted

User-user				Omitted

SS version				Omitted

CONNECT ACKNOWLEDGE

Information element			Value/remark

none but message head

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

Information element			Value/remark

Ciphering key sequence number		Value assigned to MS in the initial conditions

Spare half octet				(spare bits)

Mobile station classmark 2			as given by PICS

Mobile identity				specifies TMSI of MS

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1				Appropriate for the teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for teleservice selected for the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (SS to MS)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1				Appropriate for teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

Signal						Omitted

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Appropriate for the teleservice selected for the test

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Appropriate for the teleservice selected for the test

HLC repeat indicator			Appropriate for the teleservice selected for the test

High layer compatibility i			Appropriate for the teleservice selected for the test

High layer compatibility ii			Appropriate for the teleservice selected for the test

User-user				Omitted