20	Cell selection and reselection tests

In the following paragraphs are described the general behaviour of the SS and some explanatory text concerning the nature of the tests in this section.

The SS transmits one BCCH carrier per cell. These are referred as carrier 1, carrier 2, etc.

Unless otherwise stated in the method of test, in all of the tests of this section, the SS in continuously paging the MS on all carriers, except that at every response to paging the SS stops paging on that carrier and starts or continues on all other carriers. The SS continues the establishment of a dedicated resource and then immediately performs normal call clearing.

After the final procedural step has been completted, the SS shall wait [30] seconds and monitors for any false random accesses on the test RF carriers.

Since the conformance requirements of most of the tests in this section can not be tested explicitly, testing is done implicitly by testing the MS behaviour from its responses to SS.

The conformance requirements are direct extracts from the core recommendations. However, for each test only the relevant part of the paragrahp/sentence in question have been included to the conformance requirements. This explains the "..." marks in the conformance requirements.

To guarantee the sufficient marging for C1 values the minimum values used are +8 or -8, above and below zero, respectively. Also, for the same reason the relatives differences of C1 (and C2) values are at least 5.

Note :	Random selection of carrier can not be used because of repeatability of tests.

The default values of the system information data fields are given in table 20.1. The differencies from the defaults values are given in the beginning of each test.

Table 20.1.: Default values of the system information fields

Parameter	GSM 04.08	Abbr.	Normal Setting

reference

Cell channel description	10.5.2.1	 -	Any values

MAX retrans	10.5.2.29	 -	1

TX-integer	10.5.2.29	 -	Any value

CELL_BAR_QUALIFY	10.5.2.35	CBQ	0

CELL_BAR_ACCESS	10.5.2.29	CBA	0 (not barred)

AC CN	10.5.2.29	AC	All 0

RE	10.5.2.29	RE	0 (re-establishment allowed)

NCC	10.5.2.2	NCC	Any value

Cell Identity	10.5.1.1	 -	Any value

MCC, MNC	10.5.1.3	PLMN	MS Home PLMN

LAC	10.5.1.3	LAC	1111 (Hex)

ATT	10.5.2.11	 -	Any values

BS_AG_BLKS_RES	10.5.2.11	 -	Any values

T3212	10.5.2.11	 -	Any values

BS_PA_MFRMS	10.5.2.11	BPM	5 frames

Cell Options	10.5.2.3	 -	Any values

CELL_RESELECT_HYSTERESIS	10.5.2.4	CRH	4 dB

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Max. output power of MS

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-90 dBm

CELL_RESELECT_OFFSET	10.5.2.35	CRO	0

TEMPORARY_OFFSET	10.5.2.35	TO	0

PENALTY_TIME	10.5.2.35	PT	0

BA ARFCN	10.5.2.22	BA	All 0 except :

For GSM ARFCNs

3,9,18,25,41,43,49,50,54,58,

62,66,70,80,92,124

For DCS1800 ARFCNs

512,543,568,589,602,641,662,

683,696,711,732,754,794,851,

870,871,872,884

20.1	Basic cell selection

20.1.1	Definition and applicability

Cell selection is a process in which a MS, whenever a new PLMN is selected, attempts to find a suitable cell of that PLMN to camp on. The process ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink. Once the MS is camped on a cell, access to the network is allowed.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.1.2	Conformance requirement

1.	When optional storage of BCCH carrier information is not used, the MS shall search all RF channels in the system (124 for GSM, 174 for E-GSM and 374 for DCS1800); GSM 05.08, 6.2

2.	MS shall be able to calculate correctly the path loss criterion parameter C1 used for cell selection and reselection; GSM 05.08, 6.4.

3.	There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

(i)	It should be a cell of the selected PLMN

(ii)	It should not be "barred" (see Section 3.5.1)

(iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in Section 3.6.

Initially the MS looks for a cell which satisfies these 4 constraints ("suitable cell") by checking cells in descending order of received signal strength. If a suitable cell is found, the MS camps on it; GSM 03.22, 3.2.1.

Note :	Criteria (iii) is not applicable for Cell Selection

4.	MS shall be able to select the correct (fourth strongest) cell and be able to respond to paging on that cell within 30 seconds when the three strongest cells are not suitable. This assumes ideal radio conditions; GSM 05.08, 6.1.

20.1.3	Test purpose

1.	To verify that MS meets conformance requirement 1.

2.	To verify that MS meets conformance requirement 2.

3.	To verify that MS meets conformance requirement 3.

4.	To verify that MS meets conformance requirement 4.

20.1.4	Method of test

20.1.4.1	Initial conditions

Parameters changed from Default values Table

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	48 / -65	38 / -75	33 / -80	23 / -90	OFF	OFF

CBA	1	0	0	0

RXLEV_ACCESS_MIN

(dBm)	-90	-70	-100	-95

MNC			001

NCC			02

C1	25	-5	20	5

C2	25	-5	20	5

20.1.4.2	Procedure

a)	The MS is switched on

b)	The SS monitors all RA requests from MS on carriers 1 to 4

c)	The MS is switched off

d)	Carrier 1 is modified to have CBA=0 (cell not barred) and the MS is switched on

20.1.5	Test requirements

a)	MS shall respond only on carrier 4 within 30 s of the completion of the MS boot time as declared by the manufacturer, since this is the only carrier which fulfils all the conformance criteria.

b)	After test step d) the MS shall access on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer

Note :	The assumption is made that if the MS selects carrier 4 over carrier 2, then the C1 parameter is being calculated correctly.

Note :	By using random ARFCN values for carriers 1 to 4 in this test, it is virtually ensured that the MS does indeed search all available carriers

Note :	Test steps c) and d) are to verify that the MS calculates C1 correctly when there are more than one C1 > 0.

20.2	Cell selection with varying signal strength values

20.2.1	Definition and applicability

For definition see conformance requirement.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.2.2	Conformance requirement

The MS shall ... take readings of received RF signal strength ... and calculate the received level average for each. The averaging is based on at least five measurement samples per RF carrier spread over 3 to 5 s, the measurement samples from the different RF carriers being spread evenly during this period; GSM 05.08, 6.2.

20.2.3	Test purpose

1.	To verify the MS performs at least five RF signal strength measurements per averaging period.

2.	To verify the averaging period is between 3 and 5 seconds.

3.	To verify that the measurement samples are evenly spread over the averaging period

20.2.4	Method of test

20.2.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	23 / -90	66 / -47	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBuV emf() / dBm)	13 / -100	13 / -100

C1	10	53

20.2.4.2	Procedure

a)	The SS transmits on carriers 1 and 2. After 1.2 seconds carrier 2 reduces its transmit level to -97 dBm (16 dBuV emf( )). After a further 2.2 seconds, carrier 2 increases its transmit level again to -47 dBm (66 dBuV emf( )). Switching of carrier 2 continues with these levels and duty cycle until the end of the test.

b)	The MS is switched on

c)	The SS monitors all RA requests from MS on carriers 1 to 3

20.2.5	Test requirements

The MS shall respond only on carrier 2.

Note :	With the selected levels and duty cycle it can be guaranteed that a "good" MS passes the test even at the worst case situation, which happens with 3 s averaging time if 4 out of 5 samples hit the low signal level (result is an average level of 26 dBuV which is still 6 dB above carrier 1 level).

Note :	With the selected levels and duty cycle the probability that a "bad" MS (i.e. MS that averages over shorter period than 3 s) failes the test is maximised. However, it can not be guaranteed that all the MSs not fulfilling the conformance requirement of averaging or uniform sampling will fail this test.

Note :	The 50 dB change in the levels is needed to guarantee a good MS pass in all situations. However, it has to be verified that the SS is capable of doing a 50 dB step with a short settling time (compared to the sampling).

20.3	Basic cell reselection

20.3.1	Definition and applicability

While camped on a cell of the selected PLMN the MS may need to select a different cell in order to fulfil the normal service state. This ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.3.2	Conformance requirement

1.	While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

(iii)	The cell camped on (current serving cell) has become barred.

(iv)	There is a better cell (in terms of the path loss criterion C2) in the same LA, or a much better cell in another LA of the selected PLMN (using the CRH parameter).

The MS will then reselect a new cell in order to fulfil the process goal.; GSM 03.22, 4.5.

Note :	Criterion (i) is tested in section 20.6 (Cell reselection when C1(serving cell) < 0 for 5 seconds

Note :	Criterion (ii) is tested section 20.15 (Downlink Signalling Failure test)

Note :	Criterion (v) is tested in section 20.4.

Note :	Criterion (vi) is tested in section 20.14 (location area not allowed test)

2.	There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

(ii)	It should not be "barred"

(iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. GSM 03.22, 3.2.1.

Note :	Criterion (i) is not relevant for cell reselection and for cell selection it is tested in section 20.1.

Note :	Criterion (iii) is tested in section 20.14. (location area not allowed for national roaming)

Note :	Criterion (iv) refers to the C2 parameter

3.	MS shall be able to calculate correctly the path loss criterion parameter C2 used for cell reselection; GSM 05.08, 6.4.

4.	.... the MS shall calculate the value of ... C2 for the serving cell and recalculate ... C2 values for non serving cells (if necessary). The MS shall then check whether:

ii)	the calculated value of C2 for a nonserving suitable cell exceeds the value of C2 for the serving cell ..., except in the case of the new cell being in a different location area in which case the C2 value for the new cell shall exceed the C2 value of the serving cell by at least CELL_RESELECT_HYSTERESIS dB as defined by the BCCH data from the current serving cell, .... This indicates that it is a better cell; GSM 05.08, 6.6.2.

20.3.3	Test purpose

1.	To verify that if the current serving cell becomes barred, cell reselection is triggered in the MS.

2.	To verify that MS makes cell reselection if the value of C2 for a non-serving suitable cell (in the same LA) exceeds the value of C2 for the serving cell

3.	To verify that the MS takes into account the CELL_RESELECT_OFFSET, TEMPORARY_OFFSET and PENALTY_TIME parameters when calculating C2

4.	To verify that the MS takes into account the CELL_RESELECT_HYSTERESIS parameter when deciding on cell reselection to a cell in a different location area

5.	To verify that an MS will not perform cell reselection and camp on a neighbour cell which is barred.

20.3.4	Method of test

20.3.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	53 / -60	43 / -70	33 / -80	23 / -90	OFF	OFF

RXLEV_ACCESS_MIN

(dBuv emf() / dBm)	33 / -80	33 / -80	23 / -90	13 / -100

Penalty time	11111	11111	30 s	60 s

CRH	10 dB	10 dB	10 dB	10 dB

CRO	20	5	5	10

Temporary offset	10	10	15	20

LAC				different

from other

carriers

C1	20	10	10	10

C2	0	5	0 -> 15	0 -> 20

20.3.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 to 4

c)	60 seconds (30 seconds after penalty time of carrier 4 expires) after the MS accesses on carrier 3, the SS shall change the CBA parameter on carriers 2, 3 & 4 to 1

d)	The SS monitors all RA requests from MS on carriers 1 to 4

20.3.5	Test requirements

a)	The MS should respond firstly on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer. It shall then reselect and respond on carrier 2.

b)	The MS shall then reselect and access onto Carrier 3 after 30 seconds (penalty time of carrier 3) but shall not respond on carrier 4 after longer penalty time due to CRH parameter.

c)	The MS shall access on carrier 1 within 50 seconds of the CBA parameters on carriers 2, 3 and 4 being set to 1, but the MS shall not respond on carriers 2 or 4.

Note :	The access should not take longer than 40 s. (30 seconds to read own channel BCCH, 3 * 2.4 seconds to synchronise to and decode the BCCH data for carriers 4, 2 and 1), 50 s is a safe time to wait,

20.4	Cell reselection timings

20.4.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.4.2	Conformance requirement

At least every 5 s the MS shall calculate the value of ... C2 for the serving cell and re-calculate ... C2 values for non serving cells (if necessary). The MS shall then check whether:...

ii)	The calculated value of C2 for a non-serving suitable cell exceeds the value of C2 for the serving cell for a period of 5 seconds, ...

In case ii) above, cell reselection shall not take place if there was a cell reselection within the previous 15 seconds. Cell reselection for any other reason shall take place immediately, but the cell that the MS was camped on shall not be returned to within 5 seconds if another suitable cell can be found.; GSM 05.08, 6.6.2.

20.4.3	Test purpose

1.	To verify that MS does not make cell reselection if value of C2 for a nonserving suitable cell exceeds the value of C2 for the serving cell for a period of less than 5 seconds,

2.	To verify that MS makes cell reselection if value of C2 for a non-serving suitable cell exceeds the value of C2 for the serving cell for a period of 5 seconds,

3.	To verify that MS does not make cell reselection within 15 seconds of the previous cell reselection in case the value of C2 of the non-serving suitable cell exceeds the value of C2 of the serving cell.

4.	To verify that MS makes cell reselection immediately in case of any other reason than that in test purpose 3. eg:

"While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

(v)	A random access attempt is still unsuccessful after "Max retrans" repetitions; "Max retrans" being a parameter broadcast on BCCH." (GSM 03.22, 4.5)

20.4.4	Method of test

20.4.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	56 / -57	46 / -67	46 / -67	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBuV emf() / dBm)	29 / -84	33 / -80	38 / -75

Max. Retrans	00	00	00

C1	27	13	8

C2	27	13	8

(??? Carrier 3  is meant to be "another suitable cell" in the conformance requirement, but it needs  to be checked if the usage is OK???)

20.4.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 and 2.

c)	After the SS has received an access on carrier 1, increase transmit level on carrier 2 to 70 dBuV emf( ) for 4 s so that C2 becomes 37, then drop level back to original.

Note :	With the selected RF level values and assuming a correct RF level sampling and averaging, the C2 of carrier 2 does not exceed that of carrier 1 for longer than about 4 s.

d)	The SS monitors all RA requests from MS on carriers 1 and 2.

e)	Increase transmit level on carrier 2 to 70 dBuV emf( ) so that C2 becomes 37 and wait for access from MS on carrier 2.

f)	As soon as an access is received on carrier 2, the transmit level of carrier 1 shall be increased to 76 dBuV emf( ) so that C2 becomes 43.

g)	The SS monitors all RA requests from MS on carriers 1 and 2, but shall not respond to any RA request on carrier 1. The SS shall always respond normally on carrier 2.

20.4.5	Test requirements

a)	The MS accesses on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

b)	There should be no access on carrier 2 within 25 seconds. (5 s for rxlev averages, 5 s for calculation of C2 (absolute worst case scenario), 5 s for C2 check duration and 10 s for possible faulty access).

c)	The MS shall access on carrier 2 within 25 seconds of increasing the level in step (e).

d)	The MS should not access carrier 1 within 13 s (15 seconds for reselection and 2 seconds for possible differences in random access/paging timings) of the MS access on carrier 2.

e)	The MS shall then access carrier 1. (This access should probably be fairly soon after the expiry of the 15 s, probably safe to allow say 20s.)

f)	The MS shall not access on carrier 2 within 5 seconds of accessing on carrier 1 in test step (g).

20.5	Priority of cells

20.5.1	Definition and applicability

In general, cell prioritisation is a means of encouraging MSs to select some suitable cells in preference to others. Since the priority comparison is only between suitable cells, prioritisation does not affect coverage. Operators may prefer a certain type of cell not to be selected unless it is the only suitable type. For example, umbrella cells due to their large frequency reuse distance, or microcells because the MS could be travelling too fast for them.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.5.2	Conformance requirement

1.	During cell selection a cell with low priority indication will only be selected if a suitable cell of normal priority cannot be found; GSM 03.22, 3.5.2.1.

2.	Table 1a. Parameters affecting cell priority for cell selection

CELL_BAR QUALIFY	CELL_BAR ACCESS	Cell selection priority	Status for cell reselection

0	0	normal	normal

0	1	barred	barred

1	0	low	normal (see note 2)

1	1	low	normal (see note 2)

;GSM 05.08, Table 1.a

3.	If all the following conditions are met then the "Cell selection priority" and the "Status for cell reselection" shall be set to normal:

- the cell belongs to the MS HPLMN

- the MS is in cell test operation mode

- the CELL_BAR_ACCESS is set to "1"

- the CELL_BAR_QUALIFY is set to "0"

- the Access Control class 15 is barred

;GSM 05.08, Table 1.a

20.5.3	Test purpose

1.	To verify that MS meets conformance requirement 1.

2.	To verify that MS meets conformance requirement 2.

3.	To verify that MS meets conformance requirement 3.

20.5.4	Method of test

20.5.4.1	Initial conditions

The MS is in cell test operation mode

Note :	The cell test mode is a mode of operation defined in SIM administrative data field (see 11.11). It is used to set up a cell for test purposes without being used by normal subscribers.

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	53 / -60	43 / -70	33 / -80	23 / -90	OFF	OFF

RXLEV_ACCESS_MIN

(dBuv emf() / dBm)	13 / -100	13 / -100	13 / -100	13 / -100

CRO	0	20	0	20

TO	40	0	0	20

PT	100 s (4)	11111	11111	60 s (2)

CBA	0	1	1	0

CBQ	1	1	0	0

Access class 15	barred	barred	barred	barred

C1	40	30	20	10

C2	0 -> 40	10	20	10 -> 30

20.5.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 to 4.

20.5.5	Test requirements

a)	The MS shall access onto carrier 3 within 30 seconds of the boot up time of MS

b)	The MS shall access onto carrier 4 within 20 seconds of expiry of penalty time on carrier 4

c)	The MS shall access onto carrier 1 within 20 seconds of expiry of penalty time on carrier 1

d)	MS shall not access on carrier 2 during this test

Note :	The accesses to carriers 4 and 1 should not take much longer than 10 s after expiry of the penalty time (5 s for rxlev averages, 5 s for C2 check duration, 2.4 s for sync and BCCH data read on new carrier) 20 seconds is a safe time to wait.

20.6	Cell reselection when C1(serving cell) < 0 for 5 seconds

20.6.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.6.2	Conformance requirement

1.	At least every 5 s .... the MS shall ... check whether:

i)	The path loss criterion (C1) for current serving cell falls below zero for a period of 5 seconds. This indicates that the path loss to the cell has become too high; 05.08, 6.6.2.

2.	While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

(i)	The path loss criterion parameter C1 (see Section 3.6) indicates that the path loss to the cell has become too high.; GSM 03.22, 4.5.

20.6.3	Test purpose

1.	To verify that when the MS does not make cell reselection if value of C1 for the serving cell falls below zero for a period of less than 5 seconds

2.	To verify that the MS makes cell reselection if value of C1 for the serving cell falls below zero for a period of greater than 5 seconds

20.6.4	Method of test

20.6.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	49 / -64	28 / -85	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBuV emf() / dBm)	33 / -80	18 / -95

C1	16	10

C2	16	10

Note :	This test assumes that the averaging of the serving cell RF level is a running average.

Note :	The RXLEV_ACCESS_MIN level for carrier 1 has to be above the RF level of carrier 2 to guarantee that the reselection is done because C1 of carrier 1 goes below zero and not because C2 of carrier 2 becomes greater than C2 of carrier 1.

20.6.4.2	Procedure

a)	The MS is switched on.

b)	After 30 seconds, SS reduces signal level on carrier 1 to -100 dBm / 13 dBuV emf() for 4 s. Then, raises back to -64 dBm / 49 dBuV emf().

c)	After a further 30 seconds, SS reduces signal level on carrier 1 to -100 dBm / 13 dBuV emf()

20.6.5	Test requirements

a)	The MS shall access on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer. There shall be no access on carrier 2 within this time.

b)	There shall be no access on carrier 2 in the 30 seconds following the signal level changes in test step b).

c)	The MS shall access on carrier 2 within 20 seconds following the signal level changes in test step c).

Note :	The access should not take much longer than 10 s (5 s for rxlev averages, 5 s for C1 check duration on carrier 1, 2.4 s for sync and BCCH data read on carrier 2) 20 seconds is a safe time to wait.

20.7	Running average of the surrounding cell BCCH carrier signal levels

20.7.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.7.2	Conformance requirement

1.	Whilst in idle mode an MS shall continue to monitor all BCCH carriers as indicated by the BCCH allocation (BA - See table 1). A running average of received level in the preceding 5 to

Max. {5 , ((5 * N + 6) DIV 7) * BS_PA_MFRMS / 4}

seconds shall be maintained for each carrier in the BCCH allocation. N is the number of non-serving cell BCCH carriers in BA and the parameter BS_PA_MFRMS is defined in GSM 05.02; GSM 05.08, 6.6.1.

2.	The same number of measurement samples shall be taken for all non-serving cell BCCH carriers and the samples allocated to each carrier shall as far as possible be uniformly distributed over each evaluation period.; GSM 05.08, 6.6.1

20.7.3	Test purpose

1.	To verify that if the MS calculates a received level average (over 5 seconds) for a non-serving suitable cell which results in the value of C2 exceeding the value of C2 for the serving cell, then cell reselection takes place to the non-serving cell.

2.	By using suitable varying levels of signal strength for non serving cells, to verify that samples on adjacent non serving channels are as far as possible distributed uniformly over each evaluation period.

20.7.4	Method of test

20.7.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	40 / -73	33 / -80	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBuv emf() / dBm)	30 / -83	33 / -80

N	5	5

BS_PA_MFRMS	5	5

C1	10	0

C2	10	0

Note :	N=5 and BS_PA_MFRMS=5 such that 5 s is the bigger value and hence the averaging limit

20.7.4.2	Procedure

a)	The MS is switched on

b)	After 30 seconds, the level of Carrier 2 is increased to -50 dBm for 10 seconds and shall then switch between -80 and -50 dBm every 5 seconds.

c)	The SS monitors all RA requests from MS on carriers 1 & 2

20.7.5	Test requirements

1.	The MS accesses on carrier 1 within 30 s.of the completion of the MS boot time as declared by the manufacturer.

2.	After the level of carrier 2 has been increased for the first time, the MS shall access on carrier 2 within 20 seconds

Note :	The access should not take much longer than 10 s (5 s for rxlev average, 5 s for C2 check duration, 2.4 s for sync and BCCH data read in carrier 2). 20 s is a safe  time to wait

3.	After accessing carrier 2 the MS shall not access again on carrier 1

20.8	Running average of the serving cell BCCH carrier signal level

ASK SMG2 IF THIS IS A RUNNING AVERAGE OR NOT??? the present test is not a running average test

20.8.1	Definition and applicability

The MS is required to monitor continuously the BCCH carrier signal level of the serving cell (and to compare it to the BCCH carrier signal levels of the non-serving cells) to guarantee that it is camped on the best suitable cell in terms of signal strength.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.8.2	Conformance requirement

1.	For the serving cell, receive level measurement samples shall be taken at least for each paging block of the MS and the receive level average shall be determined using samples collected over a period of 5 s or five consecutive paging blocks of that MS, whichever is the greater period.

At least 5 received level measurement samples are required per receive level average value. New sets of receive level average values shall be calculated as often as possible; GSM 05.08, 6.6.1.

20.8.3	Test purpose

1.	To verify that if the BCCH carrier signal level average value of the serving cell changes so that the value of C2 for a non-serving suitable cell exceeds the value of C2 for the serving cell, the MS makes cell reselection.

20.8.4	Method of test

20.8.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	58 / -55	48 / -65	OFF	OFF	OFF	OFF

RxLev_Access_Min

(dBuV emf() / dBm)	43 / -70	43 / -70

BS_BA_MFRMS	0	0

Note :	With BS_BA_MFRMS value 0, i.e. the minimum value the parameter can have, the averaging time of the MS becomes 5 s (see conformance requirement 1).

20.8.4.2	Procedure

a)	The MS is switched on

b)	The SS waits the MS to access on carrier 1 within 30 s.

Note :	The C1s of he two carriers are 15 for Carrier 1 and 5 for carrier 2.

c)	The SS starts to change the RF signal level of carrier 1 in 5 s periods as follows: -66 dBm for 2 s, -56 dBm for 3 s, -66 dBm for 2 s, -56 dBm for 3 s, -66 dBm for 2 s, etc.

Note :	In this step the C2 of carrier 1 is 10, and the C2 of carrier 2 is 5.

d)	The SS waits at least 25 s to check the the MS does not make access to carrier 2.

Note :	5 s for rxlev averages, 5 s for calculation of C2, 5 s for C2 check duration and 10 s for possible faulty access (25 s).

e)	The SS starts to change the RF signal level of carrier 1 in 5 s periods as follows: -81 dBm for 2 s, -71 dBm for 3 s, -81 dBm for 2 s, -71 dBm for 3 s, -81 dBm for 2 s, etc.

Note :	In this step the C2 of carrier 1 is -5 and the C2 of carrier 2 is 5.

f)	The SS waits the MS to access on carrier 2 within 25 s.

Note :	The access should not take much longer than 15 s (5 s for rxlev averages, 5 s for calculation of C2, 5 s for C2 to be lower for reselection, 2.4 s for sync and BCCH data read in carrier 2), 25 s is a safe time to wait.

20.8.5	Test requirement

The MS shall calculate correctly BCCH carrier signal level average values for the serving cell in steps d) and f) and act accordingly.

20.9	Updating the list of six strongest neighbor carriers and decoding the BCCH information of a new carrier on the list

20.9.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.9.2	Conformance requirement

1.	The list of the 6 strongest non-serving carriers shall be updated at least as often as duration of the running average defined for measurements on the BCCH allocation of the serving cell, and may be updated more frequently; GSM 05.08, 6.6.1.

2.	When the MS recognizes that a new BCCH carrier has become one of the 6 strongest, the BCCH data shall be decoded for the new carrier within 30 seconds; GSM 05.08, 6.6.1.

20.9.3	Test purpose

1.	To verify that MS meets conformance requirement 1.

2.	To verify that MS meets conformance requirement 2.

20.9.4	Method of test

20.9.4.1	Initial conditions

Six BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm)	53 / -60	48 / -65	43 / -70	38 / -75	33 / -80	28 / -85

BA ARFCN	8 carriers	8 carriers	8 carriers	8 carriers	8 carriers	8 carriers

on BA list	on BA list	on BA list	on BA list	on BA list	on BA list

BPM	3	3	3	3	3	3

Note :	The combination of 8 carriers on the BA list and BS_PA_MFRMS = 3 gives a value 4.5 from the averaging tme formula, which being smaller than 5 s leads to averaging time of 5 seconds. Hence 5 seconds is also the updating time of the list of six strongest neighbour carriers.

Note :	The C1 (and C2) values of the carriers 1 to 6 are 30, 25, 20, 15, 10 and 5, correspondingly.

20.9.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits the MS to access on carrier 1 (max 30 s)

c)	The SS switches on a seventh BCCH carrier whose signal level is 38 dBuV emf() / -75 dBm, and with the default system information contents except that RXLEV_ACCESS_MIN is -110 dBm.

Note :	The C2 value of carrier 7 is 35 due to the low RXLEV_ACCESS_MIN which is transmitted in the BCCH data.

c)	The SS monitors all RA requests from the MS on all carriers (max 55 s).

20.9.5	Test requirements

The MS shall access on carrier 7 within 55 s of switching carrier 7 on.

Note :	The access should not take much longer than 45 s (5 s to notice new strongest carrier in top 6 (because the updating time for six strongest is 5 s), 30 s to read BCCH, 5 s for calculation of C2, 5s for C2 to be lower for reselection, 2.4 s for sync and BCCH data read in carrier 2), 55 s is a safe time to wait.

20.10	Decoding the serving cell BCCH information

20.10.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.10.2	Conformance requirement

1.	The MS shall attempt to decode the full BCCH data of the serving cell at least every 30 seconds; GSM 05.08, 6.6.1.

20.10.3	Test purpose

1.	To verify that if the BCCH data of the serving cell changes such that the value of C1 for the serving cell falls below zero the MS makes cell reselection.

20.10.4	Method of test

20.10.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.10.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits the MS to access on carrier 1 (max 30 s)

c)	The SS changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 1 to be -67 dBm.

Note :	With the above change to RXLEV_ACCESS_MIN, the C1 of carrier 1 becomes -8.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 50 s).

20.10.5	Test requirements

The MS shall access on carrier 2 within 50 s of the change of RXLEV_ACCESS_MIN on carrier 1.

Note :	The access should not take much longer than 40 s (30 s for decode of BCCH, 5 s for calculation of C1, 5 s for C1<0 for reselection, 2.4 s for sync and BCCH data read in carrier 2), 50 s is a safe time to wait.

20.11	Decoding the BCCH information of the neighbor carriers on the list of six strongest neighbor carriers

20.11.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.11.2	Conformance requirement

1.	The MS shall attempt to decode the BCCH data block that contains the parameters affecting cell reselection for each of the 6 strongest non─serving cell BCCH carriers at least every 5 minutes; GSM 05.08, 6.6.1.

20.11.3	Test purpose

1.	To verify that if the BCCH data of the non-serving cell, which is in the list of six strongest neighbour cells, changes such that the value of C2 for that cell exceeds the value of C2 of the serving cell, the MS makes cell reselection.

20.11.4	Method of test

20.11.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.11.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits for the MS to access on carrier 1 (max 30 s).

c)	The SS changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 2 to be -100 dBm.

Note :	With the above change the C2 of carrier 2 becomes 20 whereas the C2 of carrier 1 stays at 15.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 320 s).

20.11.5	Test requirements

The MS shall access on carrier 2 within 320 s of the change in the BCCH data of carrier 2.

Note :	The access should not take much longer than 310s (300 s for decode of BCCH of carrier 2, 5 s for calculation of C2, 5s for C2 of carrier 2 to be higher than C2 of carrier 1 for reselection, 2.4 s for sync and BCCH data read in carrier 2), 320 s is a safe time to wait.

20.12	Decoding the BSIC of the neighbor carriers on the list of six strongest neighbor carriers

20.12.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.12.2	Conformance requirement

1.	The MS shall attempt to check the BSIC for each of the 6 strongest non-serving cell BCCH carriers at least every 30 seconds, to confirm that it is monitoring the same cell. If a change of BSIC is detected then the carrier shall be treated as a new carrier and the BCCH data redetermined; GSM 05.08, 6.6.1.

20.12.3	Test purpose

1.	To verify that if the BSIC and the BCCH data of the non-serving cell, which is in the list of six strongest neighbour cells, changes such that the value of C2 for that cell exceeds the value of C2 of the serving cell, the MS makes cell reselection.

20.12.4	Method of test

20.12.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.12.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits for the MS to access on carrier 1 (max 30 s).

c)	The SS changes the BSIC of carrier 2 by changing the Base Station Colour Code (BCC) part of the BSIC. The SS also changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 2 to be -100 dBm.

Note :	It is left open how the BSIC is changed. A possible solution is to inverse the bits of the BCC part of the BSIC. In any case for the purpose of this test the Network Colour code (NCC) part of the BSIC shall not be changed.

Note :	With the above change to the BCCH data the C2 of carrier 2 becomes 20 whereas the C2 of carrier 1 stays at 15.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 80 s).

20.12.5	Test requirements

The MS shall access on carrier 2 within 80 s of the change in the BSIC value (and BCCH data) of carrier 2.

Note :	The access should not take much longer than 70s (30 s for check of BSIC on carrier 2, 30 s for decode of BCCH of carrier 2, 5 s for calculation of C2, 5s for C2 of carrier 1 to be lower than C2 of carrier 2, 2.4 s for sync and BCCH data read on carrier 2), 80 s is a safe time to wait.

20.13	Emergency calls

20.13.1	Definition and applicability

This test is applicable for GSM 900 and DCS 1800 MSs supporting speech.

20.13.2	Conformance requirement

1.	When in a limited service state and if not camped on a cell, the MS shall monitor the signal strength of all 124 (for GSM), all 174 (for E-GSM) or all 374 (for DCS 1800) RF channels, and search for a BCCH carrier which has C1>0 and which is not barred. When such a carrier is found, the MS shall camp on that cell, irrespective of the PLMN identity.

The MS shall perform cell reselection at least among the cells of the PLMN of the cell on which the MS has camped, according to the algorithm of GSM 03.22, except that a zero value of CELL_RESELECT_HYSTERESIS shall be used; GSM 05.08, 6.8.

2.	When in a limited service state, the MS shall be able to initiate emergency calls; GSM 05.08, 6.8.

20.13.3	Test purpose

1.	To verify that the MS shall be able to initiate emergency calls when no suitable cells of the selected PLMN are available, but at least one acceptable cell is available.

2.	To verify that the MS shall be able to initiate emergency calls without SIM card.

3.	To verify that the MS, when making the cell reselection, ignores the CELL_RESELECT_HYSTERESIS parameter values.

20.13.4	Method of test

20.13.4.1	Initial conditions

Three BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	33 / -80	OFF	OFF	OFF

RxLev_Access_Min

(dBuV emf() / dBm)	23 / -90	43 / -70	23 / -90

CELL_BAR_ACCESS	1 (barred)	0	0

MCC,MNC	forbidden	forbidden	forbidden

CELL_RESELECT_HYST	0	0	14 dB

C1	15	-10	10

Note :	All the BCCH carriers belong to the same PLMN, which is not the MS's home PLMN and is in the SIM's forbidden PLMN's list.

Note :	The MS should select carrier 3 since carrier 1 is barred and carrier 2 has C1<0.

No network selection shall be done during this test.

20.13.4.2	Procedure

a)	The MS is switched on.

b)	After 40 s an emergency call is initiated on the MS.

Note :	30 s for cell selection, 5 s for calculation of C2, 5s for C2 verification.

c)	The SS shall monitor the MS accesses on all carriers (max 10 s).

d)	The SS changes the CBA of carrier 1 to 0.

Note :	The MS should reselecet to carrier 1 because it should not take into account the CELL_RESELECT_HYST value of 14 but use 0 instead.

e)	After 40 s an emergency call is initiated on the MS.

Note :	30 s for cell selection, 5 s for calculation of C2, 5s for C2 verification.

f)	The SS shall monitor the MS accesses on all carriers (max 10 s).

g)	The MS is switched off, the SIM is removed and carrier 1 is changed back to barred (CBA = 1).

h)	The MS is switched on.

i)	After 40 s an emergency call is initiated on the MS.

Note :	30 s for cell selection, 5 s for calculation of C2, 5s for C2 verification.

j)	The SS shall monitor the MS accesses on all carriers (max 10 s).

20.13.5	Test requirements

a)	The MS shall respond on carrier 3 in test step c).

b)	The MS shall respond on carrier 1 in test step f).

b)	The MS shall respond on carrier 3 in test step j).

20.14	Cell reselection due to MS rejections "LA not allowed" & "National Roaming not allowed in this LA"

20.14.1	Definition and applicability

While camped on a cell of the selected PLMN the MS may need to select a different cell in order to fulfil the normal service state. This ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink.

The MS looks for suitable neighbour cells which satisfies 4 constraints including that It should not be in an LA which is in the list of "forbidden LAs for national roaming", or in the list of "forbidden LAIs for regional provision of service"

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.14.2	Conformance requirement

1.	There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

(iii)	It should not be in an LA which is in the list of "forbidden LAs for national roaming", or in the list of "forbidden LAIs for regional provision of service"

The MS will then reselect a new cell in order to fulfil the process goal.; GSM 03.22, 3.2.1 & 4.5.

2.	While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

(vi)	An LU request has been rejected with the cause "LA not allowed"; GSM 03.22, 4.5

3.	To prevent repeated attempts to have national roaming service on a not allowed LA, when the MS is informed that an LA is forbidden, the LA is added to a list of "forbidden LAs for national roaming" which is stored in the MS. This list is deleted when the MS is switched off or when the SIM is removed, GSM 03.22; 3.1.

Note :	LA stands for "Location Area" and LU stands for "Location Update".

20.14.3	Test purpose

1.	To verify that if an LU is rejected with cause "LA not allowed" that cell reselection is triggered in the MS

2.	To verify that if an LU is rejected with cause "LA not allowed" or "National Roaming not allowed in this LA", that the LAI of that cell is written into a forbiddenn list which prevents the MS reselecting onto another cell in that LA until powered off

3.	To verify that if an LAI is in a forbidden list in the MS, the list is deleted on powering off the MS.

20.14.4	Method of test

20.14.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBuV emf() / dBm )	63 / -50	53 / -60	63 / -50	53 / -60	OFF	OFF

RXLEV_ACCESS_MIN

(dBuV emf() / dBm)	23 / -90	23 / -90	43 / -70	43 / -70

CRH	0	0	0	0

LAC	H1111	H2222	H1111	H2222

ATT	1	1	1	1

C1	40	30	20	10

C2	40	30	20	10

20.14.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 & 2

c)	When the MS performs an IMSI attach onto carrier 1, the SS shall reject it with cause "LA not allowed"

d)	The SS shall accept any LU on carrier 2.

e)	10 seconds after completion of the LU procedure on carrier 2, the SS shall reduce the level of carrier 2 to 13 dBuV emf() so that C1<-10.

f)	The SS monitors all RA requests from MS on carriers 1 to 4.

g)	After the MS has successfully location updated onto carrier 4, the MS shall be switched off and on.

h)	The SS monitors all RA requests from MS on carriers 1 to 4 for 30 seconds.

i)	Repeat steps (a) to (h), but in step (c), the LU reject cause shall be "National Roaming not allowed in this LA"

20.14.5	Test requirements

a)	The MS should respond on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

b)	After LU reject, the MS shall then reselect and access onto Carrier 2 within 25 seconds.

c)	After reducing the level of carrier 2, the MS shall reselect and access onto carrier 4 within [25] seconds.

d)	After powering off and on the MS, it shall access onto carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

20.15	Downlink signalling failure

20.15.1	Definition and applicability

See conformance requirement.

20.15.2	Conformance requirement

The downlink signalling failure criterion is based on the downlink signalling failure counter DSC.  When the MS camps on a cell, DSC shall be initialized to a value equal to the nearest integer to 90/N where N is the BS_PA_MFRMS parameter for that cell (see GSM 05.02).  Thereafter, whenever the MS attempts to decode a message in its paging subchannel; if a message is successfully decoded DSC is increased by 1, (however never beyond the nearest integer to 90/N), otherwise DSC is decreased by 4. When DSC reaches 0, a downlink signalling failure shall be declared.

Note :	The network sends the paging subchannel for a given MS every BS_PA_MFRMS multiframes. The requirement for network transmission on the paging subchannel is specified in GSM 04.08. The MS is required to attempt to decode a message every time its paging subchannel is sent.

A downlink signalling failure shall result in cell reselection, GSM 05.08, 6.5.

20.15.3	Test purpose

1.	To verify that the MS fulfills the conformance requirement

20.15.4	Method of test

20.15.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	43 / -70	33 / -80	OFF	OFF	OFF	OFF

C1 = C2	20	10

Note :	The DSC counter will have a value 18 (90/5).

20.15.4.2	Procedure

a)	The MS is switched on.

b)	After 40 s the SS sends random data in four successive paging blocks to carrier 1 and then reverts to sending normal data.

Note :	Sending random, i.e. non-decodable data on four successive paging blocks should decrease the DSC to 2.

c)	The SS monitors all accesses on both carriers for 30 s.

d)	The SS sends random data in five successive paging blocks to carrier 1 and then reverts to sending normal data.

Note :	Sending random, data on five successive paging blocks should decrease the DSC to -2 and caues a cell reselection.

e)	The SS monitors all accesses on both carriers for 30 s.

20.15.5	Test requirements

a)	There shall be no access to carrier 2 at test step c).

b)	The MS shall access on carrier 2 at test step e).

20.16	Cell selection if no suitable cell found in 10 s

20.16.1	Definition and applicability

See conformance requirement.

20.16.2	Conformance requirement

If no suitable cell is found in cell reselection process within 10 seconds, the cell selection algorithm of GSM 03.22 shall be performed, GSM 05.08; 6.6.2.

20.16.3	Test purpose

1.	To verify that the MS fulfills the conformance requirement

20.16.4	Method of test

20.16.4.1	Initial conditions

One BCCH carrier is established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	43 / -70	OFF	OFF	OFF	OFF	OFF

C1 = C2	20

20.16.4.2	Procedure

a)	The MS is switched on.

b)	After the SS has received an access on carrier 1 (max 30 s) the SS reduces the transmit level of carrier 1 to 13 dBuV emf() (so that C1 of carrier 1 becomes -10) and turnes on a new carrier (carrier 2) at a level of 33 dBuV emf(). Carrier 2 shall not be in the MS BA list (i.e. it shall not be one of the carriers that MS has been monitoring after camped on carrier 1)

c)	The SS shall monitor all accesses on carriers 1 and 2 for 60 s.

Note :	The access on carrier 2 should not take longer than 50 s. (5 seconds to rxlev averages, 5 s for C1<0 duration, 10 s for searching another suitable cell, 30 s for cell selection), 60 s is a safe time to wait,

20.16.5	Test requirements

a)	The MS shall access on carrier 2 at test step c) within 60 s.