34	Short message service (SMS)

Ref : GSM 03.40, 04.11 (point to point)

GSM 03.41, 04.12 (cell broadcast)

34.1	General

The purpose of these tests is to verify that the MS can handle GSM functions when submitting or receiving Short Messages (SM) between MS and a short message service centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services. The SMS point to point services shall work in an active MS at any time independent of whether or not there is a speech or data call in progress. The SMS cell broadcast service only works when the MS is in idle mode.

Since the timer TC1M currently is not standardised, the value of TC1M shall be declared by the manufacturer (to be used in 34.2.1 and 34.2.2).

The manufacturer shall declare whether SMS messages are stored in the SIM and/or the ME. This shall be referred to as the SMS message store in the following tests.

34.2	Short message service point to point

34.2.1	SMS mobile terminated

34.2.1.1 	Conformance requirements

An active MS shall be able to receive short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the SC, confirming that the MS has received the short message.

Reference

GSM 03.40; 3.1

34.2.1.2 	Test purpose

To verifiy the ability of a MS to receive and decode the SMS where provided for the point to point service.

34.2.1.3 	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

The number of CP-DATA retransmissions allowed is set to 1.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Support for call control state U10

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS initiates the transmission of a short message using a paging request. Upon response of the MS to the paging the SS assigns an SDCCH, authenticates the MS and activates ciphering. Then the SS establishes SAPI 3 by sending a SABM frame with SAPI 3 on the SDCCH.

When a UA frame (SAPI 3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA.

b)	The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

c)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

d)	Steps a), b) and c) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

e)	Steps a) and b) are repeated but the CP-DATA messages from the MS is not acknowledged. The SS then initiates the channel release.

f)	The SMS message store shall be cleared.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH.

When a UA frame (SAPI-3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA. The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

h)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release. The SMS message store shall be cleared.

i)	Steps g) and h) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

j)	Step g) is repeated but the CP-DATA messages from the MS are not acknowledged. The SS then initiates the channel release.

k)	A data or speech call is established on a TCH with the SS and the state U10 of call control shall be entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the speech call is cleared by the SS with a disconnect message. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA message. The information element of the CP-DATA message is RP-DATA.

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared.

l)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the SS waits 15 seconds. During this time the speech call shall be cleared from the MS. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA message. The information element of the CP-DATA message is RP-DATA.

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

9		SS	SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3) 	MS shall respond to SABM in step 10

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

13		SS 	Waits max 25 seconds for CP-ACK

14	MS -> SS	CP-ACK

15		SS 	Waits max 60 seconds for RP-ACK RPDU

16	MS -> SS	CP-DATA	Contains RP-ACK RPDU

17	SS -> MS	CP-ACK	Within TC1M after step 16, no further

CP-DATA messages

18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

19		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

20	SS -> MS	PAGING REQUEST

21	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

22	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

23	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

24	SS -> MS	AUTHENTICATION REQ

25	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

26	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

27	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

28		SS	SS starts ciphering.

29	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

30	MS -> SS	UA (SAPI=3)	The MS shall respond to the SABM

31	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

32		SS 	Waits max 25 seconds for CP-ACK

33	MS -> SS	CP-ACK

34		SS 	Waits max 60 seconds for RP-ACK RPDU

35	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK

RPDU

36		SS	First CP-DATA message not acknowledged by

SS

37	MS -> SS	CP-DATA	Repeated CP-DATA from MS within twice

TC1M, contains RP-ACK RPDU

38	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged

within TC1M after step 37, no further

CP-DATA messages

39	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

40		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

41	SS -> MS	PAGING REQUEST

42	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

43	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

44	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

45	SS -> MS	AUTHENTICATION REQ

46	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

47	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

48	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

49		SS	SS starts ciphering.

50	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

51	MS -> SS	UA (SAPI=3) 	The MS shall respond to the SABM

52	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

53		SS 	Waits max 25 seconds for CP-ACK

54	MS -> SS	CP-ACK

55		SS 	Waits max 60 seconds for RP-ACK RPDU

56	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK

RPDU

57		SS	First CP-DATA message not acknowledged by

SS

58	MS -> SS	CP-DATA	Repeated CP-DATA from MS within twice

TC1M, contains RP-ACK RPDU

59		SS 	Second CP-DATA message not acknowledged

by SS

60		MS	The MS shall not send any more CP-DATA

messages

61	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

62		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

63		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

64	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

65	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

66	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

67		SS	Waits max 25 seconds for CP-ACK

68	MS -> SS	CP-ACK

69		SS	Waits max 60 seconds for RP-ACK RPDU

70	MS -> SS	CP-DATA	Contains RP-ACK RPDU

71	SS -> MS	CP-ACK	Within TC1M after step 70, no further

CP-DATA messages

72	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

73		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

74		MS	Clear the SMS message store

75		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

76	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

77	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

78	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

79		SS	Waits max 25 seconds for CP-ACK

80	MS -> SS	CP-ACK

81		SS	Waits max 60 seconds for RP-ACK RPDU

82	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK

RPDU

83		SS	First CP-DATA message not acknowledged by

SS

84	MS -> SS	CP-DATA	Repeated CP-DATA message within twice

TC1M, contains RP-ACK RPDU

85	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged

within TC1M after step 84, no further

CP-DATA messages

86	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

87		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

88		MS	Clear the SMS message store

89		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

90	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

91	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

92	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

93		SS	Waits max 25 seconds for CP-ACK

94	MS -> SS	CP-ACK

95		SS	Waits max 60 seconds for RP-ACK RPDU

96	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK

RPDU

97		SS	First CP-DATA message not acknowledged by

SS

98	MS -> SS	CP-DATA	Repeated CP-DATA message within twice

TC1M, contains RP-ACK RPDU

99		SS 	Second CP-DATA message not acknowledged

by SS

100		MS	The MS shall not send any more CP-DATA

messages

101	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

102		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

103		MS	Clear the SMS message store

104		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

105	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

106	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

107	SS -> MS	DISCONNECT	The speech call is cleared by the SS. The call

clearing is continued on the FACCH in parallel

to the following exchange of messages related

to SMS.

108	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

109		SS	Waits max 25 seconds for CP-ACK

110	MS -> SS	CP-ACK

111		SS	Waits max 60 seconds for RP-ACK RPDU

112	MS -> SS	CP-DATA	Contains RP-ACK RPDU

113	SS -> MS	CP-ACK	Within TC1M after step 112, no further

CP-DATA

114	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

115		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

116		MS	Clear the SMS message store

117		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

118	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

119	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

120		SS	Waits 15 seconds

121		MS	During the SS wait time in step 120 the

speech call shall be cleared from the MS. The

call clearing is continued on the FACCH in

parallel to the following exchange of messages

related to SMS.

122	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVE

TPDU)

123		SS	Waits max 25 seconds for CP-ACK

124	MS -> SS	CP-ACK

125		SS	Waits max 60 seconds for RP-ACK RPDU

126	MS -> SS	CP-DATA	Contains RP-ACK RPDU

127	SS -> MS	CP-ACK	Within TC1M after step 126, no further

CP-DATA

128	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

129		MS	The MS shall indicate that an SM has arrived.

If the MS provides the functionality to display

MT messages, it is checked that the correct

message is displayed

130		MS	Clear the SMS message store

NOTE 1:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS DELIVER

Information element	Comment	Value

TP-MTI 	MT

TP-MMS 	more messages are waiting

TP-RP	no reply path	0

TP-SRI	no status report returned	0

TP-OA 	an international number coded E.164

TP-PID 	default	0

TP-DCS 	default alphabet	0

TP-SCTS 	any legal value (cf. GSM 03.40)

TP-UDL 		160

TP-UD (140 octets)	text of message (160 characters)

NOTE 2:	The 160 characters shall include at least one occurrence of each character in the default alphabet (see GSM 03.40 annex 2).

34.2.2	SMS mobile originated

34.2.2.1	Conformance requirements

An active MS shall be able to submit short message TPDU (SMS-SUBMIT) at any time, independently of whether or not there is a speech or data call in progress.

Reference

GSM 03.40; 3.1

34.2.2.2	Test purpose

To verify that the MS is able to correctly send a short message where the SMS is provided for the point to point service. The yesy also verifies that the MS is capable of simultaneously receive a network originated SM whilst sending a mobile originated SM.

34.2.2.3	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MO/PP

Description of the basic procedures to display a mobile originated short message

Support for state U10 of call control

The value of timer TC1M

Maximum length (characters) of a mobile originated short message

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS shall be set up to send a SM to the SS. The SS responds to the channel request message by allocating an SDCCH. The SS answers correctly to the SABM on SAPI 0 and then performs the authentication and ciphering procedures.

b)	The SS responds with a UA frame SAPI-3 to the MS.

c)	The SS responds to the CP-DATA from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message.

d)	The SS sends a channel release message to the MS.

e)	Steps a) and b) are repeated with the SS configured not to send the CP-ACK. After a duration of 3*TC1M the SS initiates channel release.

f)	Steps a) and b) are repeated. On receipt of the CP-DATA from the MS the SS sends a CP-ERROR message within TC1M containing a "Network Failure" cause. Then the SS initiates channel release.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The MS is setup to send an SM to the SS. After the reception of the CM SERVICE REQUEST, the SS sends a CM SERVICE ACCEPT message. The SS responds with a UA frame SAPI-3 to the SABM with SAPI-3 received from the MS.

h)	The SS responds to the CP-DATA from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message. Then the SS sends a channel release message to the MS.

i)	Step g) is repeated. The SS is configured not to send the CP-ACK message. After a duration of 3*TC1M the SS initiates channel release.

j)	The SS is configured to receive a mobile originated SM. Step b) is repeated and, using the end of the CP-DATA message from the MS as a trigger, the SS sends a SM to the MS. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

k)	The MS is set up to send an SM to the SS. On receipt of the CM SERVICE REQUEST the SS sends a CM SERVICE REJECT message with the reject cause set to "Service Option not supported" or "Service Option temporarily out of order". After 5 seconds the SS initiates channel release.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

3	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0. CM

service type set to "Short message transfer"

4	SS -> MS	AUTHENTICATION REQ

5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

8		SS	SS starts ciphering.

9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

10	SS -> MS	UA (SAPI=3)

11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

12	SS -> MS	CP-ACK	Sent within TC1M after step 11

13	SS -> MS	CP-DATA	Contains RP-ACK RPDU

14		SS			Waits max 25 seconds for CP-ACK

15	MS -> SS	CP-ACK

16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

17	MS -> SS	DISC (SAPI=0)	MS shall respond to channel release

18	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

20	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0.

21	SS -> MS	AUTHENTICATION REQ

22	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

23	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

24	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

25		SS	SS starts ciphering.

26	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

27	SS -> MS	UA (SAPI=3)

28	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

29		SS 	SS configured not to send CP-ACK

30	MS -> SS	CP-DATA	Repeated CP-DATA message TC1M after step

28

31		MS	The MS shall retransmit CP-DATA only once

32	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a

duration of 3*TC1M.

33	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

34	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

35	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service

type set to "short message transfer"

36	SS -> MS	AUTHENTICATION REQ

37	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

38	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

39	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

40		SS	SS starts ciphering.

41	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

42	SS -> MS	UA (SAPI=3)

43	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

44	SS -> MS	CP-ERROR	Sent within TC1M containing "Network

Failure" cause.

45	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

46		SS	A data or speech call is established on a TCH

and the state U10 of call control is entered.

47		MS	The MS is set up to send an SM

48	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM

service type set to "short message transfer"

49	SS -> MS	CM SERVICE ACCEPT

50	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

51	SS -> MS 	UA (SAPI=3)

52	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

53	SS -> MS	CP-ACK	Sent within TC1M after step 52

54	SS -> MS	CP-DATA	Contains RP-ACK RPDU

55		SS	Waits max 25 seconds for CP-ACK

56	MS -> SS	CP-ACK

57	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

58	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release

59	SS		A data or speech call is established on a TCH

and the state U10 of call control is entered.

60	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM

service type set to "short message transfer"

61	SS -> MS	CM SERVICE ACCEPT

62	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

63	SS -> MS 	UA (SAPI=3)

64	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

65		SS	SS configured not to send CP-ACK

66	MS -> SS	CP-DATA	Repeated CP-DATA message TC1M after step

64

67		MS	The MS shall retransmit CP-DATA only once

66	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a

duration of 3*TC1M.

67	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release

68	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

69	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

70	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service

type set to "short message transfer"

71	SS -> MS	AUTHENTICATION REQ

72	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

73	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

74	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

75		SS	SS starts ciphering.

76	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

77	SS -> MS	UA (SAPI=3)

78	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

79		SS	The SS sends an SM to the MS triggered by

the end of the CP-DATA message from the MS

79	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

80		MS	The MS shall correctly receive the SM and

indicate that a message has arrived. If the MS

provides the functionality to display MT

messages, it is checked that the correct

message is displayed. In the MO case the MS

shall send the CP-ACK message with

transaction identifier assigned to this transfer.

In the MT case the MS shall send a CP-ACK

message and a CP-DATA message containing

the RP-ACK RPDU. The transaction identifier

shall be the same as chosen by the SS for the

MT transfer.

81	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

82	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

83	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service

type set to "short message transfer"

84	SS -> MS	CM SERVICE REJ	Reject cause set to "Service Option not

supported" or "Service Option temporarily out

of order"

85		MS	The MS shall not establish SAPI-3

86	SS -> MS	CHANNEL RELEASE	Sent 5 seconds after CM SERVICE REJ

NOTE 3:	Time values for SS wait times are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS SUBMIT

Information element	Comment	Value

TP-MTI 	MO

TP-VPF 	"TP-VP field present and integer

represented (relative)"

TP-DA 	an E164 number

TP-PID 	all zeros	0

TP-DCS 	standard alphabet	0

TP-VP 	24 hours

TP-UDL 	as applicable

TP-UD (140 octets max)	maximum number of characters

(text of message) as defined

by the manufacturer (see PICS/PIXIT)

34.2.3	Test of memory full condition and memory available notification:

The Memory Available Notification provides a means for the MS to notify the network that it has memory available to receive one or more short messages. The SMS status field in the SIM contains status information on the "memory available" notification flag.

34.2.3.1	Conformance requirement

1.	When a mobile terminated message in Class 2, the MS shall ensure that the message has been transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a protocol error message if the short message cannot be stored in the SIM and there is other short message storage available in the MS. If all the short message storage in the MS is already in use, the MS shall return "memory capability exceeded".

2.	When the MS rejects a short message due to lack of available memory capability the need to transfer notification shall be stored in the SIM.

3.	If the memory capability becomes available because memory is cleared, the value of the memory capability exceeded notification flag in the SIM is read. If the flag is set, the MS notifies the network that memory capability is now available. After a positive acknowledgement from the network, the ME unsets the memory capability exceeded notification flag in the SIM.

References

GSM 03.40, 9.2.3.10, GSM 03.38, 4.

GSM 03.40, 10.3 (operation 14)

GSM 03.40, 10.3 (operation 14)

34.2.3.2	Test purpose

1.	To verify that the MS sends the correct acknowledgement when its memory in the SIM becomes full.

2.	To verify that the MS sends the correct acknowledgement when its memory in the ME and the SIM becomes full, and sets the "memory exceeded" notification flag in the SIM.

3.	To verify that the MS performs the "memory available" procedure when its message store becomes available for receiving short messages, and only at this moment.

34.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The SMS message storage shall be empty.

The MS shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least one record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

Whether SMS messages are stored in the SIM and/or the ME

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	step a) of 34.2.5.3 (test of Class 2 Short Messages) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

b)	a Class 1 Short Message is sent to the MS.

c)	step b) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

d)	a Short Message is sent to the MS with the DCS field of the SMS-DELIVER TPDU set to 0.

e)	the SS prompts the operator to read a short message and to remove it from the message store of the MS.

f)	the SS waits for a CHANNEL REQUEST from the MS, and sends an IMMEDIATE ASSIGNMENT allocating an SDCCH.

g)	the SS answers correctly to the SABM on SAPI 0.

h)	the SS answers correctly to the SABM on SAPI 3.

i)	the SS answers to the RP-SMMA from the MS with a CP-DATA containing a RP-ACK RPDU.

j)	after the MS has acknowledged the CP-DATA with a CP-ACK, the SS releases the channel with a CHANNEL RELEASE message. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been unset on the SIM.

k)	step i) is repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU), Class 2 Short Message

13	SS 		Waits max 25 seconds for CP-ACK

14	MS -> SS	CP-ACK

15	SS 		Waits max 60 seconds for RP-ACK RPDU

16	MS -> SS	CP-DATA	Contains RP-ACK RPDU

17	SS -> MS	CP-ACK	Within TC1M after step 16

18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Step 12-18 is repeated until MS sends a

negative acknowledgement (RP-ERROR) in step

16. The RP-ERROR RPDU cause field

shall be "Protocol error, unspecified" if there is

message capability in the ME, or "Memory

capability exceeded" if there is no message

capability in the ME. If the total memory store

of the MS is full, the ME shall set the

"memory capability exceeded" notification

flag.

19	SS -> MS	PAGING REQUEST

20	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

21	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

22	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

23	SS -> MS	AUTHENTICATION REQ

24	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

25	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

26	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

27	SS		SS starts ciphering.

28	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

29	MS -> SS	UA (SAPI=3)

30	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU), Class 1 Short Message

31	SS 		Waits max 25 seconds for CP-ACK

32	MS -> SS	CP-ACK

33	SS 		Waits max 60 seconds for RP-ACK RPDU

34	MS -> SS	CP-DATA	Shall contain RP-ACK RPDU if there is memory

capability in the ME. If not it shall contain

RP-ERROR RPDU which cause field shall be

"memory capability exceeded". If the total

memory store of the MS now becomes full at

this step, the ME shall set the

"memory cap. exceed" notification flag.

35	SS -> MS	CP-ACK	Within TC1M after step 34

36	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Step 19-36 is repeated until the MS sends an

RP-ERROR. The SIM simulator shall indicate if

the "memory capability exceeded" notification

flag has been set on the SIM.

37	SS -> MS	PAGING REQUEST

38	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

39	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

40	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

41	SS -> MS	AUTHENTICATION REQ

42	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

43	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

44	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

45	SS		SS starts ciphering.

46	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

47	MS -> SS	UA (SAPI=3)

48	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU) with TP-DCS set to 0

49	SS 		Waits max 25 seconds for CP-ACK

50	MS -> SS	CP-ACK

51	SS 		Waits max 60 seconds for RP-ACK RPDU

52	MS -> SS	CP-DATA	Shall contain RP-ERROR RPDU with error

cause "memory capability exceeded".

53	SS -> MS	CP-ACK	Within TC1M after step 52

54 	SS -> MS	CHANNEL RELEASE	The maun signalling link is released.

55	SS		Prompts the operator to read a short message

and to remove it from the message store of

the MS.

57	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which

can be completed with an SDCCH".

58	SS -> MS	IMMEDIATE ASSIGNMENT	SS alloctes an SDCCH

59	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. CM service

type information elementis set to "Short

message transfer".

60	SS -> MS	CM SERVICE ACCEPT (UA)	SAPI 0

61	MS -> SS	SABM (SAPI=3)	MS shall establish SAPI 3

62	SS -> MS	UA (SAPI=3)

63	MS -> SS	CP-DATA	Contains RP-SMMA RPDU

64	SS -> MS	CP-DATA	Contains RP-ACK RPDU

65	MS -> SS	CP-ACK	Acknowledge of CP-DATA containing the

RP-ACK RPDU. The ME shall unset the

"memory capability exceeded" notification flag

on the SIM.

66	SS -> MS	CHANNEL RELEASE	The main signalling link is released. The SIM

simulator shall indicate if the "memory

capability exceeded" notification flag has been

unset on the SIM.

67	SS -> MS	CP-DATA	Contains RP-ACK RPDU

68		MS	Shall not attempt to send a RP-SMMA RPDU.

This is verified by checking that the MS does

not send a CHANNEL REQUEST message with

the establishment cause "Other services

which can be completed with an SDCCH"

NOTE 4:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS-DELIVER in step 12

same as in 34.2.5.3

SMS-DELIVER in step 30

same as in 34.2.5.2

SMS-DELIVER in step 48

same as in 34.2.1

34.2.4	Test of the status report capabilities and of SMS-COMMAND:

34.2.4.1	Conformance requirement

The SMS offers the SC the capabilities of informing the MS of the status of a previously sent mobile originated short message. This is achieved by the SC returning a status report TPDU (SMS-STATUS-REPORT) to the originating MS.

References

GSM 03.40; 3.2.9

34.2.4.2	Test purpose

1)	To verify that the MS is able to accept a SMS-STATUS-REPORT TPDU.

2)	To verify that the MS is able to use the SMS-COMMAND functionality correctly and sends an SMS-COMMAND TPDU with the correct TP-Message-Reference.

34.2.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support of SMS MO/PP and MT/PP

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS is made to send a Mobile Originated short message as in steps a) to e) of test 34.2.2 	(SMS Mobile originated).

b)	The SS establishes a data link on SAPI-3 with the MS, then sends a CP-DATA message containing a RP-DATA RPDU itself containing an SMS-STATUS-REPORT TPDU.

c)	The SS sends a CHANNEL RELEASE message.

d)	The MS is made to send an SMS-COMMAND message enquiring about the previously submitted short message.

e)	The SS responds to the MS so as to enable it to establish a data link on SAPI-3 on an SDCCH.

f)	The SS acknowledges the CP-DATA message from the MS with a CP-ACK followed by a CP-DATA message containing an RP-ACK RPDU

g)	After receiving the CP-ACK from the MS, the SS releases the channel by using a CHANNEL RELEASE message.

h)	The MS is made to send an SMS-COMMAND message requiring to delete the previously submitted short message.

i)	step e) is repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services

requested by the mobile user"

2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

3	MS -> SS	CM SERVICE REQ	Message is contained in SABM.

4	SS -> MS	AUTHENTICATION REQ

5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

8		SS	SS starts ciphering.

9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

10	SS -> MS	UA (SAPI=3)

11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT

TPDU)

12	SS -> MS	CP-ACK	Sent within TC1M after step X

13	SS -> MS	CP-DATA	Contains RP-ACK RPDU

14		SS	Waits max 25 seconds for CP-ACK

15	MS -> SS	CP-ACK

16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

17	SS -> MS	PAGING REQUEST

18	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

20	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

22	SS -> MS	AUTHENTICATION REQ

23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

26	SS		SS starts ciphering.

27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

28	MS -> SS	UA (SAPI=3)

29	SS -> MS	CP-DATA	Contains RP-DATA RPDU

(SMS-STATUS-REPORT TPDU)

30	MS -> SS	CP-ACK

31	MS -> SS 	CP-DATA	Contains RP-ACK RPDU

32	SS -> MS	CHANNEL RELEASE

33		MS	The MS is made to send an SMS-COMMAND

message enquiring about the previously

submitted SM

34	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which

can be completed with an SDCCH".

35  	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

36	MS -> SS	CM SERVICE REQ	Message is contained in SABM.

37	SS -> MS	AUTHENTICATION REQ

38	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

39	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

40	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

41		SS	SS starts ciphering.

42	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

43	SS -> MS	UA (SAPI=3)

44	MS -> SS	CP-DATA	Contains RP-DATA RPDU

(SMS-COMMAND TPDU) which shall contain

the correct TP-MR

45	SS -> MS	CP-ACK

46	SS -> MS	CP-DATA	Contains RP-ACK RPDU

47	MS -> SS	CP-ACK

48 	SS -> MS	CHANNEL RELEASE

49		MS	The MS is made to send an SMS-COMMAND

message requiring to delete the previously

submitted SM.

50	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which

can be completed with an SDCCH".

51	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

52	MS -> SS	CM SERVICE REQ	Message is contained in SABM.

53	SS -> MS	AUTHENTICATION REQ

54	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

55	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

56	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

57		SS	SS starts ciphering.

58	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

59	SS -> MS	UA (SAPI=3)

60	MS -> SS	CP-DATA	Contains RP-DATA RPDU

(SMS-COMMAND TPDU) which shall contain

the correct TP-MR

61	SS -> MS	CP-ACK

62	SS -> MS	CP-DATA	Contains RP-ACK RPDU

63	MS -> SS	CP-ACK

64 	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS-STATUS-REPORT (SS to MS):

TP-MTI	SMS-STATUS-REPORT

TP-MR	same as previous SMS-SUBMIT

TP-MMS	no more messages	1

TP-RA	same as the Destination adress of the SMS-SUBMIT

TP-SCTS	any legal value (cf. GSM 03.40)

TP-RCT	any legal value (cf. GSM 03.40)

TP-ST		0

first SMS-COMMAND (MS to SS)

TP-MTI	SMS-COMMAND

TP-MR	TP-MR in previous SMS-SUBMIT plus "1"

TP-PID	default	0

TP-CT	Enquiry relating to previously submitted short message	0

TP-MN	not checked

TP-CDL	not checked

TP-CD	not checked

second SMS-COMMAND (MS to SS)

TP-MTI	SMS-COMMAND

TP-MR	TP-MR in previous SMS-COMMAND plus "1"

TP-PID	default	0

TP-CT	Delete previously submitted short message	2

TP-MN	not checked

TP-CDL	not checked

TP-CD	not checked

34.2.5	Test of message class 0 to 3

The tests under this section only apply to a MS capable of displaying short messages (see PICS/PIXIT).

34.2.5.1	Short message class 0

34.2.5.1.1	Conformance requirement

When a mobile terminated message is class 0 and the MS has the capability of displaying short messages, the MS shall display the message immediately and send an acknowledgement to the SC when the message has successfully reached the MS irrespective of whether there is memory available in the SIM or ME. The message shall not be automatically stored in the SIM or ME.

References

GSM 03.38, 4.

34.2.5.1.2	Test purpose

To verify that the MS will accept and display but not store a class 0 message, and that it will accept and display a class 0 message if its message store is full.

NOTE 5:	failure of this test in a mobile could cause it to reject a class 0 message when its SMS memory becomes full. This could lead to unwanted repetitions between the MS and the service center.

34.2.5.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS sends a class 0 message by using the method described in step a) of section 34.2.1 but with the TPDU described in this section.

b)	The MS message store shall be filled (for example by using the method of 34.2.3 (test of the memory available notification) with the same SMS-DELIVER TPDU.

c)	The SS sends a class 0 message as in step a).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

9		SS	SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU),

Class 0 Short Message

13	MS -> SS	CP-ACK

14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

15	SS -> MS	CP-ACK

16	SS -> MS	CHANNEL RELEASE

17		MS	The content of the short message shall be

displayed by the ME.

18		SS	The MS message store shall be filled

(for example by

using the method of 34.2.3) with the same

SMS-DELIVER TPDU.

19		MS	The MS shall not store the short

message. This can

be checked by verifying that it is impossible

to retreive any short messages from

the MS message

store.

20	SS -> MS	PAGING REQUEST

21	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

22	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

23	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

24	SS -> MS	AUTHENTICATION REQ

25	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

26	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

27	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

28		SS	SS starts ciphering.

29	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

30	MS -> SS	UA (SAPI=3)

31	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU),

Class 0 Short Message

32	MS -> SS	CP-ACK

33	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

34	SS -> MS	CP-ACK

35	SS -> MS	CHANNEL RELEASE

34		MS	The content of the short

message shall be displayed

by the ME.

Specific Message Contents:

SMS-DELIVER (containing a class 0 message) (SS to MS):

TP-MTI	SMS-DELIVER

TP-MMS	more messages are waiting	0

TP-RP	no reply path	0

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	0

TP-DCS	default alphabet, class 0	binary 1111 0000

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

34.2.5.2	Test of class 1 short messages

[Editorial note: It cannot be tested, whether the MS acknowledges the SM before or after having stored it]

This test shall apply to MSs which support

-	storing of received Class 1 Short Messages and

-	displaying of stored Short Messages.

34.2.5.2.1	Conformance requirement

When a mobile terminated message is class 1, the MS shall send an acknowledgement to the SC when the message has successfully reached the MS and can be stored, either in the ME or in the SIM.

References

GSM 03.38, 4.

34.2.5.2.2	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 1 message, i.e. that it stores the message in the ME or SIM and sends an acknowledgement (at RP and CP-Layer).

34.2.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

For storing of class 1 Short Messages, the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 1 to the MS as specified in section 34.2.1, step a).

b)	the Short Message is recalled (e.g. by means of the MMI).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

9		SS	SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU), Class 1 Short Message

13	MS -> SS	CP-ACK

14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

15	SS -> MS	CHANNEL RELEASE

16		MS	The short message shall be recalled

and displayed at the MS.

Specific Message Contents:

SMS-DELIVER

same as 34.2.1, but

TP-DCS	default alphabet, class 1	binary 11110001

34.2.5.3	Test of class 2 short messages

34.2.5.3.1	Definition and applicability

Class 2 Short Messages are defined as SIM specific, and the MS shall ensure that a message of this class is stored on the SIM.

This test shall apply to MSs which support

-	storing of received Class 2 Short Messages in the SIM and

-	displaying of Short Messages stored in the SIM.

34.2.5.3.2	Conformance requirement

When a mobile terminated message is Class 2, the MS shall ensure that the message has been correctly  transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a "protocol error, unspecified" error message if the short message cannot be stored in the SIM and there is other short message storage available at the MS.

Reference(s)

GSM 03.40, 9.2.3.10; GSM 03.38, 4. GSM 11.11, 10.3.3

34.2.5.3.3	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 2 message, i.e. that it stores the message correctly in the SIM, and if this is not possible, returns a protocol error message to the network.

34.2.5.3.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The ME message store shall be empty.

The ME shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least two free records and one full record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 2 to the MS as specified in section 34.2.1, step b).

b)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "OK" ('90 00').

c)	step a) is repeated.

d)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "memory problem" ('92 40').

e)	the SIM simulator indicates if an attempt was made in steps a) and c) to store the messages and if the messages are stored according to the requirement.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

9		SS	SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU), Class 2 Short Message

13	MS -> SS	CP-ACK

14		ME	The ME shall correctly store the

short message in a

free record of EFSMS in the SIM, i.e.

-	the ME shall use a free record

-	the first byte of the record

shall indicate "message received by MS

from network"

-	the TS-Service-Centre-Address shall be

correctly stored

-	the TPDU shall be identical

to that sent by the SS

-	bytes following the TPDU

shall be set to 'FF'

15		SIM	The SIM simulator returns the status

response "OK"

('90 00'). The SIM simulator shall indicate if an

attempt was made by the ME to store the

short message in the SIM.

16	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

17	SS -> MS	CHANNEL RELEASE

18	SS -> MS	PAGING REQUEST

19	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

20	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

21	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

22	SS -> MS	AUTHENTICATION REQ

23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

26		SS			SS starts ciphering.

27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

28	MS -> SS	UA (SAPI=3)

29	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU), Class 2 Short Message

30	MS -> SS	CP-ACK

31		ME	The ME shall attempt to store the

short message in a

free record of EFSMS in the SIM.

32		SIM	The SIM simulator returns the status

response "memory problem" ('92 40').

The SIM simulator shall

indicate if an attempt was made by the ME to

store the short message in the SIM.

33	MS -> SS 	CP-DATA	Contains RP-ERROR RPDU with error cause

"protocol error, unspecified".

34	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS-DELIVER

same as 34.2.1, but

TP-DCS	default alphabet, class 2	binary 11110010

34.2.5.4	Test of class 3 short messages

[Editorial note: For this test a configuration with LTE is needed. Therefore this test should be specified together with the other 07.05 tests.]

For further study

34.2.6	Test of short message type 0

[Editorial note: The only requirement for this message type is that it must be acknowledged. But this is a general requirement for all SMs, which means that there is no specific requirement for type 0 SMs to be tested]

For further study

34.2.7	Test of the replace mechanism for SM type 1-7

34.2.7.1	Definition and applicability

This test shall apply to MSs which support

-	Replace Short Messages and

-	display of received Short Messages.

34.2.7.2	Conformance requirement

Reference(s)

34.2.7.3	Test purpose

This procedure verifies the correct implementation of the replace mechanism for Replace Short Messages.

34.2.7.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	two different numbers n and m are drawn randomly between 1 and 7.  Two different addresses for TP-Originating-Address (TPOA1 and TPOA2) are drawn.  Two different addresses for RP-Originating-Address (RPOA1 and RPOA2) are drawn.

b)	the SS delivers a short message to the MS as specified in section 34.2.1 step a).  In the SMS-DELIVER TPDU, the TP-Protocol-Identifier parameter is "Replace Short Message Type n", the TP-Originating-Address is TPOA1, and the RP-Originating-Address is RPOA1.

c)	step b) is repeated but with a different TP-Originating-Address (TPOA2), and different contents of TP-User-Data in the SMS-DELIVER TPDU.  The other parameters are the same as in step b).

d)	step c) is repeated but with RPOA2 in the RP-Originated-Address, and contents of TP-User-Data different from the former two messages.  The other parameters are the same as in step c).

e)	step d) is repeated but with the TP-Protocol-Identifier equal to "Replace Short Message Type m", and contents of TP-User-Data different from the former three messages.  The other parameters are the same as in step d).

f)	step e) is repeated but the contents of TP-User-Data are different from that used in step e).

g)	the SS prompts the operator to display the Short Messages stored in the MS.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages

shall be sent enciphered.

9		SS	SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

TP-PID is "Replace Short Message

Type n", TP-OA is TPOA1 and

RP-OA is RPOA1

13	MS -> SS	CP-ACK

14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

15	SS -> MS	CHANNEL RELEASE

16	SS -> MS	PAGING REQUEST

17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

20	SS -> MS	AUTHENTICATION REQ

21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages

shall be sent enciphered.

24		SS	SS starts ciphering.

25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

26	MS -> SS	UA (SAPI=3)

27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)

TP-PID is "Replace Short Message Type n",

TP-OA is TPOA2 an RP-OA is RPOA1,

TP-UD different from step 12

28	MS -> SS	CP-ACK

29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

30	SS -> MS	CHANNEL RELEASE

31	SS -> MS	PAGING REQUEST

32	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

34	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

35	SS -> MS	AUTHENTICATION REQ

36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages

shall be sent enciphered.

39		SS	SS starts ciphering.

40	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

41	MS -> SS	UA (SAPI=3)

42	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)

TP-PID is "Replace Short Message Type n",

TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD

different from step 12 and 29

43	MS -> SS	CP-ACK

44	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

45	SS -> MS	CHANNEL RELEASE

46	SS -> MS	PAGING REQUEST

47	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

48	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

49	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

50	SS -> MS	AUTHENTICATION REQ

51	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

52	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

53	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

54		SS	SS starts ciphering.

55	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

56	MS -> SS	UA (SAPI=3)

57	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU) TP-PID is "Replace Short Message

Type m", TP-OA is TPOA2 and RP-OA is

RPOA2, TP-UD different

from step 12, 27 and 42

57	MS -> SS	CP-ACK

59	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

60	SS -> MS	CHANNEL RELEASE

61	SS -> MS	PAGING REQUEST

62	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

63	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

64	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

65	SS -> MS	AUTHENTICATION REQ

66	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

67	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

68	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages

shall be sent enciphered.

69		SS	SS starts ciphering.

70	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

71	MS -> SS	UA (SAPI=3)

72	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER

TPDU)

TP-PID is "Replace Short Message

Type m", TP-OA is TPOA2 and RP-OA is RPOA2,

TP-UD different from step 57

73	MS -> SS	CP-ACK

74	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

75	SS -> MS	CHANNEL RELEASE

76	SS		Prompts the operator to display the

ShortMessages stored in the MS. Only the

Short Messages delivered in step 12, 27 and

72 shall be retrievable and displayed

Specific Message Contents:

SMS-DELIVER

TP-MTI	SMS-DELIVER

TP-MMS	no more messages are waiting in SC	1

TP-RP	no Reply Path	0

TP-SRI	no Status Report returned	0

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	binary 01000xxx, xxx represents n resp. m (see test method 	description)

TP-DCS	default alphabet	0

TP-SCTS	the time when the message was submitted according to GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

34.2.8	Test of the reply path scheme

34.2.8.1	Definition and applicability

This test applies to MSs which support

-	reply procedures (the class of MSs for which this is mandatory is described in GSM 03.40, Annex 4)

-	displaying of received Short Messages and

-	submitting Short Messages.

Steps b) and d) are only executed for MSs which support storing of Short messages.

34.2.8.2	Conformance requirement

When a replying MS receives an original mobile terminated short message it has:

originating SME = TP-Originating Address in the SMS-DELIVER TPDU

original SC = RP-Originating Address in the RP-MT-DATA

When submitting the reply mobile originated short message, the replying MS should use parameters as follows:

TP-Destination Address in SMS-SUBMIT TPDU = originating SME

RP-Destination Address in RP-MO-DATA = original SC

Reference(s)

GSM 03.40 Annex 4, sect. 5, 6.

34.2.8.3	Test purpose

This procedure verifies that the MS is able to send a Reply Short Message back to the correct originating SME even if in the meantime it receives another Short Message.

34.2.8.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Support for Short message MO/PP

Description of the basic procedures to display a mobile terminated short message

Description of the basic procedures to send a mobile originated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message as specified in section 34.2.1, step b) with TP-Reply-Path set to 1.

b)	step a) is repeated but with

-	different TP-Originating-Address for the originating SME

-	different RP-Originating-Address for the original SC and

-	different message contents TP-User-Data.

c)	one of the two Short Messages is displayed (e.g. by means of the MMI) and the Reply Short Message is submitted (e.g. by means of the MMI).

d)	step c) is repeated for the other Short Message.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

11	MS -> SS	UA (SAPI=3)

12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)

TP-RP set to 1

13	MS -> SS	CP-ACK

14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

15	SS -> MS	CHANNEL RELEASE

16	SS -> MS	PAGING REQUEST

17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"

18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

20	SS -> MS	AUTHENTICATION REQ

21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages

shall be sent enciphered.

24		SS	SS starts ciphering.

25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

26	MS -> SS	UA (SAPI=3)

27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)

TP-OA, RP-OA and TP-UD different from step 12

28	MS -> SS	CP-ACK

29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU.

30	SS -> MS	CHANNEL RELEASE

31	MS		One of the two Short Messages is displayed

and the Reply Short Message is submitted.

32	MS -> SS	CHANNEL REQUEST

33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

34	MS -> SS	CM SERVICE REQ	Message is contained in SABM.

35	SS -> MS	AUTHENTICATION REQ

36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.

38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages

shall be sent enciphered.

39		SS	SS starts ciphering.

40	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

41	SS -> MS	UA (SAPI=3)

42	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)

RP-DA = RP-OA corresponding to the

message displayed

TP-DA = TP-OA corresponding to the

message displayed

43	SS -> MS	CP-ACK	Sent within TC1M after step 42

44	SS -> MS	CP-DATA	Contains RP-ACK RPDU

45	SS		Waits max 25 seconds for CP-ACK

46	MS -> SS	CP-ACK

47	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

48	MS		The other Short Message is displayed and the

Reply Short Message is submitted.

49	MS -> SS	CHANNEL REQUEST

50	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

51	MS -> SS	CM SERVICE REQ	Message is contained in SABM.

52	SS -> MS	AUTHENTICATION REQ

53	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

54	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the

message.

55	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following

messages shall be sent enciphered.

56	SS		SS starts ciphering.

57	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

58	SS -> MS	UA (SAPI=3)

59	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)

RP-DA = RP-OA corresponding to the

Message displayed

TP-DA = TP-OA corresponding to the

message displayed

60	SS -> MS	CP-ACK	Sent within TC1M after step 59

61	SS -> MS	CP-DATA	Contains RP-ACK RPDU

62	SS		Waits max 25 seconds for CP-ACK

63	MS -> SS	CP-ACK

64	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

SMS-DELIVER

TP-MTI	SMS-DELIVER

TP-MMS	no more messages are waiting in SC	1

TP-RP	Reply Path exists	1

TP-SRI	no Status Report returned	0

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	default	0

TP-DCS	default alphabet	0

TP-SCTS	the time when the message was submitted according to 	GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

34.3	Short message service cell broadcast

34.3.1	Test purpose

This test verifies that an MS is able to respond to a paging requested during the transmission of a cell broadcast short message.

34.3.2	Test method

Initial conditions

System Simulator:

1 cell, default parameters

The SS provides a BCCH/CCCH to support the MS in idle mode.

Periodic location updating is disabled.

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support for short message transmission cell broadcast

Description of the basic procedures to display a cell broadcasted short message

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	Three Cell Broadcast (CB) messages are sent by the SS on the CBCH with sequence numbers 0,1,1.

b)	Step a) is repeated, but the SS pages the MS during the transmission of the second CB message.

The MS shall respond to the page.

Maximum Duration of Test

-

Expected Sequence

Since the SMS-CB messages are sent continously, a table is not applicable in this test.

Specific Message Contents:

Cell broadcast test message content

Information element	Comment	Value

Sequence no.	See above

Message identifier		0

Alpha/language identifier	0

Page parameter		0001 0001

Characters of message	93 user characters

using all characters

of default 7 bit

coded alphabet