# 34	Short message service (SMS)

Ref : GSM 03.40, 04.11 (point to point)
GSM 03.41, 04.12 (cell broadcast)

## 34.1	General

The purpose of these tests is to verify that the MS can handle GSM functions when submitting or receiving Short Messages (SM) between MS and a short message service centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services. The SMS point to point services shall work in an active MS at any time independent of whether or not there is a speech or data call in progress. The SMS cell broadcast service only works when the MS is in idle mode.

Since the timer TC1M currently is not standardised, the value of TC1M shall be declared by the manufacturer (to be used in 34.2.1 and 34.2.2).

The manufacturer shall declare whether SMS messages are stored in the SIM and/or the ME. This shall be referred to as the SMS message store in the following tests.

Unless otherwise stated default message contents from section 26.6.14 applies for GSM900 and default message contents from section 26.6.15 applies for DCS1800.

## 34.2	Short message service point to point

### 34.2.1	SMS mobile terminated

#### 34.2.1.1 	Conformance requirements

An active MS shall be able to receive short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the SC, confirming that the MS has received the short message.

Reference

GSM 03.40; 3.1

#### 34.2.1.2 	Test purpose

To verifiy the ability of a MS to receive and decode the SMS where provided for the point to point service.

#### 34.2.1.3 	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Maximum number of CP-DATA message retransmissions implemented.

Support for call control state U10

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS initiates the transmission of a short message using a paging request. Upon response of the MS to the paging the SS assigns an SDCCH, authenticates the MS and activates ciphering. Then the SS establishes SAPI 3 by sending a SABM frame with SAPI 3 on the SDCCH.

When a UA frame (SAPI 3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU).

b)	The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

c)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

d)	Steps a), b) and c) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

e)	Steps a) and b) are repeated maximum CP-DATA retransmission times but the CP-DATA messages from the MS is not acknowledged. The SS then initiates the channel release.

f)	The SMS message store shall be cleared manually by the operator.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH.

When a UA frame (SAPI-3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU). The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

h)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release. The SMS message store shall be cleared manually by the operator.

i)	Steps g) and h) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

j)	Step g) is repeated maximum CP-DATA retransmission times but the CP-DATA messages from the MS are not acknowledged. The SS then initiates the channel release.

k)	A data or speech call is established on a TCH with the SS and the state U10 of call control shall be entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the speech call is cleared by the SS with a disconnect message. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA RPDU (SMS DELIVER TPDU) message. The information element of the CP-DATA message is RP-DATA.

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

l)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the SS waits 15 seconds. During this time the speech call shall be cleared from the MS. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA message. The information element of the CP-DATA message is RP-DATA RPDU (SMS DELIVER TPDU).

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST		
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 	MS shall respond to SABM in step 10
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	 13	SS 		Waits max 25 seconds for CP-ACK
	 14	MS -> SS	CP-ACK	
	 15	SS 		Waits max 60 seconds for RP-ACK RPDU
	 16	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	 17	SS -> MS	CP-ACK	Within TC1M after step 16, no further 
				CP-DATA messages
	 18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	19	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	  20	SS -> MS	PAGING REQUEST
	  21	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  22	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  23	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  24	SS -> MS	AUTHENTICATION REQ
	  25	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  26	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  27	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 28	SS		SS starts ciphering.
	29	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	30	MS -> SS	UA (SAPI=3)	The MS shall respond to the SABM 
	31	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	 32	SS 		Waits max 25 seconds for CP-ACK
	 33	MS -> SS	CP-ACK
	 34	SS 		Waits max 60 seconds for RP-ACK RPDU
	 35	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	36	SS		First CP-DATA message not acknowledged by
				SS
	37	MS -> SS	CP-DATA	Transmitted CP-DATA from MS within 
				TC1M, after step 35, contains RP-ACK RPDU
	 38	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged
				within TC1M after step 37, no further 
				CP-DATA messages
	 39	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

40	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	  41	SS -> MS	PAGING REQUEST
	  42	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  43	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  44	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  45	SS -> MS	AUTHENTICATION REQ
	  46	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  47	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  48	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 49	SS		SS starts ciphering.
	50	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	51	MS -> SS	UA (SAPI=3) 	The MS shall respond to the SABM
	52	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	 53	SS 		Waits max 25 seconds for CP-ACK
	 54	MS -> SS	CP-ACK
	 55	SS 		Waits max 60 seconds for RP-ACK RPDU
	 56	MS -> SS	CP-DATA	Contains RP-ACK 
				RPDU
	57	SS		First CP-DATA message not acknowledged by
				SS
	58	MS -> SS	CP-DATA	Repeated CP-DATA from MS within twice
				TC1M, contains RP-ACK RPDU
	 59	SS 		Second CP-DATA message not acknowledged
				by SS
	60	MS		The MS shall not send any more CP-DATA
				messages
	 61	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	62	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	63	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	64	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	65	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	66	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	67	SS		Waits max 25 seconds for CP-ACK
	68	MS -> SS	CP-ACK
	69	SS		Waits max 60 seconds for RP-ACK RPDU
	70	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	71	SS -> MS	CP-ACK	Within TC1M after step 70, no further
				CP-DATA messages
	72	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	73	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	74	MS		Clear the SMS message store
	75	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	76	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH

77	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	78	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	79	SS		Waits max 25 seconds for CP-ACK
	80	MS -> SS	CP-ACK
	81	SS		Waits max 60 seconds for RP-ACK RPDU
	 82	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	83	SS		First CP-DATA message not acknowledged by
				SS
	84	MS -> SS	CP-DATA	Repeated CP-DATA message within twice
				TC1M, contains RP-ACK RPDU
	 85	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged
				within TC1M after step 84, no further 
				CP-DATA messages
	86	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	87	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	88	MS		Clear the SMS message store
	89	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	90	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	91	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	92	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	93	SS		Waits max 25 seconds for CP-ACK
	94	MS -> SS	CP-ACK
	95	SS		Waits max 60 seconds for RP-ACK RPDU
	96	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	97	SS		First CP-DATA message not acknowledged by
				SS
	98	MS -> SS	CP-DATA	Transmitted CP-DATA message within 
				TC1M after step 96, contains RP-ACK RPDU
	 99	SS 		Retransmitted CP-DATA message not 
				acknowledged by SS
	100	MS		Depending on the maximum number of CP-DATA
				retransmissions implemented, step 98-99 may be
				repeated. The number of repetitions shall 
				correspond to the related PICS/PIXIT statement.
 	101	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	102	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	103	MS		Clear the SMS message store
	104	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	105	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	106	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	107	SS -> MS	DISCONNECT	The speech call is cleared by the SS. The call
				clearing is continued on the FACCH in parallel
				to the following exchange of messages related
				to SMS.
	108	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	109	SS		Waits max 25 seconds for CP-ACK
	110	MS -> SS	CP-ACK
	111	SS		Waits max 60 seconds for RP-ACK RPDU

112	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	113	SS -> MS	CP-ACK	Within TC1M after step 112, no further 
				CP-DATA 
	114	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	115	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	116	MS		Clear the SMS message store
	117	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	118	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	119	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	120	SS		Waits 15 seconds
	121	MS		During the SS wait time in step 120 the
				speech call shall be cleared from the MS. The
				call clearing is continued on the FACCH in
				parallel to the following exchange of messages
				related to SMS.
	122	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVE
				TPDU)
	123	SS		Waits max 25 seconds for CP-ACK
	124	MS -> SS	CP-ACK
	125	SS		Waits max 60 seconds for RP-ACK RPDU
	126	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	127	SS -> MS	CP-ACK	Within TC1M after step 126, no further 
				CP-DATA 
	128	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	129	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	130	MS		Clear the SMS message store

NOTE 11:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS DELIVER

Information element	Comment	Value

TP-MTI 	MT

TP-MMS 	more messages are waiting

TP-RP	no reply path	0

TP-SRI	no status report returned	0

TP-OA 	an international number coded E.164

TP-PID 	default	0

TP-DCS 	default alphabet	0

TP-SCTS 	any legal value (cf. GSM 03.40)

TP-UDL 		160

TP-UD (140 octets)	text of message (160 characters)

NOTE 22:	The 160 characters shall include at least one occurrence of each character in the default alphabet (see GSM 03.40 annex 2).

### 34.2.2	SMS mobile originated

#### 34.2.2.1	Conformance requirements

An active MS shall be able to submit short message TPDU (SMS-SUBMIT) at any time, independently of whether or not there is a speech or data call in progress.

Reference

GSM 03.40; 3.1

#### 34.2.2.2	Test purpose

To verify that the MS is able to correctly send a short message where the SMS is provided for the point to point service. The yesy also verifies that the MS is capable of simultaneously receive a network originated SM whilst sending a mobile originated SM.

#### 34.2.2.3	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MO/PP

Description of the basic procedures to display a mobile originated short message

Support for state U10 of call control

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Maximum length (characters) of a mobile originated short message.

Maximum number of CP-DATA message retransmissions implemented.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS shall be set up to send a SM to the SS. The SS responds to the channel request message by allocating an SDCCH. The SS answers correctly to the SABM on SAPI 0 and then performs the authentication and ciphering procedures.

b)	The SS responds with a UA frame SAPI-3 to the MS.

c)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message.

d)	The SS sends a channel release message to the MS.

e)	Steps a) and b) are repeated maximum number of CP-DATA retransmission times with the SS configured not to send the CP-ACK. After a duration of TC1M + 5 seconds after the last CP-DATA restransmission the SS initiates channel release. The 5 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions.

f)	Steps a) and b) are repeated. On receipt of the CP-DATA from the MS the SS sends a CP-ERROR message within TC1M containing a "Network Failure" cause. Then the SS initiates channel release.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The MS is setup to send an SM to the SS. After the reception of the CM SERVICE REQUEST, the SS sends a CM SERVICE ACCEPT message. The SS responds with a UA frame SAPI-3 to the SABM with SAPI-3 received from the MS.

h)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message. Then the SS sends a channel release message to the MS.

i)	Step g) is repeated repeated maximum number of CP-DATA retransmission times. The SS is configured not to send the CP-ACK message. After a duration of TC1M + 15 seconds after the last CP-DATA restransmission the SS initiates channel release. The 15 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions (during a call in progress).

j)	The SS is configured to receive a mobile originated SM. Step b) is repeated and, using the end of the CP-DATA message from the MS as a trigger, the SS sends a SM to the MS. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

k)	The MS is set up to send an SM to the SS. On receipt of the CM SERVICE REQUEST the SS sends a CM SERVICE REJECT message with the reject cause set to "Service Option not supported" or "Service Option temporarily out of order". After 5 seconds the SS initiates channel release.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  3	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0. CM
				service type set to "Short message transfer"
	  4	SS -> MS	AUTHENTICATION REQ
	  5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 8		SS	SS starts ciphering.
	9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	10	SS -> MS	UA (SAPI=3) 
	11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	 12	SS -> MS	CP-ACK	Sent within TC1M after step 11
	 13	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	14		SS			Waits max 25 seconds for CP-ACK
	 15	MS -> SS	CP-ACK
	 16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	17	MS -> SS	DISC (SAPI=0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0
	  18	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  20	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0.
	  21	SS -> MS	AUTHENTICATION REQ
	  22	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  23	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  24	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 25		SS	SS starts ciphering.
	26	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	27	SS -> MS	UA (SAPI=3) 
	28	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	 29		SS 	SS configured not to send CP-ACK
	30	MS -> SS	CP-DATA	Transmitted CP-DATA message TC1M after step
				28
	31		MS	Depending on the maximium number of CP-DATA
				retransmissions implemented, step 30 may be 
				repeated. The number of repetitions shall 
				correspond to the related PICS/PIXIT statement.
	 32	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a
				duration of TC1m + 5 seconds after the last 
				CP-DATA retransmission.
	32a	MS -> SS	DISC (SAPI = 0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0.
	  33	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  34	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  35	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	  36	SS -> MS	AUTHENTICATION REQ
	  37	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

38	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  39	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 40		SS	SS starts ciphering.
	41	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3

42	SS -> MS	UA (SAPI=3) 
	43	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	 44	SS -> MS	CP-ERROR	Sent within TC1M containing "Network
				Failure" cause.
 45	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	45a	MS -> SS	DISC (SAPI = 0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0.
	46		SS	A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	47		MS	The MS is set up to send an SM
	48	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM
				service type set to "short message transfer"
	49	SS -> MS	CM SERVICE ACCEPT
	50	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	51	SS -> MS 	UA (SAPI=3)
	52	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	53	SS -> MS	CP-ACK	Sent within TC1M after step 52
	 54	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	55		SS	Waits max 25 seconds for CP-ACK
	 56	MS -> SS	CP-ACK
	 57	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	58	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release with a 
				layer 2 DISC frame with SAPI 0.
	59	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	60	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM
				service type set to "short message transfer"
	61	SS -> MS	CM SERVICE ACCEPT
	62	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	63	SS -> MS 	UA (SAPI=3)
	64	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	65	SS		SS configured not to send CP-ACK
	66	MS -> SS	CP-DATA	Transmitted CP-DATA message TC1M after step
				64
	67	MS		Depending on the maximum number of CP-DATA
				retransmissions implemented, step 66 may be
				repeated. The number of repetitions shall
				correspond to the related PICS/PIXIT statement
	 68	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a
				duration of TC1m + 15 seconds after the last 
				CP-DATA retransmission.
	69	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release
	  70	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  71	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  72	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	  73	SS -> MS	AUTHENTICATION REQ
	  74	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

75	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  76	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 77	SS		SS starts ciphering.
	78	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	79	SS -> MS	UA (SAPI=3) 
	80	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	81	SS		The SS sends an SM to the MS triggered by
				the end of the CP-DATA message from the MS
	82	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	83	MS		The MS shall correctly receive the SM and
				indicate that a message has arrived. If the MS
				provides the functionality to display MT
				messages, it is checked that the correct
				message is displayed. In the MO case the MS
				shall send the CP-ACK message with
				transaction identifier assigned to this transfer.
				In the MT case the MS shall send a CP-ACK
				message and a CP-DATA message containing
				the RP-ACK RPDU. The transaction identifier
				shall be the same as chosen by the SS for the
				MT transfer. 


	  84	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  85	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  86	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	87	SS -> MS	CM SERVICE REJ	Reject cause set to "Service Option not
				supported" or "Service Option temporarily out
				of order"
	88		MS	The MS shall not establish SAPI-3
	89	SS -> MS	CHANNEL RELEASE	Sent 5 seconds after CM SERVICE REJ

NOTE 33:	Time values for SS wait times are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS SUBMIT

Information element	Comment	Value

TP-MTI 	MO

TP-VPF 	"TP-VP field present and integer
	represented (relative)"

TP-RD		0

TP-DA 	an E164 number

TP-PID 	all zeros	0

TP-DCS 	standard alphabet	0

TP-VP 	24 hours

TP-UDL 	as applicable

TP-UD (140 octets max)	maximum number of characters

(text of message) as defined

by the manufacturer (see PICS/PIXIT)

### 34.2.3	Test of memory full condition and memory available notification:

The Memory Available Notification provides a means for the MS to notify the network that it has memory available to receive one or more short messages. The SMS status field in the SIM contains status information on the "memory available" notification flag.

#### 34.2.3.1	Conformance requirement

1.	When a mobile terminated message in Class 2, the MS shall ensure that the message has been transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a protocol error message if the short message cannot be stored in the SIM and there is other short message storage available in the MS. If all the short message storage in the MS is already in use, the MS shall return "memory capability exceeded".

2.	When the MS rejects a short message due to lack of available memory capability the need to transfer notification shall be stored in the SIM.

3.	If the memory capability becomes available because memory is cleared, the value of the memory capability exceeded notification flag in the SIM is read. If the flag is set, the MS notifies the network that memory capability is now available. After a positive acknowledgement from the network, the ME unsets the memory capability exceeded notification flag in the SIM.

References

GSM 03.40, 9.2.3.10, GSM 03.38, 4.

GSM 03.40, 10.3 (operation 14)

GSM 03.40, 10.3 (operation 14)

#### 34.2.3.2	Test purpose

1.	To verify that the MS sends the correct acknowledgement when its memory in the SIM becomes full.

2.	To verify that the MS sends the correct acknowledgement when its memory in the ME and the SIM becomes full, and sets the "memory exceeded" notification flag in the SIM.

3.	To verify that the MS performs the "memory available" procedure when its message store becomes available for receiving short messages, and only at this moment.

#### 34.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The SMS message storage shall be empty.

The MS shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least one record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

Whether SMS messages are stored in the SIM and/or the ME

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	step a) of 34.2.5.3 (test of Class 2 Short Messages) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

b)	a Class 1 Short Message is sent to the MS.

c)	step b) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

d)	a Short Message is sent to the MS with the DCS field of the SMS-DELIVER TPDU set to 0.

e)	the SS prompts the operator to read a short message and to remove it from the message store of the MS.

f)	the SS waits for a CHANNEL REQUEST from the MS, and sends an IMMEDIATE ASSIGNMENT allocating an SDCCH.

g)	the SS answers correctly to the SABM on SAPI 0.

h)	the SS answers correctly to the SABM on SAPI 3.

i)	the SS answers to the RP-SMMA from the MS with a CP-DATA containing a RP-ACK RPDU.

j)	after the MS has acknowledged the CP-DATA with a CP-ACK, the SS releases the channel with a CHANNEL RELEASE message. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been unset on the SIM.

k)	step e) is repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message
	 13	SS 		Waits max 25 seconds for CP-ACK
	 14	MS -> SS	CP-ACK	
	 15	SS 		Waits max 60 seconds for RP-ACK RPDU
	 16	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	 17	SS -> MS	CP-ACK	Within TC1M after step 16
	18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
				Step 1-18 is repeated until MS sends a
				negative acknowledgement (RP-ERROR) in step
				16. The RP-ERROR RPDU cause field
				shall be "Protocol error, unspecified" if there is
				message capability in the ME, or "Memory
				capability exceeded" if there is no message 
				capability in the ME. If the total memory store 
				of the MS is full, the ME shall set the 
				"memory capability exceeded" notification
				flag on the SIM.
	  19	SS -> MS	PAGING REQUEST
	  20	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  21	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  22	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  23	SS -> MS	AUTHENTICATION REQ
	  24	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  25	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	  26	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following 
				messages shall be sent enciphered.
	 27	SS		SS starts ciphering.
	28	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	29	MS -> SS	UA (SAPI=3) 
	30	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 1 Short Message
	 31	SS 		Waits max 25 seconds for CP-ACK
	 32	MS -> SS	CP-ACK
	 33	SS 		Waits max 60 seconds for RP-ACK RPDU

34	MS -> SS	CP-DATA	Shall contain RP-ACK RPDU if there is memory
				capability in the ME. If not it shall contain 
				RP-ERROR RPDU which cause field shall be 
				"memory capability exceeded". If the total
				memory store of the MS now becomes full at
				this step, the ME shall set the
				"memory cap. exceed" notification flag on the SIM.
	 35	SS -> MS	CP-ACK	Within TC1M after step 34
	36	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	 
				Step 19-36 is repeated until the MS sends an
				RP-ERROR. The SIM simulator shall indicate if
				the "memory capability exceeded" notification
				flag has been set on the SIM.
	  37	SS -> MS	PAGING REQUEST
	  38	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  39	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  40	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  41	SS -> MS	AUTHENTICATION REQ
	  42	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  43	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  44	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 45	SS		SS starts ciphering.
	46	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	47	MS -> SS	UA (SAPI=3) 
	48	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU) with TP-DCS set to 0
	 49	SS 		Waits max 25 seconds for CP-ACK
	 50	MS -> SS	CP-ACK
	 51	SS 		Waits max 60 seconds for RP-ACK RPDU
	 52	MS -> SS	CP-DATA	Shall contain RP-ERROR RPDU with error
				cause "memory capability exceeded".
	 53	SS -> MS	CP-ACK	Within TC1M after step 52
	54 	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	55	SS		Prompts the operator to read a short message
				and to remove it from the message store of
				the MS.
	57	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH".
	58	SS -> MS	IMMEDIATE ASSIGNMENT	SS alloctes an SDCCH
	59	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. CM service
				type information elementis set to "Short
				message transfer".
	60	SS -> MS	CM SERVICE ACCEPT (UA)	SAPI 0
	61	MS -> SS	SABM (SAPI=3)	MS shall establish SAPI 3
	62	SS -> MS	UA (SAPI=3)
	63	MS -> SS	CP-DATA	Contains RP-SMMA RPDU
	64	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	65	MS -> SS	CP-ACK	Acknowledge of CP-DATA containing the 
				RP-ACK RPDU. The ME shall unset the 
				"memory capability exceeded" notification flag
				on the SIM.
	66	SS -> MS	CHANNEL RELEASE	The main signalling link is released. The SIM 
				simulator shall indicate if the "memory
				capability exceeded" notification flag has been
				unset on the SIM.

67	SS		Prompts the operator to read a short message and 
				to remove it from the message store of the MS.
	68		MS	Shall not attempt to send a RP-SMMA RPDU.
				This is verified by checking that the MS does
				not send a CHANNEL REQUEST message with
				the establishment cause "Other services
				which can be completed with an SDCCH"

NOTE 44:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS-DELIVER in step 12

same as in 34.2.5.3

SMS-DELIVER in step 30

same as in 34.2.5.2

SMS-DELIVER in step 48

same as in 34.2.1

### 34.2.4	Test of the status report capabilities and of SMS-COMMAND:

This test applies to MSs which support the status report capabilities.

#### 34.2.4.1	Conformance requirement

The SMS offers the SC the capabilities of informing the MS of the status of a previously sent mobile originated short message. This is achieved by the SC returning a status report TPDU (SMS-STATUS-REPORT) to the originating MS.

SMS-COMMAND eneables an MS to invoke an operation at the SC.

The MS shall increment TP-MR by 1 for each SMS-SUBMIT or SMS-COMMAND being submitted.

References

GSM 03.40; 3.2.9

GSM 03.40; 9.2.3.6

#### 34.2.4.2	Test purpose

1)	To verify that the MS is able to accept a SMS-STATUS-REPORT TPDU.

2)	To verify that the MS is able to use the SMS-COMMAND functionality correctly and sends an SMS-COMMAND TPDU with the correct TP-Message-Reference.

#### 34.2.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support of SMS MO/PP and MT/PP

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS is made to send a Mobile Originated short message as in steps a) to e) of test 34.2.2 	(SMS Mobile originated).

b)	The SS establishes a data link on SAPI-3 with the MS, then sends a CP-DATA message containing a RP-DATA RPDU itself containing an SMS-STATUS-REPORT TPDU.

c)	The SS sends a CHANNEL RELEASE message.

d)	The MS is made to send an SMS-COMMAND message enquiring about the previously submitted short message.

e)	The SS responds to the MS so as to enable it to establish a data link on SAPI-3 on an SDCCH.

f)	The SS acknowledges the CP-DATA message from the MS with a CP-ACK followed by a CP-DATA message containing an RP-ACK RPDU

g)	After receiving the CP-ACK from the MS, the SS releases the channel by using a CHANNEL RELEASE message.

h)	The MS is made to send an SMS-COMMAND message requiring to delete the previously submitted short message.

i)	step e) is repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other services
				requested by the mobile user"
	  2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  3	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	  4	SS -> MS	AUTHENTICATION REQ
	  5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 8		SS	SS starts ciphering.
	9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	10	SS -> MS	UA (SAPI=3) 
	11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	 12	SS -> MS	CP-ACK	Sent within TC1M after step 11
	 13	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	14		SS	Waits max 25 seconds for CP-ACK
	 15	MS -> SS	CP-ACK
	 16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	  17	SS -> MS	PAGING REQUEST
	  18	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  20	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  22	SS -> MS	AUTHENTICATION REQ
	  23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 26	SS		SS starts ciphering.
	27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	28	MS -> SS	UA (SAPI=3) 
	29	SS -> MS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-STATUS-REPORT TPDU)
	30	MS -> SS	CP-ACK
	31	MS -> SS 	CP-DATA	Contains RP-ACK RPDU
	32	SS -> MS	CHANNEL RELEASE
	33		MS	The MS is made to send an SMS-COMMAND 
				message enquiring about the previously
				submitted SM	
	34	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH".
	35  	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	36	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	37	SS -> MS	AUTHENTICATION REQ
	38	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	39	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	40	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 41		SS	SS starts ciphering.
	42	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	43	SS -> MS	UA (SAPI=3)

44	MS -> SS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-COMMAND TPDU) which shall contain
				the correct TP-MR
	45	SS -> MS	CP-ACK
	46	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	47	MS -> SS	CP-ACK
	48 	SS -> MS	CHANNEL RELEASE
	49		MS	The MS is made to send an SMS-COMMAND 
				message requiring to delete the previously
				submitted SM.
	50	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH".
	51	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	52	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	53	SS -> MS	AUTHENTICATION REQ
	54	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	55	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	56	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 57		SS	SS starts ciphering.
	58	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	59	SS -> MS	UA (SAPI=3) 
	60	MS -> SS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-COMMAND TPDU) which shall contain
				the correct TP-MR
	61	SS -> MS	CP-ACK
	62	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	63	MS -> SS	CP-ACK
	64 	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS SUBMIT

As in 34.2.3 but with TP-SRR set to 1.

SMS-STATUS-REPORT (SS to MS):

TP-MTI	SMS-STATUS-REPORT

TP-MR	same as previous SMS-SUBMIT

TP-MMS	no more messages	1

TP-SRQ	result of SMS-SUBMIT

TP-RA	same as the Destination adress of the SMS-SUBMIT

TP-SCTS	any legal value (cf. GSM 03.40)

TP-RCT	any legal value (cf. GSM 03.40)

TP-ST		0

first SMS-COMMAND (MS to SS in step 44)

TP-MTI	SMS-COMMAND

TP-MR	TP-MR in previous SMS-SUBMIT plus "1"

TP-PID	default	0

TP-CT	Enquiry relating to previously submitted short message	0

TP-MN	not checked (TP-MR in previous SMS-SUBMIT)

TP-DA	not checked (an E164 number)

TP-CDL	not checked

TP-CD	not checked

second SMS-COMMAND (MS to SS in step 60)

TP-MTI	SMS-COMMAND

TP-MR	TP-MR in previous SMS-COMMAND plus "1"

TP-PID	default	0

TP-CT	Delete previously submitted short message	2

TP-MN	not checked (TP-MR in previous SMS-SUBMIT)

TP-DA	not checked (an E164 number)

TP-CDL	not checked

TP-CD	not checked

### 34.2.5	Test of message class 0 to 3

The tests under this section only apply to a MS capable of displaying short messages (see PICS/PIXIT).

#### 34.2.5.1	Short message class 0

##### 34.2.5.1.1	Conformance requirement

When a mobile terminated message is class 0 and the MS has the capability of displaying short messages, the MS shall display the message immediately and send an acknowledgement to the SC when the message has successfully reached the MS irrespective of whether there is memory available in the SIM or ME. The message shall not be automatically stored in the SIM or ME.

References

GSM 03.38, 4.

##### 34.2.5.1.2	Test purpose

To verify that the MS will accept and display but not store a class 0 message, and that it will accept and display a class 0 message if its message store is full.

NOTE 55:	failure of this test in a mobile could cause it to reject a class 0 message when its SMS memory becomes full. This could lead to unwanted repetitions between the MS and the service center.

##### 34.2.5.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS sends a class 0 message by using the method described in step a) of section 34.2.1 but with the TPDU described in this section.

b)	The MS message store shall be filled (for example by using the method of 34.2.3 (test of the memory available notification) with the same SMS-DELIVER TPDU.

c)	The SS sends a class 0 message as in step a).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 9		SS	SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU),
				Class 0 Short Message
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CP-ACK
	16	SS -> MS	CHANNEL RELEASE
	17	MS		The content of the short message shall be
				displayed by the ME. The MS shall not store the
				the message. This can be checked by verifying 
				that it is impossible to retrieve any short messages
				from the MS message store.
	18	SS		The MS message store shall be filled
				(for example by
				using the method of 34.2.3) with the same
				SMS-DELIVER TPDU. 
				
				
				
				
				
				
	 19	SS -> MS	PAGING REQUEST	
	  20	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  21	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  22	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  23	SS -> MS	AUTHENTICATION REQ
	  24	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  25	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
					message.
	  26	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 27		SS	SS starts ciphering.
	28	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	29	MS -> SS	UA (SAPI=3) 
	30	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU),
				Class 0 Short Message
	31	MS -> SS	CP-ACK
	32	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	33	SS -> MS	CP-ACK
	34	SS -> MS	CHANNEL RELEASE
	35		MS	The content of the short
				message shall be displayed
				by the ME.

Specific Message Contents:

SMS-DELIVER (containing a class 0 message) (SS to MS):

TP-MTI	SMS-DELIVER

TP-MMS	more messages are waiting	0

TP-RP	no reply path	0

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	0

TP-DCS	default alphabet, class 0	binary 1111 0000

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

#### 34.2.5.2	Test of class 1 short messages

[Editorial note: It cannot be tested, whether the MS acknowledges the SM before or after having stored it]

This test shall apply to MSs which support

-	storing of received Class 1 Short Messages and

-	displaying of stored Short Messages.

##### 34.2.5.2.1	Conformance requirement

When a mobile terminated message is class 1, the MS shall send an acknowledgement to the SC when the message has successfully reached the MS and can be stored, either in the ME or in the SIM.

References

GSM 03.38, 4.

##### 34.2.5.2.2	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 1 message, i.e. that it stores the message in the ME or SIM and sends an acknowledgement (at RP and CP-Layer).

##### 34.2.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

For storing of class 1 Short Messages, the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 1 to the MS as specified in section 34.2.1, step a).

b)	the Short Message is recalled (e.g. by means of the MMI).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST		
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 9		SS	SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 1 Short Message
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CHANNEL RELEASE
	16		MS	The short message shall be recalled
				and displayed at the MS.

Specific Message Contents:

SMS-DELIVER

same as 34.2.1, but

TP-DCS	default alphabet, class 1	binary 11110001

#### 34.2.5.3	Test of class 2 short messages

##### 34.2.5.3.1	Definition and applicability

Class 2 Short Messages are defined as SIM specific, and the MS shall ensure that a message of this class is stored on the SIM.

This test shall apply to MSs which support

-	storing of received Class 2 Short Messages in the SIM and

-	displaying of Short Messages stored in the SIM.

##### 34.2.5.3.2	Conformance requirement

When a mobile terminated message is Class 2, the MS shall ensure that the message has been correctly  transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a "protocol error, unspecified" error message if the short message cannot be stored in the SIM and there is other short message storage available at the MS.

Reference(s)

GSM 03.40, 9.2.3.10; GSM 03.38, 4. GSM 11.11, 10.3.3

##### 34.2.5.3.3	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 2 message, i.e. that it stores the message correctly in the SIM, and if this is not possible, returns a protocol error message to the network.

##### 34.2.5.3.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The ME message store shall be empty.

The ME shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least two free records and one full record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 2 to the MS as specified in section 34.2.1, step b).

b)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "OK" ('90 00').

c)	step a) is repeated.

d)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "memory problem" ('92 40').

e)	the SIM simulator indicates if an attempt was made in steps a) and c) to store the messages and if the messages are stored according to the requirement.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message
	13	MS -> SS	CP-ACK
	14	ME		The ME shall correctly store the short message 
				in a free record of EFSMS in the SIM, i.e.
				-	the ME shall use a free record
				-	the first byte of the record
					shall indicate "message received by MS
					 from network"
				-	the TS-Service-Centre-Address shall be
					correctly stored
				-	the TPDU shall be identical
					to that sent by the SS
				-	bytes following the TPDU
					shall be set to 'FF'
	15	SIM		The SIM simulator returns the status response 
				"OK" ('90 00'). The SIM simulator shall indicate 
				if an attempt was made by the ME to store the
				short message in the SIM.
	16	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	17	SS -> MS	CHANNEL RELEASE 
	 18	SS -> MS	PAGING REQUEST		
	 19	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	 20	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	 21	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	 22	SS -> MS	AUTHENTICATION REQ
	 23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	 24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	 25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 26	SS		SS starts ciphering.
	27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	28	MS -> SS	UA (SAPI=3) 
	29	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message

30	MS -> SS	CP-ACK
	31	ME		The ME shall attempt to store the short 
				message in a free record of EFSMS in the SIM.
	32	SIM		The SIM simulator returns the status response 
				"memory problem" ('92 40'). The SIM simulator 
				shall indicate if an attempt was made by the ME 
				to store the short message in the SIM.

33	MS -> SS 	CP-DATA	Contains RP-ERROR RPDU with error cause
				"protocol error, unspecified".
	34	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS-DELIVER

same as 34.2.1, but

TP-DCS	default alphabet, class 2	binary 11110010

#### 34.2.5.4	Test of class 3 short messages

[Editorial note: For this test a configuration with LTE is needed. Therefore this test should be specified together with the other 07.05 tests.]

For further study

### 34.2.6	Test of short message type 0

[Editorial note: The only requirement for this message type is that it must be acknowledged. But this is a general requirement for all SMs, which means that there is no specific requirement for type 0 SMs to be tested]

For further study

### 34.2.7	Test of the replace mechanism for SM type 1-7

#### 34.2.7.1	Definition and applicability

This test shall apply to MSs which support

-	Replace Short Messages and

-	display of received Short Messages.

#### 34.2.7.2	Conformance requirement

On receipt of a short message, the MS shall check to see if the associated Protocol Identifier contains a Replace Short Message Type code. If such a code is present, then the MS will check the associated SC address (RP-OA) and originating address (TP-OA) and replace any existing stored message having the same Protocol Identifier code, SC address and originating address with the new short message.

Reference(s)

GSM 03.40; 9.2.3.9

#### 34.2.7.3	Test purpose

This procedure verifies the correct implementation of the replace mechanism for Replace Short Messages.

#### 34.2.7.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	two different numbers n and m are drawn randomly between 1 and 7.  Two different addresses for TP-Originating-Address (TPOA1 and TPOA2) are drawn.  Two different addresses for RP-Originating-Address (RPOA1 and RPOA2) are drawn.

b)	the SS delivers a short message to the MS as specified in section 34.2.1 step a).  In the SMS-DELIVER TPDU, the TP-Protocol-Identifier parameter is "Replace Short Message Type n", the TP-Originating-Address is TPOA1, and the RP-Originating-Address is RPOA1.

c)	step b) is repeated but with a different TP-Originating-Address (TPOA2), and different contents of TP-User-Data in the SMS-DELIVER TPDU.  The other parameters are the same as in step b).

d)	step c) is repeated but with RPOA2 in the RP-Originated-Address, and contents of TP-User-Data different from the former two messages.  The other parameters are the same as in step c).

e)	step d) is repeated but with the TP-Protocol-Identifier equal to "Replace Short Message Type m", and contents of TP-User-Data different from the former three messages.  The other parameters are the same as in step d).

f)	step e) is repeated but the contents of TP-User-Data are different from that used in step e).

g)	the SS prompts the operator to display the Short Messages stored in the MS.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 9		SS	SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
 				TPDU)
				TP-PID is "Replace Short Message
				Type n", TP-OA is TPOA1 and
 				RP-OA is RPOA1
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CHANNEL RELEASE 


	 16	SS -> MS	PAGING REQUEST	
	 17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  20	SS -> MS	AUTHENTICATION REQ
	  21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 24		SS	SS starts ciphering.
	25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	26	MS -> SS	UA (SAPI=3) 
	27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-PID is "Replace Short Message Type n", 
				TP-OA is TPOA2 and RP-OA is RPOA1, 
				TP-UD different from step 12
	28	MS -> SS	CP-ACK
	29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	30	SS -> MS	CHANNEL RELEASE 


	 31	SS -> MS	PAGING REQUEST	
	 32	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  34	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  35	SS -> MS	AUTHENTICATION REQ
	  36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 39		SS	SS starts ciphering.
	40	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

41	MS -> SS	UA (SAPI=3) 
	42	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-PID is "Replace Short Message Type n",
				TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD
				different from step 12 and 27
	43	MS -> SS	CP-ACK
	44	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	45	SS -> MS	CHANNEL RELEASE 


	 46	SS -> MS	PAGING REQUEST
	 47	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  48	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  49	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  50	SS -> MS	AUTHENTICATION REQ
	  51	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  52	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  53	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 54		SS	SS starts ciphering.
	55	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	56	MS -> SS	UA (SAPI=3) 
	57	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU) TP-PID is "Replace Short Message
				Type m", TP-OA is TPOA2 and RP-OA is
				RPOA2, TP-UD different 
				from step 12, 27 and 42
	58	MS -> SS	CP-ACK
	59	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	60	SS -> MS	CHANNEL RELEASE 

	 61	SS -> MS	PAGING REQUEST	
	 62	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  63	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  64	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  65	SS -> MS	AUTHENTICATION REQ
	  66	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  67	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  68	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 69		SS	SS starts ciphering.
	70	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	71	MS -> SS	UA (SAPI=3) 
	72	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
				TP-PID is "Replace Short Message
 				Type m", TP-OA is TPOA2 and RP-OA is RPOA2, 
				TP-UD different from step 57
	73	MS -> SS	CP-ACK
	74	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	75	SS -> MS	CHANNEL RELEASE 

	76	SS		Prompts the operator to display the
				ShortMessages stored in the MS. Only the
				Short Messages delivered in step 12, 27, 42 and
				72 shall be retrievable and displayed

Specific Message Contents:

SMS-DELIVER

TP-MTI	SMS-DELIVER

TP-MMS	no more messages are waiting in SC	1

TP-RP	no Reply Path	0

TP-SRI	no Status Report returned	0

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	binary 01000xxx, xxx represents n resp. m (see test method 	description)

TP-DCS	default alphabet	0

TP-SCTS	the time when the message was submitted according to GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

### 34.2.8	Test of the reply path scheme

#### 34.2.8.1	Definition and applicability

This test applies to MSs which support

-	reply procedures (the class of MSs for which this is mandatory is described in GSM 03.40, Annex 4)

-	displaying of received Short Messages and

-	submitting Short Messages.

Steps b) and d) are only executed for MSs which support storing of Short messages.

#### 34.2.8.2	Conformance requirement

When a replying MS receives an original mobile terminated short message it has:

originating SME = TP-Originating Address in the SMS-DELIVER TPDU

original SC = RP-Originating Address in the RP-MT-DATA

When submitting the reply mobile originated short message, the replying MS should use parameters as follows:

TP-Destination Address in SMS-SUBMIT TPDU = originating SME

RP-Destination Address in RP-MO-DATA = original SC

Reference(s)

GSM 03.40 Annex 4, sect. 5, 6.

#### 34.2.8.3	Test purpose

This procedure verifies that the MS is able to send a Reply Short Message back to the correct originating SME even if in the meantime it receives another Short Message.

#### 34.2.8.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Support for Short message MO/PP

Description of the basic procedures to display a mobile terminated short message

Description of the basic procedures to send a mobile originated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message as specified in section 34.2.1, step b) with TP-Reply-Path set to 1.

b)	step a) is repeated but with

-	different TP-Originating-Address for the originating SME

-	different RP-Originating-Address for the original SC and

-	different message contents TP-User-Data.

c)	one of the two Short Messages is displayed (e.g. by means of the MMI) and the Reply Short Message is submitted (e.g. by means of the MMI).

d)	step c) is repeated for the other Short Message.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-RP set to 1
	13	MS -> SS	CP-ACK	Sent within TC1M after step 12
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CHANNEL RELEASE 
	 16	SS -> MS	PAGING REQUEST	
	 17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	  18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  20	SS -> MS	AUTHENTICATION REQ
	  21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	  23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	 24		SS	SS starts ciphering.
	25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	26	MS -> SS	UA (SAPI=3) 
	27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-OA, RP-OA and TP-UD different from step 12
	28	MS -> SS	CP-ACK	Sent within TC1M after step 12
	29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	30	SS -> MS	CHANNEL RELEASE 
	31	MS		One of the two Short Messages is displayed
				and the Reply Short Message is submitted.
	  32	MS -> SS	CHANNEL REQUEST	
	  33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  34	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	  35	SS -> MS	AUTHENTICATION REQ
	  36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	  38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	 39		SS	SS starts ciphering.
	40	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	41	SS -> MS	UA (SAPI=3) 
	42	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)
				RP-DA = RP-OA corresponding to the
				message displayed
				TP-DA = TP-OA corresponding to the
				message displayed

43	SS -> MS	CP-ACK	Sent within TC1M after step 42
	 44	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	45	SS		Waits max 25 seconds for CP-ACK
	 46	MS -> SS	CP-ACK	
	 47	SS -> MS	CHANNEL RELEASE	The main signalling link is released.


	48	MS		The other Short Message is displayed and the 
				Reply Short Message is submitted.
	  49	MS -> SS	CHANNEL REQUEST	
	  50	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	  51	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	  52	SS -> MS	AUTHENTICATION REQ
	  53	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  54	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	  55	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following 
				messages shall be sent enciphered.
	 56	SS		SS starts ciphering.
	57	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	58	SS -> MS	UA (SAPI=3) 
	59	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)
				RP-DA = RP-OA corresponding to the
				Message displayed
				TP-DA = TP-OA corresponding to the
				message displayed
	 60	SS -> MS	CP-ACK	Sent within TC1M after step 59
	 61	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	62	SS		Waits max 25 seconds for CP-ACK
	 63	MS -> SS	CP-ACK	
	 64	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

SMS-DELIVER

TP-MTI	SMS-DELIVER

TP-MMS	no more messages are waiting in SC	1

TP-RP	Reply Path exists	1

TP-SRI	no Status Report returned	0

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	default	0

TP-DCS	default alphabet	0

TP-SCTS	the time when the message was submitted according to 	GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

## 34.3	Short message service cell broadcast

This test only applies to MSs which support SMS-Cell Broadcast.

### 34.3.1	Conformance requirements

The MS is responsible for recombination of the four blocks received via the redio path into a single block which constitutes the cell broadcase short message.

In idle mode, the MS listens to the BCCH and to the paging sub-channel for the paging group it belongs to. The MS is required to receive and analyse the paging messages and immediate assignment messages sent on the paging subchannel corresponding to its paging subgroup.

Reference

GSM 03.41; 8

GSM 04.08; 3.2.1, 3.3.2.1

### 34.3.2	Test purpose

This test verifies that an MS is able to receive SMS-CB messages.

This test verifies that an MS is able to respond to a paging requested during the transmission of a cell broadcast short message.

### 34.3.3	Test method

Initial conditions

System Simulator:

1 cell, default parameters

The SS provides a BCCH/CCCH to support the MS in idle mode.

Periodic location updating is disabled.

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support for short message transmission cell broadcast

Description of the basic procedures to display a cell broadcasted short message

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	Three Cell Broadcast (CB) messages are sent by the SS on the CBCH with sequence numbers 0,1,1.

b)	Step a) is repeated, but the SS pages the MS during the transmission of the second CB message.

The MS shall respond to the page.

Maximum Duration of Test

-

Expected Sequence

Since the SMS-CB messages are sent continously, a table is not applicable in this test.

Specific Message Contents:

Cell broadcast test message content

Information element	Comment	Value

Serial Number	Message code	See above

Message identifier		0

Data Coding Scheme		0

Page parameter		0001 0001

Contents of message	93 user characters

using all characters

of default 7 bit

coded alphabet

SYSTEM INFORMATION TYPE 4

As default except:

Information element				Value/remark

CBCH Channel Description

- Channel type and TDMA offset		SDDCH/8 + SACCH/C8 or CBCH (SDDCH/8)

- Timeslot number				Arbitrary but mot zero

- Training sequence code			Arbitrary

- Hopping channel				Single RF channel

-Channel selector				Channel number 20 (for GSM900 MS)

Channel number 590 (for DCS1800 MS)

CBCH Mobile Allocation				Empty

# 35	Low battery voltage detection

## 35.1	Definition and applicability

Low battery or shutdown voltage detection is used to trigger inhibition of all RF transmission before the MS supply voltage reaches a level where effective use of the radio frequency spectrum is no longer guaranteed.

The requirements and this test apply to all types of GSM900 and DCS1800 MS.

## 35.2	Conformance requirement

1.	The MS shall not make ineffective use of the radio frequency spectrum. In no case shall the MS exceed the transmitted levels as defined in GSM 05.05 for extreme operation.

GSM 05.05, Annex D.2.2

2.	The MS shall inhibit all RF transmission when the power supply voltage is below the manufacturer declared approximate shutdown voltage.

GSM 05.05, Annex D.2.2

## 35.3	Test purpose

1.	To verify that the MS does not make ineffective use of the RF spectrum.

2.	To verify that the MS inhibits all RF transmission when the battery voltage falls below the manufacturer declared shutdown level.

## 35.4	Method of test

### 35.4.1	Initial conditions

The SS transmits a BCCH with a location updating time set to 0.1 hours.

The SS sends a paging request message to the MS.

The MS responds with a channel request message.

The SS sends an immediate assignment message establishing an SDCCH.

### 35.4.2	Procedure

a)	The SS gradually reduces the power supply voltage until the MS ceases the production of RF output.

The RF output spectrum shall be monitored for any anomalies while the supply voltage is being reduced.

NOTE :	The declared approximate shutdown voltage gives an indication of the voltage where the MS will cease RF output.

NOTE :	If any anomalies occur, then additional testing using the transmitter tests at the voltage where the anomaly occured is performed to determine in an objective manner, whether or not the conformance requirement is met.

c)	After 7 minutes, the SS sends a paging message to the MS.

d)	The SS observes whether or not the MS produces any RF output.

This measurement is performed over the relevant  transmit band.

The spectrum analyser is set to:

Bandwidth: 3 MHz

Peak Hold

e)	The SS modifies the location area of the BCCH.

f)	For 7 minutes, the SS observes whether or not the MS produces any RF output.

NOTE :	It is anticipated that the MS might attempt location updating.

g)	The MS is switched off and on.

h)	The SS pages the MS.

i)	The SS observes whether or not the MS produces any RF output.

## 35.5	Test requirement

1.	In step a) no anomalies shall occur.

2.	In step a), the MS shall cease the production of RF output.

3.	In steps d), f) and i), the MS shall not produce any RF output above -30 dBm.