# 21	Received signal measurements

For evaluating the reception quality (the basis for handover and power control) the following two criteria are used:

-	signal strength (RXLEV)

-	signal quality (RXQUAL).

## 21.1	Signal strength

21.1.1	Definition and applicability

The MS reports RXLEV values related to the apparent received RF signal strength. It is necessary for these levels to attain sufficient accuracy for the correct functioning of the system.

The requirements and this test apply to all types of GSM900, DCS1800 and multiband GSM900 / DCS1800 MS.

21.1.2	Conformance requirement

1.	The RMS received signal level at the receiver input shall be measured by the MS over the full range of -110 dBm to -48 dBm with a relative accuracy between signals with levels up to 20 dB difference according to table 21-1

1.1	under normal conditions, GSM 05.08, 8.1.2 and GSM 05.05, 6.2.

1.2	under extreme conditions, GSM 05.08, 8.1.2, GSM 05.05, Annex D.1, D.2.

Table 21-1: Tolerance for relative accuracy of receive signal measurement

Absolute level of	Tolerance
	lower level signal dBm	dB

GSM 	GSM 	DCS1800	DCS1800	Lower limit	Upper limit
	Small MS	Other MS	Class 1 & 2	Class 3	Single    Multi	Single     Multi

>= -88	>= -90	>= -86	>= -88	2         4	2         4
	>= -101	>= -103	>= -99	>= -101	3         5	2         5
	< -101	< -103	< -99	< -101	4         6	2         6

Single means that the measurements are on the same or different RF channel within the same frequency band.

Multi means that the measurements are on different RF channel on different frequency bands.

For measurements between ARFCN in different bands the ‘Absolute level of lower level signal’  column for the band including the lower level signal shall be used to determine which tolerance applies.

At extreme temperature conditions an extra 2 dB shall be added to the Multi limits in above table.

2.	The RMS received signal level at the receiver input shall be measured with an absolute accuracy of +/- 4 dB from -110 dBm to -70 dBm under normal conditions; GSM 05.08, 8.1.2.

3.	The RMS received signal level at the receiver input shall be measured with an absolute accuracy of +/- 6 dB over the full range of -110 dBm to -48 dBm under both normal and extreme conditions; GSM 05.08, 8.1.2.

4.	If the received signal level falls below the reference sensitivity level for the type of MS then the MS shall report a level between the reference sensitivity level and the actual received level, but with the tolerances given in conformance requirements 2. and 3. above.

5.	The measured signal level shall be mapped to an RXLEV value between 0 and 63 as specified in GSM 05.08; GSM 05.08 8.1.4.

21.1.3	Test purpose

1.	To verify that the RXLEV reported by the MS does not exceed conformance requirement 1.

1.1	under normal conditions;

1.2	under extreme conditions.

2.	To verify that the RXLEV reported by the MS does not exceed conformance requirement 2 under normal conditions.

3.	To verify that the RXLEV reported by the MS does not exceed conformance requirement 3 under extreme conditions and under normal conditions from -48 dBm to -70 dBm.

4.	To verify that the RXLEV reported by the MS does not exceed conformance requirement 4.

NOTE:	Conformance requirement 5 is inherently tested in each of the test purposes 1. to 4.

21.1.4	Method of test

21.1.4.1	Initial conditions

The SS is set to produce the BCCH of the serving cell at 63 dBVemf( ) and the BCCHs of 6 surrounding cells at 28 dBVemf(  ). The BCCH of the serving cell indicates these BCCHs, but not the BCCH of the serving cell. The ARFCN of the serving cell BCCH is chosen so as not to interfere with the other channels as shown in table 21-1. The fading profile for the BCCHs of the serving and surrounding cells will be set to static.

After 30 s, a call is set up according to the generic call set up procedure to an ARFCN, within the supported band of operation. The SACCH indicates the same surrounding cell BCCHs as the BCCH of the serving cell.

NOTE:	The 30 s is to allow the MS to scan and find all BCCHs.

21.1.4.2	Procedure

a)	The levels of the TCH and BCCHs are set according to table 21.2 step 1. The SS waits 20 s before continuing.

Table 21-2: Signal levels at receiver input in dBVemf(  )

ARFCN	TCH	BCCH1	BCCH2	BCCH3	BCCH4	BCCH5	BCCH6
	Step	GSM900:	1	62	124	20	40	80	100
		DCS1800	512	700	885	585	660	790	83

Multiband	1	885	62	124	40	100	512

1+m*21	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10	64.5-m*10
	2+m*21	54.5-m*10	63.5-m*10	54.5-m*10	54.5-m*10	54.5-m*10	54.5-m*10	54.5-m*10
	3+m*21	54.5-m*10	62.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10
	.	.	.	.	.	.	44.5-m*10	44.5-m*10
	17+m*21	54.5-m*10	.	.	.	.	44.5-m*10	44.5-m*10
	18+m*21	44.5-m*10	.	.	.	.	44.5-m*10	44.5-m*10
	.	.	.	.	.	.	44.5-m*10	44.5-m*10
	21+m*21	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10	44.5-m*10

m = 0, 1, 2, 3, 4.

b)	The measurement is done in 105 steps. The initial signal levels of the TCH of the serving cell and the BCCHs of the surrounding cells are adjusted according to table 21-2. At each step the SS keeps the signal levels stable for one reporting period, except at steps 21+m*21 where the level is held stable for 1.75 reporting periods. The RXLEV value for the period in which the change occurs (reported in the following period) is discarded. The SS records the RXLEV values reported for the surrounding cell BCCHs in steps 1 + m * 21 and 21 + m * 21. The RXLEV values for BCCH 1 are recorded by the SS for all 105 steps.

NOTE:	This extension at steps 21+m*21 is to allow an extra quarter reporting period for the MS to stabilise for steps 1+m*21.

At steps 1 to 30 the SS simulates a base station with DTX off and at steps 31 to 105 the SS simulates a base station with DTX on.

At steps 1 to 30 the SS checks the accuracy of the measured signal strength of TCH by checking the values of the parameters RXLEV-FULL and RXLEV-SUB. At steps 31 to 105 the SS shall check only the value of the parameter RXLEV-SUB.

At step 64, within every 480 ms reporting period, out of the 4 SACCH and 8 SID timeslots the SS transmits the first six active timeslots of the TCH with signal level 39.5 dBVemf(  ) and the last six active timeslots of the TCH with signal level 29.5 dBVemf(  ).

c)	Step b) is repeated under extreme conditions (Annex 1, TC2,2 and TC3).

21.1.5	Test requirements

21.1.5.1	Relative accuracy of measurements on different ARFCN

For normal and each of the 4 extreme conditions tested the following applies;

a)	For each of the steps 1, 21, 22, 42, 43, and 64, of the 7 reported RXLEV values checked, the difference between the minimum reported RXLEV value and the maximum reported RXLEV value shall be no more than 4 if the measurements are on the same or on different RF channel within the same frequency band and no more than 8 (12 for extreme temperature conditions) if the measurements are on different frequency bands.

b)	For each of the steps 63 and 85, of the 7 reported RXLEV values checked, the difference between the minimum reported RXLEV value and the maximum reported RXLEV value shall be no more than 5 for small MS or 4 for other MS if the measurements are on the same or on different RF channel within the same frequency band and no more than 9 for small MS or 8 for other MS (13 and 12 for extreme temperature conditions) if the measurements are on different frequency bands.

c)	For step 64, of the 7 reported RXLEV values checked, the difference between the minimum reported RXLEV value and the maximum reported RXLEV value shall be no more than 5 if the measurements are on the same or on different RF channel within the same frequency band and no more than 9 (13 for extreme temperature conditions) if the measurements are on different frequency bands.

d)	For step 105, of the reported RXLEV values checked, the difference between the minimum reported RXLEV value and the maximum reported RXLEV value shall be no more than 6 if the measurements are on the same or on different RF channel within the same frequency band and no more than 10 (14 for extreme temperature conditions) if the measurements are on different frequency bands.

NOTE:	It is not mandatory for the MS to report any of the BCCHs in step 105.

21.1.5.2	Relative accuracy at a single frequency (BCCH1)

For normal and each of the 4 extreme conditions tested the following applies;

For n <= 21 and RXLEV1 = 63

RXLEVn - (63 - n + r) shall be between:

-2 and +2

NOTE:	This formula allows for an MS with an absolute accuracy worse than +0.5 dB and therefore reporting an RXLEV of 63 for more than one step. The formula checks the relative accuracy from the lowest input level for which the MS last reports RXLEV of 63.

Otherwise

RXLEV(m*21+1) - RXLEV(m*21+n) - n + 1 shall be between:

-2 and +2

for steps 2 to 62 and 65 to 71 for DCS 1800 class 1/2 MS or to 73 for DCS 1800 class 3 and Small GSM MS; or 2 to 75 for other MS.

-3 and +2

for steps 63 and 74 to 96 for DCS 1800 class 1/2 MS or to 98 for DCS 1800 class 3 and Small GSM MS; or 76 to 100 for other MS.

-4 and +2

for steps 97 to 105 for DCS 1800 class 1/2 MS; or steps 99 to 105 for DCS 1800 class 3 and Small GSM MS; or 101 to 105 for other MS.

where: 1 < n <= 21 and 0 <= m <= 4 as identified in table 21-2, and r is the number of the last step where RXLEV of 63 was reported.

NOTE:	It is not mandatory for the MS to report BCCH1 for steps greater than 99 for GSM900 Small MS or 101 for other GSM MS or 97 for a DCS1800 Class 1 or Class 2 MS and 99 for DCS1800 Class 3 MS. If the MS reports a level and the upper limit for this step in the above formulae implies a level below the reference sensitivity level for the type of MS, then the upper limit shall be considered as equal to a value corresponding to the reference sensitivity level.

21.1.5.3	Absolute accuracy

For each BCCH reported, |RXLEVMS + m * 10 - 62| shall be no more than:

4	for steps 64 and 85 under normal conditions.

6	for steps 64 and 85 under extreme conditions.

6	for steps 1, 22 and 43 under normal and extreme conditions.

where: 0 <= m <= 4 as identified in table 21-2.

## 21.2	Signal strength selectivity

21.2.1	Definition and applicability

The signal strength selectivity is a measure of the ability of the signal strength measuring part of the MS to discriminate against RF power from adjacent ARFCN.

The requirements and this test apply to all types of GSM900 and DCS1800 MS.

21.2.2	Conformance requirement

The selectivity of the received signal measurement shall be as follows:

-	for adjacent (200 kHz) channel; >= 16 dB

-	for adjacent (400 kHz) channel; >= 48 dB

-	for adjacent (600 kHz) channel; >= 56 dB

GSM 05.08, 8.1.2

21.2.3	Test purpose

To verify that the MS meets the conformance requirement at the 200 kHz adjacent channel above and below the wanted.

21.2.4	Method of test

21.2.4.1	Initial conditions

For GSM900:

A call is set up according to the generic call set up procedure on ARFCN 40 and with surrounding cell BCCH3 indicated in the BA list at ARFCN 80.

The RF level of the TCH and BCCH3 is set to 20 dB above reference sensitivity level(  ).

BCCH1 and 2 at ARFCN 41 and 79 are off.

These conditions are kept for 30 s to ensure the MS has time to decode the BCCH.

For DCS1800:

A call is set up according to the generic call set up procedure on ARFCN 690 and with surrounding cell BCCH3 indicated in the BA list at ARFCN 790.

The RF level of the TCH and BCCH3 is set to 20 dB above reference sensitivity level(  ).

BCCH1 and 2 at ARFCN 691 and 789 are off.

These conditions are kept for 30 s to ensure the MS has time to decode the BCCH.

21.2.4.2	Procedure

a)	The SS records the RXLEV values reported for the TCH and BCCH3.

b)	BCCH1 and 2 are set to 9 dB above the signal level of the TCH and BCCH3.

c)	These conditions are kept for 30 s.

d)	The SS records the RXLEV values reported for the TCH and BCCH3.

21.2.5	Test requirements

The values of RXLEV recorded in step d) shall be no more than 1 above the values recorded in step a).

NOTE:	This one change in the reported value of RXLEV is because the MS may have been on the edge between two reported levels.

## 21.3	Signal quality under static conditions

21.3.1	Definition and applicability

The MS must be capable of measuring the received signal quality, which is specified in terms of bit error ratio (BER) before channel decoding averaged over the reporting period of length of one SACCH multiframe defined in section 8.4 of GSM 05.08. The MS has to map this BER into RXQUAL values using the coding scheme defined in section 8.2.4 of GSM 05.08.

In order to assess the correct operation of the quality reporting algorithm in the MS, the SS will measure the BER of class II bits and will assume that it is equal to that before channel coding.

The test is performed for two channel propagation conditions: static and TU50. In static conditions two kind of channels are considered: full rate and DTX. In TU50 conditions, only the full-rate channel is considered.

This test is only applicable to MS supporting speech.

21.3.2	Conformance requirement

1.	The received signal quality shall be measured by the MS in a manner that can be related to an equivalent average BER before channel decoding (i.e. chip error ratio), assessed over the reporting period of 1 SACCH block. When the quality is assessed over the full-set and sub-set of frames, eight levels of RXQUAL are defined and shall be mapped to the equivalent BER before channel decoding as per the table in GSM 05.08 8.2.4; GSM 05.08 8.2.2, 8.2.4;

2.	The reported parameters (RXQUAL) shall be the received signal quality, averaged over the reporting period of length one SACCH multiframe; GSM 05.08 8.2.3.

21.3.3	Test purpose

1.	To verify that the received signal quality is measured and reported to the eight levels of RXQUAL_FULL and RXQUAL_SUB by the MS in a manner that can be related to an equivalent average BER before channel decoding (i.e. chip error ratio), assessed over the reporting period of 1 SACCH block (104 TDMA frames) for the TCH/FS. The probability that the correct RXQUAL band is reported shall meet the values given in the specification , under static conditions, GSM 05.08 8.2.4.

2.	To verify that the reported parameters (RXQUAL) is the received signal quality, averaged over the reporting period of length one SACCH block (104 TDMA frames on the TCH).

21.3.4	Method of test

a)	A call shall be set up according to the generic call set up procedure on a channel in the mid ARFCN range. The radio link timeout is set to maximum.

The SS is set to produce a wanted signal and an independent uncorrelated interfering (unwanted) signal at the same time, both with static propagation characteristics. The wanted signal shall be the standard test signal C1. It shall be at the nominal frequency of the receiver and its level shall be 28 dBVemf(  ). The unwanted signal shall have no fixed relationship with the bit transitions of the wanted signal, and it shall be modulated with random data. The unwanted signal shall be continuous, with a nominal frequency 200 kHz above the nominal frequency of the wanted signal. The MS shall be operated in the encrypted mode.

b)	The SS commands the MS to create the loop back facility from the receiver channel decoder output to the transmitter encoder input.

c)	The SS performs a calibration of C/I to class II RBER over the range of the RXQUAL cases in the table in step d). It is expected that the range of the unwanted signal level will be approximately 35 dBV emf(  ) to 50 dBV emf(  ) and to calibrate over 2000 errors would give sufficient accuracy.

d)	A curve of BER of class II bits is drawn vs the ratio of the level of the wanted to the unwanted signal (C/I) and the correspondence of the table below is derived from it.

CASE	AVERAGE BER	C/I

0	0.09%	(C/I)0
	1	0.28%	(C/I)1
	2	0.57%	(C/I)2
	3	1.13%	(C/I)3
	4	2.26%	(C/I)4
	5	4.53%	(C/I)5
	6	9.05%	(C/I)6
	7	18.10%	(C/I)7

e)	The level of the unwanted signal is set at a value for which C/I = (C/I)i, with i equal to one out of the numbers 0, 1, ... 7. At such C/I the expected quality band reported by the MS is RXQUAL_i.

f)	At the end of each SACCH block, the SS increases by 1 a counter of the number of errors in reporting the RXQUAL band only if such band is different from the expected one (that is RXQUAL_i). Moreover a counter of the number of errors of the class II bits is properly increased.

g)	The procedure stated in f) is repeated 350 times (see note 3 below). If the ratio between the number of errors of the class II bits and the number of the class II bits occurring within 350 (see note 2 below) SACCH blocks is within +7% (or -7%) (see note 1 below) from the value of the BER corresponding to the case i, the value of max-events is increased by the recorded number of errors in reporting the RXQUAL band and max-samples is increased by 350. If outside the +/- 7% the level of the unwanted signal may be changed or not (see note 1 below).

NOTE 1:	For RXQUAL 0, if it is not possible to adjust the BER for the +/- 7% band, it is acceptable to use the nearest RF power level which gives BER better than 0.1% (which is the value specified in GSM 05.08 section 8.2.4).

NOTE 2:	If the BER measured over the 350 (see note 3 below) SACCH blocks is close to the +/- 7% target it may be preferable not to adjust the level but to repeat the sample.

h)	Steps f) and g) shall be repeated until the number of samples of case i is equal to max-samples_i, or the number of errors in reporting the RXQUAL band exceeds max-events_i.

i)	The SS is set to simulate a base station in DTX mode according to section 8.3 of GSM 05.08 and step f) is repeated until the number of samples of case i is equal to max-samples i or the number of errors in reporting the RXQUAL band exceeds max-events i.

j)	The SS is set to simulate a base station in non DTX mode.

k)	Steps from e) to j) shall be repeated for each of the (C/I)i values, with i = 0, 1, ... 7.

NOTE 3:	Due to the high error rates involved with testing RXQUAL 7, the MS would experience a radio link timeout. To avoid this the test is performed using 60 SACCH blocks instead of 350. In between measurements, at least 35 SACCH blocks, are transmitted with a lower level of the unwanted signal.

21.3.5	Test requirements

The number of errors in reporting the RXQUAL bands recorded in each of the considered cases shall not exceed the corresponding value of max-events shown in the following tables, when the number of samples relevant to the case under test is equal to max-samples.

The two tables below refer to RXQUAL_FULL bands (relevant to the full-rate channel quality evaluation) and RXQUAL_SUB bands (relevant to the quality evaluation of the channel operating in DTX mode) respectively.

CASE	Expected	Specified	Max-events	Max-samples
		RXQUAL_FULL	reporting
			error rate

0	RXQUAL_0	10%	211	1750
	1	RXQUAL_1	25%	211	700
	2	RXQUAL_2	15%	254	1400
	3	RXQUAL_3	10%	211	1750
	4	RXQUAL_4	10%	211	1750
	5	RXQUAL_5	5%	213	3500
	6	RXQUAL_6	5%	213	3500
	7	RXQUAL_7	5%	200	3300

CASE	Expected	Specified	Max-events	Max-samples
		RXQUAL_SUB	reporting
			error rate

0	RXQUAL_0	35%	300	750
	1	RXQUAL_1	65%	400	560
	2	RXQUAL_2	55%	400	660
	3	RXQUAL_3	55%	400	660
	4	RXQUAL_4	40%	300	660
	5	RXQUAL_5	30%	300	870
	6	RXQUAL_6	20%	200	835
	7	RXQUAL_7	15%	200	1105

## 21.4	Signal quality under TU50 propagation conditions

21.4.1	Definition and applicability

21.4.2	Conformance requirement

1.	The received signal quality shall be measured by the MS in a manner that can be related to an equivalent average BER before channel decoding (i.e. chip error ratio), assessed over the reporting period of 1 SACCH block, in TU50 conditions. When the quality is assessed over the full-set and sub-set of frames, eight levels of RXQUAL are defined and shall be mapped to the equivalent BER before channel decoding as required in the in GSM 05.08 8.2.4.

1)	GSM 05.08 8.2.2;

2.	The reported parameters (RXQUAL) shall be the received signal quality, averaged over the reporting period of length one SACCH block.

2)	GSM 05.08 8.2.3.

21.4.3	Test purpose

1.	To verify, under TU50 conditions, that the received signal quality is measured and reported to the eight levels of RXQUAL_FULL by the MS in a manner that can be related to an equivalent average BER before channel decoding (i.e. chip error ratio), assessed over the reporting period of 1 SACCH block (104 TDMA frames) for the TCH/FS. The probability that the correct RXQUAL band is reported shall meet the values referred in the specification GSM 05.08 8.2.4, and given in the specification GSM 11.10 21.2.2.2.

2.	To verify that the reported parameters (RXQUAL) is the received signal quality, averaged over the reporting period of length one SACCH  block (104 TDMA frames on the TCH).

21.4.4	Method of test

a)	A call shall be set up according to the generic call set up procedure on a channel in the mid ARFCN range

The SS is set to produce the standard test signal C1 with TU50 propagation profile. It shall be at the nominal frequency of the receiver at a median level of 28 dBVemf(  ).

b)	The SS commands the MS to create the loop back facility from the receiver channel decoder output to the transmitter encoder input.

c)	The SS counts the number of errors of the class II bits (occurring in 50 SACCH blocks) and the relevant BER is computed. If such BER does not correspond to one out of the 8 different cases shown in the table below, the test shall be continued from step f). On the contrary, if such BER corresponds to one out of the 8 different cases shown in the table below, let us say case i (with i equal to 1 out of the 8 numbers 0, 1, ... 7), the procedure stated from step d) on shall be adopted.

d)	The SS records all the RXQUAL bands reported at the end of each of the 50 SACCH blocks. A counter max-samples_i shall be increased by 50, and the SS verifies whether each of the quality band reported by the MS is equal to RXQUAL_i (with i = 0, 1, ... 7), or to one of the adjacent bands RXQUAL_(i-1) (with i = 1, 2, ..., 7) and RXQUAL_(i+1) (with i = 0, 1, ..., 6). For each failure (to be in the correct or one of the adjacent bands) that is found, if any, a counter max-events_i shall be increased by one.

CASE	Average BER (%) of the class II bits

0	< 0.10
	1	0.26 to 0.30
	2	0.51 to 0.64
	3	1.0 to 1.3
	4	1.9 to 2.7
	5	3.8 to 5.4
	6	7.6 to 11
	7	> 15

e)	Step c) shall be repeated 20 times.

f)	The SS is set to produce also an independent, uncorrelated interfering (unwanted) signal with TU50 propagation profile. The unwanted signal shall be on the same channel as the wanted channel but shall have no fixed relationship with the bit transitions of the wanted signal and it shall be modulated with random data. Its median level shall be 25 dB below the median level of the wanted signal.

g)	The SS counts the number of error of the class II bits occurring in 50 SACCH blocks and the relevant BER is computed.

h)	If such BER does not correspond to one out of the 8 cases shown in the table above, the median level of the unwanted signal shall be increased (or decreased) by 0.6 dB and step g) shall be repeated. On the contrary, if the computed BER corresponds to one out of the 8 cases shown in the table above, step d) shall be repeated.

i)	Steps g) and h) shall be repeated 20 times.

j)	The median level of the unwanted signal is increased at step of 2 dB up to a value of 5 dB above that of the wanted signal and, at each step, the procedure stated from g) to i) shall be repeated.

k)	Step j) shall be stopped when max-samples is greater than 200.

NOTE:	If, at the end of the whole test, one or more max-samples_i (with i = 0, 1, ...,7) is lower than 200, the corresponding case shall be skipped out, being statistically not significant.

21.4.5	Test requirements

For each of the examined cases of the table above, the error rate in reporting the quality band is computed as the ratio between the recorded number of reporting errors max-events_i, and the corresponding number of the samples max-samples_i. The test is passed if the computed reporting error rate does not exceed the max_reporting error rate shown in the table below.

CASE	Expected	Specified reporting	Max_reporting
		RXQUAL_FULL	error rate	error rate

0	RXQUAL_0/1	15%	18%
	1	RXQUAL_1/0/2	15%	18%
	2	RXQUAL_2/1/3	15%	18%
	3	RXQUAL_3/2/4	10%	12%
	4	RXQUAL_4/3/5	10%	12%
	5	RXQUAL_5/4/6	10%	12%
	6	RXQUAL_6/5/7	10%	12%
	7	RXQUAL_7/6	10%	12%