# 11	General tests

## 11.1	Verification of support and non-support of services (multiple numbering scheme or ISDN)

### 11.1.1	Mobile Terminated (MT) calls

11.1.1.1	Definition and applicability

This test applies to all MS for all Mobile Terminated Bearer Services / Teleservices according to TS GSM 02.02 and 02.03 except Teleservices 21, 22 and 23.

11.1.1.2	Conformance requirement

1.	The MS shall check the Information Elements for Bearer Capability in a received SETUP message, and if it agrees to the proposed set, it shall respond with a CALL CONFIRMED message.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3

2.	The MS in the "Null" state, U0, ready to receive a SETUP shall reject a SETUP with Information Elements for Bearer Capability which are incompatible with the Bearer Services / Teleservices supported by the MS, and shall send a RELEASE COMPLETE message.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3.1; GSM 07.01, 8.3.1; GSM 04.08, B.3.2

11.1.1.3	Test purpose

1.	To verify that the MS, for the case of the Multinumbering scheme or ISDN, accepts a SETUP message, where the Information Elements for Bearer Capability are compatible with the Bearer Services / Teleservices declared as supported by the MS, by sending a CALL CONFIRMED message.

This is verified for all Mobile Terminated Bearer Services / Teleservices declared as supported by the MS.

2.	To verify that the MS in the "Null" state, U0, when receiving a SETUP message containing incompatible Information Elements for Bearer Capability will respond with a RELEASE COMPLETE message.

This is verified for all Mobile Terminated Bearer Services / Teleservices not declared as supported by the MS.

11.1.1.4	Method of test

11.1.1.4.1	Initial conditions

For an MS with an external interface the interface shall be setup in such a way that the MS is able to successfully receive the call for the service in question. The manufacturer shall state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted.

The PIXIT statement for the service in question shall be consistent with the PICS statement made by the manufacturer and will result for this tests in one or several valid BC codings as presented in section 11.8.

The generic call set-up procedure shall be followed up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

11.1.1.4.2	Procedure

a)	For a Mobile Terminated Bearer Service / Teleservice declared as supported by the MS. The SS transmits a SETUP message.

The SETUP shall contain a single or dual BC-IE where the parameter values are arbitrarily selected among those declared as supported by the MS in PIXIT statements and corresponding to the Bearer Service / Teleservice being tested.

b)	If more than one BC-IE (or pair of) correspond to the Bearer Service / Teleservice being tested, step a) is repeated once (and only once) with another single or dual BC-IE. The BC-IE shall be chosen in such a way that as many parameters as possible are different from the previous BC-IE. In particular, if more than one value for the "Connection Element" parameter is possible, the new BC-IE shall contain a different value from the previous one for this parameter.

c)	Step a) and b) are repeated for all Bearer Services / Teleservices declared as supported by the MS.

d)	For an Mobile Terminated Bearer Service / Teleservice not declared as supported by the MS. The SS transmits SETUP. If the MS supports TS62 but not TS61, then TS61 is not tested.

The SETUP shall contain a single or dual BC-IE where the parameter values are arbitrarily selected among those defined in TS GSM 07.01 Annex II and corresponding to the Bearer Service / Teleservice being tested. The complete coding of the corresponding BC-IE(s) can be found in section 11.8.

e)	Step d) is repeated for all Bearer Services / Teleservices not declared as supported by the MS.

11.1.1.5	Test requirement

1)	After steps a), b) and c), the MS shall send a CALL CONFIRMED message. The MS may contain a single or dual BC-IE. If present these IEs are not checked.

2)	After steps d) and e), the MS shall send a RELEASE COMPLETE message with cause value 88 - incompatible destination..

### 11.1.2	Mobile Originated (MO) calls

11.1.2.1	Definition and applicability

This test applies to all MS for all Mobile Originated Bearer Services / Teleservices according to TS GSM 02.02 and 02.03 except Teleservices 21, 22 and 23.

11.1.2.2	Conformance requirement

1.	The MS shall set up a call with a SETUP message containing a single or multiple BC-IE and a single LLC according to the actual configuration of the MS. Two bearer capabilities can be present only in the cases described in GSM 07.01.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3.1; GSM 04.08, 9.3.2; GSM 07.01, 8.3.3

2.	The Repeat Indicator Information Element shall be included in the SETUP message, when the in-call modification procedure is used.

GSM 04.08, 9.3.2

11.1.2.3	Test purpose

1.	To verify that the MS generates a SETUP message which includes a single or multiple Bearer Capability and a single LLC, according to the actual configuration on the MS.

This is verified for all Mobile Originated Bearer Services / Teleservices described in GSM 07.01 and declared as supported by the MS.

2.	To verify that the MS includes a correctly encoded Repeat Indicator if it includes multiple Bearer Capabilities in the SETUP message.

11.1.2.4	Method of test

11.1.2.4.1	Initial conditions

If possible, the MS shall be configured to initiate an outgoing call with a specified BC and with the corresponding LLC when the ITC value is "unrestricted digital" or "3.1 kHz ex PLMN", in the SETUP message. The manufacturer must state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an outgoing call can be initiated.

The PIXIT statement for the service in question shall be consistent with the PICS statement made by the manufacturer and will result for this test in one or several valid BC codings as presented in section 11.8.

11.1.2.4.2	Procedure

a)	The MS shall be made to initiate a call.

b)	If the MS can be configured to send a specific BC, the test is repeated with the MS configured for all possible preferred Bearer Services and Teleservices declared as supported by the MS. The complete coding of the corresponding BC-IE(s) can be found in section 11.8.

11.1.2.5	Test requirement

The MS shall send a SETUP message, which shall contain the BC among those declared as supported by the MS and the corresponding LLC in the case of ITC value "unrestricted digital" or "3.1 kHz ex PLMN". If the MS is configured to send a specific BC, the SETUP message shall contain this particular BC. The BC-IE(s) and the corresponding LLC shall be set according to TS GSM 07.01.

Where two BCs are contained in the SETUP message, it shall be checked that the combination is allowed, according to TS GSM 07.01 and that a REPEAT INDICATOR is also included.

## 11.2	Verification of support of the single numbering scheme

11.2.1	Definition and applicability

This test applies to all MS.

11.2.2	Conformance requirement

1.	The MS shall respond to a SETUP message containing no BC-IE with a CALL CONFIRMED message including the single or multiple Bearer Capability, according to the actual configuration of the MS. Two bearer capabilities can be present only in the cases described in GSM 07.01.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3.1; GSM 04.08, 9.3.2; GSM 07.01, 8.3.3

2.	The Repeat Indicator Information Element shall be included in the CALL CONFIRMED message, when the in-call modification procedure is used, and no Bearer Capability Information Element is included in the received SETUP message.

GSM 04.08, 9.3.2

11.2.3	Test purpose

1.	To verify that the MS, for the case of the Single Numbering Scheme, accepts a SETUP message, where the Information Elements for Bearer Capability and Lower and Higher Layer Compatibility are not present by sending a CALL CONFIRMED message, which includes the single or multiple Bearer Capabilities, according to the actual configuration on the MS.

This is verified for one Mobile Terminated Bearer Service / Teleservice described in GSM 07.01 and declared as supported by the MS.

2.	To verify that the MS includes a correctly encoded Repeat Indicator if it includes multiple Bearer Capabilities in the CALL CONFIRMED message.

11.2.4	Method of test

11.2.4.1	Initial conditions

The MS is setup to receive a call. If possible, the MS shall be configured to respond to an incoming call with a specified BC selected arbitrarily from those declared as supported by the MS, in the CALL CONFIRMED message, in reply to a SETUP message with no BC, LLC or HLC elements. The manufacturer must state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted.

The generic call set-up procedure shall be followed up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

The PIXIT statement for the service in question shall be consistent with the PICS statement made by the manufacturer and will result for this tests in one or several valid BC codings as presented in section 11.18.

11.2.4.2	Procedure

The SS transmits a SETUP message with no BC, LLC or HLC elements.

11.2.5	Test requirement

The MS shall send a CALL CONFIRMED message, which shall contain the BC among those declared as supported by the MS. If the MS is configured to respond with a specific BC, the CALL CONFIRMED message shall contain this particular BC. The BC-IE shall be coded according to TS GSM 07.01.

Where two BCs are contained in the CALL CONFIRMED message, it shall be checked that the combination is allowed, according to TS GSM 07.01 and that a REPEAT INDICATOR is also included.

## 11.3	Verification of non-support of services (Advice of Charge Charging (AOCC))

11.3.1	Definition and applicability

If the manufacturer of the MS under test claims that the mobile supports Advice of Charge Charging then the tests described in GSM 11.10 section 31.6 shall be carried out. However if the manufacturer claims that the mobile does not support AOCC it shall be verified that the MS under test does not support AOCC. The test to verify non support of AOCC is described below.

11.3.2	Conformance requirement

1.	An MS claiming to not support AOCC and in the outgoing call / U4 call delivered state, on receipt of a CONNECT message containing AOCC information shall acknowledge the CONNECT message but ignore and not acknowledge the AOCC information sent within the CONNECT.

2.	An MS claiming to not support AOCC and in the outgoing call / U4 call delivered state, on receipt of a FACILITY message containing AOCC information shall ignore and not acknowledge the AOCC information contained within the FACILITY.

3.	An MS claiming to not support AOCC and in the incoming call / U9 call confirmed state, on receipt of a FACILITY message containing AOCC information shall ignore and not acknowledge the AOCC information contained within the FACILITY.

4.	An MS claiming to not support AOCC and in the U10 call active state, on receipt of a FACILITY message containing AOCC information shall ignore and not acknowledge the AOCC information contained within the FACILITY.

GSM 03.86 sections 1.2, 1.3, 2.2, 2.3; GSM 04.86 section 2.

11.3.3	Test purpose

1.	To verify that an MS claiming to not support AOCC and in the outgoing call / U4 call delivered state, on receipt of a CONNECT message containing AOCC information acknowledges the CONNECT message but ignores and does not acknowledge the AOCC information sent within the CONNECT.

2.	To verify that an MS claiming to not support AOCC and in the outgoing call / U4 call delivered state, on receipt of a FACILITY message containing AOCC information ignores and does not acknowledge the AOCC information contained within the FACILITY.

3.	To verify that an MS claiming to not support AOCC and in the incoming call / U9 call confirmed state, on receipt of a FACILITY message containing AOCC information ignores and does not acknowledge the AOCC information contained within the FACILITY.

4.	To verify that an MS claiming to not support AOCC and in the U10 call active state, on receipt of a FACILITY message containing AOCC information ignores and does not acknowledge the AOCC information contained within the FACILITY.

11.3.4	Method of test

11.3.4.1	Initial conditions

The generic call set up procedures are followed up to and including the reception, or transmission, of the ALERTING message by the MS.

11.3.4.2	Procedure

a)	For an Mobile Originated call in the U4 state the SS transmits CONNECT containing AOCC information.

b)	For an Mobile Originated call in the U4 state the SS transmits FACILITY containing AOCC information.

c)	For an Mobile Terminated call in the U9 state the SS transmits a FACILITY containing AOCC information.

d)	For a call in the U10 state the SS transmits a FACILITY containing AOCC information

11.3.5	Test requirement

The MS shall ignore the AOCC information sent to it in the Facility information elements as part of the CONNECT/FACILITY messages and not send any AOCC information acknowledgement. It shall be checked for 15 seconds that the MS does not transmit any AOCC information acknowledgement after the receipt of AOCC information.

## 11.4	Verification of non-support of services (call hold)

11.4.1	Definition and applicability

Applicable for MSs that claim to not support the Call-Hold supplementary service. [This test applies only to MSs that claim to support AOCC.]

11.4.2	Conformance requirement

An MS claiming to not support the Call Hold supplementary service and in the U10 call active state shall, when the appropriate Call Hold MMI command is entered:

-	Fail to put the first call on hold

-	Fail to place the second call

-	Optionally provide some indication to the user of an error.

GSM 02.83; GSM 04.83

11.4.3	Test purpose

To verify that an MS claiming to not support the Call Hold supplementary service and in the U10 call active state, reacts in the following manner when the appropriate call hold MMI command is entered:

-	MS fails to put the first call on hold

-	MS fails to place the second call.

-	Optionally provides some indication to the user of an error.

11.4.4	Method of test

11.4.4.1	Initial conditions

The mobile originating generic call set up procedures shall be followed up to and including the transmission by the MS of the CONNECT ACKNOWLEDGE to place the call in the U10 call active state.

11.4.4.2	Procedure

A second directory number is entered followed by "SEND" via the MMI.

11.4.5	Test requirement

The MS shall not send any HOLD messages on the dedicated channel. This is checked for 3 seconds.

The MS may however send other messages.

The MS may also give the user an indication of the error that has occurred.

## 11.5	Verification of non-support of services (multiparty)

11.5.1	Definition and applicability

Applicable for MSs that claim to support the Call-Hold supplementary service but not the MultiParty supplementary service.[This test applies only to MSs that claim to support AOCC.]

11.5.2	Conformance requirement

An MS claiming to not support the MultiParty supplementary service and in the U10 call active state with one call and in the held state with another call shall, when the appropriate MultiParty MMI command is entered:

-	Fail to combine the three parties in a MultiParty call.

-	Optionally provide some indication to the user of an error.

GSM 02.83, 02.84; GSM 04.83, 04.84

11.5.3	Test purpose

To verify that an MS claiming to not support the MultiParty supplementary service and in the U10 call active state with one call and another call on hold, reacts in the following manner when the appropriate MultiParty MMI command is entered:

-	Fails to combine the three parties in a MultiParty call.

-	Optionally provides some indication to the user of an error.

11.5.4	Method of test

11.5.4.1	Initial conditions

The mobile originating generic call set up procedures shall be followed up to and including the transmission by the MS of the CONNECT ACKNOWLEDGE to place the call in the U10 call active state. A second directory number is then entered followed by send to put the first call on hold and place a second call.

11.5.4.2	Procedure

"3" followed by "SEND" is entered via the MMI.

11.5.5	Test requirement

The MS shall not send a FACILITY message, containing the build multiparty request, on the dedicated channel. This is checked for 3 seconds.

The MS may however send other messages.

The MS may also give the user an indication of the error that has occurred.

## 11.6	Verification of non-support of feature (Fixed Dialling Number (FDN))

11.6.1	Definition and applicability

If the manufacturer of the Mobile equipment under test claims that the mobile supports Fixed Dialling Number then the tests in section 27.18 shall be carried out. However if the manufacturer claims that the mobile does not support FDN it shall be verified that the MS under test does not support FDN. The test to verify non support of FDN is described below.

11.6.2	Conformance requirement

1.	An MS claiming to not support FDN that has a SIM with FDN allocated and activated in its SIM Service Table (Service Number 3) and has FDN "enabled" shall refuse a request from the user to attempt an outgoing call.

2.	An MS claiming to not support FDN that has a SIM with FDN allocated and activated in its SIM Service Table (Service Number 3) and has FDN "enabled" shall not respond to paging.

3.	An MS claiming not to support FDN that has a SIM with FDN allocated and activated shall not attempt to rehabilitate the IMSI and Location Information Elementary Files of the SIM.

GSM 02.07 section B3.2, GSM 11.11 section 11.2.1.

11.6.3	Test purpose

1.	To verify that an MS claiming to not support FDN and that has a SIM with FDN allocated and activated in its SIM Service Table and has FDN “enabled”. i.e. AND, IMSI and Location Information Elementary Files are Invalidated inserted, it refuses an attempt to make an outgoing call made by the user.

2.	To verify that an MS claiming to not support FDN and that has a SIM with FDN allocated and activated in its SIM Service Table and has FDN “enabled”. i.e. AND, IMSI and Location Information Elementary Files are Invalidated inserted, it does not answer to paging.

3.	To verify that an MS claiming not to support FDN and that has a SIM with FDN allocated and activated in its SIM Service Table and has FDN “enabled”. i.e. AND, IMSI and Location Information Elementary Files are Invalidated inserted, does not attempt to rehabilitate IMSI and Location Information.

11.6.4	Method of test

11.6.4.1	Initial conditions

The ME is powered off. No SIM is inserted in the ME.

11.6.4.2	Procedure

a)	A SIM with FDN allocated and activated in its SIM Service Table and has FDN "enabled" is inserted in the ME and the MS is powered on.

b)	An outgoing CM connection is attempted by the user.

c)	The MS paged with its IMSI.

d)	The MS is powered off and the SIM is examined using a suitable tool to determine if the IMSI and Location Information Elementary Files have been Rehabilitated.

11.6.5	Test requirement

1)	in step b), the MS shall not send a CHANNEL REQUEST message.

2)	in step c), the MS shall not send a CHANNEL REQUEST message.

3)	in step d), the IMSI an Location Information Elementary Files shall be Invalidated.

## 11.7	IMEI Security

11.7.1	Conformance requirements

11.7.1.1	Programming security

The manufacturer declares that concerning the programming security he has taken necessary and sufficient steps to ensure that either, the IMEI cannot be electrically changed once initially programmed, or, secure password techniques have been adopted to prevent unauthorised re-programming of the IMEI.

GSM 02.09, GSM 03.20

11.7.1.2	Mechanical security

The manufacturer declares that he has taken necessary and sufficient steps to ensure that unauthorised individuals or organisations cannot economically replace the component in which the IMEI is stored and thereby provide the MS with a different IMEI.

GSM 02.09, GSM 03.20

11.7.1.3	Technical knowledge and availability of programme

The manufacturer declares that he takes the necessary measurers to ensure that the technical knowledge for changing the IMEI (where applicable) will remain securely under his control and that the knowledge of the programme content will be restricted to his authorised representative(s) on a need to know basis.

GSM 02.09, GSM 03.20

11.7.2	Test purpose

To verify that the physical protection of the IMEI is sufficiently secure.

11.7.3	Method of test

The manufacturer submitting his equipment for Type Approval shall make the declarations listed in the "Conformance Requirements".

The manufacturer shall give justification to support a claim that the requirements are met.

## 11.8	Coding of the Bearer Capability information element

### 11.8.1	Network to MS Direction

#### 11.8.1.1	BS 21 to 26 - Asynchronous Service

##### 11.8.1.1.1	BS 21

11.8.1.1.1.1	3.1 kHz Audio, Transparent

BC GSM = 04 07 A2 B8 81 21 X1 4X  81

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 1 | Modem Type : V.21 |

11.8.1.1.1.2	3.1 kHz Audio, Non Transparent

BC GSM = 04 0X A2 88 81 21 X1 6X A1 (CX)

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | x | x | x | x | Length (7 or 8) depending on presence of octet 7. |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU Integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 1 | Modem Type : V.21 |
| Octet 7 (may not be present) | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 1 | x | 0 | 0 | User Inform. layer 2 protocol, Depending on the TE Configuration |

Depending of the type of flow control supported by the TE, the coding of octet 7 is different. The value ISO 6429 (0 1000) means "Inband flow control" and the value COPnoFLCT (0 1100) means "No flow control".

11.8.1.1.1.3	UDI, Transparent

BC GSM = 04 07 A1 B8 89 21 X1 4X  80

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 1 |  |  |  | Rate Adaptation : V.110 |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |

11.8.1.1.1.4	UDI, Non Transparent

BC GSM = 04 0X A1 88 89 21 X1 6X  A0 (CX)

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | x | x | x | x | Length (7 or 8) depending on presence of octet 7. |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU Integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 1 |  |  |  | Rate Adaptation : V.110 |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |
| Octet 7 (may not be present) | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 1 | x | 0 | 0 | User Inform. layer 2 protocol, Depending of the TE Configuration |

Depending of the type of flow control supported by the TE, octet 7 is present or not. If not present, it means "outband flow control". If octet 7 is present, the value ISO 6429 (0 1000) means "Inband flow control" and the value COPnoFLCT (0 1100) means "No flow control".

##### 11.8.1.1.2	BS 22

Same as BS 21 except

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If different from none, | Modem Type Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 0 | Modem V.22 |

In case of 3.1kHz Audio non transparent service, depending of the type of flow control supported by the TE, octet 7 is present or not. If not present, it means "outband flow control".

##### 11.8.1.1.3	BS 23

Same as BS 21 except

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If different from none, | Modem Type Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 1 | Modem V.22bis |

In case of 3.1kHz Audio non transparent service, depending of the type of flow control supported by the TE, octet 7 is present or not. If not present, it means "outband flow control".

##### 11.8.1.1.4	BS 24

Same as BS 21 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If different from none, | Modem Type Octet 6c : | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

In case of 3.1kHz Audio non transparent service, depending of the type of flow control supported by the TE, octet 7 is present or not. If not present, it means "outband flow control".

##### 11.8.1.1.5	BS 25

Same as BS 21 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Interm. Rate in Octet 6b : | - | 1 | 1 | - | - | - | - | - | 16 kbit/s |
| If different from none, | Modem Type Octet 6c : | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

In case of 3.1kHz Audio non transparent service, depending of the type of flow control supported by the TE, octet 7 is present or not. If not present, it means "outband flow control".

##### 11.8.1.1.6	BS 26

For MOC only.

#### 11.8.1.2	BS 31 to 34 - Synchronous Service

##### 11.8.1.2.1	BS 32

11.8.1.2.1.1	3.1 kHz Audio, Transparent, non-X.32 case

BC GSM = 04 07 A2 B8 81 20 13 43 83

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 1 | 1 | Modem Type : V.22 bis |

11.8.1.2.1.2	UDI, Transparent mode, non-X.32 case

BC GSM = 04 07 A1 B8 89 20 13 43  80

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 1 |  |  |  | Rate Adaptation : V.110 |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : none |

11.8.1.2.1.3	3.1 kHz Audio, Transparent mode, X.32 case (Packet Service)

BC GSM = 04 07 A2 B8 86 20 13 43 83

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 1 | 1 | 0 | Signalling Access Protocol : X.32 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 1 | 1 | Modem Type : V.22 bis |

11.8.1.2.1.4	3.1 kHz Audio, Non Transparent mode, X.32 case (Packet Service)

BC GSM = 04 07 A2 88 86 20 13 63 A3 C6

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 1 | 1 | 0 | Signalling Access Protocol : X.32 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non Transparent |
|  |  |  |  | 0 | 0 | 0 | 1 | 1 | Modem Type : V.22 bis |
| Octet 7 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 0 | 1 | 1 | 0 | X.25 |

11.8.1.2.1.5	UDI, Non Transparent mode, X.32 case (Packet Service)

BC GSM = 04 08 A1 88 96 20 13 63 A0 C6

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 1 | 1 | 0 | Signalling Access Protocol : X.32 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |
| Octet 7 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 0 | 1 | 1 | 0 | X.25 |

##### 11.8.1.2.2	BS 31

For non X.32 case only, same as BS 32 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If different from none, | Modem Type Octet 6c : | - | - | - | 0 | 0 | 0 | 1 | 0 | Modem V.22 |

BS31 for Packet Service does not exist.

##### 11.8.1.2.3	BS 33

Same as BS 32 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If different from none, | Modem Type Octet 6c : | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

##### 11.8.1.2.4	BS 34

Same as BS 32 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Interm. Rate in Octet 6b : | - | 1 | 1 | - | - | - | - | - | 16 kbit/s |
| If different from none, | Modem Type Octet 6c : | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

#### 11.8.1.3	BS 41 to 46 - PAD Access Asynchronous

##### 11.8.1.3.1	BS 41

11.8.1.3.1.1	UDI, Transparent

BC GSM = 04 07 A1 B8 8C 21 X1 4X  80

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 1 |  |  |  | Rate Adaptation : V.110 |
|  |  |  |  |  |  | 1 | 0 | 0 | Signalling Access Protocol : X.28 dedicated universal NUI |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Intermediate Rate : 8kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |

11.8.1.3.1.2	UDI, Non transparent

BC GSM = 04 08 A1 88 8C 21 X1 6X  A0 C8

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | Length. |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU Integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 1 |  |  |  | Rate Adaptation : V.110 |
|  |  |  |  |  |  | 1 | 0 | 0 | Signalling Access Protocol : X.28 dedicated PAD, universal NUI |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | 0 | 0 | 0 | 1 | User Rate : 0.3 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |
| Octet 7 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 1 | 0 | 0 | 0 | User Inform. layer 2 protocol : ISO6429 |

##### 11.8.1.3.2	BS 42

Same as BS 41 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 11.8.1.3.3	BS 43

Same as BS 41 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 11.8.1.3.4	BS 44

Same as BS 41 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 11.8.1.3.5	BS 45

Same as BS 41 except

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Interm. Rate in Octet 6b: | - | 1 | 1 | - | - | - | - | - | 16 kbit/s |

##### 11.8.1.3.6	BS 46

For MOC only.

#### 11.8.1.4	BS 51 to 53 - Packet Service Synchronous

##### 11.8.1.4.1	BS 51

BC GSM = 04 08 A1 88 96 20 13 63 A0 C6

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 0 | 1 | Info. Transfer Cap. : UDI |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 1 | 0 |  |  |  | Rate Adaptation : X.31 flag. |
|  |  |  |  |  |  | 1 | 1 | 0 | Signalling Access Protocol : X.32 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | 0 | 0 | 1 | 1 | User Rate : 2.4 kbit/s |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |
| Octet 7 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 0 | 1 | 1 | 0 | X.25 |

##### 11.8.1.4.2	BS 52

Same as BS 51 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 11.8.1.4.3	BS 53

Same as BS 51 except

|  | User Rate in Octet 6a : | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### 11.8.1.5	BS 61 - Alternate Speech / Data

The first BC in the Setup message is coded "Speech" : 04 01 A0

The repeat Indicator in the Setup message is coded "Circular for successive selection (alternate)" : D3

The second BC in the Setup message is coded as described below.

##### 11.8.1.5.1	Speech/Asynchronous Data, Transparent

BC GSM = 04 07 A2 B8 81 21 XX XX  8X

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | x | x | x | x | User Rate |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | x |  |  |  |  |  | Intermediate Rate |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | x | x | x | x | Modem Type |

Depending of the user rate supported by the MS, the user rate, the modem type and the intermediate rate change :

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 0 | 1 | 0,3 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 0 | 1 | Modem V.21 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 0 | Modem V.22 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 1 | Modem V.22bis |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 1 | - | - | - | - | - | 16 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

##### 11.8.1.5.2	Speech/Asynchronous Data, Non Transparent

BC GSM = 04 0X A2 88 81 21 XX 6X AX (CX)

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | x | x | x | x | Length (7 or 8) depending on presence of octet 7. |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 0 | 0 |  |  |  |  | Structure : SDU Integrity |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 1 | Synchronous / Asynchronous : Asynchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | x |  |  |  |  |  |  | Number of Stop Bits, Depending of the TE Configuration |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | x |  |  |  |  | Number of Data Bits, Depending of the TE Configuration |
|  |  |  |  |  | x | x | x | x | User Rate |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 1 |  |  |  |  |  | Intermediate Rate : 16kbit/s |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | x | x | x | Parity, Depending of the TE Configuration |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Connection Element : Non transparent |
|  |  |  |  | 0 | x | x | x | x | Modem Type |
| Octet 7 (may not be present) | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | 0 |  |  |  |  |  | Layer 2 Id. |
|  |  |  |  | 0 | 1 | x | 0 | 0 | User Inform. layer 2 protocol, Depending of the TE Configuration |

Depending of the type of flow control supported by the TE, the coding of octet 7 is different. The value ISO 6429 (0 1000) means "Inband flow control" and the value COPnoFLCT (0 1100) means "No flow control". The "Outband Flow control is not allowed with V.21 modem).

Depending of the user rate supported by the MS, the user rate and the modem type change :

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 0 | 1 | 0,3 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 0 | 1 | Modem V.21 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 0 | Modem V.22 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 1 | Modem V.22bis |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

##### 11.8.1.5.3	Speech/Synchronous Data

BC GSM = 04 07 A2 B8 81 20 1X X3  8X

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 0 | Info. Transfer Cap. : 3.1 kHz Audio |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : I.440 / I.450 |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : Synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | x | x | x | x | User Rate |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | x |  |  |  |  |  | Intermediate Rate |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | x | x | x | x | Modem Type |

Depending of the user rate supported by the MS, the user rate, the modem type and the intermediate rate change :

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 0 | 1.2 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 0 | Modem V.22 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 1 | Modem V.22bis |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 0 | - | - | - | - | - | 8 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intermediate rate in Octet 6b: | - | 1 | 1 | - | - | - | - | - | 16 kbit/s |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

#### 11.8.1.6	BS 81 - Speech followed by Data

The first BC in the Setup message is coded "Speech" : 04 01 A0

The repeat Indicator in the Setup message is coded "Sequential for successive selection (followed by)" : D3

The second BC in the Setup message is coded as described below.

##### 11.8.1.6.1	Speech followed by Asynchronous Data

See chapters 1.5.1. and 1.5.2.

##### 11.8.1.6.2	Speech followed by Synchronous Data

See chapter 1.5.3.

#### 11.8.1.7	TS 61 - Alternate Speech / Facsimile group 3

The first BC in the Setup message is coded "Speech" : 04 01 A0

The repeat Indicator in the Setup message is coded "Circular for successive selection (alternate)" : D3

The second BC in the Setup message is coded as described below.

BC GSM = 04 07 A3 B8 81 20 1X X3  80

| Octet 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | Information Element : Bearer Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Length |
| Octet 3 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Radio Channel Requirement : Spare |
|  |  |  |  | 0 |  |  |  |  | Coding Standard : GSM |
|  |  |  |  |  | 0 |  |  |  | Transfer Mode : Circuit |
|  |  |  |  |  |  | 0 | 1 | 1 | Info. Transfer Cap. : FAX3 |
| Octet 4 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Spare |
|  |  |  | 1 | 1 |  |  |  |  | Structure : Unstructured |
|  |  |  |  |  | 1 |  |  |  | Duplex Mode : Full duplex |
|  |  |  |  |  |  | 0 |  |  | Configuration : Point to Point |
|  |  |  |  |  |  |  | 0 |  | Negotiation of Intermediate Rate Requested : No meaning |
|  |  |  |  |  |  |  |  | 0 | Establishment : Demand |
| Octet 5 | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Access Id |
|  |  |  |  | 0 | 0 |  |  |  | Rate Adaptation : No |
|  |  |  |  |  |  | 0 | 0 | 1 | Signalling Access Protocol : NA |
| Octet 6 | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 1 |  |  |  |  |  | Layer 1 Id : Default |
|  |  |  |  | 0 | 0 | 0 | 0 |  | User Information Layer 1 Protocol |
|  |  |  |  |  |  |  |  | 0 | Synchronous / Asynchronous : synchronous |
| Octet 6a | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 0 |  |  |  |  |  |  | Number of Stop Bits : NA |
|  |  |  | 0 |  |  |  |  |  | Negotiation : In band Negotiation not possible |
|  |  |  |  | 1 |  |  |  |  | Number of Data Bits : NA |
|  |  |  |  |  | x | x | x | x | User Rate |
| Octet 6b | 0 |  |  |  |  |  |  |  | Extension |
|  |  | 1 | x |  |  |  |  |  | Intermediate Rate |
|  |  |  |  | 0 |  |  |  |  | NIC on TX : Not Required |
|  |  |  |  |  | 0 |  |  |  | NIC on RX : Not Supported |
|  |  |  |  |  |  | 0 | 1 | 1 | Parity : NA |
| Octet 6c | 1 |  |  |  |  |  |  |  | Extension |
|  |  | 0 | 0 |  |  |  |  |  | Connection Element : Transparent |
|  |  |  |  | 0 | 0 | 0 | 0 | 0 | Modem Type : None |

Depending of the user rate supported by the MS, the user rate and the intermediate rate change :

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 0 | 1 | 1 | 2.4 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 0 | 1 | 1 | Modem V.22bis |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 0 | 4.8 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

|  | User Rate in Octet 6a: | - | - | - | - | 0 | 1 | 0 | 1 | 9.6 kbit/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modem type in Octet 6c: | - | - | - | 0 | 0 | 1 | 1 | 0 | Modem V.32 |

#### 11.8.1.8	TS 62 - Automatic Facsimile group 3

The repeat Indicator in the Setup message is not available.

The BC GSM is coded as described in chapter 1.7.