## 26.11	Multiband signalling

### 26.11.1	General considerations

This section applies only to Multiband mobile stations, as defined in GSM 02.06 section 3.2.1.

Conformance requirements of section 26 fully apply to any Multiband GSM/DCS MS in the whole supported band(s) of operation of the mobile station.

A Multiband mobile station has the functionality to perform handover, channel assignment, cell selection and re-selection between all its bands of operation within a PLMN.

A Multiband mobile station shall meet all requirements specified for each individual band. In addition it shall meet the extra functional requirements for multiband mobile stations.

The purpose of this section is to test these extra functional requirements for a multiband mobile station.

### 26.11.2	Multiband signalling / RR

#### 26.11.2.1	Multiband signalling / RR / Immediate assignment procedure

To inform the multiband network of the MSs additional frequency and power capability, the multiband MS has to send a CLASSMARK CHANGE as soon as possible in a connection establishment.

26.11.2.1.1	Conformance requirement

Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on a supported channel  described in the IMMEDIATE ASSIGNMENT message. On the MS side, the procedure is terminated when the establishment of the main signalling link is confirmed. When the ES bit is set to 1 in the Classmark 1 or the Classmark 2 information element and the Early Sending Classmark Control bit is set to 0 in SI3 Rest Octets, then the MS shall send, on the first occasion, the CLASSMARK CHANGE message.

During a contention resolution procedure, if the last timeslot of the block containing a L2 UA frame occurs at time T, then the MS shall be ready to transmit the CLASSMARK CHANGE before T + 40 ms.

Reference(s)

GSM 04.08 section 3.3.1.2.1 , 3.3.1.4

GSM 04.13 section 5.2.11

26.11.2.1.2	Test purpose

To verify that the MS can correctly set up a dedicated control channel and that a multi band MS is able to perform early sending of CLASSMARK CHANGE.

To verify the performance requirement on early sending of the CLASSMARK CHANGE message.

26.11.2.1.3	Method of test

Initial Conditions

System Simulator:

1 cell,

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

BCCH carrier number 20

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

Type of MS (Multiband).

Frequency bands supported

Support rate(s) of TCH: TCH/F and/or TCH/H.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The test is performed twice, first time in the GSM900 band (BCCH carrier number 20) and second time in the DCS1800 band (BCCH carrier number 590).

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message followed by a CLASSMARK CHANGE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Before the procedure is repeated, the SS is reconfigured to transmit BCCH carrier number 590.

Maximum Duration of Test

6 seconds per value of the execution timer and 1 min for reconfiguring the SS.

Expected Sequence

This sequence is performed for execution counter k = 1, 2.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAG REQ TYPE 1 |  |
| 2 | MS -> SS | CHANNELREQ |  |
| 3 | SS -> MS | IMM ASSIGNMENT | Channel Type : SDCCH/8 |
| 4 | MS -> SS | SABM (PAGING RESPONSE) | Shall be sent on the correct channel |
| 5 | SS -> MS | UA (PAGING RESPONSE) |  |
| 6 | MS -> SS | CLASSMARK CHANGE | Shall be ready to be transmitted within 40 ms after the completion of step 4. Shall indicate the MS frequency and power capabilities |
| 7 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents

IMMEDIATE ASSIGNMENT

as default except:

| Information element | Value/remark |
| --- | --- |
| Channel description |  |
| - Channel Type | SDCCH/8 |
| - Timeslot number | Arbitrary but not zero |
| - Training sequence code | Arbitrary |
| - Hopping channel | Single RF |
| - Channel selector | k=1; ARFCN 20 k=2; ARFCN 590 |
| Mobile Allocation | empty |

PAGING RESPONSE

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Ciphering Key Sequence number |  |
| - Key Sequence | Key sequence number previously allocated to MS, or ‘111’ if no key is available |
| Mobile station Classmark 2 |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use k=1; GSM900 power capability k=2; DCS1800 power capability |
| Mobile Identity |  |
| - odd/even | Even |
| - Type of identity | TMSI |
| - Identity digits | TMSI previously allocated to MS |

CLASSMARK CHANGE

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Mobile station Classmark 2 |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use k=1; GSM900 power capability k=2; DCS1800 power capability |
| Additional MS Classmark information |  |
| - Band 1 (P-GSM) supported | According to PICS/PIXIT statement |
| - Band 2 (E-GSM) supported | According to PICS/PIXIT statement |
| - Band 3 (DCS1800) supported | According to PICS/PIXIT statement |
| - Associated radio capability 1 | Corresponding to GSM900 band |
| - Associated radio capability 2 | Corresponding to DCS1800 band |

#### 26.11.2.2	Multiband signalling / RR / Handover

This section only applies to GSM900 / DCS1800 multiband mobile stations.

The purpose of this extra section is to test the handover where the target cell uses frequencies outside the frequency band of the serving cell.

##### 26.11.2.2.1	Multiband signalling / RR / Handover / successful / active call / non-synchronized

26.11.2.2.1.1	Conformance requirements

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping in one band towards a TCH/F without frequency hopping in another band.

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping in one band towards a TCH/F with frequency hopping in another band.

References

GSM 04.08 section 3.4.4 and 9.1.15

GSM 04.13 section 5.2.6.2.

26.11.2.2.1.2	Test purpose

To test that when the MS is ordered to make a non-synchronized handover it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly handles the Timing Advance IE in the PHYSICAL INFORMATION message. To test that the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay.

26.11.2.2.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with same LAI, default parameters except::

Cell A has :

BCCH ARFCN = 20

Cell Allocation = ( 10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114 )

Cell B has :

BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the

antenna connector.

Mobile Station:

The MS is in the active state (U10) of a call on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Support for speech : yes/no

Support of multiband functionality

Foreseen Final State of the MS

The active state (U10) of a call on cell A.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts on the new DCCH of the target cell. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message, before 'x' ms after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for an execution counter M = 1, 2 for an MS which only supports TCH/F.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 0 | MS -> SS |  | The MS and SS are in the active state of a call on the channel described below. |
| 1 | SS -> MS | HANDOVER COMMAND | See Specific message contents |
| 2 | MS -> SS | HANDOVER ACCESS | Repeated on every burst of the uplink main DCCH until reception of PHYSICAL INFORMATION. Handover Reference as included in the HANDOVER COMMAND. |
| 3 | SS -> MS | PHYSICAL INFO | Sent after reception of n HANDOVER ACCESS messages. See specific message contents. |
| 4 | MS -> SS | SABM | Sent without information field |
| 5 | SS -> MS | UA |  |
| 6 | MS -> SS | HANDOVER COMPLETE | The message shall be ready to be transmitted before 'x' ms after the completion of step 3. |
| 7 | MS -> SS |  | The MS and SS are in the active state of a call on the channel described below. |

Specific Message Contents For Mobiles Supporting Speech

For M = 1 :

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Channel description |  |
| - Channel type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily,but not zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | Single RF. |
| - ARFCN | Chosen arbitrarily from Cell Allocation for cell B. |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non hopping mode on cell B.

For M = 2 :

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 20 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily,but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Channel Sequence IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Synchronisation Indication IE is not included. |  |
| Channel Mode IE is not included. |  |
| Frequency Channel Sequence after time |  |
| - Frequency Channel Sequence | Allocates the following 12 frequencies (10, 17, 20, 26, 59, 66, 73, 74, 75, 76,108, 114) |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in hopping mode on cell A.

##### 26.11.2.2.2	Multiband signalling / RR / Handover / layer 1 failure

26.11.2.2.2.1	Conformance requirements

During a handover from a channel in the GSM900 band to a channel in the DCS1800 band, or the contrary, the MS shall correctly return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell.

References

GSM 04.08 section 3.4.4.

26.11.2.2.2.2	Test purpose

To check that the MS correctly returns to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell, if the origin is in the GSM900 band and the target is in the DCS1800 band or the contrary.

26.11.2.2.2.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with same LAI, default parameters except::

Cell A has :

BCCH ARFCN = 20

Cell B has :

BCCH ARFCN = 764

Mobile Station:

The MS is in the active state (U10) of a call on cell A. Used power level is the maximum supported by the MS in the band in use.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Support for multiband functionality

Foreseen Final State of the MS

The active state (U10) of a mobile call on cell A. Used power level is the maximum supported by the MS in the band in use.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 0 | SS | - | The SS records the power level in the layer 1 header of the last SACCH message sent by the MS before step 1. |
| 1 | SS -> MS | HANDOVER COMMAND | Channel description : non-hopping, full rate. SynchronisationIndication : non synchronized. |
| 2 | MS -> SS | HANDOVER ACCESS | Several messages are sent, all with correct Handover References. |
| 3 | MS -> SS | HANDOVER FAILURE | Sent on old channel, RR cause value ="Abnormal release, unspecified", "Abnormal release, channel unacceptable", "Abnormal release, timer expired", "Abnormal release, no activity on the radio path" or "Protocol error unspecified". Layer 1 header has the same power level as the layer 1 header in step 1. Shall be sent within 3 seconds from the transmission of HANDOVER COMMAND. |
| 4 | SS | - | The SS checks that the power level reported in the layer 1 header of the SACCH message that is sent in the first SACCH multiframe following the SABM is the same as in step 0. |

Specific Message Contents

None

#### 26.11.2.3	Multiband signalling / RR / Measurement reporting

This test applies to multiband GSM900/DCS1800 MSs supporting simultaneous multiband operation.

26.11.2.3.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for up to the 6 strongest BCCH carriers among those with known and allowed NCC part of BSIC on which the mobile is asked to report. For a multi band MS the number of neighbour cells, for each frequency band supported, which shall be included is indicated by the parameter MULTIBAND_REPORTING.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

26.11.2.3.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

26.11.2.3.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

| Transmitter | Level | NCC | BSCC | ARFCN | Cell identity |
| --- | --- | --- | --- | --- | --- |
| Serving, S1 | -60 | 1 | 3 | 002 | 0001H |
| Neighbour, N1 | -85 | 1 | 5 | 520 | 0002H |
| Neighbour, N2 | -80 | 1 | 7 | 014 | 0003H |
| Neighbour, N3 | -75 | 1 | 1 | 020 | 0004H |
| Neighbour, N4 | -55 | 1 | 3 | 780 | 0005H |
| Neighbour, N5 | -50 | 1 | 5 | 032 | 0006H |
| Neighbour, N6 | -45 | 1 | 7 | 880 | 0007H |
| Neighbour, N7 | -40 | 1 | 1 | 044 | 0008H |

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1, 2 and 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (multiband GSM900/DCS1800 MS supporting simultaneous multiband operation).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5ter & 6 on the SACCH. All 8 of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest   carriers, on which the mobile is asked to report (indicated by the parameter MULTIBAND_REPORTING), have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 5ter, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

SYSTEM INFORMATION TYPE 2ter:

| Information Element | value/remark |
| --- | --- |
| Additional Multiband information |  |
| - Multiband reporting | k=1: Normal reporting of six strongest cells, irrespective of the band used k=2: Minimum 3 cells reported from each band supported excluding the frequency band of the serving cell. |
| Extension of the BCCH Frequency list |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 780, 880 |
| - EXT IND | Information Element carries the complete BA. |

SYSTEM INFORMATION TYPE 3:

| Information Element | value/remark |
| --- | --- |
| as default except: | SI 3 rest octets |
| - SI 2ter indicator | System Information 2ter is available |
| - Early Classmark Sending Control | Early Sending is explicitely accepted |

SYSTEM INFORMATION TYPE 5:

| Information Element | value/remark |
| --- | --- |
| Protocol Descriminator | RR management |
| Message Type | Sys Info 5. |
| Neighbour Cells Description |  |
| - Format Identifier | Bit map 0 |
| - BCCH Allocation Sequence | 1 |
| - BCCH Allocation ARFCN | ARFCN 14, 20, 32, 44 |
| - EXT IND | Information Element carries the complete BA. |

SYSTEM INFORMATION TYPE 5ter:

| Information Element | value/remark |
| --- | --- |
| Protocol Descriminator | RR management |
| Message Type | Sys Info 5ter. |
| Additional Multiband information |  |
| - Multiband reporting | k=1: Normal reporting of six strongest cells, irrespective of the band used k=2: Minimum 3 cells reported from each band supported excluding the frequency band of the serving cell. |
| Extension of the BCCH Frequency list |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 780, 880 |
| - EXT IND | Information Element carries only a part of the BA. |

MEASUREMENT REPORT:

| Information Element | value/remark |
| --- | --- |
| Protocol Discriminator | RR Management |
| Transaction Identifier | 0000 |
| Message Type | MEASUREMENT REPORT |
| Measurement Results |  |
| BA-used | 1 |
| DTX-used | DTX was not used |
| RXLEV-FULL-SERVING-CELL | See note 1 |
| RXLEV-SUB-SERVING-CELL | See note 1 |
| MEAS-VALID | See note 3 |
| RXQUAL-FULL-SERVING-CELL | See note 1 |
| RXQUAL-SUB-SERVING-CELL | See note 1 |
| NO-NCELL-M | 6 neighbour cell measurement results |
| RXLEV-NCELL-1 | See note 1 |
| BCCH-FREQ-NCELL-1 | See note 2 |
| BSIC-NCELL-1 | Corresponds to that of BCCH-FREQ-NCELL-1 |
| RXLEV-NCELL-2 | See note 1 |
| BCCH-FREQ-NCELL-2 | See note 2 |
| BSIC-NCELL-2 | Corresponds to that of BCCH-FREQ-NCELL-2 |
| RXLEV-NCELL-3 | See note 1 |
| BCCH-FREQ-NCELL-3 | See note 2 |
| BSIC-NCELL-3 | Corresponds to that of BCCH-FREQ-NCELL-3 |
| RXLEV-NCELL-4 | See note 1 |
| BCCH-FREQ-NCELL-4 | See note 2 |
| BSIC-NCELL-4 | Corresponds to that of BCCH-FREQ-NCELL-4 |
| RXLEV-NCELL-5 | See note 1 |
| BCCH-FREQ-NCELL-5 | See note 2 |
| BSIC-NCELL-5 | Corresponds to that of BCCH-FREQ-NCELL-5 |
| RXLEV-NCELL-6 | See note 1 |
| BCCH-FREQ-NCELL-6 | See note 2 |
| BSIC-NCELL-6 | Corresponds to that of BCCH-FREQ-NCELL-6 |

NOTE 1:	These actual values are not checked.

NOTE 2:	k=1; report on ARFCNs 2, 20, 32, 44, 780, 880

k=2; report on ARFCNs 2, 32, 44, 520, 780, 880

NOTE 3:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

### 26.11.3	Multiband signalling / MM

### 26.11.3.1	Multiband signalling / MM / Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, no call can be established.

#### 26.11.3.1.1	Location updating / accepted

This test is applicable for Multiband GSM900 / DCS1800 MSs supporting simultaneous multiband operation.

26.11.3.1.1.1	Conformance requirement

If the network accepts a location updating from the Mobile, the Mobile Station shall, after receiving a Location updating Accept message, store the received LAI, stop timer T3210,reset the attempt counter and set the update status in the SIM to updated.

Reference(s)

GSM 04.08 section 4.4.4.6.

26.11.3.1 1.2	Test purpose

To test the behaviour of the MS if the network accepts the location updating of the MS, irrespective of frequency band used.

26.11.3.1.1.3	Method of test

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN and using frequencies from different frequency bands.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

Frequency bands supported (P-GSM, E-GSM, DCS1800)

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle, updated" on cell A.

Test Procedure

The MS is made to select cell B. A normal location updating is performed in cell B. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A.

The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI.

Maximum duration of test

3 minutes

Expected sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS |  | The RF level of cell A is lowered until the MS selects cell B. |
| 2 | MS -> SS | CHANNEL REQUEST | "Establishment cause" : Location updating. |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | LOCATION UPDATING REQ | "location updating type” = normal, "CKSN” = CKSN1, "location area identification” = a and "mobile identity” = TMSI. |
| 5 | SS -> MS | UA(LOC UPD REQ) |  |
| 6 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 7 | SS -> MS | LOCATION UPDATING ACC | "Mobile identity” IE not included. |
| 8 | MS -> SS | TMSI REALLOCATION CMP |  |
| 9 | SS -> MS | CHANNEL RELEASE | After the sending of this message, the SS waits for the disconnection of the main signalling link. |
|  | 10 | SS | The RF level of cell B is lowered until the MS selects cell A. |
| 11 | MS -> SS | CHANNEL REQUEST | "Establishment cause" : Location updating |
| 12 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 13 | MS -> SS | LOCATION UPDATING REQ | "location updating type” = normal, "CKSN” = CKSN1, "location area identification” = b and "mobile identity” = TMSI. |
| 14 | SS -> MS | UA(LOC UPD REQ) |  |
| 15 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 16 | SS -> MS | LOCATION UPDATING ACC | "Mobile identity" IE not included |
| 17 | SS -> MS | CHANNEL RELEASE | After the sending of this message, the SS waits for the disconnection of the main signalling link. |

Specific message contents:

LOCATION UPDATING REQUEST

| Information element | Value/remark |
| --- | --- |
| as default except: |  |
| Mobile station Classmark 1 |  |
| - ES IND | Controlled Early Classmark Sending option is implemented |
| - RF power capability | corresponding to frequency band used |

CLASSMARK CHANGE

| Information element | Value/remark |
| --- | --- |
| as default except: |  |
| Mobile station Classmark 2 |  |
| -ES IND | Controlled Early Classmark Sending is implemented. |
| -RF power capability | corresponding to the frequency band in use |
| Additional MS Classmark information |  |
| -Band 1 (P-GSM) supported | According to PICS/PIXIT statement |
| -Band 2 (E-GSM) supported | According to PICS/PIXIT statement |
| -Band 3 (DCS) supported | According to PICS/PIXIT statement |
| -Associated radio capability | Corresponding to GSM900 band |
| -Associated radio capability | Corresponding to DCS1800 band |

#### 26.11.3.1.2	Location updating / periodic

This test is applicable for Multiband GSM900 / DCS1800 MSs supporting simultaneous multiband operation.

26.11.3.1.2.1	Conformance requirement

1)	If the Mobile Station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

2)	The T3212 timeout value shall not be changed in the NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH and PLMN SEARCH-NORMAL SERVICE states.

3)	If the selected cell is in the location area where the mobile station is registered and IMSI ATTACH is not required and timer T3212 has not expired, then the state is NORMAL SERVICE.

References

1	GSM 04.08 section 4.4.2.

2	GSM 04.08 section 4.4.2.

3	GSM 04.08 section 4.2.1.1

26.11.3.1.2.2	Test purpose

1)	To check that if the PLU timer expires while the MS is out of coverage, the MS informs the network of its return to coverage, irrespective of frequency band used.

2)	To check that the PLU timer is not disturbed by cells of forbidden PLMNs.

3)	To check that if the PLU timer does not expire while out of coverage and if the mobile returns to the LA where it is updated, the mobile does not inform the network of its return to coverage.

26.11.3.1.2.3	Method of test

Initial conditions

System Simulator:

Two cells, A and B, belonging to the same location area but using different frequency bands.

Cell A is switched on and cell B is switched off.

T3212 is set to 12 minutes on cell A and cell B.

IMSI attach is allowed in both cells.

Mobile Station:

The MS is deactivated.

Related PICS/PIXIT statements

Frequency bands supported (P-GSM, E-GSM, DCS1800)

Foreseen final state of the MS

The MS is "idle updated" on cell A.

Test procedure

The MS is activated and placed in automatic network selection mode. It performs IMSI attach. 1 minute after the end of the IMSI attach procedure, cell A is switched off. 8 minutes after the end of the IMSI attach procedure, cell B is switched on. The MS shall not location update on cell B before 11.75 minutes after the end of the IMSI attach procedure. The MS shall perform a periodic location update on cell B between 11.75 minutes and 12.25 minutes after the end of the IMSI attach procedure.

3 minutes after the end of the periodic location updating procedure, cell B is switched off. 14 minutes after the end of the periodic location updating procedure, cell A is switched on. The MS shall perform a location update on cell A before 17 minutes after the end of the periodic location updating procedure.

Maximum duration of test

35 minutes

Expected sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The MS is activated in automatic network selection mode |
| 2 | MS -> SS | CHANNEL REQUEST |  |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | LOCATION UPDATING REQ | "location updating type": IMSI attach |
| 5 | SS -> MS | UA(LOC UPD REQ) |  |
| 6 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 7 | SS -> MS | LOCATION UPDATING ACC |  |
| 8 | SS -> MS | CHANNEL RELEASE | After the sending of this message, the SS waits for the disconnection of the main signalling link. |
| 9 | SS |  | 1 minute after step 6, cell A is switched off |
| 10 | SS |  | 8 minutes after step 6, cell B is switched on |
| 11 | MS -> SS | CHANNEL REQUEST | This message shall be sent on cell B by the MS between 11 minutes 45s and 12 minutes 15s after step 6. |
| 12 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 13 | MS -> SS | LOCATION UPDATING REQ | "location updating type": periodic updating |
| 14 | SS -> MS | UA(LOC UPD REQ) |  |
| 15 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 16 | SS -> MS | LOCATION UPDATING ACC |  |
| 17 | SS -> MS | CHANNEL RELEASE | After the sending of this message, the SS waits for the disconnection of the main signalling link. |
| 18 | SS |  | 3 minutes after step 13, cell B is switched off |
| 19 | SS |  | 14 minutes after step 13, cell A is switched on. |
| 20 | MS -> SS | CHANNEL REQUEST | This message shall be sent on cell A by the MS before 17 minutes after step 13. |
| 21 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 22 | MS -> SS | LOCATION UPDATING REQ | "Location updating type” = periodic |
| 23 | SS -> MS | UA(LOC UPD REQ) |  |
| 24 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 25 | SS -> MS | LOCATION UPDATING ACC |  |
| 26 | SS -> MS | CHANNEL RELEASE | After the sending of this message, the SS waits for the disconnection of the main signalling link. |

Specific message contents

LOCATION UPDATING REQUEST

| Information element | Value/remark |
| --- | --- |
| as default except: |  |
| Mobile station Classmark 1 |  |
| - ES IND | Controlled Early Classmark Sending option is implemented |
| - RF power capability | corresponding to frequency band used |

CLASSMARK CHANGE

| Information element | Value/remark |
| --- | --- |
| as default except: |  |
| Mobile station Classmark 2 |  |
| -ES IND | Controlled Early Classmark Sending is implemented. |
| -RF power capability | corresponding to the frequency band in use |
| Additional MS Classmark information |  |
| -Band 1 (P-GSM) supported | According to PICS/PIXIT statement |
| -Band 2 (E-GSM) supported | According to PICS/PIXIT statement |
| -Band 3 (DCS) supported | According to PICS/PIXIT statement |
| -Associated radio capability | Corresponding to GSM900 band |
| -Associated radio capability | Corresponding to DCS1800 band |

### 26.11.4	Multiband signalling / CC

Reserved for future use.

### 26.11.5	Multiband signalling / Structured procedures

These tests applies only to multiband mobile stations.

#### 26.11.5.1	Multiband signalling / Structured procedures / MS originated call / early assignment

26.11.5.1.1	Conformance requirements

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	After the initial message the multiband MS shall send a CLASSMARK CHANGE message in the uplink block followed direct after the Layer 2 UA message sent from the network. The CLASSMARK CHANGE message shall contain information elements Mobile Station Classmark 2 and Mobile Station Classmark 3.

4)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

5)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

6)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

7)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

References

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 3.3.1.4

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.

Conformance requirement 5:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 6:	TS GSM 04.08 section 5.4.4.

Conformance requirement 7:	TS GSM 04.08, section 3.5.1.

26.11.5.1.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that a multiband MS is able to send an early CLASSMARK CHANGE on the DCCH uplink.

4)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

5)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an HANDOVER COMMAND message allocating an appropriate TCH in another band, after having completed the traffic channel early assignment procedure by replying with the HANDOVER COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

26.11.5.1.3	Method of test

Related PICS/PIXIT Statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Frequency bands supported

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI.

Cell A is a GSM 900 cell with default parameters.

Cell B is a DCS1800 cell with default parameters.

Mobile Station:

The MS is in MM-state idle, updated on cell A with valid TMSI.

Foreseen Final State of the MS

The MS is in MM-state idle, updated on cell B with valid TMSI.

Test procedure

The following test is performed for one teleservice supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call on any frequency band supported by the MS. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" is entered |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "originating call and the network does not set the NECI bit to 1". |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. Indicating early sending of CLASSMARK CHANGE |
| 5 | SS -> MS | UA (CM SERVICE REQUEST) |  |
| 6 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 7 | SS -> MS | AUTHENTICATION REQ |  |
| 8 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 9 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 10 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 11 | SS |  | SS starts ciphering. |
| 12 | MS -> SS | SETUP |  |
| 13 | SS -> MS | CALL PROCEEDING |  |
| 14 | SS -> MS | ALERTING |  |
| 15 | MS |  | Depending on the PICS, an alerting indication is given |
| 16 | SS -> MS | HANDOVER COMMAND | See specific message contents below. |
| 17 | MS -> SS | HANDOVER ACCESS | May or may not be sent.The sending of the |
|  | MS -> SS | HANDOVER ACCESS | HANDOVER ACCESS is optional as indicated in |
|  | MS -> SS | HANDOVER ACCESS | HANDOVER COMMAND. |
|  | MS -> SS | HANDOVER ACCESS | Handover Reference is included in the HANDOVER COMMAND. |
| 18 | MS -> SS | SABM | Sent without information field |
| 19 | SS -> MS | UA |  |
| 20 | MS -> SS | HANDOVER COMPLETE |  |
| 21 | SS -> MS | CONNECT |  |
| 22 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 23 | MS |  | The appropriate bearer channel is through connected in both directions. |
| 24 | SS -> MS | DISCONNECT |  |
| 25 | MS -> SS | RELEASE |  |
| 26 | SS -> MS | RELEASE COMPLETE |  |
| 27 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

CM SERVICE REQUEST

as default except:

| Information element | Value/remark |
| --- | --- |
| Mobile station Classmark 2 |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use |

CLASSMARK CHANGE

as default except:

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Mobile station Classmark 2 |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use |
| Additional MS Classmark information |  |
| - Band 1 (P-GSM) supported | According to PICS/PIXIT statement |
| - Band 2 (E-GSM) supported | According to PICS/PIXIT statement |
| - Band 3 (DCS1800) supported | According to PICS/PIXIT statement |
| - Associated radio capability 1 | Corresponding to GSM900 band |
| - Associated radio capability 2 | Corresponding to DCS1800 band |

HANDOVER COMMAND

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Cell Description |  |
| - NCC | 1 |
| - BCC | 5 |
| - BCCH Carrier Number | 590 |
| Channel description |  |
| - Channel type | TCH/F + ACCH’s |
| - Timeslot number | Arbitrary value |
| - Training sequence code | Chosen arbitrarily |
| - Hopping | Single RF channel |
| - ARFCN | 650 |
| Handover Reference | Chosen arbitrarily from the range (0,1...255) |
| Power Command & Access type |  |
| - Power level | Arbitrarily chosen, but different to the one already in use and within the range supported by the MS. |
| - Access type control | Sending of HANDOVER ACCESS is optional |
| Synchronization Indication | pre-synchronized; ROT=0; NCI=0. |
| Timing Advance | same as in IMMEDIATE ASSIGNMENT |

#### 26.11.5.2	Structured procedures / MS terminated call / late assignment

26.11.5.2.1	Conformance requirement

1)	After the initial message the multiband MS shall send a CLASSMARK CHANGE message in the uplink block followed direct after Layer 2 UA message sent from the network.

2)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

4)	The MS indicates acceptance of a call by sending a CONNECT message.

5)	HANDOVER COMMAND is answered by HANDOVER COMPLETE.

6)	For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirement 1:		TS GSM 04.08, section 3.3.1.4

Conformance requirements 2, 3, 4:	TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 3.4.3.1.

Conformance requirement 6:		TS GSM 04.08, section 5.2.2.9.

26.11.5.2.2	Test purpose

1)	To verify that a multiband MS is able to send an early CLASSMARK CHANGE message on the DCCH uplink.

2)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

3)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

4)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

5)	To verify that subsequently after receipt of an HANDOVER COMMAND ALLOCATING A tch IN another band, the MS sends an HANDOVER COMPLETE message.

6)	To verify that subsequently the MS

-	if the call is a speech call: after sending the HANDOVER COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) HANDOVER COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

26.11.5.2.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Frequency bands supported

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI.

Cell A is a GSM 900 cell with default parameters.

Cell B is a DCS1800 cell with default parameters.

Mobile Station:

The MS is in MM-state idle, updated on cell A with valid TMSI.

Foreseen Final State of the MS

The MS is in MM-state idle, updated on cell B with valid TMSI.

Test procedure

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged on any frequency band supported by the MS and a MT call is established with late assignment (after CONNECT). The release of the call is initiated by the MS.

Maximum Duration of Test

40 seconds

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST TYPE 1 | Sent on the correct paging subchannel |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause indicates "answer to paging". |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | UA (PAG RESP) |  |
| 6 | MS -> SS | CLASSMARK CHANGE | Indicating the frequency and power capability of the MS |
| 7 | SS -> MS | AUTHENTICATION REQ |  |
| 8 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 9 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 10 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 11 | SS |  | SS starts ciphering. |
| 12 | SS -> MS | SETUP | Message contains the signal IE. |
| 13 | MS -> SS | CALL CONFIRMED |  |
| A14 | MS -> SS | CONNECT |  |
| B14 | MS -> SS | ALERTING |  |
| B15 | MS |  | An alerting indication as defined in an PICS/PIXIT statement is given by the MS. |
| B16 | MS |  | The MS is made to accept the call in the way described in a PICS/PIXIT statement. |
| B18 | MS -> SS | CONNECT |  |
| 19 | SS -> MS | HANDOVER COMMAND | See specific message contents below. |
| 20 | MS -> SS | HANDOVER ACCESS | Handover Reference is included in the |
| 21 | MS -> SS | HANDOVER ACCESS | HANDOVER COMMAND. |
| 22 | MS -> SS | HANDOVER ACCESS |  |
| 23 | MS -> SS | HANDOVER ACCESS |  |
| 24 | MS -> SS | SABM | Sent without information field |
| 25 | SS -> MS | UA |  |
| 26 | MS -> SS | HANDOVER COMPLETE |  |
| 27 | MS |  | If the call is a speech call, the TCH shall be through connected in both directions. |
| 28 | SS -> MS | CONNECT ACKNOWLEDGE |  |
| 29 | MS |  | If the call is a data call, the MS shall through connect the TCH in both directions |
| 30 | MS |  | The MS is made to release the call. |
| 31 | MS -> SS | DISCONNECT |  |
| 32 | SS -> MS | RELEASE |  |
| 33 | MS -> SS | RELEASE COMPLETE |  |
| 34 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

PAGING RESPONSE

| Information element | Value/remark |
| --- | --- |
| Mobile station Classmark 2 |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use |

CLASSMARK CHANGE

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Mobile station Classmark |  |
| - ES IND | Shall indicate early autonomous sending of CLASSMARK CHANGE |
| - RF power capability | corresponding to the frequency band in use |
| Additional MS Classmark information |  |
| - Band 1 (P-GSM) supported | According to PICS/PIXIT statement |
| - Band 2 (E-GSM) supported | According to PICS/PIXIT statement |
| - Band 3 (DCS1800) supported | According to PICS/PIXIT statement |
| - Associated radio capability 1 | Corresponding to GSM900 band |
| - Associated radio capability 2 | Corresponding to DCS1800 band |

HANDOVER COMMAND

| Information element | Value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Cell Description |  |
| - NCC | 1 |
| - BCC | 5 |
| - BCCH Carrier Number | 590 |
| Channel description |  |
| - Channel type | TCH/F + ACCH’s |
| - Timeslot number | Arbitrary value |
| - Training sequence code | Chosen arbitrarily |
| - Hopping | Single RF channel |
| - ARFCN | 650 |
| Handover Reference | Chosen arbitrarily from the range (0,1...255) |
| Power Command & Access type |  |
| - Power level | Arbitrarily chosen, but different to the one already in use and within the range supported by the MS. |
| - Access type control | Sending of HANDOVER ACCESS is mandatory. |
| Synchronization Indication | pre-synchronized; ROT=0; NCI=0. |
| Timing Advance | same as in IMMEDIATE ASSIGNMENT |

### 26.11.6	Multiband signalling / Default messages contents

Default SYSTEM INFORMATION

The following parameters shall be coded into the system information messages. Parameters shall be coded according to GSM 04.08.

SYSTEM INFORMATION TYPE 2bis, SYSTEM INFORMATION TYPE 5bis, SYSTEM INFORMATION TYPE 7 and SYSTEM INFORMATION TYPE 8 messages are not used.

SYSTEM INFORMATION TYPE 1

| Information Element | Value/remark |
| --- | --- |
| Cell channel description |  |
| For Cell A |  |
| - Format identifier | Bit map 0 |
| - Cell Allocation ARFCN | ARFCN 20, 30, 50 and 70 |
| For Cell B |  |
| - Format identifier | Range 512 |
| - Cell Allocation ARFCN | ARFCN 590, 650, 750 and 850 |
| RACH control parameters | see below |
| SI1 rest octets | see below |

SYSTEM INFORMATION TYPE 2

| Information Element | Value/remark |
| --- | --- |
| BCCH Frequency list |  |
| For Cell A |  |
| - Format identifier | Bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |
| For Cell B |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 590, 600, 700, 780, 810 and 870 |
| - EXT-IND | This IE carries the complete BA |
| NCC permitted | see below |
| RACH control parameters | see below |

SYSTEM INFORMATION TYPE 2ter

| Information Element | Value/remark |
| --- | --- |
| Additional Multiband information | 0 |
| Extension of the BCCH Frequency list |  |
| For Cell A |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 590, 600, 700, 780, 810 and 870 |
| - EXT-IND | This IE does not carry the complete BA |
| For Cell B |  |
| - Format identifier | Range 1024 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |
| SI 2ter rest octets | see below |

SYSTEM INFORMATION TYPE 3

| Information Element | Value/remark |
| --- | --- |
| Cell identity | see below |
| Location Area Identification | see below |
| Control Channel Description | see below |
| Cell options | see below |
| Cell selection parameters | see below |
| RACH control parameters | see below |
| SI3 rest octets |  |
| SI 2ter Indicator | System Information 2ter is available |
| Early Sending Classmark Control | Early Sending is explicitely accepted |

SYSTEM INFORMATION TYPE 4

| Information Element | Value/remark |
| --- | --- |
| Location Area Identification | see below |
| Cell selection parameters | see below |
| RACH control parameters | see below |
| CBCH Channel Description | see below |
| CBCH Mobile Allocation | see below |
| SI4 rest octets | see below |

SYSTEM INFORMATION TYPE 5

| Information Element | Value/remark |
| --- | --- |
| BCCH Frequency list |  |
| For Cell A |  |
| - Format identifier | Bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |
| For Cell B |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 590, 600, 700, 780, 810 and 870 |
| - EXT-IND | This IE carries the complete BA |

SYSTEM INFORMATION TYPE 5ter

| Information Element | Value/remark |
| --- | --- |
| Additional Multiband information | 0 |
| Extension of the BCCH Frequency list |  |
| For Cell A |  |
| - Format identifier | Range 512 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 520, 590, 600, 700, 780, 810 and 870 |
| - EXT-IND | This IE does not carry the complete BA |
| For Cell B |  |
| - Format identifier | Range 1024 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | ARFCN 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |

SYSTEM INFORMATION TYPE 6

| Information Element | Value/remark |
| --- | --- |
| Cell identity | see below |
| Location Area Identification | see below |
| Cell options | see below |
| NCC permitted | see below |

Common contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages.

| CBCH Channel Description | Not present |
| --- | --- |
| CBCH Mobile Allocation | Not present |
| Cell identity |  |
| CI VALUE | 0001H for cell A, 0002H for cell B |
| Cell options |  |
| Power Control Indicator | power control indicator is not set |
| DTX Indicator | MS shall not use DTX |
| RADIO LINK TIMEOUT | 8 SACCH blocks |
| Cell selection parameters |  |
| CELL RESELECT HYSTERESIS | 12 dB |
| MS-TXPWR-MAX-CCH | Minimum level |
| RXLEV-ACCESS-MIN | Minimum level |
| ACS | There are no additional cell parameters included in SI7 and SI8 |
| NECI | New establishment cause not supported |
| Control Channel Description |  |
| ATT | No Attach/Detach |
| BS-AG-BLKS-RES | 0 blocks reserved |
| CCCH-CONF | Combined CCCH/SDCCH |
| BS-PA-MFRMS | 5 multiframes |
| T3212 | Infinite |
| L2 pseudo length |  |
| SI 1 | 21 |
| SI 2 | 22 |
| SI 2ter | 18 |
| SI 3 | 18 |
| SI 4 | 12 |
| Location Area Identification |  |
| MCC | 001 decimal |
| MNC | 01 decimal |
| LAC | 0001H |
| Message Type |  |
| SI 1 | 00011001 |
| SI 2 | 00011010 |
| SI 2ter | 00000011 |
| SI 3 | 00011011 |
| SI 4 | 00011100 |
| SI 5 | 00011101 |
| SI 5ter | 00000110 |
| SI 6 | 00011110 |
| NCC permitted | 00000010 |
| RACH control parameters |  |
| MAX RETRANS | Max 1 retrans |
| TX-INTEGER | 5 slots used |
| CELL BAR ACCESS | Not barred |
| CALL RE-ESTABLISHMENT | Not Allowed |
| EMERGENCY CALL | Allowed |
| ACCESS CONTROL CLASS | None Barred |
| (0...9, 11...15) |  |
| SI 1 rest octets | Not used (all bits are set to spare) |
| SI 2 rest octets | Not used (all bits are set to spare) |
| SI 2ter rest octets | Not used (all bits are set to spare) |
| SI 4 rest octets | Not used (all bits are set to spare) |

Default settings for cell A

| Downlink input level | 63 dBmicroVolt emf |
| --- | --- |
| Uplink output power | Minimum supported by the MS |
| Propagation profile | Static |
| BCCH/CCCH | ARFCN 20 |

Default settings for cell B

| Downlink input level | 53 dBmicroVolt emf |
| --- | --- |
| Uplink output power | Minimum supported by the MS |
| Propagation profile | Static |
| BCCH/CCCH | ARFCN 590 |

Default message contents for other messages

For section 26.11.2	same as in 26.6.14 for GSM900 messages and 26.6.15 for DCS1800 messages

For section 26.11.3	same as in 26.9.7