OSDCU monitor user documentation

The creator of the OSDCU follows the philosophy that every microprocessor

system should begin life with a processor level debug ROM monitor, and that

this monitor should not only be for the original manufacturing or engineering

company's internal use, but should be part of the final product.

On the OSDCU this monitor is based on MON302, Harhan Engineering Co.'s ROM

monitor for MC68x302 microprocessor systems.  The present document is a user's

guide to MON302 as implemented on the OSDCU.

NOTE: as MON302 is fundamentally a processor level debug monitor, an

understanding of the M68K architecture is required and will be assumed

throughout the rest of this document.

The monitor may be entered in one of two ways: on power-up (boot) and upon an

M68K exception which traps to the monitor.  The monitor's own code is stack-

based (i.e., uses the stack for its own data storage) and the monitor always

executes in an exception context.  The exception context is the stack frame

created by whatever exception has caused the monitor to be entered.  This stack

frame consists of saved contents of all CPU registers at the time of the

exception.  PC and SR are saved by the CPU itself; all other registers are saved

by the exception entry code.

The MON302 exception stack frame has the following format:

Offset	Register

00	D0

04	D1

08	D2

0C	D3

10	D4

14	D5

18	D6

1C	D7

20	A0

24	A1

28	A2

2C	A3

30	A4

34	A5

38	A6

3C	SR (extended to a longword)

40	PC

Note that register A7 or SP (the stack pointer) is not part of the structure.

As the structure itself resides on the stack, a valid stack pointer is required

in order for it to be created and accessed in the first place and thus saving it

inside is pointless.  The value of SP at the time of the exception can be

directly deduced from the location of the exception frame and the return-from-

exception mechanism does not allow it to be changed easily.

The stack pointer relevant to the placement of the exception context structure

in memory and the monitor's own subsequent stack usage is the supervisor stack

pointer (SSP).  As MC68x302 systems typically never enter M68K user mode, SSP

will normally be the only stack pointer of any relevance; however, monitor

commands are provided to read and write USP.

MON302 *always* runs in an exception context; if the monitor has been entered on

power-up boot rather than as a result of an exception, a placeholder exception

context is created at the default stack bottom location.  Furthermore, one can

always revert to this "clean" placeholder exception context and reset the

monitor's own stack pointer with the RESTACK command.

Monitor commands

The author is a great fan of the VAX architecture and Harhan Engineering Co.'s

ROM monitors for other processor architectures have been heavily influenced by

the VAX and MicroVAX consoles.  Although the OSDCU monitor prompt is MON>

rather than >>>, the command syntax, the repertoire of available commands and

the overall "feel" of the monitor are very reminiscent of a MicroVAX console.

Some commands take no arguments and consist of just the command keyword, others

have parameters and/or qualifiers.  Arguments are separated by white space,

qualifiers are introduced by the slash character '/'.  Qualifiers may appear

anywhere in the command (after the command keyword or after any of the

arguments) and have the same effect regardless of their placement.  Command and

qualifier keywords may be abbreviated.

All numbers are hexadecimal unless noted otherwise.  All keywords are case-

insensitive.

Memory peeks and pokes

The monitor's most basic and fundamental commands are EXAMINE (E) and DEPOSIT

(D).  Just like on DEC systems they mean raw memory peek and raw memory poke,

respectively.  The two commands use exactly the same syntax as their

counterparts on VAX/MicroVAX consoles:

* A /B, /W or /L qualifier specifies the data size of the read/write operation:

byte (8 bits), word (16 bits) or longword (32 bits), respectively. If not

specified, the data size is remembered from the last EXAMINE or DEPOSIT

command.

* The first argument after the command keyword is the hex memory address to

read or write. For EXAMINE it's the only argument. For DEPOSIT data values

follow.

Just like on VAXen, the hex memory address to read or write may be replaced

with one of the following special symbols:

*	Address from last EXAMINE or DEPOSIT command

+	Last address incremented by the data size of the PREVIOUS access

-	Last address decremented by the data size (or step) of THIS access

For EXAMINE the address argument may be omitted, which is equivalent to +.

Examples

Suppose you want to read memory location 4000 as a longword.  You do:

MON>E/L 4000

The monitor responds:

P 00004000 55AA2233

Now try reading the same location as a 16-bit word:

MON>E/W 4000

P 00004000 55AA

Now say you want to change it to 1234:

MON>D/W 4000 1234

You can also shorten it to:

MON>D * 1234

Verify it:

MON>E/W 4000

P 00004000 1234

The following word in memory should not have been affected:

MON>E/W 4002

P 00004002 2233

MON>E/L 4000

P 00004000 12342233

The alignment required by the EXAMINE and DEPOSIT commands is the same as that

required by the M68K architecture itself: words must be naturally aligned, but

longwords only need to be word-aligned.  Thus the following is legal:

MON>E/L 4002

but the following aren't:

MON>E/W 4001

MON>E/L 4001

The monitor will catch the latter and give you an error message instead of

letting you experience an M68K address error exception, but it's one of the

very few things it catches.  In general MON302 is no more forgiving of user

errors than 'cd /; rm -rf *' on a UNIX box as root.

It is also possible to read or write more than a single memory location in a

single EXAMINE or DEPOSIT command.  Both commands take an optional qualifier of

the form /N:nnn where nnn is a hexadecimal number specifying how many

additional memory locations should be read or written in addition to the one

specified explicitly.  Note that the value of the /N:n qualifier is the number

of *additional* memory locations to access beyond the starting one, thus E/N:1

will examine and display two memory locations, not one.  The address separation

between successive reads or writes is normally equal to the data size, but may

be overridden with the /STEP:n qualifier.

The action of the /N:n qualifier is straightforward for EXAMINE, but what about

DEPOSIT?  There are two different ways to write multiple memory locations with a

single DEPOSIT command:

* One can give more than 2 arguments to a D command. In this case the additional

arguments are interpreted as additional data values to be written at

increasing addresses after the first one. The /N:n qualifier may not be used

in this case.

* One can use the /N:n qualifier in a D command with only one data value

specified. In this case the same data value is written to all memory

locations.

The two methods cannot be combined, i.e., it is an error to give a /N:n

qualifier (n != 0) to a DEPOSIT command with more than one data value specified.

In addition the D/B command (byte-sized DEPOSIT) accepts quoted strings as

arguments (in double quotes) in addition to hex values.  In this case the ASCII

characters of the quoted string are written to successive memory locations as

if their hexadecimal values were given in the command instead.  No terminating

null byte is inserted automatically, but one may be specified explicitly after

the quoted string.  (Quoted ASCII string and hex value arguments may be freely

intermixed in a single D/B command.)

Other "address spaces"

Just like on VAXen, MON302's EXAMINE and DEPOSIT commands can read and write

more than just memory.  The following additional "address spaces" can also be

accessed with these commands under MON302:

* Saved register values in the exception context

* USP

To examine a saved register value in the exception context, do this:

MON>E/X nn

where nn is the standard M68K register name (D0 thru D7, A0 thru A6, PC or SR).

To modify it, do this:

MON>D/X nn newvalue

Although SR is a word-sized register in the actual CPU, the stack frame that's

being manipulated by these E/X and D/X commands allocates a longword slot for

it just like for all other registers, thus all E/X and D/X operations are always

longword-sized.

You can also dump the entire exception context with this command:

MON>SHOW EXCEPTION

The following commands will read and write USP (the actual CPU register):

MON>E USP

U 00000000 xxxxxxxx

MON>D USP newvalue

Control transfer commands

CONTINUE

Minimal abbreviation: C

This command directs the monitor to restore all saved registers from the

exception context and execute the RTE (return from exception) instruction.

Control is transferred to the PC value in the exception context, all other

registers are restored from the exception context as well, and the exception

context frame is popped off the stack.

START addr

Minimal abbreviation: S

This command is similar to C, but the saved PC value in the exception context

is replaced with the argument before executing the return from exception, hence

control is transferred to the specified address.  Values to be restored into

all other registers are still determined by the exception context though, and

the exception frame is still popped off the stack just like by C.

CALL addr

Minimal abbreviation: CA

This is a different way of transferring control to a specified address.  Instead

of being returned to, the specified address is called with a subroutine call

instruction.  Control is transferred to the same address, but the effect on the

stack is the opposite: CALL does a call whereas CONTINUE and START do a return.

Other useful commands

HEXDUMP start count

This command performs a hex dump of a memory region starting at <start> and

extending for <count> bytes.

Optional qualifier: /ASCII

augments the hex dump with ASCII on the right.  Bytes not corresponding to

printable ASCII characters are printed as '.' on the ASCII side of the dump.

REBOOT

Simulates a board reboot as close as possible to power-up.  (See software

design specification section 6 for how it's implemented.)

REINIT

This command executes an M68K RESET instruction, then repeats the initialization

sequence performed on boot.  However, it keeps you in the monitor and does not

disturb the exception context, so it isn't the same as REBOOT.

RESTACK

This command restores the stack pointer to its initial value after boot (end of

the first 64 KB of memory) and restores the associated placeholder exception

context.

If you have used the monitor to run some program and it trapped back to the

monitor on some exception with the system state uncertain, use the following

sequence to restore to a clean state like after boot:

MON>RESTACK

MON>REINIT

You can also use REBOOT of course, but that will proceed with operational code

autoboot if OPER.CMD is found in FFS.

SET CONSOLE newbaud

This command changes the serial console baud rate.  The argument is the new

baud rate.  This command is an exception in that the argument is decimal rather

than hex.  The supported baud rates are 1200, 2400, 4800, 9600, 19200, 38400,

57600 and 115200.

A delay of 1 s is provided before the monitor prompt is redisplayed at the new

baud rate for the user to change the baud rate on his side.

Please note that this baud rate change is not persistent and that the console

baud rate will revert to 9600 baud on the next power cycle, REBOOT or REINIT.

This ability to change the console baud rate is primarily intended for those

who retrieve large volumes of data (such as SDSL bit stream captures) from the

OSDCU via HEXDUMP and prefer a faster baud rate for that.

SET IPL

Sets the current M68K IPL to the specified argument.

SHOW IPL

Displays the current M68K IPL.

Loading code into the unit

As the OSDCU has no Ethernet port, the only way to load code into the unit from

the outside world is over the serial console port.  The usual user-friendly way

is to use the Unix/Linux host-based uploader utility described in the user's

manual, but what it actually does under the hood is first load the binary data

into scratch RAM using the low-level method described here and then store it in

FFS as described in the next section.

The low-level method has the following features:

* Code is loaded into RAM, not flash;

* The user must specify the M68K absolute address at which the code is to be

loaded;

* The facility is built into the base monitor and thus does not require valid

FFS or any assistant programs.

Binary loading over the serial console port is implemented like on the VAX

console via the X command:

MON>X address count

The first argument is the address in RAM to which the code should be loaded and

the second argument is the number of bytes to load.  Only binary load is

supported, not binary unload.

The protocol following the X command is exactly as on the VAX console.  The

carriage return terminating the command needs to be immediately followed by a

binary byte giving the command checksum (computed exactly as on the VAX

console).  Upon receiving a valid X command MON302 waits for this checksum byte.

The next byte received on the console port is assumed to be this checksum and

is checked for correctness.  If the checksum doesn't match, an error message is

printed and the monitor prompt returns.

If the command checksum matches, MON302 prepares to receive binary data into

the specified buffer and prints a >>> prompt when it's ready.  Upon receiving

this second prompt the host needs to start sending the binary data.  After

sending the number of bytes equal to the second argument of the original X

command the host needs to send one additional checksum byte exactly as with the

VAX console; this second checksum byte concludes the binary load protocol.

MON302 implements the X command by temporarily reprogramming the receive side of

SCC3 to receive directly into the destination memory buffer instead of the

1-character buffers used by the getchar() implementation.  The final checksum

byte is also stored into memory in this implementation, hence even though count

bytes of real data are received, count+1 bytes of memory are overwritten.  The

reprogramming of the SCC3 receive buffers requires that reception be disabled

for a brief moment, which is why the host must wait for the >>> second prompt

before sending the data.

Aborting binary load: if you get into the binary load mode by mistake or if a

load gets interrupted in the middle, how do you get out?  If you enter a valid

X command from the keyboard, the monitor will enter the wait for the command

checksum byte.  This wait has no timeout.  Type any character; with 255/256

probability it won't be the correct checksum, you'll get an error message and

you'll be back at the MON> prompt.  If you hit that 1/256 chance and get a

valid checksum, the binary receive state will be entered.  All bytes are valid

from this point on.  However, there is a timeout implemented.  Because of the

way the SCC is programmed, it will wait forever for the very first byte of data,

but once that first byte is received, the delay between successive bytes (idle

on the line) cannot exceed 9600 character times.  That's 10 s at 9600 baud or

less if you've switched to a faster baud rate with SET CONSOLE.  Timeout at

this point will cause an error message and return to the MON> prompt.  Thus if

you are stuck in the binary load mode, type one character, then go idle and

wait for the timeout.

FFS operations

The OSDCU uses a flash file system (FFS) to store all code and data related to

the unit's end functionality which is not part of the base monitor.  The monitor

provides commands to list FFS contents, load code from FFS into RAM, create and

delete files.

FFS consists of 7 segments; each segment corresponds to an erase block (a block

of flash memory which may be erased only as a single unit) in the underlying

flash chips.  Each segment is further divided into allocation units (AUs) of

512 bytes each; as files are stored in a segment, these allocation units are

used up.  When a file is deleted, the associated directory entries are

invalidated so that a new file may be created with the same name, but the actual

space in the segment is not reclaimed immediately, instead those AUs transition

from the active state to the dirty state.  When a segment becomes filled with

dirty AUs (deleted files), it needs to be erased with the FFS ERASE command

before one can store new files in it.

FFS commands

FFS INIT segno

The argument is the segment number between 1 and 7.  This command turns a blank

flash erase block (containing all FFs) into a valid RT11FFS segment by writing

the necessary signature into the first few bytes.  Should generally be

unnecessary as this operation has already been done at the factory for all 7

segments.

FFS STORE datasrc nblocks filename

This command stores the contents of a region of memory in FFS, creating a new

file.  The first argument is the starting address in memory (must be even), the

second argument is the number of 512-byte blocks (AUs) to store (in hexadecimal

like most numeric arguments) and the third argument is the new file name.

File names in RT11FFS follow the original RT-11 file system rules: a base

filename of up to 6 characters, a period and a 3-character extension or

filetype; the set of valid characters for both the filename and the filetype

consists of letters, digits and the '$' character (the RADIX-50 character set).

Optional qualifier: /CONTIGUOUS

Normally files are allowed to be split between segments in FFS, so that an FFS

STORE operation will always succeed if there is enough free space across all

segments, even if no single segment has enough free space.  The /CONTIGUOUS

qualifier changes this behaviour and forces the new file to be contiguous; if

there isn't enough contiguous free space in any segment, the operation will

fail.

Optional qualifier: /SEGMENT:n

This qualifier forces the new file creation into a specific FFS segment.  If

used together with /CONTIGUOUS, only the specified segment will be considered,

otherwise the search for possibly fragmented free space will begin with the

specified segment instead of segment 1.

FFS DELETE filename

Deletes the specified file.

FFS ERASE segno

This command erases an FFS segment, turning all dirty AUs into free space.  All

active files in the segment must be deleted first.  The argument is the segment

number between 1 and 7.

FFS DIR

This command displays an FFS directory listing.  All active (not deleted) files

are listed along with AU usage statistics for each segment.

FFS HEXDUMP filename

Dumps the contents of a file in hex.

Optional qualifier: /ASCII

Works just like with the regular HEXDUMP command.

FFS TEXTDUMP filename

Dumps the contents of a text file; useful for command scripts and configuration

files.

LOAD filename

Loads the contents of a file into RAM.  If no extension (filetype) is specified,

it defaults to .BIN.  The load address is taken from the program header.

Optional qualifier: /START:addr

Overrides the load address in the program header and allows loading files that

have no such header at all.

RUN filename

Loads the contents of a file into RAM like LOAD and calls the entry point

listed in the program header like CALL.

Command script facility

The monitor provides a mechanism to execute very simple command scripts from

FFS.  This mechanism works by locating a file in FFS which consists of ASCII

text lines (terminated with either CR LF or LF only) and executing each line as

a monitor command, as if it had been typed from the console.  Such command

scripts may be nested.  The current position in the command script stack after

each command is remembered in a set of private global variables maintained by

the monitor; this state is reset to read commands from the console upon all

monitor entry points (power-up boot and exceptions).  Script execution is

terminated if any command encounters an error or if the monitor detects a ^C

character on the console.

To run a script, type this:

MON>@scrnam

The monitor will look for scrnam.CMD in FFS and execute it if found.

Board control commands

Although the monitor purposely does not contain full SDSL functionality and

therefore cannot perform any useful end function by itself, it does provide a

minimal set of commands to control and monitor clocks, data path multiplexing

and modem control lines.

MUX signal source

This command controls the 4 data path multiplexors found on the OSDCU board;

see hardware design specification section 2.2.  The first argument identifies

one of the 4 MUXes; the second argument selects the signal input on that MUX.

The options for the first argument are:

Keyword		MUX being set

114		CCITT circuit 114 (DCE-source Tx clock) on the EIA-530 port

115		CCITT circuit 115 (DCE-source Rx clock) on the EIA-530 port

RXD		RxD line on the EIA-530 port

SDSL		SDSL bitpump transmit data stream

The options for the SDSL bitpump transmit data stream source are:

Keyword		Data source

DCE		EIA-530 DCE port TxD line

INV		Inverted version of the above

SCC1		MC68302 SCC1

FPGA		FPGA

The options for the RxD line MUX are:

Keyword		Data source

SDSL		Bit stream from the SDSL bitpump

INV		Inverted version of the above

SCC2		MC68302 SCC2

FPGA		FPGA

The options for the 114 and 115 MUXes are:

Keyword		Clock source

BCLK		BCLK

BLCK180		BCLK rotated 180

SWCLOCK		Software clock from BRG1

FPGA		FPGA

SHOW MUX

This command displays the current configuration of the MUXes.

SHOW MCTL

This command displays the current state of all 5 EIA-530 modem control and

status lines, both inputs and outputs: DTR, DSR, CD, RTS and CTS.

SET DSR state

SET CD state

SET CTS state

These commands allow direct control over the modem status lines for debugging

purposes.  The first argument is the name of the modem status line to be

affected (DSR, CD or CTS), the second argument is ON or OFF.

SET BCLK freq

This command sets the BCLK frequency (which is normally output on CCITT circuits

114 and 115) to the specified value in kHz, which must also be a valid SDSL data

rate in kbps.  This command is an exception in that the argument is decimal

rather than hex.

SET BITPUMP UP

This command brings the SDSL bitpump into the fully powered-up and initialized

state, leaving BCLK at its current frequency.  It is admittedly of little

usefulness as the base monitor doesn't do anything with the bitpump; one needs

to load and run some application program in order to do anything interesting.

SET BITPUMP DOWN

This command puts the SDSL bitpump into the low power mode (functionally

disabled) with BCLK ticking at the slowest allowed frequency of 144 kHz.

SET LSLED state

This command sets the Line Status LED to the indicated state: OFF, GREEN, RED

or ORANGE.

FPGA control commands

The base monitor is responsible for loading configuration bits into the FPGA and

mapping the FPGA into the M68K address space.  The commands are:

FPGA LOAD filename

or

FPGA LOAD/FFS filename

This command locates the specified file in FFS and uses it as the configuration

bit image for the FPGA.  If no extension (filetype) is specified, it defaults

to .RBF, as used by Altera.  The FPGA is put through the configuration sequence

prescribed by Altera, using the file content from FFS as the configuration bits.

FPGA LOAD/MEMORY address

Same as regular FPGA LOAD, but instead of being sought in FFS, the RBF image is

assumed to be in memory at the specified address.  The image is assumed to be

51200 bytes long.

FPGA MAP

This command programs the BR3 and OR3 registers in the MC68302 so that the FPGA

address window may be accessed as intended.

FPGA KILL

This command sets the nCONFIG signal low, causing the FPGA to revert to its

unconfigured power-up state.