Guide to using flash memory on the Hack-o-Rocket

As the CR201 hardware platform has 4 times more flash memory than RAM (64 KB of

RAM and 256 KB of flash specifically), flash memory is very important on the

Hack-o-Rocket.  Programs which need to use the scarce available RAM for data

buffers need to be designed to keep their own code in flash and to run directly

from there without copying to RAM first.  The basic processor monitor and the

IP router application can only run from flash; the line debug utilities are

built in both run-from-RAM and run-from-flash versions.

Flash usage and layout

No flash filesystem is implemented on the Hack-o-Rocket, instead the flash

memory is used "raw".  Each program that is supposed to run from flash must be

programmed into flash at a specific address; all programs are linked with

absolute addresses and no PIC is used.  As a result of these considerations, a

specific layout has been arrived at as to what goes where in the flash bank.

For historical reasons this layout is slightly different between the IDSL and

SDSL Hack-o-Rockets.

The Hack-o-Rocket flash usage layout takes advantage of the fact that the flash

bank consists of 8 equal erase blocks (sometimes called flash sectors) of 32 KB

each.  An erase block is a part of the flash memory array that may be erased

and reprogrammed without affecting other blocks.  Two 29F010 type flash chips

must be used in order for the expected erase block arrangement to be present.

The following flash layout is used on the IDSL Hack-o-Rocket:

Sectors		Start addr	End addr	Usage

0		200000		207FFF		Monitor

1		208000		20FFFF		IP router or other operational

code

2-5		210000		22FFFF		Usused, available for growth

6		230000		237FFF		IP router configuration table

7		238000		23FFFF		IDEBUG utility

The following flash layout is used on the SDSL Hack-o-Rocket:

Sectors		Start addr	End addr	Usage

0		200000		207FFF		Monitor

1-2		208000		217FFF		IP router or other operational

code

3-4		218000		227FFF		Unused, available for growth

5-6		228000		237FFF		SDEBUG utility

7		238000		23FFFF		IP router configuration table

Flash programming

The design of most flash memories including 29F010s used on the rocket is such

that one cannot run any code out of the flash bank while an erase or program

operation is in progress, even if it's in a different erase block.  Therefore,

most code on the Hack-o-Rocket (including MON302) which is based on a run-from-

flash architecture can only read flash, but not write to it.

Writing to flash (more precisely, flash erase and program operations) is done

via a special flash writer program, flashwr.bin.  It is a RAM-based utility

which needs to be loaded into RAM via the monitor and then run.  When this

program is run, it establishes a fully RAM-based environment independent of the

monitor in flash (sets up RAM-based handlers for all interrupts and exceptions

replacing the flash-based ones) and provides its own command monitor that is

similar to the main Hack-o-Rocket monitor, but is RAM-based instead.  It is

then safe to perform flash erase and program operations, and commands are

provided to do that.  The program is also small and simple enough that it can

run from RAM and yet leave plenty of free RAM to play in.

Load and run the flash writer utility as follows:

HACK>SET IP ip.addr.for.rocket

HACK>TFTP tftp.server.ip.address flashwr.bin 600

HACK>S 600

Basically load the contents of flashwr.bin into RAM at hex address 600 and

transfer control to that address.  If everything works correctly, the first LED

on the unit should light orange and you should see a RAM> prompt on the console.

The RAM-based monitor in the flash writer utility has a RAM> prompt instead of

HACK> to tell you that you are in a RAM-based monitor and can write to flash.

The RAM-based monitor implements a subset of full MON302 functionality.

Specifically, the following commands work exactly as in the main (flash-based)

monitor and are documented further in mon302.doc:

* E and D (memory only)

* HEXDUMP

* REBOOT

* SET IP

* SET IPL

* SHOW IP

* SHOW IPL

* TFTP

You can use E and D commands to peek and poke at the flash chips, but for

practical use higher level erase and program commands are provided.

Erase operations

Because programming operations can only change a bit in flash from 1 to 0 but

not vice-versa, a flash region must contain all 1s prior to programming.  Blank

flash chips contain all 1s from the factory, but if a flash region has been

programmed before, it must be erased before it can be programmed with something

different.  Flash memory can only be erased in large blocks called sectors or

erase blocks; the flash bank on the Hack-o-Rocket is divided into 8 such blocks.

The erase command has the following syntax:

RAM>ERASE sector

where the argument is the erase block (sector) number between 0 and 7,

inclusive.

Multiple sectors can be erased at once by listing them in the ERASE command as

additional arguments separated by white space (e.g., ERASE 1 2) or one can

specify a range like ERASE 1-3.  The two notations can also be combined:

RAM>ERASE 1-2 7

One can erase the entire flash bank with ERASE 0-7, but this approach is not

recommended, instead it's better to treat the monitor in erase block 0 as

separate from the rest.

Corresponding sectors in the even and odd flash chips are always erased

together.

Program operations

The flash writer utility provides two versions of the basic program operation:

one implemented as a memory-to-memory copy and the other as an extension to the

TFTP load command.

The command for flash programming as a memory-to-memory copy operation is as

follows:

RAM>PROGRAM src dest len

where src is the source address (must be in RAM), dest is the destination

address (must be in flash) and len is the number of bytes to copy.  Both

addresses and the length must be even; programming is implemented as a

word-sized operation and the even and odd chips are programmed in tandem.

However, the more common way to program flash on the Hack-o-Rocket is to load

directly into flash from a TFTP server without storing the entire image in RAM

first.  Indeed, some run-from-flash program images are larger than all available

RAM and thus cannot be program in any other way.

The flash writer utility implements the TFTP command very much like the main

(flash-based) monitor (see mon302.doc), but with one important difference: the

destination address for the load may be in either RAM or flash.  TFTP transfers

data in 512-byte chunks, and when the destination is in flash, the TFTP command

programs each 512-byte chunk into flash as it's received from the network, thus

the entire image doesn't need to fit into RAM.  The flash region to be

programmed must still be erased first though.

The normal procedure for programming something into flash is thus as follows:

RAM>ERASE right_sector_range

RAM>TFTP tftp.server.ip.addr filename.bin target_flash_addr

When you are done, reboot the rocket with the REBOOT command.

Reprogramming the monitor

Erase block 0 containing the Hack-o-Rocket monitor can be erased and

reprogrammed using the flash writer utility just like everything else.

However, you need to be extra-careful with it!  If you do the following:

RAM>ERASE 0

RAM>REBOOT

You will get a dead rocket and the only way to recover from that state is to

open the box, pop the flash chips out of the sockets and reprogram them with a

device programmer.  If you don't have access to a device programmer, you are

out of luck.

You should only erase sector 0 if you really need to reprogram the monitor and

you already have the new monitor image on your TFTP server.  Erase the sector

and then reprogram it with the new monitor image right away.  If power goes out

in the middle of the operation, too bad.

Flash device support

The erase and program routines in the flash writer utility are written for

29F010 type flash chips.  Here at Harhan we use ST M29F010B70K6 parts (used to

be 497-1700-1-ND from Digi-Key until the environazis discontinued them in favor

of RoHS crap), but any "generic" 29F010 should work (hopefully).  There is no

JEDEC ID check and the classic AMD-style programming algorithm is used (most

basic form without unlock bypass or buffers).