Hack-o-Rocket IP router user documentation

The IP router is the only part of the Hack-o-Rocket software suite that is

potentially useful to an end user rather than merely for SDSL hacking for its

own sake.  It's a very rudimentary IP gateway implementation (nowhere close to

complying with RFC 1009) with two network interfaces: one Ethernet and one

SDSL/IDSL WAN.  Logically the Ethernet interface is classic IP/ARP; the WAN

interface is routed RFC 1490 or routed RFC 1483 depending on the version.

The HoR router comes in 3 different versions conditionally compiled from the

same source: IDSL, SDSL Flavor B and Nokia SDSL.  The first runs on CR201i

hardware, the other two run on CR201s hardware.  The three versions are quite

different in their quality status.  The IDSL version has been proven to work

correctly on a Covad IDSL circuit cross-connected to a 3rd party ISP who

provided an IP address block in a routed encapsulation and is currently the

only version known to work for certain.

The SDSL Flavor B version has not been tested on a real SDSL line as no such

line could be found - it appears that no one serves SDSL Flavor B any more.

Tests against a Netopia R7100 configured to emulate a CM DSLAM with FR

encapsulation were unsuccessful - the Hack-o-Rocket was receiving packets from

the R7100 successfully, but the R7100 claimed to be getting nothing back.  It

is suspected that the problem has something to do with CMCP, and the latter

can't be disabled on the R7100.  We have not tried testing against a real CM

DSLAM - although we have one, we have no place to set it up currently.

The Nokia SDSL version has been tested on a real Covad line and found NOT to

work.  We have high expectations of getting it working in the next release

though, see nokia-notes for the details.

Although IDSL is currently the only version of the HoR router which works in a

useful manner, all 3 versions are documented here as they are virtually

identical in terms of configuration and operation.

Router configuration

In order to use the HoR IP router, one first needs to configure it.  Given very

tight constraints on software complexity imposed by the small RAM and flash

sizes on CR201 hardware, there is no "live" configuration in the Hack-o-Rocket

router.  In other words, one cannot run the router and then configure it by

interacting with the running thing.  Instead one must prepare the configuration

table on an external UNIX host, compile it into binary form with a special

UNIX-based configuration compiler utility, put it on a TFTP server, then use

the Hack-o-Rocket monitor to load the flash writer utility, then use the latter

to program the router binary and its configuration table into the right place

in flash.  Yes, it's a pain, but it's the best we can do without putting a real

OS on the rocket which is too small to run one.

We'll start by describing the text-based configuration file you need to write.

The configuration file defines one setting per line.  Each line starts with a

keyword that identifies the setting, followed by the requisite number of

arguments.  Blank lines and comment lines starting with '#' are ignored.

All keywords are case-insensitive.  At the minimum a valid configuration table

must contain the LAN setting and a WAN type setting (IDSL or SDSL).  Each

setting is described below.

IDSL setting

This setting declares the WAN interface to be IDSL rather than SDSL and

specifies the IDSL channel configuration.  It should be given before all other

settings pertaining to the WAN interface.

The argument specifies the IDSL channel configuration.  IDSL is DSL carried in

the ISDN electrical signal format, hence the signal format on the line has bit

slots corresponding to B1, B2 and D channels.  The IDSL channel configuration

is the description of how the IDSL bit stream is mapped to these bit slots.

It is specified as a list of channel keywords concatenated with '+' signs.

For example, the following specification:

IDSL B1+B2+D

indicates that the IDSL bit stream fills the B1 slot first, then the B2 slot,

then the D slot.  This is the correct configuration for Covad and also the most

natural configuration, but it is also possible to omit some bit slots (giving a

payload bit rate of less than 144 kbps) or to fill the slots in a different

order.

SDSL setting

This setting declares the WAN interface to be SDSL rather than IDSL and

specifies the SDSL flavor, terminal type and data rate.  It should be given

before all other settings pertaining to the WAN interface.

3 arguments must follow the SDSL keyword.  The first is the SDSL flavor keyword

which must be either "B" or "N1" ("Nokia" is also a synonym for "N1"), the

second is the terminal type keyword ("HTU-C" or "HTU-R"), and the third is the

data rate in kbps in decimal.  HTU-R is the normal CPE mode, HTU-C means acting

as central office equipment.

Only fixed data rate operation is supported, no pre-activation signaling

protocols.  The implication for HTU-R operation is that you must set the data

rate correctly or else; the implication for HTU-C operation is that one cannot

emulate a CM DSLAM and link up with standard CM CPE on the other end for

example.  (However, the Nokia SDSL version emulates a DSLAM well enough to link

up with "official" Flavor N1 CPE devices.)

DLCI setting

This setting should be self-explanatory.  It is only meaningful for Frame Relay

WAN types, i.e., IDSL and SDSL Flavor B.  The default is 16, which should be

correct for most DSL installations with FR encapsulation.

VPI setting

This setting should be self-explanatory.  It is only meaningful for the Nokia

SDSL flavor.  The default is 0; it would be super-unusual to have a different

VPI on ATM DSL.

VCI setting

This setting should be self-explanatory.  It is only meaningful for the Nokia

SDSL flavor.  The default is 38, which is correct for Covad.

QUAT setting

This setting is only valid for SDSL Flavor B.  It selects the quat orientation:

sign bit first or magnitude bit first.  The sign bit first orientation is more

natural and is the default, but Copper Mountain uses magnitude bit first.

The syntax is:

quat signfirst

or

quat magfirst

LAN setting

This setting is always required.  It configures the router's Ethernet interface,

specifically it sets the HoR router's own IP address on the Ethernet side and

the net/subnet mask in force on this Ethernet segment.  The syntax is:

LAN ip netmask

Both arguments are given in the standard dotted-decimal notation.

Limitation note: the Ethernet ARP implementation in the current HoR router code

requires that the net/subnet mask be no wider than 255.255.255.0.

WAN setting

This setting is optional.  It specifies the HoR router's own IP address on the

WAN interface.  If your WAN setup is the "unnumbered interface" configuration,

omit this setting.

ADDIP setting

This optional setting configures an additional IP address which the HoR router

should treat as belonging to itself (in addition to those given by LAN and WAN

settings).  The router will "swallow" packets addressed to this IP number

instead of forwarding them, and it'll respond to pings of this IP address.

This setting may be given more than once.

ROUTE setting

This optional setting adds entries to the routing table.  The syntax is as

follows:

ROUTE destination route

The first argument specifies the route destination.  It can take one of the

following forms:

* A single IP address (dotted-decimal notation) - specifies a host route;

* A netblock specified in the CIDR notation;

* The "default" keyword for the default route.

The second argument is one of the following keywords:

* "lan": send packets to a gateway on the Ethernet side. A third argument is

required giving the IP address of the gateway; this IP address must be within

the netblock configured by the LAN setting.

* "wan": send packets to the WAN interface.

* "drop": packets go to a black hole. Use this route if you have a netblock

which is yours, but which you have nowhere to take at the moment.

A default route is always enabled; if one isn't specified explicitly, it goes

to the WAN interface.

Compiling the configuration table

After the configuration table is prepared, it needs to be compiled into binary

form; given the available memory resources it would be too extravagant to parse

ASCII configuration files directly on the target.  The compilation is done with

the hortconfig utility, which in turn needs to be compiled and run on a UNIX

host.

Suppose your configuration table source is in a text file named mycfg.  If you

run:

hortconfig mycfg

you should get a file named mycfg.bin.  You can also specify a different output

file name as the second argument.

Copy the compiled binary configuration file to your TFTP server.

Programming flash

It is now time to program the router code binary (irouter.bin for IDSL,

sbrouter.bin for SDSL Flavor B or nsrouter.bin for Nokia SDSL) and its

configuration table into flash on the Hack-o-Rocket.  See flash.doc for

instructions on how to load and run the flash writer utility and for the

description of what goes where in the flash.

Assuming that all the necessary binaries are on your TFTP server and that you

have already loaded and run the flash writer utility successfully, program the

IDSL router as follows:

RAM>ERASE 1

RAM>TFTP tftp.server.ip.addr irouter.bin 208000

RAM>ERASE 6

RAM>TFTP tftp.server.ip.addr mycfg.bin 230000

Program the SDSL Flavor B router as follows:

RAM>ERASE 1-2

RAM>TFTP tftp.server.ip.addr sbrouter.bin 208000

RAM>ERASE 7

RAM>TFTP tftp.server.ip.addr mycfg.bin 238000

Program the Nokia SDSL router as follows:

RAM>ERASE 1-2

RAM>TFTP tftp.server.ip.addr nsrouter.bin 208000

RAM>ERASE 7

RAM>TFTP tftp.server.ip.addr mycfg.bin 238000

Don't try running the IDSL router on an SDSL rocket or vice-versa!  Each router

binary supports only its own flavor and must only be run on the hardware it's

designed for.  It will bomb out and refuse to run if the configuration table

isn't found at the right location or if the configured WAN type in that table

doesn't match the router code version, but there is no guard against running on

the wrong hardware version and very bad things can happen in that case.

After the router is programmed into flash, rebooting the rocket (REBOOT command

or unplugging the power supply and plugging it back in) will cause the router

to autoboot.  If you want to do something other than boot the router (e.g.,

load and run the flash writer utility to change the configuration or run the

line debug utility), you can stop the autoboot with ^C within a 5 s window.

If you don't want the autoboot function, you can overwrite the first 4 bytes at

208000 with zeros (see flash.doc); these bytes contain the operational code

autoboot signature.  In that case run the router manually with S 208004 from

the monitor.

Router operation

Once the router boots and locates a valid configuration table, it runs and does

its job.  The HoR router can be viewed as consisting of the following mostly

independent processes:

* SDSL activation state machine (SDSL versions only): brings the SDSL link up,

monitors it and strives to keep it up.  This process produces a good deal of

output on the console.

* Router core: receives, processes and forwards IP and ARP packets on the LAN

and WAN interfaces.  This process is normally completely silent, the only way

to observe it is to see the router respond to ping.

* Craft terminal process: mostly a frill, allows one to enter a few commands

over the serial console port to do things like display system status and

statistics counters.

Be forewarned that the router is VERY primitive.  It only implements a minimal

subset of IP (notably missing are IP options and fragmentation/reassembly) and

a minimal subset of ICMP which consists only of generating a few error datagrams

on some conditions and responding to ping.  It does NOT implement TCP or even

UDP, so don't even think about telnetting to it.  (As primitive and minimal as

it is though, to the author's knowledge it's the only fully open source router

in the world with direct connectivity to an SDSL WAN interface at the present

time.)

Craft terminal interface

Once it's programmed into flash along with the configuration table, the HoR

router is designed to run headless, i.e., the serial console doesn't need to be

connected.  There is however a little craft terminal interface implemented,

mostly to let the user see through the serial console port that the router is

alive and allow minimal diagnostics.  The only commands provided are frills

like showing the system status and statistics counters, nothing substantial

like changing any configuration settings on the fly.

Because the router is designed to run headless, the craft terminal interface is

guarded against line noise and stale state.  Specifically, all input characters

are ignored until one sends a carriage return, which causes a ROUTER> prompt to

appear.  One must then enter the command at this prompt and send another

carriage return; there may be no more than 10 s delay between characters or the

command prompt times out.  Thus a command is acted upon only if it's both

preceded and followed by a carriage return and there is no more than 10 s delay

between characters.

The HELP command displays the list of available commands with short one-line

descriptions; all of these commands are simple enough that no further

documentation is deemed necessary.

To learn more about the HoR router from the viewpoint of internal

implementation, see router-impl.doc and the source code.