Hack-o-Rocket monitor user documentation

The author of the Hack-o-Rocket software suite follows the philosophy that

every microprocessor system should begin life with a processor level debug ROM

monitor, and that this monitor should not only be for the original manufacturing

or engineering company's internal use, but should be part of the final product.

On the Hack-o-Rocket this monitor is based on MON302, Harhan Engineering Co.'s

ROM monitor for MC68x302 microprocessor systems.  The present document is a

user's guide to MON302 as implemented on the Hack-o-Rocket.

NOTE: as MON302 is fundamentally a processor level debug monitor, an

understanding of the M68K architecture is required and will be assumed

throughout the rest of this document.

The monitor may be entered in one of two ways: on power-up (boot) and upon an

M68K exception which traps to the monitor.  The monitor's own code is stack-

based (i.e., uses the stack for its own data storage) and the monitor always

executes in an exception context.  The exception context is the stack frame

created by whatever exception has caused the monitor to be entered.  This stack

frame consists of saved contents of all CPU registers at the time of the

exception.  PC and SR are saved by the CPU itself; all other registers are saved

by the exception entry code.

The MON302 exception stack frame has the following format:

Offset	Register

00	D0

04	D1

08	D2

0C	D3

10	D4

14	D5

18	D6

1C	D7

20	A0

24	A1

28	A2

2C	A3

30	A4

34	A5

38	A6

3C	SR (extended to a longword)

40	PC

Note that register A7 or SP (the stack pointer) is not part of the structure.

As the structure itself resides on the stack, a valid stack pointer is required

in order for it to be created and accessed in the first place and thus saving it

inside is pointless.  The value of SP at the time of the exception can be

directly deduced from the location of the exception frame and the return-from-

exception mechanism does not allow it to be changed easily.

The stack pointer relevant to the placement of the exception context structure

in memory and the monitor's own subsequent stack usage is the supervisor stack

pointer (SSP).  As MC68x302 systems typically never enter M68K user mode, SSP

will normally be the only stack pointer of any relevance; however, monitor

commands are provided to read and write USP.

MON302 *always* runs in an exception context; if the monitor has been entered on

power-up boot rather than as a result of an exception, a placeholder exception

context is created at the bottom of the main memory.  Furthermore, one can

always revert to this "clean" placeholder exception context and reset the

monitor's own stack pointer with the RESTACK command.

Monitor commands

The author is a great fan of the VAX architecture and Harhan Engineering Co.'s

ROM monitors for other processor architectures have been heavily influenced by

the VAX and MicroVAX consoles.  Although the Hack-o-Rocket monitor prompt is

HACK> rather than >>>, the command syntax, the repertoire of available commands

and the overall "feel" of the monitor are very reminiscent of a MicroVAX

console.

Some commands take no arguments and consist of just the command keyword, others

have parameters and/or qualifiers.  Arguments are separated by white space,

qualifiers are introduced by the slash character '/'.  Qualifiers may appear

anywhere in the command (after the command keyword or after any of the

arguments) and have the same effect regardless of their placement.  Command and

qualifier keywords may be abbreviated.

All numbers are hexadecimal unless noted otherwise.  All keywords are case-

insensitive.

Memory peeks and pokes

The monitor's most basic and fundamental commands are EXAMINE (E) and DEPOSIT

(D).  Just like on DEC systems they mean raw memory peek and raw memory poke,

respectively.  The two commands use exactly the same syntax as their

counterparts on VAX/MicroVAX consoles:

* A /B, /W or /L qualifier specifies the data size of the read/write operation:

byte (8 bits), word (16 bits) or longword (32 bits), respectively. If not

specified, the data size is remembered from the last EXAMINE or DEPOSIT

command.

* The first argument after the command keyword is the hex memory address to

read or write. For EXAMINE it's the only argument. For DEPOSIT data values

follow.

Just like on VAXen, the hex memory address to read or write may be replaced

with one of the following special symbols:

*	Address from last EXAMINE or DEPOSIT command

+	Last address incremented by the data size of the PREVIOUS access

-	Last address decremented by the data size (or step) of THIS access

For EXAMINE the address argument may be omitted, which is equivalent to +.

Examples

Suppose you want to read memory location 4000 as a longword.  You do:

HACK>E/L 4000

The monitor responds:

P 00004000 55AA2233

Now try reading the same location as a 16-bit word:

HACK>E/W 4000

P 00004000 55AA

Now say you want to change it to 1234:

HACK>D/W 4000 1234

You can also shorten it to:

HACK>D * 1234

Verify it:

HACK>E/W 4000

P 00004000 1234

The following word in memory should not have been affected:

HACK>E/W 4002

P 00004002 2233

HACK>E/L 4000

P 00004000 12342233

The alignment required by the EXAMINE and DEPOSIT commands is the same as that

required by the M68K architecture itself: words must be naturally aligned, but

longwords only need to be word-aligned.  Thus the following is legal:

HACK>E/L 4002

but the following aren't:

HACK>E/W 4001

HACK>E/L 4001

The monitor will catch the latter and give you an error message instead of

letting you experience an M68K address error exception, but it's one of the

very few things it catches.  In general MON302 is no more forgiving of user

errors than 'cd /; rm -rf *' on a UNIX box as root.

It is also possible to read or write more than a single memory location in a

single EXAMINE or DEPOSIT command.  Both commands take an optional qualifier of

the form /N:nnn where nnn is a hexadecimal number specifying how many

additional memory locations should be read or written in addition to the one

specified explicitly.  Note that the value of the /N:n qualifier is the number

of *additional* memory locations to access beyond the starting one, thus E/N:1

will examine and display two memory locations, not one.  The address separation

between successive reads or writes is normally equal to the data size, but may

be overridden with the /STEP:n qualifier.

The action of the /N:n qualifier is straightforward for EXAMINE, but what about

DEPOSIT?  There are two different ways to write multiple memory locations with a

single DEPOSIT command:

* One can give more than 2 arguments to a D command. In this case the additional

arguments are interpreted as additional data values to be written at

increasing addresses after the first one. The /N:n qualifier may not be used

in this case.

* One can use the /N:n qualifier in a D command with only one data value

specified. In this case the same data value is written to all memory

locations.

The two methods cannot be combined, i.e., it is an error to give a /N:n

qualifier (n != 0) to a DEPOSIT command with more than one data value specified.

In addition the D/B command (byte-sized DEPOSIT) accepts quoted strings as

arguments (in double quotes) in addition to hex values.  In this case the ASCII

characters of the quoted string are written to successive memory locations as

if their hexadecimal values were given in the command instead.  No terminating

null byte is inserted automatically, but one may be specified explicitly after

the quoted string.  (Quoted ASCII string and hex value arguments may be freely

intermixed in a single D/B command.)

Other "address spaces"

Just like on VAXen, MON302's EXAMINE and DEPOSIT commands can read and write

more than just memory.  The following additional "address spaces" can also be

accessed with these commands on the Hack-o-Rocket MON302:

* Saved register values in the exception context

* CS8900 Ethernet chip registers (access via the I/O space window)

* USP

To examine a saved register value in the exception context, do this:

HACK>E/X nn

where nn is the standard M68K register name (D0 thru D7, A0 thru A6, PC or SR).

To modify it, do this:

HACK>D/X nn newvalue

Although SR is a word-sized register in the actual CPU, the stack frame that's

being manipulated by these E/X and D/X commands allocates a longword slot for

it just like for all other registers, thus all E/X and D/X operations are always

longword-sized.

You can also dump the entire exception context with this command:

HACK>SHOW EXCEPTION

The following commands will read and write USP (the actual CPU register):

HACK>E USP

U 00000000 xxxxxxxx

HACK>D USP newvalue

The /E address space qualifier selects the space of CS8900 Ethernet chip

registers.  These accesses are always word-sized and go through CS8900's I/O

space window.  This access method is thus available even when the memory space

window is disabled and was used during the development of the monitor itself.

These commands are specifically intended to read and write CS8900 control and

status registers and perform byte swaps to make the latter user-friendly; they

are not designed to read or write other areas of the chip.

Control transfer commands

CONTINUE

Minimal abbreviation: C

This command directs the monitor to restore all saved registers from the

exception context and execute the RTE (return from exception) instruction.

Control is transferred to the PC value in the exception context, all other

registers are restored from the exception context as well, and the exception

context frame is popped off the stack.

START addr

Minimal abbreviation: S

This command is similar to C, but the saved PC value in the exception context

is replaced with the argument before executing the return from exception, hence

control is transferred to the specified address.  Values to be restored into

all other registers are still determined by the exception context though, and

the exception frame is still popped off the stack just like by C.

CALL addr

Minimal abbreviation: CA

This is a different way of transferring control to a specified address.  Instead

of being returned to, the specified address is called with a subroutine call

instruction.  Control is transferred to the same address, but the effect on the

stack is the opposite: CALL does a call whereas CONTINUE and START do a return.

Other useful commands

HEXDUMP start count

This command performs a hex dump of a memory region starting at <start> and

extending for <count> bytes.

Optional qualifier: /ASCII

augments the hex dump with ASCII on the right.  Bytes not corresponding to

printable ASCII characters are printed as '.' on the ASCII side of the dump.

REBOOT

Simulates a complete power cycle as closely as possible by pulling the PB7/WDOG

pin low.  On the CopperRocket hardware this pin is connected to the master reset

net; this net going low causes the M68K HALT and RESET nets to be pulsed low

which in turn causes the MC68LC302 to perform full power-up reset.  (The latter

action releases the PB7/WDOG pin back high, preventing a dead hang.)

This action is very close but not absolutely identical to a power-on reset.  The

output of the power supply monitor is separated from the master reset net in

question by a 5 kOhm resistor, so the net before that resistor is an even higher

level master reset that cannot be asserted by software.  That higher level reset

does not appear to be used on the SDSL rocket, but on the IDSL rocket it resets

the MC145572.  The latter is thus the only component not reset by this REBOOT

operation.

REINIT

This command executes an M68K RESET instruction, then repeats the initialization

sequence performed on boot.  However, it keeps you in the monitor and does not

disturb the exception context, so it isn't the same as REBOOT.

RESTACK

This command restores the stack pointer to its initial value after boot (end of

memory) and restores the associated placeholder exception context.

If you have used the monitor to run some program and it trapped back to the

monitor on some exception with the system state uncertain, use the following

sequence to restore to a clean state like after boot:

HACK>RESTACK

HACK>REINIT

You can also use REBOOT of course, but that will proceed with operational code

autoboot if any operational code is found in the flash.

SET CONSOLE newbaud

This command changes the serial console baud rate.  The argument is the new

baud rate.  This command is an exception in that the argument is decimal rather

than hex.  The supported baud rates are 1200, 2400, 4800, 9600, 19200 and 38400.

A delay of 1 s is provided before the monitor prompt is redisplayed at the new

baud rate for the user to change the baud rate on his side.

Please note that this baud rate change is not persistent and that the console

baud rate will revert to 9600 baud on the next power cycle, REBOOT or REINIT.

This ability to change the console baud rate is primarily intended for those

who retrieve large volumes of data (such as DSL bit stream captures) from the

Hack-o-Rocket via HEXDUMP and prefer a faster baud rate for that.

SET IP

See the section "Loading code into the rocket" below.

SET IPL

Sets the current M68K IPL to the specified argument.

SHOW EEPROM

Dumps the contents of the 93C46 EEPROM.

SHOW IP

See the section "Loading code into the rocket" below.

SHOW IPL

Displays the current M68K IPL.

Loading code into the rocket

The HoR monitor supports two methods for loading code from the outside world:

over the serial console port and over Ethernet via TFTP.

Binary loading over the serial console port is implemented like on the VAX

console via the X command:

HACK>X address count

The first argument is the address in RAM to which the code should be loaded and

the second argument is the number of bytes to load.  Only binary load is

supported, not binary unload.

The protocol following the X command is exactly as on the VAX console.  The

carriage return terminating the command needs to be immediately followed by a

binary byte giving the command checksum (computed exactly as on the VAX

console).  Upon receiving a valid X command MON302 waits for this checksum byte.

The next byte received on the console port is assumed to be this checksum and

is checked for correctness.  If the checksum doesn't match, an error message is

printed and the monitor prompt returns.

If the command checksum matches, MON302 prepares to receive binary data into

the specified buffer and prints a >>> prompt when it's ready.  Upon receiving

this second prompt the host needs to start sending the binary data.  After

sending the number of bytes equal to the second argument of the original X

command the host needs to send one additional checksum byte exactly as with the

VAX console; this second checksum byte concludes the binary load protocol.

MON302 implements the X command by temporarily reprogramming the receive side of

SCC2 to receive directly into the destination memory buffer instead of the

1-character buffers used by the getchar() implementation.  The final checksum

byte is also stored into memory in this implementation, hence even though count

bytes of real data are received, count+1 bytes of memory are overwritten.  The

reprogramming of the SCC2 receive buffers requires that reception be disabled

for a brief moment, which is why the host must wait for the >>> second prompt

before sending the data.

Aborting binary load: if you get into the binary load mode by mistake or if a

load gets interrupted in the middle, how do you get out?  If you enter a valid

X command from the keyboard, the monitor will enter the wait for the command

checksum byte.  This wait has no timeout.  Type any character; with 255/256

probability it won't be the correct checksum, you'll get an error message and

you'll be back at the HACK> prompt.  If you hit that 1/256 chance and get a

valid checksum, the binary receive state will be entered.  All bytes are valid

from this point on.  However, there is a timeout implemented.  Because of the

way the SCC is programmed, it will wait forever for the very first byte of data,

but once that first byte is received, the delay between successive bytes (idle

on the line) cannot exceed 9600 character times.  That's 10 s at 9600 baud or

less if you've switched to a faster baud rate with SET CONSOLE.  Timeout at

this point will cause an error message and return to the HACK> prompt.  Thus if

you are stuck in the binary load mode, type one character, then go idle and

wait for the timeout.

Loading code into the rocket over Ethernet

MON302 on the Hack-o-Rocket implements TFTP for loading code over Ethernet.

In order to use TFTP one first needs to assign a local IP address to the rocket

for the duration of the TFTP operation.  The TFTP implementation in the monitor

is completed independent of the IP router application and thus has its own IP

address setting.  SET IP and SHOW IP commands set and display a variable which

is used only for this purpose.  The variable is located in RAM (LC302 "user RAM"

area specifically) and does not persist across reboots.

After you have set the local IP address with the SET IP command, the actual

TFTP command is as follows:

HACK>TFTP serverip filename load_address

The first argument is the TFTP server's IP address in the standard dotted-

decimal notation (must be located on the directly attached Ethernet, no

gateways), the second argument is the file name to request, and the third

argument is the memory address where the data should be loaded.

The filename argument may be given unquoted if it consists only of letters,

digits, '.' and '_' characters.  If any other characters are present (such as

'-' hyphens or UNIX '/' path separators), the argument needs to be enclosed in

double quotes '"'.

Both load methods (the X command and TFTP) can only load code or data into RAM,

not into flash.  To load into flash one must use the flash writer utility

documented in flash.doc.

Other monitor commands

The monitor implements a few additional commands which were used during

development and are no longer of much use.  They are documented below for the

sake of completeness.

ARP ipaddr

This command performs an ARP query to translate an IP address into an Ethernet

MAC address.  Used internally by the TFTP code.

ETHTEST

A test of the minimal Ethernet handling code used by the TFTP implementation.

Brings the Ethernet interface up and dumps all received frames (individually

addressed to us or broadcasts) on the console in hex.

STROBE

Certain black magic needed to put the CS8900 chip in the 16-bit mode.  Now

invoked automatically during boot time initialization.