User documentation for SDSL and IDSL debug & experimentation utilities

SDEBUG and IDEBUG utilities (for CR201s and CR201i, respectively) are a core

part of the Hack-o-Rocket project.

On the IDSL rocket IDEBUG allows one to peek and poke at MC145572 transceiver

registers, to observe the status and progress of the automatic line activation

and to configure the digital 2B+D interface for IDSL operation with any channel

combination.  On the SDSL rocket SDEBUG allows one to exercise all functions of

the ZipWire bitpump control library.  One can program the clock generator for a

number of different symbol/data rates (including some never supported by CM!),

configure a number of other libzw settings, perform HTU-C or HTU-R startup and

exercise a number of test modes.  Both IDEBUG and SDEBUG also allow one to

program MC68LC302's SCC1 (the one connected to the DSL transceiver) for HDLC

frame reception/transmission or for transparent bit stream capture.

NOTE: these are *debug* utilities and are intended for qualified hackers, not

for end users.  Using IDEBUG requires a good understanding of the workings of

the ISDN BRI U interface and of the MC145572 transceiver chip.  Using SDEBUG

requires a good understanding of the Brooktree/Rockwell/Conexant HDSL/SDSL

bitpump chips (Bt8970 is used on CR201s) and of the ZipWire control code.

(Required reading: bitpump chip datasheet and ZipWire Software User Guide.)

Relevant pages of the Open SDSL Connectivity Project website are also

recommended reading of course.

Running IDEBUG and SDEBUG

Each utility (IDEBUG and SDEBUG) is linked in two versions: run-from-RAM and

run-from-flash.  The run-from-RAM version is more convenient for development,

whereas the run-from-flash version is more convenient for stable use.  However,

SDEBUG is considerably larger than IDEBUG (the SDSL bitpump requires extensive

software babysitting in the form of libzw, whereas the ISDN transceiver chip is

mostly autonomous and IDEBUG implements only a little bit of passive monitoring

code), and this difference creates one additional consideration for the choice

of RAM-based or flash-based version.  The RAM-based version of SDEBUG takes up

almost all available RAM for its own code, thus no RAM is left for buffers.

Therefore, if one wants to perform SCC experiments on an SDSL rocket, the

flash-based version of SDEBUG must be used.  This problem does not occur with

IDEBUG however: the RAM-based version of IDEBUG is small enough to leave plenty

of RAM for buffers.  Both programs are still provided in both versions though

for the sake of symmetry.

The run-from-RAM version binary is called [is]debug-ram.bin and needs to be

loaded at address 600 via the monitor.  It then needs to be run via the CALL

command.  Example for IDEBUG:

HACK>TFTP tftp.server.ip.addr "idebug-ram.bin" 600

HACK>CA 600

CALL (CA) must be used rather than START (S) if you want to be able to return

to the monitor with the EXIT command.

The run-from-flash version of IDEBUG needs to be programmed into flash at

address 238000 (erase block 7); the run-from-flash version of SDEBUG needs to

be programmed into flash at address 228000 (erase blocks 5 and 6).  See

flash.doc for instructions on how to run the flash writer utility and program

flash.  Load IDEBUG into flash as follows:

RAM>ERA 7

RAM>TFTP tftp.server.ip.addr "idebug-flash.bin" 238000

Load SDEBUG into flash as follows:

RAM>ERA 5 6

RAM>TFTP tftp.server.ip.addr "sdebug-flash.bin" 228000

Then reboot the rocket:

RAM>REBOOT

You can now run the line debug utility from the Hack-o-Rocket monitor whenever

you want with a simple CALL command: CA 238000 for IDEBUG or CA 228000 for

SDEBUG.

When IDEBUG is run, it configures the SCP interface on the LC302 (controls the

MC145572 and used only on the IDSL rocket), sets up a handler for IRQ1 (the

MC145572 interrupt line), enables all 4 interrupts in the MC145572 and lowers

the IPL to 0.  The IRQ handler installed by IDEBUG simply reads the registers

whose change has caused an interrupt and prints messages on the console

indicating the status change.  The action of reading these registers also

clears the interrupt.  After the interrupt handler is installed and interrupts

are enabled, an IDEBUG> command prompt is presented.

When SDEBUG is run, it initializes libzw first.  This initialization includes

setting up a bitpump interrupt (IRQ1) handler which is part of libzw.  The IPL

is then lowered to 0 and an SDEBUG> command prompt is presented.  While waiting

for console command input, SDEBUG repeatedly calls _BtMain().

There is a major difference in how IDEBUG and SDEBUG work due to a corresponding

difference in the functionality implemented in the MC145572 and Bt8970 DSL

transceiver chips.  On the IDSL rocket the MC145572 chip is configured in

hardware to act as an NT1 (as opposed to LT) and it performs line activation

completely on its own.  If you want to make your CR201i activate the IDSL link,

simply plug the IDSL jack in and apply power.  There is no need to run IDEBUG,

in fact the CPU doesn't need to be functioning at all!  If you do run IDEBUG,

its function will consist solely of passive monitoring of the automatic link

bring-up.  As the ISDN transceiver chip goes through its internally controlled

activation sequence, it will generate a few interrupts reporting status changes.

IDEBUG will print messages on the console in response to these interrupts, so

you'll see what's happening.  But you don't need to actually *do* anything,

i.e., you don't need to give IDEBUG any commands.  The latter commands are

primarily useful for playing with the LC302 SCC connected to the MC145572 and

the digital 2B+D interface in between.

The SDSL rocket is different.  The Bt8970 bitpump won't utter a peep on the line

by itself, instead the work of making it talk on the line is done by the ZipWire

library (libzw) which is linked into SDEBUG.  Thus running SDEBUG is necessary

in order to play with the SDSL functionality.  When SDEBUG is run, it

initializes libzw and provides a command prompt at which the user can give

commands corresponding to libzw API calls.  Nothing will happen at first, one

must set a few libzw parameters with the SET command and then issue an ACTIVATE

command for the bitpump activation sequence to start.

The way SDEBUG is implemented (calling _BtMain() while waiting for console

command input) creates the appearance of multi-threaded operation.  The

interactive part of SDEBUG which displays the SDEBUG> prompt and interprets

user commands is one "thread" and _BtMain() is another "thread"; the two

"threads" appear to run in parallel, mixing their output on the console.  The

_BtMain() "background thread" is initially idle, but once it's set into action

by an ACTIVATE command or by one of the analog loopback commands (analog

loopback involves a local mini-activation), it will run on its own while you

can do other things in the interactive part of SDEBUG at the same time.

The same also holds for IDEBUG although to a lesser degree.  In IDEBUG the

"background thread" analogue is the ISDN interrupt handler, but all it does is

print a few short messages when certain status change events take place inside

the MC145572.

IDEBUG/SDEBUG commands

The following commands are common between IDEBUG and SDEBUG:

* DEPOSIT

* EXAMINE

* EXIT

* HEXDUMP

* SCC

* SET IPL

* SHOW IPL

Memory peek and pokes (EXAMINE and DEPOSIT), HEXDUMP, SET IPL and SHOW IPL work

exactly as in the ROM monitor, see mon302.doc.  EXIT is self-explanatory: exits

IDEBUG/SDEBUG, returning to whoever invoked the program (normally the monitor).

The program must have been invoked with a CALL in order for EXIT to work.  SCC

is described further in this document.

Although SET IPL and SHOW IPL commands are provided to manipulate the IPL

explicitly, IDEBUG and SDEBUG are designed to run at IPL 0.  The IPL is set to

0 on entry and EXIT sets it to 1 before returning so that the IRQ1 handler

installed by the program doesn't get invoked after exit (especially important

in the RAM-based version).

EXAMINE & DEPOSIT modifications in IDEBUG

In SDEBUG the EXAMINE (E) and DEPOSIT (D) commands work exactly as in MON302

(see mon302.doc) for memory peeks and pokes.  /P (physical address space) is

the only valid address space qualifier and need not be specified in SDEBUG.

In IDEBUG however, these commands have been extended to read and write MC145572

nibble and byte registers accessed via SCP, while some other features have been

sacrificed for simplicity.  The EXAMINE command can take the following forms in

IDEBUG:

Read a memory location (byte):		E/P/B memaddr

Read a memory location (word):		E/P/W memaddr

Read a memory location (longword):	E/P/L memaddr

Read an MC145572 nibble register:	E/N nrn

Read an MC145572 byte register:		E/B brn

where memaddr is an M68K memory address, nrn is a nibble register number (0

through 5) and brn is a byte register number (0 through F).  All addresses and

register numbers are hexadecimal.

DEPOSIT is similar: replace 'E' with 'D' and add a second argument with the data

value to be written (in hex).

The following features have been sacrificed in IDEBUG's implementation of

EXAMINE and DEPOSIT:

* '*', '+' and '-' addresses

* Reading and writing more than a single location at a time

* /N:n and /STEP:n qualifiers

* Quoted strings as arguments for byte-sized DEPOSIT

IDSL timeslot assigner configuration

The digital 2B+D interface coming out of the MC145572 chip is normally either

IDL or GCI, but it also supports a non-standard timeslot assigner (TSA) mode.

The power-up default configuration is IDL, but the CR201i hardware design is

such that the TSA mode must be configured and used for IDSL to work.  The

following web page describes the situation in greater detail:

http://ifctfvax.Harhan.ORG/OpenSDSL/CM/Rocket/idsl-if.html

The details of TSA operation are not described here, instead the MC145572 chip

documentation is required reading in order to use IDEBUG past this point.

IDEBUG provides a TSA command to configure the MC145572 timeslot assigner:

IDEBUG>TSA b1 b2 d

The three arguments correspond to B1, B2 and D channels in this order.  Each

argument is either a timeslot number (in hex) to be assigned to the respective

channel, or the keyword OFF to disable the respective channel.

A straightforward B1+B2+D IDSL-144 configuration can be set as follows:

IDEBUG>TSA 0 4 8

A B1+B2 IDSL-128 configuration would be as follows:

IDEBUG>TSA 0 4 OFF

If you feel like connecting the IDSL Hack-o-Rocket to a standard ISDN (not IDSL)

line, do this:

IDEBUG>TSA OFF OFF 0

The above command would select the D channel only.  You won't be able to get to

the B channels, but you'll be able to see the D channel packets with your own

eyes and maybe even send some.  (The CR201i hardware is really designed for

IDSL, not ISDN.  There is no way to access both the D channel and one or both B

channels while keeping them separate as would be required for standard ISDN.)

The TSA command also sets the SIMODE register in the MC68LC302 for the PCM

highway mode, i.e., it performs all preparations necessary for the IDSL bit

stream to appear at SCC1.

Libzw control commands in SDEBUG

The primary function of SDEBUG is to exercise the functionality and API of the

bitpump control library libzw.  Conexant's ZipWire Software User Guide is

required reading before this section can be understood.

SDEBUG provides a number of commands which correspond directly to _BtControl()

or _BtStatus() API calls as listed below.  _BtMain() is called repeatedly while

waiting for console command input.

Setting libzw parameters

The SET command has the following syntax:

SDEBUG>SET parameter value

The following parameters can be set:

Parameter keyword	_BtControl() setting	Value given as

AUTOTR			_AUTO_TIP_RING		0/1

DATARATE		_SYM_RATE		Decimal

LOST			_LOST_TIME_PERIOD	Decimal

NLEC			_OPERATE_NLEC		0/1

OTHERBT			_BT_OTHER_SIDE		0/1

RXDESCR			_RECEIVE_DESCR		0/1

QUAT			_FRAMER_FORMAT		Keyword: SIGNFIRST or MAGFIRST

STARTUPSEQ		_STARTUP_SEQ_SOURCE	Keyword: EXTERNAL or INTERNAL

SYMRATE			_SYM_RATE		Decimal

TERMINAL		_TERMINAL_TYPE		Keyword: HTUC or HTUR

TIMEOUT			_ACTIVATION_TIMEOUT	Keyword: 30SEC / 52SEC / VAR

TRREV			_REVERSE_TIP_RING	0/1

TXSCR			_TRANSMIT_SCR		0/1

Data rate / symbol rate setting

With 2B1Q transmission the data rate is always equal to twice the symbol rate,

i.e., the two are really the same setting.  You can either say SET DATARATE and

give the data rate in kbps, or say SET SYMRATE and give the symbol rate in

kilobaud, the resulting _BtControl() API call is the same.

Changing the symbol rate involves a complete reset of the bitpump, and the

current libzw implementation is unchanged from Brooktree in that this

_BtControl() API call resets all other software settings as well.  Therefore,

you must give the SET DATARATE or SET SYMRATE command before all other settings.

As the CR201s board has a Bt8970 bitpump on it rather than RS8973 and an

external ICD2053 clock generator is used for multirate operation, the set of

supported data/symbol rates is limited to those for which ICD2053 programming

is implemented.  This release supports all rates that have ever been used for

SDSL to our project's knowledge, see sdsl-data-rates for the list.

SDSL defaults

As explained in section 3.3 of Conexant's ZipWire Software User Guide, the

correct libzw settings are quite different for an HDSL system with a channel

unit versus an SDSL system without one.  Libzw has not yet been changed in this

regard and its internal defaults after initialization or after a symbol rate

change are the HDSL ones.  These settings expect an external startup sequence

source and thus aren't useful on the CR201s board where no such external source

exists.  However, SDEBUG provides a SET SDSLDEF command which sets more sensible

defaults for SDSL.  SET SDSLDEF is equivalent to:

SET STARTUPSEQ INTERNAL

SET TXSCR 1

SET RXDESCR 1

SET QUAT SIGNFIRST

SET OTHERBT 1

SET LOST 40

SET AUTOTR 1

SET TIMEOUT VARIABLE

Note that this command does not set the terminal type or the symbol/data rate.

The following command sequence should be used to set the complete libzw

configuration for SDSL ready for activation:

SET DATARATE desired_data_rate

SET TERMINAL HTUC (or HTUR as needed)

SET SDSLDEF

If you need to change some of the settings from SDEBUG's defaults (e.g., if you

need magnitude bit first), it's probably best to set the defaults first, then

change individual settings, e.g.:

SET SDSLDEF

SET QUAT MAGFIRST

Activation management commands

SDEBUG command	Abbreviation	Corresponding _BtControl() API call

ACTIVATE	ACT		_ACTIVATE

DEACTIVATE	DEA		_DEACTIVATE

EXTDATA		EXT		_TRANSMIT_EXT_DATA

Once the essential libzw settings for SDSL have been set (correct symbol rate,

HTU-C vs. HTU-R selection, internal startup sequence source), you can tell it

to proceed with activation:

SDEBUG>ACT

The ACTIVATE command is simply a _BtControl(0, _ACTIVATE, 0) call.  This API

call returns right away and the SDEBUG> prompt reappears.  The real work is

then done in _BtMain() which is called repeatedly as SDEBUG awaits further

console command input.

Libzw does not implement a high-level activation state machine.  If the

activation timeout interval (selected with SET TIMEOUT) expires before

successful activation is achieved, the only thing that happens is that a timeout

flag is set which can be checked with SHOW STATUS (_BtStatus() parameter

_STARTUP_STATUS).  In complete applications some higher level code would detect

this condition and respond with a deactivation request followed by restart, but

in SDEBUG you get to do it all manually.

After a successful activation using the internal startup sequence source the

bitpump will be transmitting 4-level scrambled 1s.  If you have configured the

SCC to transmit something more interesting, you can use the EXTDATA command to

tell the bitpump to start transmitting whatever is on its TDAT pin.  Note that

since the MC68x302 SCCs output continuous mark (1) until configured, EXTDATA is

effectively a no-op unless the Tx side of SCC1 is configured and enabled.

The DEACTIVATE (DEA) command turns the bitpump transmitter off immediately

regardless of prior activation state.

SDSL status display

SDEBUG's SHOW command provide access to information returned by _BtStatus() API

calls.  The following SHOW subcommands are implemented:

SHOW AAGC

_BtStatus() opcode: _AAGC_VALUE

Shows the current state of the AAGC control bits.

SHOW CONFIG

_BtStatus() opcode: _CONFIGURATION

This command shows the current state of several libzw configuration settings

obtainable via the _CONFIGURATION opcode of the _BtStatus() API.

SHOW IPL

Not related to the other SHOW subcommands; shows the current M68K IPL just like

in MON302.

SHOW LINE

Shows current line conditions.  Displays the information returned by

_BtStatus() opcodes _SLM, _DC_METER, _FELM, _NMR and _TIMING_RECOVERY_CONTROL

in a straightforward manner.

SHOW STATUS

_BtStatus() opcode: _STARTUP_STATUS

Shows current status of the activation process.

SHOW TXCAL

_BtStatus() opcode: _READ_TX_GAIN (_CALIBRATION)

Reads the Tx calibration register.

SHOW TXGAIN

_BtStatus() opcode: _READ_TX_GAIN (_GAIN)

Reads the Tx gain register.

SHOW VERSION

Displays version numbers returned by the _BtStatus() opcode _VERSION.

Operating the SCC

This section applies to both IDEBUG and SDEBUG; the SCC command is implemented

almost identically in both utilities.

The digital interface coming out of the DSL transceiver (Bt8970 bitpump or

MC145572 ISDN chip) is connected to the SCC1 unit in the MC68LC302 processor.

IDEBUG and SDEBUG allow you to configure SCC1 for one of the following two

modes:

* HDLC frame reception & transmission

* Transparent bit stream capture

HDLC operation

The SCC HDLC command sets SCC1 up for HDLC operation.  In IDEBUG one must

configure the 2B+D interface with the TSA command first before issuing this

command.  In SDEBUG the SCC command can be given at any time (NMSI doesn't need

any configuration and is always up), but the received bit stream will be garbage

until the SDSL link is brought fully up via the activation process.

Using the SCC command also requires some understanding of the M68K environment

one is running in.  Specifically, the user must pick a place in RAM where the

Rx buffers will be set up, i.e., specify a hex address; the picked location

must be in physically present RAM that isn't used by anything else.

The SCC HDLC command has the following syntax:

SCC HDLC start_address maxframe

The first argument is the starting address in RAM where the Rx buffers are to

be placed.  The second argument is the maximum Rx frame length the SCC should

be programmed for.  It includes the CRC octets, i.e., everything between the

flags, but not the opening and closing flags themselves.  Both arguments are

given in hex and must be even.

The SCC HDLC command sets up 8 consecutive Rx buffers starting at the specified

address; the length of each buffer is equal to the specified maximum frame

length.  The SCC parameter RAM is initialized per Motorola instructions and the

RxBD queue is set up to arrange the 8 buffers in a ring.  The SCC receiver is

enabled.

After the SCC HDLC command is given, whenever the SCC receives a frame (an RxBD

becomes marked non-empty), the received frame length, status and content in hex

are dumped on the console.  (SDEBUG implements a poll in the same custom getchar

routine which calls _BtMain() while waiting for console command input; IDEBUG

uses the SCC interrupt.)  The dump includes every octet between the flags; no

header structure is imposed.  The CRC octets (if any) are dumped just like

everything else; a flag in the frame status word indicates whether or not the

SCC detected good CRC at the end.

The SCC HDLC command also enables the SCC transmitter and sets up a ring of 8

TxBDs, however, all 8 are marked as not-ready, so initially nothing will happen

and the TxD1 pin will remain steady mark (1) as the command is given.  This

setup makes it easy for the user to transmit a frame: simply construct the

desired Tx frame in memory, then use DEPOSIT commands to stick its address in a

TxBD and mark it as ready.  The frame will then be instantly transmitted.

The HDLC operation started by SCC HDLC is stopped by the SCC STOP command.

Issue this command before deactivating the SDSL link, or else the SCC will

start receiving garbage and that garbage will start pouring on the console.

SCC STOP restores the SCC to its state upon entry to IDEBUG or SDEBUG.  This

operation is also performed automatically when you exit the program with the

EXIT command.  However, you must issue an explicit SCC STOP before you can

issue another SCC HDLC or SCC CAPTURE command.

Bit stream capture

IDEBUG and SDEBUG can perform a bit stream capture by putting the SCC in the

Motorola-supported transparent mode (BISYNC mode with NTSYN and EXSYN bits set

in SCM).  This is done with the SCC CAPTURE command, which has the same syntax

as SCC HDLC:

SCC CAPTURE buffer_addr buffer_len

The same caveats apply as with SCC HDLC: the user must pick a sensible buffer

location in RAM and the command must not be given until the SCC is receiving a

valid bit stream (precede with TSA on IDSL or activation sequence on SDSL).

However, only one buffer is set up instead of 8 as the capture is a one-shot

operation.

Capture initiation

The SCC CAPTURE command sets the EXSYN bit in SCM so that the capture can be

performed without *any* a priori knowledge of the bit stream format.  (The other

mode would require knowing some 16-bit pattern which occurs in the bit stream.)

On the IDSL rocket the interface between the SCC and the ISDN chip is PCM

highway, and the MC68x302 implementation is such that the capture will begin

immediately as soon as the SCC CAPTURE command is given and the first bit stored

in the buffer will be the first bit of a TSA frame.  Each TSA frame has a fixed

mapping to an 18-bit 2B+D block on the wire, thus the captured data has a known

phase relationship to the ISDN channel structure.  This is very convenient.

On the SDSL rocket however the situation is more complicated due to the SCC

interface being NMSI.  The MC68x302 design is such that transparent mode

reception over NMSI with EXSYN set doesn't start until the CD1 pin is sampled

low; the first bit stored in the buffer will be the bit just before.  As

Murthy's Law would have it, the CD1 pin is pulled up on an unmodified CR201s.

Therefore, if you issue an SCC CAPTURE command on an SDSL rocket without some

additional hardware hacking, the capture will never commence.

The solution is to do one of the following:

* Issue the SCC CAPTURE command on a rocket with its cover off, then manually

ground pin 50 (CD1) on the MC68LC302 for a moment, being careful not to short

anything else in the process.

* Make a permanent modification to the board to connect this pin to ground, or

even better to the QCLK output from the bitpump (Bt8970 pin 87).

Connecting CD1 to QCLK is even better than grounding it because this way the

bit stream capture will have a known quat alignment, specifically it will always

start in the middle of a quat.  This modification should have no effect on SCC

operation in any other modes as the SCC can be (and is) configured to ignore the

CD signal in all modes except totally transparent.

Either way some hardware hacking skill is required.

Once the capture is initiated, it will proceed to fill the entire buffer up to

the specified length no matter what.  The MC68x302 default is to pack received

bits into bytes least significant bit first; if you want the opposite bit

packing order (MSB first), specify an /MSB qualifier in the SCC CAPTURE command.

You can check the status of the capture operation by reading the word at address

700400 (E/P/W 700400).  This is the flags word of the RxBD set up for the

capture buffer.  The SCC CAPTURE command sets it to C000.  If the capture is

held off (CD1 is high on CR201s), the word will remain C000.  However, once the

SCC performs the capture and fills the buffer, the word should change to 4000.

Once the word at 700400 reads as 4000, the capture is done and nothing more

will happen.  Unlike SCC HDLC, the SCC CAPTURE command doesn't enable any

interrupts or any poll function.  However, you should still clean the SCC up

with an SCC STOP command; doing so is required before you can use the SCC for

anything else (another capture or HDLC).

The captured booty can be dumped with HEXDUMP.

SDSL bitpump test modes

SDEBUG includes a number of commands which exercise libzw functions related to

bitpump test modes.

The following test modes are describes in greater detail in section 8.3 of

Conexant's ZipWire Software User Guide:

SDEBUG command		Libzw test mode			Description

LOOP EXTANALOG		_ANALOG_LOOPBACK		External analog lb

LOOP INTANALOG		_INTERNAL_ANALOG_LOOPBACK	Internal analog lb

LOOP ISOANALOG		_ISOLATED_ANALOG_LOOPBACK	Isolated analog lb

LOOP NEARDIG		_NEAR_LOOPBACK			Near digital lb

LOOP FARDIG		_FAR_LOOPBACK			Far digital lb

TEST 2LSCR		_TWO_LEVEL_SCR			Tx 2-level scrambled 1s

TEST 4LSCR		_FOUR_LEVEL_SCR			Tx 4-level scrambled 1s

TEST ALT1S		_ALTERNATING_SYMBOLS_1		Tx alternating +1/-1 *

TEST ALT3S		_ALTERNATING_SYMBOLS_3		Tx alternating +3/-3 *

TEST ISOM1		_ISOLATED_PULSE_MINUS1		Tx isolated -1 pulses

TEST ISOM3		_ISOLATED_PULSE_MINUS3		Tx isolated -3 pulses

TEST ISOP1		_ISOLATED_PULSE_PLUS1		Tx isolated +1 pulses

TEST ISOP3		_ISOLATED_PULSE_PLUS3		Tx isolated +3 pulses

TEST VCXOMIN		_VCXO_MIN			Set min VCXO freq

TEST VCXOMAX		_VCXO_MAX			Set max VCXO freq

TEST VCXONOMINAL	_VCXO_NOMINAL			Set nominal VCXO freq

TEST EXIT		_EXIT_TEST_MODE			Exit test modes

* This test mode (alternating symbols) is not supported on the Bt8970 bitpump.

(It's an RS8973 feature.)

SELFTEST command

This command exercises libzw's _SelfTest() function.  It was added to test for

the potential problem described in section 5.9.2 of Conexant's ZipWire Software

User Guide.

BER meter support

SDEBUG provides the following commands to support the BER meter test:

BER START

BER STOP

BER STATUS

Reading section 8.4 of Conexant's ZipWire Software User Guide should make their

function obvious.

ERLE test

This test is described in section 8.5 of Conexant's ZipWire Software User Guide.

It is intended strictly for hardware hackers, specifically those who build their

own SDSL hardware.  The implementation of this test on the Hack-o-Rocket allows

comparison between CR201s and other hardware in terms of hybrid and echo

canceler performance.

Set the desired symbol rate with SET SYMRATE or SET DATARATE, then run the ERLE

test, e.g.:

SDEBUG>SET DATARATE 1168

SDEBUG>ERLE

The ERLE command defaults are AAGC = 12 dB, 4-level, NLEC off as recommended by

Conexant.  They can be overridden with /AAGC:x, /LEVEL:x and /NLEC:x qualifiers.

The qualifier value is the AAGC setting in decimal, 2 or 4, 0 or 1,

respectively.

The canonical form for ERLE test results is floating point, but the latter is

not included in the Hack-o-Rocket M68K code suite.  Instead SDEBUG prints the

integer results returned from libzw; capture the console output in a log and

use the erle-float program from the osdcu-unixhost-tools package to post-process

it on a UNIX host.